3GPP TSG-RAN WG2 Meeting #131bis	R2-2507271

Prague, Czech Republic, 13th –17th October, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining User Plane issues of NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

The MAC Rapporteur has listed two open issues on the MAC spec as follows:

MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587) MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

In the RAN1#122 meeting, RAN1 has sent an LS to RAN2 and replied RAN2 that there is no proper place to capture the impact on f-id in RAN1 specification [1]:

RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id. RAN1 kindly requests RAN2 to take the above into account, as well as the below RAN1 agreements, with regards to the impact on f_id of valid RO in TS38.321: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

In this contribution, we will discuss the remaining issues and provide the corresponding TP.

# Discussion

MAC issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

The calculation of RA-RNTI is related to the PRACH occasion (time and frequency resource) where UE sends the preamble. It is calculated based on the following formula and specified in TS 38.321 [2]:

Cited from TS 38.321 The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

For the additional PRACH resources, RAN1 has agreed that when an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping. And the maximum FDM RO is not larger than 8. The above restriction can guarantee the legacy RO is not impacted. In RAN2#130 meeting, RAN2 has agreed that additional RO, when the additional RO and the legacy RO are in the same time domain, the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDM-ed legacy ROs. When the additional RO and the legacy RO are in the different time domain, the calculation of RA-RNTI for additional is the same as legacy ROs and no spec impact is foreseen. When the additional RO and the legacy RO are in the same time and frequency domain, the additional ROs are invalid and no spec impact is foreseen.

Since RAN1 has no concern on the calculation of additional RO’s RA-RNTI and cannot capture this solution in RAN1’s spec, RAN2 could capture the solution in RAN2 MAC spec. For example, we could add a note in current spec to capture the specific FDM index determination of the additional ROs. There is no need to add more procedural text on how to determine the additional ROs FDM index. A note in the RA-RNTI calculation part is sufficient.

- [MAC-1] Add a note in MAC to capture specific FDM index determination of the additional ROs. Adopt the TP in the Annex.

MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

In the existing spec, RedCap UEs could be configured with Msg1 resources for initialUplinkBWP-RedCap in si-RequestConfigRedCap IE, by which RedCap UEs can request SI-messages. The SI request procedure is specified as follows:

Cited from TS 38.331 1> else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigRedCap-MSG1-Repetition and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 and criteria to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigRedCap-MSG1-Repetition are met: 2> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in si-RequestConfigRedCap-MSG1-Repetition corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 2> if acknowledgement for SI request is received from lower layers: 3> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately; 1> else if SIB1 includes si-SchedulingInfo containing si-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigSUL corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 2> if acknowledgement for SI request is received from lower layers: 3> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately; 1> else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigRedCap and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 2> if acknowledgement for SI request is received from lower layers: 3> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately; 1> else: 2> if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink as defined in TS 38.321 [3], clause 5.1.1 and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met; or 2> if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink as defined in TS 38.321 [3], clause 5.1.1 and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met: 3> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in si-RequestConfigMSG1-Repetition corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 3> if acknowledgement for SI request is received from lower layers: 4> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately; 2> else if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met; or 2> if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met: 3> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfig corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 3> if acknowledgement for SI request is received from lower layers: 4> acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately; Unchanged Text omitted SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } SchedulingInfo ::= SEQUENCE { si-BroadcastStatus ENUMERATED {broadcasting, notBroadcasting}, si-Periodicity ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-Mapping } SI-SchedulingInfo-v1700 ::= SEQUENCE { schedulingInfoList2-r17 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17, dummy SI-RequestConfig OPTIONAL } SI-SchedulingInfo-v1740 ::= SEQUENCE { si-RequestConfigRedCap-r17 SI-RequestConfig OPTIONAL -- Cond REDCAP-MSG-1 } SI-SchedulingInfo-v1800 ::= SEQUENCE { si-RequestConfigMSG1-Repetition-r18 SI-RequestConfigRepetition-r18 OPTIONAL, -- Cond MSG-1 si-RequestConfigRedCap-MSG1-Repetition-r18 SI-RequestConfigRepetition-r18 OPTIONAL, -- Cond REDCAP-MSG-1 si-RequestConfigSUL-MSG1-Repetition-r18 SI-RequestConfigRepetition-r18 OPTIONAL -- Cond SUL-MSG-1 }

From the above SI request procedures and configurations, we can observe that when the RedCap specific initial uplink BWP and RedCap specific SI request resource are not configured, RedCap UEs can use the common SI request resource to request SI. When the RedCap specific initial uplink BWP and RedCap specific SI request resource is configured, RedCap UEs can use RedCap specific SI request resource to request SI.

The RedCap specific SI request resource is only applied to a normal UL carrier. When a RedCap UE initiates the SI request on SUL, the common SI request resource can be utilized.

Observation 1: When the RedCap specific initial uplink BWP and RedCap specific SI request resource are not configured, RedCap UEs can use the common SI request resource to request SI. When the RedCap specific initial uplink BWP and RedCap specific SI request resource are configured, RedCap UEs can use RedCap specific SI request resource to request SI.

In the MAC spec, the response of SI request of PDCCH monitoring behaviour is specified as follows:

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if initialUplinkBWP-RedCap is configured for the selected carrier: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap. 1> else: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP. 1> if initialDownlinkBWP-RedCap is configured: 2> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

When a RedCap UE initiates the SI request on the SUL carrier or if RedCap specific initial downlink BWP is not configured, the RedCap UE should monitor the PDCCH on the common downlink BWP. When the SI request is initiated by using the RedCap specific SI request resource on the NUL carrier, RedCap UEs should monitor the PDCCH on the RedCap specific BWP.

Regarding the question in the open issue 2, when the NW receives the SI request initiated by using the RedCap specific SI request resource on the NUL carrier, the NW could respond a corresponding PDCCH on the RedCap specific downlink BWP. There is no ambiguity for NW/UE to send/receive the PDCCH on the RedCap specific BWP.

To achieve this, we should introduce RedCap specific OD-SIB1 request configuration and specify the corresponding OD-SIB1 request procedure following the OSI request mechanism in TS 38.331. If not, there is no need for the NW/UE to send/receive the PDCCH on the RedCap specific BWP. Monitoring the PDCCH on the common BWP is also supported to RedCap UEs if the common BWP is applied to RedCap UEs.

- [MAC-2] Introduce a RedCap specific OD-SIB1 request configuration and specify the corresponding OD-SIB1 request procedure, by which the NW/UE could know whether to send/receive the PDCCH on the RedCap specific BWP.

# Conclusion

In this contribution, we discussed User Plane issues of NES, and made the following observations and proposals:

Proposal 1:	[MAC-1] Add a note in MAC to capture specific FDM index determination of the additional ROs. Adopt the TP in the Annex.

Observation 1: When the RedCap specific initial uplink BWP and RedCap specific SI request resource are not configured, RedCap UEs can use the common SI request resource to request SI. When the RedCap specific initial uplink BWP and RedCap specific SI request resource are configured, RedCap UEs can use RedCap specific SI request resource to request SI.

Proposal 2:	[MAC-2] Introduce a RedCap specific OD-SIB1 request configuration and specify the corresponding OD-SIB1 request procedure, by which the NW/UE could know whether to send/receive the PDCCH on the RedCap specific BWP.

# References

- R1-2506587 Reply LS on RA-RNTI for PRACH adaptation

- TS 38.321 v18.6.0

# Annex: TP for 38.321

<<<<<<<<<<<<<<<<<<<<<<<<<<<<unchanged text omitted>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 5.1.3	Random Access Preamble transmission

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if LBT failure indication was not received from lower layers for the last Random Access Preamble transmission; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission; and

1>	if the Random Access procedure is not initiated by the PDCCH order for an LTM candidate cell:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell as preamble re-transmission; and

1>	if the PDCCH order indicates the same LTM candidate cell and the same SSB as the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

1>	except for contention-free Random Access Preamble for beam failure recovery request and contention-free Random Access Preamble triggered by a PDCCH order for an LTM candidate cell, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX, and PREAMBLE_RECEIVED_TARGET_POWER.

1>	if the Random Access Procedure is triggered by a PDCCH order for an LTM candidate cell:

2>	consider this Random Access procedure completed.

1>	if LBT failure indication is received from lower layers for this Random Access Preamble transmission:

2>	if lbt-FailureRecoveryConfig is configured:

3>	perform the Random Access Resource selection procedure (see clause 5.1.2).

2>	else:

3>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

3>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

4>	if the Random Access Preamble is transmitted on the SpCell:

5>	indicate a Random Access problem to upper layers;

5>	if this Random Access procedure was triggered for SI request:

6>	consider the Random Access procedure unsuccessfully completed.

4>	else if the Random Access Preamble is transmitted on an SCell:

5>	consider the Random Access procedure unsuccessfully completed.

3>	if the Random Access procedure is not completed:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2).

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

Note X: If an RO selected for preamble transmission is configured by addlRACH-Config-Adapt, the determination of f_id for the RO is FDM index in addlRACH-Config-Adapt plus the number of FDM-ed index not in addlRACH-Config-Adapt.