3GPP TSG-RAN WG2 Meeting #131bis	R2-2507286

Prague, Czechia, 13th – 19th Oct, 2025

Agenda item:	8.9.3

Source:	Samsung

Title:	Various MAC Corrections on CB-Msg3-EDT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where Capacity enhancements for uplink:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In this contribution, we provide some correction on MAC.

# Discussion

## CB-RNTI for Msg3

In the last meeting, the final agreements on CB-RNTI were taken for the basic design:

13.	CB-RNTI formula:

For eMTC

-	floor (start SFN_id of Tx window/WP) modulo (32) + 32*CE_level + 2401

-	the CE_level is the selected enhanced coverage level (0 <= CE_level < 2)

For NB-IoT

-	floor (start SFN_id of Tx window/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097

-	the CE_level is the selected enhanced coverage level (0 <= CE_level < 3)

-	the carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

WP is the configured CB-MSG3 transmission window periodicity in unit of 10ms.

However, what has been missed in the implementation so far, is that the CB-RNTI should also be used for CB-Msg3 scrambling. This means that the CB-RNTI needs to be determined not after the CB-Msg3 transmission, but after the CB-Msg3 resource have been determined.

This needs to be captured in MAC specifications as otherwise PHY-related specifications may need to capture how the RNTI is determined based on the resource-based formula as RAN2 has already captured in MAC.

Proposal 1: Confirm CB-RNTI is also used for CB-Msg3 scrambling and that it is captured in MAC specifications.

## Clarification on ending CB-Msg3-EDT procedures

In RAN2#130, the following was agreed:

3.	A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

One part of this has been implemented in 36.321 as follows:

-------------------------------------- R2-2506493 --------------------------------------

-	if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s):

... OMITTED ...

-	if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4:

-	indicate the successful completion of CB-Msg3-EDT procedure to the upper layers.

-------------------------------------- R2-2506493 --------------------------------------

The “indicate the successful completion of CB-Msg3-EDT procedure to the upper layers.” is only used for the specific case where the CB-Msg3-EDT procedure is completed without an RRC message, for the CP solution. This may be confusing to an implementer, as it seems to indicate that the only way to successfully complete the procedure is to not include a C-RNTI nor a MAC SDU. Therefore we think it would be better to use language to indicate that this is a special condition, such as “successful completion of CB-Msg3-EDT without RRC message”.

Proposal 2: On completing CB-Msg3-EDT without RRC message: clarify that successful completion is for the specific case of no RRC message received.

Proposal 3: Agree text proposal in Annex A.

## CQI and AS RAI reporting in Msg3

However, for reporting CQI and AS RAI in Msg3 for eMTC, the condition for triggering a Msg3 DCQR is that mpdcch-CQI-Reporting is configured and that an uplink grant has been received on an RA-RNTI.

This specific text has been introduced to support reporting CQI and AS RAI in Msg3 only for initial access purposes. The current text clearly does not support reporting in the CQI or AS RAI in CB-Msg3. Therefore, we believe that a further condition would be that the UE has triggered the CB-Msg3-EDT procedures.

Proposal 4: To support CQI (DCQR) and AS RAI reporting in CB-Msg3-EDT, add another condition in MAC to “Msg3 DCQR”.

Proposal 5: Agree text proposal in Annex B.

# Conclusion

In this contribution we discussed MAC corrections:

Proposal 1: Confirm CB-RNTI is also used for CB-Msg3 scrambling and that it is captured in MAC specifications.

Proposal 2: On completing CB-Msg3-EDT without RRC message: clarify that successful completion is for the specific case of no RRC message received.

Proposal 3: Agree text proposal in Annex A.

Proposal 4: To support CQI (DCQR) and AS RAI reporting in CB-Msg3-EDT, add another condition in MAC to “Msg3 DCQR”.

Proposal 5: Agree text proposal in Annex B.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek), Mediatek, RAN2#129bis, Wuhan, China, April 2025.

# Annex A

-------------------------------------- R2-2506493 --------------------------------------

#### 5.1x.3	CB-Msg4 reception

Once all the CB-Msg3s of a CB-Msg3 transmission window are transmitted, the MAC entity shall monitor the PDCCH of the SpCell, while CB-Msg3ResponseTimer is running, for CB-Msg4(s) identified by the CB-RNTI defined below.

For BL UEs and UEs in CE mode A, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 2401

Where:

-	SFN_id is the first SFN of the selected CB-Msg3 transmission window.

-	WP is the configured CB-Msg3 transmission window periodicity in unit of 10ms provided by windowPeriodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 2).

For NB-IoT UEs, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = floor (SFN_id/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097

Where:

-	SFN_id is the first SFN of the selected CB-Msg3 transmission window.

-	WP is the configured CB-Msg3 transmission window periodicity in unit of 10ms provided by windowPeriodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 3).

-	carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

After the CB-Msg3 transmission, the MAC entity shall:

-	start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT;

-	monitor the PDCCH while CB-Msg3ResponseTimer is running;

-	if notification of a reception of a PDCCH transmission is received from lower layers; and

-	if the CB-Msg4 is successfully decoded:

-	if the CB-Msg4 contains a CB Backoff Indicator subheader:

-	set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else:

-	set the CB-Msg3-EDT backoff parameter value to 0 ms.

-	if the CB-Msg4 contains one or more MAC CMRs; and

-	if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s):

-	consider CMR reception successful;

-	discard the CB-RNTI;

-	stop CB-Msg3ResponseTimer if running;

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	if the corresponding CMR contains a Timing Advance Command field:

-	process the received Timing Advance Command (see clause 5.2).

-	start the timeAlignmentTimer associated with this TAG;

-	if the corresponding CMR contains a C-RNTI field:

-	set the C-RNTI to the value of the C-RNTI field.

-	finish the disassembly and demultiplexing of the MAC PDU;

-	consider CB-Msg3 transmission successfully completed;

-	if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4:

-	indicate the successful completion of CB-Msg3-EDT procedure without an RRC message to the upper layers.

NOTE:	The UE can receive CB-Msg4 after the CB-Msg3ResponseTimer expired if the corresponding PDCCH is received before the CB-Msg3ResponseTimer expired.

-------------------------------------- R2-2506493 --------------------------------------

# Annex B

-------------------------------------- R2-2506493 --------------------------------------

## 5.25 Transmission of Downlink Channel Quality Report

The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur:

-	DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR";

-	for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR".

If any type of DCQR has been triggered:

-	start performing DL channel quality measurements according to TS 36.133 [9].

If "Regular DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	cancel the triggered "Regular DCQR".

If "Msg3 DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI or if the uplink grant is for CB-Msg3-EDT:

-	if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	else if the uplink grant is not for EDT:

-	if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1;

-	cancel the triggered "Msg3 DCQR".

-------------------------------------- 36.321 V18.4.0 --------------------------------------