3GPP TSG-RAN WG2 Meeting #131bis	R2-2507321

Prague, Czech Republic, Oct. 13th-17th 2025

Agenda item:	10.3.2 (Control plane)

Source:	Sony

Title:	Discussion on 6GR Rel-20 control plane aspects

Document for:	Discussion and Decision

# 1 Introduction

At RANP#108, the new SID on 6G radio study RP-251881 was agreed [1], with objective on radio interface protocol architecture and procedures for 6GR as follows.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this paper, we discuss the control plane aspects e.g., initial access, BWP management, RRC states, system information and paging to be addressed in RAN2 6GR study.

# 2 Discussion

## 2.1 Common initial access for all device types

Support of different device types requires the base station to know the capabilities of these devices earlier in the process of initial access procedure. This has led to a common solution of RACH resource partitioning and broadcasting these partitioning of resources. RACH partitioning was introduced for features like Redcap, slicing, SBFD, SDT etc. Resource partitioning is not a good practice from resource utilisation and feature implementation point of view and should be avoided in our view.

In order to avoid any early recognition of UE capabilities during initial access, common initial access procedure should be designed, and all device types should be able to perform initial access to any base station. UE capability for such diverse device types should be signalled later. UE may be able to indicate its capability either in MSG3 or MSG5 or later.

Proposal 1	RAN2 to agree that common initial access procedure should be supported for different device types. This may be achieved without RACH resource partitioning.

## 2.2 BWP management

In terms of resources, initial access should be performed on initial BWP. We think that initial and dedicated BWPs should be supported for power saving purposes. For TDD with SBFD, a simple configuration and switching is necessary at the UE such as treating the subband as either UL BWP or DL BWP during transmission or reception. The need for configuring 4 BWPs should be discussed further and any simplification of ASN.1 should be studied for BWP related parts. Also, the need for simultaneous activation of more than one BWP should be discussed as this was never used in NR.

Proposal 2	Initial and dedicated BWP should be supported. Need for multiple dedicated BWPs and simultaneous activation of more than one dedicated BWP should be studied further.

## 2.3 RRC states

All NR RRC states: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED should be supported as baseline. Inactive state was not implemented in NR but with features like SDT (Small Data Transmission), we think inactive state can offer better results for UE and network energy efficiency and NR inactive state should be the baseline.

Proposal 3	RAN2 to study all three RRC states: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED for 6GR.

## 2.4 System information

In Rel-15 NR, Other SI which includes all SIBs not broadcast in the Minimum SI can be broadcast on-demand on DL-SCH upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED. In Rel-19 NR, on-demand SIB1 was introduced, where network can broadcast SIB1 after receiving OD-SIB1 request from UEs in RRC_IDLE, RRC_INACTIVE or some RRC_CONNECTED UEs. The reduction of always-on SIs broadcast in the downlink data channel would be beneficial to the network energy saving performance, therefore NR on-demand SI/SIB1 should be the baseline for 6GR system information handling.

Proposal 4	NR on-demand SI/SIB1 is the baseline for 6GR system information handling.

In Rel-19 NES OD-SIB1, it is required to evaluate the impact to the legacy UEs and the negative impact to the legacy UEs should be minimized. That’s one of the reasons that in Rel-19 on-demand SIB1, only Case 2 where UE obtains OD-SIB1 request configuration from Cell A and transmits OD-SIB1 request to NES Cell and receives OD-SIB1 from NES Cell, is specified. But the limitation because of the co-existence with legacy UEs is not applicable to 6GR anymore. We support to further study the other cases which has been discussed during Rel-19 NES OD-SIB1 SI phase in 6GR, as we see the needs to reduce the signalling exchanges among different cells as well as to enable more flexible NES cell deployment. Furthermore, in Rel-19 OD-SIB1, no modification of SSB is assumed. The design of SSB may be discussed in 6GR and its impact to OD-SIB1 as well as the potential combination of SSB modification and OD-SIB1 should be further studied in RAN2 after RAN1 makes any progress on the study of SSB design.

Proposal 5	Other on-demand SIB1 cases should be further studied in 6GR in order to reduce the signalling exchange among cells as well as to enable more flexible NES cell deployment.

## 2.5 Paging

Paging as performed in NR RRC_IDLE and RRC_INACTIVE states is still power consuming and complex. We think the paging concept can be reviewed and improved when entering 6GR to support better energy efficiency in the UE.

Paging or paging indication using the LP-WUS signaling should be studied in RAN as this can improve the power consumption performance for all UEs. The power consumption of a LP-WUR receiver in the UE is substantially lower than the normal UE receiver, as the receiver does not need accurate synchronization and the active time of the UE could also be reduced. When the paging can be performed using the LP-WUR, it should also be studied whether the LP-WUR can be used for mobility measurements in RRC_IDLE state.

Proposal 6	Study usage and support of using LP-WUR for mobility and reception of paging indications.

# 3 Conclusion

In this contribution, our proposals are as follows:

Proposal 1	RAN2 to agree that common initial access procedure should be supported for different device types. This may be achieved without RACH resource partitioning.

Proposal 2	Initial and dedicated BWP should be supported. Need for multiple dedicated BWPs and simultaneous activation of more than one dedicated BWP should be studied further.

Proposal 3	RAN2 to study all three RRC states: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED for 6GR.

Proposal 4	NR on-demand SI/SIB1 is the baseline for 6GR system information handling.

Proposal 5	Other on-demand SIB1 cases should be further studied in 6GR in order to reduce the signalling exchange among cells as well as to enable more flexible NES cell deployment.

Proposal 6	Study usage and support of using LP-WUR for mobility and reception of paging indications.

# 4 References

- RP-251881. New SID: Study on 6G Radio.