3GPP TSG-RAN WG2 Meeting #131bis	R2-2507386

Prague, Czechia, 13th – 17th of October 2025

Agenda item:	7.0.2.22

Source:	Nokia

Title:	On inter-DU Layer 2 Reset in LTM

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and Decision

# 1	Introduction

This paper presents our view regarding the recent LS that RAN WG3 [1] has sent to RAN WG2 regarding the L2 reset during LTM.

# 2	Discussion

In the LS RAN3 [1] has stated the following:

1. Overall Description: RAN3 has discussed the handling of L2 reset in the context of inter-DU LTM for Rel-18. The common understanding within RAN3 is that L2 reset is always performed when the UE switches between the DUs, i.e., inter-DU LTM. 2. To RAN2 ACTION: RAN3 kindly requests RAN2 to confirm whether any issue is foreseen with the above common understanding.

As can be noticed, RAN3 has discussed about the principles of Release 18 LTM. More specifically, whether Layer 2 protocols are reset when the UE performs an inter-DU LTM.

Observation 1: RAN3 asks in R3-255907 whether Layer 2 protocols are reset when the UE performs an inter-DU LTM.

RAN2 has spent countless hours in Rel-18 discussing what and when shall be reset when the UE performs an LTM handover. It was questionable if the gains provided by keeping L2 protocols are sufficient to enable such functionality. Eventually, it was decided to introduce parameters ltm-NoResetID (defined per each LTM-Candidate) and LTM-ServingCellNoResetID (stored within UE’s variables) which are used to control if for this particular LTM candidate the UE shall perform L2 reset.

Observation 2: Release 18 LTM introduced parameters ltm-NoResetID (defined per each LTM-Candidate) and ltm-ServingCellNoResetID (stored within UE’s variables) which are used to control if for this particular LTM candidate the UE shall perform an L2 reset.

It is commonly understood in RAN2 that avoiding the L2 reset is actually feasible and done only when LTM is executed between source and the target cells belonging to the same DU. Thus, in inter-DU LTM scenario the UE would always reset Layer 2 protocols.

Observation 3: Avoiding L2 reset is feasible and done only when LTM is executed between source and the target cells belonging to the same DU.  In inter-DU LTM scenario the UE would always reset Layer 2 protocols.

The aforementioned observation should be shared with RAN3 in our response LS. A draft is available in the Annex.

Proposal 1: RAN2 responds to RAN3 and indicates avoiding L2 reset is feasible and done only when LTM is executed between source and the target cell belonging to the same DU.

Proposal 2: RAN2 considers the draft response LS in the Annex.

# 3 	Conclusion

In this paper we have briefly summarized the aspects of Layer 2 protocol reset when inter-DU LTM is executed. The following observations and proposals have been made:

Observation 1: RAN3 asks in R3-255907 whether Layer 2 protocols are reset when the UE performs an inter-DU LTM.

Observation 2: Release 18 LTM introduced parameters ltm-NoResetID (defined per each LTM-Candidate) and ltm-ServingCellNoResetID (stored within UE’s variables) which are used to control if for this particular LTM candidate the UE shall perform an L2 reset.

Observation 3: Avoiding L2 reset is feasible and done only when LTM is executed between source and the target cells belonging to the same DU.  In inter-DU LTM scenario the UE would always reset Layer 2 protocols.

Proposal 1: RAN2 responds to RAN3 and indicates avoiding L2 reset is feasible and done only when LTM is executed between source and the target cell belonging to the same DU.

Proposal 2: RAN2 considers the draft response LS in the Annex.

# References

- R3-255907 LS on the handling of inter-DU L2 reset for LTM 3GPP TSG-RAN WG3 Meeting #129 Bengaluru, India, 25 – 29 August, 2025

# Annex – Draft Response LS

3GPP TSG-RAN WG2 Meeting #131bis	R2-250xxxx

Prague, Czech Republic, 13 – 17 October 2025

Title:	Response LS on the handling of inter-DU L2 reset for LTM

Response to:	R3-255907

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

#### Name:	Jedrzej Stanczak

E-mail Address:	jedrzej.stanczak@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN3 for their LS in R3-255907 on L2 reset in LTM. RAN2 has briefly discussed the topic and would like to confirm that avoiding L2 reset is actually feasible and done only when LTM is executed between source and the target cell belonging to the same DU.

2. Actions:

To RAN WG3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take our response into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#132	from 2025-11-17	to 2025-11-21		Dallas, USA

RAN2#133	from 2026-02-09	to 2026-02-13		Stor-Göteborg , SE