3GPP RAN WG2 Meeting #131bis	R2-2507432

# Prague, Czechia,  October 13th – 17th, 2025

Agenda Item:	10.4

Source:	InterDigital, Inc.

Title:	Connected Mobility for 6GR

Document for:	Discussion

# Introduction

The study on 6G contains the following mobility related objective.

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

This contribution outlines initial views on mobility in 6G, with a focus on Connected mode operation. NR introduced multiple mobility schemes (L3 HO, CHO, LTM, DAPS), which together created signaling overhead, redundancy, and implementation complexity. In 6G, mobility should be based on a single design , delivering robust and low-latency procedures suitable for diverse services (XR, AI, mission-critical).

# Discussion

Mobility in 6G Connected mode should be built on a single design for the mobility framework that supports L3-based mobility, LTM and CHO. By consolidating existing mechanisms, streamlining measurements and reconfiguration, and enabling low-latency procedures, 6G can provide robust and efficient mobility suitable for diverse services.

## 2.1	6G Mobility Design Considerations

Connected mode mobility for 6G should be designed from the start as a lean, single design that minimizes signaling while ensuring robustness.

The following drawbacks from NR should be considered:

- Fragmentation - Mobility mechanisms such as L3 HO, CHO, and LTM were developed in parallel. Their coexistence created overlapping procedures, duplicated signaling, and heavy configuration overhead.

- Latency and interruption - In NR, handovers can still cause short service interruptions. For future services such as XR or URLLC, further reductions in latency and interruption time will be important to maintain user experience.

- Measurement duplication - L1 and L3 measurements are configured separately, resulting in duplicated reference signals and higher UE measurement burden.

- Complexity - Features such as CHO and DAPS introduced additional signaling to maintain backward compatibility. A simplified baseline in 6G can help reduce signaling load.

- Limited Deployment - Some advanced mobility features in NR have seen limited deployment so far, partly due to signaling overhead and the need for cross-vendor interoperability.

6G offers an opportunity to streamline design for wider adoption and the following design targets should be considered to achieve that:

- Simplification: Reduces signaling overhead and eases both network and device implementation.

- Robustness: A single design supporting L3/LTM improves mobility success rates and reduces handover failures.

- Low latency: RACH-less handover and early TA acquisition enable seamless mobility with minimal service interruption.

- Modular configuration: Stored and reusable RRC profiles or configuration parts avoid full reconfiguration at every handover, reducing delay and signaling.

- Service-awareness: Mobility procedures can adapt to traffic type (e.g., XR, IoT) or AI-predicted conditions, ensuring optimized performance.

- Scalability: A single framework can support a broad range of devices, from ultra-low-power IoT to high-throughput XR, and extend naturally to NTN scenarios.

Observation: NR mobility suffers from fragmentation and complexity, and is service agnostic. 6G should aim for a single, robust, low-latency and scalable framework adaptable to diverse services and devices.

## 2.2	Single design for the Mobility Framework

NR introduced multiple mobility procedures such as L3 handover, Conditional Handover (CHO), and Layer-Triggered Mobility (LTM). Each procedure came with its own signaling, configuration, and backward compatibility requirements.

The coexistence of multiple frameworks in NR (e.g., L3 HO, CHO, LTM) leads to overlapping signaling and increased complexity. A single framework in 6G can lower signaling overhead and improve interoperability.

A single design for the mobility framework that supports L3-based mobility, LTM and CHO consolidates these approaches into a common baseline, allowing a single configuration per candidate cell valid across mobility types. This reduces redundancy, simplifies specifications, and improves robustness by ensuring consistent UE behavior.

Proposal 1: Consolidate L3 HO, LTM, and conditional mobility into a single design for the mobility framework that supports both network triggered and conditional mobility based on L3/L1 measurements and L3/L2/L1 signalling.

## 2.3	Consolidated Measurement Framework

In NR, L1 and L3 measurements are configured separately (maybe even configured by different network entities, e.g., in the case of split architectures such as CU-DU), which can result in  duplicated reference signals and additional UE measurement burden. A single design framework in 6G would reduce redundancy and simplify measurement configuration.

The fragmentation of measurement frameworks also limits efficiency and prevents easy integration of mobility based on AI/ML (e.g., prediction of measurements and/or measurement events), as data from different layers cannot be harmonized.

Merging L1 and L3 measurement frameworks into one single structure reduces redundancy, simplifies configuration, and ensures consistent UE behavior. Shared RS definitions minimize overhead, while enabling AI-assisted prediction allows for proactive handovers and improved quality of experience.

Proposal 2: Merge L1 and L3 measurement frameworks into a single structure, and use common RS definitions for all RRM measurement types.

## 2.4	Separate Triggers, same Signalling Framework

Different mobility procedures (e.g., L3 HO, LTM) rely on distinct triggering conditions, typically based on quality thresholds or event conditions. In NR, these triggers are configured separately, leading to duplicated signaling and inconsistent interaction between procedures.

Maintaining separate triggers increases signaling load and makes it harder for UEs to manage multiple mobility procedures efficiently.

Designing triggers into one framework while preserving distinct criteria ensures flexibility for operators while avoiding duplicate configuration. UEs can evaluate triggers consistently, leading to smoother mobility execution and reduced signaling overhead.

Proposal 3: Maintain distinct triggers for L3 HO and LTM (e.g., quality thresholds, event-based triggers) but ensure they are configured within one single signaling framework.

## 2.5	RACH-less and Early TA Acquisition

Traditional handovers require a Random Access (RA) procedure, which introduces delay and signaling overhead. While Rel-18 introduced RACH-less LTM for intra-gNB, and Rel-19 introduced inter-gNB RACH-less procedures, they remain limited to cases with tight gNB coordination. RACH-less mobility is not yet generalized to non-ideal backhaul, inter-PLMN, inter-RAT, NTN, or multi-TRP scenarios. For 6G, a broader framework should support RACH-less transitions across all relevant deployment types.

For latency-sensitive applications (e.g. eXR, URLLC), even tens of milliseconds of interruption can cause service degradation. RACH-less mobility with early TA acquisition is needed to reduce interruption time and improve user experience.

Extending RACH-less HO to inter-gNB and inter-TA scenarios allows faster transitions without unnecessary RA signaling. Using TA prediction and pre-configuration further minimizes delay. This ensures low-latency, seamless handover performance essential for future immersive and mission-critical services.

Proposal 4: Support RACH-less HO for all relevant deployment types.

## 2.6	Simplified and Modular Reconfiguration During HO

In NR, handovers may require full RRC reconfiguration, even when only a subset of parameters changes. This leads to large signaling messages, increased latency, and higher UE processing burden. While delta signaling and reference configurations were introduced in NR (e.g. for LTM), in practice the reconfiguration is still heavy, since a full set of parameters is often constructed (e.g. a full RRC Reconfiguration for LTM).

As 6G introduces more features, reconfiguration size will only grow if the NR model is retained, further burdening devices and networks.

A better way to organize RRC configurations may be considered where the parts/modules that are likely to remain the same after mobility are grouped together. That way, only the parts/modules that need to be changed during mobility can be signalled to the UE. This will reduce the size of the reconfiguration message, which further reduces the RRC processing delay and handover interruption time, as well as overall system signaling overhead. This also simplifies specification maintenance and makes the framework more adaptable for future features.

It should also be recognized that as a UE changes from one profile or operation mode to another (e.g., due to service type, device mode, or capability update), the set of parameters that need to change based on delta configuration from that profile can differ significantly.

Proposal 5: To avoid full RRC reconfiguration at each HO, introduce RRC configuration parts (e.g., UP, security, mobility) that can persist across HO.

## 2.7	Support for NES

As 6G will be designed with energy efficiency as a baseline, mobility procedures will need to operate under conditions where both UEs and cells may be in low-power or sleep modes. This introduces new challenges that were not present in NR, at least until R19, where most reference signals and control channels are always-on.

With network energy saving (e.g., cell/beam DTX, on-demand SSB/SIB), mobility procedures cannot assume that all reference signals are continuously available. Mobility should account for the possibility that certain RS resources (SSB, CSI-RS) are intermittently available. Measurements may need validity ranges or fallback mechanisms. Handover and LTM/CHO should be possible to execute reliably even when candidate cells are in energy-saving state, e.g. by using wake-up signaling, shared configurations, or predictive measurements.

Measurements may need to adapt to cases where SSBs or CSI-RS are transmitted with longer periodicities, or only activated on-demand. Procedures for activating additional RS resources when mobility risk is detected (e.g., predicted RLF, UE entering cell edge) may therefore be needed. UEs may also reduce measurement activity based on low-power wake-up signals (LP-WUS), requiring mobility to rely on predictive or selective measurement strategies.

Without adapting mobility, there is risk of delayed or failed handover due to missing measurement opportunities, especially for latency-sensitive services. Different services (e.g. IoT vs XR) may tolerate different trade-offs between energy efficiency and robustness; mobility procedures should adapt accordingly.

Proposal 6: Mobility procedures in 6G should be designed to operate under network energy saving, e.g. adapting to candidate cells or beams that may be fully or partially sleeping with intermittently transmitted reference signals or system information.

## 2.8	Support for NTN and Multi-TRP Scenarios

In NR, Non-Terrestrial Networks (NTN) mobility support was introduced as an add-on, creating separate procedures. Similarly, Multi-TRP and cell-free deployments are often treated as extensions.

Fragmenting NTN and Multi-TRP into separate add-ons risks repeating NR’s complexity, with divergent procedures and heavy signalling to ensure backward compatibility.

By integrating NTN and Multi-TRP into the 6G mobility framework from Day 1, procedures become consistent across terrestrial and non-terrestrial deployments. This ensures scalability, avoids fragmented solutions, and supports future cell-free and NTN evolution without repeated add-ons.

Proposal 7: Mobility procedures should accommodate NTN-TN handovers and Multi-TRP/cell-free deployments from Day 1.

# Conclusion

This contribution outlines initial views on mobility in 6G, with a focus on Connected mode operation and makes the following observation and proposals:

Observation: NR mobility suffers from fragmentation and complexity, and is service agnostic. 6G should aim for a single, robust, low-latency and scalable framework adaptable to diverse services and devices.

Proposal 1: Consolidate L3 HO, LTM, and conditional mobility into a single design for the mobility framework that supports both network triggered and conditional mobility based on L3/L1 measurements and L3/L2/L1 signalling.

Proposal 2: Merge L1 and L3 measurement frameworks into a single structure, and use common RS definitions for all RRM measurement types.

Proposal 3: Maintain distinct triggers for L3 HO and LTM (e.g., quality thresholds, event-based triggers) but ensure they are configured within one single signaling framework.

Proposal 4: Support RACH-less HO for all relevant deployment types.

Proposal 5: To avoid full RRC reconfiguration at each HO, introduce RRC configuration parts (e.g., UP, security, mobility) that can persist across HO.

Proposal 6: Mobility procedures in 6G should be designed to operate under network energy saving, e.g. adapting to candidate cells or beams that may be fully or partially sleeping with intermittently transmitted reference signals or system information.

Proposal 7: Mobility procedures should accommodate NTN-TN handovers and Multi-TRP/cell-free deployments from Day 1.

# References

- RP-251881 “Study on 6G Radio”