3GPP TSG-RAN WG2 #131bis	R2-2507457

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.6.3

Source: 	Ericsson

Title:  	Discussion on remaining User Plane issues

Document for:	Discussion and Decision

# Introduction

During the review of the running CR for 38.321 some open issues have been identified. This contribution describes some of these issues and how RAN2 could proceed.

# Discussion

## Inter-CU LTM

### SP CSI-RS Measurements

In RAN2#131, the following agreement was taken:

RAN2#131 agreement: SP CSI-RS resource for target cell is deactivated after CSI reporting in the first UL transmission. How to capture it in MAC is up to MAC CR rapporteur.

As can be seen in the agreement, it was left to MAC rapporteur to update the necessary parts in specifications which was done as below:

- Running CR for MAC - 5.18.x Activation/Deactivation of Semi-persistent CSI-RS/CSI-IM resource set for candidate cell The network may activate or deactivate the configured Semi-persistent CSI-RS/CSI-IM resource sets for a candidate cell by sending the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE described in clause 6.1.3.12a. The configured Semi-persistent CSI-RS/CSI-IM resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync that is not triggered by LTM. After reconfiguration with sync that is triggered by LTM, the configured Semi-persistent CSI-RS/CSI-IM resource sets for all candidate cell(s), except the target cell, are deactivated. After CSI reporting at the target cell after or during cell switch triggered by LTM, the configured Semi-persistent CSI-RS/CSI-IM resource sets for the target cell is deactivated. The MAC entity shall: 1> if the MAC entity receives an SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE: 2> indicate to lower layers the information regarding the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

However, as we have pointed out during the review of the MAC CR, the text highlighted in yellow does not fully reflect the behaviour. The early CSI reporting in the target cell is done in the first possible uplink transmission in the target cell which is not stated in the text, otherwise according to the discussion and agreement, the normal CSI-reporting procedures applies. Therefore, we propose to change the text to (underlined text indicates adding); “After early CSI reporting at the target cell in the first available uplink transmission after or during cell switch triggered by LTM, the configured Semi-persistent CSI-RS/CSI-IM resource sets for early CSI-RS acquisition for the target cell is deactivated.”

Furthermore, as opposed to the CSI-RS resources for early CSI acquisition, resources for CSI RRM measurements can be configured in the form of NZP and ZP/CSI-IM resources. The zero power resources are more or less only gaps in neighbouring transmissions so, at the time of cell switch they will no longer be needed for the UE (since it has already transferred to a neighbour cell). The NZP however, can be available for regular CSI RRM measurements after cell switch, so it is reasonable that they are still available for the UE.

Clarify in the running CR that resources for early CSI acquisition are deactivated at cell switch.

CSI-IM resources are deactivated by the UE at cell switch, NZP resources remains active.

## Event-based L1 Measurements

### MAC-S02 Truncated vs regular MAC CE

The issue found and discussed under MAC-S02 relates to the following case:

T1	Truncated MR MAC CE sent with subset of beam information because UL grant was not big enough to accommodate all beams.

T2	Regular MAC CE is sent, and grant is now big enough to accommodate all beams.

Rapporteur argues that at T2, all beams (including the ones sent in truncated MAC CE at T1) should be included in the Regular MAC CE MR.

In our understanding, the following agreements indicates that the MR sent at T2 should not include triggered beams already reported, highlighted in yellow:

RAN2#129 agreement: Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader. RAN2#129bis agreement: Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

The previous agreements indicates that a measurement report that follows a truncated measurement report should not include triggered beams that have already been reported.

Apart from triggered beams, it is up to the UE to include other beams (type 11) up to the limit of maxNumberOfReportedBeams.

(MAC-S02) Running CR is updated to remove beams from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST when they are reported in a truncated measurement report.

### MAC-Q01 – LTM2 not treated in Measurement Report Lists

As discovered during the running CR review phase, LTM2 is not covered in the MR_LIST, BEAM_ENTERING_LIST, BEAM_LEAVING_LIST or BEAM_REPORTED_LIST since this text only mentions candidate cells, while LTM2 handles the serving cell.

Example text from running CR: - BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report;

Below is a technical proposal on how to change the text (blue underlined text indicates added text):

- 5.x.3.1 General The UE maintains the following UE variables for event triggered L1 measurement and report procedure: - MR_LIST: includes the list of event triggered L1 measurement report information, including RS resource index of LTM candidate cell(s) for evaluation, L1 measurement result, and type of reporting RS(s) as defined in 6.1.3.x, for which the L1 measurement report triggering conditions have been met for TTT. Each entry in the list is associated with a ltm-CSI-ReportConfigId; - MR_SENT_COUNTER: represents the number of event triggered L1 measurement report performed by UE if the triggering condition for the corresponding event is met for TTT for each ltm-CSI-ReportConfigId; - BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for evaluation for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report; - BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for evaluation for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report; - BEAM_REPORTED_LIST: includes the reference signaling resource index of LTM candidate cell(s) for evaluation for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have not been met for TTT

(MAC-Q01) The wording “LTM candidate” is removed from the different lists that describes beam handling according to TP above.

### The use of timeToTrigger vs TTT

In the running CR for MAC, chapter 5.x.1 introduces parameters, one of them is timeToTrigger. However, in the further introduction of the text, the term TTT is used consistently, without declaring a MAC variable for the purpose. TTT is in the list of abbreviations, but it is never explained what it is. A preferred way would be to either use timeToTrigger consistently throughout the specification as is done in the RRC specification, or state that a MAC variable called TTT is declared based on timeToTrigger. The last alternative is probably the best to both minimize impact to the specification, uphold existing semantics, and would improve for the reader.

Declare a MAC variable in running CR for 38.321 called TTT based on the value of timeToTrigger.

## Conditional LTM

### MAC-V05, MAC-K01, MAC-N01 Maintaining TA value for a previous source cell who is also a candidate

A number of agreements has led up to the current specifications which in short means that a UE maintains TA values for candidates up to a number corresponding to a capability of the UE. These TA values have been previously signalled from the network to the UE via a MAC CE. There is however one case overlooked, which is the case where a UE performs cell switch to another cell and the previous source cell is an LTM candidate. To enable RACH-less handover back to the previous cell the TA value must be known, and at the time of performing the cell switch it was known to both the network and the UE. The UE could maintain the previous TA value for use in a possible subsequent cell switch back to the original source.

It is our understanding that existing agreements to keep TA values for candidate cells covers this case and it seems feasible to introduce the changes to let the UE keep the TA value from the previous source cell.

(MAC-V05, MAC-K01, MAC-N01) A UE stores the TA value from a previous serving cell who is also a candidate cell and maintains a TA validity timer.

### MAC-O03 - Stored TA values when execution conditions are released

During RAN2#131 the following agreement was taken:

RAN2#131 agreement: Stop the running ltm-Candidate-TimeAlignmentTimer for the corresponding candidate cell and release the stored TA value for the candidate cell if the corresponding CLTM candidate configuration is released.

It has been further noticed that this agreement might not cover for the case that an existing LTM Candidate is updated with the execution conditions removed. If a candidate has the execution conditions removed, the UE can still be subject to network initiated cell switch through cell switch command, and for these cases the TA is still valid. Therefore, we propose that the UE keeps the TA value for a candidate which has the execution conditions removed.

(MAC-O03) The UE keeps any TA validity timer for an LTM candidate which has its execution conditions removed.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The previous agreements indicates that a measurement report that follows a truncated measurement report should not include triggered beams that have already been reported.

Observation 2	Apart from triggered beams, it is up to the UE to include other beams (type 11) up to the limit of maxNumberOfReportedBeams.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in the running CR that resources for early CSI acquisition are deactivated at cell switch.

Proposal 2	CSI-IM resources are deactivated by the UE at cell switch, NZP resources remains active.

Proposal 3	(MAC-S02) Running CR is updated to remove beams from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST when they are reported in a truncated measurement report.

Proposal 4	(MAC-Q01) The wording “LTM candidate” is removed from the different lists that describes beam handling according to TP above.

Proposal 5	Declare a MAC variable in running CR for 38.321 called TTT based on the value of timeToTrigger.

Proposal 6	(MAC-V05, MAC-K01, MAC-N01) A UE stores the TA value from a previous serving cell who is also a candidate cell and maintains a TA validity timer.

Proposal 7	(MAC-O03) The UE keeps any TA validity timer for an LTM candidate which has its execution conditions removed.