3GPP TSG-RAN WG2 Meeting #131bis	R2-2507458

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	9.2.2

Source:	Nokia

Title:	Ambient IoT aspects in Topology 2

WID/SID:	Ambient_IoT_Solutions_Ph2

Document for:	Discussion and Decision

# 1	Introduction

Rel-20 brings more advanced devices such as Device 2b/C, together with a focus transition to topology 2 where a UE serves as the reader. This paper summarizes our initial views in this area.

# 2	Discussion

## 2.1	Registration and Discovery

The first step in the discussion on UE readers in RAN2 is to clarify on how UE become available for the service as AIoT readers. From the direction of the SA2 discussion on Release 20 AIoT, currently in the stage of identifying Key Issues for the Topology 2 scenario, it is reasonable to assume that UE readers would be registered with a clear indication of the area they can support. While no formal conclusions with technical / specification impact have been made yet, SA2 seems to be converging to two use cases: the first one consisting in an “area of service” indication, the other one being based involving “reader ID” indication (e.g. in the SUPI form).

From RAN2 perspective, we would like to reflect this architectural choice by making minimal assumptions on the registration aspects until SA2 reach final agreements.

Proposal 1: RAN2 assumes that a UE reader registers

- for a specific service area, and/or

- under specific serving gNB identity, and/or

- under own specific UE reader identity.

Clearly, the information on spatial and temporal distribution of registered UE may not be sufficient to find readers that will be able to serve with very high likelihood specific AIoT devices or edge areas. Devices can move, be subject to adverse propagation phenomena, deactivate due to energy shortage, etc. Service area may change or be poorly define while propagation phenomena can again play role in effective coverage.

Observation 1: Registered UE readers may not satisfy operational needs in terms of specific coverage or device reachability requirements.

Thus, it would be useful to avail also of discovery methods that would permit to detect the reachability of a (specific) device or area to the AIoTF. In general, defining a novel dedicated sidelink-like discovery mechanisms for UE reader discovery over the radio interface would require novel complex procedures. This problem can be conveniently left to the application layer / AIoTF itself. More specifically, legacy procedures such as basic inventory requests can be used by the AIoT-F to observer and track association information between UE readers and devices.

Observation 2: Application layer / AIoTF service probing is sufficient to implement UE reader discovery.

Proposal 2: Successful inventory procedures are used by core network to monitor associations of UE readers and devices. FFS potential specification impact.

## 2.2	AIoT Request Delivery

The SA2 sees two options for an AIoT request to be delivered to a UE reader:

- The AF provides a target area (eg, a specific warehouse section) and the AIOTF forwards this information to serving gNB(s) which then selects the UE reader(s) on its own.

- The AF provides a specific list of reader IDs (e.g., hand-held devices carried by employees), and the gNB(s) simply select the UE reader(s) from the requested list.

In either of the above procedures, there is a need for the gNB to provide UL resources to be used for the AIoT interface. To reflect this architecture, we would like to make the following assumption:

Proposal 3: RAN2 initially assumes two options for AIoTF request delivery:

- direct delivery from the AIoTF to a UE reader,

- indirect delivery from the AIoTF to a gNB, which may delegate the request further to a selected UE reader,

and further down-selects based on SA2 progress (if necessary).

Similarly, it should be possible for the AIOTF to first select a set of candidate UE readers based on information available in the CN domain (e.g., past associations of UE readers with specific devices/areas), after which the gNB further down-selects the candidate readers using its own information such as radio strength.

Irrespective of how an AIoTF request is delivered to the UE reader, it will be the gNB that will assign the UL resources to be used on the AIoT interface. If an AIoTF request is delivered directly to the UE reader without the gNB being aware of the request details (e.g., service type and number of UEs involved), the reader will have to request the resource from the gNB. However, if the gNB has visibility into the message to the UE reader, the gNB may assign resources directly without any UE reader request.

Proposal 4: RAN2 confirms that gNB provides a set of UL resources to be used by the UE reader on the AIoT interface. FFS whether these resources can be requested by reader.

## 2.3	Multi-Reader Operation

For the Release 19 point of view, it may be possible for the AIoTF to select multiple UE readers for the same AF request, however these will not operate in parallel for the same coverage area.

In Release 20, we would like to permit the selection of both UE and gNB readers. Clearly, this process should be transparent to the device, that is, there should not be any difference for the device between the operation in Topology 1 setting with a gNB reader or Topology 2 setting with a UE reader.

Proposal 5: An AIoT device is assumed not to (be able to) distinguish between operation in Topology 1 and 2, and may be served transparently by both UE reader and gNB reader.

A similar argument holds also for multiple readers serving the same AIoTF request, that is, multiple readers paging the same device.

Proposal 6: An AIoT device may be offered the same service by multiple readers. FFS details of reader selection by device including time constraints.

To assist the device to select the most suitable UE reader, we support the reader ID indication. Without a reader ID, the device may be substantially constraint in its selection options as readers would have to use some convention in order understand who the reader selected by the device is.

For example, the last reader that paged the device prior to its response could be understood as the target reader. This simplistic approach however leaves room for error (e.g., due to the hidden / exposed node problem, slow device processing under multiple paging requests) which can be simply avoided by the reader ID usage.

Observation 3: Reader ID simplifies UE reader operation in Topology 2.

Proposal 7: Reader ID is supported.

In this context, it is noted that RAN1 discussed efficient usage of preambles for implicit ID indication, for example to deliver information associated with paging or transaction ID. The usage of preambles permits quick and simple PHY-level pre-processing of key identification parameters that could help efficiently decide whether to decode the MAC layer payload, a substantially more complex (and energy-demanding) task.

Observation 4: Preamble-based ID indication permits efficient decision-making without energy/time-consuming decoding of MAC payload.

Proposal 8: RAN2 to send an LS to RAN1 whether preambles can be used to indicate paging/reader/transaction ID.

## 2.4	UE Reader Signalling

The usage of UE reader poses particular challenges to AIoT operation. One of them is UE mobility and the dynamic nature of Topology 2 operation in general. Another one consists in efficient multi-reader operation in such settings.

Observations 5: UE reader availability and / or reachability is transient in Topology 2.

To this end, it is necessary to define clear start and end of AIoT session by the UE reader. Open sessions create the need for various back-up mechanisms like expiry timers whose (de)activation may depend on various prior conditions. Such state-oriented management is inefficient.

The reader can use simple end-of-session signalling to indicate resource expiry, end/start of monitoring of existing/new session, as well as provide inputs for failure/re-access determination and buffer data management. Clearly, end-of-session signalling can apply to both UE and gNB readers in Release 20.

Proposal 9: RAN2 to define end-of-session signalling by readers.

Similarly to R19, we do not see the need for devices to report their individual capabilities or specific capability summaries. The only thing that matters in AIoT operation is the ability of a device to harvest energy and use it for communications with a reader. This ability may depend on hardware and operational options of the device but can be summarized in one meta parameter - the device energy status. Tracking of the energy device status then allows better control and management of AIoT requests and re-transmissions in time.

We see therefore the need to report device energy status for efficient AIoT function.

Proposal 10: Device energy status is reported by the device.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Registered UE readers may not satisfy operational needs in terms of specific coverage or device reachability requirements.

Observation 2: Application layer / AIoTF service probing is sufficient to implement UE reader discovery.

Observation 3: Reader ID simplifies UE reader operation in Topology 2.

Observation 4: Preamble-based ID indication permits efficient decision-making without energy/time-consuming decoding of MAC payload.

Observations 5: UE reader availability and / or reachability is transient in Topology 2.

Proposal 1: RAN2 assumes that a UE reader registers

- for a specific service area, and/or

- under specific serving gNB identity, and/or

- under own specific UE reader identity.

Proposal 2: Successful inventory procedures are used by core network to monitor associations of UE readers and devices. FFS potential specification impact.

Proposal 3: RAN2 initially assumes two options for AIoTF request delivery:

- direct delivery from the AIoTF to a UE reader,

- indirect delivery from the AIoTF to a gNB, which may delegate the request further to a selected UE reader,

and further down-selects based on SA2 progress (if necessary).

Proposal 4: RAN2 confirms that gNB provides a set of UL resources to be used by the UE reader on the AIoT interface. FFS whether these resources can be requested by reader.

Proposal 5: An AIoT device is assumed not to (be able to) distinguish between operation in Topology 1 and 2, and may be served transparently by both UE reader and gNB reader.

Proposal 6: An AIoT device may be offered the same service by multiple readers. FFS details of reader selection by device including time constraints.

Proposal 7: Reader ID is supported.

Proposal 8: RAN2 to send an LS to RAN1 whether preambles can be used to indicate paging/reader/transaction ID.

Proposal 9: RAN2 to define end-of-session signalling by readers.

Proposal 10: Device energy status is reported by the device.