3GPP TSG-RAN WG2 #131-bis					      		 R2-2507466

Prague, Czech Republic – October 13 - 17, 2025

Agenda item:	10.3.2

Source: 	Kyocera

Title: 	Initial consideration for RRC modeling in 6GR

Document for:	Discussion

# Introduction

In this contribution, we consider some aspects of RRC that should be addressed by RAN2 for 6G.

# Discussion

During RAN#108, the 6G Radio study has been approved [1], which included the study of radio interface protocol architecture and procedures, including the control plane architecture (including RRC states) and protocol design.  In the following section, the structure and contents of the RRC specification are considered.

## 2.1	RRC fundamentals for 6GR

TS 38.331 has become a very lengthy specification and very challenging to maintain alignment with dependent specifications. Change requests addressing critical issues often take time to be standardized and integrated, leaving prolonged windows for inconsistencies in implementation.

- Observation 1	The current RRC specification is far too big/extended and complicated even before the introduction of 6GR requirements.

And with each new release, additional functionalities are added to the specification which further complicates the specification which also results in additional complexities for both UE vendors and NW vendors.  During the RAN plenary workshop for 6G, many companies proposed for simplicity as one of the key criteria for 6GR specifications, with preference for the adoption of standalone specifications for 6GR [2][3][4].  Several companies also supported the modular protocol design, including the RRC specification [2][5][6][7].   Furthermore, RAN2 should consider which RRC features should be included in the baseline RRC functions.  To form the baseline RRC functions, we can choose a common set of features that all devices would likely support or choose a particular device type that may be considered to have the lowest set of capabilities.  Candidate for such a UE type could include IoT devices such as RedCap UEs, that typically require less complicated RRC configurations with a single connection (i.e., no CA support), although it does require certain features that are not required by some higher capability devices, such as eDRX, RRM relaxation. For example, URLLC devices such as sensors, autonomous vehicles may not need eDRX and RRM relaxation, while these power saving techniques may still be helpful to eMBB devices such as smartphones. Other considerations may include the need to support inter-RAT mobility between 6G and 5G, but in our view this should be handled after the baseline RRC functions are adopted.

- Proposal 1	RAN2 should adopt a new RRC specification only for standalone 6GR in a modular approach and decide whether the baseline RRC functions should be based on a common set of features that all devices would support or choose a particular UE type with the lowest set of capabilities.

## 2.2	RRC features for improvements

Beyond the discussion for determining the baseline RRC functions, RAN2 should also consider some of the existing features that could be improved.

2.2.1 RRC States:

RAN2 has depended on characterizing the UE behaviour using the three RRC states, CONNECTED, INACTIVE and IDLE. The advantages of defining the 3 states are numerous.  For example, when the UE has no activities, the gNB may release the UE to IDLE or INACTIVE to maximize device battery life.  Signalling overhead is also reduced when the UE is not in RRC CONNECTED since there are no dedicated signalling between the UE and the gNB when in IDLE or INACTIVE.  However, there are also drawbacks to the RRC states.  In one case, the adoption of RRC states increases standardization complexities as the UE behaviour must be well defined for each state, along with the conditions for transitions among RRC states, either by gNB order or decided by the UE e.g., when the radio condition deteriorates.  With each new feature, separate UE behaviours may be needed for each RRC state. When the UE is in IDLE or INACTIVE dedicated signalling is not applicable and the UE relies on SIB signalling that cannot be catered to one particular UE. Latency in connection establishment is also significant which may work against the ultra-low latency requirements in 6GR. Even from the network’s perspective, managing all the UEs, along with their RRC states requires sophisticated network algorithms.

- Observation 2	While the UEs with no activity may be transitioned to RRC IDLE/INACTIVE to save power and reduce signalling, the UEs in RRC IDLE/INACTIVE are not easily controllable without dedicated signalling and the increased latency in connection establishment needs to be addressed.

- Despite some of the drawbacks in the three RRC states currently defined in 5G NR, it should still remain as the baseline assumption of how RRC should operate.  However, it should be further considered whether the existing states can be improved to retain some of the advantages on low-power consumption in IDLE/INACTIVE while still be controllable by the network with minimal signalling complication.

- Proposal 2	RAN2 should improve the current RRC IDLE/INACIVE states to retain the low-power consumption while still allow for better controllability by the network.

Furthermore, with the anticipated optimization needed in latency reduction for initial access, it could be further considered if the Inactive state is really necessary in addition to the Idle state.  The Inactive state was initially developed in Rel-15 NR to allow for faster data resumption since the UE context is kept at the network, while keeping some of the benefits of low power consumption as in IDLE. Other than the faster data resumption, there is essentially no difference between IDLE and INACTIVE from the UE’s perspective.  It also adds more complexity at the network side. The addition of the Inactive state increased overall RRC state machine complexity, requiring additional handling of context suspension, resume, and mobility within the RNA.  The network will also need to manage the UE’s RAN Notification Area (RNA) for paging purposes which is separate from the CN defined paging for IDLE UEs [8].   If RAN2 decides to keep the Inactive state, improved differentiation between IDLE and INACTIVE should be defined, considering the complexities the Inactive state adds to the specification.

- Proposal 3	RAN2 should also improve the differentiation between Inactive state and Idle state beyond what is already defined in 5G NR, considering the complexities the inactive state adds to the specification and network implementation.

2.2.2 Message structure:

The current RRC specification contain numerous dedicated signalling messages from the gNB to the UE and vice versa. Within many work items, decisions are made independently on whether new feature(s) that require dedicated signalling message should reuse one of the existing messages (mainly for backwards compatibility) or create a new RRC message. For example, ConditionalReconfiguration which is used for mobility, such as CHO, is included within RRCReconfiguration message as an IE rather than a new dedicated message.  Discussions were held on the pros and cons for both options [9]. While some companies thought a new RRC message would be cleaner and separate from the existing non-CHO HO configuration, most companies decided that it is easier just to add a new IE to the RRCReconfiguration message, mainly for satisfying backwards compatibility.  In another example, during multipath sidelink relay discussion, it was decided that upon detection of indirect path failure, the multipath remote UE should inform the network of the failure via the IndirectPathFailureInformation message rather than the existing SCGFailureInformation or MCGFailureInformation. Going forward, for 6GR, there should be a more consistent approach on how new or existing message should be used that would be applicable for all work items, esp. since backwards compatibility is no longer needed.

- Proposal 4	RAN2 consider a more consistent approach in designing the message structure across all work items.

Another area of signalling that could be considered is related to the current structure for handover or (re)establishment procedure such as Resume procedure and connection establishment procedure, it typically starts with a request message, followed by the response from the gNB.  Then a third message is sent by the UE for confirmation, e.g., RRCResumeComplete. If this message can be eliminated, both latency and signalling overhead may be improved.  For example, within the RACH procedure in MAC, both 4-step RACH and 2-step RACH are used whereby the 2-step RACH reduces latency at the cost of potential preamble collision that prevents RRC Connection Request in Msg A from successful reception at the gNB.  RAN2 should consider whether a simplified procedure might be possible e.g., elimination of the Complete message.

- Proposal 5	RAN2 should consider whether the existing message structure can be improved to reduce latency and for overhead reduction.

2.2.3 On-demand SIB Request:

While On-Demand SIB Request saves broadcast bandwidth and can reduce energy consumption in low-traffic scenarios, it presents practical disadvantages related to system load, delay, resource contention. If preambles are scarce or random-access resources are limited; this can lead to preamble collisions and failed requests. UEs must also wait for the base station to acknowledge requests and then schedule the required SIB, introducing delay in scenarios where immediate SIB information is needed—for example, during rapid mobility or emergency procedures. Furthermore, for cell reselection, the UE may need to prioritize reselection based on services that are available on neighbor cells.  For example, for slice-based cell reselection, the UE may not be able to determine cell reselection priorities for slicing without receiving SIB16 which may not always be broadcasted without On-demand request.  Another example is for service continuity while the UE is receiving ongoing MBS service while in INACTIVE state.  The UE may need to determine which neighbor cell supports MBS service by receiving SIB20, which may not always be broadcasted periodically. Also, UEs in IDLE/INACTIVE may prioritize cells that provide V2X services based on receiving SIB12 which are also not always broadcasted periodically. RAN2 should consider how the existing on-demand SIB request can be improved or if it is still needed.

- Proposal 6	RAN2 should consider how the existing on-demand SIB request can be improved or if it’s still needed.

2.2.3 Timer structure:

The current RRC timers are quite complicated. The description of each timer e.g., T310, T304 has grown over the years with extension for multiple new features. Latency reduction is also highly dependent on the proper configuration of certain timers.  For example, timers (e.g., T304) that govern handover execution timing, if not properly configured, and the timer expires before successful completion, UE will trigger reestablishment, potentially causing additional delays.  In another example, the T300 timer, which is used connection setup waits for the network's response to an RRC connection request and may be configured up to 2 seconds.  If the timer expires without a proper response, the setup is considered as failed, causing retries and delays.  With all the efforts in latency reduction under initial access improvements anticipated for 6GR, improper timer management can easily lead to unnecessary delay in acquisition of service.

- Proposal 7	RAN2 should consider whether some of the existing timers are needed or if they can be improved.

# Conclusion

In this contribution, we addressed some of the issues related to RRC modeling for 6GR. RAN2 is kindly asked to take into account of the observations and proposals below.

- Observation 1	The current RRC specification is far too big/extended and complicated even before the introduction of 6GR requirements.

- Proposal 1	RAN2 should adopt a new RRC specification only for standalone 6GR in a modular approach and decide whether the baseline RRC functions should be based on a common set of features that all devices would support or choose a particular UE type with the lowest set of capabilities.

- Observation 2	While the UEs with no activity may be transitioned to RRC IDLE/INACTIVE to save power and reduce signalling, the UEs in RRC IDLE/INACTIVE are not easily controllable without dedicated signalling and the increased latency in connection establishment needs to be addressed.

- Proposal 2	RAN2 should improve the current RRC IDLE/INACIVE states to retain the low-power consumption while still allow for better controllability by the network.

- Proposal 3	RAN2 should also improve the differentiation between Inactive state and Idle state beyond what is already defined in 5G NR, considering the complexities the inactive state adds to the specification and network implementation.

- Proposal 4	RAN2 consider a more consistent approach in designing the message structure across all work items.

- Proposal 5	RAN2 should consider whether the existing message structure can be improved to reduce latency and for overhead reduction.

- Proposal 6	RAN2 should consider how the existing on-demand SIB request can be improved or if it’s still needed.

- Proposal 7	RAN2 should consider whether some of the existing timers are needed or if they can be improved.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#108

- 6GWS-250087, “Views on 6G roles and beyond evolution”, Kyocera

- 6GWS-250061, “Lenovo views on 6G RAN Vision and Priorities”. Lenovo

- 6GWS-250144, “CMCC's views on 6G RAN study”, CMCC

- 6GWS-250004, “Nokia's view on 6G Radio and RAN”, Nokia

- 6GWS-250032, “OPPO's view on 6G RAN technology”, OPPO

- 6GWS-250034, “Panasonic view on 6G RAN”, Panasonic

- R2-2507503, “Initial consideration for 6G initial access aspects”, Kyocera

- R2-1911733, “Summary of [106#42] [NR/LTE/mob enh] CHO configuration”, OPPO