3GPP TSG-RAN WG2 Meeting #131bis	R2-2507471

# Prague, Czech Republic, 13th – 17th October 2025

Agenda Item:	8.7.3

Source:	Ericsson

Title:	RLC-E01, RLC-X01

Document for:	Discussion, Decision

# 1	Introduction

The following are the list of open issues as contributed by companies and the opinion of the rapp.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Ericsson | The current implementation for the t-RxDiscard configured and not running is if RX_Next_Highest > RX_Next + 1 or if RX_Next_Highest = RX_Next + 1. However, the second condition might be too aggressive and could result in a situation where the RLF detection procedure is affected. For example, if the initial state is SN = 0 with segmentation and the RLC Rx entity receives byte segments out of order. Then, RX_Next = 0, RX_Next_Highest = 1, RX_Next_Discard_Trigger = 1 If the last segment of SN = 0 is received but others are not by the time t-RxDiscard expires, then the RLC Rx window advances with Rx_Next = 1 and sends an ACK for SN = 1. This behaviour could keep happening with the Tx entity assuming things are working well but the Rx entity seeing virtually no data. [Rapp]: If t-RxDiscard expires and the SDU with SN = 0 in your case has not been fully received by the receiver, it is considered outdated and no longer needs to be transmitted. This is exactly how the “avoid unnecessary retransmission” mechanism is intended to work, so we do not see any issue here. But the proponent is welcome to rais proposal on this. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: RLC-E01 |
| Xiaomi | A open issue we put on the post email discussion: In last meeting, we have agreed: - [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC If all the RLC SDUs that RLC needs to calculated are indicated from PDCP, then RLC can indeed calculate the data based on PDCP’s indication. Then RLC calculation of the none delay reporting data can be removed. However, is that possible that RLC put some packets which are not know in PDCP before the data of one portion or is there possible that before DSR is triggered, there has already some data before one portion? Then in this way, since PDCP is not aware of data in RLC’s buffer, then I guess the part that RLC calculation of the none delay reporting data still can not be removed. And Rapp’s response is: “ My understanding is all SDU in RLC comes from PDCP, so PDCP is aware of all packets status (remaining time). RLC just needs to determine which one/segments should be calculated in the DSR, e.g. based on whether delivery to lower layer/retx. It seems no problem for this. We could think more if any issue, and further discuss it during maintenance phase. ” In last F2F meeting offline in India, after checking with LG, Sharp, we are not sure on this. We would like to hear more opinions from other companies. [Rapp] The issue is related to the implementation on RLC handling, e.g. whether PDCP and RLC have the same packet placement order, and whether PDCP and RLC share single IP packet copy in physical memory but maintaining respective protocol headers at UE side. The reasonable implementation way should be: SDU handling in RLC layer follows the same order as in PDCP layer. But companies are welcome to provide views on this issue. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-X01 |

In this contribution, we discuss the open issues as listed above.

# 2	Discussion

## 2.1 	RLC-E01

The description of the issue and the rapp’s response is as shown below:

The current implementation for the t-RxDiscard configured and not running is if RX_Next_Highest > RX_Next + 1 or if RX_Next_Highest = RX_Next + 1.

However, the second condition might be too aggressive and could result in a situation where the RLF detection procedure is affected.

For example, if the initial state is SN = 0 with segmentation and the RLC Rx entity receives byte segments out of order. Then, RX_Next = 0, RX_Next_Highest = 1, RX_Next_Discard_Trigger = 1

If the last segment of SN = 0 is received but others are not by the time t-RxDiscard expires, then the RLC Rx window advances with Rx_Next = 1 and sends an ACK for SN = 1.

This behaviour could keep happening with the Tx entity assuming things are working well but the Rx entity seeing virtually no data.

[Rapp]: If t-RxDiscard expires and the SDU with SN = 0 in your case has not been fully received by the receiver, it is considered outdated and no longer needs to be transmitted. This is exactly how the “avoid unnecessary retransmission” mechanism is intended to work, so we do not see any issue here. But the proponent is welcome to rais proposal on this.

Although the rapp’s response is technically correct in stating the behaviour is as expected, it is important to recognize that such a scenario could cause a significant drop in performance. However, the UE would not declare RLF since normal RLF detection process is interrupted.

Consider a scenario where radio conditions are poor such that the RLC Rx only receives only a small percentage of the RLC SDU or SDU segments before t-RxDiscard expires. Then, the behaviour as described above can occur repeatedly resulting in the RLC Rx receiving virtually no or very little data. While the RLC Tx is happy enough as it receives an ACK for all the RLC SDUs and would not declare RLF.

It is possible that setting t-RxDiscard to a longer value should address this scenario. However, the configuration is dependent on factors such as traffic type and t-Reordering value. Thus it is not always possible to set a longer value in every case.

As a result, our proposal is to allow for the RLC Rx to receive at least ‘full SNs’ before starting the t-RxDiscard timer to ensure the RLF mechanism is not affected. The TP is included in the Annex.

(RLC-E01) Allow the RLC Rx entity to receive ‘full SNs’ before triggering the t-RxDiscard timer i.e., start t-RxDiscard only if RX_Next_Highest> RX_Next + 1. Adopt the TP as in the Annex.

## 2.2 	RLC-X01

PDCP knows what data it has transmitted to lower layer, also what was transmitted before the DSR was triggered. The only case that there could be any issue is if RLC does reordering of the data received from higher layer. However, this should not be the case, i.e. RLC will transmit the packets in the same order as they are received from PDCP. The reordering topic has been discussed multiple times in RAN2 during XR standardization and always concluded that RLC does not do any transmission reordering. Thus, there is no issue to solve.

(RLC-X01) RAN2 confirm that current specification is sufficient regarding PDCP indicating delay reporting data to RLC.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

No table of contents entries found.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	(RLC-E01) Allow the RLC Rx entity to receive ‘full SNs’ before triggering the t-RxDiscard timer i.e., start t-RxDiscard if RX_Next_Highest> RX_Next +2.

Proposal 2	(RLC-X01) RAN2 confirm that current specification is sufficient regarding PDCP indicating delay reporting data to RLC.

# 4	Annex, TP

RLC-E01:

##### 5.2.3.2.3 Actions when an AMD PDU is placed in the reception buffer When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall: - if x >= RX_Next_Highest: - update RX_Next_Highest to x+ 1. - if all bytes of the RLC SDU with SN = x are received: - reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer; - if x = RX_Highest_Status: - update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received. - if x = RX_Next: - update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received. - if t-Reassembly is running: - if RX_Next_Status_Trigger = RX_Next; or - if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or - if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-Reassembly. - if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above): - if RX_Next_Highest> RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-Reassembly; - set RX_Next_Status_Trigger to RX_Next_Highest. - if t-RxDiscard is configured and running: - if RX_Next_Discard_Trigger = RX_Next; or - if RX_Next_ Discard_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or - if RX_Next_Discard_Trigger falls outside of the receiving window and RX_Next_Discard_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-RxDiscard. - if t-RxDiscard is configured and not running (includes the case t-RxDiscard is stopped due to actions above): - if RX_Next_Highest> RX_Next +1: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.