3GPP TSG-RAN WG2 #131bis                                          	    R2-2507485

Prague, Czech Republic, October 13-17, 2025

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	LTM MAC remaining issues

Document for:	Discussion

# 1	Introduction

This paper discusses remaining open issues in MAC for Rel-19 LTM.

# 2	Open issues on SP CSI-RS/CSI-IM

## 2.1	[MAC-Q04, MAC-N02] Activated SP CSI-RS/CSI-IM resources for target cell for L1-RSRP measurement upon LTM cell switch

In previous RAN1 & RAN2 meetings, there were some agreements on sending the early CSI report to target cell and handling activated SP CSI-RS resources during LTM cell switch, and the latest MAC running CR has captured the corresponding statement as below.

RAN1#121 chair’s note For PUSCH to convey the early CSI report, - For RACH-less LTM, the first CG or DG PUSCH after CSC is used - For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A - For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B For the reporting LTM early CSI reporting, Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format RAN2#130 chair’s note - UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell. RAN2#131 chair’s note - Common SP CSI-RS Resource Set Activation/Deactivation MAC CE is used for both RRM measurement and early CSI acquisition purpose, as captured in the current MAC running CR. - SP CSI-RS resource for target cell is deactivated after CSI reporting in the first UL transmission. How to capture it in MAC is up to MAC CR rapporteur. MAC running CR snippet ### 5.18.x Activation/Deactivation of Semi-Persistent CSI-RS/CSI-IM resource set for candidate cell […] After reconfiguration with sync that is triggered by LTM, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for all candidate cell(s), except the target cell, are deactivated. After CSI reporting at the target cell after or during cell switch triggered by LTM as specified in clause 5.2.4a in TS 38.214, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for the target cell are deactivated. […] 38.214 clause 5.2.4a snippet ### 5.2.4a CSI Reporting for LTM A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, [ltm-eCSI-ReportConfig], for a candidate cell. Each Reporting Setting [ltm-eCSI-ReportConfig] is associated with either one or two Resource Settings Rapporteur response in email discussions [Rapp]: We have discussed this issue clearly in RAN2#131 meeting, and concluded that there is no need to differentiate the SP CSI-RS for L1-RSRP and for CSI. With this, it seems there is no need to have separate handling for SP CSI-RS for CSI and for L1-RSRP.

While the rapporteur mentioned that there was a conclusion in RAN2#131 about no need to differentiate SP CSI-RS for L1-RSRP and that for CSI, we believe the discussion was mainly from the SP CSI-RS activation/deactivation MAC CE point of view, which is in line with the legacy case (i.e., SP CSI-RS/CSI-IM activation/deactivation MAC CE is used for both L1-RSRP and RI/CQI/PMI measurements purposes).

Observation 1a: From rapporteur’s comment, previous RAN2 discussions/agreements on “no differentiation for SP CSI-RS between L1-RSRP and CSI acquisition” was mainly discussing the SP CSI-RS activation/deactivation MAC CE to be applicable to both L1-RSRP and CSI acquisition, which makes sense and in line with the legacy SP CSI-RS/CSI-IM activation/deactivation MAC CE.

But the issue we are raising here is that for target cell case, there is a dependency on CSI reporting at target cell for SP CSI-RS/CSI-IM resource deactivation, and the current text leaves the UE behaviour undefined when UE has activated SP CSI-RS resources for target cell for L1-RSRP but the target cell doesn’t have early CSI reporting configuration for UE. Therefore, we believe SP CSI-RS for L1-RSRP and early CSI acquisition for target cell shall be treated separately.

Observation 1b: The current text in MAC running CR is working only if UE is configured with early CSI acquisition for target cell. In case UE has activated SP CSI-RS for L1-RSRP from target cell and the target cell doesn’t configure early CSI reporting, SP CSI-RS deactivation timing is undefined.

Since L1-RSRP measurement report is meant to be sent to the serving cell, any SP CSI-RS/CSI-IM resources for L1-RSRP measurements purpose should be deactivated for all candidate cells even including target cell during LTM cell switch.

Observation 1c: L1-RSRP measurement report is only for the serving cell, so even for the target cell, activated SP CSI-RS/CSI-IM resources should be deactivated during LTM cell switch.

Proposal 1: RAN2 to agree the deactivation of SP CSI-RS for the target cell for the L1-RSRP purpose and adopt the TP for clause 5.18.x in Annex A.1.

## 2.2	[MAC-Q05] Presence indication of CSI Resource Configuration ID2 in SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE

In RAN2#131, it was pointed out that a single LTM CSI resource configuration can have either ltm-NZP-CSI-RS-ResourceSet or ltm-CSI-IM-ResourceSet-r19, not both. As a consequence, it was agreed that SP CSI-RS/CSI-IM for candidate cell MAC CE could have up to two LTM CSI resource configuration IDs – one for CSI-RS and the other for CSI-IM.

RAN2#131 chair’s note - IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id. - SP-CSI MAC CE includes up to two CSI-RS resource configuration ids. - Detailed change is left to MAC CR rapporteur. MAC running CR snippet ### 6.1.3.12a SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE […] - A/D: This field indicates whether to activate or deactivate the indicated SP CSI-RS resource set for the candidate cell(s) associated with the CSI Resource Configuration ID1 in the same octet, or SP CSI-RS and CSI-IM resource set for the candidate cell(s) associated with the CSI Resource Configuration ID2 in the same octet, respectively. The field is set to 1 to indicate activation, otherwise it indicates deactivation; - CSI Resource Configuration ID1: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-RS resource set for the candidate cell(s) for measurement. The length of the field is 7 bits; - CSI Resource Configuration ID2: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-IM resource set for the candidate cell(s). If the SP CSI-IM resource set for the candidate cell(s) is not configured in TS 38.331 [5], this field and the reserved bit in the same octet are absent. The length of the field is 7 bits; […] Figure 6.1.3.12a-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE Rapporteur response in email discussions [Rapp]: This issue was indeed discussed in RAN2#130 meeting. Everyone agreed that UE could easily identify whether this CSI Resource Configuration ID is CSI Resource Configuration ID1 or CSI Resource Configuration ID2 based on RRC configuration due to the the following agreement： - IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id.

It is worth noting the issue raised here is not about whether UE is able to identify a particular CSI Resource Configuration ID is for CSI-RS or CSI -IM. But it is rather about the ambiguity & redundancy of the presence of CSI Resource Configuration ID2 as per the current running MAC CE.

Regarding the presence of CSI Resource Configuration ID2, UE is supposed to determine the presence implicitly based on RRC configuration of SP CSI-IM resource set for candidate cell(s). There are some issues that are envisioned with the current description as follows:

- SP CSI-RS for L1-RSRP measurements don’t need the SP CSI-IM at all. But under the current description, UE would have to assume CSI Resource Configuration ID2 is present as long as UE is configured with an SP CSI-IM resource set for candidate cell(s).

- Assume UE is configured with the following LTM CSI resource configurations:

  - LTM CSI Resource Configuration ID #0: SP CSI-RS for candidate cell #A

  - LTM CSI Resource Configuration ID #1: SP CSI-IM for candidate cell #A

  - LTM CSI Resource Configuration ID #2: SP CSI-RS for candidate cell #B, #C, #D

What would be the expected UE behaviour to determine the presence of CSI Resource Configuration ID2 when CSI Resource Configuration ID1 is set to #2 (i.e., SP CSI-RS for candidate cell #B, #C, #D)? UE is configured with SP CSI-IM, but it has nothing to do with the signalled CSI Resource Configuration ID1 value.

Both issues are not properly answered by the current MAC running CR. Moreover, if the intention was to let UE validate the corresponding LTM CSI resource configuration for CSI Resource Configuration ID1 to determine the presence of CSI Resource Configuration ID2, then it will cause unnecessary UE side processing latency for the MAC CE processing.

Observation 2a: The current MAC running CR is interpreted as if UE needs to assume CSI Resource Configuration ID2 to be present as long as UE is configured with SP CSI-IM resource set for candidate cells in RRC.

Observation 2b: Such implicit determination rule would cause either unnecessary presence of CSI Resource Configuration ID2 (e.g., SP CSI-RS for L1-RSRP which doesn’t require CSI-IM at all) or unnecessary extra processing of validating the corresponding LTM CSI resource configuration to CSI Resource Configuration ID1.

Proposal 2:  RAN2 to agree replacing “A/D” to “IM” in Octet 2 for the explicit indication of the presence of CSI Resource Configuration ID2 and adopt the TP for clause 6.1.3.12a in Annex A.2.

## 2.3	[MAC-Q06] Activation of entire NZP-CSI-RS resources within an LTM CSI resource configuration

In RAN2#130, it was agreed to use LTM-CSI-ResourceConfigId to specify the set of NZP-CSI-RS resources to be activated. While implementing the activation/deactivation MAC CE design, the same principle as the legacy SP CSI-RS/CSI-IM activation/deactivation MAC CE was adopted, i.e., deliver all TCI state IDs for NZP-CSI-RS resources within the LTM CSI configuration ID.

RAN2#130 chair’s note - Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE. LTM-CSI-ResourceConfig in RRC running CR SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE in MAC running CR Rapporteur response in email discussions [Rapp]: If I understand the comment correctly, this issue was discussed in RAN2#130, and the following agreement is achieved: - Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

Regarding rapporteur’s response, it is worth noting that the decision of using LTM CSI Resource Configuration ID was inevitable due to the fact that (i) NZP-CSI-RS Resource Set ID for each candidate cell is not explicitly linked to LTM CSI Resource Configuration, and (ii) LTM NZP-CSI-RS Resource Set doesn’t have any ID associated and is subsumed into LTM CSI resource configuration as shown above.

Observation 3a: Due to the single LTM NZP-CSI-RS resource set within the LTM CSI resource configuration, the current SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE forces the serving cell to activate/deactivate the entire NZP-CSI-RS resources in the LTM CSI resource.

This “entire” activation/deactivation of NZP-CSI-RS resources within the LTM CSI resource configuration has some issues in SP CSI-RS management at serving cell:

- When the serving cell generates an LTM CSI resource configuration covering multiple candidate cells and their NZP-CSI-RS resources to cope with various UE mobility cases, then for the activation case, it has to communicate with all member candidate cells and negotiate the TCI state IDs, even though the serving cell wants to activate only a subset of NZP-CSI-RS resources of interest based on the previous L1 measurement results (e.g., SSB based measurements).

- The serving cell may choose to generate multiple LTM CSI resource configurations to avoid unnecessary SP CSI-RS activations, but this requires the serving cell to generate at least the same number of LTM CSI report configurations as those LTM CSI resource configurations. And this would lead to RRC signalling overhead and too many LTM CSI report configurations unnecessarily.

Observation 3b: This kind of entire activation/deactivation of NZP-CSI-RS resources would have some issues such that (i) the serving cell has to negotiate the TCI state IDs and NZP-CSI-RS resource IDs with all candidate cells unnecessarily if the LTM CSI resource config has many candidate cells to cover various mobility scenarios, or (ii) the serving cell has to configure many LTM CSI resource configurations covering a smaller set of candidate cells at the expense of those many LTM CSI report configurations.

Abovementioned issues are specific to LTM SP CSI-RS resources, as in the legacy SP CSI-RS, each CSI resource configuration can have a list of NZP-CSI-RS resource sets and the activation/deactivation is done per NZP-CSI-RS resource set level, which naturally allows the partial activation/deactivation. We believe the “partial” activation/deactivation of NZP-CSI-RS resources should be also discussed and supported for LTM for the sake of more flexible and efficient SP CSI-RS/CSI-IM resource management.

Legacy CSI-ResourceConfig in RRC Legacy SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

Observation 3c: For the legacy SP CSI-RS, a CSI resource configuration can have a list of NZP-CSI-RS resource sets and the activation/deactivation is done per NZP-CSI-RS resource set level, which supports the partial activation/deactivation of NZP-CSI-RS resources within the CSI resource configuration.

Proposal 3: RAN2 to agree the support of partial activation/deactivation of SP CSI-RS resources within LTM CSI resource configuration.

# 3	Open issues on event-triggered measurement reporting

## 3.1	[MAC-Q01] UE variable and measurement report procedure for Event LTM2

In MAC running CR, there are UE variables defined in clause 5.x.3 of MAC running CR, which are to be used for describing the UE procedure for LTM event evaluation and reporting in clause 5.x.3 and 5.x.4. And it was noticed that some parameters’ descriptions are only applicable to event conditions of which fulfilment is based on the candidate RS measurement, i.e., Event LTM3/4/5.

The UE variables captured in the snippet below have the description on the reference signal resource information of LTM candidate cell(s) only, therefore the information of the current beam of serving cell for Event LTM2 is not covered at all. Since these lists are used for L1 measurement reporting procedure commonly applicable to all event types in clause 5.x.4, the reference signal (current beam of serving cell) for Event LTM2 should be also included.

Unlike the candidate RS, the current beam RS is not strictly tied with a particular RS index, but rather “floating” w.r.t. the indicated TCI state based on the unified TCI state framework. Therefore, the description about the SSBRI or CRI of concerned beam(s) in clause 5.x.3.1 is not applicable to the current beam for Event LTM2, as the RS index could keep changing.

Clause 5.x.3.1 snippet in MAC running CR - MR_LIST: includes the list of event triggered L1 measurement report information, including RS resource index of LTM candidate cell(s), L1 measurement result, and type of reporting RS(s) as defined in 6.1.3.x, for which the L1 measurement report triggering conditions have been met for TTT. Each entry in the list is associated with a ltm-CSI-ReportConfigId; - BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report; - BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report; - BEAM_REPORTED_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have not been met for TTT; […] 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event: 3> if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event): 4> include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId; 3> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> if the beam is in BEAM_LEAVING_LIST: 4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId;

Observation 4a: The description on 4 lists in clause 5.x.3.1 (MR_LIST, BEAM_ENTERING_LIST, BEAM_LEAVING_LIST, BEAM_REPORTED_LIST) is not applicable to Event LTM2 as they only discuss RS index from LTM candidate cells.

Observation 4b: The procedure in clause 5.x.3.1 about the SSBRI or CRI of the concerned beam(s) is not applicable to Event LTM2 either, as the current beam RS index is not fixed but “floating” w.r.t. the indicated TCI state ID.

Proposal 4: RAN2 to agree including additional description on the serving beam information for Event LTM2 in clause 5.x.3.1 and 5.x.4 and adopt the TP in Annex B.1.

## 3.2	[MAC-Q02] Measurement results reporting for Type II in regular L1 MR MAC CE

In RAN2#128, it has been agreed that NW controls whether the beam(s) not satisfying the event could be reported or not, and the RRC parameter allowReportAnyBeam has been captured in current MAC running CR.

RAN2#128 chair’s note - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. RAN2#131 chair’s note - RAN2 will keep the current definition of Type II in MAC, i.e. any beam. Clause 5.x.4 snippet in MAC running CR ### 5.x.4 Measurement report […] - allowReportAnyBeam: whether the UE can report the measurement results for the beams not satisfying the conditions of the events; […] For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall: 1> if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled: 2> if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.x according to the measurement report information in the MR_LIST; […] 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists; Rapporteur response in email discussions [Rapp]: No issue is forseen. The corresponding description has been already captured in 6.1.3.x, while there is no Type 01/10/11 in 5.x.4.

While the rapporteur responded that the description is correctly captured in 6.1.3.x, when checking the measurement report procedure for the regular L1 MR MAC CE in clause 5.x.4, it states that the regular L1 MR MAC CE is generated according to the measurement report information in the MR_LIST only. As shown in section 3.1 earlier, MR_LIST only contains the measurement result and beam type of RS(s) that have satisfied the event condition for TTT. Therefore the current description in regular L1 MR MAC CE doesn’t support the reporting of beams with Type II, while the procedure for truncated L1 MR MAC CE does capture the reporting of beams not satisfying the event condition at all.

Observation 5: Unlike the truncated L1 MR MAC CE, the measurement report procedure for regular L1 MR MAC CE in clause 5.x.4 states to take the measurement report information in the MR_LIST only, which doesn’t support the reporting of beams that are not satisfying the event condition when allowReportAnyBeam is configured.

Proposal 5: RAN2 to agree modifying the description on the L1 MR MAC CE generation to cover reporting beams not satisfying the event condition and adopt the TP for clause 5.x.4 in Annex B.3.

# Conclusion

This contribution discussed the remaining issues in MAC for LTM. The following observations and proposals have been made:

Observation 1a: From rapporteur’s comment, previous RAN2 discussions/agreements on “no differentiation for SP CSI-RS between L1-RSRP and CSI acquisition” was mainly discussing the SP CSI-RS activation/deactivation MAC CE to be applicable to both L1-RSRP and CSI acquisition, which makes sense and in line with the legacy SP CSI-RS/CSI-IM activation/deactivation MAC CE.

Observation 1b: The current text in MAC running CR is working only if UE is configured with early CSI acquisition for target cell. In case UE has activated SP CSI-RS for L1-RSRP from target cell and the target cell doesn’t configure early CSI reporting, SP CSI-RS deactivation timing is undefined.

Observation 1c: L1-RSRP measurement report is only for the serving cell, so even for the target cell, activated SP CSI-RS/CSI-IM resources should be deactivated during LTM cell switch.

Proposal 1: RAN2 to agree the deactivation of SP CSI-RS for the target cell for the L1-RSRP purpose and adopt the TP for clause 5.18.x in Annex A.1.

Observation 2a: The current MAC running CR is interpreted as if UE needs to assume CSI Resource Configuration ID2 to be present as long as UE is configured with SP CSI-IM resource set for candidate cells in RRC.

Observation 2b: Such implicit determination rule would cause either unnecessary presence of CSI Resource Configuration ID2 (e.g., SP CSI-RS for L1-RSRP which doesn’t require CSI-IM at all) or unnecessary extra processing of validating the corresponding LTM CSI resource configuration to CSI Resource Configuration ID1.

Proposal 2:  RAN2 to agree replacing “A/D” to “IM” in Octet 2 for the explicit indication of the presence of CSI Resource Configuration ID2 and adopt the TP for clause 6.1.3.12a in Annex A.2.

Observation 3a: Due to the single LTM NZP-CSI-RS resource set within the LTM CSI resource configuration, the current SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE forces the serving cell to activate/deactivate the entire NZP-CSI-RS resources in the LTM CSI resource.

Observation 3b: This kind of entire activation/deactivation of NZP-CSI-RS resources would have some issues such that (i) the serving cell has to negotiate the TCI state IDs and NZP-CSI-RS resource IDs with all candidate cells unnecessarily if the LTM CSI resource config has many candidate cells to cover various mobility scenarios, or (ii) the serving cell has to configure many LTM CSI resource configurations covering a smaller set of candidate cells at the expense of those many LTM CSI report configurations.

Observation 3c: For the legacy SP CSI-RS, a CSI resource configuration can have a list of NZP-CSI-RS resource sets and the activation/deactivation is done per NZP-CSI-RS resource set level, which supports the partial activation/deactivation of NZP-CSI-RS resources within the CSI resource configuration.

Proposal 3: RAN2 to agree the support of partial activation/deactivation of SP CSI-RS resources within LTM CSI resource configuration.

Observation 4a: The description on 4 lists in clause 5.x.3.1 (MR_LIST, BEAM_ENTERING_LIST, BEAM_LEAVING_LIST, BEAM_REPORTED_LIST) is not applicable to Event LTM2 as they only discuss RS index from LTM candidate cells.

Observation 4b: The procedure in clause 5.x.3.1 about the SSBRI or CRI of the concerned beam(s) is not applicable to Event LTM2 either, as the current beam RS index is not fixed but “floating” w.r.t. the indicated TCI state ID.

Proposal 4: RAN2 to agree including additional description on the serving beam information for Event LTM2 in clause 5.x.3.1 and 5.x.4 and adopt the TP in Annex B.1.

Observation 5: Unlike the truncated L1 MR MAC CE, the measurement report procedure for regular L1 MR MAC CE in clause 5.x.4 states to take the measurement report information in the MR_LIST only, which doesn’t support the reporting of beams that are not satisfying the event condition when allowReportAnyBeam is configured.

Proposal 5: RAN2 to agree modifying the description on the L1 MR MAC CE generation to cover reporting beams not satisfying the event condition and adopt the TP for clause 5.x.4 in Annex B.3.

# References

- Chairman Notes; TSG RAN WG2 Meeting #130

- R2-2506229, Running MAC CR for Mob Ph4 after RAN2#131, Rapporteur (Vivo)

# Annex A.1		TP for Proposal 1

## 5.18.x	Activation/Deactivation of Semi-Persistent CSI-RS/CSI-IM resource set for candidate cell

[…]

After reconfiguration with sync that is triggered by LTM, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for all candidate cell(s) including the target cell, except the ones for CSI reporting at target cell, are deactivated. After CSI reporting at the target cell after or during cell switch triggered by LTM as specified in clause 5.2.4a in TS 38.214, the configured Semi-Persistent CSI-RS/CSI-IM resource sets for the target cell are deactivated.

[…]

# Annex A.2 	TP for Proposal 2

## 6.1.3.12a	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE

The SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields (Figure 6.1.3.12a-1):

  - A/D: This field indicates whether to activate or deactivate the indicated SP CSI-RS resource set for the candidate cell(s) associated with the CSI Resource Configuration ID1 in the same octet. The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	CSI Resource Configuration ID1: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-RS resource set for the candidate cell(s) for measurement. The length of the field is 7 bits;

  - IM: This field indicates the presence of the CSI Resource Configuration ID2. If the IM field is set to 1, the CSI Resource Configuration ID2 is present in the same octet. If IM field is set to 0 and A/D field is set to 1, then TCI State ID0 is present in the same octet;

-	CSI Resource Configuration ID2: This field indicates the index of the LTM CSI resource configuration corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. This LTM CSI resource configuration includes an SP CSI-IM resource set for the candidate cell. If the IM field is set to 1, then this field is present. The length of the field is 7 bits;

-	TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set corresponding to LTM-CSI-ResourceConfigId as specified in TS 38.331 [5]. TCI State ID0 indicates TCI State for the first resource within the Semi Persistent NZP CSI-RS resource set, TCI State ID1 for the second one and so on. If the A/D field is set to 0, the octets containing TCI State ID field(s) are not present. The length of the field is 7 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.12a-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE

# Annex B.1		TP for Proposal 4

## 5.x.3	Measurement report triggering

### 5.x.3.1	General

The UE maintains the following UE variables for event triggered L1 measurement and report procedure:

-	MR_LIST: includes the list of event triggered L1 measurement report information, including RS resource index of LTM candidate cell(s) or the indicator of current beam RS of serving cell, L1 measurement result, and type of reporting RS(s) as defined in 6.1.3.x, for which the L1 measurement report triggering conditions have been met for TTT. Each entry in the list is associated with a ltm-CSI-ReportConfigId;

-	MR_SENT_COUNTER: represents the number of event triggered L1 measurement report performed by UE if the triggering condition for the corresponding event is met for TTT for each ltm-CSI-ReportConfigId;

-	BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) or the indicator of current beam RS of serving cell for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report;

-	BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) or the indicator of current beam RS of serving cell for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report;

-	BEAM_REPORTED_LIST: includes the reference signaling resource index of LTM candidate cell(s) or the indicator of current beam RS of serving cell for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have not been met for TTT;

[…]

2>	if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. the current beam indicator or reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event:

3>	if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

3>	include the SSBRI or CRI of the concerned beam(s), or the current beam indicator in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> if the beam is in BEAM_LEAVING_LIST:

4> remove the current beam indicator or the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

4> include the current beam indicator or the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

3>	initiate the measurement reporting procedure, as specified in 5.x.4.

2>	else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST for the measurement from lower layer during TTT defined for this event:

3> if the beam is in BEAM_ENTERING_LIST:

4> remove the current beam indicator or the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> if the beam is in BEAM_REPORTED_LIST:

4> remove the current beam indicator or the concerned beam(s) in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

4> include the current beam indicator or the SSBRI or CRI of the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if reportOnLeave is set to true for this ltm-CSI-ReportConfigId:

4>	initiate the measurement reporting procedure, as specified in 5.x.4;

2>	upon expiry of the periodical reporting timer for this ltm-CSI-ReportConfigId:

3>	initiate the measurement reporting procedure, as specified in 5.x.4.

NOTE x:	TTT is not restarted if the current beam of serving cell changes and the entry or leaving condition is still met with the new current beam.

NOTE y:	To evaluate the L1 measurement reporting triggering event, the UE uses the latest L1-RSRP measurement from lower layer.

## 5.x.4	Measurement report

[…]

3> if reportAmount is configured in the LTM-EventTriggeredPeriodicReport by RRC:

4> if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled:

5>	set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId;

4> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId by 1;

4> stop the periodical reporting timer, if running;

4> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount:

5> restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

3> include the SSBRI or CRI, or the current beam indicator in the BEAM_ENTERING_LIST and the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId is empty:

4>	remove the measurement reporting entry within the MR_LIST for this ltm-CSI-ReportConfigId;

4>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running;

3> cancel the triggered L1 measurement report;

[…]

# Annex B.3		TP for Proposal 5

## 5.x.4	Measurement report

[…]

For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall:

1>	if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST and RS(s) not in BEAM_ENTERING_LIST, BEAM_LEAVING_LIST, nor BEAM_REPORTED_LIST;

[…]