3GPP TSG-RAN WG2 Meeting #131bis	R2-2507496

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.8.2

Source:	Samsung

Title:	RIL S024, S025, C006

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss RIL S024, S025, C006.

# Discussion

## S024, S025

In the last meeting, we agree to support close reference location reporting.

The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

-	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

As mentioned in the RIL, reporting in RRCReconfiguration message in our understanding is optional. There are several reasons.

- UAI can only be send after AS security activation, however, RRCReconfigurationComplete message can be sent (unprotected) prior to AS security activation. This makes UE location, which can be approximated based on close reference location, expose to security risk. Thus, UE will not report in RRCReconfigurationComplete prior to AS security activation.

- UE has to comply the processing delay requirement of sending RRCReconfigurationComplete message, while there is no requirement of sending UAI. If the reference location list is not configured in RRCReconfiguration, UE has to acquire SIB2 for the reference location list. This delay depends on the periodicity of SIB. To meet the processing delay of sending RRCReconfigurationComplete, UE may not be able to include the report in RRCReconfigurationComplete.

- The IDC issue between GNSS and NTN UL transmission may cause extra delay to measure GNSS. UE may not be able to include the report in RRCReconfigurationComplete that has to meeting the processing delay requirements.

Observation 1: RRCReconfigurationComplete message can be sent (unprotected) prior to AS security activation, and should meet the processing delay requirements. Thus, UE will not report close reference locations in RRCReconfigurationComplete that is sent prior to AS security activation with the security risk, and may not be able to include the report in RRCReconfigurationComplete when there is extra delay to acquire SIB2 or due to GNSS IDC issue, etc.

Based on the above, reporting close reference locations in RRCReconfigurationComplete message should not be mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available.

Proposal 1a: Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available.

In addition, we notice the how to report in RRCReconfigurationComplete and in UAI should be aligned. And if the first report has not been sent in RRCReconfigurationComplete, UE should report via UAI. Thus, we propose the following procedural change.

In RRCConfigurationComplete:

2>	if the UE is configured in OtherConfig in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport, if available;

In UAI:

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so, and upon determining that the closest reference location(s) have changed compared with the last reported values.

…

1>	if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

2>	if the UE did not include referenceLocationReport in a RRCReconfigurationComplete message or in a UEAssistanceInformation message since it has been configured to provide location information; or

2>	if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration;

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide location information for assisted SMTC configuration in RRC_CONNECTED state according to 5.7.4.2;

2>	include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport;

One comment is that re-entering connected mode also triggers report in UAI. Since the report configuration is released upon RRC release, in our understanding UE will only report after connection setup/resume if the report configuration is included in this RRCReconfiguration.

Proposal 1b: Adopt the above changes for RIL S024, S025.

## C006

In RIL C006, it is proposed to remove the sentence “The total number of configurable SMTCs across smtc4list and smtc5list is 6.” in the field description of smtc5list. The maximum number configured SMTCs in smtc5list for idle/inactive is 6 can be restricted by the sequence length of smtc5list naturally.

We share the same view. The reason is that the total number of elements in smtc4list and smtc5list can be larger than 6. For example, smtc4list={a,b,c}, smtc5list={-,d,e,f,g,h}, and UE in this case applies {a,d,e,f,g,h}. Since the legacy UE can only apply smtc4list, NW will configure smtc4list with the restriction of only one periodicity for neighbor cell measurement. While Rel-19 DL-CE capable UE can apply smtc5list with two periodicities.

Observation 2: The legacy UE can only apply smtc4list, and NW will configure smtc4list with the restriction of only one periodicity for neighbor cell measurement. While Rel-19 DL-CE capable UE can apply smtc5list with two periodicities. The total number of elements in smtc4list and smtc5list in SIB can be larger than 6, and the maximum elements in smtc5list in SIB for idle/inactive is 6, and the maximum elements in smtc5list in MO for connected is 3.

We should clarify that UE shall setup SMTC according to smtc5list if both smtc4list and smtc5list are configured.

Proposal 2: Agree C006, and clarify in FD that UE shall setup SMTC according to smtc5list if both smtc4list and smtc5list are configured in SIB.

# Conclusion

Based on the discussion, we have the following proposal.

S024, S025:

Observation 1: RRCReconfigurationComplete message can be sent (unprotected) prior to AS security activation, and should meet the processing delay requirements. Thus, UE will not report close reference locations in RRCReconfigurationComplete that is sent prior to AS security activation with the security risk, and may not be able to include the report in RRCReconfigurationComplete when there is extra delay to acquire SIB2 or due to GNSS IDC issue, etc.

Proposal 1a: Reporting close reference locations in RRCReconfigurationComplete message is not mandatory to UE. UE only includes the report in RRCReconfigurationComplete when it is available.

Proposal 1b: Adopt the below proposed changes for RIL S024, S025.

In RRCConfigurationComplete:

2>	if the UE is configured in OtherConfig in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport, if available;

In UAI:

1>	if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

2>	if the UE did not include referenceLocationReport in a RRCReconfigurationComplete message or in a UEAssistanceInformation message since it has been configured to provide location information; or

2>	if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration;

C006:

Observation 2: The legacy UE can only apply smtc4list, and NW will configure smtc4list with the restriction of only one periodicity for neighbor cell measurement. While Rel-19 DL-CE capable UE can apply smtc5list with two periodicities. The total number of elements in smtc4list and smtc5list in SIB can be larger than 6, and the maximum elements in smtc5list in SIB for idle/inactive is 6, and the maximum elements in smtc5list in MO for connected is 3.

Proposal 2: Agree C006, and clarify in FD that UE shall setup SMTC according to smtc5list if both smtc4list and smtc5list are configured in SIB.

# References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- R2-250xxxx RIL review