3GPP RAN WG2 Meeting #131bis	R2-2507504

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	8.4.2

Source:	InterDigital, Inc.

Title:	RRC issues on LP-WUS

Document for:	Discussion

# 1. Discussion

In RAN2#130 following agreement related condition configuration was made [1].

It is up to NW to configure the condition for LP-WUS monitoring and/or R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading, as in the current RRC running CR.

Merging thresholds

In the remaining open issues, the RRC-8 is not closed [2]. The issue is FFS on whether/how to reduce the threshold number for LP-WUS/WUR.

During the RAN2#130 meeting, RAN2 already discussed this issue and has not been reached a conclusion [3]. From our view, RAN2 does not consider merging entry/exit conditions for LP-WUS monitoring and RRM measurement relaxation/offloading (e.g., seving/neighboring relaxation and/or serving cell offloading).

The LP-WUS monitoring and RRM measurement for relaxation/offloading have different objectives. First one is to monitor LP-WUS signal to wake-up MR and the other is to perform relaxed RRM measurement or measurement offoading with MR.

If two conditions are merged, this has a limitation of providing configurations when a UE is equipped with two LP-WURs. For example, a UE is configured to monitor LP-WUS with OOK-based WUR and configured to perform measurement with OFDM-based WUR. In this case, the network should ensure to configure two different thresholds for different LP-WURs, respectively.

Moreover, this merging thresholds or conditions can be handled by network implementation. For example, the network can configure the same/common value for both entry conditions of LP-WUS monitoring and RRM measurement relaxation/offloading.

Proposal 1: Do not merge thresholds for LP-WUS monitoring and RRM measurement relaxation/offloading.

# 2. Conclusion

Proposal 1: Do not merge thresholds for LP-WUS monitoring and RRM measurement relaxation/offloading.

# 3. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #130.

[2] R2-2505393, Discussion summary and list of RRC open issue for LP-WUS WUR (vivo)

[3] R2-2504739, Discussion report on [AT130][205][LPWUS] (vivo)