3GPP TSG-RAN WG2 Meeting #131bis	R2-2507510

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell, Huawei

Title:	RIL N091 and S038 on UAI for measurement gap skipping

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In the previous RAN4/RAN2 meeting, it was agreed to introduce UAI for measurement gap cancellation preference recommendation, which can include preference for multiple measurement gap configurations, and it is indicated per MG. Besides, a single prohibit timer was agreed for the UAI for measurement gap cancellation preference recommendation.

In this contribution, we discuss the issues identified during the ASN.1 review process: N091 and S038:

## N091 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. N091; XR; 1; UAI for measurement gap cancellation preference; R2-25xxxxx; Chunli WU (Nokia); V04; ToDo. [Description]: Current conditions for UAI triggering allow the UE to send the UAI if it has not sent it before or otherwise when the prohibit timer is not running. Considering now that we agreed single prohibit timer while the the UAI can include preference for mutliple measurement gaps, it makes sense to allow UAI to be sent for a newly configured measurement gap without delay even if UAI has been sent for other MG before. [Proposed Change]: Revise the first condition to allow such case or add the condition to allow UAI for a newly configured measuremeng gap with preference becomes available. 1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the preference for gap occasion cancellation ratio becomes available for at least one measurement gap configuration for which preference for gap occasion cancellation ratio has not been included in any UEAssistanceInformation message with gapOccasionCancelRatio previously transmitted by the UE; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio..

## S038 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S038; XR; 1; Checking T346o running status; Vinay (Samsung); V07; ToDo. [Description]: The timer not running condition should be addressed in bullet 1>, as it is a general condition and it is also consistent description as in the legacy spec #### 5.7.4.2 Initiation …. 1> if configured to provide its preference for gap occasion cancellation ratio: - 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or - 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: - 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.. [Proposed Change]: Revise to have T346o not running condition check as general and aligned with legacy. #### 5.7.4.2 Initiation …. 1> if configured to provide its preference for gap occasion cancellation ratio and T346o is not running: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio..

# 2	Discussion

As pointed out by RIL N091, the UAI can include preference recommendations for multiple MG configurations. The first bullet of the conditions states the UE initiates transmission of UAI if it did not transmit it before and starts the prohibit timer. When a new MG is configured, the previously transmitted UAI would prevent the UE from sending preference for the new MG even though preference for that MG has not been transmitted before, which we believe should not be the case. The intended behaviour should be the UAI could be sent for any newly MG configuration, then guarded by prohibit timer.

To solve the issue, the first bullet could be revised a bit to cover the case of not referring to the UAI message but refer to the UAI for the measurement gap configuration as shown in following TP:

1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration and if the UE has not transmitted a preference for gap occasion cancellation ratio for this measurement gap configuration in any UEAssistanceInformation message since it was configured to do so; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

Proposal 1: (RIL-N091) Adopt the TP above to change the first condition to transmission of UAI for preference for a measurement gap configuration other than the UAI message itself.

RIL S038 proposed to move the prohibit timer not running to the 1> level, which does not work well for the case above, unless the prohibit timer is reset when a new measurement gap configuration is configured.

Proposal 2: (RIL-S038) If the prohibit timer T346o checking is moved to the 1> level, it should be reset upon new MG configuration to avoid preventing reported preference for the newly configured MG, otherwise the specification should be left as it is.

# 3	Conclusion

RIL N091 and S038 are addressed in this contribution with the following proposals proposed:

Proposal 1: (RIL-N091) Adopt the TP above to change the first condition to transmission of UAI for preference for a measurement gap configuration other than the UAI message itself.

Proposal 2: (RIL-S038) If the prohibit timer T346o checking is moved to the 1> level, it should be reset upon new MG configuration to avoid preventing reported preference for the newly configured MG, otherwise the specification should be left as it is.