3GPP TSG-RAN WG2 Meeting #131bis	R2-2507512

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.3.1

Source:	Hanbat National University

Title:	Initial Considerations to Support User Plane Function in the AI-native RAN Architecture

Document for:	Discussion and Decision

# Introduction

In the SID of Study on 6G Radio [1], the following objective to study of radio interface protocol architecture and procedures for 6GR is defined.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we provide the user plane function and architecture of 6G Radio Access Network.

# Support of User Plane Function in the AI-native RAN Architecture

The traditional rule-based RAN management cannot adequately address the dynamic and heterogeneous requirements expected in 6G networks. Rapid traffic fluctuations, high mobility, and multi-cell interference expose the limitations of static algorithms, leading to degraded QoS and inefficient resource utilization. To overcome these challenges, the AI/ML framework for air interface was discussed in [2], however, it has a limitation to fully support AI-native functions in RAN-specific scenarios. Hence, in this contribution, we identify the AI-native architecture to support various intelligent user plane services.

In the AI-native RAN architecture, the RAN is essential to include the followings to support intelligent user plane services.

- Analytic and Intelligence Function Manager: orchestration, model management, inference control.

- Model Training: federated learning hub, deployed either inside RAN nodes or external servers.

- Inference Engine: real-time decision-making using local or global models.

- Data Processing Unit: handles measurement reports, KPIs, and training data.

Although the AI-native RAN provides system-level optimization, certain limitations remain in latency, personalization, and scalability in UE-perspective. Hence, to address these challenges, we consider the embedding AI-native functions within the UE. Local inference enables immediate decision-making for ultra-low latency services, avoiding round-trip delays between the UE and RAN. On-device training allows personalized optimization based on unique user conditions while preserving privacy by keeping raw data local. Furthermore, UE-side intelligence reduces signalling overhead by reporting only model updates or performance indicators instead of raw measurements. Finally, UEs with local intelligence enhance system resilience, maintaining autonomous operation even under weak or intermittent connectivity.

To support the AI-native RAN architecture based on the above functionalities, in the followed subsections, we propose information exchange function between the RAN and UE.

## Information Exchange in the AI-native RAN Architecture

To enable the intelligence of AI-native RAN, a variety of information related to the model distribution, training, inference, and performance monitoring is exchanged between the RAN and UE in order to ensure consistency and efficiency across both entities. From the network side, the RAN delivers trained global models and associated operational policies to the UE. This ensures that the UE can execute inference with up-to-date models while adhering to consistent operational policies. In the opposite direction, the UE provides the model update parameters forwarded to the network for aggregation and further processing in the global training loop, thereby contributing to federated model improvement across multiple nodes.

In addition to the model-related information, the RAN supports the exchange of inference requests and results. The UE may request inference execution by specifying the method and data context, while the RAN provides inference results when computations are performed on the network side. This bidirectional flow allows dynamic adjustment of where inference is executed depending on latency, computational capacity, or resource constraints. Moreover, the UE can report its own resource and capability status, including computational limitations and battery conditions, to the RAN so that the network can provide model distribution and inference policies accordingly.

Finally, performance indicators related to both training and inference are also communicated between UE and RAN. The UE provides information such as inference accuracy, latency, failure ratios, and learning convergence metrics, enabling the RAN to monitor service quality and feed these measurements back into the global learning cycle. In this way, the RAN establishes a closed feedback loop in which models, policies, requests, results, and performance metrics are continuously exchanged, ensuring that both the UE and the RAN maintain aligned, adaptive, and intelligent operations.

Proposal 1: RAN2 is encouraged to include the information exchange function in the AI-native RAN architecture. The information exchange function establishes a closed-loop feedback in which models, policies, inference requests and results, resource status, and performance metrics are continuously exchanged between the UE and RAN. This bidirectional exchange ensures that inference and learning are dynamically optimized according to latency, computational capacity, and service requirements, thereby maintaining consistent, adaptive, and intelligent RAN operations.

## Support the Reduced UE Capability for Information Exchange in the AI-native RAN Architecture

The AI-native RAN architecture should allow the reduced capability UE in which the local model training and/or inference function is removed. If the UE has no local model training, the RAN delivers trained global models and associated operational policies to the UE, ensuring that the UE can execute intelligent functions without performing local training. In the opposite direction, the UE provides operational data and performance indicators to the RAN. If the UE doesn’t have both local model training and inference function, the RAN executes inference on behalf of the UE. The RAN performs real-time inference using its global models, and the results are transmitted back to the UE for decision-making. This approach allows the location of intelligence to be optimized according to computational capacity, latency constraints, and service requirements, while significantly reducing the processing and energy burden on the UE.

The performance indicators related to service execution and UE operations are also communicated from the UE to the RAN. These indicators include service latency, reliability status, and other execution metrics, which enable the RAN to monitor quality of service and feed the measurements back into the training and inference cycles. In this way, the system establishes a closed-loop feedback process in which the UE contributes data and performance reports, while the RAN assumes responsibility for learning and inference, thereby ensuring consistent, adaptive, and intelligent operations across the network.

Proposal 2: RAN2 is encouraged to include the information exchange function to support the reduced capability UE operating without local model training and/or inference functions in the AI-native RAN architecture. In such cases, the RAN shall provide global models and policies, perform inference on behalf of the UE when needed, and establish closed-loop feedback by collecting operational data and performance indicators from the UE. This enables dynamic optimization of intelligence placement while reducing UE complexity and energy consumption, thereby ensuring consistent, adaptive, and intelligent RAN operations.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 is encouraged to include the information exchange function in the AI-native RAN architecture. The information exchange function establishes a closed-loop feedback in which models, policies, inference requests and results, resource status, and performance metrics are continuously exchanged between the UE and RAN. This bidirectional exchange ensures that inference and learning are dynamically optimized according to latency, computational capacity, and service requirements, thereby maintaining consistent, adaptive, and intelligent RAN operations.

Proposal 2: RAN2 is encouraged to include the information exchange function to support the reduced capability UE operating without local model training and/or inference functions in the AI-native RAN architecture. In such cases, the RAN shall provide global models and policies, perform inference on behalf of the UE when needed, and establish closed-loop feedback by collecting operational data and performance indicators from the UE. This enables dynamic optimization of intelligence placement while reducing UE complexity and energy consumption, thereby ensuring consistent, adaptive, and intelligent RAN operations.

# References

[1] RP-251881	New SID: Study on 6G Radio.

[2] TR 38.843 AI/ML for NR air interface.