3GPP TSG-RAN WG2 Meeting #131bis	R2-2507535

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.2.2

Source:			ASUSTeK

Title:	Discussion on Ambient IoT open issues

Document for: 	Discussion and Decision

- Introduction

In RAN2#131 meeting, the A-IoT R19 WI was considered completed from RAN2 point of view. There are some remaining open issues discussed in the post-meeting offline discussion [1]. In this contribution, we would like to discuss some of the open issues.

- Discussion

2.1 		Issue 3-7: More data indication handling

RAN2 has been discussing the scenario of delayed response for Write command. In TS 38.391, upon reception of an R2D Upper Layer Data Transfer message, the device would send a D2R Upper Layer Data Transfer message with 0 SDU and set MDI to 0 when upper layer data is unavailable. The reader, in response to receiving the D2R message with 0 SDU, may send a follow-up R2D Upper Layer Data Transfer message later to schedule another D2R Upper Layer Data Transfer message from the device without including the original command message.

In the last meeting, it was agreed to check if there is a case where NAS doesn’t provide a response at all. If such case exists, deadlock issue may happen since the reader may request the device to provide a response repeatedly while there is no response at all.

In TS 24.369 [2], responses from the device are required for commands including Read, Write and Disable. In response to receiving a command message, the device shall transmit a complete message or reject message. However, there are exceptions that the device ignores the AIoT NAS message (i.e., no response from upper layer) as quoted below.

# 6 Handling of unknown, unforeseen, and erroneous protocol data … ## 6.2 Message too short or too long ### 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3]. … ## 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.

In our understanding, the exceptions are specified for error handling, which does not correspond to a normal use case. We believe these errors are not intended by the network, and they are supposed to be corner cases. Moreover, if the corner cases really happen, it can still be solved by reader implementation. For example, if the device finds that a message is too short to decode in the upper layer, the device would ignore the message. For MAC layer operation, since the upper layer data is not available, the device sends a D2R Upper Layer Data Transfer message with 0 SDU and set MDI to 0. In response to receiving this D2R Upper Layer Data Transfer message, the reader may send a R2D Upper Layer Data Transfer message later to schedule another D2R Upper Layer Data Transfer message for the response. After receiving the R2D message, since the device does not have upper layer data, 0 SDU would still be sent. With several back and forth transmissions, the reader could determine that error occurs at the device and separate it from the delay response scenario. The reader should not provide follow up R2D Upper Layer Data Transfer message, and thus a deadlock would not happen.

Observation 1: The “no response” case from NAS would only happen for error handling, and this corner case could be handled by network implementation.

Proposal 1: (Issue 3-7) RAN2 does not handle issues that there is no response from the upper layer.

2.2 		Issue 5-4: End of R2D Upper Layer Data Transfer message monitoring

During the email discussion [1], some companies suggested to add the following text in TS 38.391.

## 5.4 A-IoT upper layer data procedure ### 5.4.1 General The purpose of this procedure is for a device to transmit or receive upper layer data. A device monitors for R2D Upper Layer Data Transfer message after transmission of D2R Upper Layer Data Transfer message until reception of a A-IoT Paging message if the device does not have a stored AS ID or, until the AS ID is released if the device has a stored AS ID.

Nonetheless, we think monitoring R2D messages is more of a A-IoT PHY behaviour. It is A-IoT MAC’s decision on whether to process the received messages or not, and such decision may not be restricted to whether AS ID is stored or not. Moreover, the suggested text seems to be confusing in some cases. For example, if the device receives NACK, it should release AS ID and not monitor for R2D Upper Layer Data Transfer message, according to the latter part of the TP. However, since the device does not have a stored AS ID now, it should monitor for R2D Upper Layer Data Transfer message, according to the former part of the TP. In fact, the device works fine under current text. Therefore, no spec change is needed.

Proposal 2: (Issue 5-4) Spec change for monitoring behaviour for R2D Upper Layer Data transfer message is not needed.

2.3 		Clarification on MAC PDU content

In 38.391, there seems to be ambiguity whether one R2D MAC PDU could include multiple R2D messages; for example, one R2D MAC PDU includes both NACK Feedback message and Access Trigger message. In RAN2#126 meeting, it was agreed that multiple “AIoT logical channels” for upper layer data were not supported. Moreover, since there is no length field for a R2D message with variable size, it’s not possible for the device to decode a R2D MAC PDU with multiple R2D messages. As a result, one R2D MAC PDU can include only one R2D message, which could be specified in the spec.

Proposal 3: Adopt the following TP to clarify that one R2D MAC PDU includes one R2D message.

### 6.1.1 General … The R2D message type represents the set of A-IoT MAC messages that are sent from the reader to the device on the R2D transport channel. One R2D MAC PDU includes one R2D message (i.e., multiplexing different R2D messages in one MAC PDU is not allowed.) The R2D message names and the values of R2D message type are specified in Table 6.1-1.

- Conclusion

We discuss some remaining issues and have the following observation and proposals for A-IoT MAC:

Observation 1: The “no response” case from NAS would only happen for error handling, and this corner case could be handled by network implementation.

Proposal 1: (Issue 3-7) RAN2 does not handle issues that there is no response from the upper layer.

Proposal 2: (Issue 5-4) Spec change for monitoring behaviour for R2D Upper Layer Data transfer message is not needed.

Proposal 3: Adopt the following TP to clarify that one R2D MAC PDU includes one R2D message.

### 6.1.1 General … The R2D message type represents the set of A-IoT MAC messages that are sent from the reader to the device on the R2D transport channel. One R2D MAC PDU includes one R2D message (i.e., multiplexing different R2D messages in one MAC PDU is not allowed.) The R2D message names and the values of R2D message type are specified in Table 6.1-1.

- Reference

- R2-2507030, “Summary of A-IoT MAC open issues (outcome of [POST131][021][AIoT] MAC spec)”

- TS 24.369, v1.0.0, “Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3; (Release 19)”