3GPP TSG-RAN WG2 Meeting #131bis	R2-2507545

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		10.3.3

Source:		Futurewei

Title:		Discussion on 6G Data Collection and Management

Document for:		Discussion and Decision

# Introduction

According to 6G SID [1], RAN2 is the lead WG responsible for the following common aspects of 6G radio interface design:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this paper, we discuss the data collection and management aspect of the AI/ML framework for 6G RAN.

# Discussions

## AI/ML Framework defined in 5G

In R18 and R19, RAN has done extensive study and normative work on the application of AI/ML to the NR air interface [3]. The study establishes a foundational framework for integrating AI/ML technologies, evaluates their benefits through specific use cases, and assesses the potential impact on 3GPP specifications.

The core of the study is the development of a general AI/ML framework, encompassing a detailed Life Cycle Management (LCM) process for AI/ML models. It encompasses the following functional blocks, as shown below in Figure 1 [3].

- Data Collection: Gathering data for training, inference, monitoring, and updates.

- Model Training: The process of creating a trained model from data.

- Model Delivery/Transfer: Procedures for moving a model from one entity to another (e.g., network to UE).

- Model Inference: Using a trained model to generate outputs from new inputs.

- Model Management: Oversees the operation (selection, activation, fallback) and monitoring of AI/ML models and functionalities.

- Model Storage: A logical function for storing trained and updated models.

Figure 1. Functional framework for AI/ML for NR Air Interface

Three key use cases were selected to validate the framework and quantify performance gains for the PHY air interface, CSI Feedback Enhancement, Beam Management (BM), and Positioning Accuracy Enhancements.

In R19, RAN2 has also studied the application of AI/ML to enhance UE mobility in NR systems [4]. The study's primary goals were to investigate AI/ML's potential to reduce UE measurement efforts and to improve handover (HO) performance, specifically by reducing the Handover Failure (HOF) rate. The study focused on three main use cases:

- RRM measurement prediction,

- measurement event prediction, and

- Radio Link Failure (RLF) prediction, with the latter being deprioritized.

Evaluations conducted via extensive simulations demonstrate that AI/ML models significantly outperform non-AI baseline methods (e.g., sample-and-hold) in prediction accuracy, particularly for longer prediction windows and higher UE speeds.

Both the PHY air interface project and the AI/ML-based mobility project have left some problems unsolved or areas that need further enhancements in the future releases. In the rest part of this section, we will discuss these areas/aspects, focusing on the aspects related to data collection and transfer, and propose working directions for 6G.

## 5G Data Collection and Limitations

Data collection is a key function within the general AI/ML framework. It is defined as the process of collecting data by NW nodes, a management (e.g., training, inference, and monitoring) entity, or the UE for purposes such as AI/ML model training, data analytics, and inference.

In 5G/5G-A, data collection for both NW-side and UE-side models have been studied.

NW-side data collection supports models where inference is performed entirely at the network or for NW-side model training, monitoring, and inference. NW-side data collection needs generally consider UE support for data logging and the ability to report data periodically, based on events, or on-demand. Practical constraints include signaling overhead, energy consumption, processing power, and UE memory size.

UE-side data collection supports models where inference is performed entirely at the UE, and is essential for training, monitoring, or inference of UE-side models or UE parts of two-sided models. The primary focus for UE-side model training data transfer is the process of collecting data at the UE and delivering it to a training entity. RAN2 studied several methods for transferring training data collected by the UE; both UP and CP solutions have been considered.

- Option 1a (Direct to OTT Server): The UE collects and directly transfers data to an entity outside the MNO (e.g., an OTT server). This typically uses a UP tunnel if transferred via the 3GPP network, and no 3GPP specification impact is expected.

- Option 1b (MNO Server): The UE collects data and transfers it to a server for UE-side model training inside the MNO, which may then optionally transfer it to an OTT server. This path typically utilizes a UP tunnel.

- Option 2 (Via Core Network): The UE collects data and transfers it to the CN, which forwards it to the training server/OTT server. A CP tunnel may be used if the data volume is manageable within NAS signaling capacity.

- Option 3 (Via OAM): The UE collects data and transfers it to the OAM (via the gNB), which then forwards it to the training server/OTT server. A CP tunnel may be used if the data volume is within RRC signaling capacity.

Due to time constraints and the fact that the feasibility of some of the mechanisms listed above need confirmation from other WGs and even other TSGs (such as SA), there are some areas need further studies in 6G. For example, for UE-side model training,

- For Option 1a, the NW has neither controllability nor visibility, which are demanded by the carriers. In addition, it is not clear how to configure/activate/deactivate data collection, as data collection happens in the UP and hence outside 3GPP.

- For Option 1b, the following additional aspects are not clear

  - Where the Server for Data Collection resides; some think inside the MNO, but others are not sure.

  - How the UP tunnel will be established (currently under SA study).

  - How controllability and visibility can be achieved.

- For Option 2 and 3, the following additional aspects are not clear

  - While CP solutions have been agreed (NAS layer for Option 2 and RRC layer for Option 3), its feasibility has not be concluded.

  - CP solution is not ideal for big-size data (multiple companies raised the issue).

  - Using CP for data collection could congest the CP for other control messages.

  - RAN2 cannot reach consensus the feasibility of UP solutions (UP tunnels), which is under SA study.

  - RAN2 cannot reach consensus on controllability and visibility.

## Business drivers and challenges for 6G data services

As we move into the era of 6G, collecting data from wireless networks faces challenges from both the business side and the technical side.

From the business side, 6G's inherent perception and intelligence, enabled digital twins, holographic communications, industrial IoT and other new applications bring massive perception data and AI data, as well as network operation data. These non-communication session data contain huge value and require efficient data control mechanisms to realize data monetization. As a core asset, 6G networks will drive network intelligence, data sharing and monetization, improve the network's own operational efficiency.

From the technical side, 6G's new data types bring challenges to the existing user plane; the existing communication network lacks a normalized data service framework and cannot process the newly added perception data and AI model data of 6G. In addition, the data collection mechanism and processing functions in the existing architecture are rigid and lack flexibility and cannot meet the needs of "on-path computing" (i.e., processing functions exist in the relaying nodes when delivering data from one end point to another) and "arbitrary topology" (i.e., any node in the network can be the data producer/source and data consumer/destination).

Proposal 1. Study traffic distribution of 6G and whether existing session-centric framework for data collection and transfer (i.e., using UP tunnels and PDU sessions) needs to be enhanced/changed (e.g., to session-less framework) to accommodate the new traffic distribution (if there is one) and diverse types of data/services.

Proposal 2. Study the necessity of on-path computing/processing (e.g., functions such as data fusion and de-privacy) for data collection and transfer in 6G.

Proposal 3. Study the necessity of supporting arbitrary topology for data collection/delivery and service in 6G.

## 5G Data Collection Framework

Under the data collection framework of 5G/5G-A, the CP handles tasks such as establishing connections and controlling the forwarding of UP data (commonly referred to as session management within the 5G core). The UP is designed to forward network packets to their intended destinations efficiently. Both the CP and UP operate within a communication session-centric framework; the CP sets up UP data tunnels for each session, facilitating signaling message exchange among various entities, including UEs, BSs, and CN functions, to maintain state synchronization.

Figure 2. The architecture of 5G

As shown in Figure 2, the SBI acts as a communication bus, using the HTTP/2 protocol for communication across all the CP functions. HTTP/2 connections are peer-to-peer connections, with messages flowing in both directions between NFs.

Figure 3. Data transfer in 5G

As shown in Figure 3, data traffic on the UP is carried by protocol data unit (PDU) sessions established between the UEs and the user plane function (UPF) deployed in the core network (CN) across the radio access network (RAN).

This existing CP/UP framework/collaboration for data collection may not be enough to support data collection needs in 6G.

Proposal 4. If session-less framework is necessary for 6G, study the changes/enhancements needed for CP and UP, or whether a new plane dedicated to data collection/management (i.e., data plane) is needed.

Proposal 5. Study necessary enhancements to network elements (e.g., NB, UE and CN) to enable session-less transmission and support of arbitrary topology (if supported) for data collection/delivery and service in 6G.

## Unified Framework for Data Collection/Delivery

Because data collection in 6G involves diverse types of data (e.g., AI/ML, sensing etc) and supports different types/levels of services (e.g., for wireless network itself, for service promoted to the application level etc), a unified framework for data collection and management is required to avoid multiple data collection systems. In fact, this has been a topic for 5G/5G-A and RAN didn’t have enough time to explore within the 5G timeframe. We believe the topic should be further studied in 6G.

Proposal 6. Support a unified framework for data collection/delivery and model transfer in 6G.

## Potential inter-WG and/or inter-TSG dependencies/impacts

Based on our experience with past AI/ML data collection related studies in 5G/5G-A, most of the use cases in RAN involve collaborations among RAN1, RAN2 and RAN4. Dependencies on SA work are also anticipated for topics such as unified data collection framework, which is also a topic under SA study for 6G.

# Conclusions

Proposal 1. Study traffic distribution of 6G and whether existing session-centric framework for data collection and transfer (i.e., using UP tunnels and PDU sessions) needs to be enhanced/changed (e.g., to session-less framework) to accommodate the new traffic distribution (if there is one) and diverse types of data/services.

Proposal 2. Study the necessity of on-path computing/processing (e.g., functions such as data fusion and de-privacy) for data collection and transfer in 6G.

Proposal 3. Study the necessity of supporting arbitrary topology for data collection/delivery and service in 6G.

Proposal 4. If session-less framework is necessary for 6G, study the changes/enhancements needed for CP and UP, or whether a new plane dedicated to data collection/management (i.e., data plane) is needed.

Proposal 5. Study necessary enhancements to network elements (e.g., NB, UE and CN) to enable session-less transmission and support of arbitrary topology (if supported) for data collection/delivery and service in 6G.

Proposal 6. Support a unified framework for data collection/delivery and model transfer in 6G.

# References

RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, et al., RAN#109.

TR 33.809 V18.1.0, Study on 5G security enhancements against False Base Stations (FBS).

TR 38.843 V19.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,  2025-09

TR 38.744 V1.2.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, 2025-09