3GPP TSG-RAN WG2 Meeting #131bis	R2-2507579

# Prague, Czech Republic, Oct 13th - 17th, 2025

Source: KDDI Corporation

Title:　 Considerations on User Plane for 6GR

Agenda Item:	10.3.1

Document for:	Discussion

- Introduction

The new SID for “study on 6G Radio” was approved in RAN#108 meeting and was revised in RAN#109 meeting, Its objective is to develop 6G Radio (6GR) to meet the requirements of the next generation of RAN, including those related to the control plane. In this contribution, we would like to share our initial considerations on several aspects of the user plane design for 6GR.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Discussion

  - General principle for 6GR protocols stack

According to the report provided by Ericsson[1], mobile traffic is expected to increase significantly, with video traffic remaining a dominant contributor. Although video coding technology has advanced, it is still of interest to explore mechanisms within the U-Plane to further reduce protocol overhead. When reviewing the proposals from the 6G workshop, some are proposing to eliminate SDAP, some are proposing to merge PDCP and RLC, and there are also some voicing that  maybe we do not need MAC. We understand that the purpose of all the proposals is trying to minimize the overall U-plane overhead to support scalable and efficient data transmission in 6G.

Proposal 1: RAN2 aims to reconsider the functions of each layer within the protocol stack with the purpose of minimizing the overall U-plane overhead.

  - General principle for 6GR MAC layer

Up to 82 MAC CEs have been defined in 5G, but many of them are rarely used in practice. This complexity not only increases the implementation burden but also leads to higher costs due to the need for extensive coordination across various network components. In Rel-19, MAC CE containing key information was specified to be transmitted to the UE without any ciphering mechanism. This raises concerns about potential security risks in future 6G systems.

Figure 1 mobile traffic by application category [1]

Moreover, it appears that MAC CEs are gradually taking over some of the responsibilities traditionally handled by the RRC layer—except for configurations.

At the initial phase of 6GR discussions, it is important to reconsider the actual role of the MAC layer. Questions arise as to whether MAC should carry such a broad set of responsibilities, whether the current number of MAC CEs is truly necessary, and whether it is appropriate for MAC CEs to carry key information without adequate security protection.

We suggest that appropriate security mechanisms for MAC CEs be considered. However, this may result in redundant security implementations across both the RRC and MAC layers, which could further increase complexity and implementation cost.

Proposal 2: RAN2 reconsider the responsibility of MAC layer, if key information is required to be delivered in MAC, appropriate security mechanism, taking into account of the complexity of implementation cost, should be introduced.

- 3. Conclusion

Proposal 1: RAN2 aims to reconsider the functions of each layer within the protocol stack with the purpose of minimizing the overall U-plane overhead.

Proposal 2: RAN2 reconsider the responsibility of MAC layer, if key information is required to be delivered in MAC, appropriate security mechanism, taking into account of the complexity of implementation cost, should be introduced.

- 4. Reference

[1] Ericsson Mobility Visualizer - Mobility Report - Ericsson