3GPP TSG-RAN WG2 Meeting #131bis	R2-2507629

Prague, Czech Republic, Oct 13th - 17th, 2025

Agenda item:		8.7.2

Source:		Samsung

Title:			RRC Corrections for XR

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we discuss the following RILs for XR RRC:

- [N091] and [S038] pertain to UAI mechanism for preference for gap cancellation ratio

- [V050] concerns the coexistance of remaining time based RLC retransmission and polling

- [V051] relates to restriction on ul-RateQueryConfigList-r19

# 2 Discussion

## 2.1 [N091] and [S038]

These two RILs concern the UAI for preference for gap occasion cancellation ratio.

[N091] argues to allow UAI to be sent for a newly configured measurement gap “without delay” even if UAI has been sent for other MG before.

1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the preference for gap occasion cancellation ratio becomes available for at least one measurement gap configuration for which preference for gap occasion cancellation ratio has not been included in any UEAssistanceInformation message with gapOccasionCancelRatio previously transmitted by the UE; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

Whereas, [S038] reasons that the T346o not running check should be addressed in bullet 1>, as it is a “general condition” and it is consistent description as in the legacy specification.

1> if configured to provide its preference for gap occasion cancellation ratio and T346o is not running: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

As far as [N091] for sending a UAI for a newly configured measurement gap is concerned, in our understanding this should also be regulated with the T346o running condition. It can be noted that the UAI for gap occasion cancellation ratio is not a critical information and there is no reason this needs to violate the T346o not running condition check.

Further, we understand the below existing specification text is encompassing any further or new configuration for gap occasion ratio as well.

2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration

Therefore, there is no need to make any amendment as asked in [N091]. While adopting [S038] also addresses this generic configuration scenario as well and ensures the conformity with the legacy specifications for T346o description.

Proposal 1: [N091] No specification change is considered for UAI to be sent without delay for a new configuration of gap occasion ratio.

Proposal 2: [S038] Agree to keep T346o not running check as general condition and place it in 1> bullet. (Adopt TP1)

## 2.2 [V050]

It is proposed that that when both remaining time based RLC polling and remaining time based RLC retransmission are configured for the same Tx RLC entity, the threshold for enabling remaining time based RLC retransmission should be set as lower than that for enabling remaining time based RLC polling.

remaingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds. The network configures remaingTimeThresholdRLC-Polling to be lower than remainingTimeThresholdRLC-ReTx, if it is configured.

In our understanding, there is no need to be explicit about setting any conditions on the value configurations for remaining time based ReTX and polling. Configured in either order, each of these triggers can be effective in avoiding retransmission delay. That is, remaining time based polling before ReTX may avail STATUS if succesful or may still require ReTX when STATUS is received. Also, remaining time based ReTX before polling may help if succesful or may still be benefitied with polling upon failure. It is, therefore, reasonable and prudent to leave the remaining time based ReTX and polling configuration settings to the network implementation.

Proposal 3: [V050] Not agree to specify any conditions on the value configurations for remaining time based ReTX and polling. Leave this to network implementation.

## 2.3 [V051]

The point is raised that when both ul-RateControlConfigList and ul-RateQueryConfigList are configured, the QoS flow configured in the rate query configuration should be the subset of QoS flow configured for the rate control. Otherwise, the QoS flow in the rate control query MAC CE is useless.

ul-RateQueryConfigList Includes the list of QoS flows for which the UL rate query is supported. The QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control.

While it seems to make sense that only those QoS flow with rate query(ies) should be configured in ul-RateQueryConfigList for which network has intention for applying rate control as configured via ul-RateControlConfigList. That is, QoS flow(s) configured in rate query would be the subset of QoS flow(s) configured for rate control. However, this does not warrant an explicit specificaiton and can be safely left to network implementation.

Proposal 4: [V051] Not agree to specify that QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control. Leave this to network implementation.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

Proposal 1: [N091] No specification change is considered for UAI to be sent without delay for a new configuration of gap occasion ratio.

Proposal 2: [S038] Agree to keep T346o not running check as general condition and place it in 1> bullet. (Adopt TP1)

Proposal 3: [V050] Not agree to specify any conditions on the value configurations for remaining time based ReTX and polling. Leave this to network implementation.

Proposal 4: [V051] Not agree to specify that QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control. Leave this to network implementation.

# 4 Annex (Text Proposals)

TP1:

#### 5.7.4.2 Initiation …. 1> if configured to provide its preference for gap occasion cancellation ratio and T346o is not running: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.