3GPP TSG-RAN WG2 Meeting #131bis 	 R2-2507648

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	8.8.1

Source:	THALES

Title:	Report of [Post131][301][R19 NR NTN] Open issues for Stage2 (Thales)

Document for:	Discussion and Decision

# Introduction

This contribution is intended to kick off Phase 2 of the following post-email discussion, and is aimed at collecting open issues related to stage 2 description for NR NTN phase 3.

- [Post131][301][R19 NR NTN] Stage2 CR (Thales)

Scope: finalize the running Stage 2 CR and merge with RAN3 CR

Intended outcome: Agreed CR

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

Companies are invited to provide input on the open issue list by Oct. 1st 00:00 UTC.

# Open issues

From the rapporteur's perspective, there are currently no open issues related to stage 2. Companies are invited to provide inputs on descriptions of their identified open issues and the corresponding proposed solutions to the CR [1].

| Issue (Company) | Identified open issues | Proposed solutions | Rapporteur’s comments |
| --- | --- | --- | --- |
| Issue#1 (vivo) | RAN1 has agreed to apply common PDCCH repetition and SIB1 repetition in TN. However, the current stage-2 spec restrict the use of common PDCCH repetition and SIB1 repetition to FR1-NTN band. This results in a misalignment between RAN1 spec and RAN2 spec. | We suggest capturing the support of common PDCCH repetition and SIB1 repetition for FR1 band in Section 19 (thus removing the common PDCCH repetition and SIB1 repetition bullets in sub-clause 16.14.9). | Agree |
| Issue#2 (vivo) | The following agreement should be captured in sub-clause 16.10.6.5.1 of stage-2 spec UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. | ##### 16.10.6.5.1 Service Continuity in RRC_IDLE or RRC_INACTIVE In RRC_IDLE and RRC_INACTIVE, the UE applies the normal cell reselection rules with the following modifications: - the UE which is receiving or interested to receive MBS broadcast service(s) via PTM and can only receive these MBS broadcast service(s) via PTM while camping on the frequency providing these MBS broadcast service(s) is allowed to make this frequency highest priority when the conditions described in TS 38.304 [10] are met; - when the MBS broadcast service(s) which the UE is interested in are no longer available (after the end of the session), or the UE is outside of the ISA(s) associated with MBS broadcast service(s) it is receiving or interested to receive, or the UE is no longer interested in receiving the service(s), the UE no longer prioritises the frequency providing these MBS broadcast service(s). | Agree |
| Issue#3 (CATT) | Stage 2 spec doesn’t capture the measurement related enhancements (including 2 SMTC periodicities, 7 SMTC offsets and N closest location report). | Propose to add these enhancements under the section “16.14.3.3 Measurements”. | Agree. The statement on the support of 2 SMTC periodicities can be revised if RAN4 cannot finish the corresponding RRM requirements. |
| Issue#4 (Samsung) | As discussed in the last open issue discussion, the issue on whether OCC can be supported for RACH-less handovers. Would be good to address it. | Add the following to Section 16.14.X: OCC is applicable to RACH-less handovers, as described in clause 9.2.3.6. | Related to the discussion in RAN2#131 : Proposal 1: RAN2 to confirm OCC is applicable for RACH-less handovers by suitable network implementation. - QC is ok with this Can be discussed in the post email discussion / in the next meeting Proposal 2: RAN2 to consider documenting OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.. In the open issue discussion in [POST131][301]: - Two companies support (including the proponent) - Two companies think it is supported by default, up to network implementation (e.g. for inter-cell, intra-gNB handover) - Two companies think it is a non-essential feature that needs RAN1/RAN4 inputs We propose to have an online discussion of the proposal, especially on the possible impact on other WGs to support this feature. |

# 3 Conclusion

In this contribution, we have identified the following open issue list and have the following proposals,

Proposal 1	Endorse R2-2507649 to capture in the stage 2 specification the following:

- The PDCCH repetition and SIB1 repetition are also supported for TN in section 19

- The deprioritization of the frequency of a service the UE is interested in when the UE is outside ISA

- The related enhancements for measurements in Downlink Coverage enhancements

Observation 1	On the applicability of OCC to RACH-less handover, two companies support this (including the proponent), two companies think it is supported by default and no specs change is needed, and two companies think it is a non-essential feature and that further RAN1 and RAN4 inputs are needed

Proposal 2a	RAN2 to confirm if OCC is applicable for RACH-less handovers by suitable network implementation.

Proposal 2b	RAN2 to discuss if an LS to RAN1 and RAN4 is needed for confirmation (to support or not)

Proposal 2c	RAN2 to discuss if a clarification in the stage 2 specification is needed if Proposal 2a is agreed. It can be revised according to the LS replies from RAN1 and RAN4 if an LS to RAN1 and RAN4 is sent

# 4 Reference

[1] R2-2506592, Introduction of NR NTN phase 3