3GPP TSG-RAN WG2 #131bis	R2-2507663

# Prague, Czech Republic, October 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5559 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Network Energy Saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the corrections identified during the ASN.1 review per WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | The corrections proposed in the RILs given below are captured: X204, X205, C181, C182, N002, H125, H126, H100, H101, H102, J003, J004, V501, L203, L204, L205, L206, L209, L210, N003, S030, S031, S035, Z103, A101, A102 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network Energy Saving (Rel-19) enhancements are not supported correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.2, 5.2.2.4.28, 5.5.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR XXXX TS 38.304 CR XXXX TS 38.306 CR XXXX TS 38.321 CR XXXX TS 38.331 CR XXXX |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Scope

The present document specifies the Radio Resource Control protocol for the radio interface between UE and NG-RAN.

The scope of the present document also includes:

-	the radio related information transported in a transparent container between source gNB and target gNB upon inter gNB handover;

-	the radio related information transported in a transparent container between a source or target gNB and another system upon inter RAT handover.

-	the radio related information transported in a transparent container between a source eNB and target gNB during E-UTRA-NR Dual Connectivity.

The RRC protocol is also used to configure the radio interface between an IAB-node and its parent node [2], and the radio interface between an NCR-node and gNB [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[6]	ITU-T Recommendation X.680 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[7]	ITU-T Recommendation X.681 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[8]	ITU-T Recommendation X.691 (08/2015) "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[9]	3GPP TS 38.215: "NR; Physical layer measurements".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[14]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[15]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[16]	3GPP TS 38.211: "NR; Physical channels and modulation".

[17]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[18]	ITU-T Recommendation X.683 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications" (Same as the ISO/IEC International Standard 8824-4).

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	3GPP TS 36.101: "E-UTRA; User Equipment (UE) radio transmission and reception".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 37.324: "Service Data Adaptation Protocol (SDAP) specification".

[25]	3GPP TS 22.261: "Service requirements for the 5G System".

[26]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[27]	3GPP TS 36.304: "E-UTRA; User Equipment (UE) procedures in idle mode".

[28]	ATIS 0700041: "WEA 3.0: Device-Based Geo-Fencing".

[29]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[30]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[31]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation".

[32]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[33]	3GPP TS 36.104:"E-UTRA; Base Station (BS) radio transmission and reception".

[34]	3GPP TS 38.101-3 "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[35]	3GPP TS 38.423: "NG-RAN, Xn application protocol (XnAP)".

[36]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[37]	3GPP TS 36.423: "E-UTRA; X2 application protocol (X2AP)".

[38]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[39]	3GPP TS 38.101-2 "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[40]	3GPP TS 36.133:"E-UTRA; Requirements for support of radio resource management".

[41]	3GPP TS 37.340: "E-UTRA and NR; Multi-connectivity; Stage 2".

[42]	3GPP TS 38.413: "NG-RAN, NG Application Protocol (NGAP)".

[43]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[44]	3GPP TR 36.816: "Evolved Universal Terrestrial Radio Access (E-UTRA); Study on signalling and procedure for interference avoidance for in-device coexistence ".

[45]	3GPP TS 25.331: "Universal Terrestrial Radio Access (UTRA); Radio Resource Control (RRC); Protocol specification".

[46]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[47]	3GPP TS 38.340: "Backhaul Adaptation Protocol (BAP) specification"

[48]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[49]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[50]	IEEE 802.11-2012, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.

[51]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.0", December 2016.

[52]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[53]	3GPP TS 38.314: "NR; layer 2 measurements".

[54]	Void.

[55]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[56]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[57]	3GPP TS 24.587: " Technical Specification Group Core Network and Terminals; Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[58]	Military Standard WGS84 Metric MIL-STD-2401 (11 January 1994): "Military Standard Department of Defence World Geodetic System (WGS)".

[59]	3GPP TS 38.101-4 "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance Requirements".

[60]	3GPP TS 33.536: "Technical Specification Group Services and System Aspects; Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[61]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[62]	3GPP TS 36.306: "User Equipment (UE) radio access capabilities".

[63]	3GPP TS 38.174: "NR; Integrated Access and Backhaul (IAB) radio transmission and reception".

[64]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[65]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[66]	3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification".

[67]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2"

[68]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[69]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[70]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[71]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[72]	3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol".

[73]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[74]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[75]	3GPP TS 38.101-5: "User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[76]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[77]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[78]	3GPP TS 24.588: "Technical Specification Group Core Network and Terminals; Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[79]	3GPP TS 38.106: "NR; NR Repeater Radio Transmission and Reception".

[80]	3GPP TS 28.405: "Technical Specification Group Services and System Aspects; Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

2Rx XR UE: Two antenna port XR UE as specified in TS 38.101-1 [15].

A2X communication: A communication to support A2X services leveraging PC5 reference points, as defined in TS 23.256 [76]. A2X services are realized by various types of A2X applications, e.g., BRID or DAA.

Additional sidelink RLC bearer: If the sidelink PDCP entity is associated with two sidelink RLC entities, the additional sidelink RLC bearer is the RLC bearer configured by sl-RLC-BearerToAddModListSizeExt in sl-ConfigDedicatedNR, or sl-RLC-BearerConfigListSizeExt in SIB12 or in SidelinkPreconfigNR.

Aerial UE: UE performing Aerial UE communication, as defined in TS 38.300 [2], clause 16.18 and TS 23.256 [76].

AM MRB: An MRB associated with at least an AM RLC bearer for PTP transmission.

Applicable AI/ML configuration: Configuration according to which an AI/ML functionality is determined to be applicable by the UE, as defined in TS 38.300 [2].

BH RLC channel: An RLC channel between two nodes, which is used to transport backhaul packets.

Broadcast MRB: A radio bearer configured for MBS broadcast delivery.

Candidate configuration: A configuration part of an RRCReconfiguration message associated with a candidate cell, e.g., for LTM or subsequent CPAC. A candidate configuration can be a complete candidate configuration or a delta configuration relatively to a reference configuration.

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

Child UE: A U2N Relay UE’s next hop in downstream direction for serving a U2N Remote UE in U2N Relay communication. Child UE can be the U2N Remote UE or a U2N Relay UE.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time, as defined in TS 23.501 [32].

Dedicated signalling: Signalling sent on DCCH logical channel between the network and a single UE.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signalling. In the dormant BWP, the UE stops monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured. For each serving cell other than the SpCell or PUCCH SCell, the network may configure one BWP as a dormant BWP.

Downstream: Direction toward child node or UE in IAB-topology or U2N Remote UE in U2N Relay communication.

Earth-fixed cell: An NTN cell fixed with respect to a certain geographic area on Earth. It can be provisioned by beam(s) continuously covering the same geographical area (e.g., the case of GSO satellites).

Earth-moving cell: An NTN cell moving on the ground. It can be provisioned by beam(s) whose coverage area slides over the Earth's surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

eRedCap UE: A UE with enhanced reduced capabilities as specified in clause 4.2.22.1 in TS 38.306 [26].

Field: The individual contents of an information element are referred to as fields.

First U2N Relay UE: An Intermediate U2N Relay UE having both PC5 connection to a parent UE and PC5 connection to a U2N Remote UE for serving the U2N Remote UE in case of multi-hop L2 U2N Relay communication.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Frequency Selection Area ID: An identity used for broadcast MBS session to guide the frequency selection of the UE as defined in TS 23.247 [67].

Global cell identity: An identity to uniquely identifying an NR cell. It is consisted of cellIdentity and plmn-Identity of the first PLMN-Identity in plmn-IdentityList in SIB1.

Information element: A structural element containing single or multiple fields is referred as information element.

Intermediate U2N Relay UE: A U2N Relay UE having both PC5 connection to a parent UE and PC5 connection to a child UE or a U2N Remote UE for serving the U2N Remote UE in case of multi-hop L2 U2N Relay communication.

Last U2N Relay UE: A U2N Relay UE having both Uu connection to the network and PC5 connection to a child UE for serving a U2N Remote UE in case of L2 U2N Relay communication. The child UE is the U2N Remote UE in case of single-hop L2 U2N Relay communication.

MBS Radio Bearer: A radio bearer that is configured for MBS delivery.

Mobile IAB-MT: mobile IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. The mobile IAB-MT uses the same procedures and behaviours specified for the IAB-MT, unless explicitly stated otherwise.

Mobile IAB-node: RAN node that supports NR access links to UEs and an NR backhaul link to a parent node, and that can conduct physical mobility across the RAN area. The mobile IAB-node function used in 38-series of 3GPP Specifications corresponds to the MBSR function defined in TS 23.501 [32]. The mobile IAB-node uses the same procedures and behaviours specified for the IAB-node, unless explicitly stated otherwise.

Multicast/Broadcast Service: A point-to-multipoint service as defined in TS 23.247 [67].

Multicast MRB: A radio bearer configured for MBS multicast delivery.

MUSIM gap: Period that the UE may use to perform MUSIM operations.

Multi-path: Mode of operation of a UE in RRC_CONNECTED configured with one direct path on which the UE connects to gNB using NR Uu, and one indirect path on which the UE connects to the same gNB via another UE using PC5 unicast link or Non-3GPP Connection.

MP remote UE: A UE configured with Multi-path. When the connectivity of indirect path is PC5 unicast link, the MP remote UE is acting as a L2 U2N Remote UE. When the connectivity of indirect path is Non-3GPP Connection, the MP remote UE is acting as a N3C remote UE.

MP relay UE: A UE that provides connectivity of indirect path to a MP remote UE. When the connectivity is PC5 unicast link, the MP relay UE is acting as a L2 U2N Relay UE. When the connectivity is Non-3GPP Connection, the MP relay UE is acting as a N3C relay UE.

NCSG: Network controlled small gap as defined in TS 38.133 [14].

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

N3C indirect path: In Multi-path, the indirect path using Non-3GPP Connection between remote UE and relay UE.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [55] and/or A2X Communication as defined in TS 23.256 [76] and/or ProSe Communication (including ProSe UE-to-Network Relay, non-Relay communication, and ProSe UE-to-UE Relay Communication including UE-to-UE Relay communication with integrated discovery) as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery, ProSe UE-to-Network Relay discovery and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink positioning: AS functionality which determines geographical or relative location and possibly velocity of a target UE or ranging via PC5 interface using SL-PRS transmission and reception as defined in TS 38.305 [73] and TS 38.355 [77].

OD-SIB1: On-demand SIB1 as defined in TS 38.300 [2].

OD-SSB: On-demand SSB as defined in TS 38.300 [2].

Parent UE: A U2N Remote UE’s or U2N Relay UE’s next hop U2N Relay UE in upstream direction for serving the U2N Remote UE in U2N Relay communication.

PNI-NPN identity: an identifier of a PNI-NPN comprising of a PLMN ID and a CAG-ID combination.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

PC5 Relay RLC channel: An RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, or between L2 U2N Relay UEs (in case of multi-hop L2 U2N relay communication), or between L2 U2U Remote UE and L2 U2U Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network relay or L2 UE-to-UE relay.

PDU Set: one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s) or video slice(s) for XR Services), as defined in TS 23.501 [32].

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH by PUCCH-Config.

PUSCH-Less SCell: An SCell configured without PUSCH.

Quasi-Earth-fixed cell: An NTN cell fixed with respect to a certain geographic area on Earth during a certain time duration. It can be provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams).

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [26].

Reference configuration: A configuration provided by the network to the UE that is common, within the same cell group, to a group of configured non-complete candidate configurations.

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Small Data Transmission: A procedure used for transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state (i.e. without the UE transitioning to RRC_CONNECTED state). The SDT procedure is considered to be ongoing once the conditions for initating SDT as specified in clause 5.3.13.1b are fulfilled until the SDT procedure is completed either successfully or unsuccessfully as specified in clause 18.0 in TS 38.300 [2].

SNPN identity: an identifier of an SNPN comprising of a PLMN ID and an NID combination.

SL indirect path: In Multi-path, the indirect path using PC5 unicast link between remote UE and relay UE.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

Split DRB: In MR-DC, a DRB that supports transmission via MCG and SCG, as well as duplication of PDCP PDUs as defined in TS 37.340 [41]; or in MP, a DRB that supports transmission via direct path and indirect path, as well as duplication of PDCP PDUs.

Split SRB: In MR-DC, an SRB that supports transmission via MCG and SCG as well as duplication of RRC PDUs as defined in TS 37.340 [41]; or in MP, a SRB that supports transmission via direct path and indirect path, as well as duplication of PDCP PDUs.

SSB Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s). Up to three L2 U2N Relay UEs (i.e. one Last U2N Relay UE and up to two Intermediate U2N Relay UEs including one First U2N Relay UE) can be configured for serving a L2 U2N Remote UE in multi-hop L2 U2N Relay communication in this release.

U2N Remote UE: A UE that communicates with the network via one or more U2N Relay UEs on an indirect path.

U2U Relay UE: A UE that provides functionality to support connectivity between two U2U Remote UEs.

U2U Remote UE: A UE that communicates with other UEs via a U2U Relay UE.

UE-to-Network Relay communication: A mode of communication in which a UE communicates with the network through an indirect path involving only one U2N Relay UE for single-hop L2 U2N Relay communication or multiple L2 U2N Relay UEs for multi-hop L2 U2N Relay communication.

UE-to-Network Relay discovery: A mode of NR sidelink discovery in which a UE discovers other UEs for U2N Relay communication.

Upstream: Direction toward parent node in IAB-topology or gNB in U2N Relay communication.

Uu Relay RLC channel: An RLC channel between L2 U2N Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network relay or for indirect path in case of MP.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.3.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [56], between nearby UEs, using E-UTRA technology but not traversing any network node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

A2X	Aircraft-to-Everything

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

ATG	Air to Ground

BAP	Backhaul Adaptation Protocol

BCD	Binary Coded Decimal

BFD	Beam Failure Detection

BH	Backhaul

BLER	Block Error Rate

BRID	Broadcast Remote Identification

BSR	Buffer Status Report

BWP	Bandwidth Part

CA	Carrier Aggregation

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CAPC	Channel Access Priority Class

CBR	Channel Busy Ratio

CCCH	Common Control Channel

CFR	Common Frequency Resources

CG	Cell Group

CHO	Conditional Handover

CLI	Cross Link Interference

CLTM	Conditional L1/L2 Triggered Mobility

CMAS	Commercial Mobile Alert Service

CP	Control Plane

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

C-RNTI	Cell RNTI

CSI	Channel State Information

DAA	Detect And Avoid

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DFN	Direct Frame Number

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

DL-SCH	Downlink Shared Channel

DM-RS	Demodulation Reference Signal

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DSR	Delay Status Report

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered Inertial

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

G-CS-RNTI	Group Configured Scheduling RNTI

GERAN	GSM/EDGE Radio Access Network

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

G-RNTI	Group RNTI

GSM	Global System for Mobile Communications

GSO	Geosynchronous Orbit

HARQ	Hybrid Automatic Repeat Request

HRNN	Human Readable Network Name

HSDN	High Speed Dedicated Network

H-SFN	Hyper SFN

HST	High Speed Train

IAB	Integrated Access and Backhaul

IAB-DU	IAB-node DU

IAB-MT	IAB Mobile Termination

IDC	In-Device Coexistence

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LBT	Listen Before Talk

LEO	Low Earth Orbit

LP-SS	Low Power Synchronization Signal

LP-WUR	Low Power Wake-Up Receiver

LP-WUS	Low Power Wake-Up Signal

LR	Low Power Wake-Up Receiver

LTM	L1/L2 Triggered Mobility

MAC	Medium Access Control

MBS	Multicast/Broadcast Service

MBS FSAI	MBS Frequency Selection Area Identity

MCCH	MBS Control Channel

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MIB	Master Information Block

MO-SDT	Mobile Originated SDT

MPE	Maximum Permissible Exposure

MP	Multi-path

MR	Main Receiver

MRB	MBS Radio Bearer

MR-DC	Multi-Radio Dual Connectivity

MTCH	MBS Traffic Channel

MT-SDT	Mobile Terminated SDT

MTSI	Multimedia Telephony Service for IMS

MUSIM	Multi-Universal Subscriber Identity Module

N3C	Non-3GPP Connection

N/A	Not Applicable

NCC	Next Hop Chaining Counter

NCR	Network-Controlled Repeater

NCR-Fwd	NCR Forwarding

NCR-MT	NCR Mobile Termination

NE-DC	NR E-UTRA Dual Connectivity

NES	Network Energy Savings

(NG)EN-DC	E-UTRA NR Dual Connectivity (covering E-UTRA connected to EPC or 5GC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to 5GC

NID	Network Identifier

NPN	Non-Public Network

NR-DC	NR-NR Dual Connectivity

NR/5GC	NR connected to 5GC

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

OD-SIB1	On-demand SIB1

OD-SSB	On-demand SSB

OOK	On-Off Keying

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

posSIB	Positioning SIB

PPW	PRS Processing Window

PRS	Positioning Reference Signal

PSCell	Primary SCG Cell

PSI	PDU Set Importance

PTM	Point to Multipoint

PTP	Point to Point

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RLM	Radio Link Monitoring

RMTC	RSSI Measurement Timing Configuration

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RS	Reference Signal

SBAS	Satellite Based Augmentation System

SBFD	Sub-Band Full Duplex

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SDL	Supplementary Downlink

SD-RSRP	Sidelink Discovery RSRP

SDT	Small Data Transmission

SFN	Single Frequency Network

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SL-PRS	Sidelink Positioning Reference Signal

SLSS	Sidelink Synchronisation Signal

SNPN	Stand-alone Non-Public Network

SpCell	Special Cell

SRAP	Sidelink Relay Adaptation Protocol

SRB	Signalling Radio Bearer

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDCP	Time Domain Channel Property

TDD	Time Division Duplex

TEG	Timing Error Group

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

TN	Terrestrial Network

TSS	Timing Synchronization Status.

U2N	UE-to-Network

U2U	UE-to-UE

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

VR	Virtual Reality

VSAT	Very Small Aperture Terminal

XR	eXtended Reality

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

# 4	General

## 4.1	Introduction

This specification is organised as follows:

-	clause 4.2 describes the RRC protocol model;

-	clause 4.3 specifies the services provided to upper layers as well as the services expected from lower layers;

-	clause 4.4 lists the RRC functions;

-	clause 5 specifies RRC procedures, including UE state transitions;

-	clause 6 specifies the RRC messages in ASN.1 and description;

-	clause 7 specifies the variables (including protocol timers and constants) and counters to be used by the UE;

-	clause 8 specifies the encoding of the RRC messages;

-	clause 9 specifies the specified and default radio configurations;

-	clause 10 specifies generic error handling;

-	clause 11 specifies the RRC messages transferred across network nodes;

-	clause 12 specifies the UE capability related constraints and performance requirements.

## 4.2	Architecture

### 4.2.1	UE states and state transitions including inter RAT

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	Monitors a Paging channel for CN paging using 5G-S-TMSI, except if the UE is acting as a L2 U2N Remote UE;

-	If configured by upper layers for MBS multicast reception, monitors a Paging channel for CN paging using TMGI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Performs measurements on L2 U2N Relay UEs and relay (re-)selection;

-	Acquires system information and can send SI request (if configured);

-	Performs logging of available measurements together with location and time for logged measurement configured UEs;

-	Performs idle/inactive measurements for idle/inactive measurement configured UEs;

-	If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data.

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for PTM transmission of MBS multicast;

-	UE controlled mobility based on network configuration;

-	The UE stores the UE Inactive AS context;

-	A RAN-based notification area is configured by RRC layer;

-	Transfer of unicast data and/or signalling to/from UE over radio bearers configured for SDT.

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	While T319a is running, monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	While SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI except if the UE is acting as a L2 U2N Remote UE;

-	While SDT procedure is not ongoing, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI, except if the UE is acting as a L2 U2N Remote UE;

-	If configured by upper layers for MBS multicast reception, while SDT procedure is not ongoing, monitors a Paging channel for paging using TMGI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Performs measurements on L2 U2N Relay UEs and relay (re-)selection;

-	Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area;

-	Acquires system information and, while SDT procedure is not ongoing, can send SI request (if configured);

-	While SDT procedure is not ongoing, performs logging of available measurements together with location and time for logged measurement configured UEs;

-	While SDT procedure is not ongoing, performs idle/inactive measurements for idle/inactive measurement configured UEs;

-	If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data;

-	If configured for MBS multicast reception in RRC_INACTIVE, acquires multicast MCCH change notification and MBS multicast control information and data;

-	Transmits SRS for Positioning.

-	RRC_CONNECTED:

-	The UE stores the AS context;

-	Transfer of unicast data to/from UE;

-	Transfer of MBS multicast data to UE;

-	At lower layers, the UE may be configured with a UE specific DRX;

-	At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for MBS multicast;

-	At lower layers, the UE may be configured with a cell specific cell DTX/DRX;

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility within NR, to/from E-UTRA, and to UTRA-FDD;

-	Network controlled mobility (path switch) between a serving cell and a L2 U2N Relay UE for single hop, or vice versa, or between a serving cell and L2 U2N Relay UEs for multi hop, or vice versa, or between a source L2 U2N Relay UE and a target L2 U2N Relay UE for single hop, or between a single hop source L2 U2N Relay UE and target L2 U2N Relay UEs for multi hop, or vice versa;

-	Network controlled MP operation.

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5), if configured;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell and/or L2 U2N relay measurements and measurement reporting;

-	Acquires system information;

-	Performs immediate MDT measurement together with available location reporting;

-	If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data;

-	Performs logging of measurements for network-side data collection, if configured.

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/5GC, E-UTRA/EPC and E-UTRA/5GC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/5GC, E-UTRA/EPC and E-UTRA/5GC

Figure 4.2.1-3 illustrates the mobility procedure supported between NR/5GC and UTRA-FDD.

Figure 4.2.1-3:	Mobility procedure supported between NR/5GC and UTRA-FDD

### 4.2.2	Signalling radio bearers

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel (except SRB0 of L2 U2N Remote UE; or except SRB0 of L2 Intermediate U2N Relay UE in case of multi hop);

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel (except SRB1 of L2 U2N Remote UE; or except SRB1 of L2 Intermediate U2N Relay UE in case of multi hop);

-	SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE; or except SRB2 of L2 Intermediate U2N Relay UE in case of multi hop). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation;

-	SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel;

-	SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

-	SRB5 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB5 has a lower priority than SRB1 and SRB3 and can only be configured by the SN serving the SCG when the UE is in NR-DC, after AS security activation.

-	SRB6 is for RRC messages which include logged measurement information for network-side data collection, all using DCCH logical channel. SRB6 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

In downlink, piggybacking of NAS messages is used only for one dependant (i.e. with joint success/failure) procedure: bearer establishment/modification/release. In uplink piggybacking of NAS message is used only for transferring the initial NAS message during connection setup and connection resume.

NOTE 1:	The NAS messages transferred via SRB2 are also contained in RRC messages, which however do not include any RRC protocol control information.

Once AS security is activated, all RRC messages on SRB1, SRB2, SRB3, SRB4, SRB5 and SRB6, including those containing NAS messages, are integrity protected and ciphered by PDCP. NAS independently applies integrity protection and ciphering to the NAS messages, see TS 24.501 [23].

Split SRB is supported for all the MR-DC options as well as MP in both SRB1 and SRB2 (split SRB is not supported for SRB0, SRB3, SRB4, SRB5 and SRB6).

For operation with shared spectrum channel access in FR1, SRB0, SRB1 and SRB3 are assigned with the highest priority Channel Access Priority Class (CAPC), (i.e. CAPC = 1) while CAPC for SRB2 is configurable.

For the NR sidelink L2 U2N relay operations not involved in MP or for the NR sidelink L2 multi hop U2N relay operations, SRB0, SRB1, SRB2 of a L2 U2N Remote UE or of a L2 Intermediate U2N Relay UE are not using Uu CCCH/DCCH logical channels. The SRB0, SRB1, SRB2 of a L2 U2N Remote UE or of a L2 Intermediate U2N Relay UE are transmitted via the PC5 Relay RLC channels over PC5 and Uu Relay RLC channels over Uu.

## 4.3	Services

### 4.3.1	Services provided to upper layers

The RRC protocol offers the following services to upper layers:

-	Broadcast of common control information;

-	Notification of UEs in RRC_IDLE, e.g. about a mobile terminating call;

-	Notification of UEs about ETWS and/or CMAS;

-	Transfer of dedicated signalling;

-	Broadcast of positioning assistance data;

-	Transfer of application layer measurement configuration and reporting.

### 4.3.2	Services expected from lower layers

In brief, the following are the main services that RRC expects from lower layers:

-	Integrity protection, ciphering and loss-less in-sequence delivery of information without duplication;

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE and RRC_INACTIVE (e.g. cell (re-)selection parameters, neighbouring cell information) and information (also) applicable for UEs in RRC_CONNECTED (e.g. common channel configuration information);

-	Including ETWS notification, CMAS notification;

-	Including positioning assistance data.

-	RRC connection control:

-	Paging;

-	Establishment/modification/suspension/resumption/release of RRC connection, including e.g. assignment/modification of UE identity (C-RNTI, fullI-RNTI, etc.), establishment/modification/suspension/resumption/release of SRBs (except for SRB0);

-	Access barring;

-	Initial AS security activation, i.e. initial configuration of AS integrity protection (SRBs, DRBs) and AS ciphering (SRBs, DRBs);

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, path switch from a PCell to a target L2 U2N Relay UE or from a L2 U2N Relay UE to a target PCell or from a source L2 U2N Relay UE to a target L2 U2N Relay UE in case of single hop or path switch from a PCell to target path via multiple L2 U2N Relay UEs or from source path via multiple L2 U2N Relay UEs to a target PCell or from a source L2 U2N Relay UE to target path via multiple L2 U2N Relay UEs or from source path via multiple L2 U2N Relay UEs to a target L2 U2N Relay UE in case of multi hop, associated AS security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes;

-	Establishment/modification/suspension/resumption/release of RBs carrying user data (DRBs/MRBs);

-	Radio configuration control including e.g. assignment/modification of ARQ configuration, HARQ configuration, DRX configuration;

-	In case of DC, cell management including e.g. change of PSCell, addition/modification/release of SCG cell(s);

-	In case of CA, cell management including e.g. addition/modification/release of SCell(s);

-	In case of MP, path management including e.g. addition/modification/release of indirect path;

-	QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration and configured grant configuration for DL and UL respectively, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB of UE and logical channel of IAB-MT.

-	Recovery from radio link failure.

-	Inter-RAT mobility including e.g. AS security activation, transfer of RRC context information;

-	Measurement configuration and reporting:

-	Establishment/modification/release of measurement configuration (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting.

-	Configuration of BAP entity and BH RLC channels for the support of IAB-node.

-	Configuration of SRAP entity and Uu/PC5 Relay RLC channels for the support of L2 U2N relay.

-	Configuration of SRAP entity and PC5 Relay RLC channels for the support of L2 U2U relay operation.

-	Other functions including e.g. generic protocol error handling, transfer of dedicated NAS information, transfer of UE radio access capability information.

-	Support of self-configuration and self-optimisation.

-	Support of measurement logging and reporting for network performance optimisation, as specified in TS 37.320 [61];

-	Support of transfer of application layer measurement configuration and reporting.

-	Configuration of side control information for NCR-node.

# 5	Procedures

## 5.1	General

### 5.1.1	Introduction

This clause covers the general requirements.

### 5.1.2	General requirements

The UE shall:

1>	process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE:	Network may initiate a subsequent procedure prior to receiving the UE's response of a previously initiated procedure.

1>	within a clause execute the steps according to the order specified in the procedural description;

1>	consider the term 'radio bearer' (RB) to cover SRBs, DRBs and MRBs unless explicitly stated otherwise;

1>	set the rrc-TransactionIdentifier in the response message, if included, to the same value as included in the message received from the network that triggered the response message;

1>	upon receiving a choice value set to setup:

2>	apply the corresponding received configuration and start using the associated resources, unless explicitly specified otherwise;

1>	upon receiving a choice value set to release:

2>	clear the corresponding configuration and stop using the associated resources;

1>	in case the size of a list is extended, upon receiving an extension field comprising the entries in addition to the ones carried by the original field (regardless of whether the network signals more entries in total); apply the following generic behaviour unless explicitly stated otherwise:

2>	create a combined list by concatenating the additional entries included in the extension field to the original field while maintaining the order among both the original and the additional entries;

2>	for the combined list, created according to the previous, apply the same behaviour as defined for the original field.

### 5.1.3	Requirements for UE in MR-DC

In this specification, the UE considers itself to be in:

-	EN-DC, if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10], and it is connected to EPC,

-	NGEN-DC, if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10], and it is connected to 5GC,

-	NE-DC, if and only if it is configured with mrdc-SecondaryCellGroup set to eutra-SCG,

-	NR-DC, if and only if it is configured with mrdc-SecondaryCellGroup set to nr-SCG,

-	MR-DC, if and only if it is in (NG)EN-DC, NE-DC or NR-DC.

NOTE:	This use of these terms deviates from the definition in TS 37.340 [41] and other specifications. In TS 37.340, these terms include also the case where the UE is configured with E-UTRA or NR MCG only (i.e. no NR or E-UTRA SCG) but with one or more bearers terminated in a secondary node (i.e. using NR PDCP).

The UE in (NG)EN-DC only executes a subclause of clause 5 in this specification when the subclause:

-	is referred to from a subclause under execution, either in this specification or in TS 36.331 [10]; or

-	applies to a message received on SRB3 (if SRB3 is established); or

-	applies to field(s), IE(s), UE variable(s) or timer(s) in this specification that the UE is configured with.

When executing a subclause of clause 5 in this specification, the UE follows the requirements in clause 5.1.2 and in all subclauses of this specification applicable to the messages (including processing time requirements), fields, IEs, timers and UE variables indicated in the subclause under execution.

## 5.2	System information

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB;

NOTE 1:	If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB.

-	the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 and posSIBs are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs or posSIBs having the same periodicity can be mapped to the same SI message. SIBs and posSIBs are mapped to different SI messages, i.e. an SI message contains either only SIBs or only posSIBs. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted. An SI message may be repeated with the same content a number of times within the SI-window. Any SIB or posSIB except SIB1 can be configured to be cell specific or area specific, using an indication in SIB1. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells and is identified by systemInformationAreaID;

-	The mapping of SIBs to SI messages is configured in schedulingInfoList and schedulingInfoList2, while the mapping of posSIBs to SI messages is configured in posSchedulingInfoList and schedulingInfoList2.
Each SIB and each posSIB is mapped to a single SI message. posSIBs of the same posSibType carrying GNSS Generic Assistance Data for different GNSS/SBAS (identified by gnss-id/sbas-id, see TS 37.355 [49]) are mapped to different SI messages.
Each SIB and posSIB is contained at most once in an SI message.
For SIBs and posSIBs with segments, the segments contained in SI messages are transmitted according to the SI message periodicity, with one segment of a particular sibType/posSibType in each SI message;

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information, paging, or upon request from the UE;

-	For a UE in RRC_CONNECTED, the network may provide multiple segments of a posSIB of the same posSIB type through dedicated signalling using the RRCReconfiguration message;

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, the network releases and adds the concerned SCell. For PSCell, the required SI can only be changed with Reconfiguration with Sync.

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

### 5.2.2	System information acquisition

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB17 or SIB17bis (if the UE is using TRS resources for power saving in RRC_IDLE and RRC_INACTIVE), SIB19 (if UE is accessing NR via NTN access), SIB22 (for ATG access), and SIB23 (if UE is capable of NR sidelink positioning and is configured by upper layers to receive or transmit SL-PRS). The UE supporting OD-SIB1 shall ensure having a valid version of SIB26 (for OD-SIB1 acquisition).

The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20, regardless of the RRC state the UE is in.

The UE shall ensure having a valid version of the posSIB requested by upper layers.

#### 5.2.2.2	SIB validity and need to (re)-acquire SIB

##### 5.2.2.2.1	SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB.

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49].

A L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected parent L2 U2N Relay UE as defined in clause 5.8.9.8.2 and receive the SIB(s) from the parent L2 U2N Relay UE as defined in clause 5.8.9.9.3. A L2 U2N Remote UE in RRC_CONNECTED receives SIB1 and other SIB(s) in RRCReconfiguration message and performs on-demand SI request if required, as defined in clause 5.2.2.3.5 and 5.2.2.3.6. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED (when MP is not configured) is not required to obtain SI over Uu interface, but it may decide to perform the SI acquisition procedure over Uu interface as defined in clause 5.2.2.3 by UE implementation.

NOTE:	The storage and management of the stored SIBs in addition to the SIBs valid for the current serving cell is left to UE implementation.

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell:

4>	if the first NPN identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

5>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

1>	for each stored version of a posSIB:

2>	if the areaScope is associated and its value for the stored version of the posSIB is the same as the value received in the posSIB-MappingInfo for that posSIB from the serving cell and the systemInformationAreaID included in the si-SchedulingInfo is identical to the systemInformationAreaID associated with the stored version of that posSIB:

3>	if the valueTag (see TS 37.355 [49]) for the posSIB received from the serving cell is identical to the valueTag associated with the stored version of that posSIB; or if the expirationTime (see TS 37.355 [49]) associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the posSIB and the areaScope value is not included in the posSIB-MappingInfo for that posSIB from the serving cell and the cellIdentity for the posSIB received from the serving cell is identical to the cellIdentity associated with the stored version of that posSIB:

3>	if the valueTag (see TS 37.355 [49]) for the posSIB received from the serving cell is identical to the valueTag associated with the stored version of that posSIB; or if the expirationTime (see TS 37.355 [49]) associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

##### 5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI message (other than SI message for ETWS, CMAS, positioning assistance data, and some NTN-specific information as specified in the field descriptions) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SIB1, and UE is configured with eDRX, modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m = 0.

For UEs in RRC_IDLE or RRC_INACTIVE configured to use an IDLE eDRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 = 0.

The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period or within preceding eDRX acquisition period. SI change indication is not applicable for SI messages containing posSIBs.

UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion except those only for UEs that support paging adaptation, see TS 38.300 [2], at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

For UEs in RRC_INACTIVE while SDT procedure is ongoing:

-	if T319a is not running and if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), the UE shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20];

-	in other cases, the UE shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

During a modification period where ETWS or CMAS transmission is started or stopped, the SI messages carrying the posSIBs scheduled in posSchedulingInfoList may change, so the UE might not be able to successfully receive those posSIBs in the remainder of the current modification period and next modification period according to the scheduling information received prior to the change.

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion except those only for UEs that support paging adaptation, see TS 38.300 [2], at least once every defaultPagingCycle if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging.

For ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing: :

-	if T319a is not running and if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), the UE shall monitor for indications about PWS notification in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20];

-	in other cases, the UE shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

For Short Message reception in a paging occasion, the UE monitors the PDCCH monitoring occasion(s) for paging as specified in TS 38.304 [20] and TS 38.213 [13].

A L2 U2N Remote UE is not required to monitor paging occasion for SI modifications and/or PWS notifications. It obtains the updated system information and SIB6/7/8 from the connected L2 U2N Relay UE as defined in clause 5.8.9.9.3.

If the UE receives a Short Message, the UE shall:

1>	if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceSIB1 and searchSpaceOtherSystemInformation on the active BWP or the initial BWP:

2> immediately re-acquire the SIB1;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6:

3>	acquire SIB6, as specified in clause 5.2.2.3.2, immediately;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7:

3>	acquire SIB7, as specified in clause 5.2.2.3.2, immediately;

2>	if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8:

3>	acquire SIB8, as specified in clause 5.2.2.3.2, immediately;

NOTE:	In case SIB1, SIB6, SIB7, or SIB8 overlap with a measurement gap it is left to UE implementation how to immediately acquire SIB1, SIB6, SIB7, or SIB8.

1>	if the UE does not operate an IDLE eDRX cycle longer than the modification period and the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next modification period;

1>	if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary.

#### 5.2.2.3	Acquisition of System Information

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.28:

4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

4>	if the UE is in RRC_CONNECTED while T311 is running:

5>	if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

6>	acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

5>	else:

6>	perform the actions as specified in clause 5.2.2.3.3b;

3>	else:

4>	perform the actions as specified in clause 5.2.2.5.

NOTE 1:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 2:	UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]).

NOTE 3:	It is up to UE implementation to check if SIB1 is currently being broadcasted for a cell before triggering SIB1 request procedure of that cell.

##### 5.2.2.3.2	Acquisition of an SI message

For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation. If searchSpaceOtherSystemInformation is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space indicated by searchSpaceOtherSystemInformation. PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	if the concerned SI message is configured in the schedulingInfoList:

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

2>	else if the concerned SI message is configured in the schedulingInfoList2;

3>	determine the integer value x = (si-WindowPosition -1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

2>	else if the concerned SI message is configured in the posSchedulingInfoList and offsetToSI-Used is not configured:

3>	create a concatenated list of SI messages by appending the posSchedulingInfoList in posSI-SchedulingInfo in SIB1 to schedulingInfoList in si-SchedulingInfo in SIB1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the posSI-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

2>	else if the concerned SI message is configured by the posSchedulingInfoList and offsetToSI-Used is configured:

3>	determine the number m which corresponds to the number of SI messages with an associated si-Periodicity of 8 radio frames (80 ms), configured by schedulingInfoList in SIB1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by posSchedulingInfoList in SIB1;

3>	determine the integer value x = m × w + (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N) +8, where T is the posSI-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

1>	receive the PDCCH containing the scheduling RNTI, i.e. SI-RNTI in the PDCCH monitoring occasion(s) for SI message acquisition, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received;

1>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message in the current modification period;

1>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

2>	stop timer T350, if running;

NOTE 1:	The UE is only required to acquire broadcasted SI message if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 2:	The UE is not required to monitor PDCCH monitoring occasion(s) corresponding to each transmitted SSB in SI-window.

NOTE 3:	If the concerned SI message was not received in the current modification period, handling of SI message acquisition is left to UE implementation.

NOTE 4:	A UE in RRC_CONNECTED may stop the PDCCH monitoring during the SI window for the concerned SI message when the requested SIB(s) are acquired.

NOTE 5:	A UE capable of NR sidelink communication/discovery and configured by upper layers to perform NR sidelink communication/discovery on a frequency, may acquire SIB12 or SystemInformationBlockType28 from a cell other than current serving cell (for RRC_INACTIVE or RRC_IDLE) or current PCell (for RRC_CONNECTED), if SIB12 of current serving cell (for RRC_INACTIVE or RRC_IDLE) or current PCell (for RRC_CONNECTED) does not provide configuration for NR sidelink communication/discovery for the frequency, and if the other cell providing configuration for NR sidelink communication/discovery for the frequency meets the S-criteria as defined in TS 38.304 [20] or TS 36.304 [27].

1>	perform the actions for the acquired SI message as specified in clause 5.2.2.4.

##### 5.2.2.3.3	Request for on demand system information

The UE shall, while SDT procedure is not ongoing:

1>	if SIB1 includes si-SchedulingInfo containing si-RequestConfigSUL-MSG1-Repetition and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 and criteria to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigSUL-MSG1-Repetition are met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in si-RequestConfigSUL-MSG1-Repetition corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigRedCap-MSG1-Repetition and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 and criteria to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigRedCap-MSG1-Repetition are met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in si-RequestConfigRedCap-MSG1-Repetition corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else if SIB1 includes si-SchedulingInfo containing si-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigSUL corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigRedCap and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else:

2>	if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink as defined in TS 38.321 [3], clause 5.1.1 and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met; or

2>	if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigMSG1-Repetition and criteria to select normal uplink as defined in TS 38.321 [3], clause 5.1.1 and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned si-RequestConfigMSG1-Repetition are met:

3>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in si-RequestConfigMSG1-Repetition corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

3>	if acknowledgement for SI request is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

2>	else if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met; or

2>	if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

3>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfig corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

3>	if acknowledgement for SI request is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

2>	else:

3>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

3>	apply the default MAC Cell Group configuration as specified in 9.2.2;

3>	apply the timeAlignmentTimerCommon included in SIB1;

3>	apply the CCCH configuration as specified in 9.1.1.2;

3>	initiate transmission of the RRCSystemInfoRequest message with rrcSystemInfoRequest in accordance with 5.2.2.3.4;

3>	if acknowledgement for RRCSystemInfoRequest message with rrcSystemInfoRequest is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	if cell reselection occurs while waiting for the acknowledgment for SI request from lower layers:

2>	reset MAC;

2>	if SI request is based on RRCSystemInfoRequest message with rrcSystemInfoRequest:

3>	release RLC entity for SRB0.

NOTE:	After RACH failure for SI request it is up to UE implementation when to retry the SI request.

##### 5.2.2.3.3a	Request for on demand positioning system information

The UE shall, while SDT procedure is not ongoing:

1>	if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfigSUL-MSG1-Repetition and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 and criteria to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned posSI-RequestConfigSUL-MSG1-Repetition are met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in posSI-RequestConfigSUL-MSG1-Repetition corresponding to the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfigRedCap-MSG1-Repetition and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 and criteria to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned posSI-RequestConfigRedCap-MSG1-Repetition are met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in posSI-RequestConfigRedCap-MSG1-Repetition corresponding to the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in posSI-RequestConfigSUL corresponding to the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfigRedCap and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in posSI-RequestConfigRedCap corresponding to the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else:

2>	if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfigMSG1-Repetition and criteria to select normal uplink as defined in TS 38.321 [3], clause 5.1.1 and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned posSI-RequestConfigMSG1-Repetition are met; or

2>	if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfigMSG1-Repetition and criteria to select normal uplink as defined in TS 38.321 [3], clause 5.1.1, and to apply MSG1 repetition as defined in TS 38.321[3], clause 5.1.1e for the concerned posSI-RequestConfigMSG1-Repetition are met:

3>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) associated with the applicable MSG1 repetition number in posSI-RequestConfigMSG1-Repetition corresponding to the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting;

3>	if acknowledgement for SI request is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

2>	else if the UE is neither a RedCap nor an eRedCap UE and if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met; or

2>	if the UE is an (e)RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes posSI-SchedulingInfo containing posSI-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

3>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in posSI-RequestConfig corresponding to the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting;

3>	if acknowledgement for SI request is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

2>	else:

3>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

3>	apply the default MAC Cell Group configuration as specified in 9.2.2;

3>	apply the timeAlignmentTimerCommon included in SIB1;

3>	apply the CCCH configuration as specified in 9.1.1.2;

3>	initiate transmission of the RRCSystemInfoRequest message with rrcPosSystemInfoRequest in accordance with 5.2.2.3.4;

3>	if acknowledgement for RRCSystemInfoRequest message with rrcPosSystemInfoRequest is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	if cell reselection occurs while waiting for the acknowledgment for SI request from lower layers:

2>	reset MAC;

2>	if SI request is based on RRCSystemInfoRequest message with rrcPosSystemInfoRequest:

3>	release RLC entity for SRB0.

NOTE:	After RACH failure for SI request it is up to UE implementation when to retry the SI request.

##### 5.2.2.3.3b	Request for on-demand SIB1

The UE shall, while SDT procedure is not ongoing:

1>	if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig included in od-sib1-Config for this cell in stored valid version of SIB26;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfigSUL included in od-sib1-Config for this cell in stored valid version of SIB26;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfigSUL is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if cell reselection occurs while waiting for the acknowledgment for SIB1 request from lower layers:

2>	reset MAC;

1>	if the UE is unable to acquire the SIB1:

2>	perform the actions as specified in clause 5.2.2.5.

##### 5.2.2.3.4	Actions related to transmission of RRCSystemInfoRequest message

The UE shall set the contents of RRCSystemInfoRequest message as follows:

1>	if the procedure is triggered to request the required SI message(s) other than positioning:

2>	set the requested-SI-List to indicate the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

1>	else if the procedure is triggered to request the required SI message(s) for positioning:

2>	set the requestedPosSI-List to indicate the SI message(s) that the UE upper layers require for positioning operations, and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting.

The UE shall submit the RRCSystemInfoRequest message to lower layers for transmission.

##### 5.2.2.3.5	Acquisition of SIB(s) or posSIB(s) in RRC_CONNECTED

The UE shall:

1>	if the UE is in RRC_CONNECTED with an active BWP not configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, or

1>	if the UE is in RRC_CONNECTED and acting as a L2 U2N Remote UE and the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1:

2>	for the SI message(s) that, according to the si-SchedulingInfo or posSI-SchedulingInfo in the stored SIB1, contain at least one required SIB or requested posSIB:

3>	if onDemandSIB-Request is configured and timer T350 is not running:

4>	initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6;

4>	start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer;

1>	else if the UE is in RRC_CONNECTED with an active BWP configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1:

2>	if ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell and if the UE has a stored valid version of od-sib1-Config for this cell:

3>	a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one;

2>	for the SI message(s) that, according to the si-SchedulingInfo in the stored SIB1, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

3>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

2>	for the SI message(s) that, according to the si-SchedulingInfo in the stored SIB1, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

3>	if onDemandSIB-Request is configured and timer T350 is not running:

4>	initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6;

4>	start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer;

4>	acquire the requested SI message(s) corresponding to the requested SIB(s) as defined in clause 5.2.2.3.2.

2>	for the SI message(s) that, according to the posSI-SchedulingInfo or si-SchedulingInfo-v1700, if present, in the stored SIB1, contain at least one requested posSIB and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to broadcasting:

3>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

2>	for the SI message(s) that, according to the posSI-SchedulingInfo or si-SchedulingInfo-v1700, if present, in the stored SIB1, contain at least one requested posSIB and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting:

3>	if onDemandSIB-Request is configured and timer T350 is not running:

4>	initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6;

4>	start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer;

4>	acquire the requested SI message(s) corresponding to the requested posSIB(s) as defined in clause 5.2.2.3.2.

NOTE:	UE may include on demand request for SIB and/or posSIB(s) in the same DedicatedSIBRequest message.

##### 5.2.2.3.6	Actions related to transmission of DedicatedSIBRequest message

The UE shall set the contents of DedicatedSIBRequest message as follows:

1>	if the procedure is triggered to request the required SIB(s):

2>	include requestedSIB-List in the onDemandSIB-RequestList to indicate the requested SIB(s);

1>	if the procedure is triggered to request the required posSIB(s):

2>	if onDemandPosSIB-RequestCtrlParam is configured:

3>	if periodic delivery of the posSIB(s) is required by the upper layers; or

3>	if the periodic delivery previously requested is no longer required by the upper layers:

4>	include the posSIB(s) in the requestedPeriodicAD-PosSIB-List-r19 and set the deliveryStatusReq to the requirement for the posSIB(s) by the upper layers;

3>	else:

4>	include the posSIB(s) in the requestedPosSIB-List-r16 in the onDemandSIB-RequestList to indicate the requested posSIB(s);

2>	else:

3>	include the posSIB(s) in the requestedPosSIB-List-r16 in the onDemandSIB-RequestList to indicate the requested posSIB(s).

The UE shall submit the DedicatedSIBRequest message to lower layers for transmission.

#### 5.2.2.4	Actions upon receipt of System Information

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the access is not for NTN or the UE is not capable of NTN; and

2>	if the UE does not support nes-CellDTX-DRX; and

2>	if the access is not for ATG or the UE is not capable of ATG; and

2>	if the UE is not IAB-MT; and

2>	if the UE is not NCR-MT; and

2>	if the cellBarred in the acquired MIB is set to barred:

3>	if the UE is an (e)RedCap UE or a 2Rx XR UE and if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]):

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

NOTE 1:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

NOTE 2:	A UE capable of ATG access should acquire SIB1 to determine whether the cell is an ATG cell.

NOTE 3:	A UE indicating any of the values in nes-CellDTX-DRX should acquire SIB1 to determine the cell barring status when the cellBarred in MIB is set to barred.

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the access is for NTN:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

3>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	if the UE is a fixed VSAT UE and the cellBarredFixedVSAT in the acquired SIB1 is set to barred or the cellBarredFixedVSAT is not included in the acquired SIB1, or

3>	if the UE is a mobile VSAT UE and the cellBarredMobileVSAT in the acquired SIB1 is set to barred or the cellBarredMobileVSAT is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the access is for ATG:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running; and

2>	if the cellBarredATG in the acquired SIB1 is set to barred or the cellBarredATG is not included in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1; or

2>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

2> else:

3>	if the cellBarredRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE supports 1 Rx branch; or

3>	if the cellBarredRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE supports 2 Rx branches:

4>	evaluate the cell barring criteria in accordance with TS 38.304 [20];

4>	if the cell is considered as barred;

5>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is a 2Rx XR UE and is in RRC_IDLE or in RRC_INACTIVE, or if the 2Rx XR UE is in RRC_CONNECTED while T311 is running:

2>	if the cellBarred2RxXR is present in the acquired SIB1:

3>	evaluate the cell barring criteria in accordance with TS 38.304 [20];

3>	if the cell is considered as barred;

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20] upon which the procedure ends;

1>	if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running:

2>	if cellBarred in the acquired MIB is set to barred:

3>	if cellBarredNES is absent in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is an eRedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the eRedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselection-eRedCap is not present in SIB1; or

2>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

2>	else:

3>	if the cellBarred-eRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE supports 1 Rx branch; or

3>	if the cellBarred-eRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE supports 2 Rx branches:

4>	evaluate the cell barring criteria in accordance with TS 38.304 [20];

4>	if the cell is considered as barred;

5>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20] upon which the procedure ends;

1>	if the cellAccessRelatedInfo contains an entry of a selected SNPN or PLMN and in case of PLMN the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN;

1>	else if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1>	if the UE in RRC_INACTIVE is configured for feature(s) that it does not support in current serving cell:

2>	not use the corresponding configuration in current serving cell;

NOTE 0:	The requirement above applies only to UE that indicates different support of UE capabilities for TN and NTN.

1>	if in RRC_CONNECTED while T311 is not running:

2>	disregard the frequencyBandList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers, if included;

2>	forward the trackingAreaList to upper layers, if included;

2>	forward the received posSIB-MappingInfo to upper layers, if included;

2>	apply the configuration included in the servingCellConfigCommon;

2>	if the UE has a stored valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

3>	use the stored version of the required SIB or posSIB;

2>	else:

3>	acquire the required SIB or posSIB requested by upper layer as defined in clause 5.2.2.3.5;

NOTE 1:	Void.

1>	else:

2>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList or frequencyBandListAerial for downlink for TDD, or one or more of the frequency bands indicated in the frequencyBandList or frequencyBandListAerial for uplink for FDD, and they are not downlink only bands, and

2>	if the UE is IAB-MT or wide area NCR-MT (see TS 38.106 [79]) or supports at least one additionalSpectrumEmission in the nr-NS-PmaxList or nr-NS-PmaxListAerial for a supported band in the downlink for TDD, or a supported band in uplink for FDD, and

2>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

-	is smaller than or equal to the carrierBandwidth (indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP or, for (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured, and

2>	if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

-	is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured), and which

-	is wider than or equal to the bandwidth of the initial downlink BWP or, for (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured, and

2>	if frequencyShift7p5khz is present and the UE supports corresponding 7.5kHz frequency shift on this band; or frequencyShift7p5khz is not present, and

2>	if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on this band:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	else if UE is IAB-MT but not a mobile IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN nor SNPN of the equivalent SNPN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	else if UE is NCR-MT and if ncr-Support is not provided:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	else if UE is a mobile IAB-MT and if mobileIAB-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN nor SNPN of the equivalent SNPN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	else:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UEs, RedCap-specific initial uplink BWP, if configured, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink or, for a (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UEs, RedCap-specific initial downlink BWP, if configured, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured;

4>	if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which SIB1 includes nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial:

5>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which SIB1 includes nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial;

4>	else if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which SIB1 does not include nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxList within frequencyBandList for the same NR frequency band number:

5>	select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which SIB1 does not include nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxList within frequencyBandList for the same NR frequency band number;

4>	else if the UE is aerial UE and SIB1 includes frequencyBandListAerial:

5>	consider the cell as barred in accordance with TS 38.304 [20];

5>	perform cell re-selection to other cells on the same frequency as the barred cell as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs is set to notAllowed as specified in TS 38.304 [20], upon which the procedure ends;

4>	else:

5>	select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the trackingAreaList to upper layers, if included;

4>	forward the received posSIB-MappingInfo to upper layers, if included;

4>	forward the PLMN identity or SNPN identity or PNI-NPN identity to upper layers;

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.13.8;

5>	if configured to receive MBS multicast in RRC_INACTIVE:

6>	if SIB24 is not scheduled in SIB1 in the new cell after cell selection (i.e., different from the cell where the UE was configured to receive multicast in RRC_CONNECTED) or in the cell after cell reselection; or

6>	if the PTM configuration is not available on the multicast MCCH in the new cell after cell selection (i.e., different from the cell where the UE was configured to receive multicast in RRC_CONNECTED) or in the cell after cell reselection for at least one multicast session that the UE has joined and for which the UE was indicated to stop monitoring G-RNTI;

7>	initiate RRC connection resume procedure for multicast reception as specified in 5.3.13.1d;

4>	forward the ims-EmergencySupport to upper layers, if present;

4>	forward the eCallOverIMS-Support to upper layers, if present;

4>	forward the UAC-AccessCategory1-SelectionAssistanceInfo or UAC-AC1-SelectAssistInfo for the selected PLMN/SNPN to upper layers, if present and set to a, b or c;

4>	if the UE is in SNPN access mode:

5>	forward the imsEmergencySupportForSNPN indicators with the corresponding SNPN identities to upper layers, if present;

4>	apply the configuration included in the servingCellConfigCommon;

4>	apply the specified PCCH configuration defined in 9.1.1.3;

4>	if the UE has a stored valid version of a SIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

5>	use the stored version of the required SIB;

4>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s), in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3;

4>	if the UE has a stored valid version of a posSIB, in accordance with clause 5.2.2.2.1, of one or several required posSIB(s), in accordance with clause 5.2.2.1:

5>	use the stored version of the required posSIB;

4> if the UE has not stored a valid version of a posSIB, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the posSI-SchedulingInfo or si-SchedulingInfo-v1700, if present, contain at least one requested posSIB and for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the posSI-SchedulingInfo or si-SchedulingInfo-v1700, if present, contain at least one requested posSIB for which posSI-BroadcastStatus in posSchedulingInfoList in posSI-SchedulingInfo or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3a;

4>	if the UE is aerial UE and, for the selected frequency band, it supports at least one additionalSpectrumEmission value indicated by nr-NS-PmaxListAerial within frequencyBandListAerial in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD:

5>	apply the first listed additionalSpectrumEmission which it supports among the values indicated by nr-NS-PmaxListAerial for the selected frequency band within frequencyBandListAerial in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD;

4>	else:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList or nr-NS-PmaxListAerial:

5>	apply the additionalPmax for UL;

4>	else:

5>	apply the p-Max in uplinkConfigCommon for UL;

4>	if supplementaryUplink is present in servingCellConfigCommon; and

4>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and

4>	if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and

4>	if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and

4>	if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

-	is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial uplink BWP of the SUL:

5>	consider supplementary uplink as configured in the serving cell;

5>	select the first frequency band in the frequencyBandList for the supplementaryUplink which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

5>	apply a supported supplementary uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which

-	is wider than or equal to the bandwidth of the initial BWP of the SUL;

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList for the supplementaryUplink;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList for the supplementaryUplink:

6>	apply the additionalPmax in supplementaryUplink for SUL;

5>	else:

6>	apply the p-Max in supplementaryUplink for SUL;

NOTE 2:	For an out of coverage L2 U2N Remote UE or L2 Intermediate U2N Relay UE in RRC_IDLE or RRC_INACTIVE receiving SIB1 from its connected parent L2 U2N Relay UE, it is up to Remote UE implementation whether to consider and apply the following parameters: frequencyBandList, carrierBandwidth, frequencyShift7p5khz, frequency band, channel bandwidth, the configuration included in the servingCellConfigCommon, the specified PCCH configuration, additionalSpectrumEmission, additionalPmax, and p-Max.

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform cell re-selection to other cells on the same frequency as the barred cell as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, or intraFreqReselection2RxXR for 2Rx XR UEs is set to notAllowed as specified in TS 38.304 [20];

##### 5.2.2.4.3	Actions upon reception of SIB2

Upon receiving SIB2, the UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if, for the entry in frequencyBandList with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within the frequencyBandList; or

2>	if, for the entry in frequencyBandListAerial with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission value indicated by the nr-Ns-PmaxListAerial within the frequencyBandListAerial:

3>	if the UE is aerial UE and it supports at least one additionalSpectrumEmission value indicated by nr-NS-PmaxListAerial:

4>	apply the first listed additionalSpectrumEmission which it supports among the values indicated by nr-NS-PmaxListAerial;

3>	else:

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList or nr-NS-PmaxListAerial:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

3>	if the UE selects a frequency band (from the procedure in clause 5.2.2.4.2) for the supplementary uplink:

4>	if, for the entry in frequencyBandListSUL with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within the frequencyBandListSUL:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandListSUL;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

4>	else:

5>	apply the p-Max.

2>	else:

3>	apply the p-Max;

##### 5.2.2.4.4	Actions upon reception of SIB3

No UE requirements related to the contents of this SIB3 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

##### 5.2.2.4.5	Actions upon reception of SIB4

Upon receiving SIB4 the UE shall:

1>	if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	for each entry in the interFreqCarrierFreqList:

3>	if the UE is neither a RedCap nor an eRedCap UE nor 2Rx XR UE; or

3>	if the UE is a RedCap UE and the interFreqCarrierFreqList-v1700 is absent; or

3>	if the UE is an eRedCap UE and the interFreqCarrierFreqList-v1800 is absent; or

3>	if the UE is a 2Rx XR UE and the interFreqCarrierFreqList-v1800 is absent; or

3>	if the UE is a RedCap UE and redCapAccessAllowed is present in interFreqCarrierFreqList-v1700; or

3>	if the UE is an eRedCap UE and eRedCapAccessAllowed is present in interFreqCarrierFreqList-v1800; or

3>	if the UE is a 2Rx XR UE and accessAllowed2RxXR is present in interFreqCarrierFreqList-v1800:

4>	select the first frequency band in the frequencyBandList (or for aerial UE frequencyBandListAerial), and frequencyBandListSUL, if present, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList (or for aerial UE as indicated by nr-NS-PmaxListAerial), if present:

4>	if, the frequency band selected by the UE in frequencyBandList or frequencyBandListAerial to represent a non-serving NR carrier frequency is not a downlink only band:

5>	if, for the selected frequency band, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within the frequencyBandList; or

5>	if, for the selected frequency band, the UE supports at least one additionalSpectrumEmission value indicated by the nr-NS-PmaxListAerial within the frequencyBandListAerial:

6>	if the UE is aerial UE and it supports at least one additionalSpectrumEmission value indicated by nr-NS-PmaxListAerial:

7>	apply the first listed additionalSpectrumEmission which it supports among the values indicated by nr-NS-PmaxListAerial;

6>	else:

7>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList;

6>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList or nr-NS-PmaxListAerial:

7>	apply the additionalPmax;

6>	else:

7>	apply the p-Max;

6>	if frequencyBandListSUL is present in SIB4 and, for the frequency band selected in frequencyBandListSUL, the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList within FrequencyBandListSUL:

7>	apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandListSUL;

7>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList:

8>	apply the additionalPmax;

7>	else:

8>	apply the p-Max;

6>	else:

7>	apply the p-Max;

5>	else:

6>	apply the p-Max;

1>	if in RRC_IDLE or RRC_INACTIVE, and T331 is running:

2>	perform the actions as specified in 5.7.8.1a;

##### 5.2.2.4.6	Actions upon reception of SIB5

No UE requirements related to the contents of this SIB5 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

##### 5.2.2.4.7	Actions upon reception of SIB6

Upon receiving the SIB6 the UE shall:

1>	forward the received warningType, messageIdentifier, serialNumber, and warningAreaCoordinates (if any) to upper layers;

##### 5.2.2.4.8	Actions upon reception of SIB7

Upon receiving the SIB7 the UE shall:

1>	if there is no current value for messageIdentifier and serialNumber for SIB7; or

1>	if either the received value of messageIdentifier or of serialNumber, or of both messageIdentifier and serialNumber are different from the current values of messageIdentifier and serialNumber for SIB7:

2>	use the received values of messageIdentifier and serialNumber for SIB7 as the current values of messageIdentifier and serialNumber for SIB7;

2>	discard any previously buffered warningMessageSegment;

2>	discard any previously buffered warningAreaCoordinatesSegment;

2>	if all segments of a warning message and geographical area coordinates (if any) have been received:

3>	assemble the warning message from the received warningMessageSegment(s);

3>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

3>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

3>	stop reception of SIB7;

3>	discard the current values of messageIdentifier and serialNumber for SIB7;

2>	else:

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	continue reception of SIB7;

1>	else if all segments of a warning message have been received:

2>	assemble the warning message from the received warningMessageSegment(s);

2>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

2>	forward the received complete warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

2>	stop reception of SIB7;

2>	discard the current values of messageIdentifier and serialNumber for SIB7;

1>	else:

2>	store the received warningMessageSegment;

2>	store the received warningAreaCoordinatesSegment (if any);

2>	continue reception of SIB7;

The UE should discard any stored warningMessageSegment and warningAreaCoordinatesSegment (if any) and the current value of messageIdentifier and serialNumber for SIB7 if the complete warning message has not been assembled within a period of 3 hours.

##### 5.2.2.4.9	Actions upon reception of SIB8

Upon receiving the SIB8 the UE shall:

1>	if the SIB8 contains a complete warning message and the complete geographical area coordinates (if any):

2>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and the geographical area coordinates (if any) to upper layers;

2>	continue reception of SIB8;

1>	else:

2>	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message and the geographical area coordinates (if any) are currently being assembled:

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	if all segments of a warning message and geographical area coordinates (if any) have been received:

4>	assemble the warning message from the received warningMessageSegment;

4>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

4>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

4>	stop assembling a warning message and geographical area coordinates (if any) for this messageIdentifier and serialNumber and delete all stored information held for it;

3>	continue reception of SIB8;

2>	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3>	start assembling a warning message for this messageIdentifier and serialNumber pair;

3>	start assembling the geographical area coordinates (if any) for this messageIdentifier and serialNumber pair;

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	continue reception of SIB8;

The UE should discard warningMessageSegment and warningAreaCoordinatesSegment (if any) and the associated values of messageIdentifier and serialNumber for SIB8 if the complete warning message and the geographical area coordinates (if any) have not been assembled within a period of 3 hours.

NOTE:	The number of warning messages that a UE can re-assemble simultaneously is a function of UE implementation.

##### 5.2.2.4.10	Actions upon reception of SIB9

Upon receiving SIB9 with referenceTimeInfo, the UE may perform the related actions in clause 5.7.1.3 except for ignoring all further referenceTimeInfo received in SIB9.

Upon receiving SIB9 with eventID-TSS, the UE shall perform the actions below if requested by upper layers:

1>	if the UE is NPN capable and the cell is an NPN-only cell:

2>	derive a gNB identity from gNB-ID-Length and cellIdentity of the first NPN-IdentityInfo list entry of NPN-IdentityInfoList in SIB1, and derive the plmn-Identity of the first NPN-Identity list entry of NPN-IdentityList in SIB1 included in the first NPN-IdentityInfo list entry of NPN-IdentityInfoList in SIB1, as defined in TS 38.413 [42];

1>	else:

2>	derive a gNB identity from gNB-ID-Length and cellIdentity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, and derive the plmn-Identity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, as defined in TS 38.413 [42];

1>	if VarTSS-Info does not have a storedEventID; or

1>	if the storedEventID within VarTSS-Info is different from the eventID-TSS received within SIB9; or

1>	if the storedGlobalGnbID within the VarTSS-Info is different from the global gNB identity derived from SIB1:

2>	notify upper layers that clockQualityDetailsLevel may have changed;

2>	store the eventID-TSS received within SIB9 as the storedEventID within VarTSS-Info;

2>	store the global gNB identity derived from SIB1 as the storedGlobalGnbID within VarTSS-Info.

##### 5.2.2.4.11	Actions upon reception of SIB10

Upon receiving SIB10, the UE shall:

1>	Forward the HRNN-list entries with the corresponding PNI-NPN and SNPN identities to upper layers;

##### 5.2.2.4.12	Actions upon reception of SIB11

Upon receiving SIB11, the UE shall:

1>	if in RRC_IDLE or RRC_INACTIVE, and T331 is running:

2>	perform the actions as specified in 5.7.8.1a;

##### 5.2.2.4.13	Actions upon reception of SIB12

Upon receiving SIB12, the UE shall:

1>	if the UE has stored at least one segment of SIB12 and the value tag of SIB12 has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB12-IEs from the received segments;

2>	if sl-FreqInfoList/sl-FreqInfoListSizeExt is included in SIB12-IEs:

3>	if configured to receive NR sidelink communication:

4>	use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7;

3>	if configured to transmit NR sidelink communication:

4>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

4>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.5.3.1;

4>	use the synchronization configuration parameters for NR sidelink communication on frequencies included in sl-FreqInfoList/sl-FreqInfoListSizeExt, as specified in 5.8.5;

3>	if configured to perform SL-PRS measurement:

4>	use the resource pool(s) indicated by sl-RxPool for SL-PRS measurement, as specified in 5.8.18.2;

3>	if configured to transmit SL-PRS:

4>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, or sl-TxPoolExceptional for SL-PRS transmission, as specified in 5.8.18.3;

4>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal or sl-TxPoolExceptional for SL-PRS, as specified in 5.5.3.1;

4>	use the synchronization configuration parameters for NR sidelink positioning on frequencies included in sl-FreqInfoList/sl-FreqInfoListSizeExt, as specified in 5.8.5;

3>	if configured to receive NR sidelink discovery:

4>	use the resource pool(s) indicated by sl-DiscRxPool or sl-RxPool for NR sidelink discovery reception, as specified in 5.8.13.2;

3>	if configured to transmit NR sidelink discovery:

4>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-MH-Relay is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages and sl-L3U2N-RelayDiscovery is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink non-relay discovery messages and sl-NonRelayDiscovery is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L2 U2U relay discovery messages and sl-L2-U2U-Relay is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L3 U2U relay discovery messages and sl-L3-U2U-RelayDiscovery is included in SIB12:

5>	use the resource pool(s) indicated by sl-DiscTxPoolSelected, sl-TxPoolExceptional or sl-TxPoolSelectedNormal for NR sidelink discovery transmission, as specified in 5.8.13.3;

5>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-DiscTxPoolSelected or sl-TxPoolExceptional for NR sidelink discovery transmission, as specified in 5.5.3.1;

5>	use the synchronization configuration parameters for NR sidelink discovery on frequencies included in sl-FreqInfoList, as specified in 5.8.5;

2>	if sl-RadioBearerConfigList or sl-RLC-BearerConfigList is included in sl-ConfigCommonNR:

3>	perform sidelink DRB addition/modification/release as specified in 5.8.9.1a.1/5.8.9.1a.2;

3>	if sl-RLC-BearerConfigListSizeExt is included in SIB12-IEs:

4>	perform additional sidelink RLC bearer addition/modification/release as specified in 5.8.9.1a.5/5.8.9.1a.6;

2> if sl-MeasConfigCommon is included in sl-ConfigCommonNR:

3> store the NR sidelink measurement configuration;

2>	if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

3>	store the NR sidelink DRX configuration and configure lower layers to perform sidelink DRX operation for groupcast and broadcast as specified in TS 38.321 [3];

1>	if the UE is acting as L2 U2N Remote UE:

2>	if the sl-TimersAndConstantsRemoteUE is included in SIB12:

3>	use values for timers T300, T301 and T319 as included in the sl-TimersAndConstantsRemoteUE received in SIB12;

2>	else:

3>	use values for timers T300, T301 and T319 as included in the ue-TimersAndConstants received in SIB1;

The UE should discard any stored segments for SIB12 if the complete SIB12 has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB12 upon cell (re-)selection.

NOTE:	The L2 U2U UE is allowed to use previous configuration based on SIB12 before receiving dedicated configuration during and immediately after state transition from idle/inactive to connected.

##### 5.2.2.4.14	Actions upon reception of SIB13

Upon receiving SIB13, the UE shall perform the actions upon reception of SystemInformationBlockType21 as specified in 5.2.2.28 in TS 36.331 [10].

##### 5.2.2.4.15	Actions upon reception of SIB14

Upon receiving SIB14, the UE shall perform the actions upon reception of SystemInformationBlockType26 as specified in 5.2.2.33 in TS 36.331 [10].

##### 5.2.2.4.16	Actions upon reception of SIBpos

No UE requirements related to the contents of the SIBpos apply other than those specified elsewhere e.g. within TS 37.355 [49], and/or within the corresponding field descriptions.

##### 5.2.2.4.17	Actions upon reception of SIB15

Upon receiving SIB15, the UE shall:

1>	forward the applicable disaster roaming information for each PLMN sharing the cell to upper layers.

##### 5.2.2.4.18	Actions upon reception of SIB16

Upon receiving SIB16 with cell reselection priorities for slicing, the UE shall perform the actions as specified in TS 38.304 [20].

##### 5.2.2.4.19	Actions upon reception of SIB17

Upon receiving SIB17, the UE shall:

1>	if the UE has stored at least one segment of SIB17 and the value tag of SIB17 has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB17-IEs from the received segments.

The UE should discard any stored segments for SIB17 if the complete SIB17 has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB17 upon cell (re-) selection.

##### 5.2.2.4.19a	Actions upon reception of SIB17bis

Upon receiving SIB17bis, the UE shall:

1>	if the UE has stored at least one segment of SIB17bis and the value tag of SIB17bis has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB17bis-IEs from the received segments.

The UE should discard any stored segments for SIB17bis if the complete SIB17bis has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB17bis upon cell (re-) selection.

##### 5.2.2.4.20	Actions upon reception of SIB18

Upon receiving SIB18, the UE shall:

1>	forward the Group IDs for Network selection (GINs) in SIB18 with the corresponding SNPN identities to upper layers;

##### 5.2.2.4.21	Actions upon reception of SIB19

Upon receiving SIB19 in an NTN cell, the UE in RRC_CONNECTED shall:

1>	start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell.

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

##### 5.2.2.4.22	Actions upon reception of SIB20

No UE requirements related to the contents of SIB20 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

##### 5.2.2.4.23	Actions upon reception of SIB21

No UE requirements related to the contents of SIB21 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

##### 5.2.2.4.24	Actions upon reception of SIB22

No UE requirements related to the contents of SIB22 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

##### 5.2.2.4.25	Actions upon reception of SIB23

Upon receiving SIB23, the UE shall:

1>	if the UE has stored at least one segment of SIB23 and the value tag of SIB23 has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB23-IEs from the received segments;

2>	if sl-PosFreqInfoList is included in sl-PosConfigCommonNR:

3>	if configured to perform SL-PRS measurement:

4>	use the resource pool(s) indicated by sl-PRS-RxPool for SL-PRS measurement, as specified in 5.8.18.2;

3>	if configured to transmit SL-PRS:

4>	use the resource pool(s) indicated by sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional for SL-PRS transmission, as specified in 5.8.18.3;

4>	perform CBR measurement on the transmission resource pool(s) indicated by sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional for SL-PRS transmission, as specified in 5.5.3.1;

4>	use the synchronization configuration parameters for NR sidelink positioning on frequencies included in sl-PosFreqInfoList, as specified in 5.8.5;

The UE should discard any stored segments for SIB23 if the complete SIB23 has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB23 upon cell (re-)selection.

##### 5.2.2.4.26	Actions upon reception of SIB24

No UE requirements related to the contents of SIB24 apply other than those specified elsewhere e.g., within procedures using the concerned system information, and/or within the corresponding field descriptions.

##### 5.2.2.4.27	Actions upon reception of SIB25

No UE requirements related to the contents of SIB25 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

##### 5.2.2.4.28	Actions upon reception of SIB26

Upon receiving SIB26, the UE shall:

1>	store the SIB26;

1>	consider the stored SIB26 is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.2.3.1;

1>	consider the stored SIB26 is valid for cell indicated in validSIB26-Cells for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIB26 is a valid version for that cell in accordance with clause 5.2.2.2.1.

##### 5.2.2.4.29	Actions upon reception of SIB27

No UE requirements related to the contents of SIB27 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB or SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20].

NOTE 1:	The SIB19 is essential for NTN access. If UE is unable to acquire the SIB19 for NTN access, the action is up to UE implementation (e.g., cell re-selection to other cells).

NOTE 2: The SIB22 is essential for ATG access. If UE is unable to acquire the SIB22 for ATG access, the action is up to UE implementation (e.g., cell re-selection to other cells).

#### 5.2.2.6	T430 expiry

The UE shall:

1>	if T430 for serving cell expires and if in RRC_CONNECTED:

2>	inform lower layers that UL synchronisation is lost;

2>	acquire SIB19 as defined in clause 5.2.2.3.2;

2>	upon successful acquisition of SIB19:

3>	inform lower layers when UL synchronisation is obtained;

NOTE:	The exact time when UL synchronisation is obtained (after SIB19 is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime.

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful AS security activation. However, the UE only accepts a re-configuration with sync message when AS security has been activated.

Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial AS security activation procedure. The RRC messages to activate AS security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate AS security is not ciphered, while the subsequent messages (e.g. used to establish SRB2, DRBs and multicast MRBs) are both integrity protected and ciphered. After having initiated the initial AS security activation procedure, the network may initiate the establishment of SRB2 and DRBs and/or multicast MRBs, i.e. the network may do this prior to receiving the confirmation of the initial AS security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2, DRBs and/or multicast MRBs. The network should release the RRC connection if the initial AS security activation and/ or the radio bearer establishment fails. A configuration with SRB2 without DRB or multicast MRB, or with DRB or multicast MRB without SRB2 is not supported (i.e., SRB2 and at least one DRB or multicast MRB must be configured in the same RRC Reconfiguration message, and it is not allowed to release all the DRBs and multicast MRBs without releasing the RRC Connection). For IAB-MT and NCR-MT, a configuration with SRB2 without any DRB/MRB is supported.

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an E-UTRA carrier frequency.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN or for SDT or for multicast reception. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s) and/or multicast MRB(s), if configured.

Upon initiating the resume procedure for SDT, AS security (both ciphering and integrity protection) is re-activated for SRB2 (if configured for SDT) and for SRB1. In addition, AS security is also re-activated (if security is configured) for all the DRBs configured for SDT. Further, the PDCP entities of SRB1 and PDCP entities of the radio bearers configured for SDT are re-established and resumed whilst the UE remains in RRC_INACTIVE state. Transmission and reception of data and/or signalling messages over radio bearers configured for SDT can happen whilst the UE is in RRC_INACTIVE state and SDT procedure is ongoing.

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

NOTE:	In case the UE receives the configurations for NR sidelink communication via the E-UTRA, the configurations for NR sidelink communication in SIB12 and sl-ConfigDedicatedNR within RRCReconfiguration used in clause 5.3 are provided by the configurations in SystemInformationBlockType28 and sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331[10], respectively.

#### 5.3.1.2	AS Security

AS security comprises of the integrity protection and ciphering of RRC signalling (SRBs) and user data (DRBs).

RRC handles the configuration of the AS security parameters which are part of the AS configuration: the integrity protection algorithm, the ciphering algorithm, if integrity protection and/or ciphering is enabled for a DRB and two parameters, namely the keySetChangeIndicator and the nextHopChainingCount, which are used by the UE to determine the AS security keys upon reconfiguration with sync (with key change), connection re-establishment and/or connection resume.

The integrity protection algorithm is common for SRB1, SRB2, SRB3 (if configured), SRB4 (if configured), SRB5 (if configured), SRB6 (if configured) and DRBs configured with integrity protection, with the same keyToUse value. The ciphering algorithm is common for SRB1, SRB2, SRB3 (if configured), SRB4 (if configured), SRB5 (if configured), SRB6 (if configured) and DRBs configured with the same keyToUse value. Neither integrity protection nor ciphering applies for SRB0.

NOTE 0:	All DRBs related to the same PDU session have the same enable/disable setting for ciphering and the same enable/disable setting for integrity protection, as specified in TS 33.501 [11].

RRC integrity protection and ciphering are always activated together, i.e. in one message/procedure. RRC integrity protection and ciphering for SRBs are never de-activated. However, it is possible to switch to a 'NULL' ciphering algorithm (nea0).

The 'NULL' integrity protection algorithm (nia0) is used only for SRBs and for the UE in limited service mode, see TS 33.501 [11] and when used for SRBs, integrity protection is disabled for DRBs. In case the ′NULL' integrity protection algorithm is used, 'NULL' ciphering algorithm is also used.

NOTE 1:	Lower layers discard RRC messages for which the integrity protection check has failed and indicate the integrity protection verification check failure to RRC.

The AS applies four different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc), one for integrity protection of user data (KUPint) and one for the ciphering of user data (KUPenc). All four AS keys are derived from the KgNB key. The KgNB key is based on the KAMF key (as specified in TS 33.501 [11]), which is handled by upper layers.

The integrity protection and ciphering algorithms can only be changed with reconfiguration with sync. The AS keys (KgNB, KRRCint, KRRCenc, KUPint and KUPenc) change upon reconfiguration with sync (if masterKeyUpdate is included), and upon connection re-establishment and connection resume.

For each radio bearer an independent counter (COUNT, as specified in TS 38.323 [5]) is maintained for each direction. For each radio bearer, the COUNT is used as input for ciphering and integrity protection.

It is not allowed to use the same COUNT value more than once for a given security key. As specified in TS 33.501 clause 6.9.4.1 [11], the network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same key, e.g. due to the transfer of large volumes of data, release and establishment of new RBs, and multiple termination point changes for RLC-UM bearers and multiple termination point changes for RLC-AM bearer with SN terminated PDCP re-establishment (COUNT reset) due to SN only full configuration whilst the key stream inputs (i.e. bearer ID, security key) at MN have not been updated. In order to avoid such re-use, the network may e.g. use different RB identities for RB establishments, change the AS security key, or an RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE and then to RRC_CONNECTED transition.

In order to limit the signalling overhead, individual messages/ packets include a short sequence number (PDCP SN, as specified in TS 38.323 [5]). In addition, an overflow counter mechanism is used: the hyper frame number (HFN, as specified in TS 38.323 [5]). The HFN needs to be synchronized between the UE and the network.

For each SRB, the value provided by RRC to lower layers to derive the 5-bit BEARER parameter used as input for ciphering and for integrity protection is the value of the corresponding srb-Identity with the MSBs padded with zeroes.

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

### 5.3.2	Paging

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit paging information to a UE in RRC_IDLE or RRC_INACTIVE.

-	to transmit paging information for a L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE to its serving L2 U2N Relay UE in any RRC state.

#### 5.3.2.2	Initiation

The network initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in TS 38.304 [20]. The network may address multiple UEs within a Paging message by including one PagingRecord for each UE. The network may also include one or multiple TMGI(s) in the Paging message to page UEs for specific MBS multicast session(s).

#### 5.3.2.3	Reception of the Paging message by the UE or PagingRecord by the L2 U2N Remote UE

Upon receiving the Paging message by the UE or receiving PagingRecord from its connected parent L2 U2N Relay UE by a L2 U2N Remote UE, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_IDLE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected parent L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

NOTE 1:	If the parent L2 U2N Relay UE supports the MUSIM feature, it can forward the paging cause to the connected L2 U2N Remote UE or to the child UE.

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected parent L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

3>	else if mt-SDT indication was included in the Paging message and if the conditions for initiating SDT for a resume procedure initiated in response to RAN paging according to 5.3.13.1b are fulfilled:

4>	if pagingGroupList was not included in the Paging message; or

4>	if pagingGroupList was included in the Paging message but the UE has not joined any MBS session(s) indicated by the TMGI(s) included in the pagingGroupList; or

4>	if pagingGroupList was included in the Paging message, all the MBS session(s) indicated by the TMGI(s) included in the pagingGroupList that the UE has joined are configured to be received in RRC_INACTIVE, and inactiveReceptionAllowed was included for all these MBS session(s):

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-SDT;

NOTE 1a:	If a UE receives a Paging message including mt-SDT indication and inactiveReceptionAllowed indications for all the multicast session(s) the UE has joined and the UE initiates RRC connection resume, the UE starts monitoring the corresponding G-RNTI(s), if configured, and if multicast MCCH is present, the UE starts monitoring the Multicast MCCH-RNTI and acquires the MBSMulticastConfiguration message on multicast MCCH.

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE 2:	If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.

NOTE 3:	A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [23].

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other';

1>	if in RRC_IDLE, for each TMGI included in pagingGroupList, if any, included in the Paging message:

2>	if the UE has joined an MBS session indicated by the TMGI included in the pagingGroupList:

3>	forward the TMGI to the upper layers;

1>	if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList:

2>	if PagingRecordList is not included in the Paging message; or

2>	if none of the ue-Identity included in any of the PagingRecord matches the UE identity allocated by upper layers or the UE's stored fullI-RNTI:

3>	if the UE is not configured to receive multicast in RRC_INACTIVE for at least one of the MBS sessions indicated by the TMGI(s) included in pagingGroupList that the UE has joined; or

3>	if inactiveReceptionAllowed is not included for at least one of the MBS sessions indicated by the TMGI(s) included in pagingGroupList that the UE has joined:

4>	initiate the RRC connection resumption procedure according to 5.3.13.1d;

3>	else:

4>	start monitoring the G-RNTI(s), if configured, corresponding to the TMGI(s);

4> if the UE was notified to stop monitoring the G-RNTI(s) for all the joined multicast sessions that are configured for reception in RRC_INACTIVE:

5>	apply the multicast PTM configuration provided in RRCRelease;

5>	if multicast MCCH is present:

6>	start monitoring the Multicast MCCH-RNTI;

6>	acquire the MBSMulticastConfiguration message on multicast MCCH;

4>	else if the UE was notified to stop monitoring the G-RNTI for at least one of the multicast sessions indicated by the TMGI(s) included in pagingGroupList for which the PTM configuration was not included in RRCRelease message:

5>	apply the multicast PTM configuration provided in RRCRelease;

5>	acquire the MBSMulticastConfiguration message on multicast MCCH;

2>	else if the ue-Identity included in any of the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the TMGI(s) to the upper layers;

1>	if the UE is acting as a L2 U2N Relay UE in case of single hop or L2 Last U2N Relay UE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord in the Paging message matches the UE identity in sl-PagingIdentityRemoteUE included in sl-PagingInfo-RemoteUE received in RemoteUEInformationSidelink message from a L2 U2N Remote UE or from a child L2 U2N Relay UE:

3>	inititate the Uu Message transfer in sidelink to that UE as specified in 5.8.9.9;

### 5.3.3	RRC connection establishment

#### 5.3.3.1	General

Figure 5.3.3.1-1: RRC connection establishment, successful

Figure 5.3.3.1-2: RRC connection establishment, network reject

The purpose of this procedure is to establish an RRC connection. RRC connection establishment involves SRB1 establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network.

The network applies the procedure e.g.as follows:

-	When establishing an RRC connection;

-	When UE is resuming or re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context. In this case, UE receives RRCSetup and responds with RRCSetupComplete.

#### 5.3.3.1a	Conditions for establishing RRC Connection for NR sidelink communication/discovery/V2X sidelink communication/MP operation

For NR sidelink communication/discovery, an RRC connection establishment is initiated only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to transmit NR sidelink discovery and related data is available for transmission:

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2U relay discovery messages and sl-L2-U2U-Relay-r18 is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2U relay discovery messages and sl-L3-U2U-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages and sl-L3U2N-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink non-relay discovery messages and sl-NonRelayDiscovery is included in SIB12:

3>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 includes neither sl-DiscTxPoolSelected nor sl-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_IDLE, an RRC connection establishment is initiated in the following cases:

1>	if any message is received from a L2 U2N Remote UE or from a child U2N Relay UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4; or

1>	if RemoteUEInformationSidelink containing the connectionForMP is received from a L2 U2N Remote UE as specified in 5.8.9.8.3;

For V2X sidelink communication, an RRC connection is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE 1:	Upper layers initiate an RRC connection (except if the RRC connection is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE or from a child U2N Relay UE via SL-RLC0 or SL-RLC1, or upon reception of RemoteUEInformationSidelink message containing the connectionForMP). The interaction with NAS is left to UE implementation.

For N3C relay UE in RRC_IDLE, an RRC connection establishment is initiated when a N3C remote UE indicates it to enter RRC_CONNECTED state.

NOTE 2:	How/when the N3C remote UE to indicate N3C relay UE to enter RRC_CONNECTED state is left to UE implementation, e.g. before reporting relay UE information with non-3GPP connection(s).

#### 5.3.3.1b	Void

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information, or for sidelink communication as specified in clause 5.3.3.1a.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

2>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and/or in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

NOTE:	If there are multiple NSAGs with the same highest NAS-provided NSAG priority identified for access attempt as above, it is left to UE implementation to select the NSAG to be applied in the Random Access procedure.

1>	if the UE is acting as L2 U2N Remote UE or is acting as L2 Intermediate U2N Relay UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

#### 5.3.3.3	Actions related to transmission of RRCSetupRequest message

The UE shall set the contents of RRCSetupRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	set the ue-Identity to ng-5G-S-TMSI-Part1;

2>	else:

3>	draw a 39-bit random value in the range 0..239-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the 5G-S-TMSI if the UE is registered in the TA of the current cell.

1>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN):

2>	set the establishmentCause to mps-PriorityAccess;

1>	else:

2>	set the establishmentCause in accordance with the information received from upper layers;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection establishment triggered either by reception of message from a L2 U2N Remote UE or from a child UE via SL-RLC0 or SL-RLC1, or by reception of message RemoteUEInformationSidelink containing the connectionForMP, as specified in 5.3.3.1a, the L2 U2N Relay UE sets the establishmentCause by implementation, but: (1) for SL-RLC0, it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as establishmentCause if the same cause value is in the message received from the L2 U2N Remote UE or from a child UE via SL-RLC0; and (2) for SL-RLC1, it sets the establishmentCause to emergency if the message received from the L2 U2N Remote UE or from a child UE via SL-RLC1 is over PC5 link established for emergency service as indicated by upper layer [72].

1>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

If the UE is an (e)RedCap UE and the RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.6.

NOTE 3:	For L2 U2N Remote UE or L2 Intermediate U2N Relay UE in RRC_IDLE, the cell (re)selection procedure as specified in TS 38.304 [20] and relay (re)selection procedure as specified in 5.8.15.3 are performed independently and up to UE implementation to select either a cell or a L2 U2N Relay UE.

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if the UE is NCR-MT:

3>	indicate to NCR-Fwd to cease forwarding;

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

3>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG indicated by tag-Id, if it is not running;

2>	if srs-PosRRC-Inactive is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running;

2>	if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT, if it is running;

2>	if the UE is configured to receive MBS multicast in RRC_INACTIVE:

3>	reset MAC;

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs;

2>	indicate to upper layers fallback of the RRC connection;

2>	for each application layer measurement configuration with appLayerIdleInactiveConfig absent:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301, T319;

1>	if T319a is running:

2>	stop T319a;

2>	consider SDT procedure is not ongoing;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	if T331 is running:

3>	stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

2>	stop relay (re)selection procedure if any for L2 U2N Remote UE or L2 Intermediate U2N Relay UE;

1>	consider the current cell to be the PCell;

1>	perform the L2 U2N Remote UE or L2 Intermediate U2N Relay UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	perform the sidelink dedicated configuration procedure in accordance with the received sl-ConfigDedicatedNR as specified in 5.3.5.14;

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

2>	if reconnectCellId in VarRLF-Report is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	if the UE supports RLF-Report for conditional handover and if choCellId in VarRLF-Report is set; or

3>	if the UE supports RLF-Report for MCG LTM cell switch and if ltm-RecoveryCellId in VarRLF-Report is set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the radio link failure or reconfiguration with sync failure experienced in the failedPCellId stored in VarRLF-Report;

3>	else:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or reconfiguration with sync failure;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

2>	if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link failure or handover failure in LTE;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	if upper layers selected an SNPN or a PLMN and in case of PLMN UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

3>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

2>	else:

3>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	if upper layers provide onboarding request indication:

3>	include the onboardingRequest;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

2>	if connecting as an IAB-node but not as a mobile IAB-node:

3>	include the iab-NodeIndication;

2>	else if connecting as a mobile IAB-node:

3>	include the mobileIAB-NodeIndication;

2>	if connecting as an NCR-node:

3>	include the ncr-NodeIndication;

2>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

2>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

3>	include the idleMeasAvailable;

2>	if the SIB1 contains reselectionMeasurementsNR:

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig and the UE has NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; or

3>	if measReselectionCarrierListNR is not present in VarMeasReselectionConfig and if the UE has NR reselection measurements available:

4>	include the reselectionMeasAvailable;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

2>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included; or

2>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

3>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the current registered SNPN identity is equal to snpn-Identity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report, or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10], and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarRLF-Report:

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

3>	include successPSCell-InfoAvailable in the RRCSetupComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCSetupComplete message;

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	include measConfigReportAppLayerAvailable in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation according to the network indication rrc-SegAllowed:

3>	may include the ul-RRC-Segmentation in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation according to the network indication rrc-MaxCapaSegAllowed:

3>	include the ul-RRC-MaxCapaSegments in the RRCSetupComplete message;

2>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

3>	if speedStateReselectionPars is configured in the SIB2:

4>	include the mobilityState in the RRCSetupComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCSetupComplete message upon determining it has temporary capability restriction;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

NOTE:	Upon reception of musim-CapRestrictionInd in RRCSetupComplete, it is up to network implementation to configure the UE with a limited configuration that is used until network sends RRCReconfiguration based on the actual restricted UE capabilities included in UEAssistanceInformation.

#### 5.3.3.5	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15;

#### 5.3.3.6	Cell re-selection or cell selection or relay (re)selection while T390, T300 or T302 is running (UE in RRC_IDLE)

The UE shall:

1>	if cell reselection occurs while T300 or T302 is running; or

1>	if relay (re)selection or cell selection by a L2 U2N Remote UE or by a L2 Intermediate U2N Relay UE, occurs while T300 is running; or

1>	if cell changes due to relay (re)selection or cell selection by a L2 U2N Remote UE or L2 Intermediate U2N Relay UE while T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC connection failure';

1>	else:

2>	if cell selection or reselection occurs while T390 is running; or

2>	cell change due to relay selection or reselection occurs while T390 is running:

3>	stop T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4.

#### 5.3.3.7	T300 expiry

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except broadcast MRBs);

2>	if the UE supports RRC Connection Establishment failure with temporary offset and the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	if the UE supports multiple CEF report:

3>	if the UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity in networkIdentity stored in VarConnEstFailReport; or

3>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport:

4>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

5>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

2>	if the UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity in networkIdentity stored in VarConnEstFailReport; or

2>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is not equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReportList and if the RPLMN is not equal to plmn-identity in networkIdentity stored in any entry of VarConnEstFailReportList;or

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReportList and if the registered SNPN identity is not equal to snpn-Identity in networkIdentity stored in any entry of VarConnEstFailReportList:

3>	clear the content included in VarConnEstFailReportList;

2>	clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	if the UE is not in SNPN access mode:

4>	set the plmn-Identity in networkIdentity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

3>	else if the UE is in SNPN access mode:

4>	set the snpn-Identity in networkIdentity to include the SNPN identity selected by upper layers (see TS 24.501 [23]) from the list of SNPN(s) included in the npn-IdentityInfoList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection establishment failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

NOTE 3:	Which location information related configuration is used by the UE to make the locationInfo available for inclusion in the VarConnEstFailReport is left to UE implementation.

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

3>	if the numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure or connection resume failure information, i.e. release the UE variable VarConnEstFailReport and the UE variable VarConnEstFailReportList, 48 hours after the last connection establishment failure is detected.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the child UE(s) in accordance with 5.8.9.10.

#### 5.3.3.8	Abortion of RRC connection establishment

If upper layers abort the RRC connection establishment procedure, due to a NAS procedure being aborted as specified in TS 24.501 [23], while the UE has not yet entered RRC_CONNECTED, the UE shall:

1>	stop timer T300, if running;

1>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except broadcast MRBs).

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the child UE(s) in accordance with 5.8.9.10.

The L2 U2N Remote UE or the L2 First U2N Relay UE or L2 Intermediate U2N Relay UE indicates to upper layers to trigger PC5 unicast link release with its connected parent L2 U2N Relay UE.

### 5.3.4	Initial AS security activation

#### 5.3.4.1	General

Figure 5.3.4.1-1: Security mode command, successful

Figure 5.3.4.1-2: Security mode command, failure

The purpose of this procedure is to activate AS security upon RRC connection establishment.

#### 5.3.4.2	Initiation

The network initiates the security mode command procedure to a UE in RRC_CONNECTED. Moreover, the network applies the procedure as follows:

-	when only SRB1 is established, i.e. prior to establishment of SRB2, multicast MRBs and/ or DRBs.

#### 5.3.4.3	Reception of the SecurityModeCommand by the UE

The UE shall:

1>	derive the KgNB key, as specified in TS 33.501 [11];

1>	derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	configure lower layers to apply SRB integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2>	configure lower layers to apply SRB ciphering using the indicated algorithm, the KRRCenc keyafter completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	consider AS security to be activated;

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	continue using the configuration used prior to the reception of the SecurityModeCommand message, i.e. neither apply integrity protection nor ciphering.

2>	submit the SecurityModeFailure message to lower layers for transmission, upon which the procedure ends.

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs/BH RLC channels/Uu Relay RLC channels/PC5 Relay RLC channels, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, to add/modify/release conditional reconfiguration configuration, to add/modify/release LTM configuration, and to add/modify/release MP configuration. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators;

-	reconfiguration with sync for DAPS and security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: refresh of security and re-establishment of RLC and PDCP triggered by explicit indicators;

-	for DAPS bearer: establishment of RLC for the target PCell, refresh of security and reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: refresh of security and establishment of RLC and PDCP for the target PCell;

-	reconfiguration with sync for DAPS but without security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators.

-	for DAPS bearer: establishment of RLC for target PCell, reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: establishment of RLC and PDCP for the target PCell.

-	reconfiguration with sync for direct-to-indirect path switch or indirect-to-indirect path switch, not involving RA at target side, involving re-establishment of PDCP /PDCP data recovery (for AM DRB) triggered by explicit indicators;

-	reconfiguration with sync for LTM cell switch, and

-	depending on a network indication, involving or not involving security key refresh;

-	involving or not involving RA to the target LTM candidate SpCell according to a network indication;

-	MAC reset;

-	depending on a network indication, involving or not involving re-establishment of RLC and PDCP data recovery (for AM DRB), or re-establishment of RLC and re-establishment of PDCP.

In (NG)EN-DC and NR-DC, SRB3 can be used for measurement configuration and reporting, for UE assistance (re-)configuration and reporting for power savings, for IP address (re-)configuration and reporting for IAB-nodes, to (re-)configure MAC, RLC, BAP, physical layer and RLF timers and constants of the SCG configuration, to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, to reconfigure SDAP for DRBs associated with S-KgNB in NGEN-DC and NR-DC, to add/modify/release conditional PSCell change configuration or subsequent CPAC configuration, and to add/modify/release the LTM configuration associated with the SCG (only in NR-DC), provided that the (re-)configuration does not require any MN involvement, and to transmit RRC messages between the MN and the UE during fast MCG link recovery. In (NG)EN-DC and NR-DC, only measConfig, radioBearerConfig, conditionalReconfiguration, ltm-Config (only in NR-DC), bap-Config, iab-IP-AddressConfigurationList, otherConfig, appLayerMeasConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3, except when RRCReconfiguration is received within DLInformationTransferMRDC.

When a clause of 5.3.5 is executed due to an LTM cell switch execution (i.e., as specified in 5.3.5.18.6) or due to a conditional reconfiguration execution for subsequent CPAC (i.e., as specified in 5.3.5.13.8), every appearance of "the received" before RRCReconfiguration message, before a field name, or before an IE name, refers to the RRCReconfiguration message that the UE applies, as specified in 5.3.5.18.6, 5.3.5.13.8, or the field or IE in that RRCReconfiguration message.

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the establishment of BH RLC Channels for IAB is performed only when AS security has been activated;

-	the configuration of NCR-Fwd is performed only when AS security has been activated;

-	the establishment of Uu Relay RLC channels and PC5 Relay RLC channels (other than SL-RLC0 and SL-RLC1) for L2 U2N Relay UE in case of single hop or for L2 Last U2N Relay UE is performed only when AS security has been activated, and the establishment of PC5 Relay RLC channels for L2 U2N Remote UE or for L2 Intermediate U2N Relay UE (other than SL-RLC0 and SL-RLC1) is performed only when AS security has been activated;

-	the establishment of PC5 Relay RLC channels for L2 U2U Relay UE and L2 U2U Remote UE is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one RLC bearer or BH RLC channel is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have been setup and not suspended;

-	the conditionalReconfiguration for CPC is included only when at least one RLC bearer is setup in SCG;

-	the conditionalReconfiguration for CHO, CPA, or subsequent CPAC is included only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB or, for IAB, SRB2, are setup and not suspended;

-	the addition of indirect path for MP is performed only when AS security has been activated;

-	the ltm-Config for LTM on the MCG is included only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended;

-	the ltm-Config or ltm-ConfigNRDC for LTM on the SCG is included only when at least one RLC bearer is setup in SCG.

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

1>	if the RRCReconfiguration includes the daps-SourceRelease:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	if the RRCReconfiguration is received while the timer T348 is running:

2>	if the configuration does not exceed UE temporary capability restriction indicated via musim-CapRestriction included in the last transmission of UEAssistanceInformation:

3>	stop the timer T348;

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure which requires an update of the master security key, according to clause 5.3.5.18.6:

3>	if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release:

4>	perform security key update procedure as specified in 5.3.5.7;

2>	else if this RRCReconfiguration message is not applied due to an LTM cell switch execution procedure:

3>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

NOTE 00:	If the UE receives, within an LTM candidate configuration, an mrdc-SecondaryCellGroupConfig set to release even if no SCG is currently configured at the UE, the UE does not consider this as an invalid configuration.

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

NOTE 0:	If this RRCReconfiguration is associated to the MCG and includes reconfigurationWithSync in spCellConfig and dedicatedSIB1-Delivery, the UE initiates (if needed) the request to acquire required SIBs, according to clause 5.2.2.3.5, only after the random access procedure or the LTM cell switch execution towards the target SpCell is completed.

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the dedicatedPosSysInfoDelivery:

2>	perform the action upon reception of the contained posSIB(s), as specified in clause 5.2.2.4.16;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	if the RRCReconfiguration message includes the bap-Config:

2>	perform the BAP configuration procedure as specified in 5.3.5.12;

1>	if the RRCReconfiguration message includes the iab-IP-AddressConfigurationList:

2>	if iab-IP-AddressToReleaseList is included:

3>	perform release of IP address as specified in 5.3.5.12a.1.1;

2>	if iab-IP-AddressToAddModList is included:

3>	perform IAB IP address addition/update as specified in 5.3.5.12a.1.2;

1>	if the RRCReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.13;

1>	if the RRCReconfiguration message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	if the RRCReconfiguration message includes the onDemandSIB-Request:

2>	if onDemandSIB-Request is set to setup:

3>	consider itself to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

2>	else:

3>	consider itself not to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

NOTE 0a:	If the sl-ConfigDedicatedNR was received embedded within an E-UTRA RRCConnectionReconfiguration message, the UE does not build an NR RRCReconfigurationComplete message for the received sl-ConfigDedicatedNR.

1>	if the RRCReconfiguration message includes the sl-L2RelayUE-Config:

2>	perform the L2 U2N or U2U Relay UE configuration procedure as specified in 5.3.5.15;

1>	if the RRCReconfiguration message includes the sl-L2RemoteUE-Config:

2>	perform the L2 U2N or U2U Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCReconfiguration message includes the dedicatedPagingDelivery:

2>	perform the Paging message reception procedure as specified in 5.3.2.3;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedEUTRA-Info:

2>	perform related procedures for V2X sidelink communication in accordance with TS 36.331 [10], clause 5.3.10 and clause 5.5.2;

1>	if the RRCReconfiguration message includes the ul-GapFR2-Config:

2>	perform the FR2 UL gap configuration procedure as specified in 5.3.5.13c;

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

1>	if the RRCReconfiguration message includes the appLayerMeasConfig:

2>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

4>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

4>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

4>	consider itself not to be configured to send application layer measurement report for the measConfigAppLayerId;

2>	if idleInactiveReportAllowed is included in the RRCReconfiguration message:

3> if the UE is configured with at least one application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	initiate the procedure in 5.7.16.2 after the RRCReconfigurationComplete has been transmitted;

2>	else:

3>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

4>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

4>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

4>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCReconfiguration message includes the ue-TxTEG-RequestUL-TDOA-Config:

2>	if ue-TxTEG-RequestUL-TDOA-Config is set to setup:

3>	perform the UE positioning assistance information procedure as specified in 5.7.14;

2>	else:

3>	release the configuration of UE positioning assistance information;

1>	if the RRCReconfiguration message includes the aerial-Config:

2>	(re)configure the aerial parameters in accordance with the included aerial-Config;

1>	if the RRCReconfiguration message includes the sl-IndirectPathAddChange:

2>	perform the SL indirect path specific configuration procedure as specified in 5.3.5.17.2.2;

1>	if the RRCReconfiguration message includes the n3c-IndirectPathAddChange or n3c-ExtIndirectPathAddChange:

2>	perform configuration procedure for the remote UE part of N3C indirect path as specified in 5.3.5.17.3.2;

1>	if the RRCReconfiguration message includes the n3c-IndirectPathConfigRelay:

2>	perform the configuration procedure for the relay UE part of N3C indirect path as specified in 5.3.5.17.3.3;

1>	if the RRCReconfiguration message includes the ltm-Config:

2>	if the ltm-Config is set to setup:

3>	perform the LTM configuration procedure as specified in 5.3.5.18.1;

2>	else:

3>	perform the LTM configuration release procedure as specified in clause 5.3.5.18.7;

1>	if the RRCReconfiguration message includes the ltm-ConfigNRDC:

2>	if the ltm-ConfigNRDC is set to setup:

3>	if ltm-ConfigNRDC includes ltm-ConfigurationSCG:

4>	perform the LTM configuration procedure as specified in clause 5.3.5.18.1;

3>	if ltm-ConfigNRDC includes ltm-SK-CounterConfigToReleaseList:

4>	perform the LTM sk-Counter configuration release as specified in clause 5.3.5.18.10;

3>	if ltm-ConfigNRDC includes ltm-SK-CounterConfigToAddModList:

4>	perform the LTM sk-Counter configuration addition/modification as specified in clause 5.3.5.18.9;

2>	else:

3>	perform the LTM configuration release procedure as specified in clause 5.3.5.18.7;

1>	if the RRCReconfiguration message includes the srs-PosResourceSetAggBW-CombinationList:

2>	if srs-PosResourceSetAggBW-CombinationList is set to setup:

3>	perform the SRS for positioning transmission using bandwidth aggregation provided in configuration SRS-PosResourceSetLinkedForAggBW as specified in TS 38.211 [16];

2>	else:

3>	release all the configuration of SRS-PosResourceSetLinkedForAggBW;

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

NOTE 0b:	The UE does not expect that the reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier is received in both masterCellGroup and in secondaryCellGroup. Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

3>	if the RRCReconfiguration message is applied due to conditional reconfiguration execution and the RRCReconfiguration message does not include the reconfigurationWithSync in the masterCellGroup:

4>	include in the selectedCondRRCReconfig the condReconfigId for the selected cell of conditional reconfiguration execution;

4>	if a new sk-Counter value has been selected due to the conditional reconfiguration execution for subsequent CPAC:

5>	include selectedSK-Counter and set its value to the selected sk-Counter value;

3>	if the RRCReconfiguration message is applied due to conditional reconfiguration execution and condExecutionCondPSCell is configured for the selected PSCell:

4>	include in the selectedPSCellForCHO-WithSCG and set it to the information of the selected PSCell;

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

3>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in the VarLogMeasReport:

4>	include the logMeasAvailable in the RRCReconfigurationComplete message;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

4>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

3>	if the sigLoggedMeasType in VarLogMeasReport is included; or

3>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

4>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

5>	set sigLogMeasConfigAvailable to true in the RRCReconfigurationComplete message;

4>	else:

5>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

6>	set sigLogMeasConfigAvailable to false in the RRCReconfigurationComplete message;

3>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

3>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

4>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

4>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE was configured with successHO-Config when connected to the source PCell:

4>	if the applied RRCReconfiguration is not due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3, and the applied RRCReconfiguration is not due to an LTM cell switch execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3; or

4>	if the applied RRCReconfiguration is not received when T316 was running:

5>	perform the actions for the successful handover report determination as specified in clause 5.7.10.6, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the MCG, or upon an indication from lower layer that the LTM cell switch execution has been successfully completed;

4>	if applied RRCReconfiguration is received when T316 was running:

5>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell;

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

3>	release successPSCell-Config configured by the source PCell, if available;

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

4>	include successPSCell-InfoAvailable in the RRCReconfigurationComplete message;

3>	if the RRCReconfiguration includes retainLoggedMeasurements:

4>	if the UE has logged measurement entries available in VarCSI-LogMeasReport:

5>	include csi-LogMeasAvailable in the RRCReconfigurationComplete message;

3>	else:

4>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

2>	if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume:

3>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapsConfigNR; or

4>	if the NeedForGapsInfoNR information is changed compared to last time the UE reported this information; or

4>	if the RRCReconfiguration message includes the needForInterruptionConfigNR and set it to enabled; or

4>	if the needForInterruptionConfigNR is enabled and the NeedForInterruptionInfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapsInfoNR and set the contents as follows:

6>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNR is configured:

7>	for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band;

6>	else:

7>	include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

5>	if the needForInterruptionConfigNR is enabled:

6>	include the needForInterruptionInfoNR and set the contents as follows:

7>	include intraFreq-needForInterruption with the same number of entries, and listed in the same order, as in intraFreq-needForGap;

7> for each entry in intraFreq-needForInterruption:

8>	include interruptionIndication and set the interruption requirement information if the corresponding entry in intraFreq-needForGap is set to no-gap;

7>	include interFreq-needForInterruption with the same number of entries, and listed in the same order, as in interFreq-needForGap;

7> for each entry in interFreq-needForInterruption:

8>	include interruptionIndication and set the interruption requirement information if the corresponding entry in interFreq-needForGap is set to no-gap;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR; or

4>	if the needForGapNCSG-InfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

6>	include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNCSG-NR is configured:

7>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

6>	else:

7>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA; or

4>	if the needForGapNCSG-InfoEUTRA information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

6>	if requestedTargetBandFilterNCSG-EUTRA is configured, for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band; otherwise, include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

2>	if the UE has (updated) flight path information available:

3>	if the UE had not provided a flight path information since last entering RRC_CONNECTED state; or

3>	if at least one waypoint or a timestamp corresponding to a waypoint location that was not previously provided since last entering RRC_CONNECTED state is available; or

3>	if at least one upcoming waypoint or a timestamp corresponding to a waypoint location that was previously provided since last entering RRC_CONNECTED state is to be removed; or

3>	if flightPathUpdateDistanceThr is configured and, for at least one waypoint, the 3D distance between the previously provided location and the new location is more than the distance threshold configured by flightPathUpdateDistanceThr; or

3> if flightPathUpdateTimeThr is configured and, for at least one waypoint, the time difference between the previously provided timestamp and the new timestamp, if available, is more than the time threshold configured by flightPathUpdateTimeThr:

4>	include flightPathInfoAvailable;

NOTE 0c:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to include flightPathInfoAvailable when updated flight path information is available.

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured which has not been successfully transmitted since entering RRC_CONNECTED state:

3>	include measConfigReportAppLayerAvailable;

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

3>	if this RRCReconfiguration message was received via SRB1 but not within the nr-SCG within mrdc-SecondaryCellGroup:

4>	if a new sk-Counter value has been selected due to the LTM cell switch execution procedure as specified in 5.3.5.18.6:

5>	include selectedSK-Counter and set its value to the selected sk-Counter value;

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or

2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or

2>	if the RRCReconfiguration message was received via E-UTRA RRC message RRCConnectionReconfiguration within MobilityFromNRCommand (handover from NR standalone to (NG)EN-DC);

3>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SecondaryCellGroupConfig specified in TS 36.331 [10]:

4>	submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a.

3>	else if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

4>	submit the RRCReconfigurationComplete message via E-UTRA embedded in E-UTRA RRC message RRCConnectionResumeComplete as specified in TS 36.331 [10], clause 5.3.3.4a;

3>	else:

4>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4/5.4.2.3;

3>	if the scg-State is not included in the E-UTRA message (RRCConnectionReconfiguration or RRCConnectionResume) containing the RRCReconfiguration message:

4>	perform SCG activation as specified in 5.3.5.13a;

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else if the SCG was deactivated before the reception of the E-UTRA RRC message containing the RRCReconfiguration message:

5>	if bfd-and-RLM was not configured to true before the reception of the E-UTRA RRCConnectionReconfiguration or RRCConnectionResume message containing the RRCReconfiguration message or if lower layers indicate that a Random Access procedure is needed for SCG activation:

6>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

5>	else the procedure ends;

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

2>	if the RRCReconfiguration message was received within nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration message received via SRB3 within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4;

3>	if the scg-State is not included in the RRCConnectionReconfiguration:

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC or subsequent CPAC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup; or

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution and is configured via an LTM-Config IE contained in nr-SCG within mrdc-SecondaryCellGroup:

3>	submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

2>	if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message:

3>	perform SCG activation as specified in 5.3.5.13a;

3>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

4>	if the RRCReconfiguration message is not applied due to an LTM cell switch execution for which lower layer indicate to skip the Random Access procedure:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	if the UE was configured with successPSCell-Config when connected to the source PSCell (for PSCell change) or to the PCell (for PSCell addition or change):

5>	perform the actions for the successful PSCell change or addition report determination as specified in clause 5.7.10.7, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

3>	else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message:

4>	if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or

4>	if lower layers indicate that a Random Access procedure is needed for SCG activation:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else the procedure ends;

2>	else

3>	perform SCG deactivation as specified in 5.3.5.13b;

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if the scg-State is not included in the RRCReconfiguration message containing the RRCReconfiguration message:

5>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

6>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

6>	if the UE was configured with successPSCell-Config when connected to the source PSCell (for PSCell change) or to the PCell (for PSCell addition or change):

7>	perform the actions for the successful PSCell change report determination as specified in clause 5.7.10.7, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

5>	else:

6>	the procedure ends;

4>	else:

5>	perform SCG deactivation as specified in 5.3.5.13b;

5>	the procedure ends;

3>	else:

4>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

5>	if the RRCReconfiguration includes the scg-State:

6>	perform SCG deactivation as specified in 5.3.5.13b;

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig for the SCG; and

3>	if the UE was configured with successPSCell-Config when connected to the source PSCell (for PSCell change):

4>	perform the actions for the successful PSCell change report determination as specified in clause 5.7.10.7, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

4>	include successPSCell-InfoAvailable in the RRCReconfigurationComplete message;

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	if the UE is in NR-DC and;

2>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

3>	if the RRCReconfiguration includes the scg-State:

4>	perform SCG deactivation as specified in 5.3.5.13b;

3>	else:

4>	perform SCG activation without SN message as specified in 5.3.5.13b1;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

4>	indicate TA report initiation to lower layers;

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2, SRB4, SRB6, DRBs, multicast MRB, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE in case of single hop or for L2 Last U2N Relay UE, that are suspended;

1>	if sl-IndirectPathAddChange was included in RRCReconfiguration message:

2>	if SRB1 is configured as split SRB and pdcp-Duplication is configured:

3>	when successfully sending RRCReconfigurationComplete message via SL indirect path (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE):

4>	stop timer T421;

2> else (i.e. split SRB1 with duplication is not configured):

3> when receiving RRCReconfigurationCompleteSidelink message from target L2 U2N Relay UE:

4>	stop timer T421;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or,

1>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2>	stop timer T304 for that cell group if running;

2>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

2>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

3>	release dedicated preambles provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

2>	if sl-PathSwitchConfig was included in reconfigurationWithSync:

3>	if the sl-IndirectPathMaintain is not included in reconfigurationWithSync:

4>	stop timer T420;

4>	release all radio resources, including release of the RLC entities and the MAC configuration at the source side;

4>	reset MAC used in the source cell;

3>	else (sl-IndirectPathMaintain is included):

4>	release radio resources on the direct path, including release of the RLC entities and the MAC configuration;

4>	reset MAC used in the source cell;

2>	if rach-LessHO was included in reconfigurationWithSync and cg-RRC-Configuration was configured:

3>	release the uplink grant configured for RACH-less handover;

NOTE 2b:	PDCP and SDAP configured by the source prior to the path switch that are reconfigured and re-used by target when delta signalling is used, are not released as part of this procedure.

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the RRCReconfiguration message is contained in an entry in MCG VarConditionalReconfig that includes the subsequentCondReconfig:

3>	for each entry in the condReconfigList within the MCG VarConditionalReconfig:

4>	if there is an entry in condExecutionCondToAddModList within the subsequentCondReconfig that has subsequentCondReconfigId matching the condReconfigId in the entry of the condReconfigList:

5>	if subsequentCondExecutionCondSCG is included in the entry of the condExecutionCondToAddModList:

6>	store in the condExecutionCondSCG in the entry of the condReconfigList the value of subsequentCondExecutionCondSCG in the entry of the condExecutionCondToAddModList;

2>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the RRCReconfiguration message is contained in an entry in SCG VarConditionalReconfig that includes the subsequentCondReconfig:

3>	for each entry in the condReconfigList within the SCG VarConditionalReconfig:

4>	if there is an entry in condExecutionCondToAddModList within the subsequentCondReconfig that has subsequentCondReconfigId matching the condReconfigId in the entry of the condReconfigList:

5>	if subsequentCondExecutionCond is included in the entry of the condExecutionCondToAddModList:

6>	store in the condExecutionCond in the entry of the condReconfigList the value of subsequentCondExecutionCond in the entry of the condExecutionCondToAddModList;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA, CPC, or subsequent CPAC was configured:

3>	remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

3>	remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

3>	for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig; and

4>	if the measObjectId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

4>	start or restart the leave without response timer (if exists) with the timer value set to the value in the musim-LeaveAssistanceConfig or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

3>	if SIB12 is provided by the target PCell, and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication/discovery related parameters relevant in target PCell during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication/discovery and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

3>	if any application layer measurement report container has been received from upper layers for which the successful transmission of the MeasurementReportAppLayer message or at least one segment of the message via SRB4 (if reconfigurationWithSync was included in masterCellGroup) or SRB5 (if reconfigurationWithSync was included in secondaryCellGroup) has not been confirmed by lower layers:

4>	if RRC segmentation was used for the MeasurementReportAppLayer message:

5>	if RRC segmentation is enabled based on the field rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 for the reportingSRB (or SRB4 if reportingSRB is not configured):

6>	re-submit all segments of the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

5>	else:

6>	discard all segments of the MeasurementReportAppLayer message;

4>	else:

5>	re-submit the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

2>	if reconfigurationWithSync was included in masterCellGroup and SRB4 is configured in the target cell:

3>	for each application layer measurement configuration in the UE:

4>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, if transmissionOfSessionStartStop is set to true for the application layer measurement configuration and if the session status has changed since the UE was configured with the conditional reconfiguration:

5>	initiate transmission of a MeasurementReportAppLayer message including appLayerSessionStatus, via SRB4 for the application layer measurement in accordance with 5.7.16.2;

2>	if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21 or provides SIB1 including nonServingCellMII:

3>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message:

4>	initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA (i.e. (NG)EN-DC case) or NR (i.e. NR-DC case):

2>	reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2>	for each BH RLC channel that is part of the SCG configuration:

3>	perform BH RLC channel release procedure as specified in 5.3.5.5.10;

2>	release the SCG configuration;

2>	for each application layer measurement configuration that is part of the SCG configuration:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig, if stored;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	discard any application layer measurement reports which were configured to be reported via SRB5 and which were not yet fully submitted to lower layers for transmission;

2>	remove all the entries within the SCG VarConditionalReconfig, if any;

2>	if SCG release was triggered by NR (i.e. NR-DC case):

3>	remove all the entries in the condReconfigList within the MCG VarConditionalReconfig for which subsequentCondReconfig is not present and the RRCReconfiguration within condRRCReconfig does not include the masterCellGroup with reconfigurationWithSync, if any;

3>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

3>	for each entry in the condReconfigList within the MCG VarConditionalReconfig for which subsequentCondReconfig is present:

4>	remove the condExecutionCondSCG, if stored;

2>	else (i.e. EN-DC case):

3>	perform VarConditionalReconfiguration CPC removal as specified in TS 36.331 [10] clause 5.3.5.9.7;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T312 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running.

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released.

#### 5.3.5.5	Cell Group configuration

##### 5.3.5.5.1	General

The network configures the UE with Master Cell Group (MCG), and zero or one Secondary Cell Group (SCG). In (NG)EN-DC, the MCG is configured as specified in TS 36.331 [10], and for NE-DC, the SCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2>	perform Reconfiguration with sync according to 5.3.5.5.2;

2>	resume all suspended radio bearers except the SRBs for the source cell group, and resume SCG transmission for all radio bearers, and resume BH RLC channels and resume SCG transmission for BH RLC channels for IAB-MT, if suspended;

NOTE 1:	If the SCG is deactivated, resuming SCG transmission for all radio bearers does not imply that PDCP PDUs can be transmitted or received on SCG RLC bearers.

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList or rlc-BearerToReleaseListExt:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2>	perform SCell addition/modification as specified in 5.3.5.5.9;

1>	if the CellGroupConfig contains the bh-RLC-ChannelToReleaseList:

2>	perform BH RLC channel release as specified in 5.3.5.5.10;

1>	if the CellGroupConfig contains the bh-RLC-ChannelToAddModList:

2>	perform the BH RLC channel addition/modification as specified in 5.3.5.5.11;

1>	if the CellGroupConfig contains the uu-RelayRLC-ChannelToReleaseList:

2>	perform Uu Relay RLC channel release as specified in 5.3.5.5.12;

1>	if the CellGroupConfig contains the uu-RelayRLC-ChannelToAddModList:

2>	perform the Uu Relay RLC channel addition/modification as specified in 5.3.5.5.13;

1>	if the CellGroupConfig contains the ncr-FwdConfig:

2>	perform the NCR-Fwd configuration as specified in 5.3.5.5.14;

1>	if the CellGroupConfig contains the autonomousDenialParameters:

2>	consider itself to be allowed to deny any transmission in a particular UL slot if during the number of slots indicated by autonomousDenialValidity, preceding and including this particular slot, it autonomously denied fewer UL slots than indicated by autonomousDenialSlots within the same cell group;

NOTE 2:	When counting the number of denied UL slots, the UE sums up the denied UL slots across all serving cells within the same cell group. When counting the number of slots indicated by autonomousDenialValidity, the UE sums up the UL slots across all serving cells within the same cell group.

NOTE 3:	When multiple denied UL slots across all serving cells partially or fully overlap in the time domain, the number of denied UL slots across all serving cells is counted as one denied UL slot, based on the longest slot.

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T430 if running;

1>	if no DAPS bearer is configured:

2>	stop timer T310 for the corresponding SpCell, if running;

1>	if this procedure is executed for the MCG:

2>	if timer T316 is running;

3>	stop timer T316;

3>	if the UE supports RLF-Report for fast MCG recovery procedure as specified in TS 38.306 [26]:

4>	set the elapsedTimeT316 in the VarRLF-Report to the value of the elapsed time of the timer T316;

4>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

3>	else:

4>	clear the information included in VarRLF-Report, if any;

2>	resume MCG transmission, if suspended.

1>	stop timer T312 for the corresponding SpCell, if running;

1>	if sl-PathSwitchConfig is included:

2>	apply the value of the newUE-Identity as the C-RNTI;

2>	if sl-IndirectPathMaintain is not included in reconfigurationWithSync:

3>	if the UE is L2 U2N remote UE at source side:

4>	indicate to upper layer to trigger PC5 unicast link release with the source L2 U2N Relay UE;

3>	consider the target L2 U2N Relay UE to be the one indicated by the targetRelayUE-Identity in the sl-PathSwitchConfig;

3>	start timer T420 for the corresponding target L2 U2N Relay UE with the timer value set to t420, as included in the sl-PathSwitchConfig;

3>	indicate to upper layer (to trigger the PC5 unicast link establishment) with the target L2 U2N Relay UE indicated by the targetRelayUE-Identity;

3>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	else:

3>	consider the connected L2 U2N Relay UE on the indirect path as the target L2 U2N relay UE, and maintain the PC5 connection with the L2 U2N Relay UE;

1>	else (sl-PathSwitchConfig is not included):

2>	if this procedure is executed for the MCG or if this procedure is executed for an SCG not indicated as deactivated in the E-UTRA or NR RRC message in which the RRCReconfiguration message is embedded:

3>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

2>	if the frequencyInfoDL is included:

3>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

2>	else:

3>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

2>	if this procedure is performed due to an LTM cell switch execution:

3>	if UE is performing LTM cell switch conditions evaluation based on L1 measurements:

4>	request lower layers to stop the LTM conditions evaluation based on L1 measurements for all the LTM candidate configurations;

3>	if UE is performing LTM cell switch conditions evaluation based on L3 measurements:

4>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations;

3>	start synchronising to the DL of the indicated LTM candidate cell, if no DL synchronization for the indicated LTM candidate cell has been already acquired;

2>	else:

3>	if the target SpCell is different from current SpCell:

4>	stop the LTM conditions evaluation, if any, for all the LTM candidate configurations;

4>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

5>	request lower layers to stop the LTM cell switch conditions evaluation for all LTM candidate configurations;

3>	start synchronising to the DL of the target SpCell;

2>	apply the specified BCCH configuration defined in 9.1.1.1 for the target SpCell;

2>	acquire the MIB of the target SpCell, which is scheduled as specified in TS 38.213 [13];

2>	if NTN-Config is configured for the target cell:

3>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime, according to the target cell NTN-Config;

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access, or if not needed for RACH-less initial UL transmission.

NOTE 2a:	A UE with DAPS bearer does not monitor for system information updates in the source PCell.

2>	If any DAPS bearer is configured:

3>	create a MAC entity for the target cell group with the same configuration as the MAC entity for the source cell group;

3>	for each DAPS bearer:

4>	establish an RLC entity or entities for the target cell group, with the same configurations as for the source cell group;

4>	establish the logical channel for the target cell group, with the same configurations as for the source cell group;

NOTE 2b:	In order to understand if a DAPS bearer is configured, the UE needs to check the presence of the field daps-Config within the RadioBearerConfig IE received in radioBearerConfig or radioBearerConfig2.

3>	for each SRB:

4>	establish an RLC entity for the target cell group, with the same configurations as for the source cell group;

4>	establish the logical channel for the target cell group, with the same configurations as for the source cell group;

3>	suspend SRBs for the source cell group;

NOTE 3:	Void

3>	apply the value of the newUE-Identity as the C-RNTI in the target cell group;

3>	configure lower layers for the target SpCell in accordance with the received spCellConfigCommon;

3>	configure lower layers for the target SpCell in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

2>	else:

3>	reset the MAC entity of this cell group;

3>	consider the SCell(s) of this cell group, if configured, that are not included in the SCellToAddModList in the RRCReconfiguration message, to be in deactivated state;

3>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

3>	configure lower layers in accordance with the received spCellConfigCommon;

3>	if rach-LessHO is included:

4>	configure lower layers in accordance with rach-LessHO for the target SpCell;

3>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

2>	if the UE is acting as L2 U2N Remote UE at the source side:

3>	if the sl-IndirectPathMaintain is not included in reconfigurationWithSync:

4>	indicate upper layer to trigger PC5 unicast link release.

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the child UE(s) in accordance with 5.8.9.10.

NOTE 4:	The MP direct path release is realized by direct-to-indirect path switch procedure (i.e. sl-PathSwitchConfig and sl-indirectPathMaintain included in RRCReconfiguration message), where MP is configured in source side.

##### 5.3.5.5.3	RLC bearer release

The UE shall:

1>	for each logicalChannelIdentity/LogicalChannelIdentityExt value included in the rlc-BearerToReleaseList/rlc-BearerToReleaseListExt that is part of the current UE configuration within the same cell group (LCH release); or

1>	for each logicalChannelIdentity value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.4	RLC bearer addition/modification

For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity/LogicalChannelIdentityExt within the same cell group:

2>	if the RLC bearer is associated with an DAPS bearer, or

2>	if any DAPS bearer is configured and the RLC bearer is associated with an SRB:

3>	reconfigure the RLC entity or entities for the target cell group in accordance with the received rlc-Config;

3>	reconfigure the logical channel for the target cell group in accordance with the received mac-LogicalChannelConfig;

2>	else:

3>	if reestablishRLC is received:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

3>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

3>	if servedMBS-RadioBearer is received:

4>	associate this logical channel with the PDCP entity identified by servedMBS-RadioBearer;

NOTE 1:	For DRB and SRB, the network does not re-associate an already configured logical channel with another radio bearer. Hence, servedRadioBearer is not present in this case. For MRB, the network does not re-associate an already configured logical channel with DRB or SRB or another MRB (i.e. MRB with another PDCP entity). Hence multicastRLC-BearerConfig is not present in this case. If a radio bearer is released and another is added with the same radio bearer identity, it is considered as a new (different) radio bearer. Hence, the network also releases the RLC bearer(s) associated with the released radio bearer.

NOTE 2:	In DAPS handover, the UE may perform RLC entity re-establishment (if reestablishRLC is set) for an RLC bearer associated with a non-DAPS bearer when indication of successful completion of random access towards target cell is received from lower layers as specified in TS 38.321 [3].

1>	else (a logical channel with the given logicalChannelIdentity/LogicalChannelIdentityExt is not configured within the same cell group, including the case when full configuration option is used):

2>	if the servedRadioBearer associates the logical channel with an SRB and rlc-Config is not included:

3>	establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	establish an RLC entity in accordance with the received rlc-Config;

2>	if the servedRadioBearer associates the logical channel with an SRB and if mac-LogicalChannelConfig is not included:

3>	configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig;

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer or servedMBS-RadioBearer.

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

1>	if any DAPS bearer is configured:

2>	reconfigure the MAC main configuration for the target cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	else:

2>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer.

##### 5.3.5.5.6	RLF Timers & Constants configuration

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

2>	if any DAPS bearer is configured:

3>	use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1;

2>	else:

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	else:

2>	if any DAPS bearer is configured:

3>	configure the value of timers and constants for the target cell group in accordance with received rlf-TimersAndConstants;

2>	else:

3>	(re-)configure the value of timers and constants in accordance with received rlf-TimersAndConstants;

3>	stop timer T310 for this cell group, if running;

3>	stop timer T312 for this cell group, if running;

3>	reset the counters N310 and N311.

##### 5.3.5.5.7	SpCell Configuration

The UE shall:

1>	if the UE is acting as L2 U2N Remote UE and is not configured with MP:

2>	if the SpCellConfig contains the rlf-TimersAndConstants which is set to setup:

3>	use value for timers T311 as received in rlf-TimersAndConstants;

2>	else if rlf-TimersAndConstants is not configured for this cell group or SpCellConfig contains the rlf-TimersAndConstants which is set to release:

3>	use value for timers T311, as included in ue-TimersAndConstants received in SIB1;

1>	else

2>	if the SpCellConfig contains the rlf-TimersAndConstants:

3>	configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6;

2>	else if rlf-TimersAndConstants is not configured for this cell group:

3>	if any DAPS bearer is configured:

4>	use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1;

3>	else

4>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

2>	if the SpCellConfig contains spCellConfigDedicated:

3>	configure the SpCell in accordance with the spCellConfigDedicated;

3>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id, if included in the spCellConfigDedicated, to be the active uplink bandwidth part;

3>	if the firstActiveDownlinkBWP-Id is included in the spCellConfigDedicated:

4>	if the SpCellConfig is included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated:

5>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the bandwidth part for Radio Link Monitoring, Beam Failure Detection and measurements;

4>	else:

5>	consider the bandwith part indicated in firstActiveDownlinkBWP-Id to be the active downlink bandwidth part;

3>	if csi-LoggedMeasurementConfigToAddModList is included within csi-MeasConfig in spCellConfigDedicated:

4>	perform logging of measurements for network-side data collection as specified in 5.5a;

3>	if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

4>	stop timer T310 for the corresponding SpCell, if running;

4>	stop timer T312 for the corresponding SpCell, if running;

4>	reset the counters N310 and N311.

1>	if the SpCellConfig contains the lowMobilityEvaluationConnected:

2>	the UE may perform the evaluation of the low mobility criterion for this cell group as specified in 5.7.13.1;

1>	if the SpCellConfig contains the goodServingCellEvaluationRLM:

2>	the UE may perform the evaluation of the good serving cell quality criterion for this SpCell as specified in 5.7.13.2;

1>	if the SpCellConfig contains the goodServingCellEvaluationBFD:

2>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2;

##### 5.3.5.5.8	SCell Release

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell.

##### 5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	if the sCellState is included:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

2>	if the SCellConfig contains the goodServingCellEvaluationBFD:

3>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2;

2>	if csi-LoggedMeasurementConfigToAddModList is included within csi-MeasConfig in sCellConfigDedicated:

3>	perform logging of measurements for network-side data collection as specified in 5.5a.

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated;

2>	if the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync, or received in an RRCResume message, or received in an RRCReconfiguration message including reconfigurationWithSync embedded in an RRCResume message or embedded in an RRCReconfiguration message or embedded in an E-UTRA RRCConnectionReconfiguration message or embedded in an E-UTRA RRCConnectionResume message, or received in an RRCReconfiguration message embedded in an RRCReconfiguration message or embedded in an E-UTRA RRCConnectionReconfiguration message activating deactivated SCG:

3>	if the sCellState is included:

4>	configure lower layers to consider the SCell to be in activated state;

3>	else:

4>	configure lower layers to consider the SCell to be in deactivated state.

2>	if the SCellConfig contains the goodServingCellEvaluationBFD:

3>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2;

2>	if csi-LoggedMeasurementConfigToAddModList is included within csi-MeasConfig in sCellConfigDedicated:

3>	perform logging of measurements for network-side data collection as specified in 5.5a.

##### 5.3.5.5.10	BH RLC channel release

The IAB-node shall:

1>	for each BH-RLC-ChannelID value included in the bh-RLC-ChannelToReleaseList that is part of the current IAB-node configuration within the same cell group (LCH release); or

1>	for each BH-RLC-ChannelID value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.11	BH RLC channel addition/modification

For each BH-RLC-ChannelConfig received in the bh-RLC-ChannelToAddModList IE the IAB-node shall:

1>	if the current configuration contains a BH RLC Channel with the received bh-RLC-ChannelID within the same cell group:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

1>	else (a backhaul logical channel with the given BH-RLC-ChannelID was not configured before within the same cell group):

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig.

##### 5.3.5.5.12	Uu Relay RLC channel release

The L2 U2N Relay UE or L2 Last U2N Relay UE or N3C relay UE shall:

1>	for each Uu-RelayRLC-ChannelID value included in the uu-RelayRLC-ChannelToReleaseList that is part of the current configuration within the same cell group (LCH release):

2>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.13	Uu Relay RLC channel addition/modification

For each Uu-RelayRLC-ChannelConfig received in the uu-RelayRLC-ChannelToAddModList the L2 U2N Relay UE or L2 Last U2N Relay UE or N3C relay UE shall:

1>	if the current configuration contains a Uu Relay RLC channel with the same uu-RelayRLC-ChannelID within the same cell group:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

1>	else (a logical channel with the given uu-RelayRLC-ChannelID was not configured before within the same cell group):

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig.

##### 5.3.5.5.14	NCR-Fwd configuration

The NCR-MT shall:

1>	if ncr-FwdConfig is set to setup:

2>	if ncr-FwdConfig includes periodic forwarding resource configuration:

3>	indicate to NCR-Fwd to forward in accordance with the configured periodic forwarding resource set(s);

1>	else (ncr-FwdConfig is set to release):

2>	release ncr-FwdConfig;

2>	indicate to NCR-Fwd to cease forwarding.

#### 5.3.5.6	Radio Bearer configuration

##### 5.3.5.6.1	General

The UE shall perform the following actions based on a received RadioBearerConfig IE:

1>	if the RadioBearerConfig includes the srb3-ToRelease, srb4-ToRelease, srb5-ToRelease or srb6-ToRelease:

2>	perform the SRB release as specified in 5.3.5.6.2;

1>	if the RadioBearerConfig includes the srb-ToAddModList, srb4-ToAddMod, srb5-ToAddMod or srb6-ToAddMod or if any DAPS bearer is configured:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5;

1>	if the RadioBearerConfig includes the mrb-ToReleaseList:

2>	perform multicast MRB release as specified in 5.3.5.6.6;

1>	if the RadioBearerConfig includes the mrb-ToAddModList:

2>	perform multicast MRB addition or reconfiguration as specified in 5.3.5.6.7;

1>	release all SDAP entities established for the PDU sessions, if any, that have no associated DRB as specified in TS 37.324 [24] clause 5.1.2, and indicate the release of the user plane resources for PDU Sessions associated with the released SDAP entities to upper layers;

1>	release all SDAP entities established for the MBS multicast sessions, if any, that have no associated multicast MRB as specified in TS 37.324 [24] clause 5.1.2, and indicate the release of user plane resources for these MBS multicast sessions to upper layers.

##### 5.3.5.6.2	SRB release

The UE shall:

1>	if srb3-ToRelease is included:

2>	release the PDCP entity and the srb-Identity of the SRB3;

1>	if srb4-ToRelease is included

2>	release the PDCP entity and the srb-Identity of the SRB4;

1>	if srb5-ToRelease is included:

2>	release the PDCP entity and the srb-Identity of the SRB5;

1>	if srb6-ToRelease is included:

2>	release the PDCP entity and the srb-Identity of the SRB6.

##### 5.3.5.6.3	SRB addition/modification

The UE shall:

1>	If any DAPS bearer is configured, for each SRB:

2>	establish a PDCP entity for the target cell group as specified in TS 38.323 [5], with the same configuration as the PDCP entity for the source cell group;

2>	if the masterKeyUpdate is received:

3>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KgNB);

2>	else:

3>	configure the PDCP entity for the target cell group with state variables continuation as specified in TS 38.323 [5], and with the same security configuration as the PDCP entity for the source cell group;

1>	for each srb-Identity value included in the srb-ToAddModList, srb4-ToAddMod, srb5-ToAddMod or srb6-ToAddMod that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity;

2>	if AS security has been activated:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC, but not capable of NGEN-DC:

5>	configure the PDCP entity with the security algorithms and keys (KRRCenc and KRRCint) configured/derived as specified in TS 36.331 [10];

4>	else (i.e., UE capable of NGEN-DC):

5>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC):

4>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 [10] includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

1>	if any DAPS bearer is configured, for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity for the target cell group in accordance with the received pdcp-Config;

1>	else, for each srb-Identity value included in the srb-ToAddModList, srb4-ToAddMod, srb5-ToAddMod or srb6-ToAddMod that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC, but not capable of NGEN-DC:

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key configured/derived as specified in TS 36.331 [10], i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	else (i.e., a UE capable of NGEN-DC):

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	else (i.e., UE connected to NR or UE in EN-DC):

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB), as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

2>	else, if the discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

##### 5.3.5.6.4	DRB release

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the DRB is associated with an eps-BearerIdentity:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10];

4>	else (i.e., a UE capable of NGEN-DC):

5>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the key (KUPenc) associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC):

4>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB/S-KeNB) as indicated in keyToUse;

2>	if the PDCP entity of this DRB is configured with integrityProtection:

3>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2>	if an sdap-Config is included:

3>	if an SDAP entity with the received pdu-Session does not exist:

4>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

4>	if an SDAP entity with the received pdu-Session did not exist prior to receiving this reconfiguration:

5>	indicate the establishment of the user plane resources for the pdu-Session to upper layers;

3>	configure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24] and associate the DRB with the SDAP entity;

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

2>	if the DRB is associated with an eps-BearerIdentity:

3>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

4>	associate the established DRB with the corresponding eps-BearerIdentity;

3>	else:

4>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

2>	if the n3c-BearerAssociated is included for a DRB:

3>	consider this radio bearer to be associated with the N3C indirect path;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and configured as DAPS bearer:

2>	reconfigure the PDCP entity to configure DAPS with the ciphering function, integrity protection function and ROHC function of the target cell group as specified in TS 38.323 [5] and configure it in accordance with the received pdcp-Config;

2>	if the masterKeyUpdate is received:

3>	if the ciphering function of the target cell group PDCP entity is not configured with cipheringDisabled:

4>	configure the ciphering function of the target cell group PDCP entity with the ciphering algorithm according to securityConfig and apply the KUPenc key associated with the master key (KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received from the target cell group and sent to the target cell group by the UE;

3>	if the integrity protection function of the target cell group PDCP entity is configured with integrityProtection:

4>	configure the integrity protection function of the target cell group PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) as indicated in keyToUse;

2>	else:

3>	configure the ciphering function and the integrity protection function of the target cell group PDCP entity with the same security configuration as the PDCP entity for the source cell group;

2>	if the sdap-Config is included and when indication of successful completion of random access towards target cell is received from lower layers as specified in [3]:

3>	reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24];

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	if drb-ContinueEHC-UL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-UL is configured;

3>	if drb-ContinueUDC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueUDC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

2>	if the sdap-Config is included:

3>	reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS37.324 [24];

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

2>	if the n3c-BearerAssociated is included for a DRB:

3>	consider this radio bearer to be associated with the N3C indirect path;

NOTE 1:	Void.

NOTE 2:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in. To re-associate a DRB with a different key (KeNB to S-KgNB, KgNB to S-KeNB, KgNB to S-KgNB, or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE 3:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 4:	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 5: Ciphering and integrity protection can be enabled or disabled for a DRB. The enabling/disabling of ciphering or integrity protection can be changed only by releasing and adding the DRB.

NOTE 6:	In DAPS handover, the UE may perform PDCP entity re-establishment (if reestablishPDCP is set) or the PDCP data recovery (if recoverPDCP is set) for a non-DAPS bearer when indication of successful completion of random access towards target cell is received from lower layers as specified in TS 38.321 [3]. In this case, the UE suspends data transmission and reception for all non-DAPS bearers in the source MCG for duration of the DAPS handover.

##### 5.3.5.6.6	Multicast MRB release

The UE shall:

1>	for each mrb-Identity value included in the mrb-ToReleaseList that is part of the current UE configuration; or

1>	for each mrb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the mrb-Identity;

NOTE 1:	The UE does not consider the message as erroneous if the mrb-ToReleaseList includes any mrb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.7	Multicast MRB addition/modification

The UE shall for each element in the order of entry in the list mrb-ToAddModList:

1>	if mrb-Identity value included in the mrb-ToAddModList is part of the UE configuration:

2>	if mrb-Identity value included in the mrb-ToAddModList for which mrb-IdentityNew is included (i.e., multicast MRB ID change):

3>	update the mrb-Identity to the value mrb-IdentityNew;

2>	if the reestablishPDCP is set:

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

1>	else if mrb-Identity value included in the mrb-ToAddModList is not part of the UE configuration (i.e., multicast MRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	associate the established multicast MRB with the corresponding mbs-SessionId;

2>	if an SDAP entity with the received mbs-SessionId does not exist:

3>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

3>	if an SDAP entity with the received mbs-SessionId did not exist prior to receiving this reconfiguration:

4>	indicate the establishment of the user plane resources for the mbs-SessionId to upper layers.

NOTE 1:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. The network does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 2:	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 3:	When updating the mrb-Identity, the network ensures new MRBs are listed at the end of the mrb-ToAddModList if they have the same MRB ID as in the existing UE configuration.

#### 5.3.5.7	AS Security key update

The UE shall:

1>	if UE is connected to E-UTRA/EPC or E-UTRA/5GC:

2>	upon reception of sk-Counter as specified in TS 36.331 [10]:

3>	update the S-KgNB key based on the KeNB key and using the received sk-Counter value, as specified in TS 33.401 [30] for EN-DC, or TS 33.501 [11] for NGEN-DC;

3>	derive the KRRCenc and KUPenc keys as specified in TS 33.401 [30] for EN-DC, or TS 33.501 [11] for NGEN-DC;

3>	derive the KRRCint and KUPint keys as specified in TS 33.401 [30] for EN-DC or TS 33.501 [11] for NGEN-DC.

1>	else if this procedure was initiated due to reception of the masterKeyUpdate:

2>	if the nas-Container is included in the received masterKeyUpdate:

3>	forward the nas-Container to the upper layers;

2>	if the keySetChangeIndicator is set to true:

3>	derive or update the KgNB key based on the KAMF key, as specified in TS 33.501 [11];

2>	else:

3>	derive or update the KgNB key based on the current KgNB key or the NH, using the nextHopChainingCount value indicated in the received masterKeyUpdate, as specified in TS 33.501 [11];

2>	store the nextHopChainingCount value;

2>	derive the keys associated with the KgNB key as follows:

3>	if the securityAlgorithmConfig is included in SecurityConfig:

4>	derive the KRRCenc and KUPenc keys associated with the cipheringAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

4>	derive the KRRCint and KUPint keys associated with the integrityProtAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

3>	else:

4>	derive the KRRCenc and KUPenc keys associated with the current cipheringAlgorithm, as specified in TS 33.501 [11];

4>	derive the KRRCint and KUPint keys associated with the current integrityProtAlgorithm, as specified in TS 33.501 [11].

NOTE 1:	Ciphering and integrity protection are optional to configure for the DRBs.

1>	else if this procedure was initiated due to an LTM cell switch execution procedure which requires updating the master key, as specified in section 5.3.5.18.6:

2>	derive or update the KgNB key based on the current KgNB key or the NH, using the NCC value indicated by lower layer, as specified in TS 33.501 [11];

2>	replace the nextHopChainingCount with the NCC value indicated by lower layers;

2>	derive the keys associated with the KgNB key as follows:

3>	derive the KRRCenc and KUpenc keys associated with the current cipheringAlgorithm, as specified in TS 33.501 [11];

3>	derive the KRRCint and KUpint keys associated with the current integrityProtAlgorithm, as specified in TS 33.501 [11].

1>	else if this procedure was initiated due to reception of the sk-Counter (UE is in NE-DC, or NR-DC, or is configured with SN terminated bearer(s)) or if the procedure was initiated due to selection of an sk-Counter as specified in clause 5.3.5.13.8 or clause 5.3.5.18.6 (UE is in NR-DC):

2>	derive or update the secondary key (S-KgNB or S-KeNB) based on the KgNB key and using the received or selected sk-Counter value, as specified in TS 33.501 [11];

2>	derive the KRRCenc key and the KUPenc key as specified in TS 33.501 [11] using the ciphering algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse;

2>	derive the KRRCint key and the KUPint key as specified in TS 33.501 [11] using the integrity protection algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse;

NOTE 2:	If the UE has no radio bearer configured with keyToUse set to secondary and receives the sk-Counter or an sk-Counter is selected for subsequent CPAC or LTM without any RadioBearerConfig with keyToUse set to secondary, the UE does not consider it as an invalid reconfiguration.

#### 5.3.5.8	Reconfiguration failure

##### 5.3.5.8.1	Void

##### 5.3.5.8.2	Inability to comply with RRCReconfiguration

NOTE 00:	The UE behaviour specified in this clause does not apply to the following, and the UE ignores, i.e. does not take an action on and does not store, the fields that it does not support or does not comprehend:

-	The fields in ServingCellConfigCommon that are defined in Rel-16 and later.

-	The fields of searchSpaceMCCH and searchSpaceMTCH in PDCCH-ConfigCommon that are defined in Rel-17 and later.

The UE shall:

1>	if the UE is in (NG)EN-DC:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

3>	else:

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends;

2>	else, if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB1;

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends.

1>	else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC):

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

NOTE 0:	This case does not apply in NE-DC.

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration; or,

3>	if the RRCReconfiguration message was received as part of ltm-Config:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

3>	else:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7, upon which the connection reconfiguration procedure ends;

2>	else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1 or if the upper layers indicate that the nas-Container is invalid:

NOTE 0a:	The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc-SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations.

NOTE 0b:	The compliance also covers the V2X sidelink configuration carried within an octet string, e.g. field sl-ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded V2X sidelink configuration.

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration; or,

3>	if the RRCReconfiguration message was received as part of ltm-Config or ltm-ConfigNRDC:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'

3>	else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends;

1>	else if RRCReconfiguration is received via other RAT (Handover to NR failure):

2>	if the UE is unable to comply with any part of the configuration included in the RRCReconfiguration message or if the upper layers indicate that the nas-Container is invalid:

3>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure, except for the MUSIM case (i.e. the UE is configured to provide MUSIM assistance information for temporary capability restriction and is unable to apply (part of) the configuration resulting from RRCReconfiguration message due to UE temporary capability restriction for MUSIM operation). For the MUSIM case, the UE does not apply above failure handling, and it is up to UE implementation how to apply RRCReconfiguration message. If UE does not perform RRC reconfiguration failure in this case, UE will provide MUSIM assistance information for temporary capability restriction as specified in 5.7.4 and still considers the configuration resulting from RRCReconfiguration message as the current configuration and baseline for delta configuration for future reconfigurations.

NOTE 3:	It is up to UE implementation whether the compliance check for an RRCReconfiguration received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO, CPA, CPC, and subsequent CPAC execution (when the message is required to be applied).

NOTE 4:	It is up to UE implementation whether the compliance check for an RRCReconfiguration message received as part of an LTM-Config IE is performed upon the reception of the message or during an LTM cell switch procedure (when the message is required to be applied).

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1>	if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to clause 5.3.10.3:

3>	reset MAC for the target PCell and release the MAC configuration for the target PCell;

3>	for each DAPS bearer:

4>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

3>	for each SRB:

4>	if the masterKeyUpdate was not received:

5>	configure the PDCP entity for the source PCell with state variables continuation as specified in TS 38.323 [5];

4>	release the PDCP entity for the target PCell;

4>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	trigger the PDCP entity for the source PCell to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity for the source PCell;

3>	release the physical channel configuration for the target PCell;

3>	discard the keys used in target PCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

3>	resume suspended SRBs in the source PCell;

3>	for each non-DAPS bearer:

4>	revert back to the UE configuration used for the DRB or multicast MRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ;

3>	revert back to the UE measurement configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the failure information procedure as specified in clause 5.7.5 to report DAPS handover failure.

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers or fulfilment of LTM cell switch execution conditions, as described in 5.3.5.18.6:

4>	revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG;

3>	else:

4>	revert back to the UE configuration used in the source PCell;

3>	if the associated T304 was not initiated upon cell selection performed while timer T311 was running, as defined in clause 5.3.7.3:

4>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	if MCG transmission is not suspended:

3>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated, if configured;

3>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

2>	else:

3>	if the UE is in NR-DC:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

3>	else (the UE is in (NG) EN-DC):

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group, and its preference on cell DTX/DRX related parameters for PCell if cell DTX/DRX is configured, in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group or its preference on cell DTX/DRX related parameters for PCell, and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxBW-PreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxMIMO-LayerPreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes minSchedulingOffsetPreferenceConfigExt:

4>	consider itself to be configured to provide its preference on the minimum scheduling offset for 480 kHz SCS and/or 960 kHz SCS for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the lpwus-OffsetPreferenceConfig:

2>	if lpwus-OffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on time offset for LP-WUS monitoring in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on time offset for LP-WUS monitoring and stop timer T346p, if running;

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report, SCGFailureInformation, successful handover report, and successful PSCell change or addition report (if received for the associated cell group);

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the assisted-SSB-MTC-Config:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state;

1>	if the received otherConfig includes the referenceTimePreferenceReporting:

2>	consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide UE reference time assistance information;

1>	if successHO-Config is set to setup:

2>	consider itself to be configured to provide the successful handover information in accordance with 5.7.10.6;

1>	else:

2>	consider itself not to be configured to provide the successful handover information.

1>	if sn-initiatedPSCellChange is not included in the received otherConfig and if the successPSCell-Config in the received otherConfig is set to setup:

2>	consider itself to be configured by the corresponding cell group to provide the successful PSCell change or addition information in accordance with 5.7.10.7;

1>	if sn-initiatedPSCellChange is included in the received otherConfig and if the received otherConfig includes successPSCell-Config set to setup and thresholdPercentageT304-SCG is not included; or

1>	if sn-initiatedPSCellChange is included in received otherConfig and successPSCell-Config is already configured for the SCG:

2>	consider itself to be configured by the source PSCell to provide the successful PSCell change or addition information in accordance with 5.7.10.7;

1>	if successPSCell-Config in the received otherConfig is set to setup and thresholdPercentageT304-SCG is included:

2>	consider itself to be configured by the target PSCell to provide the successful PSCell change or addition information in accordance with 5.7.10.7

1>	if the successPSCell-Config received in otherConfig is set to release:

2>	consider itself not to be configured by the corresponding cell group to provide the successful PSCell change or addition information.

1>	if the received otherConfig includes the ul-GapFR2-PreferenceConfig:

2>	consider itself to be configured to provide its preference on FR2 UL gap in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide its preference on FR2 UL gap;

1>	if the received otherConfig includes the musim-GapAssistanceConfig:

2>	if musim-GapAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for gap preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for gap preference and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

1>	if the received otherConfig includes the musim-GapPriorityAssistanceConfig:

2>	consider itself to be configured to provide MUSIM assistance information for gap(s) priority in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide MUSIM assistance information for gap(s) priority;

1>	if the received otherConfig includes the musim-CapabilityRestrictionConfig:

2>	if musim-CapabilityRestrictionConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for capability restriction in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for capability restriction and stop timer T348 and T346n, if running;

1>	if the received otherConfig includes the rlm-RelaxationReportingConfig:

2>	if rlm-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of RLM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of RLM measurements and stop timer T346j associated with the cell group, if running;

1>	if the received otherConfig includes the bfd-RelaxationReportingConfig:

2>	if bfd-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of BFD measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of BFD measurements and stop timer T346k associated with the cell group, if running;

1>	if the received otherConfig includes the scg-DeactivationPreferenceConfig:

2>	if the scg-DeactivationPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its SCG deactivation preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its SCG deactivation preference and stop timer T346i, if running.

1>	if the received otherConfig includes the propDelayDiffReportConfig:

2>	if the propDelayDiffReportConfig is set to setup:

3>	consider itself to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s) in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s).

1>	if the received otherConfig includes the rrm-MeasRelaxationReportingConfig:

2>	if the rrm-MeasRelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the fulfilment of the criterion for relaxing RRM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the fulfilment of the criterion for relaxing RRM measurements.

1>	if the received otherConfig includes the multiRx-PreferenceReportingConfigFR2:

2>	if the multiRx-PreferenceReportingConfigFR2 is set to setup:

3>	consider itself to be configured to provide its preference on multi-Rx operation for FR2 in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on multi-Rx operation for FR2 and stop timer T346m, if running.

1>	if the received otherConfig includes the aerial-FlightPathAvailabilityConfig:

2>	consider itself to be configured to indicate the availability of flight path information in accordance with 5.7.4;

1>	if the received otherConfig includes the ul-TrafficInfoReportingConfig:

2>	if ul-TrafficInfoReportingConfig is set to setup:

3>	consider itself to be configured to provide UL traffic information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide UL traffic information and stop all instances of timer T346l, if running;

1>	if the received otherConfig includes n3c-RelayUE-InfoReportConfig:

2>	consider itself to be configured to report relay UE information with non-3GPP connection(s).

1>	if the received otherConfig includes gapOccasionCancelRatioReportConfig:

2>	if gapOccasionCancelRatioReportConfig is set to setup:

3>	consider itself to be configured to provide UE's preference for gap occasion cancellation ratio in accordance with Clause 5.7.4.

2>	else:

3>	consider itself not to be configured to provide UE's preference for gap occasion cancellation ratio;

3>	stop the timer T346o, if running.

1>	if the received otherConfig includes applicabilityReportConfig:

2>	if applicabilityReportConfig is set to setup:

3>	consider itself to be configured to report applicability information of configurations subject to the applicability determination procedure in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report applicability information of configurations subject to the applicability determination procedure in accordance with 5.7.4;

1>	if the received otherConfig includes dataCollectionPreferenceConfig:

2>	if dataCollectionPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on being configured with radio measurement resources for UE data collection in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on being configured with radio measurement resources for UE data collection;

1>	if the received otherConfig includes loggedDataCollectionAssistanceConfig:

2>	if loggedDataCollectionAssistanceConfig is set to setup:

3>	consider itself to be configured to report assistance information related to logging of radio measurements for network-side data collection in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report assistance information related to logging of radio measurements for network-side data collection.

#### 5.3.5.9a	MUSIM gap configuration

The UE shall:

1>	if musim-GapConfig is set to setup:

2>	for each musim-GapId included in the received musim-GapToReleaseList:

3>	release the periodic MUSIM gap configuration associated with the musim-GapId;

2>	for each MUSIM-Gap included in the received musim-GapToAddModList:

3>	setup periodic MUSIM gap configuration indicated by the MUSIM-Gap in accordance with the received musim-GapRepetitionAndOffset (providing musim-GapRepetition and Offset value for the following condition) i.e. the first subframe of each periodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN mod T = FLOOR(Offset/10);

subframe = Offset mod 10;

with T = musim-GapRepetition/10;

3>	set the MUSIM gap priority configuration indicated by musim-GapToAddModListExt, if configured, for each periodic MUSIM gap;

NOTE:	If network does not configure the relative priorities among MUSIM gaps as indicated by the UE, UE behaviour is not specified.

2>	if musim-AperiodicGap is included:

3>	setup aperiodic MUSIM gap configuration indicated by the musim-AperiodicGap in accordance with the received musim-Starting-SFN-AndSubframe, i.e. the first subframe of aperiodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN = starting-SFN;

subframe = startingSubframe;

2>	keep all colliding MUSIM gaps as specified in TS 38.133 [14], if musim-GapKeep is configured;

1>	else if musim-GapConfig is set to release:

2>	release the MUSIM gap configuration.

#### 5.3.5.10	MR-DC release

The UE shall:

1>	as a result of MR-DC release triggered by E-UTRA or NR:

2>	release SRB3, if established, as specified in 5.3.5.6.2;

2>	release SRB5, if established, as specified in 5.3.5.6.2;

2>	release measConfig associated with SCG;

2>	if the UE is configured with NR SCG:

3>	release the SCG configuration as specified in clause 5.3.5.4;

3>	if this procedure is initiated due to the reception of mrdc-ReleaseAndAdd:

4> release otherConfig associated with the SCG except the successPSCell-Config configured by the source PSCell, if configured;

3>	else:

4>	release otherConfig associated with the SCG, if configured;

4>	release successPSCell-Config configured by the PCell in the otherConfig, if configured;

3>	stop timers T346a, T346b, T346c, T346d, T346e, T346j and T346k associated with the SCG, if running;

3>	release bap-Config associated with the SCG, if configured;

3>	release the BAP entity as specified in TS 38.340 [47], if there is no configured bap-Config;

3>	release iab-IP-AddressConfigurationList associated with the SCG, if configured;

3>	perform the LTM configuration release procedure for the SCG as specified in clause 5.3.5.18.7;

2>	else if the UE is configured with E-UTRA SCG:

3>	release the SCG configuration as specified in TS 36.331 [10], clause 5.3.10.19 to release the E-UTRA SCG;

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the SRB1/SRB2 configurations and DRB/multicast MRB configurations as configured by radioBearerConfig or radioBearerConfig2.

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. Radio configuration also includes the RLC bearer configurations as configured by RLC-BearerConfig, PC5 Relay RLC channel as configured by SL-RLC-ChannelConfig, and Uu Relay RLC channel as configured by Uu-RelayRLC-ChannelConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10.

NOTE 1a:	For NR sidelink communication/discovery, the radio configuration includes the sidelink RRC configuration received from the network, but does not include the sidelink RRC reconfiguration and sidelink UE capability received from other UEs via PC5-RRC. In addition, the UE considers the new NR sidelink configurations as full configuration, in case of state transition and change of system information used for NR sidelink communication/discovery.

NOTE 1b:	To establish the RLC bearer of SRB(s) after release due to fullConfig, the network can include the srb-Identity within srb-ToAddModList (i.e. the UE applies RLC default configuration) and/or provide rlc-BearerToAddModList of concerned SRB(s) explicitly.

-	the logged measurement configuration;

-	the successHO-Config;

-	the successPSCell-Config.

1>	if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync:

2>	release/ clear all current common radio configurations;

2>	if sl-PathSwitchConfig was included in reconfigurationWithSync:

3>	use the default values specified in 9.2.3 for timer T311;

2>	else:

3>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311;

1>	else (full configuration after re-establishment or during RRC resume):

2>	if the UE is acting as L2 U2N Remote UE:

3>	use value for timer T311, as included in ue-TimersAndConstants received in SIB1

2>	else:

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	if no measConfigAppLayerId is included:

2>	inform upper layers about the release of all application layer measurement configurations;

2>	release all application layer measurement configurations including their fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig, if stored;

2>	discard any received application layer measurement reports from upper layers;

2>	consider itself not to be configured to send application layer measurement reports;

1>	if the UE is acting as L2 U2N Remote UE at the target side during reconfiguration with sync, or after re-establishment, or during RRC resume:

2>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the following:

-	parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

3>	establish an RLC entity for the corresponding SRB;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 2:	This is to get the SRBs (SRB1 and SRB2 for reconfiguration with sync and SRB2 for resume and reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

1>	for each pdu-Session that is part of the current UE configuration:

2>	release the SDAP entity (clause 5.1.2 in TS 37.324 [24]);

2>	release each DRB associated to the pdu-Session as specified in 5.3.5.6.4;

NOTE 3:	This will retain the pdu-Session but remove the DRBs including drb-identity of these bearers from the current UE configuration. Setup of the DRBs within the AS is described in clause 5.3.5.6.5 using the new configuration. The pdu-Session acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

1>	for each mbs-SessionId that is part of the current UE configuration and associated to a multicast MRB:

2>	release the SDAP entity (clause 5.1.2 in TS 37.324 [24]);

2>	release each multicast MRB associated to the mbs-SessionId as specified in 5.3.5.6.6;

NOTE 4:	This will retain the mbs-SessionId but remove the multicast MRBs including mrb-identity of these bearers from the current UE configuration. Setup of the multicast MRBs within the AS is described in clause 5.3.5.6.7 using the new configuration. The mbs-SessionId acts as the anchor for associating the released and re-setup multicast MRB. In the AS the multicast MRB re-setup is equivalent with a new multicast MRB setup (including new PDCP and logical channel configurations).

1>	for each pdu-Session that is part of the current UE configuration but not added with same pdu-Session in the drb-ToAddModList:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers immediately;

1>	for each mbs-SessionId that is part of the current UE configuration but not added with the same mbs-SessionId in the mrb-ToAddModList:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the user plane resources for the mbs-SessionId to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the user plane resources for the mbs-SessionId to upper layers immediately.

#### 5.3.5.12	BAP configuration

The IAB-MT shall:

1>	if the bap-Config is set to setup:

2>	if no BAP entity is established:

3>	establish a BAP entity as specified in TS 38.340 [47];

2>	if bap-Address is included:

3>	configure the BAP entity to use the bap-Address as this node's BAP address;

2>	if defaultUL-BAP-RoutingID is included:

3>	configure the BAP entity to apply the default UL BAP routing ID according to the configuration;

2>	if defaultUL-BH-RLC-Channel is included

3>	configure the BAP entity to apply the default UL BH RLC channel according to the configuration;

2>	if flowControlFeedbackType is included:

3>	configure the BAP entity to apply the flow control feedback according to the configuration;

1>	if the bap-Config is set to release:

2>	release the concerned bap-Config;

2>	if there is no other configured bap-Config for the MCG or for the SCG

3>	release the BAP entity as specified in TS 38.340 [47].

#### 5.3.5.12a	IAB Other Configuration

##### 5.3.5.12a.1	IP address management

###### 5.3.5.12a.1.1	IP Address Release

The IAB-MT shall:

1>	if the release is triggered by reception of the iab-IP-AddressToReleaseList:

2>	for each iab-IP-AddressIndex value included in iab-IP-AddressToReleaseList:

3>	release the corresponding IAB-IP-AddressConfiguration.

###### 5.3.5.12a.1.2	IP Address Addition/Modification

The IAB-MT shall:

1>	for each iab-IP-AddressIndex value included in the iab-IP-AddressToAddModList that is not part of the current IAB-MT configuration:

2>	add the IP address indicated in iab-IP-Address, corresponding to the iab-IP-AddressIndex.

2>	if added IP address is iPv4-Address:

3>	if iab-IP-Usage is set to f1-C:

4>	store the received IPv4 address for F1-C traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else if iab-IP-Usage is set to f1-U:

4>	store the received IPv4 address for F1-U traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else if iab-IP-Usage is set to non-F1:

4>	store the received IPv4 address for non-F1 traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else:

4>	store the received IPv4 address for all traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

2>	else if iPv6-Address is included:

3>	if iab-IP-Usage is set to f1-C:

4>	store the received IPv6 address for F1-C traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else if iab-IP-Usage is set to f1-U:

4>	store the received IPv6 address for F1-U traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else if iab-IP-Usage is set to non-F1:

4>	store the received IPv6 address for non-F1 traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else:

4>	store the received IPv6 address for all traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

2>	else if iPv6-Prefix is included in iab-IP-AddressToAddModList:

3>	if iab-IP-Usage is set to f1-C:

4>	store the received IPv6 address prefix for F1-C traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else if iab-IP-Usage is set to f1-U:

4>	store the received IPv6 address prefix for F1-U traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else if iab-IP-Usage is set to non-F1:

4>	store the received IPv6 address prefix for non-F1 traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

3>	else:

4>	store the received IPv6 address prefix for all traffic together with the IAB-donor-DU BAP address corresponding to the iab-IP-AddressIndex.

1>	for each iab-IP-AddressIndex value included in the iab-IP-AddressToAddModList that is part of the current IAB-MT configuration:

2>	modify the IP address configuration(s) in accordance with the IAB-IP-AddressConfiguration corresponding to the iab-IP-AddressIndex.

#### 5.3.5.13	Conditional Reconfiguration

##### 5.3.5.13.1	General

The network configures the UE with one or more candidate target SpCells in the conditional reconfiguration. The UE evaluates the condition of each configured candidate target SpCell. The UE applies the conditional reconfiguration associated with one of the target SpCells which fulfils associated execution condition.

The network can also configure the UE with one or more candidate target PCells associated with one or more candidate target PSCells. The UE evaluates the conditions for the candidate target PCells and the associated candidate target PSCells in parallel and applies a target configuration that include PCell and PSCell for which the associated execution conditions are fulfilled. If there are multiple candidate PSCells associated with one candidate target PCell, the network provides multiple conditional configurations for the same candidate target PCell, i.e., each configuration contains one MCG configuration (for the same candidate target PCell) and one SCG configuration (for one of the multiple associated candidate PSCells). For this case, the network may also provide a complementary CHO only configuration, i.e., there is execution condition only for candidate PCell.

The network provides the configuration parameters for the target SpCell(s) in the condRRCReconfig.

In NR-DC, the UE may receive two independent conditionalReconfiguration:

-	a conditionalReconfiguration associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a conditionalReconfiguration, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

In this case:

-	the UE maintains two independent VarConditionalReconfig, one associated with each conditionalReconfiguration;

-	the UE independently performs all the procedures in clause 5.3.5.13 for each conditionalReconfiguration and the associated VarConditionalReconfig, unless explicitly stated otherwise;

-	the UE performs the procedures in clause 5.5 for the VarConditionalReconfig associated with the same cell group like the measConfig.

In EN-DC, the VarConditionalReconfig is associated with the SCG.

In NE-DC and when no SCG is configured, the VarConditionalReconfig is associated with the MCG.

The UE performs the following actions based on a received ConditionalReconfiguration IE:

1>	if the ConditionalReconfiguration contains the condReconfigToRemoveList:

2>	perform conditional reconfiguration removal procedure as specified in 5.3.5.13.2;

1>	if the ConditionalReconfiguration contains the condReconfigToAddModList:

2>	perform conditional reconfiguration addition/modification as specified in 5.3.5.13.3;

1>	if the ConditionalReconfiguration contains the scpac-ReferenceConfiguration:

2>	perform subsequent CPAC reference configuration addition/removal as specified in 5.3.5.13.6;

1>	if the ConditionalReconfiguration contains the sk-CounterConfiguration:

2>	perform sk-CounterList addition/modification/removal as specified in 5.3.5.13.7;

1>	if the ConditionalReconfiguration contains the servingSecurityCellSetId:

2>	if the current VarServingSecurityCellSetID includes servingSecurityCellSetId:

3>	replace the servingSecurityCellSetId value within VarServingSecurityCellSetID with the received servingSecurityCellSetID;

2>	else:

3>	store the received servingSecurityCellSetId within VarServingSecurityCellSetID.

##### 5.3.5.13.2	Conditional reconfiguration removal

The UE shall:

1>	for each condReconfigId value included in the condReconfigToRemoveList that is part of the current UE conditional reconfiguration in VarConditionalReconfig:

2>	remove the entry with the matching condReconfigId from the VarConditionalReconfig;

NOTE 1:	The UE does not consider the message as erroneous if the condReconfigToRemoveList includes any condReconfigId value that is not part of the current UE configuration.

NOTE 2:	Void.

##### 5.3.5.13.3	Conditional reconfiguration addition/modification

For each condReconfigId received in the condReconfigToAddModList IE the UE shall:

1>	if an entry with the matching condReconfigId exists in the condReconfigToAddModList within the VarConditionalReconfig:

2>	if the entry in condReconfigToAddModList includes an condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell;

3>	replace condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell within the VarConditionalReconfig with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes subsequentCondReconfig containing condExecutionCondToAddModList:

3>	for each condReconfigId received in condExecutionCondToAddModList:

4>	if an entry with the matching condReconfigId exists in the condExecutionCondToAddModList within VarConditionalReconfig;

5>	replace the entry in condExecutionCondToAddModList within VarConditionalReconfig with the value received for this condReconfigId;

4>	else:

5>	add a new entry in condExecutionCondToAddModList within VarConditionalReconfig with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes subsequentCondReconfig containing condExecutionCondToReleaseList:

3>	for each condReconfigId received in condExecutionCondToReleaseList that is part of current stored condExecutionCondToAddModList within VarConditionalReconfig:

4>	remove the entry in condExecutionCondToAddModList within VarConditionalReconfig with the value received for this condReconfigId;

NOTE 1:	The UE does not consider the message as erroneous if the condExecutionCondToReleaseList includes any condReconfigId value that is not part of the current UE configuration.

2>	if the entry in condReconfigToAddModList includes a securityCellSetId;

3>	replace securityCellSetId within the VarConditionalReconfig with the value received for this condReconfigId;

NOTE 2:	The UE should release the entry within VarServingSecurityCellSetID in case all the subsequent CPAC configurations are released.

2>	if the entry in condReconfigToAddModList includes a condRRCReconfig;

3>	replace condRRCReconfig within the VarConditionalReconfig with the value received for this condReconfigId;

1>	else:

2>	add a new entry for this condReconfigId within the VarConditionalReconfig;

1>	perform conditional reconfiguration evaluation as specified in 5.3.5.13.4;

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

3>	if the associated condExecutionCondPSCell is configured:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; and

4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the nr-SCG within the received condRRCReconfig to be applicable cell;

3>	else:

4>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

2>	else if the RRCReconfiguration within condRRCReconfig includes the secondaryCellGroup including the reconfigurationWithSync:

3>	if the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the received condRRCReconfig is not the PSCell:

4>	if subsequentCondReconfig is not included for the condReconfigId; or

4>	if subsequentCondReconfig is not included for the PSCell; or

4>	if subsequentCondReconfig is included for the condReconfigId and there is a subsequentCondReconfig for the PSCell with a matching condReconfigId value in condExecutionCondToAddModList:

5>	consider the cell to be applicable cell;

NOTE 0:	For CHO configured with only condEventD1, condEventD2 or condEventT1 (without any RRM measurement event), it is up to UE implementation whether and how to detect the applicable cell.

2>	if condExecutionCondSCG is configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondSCG as a measId in the VarMeasConfig associated with the SCG measConfig;

2>	if the condExecutionCondPSCell is configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondPSCell as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	if condExecutionCond is configured:

3>	if it is configured via SRB3 or configured within nr-SCG or within nr-SecondaryCellGroupConfig (specified in TS 36.331[10]) via SRB1:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the SCG measConfig;

3>	else:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond, condExecutionCondSCG, or condExecutionCondPSCell of the condReconfigId:

3>	if condExecutionCond, condExecutionCondSCG, and subsequentCondReconfig are included for the condReconfigId:

4>	ignore the measId(s) in the condExecutionCond of the condReconfigId;

3>	if the condTriggerConfig is not configured with nesEvent:

4>	if the condEventId is associated with condEventT1, and if the entry condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4> if the condEventId is associated with condEventD1 or condEventD2, and if the entry conditions applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4, condEventA5, condEventA3H1, condEventA3H2, condEventA5H1 or condEventA5H2, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be fulfilled;

4>	if the measId for this event associated with the condReconfigId has been modified; or

4>	if the condEventId is associated with condEventT1, and if the leaving condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

4>if the condEventId is associated with condEventD1 or condEventD2, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

4>	if the condEventId is associated with condEventA3, condEventA4, condEventA5, condEventA3H1, condEventA3H2, condEventA5H1 or condEventA5H2, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be not fulfilled;

3>	else:

4>	if NES mode indication is received from lower layers, indicating that the NES-specific CHO execution condition of the PCell is enabled; and

4>	if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be fulfilled;

4>	if the measId for this event associated with the condReconfigId has been modified; or

4>	if NES mode indication is received from lower layers, indicating that the NES-specific CHO execution condition of the PCell is disabled; or

4>	if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

5>	consider the event associated to that measId to be not fulfilled;

2>	if condExecutionCondPSCell is not configured:

3>	if event(s) associated to all measId(s) within condTriggerConfig for the applicable cell are fulfilled:

4>	consider the applicable cell, associated to that condReconfigId, as a triggered cell;

4>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

2>	else:

3>	if event(s) associated to all measId(s), as indicated in the condExecutionCond and condExecutionCondPSCell, within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

4>	consider the target candidate PCell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered PCell;

4>	consider the target candidate PSCell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered PSCell;

4>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5.

2>	if one of the events associated to the measIds within condTriggerConfig for the applicable cell within the stored condRRCReconfig is not configured with nesEvent, and the other event associated to the measIds within condTriggerConfig for the applicable cell within the stored condRRCReconfig is configured with nesEvent, and at least one of them is fulfilled:

3>	consider the applicable cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

NOTE 1:	Up to 2 MeasId can be configured for each condReconfigId, if condExecutionCondPSCell is not configured. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

NOTE 2:	Void.

NOTE 3:	For CHO with candidate SCG(s), up to 2 MeasId can be configured for condExecutionCond and up to 2 MeasId can be configured for condExecutionCondPSCell for each condReconfigId.

##### 5.3.5.13.4a	Conditional reconfiguration evaluation of SN initiated inter-SN CPC for EN-DC

The UE shall:

1>	for each condReconfigurationId within the VarConditionalReconfiguration specified in TS 36.331[10]:

2>	for each measId included in the measIdList within VarMeasConfig indicated in the CondReconfigExecCondSCG contained in the triggerConditionSN associated to the condReconfigurationId as specified in TS 36.331[10]:

3>	if the entry condition(s) applicable for the event associated with that measId, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event associated with that measId:

4>	consider this event to be fulfilled;

3>	if the measId for this event has been modified; or

3>	if the leaving condition(s) applicable for this event associated with that measId, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event associated with that measId:

4>	consider this event associated to that measId to be not fulfilled;

2>	if trigger conditions for all events associated with the measId(s) indicated in the CondReconfigExecCondSCG contained in the triggerConditionSN as specified in TS 36.331[10]), are fulfilled:

3>	consider the target cell candidate within the RRCReconfiguration message contained in nr-SecondaryCellGroupConfig in the RRCConnectionReconfiguration message, as specified in TS 36.331[10], contained in the stored condReconfigurationToApply, associated to that condReconfigurationId as specified in TS 36.331[10]), clause 5.3.5.9.4, as a triggered cell;

3>	initiate the conditional reconfiguration execution, as specified in TS 36.331[10]), clause 5.3.5.9.5;

NOTE:	Void.

##### 5.3.5.13.5	Conditional reconfiguration execution

The UE shall:

1>	if more than one pair of triggered PCell and associated triggered PSCell exist:

2>	select one of the triggered PCell(s) and the associated triggered PSCell(s) as the selected cells for conditional reconfiguration execution;

1>	else if only one pair of triggered PCell and associated triggered PSCell exists:

2>	consider the triggered PCell and the associated triggered PSCell as the selected cells for conditional reconfiguration execution;

1>	else if more than one triggered cell exists:

2>	select one of the triggered cells as the selected cell for conditional reconfiguration execution;

1>	else:

2>	consider the triggered cell as the selected cell for conditional reconfiguration execution;

1>	for the selected cell(s) of conditional reconfiguration execution:

2>	if the subsequentCondReconfig is included in the entry in VarConditionalReconfig containing the RRCReconfiguration message for the selected cell:

3>	perform the actions as specified in 5.3.5.13.8;

2>	else:

3>	apply the stored condRRCReconfig of the selected cell and perform the actions as specified in 5.3.5.3;

NOTE:	If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

##### 5.3.5.13.6	Subsequent CPAC reference configuration addition/removal

The UE shall:

1>	if the scpac-ReferenceConfiguration is set to setup:

2>	if scpac-ReferenceConfiguration exists within the VarConditionalReconfig:

3>	replace the scpac-ReferenceConfiguration within the VarConditionalReconfig;

2>	else:

3>	store the scpac-ReferenceConfiguration within the VarConditionalReconfig;

1>	else (if scpac-ReferenceConfiguration is set to release):

2>	remove the scpac-ReferenceConfiguration within the VarConditionalReconfig;

##### 5.3.5.13.7	sk-Counter configuration addition/modification/removal

The UE shall:

1>	for each securityCellSetId received in the sk-CounterConfigToAddModList IE:

2>	if an entry with the matching securityCellSetId exists in the sk-CounterConfigToAddModList within the VarConditionalReconfig:

3>	replace the sk-CounterList within the VarConditionalReconfig with the sk-CounterList according to the received securityCellSetId;

2>	else:

3>	add a new entry for this securityCellSetId within the VarConditionalReconfig;

1>	for each securityCellSetId value included in the sk-CounterConfigToRemoveList that is part of the current sk-CounterConfigToAddModList in VarConditionalReconfig:

2>	remove the entry with the matching securityCellSetId from the sk-CounterConfigToAddModList;

##### 5.3.5.13.8	Subsequent CPAC execution

Upon the conditional reconfiguration execution for subsequent CPAC, the UE shall:

1>	if the selected subsequent CPAC candidate configuration is stored in MCG VarConditionalReconfig:

2>	for each SRB/DRB in current UE configuration:

-	keep the associated RLC, PDCP and SDAP entities, their state variables, buffers and timers;

-	release all fields related to the SRB/DRB configuration except for srb-Identity, drb-Identity, and securityConfig;

2>	release/clear all current dedicated radio configuration except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key and the secondary key;

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers;

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers;

-	the UE variables VarConditionalReconfig and VarServingSecurityCellSetID;

-	the logged measurement configuration.

2>	release/clear all current common radio configuration, except for the ServingCellConfigCommon of the PCell;

2>	apply the default MAC Cell Group configuration for MCG MAC and SCG MAC as specified in 9.2.2;

2>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications for the MCG and SCG;

1>	else:

2>	for each SRB/DRB in current UE configuration:

-	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

-	release all fields related to the SRB/DRB configuration except for srb-Identity, drb-Identity, and securityConfig;

2>	release/clear all current dedicated radio configuration associated with the SCG except for the following:

-	the AS security configurations associated with the secondary key;

-	the UE variables VarConditionalReconfig.

2>	release/clear all current common radio configuration associated with the SCG;

2>	apply the default MAC Cell Group configuration for the SCG MAC as specified in 9.2.2;

2>	use the default values specified in 9.2.3 for timer T310 and constants N310 and N311 for the SCG ;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications for the SCG;

1>	if the securityCellSetId is included in the entry in VarConditionalReconfig containing the RRCReconfiguration message:

2>	if servingSecurityCellSetId is not included within VarServingSecurityCellSetID; or

2>	if the value of the securityCellSetId is not equal to the value of servingSecurityCellSetId within VarServingSecurityCellSetID:

3>	consider the first sk-Counter value in the sk-CounterList associated with the securityCellSetId within the VarConditionalReconfig as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7;

3>	remove the selected sk-Counter value from the sk-CounterList associated with the securityCellSetId within the VarConditionalReconfig;

3>	if the current VarServingSecurityCellSetID includes servingSecurityCellSetId:

4>	replace the value of servingSecurityCellSetId within VarServingSecurityCellSetID with the value of securityCellSetId associated with the selected cell;

3>	else:

4>	store the servingSecurityCellSetId within VarServingSecurityCellSetID with the value of securityCellSetId associated with the selected cell;

1>	if the selected subsequent CPAC candidate configuration is stored in the SCG VarConditionalReconfig:

2>	for each drb-Identity value included in each RadioBearerConfig in the selected subsequent CPAC candidate configuration that is part of the current UE configuration, the UE shall perform the following actions after the end of this procedure:

3>	if the bearer is an AM DRB:

4>	trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];

3>	re-establish the corresponding RLC entity as specified in TS 38.322 [4];

1>	else:

2>	for each drb-Identity value included in each RadioBearerConfig in the selected subsequent CPAC candidate configuration that is part of the current UE configuration, the UE shall perform the following actions after the end of this procedure:

3>	if the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration; or

3>	if the bearer is associated with the secondary key (S-KgNB) as indicated by keyToUse in the current UE configuration and a new sk-Counter value has been selected due to the conditional reconfiguration execution for subsequent CPAC:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e., the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

4>	if drb-ContinueROHC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueROHC is configured;

4>	if drb-ContinueEHC-DL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-DL is configured;

4>	if drb-ContinueEHC-UL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-UL is configured;

4>	if drb-ContinueUDC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueUDC is configured;

4>	re-establish the corresponding RLC entity as specified in TS 38.322 [4];

4>	trigger the PDCP entity of the bearer to perform PDCP re-establishment as specified in TS 38.323 [5];

3>	else:

4>	if there is an associated SCG RLC bearer in the selected subsequent CPAC candidate configuration that is part of the current UE configuration:

5>	re-establish the SCG RLC entity as specified in TS 38.322 [4];

4>	if the RLC entity of the associated RLC bearer(s) is re-established; or

4>	if an associated RLC bearer is released in the selected subsequent CPAC candidate configuration:

5>	if the bearer is an AM DRB:

6>	trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];

2>	for each srb-Identity included in RadioBearerConfig that is part of the current UE configuration and if the radio bearer is SRB3 or SRB5, the UE shall perform the following actions after the end of this procedure:

3>	if a new sk-Counter value has been selected due to the conditional reconfiguration execution for subsequent CPAC:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the secondary key (S-KgNB) as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	trigger the PDCP entity of SRB to perform PDCP re-establishment as specified in TS 38.323 [5];

3>	else:

4>	trigger the PDCP entity of SRB to perform SDU discard as specified in TS 38.323 [5];

3>	re-establish the corresponding RLC entity as specified in TS 38.322 [4];

1>	if scpac-ConfigComplete is not included within the VarConditionalReconfig for the selected cell:

2>	if the subsequent CPAC candidate cell configuration is stored in MCG VarConditionalReconfig:

3>	consider scpac-ReferenceConfiguration in MCG VarConditionalReconfig to be the current UE configuration;

2>	else:

3>	consider scpac-ReferenceConfiguration in SCG VarConditionalReconfig to be the current SCG configuration;

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3.

1>	apply the stored condRRCReconfig of the selected cell(s) and perform the actions as specified in 5.3.5.3;

1>	release the radio bearer(s) and the associated logical channel(s) that are part of the current UE configuration but not part of the subsequent CPAC candidate configuration for the selected cell, or the subsequent CPAC reference configuration (in case the subsequent CPAC candidate configuration does not include scpac-ConfigComplete).

NOTE 2:	When scpac-ConfigComplete is not included for the selected cell, before a subsequent CPAC execution, a UE implementation may generate and store an RRC reconfiguration message by applying the received subsequent CPAC candidate configuration on top of the subsequent CPAC reference configuration, and the stored RRC reconfiguration message is applied for subsequent CPAC execution. The UE needs to ensure that the RRC reconfiguration applied at the time of subsequent CPAC execution is in accordance with the latest received scpac-ReferenceConfiguration and condRRCReconfig for the subsequent CPAC configuration.

#### 5.3.5.13a	SCG activation

Upon initiating the procedure, the UE shall:

1>	if the UE is configured with an SCG after receiving the message for which this procedure is initiated:

2>	if the UE was configured with a deactivated SCG before receiving the message for which this procedure is initiated:

3>	consider the SCG to be activated;

3>	resume performing radio link monitoring on the SCG, if previously stopped;

3>	indicate to lower layers to resume beam failure detection on the PSCell, if previously stopped;

3>	indicate to lower layers that the SCG is activated.

#### 5.3.5.13b	SCG deactivation

Upon initiating the procedure, the UE shall:

1>	consider the SCG to be deactivated;

1>	indicate to lower layers that the SCG is deactivated;

1>	if bfd-and-RLM is configured to true:

2>	perform radio link monitoring on the SCG;

2>	indicate to lower layers to perform beam failure detection on the PSCell;

1>	else:

2>	stop radio link monitoring on the SCG;

2>	indicate to lower layers to stop beam failure detection on the PSCell;

2>	stop timer T310 for this cell group, if running;

2>	stop timer T312 for this cell group, if running;

2>	reset the counters N310 and N311;

1>	if the UE was in RRC_CONNECTED and the SCG was activated before receiving the message for which this procedure is initiated:

2>	if SRB3 was configured before the reception of the RRCReconfiguration or of the RRCConnectionReconfiguration and SRB3 is not to be released according to any RadioBearerConfig included in the RRCReconfiguration or in the RRCConnectionReconfiguration as specified in TS 36.331[10]:

3>	trigger the PDCP entity of SRB3 to perform SDU discard as specified in TS 38.323 [5];

3>	re-establish the RLC entity of SRB3 as specified in TS 38.322 [4].

#### 5.3.5.13b1	SCG activation without SN message

Upon initiating the procedure, the UE shall:

1>	if the SCG was deactivated before the reception of the RRCReconfiguration message or the E-UTRA RRCConnectionReconfiguration message for which the procedure invoking this clause is executed:

2>	consider the SCG to be activated;

2>	indicate to lower layers that the SCG is activated;

2>	resume performing radio link monitoring on the SCG, if previously stopped;

2>	indicate to lower layers to resume beam failure detection on the PSCell, if previously stopped;

2>	if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration message or the E-UTRA RRCConnectionReconfiguration message for which the procedure invoking this clause is executed; or

2>	if lower layers indicate that a Random Access procedure is needed for SCG activation:

3>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3].

#### 5.3.5.13c	FR2 UL gap configuration

The UE shall:

1>	if ul-GapFR2-Config is set to setup:

2>	if an FR2 UL gap configuration is already setup, release the FR2 UL gap configuration;

2>	setup the FR2 UL gap configuration indicated by the ul-GapFR2-Config in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR (gapOffset/10);

if the UGRP is larger than 5ms:

subframe = gapOffset mod 10;

else:

subframe = gapOffset or (gapOffset +5);

with T = CEIL(UGRP/10).

1>	else if ul-GapFR2-Config is set to release:

2>	release the FR2 UL gap configuration.

NOTE 1:	For ul-GapFR2-Config configuration with synchronous CA, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation. For ul-GapFR2-Config configuration with asynchronous CA, the SFN and subframe of a serving cell on FR2 frequency indicated by the refFR2-ServCellAsyncCA in ul-GapFR2-Config is used in the gap calculation.

#### 5.3.5.13d	Application layer measurement configuration

The UE shall:

1>	if measConfigAppLayerToReleaseList is included in appLayerMeasConfig within RRCReconfiguration or RRCResume:

2>	for each measConfigAppLayerId value included in the measConfigAppLayerToReleaseList:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration including any RAN visible application layer measurement configuration;

3>	discard any application layer measurement reports received from upper layers;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig, if stored;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId.

1>	if measConfigAppLayerToAddModList is included in appLayerMeasConfig within RRCReconfiguration or RRCResume:

2>	for each measConfigAppLayerId value included in the measConfigAppLayerToAddModList:

3>	if measConfigAppLayerContainer is included for the corresponding MeasConfigAppLayer configuration:

4>	forward the measConfigAppLayerContainer, the measConfigAppLayerId and the serviceType to upper layers considering the serviceType;

3>	consider itself to be configured to send application layer measurement report for the measConfigAppLayerId in accordance with 5.7.16;

3>	forward the transmissionOfSessionStartStop, if configured, and measConfigAppLayerId to upper layers considering the serviceType;

3>	if ran-VisibleParameters is set to setup:

4>	forward the measConfigAppLayerId, the ran-VisiblePeriodicity, if configured, the numberOfBufferLevelEntries, if configured, and the reportPlayoutDelayForMediaStartup, if configured, to upper layers considering the serviceType;

3>	else if ran-VisibleParameters is set to release:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the RAN visible application layer measurement configuration;

4>	discard any RAN visible application layer measurement reports received from upper layers;

3>	if pauseReporting is set to true:

4>	if at least one segment, but not all segments, of a segmented MeasurementReportAppLayer message containing an application layer measurement report associated with the measConfigAppLayerId has been submitted to lower layers for transmission:

5>	submit the remaining segments of the MeasurementReportAppLayer message to lower layers for transmission;

4>	suspend submitting application layer measurement report containers to lower layers for the application layer measurement configuration associated with the measConfigAppLayerId;

4>	store any previously or subsequently received application layer measurement report containers associated with the measConfigAppLayerId for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers;

4>	if the memory reserved for storing application layer measurement report containers becomes full while the reporting is paused:

5>	if the reports are associated with a configuration including appLayerMeasPriority:

6>	discard reports in priority order where reports with the lowest priority are discarded first, and among reports with equal associated priority, discard reports in the order they were received, where older reports are discarded first;

5>	else:

6>	discard reports in the order they were received, where older reports are discarded first;

3>	else if pauseReporting is set to false and if transmission of application layer measurement report containers has previously been suspended for the application layer measurement configuration associated with the measConfigAppLayerId:

4>	submit stored application layer measurement report containers to lower layers, if any, for the application layer measurements configuration associated with the measConfigAppLayerId;

4>	resume submitting application layer measurement report containers to lower layers for the application layer measurement configuration associated with the measConfigAppLayerId;

3>	if appLayerIdleInactiveConfig is set to setup:

4>	store the received qoe-Reference, measConfigAppLayerId, serviceType, qoe-MeasurementType, qoe-AreaScope, mce-Id, availableRAN-VisibleMetrics and appLayerMeasPriority (if included) in VarAppLayerIdleConfig;

4>	if the qoe-AreaScope includes plmn-IdentityList:

5>	set plmn-IdentityList in VarAppLayerPLMN-ListConfig to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

4>	else:

5>	set plmn-IdentityList in VarAppLayerPLMN-ListConfig to include the RPLMN;

3>	else if appLayerIdleInactiveConfig is set to release:

4>	release appLayerIdleInactiveConfig.

NOTE 1:	The UE may discard reports when the memory reserved for storing application layer measurement report containers becomes full. If no or equal appLayerMeasPriority is configured, older reports are discarded first.

NOTE 2:	The transmission of RAN visible application layer measurement reports and appLayerSessionStatus is not paused when pauseReporting is set to true.

NOTE 3:	The UE may discard an application layer measurement configuration and associated unsent reports after 48 hours in RRC_IDLE. The time duration stops incrementing and is reset to zero if the UE is transferred to RRC_CONNECTED.

NOTE 4:	Release of a previously configured value in ran-VisibleParameters is needed (independently of its RRC Need code) before a new value is forwarded to upper layers.

#### 5.3.5.14	Sidelink dedicated configuration

Upon initiating the procedure, the UE shall:

1>	if sl-FreqInfoToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each entry included in the received sl-FreqInfoToReleaseList that is part of the current UE configuration:

3>	release the related configurations from the stored NR sidelink communication/discovery configurations;

1>	if sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	if configured to receive NR sidelink communication:

3>	use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7;

2>	if configured to transmit NR sidelink communication:

3>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

2>	if configured to receive NR sidelink discovery:

3>	use the resource pool(s) indicated by sl-DiscRxPool or sl-RxPool for NR sidelink discovery reception, as specified in 5.8.13.2;

2>	if configured to transmit NR sidelink discovery:

3>	use the resource pool(s) indicated by sl-DiscTxPoolSelected, sl-DiscTxPoolScheduling, sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional for NR sidelink discovery transmission, as specified in 5.8.13.3;

2>	if configured to perform SL-PRS measurement:

3>	use the resource pool(s) indicated by sl-RxPool and/or sl-PRS-RxPool for SL-PRS reception, as specified in 5.8.18.2;

2>	if configured to transmit SL-PRS:

3>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolScheduling or sl-PRS-TxPoolExceptional for SL-PRS transmission, as specified in 5.8.18.3;

2>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolScheduling, sl-DiscTxPoolSelected, sl-DiscTxPoolScheduling, sl-TxPoolExceptional or sl-PRS-TxPoolExceptional for NR sidelink communication/discovery/positioning transmission, as specified in 5.5.3;

2>	use the synchronization configuration parameters for NR sidelink communication/discovery/positioning on frequencies included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt, as specified in 5.8.5;

1>	if sl-RadioBearerToReleaseList or sl-RLC-BearerToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform sidelink DRB release as specified in 5.8.9.1a.1;

1>	if sl-RLC-BearerToReleaseListSizeExt is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform additional sidelink RLC bearer release as specified in 5.8.9.1a.5;

1>	if sl-RadioBearerToAddModList or sl-RLC-BearerToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform sidelink DRB addition/modification as specified in 5.8.9.1a.2;

1>	if sl-RLC-BearerToAddModListSizeExt is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform additional sidelink RLC bearer addition/modification as specified in 5.8.9.1a.6;

1>	if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	configure the MAC entity parameters, which are to be used for NR sidelink communication/discovery, in accordance with the received sl-ScheduledConfig;

1>	if sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	configure the parameters, which are to be used for NR sidelink communication/discovery, in accordance with the received sl-UE-SelectedConfig;

1>	if sl-MeasConfigInfoToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each SL-DestinationIndex included in the received sl-MeasConfigInfoToReleaseList that is part of the current UE configuration:

3>	remove the entry with the matching SL-DestinationIndex from the stored NR sidelink measurement configuration information;

1>	if sl-MeasConfigInfoToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each sl-DestinationIndex included in the received sl-MeasConfigInfoToAddModList that is part of the current stored NR sidelink measurement configuration:

3>	reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink measurement configuration information;

2>	for each sl-DestinationIndex included in the received sl-MeasConfigInfoToAddModList that is not part of the current stored NR sidelink measurement configuration:

3>	add a new entry for this sl-DestinationIndex to the stored NR sidelink measurement configuration.

NOTE 1:	The UE is expected to update the mapping between the Destination Layer-2 ID and the destination index for the stored NR sidelink measurement configuration after the UE updates the destination list and reports to the gNB.

1>	if sl-DRX-ConfigGC-BC is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	store the NR sidelink DRX configuration and configure lower layers to perform sidelink DRX operation for groupcast and broadcast as specified in TS 38.321 [3];

1>	if sl-DRX-ConfigUC-ToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each SL-DestinationIndex included in the received sl-DRX-ConfigUC-ToReleaseList that is part of the current UE configuration:

3>	remove the entry with the matching SL-DestinationIndex from the stored NR sidelink DRX configuration information;

1>	if sl-DRX-ConfigUC-ToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is part of the current stored NR sidelink DRX configuration:

3>	reconfigure the entry according to the value received for this sl-DestinationIndex from the stored NR sidelink DRX configuration information;

2>	for each sl-DestinationIndex included in the received sl-DRX-ConfigUC-ToAddModList that is not part of the current stored NR sidelink DRX configuration:

3>	add a new entry for this sl-DestinationIndex to the stored NR sidelink DRX configuration.

NOTE 2:	The UE is expected to update the mapping between the Destination Layer-2 ID and the destination index for the stored NR sidelink DRX configuration after the UE updates the destination list and reports to the gNB.

1>	if sl-RLC-ChannelToReleaseList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	perform PC5 Relay RLC channel release as specified in 5.8.9.7.1;

1>	if sl-RLC-ChannelToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration or RRCSetup:

2>	perform PC5 Relay RLC channel addition/modification as specified in 5.8.9.7.2;

#### 5.3.5.15	L2 U2N or U2U Relay UE configuration

##### 5.3.5.15.1	General

The network configures the L2 U2N or U2U Relay UE with relay operation related configurations. For each connected L2 U2N or U2U Remote UE indicated in sl-L2IdentityRemote, the network provides the configuration parameters used for relaying.

The L2 U2N Relay UE shall:

1>	if sl-L2RelayUE-Config is set to setup:

2>	if the sl-L2RelayUE-Config contains the sl-RemoteUE-ToReleaseList:

3>	perform the L2 U2N Remote UE release as specified in 5.3.5.15.2;

2>	if the sl-L2RelayUE-Config contains the sl-RemoteUE-ToAddModList:

3>	perform the L2 U2N Remote UE addition/modification as specified in 5.3.5.15.3;

1>	else if sl-L2RelayUE-Config is set to release:

2>	release the L2 U2N relay operation related configurations.

The L2 U2U Relay UE shall:

1>	if sl-L2RelayUE-Config is set to setup:

2>	if the sl-L2RelayUE-Config contains the sl-U2U-RemoteUE-ToReleaseList:

3>	perform the L2 U2U Remote UE release as specified in 5.3.5.15.2;

2>	if the sl-L2RelayUE-Config contains the sl-U2U-RemoteUE-ToAddModList:

3>	perform the L2 U2U Remote UE addition/modification as specified in 5.3.5.15.3;

1>	else if sl-L2RelayUE-Config is set to release:

2>	release the L2 U2U relay operation related configurations.

##### 5.3.5.15.2	L2 U2N or U2U Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	indicate upper layers to trigger PC5 unicast link release.

The L2 U2U Relay UE shall:

1>	if the release is triggered by reception of the sl-U2U-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-U2U-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5-RRC connection to a L2 U2U Remote UE with this SL-DestinationIdentity:

4>	release the configuration associated with the L2 U2U Remote UE.

##### 5.3.5.15.3	L2 U2N or U2U Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelayToAddModList if applicable;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

3>	release SL-RLC1, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

3>	if SL-RLC1 is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay and sl-SRAP-ConfigRelayToAddModList/ sl-SRAP-ConfigRelayToReleaseList if applicable;

The L2 U2U Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemoteUE value included in the sl-U2U-RemoteUE-ToAddModList that is not part of the current UE configuration (target L2 U2U Remote UE Addition):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity in accordance with one entry of the sl-SourceRemoteUE-ToAddModList:

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemote value included in the sl-U2U-RemoteUE-ToAddModList that is part of the current UE configuration (target L2 U2U Remote UE modification):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToReleaseList (source L2 U2U Remote UE Release):

3>	release the configuration associated with the source L2 U2U Remote UE;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is not part of the current UE configuration (source L2 U2U Remote UE Addition):

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is part of the current UE configuration (source L2 U2U Remote UE modification):

3>	modify the configuration in accordance with the sl-SRAP-ConfigU2U;

#### 5.3.5.16	L2 U2N or U2U Remote UE configuration

The network configures the L2 U2N or U2U Remote UE with relay operation related configurations, e.g. SRAP configuration.

The L2 U2N Remote UE shall:

1>	if sl-L2RemoteUE-Config is set to setup or received from RRCSetup message:

2>	if the sl-L2RemoteUE-Config contains the sl-SRAP-ConfigRemote:

3>	if no SRAP entity has been established:

4>	establish a SRAP entity as specified in TS 38.351 [66];

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRemote;

3>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

4>	release SL-RLC1, if established;

4> associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

3>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

4>	if SL-RLC1 is not established:

5>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

2>	if the sl-L2RemoteUE-Config contains the sl-UEIdentityRemote:

3>	use the value of the sl-UEIdentityRemote as the C-RNTI in the PCell.

1>	else if sl-L2RemoteUE-Config is set to release:

2>	release the L2 U2N relay operation related configurations.

The L2 U2U Remote UE shall:

1>	if sl-L2RemoteUE-Config is set to setup:

2>	if the sl-L2RemoteUE-Config contains the sl-U2U-RelayUE-ToReleaseList:

3>	perform the L2 U2U Relay UE release as specified in 5.3.5.16.1;

2>	if the sl-L2RemoteUE-Config contains the sl-U2U-RelayUE-ToAddModList:

3>	perform the L2 U2U Relay UE addition/modification as specified in 5.3.5.16.2;

1>	else if sl-L2RemoteUE-Config is set to release:

2>	release the L2 U2U relay operation related configurations.

##### 5.3.5.16.1	L2 U2U Relay UE Release

The L2 U2U Remote UE shall:

1>	if the release is triggered by reception of the sl-U2U-RelayUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-U2U-RelayUE-ToReleaseList:

3>	release the configuration associated with the L2 U2U Relay UE.

##### 5.3.5.16.2	L2 U2U Relay UE Addition/Modification

The L2 U2U Remote UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each L2 U2U Relay UE indicated in sl-L2IdentityRelay value included in the sl-U2U-RelayUE-ToAddModList that is not part of the current UE configuration (L2 U2U Relay UE Addition):

2>	for target L2 U2U Remote UE indicated in sl-TargetUE-Identity in accordance with one entry of the SL-PeerRemoteUE-ToAddModList:

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

1>	for each L2 U2U Relay UE indicated in sl-L2IdentityRelay value included in the sl-U2U-RelayUE-ToAddModList that is part of the current UE configuration (L2 U2U Relay UE modification):

2>	for each SL-DestinationIdentity value included in the sl-TargetRemoteUE-ToReleaseList (target L2 U2U Remote UE Release):

3>	release the configuration associated with the peer target L2 U2U Remote UE;

2>	for the target L2 U2U Remote UE indicated in sl-TargetUE-Identity included in the sl-TargetRemoteUE-ToAddModList that is not part of the current UE configuration (target L2 U2U Remote UE Addition):

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

2>	for the target L2 U2U Remote UE indicated in sl-TargetUE-Identity included in the sl-TargetRemoteUE-ToAddModList that is part of the current UE configuration (target L2 U2U Remote UE modification):

3>	modify the configuration in accordance with the sl-SRAP-ConfigU2U;

#### 5.3.5.17	MP configuration

##### 5.3.5.17.1	Introduction

In case of MP, a MP remote UE is configured with one direct path (i.e. MCG) and one indirect path. The MP remote UE connects to network on the direct path using NR Uu, and connects to network on the indirect path via a MP relay UE.

On indirect path, for the hop between the MP remote UE and the MP relay UE, when the MP remote UE connects to a L2 U2N Relay UE, i.e. the MP remote UE is acting as a L2 U2N Remote UE, the network provides the configuration of SL indirect path to the MP remote UE and the L2 U2N Relay UE as specified in 5.3.5.17.2; when the MP remote UE connects to one or more N3C relay UE(s) using non-3GPP connection, the network provides the configuration of N3C indirect path to the MP remote UE and the N3C relay UE as specified in 5.3.5.17.3.

##### 5.3.5.17.2	Configuration of SL indirect path

###### 5.3.5.17.2.1	General

For SL indirect path:

-	the L2 U2N Remote UE is provided with sidelink dedicated configuration as specified in 5.3.5.14, L2 U2N Remote UE configuration as specified in 5.3.5.16, and SL indirect path specific configuration as specified in 5.3.5.17.2.2;

-	the L2 U2N Relay UE is provided with sidelink dedicated configuration as specified in 5.3.5.14, L2 U2N Relay UE configuration as specified in 5.3.5.15, as well as Uu Relay RLC channel configuration as specified in 5.3.5.5.12 and 5.3.5.5.13.

###### 5.3.5.17.2.2	SL indirect path specific configuration

The L2 U2N Remote UE shall:

1>	if sl-IndirectPathAddChange is set to setup:

2>	if the sl-IndirectPathMaintain is included in reconfigurationWithSync:

3>	consider the source L2 U2N Relay UE to be the L2 U2N Relay UE on indirect path in MP operation;

2>	else:

3>	consider the UE indicated by the sl-IndirectPathRelayUE-Identity to be the (target) L2 U2N Relay UE and indicate to upper layer to trigger the PC5 unicast link establishment with the L2 U2N Relay UE;

3>	start timer T421 for the corresponding L2 U2N Relay UE with the timer value set to T421;

3>	if reconfigurationWithSync is not included in the same RRCReconfiguration message:

4>	indicate to upper layer (to trigger the PC5 unicast link release) with the source L2 U2N Relay UE in case of SL indirect path change (i.e. a new L2 U2N Relay UE is indicated via sl-IndirectPathRelayUE-Identity);

1>	else if sl-IndirectPathAddChange is set to release:

2>	consider the SL indirect path is released and release the corresponding configurations;

2>	indicate to upper layer (to trigger the PC5 unicast link release) with the L2 U2N Relay UE.

NOTE:	The MP direct path addition is realized by indirect-to-direct path switch procedure (i.e. ReconfigurationWithSync included in RRCReconfiguration message), where sl-IndirectPathAddChange is set to setup in RRCReconfiguration message.

###### 5.3.5.17.2.3	T421 expiry (Indirect path addition/change failure)

The L2 U2N Remote UE shall:

1> if T421 expires; or

1> if the (target) L2 U2N Relay UE (i.e., the UE indicated by sl-IndirectPathRelayUE-Identity in the received sl-IndirectPathAddChange) changes its serving cell to a different cell from the target cell (i.e. the cell indicated by sl-IndirectPathCellIdentity in the received sl-IndirectPathAddChange) during path addition or change:

2>	if MCG transmission is not suspended:

3>	initiate the indirect path failure information procedure as specified in clause 5.7.3c to report SL indirect path addition/change failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

##### 5.3.5.17.3	Configuration of N3C indirect path

###### 5.3.5.17.3.1	General

For N3C indirect path,

-	the N3C remote UE is provided with N3C indirect path configuration including relay UE identification as specified in 5.3.5.17.3.2;

-	the N3C relay UE is provided with N3C indirect path configuration including bearer mapping configurations as specified in 5.3.5.17.3.4, as well as Uu Relay RLC channel configuration as specified in 5.3.5.5.12 and 5.3.5.5.13.

NOTE: The data transmission/reception between the N3C remote UE and the N3C relay UE via the non-3GPP connection is outside the scope of 3GPP.

###### 5.3.5.17.3.2	N3C remote UE configuration

The N3C remote UE shall:

1>	if n3c-IndirectPathAddChange is set to setup:

2>	consider the non-3GPP connection with the relay UE indicated by the n3c-RelayIdentification to be used for the N3C indirect path;

2>	consider the source non-3GPP connection is not to be used in case of N3C indirect path change (i.e. a new relay UE is indicated by the n3c-RelayIdentification);

1>	else if n3c-IndirectPathAddChange is set to release:

2>	consider the indirect path is not to be used and release the corresponding configuration:

1>	if n3c-ExtIndirectPathAddChange is set to setup:

2>	consider the configured N3C relay UE(s) is(are) released;

2>	consider the relay UE(s) indicated by the n3c-ExtIndirectPathAddChange to be used for the N3C indirect path;

1>	if n3c-ExtIndirectPathAddChange is set to release:

2>	consider the indirect path is not to be used and release the corresponding configuration.

###### 5.3.5.17.3.2a	N3C Indirect path addition/change failure

The N3C remote UE shall:

1> if N3C Indirect path addition/change failure is detected:

2>	if MCG transmission is not suspended:

3>	initiate the indirect path failure information procedure as specified in clause 5.7.3c to report N3C indirect path addition/change failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

NOTE:	How to detect N3C Indirect path addition/change failure is up to UE implementation.

###### 5.3.5.17.3.3	N3C relay UE configuration

The N3C relay UE shall:

1>	if n3c-IndirectPathConfigRelay is set to setup:

2>	if n3c-MappingToReleaseList is included:

3>	perform N3C bearer mapping release as specified in 5.3.5.17.3.4.1;

2>	if n3c-MappingToAddModList is included:

3>	perform N3C bearer mapping addition/modification as specified in 5.3.5.17.3.4.2;

1>	else if n3c-IndirectPathConfigRelay is set to release:

2>	consider the indirect path with the remote UE is released and release the corresponding configuration.

###### 5.3.5.17.3.4	Bearer mapping management on N3C indirect path

5.3.5.17.3.4.1	Bearer mapping release

The UE shall:

1>	if the release is triggered by reception of n3c-IndirectPathConfigRelay set to release:

2>	for all configured SL-RemoteUE-RB-Identity value:

3>	release n3c-RLC-ChannelUu and n3c-RemoteUE-RB-Identity;

1>	else if the release is triggered by reception of the n3c-MappingToReleaseList:

2>	for each SL-RemoteUE-RB-Identity value included in n3c-MappingToReleaseList:

3>	release the corresponding n3c-RLC-ChannelUu and n3c-RemoteUE-RB-Identity;

5.3.5.17.3.4.2	Bearer mapping addition and modification

The UE shall:

1>	for each n3c-RemoteUE-RB-Identity value included in the n3c-MappingToAddModList that is not part of the current UE configuration:

2>	associate the remote UE's RB indicated in n3c-RemoteUE-RB-Identity to the Uu Relay RLC channel indicated in n3c-RLC-ChannelUu;

1>	for the n3c-RemoteUE-RB-Identity value included in the n3c-MappingToAddModList that is part of the current UE configuration:

2>	update the associated Uu Relay RLC channel to the one indicated in n3c-RLC-ChannelUu.

#### 5.3.5.18	LTM configuration and execution

##### 5.3.5.18.1	LTM configuration

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE.

An ltm-Config included within an RRCReconfiguration message received via SRB1 is for LTM on the MCG. It may include an SCG configuration and/or ltm-ServingCellNoSecurityChangeID.

An ltm-Config included within an RRCReconfiguration message either received via SRB3, or embedded in an RRCReconfiguration message received via SRB1 is for LTM on the SCG. It does not include any MCG configuration and does not include ltm-ServingCellNoSecurityChangeID.

An ltm-ConfigNRDC included within an RRCReconfiguration message received via SRB1 is for LTM on the SCG. It includes the MCG configuration and may include ltm-ServingCellNoSecurityChangeID.

In NR-DC, the UE may be configured simultaneously with an ltm-Config for MCG LTM, and an ltm-Config for SCG LTM, or be configured simultaneously with an ltm-Config for MCG LTM, and an ltm-ConfigNRDC for SCG LTM.

In this case, the following principles apply:

-	the UE maintains independently the two ltm-Config, or the ltm-Config and the ltm-ConfigNRDC;

-	the UE maintains two independent VarLTM-ServingCellNoResetID, one associated with each ltm-Config, or one associated with the ltm-Config and one associated with the ltm-ConfigNRDC;

-	the UE maintains two independent VarLTM-ServingCellUE-MeasuredTA-ID, one associated with each ltm-Config, or one associated with the ltm-Config and one associated with the ltm-ConfigNRDC;

-	the UE independently performs all the procedures in clause 5.3.5.18 for each ltm-Config, or an ltm-Config and an ltm-ConfigNRDC, and the associated VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange, unless explicitly stated otherwise.

The UE shall perform the following actions based on the received LTM-Config IE:

1>	if the received LTM-Config includes ltm-ServingCellNoResetID:

2>	if the current VarLTM-ServingCellNoResetID includes an ltm-ServingCellNoResetID:

3>	replace the ltm-ServingCellNoResetID value within VarLTM-ServingCellNoResetID with the received ltm-ServingCellNoResetID;

2>	else:

3>	store the received ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID;

1>	if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID:

2>	if the current VarLTM-ServingCellUE-MeasuredTA-ID includes an ltm-ServingCellUE-MeasuredTA-ID:

3>	replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID;

2>	else:

3>	store the received ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID;

1>	if the received LTM-Config includes ltm-ServingCellNoSecurityChangeID:

2>	if the current VarLTM-ServingCellNoSecurityChange includes an ltm-ServingCellNoSecurityChangeID:

3>	replace the ltm-ServingCellNoSecurityChangeID value within VarLTM-ServingCellNoSecurityChange with the received ltm-ServingCellNoSecurityChangeID;

2>	else:

3>	store the received ltm-ServingCellNoSecurityChangeID in VarLTM-ServingCellNoSecurityChange;

1>	if the received LTM-Config includes the ltm-CandidateToReleaseList:

2>	perform the LTM candidate configuration release as specified in 5.3.5.18.2;

1>	if the received LTM-Config includes the ltm-CandidateToAddModList:

2>	perform the LTM candidate configuration addition or modification as specified in 5.3.5.18.3;

1>	reconfigure the UE according to all other fields of the received LTM-Config IE;

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup:

2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

3>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

4>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

3>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

4>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

3>	if l3-Conditions is included within ltm-ServingCellExecutionCondition:

4>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ServingCellExecutionCondition;

3>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

4>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

1>	else (ltm-ServingCellExecutionCondition set to release):

2>	release the ltm-ServingCellExecutionCondition.

##### 5.3.5.18.2	LTM candidate configuration release

The UE shall:

1>	for each ltm-CandidateId value included in the ltm-CandidateToReleaseList that is part of the current UE configuration:

2>	remove the corresponding LTM-Candidate.

##### 5.3.5.18.3	LTM candidate configuration addition/modification

The UE shall:

1>	for each ltm-CandidateId value included in the ltm-CandidateToAddModList:

2>	if the current UE configuration contains an LTM-Candidate with the ltm-CandidateId value:

3>	reconfigure the corresponding LTM-Candidate in accordance with the received LTM-Candidate;

2>	else:

3>	add the received LTM-Candidate;

2>	if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID:

3>	if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

4>	inform lower layers that the UE is configured with UE-based TA measurements for this LTM-Candidate;

3>	else:

4>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate;

2>	else:

3>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

##### 5.3.5.18.4	Void

##### 5.3.5.18.5	Void

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2>	if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

1>	if the LTM cell switch is triggered on the MCG; or

1>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange;

-	the ltm-Config and ltm-ConfigNRDC (if configured);

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

-	the successHO-Config;

3>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

-	the ServingCellConfigCommon of the PCell;

1>	if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group(s) for which the RRCReconfiguration message is applied due to the triggered LTM cell switch procedure, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group(s) for which the RRCReconfiguration message is applied due to the triggered LTM cell switch procedure;

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	if the LTM cell switch is triggered on the MCG:

3>	update the master security key by performing the AS security key update procedure as specified in 5.3.5.7;

2>	else if the LTM cell switch is triggered on the SCG:

3>	consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange associated to the the field ltm-NoSecurityChangeID as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7;

3>	remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange;

2>	at the end of the procedure, for each drb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or

3>	if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KgNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

4>	if drb-ContinueROHC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueROHC is configured;

4>	if drb-ContinueEHC-DL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-DL is configured;

4>	if drb-ContinueEHC-UL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-UL is configured;

4>	if drb-ContinueUDC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueUDC is configured;

4>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

3>	else if LTM cell switch is triggered on the SCG and this DRB is using the master key:

4>	if the RLC entity of an RLC bearer associated with this DRB is re-established or released during LTM cell switch execution:

5>	if this DRB is an AM DRB:

6>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	at the end of the procedure, for each srb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and the SRB is using the secondary key:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

2>	if the value of field ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange:

3>	replace the value of ltm-ServingCellNoSecurityChangeID in VarLTM-ServingCellNoSecurityChange with the value of ltm-NoSecurityChangeID in the LTM-Candidate in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1>	else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

1>	if the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC that includes the LTM-Candidate IE indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config or ltm-ConfigNRDC, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config or ltm-ConfigNRDC;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config or ltm-ConfigNRDC as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.8)):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.8 or 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	if ltm-ExecutionCondition is configured within the LTM-Candidate IE for the selected LTM candidate configuration:

2>	if the field l3-Conditions is included within ltm-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the received ltm-ExecutionCondition once this procedure is completed;

2>	else if the field l1-Conditions is included within ltm-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ExecutionCondition once this procedure is completed.

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

##### 5.3.5.18.7	LTM configuration release

For the cell group for which the LTM configuration release procedure is triggered, the UE shall:

1>	remove the entry within VarLTM-ServingCellNoResetID;

1>	remove the entry within VarLTM-ServingCellUE-MeasuredTA-ID;

1> remove the entries within VarLTM-ServingCellNoSecurityChange;

1>	if the release of the LTM configuration is for the SCG:

2>	if there is an ltm-Config associated with the SCG;

3>	release the ltm-Config associated with the SCG;

2>	else if there is an ltm-ConfigNRDC:

3>	release the ltm-ConfigNRDC:

1>else:

2>	release ltm-Config associated to the MCG.

##### 5.3.5.18.8	LTM cell switch conditions evalution based on L3 measurements

The UE shall:

1>	for each entry within the LTM-ExecutionConditionList which has the l3-Conditions configured:

2>	for each measId indicated in the l3-Conditions which has a corresponding measId in the VarMeasConfig associated with the MCG measConfig:

3>	if the condEventId related to this measId is associated with condEventA3 or condEventA5, and if the entry condition applicable for this event is fulfilled for the ltm-CandidatePCI related to the ltm-CandidateId for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event:

4>	consider the event associated to this measId to be fulfilled for the ltm-CandidateId associated to the measId;

3>	if the condEventId related to this measId is associated with condEventA3 or condEventA5, and if the leaving condition applicable for this event is fulfilled for the ltm-CandidatePCI related to the ltm-CandidateId for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event:

4>	consider the event associated to this measId to be not fulfilled for the ltm-CandidateId associated to the measId;

1>	if event(s) associated with all measId(s) for an ltm-CandidateId within the LTM-ExecutionConditionList IE are fulfilled:

2> inform lower layers that an event based on L3 measurements to perform an LTM cell switch procedure is fulfilled;

2>	perform the LTM cell switch procedure for the LTM candidate configuration associated to the ltm-CandidateId according to the actions specified in 5.3.5.18.6.

##### 5.3.5.18.9	LTM sk-Counter configuration addition/modification

The UE shall:

1>	for each ltm-NoSecurityChangeID received in ltm-SK-CounterConfigToAddModList:

2>	if an entry with the matching ltm-NoSecurityChangeID exists in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange:

3>	replace the sk-CounterList within the VarLTM-ServingCellNoSecurityChange with the sk-CounterList according to the received ltm-NoSecurityChangeID;

2>	else:

3>	add a new entry for this ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange.

##### 5.3.5.18.10	LTM sk-Counter configuration release

The UE shall:

1>	for each ltm-NoSecurityChangeID value included in the ltm-SK-CounterConfigToRemoveList that is part of the current ltm-SK-Counters in VarLTM-ServingCellNoSecurityChange:

2>	remove the entry with the matching ltm-NoSecurityChangeID from the ltm-SK-Counters in VarLTM-ServingCellNoSecurityChange.

#### 5.3.5.19	T348 expiry

The UE can apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction, as specified in 5.7.4.3.

### 5.3.6	Counter check

#### 5.3.6.1	General

Figure 5.3.6.1-1: Counter check procedure

The counter check procedure is used by the network to request the UE to verify the amount of data sent/ received on each DRB. More specifically, the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by the network.

NOTE:	The procedure enables the network to detect packet insertion by an intruder (a 'man in the middle').

#### 5.3.6.2	Initiation

The network initiates the procedure by sending a CounterCheck message.

NOTE:	The network may initiate the procedure when any of the COUNT values reaches a specific value.

#### 5.3.6.3	Reception of the CounterCheck message by the UE

Upon receiving the CounterCheck message, the UE shall:

1>	for each DRB that is established:

2>	if no COUNT exists for a given direction (uplink or downlink) because it is a uni-directional bearer configured only for the other direction:

3>	assume the COUNT value to be 0 for the unused direction;

2>	if the drb-Identity is not included in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

2>	else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

1>	for each DRB that is included in the drb-CountMSB-InfoList in the CounterCheck message that is not established:

2>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero;

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends.

### 5.3.7	RRC connection re-establishment

#### 5.3.7.1	General

Figure 5.3.7.1-1: RRC connection re-establishment, successful

Figure 5.3.7.1-2: RRC re-establishment, fallback to RRC establishment, successful

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB/multicast MRB setup or, for IAB and NCR, SRB2, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.

The network applies the procedure e.g as follows:

-	When AS security has been activated and the network retrieves or verifies the UE context:

-	to re-activate AS security without changing algorithms;

-	to re-establish and resume the SRB1;

-	When UE is re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context:

-	to discard the stored AS Context and release all RBs and BH RLC channels and Uu Relay RLC channels;

-	to fallback to establish a new RRC connection.

If AS security has not been activated, the UE shall not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'other'. If AS security has been activated, but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, are not setup, the UE does not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'RRC connection failure'.

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with clause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] clause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with clause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with clause 5.7.3b.5; or

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP, in accordance with clause 5.8.9.3; or

1>	upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP or by L2 Intermediate U2N Relay UE in RRC_CONNECTED, in accordance with clause 5.8.9.10; or

1>	upon PC5 unicast link release for the serving L2 U2N Relay UE indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP while T301 is not running; or

1>	if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with clause 5.3.10 while the transmission of indirect path is suspended as specified in 5.3.5.17; or

1>	if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with 5.3.10 while MP indirect path addition or change is ongoing; or

1>	if MP is configured, upon detecting sidelink radio link failure of SL indirect path by L2 U2N Remote UE, in accordance with clause 5.8.9.3, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon reception of NotificationMessageSidelink including indicationType in accordance with clause 5.8.9.10, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon detecting the failure of N3C indirect path of all relay UEs by N3C remote UE in accordance with clause 5.7.3c, while MCG transmission (i.e. direct path) is suspended.

NOTE 0:	It is up to UE implementation whether to initiate the procedure while T346g is running.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	stop timer T421, if running;

1>	if UE is not configured with attemptCondReconfig; and

1>	if UE is not configured with attemptLTM-Switch:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE or for L2 Last U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-GapPriorityAssistanceConfig, if configured;

2>	release musim-LeaveAssistanceConfig, if configured;

2>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

2>	release maxBW-PreferenceConfigFR2-2, if configured;

2>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

2>	release minSchedulingOffsetPreferenceConfigExt, if configured;

2>	release multiRx-PreferenceReportingConfigFR2, if configured, and stop timer T346m, if running;

2>	release aerial-FlightPathAvailabilityConfig, if configured;

2>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

2>	release gapOccasionCancelRatioReportConfig, if configured, and stop timer T346o, if running;

2>	release lpwus-OffsetPreferenceConfig, if configured, and stop timer T346p, if running;

2>	release loggedDataCollectionAssistanceConfig, if configured;

2>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

2>	release applicabilityReportConfig, if configured;

2>	release dataCollectionPreferenceConfig, if configured;

2>	release assisted-SSB-MTC-Config, if configured;1>	release successHO-Config, if configured;

1>	release successPSCell-Config configured by the PCell, if configured;

1>	if any DAPS bearer is configured:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	release sl-L2RelayUE-Config for L2 U2N relay operation, if configured;

1>	release sl-L2RemoteUE-Config for L2 U2N relay operation, if configured;

1>	release the SRAP entity for L2 U2N relay operation, if configured;

1>	release ncr-FwdConfig, if configured;

1>	if the UE is NCR-MT:

2>	indicate to NCR-Fwd to cease forwarding;

1>	if SL indirect path is configured:

2>	release cell identity and relay UE ID configured in sl-IndirectPathAddChange;

2>	indicate upper layers to trigger PC5 unicast link release of the SL indirect path;

1>	if N3C indirect path is configured:

2>	release n3c-IndirectPathAddChange, n3c-ExtIndirectPathAddChange;

2> consider the non-3GPP connection is not used;

1>	if the UE is acting as a N3C relay UE:

2>	release n3c-IndirectPathConfigRelay;

2> consider the non-3GPP connection is not used;

1>	if the UE is acting as L2 U2N Remote UE and MP via L2 U2N Relay UE is not configured or is acting as L2 Intermediate U2N Relay UE:

2>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

3>	indicate upper layers to trigger PC5 unicast link release;

3>	perform either cell selection in accordance with the cell selection process as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else (i.e., maintain the PC5 RRC connection):

3>	consider the connected L2 U2N Relay UE as suitable and perform actions as specified in clause 5.3.7.3a;

NOTE 1:	It is up to Remote UE implementation whether to release or keep the current PC5 unicast link.

1> else:

2>	if the UE is capable of L2 U2N Remote UE or L2 Intermediate U2N Relay UE:

3>	perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

NOTE 2:	For L2 U2N Remote UE or L2 Intermediate U2N Relay UE, if both a suitable cell and a suitable relay are available, the UE can select either one based on its implementation.

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the relay (re)selection procedure, if ongoing;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers or fulfilment of LTM cell switch execution conditions as specified in clause 5.3.5.18.6; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG; and

1>	if at least one of the following conditions is fulfilled:

2>	the selected cell does not have the field ltm-NoSecurityChangeID configured and the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange; or

2>	the cell selection is triggered by detecting radio link failure of the MCG and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange; or

2>	the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.8 or 5.3.5.18.6 and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected:

3>	if the UE supports RLF-Report for MCG LTM cell switch, set the ltm-RecoveryCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

Editor’s note:	It need to be verified that above SON/MDT statement is in correct position.

3>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

1>	else:

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-GapPriorityAssistanceConfig, if configured;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	release multiRx-PreferenceReportingConfigFR2, if configured, and stop timer T346m, if running;

3>	release aerial-FlightPathAvailabilityConfig, if configured;

3>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

3>	release gapOccasionCancelRatioReportConfig, if configured, and stop timer T346o, if running;

3>	release loggedDataCollectionAssistanceConfig, if configured;

3>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

3>	release applicabilityReportConfig, if configured;

3>	release dataCollectionPreferenceConfig, if configured;

3>	release assisted-SSB-MTC-Config, if configured;3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0 and broadcast MRBs;

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

2>	release the PC5 RLC entity for SL-RLC0, if any;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2a:	This procedure applies also if the UE returns to the source PCell.

NOTE 3:	A L2 U2N Relay UE may re-establish (e.g. via release and establish) the SL-RLC0 and SL-RLC1 of the connected L2 U2N Remote UE(s) or child UE(s).

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.3a	Actions following relay selection while T311 is running

Upon selecting a suitable L2 U2N Relay UE, the L2 U2N Remote UE shall:

1>	indicate to upper layer to trigger the PC5 unicast link establishment with the selected L2 U2N Relay UE, if a new L2 U2N Relay UE is selected;

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the cell selection procedure, if ongoing;

1>	start timer T301;

1>	release the RLC entity for SRB0, if any;

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

1> apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4.

#### 5.3.7.4	Actions related to transmission of RRCReestablishmentRequest message

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or reconfiguration with sync failure as specified in 5.3.5.8.3; or

1>	if the procedure was initiated due to mobility from NR failure as specified in 5.4.3.5 and if voiceFallbackIndication is included in the MobilityFromNRCommand message:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source PCell (reconfiguration with sync or mobility from NR failure) or used in the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.8.2:

3>	set the reestablishmentCause to the value reconfigurationFailure;

2>	else if the re-establishment procedure was initiated due to reconfiguration with sync failure as specified in 5.3.5.8.3 (intra-NR handover failure) or 5.4.3.5 (inter-RAT mobility from NR failure):

3>	set the reestablishmentCause to the value handoverFailure;

2>	else:

3>	set the reestablishmentCause to the value otherFailure;

1>	re-establish PDCP for SRB1;

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish or re-establish (e.g. via release and add) SL RLC entity for SRB1;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default configuration of PDCP as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

1> else:

2>	re-establish RLC for SRB1;

2>	apply the default configuration defined in 9.2.1 for SRB1;

1>	configure lower layers to suspend integrity protection and ciphering for SRB1;

NOTE:	Ciphering is not applied for the subsequent RRCReestablishment message used to resume the connection. An integrity check is performed by lower layers, but merely upon request from RRC.

1>	resume SRB1;

1>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

1>	submit the RRCReestablishmentRequest message to lower layers for transmission.

#### 5.3.7.5	Reception of the RRCReestablishment by the UE

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	update the KgNB key based on the current KgNB key or the NH, using the received nextHopChainingCount value, as specified in TS 33.501 [11];

1>	store the nextHopChainingCount value indicated in the RRCReestablishment message;

1>	derive the KRRCenc and KUPenc keys associated with the previously configured cipheringAlgorithm, as specified in TS 33.501 [11];

1>	derive the KRRCint and KUPint keys associated with the previously configured integrityProtAlgorithm, as specified in TS 33.501 [11].

1>	request lower layers to verify the integrity protection of the RRCReestablishment message, using the previously configured algorithm and the KRRCint key;

1>	if the integrity protection check of the RRCReestablishment message fails:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure', upon which the procedure ends;

1>	configure lower layers to resume integrity protection for SRB1 using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to resume ciphering for SRB1 using the previously configured algorithm and, the KRRCenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	release the measurement gap configuration indicated by the measGapConfig, if configured;

1>	release the MUSIM gap configuration indicated by the musim-GapConfig, if configured;

1>	release the FR2 UL gap configuration indicated by the ul-GapFR2-Config, if configured;

1>	perform the L2 U2N Remote UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

2>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCReestablishmentComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCReestablishmentComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included; or

2>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

3>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

4>	set sigLogMeasConfigAvailable to true in the RRCReestablishmentComplete message;

3>	else:

4>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

5>	set sigLogMeasConfigAvailable to false in the RRCReestablishmentComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

3>	include successPSCell-InfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured which has not been successfully transmitted since entering RRC_CONNECTED state:

3>	include measConfigReportAppLayerAvailable in the RRCReestablishmentComplete message;

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCReestablishmentComplete message upon determining it has temporary capability restriction;

NOTE:	Upon reception of musim-CapRestrictionInd in RRCReestablishmentComplete, it is up to network implementation to configure the UE with a limited configuration that is used until network sends RRCReconfiguration based on the actual restricted UE capabilities included in UEAssistanceInformation.

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	if SIB21 is provided by the PCell; or

1>	if nonServingCellMII is provided in SIB1 by the PCell:

2>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding detection of radio link failure:

3>	initiate transmission of an MBSInterestIndication message in accordance with 5.9.4;

1>	the procedure ends.

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	if the procedure was initiated due to radio link failure or handover failure:

2>	set the noSuitableCellFound in the VarRLF-Report to true;

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.7	T301 expiry or selected cell/L2 U2N Relay UE no longer suitable

The UE shall:

1>	if timer T301 expires; or

1>	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in TS 38.304 [20]; or

1>	if the (re)selected L2 U2N Relay UE becomes unsuitable; or

1>	upon reception of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection; or

1>	upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.8	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection establishment procedure as specified in 5.3.3.4.

### 5.3.8	RRC connection release

#### 5.3.8.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	to release the RRC connection, which includes the release of the established radio bearers (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels as well as all radio resources; or

-	to suspend the RRC connection only if SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, are setup, which includes the suspension of the established radio bearers (except for broadcast MRBs).

#### 5.3.8.2	Initiation

The network initiates the RRC connection release procedure to transit a UE in RRC_CONNECTED to RRC_IDLE; or to transit a UE in RRC_CONNECTED to RRC_INACTIVE only if SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, is setup in RRC_CONNECTED; or to transit a UE in RRC_INACTIVE back to RRC_INACTIVE when the UE tries to resume (for resuming a suspended RRC connection, for RNA Update or for initiating SDT); or to transit a UE in RRC_INACTIVE to RRC_IDLE when the UE tries to resume (for resuming of a suspended RRC connection, for RNA Update or for initiating SDT). The procedure can also be used to release and redirect a UE to another frequency.

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

NOTE 0:	When the RRCRelease message is received on a HARQ process with disabled HARQ feedback, and when STATUS reporting, as defined in TS 38.322 [4], has not been triggered for a logical channel associated with the SRB1, the lower layers can be considered to have indicated that the receipt of the RRCRelease message has been successfully acknowledged.

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if timer T316 is running;

2>	stop timer T316;

2>	if the UE supports RLF-Report for fast MCG recovery procedure as specified in 38.306 [26]:

3>	set the elapsedTimeT316 in the VarRLF-Report to the value of the elapsed time of the timer T316;

3>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

2>	else:

3>	clear the information included in VarRLF-Report, if any;

1>	stop timer T350, if running;

1>	stop timer T346g, if running;

1>	stop timer T348, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

2>	if voiceFallbackIndication is included:

3>	consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]);

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry/stop;

NOTE 1a:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise.

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	if the measIdleConfig contains validityAreaList:

4>	store the received validityAreaList in VarMeasIdleConfig;

3>	if the measIdleConfig contains measReselectionCarrierListNR:

4>	store the received measReselectionCarrierListNR in VarMeasReselectionConfig;

3>	if the measIdleConfig contains measReselectionValidityDuration:

4>	store the received measReselectionValidityDuration in VarMeasReselectionConfig;

3>	if the measIdleConfig contains measIdleValidityDuration:

4>	store the received measIdleValidityDuration in VarEnhMeasIdleConfig;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer (except those associated with broadcast MRBs and multicast MRBs) that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

2>	if srs-PosRRC-Inactive is configured:

3>	apply the SRS for positioning configuration in RRC_INACTIVE and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer;

2>	if srs-PosRRC-InactiveValidityAreaNonPreConfig is set to setup:

3>	apply the SRS for positioning configuration in RRC_INACTIVE and instruct MAC to start the inactivePosSRS-ValidityAreaTAT;

2>	else if srs-PosRRC-InactiveValidityAreaNonPreConfig is set to release:

3>	release srs-PosRRC-InactiveValidityAreaNonPreConfig, if available;

2>	if srs-PosRRC-InactiveValidityAreaPreConfigList is set to setup:

3>	store srs-PosRRC-InactiveValidityAreaPreConfigList and perform actions as specified in clause 5.7.20;

2>	else if srs-PosRRC-InactiveValidityAreaPreConfigList is set to release:

3>	remove all srs-PosRRC-InactiveValidityAreaPreConfigList, if available;

NOTE 1b:	The Network should provide full configuration to UE for SRS for Positioning in RRC_INACTIVE.

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

2>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

2>	for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if the UE is NCR-MT and if ncr-FwdConfig is configured:

3>	if the ncr-FwdConfig includes periodic forwarding resource configuration:

4>	indicate to NCR-Fwd to continue forwarding only in accordance with the configured periodic forwarding resource set(s);

3>	else:

4>	indicate to NCR-Fwd to cease forwarding;

2>	if the UE is acting as L2 U2N Remote UE and is not configured with MP:

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release;

3>	else (i.e., maintain the PC5 RRC connection):

4>	establish or re-establish (e.g. via release and add) SL RLC entity for SRB1;

2>	else:

3>	re-establish RLC entities for SRB1;

2>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the RAN visible application layer measurement configuration;

3>	discard any RAN visible application layer measurement reports received from upper layers;

3>	initiate the procedure in 5.5b.1.2;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

-	aerial-Config, if configured;

-	cellDTX-DRX-Config, if configured;

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement report containers for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers;

NOTE 2:	NR sidelink communication/discovery/positioning related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s), except SRB0 and broadcast MRBs;

2>	suspend all multicast MRB(s) associated with multicast session(s) not configured for reception in RRC_INACTIVE;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs associated with multicast session(s) not configured for reception in RRC_INACTIVE;

2>	release Uu Relay RLC channel(s), if configured;

2>	release PC5 Relay RLC channel(s), if configured;

2>	release the SRAP entity, if configured;

NOTE 2a:	A L2 U2N Relay UE may re-establish the SL-RLC0, SL-RLC1 and SRAP entity after release.

2>	if SL indirect path is configured:

3>	release cell identity and relay UE ID configured in sl-IndirectPathAddChange;

3>	indicate upper layers to trigger PC5 unicast link release of the SL indirect path;

2>	if N3C indirect path is configured:

3>	release n3c-IndirectPathAddChange;

3>	consider the non-3GPP connection is not used;

2>	if the UE is acting as a N3C relay UE:

3>	release n3c-IndirectPathConfigRelay;

3>	consider the non-3GPP connection is not used;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

2>	if the suspendConfig includes resumeIndication:

3> perform the actions as if the UE received Paging message with the ue-Identity included in the PagingRecord matching the UE's stored fullI-RNTI, as specified in clause 5.3.2.3;

2>	if the multicastConfigInactive is set to setup:

3>	if the multicast PTM configuration is provided for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI and the UE selects the same cell as the one on which the multicast session was configured in RRC_CONNECTED:

4>	apply the multicast PTM configuration as specified in 5.10.3;

4>	if multicast MCCH is present:

5>	monitor the Multicast MCCH-RNTI as specified in 5.10.1.2;

2>	release CSI-LoggedMeasurementConfig, if configured;

2>	release loggedDataCollectionAssistanceConfig, if configured;

2>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

NOTE 3:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 4:	It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

#### 5.3.8.4	T320 expiry

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information.

#### 5.3.8.5	UE actions upon the expiry of DataInactivityTimer

Upon receiving the expiry of DataInactivityTimer from lower layers while in RRC_CONNECTED, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.8.6	T346g expiry

The UE shall:

1>	if T346g expires:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.9	RRC connection release requested by upper layers

#### 5.3.9.1	General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

#### 5.3.9.2	Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection as specified in TS 24.501 [23]. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>	if the upper layers indicate barring of the PCell:

2>	treat the PCell used prior to entering RRC_IDLE as barred according to TS 38.304 [20];

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.10	Radio link failure related actions

#### 5.3.10.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	if any DAPS bearer is configured, upon receiving N310 consecutive "out-of-sync" indications for the source SpCell from lower layers and T304 is running:

2>	start timer T310 for the source SpCell.

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while neither T300, T301, T304, T311, T316 nor T319 are running:

2>	start timer T310 for the corresponding SpCell.

#### 5.3.10.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell.

1>	stop timer T312 for the corresponding SpCell, if running.

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by L1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

2>	upon T312 expiry in PCell; or

2>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running and SDT procedure is not ongoing; or

2>	upon indication from MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is not ongoing; or

2>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG; or

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

3>	if the indication is from MCG RLC and CA duplication is configured and activated for MCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

4>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

3>	else:

4>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

4>	release CSI-LoggedMeasurementConfig, if configured;

4>	release loggedDataCollectionAssistanceConfig, if configured;

4>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

NOTE 1:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	if MP is configured:

6>	if T316 is configured, and MP indirect path transmission is not suspended; and

6>	if neither MP indirect path change nor MP indirect path addition is ongoing:

7>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

6>	else:

7>	initiate the connection re-establishment procedure as specified in 5.3.7.

5>	else:

6>	if the UE supports RLF-Report for fast MCG recovery procedure and if T316 is configured:

7>	if the SCG is deactivated at the moment of detecting RLF in the MCG:

8>	set the mcg-RecoveryFailureCause in the VarRLF-Report to scg-Deactivated;

8>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

7>	else if SCG transmission is suspended at the moment of detecting RLF in the MCG:

8>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

8>	set the scg-FailureCause value in the VarRLF-Report according to 5.7.3.5;

8>	set the elapsedTimeSCG-Failure in the VarRLF-Report to the time elapsed between SCG failure and the MCG failure;

6>	if T316 is configured; and

6>	if SCG transmission is not suspended; and

6>	if the SCG is not deactivated; and

6>	if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC):

7>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

6>	else:

7>	initiate the connection re-establishment procedure as specified in 5.3.7.

A L2/L3 U2N Relay UE in case of single hop or the L2 Last U2N Relay UE shall:

1>	upon detecting radio link failure:

2>	either indicate to upper layers (to trigger PC5 unicast link release with its child UE(s)) or send NotificationMessageSidelink to the connected L2/L3 U2N Remote UE(s) or to the child UE(s)) in accordance with 5.8.9.10.

A N3C Relay UE shall:

1>	upon detecting radio link failure:

2>	indicates to the associated N3C remote UE via the Non-3GPP Connection.

NOTE 2:	How the N3C Relay UE indicates Uu RLF on the Non-3GPP Connection is left to implementation.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon T312 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached; or

1>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the SCG; or

1>	upon consistent uplink LBT failure indication from SCG MAC:

2>	if the indication is from SCG RLC and CA duplication is configured and activated for SCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	if the SCG is deactivated:

4>	stop radio link monitoring on the SCG;

4>	indicate to lower layers to stop beam failure detection on the PSCell;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

3>	else:

4>	if the UE is in NR-DC:

5>	if the UE supports RLF-Report for fast MCG recovery procedure and if the UE detected SCG failure while the timer T316 was running:

6>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

6>	set the scg-FailureCause in the VarRLF-Report value according to 5.7.3.5;

6>	set the elapsedTimeSCG-Failure in the VarRLF-Report to the time elapsed between MCG failure and the SCG failure;

6>	include scg-FailedAfterMCG;

5>	initiate the connection re-establishment procedure as specified in 5.3.7;

4>	else (the UE is in (NG)EN-DC):

5>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

#### 5.3.10.4	RLF cause determination

The UE shall set the rlf-Cause in the VarRLF-Report as follows:

1>	if the UE declares radio link failure due to T310 expiry:

2>	set the rlf-Cause as t310-Expiry;

1>	else if the UE declares radio link failure due to the random access problem indication from MCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the rlf-Cause as beamFailureRecoveryFailure;

2>	else:

3>	set the rlf-Cause as randomAccessProblem;

1>	else if the UE declares radio link failure due to the reaching of maximum number of retransmissions from the MCG RLC:

2>	set the rlf-Cause as rlc-MaxNumRetx;

1>	else if the UE declares radio link failure due to consistent uplink LBT failures:

2>	set the rlf-Cause as lbtFailure;

1>	else if the IAB-MT declares radio link failure due to the reception of a BH RLF indication on BAP entity:

2>	set the rlf-Cause as bh-rlfRecoveryFailure.

1>	else if the UE declares radio link failure due to T312 expiry:

2>	set the rlf-Cause as t312-Expiry;

#### 5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. including the RPLMN);

1>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell (in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell;

2>	set the measResultLastServPSCell to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected the failure;

2>	if the UE does not support RLF-Report for fast MCG recovery procedure as specified in TS 38.306 [26] or if T316 is not configured:

3>	set pSCellId to the the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change);

1>	if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2;

2>	set distanceFromReference1 to the measured distance between the UE and the serving cell moving reference location;

1>	if measRSSI-ReportConfig is configured for the measObject indicated as the servingCellMO of the source PCell (in case HO failure) or PCell (in case of RLF), set the measResultLastServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell (in case HO failure) or PCell (in case of RLF) up to the moment the UE detected the failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

2>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell:

3>	set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell, ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

1>	if the CSI-RS based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure;

2>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell:

3>	set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure;

1>	if the UE supports RLF-Report for MCG LTM cell switch and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell (in case of HO failure) or PCell (in case of RLF) and if the SS/PBCH block-based L1-RSRP measurements performed based on LTM-CSI-ReportConfig are available:

2>	set the resultsSSB-Indexes in measResultL1-LastServCell to include all the available SS/PBCH block-based L1-RSRP values of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block L1-RSRP measurement is listed first, based on the available SS/PBCH block-based L1-RSRP collected up to the moment the UE detected failure;

1>	for each of the configured measObjectNR in which measurements are available or in which the associated reportConfigNR is configured as conditional handover with time-based or location-based trigger condition:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF) and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 0a:	For the neighboring cells included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, UE also includes the CSI-RS based measurement quantities, if available.

2>	if the CSI-RS based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 0b:	For ordering the neighboring cells based on the CSI-RS measurement quantities, UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

2>	if measurement quantities are not available:

3>	set physCellId in measResultListNR in measResultNeighCells to include the physical cell identity of the neighbour cells that are candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure and if the related MCG VarConditionalReconfig only concerns measurement-based trigger condition; or

3>	if the UE supports RLF-Report for conditional handover with time-based and location-based trigger conditions in NTN and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure; or

3>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

4>	if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2;

5>	set distanceFromReference2 to the measured distance between the UE and the moving reference location of the neighbour cell, at the moment of handover failure, or radio link failure;

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the RLF;

2>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

2>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

2>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig for the same target candidate PCell as set in pCellId:

3>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

1>	if the UE supports RLF-Report for MCG LTM cell switch, for each neighbour MCG LTM candidate cell:

2>	if SS/PBCH block-based L1-RSRP measurement quantities performed based on LTM-CSI-ReportConfig are available:

3>	set the measResultL1-NeighCells to include all the available SS/PBCH block-based L1-RSRP measurement results, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first;

3>	for each neighbour frequency included, include the optional fields that are available;

1>	for each of the configured measObjectNR associated with neighboring cells if the associated reportConfigNR includes measRSSI-ReportConfig:

2>	set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequencies other than the frequency of the source PCell (in case HO failure) or of the PCell (in case RLF), up to the moment the UE detected failure:

3>	for each neighbour frequency included, include the optional fields that are available;

1>	for each of the configured EUTRA frequencies in which measurements are available;

2>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

1>	set the c-RNTI to the C-RNTI used in the source PCell (in case HO failure) or PCell (in case RLF);

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if the UE supports RLF-Report for DAPS handover and if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure:

3>	if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover;

3>	else:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig;

3>	set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResultNeighCells;

2>	if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

3>	set lastHO-Type to cho;

2>	else if the UE supports RLF-Report for MCG LTM cell switch and the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch:

3>	set lastHO-Type to ltm;

2>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG:

3>	set lastHO-Type to choWithCandidateSCG;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover or a failed LTM cell switch;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was applied;

2>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

1>	else if the failure is detected due to Mobility from NR failure as described in 5.4.3.5, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if last MobilityFromNRCommand concerned a failed inter-RAT handover from NR to E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA (NR to EUTRA):

3>	set the eutraFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last MobilityFromNRCommand message was received;

2>	set the timeConnFailure to the elapsed time since the initialization of the handover associated to the last MobilityFromNRCommand message;

2>	if the UE supports RLF report for inter-system handover for voice fallback and if voiceFallbackIndication is included in the last MobilityFromNRCommand:

3>	include the voiceFallbackHO;

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last successfully executed RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover or an LTM cell switch and the target cell of the intra NR handover or LTM cell switch was the PCell where radio link failure is detected; and

3>	if T316 was not running before entering the PCell in which the radio link failure was detected; and

3>	if T311 was not running before entering the PCell in which the radio link failure was detected:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the source PCell of the intra NR handover or LTM cell switch concerning the last successfully executed RRCReconfiguration message including reconfigurationWithSync;

4>	if the UE supports RLF-Report for DAPS handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a DAPS handover:

5>	set lastHO-Type to daps;

4>	else if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

5>	set lastHO-Type to cho;

4>	else if the UE supports RLF-Report for MCG LTM cell switch and the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch:

5>	set lastHO-Type to ltm;

4>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG:

5>	set lastHO-Type to choWithCandidateSCG;

4>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

3>	else if the last RRCReconfiguration message including the reconfigurationWithSync concerned a handover to NR from E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

4>	include the eutraPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the E-UTRA PCell where the last RRCReconfiguration message including reconfigurationWithSync was received embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

2>	if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of declaring the radio link failure:

3>	set timeSinceCHO-Reconfig to the time elapsed between the detection of the radio link failure, and the reception, in the source PCell, of the last conditionalReconfiguration including the condRRCReconfig message;

3>	set choCandidateCellList to include the global cell identity if available, and otherwise to the physical cell identity and carrier frequency of each of all the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of radio link failure, excluding the candidate target cells included in measResultNeighCells;

1>	if connectionFailureType is rlf and the rlf-Cause is set to randomAccessProblem or beamFailureRecoveryFailure; or

1>	if connectionFailureType is rlf and the rlf-Cause is set to lbtFailure and the radio link failure is detected during the random access procedure; or

1>	if connectionFailureType is hof and if the failed handover is an intra-RAT reconfiguration with sync and if a random-access procedure was triggered for the failed reconfiguration with sync:

2>	set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

1>	if connectionFailureType is rlf and the rlf-Cause is set to lbtFailure, and the radio link failure is not detected during the random access procedure:

2>	set the locationAndBandwidth and subcarrierSpacing in bwp-Info associated to the UL BWP in which the consistent uplink LBT failure was detected;

1>	if the rlf-Cause is set to t310-Expiry or t312-Expiry:

2>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the last serving cell, if available;

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

### 5.3.11	UE actions upon going to RRC_IDLE

The UE shall:

1>	reset MAC;

1>	if the UE is NCR-MT:

2>	indicate to NCR-Fwd to cease forwarding;

1>	set the variable pendingRNA-Update to false, if that is set to true;

1>	if going to RRC_IDLE was triggered by reception of the RRCRelease message including a waitTime:

2>	if T302 is running:

3>	stop timer T302;

2>	start timer T302 with the value set to the waitTime;

2>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2'.

1>	else:

2>	if T302 is running:

3>	stop timer T302;

3>	perform the actions as specified in 5.3.14.4;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the UE is leaving RRC_INACTIVE:

2>	if going to RRC_IDLE was not triggered by reception of the RRCRelease message:

3>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities;

3>	stop the timer T320, if running;

2>	if SDT procedure is ongoing:

3>	stop timer T319a, if running;

3>	consider SDT procedure is not ongoing;

1>	stop all timers that are running except T302, T320, T325, T330, T331, T400 and T430;

1>	discard the UE Inactive AS context, if any;

1>	release the suspendConfig, if configured;

1>	release the aerial-Config, if configured;

1>	perform LTM configuration release procedure for the MCG and SCG as specified in clause 5.3.5.18.7;

1>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

1>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

1>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

2>	for the associated reportConfigId:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

3>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

1>	discard the KgNB key, the S-KgNB key, the S-KeNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if any;

1>	if SL indirect path is configured:

2>	release cell identity and relay UE ID configured in sl-IndirectPathAddChange;

2>	indicate upper layers to trigger PC5 unicast link release of the SL indirect path;

1>	if N3C indirect path is configured:

2>	release n3c-IndirectPathAddChange;

2>	consider the non-3GPP connection is not used;

1>	if the UE is acting as a N3C relay UE:

2>	release n3c-IndirectPathConfigRelay;

2>	consider the non-3GPP connection is not used;

1>	release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels and SRAP entity;

NOTE 0:	A L2 U2N Relay UE may re-establish the SL-RLC0, SL-RLC1 and SRAP entity after release.

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	for each application layer measurement configuration without appLayerIdleInactiveConfig configured:

2>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

2>	release the application layer measurement configuration;

2>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

2>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	forward the measConfigAppLayerId and inform upper layers about the release of the RAN visible application layer measurement configuration;

2>	discard any RAN visible application layer measurement reports received from upper layers;

2>	initiate the procedure in 5.5b.1.2;

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

1>	except if going to RRC_IDLE was triggered by inter-RAT cell reselection while the UE is in RRC_INACTIVE or RRC_IDLE or when selecting an inter-RAT cell while T311 was running or when selecting an E-UTRA cell for EPS fallback for IMS voice as specified in 5.4.3.5:

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_IDLE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20];

1>	release CSI-LoggedMeasurementConfig, if configured;

1>	release loggedDataCollectionAssistanceConfig, if configured;

1>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

NOTE 1:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 2:	It is left to UE implementation whether to stop T430, if running, when going to RRC_IDLE.

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

1>	if the indicated serving cell is referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig;

2> release pucch-Resource indicated in the associated CSI-ReportUE-IBR;

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances configured in SRS-Config;

1>	release SRS-PosResourceSetLinkedForAggBW, if configured;

1>	release ue-TxTEG-RequestUL-TDOA-Config, if configured.

Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall:

1>	release the configured srs-PosRRC-Inactive;

1>	release SRS-PosResourceSetLinkedForAggBW, if configured.

### 5.3.13	RRC connection resume

#### 5.3.13.1	General

Figure 5.3.13.1-1: RRC connection resume, successful

Figure 5.3.13.1-2: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.13.1-3: RRC connection resume followed by network release, successful

Figure 5.3.13.1-4: RRC connection resume followed by network suspend, successful

Figure 5.3.13.1-5: RRC connection resume, network reject

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s), DRB(s) and multicast MRB(s) or perform an RNA update. This procedure is also used to initiate SDT in RRC_INACTIVE.

#### 5.3.13.1a	Conditions for resuming RRC Connection for NR sidelink communication/positioning/discovery/V2X sidelink communication

For NR sidelink communication/positioning/discovery an RRC connection is resumed only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to transmit NR sidelink discovery and related data is available for transmission:

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2U relay discovery messages and sl-L2-U2U-Relay-r18 is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2U relay discovery messages and sl-L3-U2U-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages and sl-L3U2N-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink non-relay discovery messages and sl-NonRelayDiscovery is included in SIB12:

3>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-DiscTxPoolSelected or sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to perform NR sidelink positioning and indicated by upper layers to transmit SL-PRS:

2>	if the frequency on which the UE is configured to transmit SL-PRS is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-PRS-ResourcesSharedSL-PRS-RP-r18 in sl-TxPoolSelectedNormal for the concerned frequency; or

2>	if the frequency on which the UE is configured to transmit SL-PRS is included in sl-PosFreqInfoList within SIB23 provided by the cell on which the UE camps; and if the valid version of SIB23 does not include sl-PRS-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_INACTIVE, an RRC connection establishment is resumed in the following cases:

1>	if any message is received from the L2 U2N Remote UE or from a child UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4; or

1>	if RemoteUEInformationSidelink containing the connectionForMP is received from a L2 U2N Remote UE as specified in 5.8.9.8.3;

For V2X sidelink communication an RRC connection resume is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE:	Upper layers initiate an RRC connection resume (except if the RRC connection resume is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1, or upon reception of RemoteUEInformationSidelink message containing the connectionForMP). The interaction with NAS is left to UE implementation.

#### 5.3.13.1b	Conditions for initiating SDT

When requesting lower layers to check the conditions for initiating SDT, RRC indicates to lower layers whether the resume procedure is initiated for mobile originated or mobile terminated case.

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	for the resume procedure initiated by the upper layers (i.e. mobile originated case):

2>	SIB1 includes sdt-ConfigCommon; and

2>	sdt-Config is configured; and

2>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

2>	for an (e)RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

2>	lower layers indicate that conditions for initiating MO-SDT as specified in TS 38.321 [3] are fulfilled.

1>	for the resume procedure initiated in response to RAN paging (i.e. mobile terminated case):

2>	lower layers indicate that conditions for initiating MT-SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

#### 5.3.13.1c	Void

#### 5.3.13.1d	Conditions for resuming RRC connection for multicast reception

In RRC_INACTIVE state, if configured with MBS multicast reception in RRC_INACTIVE, the UE shall:

1>	if the RRC connection resume procedure is triggered for multicast reception at reception of SIB1, as specified in 5.2.2.4.2; or

1>	if the RRC connection resume procedure is triggered for multicast reception at reception of Paging message, as specified in 5.3.2.3; or

1>	if the PTM configuration is not available on the multicast MCCH in the new cell after cell selection (i.e., different from the cell where the UE was configured to receive multicast in RRC_CONNECTED) or in the cell after cell reselection for at least one multicast session that the UE has joined and for which the UE is not indicated to stop monitoring the G-RNTI; or

1>	if mbs-NeighbourCellList included in MBSMulticastConfiguration acquired in the previous cell indicates that at least one multicast session that the UE has joined and for which the UE is not indicated to stop monitoring the G-RNTI, is not provided for RRC_INACTIVE in the current serving cell; or

1>	if either the measured RSRP or RSRQ for serving cell as specified in TS 38.304 [20] is below the corresponding threshold indicated by thresholdIndex for a multicast session that the UE has joined and for which the UE is not indicated to stop monitoring the G-RNTI:

2>	initiate RRC connection resume procedure as specified in 5.3.13.2 with resumeCause set as below:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	set resumeCause to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	set resumeCause to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	set resumeCause to highPriorityAccess;

3>	else:

4>	set resumeCause to mt-Access.

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, upon requesting multicast reception as specified in clause 5.3.13.1d, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a, for requesting configuration for SRS for positioning, for activation of preconfigured Positioning SRS in RRC_INACTIVE, for activation of non-preconfigured Positioning SRS with type semi-persistent in RRC_INACTIVE, upon receiving RRCRelease message including resumeIndication) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging; or

1> if the resumption of the RRC connection is triggered by receiving RRCRelease message including resumeIndication; or

1>	if the resumption of the RRC connection is triggered for multicast reception as specified in clause 5.3.13.1d:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

3>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and/or in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

NOTE 0:	If there are multiple NSAGs with the same highest NAS-provided NSAG priority identified for access attempt as above, it is left to UE implementation to select the NSAG to be applied in the Random Access procedure.

2>	if the resumption occurs after release with redirect with mpsPriorityIndication:

3>	set the resumeCause to mps-PriorityAccess;

2>	else if the resumption of the RRC connection is triggered for activation of preconfigured SRS for positioning available in srs-PosRRC-InactiveValidityAreaPreConfigList and if the UE is camped in one of the cells indicated in one of srs-PosConfigValidityArea; or

2>	if the resumption of the RRC connection is triggered due to the need for SRS for positioning configuration and no stored srs-PosRRC-InactiveValidityAreaPreConfigList for the camped cell exists; or

2>	if the resumption of the RRC connection is triggered due to activation of non-preconfigured SRS for positioning with type semi-persistent available in srs-PosRRC-InactiveValidityAreaNonPreConfig and if the UE is camped in the cells indicated in srs-PosConfigValidityArea:

3>	if an emergency service is ongoing:

4>	select '2' as the Access Category;

4>	set the resumeCause to emergency;

3>	else:

4>	set the resumeCause to srs-PosConfigOrActivationReq;

2>	else:

3>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE 1:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

1>	else if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured:

2>	if the resumption of the RRC connection is triggered due to cell reselection as specified in clause 5.3.13.6:

3>	if an emergency service is ongoing:

4>	select '2' as the Access Category;

4>	set the resumeCause to emergency;

3>	else:

4>	select '8' as the Access Category;

4>	set the resumeCause to srs-PosConfigOrActivationReq;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection resume triggered either by reception of message from a L2 U2N Remote UE or from a child UE via SL-RLC0 or SL-RLC1 as specified in 5.3.13.1a, or by reception of the RemoteUEInformationSidelink message containing the connectionForMP as specified in 5.3.13.1a, the L2 U2N Relay UE sets the resumeCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as resumeCause, if the same cause value in the message received from the L2 U2N Remote UE or from a child UE via SL-RLC0.

1>	if the UE is in NE-DC or NR-DC:

2>	if the UE does not support maintaining SCG configuration upon connection resumption:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

2>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	if the UE is acting as L2 U2N Remote UE or is acting as L2 Intermediate U2N Relay UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default PDCP configuration as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

2>	apply the default SRB1 configuration as specified in 9.2.1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	release idc-AssistanceConfig from the UE Inactive AS context, if stored;

1>	release drx-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346a, if running;

1>	release maxBW-PreferenceConfig and maxBW-PreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346b, if running;

1>	release maxCC-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346c, if running;

1>	release maxMIMO-LayerPreferenceConfig and maxMIMO-LayerPreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346d, if running;

1>	release minSchedulingOffsetPreferenceConfig and minSchedulingOffsetPreferenceConfigExt for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346e, if running;

1>	release rlm-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346j, if running;

1>	release bfd-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346k, if running;

1>	release releasePreferenceConfig from the UE Inactive AS context, if stored;

1>	release wlanNameList from the UE Inactive AS context, if stored;

1>	release btNameList from the UE Inactive AS context, if stored;

1>	release sensorNameList from the UE Inactive AS context, if stored;

1>	release obtainCommonLocation from the UE Inactive AS context, if stored;

1>	stop timer T346f, if running;

1>	stop timer T346i, if running;

1>	release referenceTimePreferenceReporting from the UE Inactive AS context, if stored;

1>	release sl-AssistanceConfigNR from the UE Inactive AS context, if stored;

1>	release musim-GapAssistanceConfig from the UE Inactive AS context, if stored and stop timer T346h, if running;

1>	release musim-GapConfig from the UE Inactive AS context, if stored;

1>	release musim-GapPriorityAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-LeaveAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-CapabilityRestrictionConfig from the UE Inactive AS context, if stored and stop timer T346n, if running;

1>	release propDelayDiffReportConfig from the UE Inactive AS context, if stored;

1>	release ul-GapFR2-PreferenceConfig, if configured;

1>	release rrm-MeasRelaxationReportingConfig from the UE Inactive AS context, if stored;

1>	release multiRx-PreferenceReportingConfigFR2 if configured, and stop timer T346m, if running;

1>	release aerial-FlightPathAvailabilityConfig from the UE Inactive AS context, if stored;

1>	release ul-TrafficInfoReportingConfig from the UE Inactive AS context, if stored;

1>	release applicabilityReportConfig from the UE Inactive AS context, if stored;

1>	release dataCollectionPreferenceConfig from the UE Inactive AS context, if stored;

1>	release assisted-SSB-MTC-Config from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346l, if running;

1>	release gapOccasionCancelRatioReportConfig from the UE Inactive AS context, if stored, and stop timer T346o, if running.

1>	if the UE is acting as L2 U2N Remote UE:

2>	apply the specified configuration of SL-RLC0 used for the delivery of RRC message over SRB0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ncd-SSB-RedCapInitialBWP-SDT is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored ncd-SSB-RedCapInitialBWP-SDT:

3>	release the stored ncd-SSB-RedCapInitialBWP-SDT;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

1>	set the variable pendingRNA-Update to false;

1>	release successHO-Config from the UE Inactive AS context, if stored;

1>	release successPSCell-Config configured by the PCell from the UE Inactive AS context, if stored;

1>	release successPSCell-Config configured by the PSCell from the UE Inactive AS context, if stored;

1>	release lpwus-OffsetPreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346p, if running;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

#### 5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the resumeIdentity to the stored shortI-RNTI value;

1>	restore the RRC configuration, RoHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules and the KgNB and KRRCint keys from the stored UE Inactive AS context except for the following:

-	masterCellGroup;

-	mrdc-SecondaryCellGroup, if stored; and

-	pdcp-Config;

1>	set the resumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarResumeMAC-Input;

2>	with the KRRCint key in the UE Inactive AS Context and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	derive the KgNB key based on the current KgNB key or the NH, using the nextHopChainingCount value received in the previous RRCRelease message and stored in the UE Inactive AS Context, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

1>	configure lower layers to apply integrity protection for all radio bearers except SRB0 and MRBs using the configured algorithm and the KRRCint key and KUPint key derived in this clause immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

NOTE 1:	Only DRBs with previously configured UP integrity protection shall resume integrity protection.

1>	configure lower layers to apply ciphering for all radio bearers except SRB0 and MRBs and to apply the configured ciphering algorithm, the KRRCenc key and the KUPenc key derived in this clause, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	re-establish PDCP entities for SRB1;

1>	resume SRB1;

1>	if the resume procedure is initiated for SDT:

2>	for each radio bearer that is configured for SDT and for SRB1:

3>	restore the RLC-BearerConfig associated with the RLC bearers of masterCellGroup and pdcp-Config from the UE Inactive AS context;

3>	if the radio bearer is a DRB configured with Ethernet Header Compression:

4>	indicate to lower layer that ethernetHeaderCompression is not configured;

3>	if the radio bearer is a DRB configured with UDC:

4>	indicate to lower layer that uplinkDataCompression is not configured;

3>	if the radio bearer is a DRB configured with ROHC function:

4>	if sdt-DRB-ContinueROHC is set to cell and the resume procedure is initiated in a cell that is the same as the PCell in which the UE received the previous RRCRelease message; or

4>	if sdt-DRB-ContinueROHC is set to rna and the resume procedure is initiated in a cell belonging to the same RNA as the PCell in which the UE received the previous RRCRelease message:

5>	indicate to lower layer that drb-continueROHC is configured;

4>	else:

5>	indicate to lower layer that drb-continueROHC is not configured;

3>	re-establish PDCP entity for the radio bearer that is configured for SDT without triggering PDCP status report;

2>	resume all the radio bearers that are configured for SDT;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

NOTE 2a:	Before the lower layers first transmit the RRCResumeRequest or RRCResumeRequest1, the UE may initiate a new resume procedure if other conditions for initiation of the resume procedure as specified in 5.3.13.2 are satisfied.

If lower layers indicate an integrity check failure while T319 is running or SDT procedure is ongoing, perform actions specified in 5.3.13.5.

If the UE is an (e)RedCap UE and the RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

NOTE 3:	For L2 U2N Remote UE or L2 Intermediate U2N Relay UE in RRC_INACTIVE, the cell (re)selection procedure as specified in TS 38.304 [20] and relay (re)selection procedure as specified in 5.8.15.3 are performed independently and it is up to UE implementation to select either a cell or a L2 U2N Relay UE.

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319, if running;

1>	stop timer T319a, if running and consider SDT procedure is not ongoing;

1>	stop timer T380, if running;

1>	if T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.7.8.3;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	if the RRCResume does not include the restoreMCG-SCells:

3>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

2>	if the RRCResume does not include the restoreSCG:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

2>	restore the masterCellGroup, mrdc-SecondaryCellGroup, if stored, and pdcp-Config from the UE Inactive AS context;

2>	configure lower layers to consider the restored MCG and SCG SCell(s) (if any) to be in deactivated state;

1>	discard the UE Inactive AS context;

1>	store the used nextHopChainingCount value associated to the current KgNB;

1>	if the UE is configured to receive MBS multicast in RRC_INACTIVE:

2>	reset MAC;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

2>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG indicated by tag-Id, if it is not running;

1>	if srs-PosRRC-Inactive is configured:

2>	instruct the MAC entity to stop inactivePosSRS-TimeAlignmentTimer, if it is running;

1>	if srs-PosRRC-InactiveValidityAreaNonPreConfig is configured; or

1>	if srs-PosRRC-InactiveValidityAreaPreConfigList is configured and if the cell is not listed in srs-PosConfigValidityArea:

2>	instruct the MAC entity to stop inactivePosSRS-ValidityAreaTAT, if it is running;

1>	release the suspendConfig except the ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCResume includes the mrdc-SecondaryCellGroup:

2>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

3>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

2>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

3>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCResume message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCResume message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCResume message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	if the RRCResume message includes the appLayerMeasConfig:

2>	if idleInactiveReportAllowed is included in the RRCResume message:

3>	if the UE is configured with at least one application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	initiate the procedure in 5.7.16.2 after the RRCResumeComplete has been transmitted;

2>	else:

3>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

4>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig, if stored;

4>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

4>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCResume message includes the sl-L2RemoteUE-Config (i.e. the UE is a L2 U2N Remote UE):

2>	perform the L2 U2N Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCResume message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

1>	resume SRB2 (if suspended), SRB3 (if configured), SRB4 (if configured), SRB5 (if configured), SRB6 (if configured), all DRBs (that are suspended) and multicast MRBs (that are suspended);

NOTE 1:	If the SCG is deactivated, resuming SRB3 and all DRBs does not imply that PDCP or RRC PDUs can be transmitted or received on SCG RLC bearers.

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	stop relay reselection procedure if any for L2 U2N Remote UE;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if upper layers provides a PLMN:

3>	if the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

4>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

3>	else:

4>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	if validatedMeasurementsReq is included in the RRCResume and measIdleValidityDuration is included in VarEnhMeasIdleConfig;

5>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig;

5>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig;

5>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

5>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig;

4>	else:

5>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

5>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

5>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

5>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

4>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

2>	if the reselectionMeasurementReq is included in the RRCResume message:

3> if validatedMeasurementsReq is included in the RRCResume and measReselectionValidityDuration is included in VarMeasReselectionConfig:

4>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

5>	if the UE has valid cell reselection measurements results for any frequency listed in measReselectionCarrierListNR in VarMeasRelectionConfig:

6>	set the measResultReselectionNR in the RRCResumeComplete message to the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig;

4>	else:

5>	if the UE has valid NR cell reselection measurements results:

6>	set the measResultReselectionNR in the RRCResumeComplete message to any available valid NR measurement results, if available, and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig;

3> else:

4>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

5>	if the UE has cell reselection measurements results for any frequency listed in measReselectionCarrierListNR in VarMeasRelectionConfig:

6>	set the measResultReselectionNR in the RRCResumeComplete message to the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig;

4>	else:

5>	if the UE has NR cell reselection measurements results:

6>	set the measResultReselectionNR in the RRCResumeComplete message to any available NR measurement results, if available;

2>	else:

3>	if the SIB1 contains reselectionMeasurementsNR:

4>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig and the UE has NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; or

4>	if measReselectionCarrierListNR is not present in VarMeasReselectionConfig and if the UE has NR reselection measurements available:

5>	include the reselectionMeasAvailable;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

2>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCResumeComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCResumeComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCResumeComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included; or

2>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

3>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

4>	set sigLogMeasConfigAvailable to true in the RRCResumeComplete message;

3>	else:

4>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

5>	set sigLogMeasConfigAvailable to false in the RRCResumeComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCResumeComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity are included in snpn-IdentityList stored in VarRLF-Report; or

3>	include rlf-InfoAvailable in the RRCResumeComplete message;

2>	if the UE has successful PSCell change or addition related information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

3>	include successPSCell-InfoAvailable in the RRCResumeComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCResumeComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCResumeComplete message;

2>	if speedStateReselectionPars is configured in the SIB2:

3>	include the mobilityState in the RRCResumeComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	include measConfigReportAppLayerAvailable in the RRCResumeComplete message;

2>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

3>	include the NeedForGapsInfoNR and set the contents as follows:

4> include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNR is configured, for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band; otherwise, include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

3>	if the needForInterruptionConfigNR is enabled:

4>	include the needForInterruptionInfoNR and set the contents as follows:

5>	include intraFreq-needForInterruption with the same number of entries, and listed in the same order, as in intraFreq-needForGap;

5> for each entry in intraFreq-needForInterruption:

6>	include interruptionIndication and set the interruption requirement information if the corresponding entry in intraFreq-needForGap is set to no-gap;

5>	include interFreq-needForInterruption with the same number of entries, and listed in the same order, as in interFreq-needForGap;

5>	for each entry in interFreq-needForInterruption:

6>	include interruptionIndication and set the interruption requirement information if the corresponding entry in interFreq-needForGap is set to no-gap;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

3>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

4> include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNCSG-NR is configured:

5>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

4>	else:

5>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

3>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

4>	if requestedTargetBandFilterNCSG-EUTRA is configured:

5>	for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band;

4>	else:

5>	include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCResumeComplete message upon determining it has temporary capability restriction;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19:

3>	for each such serving cell, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19:

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

NOTE 2:	Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

NOTE 3:	Upon reception of musim-CapRestrictionInd in RRCResumeComplete, it is up to network implementation to configure the UE with a limited configuration that is used until network sends RRCReconfiguration based on the actual restricted UE capabilities included in UEAssistanceInformation.

#### 5.3.13.5	Handling of failure to resume RRC Connection

The UE shall:

1>	if timer T319 expires:

2>	if the UE supports multiple CEF report:

3>	if UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity in networkIdentity stored in VarConnEstFailReport; or

3>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is equal to snpn-identity in networkIdentity stored in VarConnEstFailReport:

4>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

5>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

2>	if the UE is not in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the UE is in SNPN access mode and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the registered SNPN identity is not equal to snpn-identity in networkIdentity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReportList and if the RPLMN is not equal to plmn-identity in networkIdentity stored in any entry of VarConnEstFailReportList:

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReportList and if the registered SNPN identity is not equal to snpn-identity in networkIdentity stored in any entry of VarConnEstFailReportList:

3>	clear the content included in VarConnEstFailReportList;

2> clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection resume failure information in the VarConnEstFailReport by setting its fields as follows:

3>	if the UE is not in SNPN access mode:

4>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

3>	else if the UE is in SNPN access mode:

4>	set the snpn-Identity in networkIdentity to include the SNPN identity selected by upper layers (see TS 24.501 [23]) from the list of SNPN(s) included in the npn-IdentityInfoList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection resume failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as in 5.3.3.7;

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

3>	if numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

1>	else if upon receiving integrity check failure indication from lower layers while T319 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

1>	else if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing; or

1>	if random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

1>	if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; or

1>	if integrity check failure indication is received from lower layers while SDT procedure is ongoing; or

1>	if T319a expires:

2>	consider SDT procedure is not ongoing;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

The UE may discard the connection resume failure or connection establishment failure information, i.e. release the UE variable VarConnEstFailReport and the UE variable VarConnEstFailReportList, 48 hours after the last connection resume failure is detected.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) to the child UE(s) in accordance with 5.8.9.10.

#### 5.3.13.6	Cell re-selection or cell selection or L2 U2N relay (re)selection while T390, T319 or T302 is running or SDT procedure is ongoing (UE in RRC_INACTIVE) or SRS transmission in RRC_INACTIVE is configured

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running or while SDT procedure is ongoing; or

1>	if relay (re)selection or cell selection by a L2 U2N Remote UE or by L2 Intermediate U2N Relay UE occurs while T319 is running; or

1>	if cell changes due to relay reselection or cell selection by a L2 U2N Remote UE or by L2 Intermediate U2N Relay UE while T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure';

1>	else if cell selection or reselection occurs while T390 is running, or cell change due to relay selection or reselection occurs while T390 is running:

2>	stop T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4.

1>	else if cell reselection occurs when srs-PosRRC-Inactive is configured:

2>	indicate to the lower layer to stop inactivePosSRS-TimeAlignmentTimer;

2>	release the srs-PosRRC-Inactive.

1>	else if cell reselection occurs when srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured and if there is an on-going SRS for positioning transmission procedure in RRC_INACTIVE:

2>	if the selected cell is not included in the srs-PosConfigValidityArea:

3>	indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT;

3>	initiate RRC connection resume procedure in 5.3.13.2;

2>	else if the cell is included in the srs-PosConfigValidityArea:

3>	if the selected cell and the previously camped cell are in the same srs-PosConfigValidityArea:

4>	if autonomousTA-AdjustmentEnabled is configured and if the Timing Advance validation requirements specified in clause 5.6.6.3 of TS 38.133 [14] is met:

5>	indicate to the lower layer to update Timing Advance and stored RSRP;

4>	instruct lower layers to continue transmitting SRS if Timing Advance validation condition as specified in TS 38.321 [3] are satisfied;

3>	if the selected cell and previously camped cell are in the different srs-PosConfigValidityArea:

4>	initiate RRC connection resume procedure in 5.3.13.2;

4>	indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT.

#### 5.3.13.7	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection setup procedure as specified in 5.3.3.4.

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2>	if T319 is not running and SDT procedure is not ongoing:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

1>	if barring is alleviated for Access Category '8' or Access Category '2', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRNA-Update is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

NOTE:	It is left to UE implementation how to behave when T380 expires while the UE is camped neither on a suitable nor on an acceptable cell.

#### 5.3.13.9	Reception of the RRCRelease by the UE

The UE shall:

1>	perform the actions as specified in 5.3.8.

#### 5.3.13.10	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15.

#### 5.3.13.11	Inability to comply with RRCResume

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCResume message;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ′RRC Resume failure′.

NOTE 1:	The UE may apply above failure handling also in case the RRCResume message causes a protocol error for which the generic error handling as defined in 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is configured (i.e., via SIB1) to send MUSIM temporary capability restriction indication, and if the UE supports MUSIM temporary capability restriction, the UE does not apply above failure handling in case the UE is unable to apply (part of) the configuration resulting from RRCResume message due to UE temporary capability restriction for MUSIM operation. If UE does not go to RRC_IDLE in this case, UE still considers the configuration resulting from the RRCResume message as the current configuration as the baseline for delta configuration for future reconfigurations. It is up to UE implementation how to apply RRCResume message. For other cases, if the UE is unable to comply with part of the configuration, it does not apply any (part of) the configuration, i.e. there is no partial success/failure.

#### 5.3.13.12	Inter RAT cell reselection

Upon reselecting to an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.14	Unified Access Control

#### 5.3.14.1	General

The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer. This procedure does not apply to IAB-MT and NCR-MT. This procedure does not apply to L2 U2N Relay UE initiating RRC connection establishment or RRC connection resume upon reception of any message from a L2 U2N remote UE via SL-RLC0 or SL-RLC1 in accordance to 5.3.3.1a or 5.3.13.1a.

After a PCell change in RRC_CONNECTED the UE shall defer access barring checks until it has obtained SIB1 (as specified in 5.2.2.2) from the target cell.

#### 5.3.14.2	Initiation

Upon initiation of the procedure, the UE shall:

1>	if timer T390 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else if timer T302 is running:

2>	if the Access Category is neither '2' nor '0'; or

2>	if the Access Category is '0' for RRC resumption triggered by multicast reception as specified in 5.3.13.2:

3>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else:

3>	if SIB1 includes uac-BarringPerPLMN-List that contains a UAC-BarringPerPLMN for the selected PLMN or SNPN:

4>	if the procedure in 5.2.2.4.2 for a selected PLMN resulted in use of information in npn-IdentityInfoList and UAC-BarringPerPLMN has an entry with the plmn-IdentityIndex corresponding to used information in this list:

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to used information in the npn-IdentityInfoList;

4>	else:

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the selected PLMN and the PLMN-IdentityInfo, if any, or the selected SNPN and the npn-IdentityInfoList;

3>	if any UAC-BarringPerPLMN entry is selected:

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SIB1;

3>	else if SIB1 includes uac-BarringForCommon:

4>	in the remainder of this procedure use the uac-BarringForCommon (i.e. presence or absence of these parameters) included in SIB1;

3>	else:

4>	consider the access attempt as allowed;

3>	if uac-BarringForCommon is applicable or the uac-ACBarringListType indicates that uac-ExplicitACBarringList is used:

4>	if the corresponding UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

5>	if the uac-BarringInfoSetList contains a UAC-BarringInfoSet entry corresponding to the selected uac-barringInfoSetIndex in the UAC-BarringPerCat:

6>	select the UAC-BarringInfoSet entry;

6>	perform access barring check for the Access Category as specified in 5.3.14.5, using the selected UAC-BarringInfoSet as "UAC barring parameter";

5>	else:

6>	consider the access attempt as allowed;

4>	else:

5>	consider the access attempt as allowed;

3>	else if the uac-ACBarringListType indicates that uac-ImplicitACBarringList is used:

4>	select the uac-BarringInfoSetIndex corresponding to the Access Category in the uac-ImplicitACBarringList;

4>	if the uac-BarringInfoSetList contains the UAC-BarringInfoSet entry corresponding to the selected uac-BarringInfoSetIndex:

5>	select the UAC-BarringInfoSet entry;

5>	perform access barring check for the Access Category as specified in 5.3.14.5, using the selected UAC-BarringInfoSet as "UAC barring parameter";

4>	else:

5>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as allowed;

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	if timer T302 is running:

4>	if timer T390 is running for Access Category '2':

5>	inform the upper layer that access barring is applicable for all access categories except categories '0', upon which the procedure ends;

4>	else

5>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2', upon which the procedure ends;

3>	else:

4>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends.

#### 5.3.14.3	Void

#### 5.3.14.4	T302, T390 expiry or stop (Barring alleviation)

The UE shall:

1>	if timer T302 expires or is stopped:

2>	for each Access Category for which T390 is not running:

3>	consider the barring for this Access Category to be alleviated:

1>	else if timer T390 corresponding to an Access Category other than '2' expires or is stopped, and if timer T302 is not running:

2>	consider the barring for this Access Category to be alleviated;

1>	else if timer T390 corresponding to the Access Category '2' expires or is stopped:

2>	consider the barring for this Access Category to be alleviated;

1>	when barring for an Access Category is considered being alleviated:

2>	if the Access Category was informed to upper layers as barred:

3>	inform upper layers about barring alleviation for the Access Category.

2>	else if the Access Category is Access Category '0' for RRC resumption triggered by multicast reception as specified in 5.3.13.2:

3>	perform actions specified in 5.3.13.1d;

2>	if barring is alleviated for Access Category '8'; or

2>	if barring is alleviated for Access Category '2':

3>	perform actions specified in 5.3.13.8;

#### 5.3.14.5	Access barring check

The UE shall:

1>	if one or more Access Identities equal to 1, 2, 11, 12, 13, 14, or 15 are indicated according to TS 24.501 [23], and

1>	if for at least one of these Access Identities the corresponding bit in the uac-BarringForAccessIdentity contained in "UAC barring parameter" is set to zero:

2>	consider the access attempt as allowed;

1>	else:

2>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN); and

2>	if the bit corresponding to Access Identity 1 in the uac-BarringForAccessIdentity contained in the "UAC barring parameter" is set to zero:

3>	consider the access attempt as allowed;

2>	else if Access Identity 3 is indicated:

3>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3>	if 'rand' is lower than the value indicated by uac-BarringFactorForAI3 included in "UAC barring parameter":

4>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as barred;

2>	else:

3>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3>	if 'rand' is lower than the value indicated by uac-BarringFactor included in "UAC barring parameter":

4>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as barred;

1>	if the access attempt is considered as barred:

2>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2>	start timer T390 for the Access Category with the timer value calculated as follows, using the uac-BarringTime included in "UAC barring parameter":

T390 = (0.7+ 0.6 * rand) * uac-BarringTime.

### 5.3.15	RRC connection reject

#### 5.3.15.1	Initiation

The UE initiates the procedure upon the reception of RRCReject when the UE tries to establish or resume an RRC connection.

#### 5.3.15.2	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if running;

1>	stop timer T319, if running;

1>	stop timer T319a, if running and consider SDT procedure is not ongoing;

1>	stop timer T302, if running;

1>	reset MAC and release the default MAC Cell Group configuration (except if the RRCReject is received in response to resuming RRC connection for multicast reception);

1>	if waitTime is configured in the RRCReject:

2>	start timer T302, with the timer value set to the waitTime;

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if RRCReject is received in response to an RRCSetupRequest:

2>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

1>	else if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

2>	if resume is triggered by upper layers:

3>	inform upper layers about the failure to resume the RRC connection;

2>	if resume is triggered due to an RNA update; or

2>	if resume is triggered for SDT and T380 has expired:

3>	set the variable pendingRNA-Update to true;

2>	discard the current KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key derived in accordance with 5.3.13.3;

2>	if resume is triggered for SDT:

3>	for SRB2, if it is resumed and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for each DRB that is not suspended:

4>	indicate PDCP suspend to lower layers;

4>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	suspend SRB1 and the radio bearers configured for SDT, if any;

2>	the procedure ends.

Upon L2 U2N Relay UE receives RRCReject, it either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the child UE(s) in accordance with 5.8.9.10.

The RRC_INACTIVE UE shall continue to monitor paging while the timer T302 is running.

NOTE:	If timer T331 is running, the UE continues to perform idle/inactive measurements according to 5.7.8.

## 5.4	Inter-RAT mobility

### 5.4.1	Introduction

Network controlled inter-RAT mobility between NR and E-UTRA, where E-UTRA can be connected to either EPC or 5GC, and from NR to UTRA-FDD is supported.

### 5.4.2	Handover to NR

#### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA.

#### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to NR procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

The network applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to re-establish SRBs and one or more DRBs;

#### 5.4.2.3	Reception of the RRCReconfiguration by the UE

The UE shall:

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	perform RRC reconfiguration procedure as specified in 5.3.5;

NOTE:	If the UE is connected to 5GC of the source E-UTRA cell, the delta configuration for PDCP and SDAP can be used for intra-system inter-RAT handover. For other cases, source RAT configuration is not considered when the UE applies the reconfiguration message of target RAT.

### 5.4.3	Mobility from NR

#### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from NR, successful

Figure 5.4.3.1-2: Mobility from NR, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using other RAT, e.g. E-UTRA, UTRA-FDD. The mobility from NR procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell;

#### 5.4.3.2	Initiation

The network initiates the mobility from NR procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport or an MCGFailureInformation message, by sending a MobilityFromNRCommand message. The network applies the procedure as follows:

-	the procedure is initiated only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB have been setup and not suspended;

-	the procedure is not initiated if any DAPS bearer is configured;

#### 5.4.3.3	Reception of the MobilityFromNRCommand by the UE

The UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	if T316 is running:

2>	stop timer T316;

2>	if the UE supports RLF-Report for fast MCG recovery procedure as specified in 38.306 [26]:

3>	set the elapsedTimeT316 in the VarRLF-Report to the value of the elapsed time of the timer T316;

3>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

2>	else:

3>	clear the information included in VarRLF-Report, if any;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the targetRAT-Type is set to eutra:

2>	consider inter-RAT mobility as initiated towards E-UTRA;

2>	forward the nas-SecurityParamFromNR to the upper layers, if included;

1>	else if the targetRAT-Type is set to utra-fdd:

2>	consider inter-RAT mobility as initiated towards UTRA-FDD;

2>	forward the nas-SecurityParamFromNR to the upper layers, if included;

1>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT.

#### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover, at the source side the UE shall:

1>	if T316 was not running at the time of receiving MobilityFromNRCommand and if the UE was configured with successHO-Config when connected to the source PCell and the targetRAT-Type is set to eutra:

2>	perform the actions for the successful handover report determination for inter-RAT handover as specified in clause 5.7.10.6.

1>	reset MAC;

1>	stop all timers that are running except T325, T330 and T400;

1>	release ran-NotificationAreaInfo, if stored;

1>	release the AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if stored;

1>	release all radio resources, including release of the RLC entity and the MAC configuration;

1>	release the associated PDCP entity and SDAP entity for all established RBs;

NOTE :	PDCP and SDAP configured by the source RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure.

1>	if the UE is configured with any application layer measurement configuration:

2>	inform upper layers about the release of all application layer measurement configurations;

2>	release all application layer measurement configurations including their fields in UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

2>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

2>	consider itself not to be configured to send application layer measurement reports;

1>	discard any logged measurement entries included in VarCSI-LogMeasReport;

1>	if the targetRAT-Type is set to eutra and the nas-SecurityParamFromNR is included: or

1>	if the targetRAT-Type is set to utra-fdd:

2>	indicate the release of the RRC connection to upper layers together with the release cause 'other'.

#### 5.4.3.5	Mobility from NR failure

The UE shall:

1>	if the UE does not succeed in establishing the connection to the target radio access technology:

2>	if the targetRAT-Type in the received MobilityFromNRCommand is set to eutra and the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

3>	store handover failure information in VarRLF-Report according to 5.3.10.5;

2>	if voiceFallbackIndication is included in the MobilityFromNRCommand message; or

2>	if the mobility from NR procedure is for emergency services fallback as specified in TS 23.502 [43]:

3>	attempt to select an E-UTRA cell:

4>	if a suitable E-UTRA cell is selected; or

4>	if no suitable E-UTRA cell is available and an acceptable E-UTRA cell supporting emergency call is selected when the UE has an ongoing emergency call:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	revert back to the configuration used in the source PCell;

5>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

NOTE:	It is left to UE implementation to determine whether the mobility from NR procedure is for emergency services fallback as specified in TS 23.502 [43].

2>	else:

3>	revert back to the configuration used in the source PCell;

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

1>	else if the UE is unable to comply with any part of the configuration included in the MobilityFromNRCommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromNRCommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2>	if the targetRAT-Type in the received MobilityFromNRCommand is set to eutra and the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

3>	store handover failure information in VarRLF-Report according to 5.3.10.5;

2>	revert back to the configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

## 5.5	Measurements

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements. The network may configure the UE to report them in accordance with the measurement configuration or to perform conditional reconfiguration evaluation in accordance with the conditional reconfiguration or to perform LTM cell switch conditions evaluation in accordance with the configured LTM cell switch conditions. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration or RRCResume.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies;

-	Inter-RAT measurements of UTRA-FDD frequencies;

-	NR sidelink measurements of L2 U2N Relay UEs.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The network may configure the UE to perform the following types of measurements for NR sidelink and V2X sidelink:

-	CBR measurements.

The network may configure the UE to report the following CLI measurement information based on SRS resources:

-	Measurement results per SRS resource;

-	SRS resource(s) indexes.

The network may configure the UE to report the following CLI measurement information based on CLI-RSSI resources:

-	Measurement results per CLI-RSSI resource;

-	CLI-RSSI resource(s) indexes.

The network may configure the UE to report the following Rx-Tx time difference measurement information based on CSI-RS for tracking or PRS:

-	UE Rx-Tx time difference measurement result.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'exclude-listed' cells and a list of 'allow-listed' cells. Exclude-listed cells are not applicable in event evaluation or measurement reporting. Allow-listed cells are the only ones applicable in event evaluation or measurement reporting.

-	The measObjectId of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single E-UTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets and a list of 'exclude-listed' cells. Exclude-listed cells are not applicable in event evaluation or measurement reporting.

-	For inter-RAT UTRA-FDD measurements a measurement object is a set of cells on a single UTRA-FDD carrier frequency.

-	For NR sidelink measurements of L2 U2N Relay UEs, a measurement object is a single NR sidelink frequency to be measured.

-	For CBR measurement of NR sidelink communication, a measurement object is a set of transmission resource pool(s) on a single carrier frequency for NR sidelink communication.

-	For CBR measurement of NR sidelink discovery, a measurement object is a set of discovery dedicated resource pool(s) or transmission resource pool(s) also used for NR sidelink discovery on a single carrier frequency for NR sidelink discovery.

-	For CBR measurement of NR sidelink positioning, a measurement object is a set of positioning dedicated resource pool(s) or transmission resource pool(s) also used for NR sidelink positioning on a single carrier frequency for NR sidelink positioning.

-	For CLI measurements a measurement object indicates the frequency/time location of SRS resources and/or CLI-RSSI resources, and subcarrier spacing of SRS resources to be measured.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each measurement reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

In case of conditional reconfiguration, each configuration consists of the following:

-	Execution criteria: The criteria the UE uses for conditional reconfiguration execution.

-	RS type: The RS that the UE uses for obtaining beam and cell measurement results (SS/PBCH block-based or CSI-RS-based), used for evaluating conditional reconfiguration execution condition.

3.	Measurement identities: For measurement reporting, a list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network. For conditional reconfiguration triggering, one measurement identity links to exactly one conditional reconfiguration trigger configuration. And up to 2 measurement identities can be linked to one conditional reconfiguration execution condition.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements.

6.	Effective measurement window: Periods that the UE may use to perform inter RAT measurements.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR measurement object(s), CLI measurement object(s), inter-RAT objects, and L2 U2N Relay objects. Similarly, the reporting configuration list includes NR, inter-RAT, and L2 U2N Relay reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) – these are the SpCell and one or more SCells.

2.	Listed cells – these are cells listed within the measurement object(s).

3.	Detected cells – these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s)/serving Relay UE (for L2 U2N Remote UE), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on the configured resources on the indicated frequency. For inter-RAT measurements object(s) of UTRA-FDD, the UE measures and reports on listed cells. For CLI measurement object(s), the UE measures and reports on configured measurement resources (i.e. SRS resources and/or CLI-RSSI resources). For L2 U2N Relay object(s), the UE measures and reports on the serving NR cell(s), as well as the discovered L2 U2N Relay UEs.

Whenever the procedural specification, other than contained in clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

In NR-DC, the UE may receive two independent measConfig:

-	a measConfig, associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a measConfig, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

In this case, the UE maintains two independent VarMeasConfig and VarMeasReportList, one associated with each measConfig, and independently performs all the procedures in clause 5.5 for each measConfig and the associated VarMeasConfig and VarMeasReportList, unless explicitly stated otherwise.

The configurations related to CBR measurements are only included in the measConfig associated with MCG.

The configurations related to Rx-Tx time difference measurement are only included in the measConfig associated with MCG.

### 5.5.2	Measurement configuration

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured;

-	to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI;

-	to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-DelayValueConfig;

-	to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-ExcessDelayConfig;

-	to ensure that, in the measConfig associated with a CG:

-	for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

-	an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value and that an smtc4list included in any measurement object with the same ssbFrequency has the same value;

-	to ensure that all measurement objects configured in this specification and in TS 36.331 [10] with the same ssbFrequency have the same ssbSubcarrierSpacing;

-	to ensure that, if a measurement object associated with the MCG has the same ssbFrequency as a measurement object associated with the SCG:

-	for that ssbFrequency, the measurement window according to the smtc1 configured by the MCG includes the measurement window according to the smtc1 configured by the SCG, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

-	if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

-	to ensure that, if a measurement object has the same ssbFrequency as a measurement object configured in TS 36.331 [10]:

-	for that ssbFrequency, the measurement window according to the smtc configured in TS 36.331 [10] includes the measurement window according to the smtc1 configured in TS 38.331, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

-	if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

-	when the UE is in NE-DC, NR-DC, or NR standalone, to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportSFTD;

For CSI-RS resources, the network applies the procedure as follows:

-	to ensure that all CSI-RS resources configured in each measurement object have the same center frequency, (startPRB+floor(nrofPRBs/2))

-	to ensure that the total number of CSI-RS resources configured in each measurement object does not exceed the maximum number specified in TS 38.214 [19].

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the quantityConfig:

2>	perform the quantity configuration procedure as specified in 5.5.2.8;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

1>	if the received measConfig includes the measGapSharingConfig:

2>	perform the measurement gap sharing configuration procedure as specified in 5.5.2.11;

1>	if the received measConfig includes the s-MeasureConfig:

2>	if s-MeasureConfig is set to ssb-RSRP, set parameter ssb-RSRP of s-MeasureConfig within VarMeasConfig to the threshold value of the RSRP indicated by the received value of s-MeasureConfig which is derived as specified in 6.3.2;

2>	else, set parameter csi-RSRP of s-MeasureConfig within VarMeasConfig to the threshold value of the RSRP indicated by the received value of s-MeasureConfig which is derived as specified in 6.3.2.

1>	if the received measConfig includes the effectiveMeasWindowConfig:

2>	perform the effective measurement window configuration as specified in 5.5.2.12;

#### 5.5.2.2	Measurement identity removal

The UE shall:

1>	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

2>	if the reportType is set to reportOnScellActivation in the reportConfig associated with this measId:

3>	indicate to lower layer to disable the measurement reporting for fast unknown SCell activation.

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

#### 5.5.2.3	Measurement identity addition/modification

The network applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured.

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId to the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE 1:	If the measId associated with reportConfig for conditional reconfiguration is modified, the conditions are considered to be not fulfilled as specified in 5.3.5.13.4.

2>	if the reportType is set to reportCGI in the reportConfig associated with this measId:

3>	if the measObject associated with this measId concerns E-UTRA:

4>	if the useAutonomousGaps is included in the reportConfig associated with this measId:

5>	start timer T321 with the timer value set to 200 ms for this measId;

4>	else:

5>	start timer T321 with the timer value set to 1 second for this measId;

3>	if the measObject associated with this measId concerns NR:

4>	if the measObject associated with this measId concerns FR1:

5>	if the useAutonomousGaps is included in the reportConfig associated with this measId:

6>	if the UE is an (e)RedCap UE with 1 Rx branch

7>	start timer T321 with the timer value set to 3 seconds for this measId;

6>	else

7>	start timer T321 with the timer value set to 2 seconds for this measId;

5>	else:

6>	start timer T321 with the timer value set to 2 seconds for this measId;

4>	if the measObject associated with this measId concerns FR2:

5>	if the useAutonomousGaps is included in the reportConfig associated with this measId:

6>	if the UE is a RedCap UE with 1 Rx branch

7>	start timer T321 with the timer value set to 6 seconds for this measId;

6>	else

7>	start timer T321 with the timer value set to 5 seconds for this measId;

5>	else:

6>	start timer T321 with the timer value set to 16 seconds for this measId.

2>	if the reportType is set to reportSFTD in the reportConfigNR associated with this measId and the drx-SFTD-NeighMeas is included:

3>	if the measObject associated with this measId concerns FR1:

4>	start timer T322 with the timer value set to 3 seconds for this measId;

3>	if the measObject associated with this measId concerns FR2:

4>	start timer T322 with the timer value set to 24 seconds for this measId.

2>	if the reportType is set to reportOnScellActivation in the reportConfig associated with this measId:

3>	indicate to lower layer to enable the measurement reporting for fast unknown SCell activation.

#### 5.5.2.4	Measurement object removal

The UE shall:

1>	for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig:

2>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer or timer T321 or timer T322, whichever is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

#### 5.5.2.5	Measurement object addition/modification

The UE shall:

1>	for each measObjectId included in the received measObjectToAddModList:

2>	if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, excludedCellsToAddModList, allowedCellsToAddModList, cellsToRemoveList, excludedCellsToRemoveList, allowedCellsToRemoveList, tx-PoolMeasToRemoveList, tx-PoolMeasToAddModList, ssb-PositionQCL-CellsToRemoveList, ssb-PositionQCL-CellsToAddModList, cca-CellsToRemoveList, and cca-CellsToAddModList;

3>	if the received measObject includes the cellsToRemoveList:

4>	for each physCellId included in the cellsToRemoveList:

5>	remove the entry with the matching physCellId from the cellsToAddModList;

3>	if the received measObject includes the cellsToAddModList:

4>	for each physCellId value included in the cellsToAddModList:

5>	if an entry with the matching physCellId exists in the cellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the cellsToAddModList;

3>	if the received measObject includes the excludedCellsToRemoveList:

4>	for each pci-RangeIndex included in the excludedCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the excludedCellsToAddModList;

NOTE 1:	For each pci-RangeIndex included in the excludedCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the exclude-list of cells only if all PCI ranges containing it are removed.

3>	if the received measObject includes the excludedCellsToAddModList:

4>	for each pci-RangeIndex included in the excludedCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the excludedCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the excludedCellsToAddModList;

3>	if the received measObject includes the allowedCellsToRemoveList:

4>	for each pci-RangeIndex included in the allowedCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the allowedCellsToAddModList;

NOTE2:	For each pci-RangeIndex included in the allowedCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the allow-list of cells only if all PCI ranges containing it are removed.

3>	if the received measObject includes the allowedCellsToAddModList:

4>	for each pci-RangeIndex included in the allowedCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the allowedCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the allowedCellsToAddModList

3>	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4>	if the UE does not support Uplink PDCP delay measurements upon MO update; or

4>	if the UE supports Uplink PDCP delay measurements upon MO update and if ul-DelayValueConfig or ul-ExcessDelayConfig is not configured for the associated reportConfig:

5>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

5>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

3>	if the received measObject includes the tx-PoolMeasToRemoveList:

4>	for each transmission resource pool indicated in tx-PoolMeasToRemoveList:

5>	remove the entry with the matching identity of the transmission resource pool from the tx-PoolMeasToAddModList;

3>	if the received measObject includes the tx-PoolMeasToAddModList:

4>	for each transmission resource pool indicated in tx-PoolMeasToAddModList:

5>	if an entry with the matching identity of the transmission resource pool exists in the tx-PoolMeasToAddModList:

6>	replace the entry with the value received for this transmission resource pool;

5>	else:

6>	add a new entry for the received identity of the transmission resource pool to the tx-PoolMeasToAddModList;

3>	if the received measObject includes the ssb-PositionQCL-CellsToRemoveList:

4>	for each physCellId included in the ssb-PositionQCL-CellsToRemoveList:

5>	remove the entry with the matching physCellId from the ssb-PositionQCL-CellsToAddModList;

3>	if the received measObject includes the ssb-PositionQCL-CellsToAddModList:

4>	for each physCellId included in the ssb-PositionQCL-CellsToAddModList:

5>	if an entry with the matching physCellId exists in the ssb-PositionQCL-CellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the ssb-PositionQCL-CellsToAddModList;

3>	if the received measObject includes the cca-CellsToRemoveList:

4>	for each physCellId included in the cca-CellsToRemoveList:

5>	remove the entry with the matching physCellId from the cca-CellsToAddModList;

3>	if the received measObject includes the cca-CellsToAddModList:

4>	for each physCellId included in the cca-CellsToAddModList:

5>	if an entry with the matching physCellId exists in the cca-CellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the cca-CellsToAddModList;

2>	else:

3>	add a new entry for the received measObject to the measObjectList within VarMeasConfig.

#### 5.5.2.6	Reporting configuration removal

The UE shall:

1>	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

#### 5.5.2.7	Reporting configuration addition/modification

The UE shall:

1>	for each reportConfigId included in the received reportConfigToAddModList:

2>	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this reportConfig;

3>	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig.

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration configured by gapFR1 is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the gapFR1 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration configured by gapFR1;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration configured by gapFR2 is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the gapFR2 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration configured by gapFR2;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration configured by gapUE is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the gapUE in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration configured by gapUE.

1>	for each measGapId included in the received gapToReleaseList:

2>	release the measurement gap configuration associated with the measGapId;

1>	for each measPosPreConfigGapId included in the received posMeasGapPreConfigToReleaseList:

2>	release the measurement gap configuration associated with the measPosPreConfigGapId;

1>	for each GapConfig received in gapToAddModList:

2>	setup measurement gap configuration indicated by the GapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	apply the measurement gap as per UE measurement gap, FR1 measurement gap, or FR2 measurement gap according to the gapType indicated by the GapConfig;

2>	associate the measurement gap with the measGapId indicated by the GapConfig;

2>	if gapSharing in the GapConfig is present:

3>	setup the gap sharing configuration for the measurement gap in accordance with the received gapSharing as defined in TS 38.133 [14];

2>	else:

3>	release the gap sharing configuration (if configured) for the measurement gap;

1>	for each PosGapConfig received in PosMeasGapPreConfigToAddModList:

2>	if a measurement gap configuration associated with the measPosPreConfigGapId indicated by the PosGapConfig is already setup:

3>	release the measurement gap configuration;

2>	setup measurement gap configuration indicated by the PosGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	configure the measurement gap as indicated by gapType;

1>	for each FR1, FR2, and per UE measurement gap that is setup:

2>	if the measurement gap is configured by GapConfig and preConfigInd-r17 in the corresponding GapConfig is present:

3>	determine whether the measurement gap is activated or not according to TS 38.133 [14];

2>	else if the measurement gap is configured by PosGapConfig:

3>	consider the measurement gap to be deactivated;

2>	else:

3>	consider the measurement gap to be activated.

NOTE 1:	For FR2 gap configuration with synchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

NOTE 2:	For FR1 gap or per UE gap configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in is used in the gap calculation. Otherwise, the SFN and subframe of the PCell is used in the gap calculation.

NOTE 3:	For FR2 gap configuration with asynchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator and refFR2ServCellAsyncCA is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency indicated by the refFR2ServCellAsyncCA is used in the gap calculation

#### 5.5.2.10	Reference signal measurement timing configuration

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the SSB-MTC configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10));

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the IAB-MT shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition.

If smtc5list is present, for cells indicated in the pci-List parameter in each SSB-MTC5 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity and offset parameter in each SSB-MTC5 configuration and use the duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition.

If smtc6list is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtc6list for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first  OD-SSB configuration for the serving cell; the UE shall setup SMTC according to the second SMTC in smtc6list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on.

If smtc7-SSBAdapt is present, the UE shall setup SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI is not received or the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; the UE shall setup SMTC according to the first SMTC in smtc7-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the first adaptive SSB periodicity in ssb-BurstPeriodicityList; the UE shall setup SMTC according to the second SMTC in smtc7-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the second adaptive SSB periodicity in ssb-BurstPeriodicityList.

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

#### 5.5.2.10a	RSSI measurement timing configuration

The UE shall setup the RSSI measurement timing configuration (RMTC) in accordance with the received rmtc-Periodicity and, if configured, with rmtc-SubframeOffset i.e. the first symbol of each RMTC occasion occurs at first symbol of an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = FLOOR(rmtc-SubframeOffset/10);

subframe = rmtc-SubframeOffset mod 10;

with T = rmtc-Periodicity/10;

On the frequency configured by rmtc-Frequency, the UE shall not consider RSSI measurements outside the configured RMTC occasion which lasts for measDurationSymbols for RSSI and channel occupancy measurements.

The UE derives the RSSI measurement duration from a combination of measDurationSymbols and ref-SCS-CP. At least for RSSI measurement confined within the active DL BWP, the UE performs RSSI measurement using the numerology of the active DL BWP during the derived measurement duration. Otherwise, the numerology used by the UE for measurements is up to UE implementation. If configured, the UE performs RSSI measurements on a bandwidth in accordance with the received rmtc-Bandwidth. If configured, the UE performs RSSI measurements according to the TCI state configured by tci-StateId in the reference BWP configured by ref-BWPId of the reference serving cell configured by ref-ServCellId (see TS 38.133 [14], clause 9.2A.7 and clause 9.3A.8).If the UE has no serving cell in FR2-2 and configured with inter-frequency RSSI measurement in FR2-2, it is up to the UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

#### 5.5.2.11	Measurement gap sharing configuration

The UE shall:

1>	if gapSharingFR1 is set to setup:

2>	if an FR1 measurement gap sharing configuration configured by gapSharingFR1 is already setup:

3>	release the FR1 measurement gap sharing configuration configured by gapSharingFR1;

2>	setup the FR1 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR1 as defined in TS 38.133 [14];

1>	else if gapSharingFR1 is set to release:

2>	release the FR1 measurement gap sharing configuration configured by gapSharingFR1;

1>	if gapSharingFR2 is set to setup:

2>	if an FR2 measurement gap sharing configuration configured by gapSharingFR2 is already setup:

3>	release the FR2 measurement gap sharing configuration configured by gapSharingFR2;

2>	setup the FR2 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR2 as defined in TS 38.133 [14];

1>	else if gapSharingFR2 is set to release:

2>	release the FR2 measurement gap sharing configuration configured by gapSharingFR2.

1>	if gapSharingUE is set to setup:

2>	if a per UE measurement gap sharing configuration configured by gapSharingUE is already setup:

3>	release the per UE measurement gap sharing configuration configured by gapSharingUE;

2>	setup the per UE measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingUE as defined in TS 38.133 [14];

1>	else if gapSharingUE is set to release:

2>	release the per UE measurement gap sharing configuration configured by gapSharingUE.

#### 5.5.2.12	Effective measurement window configuration

The UE shall:

1>	if effectiveMeasWindowConfig is set to setup:

2>	if an effective measurement window configuration is already setup:

3>	release the effective measurement window configuration;

2>	setup an effective measurement window configuration indicated by the effectiveMeasWindowConfig in accordance with the received windowOffsetPeriodicity (providing periodicity and offset for the following condition), i.e., the first subframe of each window occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(offset/10);

subframe = offset mod 10;

with T = periodicity/10;

1>	else if effectiveMeasWindowConfig is set to release:

2>	release the effective measurement window configuration;

NOTE:	The SFN and subframe of the PCell is used in the effective measurement window calculation.

### 5.5.3	Performing measurements

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements.

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact L1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the OD-SSB-Config is not configured, or:

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or:

2>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or:

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is configured and OD-SSB transmission is not activated:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is configured and OD-SSB transmission is activated:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the OD-SSB-Config is not configured, or:

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency indicates the same frequency as absoluteFrequencySSB of the serving cell, or:

2>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or:

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is configured and OD-SSB transmission is not activated:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

2>	else if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is configured and OD-SSB transmission is activated:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO-OD:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is set to reportCGI and timer T321 is running:

3>	if useAutonomousGaps is configured for the associated reportConfig:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3>	else:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods;

3>	if the cell indicated by reportCGI field for the associated measObject is an NR cell and that indicated cell is broadcasting SIB1 (see TS 38.213 [13], clause 13):

4>	try to acquire SIB1 in the concerned cell;

3>	if the cell indicated by reportCGI field is an E-UTRA cell:

4>	try to acquire SystemInformationBlockType1 in the concerned cell;

2>	if the ul-DelayValueConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB;

2>	if the ul-ExcessDelayConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding UL PDCP Excess Packet Delay delay measurement according to the configured threshold per DRB;

2>	if the reportType for the associated reportConfig is periodical; or

2>	if the reportType for the associated reportConfig is eventTriggered, and the eventId is not set to eventD1 or eventD2; or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond or in the condExecutionCondPSCell associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA, MN-initiated inter-SN CPC, or subsequent CPAC in NR-DC), and the condEventId is not set to condEventD1, condEventD2 or condEventT1; or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig (for intra-SN CPC or subsequent CPAC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig (for SN-initiated inter-SN CPC or subsequent CPAC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10] (for SN-initiated inter-SN CPC in EN-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the LTM-ExecutionConditionList for which the UE is currently performing LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8:

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to UTRA-FDD:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to L2 U2N Relay UE:

6>	perform the corresponding measurements associated to candidate Relay UEs on the frequencies indicated in the concerned measObject, as described in 5.5.3.4;

4>	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5>	perform the RSSI and channel occupancy measurements on the frequency configured by rmtc-Frequency in the associated measObject;

NOTE 0:	The network avoids configuring UEs supporting only CHO and/or Rel-16 CPC with measurements not referred to by any execution condition.

2>	if the reportType for the associated reportConfig is set to reportSFTD and the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than one:

3>	if the reportSFTD-Meas is set to true:

4>	if the measObject is associated to E-UTRA:

5>	perform SFTD measurements between the PCell and the E-UTRA PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the E-UTRA PSCell;

4>	else if the measObject is associated to NR:

5>	perform SFTD measurements between the PCell and the NR PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the NR PSCell based on SSB;

3>	else if the reportSFTD-NeighMeas is included:

4>	if the measObject is associated to NR:

5>	if the drx-SFTD-NeighMeas is included:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject using available idle periods;

5>	else:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject;

5>	if the reportRSRP is set to true:

6>	perform RSRP measurements based on SSB for the NR neighbouring cell(s) detected based on parameters in the associated measObject;

2>	if the reportType for the associated reportConfig is cli-Periodical or cli-EventTriggered:

3>	perform the corresponding measurements associated to CLI measurement resources indicated in the concerned measObjectCLI;

2>	perform the evaluation of reporting criteria as specified in 5.5.4, except if reportConfig is condTriggerConfig.

NOTE 1:	The evaluation of conditional reconfiguration execution criteria is specified in 5.3.5.13.

NOTE 1a:	The evaluation of LTM cell switch conditions execution criteria is specified in 5.3.5.18.8.

The UE acting as a L2 U2N Remote UE whenever configured with measConfig shall:

1>	perform the corresponding measurements associated to the serving L2 U2N Relay UE, as described in 5.5.3.4;

The UE capable of Rx-Tx time difference measurement when configured with measObjectRxTxDiff shall:

1>	perform the corresponding Rx-Tx time difference measurements associated with downlink reference signals indicated in the concerned measObjectRxTxDiff.

The UE capable of CBR measurement when configured to transmit NR sidelink communication/discovery/positioning shall:

1>	If the frequency used for NR sidelink communication/discovery/positioning is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12 or included in sl-PosConfigCommonNR within SIB23:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE:

3>	if configured with NR sidelink communication and the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency; or

3>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional but does not include sl-DiscTxPoolSelected for the concerned frequency:

4>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency in SIB12;

3>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-DiscTxPoolSelected for the concerned frequency:

4>	perform CBR measurement on pools in sl-DiscTxPoolSelected and sl-TxPoolExceptional for the concerned frequency in SIB12;

3>	if configured with NR sidelink positioning and the cell chosen for NR sidelink positioning provides SIB23 which includes sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional for the concerned frequency, or provides SIB12 which includes sl-TxPoolSelectedNormal, sl-TxPoolExceptional:

4>	perform CBR measurement on pool(s) in sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolExceptional, sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency;

2>	if the UE is in RRC_CONNECTED:

3>	if tx-PoolMeasToAddModList is included in VarMeasConfig:

4>	perform CBR measurements on each transmission resource pool indicated in the tx-PoolMeasToAddModList;

3>	if sl-DiscTxPoolSelected, sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolScheduling or sl-PRS-TxPoolExceptional is included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration:

4>	perform CBR measurement on pool(s) in sl-DiscTxPoolSelected, sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolScheduling and sl-PRS-TxPoolExceptional if included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration;

3>	else:

4>	if configured with NR sidelink communication and the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency; or

4>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional but does not provide sl-DiscTxPoolSelected for the concerned frequency:

5>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency in SIB12;

4>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-DiscTxPoolSelected for the concerned frequency:

5>	perform CBR measurement on pools in sl-DiscTxPoolSelected and sl-TxPoolExceptional for the concerned frequency in SIB12;

4>	if configured with NR sidelink positioning and the cell chosen for NR sidelink positioning provides SIB23 which includes sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional, or provides SIB12 which includes sl-TxPoolSelectedNormal, sl-TxPoolExceptional for the concerned frequency:

5>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional for the concerned frequency;

1>	else:

2>	if configured with NR sidelink communication and sl-TxPoolSelectedNormal is included in SidelinkPreconfigNR for the concerned frequency; or

2>	if configured with NR sidelink discovery and sl-TxPoolSelectedNormal is included in SidelinkPreconfigNR but sl-DiscTxPoolSelected is not included in SidelinkPreconfigNR for the concerned frequency:

3>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal in SidelinkPreconfigNR for the concerned frequency.

2>	if configured with NR sidelink discovery and sl-DiscTxPoolSelected is included in SidelinkPreconfigNR for the concerned frequency:

3>	perform CBR measurement on pools in sl-DiscTxPoolSelected if included in SidelinkPreconfigNR.

2>	if configured with NR sidelink positioning and sl-TxPoolSelectedNormal or sl-PRS-TxPoolSelectedNormal is included in SL-PreconfigurationNR for the concerned frequency:

3>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-PRS-TxPoolSelectedNormal in SidelinkPreconfigNR for the concerned frequency.

NOTE 2:	In case the configurations for NR sidelink communication and CBR measurement are acquired via the E-UTRA, configurations for NR sidelink communication in SIB12, sl-ConfigDedicatedNR within RRCReconfiguration used in this clause are provided by the configurations in SystemInformationBlockType28, sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331[10], respectively.

NOTE 3:	If a UE that is configured by upper layers to transmit V2X sidelink communication is configured by NR with transmission resource pool(s) and the measurement objects concerning V2X sidelink communication (i.e. by sl-ConfigDedicatedEUTRA-Info), it shall perform CBR measurement as specified in clause 5.5.3 of TS 36.331 [10], based on the transmission resource pool(s) and the measurement object(s) concerning V2X sidelink communication configured by NR.

NOTE 4:	For V2X sidelink communication, each of the CBR measurement results is associated with a resource pool, as indicated by the poolReportId (see TS 36.331 [10]), that refers to a pool as included in sl-ConfigDedicatedEUTRA-Info or SIB13.

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4:

2>	filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

NOTE 4:	For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch.

NOTE 5:	For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range.

NOTE 6:	Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

#### 5.5.3.3	Derivation of cell measurement results

The network may configure the UE in RRC_CONNECTED to derive RSRP, RSRQ and SINR measurement results per cell associated to NR measurement objects based on parameters configured in the measObject (e.g. maximum number of beams to be averaged and beam consolidation thresholds) and in the reportConfig (rsType to be measured, SS/PBCH block or CSI-RS).

The network may configure the UE in RRC_IDLE or in RRC_INACTIVE to derive RSRP and RSRQ measurement results per cell associated to NR carriers based on parameters configured in measIdleCarrierListNR within VarMeasIdleConfig for measurements performed according to 5.7.8.2a.

The UE shall:

1>	for each cell measurement quantity to be derived based on SS/PBCH block:

2>	if nrofSS-BlocksToAverage is not configured in the associated measObject in RRC_CONNECTED or in the associated entry in measIdleCarrierListNR within VarMeasIdleConfig in RRC_IDLE/RRC_INACTIVE; or

2>	if absThreshSS-BlocksConsolidation is not configured in the associated measObject in RRC_CONNECTED or in the associated entry in measIdleCarrierListNR within VarMeasIdleConfig in RRC_IDLE/RRC_INACTIVE; or

2>	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation:

3>	derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on SS/PBCH block as the linear power scale average of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage, and where each beam measurement quantity is described in TS 38.215 [9];

2>	if in RRC_CONNECTED, apply layer 3 cell filtering as described in 5.5.3.2;

1>	for each cell measurement quantity to be derived based on CSI-RS:

2>	consider a CSI-RS resource to be applicable for deriving cell measurements when the concerned CSI-RS resource is included in the csi-rs-CellMobility including the physCellId of the cell in theCSI-RS-ResourceConfigMobility in the associated measObject;

2>	if nrofCSI-RS-ResourcesToAverage in the associated measObject is not configured; or

2>	if absThreshCSI-RS-Consolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below or equal to absThreshCSI-RS-Consolidation:

3>	derive each cell measurement quantity based on applicable CSI-RS resources for the cell as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on CSI-RS as the linear power scale average of the highest beam measurement quantity values above absThreshCSI-RS-Consolidation where the total number of averaged beams shall not exceed nrofCSI-RS-ResourcesToAverage;

2>	apply layer 3 cell filtering as described in 5.5.3.2.

#### 5.5.3.3a	Derivation of layer 3 beam filtered measurement

The UE shall:

1>	for each layer 3 beam filtered measurement quantity to be derived based on SS/PBCH block;

2>	derive each configured beam measurement quantity based on SS/PBCH block as described in TS 38.215[9], and apply layer 3 beam filtering as described in 5.5.3.2;

1>	for each layer 3 beam filtered measurement quantity to be derived based on CSI-RS;

2>	derive each configured beam measurement quantity based on CSI-RS as described in TS 38.215 [9], and apply layer 3 beam filtering as described in 5.5.3.2.

#### 5.5.3.4	Derivation of L2 U2N Relay UE measurement results

A UE may be configured by network to derive NR sidelink measurement results of serving L2 U2N Relay UE or candidate L2 U2N Relay UEs associated to the measurement objects configured in the measObjectRelay.

The UE shall:

1>	for each L2 U2N Relay UE measurement quantity to be derived:

2>	derive the corresponding measurement quantity based on PSSCH/PSCCH DMRS as described in TS 38.215 [9];

2>	apply layer 3 filtering as described in 5.5.3.2;

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the corresponding reportConfig includes measRSSI-ReportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 or eventA3H1 or eventA3H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if the eventX2 is configured in the corresponding reportConfig:

5>	consider only the serving L2 U2N Relay UE to be applicable;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1, eventA2, eventD1, eventD2, eventX2, eventH1 or eventH2:

5>	if useAllowedCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if eventB1 or eventB2 is configured in the corresponding reportConfig:

5>	consider a serving cell, if any, on the associated E-UTRA frequency as neighbour cell;

4>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModListEUTRAN defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns UTRA-FDD:

4>	if eventB1-UTRA-FDD or eventB2-UTRA-FDD is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider a neighbouring cell on the associated frequency to be applicable when the concerned cell is included in the cellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns L2 U2N Relay UE:

4>	if eventY1-Relay or eventY2-Relay or eventZ1-Relay is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider any L2 U2N Relay UE fulfilling upper layer criteria detected on the associated frequency to be applicable for this measId;

2>	else if the corresponding reportConfig includes a reportType set to reportCGI:

3>	consider the cell detected on the associated measObject which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding reportConfig within the VarMeasConfig to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the reportSFTD-Meas is set to true:

5>	consider the NR PSCell to be applicable;

4>	else if the reportSFTD-NeighMeas is included:

5>	if cellsForWhichToReportSFTD is configured in the corresponding reportConfig:

6>	consider any NR neighbouring cell detected on the associated measObjectNR which has a physical cell identity that is included in the cellsForWhichToReportSFTD to be applicable;

5>	else:

6>	consider up to 3 strongest NR neighbouring cells detected based on parameters in the associated measObjectNR to be applicable when the concerned cells are not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if the reportSFTD-Meas is set to true:

5>	consider the E-UTRA PSCell to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to cli-Periodical or cli-EventTriggered:

3>	consider all CLI measurement resources included in the corresponding measObject to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to rxTxPeriodical:

3>	consider all Rx-Tx time difference measurement resources included in the corresponding measObject to be applicable;

2>	if the corresponding reportConfig concerns the reporting for NR sidelink communication/discovery (i.e. reportConfigNR-SL):

3>	consider the transmission resource pools indicated by the tx-PoolMeasToAddModList defined within the VarMeasConfig for this measId to be applicable;

2>	if the reportType is set to eventTriggered, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered, and if the corresponding reportConfig includes numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig:

3>	if the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	if the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	else:

4>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

4>	if the number of cell(s) in the cellsTriggeredList is larger than or equal to numberOfTriggeringCells:

5>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	if enteringLeavingReport is configured for the corresponding reporting configuration:

4>	store the concerned cell(s) in the cellsMetLeavingCond defined within the VarMeasReportList for this measId;

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	if the corresponding reportConfig does not include numberOfTriggeringCells; or

4>	if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s):

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove all the concerned cell(s) from cellsMetLeavingCond defined within the VarMeasReportList for this measId, if any;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable L2 U2N Relay UEs for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first L2 U2N Relay UE triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable L2 U2N Relay UEs not included in the relaysTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent L2 U2N Relay UE triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the L2 U2N Relay UEs included in the relaysTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the relaysTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools not included in the poolsTriggeredList for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent transmission resource pool triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the poolsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the leaving condition applicable for this event is fulfilled for the associated VarMeasReport within the VarMeasReportList for this measId during timeToTrigger defined within the VarMeasConfig for this event:

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove the measurement reporting entry within the VarMeasReportList for this measId;

3>	stop the periodical reporting timer for this measId, if running;

NOTE 1:	Void.

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	if the corresponding reportConfig includes measRSSI-ReportConfig:

4>	initiate the measurement reporting procedure as specified in 5.5.5 immediately when RSSI sample values are reported by the physical layer after the first L1 measurement duration;

3>	else if the corresponding reportConfig includes the ul-DelayValueConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity;

3>	else if the corresponding reportConfig includes the ul-ExcessDelayConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity(ies) according to the configured threshold per DRB identity(ies);

3>	else if the reportAmount exceeds 1:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell or for the serving L2 U2N Relay UE (if the UE is a L2 U2N Remote UE);

3>	else (i.e. the reportAmount is equal to 1):

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and for the strongest cell among the applicable cells, or for the NR SpCell and for the strongest L2 U2N Relay UEs among the applicable L2 U2N Relay UEs; or initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the serving L2 U2N Relay UE and for the strongest cell among the applicable cells, or for the serving L2 U2N Relay UE and for the strongest L2 U2N Relay UEs among the applicable L2 U2N Relay UEs (if the UE is a L2 U2N Remote UE);

2>	if, in case the corresponding reportConfig concerns the reporting for NR sidelink communication/discovery, reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and CBR measurement results become available;

2>	if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable CLI measurement resources for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first CLI measurement resource triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more CLI measurement resources not included in the cli-TriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent CLI measurement resource triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to cli-EventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the CLI measurement resources included in the cli-TriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cli-TriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to cli-Periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for at least one CLI measurement resource;

2>	if reportType is set to rxTxPeriodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon expiry of the periodical reporting timer for this measId:

3>	if reportType is set to eventTriggered and reportOnBestCellChange is configured for this measId

4>	if reportOnBestCellChange is set to n1, and the first measured neighbouring cell among cells within cellsTriggeredList according to the sorting quantity is the same as the first cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId, or

4>	if reportOnBestCellChange is set to n2 and there is only one cell included in the cellsTriggeredList for this measId, and the best measured neighbouring cell according to the sorting quantity in the cellsTriggeredList is the same as the first cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId, or

4>	if reportOnBestCellChange is set to n2 and there is more than one cell included in the cellsTriggeredList for this measId, and the best measured neighbouring cell among cells within cellsTriggeredList according to the sorting quantity is the same as the first cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId, and the second best measured neighbouring cell among cells within cellsTriggeredList according to the sorting quantity is the same as the second cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId:

5>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

5>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

6>	restart the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

4>	else:

5>	initiate the measurement reporting procedure, as specified in 5.5.5;3>	else:

4>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	if the corresponding reportConfig includes a reportType is set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the drx-SFTD-NeighMeas is included:

5>	if the quantity to be reported becomes available for each requested pair of PCell and NR cell:

6>	stop timer T322;

6>	initiate the measurement reporting procedure, as specified in 5.5.5;

4>	else

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for each requested pair of PCell and NR cell or the maximal measurement reporting delay as specified in TS 38.133 [14];

3>	else if the corresponding measObject concerns E-UTRA:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the pair of PCell and E-UTRA PSCell or the maximal measurement reporting delay as specified in TS 38.133 [14];

2>	if reportType is set to reportCGI:

3>	if the UE acquired the SIB1 or SystemInformationBlockType1 for the requested cell; or

3>	if the UE detects that the requested NR cell is not transmitting SIB1 (see TS 38.213 [13], clause 13):

4>	stop timer T321;

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

4>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon the expiry of T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	upon the expiry of T322 for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

If AS security has been activated successfully and if SCell activation(s) indication is received from lower layer, the UE shall:

1>	if reportType is set to reportOnScellActivation for any measId included in the measIdList within VarMeasConfig:

2>	if the activated SCell(s) fulfills the measurement requirement as specified in TS 38.133 [14]:

3>	include a measurement reporting entry within the VarMeasReportList for this measId:

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5.

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the NR serving cell corresponding to the associated measObjectNR associated with this event.

Inequality A1-1 (Entering condition)

Ms – Hys > Thresh

Inequality A1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated by the measObjectNR associated to this event.

NOTE:	If the SCell indicated by the measObjectNR associated to this event is not detectable, then the UE should consider for the value of Ms the lowest value of the value range of the measurement quantity as the SCell measurement.

Inequality A2-1 (Entering condition)

Ms + Hys < Thresh

Inequality A2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Inequality A3-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

NOTE 2:	The definition of Event A3 also applies to CondEvent A3.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled.

Inequality A4-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality A4-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell or the measurement result of serving PSCell (i.e., in case it is configured as candidate PSCell for CondEvent A4 evaluation) for CHO with candidate SCG(s) case, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

NOTE:	The definition of Event A4 also applies to CondEvent A4.

#### 5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE 1:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality A5-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality A5-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality A5-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

NOTE 2:	The definition of Event A5 also applies to CondEvent A5.

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell corresponding to the measObjectNR associated to this event to be the serving cell.

NOTE:	The reference signal(s) of the neighbour(s) and the reference signal(s) of the SCell are both indicated in the associated measObjectNR.

Inequality A6-1 (Entering condition)

Mn + Ocn – Hys > Ms + Ocs + Off

Inequality A6-2 (Leaving condition)

Mn + Ocn + Hys < Ms + Ocs + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within the associated measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within the associated measObjectNR), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

#### 5.5.4.8	Event B1 (Inter RAT neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition B1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B1-2, as specified below, is fulfilled.

Inequality B1-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality B1-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell, utra-FDD-Q-OffsetRange as defined within the measObjectUTRA-FDD corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell, or cellIndividualOffset as defined within reportConfigInterRAT), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. b1-ThresholdEUTRA as defined within reportConfigInterRAT for this event, b1-ThresholdUTRA-FDD as defined for UTRA-FDD within reportConfigInterRAT for this event).

Mn is expressed in dBm or in dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.9	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition B2-1 and condition B2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B2-3 or condition B2-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality B2-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality B2-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality B2-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality B2-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the inter-RAT neighbour cell, utra-FDD-Q-OffsetRange as defined within the measObjectUTRA-FDD corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell, or cellIndividualOffset as defined within reportConfigInterRAT), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. b2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. b2-Threshold2EUTRA as defined within reportConfigInterRAT for this event, b2-Threshold2UTRA-FDD as defined for UTRA-FDD within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mn is expressed in dBm or dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.10	Event I1 (Interference becomes higher than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition I1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition I1-2, as specified below, is fulfilled.

Inequality I1-1 (Entering condition)

Mi – Hys > Thresh

Inequality I1-2 (Leaving condition)

Mi+ Hys < Thresh

The variables in the formula are defined as follows:

Mi is the measurement result of the interference, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. i1-Threshold as defined within reportConfigNR for this event).

Mi, Thresh are expressed in dBm.

Hys is expressed in dB.

#### 5.5.4.11	Event C1 (The NR sidelink channel busy ratio is above a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition C1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition C1-2, as specified below, is fulfilled;

Inequality C1-1 (Entering condition)

Inequality C1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of channel busy ratio of the transmission resource pool, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR-SL for this event).

Thresh is the threshold parameter for this event (i.e. c1-Threshold as defined within reportConfigNR-SL for this event).

Ms is expressed in decimal from 0 to 1 in steps of 0.01.

Hys is expressed is in the same unit as Ms.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.12	Event C2 (The NR sidelink channel busy ratio is below a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition C2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition C2-2, as specified below, is fulfilled;

Inequality C2-1 (Entering condition)

Inequality C2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of channel busy ratio of the transmission resource pool, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR-SL for this event).

Thresh is the threshold parameter for this event (i.e. c2-Threshold as defined within reportConfigNR-SL for this event).

Ms is expressed in decimal from 0 to 1 in steps of 0.01.

Hys is expressed is in the same unit as Ms.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.13	Void

#### 5.5.4.14	Void

#### 5.5.4.15	Event D1 (Distance between UE and referenceLocation1 is above threshold1 and distance between UE and referenceLocation2 is below threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D1-1 and condition D1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D1-3 or condition D1-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D1-1 (Entering condition 1)

Ml1 - Hys > Thresh1

Inequality D1-2 (Entering condition 2)

Ml2 + Hys < Thresh2

Inequality D1-3 (Leaving condition 1)

Ml1 + Hys < Thresh1

Inequality D1-4 (Leaving condition 2)

Ml2 - Hys > Thresh2

The variables in the formula are defined as follows:

Ml1 is the distance between UE and a reference location for this event (i.e. referenceLocation1 as defined within reportConfigNR for this event), not taking into account any offsets.

Ml2 is the distance between UE and a reference location for this event (i.e. referenceLocation2 as defined within reportConfigNR for this event), not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresisLocation as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1, from a reference location configured with parameter referenceLocation1 within reportConfigNR for this event.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2, from a reference location configured with parameter referenceLocation2 within reportConfigNR for this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

NOTE:	The definition of Event D1 also applies to CondEvent D1.

#### 5.5.4.15a	Event D2 (Distance between UE and the serving cell moving reference location is above threshold1 and distance between UE and a moving reference location is below threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D2-1 and condition D2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D2-3 or condition D2-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D2-1 (Entering condition 1)

Ml1 - Hys > Thresh1

Inequality D2-2 (Entering condition 2)

Ml2 + Hys < Thresh2

Inequality D2-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D2-4 (Leaving condition 2)

Ml2 - Hys > Thresh2

The variables in the formula are defined as follows:

Ml1 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on movingReferenceLocation and the corresponding epoch time and satellite ephemeris for the serving cell broadcast in SIB19.

Ml2 is the distance between UE and a moving reference location for this event, not taking into account any offsets. The moving reference location is determined based on the parameter referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to this event.

Hys is the hysteresis parameter for this event (i.e. hysteresisLocation as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1 in reportConfigNR for this event, from a moving reference location determined based on the parameter movingReferenceLocation and the corresponding epoch time and satellite ephemeris for the serving cell broadcast in SIB19.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2 in reportConfigNR for this event, from a moving reference location determined based on the parameter referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

NOTE:	The definition of Event D2 also applies to CondEvent D2.

#### 5.5.4.16	CondEvent T1 (Time measured at UE is within a duration from threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition T1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition T1-2, as specified below, is fulfilled;

Inequality T1-1 (Entering condition)

Mt > Thresh1

Inequality T1-2 (Leaving condition)

Mt > Thresh1 + Duration

The variables in the formula are defined as follows:

Mt is the time measured at UE.

Thresh1 is the threshold parameter for this event (i.e. t1-Threshold as defined within reportConfigNR for this event).

Duration is the duration parameter for this event (i.e. duration as defined within reportConfigNR for this event).

Mt is expressed in ms.

Thresh1 is expressed in the same unit as Mt.

Duration is expressed in the same unit as Mt.

#### 5.5.4.17	Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition X1-1 and condition X1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition X1-3 or condition X1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality X1-1 (Entering condition 1)

Mr + Hys < Thresh1

Inequality X1-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality X1-3 (Leaving condition 1)

Mr – Hys > Thresh1

Inequality X1-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Mn is the measurement result of the NR cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the NR cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the NR cell).

Ocn is the cell specific offset of the NR cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the NR cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the cell.

Hys is the hysteresis parameter for this event.

Thresh1 is the threshold parameter for this event (i.e. x1-Threshold1-Relay/eventX1-SD-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. x1-Threshold2 as defined within reportConfigNR for this event).

Mr is expressed in dBm.

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mr.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.18	Event X2 (Serving L2 U2N Relay UE becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition X2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition X2-2, as specified below, is fulfilled;

Inequality X2-1 (Entering condition)

Mr + Hys < Thresh

Inequality X2-2 (Leaving condition)

Mr – Hys > Thresh

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. x2-Threshold-Relay/eventX2-SD-Threshold as defined within reportConfigNR for this event).

Mr is expressed in dBm.

Hys are expressed in dB.

Thresh is expressed in the same unit as Mr.

#### 5.5.4.19	Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition Y1-1 and condition Y1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Y1-3 or condition Y1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality Y1-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality Y1-2 (Entering condition 2)

Mr– Hys > Thresh2

Inequality Y1-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality Y1-4 (Leaving condition 2)

Mr + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mr is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. y1-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. y1-Threshold2-Relay as defined within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mr is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mr.

#### 5.5.4.20	Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition Y2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Y2-2, as specified below, is fulfilled;

Inequality Y2-1 (Entering condition)

Mr– Hys > Thresh

Inequality Y2-2 (Leaving condition)

Mr + Hys < Thresh

The variables in the formula are defined as follows:

Mr is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. y2-Threshold-Relay as defined within reportConfigInterRAT for this event).

Mr is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh is expressed in the same unit as Mr.

#### 5.5.4.20b	Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition Z1-1 and condition Z1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Z1-3 or condition Z1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality Z1-1 (Entering condition 1)

Mr + Hys < Thresh1

Inequality Z1-2 (Entering condition 2)

Mn – Hys > Thresh2

Inequality Z1-3 (Leaving condition 1)

Mr – Hys > Thresh1

Inequality Z1-4 (Leaving condition 2)

Mn + Hys < Thresh2

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Mn is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. sl-rsrp in z1-Threshold1-Relay as defined within reportConfigInterRAT if the UE measures SL-RSRP, or sd-rsrp in z1-Threshold1-Relay as defined within reportConfigInterRAT if the UE measures SD-RSRP for this event).

Thresh2 is the threshold parameter for this event (i.e. z1-Threshold2-Relay as defined within reportConfigInterRAT for this event).

Mr is expressed in dBm or dB, depending on the measurement quantity of serving L2 U2N Relay UE.

Mn is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mr.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.21	Event H1 (The Aerial UE altitude becomes higher than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H1-2, as specified below, is fulfilled;

Inequality H1-1 (Entering condition)

Ms – Hys > Thresh

Inequality H1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the Aerial UE altitude relative to the sea level.

Hys is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Ms, Hys, Thresh are expressed in meters.

#### 5.5.4.22	Event H2 (The Aerial UE altitude becomes lower than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H2-2, as specified below, is fulfilled;

Inequality H2-1 (Entering condition)

Ms + Hys < Thresh

Inequality H2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the Aerial UE altitude relative to the sea level.

Hys is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Ms, Hys, Thresh are expressed in meters.

#### 5.5.4.23	Event A3H1 (Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes higher than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A3H1-1 and condition A3H1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3H1-3 or condition A3H1-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3H1-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off

Inequality A3H1-2 (Entering condition 2)

Ms – Hys2 > Thresh

Inequality A3H1-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off

Inequality A3H1-4 (Leaving condition 2)

Ms + Hys2 < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys1 is the hysteresis parameter for this event (i.e. a3-Hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1, Ofp, Ocp, Off are expressed in dB.

Ms, Hys2, Thresh are expressed in meters.

NOTE 2:	The definition of Event A3H1 also applies to CondEvent A3H1.

#### 5.5.4.24	Event A3H2 (Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes lower than a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A3H2-1 and condition A3H2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3H2-3 or condition A3H2-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3H2-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off

Inequality A3H2-2 (Entering condition 2)

Ms + Hys2 < Thresh

Inequality A3H2-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off

Inequality A3H1-4 (Leaving condition 2)

Ms – Hys2 > Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys1 is the hysteresis parameter for this event (i.e. a3-Hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1, Ofp, Ocp, Off are expressed in dB.

Ms, Hys2, Thresh are expressed in meters.

NOTE 2:	The definition of Event A3H2 also applies to CondEvent A3H2.

#### 5.5.4.25	Event A4H1 (Neighbour becomes better than threshold1 and the Aerial UE altitude becomes higher than a threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A4H1-1 and condition A4H1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4H1-3 or condition A4H1-4, i.e. at least one of the two, as specified below, is fulfilled.

Inequality A4H1-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Thresh1

Inequality A4H1-2 (Entering condition 2)

Ms – Hys2 > Thresh2

Inequality A4H1-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Thresh1

Inequality A4H1-4 (Leaving condition 2)

Ms + Hys2 < Thresh2

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a4-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1 are expressed in dB.

Thresh1 is expressed in the same unit as Mn.

Ms, Hys2, Thresh2 are expressed in meters.

#### 5.5.4.26	Event A4H2 (Neighbour becomes better than threshold1 and the Aerial UE altitude becomes lower than a threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A4H2-1 and condition A4H2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4H2-3 or condition A4H2-4, i.e. at least one of the two, as specified below, is fulfilled.

Inequality A4H2-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Thresh1

Inequality A4H2-2 (Entering condition 2)

Ms + Hys2 < Thresh2

Inequality A4H2-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Thresh1

Inequality A4H2-4 (Leaving condition 2)

Ms – Hys2 > Thresh2

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a4-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1 are expressed in dB.

Thresh1 is expressed in the same unit as Mn.

Ms, Hys2, Thresh2 are expressed in meters.

#### 5.5.4.27	Event A5H1 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes higher than a threshold3)

The UE shall:

1>	consider the entering condition for this event to be satisfied when all of condition A5H1-1 and condition A5H1-2 and condition A5H1-3, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5H1-4 or condition A5H1-5 or condition A5H1-6, i.e. at least one of the three, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE 1:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5H1-1 (Entering condition 1)

Mp + Hys1 < Thresh1

Inequality A5H1-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys1 > Thresh2

Inequality A5H1-3 (Entering condition 3)

Ms – Hys2 > Thresh3

Inequality A5H1-4 (Leaving condition 1)

Mp – Hys1 > Thresh1

Inequality A5H1-5 (Leaving condition 2)

Mn + Ofn + Ocn + Hys1 < Thresh2

Inequality A5H1-6 (Leaving condition 3)

Ms + Hys2 < Thresh3

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a5-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh3 is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1 are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

Ms, Hys2, Thresh3 are expressed in meters.

NOTE 2:	The definition of Event A5H1 also applies to CondEvent A5H1.

#### 5.5.4.28	Event A5H2 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes lower than a threshold3)

The UE shall:

1>	consider the entering condition for this event to be satisfied when all of condition A5H2-1 and condition A5H2-2 and condition A5H2-3, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5H2-4 or condition A5H2-5 or condition A5H2-6, i.e. at least one of the three, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE 1:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5H2-1 (Entering condition 1)

Mp + Hys1 < Thresh1

Inequality A5H2-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys1 > Thresh2

Inequality A5H2-3 (Entering condition 3)

Ms + Hys2 < Thresh3

Inequality A5H2-4 (Leaving condition 1)

Mp – Hys1 > Thresh1

Inequality A5H2-5 (Leaving condition 2)

Mn + Ofn + Ocn + Hys1 < Thresh2

Inequality A5H2-6 (Leaving condition 3)

Ms – Hys2 > Thresh3

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Hys1 is the hysteresis parameter for this event (i.e. a5-Hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h2-Hysteresis as defined within reportConfigNR for this event).

Thresh3 is the threshold parameter for this event (i.e. h2-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1 are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

Ms, Hys2, Thresh3 are expressed in meters.

NOTE 2:	The definition of Event A5H2 also applies to CondEvent A5H2.

## 5.5a	Logged Measurements for Network-Side Data Collection

### 5.5a.1	Logged Measurement Configuration

#### 5.5a.1.1	General

The purpose of this procedure is to configure the UE to perform logging of measurement results while in RRC_CONNECTED. This procedure applies to UEs in RRC_CONNECTED that are capable of logged measurements for network-side data collection.

#### 5.5a.1.2	Initiation

NG-RAN initiates the logged measurement configuration procedure to UE in RRC_CONNECTED for a serving cell by sending csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell.

#### 5.5a.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5a.3.2.

### 5.5a.2	Release of Network-Side Logged Measurement Configuration

#### 5.5a.2.1	General

The purpose of this procedure is to release the logged measurement configuration for network-side data collection.

#### 5.5a.2.2	Initiation

Upon receiving csi-LoggedMeasurementConfigToReleaseList, the UE shall:

1>	for each CSI logged measurement configuration ID included in csi-LoggedMeasurementConfigToReleaseList associated with a serving cell:

2>	if the current UE configuration for the associated serving cell includes a CSI logged measurement configuration with the associated CSI logged measurement configuration ID:

3>	release the CSI logged measurement configuration.

### 5.5a.3	Measurements logging

#### 5.5a.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_CONNECTED that has a logged measurement configuration for network-side data collection.

#### 5.5a.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

The UE shall, for each entry in the VarMeasReportList:

1>	if the eventH1 or eventH2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

1>	else if the eventA3H1 or eventA3H2 or eventA4H1 or eventA4H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList associated with the same measObjectNR for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO or servingCellMO-OD:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

1>	set the servCellId within measResultServingMOList to include each NR serving cell that is configured with servingCellMO or servingCellMO-OD, if any;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

2>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>	for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting:

3>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each best non-serving cell included in the measurement report:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2, or eventA3H1, or eventA3H2, or eventA4H1, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventA3H1, or eventA3H2, or eventA4H1, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

4>	else:

5>	if SSB based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

5>	else if CSI-RS based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

4>	if results for the serving cell derived based on SSB are included:

5>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR of the serving cell;

4>	if results for the serving cell derived based on CSI-RS are included:

5>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR of the serving cell;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

6>	set the measResultNeighCellListNR within measResultServFreqListNR-SCG to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR, where availability is considered according to the measurement configuration associated with the SCG;

7>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

8>	for each best non-serving cell included in the measurement report:

9>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

1>	if the measRSSI-ReportConfig is configured within the corresponding reportConfig for this measId:

2>	set the rssi-Result to the linear average of sample value(s) provided by lower layers in the reportInterval;

2>	set the channelOccupancy to the rounded percentage of sample values which are beyond the channelOccupancyThreshold within all the sample values in the reportInterval;

1>	if the UE is acting as L2 U2N Remote UE:

2>	set the sl-MeasResultServingRelay in accordance with the following:

3>	set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the serving L2 U2N Relay UE;

3>	set the sl-RelayUE-Identity to include the Source L2 ID of the serving L2 U2N Relay;

3>	if the measurement of serving L2 U2N Relay UE is based on SL-RSRP:

4>	set the sl-MeasResult to include the SL-RSRP of the serving L2 U2N Relay UE;

4>	set the sl-MeasQuantity to sl-rsrp, if supported by the UE;

3>	else:

4>	set the sl-MeasResult to include the SD-RSRP of the serving L2 U2N Relay UE;

4>	set the sl-MeasQuantity to sd-rsrp, if supported by the UE;

NOTE 1:	In case of no data transmission from L2 U2N Relay UE to L2 U2N Remote UE, it is left to UE implementation whether to use SL-RSRP or SD-RSRP when setting the sl-MeasResultServingRelay of the serving L2 U2N Relay UE.

1>	if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	if the measurement report concerns the candidate L2 U2N Relay UE:

4>	set the sl-MeasResultsCandRelay in measResultNeighCells to include the best candidate L2 U2N Relay UEs up to maxNrofRelayMeas in accordance with the following:

5>	if the reportType is set to eventTriggered:

6>	include the L2 U2N Relay UEs included in the relaysTriggeredList as defined within the VarMeasReportList for this measId;

5>	else:

6>	include the applicable L2 U2N Relay UEs for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	for each L2 U2N Relay UE that is included in the sl-MeasResultsCandRelay:

6>	set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the concerned L2 U2N Relay UE;

6>	set the sl-RelayUE-Identity to include the Source L2 ID of the concerned L2 U2N Relay UE;

6>	set the sl-MeasResult to include the SD-RSRP of the concerned L2 U2N Relay UE;

5>	for each included L2 U2N Relay UE, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	set the sl-MeasResult to include the quantity(ies) indicated in the reportQuantityRelay within the concerned reportConfigRelay in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best L2 U2N Relay UE is included first;

6>	if the UE supports multipathRemoteUE-PC5L2 and idle/inactive relay UE reporting, and if the sl-RelayIndication is contained in the discovery message received from the concerned L2 U2N Relay UE:

7>	set the sl-RelayIndicationMP in the sl-MeasResultsCandRelay;

3>	else:

4>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

5>	if the reportType is set to eventTriggered and eventId is not set to eventD1 or eventD2 or eventH1 or eventH2:

6>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

5>	else:

6>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	for each cell that is included in the measResultNeighCells, include the physCellId;

5>	if the reportType is set to eventTriggered or periodical:

6>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

7>	if the measObject associated with this measId concerns NR:

8>	if rsType in the associated reportConfig is set to ssb:

9>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

9>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

8>	else if rsType in the associated reportConfig is set to csi-rs:

9>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

9>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	if the measObject associated with this measId concerns E-UTRA:

8>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

7>	if the measObject associated with this measId concerns UTRA-FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

8>	set the measResult to include the quantity(ies) indicated in the reportQuantityUTRA-FDD within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

6>	if reportType is set to eventTriggered and reportOnBestCellChange is configured:

7>	if reportOnBestCellChange is set to n1:

8>	set the reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the first cell listed in the measResultNeighCells;

7>	if reportOnBestCellChange is set to n2:

8>	set the first cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the first cell listed in the measResultNeighCells and the second cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the second cell listed in the measResultNeighCells, if available;

2>	else:

3>	if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>	if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), trackingAreaList (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>	include frequencyBandList if available;

5>	for each PLMN-IdentityInfo in plmn-IdentityInfoList:

6>	if the gNB-ID-Length is broadcast:

7>	include gNB-ID-Length;

4>	if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList;

5>	for each NPN-IdentityInfo in NPN-IdentityInfoList:

6>	if the gNB-ID-Length is broadcast:

7>	include gNB-ID-Length;

5>	include cellReservedForOtherUse if available;

4>	if nr-CGI-Reporting-HSDN is supported by the UE and if the hsdn-Cell for the concerned cell has been obtained:

5>	include hsdn-Cell;

4>	else if MIB indicates the SIB1 is not broadcast:

5>	include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

3>	if the cell indicated by cellForWhichToReportCGI is an E-UTRA cell:

4>	if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained:

5>	include in the cgi-Info-EPC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to EPC;

4>	if the UE is E-UTRA/5GC capable and all mandatory fields of the cgi-Info-5GC for the concerned cell have been obtained:

5>	include in the cgi-Info-5GC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to 5GC;

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	include the freqBandIndicator;

5>	if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

5>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

4>	if eutra-CGI-Reporting-HSDN is supported by the UE and if the hsdn-Cell for the concerned cell has been obtained:

5>	include hsdn-Cell;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and if enteringLeavingReport is configured:

2>	for each cell that is included in measResultNeighCells:

3>	if the measObject associated with this measId concerns NR:

4>	if event entry condition for the event that triggered this measurement report has been fulfilled for the cell and the cell has just been included to cellsTriggeredList:

5>	set entering to true for the concerned NR cell;

2>	if the field cellsMetLeavingCond within the VarMeasReportList for this measId is not empty:

3>	set cellsMetReportOnLeaveList to include the cell(s) in cellsMetLeavingCond;

1>	if the corresponding measObject concerns NR:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigNR for this measId:

3>	set the measResultSFTD-NR in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrp-Result to the RSRP of the NR PSCell derived based on SSB;

2>	else if the reportSFTD-NeighMeas is included within the corresponding reportConfigNR for this measId:

3>	for each applicable cell which measurement results are available, include an entry in the measResultCellListSFTD-NR and set the contents as follows:

4>	set physCellId to the physical cell identity of the concerned NR neighbour cell.

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true:

5>	set rsrp-Result to the RSRP of the concerned cell derived based on SSB;

1>	else if the corresponding measObject concerns E-UTRA:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigInterRAT for this measId:

3>	set the measResultSFTD-EUTRA in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrpResult-EUTRA to the RSRP of the EUTRA PSCell;

1>	if average uplink PDCP delay values are available:

2>	set the ul-PDCP-DelayValueResultList to include the corresponding average uplink PDCP delay values;

1>	if PDCP excess delay measurements are available:

2>	set the ul-PDCP-ExcessDelayResultList to include the corresponding PDCP excess delay measurements;

1>	if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows:

2>	include the locationTimestamp;

2>	include the locationCoordinate, if available;

2>	include the velocityEstimate, if available;

2>	include the locationError, if available;

2>	include the locationSource, if available;

2>	if available, include the gnss-TOD-msec,

1>	if the coarseLocationRequest is set to true in the corresponding reportConfig for this measId:

2>	include coarseLocationInfo, if available;

1>	if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultWLAN, in order of decreasing RSSI for WLAN APs;

1>	if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the BT-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultBT, in order of decreasing RSSI for Bluetooth beacons;

1>	if the includeSensor-Meas is configured in the corresponding reportConfig for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the sensor-MeasurementInformation;

2>	if available, include the sensor-MotionInformation;

1>	if the includeAltitudeUE is set to true in the corresponding reportConfig for this measId:

2>	set the altitudeUE to include the altitude of the UE;

1>	if there is at least one applicable transmission resource pool for NR sidelink communication/discovery (for measResultsSL):

2>	set the measResultsListSL to include the CBR measurement results in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the transmission resource pools included in the poolsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable transmission resource pools for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

3>	if the corresponding measObject concerns NR sidelink communication/discovery, then for each transmission resource pool to be reported:

4>	set the sl-poolReportIdentity to the identity of this transmission resource pool;

4>	set the sl-CBR-ResultsNR to the CBR measurement results on PSSCH and PSCCH of this transmission resource pool provided by lower layers, if available;

NOTE 1:	Void.

1>	if there is at least one applicable CLI measurement resource to report:

2>	if the reportType is set to cli-EventTriggered or cli-Periodical:

3>	set the measResultCLI to include the most interfering SRS resources or most interfering CLI-RSSI resources up to maxReportCLI in accordance with the following:

4>	if the reportType is set to cli-EventTriggered:

5>	if trigger quantity is set to srs-RSRP i.e. i1-Threshold is set to srs-RSRP:

6>	include the SRS resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

5>	if trigger quantity is set to cli-RSSI i.e. i1-Threshold is set to cli-RSSI:

6>	include the CLI-RSSI resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	if reportQuantityCLI is set to srs-rsrp:

6>	include the applicable SRS resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	else:

6>	include the applicable CLI-RSSI resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each SRS resource that is included in the measResultCLI:

5>	include the srs-ResourceId;

5>	set srs-RSRP-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering SRS resource is included first;

4>	for each CLI-RSSI resource that is included in the measResultCLI:

5>	include the rssi-ResourceId;

5>	set cli-RSSI-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering CLI-RSSI resource is included first;

1>	if there is at least one applicable UE Rx-Tx time difference measurement to report:

2>	set measResultRxTxTimeDiff to the latest measurement result;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical or cli-Periodical or rxTxPeriodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1>	if the measurement reporting was configured by a sl-ConfigDedicatedNR received within the RRCConnectionReconfiguration:

2>	submit the MeasurementReport message to lower layers for transmission via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified TS 36.331 [10], clause 5.6.28;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured and the SCG is not deactivated:

3>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>	else:

3>	submit the MeasurementReport message via E-UTRA embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the measurement configuration that triggered this measurement report is associated with the SCG:

3>	if SRB3 is configured and the SCG is not deactivated:

4>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

3>	else:

4>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered or reportOnScellActivation:

2>	consider the trigger quantity as the sorting quantity if available, otherwise RSRP as sorting quantity if available, otherwise RSRQ as sorting quantity if available, otherwise SINR as sorting quantity;

1>	if reportType is set to periodical:

2>	if a single reporting quantity is set to true in reportQuantityRS-Indexes;

3>	consider the configured single quantity as the sorting quantity;

2>	else:

3>	if rsrp is set to true;

4>	consider RSRP as the sorting quantity;

3>	else:

4>	consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and if absThreshSS-BlocksConsolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

3>	if includeBeamMeasurements is set to true, include the SS/PBCH based measurement results for the quantities in reportQuantityRS-Indexes for each SS/PBCH block index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and, if absThreshCSI-RS-Consolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation;

3>	if includeBeamMeasurements is set to true, include the CSI-RS based measurement results for the quantities in reportQuantityRS-Indexes for each CSI-RS index.

#### 5.5.5.3	Sorting of cell measurement results

The UE shall determine the sorting quantity according to parameters of the reportConfig associated with the measId that triggered the reporting:

1>	if the reportType is set to eventTriggered:

2>	for an NR cell, consider the quantity used in the aN-Threshold (for eventA1, eventA2, eventA4, eventA4H1 and eventA4H2) or in the a5-Threshold2 (for eventA5, eventA5H1 and eventA5H2) or in the aN-Offset (for eventA3, eventA3H1, eventA3H2 and eventA6) or in the x1-Threshold2 (for eventX1) as the sorting quantity;

2>	for an E-UTRA cell, consider the quantity used in the bN-ThresholdEUTRA as the sorting quantity;

2>	for an UTRA-FDD cell, consider the quantity used in the bN-ThresholdUTRA-FDD as the sorting quantity;

2>	for a candidate L2 U2N Relay UE, consider the y1-Threshold2-Relay (for eventY1-Relay) or y2-Threshold-Relay (for eventY2-Relay) or z1-Threshold2-Relay (for eventZ1) as the sorting quantity;

1>	if the reportType is set to periodical:

2>	determine the sorting quantity according to reportQuantityCell for an NR cell, and according to reportQuantity for an E-UTRA cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	if rsrp is set to true;

5>	consider RSRP as the sorting quantity;

4>	else:

5>	consider RSRQ as the sorting quantity;

2>	determine the sorting quantity according to reportQuantityUTRA-FDD for UTRA-FDD cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	consider RSCP as the sorting quantity.

2>	for a candidate L2 U2N Relay UE, consider the reportQuantityRelay as the sorting quantity;

### 5.5.6	Location measurement indication

#### 5.5.6.1	General

Figure 5.5.6.1-1: Location measurement indication

The purpose of this procedure is to indicate to the network that the UE is going to start/stop location related measurements towards E-UTRA or NR which require measurement gaps or start/stop detection of subframe and slot timing towards E-UTRA (eutra-FineTimingDetection) which requires measurement gaps. UE shall initiate this procedure only after successful AS security activation.

NOTE:	It is a network decision to configure the measurement gap.

#### 5.5.6.2	Initiation

The UE shall:

1>	if and only if upper layers indicate to start performing location measurements towards E-UTRA or NR or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient:

2>	if preconfigured measurement gaps for positioning and posMG-Request are configured and the UE considers that at least one of the preconfigured measurement gaps for positioning is sufficient for the location measurement when activated:

3>	trigger the lower layers to initiate the measurement gap activation request using UL MAC CE as specified in TS 38.321 [3];

2> else:

3>	initiate the procedure to indicate start as specified in clause 5.5.6.3;

NOTE 1:	The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient.

NOTE 1a:	When indication is received from upper layers for performing location measurement and there is pre-configured measurement gap configured (not preconfigured measurement gap for positioning), the UE considers this preconfigured measurement gap to be not sufficient if the measurement gap is not considered to be always activated according to clause 9.1.7.2 of TS 38.133 [14].

1>	if and only if upper layers indicate to stop performing location measurements towards E-UTRA or NR or stop subframe and slot timing detection towards E-UTRA:

2>	if there is no activated preconfigured measurement gap for positioning:

3>	if there is previously triggered UL MAC CE transmission for the measurement gap activation for positioning:

4>	indicate to the lower layers to cancel the triggered UL MAC CE transmission for the measurement gap activation as specified in TS 38.321 [3];

2>	else if there is activated preconfigured measurement gap for positioning:

3>	trigger the lower layers to deactivate all the activated measurement gap(s) for positioning as specified in TS 38.321 [3].

2>	if there is configured measurement gap used for positioning and the measurement gap is not the activated preconfigured measurement gap for positioning:

3>	initiate the procedure to indicate stop as specified in 5.5.6.3.

NOTE 2:	The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

#### 5.5.6.3	Actions related to transmission of LocationMeasurementIndication message

The UE shall set the contents of LocationMeasurementIndication message as follows:

1>	if the procedure is initiated to indicate start of location related measurements:

2>	if the procedure is initiated for RSTD measurements towards E-UTRA:

3>	set the measurementIndication to the eutra-RSTD according to the information received from upper layers;

2>	else if the procedure is initiated for positioning measurement towards NR:

3>	set the measurementIndication to the nr-PRS-Measurement according to the information received from upper layers;

1>	else if the procedure is initiated to indicate stop of location related measurements:

2>	set the measurementIndication to the value release;

1>	if the procedure is initiated to indicate start of subframe and slot timing detection towards E-UTRA:

2>	set the measurementIndication to the value eutra-FineTimingDetection;

1>	else if the procedure is initiated to indicate stop of subframe and slot timing detection towards E-UTRA:

2>	set the measurementIndication to the value release;

1>	submit the LocationMeasurementIndication message to lower layers for transmission, upon which the procedure ends.

## 5.5a	Logged Measurements

### 5.5a.1	Logged Measurement Configuration

#### 5.5a.1.1	General

Figure 5.5a.1.1-1: Logged measurement configuration

The purpose of this procedure is to configure the UE to perform logging of measurement results while in RRC_IDLE and RRC_INACTIVE. The procedure applies to logged measurements capable UEs that are in RRC_CONNECTED.

NOTE:	NG-RAN may retrieve stored logged measurement information by means of the UE information procedure.

#### 5.5a.1.2	Initiation

NG-RAN initiates the logged measurement configuration procedure to UE in RRC_CONNECTED by sending the LoggedMeasurementConfiguration message.

#### 5.5a.1.3	Reception of the LoggedMeasurementConfiguration by the UE

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	If the UE is in SNPN access mode:

2>	if the LoggedMeasurementConfiguration message includes snpn-ConfigList:

3>	set the snpn-ConfigID-List in VarLogMeasReport to include the current registered SNPN identity as well as SNPN identities in snpn-Config-List;

2>	else:

3>	set the snpn-ConfigID-List in VarLogMeasReport to include the current registered SNPN identity;

1>	else if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-NameList, if included, in VarLogMeasConfig;

1>	store the received sensor-NameList, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

1>	store the received sigLoggedMeasType, if included, in VarLogMeasReport;

1>	store the received earlyMeasIndication, if included, in VarLogMeasConfig;

1>	store the received AreaConfigurationNTN-List, if included, in VarLogMeasConfig;

#### 5.5a.1.4	T330 expiry

Upon expiry of T330 the UE shall:

1>	release VarLogMeasConfig;

The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry.

### 5.5a.2	Release of Logged Measurement Configuration

#### 5.5a.2.1	General

The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information.

#### 5.5a.2.2	Initiation

The UE shall initiate the procedure upon receiving a logged measurement configuration in same or another RAT. The UE shall also initiate the procedure upon power off or upon deregistration.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

### 5.5a.3	Measurements logging

#### 5.5a.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_IDLE and RRC_INACTIVE that has a logged measurement configuration. The actual process of logging within the UE, takes place in RRC_IDLE state could continue in RRC_INACTIVE state or vice versa.

#### 5.5a.3.2	Initiation

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if AreaConfigurationNTN-List is included in VarLogMeasConfig:

2>	if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or

2>	if location information is not available:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

1>	if not suspended, perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by cag-ConfigList in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by snpn-ConfigList in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	upon transition from any cell selection state to camped normally state in NR:

4>	if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the current camping cell is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	perform the logging;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the current serving cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig;

5>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo or any inter-RAT frequency during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

NOTE 0:	If the UE detected and resolved IDC problem during the last logging interval, the UE does not suspend measurement logging from the next logging interval and does not suspend measurement logging for the last logging interval.

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority;

5>	set the reselectedCellId to the cell UE reselected after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority before transition to any cell selection state;

4>	if the reportType is set to eventTriggered in the VarLogMeasConfig; and

4>	if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport or if the registered SNPN identity at the time of entering the any cell selection state is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

4>	else if the reportType is set to periodical in the VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority;

5>	set the reselectedCellId to the cell UE reselected after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority, if it is different from servCellIdentity;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

4>	include measurement results for at most 6 neighbouring cells on the NR serving frequency and for at most 3 cells per NR neighbouring frequency and for the NR neighbouring frequencies in accordance with the following:

5>	if interFreqTargetInfo is included in VarLogMeasConfig:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and either in measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and SIB4;

5>	else:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in either measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in SIB4;

4>	include measurement results for at most 3 neighbours per inter-RAT frequency in accordance with the following:

5>	if earlyMeasIndication is included in VarLogMeasConfig:

6>	include measurement results for inter-RAT neighbouring frequencies that are included in either measIdleCarrierListEUTRA (within the VarMeasIdleConfig) or SIB5;

5>	else:

6>	include measurement results for inter-RAT frequencies that are included in SIB5;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

NOTE 2:	For logging the measurements on frequencies (indicated in measIdleCarrierListNR/ measIdleCarrierListEUTRA) in the logged measurement, the qualityThreshold in measIdleConfig should not be applied, and how the UE logs the measurements on the frequencies is left to the UE implementation.

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

## 5.5b	Application Layer Measurements in RRC_IDLE/RRC_INACTIVE

### 5.5b.1	Handling of Application Layer Measurements in RRC_IDLE/RRC_INACTIVE

#### 5.5b.1.1	General

This procedure specifies the UE actions for a UE in RRC_IDLE and RRC_INACTIVE that has an application layer measurement configuration with appLayerIdleInactiveConfig configured. The procedure includes area handling, storing of application layer measurement report containers and cell re-selection to LTE.

#### 5.5b.1.2	Initiation

While in RRC_INACTIVE and RRC_IDLE state, the UE shall:

1>	store any previously or subsequently received application layer measurement report containers associated with the measConfigAppLayerId for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers;

NOTE:	The UE continues storing application layer measurement report containers for MBS broadcast service in RRC_CONNECTED as long as stored application layer measurement configurations/reports have not been retrieved.

1>	if the memory reserved for storing application layer measurement report containers becomes full:

2>	if the reports are associated with a configuration including appLayerMeasPriority:

3>	discard reports in priority order where reports with the lowest priority are discarded first, and among reports with equal associated priority, discard reports in the order they were received, where older reports are discarded first;

2>	else:

3>	discard reports in the order they were received, where older reports are discarded first;

1>	if qoe-AreaScope is configured:

2>	if the cell which the UE selects when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being inside the area;

2>	if the cell which the UE selects when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is not part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being outside the area.

2>	if the UE re-selects to a new cell which is part of the area indicated by qoe-AreaScope and the previous cell was not part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being inside the area;

2>	if the UE re-selects to a new cell which is not part of the area indicated by qoe-AreaScope and the previous cell was part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being outside the area.

1>	if the UE re-selects to an E-UTRA cell:

2>	inform upper layers about the release of all application layer measurement configurations;

2>	release all application layer measurement configurations including their fields in UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

2>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

2>	consider itself not to be configured to send application layer measurement reports.

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

This clause describes how the UE compiles and transfers its UE capability information upon receiving a UECapabilityEnquiry from the network.

Figure 5.6.1.1-1: UE capability transfer

#### 5.6.1.2	Initiation

The network initiates the procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information. The network should retrieve UE capabilities only after AS security activation. Network does not forward UE capabilities that were retrieved before AS security activation to the CN.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra-nr:

2> if the UE supports (NG)EN-DC or NE-DC:

3>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-MRDC-Capability and with the rat-Type set to eutra-nr;

3>	include the supportedBandCombinationList and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra:

2>	if the UE supports E-UTRA:

3>	include in the ue-CapabilityRAT-ContainerList a ue-CapabilityRAT-Container of the type UE-EUTRA-Capability associated with the terrestrial network and with the rat-Type set to eutra as specified in TS 36.331 [10], clause 5.6.3.3, according to the capabilityRequestFilter, if received;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to utra-fdd:

2>	if the UE supports UTRA-FDD:

3>	include the UE radio access capabilities for UTRA-FDD within a ue-CapabilityRAT-Container and with the rat-Type set to utra-fdd;

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

2>	consider the maximum number of UL segments the UE is allowed to use when segmenting the UECapabilityInformation message is 16;

2>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

1>	else if the RRC message segmentation is enabled based on the field rrc-MaxCapaSegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

2>	consider the maximum number of UL segments the UE is allowed to use when segmenting the UECapabilityInformation message to be the value indicated by rrc-MaxCapaSegAllowed;

2>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

#### 5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon, and rrc-SegAllowed) as defined in TS 36.331 [10], where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

NOTE 2:	In (NG)EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations.

NOTE 3:	If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

NOTE 3a:	For the usage of rrc-MaxCapaSegAllowed, if the NR or E-UTRA network does not request UE capabilities for nr, eutra-nr and eutra in a single enquiry, the feature set IDs might not be consistent across feature sets, feature set combinations and band combinations in all three UE capability containers.

The UE shall:

1>	if the UE is an eRedCap UE and the UE supports eRedCapIgnoreCapabilityFiltering:

2> ignore the value received in frequencyBandListFilter;

2>	in the remainder of this procedure assume frequencyBandListFilter indicates all the supported bands of the UE;

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell or SUL according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

NOTE 5:	Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR and srs-SwitchingAffectedBandsListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many NR-only band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR and srs-SwitchingAffectedBandsListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

NOTE 6:	This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container.

2>	if sidelinkRequest is received:

3>	for a sidelink band combination the UE included in supportedBandCombinationListSidelinkEUTRA-NR, supportedBandCombinationListSL-RelayDiscovery, supportedBandCombinationListSL-U2U-RelayDiscovery or supportedBandCombinationListSL-NonRelayDiscovery:

4>	if the UE supports partial sensing for a band of the sidelink band combination, include the partial sensing capabilities for the band using the sl-TransmissionMode2-PartialSensing-r17;

3>	set sidelinkRequested to true;

2>	include into featureSets the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received;

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR, srs-SwitchingTimesListEUTRA and srs-SwitchingAffectedBandsListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many E-UTRA-NR band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR, srs-SwitchingTimesListEUTRA and srs-SwitchingAffectedBandsListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

1>	else (if the requested rat-Type is eutra):

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

NOTE 7:	This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSetsEUTRA (in the UE-EUTRA-Capability) the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	if the UE is an eRedCap UE and the UE supports eRedCapIgnoreCapabilityFiltering, i.e., it assumes frequencyBandListFilter to be the filter containing all the supported bands of the UE:

2>	include all the supported bands of the UE in the field appliedFreqBandListFilter, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	else:

2>	include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	if the network included ue-CapabilityEnquiryExt:

2>	include the received ue-CapabilityEnquiryExt in the field receivedFilters;

#### 5.6.1.5	Void

## 5.7	Other

### 5.7.1	DL information transfer

#### 5.7.1.1	General

Figure 5.7.1.1-1: DL information transfer

The purpose of this procedure is to transfer NAS dedicated information from NG-RAN to a UE in RRC_CONNECTED or to a UE in RRC_INACTIVE during SDT, or to transfer F1-C related information from IAB Donor-CU to IAB-DU via the collocated IAB-MT in RRC_CONNECTED.

#### 5.7.1.2	Initiation

The network initiates the DL information transfer procedure whenever there is a need to transfer NAS dedicated information, or F1-C related information to an IAB-node. The network initiates the DL information transfer procedure by sending the DLInformationTransfer message.

#### 5.7.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

1>	if dedicatedNAS-Message is included:

2>	forward dedicatedNAS-Message to upper layers.

1>	if referenceTimeInfo is included:

2>	calculate the reference time based on the time, referenceSFN and timeInfoType if it is included;

2>	calculate the uncertainty of the reference time based on the uncertainty, if uncertainty is included;

2>	inform upper layers of the reference time and, if uncertainty is included, of the uncertainty;

2>	ignore all further referenceTimeInfo received in SIB9, if any.

1>	if sib9Fallback is included:

2>	apply referenceTimeInfo in SIB9.

1>	if rxTxTimeDiff-gNB is included:

2>	calculate the propagation delay based on the UE Rx-Tx time difference measurement and the received Rx-Tx time difference measurement at the gNB;

2>	inform upper layers of the propagation delay.

1>	if ta-PDC is set to activate:

2>	inform upper layers of the propagation delay determined by the accumulated Timing Advance commands.

1>	if clockQualityDetailsLevel is included:

2>	store the eventID-TSS as the storedEventID within VarTSS-Info;

2>	if the UE is NPN capable and the cell is an NPN-only cell:

3>	derive a gNB identity from gNB-ID-Length and cellIdentity of the first NPN-IdentityInfo list entry of NPN-IdentityInfoList in SIB1, and derive the plmn-Identity of the first NPN-Identity list entry of NPN-IdentityList in SIB1 included in the first NPN-IdentityInfo list entry of NPN-IdentityInfoList in SIB1, as defined in TS 38.413 [42];

2>	else:

3>	derive a gNB identity from gNB-ID-Length and cellIdentity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, and derive the plmn-Identity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, as defined in TS 38.413 [42];

2>	store the global gNB identity derived from SIB1 as the storedGlobalGnbID within VarTSS-Info;

2>	forward clockQualityDetailsLevel to upper layers.

Upon receiving DLInformationTransfer message, the IAB-MT shall:

1>	if dedicatedInfoF1c is included:

2>	forward dedicatedInfoF1c to the collocated IAB-DU.

### 5.7.1a	DL information transfer for MR-DC

#### 5.7.1a.1	General

Figure 5.7.1a.1-1: DL information transfer MR-DC

The purpose of this procedure is to transfer RRC messages from the network to the UE over SRB3 (e.g. an NR RRC reconfiguration message including reconfigurationWithSync, an E-UTRA RRC connection reconfiguration message including mobilityControlInfo, an RRC connection release message, a MobilityFromNRCommand message, or a MobilityFromEUTRACommand message) during fast MCG link recovery.

#### 5.7.1a.2	Initiation

The network initiates this procedure whenever there is a need to transfer an RRC message during fast MCG link recovery.

#### 5.7.1a.3	Actions related to reception of DLInformationTransferMRDC message

Upon receiving the DLInformationTransferMRDC, the UE shall:

1>	if the RRCReconfiguration message is included in dl-DCCH-MessageNR:

2>	perform the RRC reconfiguration procedure according to 5.3.5.3;

1>	else if the RRCRelease message is included in dl-DCCH-MessageNR:

2>	perform the RRC release procedure according to 5.3.8;

1>	else if the MobilityFromNRCommand message is included in the dl-DCCH-MessageNR:

2>	perform the mobility from NR procedure according to 5.4.3.3;

1>	else if the E-UTRA RRCConnectionReconfiguration message is included in dl-DCCH-MessageEUTRA:

2>	perform the RRC connection reconfiguration procedure as specified in TS 36.331 [10], clause 5.3.5.4;

1>	else if the E-UTRA RRCConnectionRelease message is included in dl-DCCH-MessageEUTRA:

2>	perform the RRC connection release as specified in TS 36.331 [10], clause 5.3.8;

1>	else if the MobilityFromEUTRACommand message is included in the dl-DCCH-MessageEUTRA:

2>	perform the mobility from E-UTRA procedure as specified in TS 36.331 [10], clause 5.4.3.3;

### 5.7.2	UL information transfer

#### 5.7.2.1	General

Figure 5.7.2.1-1: UL information transfer

The purpose of this procedure is to transfer NAS dedicated information from the UE to the network in RRC_CONNECTED or in RRC_INACTIVE during SDT, or to transfer F1-C related information from IAB-DU to IAB Donor-CU via the collocated IAB-MT in RRC_CONNECTED.

#### 5.7.2.2	Initiation

A UE in RRC_CONNECTED or a UE in RRC_INACTIVE during SDT initiates the UL information transfer procedure whenever there is a need to transfer NAS dedicated information. The UE initiates the UL information transfer procedure by sending the ULInformationTransfer message. In addition, an IAB-MT in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer F1-C related information. When F1-C related information has to be transferred, the IAB-MT shall initiate the procedure only if SRB2 or split SRB2 is established.

#### 5.7.2.3	Actions related to transmission of ULInformationTransfer message

The UE shall set the contents of the ULInformationTransfer message as follows:

1>	if the upper layer provides NAS PDU:

2>	set the dedicatedNAS-Message to include the information received from upper layers;

1>	for the IAB-MT, if there is a need to transfer F1-C related information:

2>	include the dedicatedInfoF1c;

1>	submit the ULInformationTransfer message to lower layers for transmission, upon which the procedure ends.

#### 5.7.2.4	Failure to deliver ULInformationTransfer message

The UE shall:

1>	if AS security is not started and radio link failure occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers; or

1>	if PDCP re-establishment or release/addition (e.g due to key refresh upon PCell or PSCell change, or RRC connection re-establishment, or failure of resume procedure initiated for SDT) occurs on an SRB on which ULInformationTransfer messages were submitted for transmission but successful delivery of these messages was not confirmed by lower layers:

2>	inform upper layers about the possible failure to deliver the information contained in the concerned ULInformationTransfer messages, unless the messages only include dedicatedInfoF1c.

### 5.7.2a	UL information transfer for MR-DC

#### 5.7.2a.1	General

Figure 5.7.2a.1-1: UL information transfer MR-DC

The purpose of this procedure is to transfer MR-DC dedicated information from the UE to the network e.g. the NR or E-UTRA RRC MeasurementReport, FailureInformation, UEAssistanceInformation, RRCReconfigurationComplete, MCGFailureInformation, or IABOtherInformation message.

#### 5.7.2a.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer for MR-DC procedure whenever there is a need to transfer MR-DC dedicated information. I.e. the procedure is not used during an RRC connection reconfiguration involving NR or E-UTRA connection reconfiguration, in which case the MR DC information is piggybacked to the RRCReconfigurationComplete message, except in the case the UE executes an intra-SN CPC.

#### 5.7.2a.3	Actions related to transmission of ULInformationTransferMRDC message

The UE shall set the contents of the ULInformationTransferMRDC message as follows:

1>	if there is a need to transfer MR-DC dedicated information related to NR:

2>	set the ul-DCCH-MessageNR to include the NR MR-DC dedicated information to be transferred (e.g., NR RRC MeasurementReport, UEAssistanceInformation, FailureInformation, RRCReconfigurationComplete, MCGFailureInformation, or IABOtherInformation message);

1>	else if there is a need to transfer MR-DC dedicated information related to E-UTRA:

2>	set the ul-DCCH-MessageEUTRA to include the E-UTRA MR-DC dedicated information to be transferred (e.g., E-UTRA RRC MeasurementReport, or MCGFailureInformation message);

1>	submit the ULInformationTransferMRDC message to lower layers for transmission, upon which the procedure ends.

### 5.7.2b	UL transfer of IRAT information

#### 5.7.2b.1	General

Figure 5.7.2b.1-1: UL transfer of IRAT information

The purpose of this procedure is to transfer from the UE to NR MCG dedicated information terminated at the NR MCG but specified by another RAT e.g. the E-UTRA MeasurementReport message, the E-UTRA SidelinkUEInformation message or the E-UTRA UEAssistanceInformation message. The specific information transferred in this message is set in accordance with:

-	the procedure specified in 5.6.10 of TS 36.331 [10] for E-UTRA UEAssistanceInformation message;

-	the procedure specified in 5.10.2 of TS 36.331 [10] for E-UTRA SidelinkUEInformation message;

-	the procedure specified in 5.5.5 of TS 36.331 [10] for E-UTRA MeasurementReport Message.

#### 5.7.2b.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer dedicated inter-RAT information as specified in TS 36.331 [10].

#### 5.7.2b.3	Actions related to transmission of ULInformationTransferIRAT message

The UE shall set the contents of the ULInformationTransferIRAT message as follows:

1>	if there is a need to transfer dedicated LTE information related to V2X sidelink communications:

2>	set the ul-DCCH-MessageEUTRA to include the V2X sidelink communication information to be transferred (e.g. the E-UTRA RRC MeasurementReport message, the E-UTRA RRC SidelinkUEInformation message, or the E-UTRA RRC UEAssistanceInformation message);

1>	submit the ULInformationTransferIRAT message to lower layers for transmission, upon which the procedure ends;

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, beam failure of the PSCell while the SCG is deactivated, failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3, SCG integrity check failure, and consistent uplink LBT failures on PSCell for operation with shared spectrum channel access.

#### 5.7.3.2	Initiation

A UE initiates the procedure to report SCG failures when neither MCG nor SCG transmission is suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with clause 5.3.10.3;

1>	upon detecting beam failure of the PSCell while the SCG is deactivated, in accordance with TS 38.321[3];

1>	upon reconfiguration with sync failure of the SCG, in accordance with clause 5.3.5.8.3;

1>	upon SCG configuration failure, in accordance with clause 5.3.5.8.2;

1>	upon integrity check failure indication from SCG lower layers concerning SRB3.

Upon initiating the procedure, the UE shall:

1>	if the procedure was not initiated due to beam failure of the PSCell while the SCG is deactivated:

2>	suspend SCG transmission for all SRBs, DRBs and, if any, BH RLC channels;

2>	reset SCG MAC;

1>	stop T304 for the SCG, if running;

1>	stop conditional reconfiguration evaluation for CPC, CPA, or subsequent CPAC, if configured;

1>	stop conditional reconfiguration evaluation for each CHO configuration that includes condExecutionCondPSCell, if configured;

1>	if the UE is in (NG)EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a.

1>	else:

2>	initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5.

#### 5.7.3.3	Failure type determination for (NG)EN-DC

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationNR message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to T312 expiry:

2>	set the failureType as any value and set the failureType-v1610 as t312-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailureSCG;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as randomAccessProblem and set the failureType-v1610 as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to SRB3 integrity check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to consistent uplink LBT failures:

2>	set the failureType as any value and set the failureType-v1610 as scg-lbtFailure;

1> else if connected as an IAB-node and the SCGFailureInformationNR is initiated due to the reception of a BH RLF indication on BAP entity from the SCG:

2>	set the failureType as any value and set failureType-v1610 as bh-RLF.

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to beam failure of the PSCell while the SCG is deactivated:

2>	set the failureType as any value and set failureType-v1610 as beamFailure.

#### 5.7.3.4	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failure as follows:

1>	for each MeasObjectNR configured on NR SCG for which a measId is configured and measurement results are available:

2>	include an entry in measResultPerMOList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

3>	if the UE supports SCG failure information for mobility robustness optimization for conditional PSCell change, for each neighbour cell, if any, included in measResultSCG-Failure:

4>	if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the secondaryCellGroup in the MCG VarConditionalReconfig (for inter-SN CPC in NR-DC) or SCG VarConditionalReconfig (for intra-SN CPC) at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change failure):

5>	if the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell corresponds to a fulfilled execution condition at the moment of SCG failure; or

5>	if the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, if available, corresponds to a fulfilled execution condition at the moment of SCG failure:

6>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell or to the execution condition condSecondEvent corresponding to the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, whichever execution condition was fulfilled first in time;

6>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled first in time, and the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the condExecutionCond or condExecutionCondSCG associated to the neighbour cell were fulfilled;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

1>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the NR SCG.

#### 5.7.3.5	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to T312 expiry:

2>	set the failureType as other and set the failureType-v1610 as t312-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailureSCG;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as other and set the failureType-v1610 as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures:

2>	set the failureType as other and set the failureType-v1610 as scg-lbtFailure;

1>	else if connected as an IAB-node and the SCGFailureInformation is initiated due to the reception of a BH RLF indication on BAP entity from the SCG:

2>	set the failureType as other and set failureType-v1610 as bh-RLF;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to beam failure of the PSCell while the SCG is deactivated:

2>	set the failureType as other and set failureType-v1610 as beamFailure;

1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4;

1>	for each MeasObjectNR configured by a MeasConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	if the UE supports SCG failure information for mobility robustness optimization for conditional PSCell change or addition, for each neighbour cell, if any, included in measResultListNR in measResultFreqList:

4>	if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the secondaryCellGroup in the MCG VarConditionalReconfig (for CPA or inter-SN CPC in NR-DC) or SCG VarConditionalReconfig (for intra-SN CPC) at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change or addition failure):

5>	if the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell corresponds to a fulfilled execution condition at the moment of SCG failure; or

5>	if the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, if available, corresponds to a fulfilled execution condition at the moment of SCG failure:

6>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell or to the execution condition condSecondEvent corresponding to the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, whichever execution condition was fulfilled first in time;

6>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled first in time, and the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the condExecutionCond or condExecutionCondSCG associated to the neighbour cell were fulfilled;

3>	if the UE supports SCG failure information for mobility robustness optimization for conditional handover with candidate SCG, for each neighbour cell, if any, included in measResultListNR in measResultFreqList:

4>	if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is associated with both condExecutionCond and condExecutionCondPSCell in the MCG VarConditionalReconfig at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change or addition failure):

5>	if the first entry of condExecutionCond associated to the neighbour cell corresponds to a fulfilled execution condition at the moment of SCG failure; or

5>	if the second entry of condExecutionCond associated to the neighbour cell, if available, corresponds to a fulfilled execution condition at the moment of SCG failure:

6>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of condExecutionCond associated to the neighbour cell or to the execution condition condSecondEvent corresponding to the second entry of condExecutionCond associated to the neighbour cell, whichever execution condition was fulfilled first in time;

6>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in condExecutionCond associated to the neighbour cell that was fulfilled first in time, and the point in time of fulfilling the condition in condExecutionCond associated to the neighbour cell that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the condExecutionCond associated to the neighbour cell were fulfilled;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

NOTE 2:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the SCG failure;

2>	else if all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled:

3>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

3>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

1>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the NR MCG.

1>	if the UE supports SCG failure for mobility robustness optimization:

2>	if the failureType is set to synchReconfigFailureSCG; or

2>	if the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running:

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5.

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the target PSCell of the failed PSCell change or failed PSCell addition;

3>	if the failure occurred during a subsequent CPC:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

3>	else:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

3>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

2>	else:

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the PSCell in which the SCG failure was declared;

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

4>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

4>	if the failure occurred after a subsequent CPC:

5>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

4>	else:

5>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG;

1>	release successPSCell-Config configured by the source PSCell, if available.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

### 5.7.3a	EUTRA SCG failure information

#### 5.7.3a.1	General

Figure 5.7.3a.1-1: EUTRA SCG failure information

The purpose of this procedure is to inform NR MN about an SCG failure on E-UTRA SN the UE has experienced (e.g. SCG radio link failure, SCG change failure), as specified in TS 36.331 [10] clause 5.6.13.2.

#### 5.7.3a.2	Initiation

A UE initiates the procedure to report EUTRA SCG failures when neither NR MCG nor EUTRA SCG transmission is suspended and in accordance with TS 36.331 [10] clause 5.6.13.2. Actions the UE shall perform upon initiating the procedure, other than related to the transmission of the SCGFailureInformationEUTRA message are specified in TS 36.331 [10] clause 5.6.13.2.

#### 5.7.3a.3	Actions related to transmission of SCGFailureInformationEUTRA message

The UE shall set the contents of the SCGFailureInformationEUTRA message as follows:

1>	include failureType within failureReportSCG-EUTRA and set it to indicate the SCG failure in accordance with TS 36.331 [10] clause 5.6.13.4;

1>	include and set measResultSCG-FailureMRDC in accordance with TS 36.331 [10] clause 5.6.13.5;

1>	for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListMRDC to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

NOTE:	Field measResultSCG-FailureMRDC is used to report available results for E-UTRAN frequencies the UE is configured to measure by E-UTRA RRC signalling.

1>	if available, set the locationInfo as in 5.3.3.7.:

The UE shall submit the SCGFailureInformationEUTRA message to lower layers for transmission.

### 5.7.3b	MCG failure information

#### 5.7.3b.1	General

Figure 5.7.3b.1-1: MCG failure information

The purpose of this procedure is to inform the network about an MCG failure the UE has experienced i.e. MCG radio link failure. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB or multicast MRB setup or, for IAB, SRB2, may initiate the fast MCG link recovery procedure in order to continue the RRC connection without re-establishment.

#### 5.7.3b.2	Initiation

In case of MR-DC, a UE configured with split SRB1 or SRB3 initiates the procedure to report MCG failures when neither MCG nor SCG transmission is suspended, the SCG is not deactivated, t316 is configured, and when the following condition is met; or

In case of MP, a MP remote UE configured with split SRB1 initiates the procedure to report direct path failures when neither MCG (i.e. direct path) nor indirect path transmission is suspended, t316 is configured, and when the following condition is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10.3, while T316 is not running.

Upon initiating the procedure, the UE shall:

1>	stop timer T310 for the PCell, if running;

1>	stop timer T312 for the PCell, if running;

1>	suspend MCG transmission for all SRBs, DRBs, multicast MRBs, except SRB0, and, if any, BH RLC channels;

1>	reset MCG MAC;

1>	stop conditional reconfiguration evaluation for CHO or CHO with candidate SCG(s), if configured;

1>	stop conditional reconfiguration evaluation for CPC or subsequent CPAC, if configured;

1>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations associated with the MCG, if configured;

1>	if LTM cell switch conditions evaluation was done based on L1 measurements:

2>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all LTM candidate configurations;

1>	initiate transmission of the MCGFailureInformation message in accordance with 5.7.3b.4.

NOTE:	The handling of any outstanding UL RRC messages during the initiation of the fast MCG link recovery is left to UE implementation.

#### 5.7.3b.3	Failure type determination

The UE shall set the MCG failure type as follows:

1>	if the UE initiates transmission of the MCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the MCGFailureInformation message due to T312 expiry:

2>	set the failureType as t312-Expiry;

1>	else if the UE initiates transmission of the MCGFailureInformation message to provide random access problem indication from MCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the MCGFailureInformation message to provide indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx:

1>	else if the UE initiates transmission of the MCGFailureInformation message due to consistent uplink LBT failures on the MCG:

2>	set the failureType as lbt-Failure;

1>	else if connected as an IAB-node and the MCGFailureInformation message is initiated due to the reception of a BH RLF indication on BAP entity from the MCG:

2>	set the failureType as bh-RLF.

#### 5.7.3b.4	Actions related to transmission of MCGFailureInformation message

The UE shall set the contents of the MCGFailureInformation message as follows:

1>	include and set failureType in accordance with 5.7.3b.3;

1>	for each MeasObjectNR configured by a measConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

1>	for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListEUTRA to include the best measured cells, ordered such that the best cell is listed first using RSRP to order the cells if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order the cells if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order the cells, based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

1>	for each UTRA-FDD frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListUTRA-FDD to include the best measured cells, ordered such that the best cell is listed first using RSCP to order the cells if RSCP measurement results are available for cells on this frequency, otherwise using EcN0 to order the cells, based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

1>	if the UE is in NR-DC:

2>	include and set measResultSCG in accordance with 5.7.3.4;

1>	if the UE is in NE-DC:

2>	include and set measResultSCG-EUTRA in accordance with TS 36.331 [10] clause 5.6.13.5;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

NOTE 2:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

NOTE 3:	Field measResultSCG-EUTRA is used to report available results for E-UTRAN frequencies the UE is configured to measure by E-UTRA RRC signalling.

1>	if SRB1 is configured as split SRB and pdcp-Duplication is not configured:

2>	if the primaryPath for the PDCP entity of SRB1 refers to the direct path when MP is configured:

3>	set the primaryPath to refer to the indirect path;

2>	else if the primaryPath for the PDCP entity of SRB1 refers to the MCG:

3>	set the primaryPath to refer to the SCG.

The UE shall:

1>	start timer T316;

1>	if SRB1 is configured as split SRB:

2>	submit the MCGFailureInformation message to lower layers for transmission via SRB1, upon which the procedure ends;

1>	else (i.e. SRB3 configured):

2>	submit the MCGFailureInformation message to lower layers for transmission embedded in NR RRC message ULInformationTransferMRDC via SRB3 as specified in 5.7.2a.3.

#### 5.7.3b.5	T316 expiry

The UE shall:

1>	if T316 expires:

2>	if the UE supports RLF-Report for fast MCG recovery procedure:

3>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

3>	set the mcg-RecoveryFailureCause in VarRLF-Report to t316-Expiry;

2>	initiate the connection re-establishment procedure as specified in 5.3.7.

### 5.7.3c	Indirect path failure information

#### 5.7.3c.1	General

Figure 5.7.3c.1-1: Indirect path failure information

The purpose of this procedure is to inform the network about an indirect path failure the MP remote UE has experienced.

#### 5.7.3c.2	Initiation

In case of MP, a MP remote UE initiates the procedure to report indirect path failures when neither MCG transmission nor indirect path transmission is suspended and when one of the following conditions is met:

1>	upon detecting a SL indirect path failure resulting from sidelink radio link failure on the PC5 unicast link or reception of NotificationMessageSidelink from the L2 U2N Relay UE (except indicating relayUE-HO), or SL indirect path addition/change failure in accordance with 5.3.5.17.2.3, or PC5-RRC connection release requested by upper layers in accordance with clause 5.8.9.5; or

1>	upon detecting a N3C indirect path failure, including N3C connection failure or Uu failure of the N3C relay UE with N3C indirect path, or N3C indirect path addition/change failure in accordance with 5.3.5.17.3.2a;

Upon initiating the procedure, the UE shall:

1>	if the procedure was initiated to report SL indirect path failure:

2>	reset the sidelink specific MAC of the L2 U2N relay UE;

2>	stop T421 if running;

1>	suspend indirect path transmission for all SRBs and DRBs;

1>	initiate transmission of the IndirectPathFailureInformation message in accordance with 5.7.3c.4;

#### 5.7.3c.3	Failure type determination

The L2 U2N Remote UE configured with SL indirect path shall set the indirect path failure type as follows:

1>	if the UE initiates transmission of the IndirectPathFailureInformation message due to T421 expiry in accordance with clause 5.3.5.16.1.3:

2>	set the failureTypeIndirectPath as t421-Expiry;

1>	else if the UE initiates transmission of the IndirectPathFailureInformation message due to reception of the NotificationMessageSidelink including indicationType, in accordance with clause 5.8.9.10.4:

2>	set the failureTypeIndirectPath as the value in indicationType received from NotificationMessageSidelink (except relayUE-CellReselection);

1>	else if the UE initiates transmission of the IndirectPathFailureInformation message due to sidelink radio link failure in accordance with clause 5.8.9.3:

2>	set the failureTypeIndirectPath as sl-Failure;

1>	else if the UE initiates transmission of the IndirectPathFailureInformation message due to PC5-RRC connection release requested by upper layers in accordance with clause 5.8.9.5:

2>	set the failureTypeIndirectPath as sl-PC5-Release;

1>	else if the UE initiates transmission of the IndirectPathFailureInformation message due to that the (target) L2 U2N Relay UE (i.e., the UE indicated by sl-IndirectPathRelayUE-Identity in the received sl-IndirectPathAddChange) changes its serving cell to a different cell from the target cell (i.e. the cell indicated by sl-IndirectPathCellIdentity in the received sl-IndirectPathAddChange) during path addition or change:

2>	set the failureTypeIndirectPath as indirectPathAddChangeFailure;

The N3C remote UE shall set the indirect path failure type as follows:

1>	if the UE initiates transmission of the IndirectPathFailureInformation message due to N3C connection failure:

2>	set the failureTypeIndirectPath as n3c-Failure;

1>	else if the UE initiates transmission of the IndirectPathFailureInformation message due to Uu radio link failure on N3C indirect path in accordance with clause 5.3.10.3:

2>	set the failureTypeIndirectPath as relayUE-Uu-RLF;

1>	if the UE initiates transmission of the IndirectPathFailureInformation message due to N3C Indirect path addition/change failure in accordance with clause 5.3.5.17.3.2a:

2>	set the failureTypeIndirectPath as indirectPathAddChangeFailure.

NOTE:	It is out of 3GPP scope how the remote UE detects N3C connection failure, or how the relay UE indicates Uu RLF to the remote UE on the N3C connection.

#### 5.7.3c.4	Actions related to transmission of IndirectPathFailureInformation message

The UE shall set the contents of the IndirectPathFailureInformation message as follows:

1>	set the failureTypeIndirectPath in accordance with 5.7.3c.3;

1>	if the procedure was initiated to report SL indirect path failure:

2>	set the sl-MeasResultServingRelay to include the measurement result for serving L2 U2N Relay UE if available;

2>	for each measObjectRelay included in MeasConfig, and for which measurement results are available:

3>	include an entry in sl-MeasResultsCandRelay;

1>	else if the procedure was initiated to report N3C indirect path failure;

2>	include n3c-relayUE-InfoList to report relay UE information with non-3GPP connection(s) if available;

1>	submit the IndirectPathFailureInformation message to lower layers for transmission.

### 5.7.4	UE Assistance Information

#### 5.7.4.1	General

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is for the UE to inform the network of:

-	its delay budget report carrying desired increment/decrement in the connected mode DRX cycle length; or

-	its overheating assistance information; or

-	its IDC assistance information; or

-	its preference on DRX parameters for power saving, and its preference on cell DTX/DRX related parameters; or

-	its preference on the maximum aggregated bandwidth for power saving; or

-	its preference on the maximum number of secondary component carriers for power saving; or

-	its preference on the maximum number of MIMO layers for power saving; or

-	its preference on the minimum scheduling offset for cross-slot scheduling for power saving; or

-	its preference on the RRC state; or

-	configured grant assistance information for NR sidelink communication; or

-	its preference in being provisioned with reference time information; or

-	its preference for FR2 UL gap; or

-	its preference to transition out of RRC_CONNECTED state for MUSIM operation; or

-	its preference on the MUSIM gaps; or

-	its preference on the MUSIM gap priority; or

-	its preference on keeping the colliding MUSIM gaps; or

-	its preference on the MUSIM temporary capability restriction; or

-	its relaxation state for RLM measurements; or

-	its relaxation state for BFD measurements; or

-	availability of data and/or signalling mapped to radio bearers which are not configured for SDT; or

-	its preference for the SCG to be deactivated; or

-	availability of uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated; or

-	change of its fulfilment status for RRM measurement relaxation criterion; or

-	service link (specified in TS 38.300 [2]) propagation delay difference between serving cell and neighbour cell(s); or

-	its preference on multi-Rx operation for FR2; or

-	availability of flight path information for Aerial UE operation; or

-	UL traffic information; or

-	the information of the relay UE(s) with which it connects via a non-3GPP connection for MP; or

-	configured grant assistance information for NR sidelink positioning; or

-	its preference for gap occasion cancellation (specified in clause 10.6 in TS 38.213 [13]) ratio; or

-	its preference on time offset for LP-WUS monitoring; or

-	applicability of configurations subject to the applicability determination procedure; or

-	its preference to be configured with or stop being configured with radio resources to perform UE-side data collection; or

-	its assistance information related to logging of measurements for network-side data collection; or

-	location information for assisted SMTC configuration in RRC_CONNECTED state.

#### 5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information.

A UE capable of providing its preference on DRX parameters of a cell group for power saving or its preference on cell DTX/DRX related parameters in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters or cell DTX/DRX related parameters and upon change of its preference on DRX parameters or cell DTX/DRX related parameters.

A UE capable of providing its preference on the maximum aggregated bandwidth of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum aggregated bandwidth preference and upon change of its maximum aggregated bandwidth preference.

A UE capable of providing its preference on the maximum number of secondary component carriers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of secondary component carriers preference and upon change of its maximum number of secondary component carriers preference.

A UE capable of providing its preference on the maximum number of MIMO layers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of MIMO layers preference and upon change of its maximum number of MIMO layers preference.

A UE capable of providing its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a minimum scheduling offset preference and upon change of its minimum scheduling offset preference.

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

A UE capable of providing configured grant assistance information for NR sidelink communication in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide traffic pattern information and upon change of traffic patterns.

A UE capable of providing an indication of its preference in being provisioned with reference time information may initiate the procedure upon being configured to provide this indication, or if it was configured to provide this indication and upon change of its preference.

A UE capable of providing an indication of its preference in FR2 UL gap may initiate the procedure if it was configured to do so, upon detecting the need of FR2 UL gap activation/deactivation.

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

A UE capable of providing MUSIM assistance information for gap priority preference and/or preference to keep the colliding MUSIM gaps may initiate the procedure if it was configured to do so, upon determining it has gap priority preference information and/or it has preference to keep the colliding MUSIM gaps.

A UE capable of providing MUSIM assistance information for leave indication may initiate the procedure if it was configured to do so upon determining that it needs to leave RRC_CONNECTED state.

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

A UE capable of relaxing its BFD measurements in serving cells of a cell group in RRC_CONNECTED shall initiate the procedure for providing an indication of its relaxation state for BFD measurements upon being configured to do so, and upon change of its relaxation state for BFD measurements in RRC_CONNECTED state.

A UE capable of SDT initiates this procedure when data and/or signalling mapped to radio bearers that are not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing).

A UE capable of providing its preference for SCG deactivation may initiate the procedure if it was configured to do so, upon determining that it prefers or does no more prefer the SCG to be deactivated.

A UE that has uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated shall initiate the procedure.

A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

A UE capable of providing service link propagation delay difference between serving cell and neighbour cell(s) shall initiate the procedure upon being configured to do so, and upon determining that service link propagation delay difference between serving cell and a neighbour cell has changed more than threshPropDelayDiff compared with the last reported value.

A UE capable of providing an indication of its preference on multi-Rx operation for FR2 may initiate the procedure if it was configured to do so, upon detecting having a preference on multi-Rx operation for FR2 and upon change of its preference on multi-Rx operation for FR2.

A UE capable of indicating the availability of flight path information may initiate the procedure, if it was configured to do so, upon determining that an initial or updated flight path information is available.

A UE capable of providing UL traffic information shall initiate the procedure when this information is available upon being configured to do so, and upon change of UL traffic information.

A UE capable of N3C remote UE operation initiates the procedure upon being configured to report relay UE information on the available non-3GPP connection(s), and upon change of its available non-3GPP connection(s).

A UE capable of providing configured grant assistance information including SL-PRS transmission periodicity, priority, bandwidth and delay budget for NR sidelink positioning in RRC_CONNECTED may initiate the procedure.

A UE capable of providing UE's preference for gap occasion cancellation ratio initiates the procedure upon being configured to do so when the UE has a preference for gap occasion cancellation ratio, or upon change of its preference.

A UE capable of providing its preference on time offset for LP-WUS monitoring may initiate the procedure in several cases, including upon being configured to provide its preference on time offset for LP-WUS monitoring and upon change of its preference on time offset for LP-WUS monitoring.

A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure may initiate the procedure in several cases, including upon being configured to report assistance information about the applicability of configurations subject to the applicability determination procedure and upon change of the applicability of the configurations subject to the applicability determination procedure. A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure shall initiate the procedure if it was configured to do so, upon determining that the applicability of a configuration subject to the applicability determination procedure changed from applicable to inapplicable.

A UE capable of providing its preference to be configured with or stop being configured with radio resources to perform UE-side data collection may initiate the procedure if it was configured to do so, upon determining that it would like to perform UE-side data collection, or upon determining a list of preferred radio resource configurations for UE-side data collection, or upon determining a list of radio resource configurations for which it prefers to stop UE-side data collection.

A UE capable of logging measurements for network-side data collection may initiate the procedure if it was configured to do so, upon determining that it has entered a low power state, or upon determining that the buffer reserved for the logging of radio measurements for network-side data collection has become full.

A UE capable of logging measurements for network-side data collection and of providing a data availability indication based on a buffer threshold may initiate the procedure if it was configured to do so, upon determining that the amount of logged data related to radio measurements for network-side data collection reached a configured buffer threshold.

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so, and upon determining that the closest reference location(s) have changed compared with the last reported values.

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

2>	if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or

2>	if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running:

3>	start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report;

1>	if configured to provide overheating assistance information:

2>	if the overheating condition has been detected and T345 is not running; or

2>	if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running:

3>	start timer T345 with the timer value set to the overheatingIndicationProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information;

1>	if configured to provide IDC assistance information based on candidateServingFreqListNR included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FDM IDC assistance information including a list of affected frequencies and/or frequency combinations;

2>	else if the current idc-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC FDM assistance information including a list of affected frequencies and/or frequency combinations;

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC enhanced FDM assistance information including a list of affected frequency ranges and/or frequency range combinations;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC enhanced FDM assistance information including a list of affected frequency ranges and/or frequency range combinations;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC TDM assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC TDM assistance information;

NOTE 1:	The term "IDC problems" refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected.

NOTE 2:	For the frequencies or frequency range(s) on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds.
For frequencies or frequency range(s) on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself.
For a non-serving frequency or frequency range(s), reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies or frequency range(s) became a serving frequency or serving frequencies or frequency range(s) then this would result in interference issues that the UE would not be able to solve by itself.

1>	if configured to provide its preference on DRX parameters of a cell group for power saving and its preference on cell DTX/DRX related parameters:

2>	if the UE has a preference on DRX parameters of the cell group or cell DTX/DRX parameters and the UE did not transmit a UEAssistanceInformation message with drx-Preference for the cell group since it was configured to provide its preference on DRX parameters of the cell group for power saving and cell DTX/DRX parameters; or

2>	if the current drx-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including drx-Preference for the cell group and timer T346a associated with the cell group is not running:

3>	start the timer T346a with the timer value set to the drx-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current drx-Preference;

1>	if configured to provide its preference on the maximum aggregated bandwidth of a cell group for power saving:

2>	if the UE has a preference on the maximum aggregated bandwidth of the cell group and the UE did not transmit a UEAssistanceInformation message with maxBW-Preference and/or maxBW-PreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum aggregated bandwidth of the cell group for power saving; or

2>	if the current maxBW-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxBW-Preference and/or maxBW-PreferenceFR2-2for the cell group and timer T346b associated with the cell group is not running:

3>	start the timer T346b with the timer value set to the maxBW-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxBW-Preference and/or maxBW-PreferenceFR2-2;

1>	if configured to provide its preference on the maximum number of secondary component carriers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of secondary component carriers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxCC-Preference for the cell group since it was configured to provide its preference on the maximum number of secondary component carriers of the cell group for power saving; or

2>	if the current maxCC-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxCC-Preference for the cell group and timer T346c associated with the cell group is not running:

3>	start the timer T346c with the timer value set to the maxCC-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxCC-Preference;

1>	if configured to provide its preference on the maximum number of MIMO layers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of MIMO layers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum number of MIMO layers of the cell group for power saving; or

2>	if the current maxMIMO-LayerPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group and timer T346d associated with the cell group is not running:

3>	start the timer T346d with the timer value set to the maxMIMO-LayerPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2;

1>	if configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving:

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling of the cell group and the UE did not transmit a UEAssistanceInformation message with minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group since it was configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving; or

2>	if the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group and timer T346e associated with the cell group is not running:

3>	start the timer T346e with the timer value set to the minSchedulingOffsetPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt;

1>	if configured to provide its release preference and timer T346f is not running:

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;

1>	if configured to provide configured grant assistance information for NR sidelink communication:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink communication;

1>	if configured to provide preference in being provisioned with reference time information:

2>	if the UE did not transmit a UEAssistanceInformation message with referenceTimeInfoPreference since it was configured to provide preference; or

2>	if the UE's preference changed from the last time UE initiated transmission of the UEAssistanceInformation message including referenceTimeInfoPreference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide preference in being provisioned with reference time information.

1>	if configured to provide its preference on FR2 UL gap:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-GapFR2-Preference since it was configured to provide its preference on FR2 UL gap information:

3>	if the UE has a preference on FR2 UL gap activation/deactivation:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference;

2>	else if the current FR2 UL gap preference is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference.

1>	if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>	if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

3>	start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer;

1>	if configured to provide MUSIM assistance information for gap preference:

2>	if configured to provide MUSIM assistance information for gap priority preference:

3>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference since it was configured to provide MUSIM assistance information for gap preference and gap priority preference and the timer T346h is not running; or

3>	if the current musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference and the timer T346h is not running:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference;

4>	start the timer T346h with the timer value set to the musim-GapProhibitTimer.

2>	else:

3>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or

3>	if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList;

4>	start the timer T346h with the timer value set to the musim-GapProhibitTimer.

NOTE 3:	The UE does not need to initiate transmission of the UEAssistanceInformation message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap.

1>	if configured to provide MUSIM assistance information for temporary capability restriction:

2>	if the UE has temporary capability restriction on the current configuration and timer T348 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList;

3>	start the timer T348 with the timer value set to the musim-WaitTimer.

2>	if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList or if the UE has temporary capability restriction on the maximum CC number, and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction and timer T346n is not running; or

2>	if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC;

3>	start the timer T346n with the timer value set to the musim-ProhibitTimer.

2>	if the UE is configured to provide the measurement gap requirement information of NR target bands and if the current measurement gap requirement information is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-NeedForGapsInfoNR or RRCReconfigurationComplete message or RRCResumeComplete message including needForGapsInfoNR:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-NeedForGapsInfoNR;

2>	if the UE has included musim-CapRestrictionInd in the RRCSetupComplete message or RRCResumeComplete or RRCReestablishmentComplete message and the temporary capability restriction is not applicable when the UE is configured to provide MUSIM assistance information for temporary capability restriction:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that there is no temporary capability restriction;

1>	if configured to provide the relaxation state of RLM measurements of a cell group and RLM measurement of the cell group is not stopped:

2>	if the UE did not transmit a UEAssistanceInformation message with rlm-MeasRelaxationState since it was configured to provide the relaxation state of RLM measurements for the cell group; or

2>	if the relaxation state of RLM measurements for the cell group is currently different from the relaxation state reported in the last transmission of the UEAssistanceInformation message including rlm-MeasRelaxationState of the cell group and timer T346j associated with the cell group is not running:

3>	start timer T346j with the timer value set to the rlm-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of RLM measurements of the cell group;

1>	if configured to provide the relaxation state of BFD measurements of serving cells of a cell group and BFD measurement of the cell group is not stopped:

2>	if the UE did not transmit a UEAssistanceInformation message with bfd-MeasRelaxationState since it was configured to provide the relaxation state of BFD measurements for the cell group; or

2>	if the relaxation state of BFD measurements in any serving cell of the cell group is currently different from the relaxation state reported in the last transmission of the UEAssistanceInformation message including bfd-MeasRelaxationState of the cell group and timer T346k associated with the cell group is not running:

3>	start timer T346k with the timer value set to the bfd-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of BFD measurements of serving cells of the cell group.

1>	if data and/or signalling mapped to radio bearers not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing):

2>	if the UE did not transmit a UEAssistanceInformation message with nonSDT-DataIndication since the initiation of the current resume procedure for SDT:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide nonSDT-DataIndication.

1>	if configured to provide its preference for SCG deactivation and timer T346i is not running;

2>	if the UE prefers the SCG to be deactivated and did not transmit a UEAssistanceInformation message with scg-DeactivationPreference since it was configured to provide its SCG deactivation preference; or

2>	if the UE preference for SCG deactivation is different from the last indicated scg-DeactivationPreference:

3>	start timer T346i with the timer value set to the scg-DeactivationPreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for SCG deactivation;

1>	if the SCG is deactivated, and,

1>	the UE has uplink data to send for an SCG RLC entity while the UE previously did not have any uplink data to send for any SCG RLC entity:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the UE has uplink data to send for a DRB whose DRB-Identity is not included in any RLC-BearerConfig in the CellGroupConfig associated with the MCG.

1>	if configured to send indications of RRM measurement relaxation criterion fulfilment:

2>	if the criterion in 5.7.4.4 is met for a period of TSearchDeltaP-StationaryConnected:

3>	if the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment; or

3>	the last UEAssistanceInformation message indicated the criterion in 5.7.4.4 is not fulfilled with rrm-MeasRelaxationFulfilment as false:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled;

2>	else:

3>	if the last UEAssistanceInformation message indicated fulfilment of the criterion in 5.7.4.4 with rrm-MeasRelaxationFulfilment as true:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled.

1>	if configured to provide service link propagation delay difference between serving cell and neighbour cell(s);

2>	if the UE did not transmit a UEAssistanceInformation message with propagationDelayDifference since it was configured to provide service link propagation delay difference between serving cell and neighbour cell(s); or

2>	for any neighbour cell in neighCellInfoList, if the service link propagation delay difference between serving cell and the neighbour cell has changed more than threshPropDelayDiff since the last transmission of the UEAssistanceInformation message including propagationDelayDifference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide service link propagation delay difference between serving cell and each neighbour cell included in the neighCellInfoList;

1>	if configured to provide its preference for multi-Rx operation and timer T346m is not running;

2>	if the UE has a preference on multi-Rx operation for FR2 and did not transmit a UEAssistanceInformation message with multiRx-PreferenceFR2 since it was configured to provide its preference on multi-Rx operation; or

2>	if the UE has a different preference on multi-Rx operation for FR2 from the last indicated multiRx-PreferenceFR2:

3>	start timer T346m with the timer value set to the multiRx-PreferenceReportingConfigFR2ProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for multi-Rx operation for FR2.

1>	if configured to indicate the availability of flight path information and the UE has (updated) flight path information available:

2>	if the UE had neither provided a flight path information nor indicated the availability of flight path information since last entering RRC_CONNECTED state; or

2>	if at least one waypoint or a timestamp corresponding to a waypoint location that was not previously provided since last entering RRC_CONNECTED state is available; or

2>	if at least one upcoming waypoint or a timestamp corresponding to a waypoint location that was previously provided since last entering RRC_CONNECTED state is to be removed; or

2>	if flightPathUpdateDistanceThr is configured and, for at least one waypoint, the 3D distance between the previously provided location and the new location is more than the distance threshold configured by flightPathUpdateDistanceThr; or

2> if flightPathUpdateTimeThr is configured and, for at least one waypoint, the time difference between the previously provided timestamp and the new timestamp, if available, is more than the time threshold configured by flightPathUpdateTimeThr:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate the availability of flight path information;

NOTE 4:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to initiate transmission of the UEAssistanceInformation message when updated flight path information is available.

1>	if configured to provide UL traffic information:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-TrafficInfo since it was configured to provide UL traffic information; or

2>	if UL traffic information included in the previous UEAssistanceInformation has changed since the last transmission of the UEAssistanceInformation message containing ul-TrafficInfo for at least one QoS flow for which timer T346l is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UL traffic information.

NOTE 5:	The UE only considers burstArrivalTime to have changed when it changes relative to the periodicity of the Data Burst arrival.

1>	if configured to report relay UE information with non-3GPP connection(s):

2>	if the UE did not transmit a UEAssistanceInformation message with n3c-relayUE-InfoList since it was configured to report available relay UE information with non-3GPP connection(s); or

2>	if the UE has new available non-3GPP connection(s); or

2>	if the non-3GPP connection(s) with the reported relay UE(s) is not available:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report relay UE information with non-3GPP connection(s) included in the n3c-relayUE-InfoList;

1>	if configured to provide configured grant assistance information for NR sidelink positioning:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink positioning;

1>	if configured to provide its preference for gap occasion cancellation ratio:

2>	if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or

2>	if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running:

3>	start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

1>	if configured to provide its preference on time offset for LP-WUS monitoring of a cell group:

2>	if [the UE has a preference on time offset for LP-WUS monitoring of the cell group and] the UE did not transmit a UEAssistanceInformation message with lpwus-OffsetPreference for the cell group since it was configured to provide its preference on time offset for LP-WUS monitoring of the cell group for power saving; or

Editor´s note: Brackets should be deleted when issue is resolved.

2>	if the current lpwus-OffsetPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including lpwus-OffsetPreference for the cell group and timer T346p associated with the cell group is not running:

3>	start the timer T346p with the timer value set to the lpwus-OffsetPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current lpwus-OffsetPreference.

1>	if configured to report assistance information about the applicability of configurations subject to the applicability determination procedure:

2>	if the applicability status of configurations subject to the applicability determination procedure has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation):

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report assistance information about the applicability of configurations subject to the applicability determination procedure;

1>	if configured to provide its preference to be configured with radio measurement resources for UE-side data collection:

2>	if the UE has a preference to be configured with radio measurement resources to perform UE-side data collection and did not transmit a UEAssistanceInformation message with dataCollectionPreference since it was configured to provide its preference to be configured with radio measurement resources to perform UE-side data collection; or

2>	if the preference to be configured with radio measurement resources to perform UE-side data collection has changed since the last transmission of the UEAssistanceInformation message including dataCollectionPreference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report the UE preference to be configured with radio measurement resources for UE-side data collection;

1>	if configured to provide assistance information related to logging of measurements for network-side data collection based on loggedDataCollectionAssistanceConfig:

2>	if the buffer reserved for the logging of radio measurements for network-side data collection has become full; or

2>	if the UE determines that it has entered a low power state; or

2>	if loggedDataCollectionBufferThreshold is included in loggedDataCollectionAssistanceConfig and the amount of logged data related to radio measurements for network-side data collection has become equal to or above the loggedDataCollectionBufferThreshold:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide assistance information related to logging of measurements for network-side data collection.

NOTE 6:	It is up to UE implementation how to determine a low power state and how to determine whether the buffer threshold is reached or if the buffer is full.

1>	if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

2>	if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration;

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3;

2>	set delayBudgetReport to type1 according to a desired value;

1>	if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2 or 5.3.5.3;

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-1:

4>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-2:

4>	include reducedMaxBW-FR2-2 in the OverheatingAssistance IE;

4>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-2;

4>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-2;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-1:

4>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2-1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2-1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-2:

4>	include reducedMaxMIMO-LayersFR2-2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxBW-FR2-2, reducedMaxMIMO-LayersFR1, reducedMaxMIMO-LayersFR2 or reducedMaxMIMO-LayersFR2-2 in OverheatingAssistance IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC FDM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA or NR-DC combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC enhanced FDM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC TDM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one candidate carrier frequency included in candidateServingFreqListNR or candidate frequency range included in candidateServingFreqRangeListNR or one supported UL CA or NR-DC combination comprising of candidate carrier frequencies included in candidateServingFreqListNR or candidate frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

NOTE 1:	When sending an UEAssistanceInformation message to inform the IDC problems, the UE includes all IDC assistance information in the idc-Assistance (IDC FDM assistance information) or idc-FDM-Assistance (IDC enhanced FDM assistance information) or idc-TDM-Assistance (IDC TDM assistance information) fields respectively (rather than providing e.g. the changed part(s) of the IDC assistance information in respective fields).

NOTE 2:	Upon not anymore experiencing a particular IDC problem that the UE previously reported, the UE provides an IDC indication with the modified contents of the UEAssistanceInformation message (e.g. by not including the IDC assistance information in the idc-Assistance or idc-FDM-Assistance or idc-TDM-Assistance fields).

1>	if transmission of the UEAssistanceInformation message is initiated to provide drx-Preference of a cell group for power saving or for DTX/DRX configuration preference according to 5.7.4.2 or 5.3.5.3:

2>	include drx-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on DRX parameters for the cell group or cell DTX/DRX related parameters:

3>	if the UE has a preference for the long DRX cycle or for cellDTX-DRX-Cycle:

4>	include preferredDRX-LongCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the DRX inactivity timer:

4>	include preferredDRX-InactivityTimer in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX cycle:

4>	include preferredDRX-ShortCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX timer:

4>	include preferredDRX-ShortCycleTimer in the DRX-Preference IE and set it to the preferred value;

2>	else (if the UE has no preference on DRX parameters for the cell group):

3>	do not include preferredDRX-LongCycle, preferredDRX-InactivityTimer, preferredDRX-ShortCycle and preferredDRX-ShortCycleTimer in the DRX-Preference IE;

NOTE 2a:	If drx-Preference contains only preferredDRX-LongCycle and it is shorter than the configured cellDTX-DRX-Cycle it implies preference for cell DTX/DRX.

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum aggregated bandwidth for the cell group:

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR1 in the cell group;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2-1:

4>	include reducedMaxBW-FR2 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2-1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2-1 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR1 and reducedMaxBW-FR2 in the MaxBW-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-PreferenceFR2-2 of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-PreferenceFR2-2 in the UEAssistanceInformation message;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2-2:

4>	include reducedMaxBW-FR2-2 in the MaxBW-PreferenceFR2-2 IE;

4>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2-2 in the cell group;

4>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2-2 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR2-2 in the MaxBW-PreferenceFR2-2 IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxCC-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxCC-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of secondary component carriers for the cell group:

3>	include reducedMaxCCs in the MaxCC-Preference IE;

3>	set reducedCCsDL to the number of maximum SCells the UE desires to have configured in downlink in the cell group;

3>	set reducedCCsUL to the number of maximum SCells the UE desires to have configured in uplink in the cell group;

2>	else (if the UE has no preference on the maximum number of secondary component carriers for the cell group):

3>	do not include reducedMaxCCs in the MaxCC-Preference IE;

NOTE 3:	The UE can implicitly indicate a preference for NR SCG release by reporting the maximum aggregated bandwidth preference for power saving of the cell group, if configured, as zero for both FR1 and FR2, and by reporting the maximum number of secondary component carriers for power saving of the cell group, if configured, as zero for both uplink and downlink.

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO-LayerPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR1-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR1-UL to the preferred maximum number of uplink MIMO layers of each FR1 serving cell that the UE operates on in the cell group;

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2-1:

4>	include reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2-1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-UL to the preferred maximum number of uplink MIMO layers of each FR2-1 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO LayerPreferenceFR2 2 of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreferenceFR2-2 in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group for FR2-2:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2 2:

4>	include reducedMaxMIMO-LayersFR2-2 in the MaxMIMO-LayerPreferenceFR2 2 IE;

4>	set reducedMIMO-LayersFR2-2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2-2 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-2-UL to the preferred maximum number of uplink MIMO layers of each FR2-2 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR2-2 in the MaxMIMO-LayerPreferenceFR2-2 IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group:

3>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK0-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK0-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK0-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK0-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K2 (TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK2-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK2-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK2-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK2-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

2>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

3>	do not include preferredK0 and preferredK2 in the MinSchedulingOffsetPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreferenceExt of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreferenceExt in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group for FR2-2:

3>	include minSchedulingOffsetPreferenceExt in the UEAssistanceInformation message;

4>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 480 kHz SCS:

5>	include preferredK0-SCS-480kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K0;

4>	if the UE has a preference for the value of K0 for cross-slot scheduling with 960 kHz SCS:

5>	include preferredK0-SCS-960kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K0;

4>	if the UE has a preference for the value of K2 for cross-slot scheduling with 480 kHz SCS:

5>	include preferredK2-SCS-480kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K2;

4>	if the UE has a preference for the value of K2 for cross-slot scheduling with 960 kHz SCS:

5>	include preferredK2-SCS-960kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K2;

3>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

4>	do not include preferredK0 and preferredK2 in the minSchedulingOffsetPreferenceExt IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide a release preference according to 5.7.4.2 or 5.3.5.3:

2>	include releasePreference in the UEAssistanceInformation message;

2>	set preferredRRC-State to the desired RRC state on transmission of the UEAssistanceInformation message;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference in being provisioned with reference time information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference in being provisioned with reference time information:

3>	set referenceTimeInfoPreference to true;

2>	else:

3>	set referenceTimeInfoPreference to false.

1>	if transmission of the UEAssistanceInformation message is initiated to provide preference on FR2 UL gap according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for FR2 UL gap configuration:

3>	set ul-GapFR2-PatternPreference to the preferred FR2 UL gap pattern;

2>	else (if the UE has no preference for the FR2 UL gap configuration):

3>	do not include ul-GapFR2-PatternPreference in the UL-GapFR2-Preference IE.

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference, or provide MUSIM assistance information for leaving RRC_CONNECTED according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

4>	if UE has a preference for MUSIM gap priority;

5>	include the musim-GapPriorityPreferenceList the UE prefers to be configured;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has a preference to keep all colliding MUSIM gaps:

3>	include the musim-GapKeepPreference;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList, musim-GapPriorityPreferenceList and musim-GapKeepPreference in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-CapRestriction according to 5.7.4.2 or 5.3.5.3:

2>	if UE has a preference for temporary capability restriction:

3>	if UE has a preference for serving cell(s), except PCell, and/or SCG to be released:

4>	include the musim-Cell-SCG-ToRelease;

5>	set musim-CellToRelease to include the serving cell(s) the UE prefers to be released;

5>	set scg-ReleasePreference to scgReleasePreferred if the UE prefers the SCG to be released;

3>	if UE has a preference to indicate the serving cells with restricted capabilities:

4>	include the musim-CellToAffectList the UE prefers to be configured;

5>	include the musim-ServCellIndex and the musim-MIMO-Layers-DL/ musim-MIMO-Layers-UL/ musim-SupportedBandwidth-DL/ musim-SupportedBandwidth-UL for the corresponding serving cell;

3>	if UE has a preference to indicate the maximum number of CCs:

4>	include the musim-CapRestriction for the musim-MaxCC the UE prefers to be configured;

5>	include the musim-MaxCC-TotalDL/ musim-MaxCC-TotalUL/ musim-MaxCC-FR1-DL/ musim-MaxCC-FR1-UL/ musim-MaxCC-FR2-1-DL/ musim-MaxCC-FR2-2-UL/ musim-MaxCC-FR2-2-DL/ musim-MaxCC-FR2-2-UL for the corresponding maximum number of CCs;

3>	if UE has a preference to indicate band(s) and/or combination(s) of bands with capabilities restricted which comprise of the band(s) that is/are indicated in musim-CandidateBandList:

4>	include the musim-AffectededBandsList the UE prefer to be configured with capabilities restricted;

5>	include the musim-bandEntryIndex for each band or each band of the combination(s) for which capabilities are restricted;

5>	include the musim-CapabilityRestricted for the corresponding band;

3>	if UE has a preference to indicate band(s) and/or combination(s) of bands to be avoided which comprise of band(s) that is indicated in musim-CandidateBandList:

4>	include the musim-AvoidedBandsList the UE prefers not to be configured;

5>	include the musim-bandEntryIndex for each band or each band of the combination(s) to be avoided;

2>	if UE has no longer preference for temporary capability restriction indicated by musim-Cell-SCG-ToRelease, musim-CellToAffectList, musim-MaxCC, musim-AffectededBandsList and/or musim-AvoidedBandsList:

3>	do not include the corresponding temporary capability restriction preference in the musim-CapRestriction;

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-NeedForGapsInfoNR according to 5.7.4.2 or 5.3.5.3:

2>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each supported NR serving cell;

2>	if the requestedTargetBandFilterNR-r16 of NeedForGapsConfigNR is configured:

3>	for each supported NR band included in requestedTargetBandFilterNR-r16, include an entry in interFreq-needForGap and set the measurement gap requirement information for that band;

2>	else:

3>	include an entry in interFreq-needForGap and set the measurement gap requirement information for each supported NR band;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of RLM measurements of a cell group according to 5.7.4.2:

2>	if the UE performs RLM measurement relaxation on the cell group according to TS 38.133 [14]:

3>	set the rlm-MeasRelaxationState to true;

2>	else:

3>	set the rlm-MeasRelaxationState to false;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of BFD measurements of a cell group:

2>	for each serving cell of the cell group:

3>	if the UE performs BFD measurement relaxation on this serving cell according to TS 38.133 [14]:

4>	set the n-th bit of bfd-MeasRelaxationState to '1', where n is equal to the servCellIndex value + 1 of the serving cell;

3>	else:

4>	set the n-th bit of bfd-MeasRelaxationState to '0', where n is equal to the servCellIndex value + 1 of the serving cell.

1>	if transmission of the UEAssistanceInformation message is initiated to indicate availability of data mapped to radio bearers not configured for SDT according to 5.7.4.2:

2>	include the nonSDT-DataIndication in the UEAssistanceInformation message;

2>	include and set the resumeCause according to the information received from the upper layers, if provided.

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference for SCG deactivation according to 5.7.4.2:

2>	include scg-DeactivationPreference in the UEAssistanceInformation message;

2>	set the scg-DeactivationPreference to scg-DeactivationPreferred if the UE prefers the SCG to be deactivated, otherwise set it to noPreference;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication that the UE has uplink data related to a deactivated SCG according to 5.7.4.2:

2>	include uplinkData in the UEAssistanceInformation message.

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication about whether the criterion for RRM relaxation for connected mode is fulfilled or not fulfilled:

2>	if the criterion for RRM measurement relaxation for connected mode is fulfilled:

3>	set the rrm-MeasRelaxationFulfilment to true;

2>	else:

3>	set the rrm-MeasRelaxationFulfilment to false.

1>	if transmission of the UEAssistanceInformation message is initiated to provide the service link propagation delay difference between serving cell and neighbour cell(s) according to 5.7.4.2;

2>	include the propagationDelayDifference for each neighbour cell in the neighCellInfoList;

1>	if transmission of the UEAssistanceInformation message is initiated to provide preference on multi-Rx operation for FR2 according to 5.7.4.2:

2>	if the UE has a preference for not operating on multi-Rx (i.e. not supporting simultaneous reception with different QCL-typeD) for FR2:

3>	set multiRx-PreferenceFR2 to single;

2>	else (if the UE has the preference for operating on multi-Rx for FR2):

3>	set multiRx-PreferenceFR2 to multiple.

1>	if transmission of the UEAssistanceInformation message is initiated to indicate the availability of flight path information according to 5.7.4.2 or 5.3.5.3;

2>	include the flightPathInfoAvailable;

1>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.7.4.2 or 5.3.5.3:

2>	for each PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

3>	set pdu-SessionID to the value of the concerned PDU session ID;

3>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.3.5.3:

4>	stop timer T346l for each QoS flow of this PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message;

3>	for each QoS flow of this PDU session for which timer T346l is not running and for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

4>	start timer T346l associated to this QoS flow with the timer value set to the value of ul-TrafficInfoProhibitTimer;

4>	set qfi to the value of the concerned QFI;

4>	if the jitter range measurement is available; and

4>	if the UE did not provide jitter range since it was configured to provide UL traffic information, or if the measured jitter range has changed since the last transmission of the UEAssistanceInformation message containing jitterRange:

5>	set jitterRange to the latest measured value of the jitter range;

4>	if the burst arrival time measurement is available; and

4>	if the UE did not provide burst arrival time since it was configured to provide UL traffic information, or if the measured burst arrival time has changed since the last transmission of the UEAssistanceInformation message containing burstArrivalTime:

5>	set burstArrivalTime to the latest measured value of the burst arrival time;

4>	if the traffic periodicity measurement is available; and

4>	if the UE did not provide traffic periodicity since it was configured to provide UL traffic information, or if the measured traffic periodicity has changed since the last transmission of the UEAssistanceInformation message containing trafficPeriodicity:

5>	set trafficPeriodicity to the latest measured value of the traffic periodicity;

4>	if the UE did not provide pdu-SetIdentification since it was configured to provide UL traffic information, or if the information previously provided in pdu-SetIdentification has changed since the last transmission of the UEAssistanceInformation message containing pdu-SetIdentification:

5>	if the UE is able to identify PDU Set(s) for the QoS flow:

6>	set pdu-SetIdentification to true;

5>	else:

6>	set pdu-SetIdentification to false.

4>	if the UE did not provide psi-Identification since it was configured to provide UL traffic information, or if the information previously provided in psi-Identification has changed since the last transmission of the UEAssistanceInformation message containing psi-Identification:

5>	if the UE is able to identify PSI(s) for the QoS flow:

6>	set psi-Identification to true;

5>	else:

6>	set psi-Identification to false.

1>	if transmission of the UEAssistanceInformation message is initiated to report relay UE information with non-3GPP connection(s) according to 5.7.4.2:

2>	include n3c-relayUE-InfoList in the UEAssistanceInformation message;

1>	if transmission of the UEAssistanceInformation message is initiated to report the UE's preference for gap occasion cancellation ratio according to 5.7.4.2:

2>	if gapUE is configured under measGapConfig; and

2>	if the UE has the preference for gap occasion cancellation ratio for per UE gap:

3>	set perUE to the preferred gap occasion cancellation ratio;

2>	if gapFR1 or gapFR2 is configured under measGapConfig:

3>	if the UE has the preference for gap occasion cancellation ratio for FR1:

4>	set fr1 to the preferred gap occasion cancellation ratio;

3>	if the UE has the preference for gap occasion cancellation ratio for FR2:

4>	set fr2 to the preferred gap occasion cancellation ratio;

2>	if gapToAddModList is configured under measGapConfig:

3>	include the preferred gap occasion cancellation ratio for each measurement gap configuration with measGapId for which the UE has preferred gap occasion cancellation ratio;

1>	if transmission of the UEAssistanceInformation message is initiated to provide lpwus-OffsetPreference of a cell group according to 5.7.4.2 or 5.3.5.3:

2>	include lpwus-OffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on time offset for LP-WUS monitoring:

3>	set the timeOffset to the preferred offset value;

1>	if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2:

2>	include applicabilityReportList in this UEAssistanceInformation message;

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed; or

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed:

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList that changed applicability status, associated with the concerned serving cell:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId to the corresponding applicabilitySetConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

1>	if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE-side data collection according to 5.7.4.2:

2>	include dataCollectionPreference in this UEAssistanceInformation message;

2>	if the UE prefers to be configured with radio resources to perform data collection:

3>	set dataCollectionStart to start;

3>	for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s):

4>	include an entry in dataCollectionPreferredConfigurationList and set the content as follows:

5>	set the dataCollectionServCellIndex to the serving cell index of the cell;

5>	include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

2>	if the UE no longer prefers to be configured with radio resources to perform data collection:

3>	for each serving cell configured with one or more UE-side data collection configuration(s) corresponding to a CSI-ReportConfig and for which the UE prefers to stop UE-side data collection:

4>	include an entry in dataCollectionStopConfigurationList and set the content as follows:

5>	set the dataCollectionStopServCellIndex to the serving cell index of the cell;

5>	include in dataCollectionIdList the reportConfigId associated with the CSI-ReportConfig including the UE-side data collection configuration(s) that the UE prefers to stop;

1>	if transmission of the UEAssistanceInformation message is initiated to provide assistance information related to logging of measurements for network-side data collection according to 5.7.4.2:

2>	if the UE determines that it has entered a low power state:

3>	set lowPowerState to true;

2>	if the buffer reserved for the logging of radio measurements for network-side data collection has become full:

3>	set bufferStatus to full;

2>	else if loggedDataCollectionBufferThreshold is configured and if the amount of logged data related to radio measurements for network-side data collection has become equal to or above the loggedDataCollectionBufferThreshold:

3>	set bufferStatus to aboveThreshold;

1>	if transmission of the UEAssistanceInformation message is initiated to provide location information for assisted SMTC configuration in RRC_CONNECTED state according to 5.7.4.2;

2>	include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport;

The UE shall set the contents of the UEAssistanceInformation message for configured grant assistance information for NR sidelink communication or NR sidelink positioning:

1>	if configured to provide configured grant assistance information for NR sidelink:

2>	include the sl-UE-AssistanceInformationNR;

1>	if configured to provide configured grant assistance information for NR sidelink positioning:

2>	include the sl-PRS-UE-AssistanceInformationNR;

NOTE 4:	It is up to UE implementation when and how to trigger configured grant assistance information for NR sidelink communication or NR sidelink positioning.

The UE shall:

1>	if the procedure was triggered to provide configured grant assistance information for NR sidelink communication by an NR RRCReconfiguration message that was embedded within an E-UTRA RRCConnectionReconfiguration:

2>	submit the UEAssistanceInformation to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else if the procedure was triggered to provide UE preference for SCG deactivation or to indicate that the UE with a deactivate SCG has uplink data to send on a DRB for which there is no MCG RLC bearer:

2>	submit the UEAssistanceInformation via SRB1 to lower layers for transmission;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured and the SCG is not deactivated:

3>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

2>	else:

3>	submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the UE assistance configuration that triggered this UE assistance information is associated with the SCG:

3>	if SRB3 is configured and the SCG is not deactivated:

4>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

3>	else:

4>	submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

1>	else:

2>	submit the UEAssistanceInformation message to lower layers for transmission.

#### 5.7.4.3a	Setting the contents of OverheatingAssistance IE

The UE shall set the contents of OverheatingAssistance IE if initiated to provide overheating assistance indication for SCG in (NG)EN-DC according to clause 5.6.10.3 as specified in TS 36.331 [10]:

1>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers for SCG:

2>	include reducedMaxCCs in the OverheatingAssistance IE;

2>	set reducedCCsDL to the number of maximum PSCell and SCells of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedCCsUL to the number of maximum PSCell and SCells of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1 for SCG:

2>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

2>	set reducedBW-FR1-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1 of the SCG;

2>	set reducedBW-FR1-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1 of the SCG;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-1 for SCG:

2>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

2>	set reducedBW-FR2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-1 of the SCG;

2>	set reducedBW-FR2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-1 of the SCG;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-2 for SCG:

2>	include reducedMaxBW-FR2-2 in the OverheatingAssistance IE;

2>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-2 of the SCG;

2>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-2 of the SCG;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1 for SCG:

2>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-1 for SCG:

2>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2-1 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2-1 of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-2 for SCG:

2>	include reducedMaxMIMO-LayersFR2-2 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR2-2-DL to the number of maximum MIMO layers of each serving cell operating on FR2-2 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR2-2-UL to the number of maximum MIMO layers of each serving cell operating on FR2-2 of the SCG the UE prefers to be temporarily configured in uplink;

#### 5.7.4.4	Relaxed measurement criterion for a stationary (e)RedCap UE

The relaxed measurement criterion for a stationary UE is met when:

-	(SS-RSRPRefStationaryConnected – SS-RSRP) < SSearchDeltaP-StationaryConnected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the PCell based on SSB (dB).

-	SS-RSRPRefStationaryConnected = reference SS-RSRP value of the PCell (dB), set as follows:

-	at the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or

-	after MAC successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig while stationary criterion is configured; or

-	upon an indication from lower layers that a RACH-less handover or an LTM cell switch is successfully completed for the CG while stationary criterion is configured, or

-	if (SS-RSRP – SS-RSRPRefStationaryConnected) > 0; or

-	if the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected:

-	UE shall set the value of SS-RSRPRefStationaryConnected to the current SS-RSRP value of the serving cell.

### 5.7.4a	Void

### 5.7.5	Failure information

#### 5.7.5.1	General

Figure 5.7.5.1-1: Failure information

The purpose of this procedure is to inform the network about a failure detected by the UE.

#### 5.7.5.2	Initiation

A UE initiates the procedure when there is a need inform the network about a failure detected by the UE. In particular, the UE initiates the procedure when the following condition is met:

1>	upon detecting failure for an RLC bearer, in accordance with 5.3.10.3;

1>	upon detecting DAPS handover failure, in accordance with 5.3.5.8.3;

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the FailureInformation message as specified in 5.7.5.3;

#### 5.7.5.3	Actions related to transmission of FailureInformation message

The UE shall:

1>	if initiated to provide RLC failure information, set FailureInfoRLC-Bearer as follows:

2>	set logicalChannelIdentity to the logical channel identity of the failing RLC bearer;

2>	set cellGroupId to the cell group identity of the failing RLC bearer;

2>	set the failureType as rlc-failure;

1>	if initiated to provide DAPS failure information, set FailureInfoDAPS as follows:

2>	set the failureType as daps-failure;

1>	if used to inform the network about a failure for an MCG RLC bearer or DAPS failure information:

2>	submit the FailureInformation message to lower layers for transmission via SRB1;

1>	else if used to inform the network about a failure for an SCG RLC bearer:

2>	if SRB3 is configured;

3>	submit the FailureInformation message to lower layers for transmission via SRB3;

2>	else;

3>	if the UE is in (NG)EN-DC:

4>	submit the FailureInformation message via E-UTRA SRB1 embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

3>	else if the UE is in NR-DC:

4>	submit the FailureInformation message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

### 5.7.6	DL message segment transfer

#### 5.7.6.1	General

Figure 5.7.6.1-1: DL message segment transfer

The purpose of this procedure is to transfer segments of DL DCCH messages from the network to the UE.

NOTE:	The segmentation of DL DCCH message is only applicable to RRCReconfiguration and RRCResume messages in this release.

#### 5.7.6.2	Initiation

The network initiates the DL Dedicated Message Segment transfer procedure whenever the encoded RRC message PDU exceeds the maximum PDCP SDU size. The network initiates the DL Dedicated Message Segment transfer procedure by sending the DLDedicatedMessageSegment message.

#### 5.7.6.3	Reception of DLDedicatedMessageSegment by the UE

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment included in rrc-MessageSegmentContainer;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5.3 for the RRCReconfiguration message or 5.3.13.4 for the RRCResume message;

2>	discard all segments.

### 5.7.7	UL message segment transfer

#### 5.7.7.1	General

Figure 5.7.7.1-1: UL message segment transfer

The purpose of this procedure is to transfer segments of UL DCCH messages from UE to a NG-RAN in RRC_CONNECTED.

NOTE:	The segmentation of UL DCCH message is only applicable to UECapabilityInformation and MeasurementReportAppLayer in this release.

#### 5.7.7.2	Initiation

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed, rrc-MaxCapaSegAllowed, rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

#### 5.7.7.3	Actions related to transmission of ULDedicatedMessageSegment message

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5] and the maximum number of UL segments according to rrc-SegAllowed or rrc-MaxCapaSegAllowed (for segmentation of UECapabilityInformation) and rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 (for segmentation of MeasurementReportAppLayer), if received. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

### 5.7.8	Idle/inactive Measurements

#### 5.7.8.1	General

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE and RRC_INACTIVE when it has an idle/inactive measurement configuration.

#### 5.7.8.1a	Measurement configuration

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and SDT procedure is not ongoing and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveEUTRA-MeasReport:

3>	if the SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListEUTRA:

4>	store or replace the measIdleCarrierListEUTRA of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListEUTRA in VarMeasIdleConfig, if stored;

2>	if the UE supports idleInactiveNR-MeasReport:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListNR:

4>	store or replace the measIdleCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListNR in VarMeasIdleConfig, if stored;

1>	if VarEnhMeasIdleConfig does not include measIdleValidityDuration received from the RRCRelease message:

2>	if the UE supports measValidationReportEMR:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleValidityDuration:

4>	store or replace the measIdleValidityDuration of measIdleConfigSIB of SIB11 within VarEnhMeasIdleConfig;

3>	else:

4>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

#### 5.7.8.1b	Measurement configuration (reselection measurements)

The purpose of this procedure is to update the reselection measurement configuration.

The UE initiates this procedure while SDT procedure is not ongoing and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC_INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB11), e.g., due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, the UE shall:

1>	if VarMeasReselectionConfig does not include measReselectionCarrierListNR received from the RRCRelease message:

2>	if the UE supports reselection measurement reporting:

3>	if SIB11 includes the measIdleConfigSIB and contains measReselectionCarrierListNR:

4>	store or replace the measReselectionCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasReselectionConfig;

3>	else:

4>	remove the measReselectionCarrierListNR in VarMeasReselectionConfig, if stored;

1>	if VarMeasReselectionConfig does not include measReselectionValidityDuration received from the RRCRelease message:

2>	if the UE supports reselection measurement reporting:

3>	if SIB11 includes the measIdleConfigSIB and contains measReselectionValidityDuration:

4>	store or replace the measReselectionValidityDuration of measIdleConfigSIB of SIB11 within VarMeasReselectionConfig;

3>	else:

4>	remove the measReselectionValidityDuration in VarMeasReselectionConfig, if stored.

#### 5.7.8.2	Void

#### 5.7.8.2a	Performing measurements

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running and SDT procedure is not ongoing, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListEUTRA and the SIB1 contains idleModeMeasurementsEUTRA:

3>	for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

4>	if UE supports NE-DC between the serving carrier and the carrier frequency indicated by carrierFreqEUTRA within the corresponding entry:

5>	perform measurements in the carrier frequency and bandwidth indicated by carrierFreqEUTRA and allowedMeasBandwidth within the corresponding entry;

5>	if the reportQuantitiesEUTRA is set to rsrq:

6>	consider RSRQ as the sorting quantity;

5>	else:

6>	consider RSRP as the sorting quantity;

5>	if the measCellListEUTRA is included:

6>	consider cells identified by each entry within the measCellListEUTRA to be applicable for idle/inactive mode measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive measurement results for the measurement quantities indicated by reportQuantitiesEUTRA;

5>	store the derived measurement results as indicated by reportQuantitiesEUTRA within the measReportIdleEUTRA in VarMeasIdleReport in decreasing order of the sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThresholdEUTRA is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThresholdEUTRA;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports idleInactiveNR-MeasBeamReport for the FR of the carrier frequency indicated by carrierFreq within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

6>	if the reportQuantityRS-Indexes is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

6>	if the includeBeamMeasurements is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

2>	if, as a result of the procedure in this clause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

3>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and it contains an entry with carrierFreq set to the value of the serving frequency:

4>	if beamMeasConfigIdle is included in that entry, and if the UE supports idleInactiveNR- MeasBeamReport for the FR of the serving cell:

5>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

5>	if the reportQuantityRS-Indexes is set to rsrq:

6>	consider RSRQ as the beam sorting quantity;

5>	else:

6>	consider RSRP as the beam sorting quantity;

5>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

6>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included in SIB2 of serving cell, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

5>	if the includeBeamMeasurements is set to true:

6>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signaling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

NOTE 4:	When idleModeMeasVoiceFallback is included in SIB5, UE may decide to measure and report idle/inactive measurements for EUTRA carrier frequencies included in SIB5 even if it does not support NE-DC between the serving carrier and the EUTRA carrier frequencies.

#### 5.7.8.3	T331 expiry or stop

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

#### 5.7.8.4	Cell re-selection or cell selection while T331 is running

The UE shall:

1>	if intra-RAT cell selection or reselection occurs while T331 is running:

2>	if validityAreaList is configured in VarMeasIdleConfig:

3>	if the serving frequency does not match with the carrierFreq of an entry in the validityAreaList; or

3>	if the serving frequency matches with the carrierFreq of an entry in the validityAreaList, the validityCellList is included in that entry, and the physical cell identity of the serving cell does not match with any entry in validityCellList:

4>	stop timer T331;

4>	perform the actions as specified in 5.7.8.3, upon which the procedure ends.

1>	else if inter-RAT cell selection or reselection occurs while T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.7.8.3;

### 5.7.9	Mobility history information

#### 5.7.9.1	General

This procedure specifies how the mobility history information is stored by the UE, covering RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED and released by the UE upon deregistration as specified in TS 23.502 [43].

#### 5.7.9.2	Initiation

If the UE supports storage of mobility history information, the UE shall:

1>	If the UE supports PSCell mobility history information and upon addition of a PSCell:

2>	include an entry in visitedPSCellInfoList in variable VarMobilityHistoryReport possibly after performing the following, if necessary:

3>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

4>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

5>	remove the oldest entry in the visitedPSCellInfoListReport;

3>	else:

4>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

2>	for the included entry:

3>	set the field timeSpent of the entry according to following:

4>	if this is the first PSCell entry for the current PCell since entering the current PCell in RRC_CONNECTED:

5>	include the entry as the time spent with no PSCell since entering the current PCell in RRC_CONNECTED;

4>	else:

5>	include the time spent with no PSCell since last PSCell release since entering the current PCell in RRC_CONNECTED;

1>	If the UE supports PSCell mobility history information and upon change, or release of a PSCell while being connected to the current PCell, or upon release of a PSCell while entering 'camped normally' state or 'any cell selection' state or 'camped on any cell' state:

2>	include an entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after performing the following, if necessary:

3>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

4>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

5>	remove the oldest entry in the visitedPSCellInfoListReport;

3>	else:

4>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

2>	for the included entry:

3>	if the global cell identity of the previous PSCell is available:

4>	include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	if the PSCell is changed or released while being connected to the current PCell, or if the PSCell is released while entering 'camped normally' state in the same serving cell which was previously the PCell:

4>	set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the current PCell/serving cell;

4>	if the UE supports storing and reporting SCG activation information in mobility history information:

5> set the field scgActiveDuration of the entry to the accumulated time spent in the previous PSCell with SCG state set to activated during the stay in the PSCell while being connected to the current PCell/serving cell, if available;

3>	if the PSCell is released while entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE, or if the PSCell is released while entering 'camped normally' state while previously in RRC_CONNECTED state in a PCell different from the current serving cell:

4>	set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the previous PCell;

4>	if the UE supports storing and reporting SCG activation information in mobility history information:

5>	set the field scgActiveDuration of the entry to the accumulated time spent in the previous PSCell with SCG state set to activated during the stay in the PSCell while being connected to the previous PCell, if available ;

1>	Upon change of suitable cell, consisting of PCell in RRC_CONNECTED (for NR or E-UTRA cell) or serving cell in RRC_INACTIVE (for NR cell) or in RRC_IDLE (for NR or E-UTRA cell), to another NR or E-UTRA cell, or when entering 'any cell selection' state from 'camped normally' state in NR or LTE or when entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE:

NOTE:	Change of suitable cell encompasses also simultaneous change of serving cell and RRC state change.

2>	include an entry in visitedCellInfoList of the variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	if the global cell identity of the previous PCell/serving cell is available:

4>	include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	set the field timeSpent of the entry as the time spent in the previous PCell/serving cell;

3>	if the UE supports PSCell mobility history information and if the UE continues to be connected to the same PSCell during the change of the PCell in RRC_CONNECTED; or

3>	if the UE supports PSCell mobility history information and if the UE changes PSCell at the same time as the change of the PCell in RRC_CONNECTED; or

3>	if the UE supports PSCell mobility history information and if the PSCell is released at the same time as the change of the PCell in RRC_CONNECTED:

4>	include an entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after performing the following, if necessary:

5>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

6>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

7>	remove the oldest entry in the visitedPSCellInfoListReport;

5>	else:

6>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

4>	for the included entry:

5>	if the global cell identity of the PSCell (in case the UE continues to be connected to the same PSCell) or the previous PSCell (in case the UE changes PSCell, or in case PSCell is released) is available:

6>	include the global cell identity of that cell in the field visitedCellId of the entry;

5>	else:

6>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

5>	set the field timeSpent of the entry as the time spent in the PSCell, while being connected to previous PCell;

5>	if the UE supports storing and reporting SCG activation information in mobility history information:

6> set the field scgActiveDuration of the entry to the accumulated time spent in the PSCell with SCG state set to activated while being connected to the previous PCell, if available;

3>	if the UE supports PSCell mobility history information and if the UE was not configured with a PSCell at the time of change of PCell in RRC_CONNECTED; or

3>	if the UE supports PSCell mobility history information and if the UE was not configured with a PSCell at the time of entering 'camped normally' state while previously in RRC_CONNECTED state in a PCell different from the current serving cell; or

3>	if the UE supports PSCell mobility history information and if the UE was not configured with a PSCell at the time of entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE:

4>	include an entry in visitedPSCellInfoList in variable VarMobilityHistoryReport after performing the following, if necessary;

5>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

6>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

7>	remove the oldest entry in the visitedPSCellInfoListReport;

5>	else:

6>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

4>	for the included entry:

5>	set the field timeSpent of the entry as the time without PSCell according to the following:

6>	if the UE experienced a PSCell release since entering the previous PCell in RRC_CONNECTED:

7>	include the time spent with no PSCell since last PSCell release since entering the previous PCell in RRC_CONNECTED;

6>	else:

7>	include the time spent with no PSCell since entering the previous PCell in RRC_CONNECTED;

3>	if the UE supports PSCell mobility history information and if visitedPSCellInfoList exists in VarMobilityHistoryReport:

4>	include visitedPSCellInfoList in VarMobilityHistoryReport in the visitedPSCellInfoListReport within the entry of the visitedCellInfoList associated to the latest PCell entry;

4>	remove visitedPSCellInfoList from the variable VarMobilityHistoryReport;

1>	if the UE supports PSCell mobility history information and upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in RRC_CONNECTED state NR or LTE in the same serving cell which was previously the PCell while not connected to a PSCell:

2>	include an entry in visitedPSCellInfoList in variable VarMobilityHistoryReport after performing the following, if necessary;

3>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

4>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

5>	remove the oldest entry in the visitedPSCellInfoListReport;

3>	else:

4>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

2>	for the included entry:

3>	set the field timeSpent of the entry as the time without PSCell according to the following:

4>	if the UE experienced a PSCell release since entering the current serving cell in RRC_CONNECTED:

5>	include the time spent with no PSCell since last PSCell release after entering the current serving cell in RRC_CONNECTED;

4>	else:

5>	include the time spent with no PSCell since entering the current serving cell in RRC_CONNECTED;

1>	upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in 'any cell selection' state or 'camped on any cell' state in NR or LTE:

2>	include an entry in visitedCellInfoList in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	set the field timeSpent of the entry as the time spent in 'any cell selection' state and/or 'camped on any cell' state in NR or LTE.

#### 5.7.9.3	Release of Mobility History Information

If the UE supports storage of mobility history information, the UE shall:

1>	if stored, discard the mobility history information, i.e. release the UE variable VarMobilityHistoryReport upon deregistration from the network as specified in TS 23.502 [43].

### 5.7.10	UE Information

#### 5.7.10.1	General

Figure 5.7.10.1-1: UE information procedure

The UE information procedure is used by the network to request the UE to report information.

#### 5.7.10.2	Initiation

The network initiates the procedure by sending the UEInformationRequest message. The network should initiate this procedure only after successful security activation.

#### 5.7.10.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measIdleValidityDuration is included in VarEnhMeasIdleConfig;

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

3>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig;

2>	else:

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

3>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

1>	if the reselectionMeasurementReq is included in the UEInformationRequest:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig;

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement;

3> else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig;

2>	else:

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig;

3>	else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from the VarLogMeasReport starting from the entries logged first, and for each entry of the logMeasInfoList that is included, include all information stored in the corresponding logMeasInfoList entry in VarLogMeasReport;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

4>	if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableBT;

4>	if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableWLAN;

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report; or

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the registered SNPN identity is included in snpn-IdentityList stored in VarRA-Report:

2>	for each RA-Report stored in ra-ReportList in VarRA-Report that consists of failed SDT information:

3>	set timeSinceSdt-Execution to the time that elapsed since SDT execution;

2>	set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report;

2>	discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure or LTM cell switch execution failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

2>	else if the UE is capable of cross-RAT RLF reporting as defined in TS 38.306 [26] and has radio link failure information or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	set timeSinceFailure in VarRLF-Report of TS 36.331 [10] to the time that elapsed since the last radio link failure or handover failure in EUTRA;

3>	set failedPCellId-EUTRA in the rlf-Report in the UEInformationResponse message to indicate the PCell in which RLF was detected or the source PCell of the failed handover in the VarRLF-Report of TS 36.331 [10];

3>	set the measResult-RLF-Report-EUTRA in the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report of TS 36.331 [10];

3>	discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

1>	if connEstFailReportReq is set to true and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	if the UE supports multiple CEF report:

3>	for each connEstFailReport in the connEstFailReportList in VarConnEstFailReportList:

4>	set timeSinceFailure to the time that elapsed since the associated connection establishment failure or connection resume failure in NR;

2>	for each connEstFailReport in the connEstFailReportList in the UEInformationResponse message, set the value to the value of connEstFailReport in VarConnEstFailReport in VarConnEstFailReportList;

2>	discard the connEstFailReport from VarConnEstFailReport and VarConnEstFailReportList upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the mobilityHistoryReportReq is set to true:

2>	include the mobilityHistoryReport and set it to include visitedCellInfoList from VarMobilityHistoryReport;

2>	include in the mobilityHistoryReport an entry for the current PCell, possibly after removing the oldest entry if required, and set its fields as follows:

3>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PCell:

3>	set field timeSpent to the time spent in the current PCell;

3>	if the UE supports PSCell mobility history information and if visitedPSCellInfoList is present in VarMobilityHistoryReport:

4>	for the newest entry of the PCell in the mobilityHistoryReport, include visitedPSCellInfoList from VarMobilityHistoryReport;

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

6>	if the UE supports storing and reporting SCG activation information in mobility history information:

7> set the field scgActiveDuration of the entry to the accumulated time spent in the current PSCell with SCG state set to activated during the stay in the PSCell while being connected to the current PCell, if available;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since last PSCell release since connected to the current PCell in RRC_CONNECTED;

3>	else if the UE supports PSCell mobility history information:

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

6>	if the UE supports storing and reporting SCG activation information in mobility history information:

7>	set the field scgActiveDuration of the entry to the accumulated time spent in the current PSCell with SCG state set to activated while being connected to the current PCell, if available;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since connected to the current PCell in RRC_CONNECTED;

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the RPLMN is included in the plmn-IdentityList stored in VarSuccessHO-Report; or

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	if the successHO-Report in the VarSuccessHO-Report concerns a mobilityFromNRCommand:

3>	set timeSinceSHR in VarSuccessHO-Report to the time that elapsed since the execution of the MobilityFromNRCommand associated to the successful handover report in the VarSuccessHO-Report;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessPSCell-Report:

2>	set the successPSCell-Report in the UEInformationResponse message to the value of successPSCell-Report in the VarSuccessPSCell-Report;

2>	discard the VarSuccessPSCell-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the coarseLocationRequest is set to true:

2>	include coarseLocationInfo, if available;

1>	if the flightPathInfoReq is included in the UEInformationRequest and the UE has (updated) flight path information available, set the flightPathInfoReport in the UEInformationResponse message as follows:

2>	include the list of up to maxWayPointNumber waypoints, if any, along the flight path;

2>	if the includeTimeStamp is set to true, for each included waypoint:

3>	if available, set the field timestamp to the time when UE intends to arrive at the waypoint;

1>	if the csi-LogMeasReportReq is present:

2>	if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

3>	if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message:

4>	include the csi-MoreLogMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else if csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB6;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested in the same message.

#### 5.7.10.4	Actions for the Random Access report determination

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or

1>	if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the random-access procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	if the UE supports spCell ID indication:

5>	if the corresponding random-access procedure was performed on an SCell of MCG:

6>	set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on an SCell of SCG:

6>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on PSCell:

6>	if the cellId is not set to the global cell identity of the PSCell, set the spCellId to the global cell identity of the PCell;

4>	if the random-access procedure is initiated for SDT and the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

5>	set the sdt-FailureCause to the cause of SDT failure;

4>	if the conditions to initiate MO-SDT were evaluated and not fullfilled according to TS 38.321 [3]:

5>	set the sdt-DL-Rsrp-Info to the RSRP value measured at the time of SDT evaluation as specified in TS 38.321 [3];

5>	set the sdt-UL-DataVolume to the UL data volume at the time of SDT evaluation as specified in TS 38.321 [3];

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

1>	if the UE is in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of SNPN identity entries in snpn-IdentityList in identityList stored in VarRA-Report is less than maxNPN; or

2>	if the number of SNPN identity entries in snpn-IdentityList in identityList stored in VarRA-Report is equal to maxNPN and the list of equivalent SNPN(s) is subset of or equal to the snpn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the random-access procedure as a new entry in the VarRA-Report:

4>	if the list of equivalent SNPN(s) has been stored by the UE:

5>	set the snpn-IdentityList to include the list of equivalent SNPN(s) stored by the UE (i.e. includes the registered SNPN identity) without exceeding the limit of maxNPN;

4>	else:

5>	set the snpn-Identity, in snpn-IdentityList, to the SNPN identity selected by upper layers (see TS 24.501 [23]) from the SNPN identities included in the NPN-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	if the UE supports spCell ID indication:

5>	if the corresponding random-access procedure was performed on an SCell of MCG:

6>	set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on an SCell of SCG:

6>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on PSCell:

6>	if the cellId is not set to the global cell identity of the PSCell, set the spCellId to the global cell identity of the PCell;

4>	if the random-access procedure is initiated for SDT and the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

5>	set the sdt-FailureCause to the cause of SDT failure;

4>	if conditions to initiate MO-SDT were evaluated and not fullfilled according to TS 38.321 [3]:

5>	set the sdt-DL-Rsrp-Info to the RSRP value measured at the time of SDT evaluation as specified in TS 38.321 [3];

5>	set the sdt-UL-DataVolume to the UL data volume at the time of SDT evaluation as specified in TS 38.321 [3];

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure or the failed or successfully completed on-demand system information acquisition procedure or the failed or successfully completed RA-SDT procedure related information is added to the VarRA-Report.

NOTE 1:	Void.

#### 5.7.10.5	RA information determination

The UE shall, for the last successfully completed or last failed random-access procedure, set the content in ra-InformationCommon as follows:

1>	set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources used in the random-access procedure;

1>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources used in the random-access procedure;

1>	if contention based random-access resources are used in the random-access procedure:

2>	set the msgA-RO-FrequencyStart and msgA-RO-FDM and msgA-SubcarrierSpacing associated to the 2 step random- access resources if used in the random-access procedure;

2>	if msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure is available:

3>	set the msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure;

2>	else if only 2 step random-access resources are available in the UL BWP used in the random-access procedure:

3>	set the msgA-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the msgA-PRACH-ConfigurationIndex used in the 2-step random-access procedure;

2>	else:

3>	set the msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure;

2>	set the msg1-FrequencyStart associated to the 4 step random-access resources if used in the random-access procedure, and if its value is different from the value of msgA-RO-FrequencyStart if it is included in the ra-InformationCommon;

2>	set the msg1-FDM associated to the 4 step random-access resources if used in the random-access procedure, and if its value is different from the value of msgA-RO-FDM if it is included in the ra-InformationCommon;

2>	if msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure is available, and if its value is different from the value of msgA-SubcarrierSpacing if it is included in the ra-InformationCommon:

3>	set the msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure;

2> else:

3>	set the msg1-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the prach-ConfigurationIndex used in the 4-step random-access procedure, and if its value is different from the value of msgA-SCS-From-prach-ConfigurationIndex if it is included in the ra-InformationCommon;

1>	if contention free random-access resources are used in the random-access procedure:

2>	set the msg1-FrequencyStartCFRA and msg1-FDMCFRA associated to the 4 step random-access resources if used in the random-access procedure;

2> if msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure is available:

3>	set the msg1-SubcarrierSpacingCFRA associated to the 4 step random-access resources used in the random-access procedure;

2> else:

3>	set the msg1-SCS-From-prach-ConfigurationIndexCFRA to the subcarrier spacing as derived from the prach-ConfigurationIndex used in the 4 step random-access procedure;

2>	set the msgA-RO-FrequencyStartCFRA and msgA-RO-FDMCFRA associated to the 2 step contention free random access resources if used in the random-access procedure;

2>	set the msgA-MCS, the nrofPRBs-PerMsgA-PO, the msgA-PUSCH-TimeDomainAllocation, the frequencyStartMsgA-PUSCH, the nrofMsgA-PO-FDM associated to the 2 step random-access resources if used in the random-access procedure;

2>	if msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure is available:

3>	set the msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure;

2>	else if only 2 step random-access resources are available in the UL BWP used in the random-access procedure:

3>	set the msgA-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the msgA-PRACH-ConfigurationIndex used in the 2-step random-access procedure;

2>	else:

3>	set the msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure;

1>	if the random access procedure is initialized with RA_TYPE set to 2-stepRA as described in TS 38.321 [3]:

2>	set the dlPathlossRSRP to the measeured RSRP of the DL pathloss reference obtained at the time of RA_Type selection stage of the initialization of the RA procedure as captured in TS 38.321 [3];

2>	if the configuration for the random access msgA-TransMax was configured in RACH-ConfigDedicated for this random access procedure, and raPurpose is set to reconfigurationWithSync or ltm:

3>	set msgA-TransMax to the value of msgA-TransMax in RACH-ConfigDedicated;

2>	else if msgA-TransMax was configured in RACH-ConfigCommonTwoStepRA:

3>	set msgA-TransMax to the value of msgA-TransMax in RACH-ConfigCommonTwoStepRA;

2>	set the msgA-PUSCH-PayloadSize to the size of the overall payload available in the UE buffer at the time of initiating the 2 step RA procedure;

1>	if the purpose of the random access procedure is to request on-demand system information (i.e., if the raPurpose is set to requestForOtherSI or msg3RequestForOtherSI):

2>	set the intendedSIBs to indicate the SIB(s) the UE wanted to receive as a result of the SI request;

2>	set the ssbsForSI-Acquisition to indicate the SSB(s) used to receive the SI message;

2>	if the on-demand system information acquisition was successful:

3>	set the onDemandSISuccess to true;

1>	if one or more of the features including RedCap and/or Slicing and/or SDT and/or MSG3 repetitions and/or MSG1 repetitions and/or eRedCap are applicable for this random-access procedure as specified in clause 5.1.1b of TS 38.321[3]:

2>	set the triggeredFeatureCombination to indicate all the features triggering this random-access procedure as below:

3>	if this random-access procedure is triggered by RedCap, include redCap;

3>	if this random-access procedure is triggered by SDT, include smallData;

3>	if this random-access procedure is triggered by Msg3 repetitions, include msg3-Repetitions;

3>	if this random-access procedure is triggered by Msg1 repetitions, include msg1-Repetitions;

3>	if this random-access procedure is triggered by eRedCap, include eRedCap;

3>	if this random-access procedure is triggered by slicing, set nsag to the NSAG-ID(s) applied in the random-access procedure and set the triggered-S-NSSAI-List to include all the S-NSSAI(s) associated to the slices triggering the access attempt in the random-access procedure;

2>	if the value of used feature or combination of features is different from the triggeredFeatureCombination:

3>	set the usedFeatureCombination to indicate one or more features of FeatureCombination associated to the random-access resource used in the random-access procedure as below:

4>	if RedCap is part of the used FeatureCombination, include redCap;

4>	if SDT is part of the used FeatureCombination, include smallData;

4>	if Msg3 repetitions is part of the used FeatureCombination, include msg3-Repetitions;

4>	if Msg1 repetitions is part of the used FeatureCombination, include msg1-Repetitions;

4>	if eRedCap is part of the used FeatureCombination, include eRedCap;

4>	if NSAG(s) is part of the used FeatureCombination, set nsag to include the NSAG-ID(s) configured for the used FeatureCombination;

2>	set the startPreambleForThisPartition to the value of startPreambleForThisPartition in the last used FeatureCombinationPreambles associated to the used FeatureCombination;

2>	set the numberOfPreamblesPerSSB-ForThisPartition to the value of numberOfPreamblesPerSSB-ForThisPartition in the last used FeatureCombinationPreambles associated to the used FeatureCombination;

1>	set the parameters associated to the successive random-access attempts associated to the selected beam in the perRAInfoList as follows:

2>	if the random-access resource used is associated to a SS/PBCH block, set the associated random-access parameters for the successive random-access attempts associated to the same SS/PBCH block for one or more random-access attempts as follows:

3>	set the ssb-Index to include the SS/PBCH block index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnSSB to indicate the number of successive random-access attempts associated to the SS/PBCH block;

3>	if all preamble transmissions for the successive random-access attempts associated to this SS/PBCH block were blocked by LBT:

4>	include allPreamblesBlocked;

3>	else:

4>	if LBT failure indication was received from lower layers for the last random-access preamble transmission attempt in the SS/PBCH block associated to the ssb-Index, before changing the SS/PBCH block for random access preamble transmission:

5>	include lbt-Detected;

3>	for each random-access attempt performed on the random-access resource, except the random-access attempts for which LBT failure indication was received from lower layers, include the following parameters in the chronological order of the random-access attempt:

4>	if the random-access attempt is performed on the contention based random-access resource and if raPurpose is not equal to 'requestForOtherSI', include contentionDetected as follows:

5>	if contention resolution was not successful as specified in TS 38.321 [6] for the transmitted preamble:

6>	set the contentionDetected to true;

5>	else:

6>	set the contentionDetected to false;

4>	if the random access attempt is a 2-step random access attempt:

5>	if fallback from 2-step random access to 4-step random access occurred during the random access attempt:

6>	set fallbackToFourStepRA to true;

4>	if the random-access attempt is performed on the contention based random-access resource; or

4>	if the random-access attempt is performed on the contention free random-access resource and if the random-access procedure was initiated due to the PDCCH ordering:

5>	if the random access attempt is a 4-step random access attempt and the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above rsrp-ThresholdSSB; or

5>	if the random access attempt is a 2-step random access attempt and the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above msgA-RSRP-ThresholdSSB:

6>	set the dlRSRPAboveThreshold to true;

5>	else:

6>	set the dlRSRPAboveThreshold to false;

2>	else if the random-access resource used is associated to a CSI-RS, set the associated random-access parameters for the successive random-access attempts associated to the same CSI-RS for one or more random-access attempts as follows:

3>	set the csi-RS-Index to include the CSI-RS index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnCSI-RS to indicate the number of successive random-access attempts associated to the CSI-RS;

3>	if all preamble transmissions for the successive random-access attempts associated to this CSI-RS were blocked by LBT:

4>	include allPreamblesBlocked;

3>	else:

4>	if LBT failure indication was received from lower layers for the last random-access preamble transmission attempt in the CSI-RS associated to the csi-RS-Index, before changing the CSI-RS for random access preamble transmission:

5>	include lbt-Detected;

1>	if at least one LBT failure indication has been received from lower layers during the random-access procedure:

2>	set the numberOfLBT-Failures to indicate the total number of random-access attempts for which LBT failure indications have been received from lower layers in the random-access procedure.

The UE shall, for all the BWPs in which consistent LBT failures are triggered and not cancelled (according to TS 38.321 [3]) prior to successful RA completion or prior to RLF/HOF, set the below parameters in attemptedBWP-InfoList in the chronological order of BWP selection:

1>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP.

NOTE 1:	Void.

NOTE 2:	If allPreamblesBlocked is included, it is left to UE implementation how to set the numberOfPreamblesSentOnSSB-r16, numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList-r16.

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running; or:

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA; or

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the Mobility from NR to E-UTRA and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity), if available;

3>	for intra-NR handover, set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

4>	set the sourceCellMeas in sourceCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

4>	set the rsIndexResults in sourceCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config and LTM-CSI-ReportConfig associated with the source PCell when connected to the source PCell:

5>	set the resultsSSB-Indexes in sourceCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

5>	set the rlf-InSourceDAPS in sourceCellInfo to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if the UE was configured with condExecutionCond and condExecutionCondPScell, for the source PSCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied;

4>	set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, if available, of the source PSCell;

4>	set the sourcePSCellMeas in sourcePSCellInfo to include the available cell level RSRP, RSRQ and the SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync;

4>	set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync;

4>	set the targetPSCellId to the global cell identity and tracking area code, if available, of the target PSCell, and otherwise to the physical cell identity and carrier frequency of the target PSCell;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the target PCell indicated in the last applied RRCReconfiguration message including reconfigurationWithSync:

4>	set the targetPCellID in targetCellInfo to the global cell identity and tracking area code, if available, of the target PCell; otherwise, set the targetCell-PCI-ARFCN to the physical cell identity and carrier frequency of the target PCell;

NOTE 00:	If targetCell-PCI-ARFCN is included, it is left to UE implementation how to set the targetPCellID.

4>	set the targetCellMeas in targetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in targetCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config and LTM-CSI-ReportConfig associated with the target PCell when connected to the source PCell:

5>	set the resultsSSB-Indexes in targetCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last applied RRCReconfiguration message including reconfigurationWithSync was included in the stored condRRCReconfig:

5>	set the timeSinceCHO-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PCell and the reception of the last applied conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell;

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the target PCell indicated in the last applied MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA:

4>	set the targetPCellId in eutra-TargetCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell;

4>	set the targetCellMeas in eutra-TargetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available measurements collected up to the moment the UE sends RRCConnectionReconfigurationComplete message;

NOTE 0:	If eutra-TargetCellInfo is included, it is left to UE implementation how to set the targetCellInfo.

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the T304 timer, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync:

4>	set t304-cause in shr-Cause to true;

4>	if the procedure is triggered due to successful completion of RACH-based reconfiguration with sync:

5>	set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PCell, as specified in clause 5.7.10.5;

3>	if the ratio between the value of the elapsed time of the timer T310 and the configured value of the T310 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync or the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync or Mobility from NR to E-UTRA:

4>	set t310-cause in shr-Cause to true;

3>	if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure or Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the T312 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync or Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync, or Mobility from NR to E-UTRA:

4>	set t312-cause in shr-Cause to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if sourceDAPS-FailureReporting included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync is set to true, and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

4>	set sourceDAPS-Failure in shr-Cause to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectNR, configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied;or:

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for each of the measObjectNR, configured by the source PCell, in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	if measRSSI-ReportConfig is configured for the frequency of the source PCell:

5>	if the procedure is triggered due to successful completion of reconfiguration with sync:

6>	set the measResultServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell up to the moment the UE sends the RRCReconfigurationComplete message;

5>	else if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA:

6>	set the measResultServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message;

4>	for each of the configured measObjectNR if measRSSI-ReportConfig is configured for the configured frequency:

5>	if the procedure is triggered due to successful completion of reconfiguration with sync:

6>	set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the associated neighbouring frequency up to the moment the UE sends the RRCReconfigurationComplete message:

7>	for each neighbour frequency included, include the optional fields that are available;

5>	else if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA:

6>	set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the associated neighbouring frequency up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message:

7>	for each neighbour frequency included, include the optional fields that are available;

4>	if measurements are available for the measObjectNR:

5>	if the SS/PBCH block-based measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	For the neighboring cells set included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, the UE includes also the CSI-RS based measurement quantities, if available.

5>	if the CSI-RS measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell and target PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	For the neighboring cells set ordered based on the CSI-RS measurement quantities, the UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config including LTM-CSI-ReportConfig associated with the MCG when connected to the source PCell:

4>	for each neighbour MCG LTM candidate cell:

5>	if SS/PBCH block-based L1-RSRP measurement results are available:

6>	set the neighCellsMeasL1ListNR to include all the available SS/PBCH block-based L1-RSRP measurement results of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first, based on the available SS/PBCH block-based L1-RSRP measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectEUTRA, configured by the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or:

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for each of the measObjectEUTRA, configured by the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	if measurements are available for the measObjectEUTRA:

5>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, based on measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

5>	for each neighbour cell included, include the optional fields that are available;

3>	for each of the neighbour cells included in measResultNeighCells:

4>	if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration configured by the source PCell including the condExecutionCond within the conditionalReconfiguration associated to condEventA3 or condEventA5, in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

5>	set the choCandidate to true in measResultNR;

3>	if the UE supports successful handover report for MCG LTM cell switch and the procedure is triggered due to successful completion of reconfiguration with sync concerning an LTM cell switch:

4>	if the last executed LTM cell switch is a RACH-less LTM cell switch:

5>	include the rach-Less;

3>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

4>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first in time;

4>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

5>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;4>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration for the same target candidate PCell as set in pCellId:

5>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

3>	if available, set the locationInfo as in 5.3.3.7;

1>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell.

The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report.

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

The UE shall for the PSCell:

1>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message for the SCG including the reconfigurationWithSync, is greater than thresholdPercentageT304-SCG if included in the successPSCell-Config received before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG:

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG:

2>	clear the information included in VarSuccessPSCell-Report, if any;

2>	store the successful PSCell change or addition information in VarSuccessPSCell-Report and determine the content in VarSuccessPSCell-Report as follows:

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs (including the RPLMN) stored by the UE;

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPN identities (including the registered SNPN identity) stored by the UE, if available;

3>	if the procedure is triggered due to successful completion of CHO with candidate SCG:

4>	set the pCellId to the global cell identity and if available the tracking area code of the source PCell;

4>	set the targetPCellId to the global cell identity and tracking area code, if available, of the target PCell, and otherwise to the physical cell identity and carrier frequency of the target PCell;

4>	set the c-RNTI to the C-RNTI assigned by the target PCell of the successful conditional handover;

3>	else:

4>	set the pCellId to the global cell identity and, if available, tracking area code of the PCell;

3>	for the source PSCell (in case of PSCell change procedure) in which the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied:

4>	set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, and otherwise to the physical cell identity and carrier frequency of the source PSCell;

4>	set the sourcePSCellMeas in sourcePSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG;

3>	for the target PSCell indicated in the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync:

4>	set the targetPSCellID in targetPSCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PSCell;

4>	set the targetPSCellMeas in targetPSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	set the rsIndexResults in targetPSCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	if the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync was included in the stored condRRCReconfig:

5>	set the timeSinceCPAC-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PSCell and the reception of the last applied conditionalReconfiguration for the SCG including the condRRCReconfig of the target PSCell;

3>	if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT304-SCG is met:

4>	set t304-cause in spr-Cause to true;

4>	set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PSCell, as specified in clause 5.7.10.5;

3>	if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT310-SCG is met:

4>	set t310-cause in spr-Cause to true;

3>	if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT312-SCG is met:

4>	set t312-cause in spr-Cause to true;

3>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured:

4>	consider all measObjectNR configured by the source PSCell;

3>	else:

4>	consider all measObjectNR configured by the PCell;

3>	for each of the considered measObjectNR:

4>	if measurements are available for the measObjectNR:

5>	if the SS/PBCH block-based measurement quantities are available:

6>	include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PSCell (in case of PSCell change procedure) or target PSCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE successfully completed the random access procedure;

6>	for each neighbour cell included, include the optional fields that are available (including the CSI-RS based measurement quantities, if available);

5>	if the CSI-RS measurement quantities are available for the cells not yet included in measResultListNR in measResultNeighCells:

6>	include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PSCell (in case of PSCell change procedure) and target PSCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE successfully completed the random access procedure;

6>	for each neighbour cell included, include the optional fields that are available;

3>	for each of the neighbour cells included in measResultNeighCells:

4>	if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration, configured by the source PCell including the condExecutionCond within the conditionalReconfiguration associated to condEventA4 or by the source PSCell (in case of PSCell change) when the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied:

5>	set the choCandidate to true in measResultNR;

3>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

4>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first in time;

4>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

5>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

3>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured:

4>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the source PSCell;

4>	include sn-InitiatedPSCellChange;

3>	else:

4>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the PCell;

1>	release successPSCell-Config configured by the source PSCell if available and thresholdPercentageT304 if configured by the target PSCell.

The UE may discard the successful PSCell change or addition information, i.e., release the UE variable VarSuccessPSCell-Report, 48 hours after the last successful PSCell change or addition information is added to the VarSuccessPSCell-Report or upon deregistration from the network as specified in TS 23.502 [43].

### 5.7.11	Void

### 5.7.12	IAB Other Information

#### 5.7.12.1	General

Figure 5.7.12.1-1: IAB Other Information procedure

The IAB Other Information procedure is used by IAB-MT to request the IAB-donor-CU to allocate IP address or inform the IAB-donor-CU of the IP address for the collocated IAB-DU.

#### 5.7.12.2	Initiation

Upon initiation of the procedure, the IAB-MT shall:

1>	initiate transmission of the IABOtherInformation message in accordance with 5.7.12.3;

#### 5.7.12.3	Actions related to transmission of IABOtherInformation message

The IAB-MT shall set the contents of IABOtherInformation message as follows:

1>	if the procedure is used to request IP addresses:

2>	if IPv4 addresses are requested:

3>	set the iab-IPv4-AddressNumReq to the number of IPv4 addresses requested per specific usage;

2>	if IPv6 addresses or IPv6 address prefixes are requested:

3>	if IPv6 addresses are requested:

4>	set the iab-IPv6-AddressNumReq to the number of IPv6 addresses requested per specific usage;

3>	else if IPv6 address prefixes are requested:

4>	set the iab-IPv6-AddressPrefixReq to true per specific usage;

1>	if the procedure is used to report IP addresses:

2>	if IPv4 addresses are reported:

3>	include iPv4-Address in iab-IPv4-AddressReport, and for each IP address included:

4>	if IPv4 addresses are used for F1-C traffic:

5>	include these addresses in f1-C-Traffic-IP-Address.

4>	if IPv4 addresses are used for F1-U traffic:

5>	include these addresses in f1-U-Traffic-IP-Address.

4>	if IPv4 address are used for non-F1 traffic:

5>	include these addresses in non-f1-Traffic-IP-Address.

4>	if IPv4 addresses are used for all traffic:

5>	include these addresses in all-Traffic-IAB-IP-Address.

2>	if IPv6 addresses or IPv6 address prefixes are reported:

3>	if IPv6 addresses are reported:

4>	include iPv6-Address in iab-IPv6-AddressReport, and for each IP address included;

5>	if IPv6 addresses are used for F1-C traffic:

6>	include these addresses in f1-C-Traffic-IP-Address.

5>	if IPv6 addresses are used for F1-U traffic:

6>	include these addresses in f1-U-Traffic-IP-Address.

5>	if IPv6 addresses are used for non-F1 traffic:

6>	include these addresses in non-f1-Traffic-IP-Address.

5>	if IPv6 addresses are used for all traffic:

6>	include these addresses in all-Traffic-IAB-IP-Address.

3>	else if IPv6 address prefixes are reported:

4>	include these iPv6-Prefix in iab-IPv6-PrefixReport, and for each IP address prefix included;

5>	if this IPv6 address prefix is used for F1-C traffic:

6>	include this prefix in f1-C-Traffic-IP-Address.

5>	if this IPv6 address prefix is used for F1-U traffic:

6>	include this prefix in f1-U-Traffic-IP-Address.

5>	if this IPv6 address prefix is used for non-F1 traffic:

6>	include this prefix in non-f1-Traffic-IP-Address.

5>	if this IPv6 address prefix is used for all traffic:

6>	include this prefix in all-Traffic-IAB-IP-Address.

1>	if the IAB-MT is in (NG)EN-DC, or

1>	if the IAB-MT is in NR-DC and the IAB Other Information procedure is towards the IAB-donor-CU in the SN:

2>	if SRB3 is configured:

3>	submit the IABOtherInformation message via SRB3 to lower layers for transmission;

2>	else if the IAB-MT is in (NG)EN-DC:

3>	submit the IABOtherInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10];

2>	else:

3>	submit the IABOtherInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3;

1>	else:

2>	submit the IABOtherInformation message to lower layers for transmission.

### 5.7.13	RLM/BFD relaxation

#### 5.7.13.0	General

In case both low mobility criterion and good serving cell criterion are configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when both relaxed measurement criterion for low mobility and relaxed measurement criterion for good serving cell quality are met.

In case only the good serving cell quality criterion is configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when the relaxed measurement criterion for good serving cell quality is met.

When the short DRX cycle is smaller than or equal to 80 ms and the long DRX cycle is larger than 80 ms then the UE is allowed to perform RLM and/or BFD relaxation while drx-ShortCycleTimer is running and the relaxed measurement criteria are met. In this case the UE does not report change of its relaxation state for RLM and/or BFD relaxation due to start or stop of drx-ShortCycleTimer, if configured.

#### 5.7.13.1	Relaxed measurement criterion for low mobility

The relaxed measurement criterion for UE with low mobility in RRC_CONNECTED is fulfilled when:

-	(SS-RSRPRef – SS-RSRP) < SSearchDeltaP-Connected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the SpCell based on SSB (dB).

-	SS-RSRPRef = reference L3 RSRP measurement of the SpCell based on SSB (dB), set as follows:

-	After receiving low mobility criterion configuration, or

-	After MAC of the CG successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig of the CG while low mobility criterion is configured, or

-	upon an indication from lower layers that a RACH-less handover or an LTM cell switch is successfully completed for the CG while low mobility criterion is configured, or

-	If (SS-RSRP - SS-RSRPRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Connected:

-	The UE shall set the value of SS-RSRPRef to the current SS-RSRP value of the SpCell.

#### 5.7.13.2	Relaxed measurement criterion for good serving cell quality

The relaxed measurement criterion of good serving cell quality for RLM is fulfilled when the downlink radio link quality on the configured RLM-RS resource is evaluated to be better than the threshold Qin+XdB, wherein

-	Qin is specified in clause 8.1 of TS 38.133 [14].

-	X is the parameter offset in goodServingCellEvaluationRLM for the evaluated serving cell.

The relaxed measurement criterion of good serving cell quality for BFD is fulfilled when the downlink radio link quality on the configured BFD-RS resource is evaluated to be better than the threshold Qin+XdB, wherein

-	Q in is specified in clause 8.1 of TS 38.133 [14].

-	X is the parameter offset in goodServingCellEvaluationBFD for the evaluated serving cell.

### 5.7.14	UE Positioning Assistance Information

#### 5.7.14.1	General

Figure 5.7.14.1-1: UE Positioning Assistance Information procedure

The UE Positioning Assistance Information procedure is used by UE to report the UE Positioning Assistance Information. The UE reports the association between SRS resources for positioning and the UE Tx TEG ID as defined in TS 38.305 [73].

#### 5.7.14.2	Initiation

A UE capable of providing the association between SRS resource for positioning and UE Tx TEG ID in RRC_CONNECTED may initiate the procedure upon being configured to provide this association information.

Upon initiation of the procedure, the UE shall:

1>	initiate transmission of the UEPositioningAssistanceInfo message in accordance with 5.7.14.3 to provide the association.

#### 5.7.14.3	Actions related to transmission of UEPositioningAssistanceInfo message

The UE shall set the contents of the UEPositioningAssistanceInfo message as follows:

1>	if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message is configured with periodicReporting:

2>	for all the association changes store ue-TxTEG-Association corresponding to each ue-TxTEG-ID with nr-TimeStamp;

2>	include the results in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period;

2>	optionally include one ue-TxTEG-TimingErrorMarginValue for each UEPositioningAssistanceInfo message;

1>	else if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message is configured with oneShot:

2>	identify the ue-TxTEG-Association corresponding to each ue-TxTEG-ID with nr-TimeStamp;

2>	include the results in ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message only one time;

2>	optionally include one ue-TxTEG-TimingErrorMarginValue for each UEPositioningAssistanceInfo message.

The UE shall submit the UEPositioningAssistanceInfo message to lower layers for transmission.

### 5.7.15	Void

### 5.7.16	Application layer measurement reporting

#### 5.7.16.1	General

Figure 5.7.16.1-1: Application layer measurement reporting

The purpose of this procedure is to send application layer measurement reports to the network.

#### 5.7.16.2	Initiation

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement and reporting, i.e. when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network.

Upon initiating the procedure, the UE shall:

1>	for each stored application layer measurement configuration with appLayerIdleInactiveConfig configured and for which appLayerIdleInactiveConfig has not been transmitted since the UE received the RRCSetup message:

2>	set the parameters in appLayerIdleInactiveConfig in the MeasurementReportAppLayer message to the values stored in the UE variable VarAppLayerIdleConfig;

2>	if the transmissionOfSessionStartStop is set to true:

3>	if a session start or stop indication has been received from upper layers:

4>	set the appLayerSessionStatus in the MeasurementReportAppLayer message to the value of the latest received session start or stop indication;

3>	else:

4>	set the appLayerSessionStatus in the MeasurementReportAppLayer message to stop;

1>	for each measConfigAppLayerId with an application layer measurement report container which has not been transmitted since the UE received the RRCSetup message:

2>	if the application layer measurement reporting has not been suspended for the measConfigAppLayerId associated with the application layer measurement report container according to clause 5.3.5.13d:

3>	set the measConfigAppLayerId in a MeasReportAppLayer in the MeasurementReportAppLayer message(s) to the value of the measConfigAppLayerId received together with the application layer measurement report container;

3>	for each application measurement report container stored for that measConfigAppLayerId:

4>	set the measReportAppLayerContainerList in the MeasurementReportAppLayer message(s) to the stored value of the application layer measurement report container;

NOTE 0:	If the size of the application layer measurement configurations, session status indications and reports exceed the size of an RRC message, the UE includes the appLayerIdleInactiveConfig and corresponding appLayerSessionStatus in the first MeasurementReportAppLayer message after the UE received the RRCSetup message, and transmits application layer measurement report containers in the same and subsequent MeasurementReportAppLayer messages.

1>	for each measConfigAppLayerId received from upper layers:

2>	if the UE AS has received application layer measurement report container from upper layers which has not been transmitted; and

2>	if the application layer measurement reporting has not been suspended for the measConfigAppLayerId associated with the application layer measurement report container according to clause 5.3.5.13d:

3>	set the measReportAppLayerContainer in the MeasurementReportAppLayer message to the received value in the application layer measurement report container;

2>	set the measConfigAppLayerId in the MeasurementReportAppLayer message to the value of the measConfigAppLayerId received together with application layer measurement report information;

2>	if session start or stop information has been received from upper layers for the measConfigAppLayerId:

3>	set the appLayerSessionStatus in the MeasurementReportAppLayer message to the latest received value of session start or stop information;

2>	if a RAN visible application layer measurement report has been received from upper layers:

3>	for each appLayerBufferLevel value in the received RAN visible application layer measurement report:

4>	set the appLayerBufferLevel values in the appLayerBufferLevelList in the MeasurementReportAppLayer message to the buffer level values received from the upper layer in the order with the first appLayerBufferLevel value set to the newest received buffer level value, the second appLayerBufferLevel value set to the second newest received buffer level value, and so on until all the buffer level values received from the upper layer have been assigned or the maximum number of values have been set according to appLayerBufferLevel, if configured;

3>	set the playoutDelayForMediaStartup in the MeasurementReportAppLayer message to the received value of playout delay for media startup in the RAN visible application layer measurement report, if any;

3>	for each PDU session ID value indicated in the received RAN visible application layer measurement report, if any:

4>	set the PDU-SessionID in the pdu-SessionIdList in the MeasurementReportAppLayer message to the indicated PDU session ID value;

4>	for each QoS Flow ID value indicated in the received RAN visible application layer measurement report associated with the PDU Session ID, if any:

5>	set the QFI associated with the PDU session ID to the indicated QoS Flow ID value.

1>	if reportingSRB (or SRB4 if reportingSRB is absent) and ran-VisibleReportingSRB (or SRB4 if ran-VisibleReportingSRB is absent) indicates different SRBs for the measConfigAppLayerId:

2>	include measReportAppLayerContainer and appLayerSessionStatus in a separate MeasurementReportAppLayer message than ran-VisibleMeasurements;

1>	for each encoded MeasurementReportAppLayer message generated above which contains either only encapsulated application layer measurement reports or both encapsulated application layer measurement reports and RAN visible application layer measurement reports:

2>	if reportingSRB is not configured for the measConfigAppLayerId:

3>	if the encoded RRC message is larger than the maximum supported size of one PDCP SDU specified in TS 38.323 [5]:

4>	if the RRC message segmentation is enabled based on the field rrc-SegAllowedSRB4 received in appLayerMeasConfig:

5>	initiate the UL message segment transfer procedure as specified in clause 5.7.7 for transmission via SRB4;

4>	else:

5>	discard the RRC message;

3>	else:

4>	submit the MeasurementReportAppLayer message to lower layers for transmission via SRB4.

2>	else if reportingSRB is configured for the measConfigAppLayerId:

3>	if the encoded RRC message is larger than the maximum supported size of one PDCP SDU specified in TS 38.323 [5]:

4>	if the RRC message segmentation is enabled based on the field rrc-SegAllowedSRB4 received in appLayerMeasConfig and the reportingSRB is SRB4, or

4>	if RRC message segmentation is enabled based on the field rrc-SegAllowedSRB5 received in appLayerMeasConfig and the reportingSRB is SRB5:

5>	initiate the UL message segment transfer procedure as specified in clause 5.7.7 for transmission via the SRB indicated in the field reportingSRB in MeasConfigAppLayer;

4>	else:

5>	discard the RRC message;

3>	else:

4>	submit the MeasurementReportAppLayer message to lower layers for transmission via the SRB indicated in the field reportingSRB in MeasConfigAppLayer.

1>	for each encoded MeasurementReportAppLayer message generated above which contains only RAN visible application layer measurement reports:

2>	if ran-VisibleReportingSRB is not configured for the measConfigAppLayerId:

3>	submit the MeasurementReportAppLayer message to lower layers for transmission via SRB4;

2>	else if ran-VisibleReportingSRB is configured for the measConfigAppLayerId:

3>	submit the MeasurementReportAppLayer message to lower layers for transmission via the SRB indicated in the field ran-VisibleReportingSRB in MeasConfigAppLayer.

NOTE 1:	If the SRB indicated by reportingSRB is not available or if reportingSRB is absent and SRB4 is not available, the UE may store application layer measurement report containers until the SRB is available. The UE may discard reports when the memory reserved for storing application layer measurement report containers becomes full. Reports with lower appLayerMeasPriority are discarded first. If equal or no appLayerMeasPriority is configured, older reports are discarded first.

NOTE 2:	If the SRB indicated by ran-VisibleReportingSRB is not available or if ran-VisibleReportingSRB is absent and SRB4 is not available, the UE discards RAN visible application layer measurement reports.

### 5.7.17	Derivation of pathloss reference for TA validation of SRS for Positioning transmission and CG-SDT in RRC_INACTIVE

Upon request from lower layer for pathloss reference derivation for TA validation for SRS for Positioning transmission or CG-SDT in RRC_INACTIVE, the UE shall:

1>	acquire SIB2, if stored version is invalid;

1>	if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation:

2>	derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24];

1>	else:

2>	derive the downlink pathloss reference RSRP for TA validation as the linear average of the power values of up to nrofSS-BlocksToAverage of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24].

NOTE:	If the UE is configured with multiple SSB configurations, the downlink pathloss reference RSRP for TA validation is derived from the SSB configured by SIB1.

### 5.7.18	Void

### 5.7.19	Satellite switch with resynchronization

A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19.

Upon initiating the procedure, the UE shall:

1>	if t-ServiceStart is included in SIB19 and the UE supports soft satellite switch with resynchronization:

2>	start acquiring DL synchronization with the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync between the time indicated by t-ServiceStart and the time indicated by t-Service for the serving cell;

1>	upon the time indicated by t-Service:

2>	stop timer T430 if running;

2>	inform lower layers that UL synchronisation is lost due to satellite switch with resynchronization;

2>	synchronise to the DL of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync, if the UE has not previously synchronized to the DL of the SpCell;

2>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime in ntn-Config in SatSwitchWithReSync;

2>	inform lower layers when UL synchronisation is obtained.

NOTE:	UE should attempt to re-acquire SIB19 after satellite switch with resynchronization. The exact time is left to UE implementation.

### 5.7.20	Actions related to Transmission of SRS for Positioning in a validity area in RRC_INACTIVE

When configured with srs-PosRRC-InactiveValidityAreaPreConfigList, the UE shall:

1>	upon receiving RRCRelease message after sending the RRCResumeRequest message with the resumeCause set to srs-PosConfigOrActivationReq:

2>	if the current camped cell is included in any srs-PosConfigValidityArea in the srs-PosRRC-InactiveValidityAreaPreConfigList:

3>	apply the corresponding preconfigured SRS for positioning available in srs-PosRRC-InactiveValidityAreaPreConfigList and instruct lower layers to initiate SRS for Positioning transmission.

## 5.8	Sidelink

### 5.8.1	General

NR sidelink communication consists of unicast, groupcast and broadcast. For unicast, the PC5-RRC connection is a logical connection between a pair of a Source Layer-2 ID and a Destination Layer-2 ID in the AS. The PC5-RRC signalling, as specified in clause 5.8.9, can be initiated after its corresponding PC5 unicast link establishment (TS 23.287 [55]). The PC5-RRC connection and the corresponding sidelink SRBs and sidelink DRB(s) are released when the PC5 unicast link is released as indicated by upper layers.

For each PC5-RRC connection of unicast, one sidelink SRB (i.e. SL-SRB0) is used to transmit the PC5-S message(s) before the PC5-S security has been established. One sidelink SRB (i.e. SL-SRB1) is used to transmit the PC5-S messages to establish the PC5-S security. One sidelink SRB (i.e. SL-SRB2) is used to transmit the PC5-S messages after the PC5-S security has been established, which is protected. One sidelink SRB (i.e. SL-SRB3) is used to transmit the PC5-RRC signalling, which is protected and only sent after the PC5-S security has been established. One sidelink SRB (i.e. SL-SRB4) is used to transmit/receive the NR sidelink discovery messages.

For unicast of NR sidelink communication, AS security comprises of integrity protection of PC5 signalling (SL-SRB1, SL-SRB2 and SL-SRB3) and user data (SL-DRBs), and it further comprises of ciphering of PC5 signaling (SL-SRB1 only for the Direct Link Security Mode Complete message as specified in TS 24.587 [57] for V2X service or TS 24.554 [72] for Proximity-services, SL-SRB2 and SL-SRB3) and user data (SL-DRBs). The ciphering and integrity protection algorithms and parameters for a PC5 unicast link are exchanged by PC5-S messages in the upper layers as specified in TS 33.536 [60], and applied to the corresponding PC5-RRC connection in the AS. Once AS security is activated for a PC5 unicast link in the upper layers as specified in TS 33.536 [60], all messages on SL-SRB2 and SL-SRB3 and/or user data on SL-DRBs of the corresponding PC5-RRC connection are integrity protected and/or ciphered by the PDCP.

For unicast of NR sidelink communication, if the change of the key is indicated by the upper layers as specified in TS 24.587 [57] or TS 24.554 [72], UE re-establishes the PDCP entity of the SL-SRB1, SL-SRB2, SL-SRB3 and SL-DRBs on the corresponding PC5-RRC connection.

NOTE 1:	In case the configurations for NR sidelink communication are acquired via the E-UTRA, the configurations for NR sidelink communication in SIB12 and sl-ConfigDedicatedNR within RRCReconfiguration used in clause 5.8 are provided by the configurations in SystemInformationBlockType28 and sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331 [10], respectively.

NOTE 2:	In this release, there is one-to-one correspondence between the PC5-RRC connection and the PC5 unicast link as specified in TS 38.300[2].

NOTE 3:	All SL-DRBs related to the same PC5-RRC connection have the same activation/deactivation setting for ciphering and the same activation/deactivation setting for integrity protection as specified in TS 33.536 [60].

NOTE 4:	When integrity check failure concerning SL-SRB1 for a specific destination is detected, the UE sends an indication to the upper layers [57].

NOTE 5:	The selection of NULL algorithms means that the PC5 messages are considered protected for the purposes of being allowed to be sent or received.

### 5.8.2	Conditions for NR sidelink communication/discovery/positioning operation

The UE shall perform NR sidelink communication/discovery/positioning operation only if the conditions defined in this clause are met:

1>	if the UE's serving cell is suitable (RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED); and if either the selected cell on the frequency used for NR sidelink communication/discovery/positioning operation belongs to the registered or equivalent PLMN as specified in TS 24.587 [57] or TS 24.554 [72] or belongs to the Subscribed SNPN as specified in TS 24.554 [72] or the UE is out of coverage on the frequency used for NR sidelink communication/discovery/positioning operation as defined in TS 38.304 [20] and TS 36.304 [27]; or

1>	if the UE's serving cell (RRC_IDLE or RRC_CONNECTED) fulfils the conditions to support NR sidelink communication/discovery in limited service state as specified in TS 23.287 [55]; and if either the serving cell is on the frequency used for NR sidelink communication/discovery operation or the UE is out of coverage on the frequency used for NR sidelink communication/discovery operation as defined in TS 38.304 [20] and TS 36.304 [27]; or

1>	if the UE has no serving cell (RRC_IDLE).

### 5.8.3	Sidelink UE information for NR sidelink communication/discovery/positioning

#### 5.8.3.1	General

Figure 5.8.3.1-1: Sidelink UE information for NR sidelink communication/discovery/positioning

The purpose of this procedure is to inform the network that the UE:

-	is interested or no longer interested to receive or transmit NR sidelink communication/discovery/positioning,

-	is requesting assignment or release of transmission resource for NR sidelink communication/discovery/positioning,

-	is reporting QoS parameters and QoS profile(s) related to NR sidelink communication,

-	is reporting mapped frequency(ies) for each QoS flow related to NR sidelink communication,

-	is reporting associated Tx Profile for each QoS flow related to NR sidelink groupcast and broadcast communication,

-	is reporting that a sidelink radio link failure, sidelink RRC reconfiguration failure or a sidelink carrier failure has been detected,

-	is reporting the sidelink UE capability information of the associated peer UE for unicast communication,

-	is reporting the RLC mode information of the sidelink data radio bearer(s) received from the associated peer UE for unicast communication,

-	is reporting the accepted sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception,

-	is reporting the sidelink DRX assistance information received from the associated peer UE for NR sidelink unicast transmission, when the UE is configured with sl-ScheduledConfig,

-	is reporting, for NR sidelink groupcast transmission, the sidelink DRX on/off indication for the associated Destination Layer-2 ID, when the UE is configured with sl-ScheduledConfig,

-	is reporting, for NR sidelink groupcast or broadcast reception, the Destination Layer-2 ID and QoS profile(s) associated with its interested services to which sidelink DRX is applied,

-	is reporting DRX configuration reject information from its associated peer UE for NR sidelink unicast transmission, when the UE is configured with sl-ScheduledConfig,

-	is reporting parameters related to single hop or multi hop U2N relay operation,

-	is reporting parameters related to U2U relay operation.

#### 5.8.3.2	Initiation

A UE capable of NR sidelink communication or NR sidelink discovery or NR sidelink U2N relay operation including multi hop relay operation or NR sidelink U2U relay operation or NR sidelink positioning that is in RRC_CONNECTED may initiate the procedure to indicate it is (interested in) receiving or transmitting NR sidelink communication or NR sidelink discovery or NR sidelink U2N relay operation including multi hop relay operation or NR sidelink U2U relay operation or SL-PRS transmission/reception in several cases including upon successful connection establishment or resuming, upon change of interest, upon changing QoS profile(s), upon receiving UECapabilityInformationSidelink from the associated peer UE, upon RLC mode information updated from the associated peer UE or upon change to a PCell providing SIB12 including sl-ConfigCommonNR, or upon change to a PCell providing SIB23 including sl-PosConfigCommonNR. A UE capable of NR sidelink communication may initiate the procedure to request assignment of dedicated sidelink DRB configuration and transmission resources for NR sidelink communication transmission. A UE capable of NR sidelink communication may initiate the procedure to report to the network that a sidelink radio link failure, sidelink RRC reconfiguration failure or sidelink carrier failure has been declared. A UE capable of NR sidelink discovery may initiate the procedure to request assignment of dedicated resources for NR sidelink discovery transmission or NR sidelink discovery reception. A UE capable of U2N relay operation including multi hop relay operation may initiate the procedure to report/update parameters for acting as U2N Relay UE or U2N Remote UE (including L2 U2N Remote UE's source L2 ID). A UE capable of U2U relay operation may initiate the procedure to report/update parameters for acting as U2U Relay UE or U2U Remote UE. A UE capable of NR sidelink positioning may initiate the procedure to request it is interested or no longer interested in either transmitting SL-PRS or receiving SL-PRS.

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the sidelink DRX configuration received from the associated peer UE for NR sidelink unicast reception, upon accepting the sidelink DRX configuration from the associated peer UE. A UE capable of NR sidelink communication that is configured with sl-ScheduledConfig and is performing sidelink unicast transmission may initiate the procedure to report the sidelink DRX assistance information or the sidelink DRX configuration reject information received from the associated peer UE, upon receiving either of them from the associated peer UE. A UE capable of NR sidelink communication that is configured with sl-ScheduledConfig and is performing sidelink groupcast transmission may initiate the procedure to report the sidelink DRX on/off indication for the associated Destination Layer-2 ID. A UE capable of NR sidelink communication that is in RRC_CONNECTED may initiate the procedure to report the frequency(ies) and Tx Profile associated with each QoS flow for NR sidelink groupcast or broadcast transmission. A UE capable of NR sidelink communication that is in RRC_CONNECTED may initiate the procedure to report the frequency(ies) associated with each QoS flow for NR sidelink unicast transmission.

A UE capable of NR sidelink operation that is in RRC_CONNECTED may initiate the procedure to report the Destination Layer-2 ID and QoS profile(s) associated with its interested service(s) that sidelink DRX is applied, for NR sidelink groupcast or broadcast reception.

Upon initiating this procedure, the UE shall:

1>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

2>	ensure having a valid version of SIB12 for the PCell;

2>	if configured by upper layers to receive NR sidelink communication on the frequency included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqList; or if the frequency configured by upper layers to receive NR sidelink communication on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink communication reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR sidelink communication reception in accordance with 5.8.3.3;

2>	if configured by upper layers to transmit non-relay NR sidelink communication on the frequency included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12 of the PCell; or

2>	if configured by upper layer to transmit NR sidelink L3 U2U relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3-U2U-RelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqList; or if the information carried by the sl-TxResourceReqList has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink communication transmission resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink communication transmission resources in accordance with 5.8.3.3;

2>	if configured by upper layer to receive NR sidelink non-relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-NonRelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-NonRelayDiscovery; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqListDisc; or if the frequency configured by upper layers to receive NR sidelink non-relay discovery messages on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink non-relay discovery reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR sidelink non-relay discovery messages reception in accordance with 5.8.3.3;

2>	if configured by upper layer to receive NR sidelink L2 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to receive NR sidelink L3 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery; or

2>	if configured by upper layer to receive NR sidelink L2 U2U relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2-U2U-Relay; or

2>	if configured by upper layer to receive NR sidelink L3 U2U relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3-U2U-RelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case of L2 U2N relay operation; or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell providing SIB12 but not including sl-L2-U2U-Relay in case of L2 U2U relay operation; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell providing SIB12 but not including sl-L3-U2U-RelayDiscovery in case of L3 U2U relay operation; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqListDisc; or if the frequency configured by upper layers to receive NR sidelink relay discovery messages on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	if the UE is capable of U2N Relay UE or Last U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon; or

4>	if the UE is capable of Intermediate U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommonMH; or

4>	if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon; or

4>	if the UE is capable of U2U Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommonU2U; or

4>	if the UE is selecting a U2U Relay UE / has a selected U2U Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommonU2U:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay discovery reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR sidelink relay discovery messages reception in accordance with 5.8.3.3;

2>	if configured by upper layer to transmit NR sidelink non-relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-NonRelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR or connected to a PCell providing SIB12 but not including sl-NonRelayDiscovery; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqListDisc; or if the information carried by the sl-TxResourceReqListDisc has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink non-relay discovery messages resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink non-relay discovery messages resources in accordance with 5.8.3.3;

2>	if configured by upper layer to transmit NR sidelink L2 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to transmit NR sidelink L3 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery; or

2>	if configured by upper layer to transmit NR sidelink L2 U2U relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2-U2U-Relay; or

2>	if configured by upper layer to transmit NR sidelink L3 U2U relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3-U2U-RelayDiscovery:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case of L2 U2N relay operation; or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell providing SIB12 but not including sl-L2-U2U-Relay in case of L2 U2U relay operation;

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell providing SIB12 but not including sl-L3-U2U-RelayDiscovery in case of L3 U2U relay operation; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqListDisc; or if the information carried by the sl-TxResourceReqListDisc has changed since the last transmission of the SidelinkUEInformationNR message:

4>	if the UE is capable of U2N Relay UE or of Last U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE or if the Last U2N Relay UE threshold conditions as specified in 5.8.14.2 are met; or

4>	if the UE is capable of Intermediate U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommonMH; or

4>	if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE/ configured with measurement object associated to L2 U2N Relay UEs, and if SIB12 includes sl-RemoteUE-ConfigCommon, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met; or

4>	if the UE is capable of U2U Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommonU2U, and if the U2U Relay UE threshold conditions as specified in 5.8.16.2 are met; or

4>	if the UE is selecting a U2U Relay UE / has a selected U2U Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommonU2U, and if the U2U Remote UE threshold conditions as specified in 5.8.17.2 are met:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay discovery messages resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqListDisc:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink relay discovery messages resources in accordance with 5.8.3.3;

2>	if configured by upper layer to transmit NR sidelink L2 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to transmit NR sidelink L3 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery; or

2>	if configured by upper layer to transmit NR sidelink L2 U2U relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2-U2U-Relay:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case of L2 U2N relay operation; or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell providing SIB12 but not including sl-L2-U2U-Relay in case of L2 U2U relay operation; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL2U2N-Relay; or if the information carried by the sl-TxResourceReqL2U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL3U2N-Relay; or if the information carried by the sl-TxResourceReqL3U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL2-U2U; or if the information carried by the sl-TxResourceReqL2-U2U has changed since the last transmission of the SidelinkUEInformationNR message; or

3>	if configured by upper layers not to transmit either NR sidelink L2 U2N relay communication or NR sidelink L3 U2N relay communication, and if the last transmission of the SidelinkUEInformationNR message includes both sl-TxResourceReqL2U2N-Relay and sl-TxResourceReqL3U2N-Relay:

4>	if the UE is capable of U2N Relay UE; or

4>	if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE; or

4>	if the UE is capable of L2 U2U Relay UE; or

4>	if the UE is selecting a L2 U2U Relay UE / has a selected L2 U2U Relay UE:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay communication transmission resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqL2U2N-Relay or sl-TxResourceReqL3U2N-Relay or sl-TxResourceReqL2-U2U:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink relay communication transmission resources in accordance with 5.8.3.3;

2>	if configured by upper layers to perform NR sidelink reception on the frequency included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12 of the PCell and if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

3>	if the UE received a sidelink DRX configuration in the RRCReconfigurationSidelink message for NR sidelink unicast reception from the associated peer UE and the UE accepted the sidelink DRX configuration:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxDRX-ReportList; or if the information carried by sl-RxDRX-ReportList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the sidelink DRX configuration in accordance with 5.8.3.3;

3>	else:

4>	if the last transmission of the SidelinkUEInformationNR message included sl-RxDRX-ReportList:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate the sidelink DRX configuration is no longer used in accordance with 5.8.3.3;

3>	if the UE is performing NR sidelink groupcast or broadcast reception and is interested in a service that sidelink DRX is applied:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedGC-BC-DestList; or if the information carried by sl-RxInterestedGC-BC-DestList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the Destination Layer-2 ID and QoS profile(s) associated with the service(s) in accordance with 5.8.3.3;

3>	else:

4>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedGC-BC-DestList:

5>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in the service that sidelink DRX is applied in accordance with 5.8.3.3;

2>	if configured by upper layers to perform NR sidelink transmission on the frequency included in sl-FreqInfoList in SIB12 of the PCell and if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs and if the UE is configured with sl-ScheduledConfig:

3>	if the UE received a sidelink DRX assistance information or a sidelink DRX configuration reject information from the associated peer UE for NR sidelink unicast transmission:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-DRX-InfoFromRxList, or sl-FailureList; or if the information carried by sl-DRX-InfoFromRxList, or sl-FailureList has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report the sidelink DRX assistance information or the sidelink DRX configuration reject information in accordance with 5.8.3.3;

NOTE:	After including the SL-DRX reject information in sl-FailureList in the last transmission of the SidelinkUEInformationNR message, it is up to UE implementation to consider another sidelink DRX rejection of a new SL DRX configuration from the same associated peer UE as "change" of sl-FailureList.

3>	if the UE is performing NR sidelink groupcast transmission:

4>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

4>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-DRX-ConfigCommonGC-BC; or

4>	if the last transmission of the SidelinkUEInformationNR message did not include sl-DRX-Indication; or if the information carried by sl-DRX-Indication has changed since the last transmission of the SidelinkUEInformationNR message:

5>	initiate transmission of the SidelinkUEInformationNR message to report sidelink DRX on/off indication for the corresponding destination in accordance with 5.8.3.3;

1>	if SIB23 including sl-PosConfigCommonNR is provided by the PCell:

2>	ensure having a valid version of SIB23 for the PCell;

2>	if configured to perform SL-PRS measurement on the frequency included in sl-PosFreqInfoList in SIB23 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB23 including sl-PosConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-PosRxInterestedFreqList; or if the frequency configured to receive SL-PRS has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the frequency of interest for SL-PRS reception in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-PosRxInterestedFreqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in SL-PRS reception in accordance with 5.8.3.3;

2>	if configured to transmit SL-PRS on the frequency included in sl-PosFreqInfoList in SIB23 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB23 including sl-PosConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-PosTxResourceReqList; or if the information carried by the sl-PosTxResourceReqList has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink positioning transmission resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-PosTxResourceReqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink positioning transmission resources in accordance with 5.8.3.3;

#### 5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

The UE shall set the contents of the SidelinkUEInformationNR message as follows and shall include all concerned information, irrespective of what triggered the procedure:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink communication/positioning; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink communication/positioning transmission resources or to report to the network that a sidelink radio link failure, sidelink RRC reconfiguration failure or sidelink carrier failure has been declared; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX configuration for NR sidelink unicast reception; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX assistance information or the sidelink DRX configuration reject information for NR sidelink unicast transmission; or

1>	if the UE initiates the procedure to report to the network the Destination Layer-2 ID and QoS profile(s) associated with its interested service(s) that sidelink DRX is applied for NR sidelink groupcast or broadcast reception; or

1>	if the UE initiates the procedure to report to the network the Destination Layer-2 ID and the sidelink DRX on/off indication for the corresponding destination for NR sidelink groupcast transmission; or

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink discovery messages; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink discovery messages transmission resources; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink U2N or U2U relay communication transmission resources or report other parameters related to U2N or U2U relay operation:

2>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

3>	if configured by upper layers to receive NR sidelink communication:

4>	include sl-RxInterestedFreqList and set it to the frequency for NR sidelink communication reception;

3>	if configured by upper layers to transmit non-relay NR sidelink communication and/or to transmit NR sidelink relay communication; or

3>	if configured by upper layers to transmit NR sidelink L3 U2U relay communication and SIB12 includes sl-L3-U2U-RelayDiscovery:

4>	include sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-RLC-ModeIndicationList to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB(s) have been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the associated peer UE;

5>	if sl-FreqInfoListSizeExt is included in SIB12-IEs:

6>	set sl-QoS-InfoList to include the frequency(ies), and Tx Profile, if any, mapped to the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink groupcast and broadcast communication transmission;

4>	if a sidelink radio link failure or a sidelink RRC reconfiguration failure has been declared, according to clauses 5.8.9.3 and 5.8.9.1.8, respectively;

5>	include sl-FailureList and set its fields as follows for each destination for which it reports the NR sidelink communication failure:

6>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

6>	if the sidelink RLF is detected as specified in clause 5.8.9.3:

7>	set sl-Failure as rlf for the associated destination for the NR sidelink communication transmission;

6>	else if RRCReconfigurationFailureSidelink is received:

7>	set sl-Failure as configFailure for the associated destination for the NR sidelink communication transmission;

4>	if a sidelink carrier failure has been indicated by MAC layer;

5>	include sl-CarrierFailureList and set its fields as follows for each destination for which it reports the sidelink carrier failure:

6>	set sl-DestinationIdentity to the destination identity for which the concerned sidelink carrier failure is indicated;

6>	set sl-CarrierFailure to include the concerned carrier for which the sidelink carrier failure is indicated;

3>	if SIB12 includes sl-NonRelayDiscovery and if configured by upper layers to receive NR sidelink non-relay discovery messages, or if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to receive NR sidelink L2 U2N relay discovery messages, or if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to receive NR sidelink L3 U2N relay discovery messages; or

3>	if SIB12 includes sl-L2-U2U-Relay and if configured by upper layers to receive NR sidelink L2 U2U relay discovery messages; or

3>	if SIB12 includes sl-L3-U2U-RelayDiscovery and if configured by upper layers to receive NR sidelink L3 U2U relay discovery messages:

4>	include sl-RxInterestedFreqListDisc and set it to the frequency for NR sidelink discovery messages reception;

3>	if SIB12 includes sl-L2U2N-Relay and the UE is capable of L2 U2N remote UE:

4>	include sl-SourceIdentityRemoteUE and set it to the source identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

3>	if SIB12 includes sl-NonRelayDiscovery and if configured by upper layers to transmit NR sidelink non-relay discovery messages, or if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages, or if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages; or

3>	if SIB12 includes sl-L2-U2U-Relay and if configured by upper layers to transmit NR sidelink L2 U2U relay discovery messages; or

3>	if SIB12 includes sl-L3-U2U-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2U relay discovery messages:

4>	include sl-TxResourceReqListDisc and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink discovery messages resource:

5>	set sl-DestinationIdentityDisc to the destination identity configured by upper layer for NR sidelink discovery messages transmission;

5>	if the UE is acting as L2 U2N Relay UE:

6>	set sl-SourceIdentityRelayUE to the source identity configured by upper layer for NR sidelink L2 U2N relay discovery messages transmission;

5>	set sl-CastTypeDisc to the cast type of the associated destination identity for the NR sidelink discovery messages transmission;

5>	set sl-TxInterestedFreqListDisc to indicate the frequency of the associated destination for NR sidelink discovery messages transmission;

5>	set sl-TypeTxSyncListDisc to the current synchronization reference type used on the associated sl-TxInterestedFreqListDisc for NR sidelink discovery messages transmission;

5>	set sl-DiscoveryType to the current discovery type of the associated destination identity configured by the upper layer for NR sidelink discovery messages transmission;

5>	if the UE is acting as L2/L3 U2U Relay UE:

6>	include ue-TypeU2U and set it to relayUE;

5>	if the UE is acting as L2/L3 U2U Remote UE:

6>	include ue-TypeU2U and set it to remoteUE;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE transiting to RRC_CONNECTED or in RRC_CONNECTED state;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE has a selected L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to remoteUE;

3>	if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay communication:

4>	include sl-TxResourceReqL3U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L3 U2N relay communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-RLC-ModeIndicationList to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB(s) have been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink L3 U2N relay communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE if the UE is acting as NR sidelink L3 U2N Relay UE or to remoteUE otherwise;

3>	if SIB12 includes sl-L2-U2U-Relay and if configured by upper layers to transmit NR sidelink L2 U2U relay communication and the UE is acting as L2 U2U Relay UE:

4>	include sl-TxResourceReqL2-U2U and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2U relay communication resource:

5>	set sl-DestinationIdentityL2-U2U to the destination identity configured by upper layer for NR sidelink L2 U2U relay communication transmission to the target L2 U2U Remote UE;

5>	set sl-TxInterestedFreqListL2-U2U to indicate the frequency of the associated destination for NR sidelink L2 U2U relay communication transmission;

5>	set sl-TypeTxSyncListL2-U2U to the current synchronization reference type used on the associated sl-InterestedFreqListL2-U2U for NR sidelink L2 U2U relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the target L2 U2U Remote UE;

5>	include sl-U2U-InfoList and set its fields (if needed) for each entry as follows, to report the related information of the connected L2 U2U Remote UEs:

6>	include the source L2 U2U Remote UE's L2 ID in sl-SourceUE-Identity;

6>	include sl-PerSLRB-QoS-InfoList, with each entry including the per-SLRB second-hop QoS profile and the corresponding sl-RemoteUE-SLRB-Identity which is set to the same value as the sl-E2E-SLRB-Index received in UEInformationRequestSidelink message from the L2 U2U Remote UE for the same end-to-end SLRB;

5>	set sl-RLC-ModeIndicationListL2-U2U to include the RLC mode(s), if the associated bi-directional PC5 Relay RLC channel(s) has been established due to the configuration by RRCReconfigurationSidelink;

3>	if SIB12 includes sl-L2-U2U-Relay and if configured by upper layers to transmit NR sidelink L2 U2U relay communication and the UE has a selected L2 U2U Relay UE:

4>	include sl-TxResourceReqL2-U2U and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2U relay communication resource:

5>	set sl-DestinationIdentityL2-U2U to the destination identity configured by upper layer for NR sidelink L2 U2U relay communication transmission to L2 U2U Relay UE;

5>	set sl-TxInterestedFreqListL2-U2U to indicate the frequency of the associated destination for NR sidelink L2 U2U relay communication transmission;

5>	set sl-TypeTxSyncListL2-U2U to the current synchronization reference type used on the associated sl-InterestedFreqListL2-U2U for NR sidelink L2 U2U relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message received from L2 U2U Relay UE, if any;

5>	include sl-U2U-InfoList and set its fields (if needed) for each entry as follows to report the related end-to-end and the first hop information for the end-to-end PC5 connection with each target L2 U2U Remote UE:

6>	set sl-TargetUE-Identity to the destination identity configured by upper layer for NR sidelink L2 U2U relay communication transmission to the target L2 U2U Remote UE;

6>	set sl-E2E-QoS-InfoList to include end-to-end QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L2 U2U relay communication transmission to the target L2 U2U Remote UE;

6>	set sl-PerHop-QoS-InfoList to include the first-hop split PDB of the sidelink QoS flow(s) received from the sl-SplitQoS-InfoListPC5 in UEInformationResponseSidelink message for the associated destination in accordance with the received sl-TargetUE-Identity;

6>	set sl-CapabilityInformationTargetRemoteUE to include the related UE capability information received from the target L2 U2U Remote UE, if any;

3>	if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

4>	if configured by upper layers to perform NR sidelink reception:

5>	include sl-RxDRX-ReportList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-ConfigFromTx to include the accepted sidelink DRX configuration of the associated destination for NR sidelink unicast communication, if received from the associated peer UE;

5>	include sl-RxInterestedGC-BC-DestList and set its fields (if needed) as follows for each Destination Layer-2 ID for which it reports to network:

6>	set sl-RxInterestedQoS-InfoList to include the QoS profile of its interested service(s) that sidelink DRX is applied for the associated destination for NR sidelink groupcast or broadcast reception;

NOTE 1:	It is up to UE implementation to set the QoS profile in sl-RxInterestedQoS-InfoList for reception of NR sidelink discovery message or ProSe Direct Link Establishment Request message as described in TS 24.554 [72], or for reception of Direct Link Establishment Request message as described in TS 24.587 [57].

6>	set sl-DestinationIdentity to the associated destination identity configured by upper layer for NR sidelink groupcast or broadcast reception;

4>	if configured by upper layers to perform NR sidelink transmission and configured with sl-ScheduledConfig:

5>	include sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay and/or sl-FailureList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-InfoFromRxList to include the sidelink DRX assistance information of the associated destination, if any, received from the associated peer UE;

6>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

7>	set sl-Failure as drxReject-v1710 for the associated destination for the NR sidelink communication transmission;

6>	set sl-DRX-Indication to include the sidelink DRX on/off indication for the associated destination for NR sidelink groupcast transmission;

3>	if SIB12 includes sl-PRS-ResourcesSharedSL-PRS-RP:

4>	if configured to perform SL-PRS measurements:

5>	include sl-PosRxInterestedFreqList2 and set it to the frequency configured with sl-PRS-ResourcesSharedSL-PRS-RP for SL-PRS reception;

4>	if configured to transmit SL-PRS:

5>	include sl-PosTxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign SL-PRS resource:

6>	set sl-PosDestinationIdentity to the destination identity configured by upper layer for SL-PRS transmission;

6>	set sl-PosCastType to the cast type of the associated destination identity configured by the upper layer for SL-PRS transmission;

6>	set sl-PosTxInterestedFreqList2 to indicate the frequency configured with sl-PRS-ResourcesSharedSL-PRS-RP of the associated destination for SL-PRS transmission;

6>	set sl-PosTypeTxSyncList to the current synchronization reference type used on the associated sl-PosRxInterestedFreqList for SL-PRS transmission;

6>	set sl-PosQoS-InfoList to include the SL-PRS transmission QoS profile;

5>	include sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign SL-PRS resource:

6>	set sl-PosDestinationIdentity to the destination identity configured by upper layer for NR sidelink positioning transmission;

6>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the associated peer UE;

2>	if SIB23 including sl-PosConfigCommonNR is provided by the PCell;

3>	if configured to transmit SL-PRS:

4>	include sl-PosTxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign SL-PRS resource:

5>	set sl-PosDestinationIdentity to the destination identity configured by upper layer for SL-PRS transmission;

5>	set sl-PosCastType to the cast type of the associated destination identity configured by the upper layer for the SL-PRS transmission;

5>	set sl-PosTxInterestedFreqList to indicate the frequency of the associated destination for SL-PRS transmission;

5>	set sl-PosTypeTxSyncList to the current synchronization reference type used on the associated sl-PosRxInterestedFreqList for SL-PRS transmission;

5>	set sl-PosQoS-InfoList to include the SL-PRS transmission QoS profile;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the associated peer UE;

3>	if configured to perform SL-PRS measurements;

4>	include sl-PosRxInterestedFreqList and set it to the frequency for SL-PRS reception;

1>	if the UE initiates the procedure while connected to an E-UTRA PCell:

2>	submit the SidelinkUEInformationNR to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else:

2>	submit the SidelinkUEInformationNR message to lower layers for transmission.

NOTE 2:	When multiple lists are reported in SidelinkUEInformationNR, a UE can report up to maxNrofSL-Dest-r16 SL destinations in sl-TxResourceReqList, sl-TxResourceReqListDisc and sl-TxResourceReqListCommRelay in total.

### 5.8.4	Void

### 5.8.5	Sidelink synchronisation information transmission for NR sidelink communication/discovery/positioning

#### 5.8.5.1	General

Figure 5.8.5.1-1: Synchronisation information transmission for NR sidelink communication/discovery/positioning, in (partial) coverage

Figure 5.8.5.1-2: Synchronisation information transmission for NR sidelink communication/discovery/positioning, out of coverage

The purpose of this procedure is to provide synchronisation information to a UE. This procedure also applies to NR sidelink discovery.

#### 5.8.5.2	Initiation

A UE capable of NR sidelink communication/discovery/positioning and SLSS/PSBCH transmission shall, when transmitting NR sidelink communication/discovery/positioning, and if the conditions for NR sidelink communication/discovery/positioning operation are met and when the following conditions are met:

1>	if in coverage on the frequency used for NR sidelink communication/discovery/positioning, as defined in TS 38.304 [20]; and has selected GNSS or the cell as synchronization reference as defined in 5.8.6.3; or

1>	if out of coverage on the frequency used for NR sidelink communication/discovery/positioning, and the frequency used to transmit NR sidelink communication/discovery is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12; and has selected GNSS or the cell as synchronization reference as defined in 5.8.6.3:

2>	if sl-SyncFreqList is neither included in RRCReconfiguration nor in SIB12; or

2>	if sl-SyncFreqList is included in RRCReconfiguration or in SIB12; and if none of the frequency(ies) selected as specified in TS 38.321 [3] is included in the sl-SyncFreqList or the concerned frequency is selected as the synchronisation carrier frequency in accordance with 5.8.6.2; or

2>	if sl-SyncFreqList and sl-SyncTxMultiFreq are included in RRCReconfiguration or in SIB12; and if the concerned frequency has been selected for NR sidelink communication transmission as specified in TS 38.321 [3] and is included in sl-SyncFreqList; and if the UE has selected a frequency other than the concerned frequency as the synchronisation carrier frequency in accordance with 5.8.6.2; and if sl-SyncTxDisabled corresponding to the concerned frequency is not configured in RRCReconfiguration or in SIB12:

3>	if in RRC_CONNECTED; and if networkControlledSyncTx is configured and set to on; or

3>	if networkControlledSyncTx is not configured; and for the concerned frequency syncTxThreshIC is configured; and the RSRP measurement of the reference cell, selected as defined in 5.8.6.3, for NR sidelink communication/discovery transmission is below the value of syncTxThreshIC:

4>	transmit sidelink SSB on the frequency used for NR sidelink communication/discovery in accordance with 5.8.5.3 and TS 38.211 [16], including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

1>	else:

2>	if sl-SyncFreqList is not included in SL-PreconfigurationNR; or

2>	if sl-SyncFreqList is included in SL-PreconfigurationNR, and if none of the frequency(ies) selected as specified in TS 38.321 [3] is included in the sl-SyncFreqList or the concerned frequency is selected as the synchronisation carrier frequency in accordance with 5.8.6.2; or

2>	if sl-SyncFreqList and sl-SyncTxMultiFreq are included in SL-PreconfigurationNR, and if the concerned frequency has been selected for NR sidelink communication transmission as specified in TS 38.321 [3] and included in sl-SyncFreqList; and if the UE has selected a frequency other than the concerned frequency as the synchronisation carrier frequency in accordance with 5.8.6.2; and if sl-SyncTxDisabled corresponding to the concerned frequency is not configured in SL-PreconfigurationNR:

3>	for the frequency used for NR sidelink communication/discovery, if syncTxThreshOoC is included in SidelinkPreconfigNR; and the UE is not directly synchronized to GNSS, and the UE has no selected SyncRef UE or the PSBCH-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC; or

3>	for the frequency used for NR sidelink communication/discovery, if the UE selects GNSS as the synchronization reference source:

4>	transmit sidelink SSB on the frequency used for NR sidelink communication/discovery in accordance with 5.8.5.3 and TS 38.211 [16], including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

#### 5.8.5.3	Transmission of SLSS

The UE shall select the SLSSID and the slot in which to transmit SLSS as follows:

1>	if triggered by NR sidelink communication/discovery/positioning and in coverage on the frequency used for NR sidelink communication/discovery/positioning, as defined in TS 38.304 [20]; or

1>	if triggered by NR sidelink communication/discovery/positioning, and out of coverage on the frequency used for NR sidelink communication/discovery/positioning, and the concerned frequency is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-FreqInfoList/sl-FreqInfoToAddModListExt within SIB12 or SIB23 for NR sidelink positioning:

2>	if the UE has selected GNSS as synchronization reference in accordance with 5.8.6.2:

3>	select SLSSID 0;

3>	use sl-SSB-TimeAllocation1 included in the entry of configured sl-SyncConfigList corresponding to the concerned frequency, that includes txParameters and gnss-Sync;

3>	select the slot(s) indicated by sl-SSB-TimeAllocation1;

2>	if the UE has selected a cell as synchronization reference in accordance with 5.8.6.2:

3>	select the SLSSID included in the entry of configured sl-SyncConfigList corresponding to the concerned frequency, that includes txParameters and does not include gnss-Sync;

3>	select the slot(s) indicated by sl-SSB-TimeAllocation1;

1>	else if triggered by NR sidelink communication/discovery and the UE has GNSS as the synchronization reference:

2>	select SLSSID 0;

2>	if sl-SSB-TimeAllocation3 is configured for the frequency used in SidelinkPreconfigNR:

3>	select the slot(s) indicated by sl-SSB-TimeAllocation3;

2>	else:

3>	select the slot(s) indicated by sl-SSB-TimeAllocation1;

1>	else:

2>	select the synchronisation reference UE (i.e. SyncRef UE) as defined in 5.8.6;

2>	if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlockSidelink message received from this UE is set to true; or

2>	if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlockSidelink message received from this UE is set to false while the SLSS from this UE is part of the set defined for out of coverage, see TS 38.211 [16]:

3>	select the same SLSSID as the SLSSID of the selected SyncRef UE;

3>	select the slot in which to transmit the SLSS according to the sl-SSB-TimeAllocation1 or sl-SSB-TimeAllocation2 included in the preconfigured sidelink parameters corresponding to the concerned frequency, such that the timing is different from the SLSS of the selected SyncRef UE;

2>	else if the UE has a selected SyncRef UE and the SLSS from this UE was transmitted on the slot(s) indicated sl-SSB-TimeAllocation3, which is configured for the frequency used in SidelinkPreconfigNR:

3>	select SLSSID 337;

3>	select the slot(s) indicated by sl-SSB-TimeAllocation2;

2>	else if the UE has a selected SyncRef UE:

3>	select the SLSSID from the set defined for out of coverage having an index that is 336 more than the index of the SLSSID of the selected SyncRef UE, see TS 38.211 [16];

3>	select the slot in which to transmit the SLSS according to sl-SSB-TimeAllocation1 or sl-SSB-TimeAllocation2 included in the preconfigured sidelink parameters corresponding to the concerned frequency, such that the timing is different from the SLSS of the selected SyncRef UE;

2>	else (i.e. no SyncRef UE selected):

3>	if the UE has not randomly selected an SLSSID:

4>	randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage except SLSSID 336 and 337, see TS 38.211 [16];

4>	select the slot in which to transmit the SLSS according to the sl-SSB-TimeAllocation1 or sl-SSB-TimeAllocation2 (arbitrary selection between these) included in the preconfigured sidelink parameters in SidelinkPreconfigNR corresponding to the concerned frequency;

### 5.8.5a	Sidelink synchronisation information transmission for V2X sidelink communication

#### 5.8.5a.1	General

Figure 5.8.5a.1-1: Synchronisation information transmission for V2X sidelink communication, in (partial) coverage

Figure 5.8.5a.1-2: Synchronisation information transmission for V2X sidelink communication, out of coverage

The purpose of this procedure is to provide synchronisation information to a UE.

#### 5.8.5a.2	Initiation

A UE capable of V2X sidelink communication initiates the transmission of SLSS and MasterInformationBlock-SL-V2X according to the conditions and the procedures specified for V2X sidelink communication in clause 5.10.7 of TS 36.331 [10].

NOTE 1: When applying the procedure in this clause, SIB13 and SIB14 correspond to SystemInformationBlockType21 and SystemInformationBlockType26 specified in TS 36.331 [10] respectively.

### 5.8.6	Sidelink synchronisation reference

#### 5.8.6.1	General

The purpose of this procedure is to select a synchronisation reference and used when transmitting NR sidelink communication/discovery. This procedure also applies to NR sidelink discovery.

#### 5.8.6.2	Selection and reselection of synchronisation reference

The UE shall for frequency(ies) which have been selected for NR sidelink communication/discovery as specified in TS 38.321 [3]:

1>	if sl-SyncFreqList is not included in RRCReconfiguration nor in SIB12; or

1>	if sl-SyncFreqList is included in RRCReconfiguration or in SIB12, and none of the frequency(ies) selected as specified in TS 38.321 [3] is included in the syncFreqList:

2>	if the frequency used for NR sidelink communication/discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority is configured for the concerned frequency and set to gnbEnb:

3>	select a cell as the synchronization reference source as defined in 5.8.6.3:

NOTE 1:	When an out of coverage L2 U2N Remote UE receives SIB12 with sl-SyncPriority set to gnbEnb, the L2 U2N Remote UE continues using the current synchronization source until higher priority synchronization source is found or the current synchronization source becomes unreliable.

2>	else if the frequency used for NR sidelink communication/discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority for the concerned frequency is not configured or is set to gnss, and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]:

3>	select GNSS as the synchronization reference source;

2>	else if the frequency used for NR sidelink communication/discovery is included in SL-PreconfigurationNR, and sl-SyncPriority in SidelinkPreconfigNR is set to gnss and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]:

3>	select GNSS as the synchronization reference source;

2>	else:

3>	perform synchronization reference search procedure as defined in 5.8.6.2b on each frequency which is used for NR sidelink communication/discovery;

3>	if the UE has not selected any synchronization reference:

4>	for each frequency which is used for NR sidelink communication/discovery:

5>	if the UE detects one or more SLSSIDs for which the PSBCH-RSRP exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and for which the UE received the corresponding MasterInformationBlockSidelink message (candidate SyncRef UEs), or if the UE detects GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14], or if the UE detects a cell, select a synchronization reference according to the priority group order as defined in 5.8.6.2a:

1>	if sl-SyncFreqList is included in RRCReconfiguration or in SIB12, and includes at least one of the concerned frequency(ies)

2>	if the concerned frequency(ies) are included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority is configured for the concerned frequency(ies) and set to gnbEnb:

3>	select one frequency from the concerned frequency(ies) which are included in sl-SyncFreqList as the synchronisation carrier frequency;

3>	select a cell in accordance with the synchronisation carrier frequency as the synchronization reference source as defined in 5.8.6.3:

2>	else if the concerned frequency(ies) are included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority for concerned frequency(ies) are not configured or are set to gnss, and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]; or if the concerned frequency(ies) are included in SL-PreconfigurationNR, and sl-SyncPriority in SidelinkPreconfigNR is set to gnss and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]:

3>	select one frequency from the concerned frequency(ies) which are included in sl-SyncFreqList as the synchronisation carrier frequency;

3>	select GNSS in accordance with the synchronisation carrier frequency as the synchronization reference source;

2>	else:

3>	perform a synchronization reference search procedure as defined in 5.8.6.2b on each concerned frequency which is included in sl-SyncFreqList;

3>	if the UE has not selected any synchronization reference:

4>	for each concerned frequency which is included in sl-SyncFreqList:

5>	if the UE detects one or more SLSSIDs for which the PSBCH-RSRP exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and for which the UE received the corresponding MasterInformationBlockSidelink message (candidate SyncRef UEs), or if the UE detects GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14], or if the UE detects a cell,

6>	select the synchronisation reference source(s) according to the following priority group order as defined in 5.8.6.2a;

6>	select the frequency with the highest synchronisation reference source priority as the synchronisation carrier frequency, according to the following priority group order as defined in 5.8.6.2a, and consider the synchornization reference source (i.e. eNB/gNB, GNSS or SyncRef UE) that selected on the synchronisation carrier frequency as the synchronization reference:

NOTE 2:	How the UE achieves subframe boundary alignment between V2X sidelink communication and NR sidelink communication/discovery (if both are performed by the UE) is as specified in TS 38.213, clause 16.7.

#### 5.8.6.2a	Sidelink synchronization reference priority group order

1>	if sl-SyncPriority corresponding to the concerned frequency is set to gnbEnb:

2>	UEs of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, starting with the UE with the highest PSBCH-RSRP result (priority group 1);

2>	UE of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 2);

2>	GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14] (priority group 3);

2>	UEs of which SLSSID is 0, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, or of which SLSSID is 0 and SLSS is transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, starting with the UE with the highest PSBCH-RSRP result (priority group 4);

2>	UEs of which SLSSID is 0 and SLSS is not transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 5);

2>	UEs of which SLSSID is 337 and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 5);

2>	Other UEs, starting with the UE with the highest PSBCH-RSRP result (priority group 6);

1>	if sl-SyncPriority corresponding to the concerned frequency is set to gnss, and sl-NbAsSync is set to true:

2>	UEs of which SLSSID is 0, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, or of which SLSSID is 0 and SLSS is transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, starting with the UE with the highest PSBCH-RSRP result (priority group 1);

2>	UEs of which SLSSID is 0 and SLSS is not transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCHS-RSRP result (priority group 2);

2>	UEs of which SLSSID is 337 and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 2);

2>	the cell detected by the UE as defined in 5.8.6.3 (priority group 3);

2>	UEs of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, starting with the UE with the highest PSBCH-RSRP result (priority group 4);

2>	UE of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 5);

2>	Other UEs, starting with the UE with the highest S-RSRP result (priority group 6);

1>	if sl-SyncPriority corresponding to the concerned frequency is set to gnss, and sl-NbAsSync is set to false:

2>	UEs of which SLSSID is 0, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, or of which SLSSID is 0 and SLSS is transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, starting with the UE with the highest PSBCH-RSRP result (priority group 1);

2>	UEs of which SLSSID is 0 and SLSS is not transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCHS-RSRP result (priority group 2);

2>	UEs of which SLSSID is 337 and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 2);

2>	Other UEs, starting with the UE with the highest PSBCH-RSRP result (priority group 3);

#### 5.8.6.2b	Sidelink synchronization reference search

The UE shall for the indicated frequency(ies)

1>	perform a full search (i.e. covering all subframes and all possible SLSSIDs) on each indicated frequency to detect candidate SLSS, in accordance with TS 38.133 [14];

1>	when evaluating the one or more detected SLSSIDs, apply layer 3 filtering as specified in 5.5.3.2 using the preconfigured sl-filterCoefficient, before using the PSBCH-RSRP measurement results;

1>	if the UE has selected a SyncRef UE:

2>	if the PSBCH-RSRP of the strongest candidate SyncRef UE exceeds the minimum requirement TS 38.133 [14] by sl-SyncRefMinHyst and the strongest candidate SyncRef UE belongs to the same priority group as the current SyncRef UE and the PSBCH-RSRP of the strongest candidate SyncRef UE exceeds the PSBCH-RSRP of the current SyncRef UE by syncRefDiffHyst; or

2>	if the PSBCH-RSRP of the candidate SyncRef UE exceeds the minimum requirement TS 38.133 [14] by sl-SyncRefMinHyst and the candidate SyncRef UE belongs to a higher priority group than the current SyncRef UE; or

2>	if GNSS becomes reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14], and GNSS belongs to a higher priority group than the current SyncRef UE; or

2>	if a cell is detected and gNB/eNB (if sl-NbAsSync is set to true) belongs to a higher priority group than the current SyncRef UE; or

2>	if the PSBCH-RSRP of the current SyncRef UE is less than the minimum requirement defined in TS 38.133 [14]:

3>	consider no SyncRef UE to be selected;

1>	if the UE has selected GNSS as the synchronization reference for NR sidelink communication/discovery:

2>	if the PSBCH-RSRP of the candidate SyncRef UE exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and the candidate SyncRef UE belongs to a higher priority group than GNSS; or

2>	if GNSS becomes not reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]:

3>	consider GNSS not to be selected;

1>	if the UE has selected cell as the synchronization reference for NR sidelink communication/discovery:

2>	if the PSBCH-RSRP of the candidate SyncRef UE exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and the candidate SyncRef UE belongs to a higher priority group than gNB/eNB; or

2>	if the selected cell is not detected:

3>	consider the cell not to be selected;

#### 5.8.6.3	Sidelink communication transmission reference cell selection

A UE capable of NR sidelink communication/discovery that is configured by upper layers to transmit NR sidelink communication/discovery shall:

1>	for the frequency used to transmit NR sidelink communication/discovery, select a cell to be used as reference for synchronization in accordance with the following:

2>	if the frequency concerns the primary frequency:

3>	use the PCell or the serving cell as reference;

2>	else if the frequency concerns a secondary frequency:

3>	use the concerned SCell as reference;

2>	else if the UE is in coverage of the concerned frequency:

3>	use the DL frequency paired with the one used to transmit NR sidelink communication/discovery as reference;

2>	else (i.e., out of coverage on the concerned frequency):

3>	use the PCell or the serving cell as reference, if needed;

### 5.8.7	Sidelink communication reception

A UE capable of NR sidelink communication that is configured by upper layers to receive NR sidelink communication shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in RRCReconfiguration message or sl-FreqInfoList/sl-FreqInfoListSizeExt included in SIB12:

3>	if the UE is configured with sl-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

4>	configure lower layers to monitor sidelink control information and the corresponding data using the pool(s) of resources indicated by sl-RxPool;

3>	else if the cell chosen for NR sidelink communication provides SIB12:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the pool(s) of resources indicated by sl-RxPool in SIB12;

2>	else:

3>	configure lower layers to monitor sidelink control information and the corresponding data using the pool(s) of resources that were preconfigured by sl-RxPool in SL-PreconfigurationNR, as defined in clause 9.3.

### 5.8.8	Sidelink communication transmission

A UE capable of NR sidelink communication that is configured by upper layers to transmit NR sidelink communication and has related data to be transmitted shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

4>	if the UE acting as U2U Relay UE is performing U2U Relay Communication with integrated Discovery as specified in TS 23.304[65] and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2U Relay UE threshold conditions for integrated Discovery as specified in 5.8.16.2 are met based on sl-RelayUE-ConfigU2U; or

4>	if the UE capable of U2U Remote UE is performing U2U Relay Communication with integrated Discovery as specified in TS 23.304[65] and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2U Remote UE threshold conditions for integrated Discovery as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigU2U; or

4>	if the UE is performing NR sidelink communication other than U2U Relay Communication with integrated Discovery:

NOTE 0:	For U2U Relay UE, it can be up to UE implementation on cross-layer interaction for the AS layer condition check for Direct Communication Request message with integrated discovery forwarding.

5>	if the UE is configured with sl-ScheduledConfig:

6>	if T310 for MCG or T311 is running; and if sl-TxPoolExceptional is included in sl-FreqInfoList/sl-FreqInfoListSizeExt for the concerned frequency in SIB12 or included in sl-ConfigDedicatedNR in RRCReconfiguration; or

6>	if T301 is running and the cell on which the UE initiated RRC connection re-establishment provides SIB12 including sl-TxPoolExceptional for the concerned frequency; or

6>	if T304 for MCG is running and the UE is configured with sl-TxPoolExceptional included in sl-ConfigDedicatedNR for the concerned frequency in RRCReconfiguration:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection using the pool of resources indicated by sl-TxPoolExceptional as defined in TS 38.321 [3];

6>	else:

7>	configure lower layers to perform the sidelink resource allocation mode 1 for NR sidelink communication;

6>	if T311 is running, configure the lower layers to release the resources indicated by rrc-ConfiguredSidelinkGrant (if any);

5>	if the UE is configured with sl-UE-SelectedConfig:

6>	if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency included in sl-ConfigDedicatedNR within RRCReconfiguration is not available in accordance with TS 38.214 [19];

7>	if sl-TxPoolExceptional for the concerned frequency is included in RRCReconfiguration; or

7>	if the PCell provides SIB12 including sl-TxPoolExceptional in sl-FreqInfoList/sl-FreqInfoListSizeExt for the concerned frequency:

8>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection using the pool of resources indicated by sl-TxPoolExceptional as defined in TS 38.321 [3];

6>	else, if the sl-TxPoolSelectedNormal for the concerned frequency is included in the sl-ConfigDedicatedNR within RRCReconfiguration:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency;

3>	else:

4>	if the cell chosen for NR sidelink communication transmission provides SIB12:

5>	if the UE acting as U2U Relay UE is performing U2U Relay communication with integrated Discovery as specified in TS 23.304[65], and if the NR sidelink U2U Relay UE threshold conditions for integrated Discovery as specified in 5.8.16.2 are met based on sl-RelayUE-ConfigCommonU2U in SIB12; or

5>	if the UE capable of U2U Remote UE is performing U2U Relay Communication with integrated Discovery as specified in TS 23.304[65], and if the NR sidelink U2U Remote UE threshold conditions for integrated Discovery as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigCommonU2U in SIB12; or

5>	if the UE is performing NR sidelink communication other than U2U Relay Communication with integrated Discovery:

6>	if SIB12 includes sl-TxPoolSelectedNormal for the concerned frequency, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-TxPoolSelectedNormal is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency as defined in TS 38.321 [3];

6>	else if SIB12 includes sl-TxPoolExceptional for the concerned frequency:

7>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

7>	if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]:

8>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using the pool of resources indicated by sl-TxPoolExceptional for the concerned frequency;

2>	else:

3>	if the UE acting as U2U Relay UE is performing U2U Relay communication with integrated Discovery as specified in TS 23.304[65], and if the NR sidelink U2U Relay UE threshold conditions for integrated Discovery as specified in 5.8.16.2 are met based on sl-RelayUE-PreconfigU2U in SidelinkPreconfigNR; or

3>	if the UE capable of U2U Remote UE is performing U2U Relay Communication with integrated Discovery as specified in TS 23.304[65], and if the NR sidelink U2U Remote UE threshold conditions for integrated Discovery as specified in 5.8.17.2 are met based on sl-RemoteUE-PreconfigU2U in SidelinkPreconfigNR; or

3>	if the UE is performing NR sidelink communication other than U2U Relay Communication with integrated Discovery:

4>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-TxPoolSelectedNormal in SidelinkPreconfigNR for the concerned frequency.

NOTE 1:	The UE continues to use resources configured in rrc-ConfiguredSidelinkGrant (while T310 is running) until it is released (i.e. until T310 has expired). The UE does not use sidelink configured grant type 2 resources while T310 is running.

NOTE 2:	In case of RRC reconfiguration with sync, the UE uses resources configured in rrc-ConfiguredSidelinkGrant (while T304 on the MCG is running) if provided by the target cell.

NOTE 3:	It is up to UE implementation to determine, in accordance with TS 38.321[3], which resource pool to use if multiple resource pools are configured, and which resource allocation scheme is used in the AS based on UE capability (for a UE in RRC_IDLE/RRC_INACTIVE) and the allowed resource schemes sl-AllowedResourceSelectionConfig in the resource pool configuration.

NOTE 4:	In case that the network does not provide resource pools in SIB12, a UE which is out of coverage, will be unable to obtain sidelink resources to send the first UL RRC message.

If configured to perform sidelink resource allocation mode 2, the UE capable of NR sidelink communication that is configured by upper layers to transmit NR sidelink communication shall perform resource selection operation according to sl-AllowedResourceSelectionConfig on all pools of resources which may be used for transmission of the sidelink control information and the corresponding data. The pools of resources are indicated by SidelinkPreconfigNR, sl-TxPoolSelectedNormal in sl-ConfigDedicatedNR, or sl-TxPoolSelectedNormal in SIB12 for the concerned frequency, as configured above.

### 5.8.9	Sidelink RRC procedure

#### 5.8.9.1	Sidelink RRC reconfiguration

##### 5.8.9.1.1	General

Figure 5.8.9.1.1-1: Sidelink RRC reconfiguration, successful

Figure 5.8.9.1.1-2: Sidelink RRC reconfiguration, failure

The purpose of this procedure is to modify a PC5-RRC connection, e.g. to establish/modify/release sidelink DRBs or additional sidelink RLC bearer or PC5 Relay RLC channels, to add/release sidelink carrier, to (re-)configure NR sidelink measurement and reporting, to (re-)configure sidelink CSI reference signal resources, to (re)configure CSI reporting latency bound, to (re)configure sidelink DRX, to (re-)configure the latency bound of SL Inter-UE coordination report, and to indicate the SFN-DFN offset in case of single hop.

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases:

-	the release of sidelink DRBs associated with the peer UE, or peer L2 U2U Remote UE in case of L2 U2U Relay operation, as specified in clause 5.8.9.1a.1;

-	the establishment of sidelink DRBs associated with the peer UE, or peer L2 U2U Remote UE in case of L2 U2U Relay operation, as specified in clause 5.8.9.1a.2;

-	the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, or peer L2 U2U Remote UE in case of L2 U2U Relay operation, as specified in clause 5.8.9.1a.2;

-	the release of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.5;

-	the establishment of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.6;

-	the modification for the parameters included in SL-RLC-BearerConfig of additional sidelink RLC bearer associated with the peer UE, as specified in clause 5.8.9.1a.6;

-	the release of PC5 Relay RLC channels for L2 U2N/U2U Relay UE and Remote UE, as specified in clause 5.8.9.7.1;

-	the establishment of PC5 Relay RLC channels for L2 U2N/U2U Relay UE and Remote UE, as specified in clause 5.8.9.7.2;

-	the modification for the parameters included in SL-RLC-ChannelConfigPC5 of PC5 Relay RLC channels for L2 U2N/U2U Relay UE and Remote UE, as specified in clause 5.8.9.7.2;

-	the release of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.1;

-	the addition of sidelink carrier associated with the peer UE, as specified in clause 5.8.9.1b.2;

-	the (re-)configuration of the peer UE to perform NR sidelink measurement and report.

-	the (re-)configuration of the sidelink CSI reference signal resources and CSI reporting latency bound;

-	the (re-)configuration of the peer UE to perform sidelink DRX;

-	the (re-)configuration of the latency bound of SL Inter-UE coordination report;

-	the (re-)configuration of the local UE ID pair for L2 U2U Remote UE and its peer L2 U2U Remote UE by L2 U2U Relay UE.

-	the response to the request in a RemoteUEInformationSidelink message for the SFN-DFN offset from the L2 U2N Remote UE in case of single hop;

-	the change in the value of the SFN-DFN offset at the L2 U2N Relay UE.

NOTE:	It is up to L2 U2N Relay UE implementation to determine when the SFN-DFN offset has changed in value to a degree requiring an update to be sent to the L2 U2N Remote UE.

In RRC_CONNECTED, the UE applies the NR sidelink communications parameters provided in RRCReconfiguration (if any). In RRC_IDLE or RRC_INACTIVE, the UE applies the NR sidelink communications parameters provided in system information (if any). For other cases, UEs apply the NR sidelink communications parameters provided in SidelinkPreconfigNR (if any). When UE performs state transition between above three cases, the UE applies the NR sidelink communications parameters provided in the new state, after acquisition of the new configurations. Before acquisition of the new configurations, UE continues applying the NR sidelink communications parameters provided in the old state.

##### 5.8.9.1.2	Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, by upper layers, or due to end-to-end sidelink DRB release:

2>	set the entry included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	if the sidelink DRB is a per-hop sidelink DRB (i.e. the UE is performing NR sidelink communication with a peer UE without via a L2 U2U Relay UE):

3>	if a sidelink DRB is to be established:

4>	assign a new logical channel identity for the logical channel to be associated with the sidelink DRB and set sl-MAC-LogicalChannelConfigPC5 in the SLRB-Config to include the new logical channel identity;

3>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

2>	else if the sidelink DRB is an end-to-end sidelink DRB (i.e. the UE is acting as a L2 U2U Remote UE, and configure peer L2 U2U Remote UE with end-to-end SDAP and PDCP):

3>	set the SLRB-Config (excluding sl-RLC-ConfigPC5 and sl-MAC-LogicalChannelConfigPC5) included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig corresponding to the sidelink DRB;

1>	for each additional sidelink RLC bearer that is to be released, according to clause 5.8.9.1a.5.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or by upper layers:

2>	set the entry included in the sl-RLC-BearerToReleaseList corresponding to the additional sidelink RLC bearer;

1>	for each additional sidelink RLC bearer that is to be established or modified, according to clause 5.8.9.1a.6.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	if an additional sidelink RLC bearer is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the additional sidelink RLC bearer and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-BearerConfig to include the new logical channel identity;

2>	set the SL-RLC-BearerConfig included in the sl-RLC-BearerToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the additional sidelink RLC bearer;

1>	for each carrier that is to be released, according to clause 5.8.9.1b.1.1:

2>	include the corresponding sidelink carrier in the sl-CarrierToReleaseList;

1>	for each carrier that is to be added, according to clause 5.8.9.1b.2.1:

2>	include the corresponding sidelink carrier in the sl-CarrierToAddModList;

1>	set the sl-MeasConfig as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration information for this destination;

3>	if UE is in RRC_IDLE or RRC_INACTIVE:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration received from SIB12;

2>	else:

3>	set the sl-MeasConfig according to the sl-MeasPreConfig in SidelinkPreconfigNR;

1>	set the sl-LatencyBoundIUC-Report;

1>	start timer T400 for the destination;

1>	set the sl-CSI-RS-Config;

1>	set the sl-LatencyBoundCSI-Report;

1>	set the sl-ResetConfig;

NOTE 1:	Whether/how to set the parameters included in sl-LatencyBoundIUC-Report, sl-CSI-RS-Config, sl-LatencyBoundCSI-Report and sl-ResetConfig is up to UE implementation.

1>	set the sl-DRX-ConfigUC-PC5 as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED and if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

4>	set the sl-DRX-ConfigUC-PC5 according to stored NR sidelink DRX configuration information for this destination;

NOTE 2:	If UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, or in RRC_CONNECTED and sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration, it is up to UE implementation to set the sl-DRX-ConfigUC-PC5.

1>	for each PC5 Relay RLC channel that is to be released due to configuration by sl-ConfigDedicatedNR:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	for each PC5 Relay RLC channel that is to be established or modified due to receiving sl-ConfigDedicatedNR:

2>	if a PC5 Relay RLC channel is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the PC5 Relay RLC channel and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-ChannelConfigPC5 to include the new logical channel identity;

2>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the received SL-RLC-ChannelConfig corresponding to the PC5 Relay RLC channel, including setting sl-RLC-ChannelID-PC5 to the same value of sl-RLC-ChannelID received in SL-RLC-ChannelConfig;

1>	if the UE is operating as a L2 U2N Relay UE:

2>	if the destination UE is a L2 U2N Remote UE that requested the SFN-DFN offset in a previous RemoteUEInformationSidelink message:

3>	if the SFN-DFN offset has changed since a previous transmission of the RRCReconfigurationSidelink message, or no previous transmission of the RRCReconfigurationSidelink message has occurred since the reception of the RemoteUEInformationSidelink message:

4>	set the sl-SFN-DFN-Offset according to the relation between the SFN timeline of the PCell and the DFN timeline;

1>	if the UE is acting as L2 U2U Relay UE, and if the procedure is initiated to configure local ID pair to a connected L2 U2U Remote UE:

2>	if the local ID pair is to be assigned or modified for an end-to-end PC5 connection, and if the per-hop PC5-RRC connection with this L2 U2U Remote UE and the per-hop PC5-RRC connection with its peer L2 U2U Remote UE are successfully established:

3>	include an entry in sl-LocalID-PairList, and set the fields as below:

4>	set sl-RemoteUE-L2Identity to the source L2 ID of this L2 U2U Remote UE, and set sl-RemoteUE-LocalIdentity to include the new local UE ID assigned to this L2 U2U Remote UE, in the SL-SRAP-ConfigPC5;

4>	set sl-PeerRemoteUE-L2Identity to the source L2 ID of the peer L2 U2U Remote UE, and set sl-PeerRemoteUE-LocalIdentity to include the new local UE ID assigned to the peer L2 U2U Remote UE, in the SL-SRAP-ConfigPC5;

1>	if the UE is acting as L2 U2U Remote UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the first hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Relay UE (i.e. Rx UE) according to clause 5.8.9.7.1; or

1>	if the UE is acting as L2 U2U Relay UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the second hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Remote UE (i.e. Rx UE) according to clause 5.8.9.7.1:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	if the UE is acting as L2 U2U Remote UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and if the procedure is initiated to add/modify the first hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Relay UE (i.e. Rx UE), based on configuration in SIB12 or SidelinkPreconfigNR; or

1>	if the UE is acting as L2 U2U Relay UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and if the procedure is initiated to add/modify the second hop PC5 Relay RLC channel to the connected L2 U2U Remote UE (i.e. Rx UE) based on configuration in SIB12 or SidelinkPreconfigNR:

2>	if a PC5 Relay RLC channel is to be established:

3>	assign a new RLC channel ID and set sl-RLC-ChannelID-PC5 in the SL-RLC-ChannelConfigPC5 to include the new RLC channel ID;

3>	assign a new logical channel identity for the logical channel to be associated with the PC5 Relay RLC channel and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-ChannelConfigPC5 to include the new logical channel identity;

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE:

3>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the SL-RLC-BearerConfig derived based on the per-hop QoS of the end-to-end SLRB according to sl-RLC-BearerConfigList in SIB12;

2>	else if the UE is out of coverage:

3>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the SL-RLC-BearerConfig derived based on the per-hop QoS of the end-to-end SLRB according to sl-RLC-BearerPreConfigList in SidelinkPreconfigNR.

NOTE 3:	Void.

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

##### 5.8.9.1.3	Reception of an RRCReconfigurationSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationSidelink:

1>	if the RRCReconfigurationSidelink includes the sl-ResetConfig:

2>	perform the sidelink reset configuration procedure as specified in 5.8.9.1.10;

1>	if the RRCReconfigurationSidelink includes the slrb-ConfigToReleaseList:

2>	for each entry value included in the slrb-ConfigToReleaseList that is part of the current UE sidelink configuration;

3>	perform the sidelink DRB release procedure, according to clause 5.8.9.1a.1;

1>	if the RRCReconfigurationSidelink includes the slrb-ConfigToAddModList:

2>	for each slrb-PC5-ConfigIndex value included in the slrb-ConfigToAddModList that is not part of the current UE sidelink configuration:

3>	if sl-MappedQoS-FlowsToAddList is included:

4>	apply the SL-PQFI included in sl-MappedQoS-FlowsToAddList;

3>	perform the sidelink DRB addition procedure, according to clause 5.8.9.1a.2;

2>	for each slrb-PC5-ConfigIndex value included in the slrb-ConfigToAddModList that is part of the current UE sidelink configuration:

3>	if sl-MappedQoS-FlowsToAddList is included:

4>	add the SL-PQFI included in sl-MappedQoS-FlowsToAddList to the corresponding sidelink DRB;

3>	if sl-MappedQoS-FlowsToReleaseList is included:

4>	remove the SL-PQFI included in sl-MappedQoS-FlowsToReleaseList from the corresponding sidelink DRB;

3>	if the sidelink DRB release conditions as described in clause 5.8.9.1a.1.1 are met:

4>	perform the sidelink DRB release procedure according to clause 5.8.9.1a.1.2;

3>	else if the sidelink DRB modification conditions as described in clause 5.8.9.1a.2.1 are met:

4>	perform the sidelink DRB modification procedure according to clause 5.8.9.1a.2.2;

1>	if the RRCReconfigurationSidelink includes the sl-RLC-BearerToReleaseList:

2>	for each entry value included in the sl-RLC-BearerToReleaseList that is part of the current UE sidelink configuration;

3>	perform the additional sidelink RLC bearer release procedure, according to clause 5.8.9.1a.5;

1>	if the RRCReconfigurationSidelink includes the sl-RLC-BearerToAddModList:

2>	for each SL-RLC-BearerConfigIndex value included in the sl-RLC-BearerToAddModList that is not part of the current UE sidelink configuration:

3>	perform the additional sidelink RLC bearer addition procedure, according to clause 5.8.9.1a.6;

2>	for each SL-RLC-BearerConfigIndex value included in the sl-RLC-BearerToAddModList that is part of the current UE sidelink configuration:

3>	perform the additional sidelink RLC bearer modification procedure, according to clause 5.8.9.1a.6;

1>	if the RRCReconfigurationSidelink includes the sl-CarrierToReleaseList:

2>	for each entry value included in the sl-CarrierToReleaseList that is part of the current UE sidelink configuration;

3>	perform the sidelink carrier release procedure, according to clause 5.8.9.1b.1;

1>	if the RRCReconfigurationSidelink includes the sl-CarrierToAddModList:

2>	for each sl-CarrierId value included in the sl-CarrierToAddModList that is not part of the current UE sidelink configuration:

3>	perform the sidelink carrier addition procedure, according to clause 5.8.9.1b.2;

1>	if the RRCReconfigurationSidelink message includes the sl-MeasConfig:

2>	perform the sidelink measurement configuration procedure as specified in 5.8.10;

1>	if the RRCReconfigurationSidelink message includes the sl-CSI-RS-Config:

2>	apply the sidelink CSI-RS configuration;

1>	if the RRCReconfigurationSidelink message includes the sl-LatencyBoundCSI-Report:

2>	apply the configured sidelink CSI report latency bound;

1>	if the RRCReconfigurationSidelink includes the sl-RLC-ChannelToReleaseListPC5:

2>	for each SL-RLC-ChannelID value included in the sl-RLC-ChannelToReleaseListPC5 that is part of the current UE sidelink configuration;

3>	perform the PC5 Relay RLC channel release procedure, according to clause 5.8.9.7.1;

1>	if the RRCReconfigurationSidelink includes the sl-RLC-ChannelToAddModListPC5:

2>	for each sl-RLC-ChannelID-PC5 value included in the sl-RLC-ChannelToAddModListPC5 that is not part of the current UE sidelink configuration:

3>	perform the PC5 Relay RLC channel addition procedure, according to clause 5.8.9.7.2;

2>	for each sl-RLC-ChannelID-PC5 value included in the sl-RLC-ChannelToAddModListPC5 that is part of the current UE sidelink configuration:

3>	perform the PC5 Relay RLC channel modification procedure according to clause 5.8.9.7.2;

1>	if the RRCReconfigurationSidelink message includes the sl-DRX-ConfigUC-PC5; and

1>	if the UE accepts the sl-DRX-ConfigUC-PC5:

2>	configure lower layers to perform sidelink DRX operation according to sl-DRX-ConfigUC-PC5 for the associated destination as defined in TS 38.321 [3];

1>	if the RRCReconfigurationSidelink message includes the sl-LatencyBoundIUC-Report:

2>	apply the configured sidelink IUC report latency bound;

1>	if the RRCReconfigurationSidelink message includes the sl-LocalID-PairList:

2>	configure SRAP entity to perform NR sidelink L2 U2U relay operation accordingly for the end-to-end PC5 connection with the peer L2 U2U Remote UE as defined in TS 38.351 [65];

1>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfigurationSidelink (i.e. sidelink RRC reconfiguration failure):

2>	continue using the configuration used prior to the reception of the RRCReconfigurationSidelink message;

2>	set the content of the RRCReconfigurationFailureSidelink message;

3>	submit the RRCReconfigurationFailureSidelink message to lower layers for transmission;

1>	if the RRCReconfigurationSidelink message includes the sl-SFN-DFN-Offset:

2>	if the sl-SFN-DFN-Offset is set to setup:

3>	apply the configured SFN-DFN time offset;

2>	if the sl-SFN-DFN-Offset is set to release:

3>	release the received sl-SFN-DFN-Offset;

1>	else:

2>	set the content of the RRCReconfigurationCompleteSidelink message;

3>	if the UE rejects the sidelink DRX configuration sl-DRX-ConfigUC-PC5 received from the peer UE:

4>	include the sl-DRX-ConfigReject in the RRCReconfigurationCompleteSidelink message;

4>	consider no sidelink DRX to be applied for the corresponding sidelink unicast communication;

3>	submit the RRCReconfigurationCompleteSidelink message to lower layers for transmission;

NOTE 1:	When the same logical channel is configured with different RLC mode by another UE, the UE handles the case as sidelink RRC reconfiguration failure.

NOTE 2:	It is up to the UE implementation whether or not to indicate the rejection to the peer UE for a received sidelink DRX configuration.

NOTE 3:	When UE transmits SL-PRS in dedicated SL-PRS resource pool, the sidelink DRX configuration is not applied.

##### 5.8.9.1.4	Void

##### 5.8.9.1.5	Void

##### 5.8.9.1.6	Void

##### 5.8.9.1.7	Void

##### 5.8.9.1.8	Reception of an RRCReconfigurationFailureSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationFailureSidelink:

1>	stop timer T400 for the destination, if running;

1>	continue using the configuration used prior to corresponding RRCReconfigurationSidelink message;

1>	if UE is in RRC_CONNECTED:

2>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3 or clause 5.10.15 in TS 36.331 [10];

##### 5.8.9.1.9	Reception of an RRCReconfigurationCompleteSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink:

1>	stop timer T400 for the destination, if running;

1>	consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied.

2>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

3>	consider no sidelink DRX to be applied for the corresponding sidelink unicast communication.

##### 5.8.9.1.10	Sidelink reset configuration

The UE shall:

1>	release/clear current sidelink radio configuration of this destination received in the RRCReconfigurationSidelink;

1>	release the sidelink DRBs of this destination, in according to clause 5.8.9.1a.1;

1>	release the additional sidelink RLC bearer of this destination, if configured, in according to clause 5.8.9.1a.5;

1>	reset the sidelink specific MAC of this destination, except for end-to-end PC5 connection in L2 U2U Relay operation.

NOTE 1:	Sidelink radio configuration is not just the resource configuration but may include other configurations included in the RRCReconfigurationSidelink message except the sidelink DRBs of this destination.

NOTE 2:	After the sidelink DRB release procedure, UE may perform the sidelink DRB addition according to the current sidelink configuration of this destination, received in sl-ConfigDedicatedNR, SIB12 and SidelinkPreconfigNR, according to clause 5.8.9.1a.2.

#### 5.8.9.1a	Sidelink radio bearer management

##### 5.8.9.1a.1	Sidelink DRB release

5.8.9.1a.1.1	Sidelink DRB release conditions

For NR sidelink communication, a sidelink DRB release is initiated in the following cases:

1>	for groupcast, broadcast and unicast, if slrb-Uu-ConfigIndex (if any) of the sidelink DRB is included in sl-RadioBearerToReleaseList in sl-ConfigDedicatedNR; or

1>	for groupcast and broadcast, if no sidelink QoS flow with data indicated by upper layers is mapped to the sidelink DRB for transmission, which is (re)configured by receiving SIB12 or SidelinkPreconfigNR; or

1>	for groupcast, broadcast and unicast, if SL-RLC-BearerConfigIndex (if any) of the sidelink DRB is included in sl-RLC-BearerToReleaseList/sl-RLC-BearerToReleaseListSizeExt in sl-ConfigDedicatedNR; or

1>	for unicast, if no sidelink QoS flow with data indicated by upper layers is mapped to the sidelink DRB for transmission, which is (re)configured by receiving SIB12 or SidelinkPreconfigNR, and if no sidelink QoS flow mapped to the sidelink DRB, which is (re)configured by receiving RRCReconfigurationSidelink, has data; or

1>	for unicast, if SLRB-PC5-ConfigIndex (if any) of the sidelink DRB is included in slrb-ConfigToReleaseList in RRCReconfigurationSidelink or if sl-ResetConfig is included in RRCReconfigurationSidelink; or

1>	for unicast, when the corresponding PC5-RRC connection is released due to sidelink RLF being detected, according to clause 5.8.9.3; or

1>	for unicast, when the corresponding PC5-RRC connection is released due to upper layer request according to clause 5.8.9.5; or

1>	for L2 U2U relay operation, when the corresponding end-to-end PC5 connection failure is detected according to clause 5.8.9.3a; or

1>	for L2 U2U relay operation, when the corresponding end-to-end PC5-RRC connection is released due to upper layer request according to clause 5.8.9.5a.

5.8.9.1a.1.2	Sidelink DRB release operations

For each sidelink DRB, whose sidelink DRB release conditions are met as in clause 5.8.9.1a.1.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the sidelink DRB release was triggered after the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	release the PDCP entity for NR sidelink communication associated with the sidelink DRB;

2>	if SDAP entity for NR sidelink communication associated with this sidelink DRB is configured:

3>	indicate the release of the sidelink DRB to the SDAP entity associated with this sidelink DRB (TS 37.324 [24], clause 5.3.3);

2>	release SDAP entities for NR sidelink communication, if any, that have no associated sidelink DRB as specified in TS 37.324 [24] clause 5.1.2;

1>	for groupcast and broadcast; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB release was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	for each sl-RLC-BearerConfigIndex included in the received sl-RLC-BearerToReleaseList/sl-RLC-BearerToReleaseListSizeExt that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel for NR sidelink communication, associated with the sl-RLC-BearerConfigIndex.

1>	if the sidelink DRB is a per-hop sidelink DRB (i.e. the UE is performing NR sidelink communication with a peer UE without via a L2 U2U Relay UE):

2>	for unicast, if the sidelink DRB release was triggered due to the reception of the RRCReconfigurationSidelink message; or

2>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB release was triggered due to the configuration received within the SIB12, SidelinkPreconfigNR or indicated by upper layers:

3>	release the RLC entity and the corresponding logical channel for NR sidelink communication associated with the sidelink DRB;

3>	perform the sidelink UE information procedure in clause 5.8.3 for unicast if needed.

1>	if the sidelink radio link failure is detected for a specific destination:

2>	release the PDCP entity, RLC entity and the logical channel of the sidelink DRB for the specific destination.

1>	if the UE is acting as a L2 U2U Remote UE, and if the end-to-end sidelink DRB release is triggered by end-to-end PC5 connection failure due to per-hop PC5 link failure, in accordance with clause 5.8.9.3a:

2>	release the PDCP entity(ies) of the end-to-end sidelink DRB(s) for the specific end-to-end PC5 connection;

1>	if the sidelink DRB is an end-to-end sidelink DRB in L2 U2U relay operation and the UE is acting as a L2 U2U remote UE:

2>	trigger the UE information transfer procedure on sidelink as specified in 5.8.9.11.2;

##### 5.8.9.1a.2	Sidelink DRB addition/modification

5.8.9.1a.2.1	Sidelink DRB addition/modification conditions

For NR sidelink communication, a sidelink DRB addition is initiated only in the following cases:

1>	if any sidelink QoS flow is (re)configured by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR and is to be mapped to one sidelink DRB, which is not established; or

1>	if any sidelink QoS flow is (re)configured by RRCReconfigurationSidelink and is to be mapped to a sidelink DRB, which is not established;

1>	if any sidelink QoS flow is (re)configured by source L2 U2U Remote UE and is mapped to an end-to-end sidelink DRB for transmission when the UE is acting as L2 U2U Relay UE;

The above conditions also apply to L2 U2U Remote UE for end-to-end sidelink DRB addition.

For NR sidelink communication, a sidelink DRB modification is initiated only in the following cases:

1>	if any of the sidelink DRB related parameters is changed by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or RRCReconfigurationSidelink for one sidelink DRB, which is established;

The above conditions also apply to L2 U2U Remote UE for end-to-end sidelink DRB modification.

5.8.9.1a.2.2	Sidelink DRB addition/modification operations

For the sidelink DRB, whose sidelink DRB addition conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the sidelink DRB addition was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB addition was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	if an SDAP entity for NR sidelink communication associated with the destination and the cast type of the sidelink DRB does not exist:

3>	establish an SDAP entity for NR sidelink communication as specified in TS 37.324 [24] clause 5.1.1;

2>	(re)configure the SDAP entity in accordance with the sl-SDAP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-SDAP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, associated with the sidelink DRB;

2>	establish a PDCP entity for NR sidelink communication and configure it in accordance with the sl-PDCP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-PDCP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, associated with the sidelink DRB;

2>	for a per-hop sidelink DRB (i.e. the UE is performing NR sidelink communication with a peer UE without via a L2 U2U Relay UE):

3>	establish a RLC entity for NR sidelink communication and configure it in accordance with the sl-RLC-ConfigPC5 received in the RRCReconfigurationSidelink or sl-RLC-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, associated with sidelink DRB;

3>	if this procedure was due to the reception of a RRCReconfigurationSidelink message:

4>	configure the MAC entity with a logical channel in accordance with the sl-MAC-LogicalChannelConfigPC5 received in the RRCReconfigurationSidelink associated with the sidelink DRB, and perform the sidelink UE information procedure in clause 5.8.3 for unicast if needed;

3>	else if this procedure was due to the reception of a RRCReconfigurationCompleteSidelink message:

4>	configure the MAC entity with a logical channel associated with the sidelink DRB, in accordance with the sl-MAC-LogicalChannelConfig received in the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR;

3>	else (i.e. for groupcast/broadcast):

4>	configure the MAC entity with a logical channel associated with the sidelink DRB, in accordance with the sl-MAC-LogicalChannelConfig received in the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR and assign a new LCID to this logical channel.

1>	if the DRB is an end-to-end sidelink DRB and the UE is acting as L2 U2U Remote UE:

2>	trigger the UE information transfer procedure on sidelink as specified in 5.8.9.11.2;

NOTE 1:	When a sidelink DRB addition is due to the configuration by RRCReconfigurationSidelink, it is up to UE implementation to select the sidelink DRB configuration as necessary transmitting parameters for the sidelink DRB, from the received sl-ConfigDedicatedNR (if in RRC_CONNECTED), SIB12 (if in RRC_IDLE/INACTIVE), SidelinkPreconfigNR (if out of coverage) with the same RLC mode as the one configured in RRCReconfigurationSidelink.

For the sidelink DRB, whose sidelink DRB modification conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the sidelink DRB modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink DRB modification was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	reconfigure the SDAP entity of the sidelink DRB, in accordance with the sl-SDAP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-SDAP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the PDCP entity of the sidelink DRB, in accordance with the sl-PDCP-ConfigPC5 received in the RRCReconfigurationSidelink or sl-PDCP-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the RLC entity of the sidelink DRB, in accordance with the sl-RLC-ConfigPC5 received in the RRCReconfigurationSidelink or sl-RLC-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the logical channel of the sidelink DRB, in accordance with the sl-MAC-LogicalChannelConfigPC5 received in the RRCReconfigurationSidelink or sl-MAC-LogicalChannelConfig received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

1>	if the DRB is an end-to-end sidelink DRB and the UE is acting as L2 U2U Remote UE:

2>	trigger the UE information transfer procedure on sidelink as specified in 5.8.9.11.2.

##### 5.8.9.1a.3	Sidelink SRB release

The UE shall:

1>	if a PC5-RRC connection release for a specific destination is requested by upper layers or AS layer; or

1>	if the sidelink radio link failure is detected for a specific destination:

2>	release the PDCP entity, RLC entity and the logical channel of the sidelink SRB for PC5-RRC message of the specific destination;

2>	consider the PC5-RRC connection is released for the destination.

1>	if PC5-S transmission for a specific destination is terminated in upper layers:

2>	release the PDCP entity, RLC entity and the logical channel of the sidelink SRB(s) for PC5-S message of the specific destination if any;

1>	if discovery transmission for a specific destination is terminated in upper layers:

2>	release the PDCP entity, RLC entity and the logical channel of the sidelink SRB4 for discovery message of the specific destination;

1>	if an end-to-end PC5-RRC connection release/failure is detected:

2>	if the UE is acting L2 U2U Remote UE:

3>	release the PDCP entity of the end-to-end sidelink SRB for the end-to-end PC5 connection;

2>	release the association between the end-to-end sidelink SRB and the egress PC5 Relay RLC channel, and reconfigure SRAP configuration.

##### 5.8.9.1a.4	Sidelink SRB addition

The UE shall:

1>	if transmission of PC5-S message for a specific destination is requested by upper layers for sidelink SRB:

2>	establish PDCP entity, RLC entity and the logical channel of a sidelink SRB for PC5-S message if needed, as specified in clause 9.1.1.4;

2>	if in coverage on the frequency used for the NR sidelink communication as defined in TS 38.304 [20]:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition as indicated in sl-FreqInfoList, to lower layer;

2>	else:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition as indicated in sl-PreconfigFreqInfoList, to lower layer;

1>	if transmission of discovery message for a specific destination is requested by upper layers for sidelink SRB:

2>	establish PDCP entity, RLC entity and the logical channel of a sidelink SRB4 for discovery message, as specified in clause 9.1.1.4;

1>	if a PC5-RRC connection establishment for a specific destination is indicated by upper layers:

2>	establish PDCP entity, RLC entity and the logical channel of a sidelink SRB for PC5-RRC message of the specific destination if needed, as specified in clause 9.1.1.4;

2>	consider the PC5-RRC connection is established for the destination;

2>	if in coverage on the frequency used for the NR sidelink communication as defined in TS 38.304 [20]:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition, as indicated in sl-FreqInfoList, to lower layer;

2>	else:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition as specified in clause 5.8.9.1.9, as indicated in sl-PreconfigFreqInfoList, to lower layer;

1>	for end-to-end SRB0/1/2/3:

2>	if the UE is acting L2 U2U Remote UE:

3>	consider the SL-U2U-RLC as specified in clause 9.1.1.4 as the egress PC5 Relay RLC channel;

4>	associate this end-to-end sidelink SRB with the SL-U2U-RLC and configure the mapping between the end-to-end sidelink SRB and the egress PC5 Relay RLC channel to SRAP.

##### 5.8.9.1a.5	Additional Sidelink RLC Bearer release

###### 5.8.9.1a.5.1	Additional Sidelink RLC Bearer release conditions

For NR sidelink communication, additional sidelink RLC bearer release is initiated in the following cases:

1>	for sidelink DRB, the release conditions are met as in clause 5.8.9.1a.1.1 for the associated sidelink DRB; or

1>	for sidelink SRB 1/2/3, the release conditions are met as in clause 5.8.9.1a.3 for the associated sidelink SRB; or

1>	for sidelink DRB, if SL-RLC-BearerConfigIndex (if any) of the sidelink DRB is included in sl-RLC-BearerToReleaseList in RRCReconfigurationSidelink; or

1>	for sidelink DRB, if SL-RLC-BearerConfigIndex (if any) of the additional Sidelink RLC Bearer is included in sl-RLC-BearerToReleaseListSizeExt in sl-ConfigDedicatedNR;

###### 5.8.9.1a.5.2	Additional Sidelink RLC Bearer release operation

The UE shall:

1>	for groupcast and broadcast; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the additional sidelink RLC bearer release was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	for each sl-RLC-BearerConfigIndex included in the received sl-RLC-BearerToReleaseListSizeExt in sl-ConfigDedicatedNR that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel of the additional sidelink RLC bearer for NR sidelink communication, associated with the sl-RLC-BearerConfigIndex.

1>	for unicast, if the additional sidelink RLC bearer release was triggered due to the reception of the RRCReconfigurationSidelink message:

2>	for each SL-RLC-BearerConfigIndex included in received sl-RLC-BearerToReleaseList in RRCReconfigurationSidelink that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel of the additional sidelink RLC bearer for NR sidelink communication associated with the SL-RLC-BearerConfigIndex;

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the additional sidelink RLC bearer release was triggered due to the configuration received within the SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	for each SL-RLC-BearerConfigIndex included in transmitted sl-RLC-BearerToReleaseList in RRCReconfigurationSidelink that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel of the additional sidelink RLC bearer for NR sidelink communication associated with the SL-RLC-BearerConfigIndex;

##### 5.8.9.1a.6	Additional Sidelink RLC Bearer addition/modification

###### 5.8.9.1a.6.1	Additional Sidelink RLC Bearer addition/modification conditions

For NR sidelink communication, additional sidelink RLC bearer addition is initiated only in the following cases:

1>	for unicast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerToAddModList in the RRCReconfigurationSidelink for a slrb-PC5-ConfigIndex; or

1>	for groupcast and broadcast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerToAddModListSizeExt in sl-ConfigDedicatedNR for a sl-ServedRadioBearer; or

1>	for unicast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerToAddModListSizeExt in sl-ConfigDedicatedNR for a sl-ServedRadioBearer; or

1>	for groupcast and broadcast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerConfigListSizeExt in SIB12 or in SidelinkPreconfigNR for a sl-ServedRadioBearer, if the sidelink DRB has been established as in clause 5.8.9.1a.2 and has not been released as in clause 5.8.9.1a.1, and if the SL-TxProfile of all associated QoS flow(s) for the sl-ServedRadioBearer indicates backwardsIncompatible; or

1>	for groupcast and broadcast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerConfigListSizeExt in SIB12 or in SidelinkPreconfigNR for a sl-ServedRadioBearer, if the sidelink DRB has been established as in clause 5.8.9.1a.2 and has not been released as in clause 5.8.9.1a.1, and if the SL-TxProfile of at least one QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible and UE decides to use PDCP duplication; or

1>	for unicast, for sidelink DRB, if SL-RLC-BearerConfig is received in sl-RLC-BearerConfigListSizeExt in SIB12 or in SidelinkPreconfigNR for a sl-ServedRadioBearer, and if both UEs support PDCP duplication; or

1>	for unicast, for sidelink SRB 1/2/3, if both UEs support PDCP duplication and UE decides to use PDCP duplication;

For NR sidelink communication, additional sidelink RLC bearer modification is initiated only in the following cases:

1>	if any of the additional sidelink RLC bearer related parameters is changed by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or RRCReconfigurationSidelink for one additional sidelink RLC bearer, which is established;

###### 5.8.9.1a.6.2	Additional Sidelink RLC Bearer addition/modification operation

For the additional Sidelink RLC bearer, whose addition conditions are met as in clause 5.8.9.1a.6.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the additional Sidelink RLC bearer addition was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, for DRB, after receiving the RRCReconfigurationCompleteSidelink message, if the additional Sidelink RLC bearer addition was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers; or

1>	for unicast, for SRB, after receiving the RRCReconfigurationCompleteSidelink message, if the additional Sidelink RLC bearer addition was decided by UE:

2>	establish an additional RLC entity for NR sidelink communication and configure it in accordance with each sl-RLC-ConfigPC5 received in the RRCReconfigurationSidelink or sl-RLC-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR for SL DRB, or as specified in clause 9.1.1.4 for SL SRB;

2>	if this procedure was for Sidelink DRB:

3>	if this procedure was due to the reception of a RRCReconfigurationSidelink message:

4>	configure the MAC entity with a logical channel in accordance with the sl-MAC-LogicalChannelConfigPC5 received in the RRCReconfigurationSidelink associated with the sidelink RLC entity;

3>	else if this procedure was due to the reception of a RRCReconfigurationCompleteSidelink message:

4>	configure the MAC entity with a logical channel associated with the sidelink RLC entity, in accordance with the sl-MAC-LogicalChannelConfig received in the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR;

3>	else (i.e. for groupcast/broadcast):

4>	configure the MAC entity with a logical channel associated with the sidelink RLC entity, in accordance with the sl-MAC-LogicalChannelConfig received in the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR and assign a new LCID to this logical channel.

3>	if the UE is in RRC_CONNECTED:

4>	indicate the allowed carriers for the two RLC bearers of the DRB, as indicated in sl-AllowedCarriers, to lower layer;

3>	else:

4>	indicate the allowed carriers for the two RLC bearers of the DRB, decided by UE implementation, from the sidelink carrier(s) mapped to the sidelink QoS flow(s) configured by the upper layer which is allowed carrier as indicated in sl-FregInfoList as specified in 5.8.9.1a.4 or is additional allowed carrier(s) as specified in 5.8.9.1b.1.2 and 5.8.9.1b.2.2, to lower layer, where the carrier indicated in sl-FreqInfoList is used for the RLC bearer if the SL-TxProfile of at least one associated QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible, and the allowed carriers for the two RLC bearers are not overlapping with each other;

2>	else (i.e., if this procedure was for Sidelink SRB):

3>	configure the MAC entity with a logical channel associated with the sidelink RLC entity, as specified in clause 9.1.1.4.

3>	if the UE is in RRC_CONNECTED and if sl-AllowedCarrierFreqSet1/sl-AllowedCarrierFreqSet2 are configured:

4>	indicate the allowed carriers for the two RLC bearers of the SRB, as indicated in sl-AllowedCarrierFreqSet1/sl-AllowedCarrierFreqSet2, to lower layer;

3>	else:

4>	indicate the allowed carriers for the two RLC bearers of the SRB, decided by UE implementation, from the allowed carrier as indicated in sl-FregInfoList as specified in 5.8.9.1a.4 and additional allowed carrier(s) as specified in 5.8.9.1b.1.2 and 5.8.9.1b.2.2, to lower layer, where the allowed carriers for the two RLC bearers are not overlapping with each other;

For the additional Sidelink RLC bearer, whose modification conditions are met as in clause 5.8.9.1a.6.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

1>	for groupcast and broadcast; or

1>	for unicast, if the additional Sidelink RLC bearer modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the additional Sidelink RLC bearer modification was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	reconfigure the RLC entity of the sidelink DRB, in accordance with the sl-RLC-ConfigPC5 received in the RRCReconfigurationSidelink or sl-RLC-Config received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included;

2>	reconfigure the logical channel of the sidelink DRB, in accordance with the sl-MAC-LogicalChannelConfigPC5 received in the RRCReconfigurationSidelink or sl-MAC-LogicalChannelConfig received in sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, if included.

#### 5.8.9.1b	Sidelink Carrier Configuration

##### 5.8.9.1b.1	Sidelink Carrier Release

###### 5.8.9.1b.1.1	Sidelink Carrier Release Condition

For NR sidelink communication, sidelink carrier release is initiated in the following cases:

1>	for unicast, if sl-CarrierId of the sidelink carrier is received in sl-CarrierToReleaseList in the RRCReconfigurationSidelink; or

1>	for unicast, if a sidelink carrier failure has been indicated by MAC layer; or

1>	for unicast, if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or upper layer; or

1>	for unicast, if the sidelink SRB(s), DRB(s) or additional sidelink RLC bearer(s), which was associated with the sidelink carrier(s), are released according to clause 5.8.9.1a.3.1, 5.8.9.1a.1.1 or 5.8.9.1a.5.1;

5.8.9.1b.1.2	Sidelink Carrier Release Operation

The UE shall:

1>	for unicast, if the sidelink carrier release was triggered due to the reception of the RRCReconfigurationSidelink message:

2>	for each sl-CarrierId value included in the sl-CarrierToReleaseList:

3>	if the current UE configuration includes a sidelink carrier with value sl-CarrierId:

4>	release the sidelink carrier for reception.

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink carrier release was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	for each sl-CarrierId value included in the sl-CarrierToReleaseList:

3>	if the current UE configuration includes a sidelink carrier with value sl-CarrierId:

4>	release the sidelink carrier for transmission;

4>	indicate the sidelink carrier not as allowed carrier for the RLC bearer of the SRB to lower layer;

4>	indicate the sidelink carrier not as additional allowed carrier for the RLC bearer of the DRB(s), to lower layer.

##### 5.8.9.1b.2	Sidelink Carrier Addition

###### 5.8.9.1b.2.1	Sidelink Carrier Addition Condition

For NR sidelink communication, sidelink carrier addition is initiated in the following cases:

1>	for unicast, if sl-CarrierId of the sidelink carrier is received in sl-CarrierToAddModList in the RRCReconfigurationSidelink; or

1>	for unicast, if a sidelink SRB(s), DRB(s) or additional sidelink RLC bearer(s), which is associated with the sidelink carrier(s) (taking into account at least carrier(s) mapped to the sidelink QoS flow(s) configured by the upper layer, carriers configured in sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR, and carrier(s) supported by both UEs), are established according to clause 5.8.9.1a.4, 5.8.9.1a.2.1 or 5.8.9.1a.6.1;

5.8.9.1b.2.2	Sidelink Carrier Addition Operation

The UE shall:

1>	for unicast, if the sidelink carrier addition was triggered due to the reception of the RRCReconfigurationSidelink message:

2>	for each sl-CarrierId value included in the sl-CarrierToAddModList that is not part of the current UE configuration (sidelink carrier addition):

3>	add the sidelink carrier, corresponding to the sl-CarrierId, in accordance with the sl-OffsetToCarrier, subcarrierSpacing, carrierBandwidth and sl-AbsoluteFrequencyPointA for reception;

1>	for unicast, after receiving the RRCReconfigurationCompleteSidelink message, if the sidelink carrier addition was triggered due to the configuration received within the sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or indicated by upper layers:

2>	for each sl-CarrierId value included in the sl-CarrierToAddModList that is not part of the current UE configuration (sidelink carrier addition):

3>	add the sidelink carrier, corresponding to the sl-CarrierId, in accordance with the sl-AbsoluteFrequencyPointA for transmission;

3>	indicate the sidelink carrier as additional allowed carrier for the RLC bearer of the SRB, in addition to carrier as indicated in sl-FreqInfoList, from carrier(s) mapped to the sidelink QoS flow(s) configured by the upper layer, to lower layer;

3>	indicate the sidelink carrier as additional allowed carrier for the RLC bearer of the DRB(s), for which the sidelink carrier is mapped to the sidelink QoS flow(s) configured by the upper layer, to lower layer;

#### 5.8.9.2	Sidelink UE capability transfer

##### 5.8.9.2.1	General

This clause describes how the UE compiles and transfers its sidelink UE capability information for unicast to the initiating UE.

Figure 5.8.9.2.1-1: Sidelink UE capability transfer

##### 5.8.9.2.2	Initiation

The UE may initiate the sidelink UE capability transfer procedure upon indication from upper layer when it needs (additional) UE radio access capability information.

##### 5.8.9.2.3	Actions related to transmission of the UECapabilityEnquirySidelink by the UE

The initiating UE shall set the contents of UECapabilityEnquirySidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink, if needed;

NOTE 1:	It is up to initiating UE to decide whether ue-CapabilityInformationSidelink should be included.

1>	set frequencyBandListFilterSidelink to include frequency bands for which the peer UE is requested to provide supported bands and band combinations;

NOTE 2:	The initiating UE is not allowed to send the UECapabilityEnquirySidelink message without including the field frequencyBandListFilterSidelink except for end-to-end L2 U2U relay NR sidelink communication.

1>	submit the UECapabilityEnquirySidelink message to lower layers for transmission.

##### 5.8.9.2.4	Actions related to reception of the UECapabilityEnquirySidelink by the UE

The peer UE shall set the contents of UECapabilityInformationSidelink message as follows:

1>	include UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink;

1>	compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilterSidelink, and prioritized in the order of frequencyBandListFilterSidelink (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on).

1>	include into supportedBandCombinationListSidelinkNR as many band combinations as possible from the list of "candidate band combinations", starting from the first entry;

1>	include the received frequencyBandListFilterSidelink in the field appliedFreqBandListFilter of the requested UE capability;

1>	submit the UECapabilityInformationSidelink message to lower layers for transmission.

NOTE 1:	If the UE cannot include all band combinations due to message size or list size constraints, it is up to UE implementation which band combinations it prioritizes.

NOTE 2:	It is up to UE implementation whether to include fields other than pdcp-ParametersSidelink in the UECapabilityInformationSidelink message for end-to-end L2 U2U relay NR sidelink communication.

#### 5.8.9.3	Sidelink radio link failure related actions

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from MAC entity that HARQ-based Sidelink RLF for a specific destination has been detected; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination; or

1>	upon indication of consistent sidelink LBT failures for all RB sets for a specific destination from MAC entity:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs (if any) of this destination, according to clause 5.8.9.1a.1;

2>	release the SRBs of this destination, according to clause 5.8.9.1a.3;

2>	release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination, except for end-to-end PC5 connection in L2 U2U Relay operation;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	if the UE is acting as L2 U2N Remote UE for the destination:

4>	if MP is configured, and neither MCG transmission nor indirect path transmission is suspended:

5>	initiate the indirect path failure information procedure as specified in 5.7.3c;

4>	else (i.e., MP is not configured, or MP is configured and MCG transmission or indirect path transmission is suspended):

5>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3>	else:

4>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

2>	if the UE is acting as L2 U2U Relay UE for the destination which identifies a connected L2 U2U Remote UE:

3>	consider the end-to-end PC5 connection failure for the end-to-end PC5 connection(s) over the per-hop PC5 link established with the L2 U2U Remote UE;

3>	send NotificationMessageSidelink to the peer L2 U2U Remote UE(s) of the end-to-end PC5 connection(s), in accordance with 5.8.9.10.

3>	initiate the end-to-end PC5 connection failure related actions as specified in 5.8.9.3b;

2>	if the UE is acting as L2 U2U Remote UE for the destination which identifies a connected L2 U2U Relay UE:

3>	consider the end-to-end PC5 connection failure for the end-to-end PC5 connection(s) over the per-hop PC5 link established with the L2 U2U Relay UE;

3>	initiate the end-to-end PC5 connection failure related actions as specified in 5.8.9.3a;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

#### 5.8.9.3a	End-to-end PC5 connection failure related actions performed by L2 U2U Remote UE

The UE acting as NR sidelink L2 U2U Remote UE shall:

1>	upon detection of end-to-end PC5 connection failure due to per-hop PC5 link failure, in accordance with clause 5.8.9.3; or

1>	upon detection of end-to-end PC5 connection failure due to per-hop PC5 link release, in accordance with clause 5.8.9.5; or

1>	upon T400 expiry for an end-to-end PC5 connection; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for an end-to-end PC5 connection; or

1>	upon detection end-to-end PC5 connection failure due to reception of NotificationMessageSidelink indicating PC5 RLF from the L2 U2U Relay UE for a specific destination based on the received sl-DestinationIdentityRemoteUE, in accordance with clause 5.8.9.10.4:

2>	release the end-to-end DRBs for this end-to-end PC5 connection, according to clause 5.8.9.1a.1;

2>	release the end-to-end SRBs for this end-to-end PC5 connection, according to clause 5.8.9.1a.3;

2>	discard the end-to-end NR sidelink communication related radio resources and configuration for this end-to-end PC5 connection, including local ID pair in SRAP configuration;

2>	consider the end-to-end PC5-RRC connection is released for this end-to-end PC5 connection;

2>	indicate the release of the end-to-end PC5-RRC connection to the upper layers;

2>	if the end-to-end PC5 connection failure is due to T400 expiry or integrity check failure of SL-SRB2 or SL-SRB3:

3>	send RemoteUEInformationSidelink message to the L2 U2U Relay UE of the end-to-end PC5 connection(s) in accordance with 5.8.9.8.2;

1>	if the PC5 link between the UE and L2 U2U relay UE is not needed for U2U relay operation, but not released:

2>	if the UE is in RRC_IDLE or RRC_INACTIVE or out of coverage:

3>	trigger the PC5 Relay RLC channel derivation procedure, according to clause 5.8.9.7.0.

#### 5.8.9.3b	End-to-end PC5 connection failure/release related actions performed by L2 U2U Relay UE

The UE acting as NR sidelink L2 U2U Relay UE shall:

1>	upon detection end-to-end PC5 connection failure due to per-hop PC5 link failure, in accordance with clause 5.8.9.3; or

1>	upon detection end-to-end PC5 connection failure due to per-hop PC5 link release, in accordance with clause 5.8.9.5; or

1>	upon reception of RemoteUEInfomationSidelink indicating end-to-end connection release or failure for a specific destination based on the received sl-DestinationIdentityRemoteUE, in accordance with clause 5.8.9.8.3:

2>	discard the end-to-end NR sidelink communication related radio resources and configuration for this end-to-end PC5 connection, including end-to-end SRB/DRB related configuration, QoS related configuration, SRAP configuration;

2>	if the PC5 link between L2 U2U relay UE and the peer L2 U2U remote UE is still available:

3>	if the UE is in RRC_IDLE or RRC_INACTIVE or out of coverage:

4>	trigger the PC5 Relay RLC channel deriviation procedure, according to clause 5.8.9.7.0;

3>	if the UE is in RRC_CONNECTED:

4>	initiate sidelink UE information procedure according to clause 5.8.3.2.

#### 5.8.9.4	Sidelink common control information

##### 5.8.9.4.1	General

The sidelink common control information is carried by MasterInformationBlockSidelink. The sidelink common control information may change at any transmission, i.e. neither a modification period nor a change notification mechanism is used. This procedure also applies to NR sidelink discovery.

A UE configured to receive or transmit NR sidelink communication/discovery/positioning shall:

1>	if the UE has a selected SyncRef UE, as specified in 5.8.6:

2>	ensure having a valid version of the MasterInformationBlockSidelink message of that SyncRef UE;

##### 5.8.9.4.2	Actions related to reception of MasterInformationBlockSidelink message

Upon receiving MasterInformationBlockSidelink, the UE shall:

1>	apply the values included in the received MasterInformationBlockSidelink message.

##### 5.8.9.4.3	Transmission of MasterInformationBlockSidelink message

The UE shall set the contents of the MasterInformationBlockSidelink message as follows:

1>	if in coverage on the frequency used for the NR sidelink communication/positioning as defined in TS 38.304 [20].

2>	set inCoverage to true;

2>	if tdd-UL-DL-ConfigurationCommon is included in the received SIB1:

3>	set sl-TDD-Config to the value representing the same meaning as that is included in tdd-UL-DL-ConfigurationCommon, as described in TS 38.213, clause 16.1 [13];

2>	else:

3>	set sl-TDD-Config to the value as specified in TS 38.213 [13], clause 16.1;

2>	if syncInfoReserved is included in an entry of configured sl-SyncConfigList corresponding to the concerned frequency from the received SIB12:

3>	set reservedBits to the value of syncInfoReserved in the received SIB12;

2>	else:

3>	set all bits in reservedBits to 0;

1>	else if out of coverage on the frequency used for NR sidelink communication/positioning as defined in TS 38.304 [20]; and the concerned frequency is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in RRCReconfiguration or in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12:

2>	set inCoverage to true;

2>	set reservedBits to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SidelinkPreconfigNR defined in 9.3);

2>	set sl-TDD-Config to the value representing the same meaning as that is included in the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SL-PreconfigurationNR defined in 9.3) as described in TS 38.213, clause 16.1 [13];

1>	else if out of coverage on the frequency used for NR sidelink communication/positioning as defined in TS 38.304 [20]; and the UE selects GNSS as the synchronization reference and sl-SSB-TimeAllocation3 is not configured for the frequency used in SidelinkPreconfigNR:

2>	set inCoverage to true;

2>	set reservedBits to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SidelinkPreconfigNR defined in 9.3);

2>	set sl-TDD-Config to the value representing the same meaning as that is included in the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SL-PreconfigurationNR defined in 9.3) as described in TS 38.213, clause 16.1 [13];

1>	else if the UE has a selected SyncRef UE (as defined in 5.8.6):

2>	set inCoverage to false;

2>	set sl-TDD-Config and reservedBits to the value of the corresponding field included in the received MasterInformationBlockSidelink;

1>	else:

2>	set inCoverage to false;

2>	set reservedBits to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SidelinkPreconfigNR defined in 9.3);

2> set sl-TDD-Config to the value representing the same meaning as that is included in the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SL-PreconfigurationNR defined in 9.3) as described in TS 38.213, clause 16.1 [13];

1>	set directFrameNumber and slotIndex according to the slot used to transmit the SLSS, as specified in 5.8.5.3;

1>	submit the MasterInformationBlockSidelink to lower layers for transmission upon which the procedure ends;

#### 5.8.9.5	Actions related to PC5-RRC connection release requested by upper layers

The UE initiates the procedure when upper layers request the release of the PC5-RRC connection as specified in TS 24.587 [57] or TS 24.554 [72]. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>	if the PC5-RRC connection release for the specific destination is requested by upper layers:

2>	discard the NR sidelink communication related configuration of this destination;

2>	release the DRBs of this destination if configured, in according to clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to clause 5.8.9.1a.3;

2>	release the PC5 Relay RLC channels if configured, in according to clause 5.8.9.7.1;

2>	reset the sidelink specific MAC of this destination except for end-to-end PC5-RRC connection in L2 U2U relay operation.

2>	consider the PC5-RRC connection is released for the destination;

2>	if the UE is acting as MP remote UE, and this destination identifies a connected MP relay UE:

3>	if neither MCG transmission nor indirect path transmission is suspended:

4>	initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure;

3>	else if T301 is not running:

4>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

2>	if the UE is acting as L2 U2U Remote UE, and this destination identifies a connected L2 U2U Relay UE:

3>	consider the end-to-end PC5 connection failure for the end-to-end PC5 connection(s) over the per-hop PC5 link established with the L2 U2U Relay UE;

3>	initiate the end-to-end PC5 connection failure related actions as specified in 5.8.9.3a;

2>	if the UE is acting as L2 U2U Relay UE, and this destination identifies a connected L2 U2U Remote UE:

3>	consider the end-to-end PC5 connection failure for the end-to-end PC5 connection(s) over the per-hop PC5 link established with the L2 U2U Remote UE;

3>	send NotificationMessageSidelink message to the peer L2 U2U Remote UE(s) for the end-to-end PC5 connection(s) in accordance with 5.8.9.10;

3>	initiate the end-to-end PC5 connection failure related actions as specified in 5.8.9.3b;

#### 5.8.9.5a	Actions related to end-to-end PC5-RRC connection release performed by L2 U2U Remote UE

The UE acting as sidelink L2 U2U Remote UE shall:

1>	if the end-to-end PC5-RRC connection release is requested by upper layers as specified in TS 23.304 [65]:

2>	release the end-to-end DRBs for this end-to-end PC5-RRC connection if configured, in according to clause 5.8.9.1a.1;

2>	release the end-to-end SRBs for this end-to-end PC5-RRC connection, in according to clause 5.8.9.1a.3;

2>	discard all the radio resources and configuration for this end-to-end PC5-RRC connection, including local ID pair in SRAP configuration;

2>	consider the end-to-end PC5-RRC connection is released for this end-to-end PC5-RRC connection;

2>	send RemoteUEInformationSidelink message to the L2 U2U Relay UE of the end-to-end PC5 connection(s) in accordance with 5.8.9.8.2;

#### 5.8.9.6	Sidelink UE assistance information

##### 5.8.9.6.1	General

Figure 5.8.9.6.1-1: Sidelink UE assistance information

The purpose of this procedure is for a UE to inform its peer UE of the sidelink DRX assistance information used to determine the sidelink DRX configuration for unicast communication.

For sidelink unicast, a UE may include its desired sidelink DRX configurations in the UEAssistanceInformationSidelink as the sidelink DRX assistance information which is transmitted to its peer UE.

NOTE:	It is up to UE implementation to determine its desired sidelink DRX configurations for unicast communication.

##### 5.8.9.6.2	Initiation

For sidelink unicast, if both a RX UE and its peer TX UE for a direction of sidelink communication are capable of sidelink DRX, the RX UE that is interested in sending the sidelink DRX assistance information may send the UEAssistanceInformationSidelink as the sidelink DRX assistance information to its peer UE when the sidelink DRX assistance information has not been sent previously or when the previously transmitted sidelink DRX assistance information has changed.

##### 5.8.9.6.3	Actions related to reception of UEAssistanceInformationSidelink message

For sidelink unicast, when a UE is in RRC_CONNECTED and is performing sidelink operation with resource allocation mode 1, it may report the sidelink DRX assistance information received within the UEAssistanceInformationSidelink from its peer UE to the network as specified in 5.8.3. For sidelink unicast, when a UE is in RRC_CONNECTED and is performing sidelink operation with resource allocation mode 2 or is in RRC_IDLE or RRC_INACTIVE or out of coverage, regardless of whether the UE has obtained the sidelink DRX assistance information from the UEAssistanceInformationSidelink transmitted from its peer UE or not, it may determine the sidelink DRX configuration SL-DRX-ConfigUC for its peer UE.

NOTE:	When UE determines the sidelink DRX configuration for its peer UE, it may take the sidelink DRX assistance information received from its peer UE into account.

#### 5.8.9.7	PC5 Relay RLC channel management for L2 U2N or U2U relay

##### 5.8.9.7.0	Deriviation of PC5 Relay RLC channel configuration

An L2 U2U Remote UE/L2 U2U Relay UE in RRC_IDLE or in RRC_INACTIVE or out of coverage shall derive PC5 Relay RLC channel configuration based on the per-hop QoS requirements. The update of the PC5 Relay RLC channel configuration can be triggered upon reception of the UEInformationRequestSidelink or UEInformationResponseSidelink due to the update of the per-hop QoS requirements, as described in clause 5.8.9.11.3 and 5.8.9.11.4, or upon detection of end-to-end PC5 connection failure/release, as described in clause 5.8.9.3a and 5.8.9.3b. The UE shall perform PC5 Relay RLC channel release/addition/modification procedure when the corresponding PC5 Relay RLC channel configuration is released/added/modified.

The source L2 U2U Remote UE and L2 U2U Relay UE derive the configuration for the corresponding PC5 Relay RLC channel based on SIB12/Preconfiguration, as follows:

-	The source L2 U2U Remote UE derives the configuration for the PC5 Relay RLC channel(s) between the source L2 U2U Remote UE and L2 U2U Relay UE (i.e. the first hop PC5 Relay RLC channel(s)), by aggregating the QoS profile(s) of the QoS flow(s) with split QoS information on the first hop into a per-SLRB level QoS profile for each end-to-end sidelink DRB as described in clause 5.8.9.11.4, and considering the SL-RLC-BearerConfig in either sl-RLC-BearerConfigList or sl-RLC-BearerPreConfigList (linked to the SL-RadioBearerConfig which matches the per-SLRB level QoS profile) as the first hop PC5 Relay RLC channel configuration.

-	The L2 U2U Relay UE derives the configuration for the PC5 Relay RLC channel(s) between L2 U2U Relay UE and the target L2 U2U Remote UE (i.e. the second hop PC5 Relay RLC channel(s)), by aggregating the QoS profile(s) of the QoS flow(s) with split QoS information on the second hop into a per-SLRB level QoS profile for each end-to-end sidelink DRB as described in clause 5.8.9.11.3, and considering the SL-RLC-BearerConfig in either sl-RLC-BearerConfigList or sl-RLC-BearerPreConfigList (linked to the SL-RadioBearerConfig which matches the per-SLRB level QoS profile) as the second hop PC5 Relay RLC channel configuration.

##### 5.8.9.7.1	PC5 Relay RLC channel release

The UE shall:

1>	if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR:

2>	for each SL-RLC-ChannelID in sl-RLC-ChannelToReleaseList received in sl-ConfigDedicatedNR within RRCReconfiguration, or for each SL-RLC-ChannelID included in the received sl-RLC-ChannelToReleaseListPC5 that is part of the current UE sidelink configuration:

3>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID;

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered based on the derivation of PC5 Relay RLC channel configuration for end-to-end sidelink DRB(s) as specified in 5.8.9.7.0:

2>	release the RLC entity and the corresponding logical channel;

1>	if the PC5 Relay RLC channel release was triggered for a specific destination by upper layers as specified in 5.8.9.5, or due to sidelink RLF as specified in 5.8.9.3:

2>	release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID of the specific destination.

##### 5.8.9.7.2	PC5 Relay RLC channel addition/modification

Upon PC5-RRC connection establishment between the L2 U2N Relay UE and L2 U2N Remote UE, the L2 U2N Relay UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-RLC0 as specified in clause 9.1.1.4:

1>	apply RLC default configuration of SL-RLC1 as defined in clause 9.2.4 if the L2 U2N Relay UE is in RRC_IDLE/INACTIVE state;

Upon PC5-RRC connection establishment between two UEs for L2 U2U relay operation, the L2 U2U Relay UE and the L2 U2U Remote UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-U2U-RLC as specified in clause 9.1.1.4;

The UE shall:

1>	if the PC5 Relay RLC channel addition/modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered due to the configuration received within the sl-ConfigDedicatedNR; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered based on the derivation of PC5 Relay RLC channel configuration(s) for end-to-end sidelink DRB(s) as specified in 5.8.9.7.0:

2>	if the current configuration contains a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5; or

2>	if the configuration in SIB12 or SidelinkPreconfigNR has updated, based on which the PC5 Relay RLC channel is derived:

3>	reconfigure the sidelink RLC entity in accordance with the received sl-RLC-Config or sl-RLC-ConfigPC5;

3>	reconfigure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5;

2>	else (a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5 was not configured before):

3>	establish a sidelink RLC entity in accordance with the received sl-RLC-Config (in sl-ConfigDedicatedNR, or SIB12, or SidelinkPreconfigNR) or sl-RLC-ConfigPC5;

3>	configure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5.

#### 5.8.9.8	Remote UE information

##### 5.8.9.8.1	General

Figure 5.8.9.8.1-1: Remote UE information

This procedure is used by the L2 U2N Remote UE or L2 Intermediate U2N Relay UE in RRC_IDLE/RRC_INACTIVE to inform about the required SIB(s) /posSIB(s), provide Paging related information to the connected parent L2 U2N Relay UE, request the SFN-DFN offset from the connected L2 U2N Relay UE in case of single hop, and trigger L2 U2N Relay UE in RRC_IDLE/RRC_INACTIVE to enter RRC_CONNECTED during indirect path addition/change in MP operation. This procedure is also used by the L2 U2U Remote UE to send end-to-end PC5 connection release/failure related information to L2 U2U Relay UE.

This procedure is used by the L2 U2N Remote UE in RRC_CONNECTED to request the SFN-DFN offset from the connected L2 U2N Relay UE in case of single hop.

NOTE:	MIB is not required by a L2 U2N Remote UE.

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall:

1>	if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before):

2>	include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not stored a valid version, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) that the UE requires for a positioning operation, and the requested posSIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before, and the connected parent L2 U2N relay UE set posSIB-ForwardingSupported to supported:

2>	include sl-RequestedPosSIB-List in the RemoteUEInformationSidelink to indicate the requested posSIB(s);

1>	if the UE needs the SFN-DFN offset based on the request from upper layers and the connected L2 U2N relay UE set sfn-DFN-OffsetSupported to supported:

2>	set sl-SFN-DFN-OffsetRequested to true;

1>	if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

2>	if any paging information is received from the Child UE:

3>	include the received paging information in the sl-PagingInfo-RemoteUE-List;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

When entering RRC_CONNECTED, if L2 U2N remote UE or L2 Intermediate U2N Relay UE had sent sl-RequestedSIB-List, sl-RequestedPosSIB-List, and/or sl-PagingInfo-RemoteUE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall:

1>	set the sl-RequestedSIB-List to the value release if requested before;

1>	set the sl-RequestedPosSIB-List to the value release if requested before;

1>	set the sl-PagingInfo-RemoteUE to the value release if sent before;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

Upon any change in the need of SFN-DFN offset while in RRC_CONNECTED, the L2 U2N Remote UE shall:

1>	if the UE needs the SFN-DFN offset based on the request from upper layers and the connected L2 U2N relay UE set sfn-DFN-OffsetSupported to supported:

2>	set sl-SFN-DFN-OffsetRequested to true;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

The L2 U2N Remote UE in RRC_CONNECTED shall:

1>	if the UE is configured with sl-IndirectPathAddChange set to setup, and not configured with split SRB1 with duplication:

2>	include connectionForMP;

2>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

The L2 U2U Remote UE shall:

1>	upon end-to-end PC5-RRC connection release; or

1>	upon end-to-end PC5-RRC connection failure due to T400 expiry or integrity check failure of SL-SRB2 or SL-SRB3:

2>	include sl-DestinationIdentityRemoteUE;

2>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

##### 5.8.9.8.3	Reception of RemoteUEInformationSidelink message by the L2 U2N/U2U Relay UE

The L2 U2N Relay UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE:

2>	if the UE is in RRC_CONNECTED on an active BWP with common search space configured including pagingSearchSpace; or

2>	if the UE is in RRC_IDLE or RRC_INACTIVE:

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	monitor the Paging message at the L2 U2N Remote UE's paging occasion calculated according to sl-PagingIdentityRemoteUE and sl-PagingCycleRemoteUE included in sl-PagingInfo-RemoteUE;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

2>	else:

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	include the received sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message and perform Sidelink UE information transmission in accordance with 5.8.3;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	initiate transmission of the SidelinkUEInformationNR message to release the sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message in accordance with 5.8.3;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List:

2>	if the sl-RequestedSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List:

4>	perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2 or 5.8.9.8.2;

3>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedSIB-List is set to release:

3>	release received SIB request in sl-RequestedSIB-List;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedPosSIB-List:

2>	if the sl-RequestedPosSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of posSIB(s) indicated in sl-RequestedPosSIB-List:

4>	perform acquisition of the positioning system information indicated in sl-RequestedPosSIB-List in accordance with 5.2.2 or 5.8.9.8.2;

3>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedPosSIB-List is set to release:

3>	release received posSIB request in sl-RequestedPosSIB-List.

1>	if the RemoteUEInformationSidelink includes the connectionForMP:

2>	if the L2 U2N Relay UE is in RRC_IDLE:

3>	initiate an RRC connection establishment as specified in 5.3.3;

2>	else if the L2 U2N Relay UE is in RRC_INACTIVE:

3>	initiate an RRC connection resume as specified in 5.3.13;

The L2 U2U Relay UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-DestinationIdentityRemoteUE:

2>	consider the end-to-end PC5 connection release for the end-to-end PC5 connection between the L2 U2U Remote UE and the peer L2 U2U Remote UE identified by sl-DestinationIdentityRemoteUE;

2>	initiate the end-to-end PC5 connection failure/release related actions as specified in 5.8.9.3b;

#### 5.8.9.9	Uu message transfer in sidelink

##### 5.8.9.9.1	General

Figure 5.8.9.9.1-1: Uu message transfer in sidelink

The purpose of this procedure is to transfer Paging message and System Information from the L2 U2N Relay UE to the L2 U2N Remote UE (in case of single hop) or to the Child UE (in case of multi hop) in RRC_IDLE/RRC_INACTIVE.

##### 5.8.9.9.2	Actions related to transmission of UuMessageTransferSidelink message

The L2 U2N Relay UE initiates the Uu message transfer procedure when at least one of the following conditions is met:

1>	upon receiving Paging message related to the connected L2 U2N Remote UE or the Child UE from network (including Paging message within RRCReconfiguration message);

1>	upon acquisition of the SIB(s) requested by the connected L2 U2N Remote UE or by the Child UE (as indicated in sl-RequestedSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated SIB(s) from network which has been requested by the connected L2 U2N Remote UE or by the Child UE;

1>	upon acquisition of the posSIB(s) requested by the connected L2 U2N Remote UE or by the Child UE (as indicated in sl-RequestedPosSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated posSIB(s) from network which have been requested by the connected L2 U2N Remote UE or by the Child UE;

1>	upon unsolicited SIB1 forwarding to the connected L2 U2N Remote UE or by the Child UE or upon receiving the updated SIB1 from network;

For each associated L2 U2N Remote UE or for each associated Child UE, the L2 U2N Relay UE shall set the contents of UuMessageTransferSidelink message as follows:

1>	include sl-PagingDelivery if the Paging message received from network containing the ue-Identity of the L2 U2N Remote UE;

1>	include sl-SIB1-Delivery if any of the conditions for initiating Uu message transfer procedure related to SIB1 are met;

1>	include sl-SystemInformationDelivery if any of the conditions for initiating Uu message transfer procedure related to System Information are met;

1>	submit the UuMessageTransferSidelink message to lower layers for transmission.

NOTE:	The L2 U2N Relay UE may perform unsolicited forwarding of SIB1 to the L2 U2N Remote UE or to the Child UE based on UE implementation. A L2 U2N Remote UE configured with MP does not apply the SIB1 received from the L2 U2N Relay UE on the indirect path, if any.

##### 5.8.9.9.3	Reception of the UuMessageTransferSidelink by the L2 U2N Remote UE

Upon receiving the UuMessageTransferSidelink message, the L2 U2N Remote UE shall:

1>	if sl-PagingDelivery is included:

2>	perform the paging reception procedure as specified in clause 5.3.2.3;

1>	if sl-SystemInformationDelivery and/or sl-SIB1-Delivery is included:

2>	perform the actions specified in clause 5.2.2.4.

##### 5.8.9.9.4	Reception of the UuMessageTransferSidelink by the L2 Intermediate U2N Relay UE

Upon receiving the UuMessageTransferSidelink message from the connected L2 U2N Parent Relay UE, the L2 Intermediate U2N Relay UE shall:

1>	if sl-PagingDelivery contains the ue-Identity of the child UEs:

2>	consider the paging message of the child UE is acquired;

1>	if sl-SystemInformationDelivery requested by the child UEs and/or sl-SIB1-Delivery is included:

2>	consider the SIB requested by the child UE is acquired;

#### 5.8.9.10	Notification Message

##### 5.8.9.10.1	General

Figure 5.8.9.8.1-1: Notification message in sidelink

This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE or to the connected child UE, or used by a L2 U2U Relay UE to send notification to the L2 U2U Remote UE for an end-to-end PC5 connection when condition(s) as specified in 5.8.9.10.2 is met for the hop between the L2 U2U Relay UE and the peer L2 U2U Remote UE.

##### 5.8.9.10.2	Initiation

The Relay UE may initiate the procedure when one of the following conditions is met:

1>	if the UE is acting as U2N Relay UE or Last U2N Relay UE:

2>	upon Uu RLF as specified in 5.3.10;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon cell reselection;

2>	upon L2 U2N Relay UE's or Last U2N Relay UE’s RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF with its parent relay UE;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of a NotificationMessageSidelink from the parent while in RRC_CONNECTED;

1>	if the UE is acting as L2 U2U Relay UE:

2>	upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3;

2>	upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5;

NOTE:	The Notification Message may not be sent by an Intermediate U2N relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if the relay reselection or cell selection does not cause the change of the serving cell.

##### 5.8.9.10.3	Actions related to transmission of NotificationMessageSidelink message

The Relay UE shall set the indication type as follows:

1>	if the UE is acting as U2N Relay UE or Last U2N Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF:

3>	set the indicationType as relayUE-Uu-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

3>	set the indicationType as relayUE-HO;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection:

3>	set the indicationType as relayUE-CellReselection;

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure:

3>	set the indicationType as relayUE-Uu-RRC-Failure;

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to relay reselection:

3>	set the indicationType as relayUE-RelayReselection;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell selection:

3>	set the indicationType as relayUE-CellSelection;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to PC5 RLF with its parent Relay UE:

3>	set the indicationType as relayUE-PC5-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

3>	set the indicationType as relayUE-HO;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message upon reception of the NotificationMessageSidelink message from the parent relay UE:

3>	set the indicationType as received from the parent relay UE;

2>	submit the NotificationMessageSidelink message to lower layers for transmission.

1>	if the UE is acting as L2 U2U Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to PC5 RLF with L2 U2U Remote UE; or

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE:

3>	set the sl-IndicationType as relayUE-PC5-RLF;

3>	set the sl-DestinationIdentityRemoteUE as the associated destination for L2 U2U Remote UE;

3>	submit the NotificationMessageSidelink message to lower layers for transmission;

##### 5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the Remote UE shall:

1>	if the UE is acting as U2N Remote UE:

2>	if the indicationType is included:

3>	if the UE is L2 U2N Remote UE in RRC_CONNECTED:

4>	if MP is configured and MCG transmission (i.e. direct path) is not suspended;

5>	if the indicationType is relayUE-HO;

6>	suspend indirect path transmission;

5>	else:

6>	initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure;

4>	else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE or child UE in RRC_IDLE or RRC_INACTIVE):

4>	if the PC5-RRC connection with the U2N Relay UE or with the U2N Parent Relay UE is determined to be released:

5>	indicate upper layers to trigger PC5 unicast link release;

4>	else (i.e., maintain the PC5 RRC connection):

5>	if the UE is L2 U2N Remote UE or child UE and the indicationType is relayUE-HO or relayUE-CellReselection:

6>	consider cell re-selection occurs;

NOTE 1:	For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

NOTE 2:	The L2 U2N Remote UE may ignore the NotificationMessageSidelink if it does not release the PC5 unicast link in source side yet during an indirect-to-direct path switch, i.e. T304 is running.

1>	if the UE is acting as L2 U2U Remote UE:

2>	if sl-IndicationType is relayUE-PC5-RLF:

3>	indicate PC5 RLF received from L2 U2U Relay UE to the upper layers for the indicated L2 U2U Remote UE based on the received sl-DestinationIdentityRemoteUE;

3>	consider the end-to-end PC5 connection failure for the end-to-end PC5 connection(s) over the per-hop PC5 link between the L2 U2U Relay UE and the L2 U2U Remote UE identified by sl-DestinationIdentityRemoteUE;

3>	perform the end-to-end PC5 connection failure related actions as specified in 5.8.9.3a;

NOTE 3:	It is up to the upper layers on whether to trigger U2U Relay reselection and whether to keep or release the PC5 link with the U2U Relay UE after the PC5 RLF indication received from U2U Relay UE.

#### 5.8.9.11	UE information transfer on sidelink

##### 5.8.9.11.1	General

Figure 5.8.9.11.1-1: Sidelink UE information procedure

The purpose of this procedure is to transfer the UE information between a Remote UE and a Relay UE in sidelink. The L2 U2U Remote UE informs its end-to-end QoS information to its connected L2 U2U Relay UE in the UEInformationRequestSidelink message, and the L2 U2U Relay UE delivers the split QoS information of the first-hop to the Remote UE in the UEInformationResponseSidelink message.

##### 5.8.9.11.2	Actions related to transmission of the UEInformationRequestSidelink by the UE

For initial information transfer (e.g. for QoS split) or upon change in any of the information in the UEInformationRequestSidelink, the UE shall set the contents of UEInformationRequestSidelink message as follows and include all concerned information, irrespective of trigger condition:

1>	if the UE is acting as L2 U2U Remote UE:

2>	set sl-E2E-QoS-InfoListPC5 to include the end-to-end QoS profile(s) of the sidelink QoS flow(s) towards the peer L2 U2U Remote UE if configured by the upper layer, and for each entry:

3>	set sl-DestinationIdentityRemoteUE to include the associated identity for peer L2 U2U Remote UE;

3>	set sl-E2E-SLRB-Index to include the associated end-to-end SLRB configuration index;

3>	set sl-QoS-InfoList to include the end-to-end QoS profile(s) of the sidelink QoS flow(s);

2>	submit the UEInformationRequestSidelink message to lower layers for transmission.

##### 5.8.9.11.3	Actions related to reception of the UEInformationRequestSidelink by the UE

The UE shall perform the following actions upon reception of the UEInformationRequestSidelink:

1>	if the UE is acting as L2 U2U Relay UE:

2>	perform QoS split based on the sl-QoS-InfoList for each QoS flow to decide the split PDB value for each PC5 hop;

2>	set the contents of UEInformationResponseSidelink message as follows:

3>	set sl-SplitQoS-InfoListPC5 to include the split PDB value for each QoS flow on the first PC5 hop between L2 U2U Relay UE and L2 U2U Remote UE;

2>	submit the UEInformationResponseSidelink message to lower layers for transmission;

2>	for each sl-E2E-SLRB-Index value included in the sl-E2E-QoS-InfoListPC5:

3>	merge the split per-flow QoS on the second PC5 hop into a per-SLRB level QoS profile;

2>	if the UE is in RRC_IDLE or RRC_INACTIVE or out of coverage:

3>	derive the PC5 Relay RLC channel configuration on the second PC5 hop based on the per-SLRB level QoS profile of this end-to-end sidelink DRB, as specified in 5.8.9.7.0;

3>	(re)configure the SRAP entity to map the end-to-end DRB with PC5 Relay RLC channel, if needed;

2>	if the UE is in RRC_CONNECTED:

3>	initiate sidelink UE information procedure according to clause 5.8.3.2 to report per-SLRB level QoS.

NOTE 1:	It is left to Relay UE implementation on how to split the PDB.

NOTE 2:	For a L2 U2U Relay UE, how to merge the split per-flow QoS on the second PC5 hop into a per-SLRB level QoS profile is up to UE implementation.

##### 5.8.9.11.4	Actions related to reception of the UEInformationResponseSidelink by the UE

The UE shall perform the following actions upon reception of the UEInformationResponseSidelink:

1>	if the UE is in RRC_CONNECTED:

2>	initiate sidelink UE information procedure according to clause 5.8.3.2 to report the split QoS profile for the first PC5 hop;

1>	else (UE in RRC_IDLE or RRC_INACTIVE or out of coverage):

2>	for each sl-E2E-SLRB-Index value included in the sl-E2E-QoS-InfoListPC5 in UEInformationRequestSidelink:

3>	merge the split per-flow QoS on the first PC5 hop into a per-SLRB level QoS profile;

3>	derive the PC5 Relay RLC channel configuration for the first PC5 hop based on the per-SLRB level QoS profile of this end-to-end sidelink DRB, as specifed in 5.8.9.7.0;

3>	(re)configure the SRAP entity to map the end-to-end DRB with PC5 Relay RLC channel, if needed.

NOTE:	For a L2 U2U Remote UE in RRC_IDLE or RRC_INACTIVE or out of coverage, how to merge the split per-flow QoS on the first PC5 hop into a per-SLRB level QoS profile is up to UE implementation.

### 5.8.10	Sidelink measurement

#### 5.8.10.1	Introduction

The UE may configure the associated peer UE to perform NR sidelink measurement and report on the corresponding PC5-RRC connection in accordance with the NR sidelink measurement configuration for unicast by RRCReconfigurationSidelink message.

The NR sidelink measurement configuration includes the following parameters for a PC5-RRC connection:

1.	NR sidelink measurement objects: Object(s) on which the associated peer UE shall perform the NR sidelink measurements.

-	For NR sidelink measurement, a NR sidelink measurement object indicates the NR sidelink frequency of reference signals to be measured.

2.	NR sidelink reporting configurations: NR sidelink measurement reporting configuration(s) where there can be one or multiple NR sidelink reporting configurations per NR sidelink measurement object. Each NR sidelink reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a NR sidelink measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for NR sidelink measurement results. In this release, both DMRS and SL-PRS are supported for NR sidelink measurement.

-	Reporting format: The quantities that the UE includes in the measurement report. In this release, only RSRP measurement is supported.

3.	NR sidelink measurement identities: A list of NR sidelink measurement identities where each NR sidelink measurement identity links one NR sidelink measurement object with one NR sidelink reporting configuration. By configuring multiple NR sidelink measurement identities, it is possible to link more than one NR sidelink measurement object to the same NR sidelink reporting configuration, as well as to link more than one NR sidelink reporting configuration to the same NR sidelink measurement object. The NR sidelink measurement identity is also included in the NR sidelink measurement report that triggered the reporting, serving as a reference to the network.

4.	NR sidelink quantity configurations: The NR sidelink quantity configuration defines the NR sidelink measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that NR sidelink measurement. In each configuration, different filter coefficients can be configured for different NR sidelink measurement quantities.

Both UEs of the PC5-RRC connection maintains a NR sidelink measurement object list, a NR sidelink reporting configuration list, and a NR sidelink measurement identities list according to signalling and procedures in this specification.

#### 5.8.10.2	Sidelink measurement configuration

##### 5.8.10.2.1	General

The UE shall:

1>	if the received sl-MeasConfig includes the sl-MeasObjectToRemoveList in the RRCReconfigurationSidelink:

2>	perform the sidelink measurement object removal procedure as specified in 5.8.10.2.4;

1>	if the received sl-MeasConfig includes the sl-MeasObjectToAddModList in the RRCReconfigurationSidelink:

2>	perform the sidelink measurement object addition/modification procedure as specified in 5.8.10.2.5;

1>	if the received sl-MeasConfig includes the sl-ReportConfigToRemoveList in the RRCReconfigurationSidelink:

2>	perform the sidelink reporting configuration removal procedure as specified in 5.8.10.2.6;

1>	if the received sl-MeasConfig includes the sl-ReportConfigToAddModList in the RRCReconfigurationSidelink:

2>	perform the sidelink reporting configuration addition/modification procedure as specified in 5.8.10.2.7;

1>	if the received sl-MeasConfig includes the sl-QuantityConfig in the RRCReconfigurationSidelink:

2>	perform the sidelink quantity configuration procedure as specified in 5.8.10.2.8;

1>	if the received sl-MeasConfig includes the sl-MeasIdToRemoveList in the RRCReconfigurationSidelink:

2>	perform the sidelink measurement identity removal procedure as specified in 5.8.10.2.2;

1>	if the received sl-MeasConfig includes the sl-MeasIdToAddModList in the RRCReconfigurationSidelink:

2>	perform the sidelink measurement identity addition/modification procedure as specified in 5.8.10.2.3;

##### 5.8.10.2.2	Sidelink measurement identity removal

The UE shall:

1>	for each sl-MeasId included in the received sl-MeasIdToRemoveList that is part of the current UE configuration in VarMeasConfigSL:

2>	remove the entry with the matching sl-MeasId from the sl-MeasIdList within the VarMeasConfigSL;

2>	remove the NR sidelink measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId.

NOTE:	The UE does not consider the message as erroneous if the sl-MeasIdToRemoveList includes any sl-MeasId value that is not part of the current UE configuration.

##### 5.8.10.2.3	Sidelink measurement identity addition/modification

The UE shall:

1>	for each sl-MeasId included in the received sl-MeasIdToAddModList:

2>	if an entry with the matching sl-MeasId exists in the sl-MeasIdList within the VarMeasConfigSL:

3>	replace the entry with the value received for this sl-MeasId;

2>	else:

3>	add a new entry for this sl-MeasId within the VarMeasConfigSL;

2>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId;

##### 5.8.10.2.4	Sidelink measurement object removal

The UE shall:

1>	for each sl-MeasObjectId included in the received sl-MeasObjectToRemoveList that is part of sl-MeasObjectList in VarMeasConfigSL:

2>	remove the entry with the matching sl-MeasObjectId from the sl-MeasObjectList within the VarMeasConfigSL;

2>	remove all sl-MeasId associated with this sl-MeasObjectId from the sl-MeasIdList within the VarMeasConfigSL, if any;

2>	if a sl-MeasId is removed from the sl-MeasIdList:

3>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId.

NOTE:	The UE does not consider the message as erroneous if the sl-MeasObjectToRemoveList includes any sl-MeasObjectId value that is not part of the current UE configuration.

##### 5.8.10.2.5	Sidelink measurement object addition/modification

The UE shall:

1>	for each sl-MeasObjectId included in the received sl-MeasObjectToAddModList:

2>	if an entry with the matching sl-MeasObjectId exists in the sl-MeasObjectList within the VarMeasConfigSL, for this entry:

3>	for each sl-MeasId associated with this sl-MeasObjectId included in the sl-MeasIdList within the VarMeasConfigSL, if any:

4>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId;

3>	reconfigure the entry with the value received for this sl-MeasObject;

2>	else:

3>	add a new entry for the received sl-MeasObject to the sl-MeasObjectList within VarMeasConfigSL.

##### 5.8.10.2.6	Sidelink reporting configuration removal

The UE shall:

1>	for each sl-ReportConfigId included in the received sl-ReportConfigToRemoveList that is part of the current UE configuration in VarMeasConfigSL:

2>	remove the entry with the matching sl-ReportConfigId from the sl-ReportConfigList within the VarMeasConfigSL;

2>	remove all sl-MeasId associated with the sl-ReportConfigId from the sl-MeasIdList within the VarMeasConfigSL, if any;

2>	if a sl-MeasId is removed from the sl-MeasIdList:

3>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId.

NOTE:	The UE does not consider the message as erroneous if the sl-ReportConfigToRemoveList includes any sl-ReportConfigId value that is not part of the current UE configuration.

##### 5.8.10.2.7	Sidelink reporting configuration addition/modification

The UE shall:

1>	for each sl-ReportConfigId included in the received sl-ReportConfigToAddModList:

2>	if an entry with the matching sl-ReportConfigId exists in the sl-ReportConfigList within the VarMeasConfigSL, for this entry:

3>	reconfigure the entry with the value received for this sl-ReportConfig;

3>	for each sl-MeasId associated with this sl-ReportConfigId included in the sl-MeasIdList within the VarMeasConfigSL, if any:

4>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId;

2>	else:

3>	add a new entry for the received sl-ReportConfig to the sl-ReportConfigList within the VarMeasConfigSL.

##### 5.8.10.2.8	Sidelink quantity configuration

The UE shall:

1>	for each received sl-QuantityConfig:

2>	set the corresponding parameter(s) in sl-QuantityConfig within VarMeasConfigSL to the value of the received sl-QuantityConfig parameter(s);

1>	for each sl-MeasId included in the sl-MeasIdList within VarMeasConfigSL:

2>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId.

#### 5.8.10.3	Performing NR sidelink measurements

##### 5.8.10.3.1	General

A UE shall derive NR sidelink measurement results by measuring one or multiple DMRS/SL-PRS associated per PC5-RRC connection as configured by the peer UE associated, as described in 5.8.10.3.2. For all NR sidelink measurement results the UE applies the layer 3 filtering as specified in clause 5.5.3.2, before using the measured results for evaluation of reporting criteria and measurement reporting. In this release, only NR sidelink RSRP can be configured as trigger quantity and reporting quantity.

The UE shall:

1>	for each sl-MeasId included in the sl-MeasIdList within VarMeasConfigSL:

2>	if the sl-MeasObject is associated to NR sidelink and the sl-RS-Type is set to dmrs:

3>	derive the layer 3 filtered NR sidelink measurement result based on PSSCH DMRS for the trigger quantity and each measurement quantity indicated in sl-ReportQuantity using parameters from the associated sl-MeasObject, as described in 5.8.10.3.2.

2>	if the sl-MeasObject is associated to NR sidelink and the sl-RS-Type is set to sl-prs:

3>	derive the layer 3 filtered NR sidelink measurement result based on SL-PRS for the trigger quantity and each measurement quantity indicated in sl-ReportQuantity using parameters from the associated sl-MeasObject, as described in 5.8.10.3.2.

2>	perform the evaluation of reporting criteria as specified in 5.8.10.4.

##### 5.8.10.3.2	Derivation of NR sidelink measurement results

The UE may be configured by the peer UE associated to derive NR sidelink RSRP measurement results per PC5-RRC connection associated to the NR sidelink measurement objects based on parameters configured in the sl-MeasObject and in the sl-ReportConfig.

The UE shall:

1>	for each NR sidelink measurement quantity to be derived based on NR sidelink DMRS/SL-PRS:

2>	derive the corresponding measurement of NR sidelink frequency indicated quantity based on PSSCH DMRS/SL-PRS as described in TS 38.215 [9] in the concerned sl-MeasObject;

2>	apply layer 3 filtering as described in 5.5.3.2.

#### 5.8.10.4	Sidelink measurement report triggering

##### 5.8.10.4.1	General

The UE shall:

1>	for each sl-MeasId included in the sl-MeasIdList within VarMeasConfigSL:

2>	if the sl-ReportType is set to sl-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the sl-EventId of the corresponding sl-ReportConfig within VarMeasConfigSL, is fulfilled for NR sidelink frequency for all NR sidelink measurements after layer 3 filtering taken during sl-TimeToTrigger defined for this event within the VarMeasConfigSL, while the VarMeasReportListSL does not include a NR sidelink measurement reporting entry for this sl-MeasId (a first NR sidelink frequency triggers the event):

3>	include a NR sidelink measurement reporting entry within the VarMeasReportListSL for this sl-MeasId;

3>	set the sl-NumberOfReportsSent defined within the VarMeasReportListSL for this sl-MeasId to 0;

3>	include the concerned NR sidelink frequency in the sl-FrequencyTriggeredList defined within the VarMeasReportListSL for this sl-MeasId;

3>	initiate the NR sidelink measurement reporting procedure, as specified in 5.8.10.5;

2>	else if the sl-ReportType is set to sl-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the sl-EventId of the corresponding sl-ReportConfig within VarMeasConfigSL, is fulfilled for NR sidelink frequency not included in the sl-FrequencyTriggeredList for all NR sidelink measurements after layer 3 filtering taken during sl-TimeToTrigger defined for this event within the VarMeasConfigSL (a subsequent NR sidelink frequency triggers the event):

3>	set the sl-NumberOfReportsSent defined within the VarMeasReportListSL for this sl-MeasId to 0;

3>	include the concerned NR sidelink frequency in the sl-FrequencyTriggeredList defined within the VarMeasReportListSL for this sl-MeasId;

3>	initiate the NR sidelink measurement reporting procedure, as specified in 5.8.10.5;

2>	else if the sl-ReportType is set to sl-EventTriggered and if the leaving condition applicable for this event is fulfilled for NR sidelink frequency included in the sl-FrequencyTriggeredList defined within the VarMeasReportListSL for this sl-MeasId for all NR sidelink measurements after layer 3 filtering taken during sl-TimeToTrigger defined within the VarMeasConfigSL for this event:

3>	remove the concerned NR sidelink frequency in the sl-FrequencyTriggeredList defined within the VarMeasReportListSL for this sl-MeasId;

3>	if sl-ReportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the NR sidelink measurement reporting procedure, as specified in 5.8.10.5;

3>	if the sl-FrequencyTriggeredList defined within the VarMeasReportListSL for this sl-MeasId is empty:

4>	remove the NR sidelink measurement reporting entry within the VarMeasReportListSL for this sl-MeasId;

4>	stop the periodical reporting timer for this sl-MeasId, if running;

2>	if sl-ReportType is set to sl-Periodical and if a (first) NR sidelink measurement result is available:

3>	include a NR sidelink measurement reporting entry within the VarMeasReportListSL for this sl-MeasId;

3>	set the sl-NumberOfReportsSent defined within the VarMeasReportListSL for this sl-MeasId to 0;

3>	initiate the NR sidelink measurement reporting procedure, as specified in 5.8.10.5, immediately after the quantity to be reported becomes available for the NR sidelink frequency:

2>	upon expiry of the periodical reporting timer for this sl-MeasId:

3>	initiate the NR sidelink measurement reporting procedure, as specified in 5.8.10.5.

##### 5.8.10.4.2	Event S1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition S1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition S1-2, as specified below, is fulfilled;

1>	for this NR sidelink measurement, consider the NR sidelink frequency corresponding to the associated sl-MeasObject associated with this event.

Inequality S1-1 (Entering condition)

Ms – Hys > Thresh

Inequality S1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the NR sidelink measurement result of the NR sidelink frequency, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. sl-Hysteresis as defined within sl-ReportConfig for this event).

Thresh is the threshold parameter for this event (i.e. s1-Threshold as defined within sl-ReportConfig for this event).

Ms is expressed in dBm in case of RSRP.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

##### 5.8.10.4.3	Event S2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition S2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition S2-2, as specified below, is fulfilled;

1>	for this NR sidelink measurement, consider the NR sidelink frequency indicated by the sl-MeasObject associated to this event.

Inequality S2-1 (Entering condition)

Ms + Hys < Thresh

Inequality S2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the NR sidelink measurement result of the NR sidelink frequency, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. sl-Hysteresis as defined within sl-ReportConfig for this event).

Thresh is the threshold parameter for this event (i.e. s2-Threshold as defined within sl-ReportConfig for this event).

Ms is expressed in dBm in case of RSRP.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.8.10.5	Sidelink measurement reporting

##### 5.8.10.5.1	General

Figure 5.8.10.5.1-1: NR sidelink measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the peer UE associated.

For the sl-MeasId for which the NR sidelink measurement reporting procedure was triggered, the UE shall set the sl-MeasResults within the MeasurementReportSidelink message as follows:

1>	set the sl-MeasId to the measurement identity that triggered the NR sidelink measurement reporting;

1>	if the sl-ReportConfig associated with the sl-MeasId that triggered the NR sidelink measurement reporting is set to sl-EventTriggered or sl-Periodical:

2>	if the sl-RS-Type within sl-ReportConfig is set to dmrs:

3>	set sl-ResultDMRS within sl-MeasResult to include the NR sidelink DMRS based quantity indicated in the sl-ReportQuantity within the concerned sl-ReportConfig;

2>	if the sl-RS-Type within sl-ReportConfig is set to sl-prs:

3>	set sl-Result-SL-PRS within sl-MeasResult to include the NR SL-PRS based quantity indicated in the sl-ReportQuantity within the concerned sl-ReportConfig;

1>	increment the sl-NumberOfReportsSent as defined within the VarMeasReportListSL for this sl-MeasId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the sl-NumberOfReportsSent as defined within the VarMeasReportListSL for this sl-MeasId is less than the sl-ReportAmount as defined within the corresponding sl-ReportConfig for this sl-MeasId:

2>	start the periodical reporting timer with the value of sl-ReportInterval as defined within the corresponding sl-ReportConfig for this sl-MeasId;

1>	else:

2>	if the sl-ReportType is set to sl-Periodical:

3>	remove the entry within the VarMeasReportListSL for this sl-MeasId;

3>	remove this sl-MeasId from the sl-MeasIdList within VarMeasConfigSL;

1>	submit the MeasurementReportSidelink message to lower layers for transmission, upon which the procedure ends.

### 5.8.11	Zone identity calculation

The UE shall determine an identity of the zone (i.e. Zone_id) in which it is located using the following formulae, if sl-ZoneConfig is configured:

x1= Floor (x / L) Mod 64;

y1= Floor (y / L) Mod 64;

Zone_id = y1 * 64 + x1.

The parameters in the formulae are defined as follows:

L is the value of sl-ZoneLength included in sl-ZoneConfig;

x is the geodesic distance in longitude between UE's current location and geographical coordinates (0, 0) according to WGS84 model [58] and it is expressed in meters;

y is the geodesic distance in latitude between UE's current location and geographical coordinates (0, 0) according to WGS84 model [58] and it is expressed in meters.

NOTE:	How the calculated zone_id is used is specified in TS 38.321 [3].

### 5.8.12	DFN derivation from GNSS

When the UE selects GNSS as the synchronization reference source, the DFN, the subframe number within a frame and slot number within a frame used for NR sidelink communication/discovery are derived from the current UTC time, by the following formulae:

DFN= Floor (0.1*(Tcurrent –Tref–OffsetDFN)) mod 1024

SubframeNumber= Floor (Tcurrent –Tref–OffsetDFN) mod 10

SlotNumber= Floor ((Tcurrent –Tref–OffsetDFN)*2μ) mod (10*2μ)

Where:

Tcurrent is the current UTC time obtained from GNSS. This value is expressed in milliseconds;

Tref is the reference UTC time 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Thursday, December 31, 1899 and Friday, January 1, 1900). This value is expressed in milliseconds;

OffsetDFN is the value sl-OffsetDFN if configured, otherwise it is zero. This value is expressed in milliseconds.

μ=0/1/2/3 corresponding to the 15/30/60/120 kHz of SCS for SL, respectively.

NOTE 1:	In case of leap second change event, how UE obtains the scheduled time of leap second change to adjust Tcurrent correspondingly is left to UE implementation. How UE handles to avoid the sudden discontinuity of DFN is left to UE implementation.

NOTE 2:	Void.

### 5.8.13	NR sidelink discovery

#### 5.8.13.1	General

The purpose of this procedure is to perform NR sidelink discovery as specified in TS 23.304 [65].

#### 5.8.13.2	NR sidelink discovery monitoring

A UE capable of NR sidelink discovery that is configured by upper layers to monitor NR sidelink discovery messages shall:

1>	if the frequency used for NR sidelink discovery is included in sl-FreqInfoToAddModList in RRCReconfiguration message and sl-DiscConfig is included in RRCReconfiguration; or if the frequency used for NR sidelink discovery is included in sl-FreqInfoList included in SIB12 and sl-DiscConfigCommon is included in SIB12:

2>	if the UE is configured with sl-DiscRxPool for NR sidelink discovery reception included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

3>	configure lower layers to monitor sidelink control information and the corresponding data using the resource pool indicated by sl-DiscRxPool for NR sidelink discovery reception in RRCReconfiguration;

2>	else if the UE is configured with sl-RxPool for NR sidelink discovery reception included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

3>	configure lower layers to monitor sidelink control information and the corresponding data using the resource pool indicated by sl-RxPool for NR sidelink discovery reception in RRCReconfiguration;

2>	else if the cell chosen for NR sidelink discovery reception provides SIB12:

3>	if sl-DiscRxPool for NR sidelink discovery reception is included in SIB12:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the resource pool indicated by sl-DiscRxPool for NR sidelink discovery reception in SIB12;

3>	else if sl-RxPool for NR sidelink discovery reception is included in SIB12:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the resource pool indicated by sl-RxPool for NR sidelink discovery reception in SIB12;

1>	else:

2>	if out of coverage on the concerned frequency for NR sidelink discovery:

3>	if sl-DiscRxPool was preconfigured:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the resource pool that was preconfigured by sl-DiscRxPool for NR sidelink discovery reception in SL-PreconfigurationNR, as defined in clause 9.3;

3>	else:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the resource pool that was preconfigured by sl-RxPool for NR sidelink discovery reception in SL-PreconfigurationNR, as defined in clause 9.3;

NOTE:	If sl-DiscRxPool and sl-RxPool are both included in SIB12 or preconfigured, it is up to UE implementation whether to monitor sidelink control information and the corresponding data using the resource pool indicated by sl-RxPool for NR sidelink discovery reception.

#### 5.8.13.3	NR sidelink discovery transmission

A UE capable of NR sidelink discovery that is configured by upper layer to transmit NR sidelink discovery message shall:

1>	if the frequency used for NR sidelink discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message; or if the frequency used for NR sidelink discovery is included in sl-FreqInfoList within SIB12:

2>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

3>	if the UE is acting as NR sidelink U2N Relay UE or Last U2N Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2N Relay UE or Last U2N Relay UE threshold conditions as specified in 5.8.14.2 are met based on sl-RelayUE-Config; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE/ configured with measurement object associated to L2 U2N Relay UEs in both single hop or multi hop case and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-Config; or

3>	if the UE acting as Last U2N Relay UE is sending Discovery Response message with Model B as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.19.2 are met based on sl-RelayUE-ConfigMH; or

3>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.19.2 are met based on sl-RelayUE-ConfigMH; or

3>	if the UE is selecting NR sidelink U2U Relay UE / has a selected NR sidelink U2U Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2U Remote UE threshold conditions associated with the peer NR Sidelink U2U Remote UE as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigU2U; or

3>	if the UE acting as Target Remote UE is performing U2U Relay Discovery with Model B and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2U Remote UE threshold conditions associated with the NR sidelink U2U Relay UE as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigU2U; or

3>	if the UE acting as U2U Relay UE is performing U2U Relay Discovery with Model A as specified in TS 23.304[65], and neighbour UEs in discovery message to be transmitted meet the threshold conditions as specified in 5.8.16.3; or

3> if the UE acting as U2U Relay UE is sending Discovery Response message with Model B as specified in TS 23.304[65]; or

3>	if the UE acting as U2U Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304[65] and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2U Relay UE threshold conditions as specified in 5.8.16.2 are met based on sl-RelayUE-ConfigU2U; or

NOTE 1:	For U2U Relay UE and Target Remote UE, it can be up to UE implementation on cross-layer interaction for the AS layer condition check for discovery message forwarding.

3>	if the UE is performing NR sidelink non-relay discovery:

4>	if the UE is configured with sl-ScheduledConfig:

5>	if T310 for MCG or T311 is running; and if sl-TxPoolExceptional is included in sl-FreqInfoList for the concerned frequency in SIB12 or included in sl-ConfigDedicatedNR in RRCReconfiguration; or

5>	if T301 is running and the cell on which the UE initiated RRC connection re-establishment provides SIB12 including sl-TxPoolExceptional for the concerned frequency; or

5>	if T304 for MCG is running and the UE is configured with sl-TxPoolExceptional included in sl-ConfigDedicatedNR for the concerned frequency in RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection using the resource pool indicated by sl-TxPoolExceptional as defined in TS 38.321 [3] for NR sidelink discovery transmission;

5>	else:

6>	configure lower layers to perform the sidelink resource allocation mode 1 using the resource pool indicated by sl-DiscTxPoolScheduling or sl-TxPoolScheduling for NR sidelink discovery transmission on the concerned frequency in RRCReconfiguration;

5>	if T311 is running, configure the lower layers to release the resources indicated by rrc-ConfiguredSidelinkGrant (if any);

4>	if the UE is configured with sl-UE-SelectedConfig:

5>	if the sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency is included in the sl-ConfigDedicatedNR within RRCReconfiguration, and if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency included in sl-ConfigDedicatedNR within RRCReconfiguration is not available in accordance with TS 38.214 [19]; or

5>	if the sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency is not included in the sl-ConfigDedicatedNR within RRCReconfiguration, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency included in sl-ConfigDedicatedNR within RRCReconfiguration is not available in accordance with TS 38.214 [19];

6>	if sl-TxPoolExceptional for the concerned frequency is included in RRCReconfiguration; or

6>	if the PCell provides SIB12 including sl-TxPoolExceptional in sl-FreqInfoList for the concerned frequency:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection using the resource pool indicated by sl-TxPoolExceptional as defined in TS 38.321 [3] for NR sidelink discovery transmission;

5>	else, if the sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency is included in the sl-ConfigDedicatedNR within RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency in RRCReconfiguration;

5>	else, if the sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency is included in the sl-ConfigDedicatedNR within RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency in RRCReconfiguration;

2>	else if the cell chosen for NR sidelink discovery transmission provides SIB12:

3>	if the UE is acting as NR sidelink U2N Relay UE or Last U2N Relay UE and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Relay UE or Last U2N Relay UE threshold conditions as specified in 5.8.14.2 are met based on sl-RelayUE-ConfigCommon in SIB12; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE in both single hop or multi hop case and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE acting as Intermediate U2N Relay UE has an established PC5 link with the selected parent U2N Relay UE, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.19.2 are met based on sl-RelayUE-ConfigCommonMH; or

3>	if the UE acting as Last U2N Relay UE is sending Discovery Response message with Model B as specified in TS 23.304 [65] and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.19.2 are met based on sl-RelayUE-ConfigCommonMH; or

3>	if the UE is selecting NR sidelink U2U Relay UE / has a selected NR sidelink U2U Relay UE and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2U Remote UE threshold conditions associated with the peer NR Sidelink U2U Remote UE as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigCommonU2U in SIB12; or

3>	if the UE acting as Target Remote UE is performing U2U Relay Discovery with Model B and if the NR sidelink U2U Remote UE threshold conditions associated with the NR sidelink U2U Relay UE as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigCommonU2U in SIB12; or

3>	if the UE acting as U2U Relay UE is performing U2U Relay Discovery with Model A as specified in TS 23.304[65], and neighbor UEs in discovery message to be transmitted meet the threshold conditions as specified in 5.8.16.3; or

3> if the UE acting as U2U Relay UE is sending Discovery Response message with Model B as specified in TS 23.304[65]; or

3>	if the UE acting as U2U Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304[65] and if the NR sidelink U2U Relay UE threshold conditions as specified in 5.8.16.2 are met based on sl-RelayUE-ConfigCommonU2U in SIB12; or

3>	if the UE is performing NR sidelink non-relay discovery:

4>	if SIB12 includes sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-DiscTxPoolSelected for NR sidelink discovery transmission is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

5>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency in SIB12 as defined in TS 38.321 [3];

4>	else if SIB12 includes sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-TxPoolSelectedNormal for NR sidelink discovery transmission is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

5>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency in SIB12 as defined in TS 38.321 [3];

4>	else if SIB12 includes sl-TxPoolExceptional for the concerned frequency:

5>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

5>	if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]; or

5>	if sl-DiscTxPoolSelected for NR sidelink discovery transmission on the concerned frequency is not included in SIB12 and if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using one of the pools of resources indicated by sl-TxPoolExceptional for NR sidelink discovery transmission on the concerned frequency;

1>	else if out of coverage on the concerned frequency for NR sidelink discovery:

2>	if the UE is acting as L3 U2N Relay UE; or

2>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE in both single hop or multi hop case and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-PreconfigDiscConfig in SidelinkPreconfigNR; or

2>	if the UE acting as Intermediate U2N Relay UE has an established PC5 link with the selected parent U2N Relay UE; or

2>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.19.2 are met based on sl-PreconfigDiscConfig in SidelinkPreconfigNR; or

2>	if the UE is selecting NR sidelink U2U Relay UE / has a selected NR sidelink U2U Relay UE and if the NR sidelink U2U Remote UE threshold conditions associated with the peer NR sidelink U2U Remote UE as specified in 5.8.17.2 are met based on sl-RemoteUE-PreconfigU2U in SidelinkPreconfigNR; or

2>	if the UE acting as Target Remote UE is performing U2U Relay Discovery with Model B and if the NR sidelink U2U Remote UE threshold conditions associated with the NR sidelink U2U Relay UE as specified in 5.8.17.2 are met based on sl-RemoteUE-PreconfigU2U in SidelinkPreconfigNR; or

2>	if the UE acting as U2U Relay UE is performing U2U Relay Discovery with Model A as specified in TS 23.304[65], and neighbor UEs in discovery message to be transmitted meet the threshold conditions as specified in 5.8.16.3; or

2>	if the UE acting as U2U Relay UE is sending Discovery Response message with Model B as specified in TS 23.304[65]; or

2>	if the UE acting as U2U Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304[65] and if the NR sidelink U2U Relay UE threshold conditions as specified in 5.8.16.2 are met based on sl-RelayUE-PreconfigU2U in SidelinkPreconfigNR; or

2>	if the UE is performing NR sidelink non-relay discovery:

3>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated in sl-DiscTxPoolSelected or sl-TxPoolSelectedNormal for NR sidelink discovery transmission on the concerned frequency in SidelinkPreconfigNR.

NOTE 2:	It is up to UE implementation to determine, in accordance with TS 38.321[3], which resource pool to use if multiple resource pools are configured, and which resource allocation scheme is used in the AS based on UE capability (for a UE in RRC_IDLE/RRC_INACTIVE) and the allowed resource schemes sl-AllowedResourceSelectionConfig in the resource pool configuration.

### 5.8.14	NR sidelink U2N Relay UE operation

#### 5.8.14.1	General

This procedure is used by a UE supporting NR sidelink U2N Relay UE operation in case of single hop or by a UE supporting NR sidelink Last U2N Relay UE operation in case of multi hop configured by upper layers to transmit NR sidelink discovery messages to evaluate AS layer conditions.

#### 5.8.14.2	NR sidelink U2N Relay UE threshold conditions

A UE capable of NR sidelink U2N Relay UE or Last U2N Relay UE operation shall:

1>	if the threshold conditions specified in this clause were previously not met:

2>	if threshHighRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRelay by hystMaxRelay if configured; and

2>	if threshLowRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshLowRelay by hystMinRelay if configured:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRelay if configured; or

2>	if the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshLowRelay if configured;

3>	consider the threshold conditions not to be met (leave);

### 5.8.15	NR sidelink U2N Remote UE operation

#### 5.8.15.1	General

This procedure is used by a UE supporting NR sidelink U2N Remote UE operation configured by upper layers to transmit NR sidelink discovery message to evaluate AS layer conditions. The procedure is also used to perform selection and reselection of NR sidelink U2N Relay UE.

#### 5.8.15.2	NR Sidelink U2N Remote UE threshold conditions

A UE capable of NR sidelink U2N Remote UE operation shall:

1>	if the threshold conditions specified in this clause were previously not met:

2>	if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or

2> if the UE has no serving cell:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRemote if configured:

3>	consider the threshold conditions not to be met (leave);

The L2 U2N Remote UE not configured with MP considers the cell indicated by sl-ServingCellInfo in the SL-AccessInfo-L2U2N-r17 received from the connected L2 U2N Relay UE as the PCell/camping cell.

NOTE:	First U2N Relay UE and Intermediate U2N Relay UE first connects to the network via the parent U2N Relay UE(s) acting as a U2N Remote UE after checking the Remote UE threshold conditions above.

#### 5.8.15.3	Selection and reselection of NR sidelink U2N Relay UE

A UE capable of NR sidelink U2N Remote UE operation that is configured by upper layers to search for a NR sidelink U2N Relay UE shall:

1>	if the UE has no serving cell; or

1>	if the RSRP measurement of the cell on which the UE camps (for L2 and L3 U2N Remote UE in RRC_IDLE or RRC_INACTIVE)/ the PCell (for L3 U2N Remote UE in RRC_CONNECTED) is below threshHighRemote within sl-RemoteUE-ConfigCommon/sl-RemoteUE-Config:

2>	if the UE does not have a selected NR sidelink U2N Relay UE; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is available and is below sl-RSRP-Thresh; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is not available, and SD-RSRP of the currently selected U2N Relay UE is below sl-RSRP-Thresh; or

NOTE 1:	U2N Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE. If SD-RSRP is used, the discovery procedure will be performed between the U2N Remote UE and the selected U2N Relay UE.

2>	if the UE has a selected NR sidelink U2N Relay UE, and upper layers indicate not to use the currently selected NR sidelink U2N Relay UE; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and upper layers request the release of the PC5-RRC connection; or

2>	if the UE has a selected NR sidelink U2N Relay UE, and sidelink radio link failure is detected on the PC5-RRC connection with the current U2N Relay UE as specified in clause 5.8.9.3:

3>	perform NR sidelink discovery procedure as specified in clause 5.8.13 in order to search for candidate NR sidelink U2N Relay UEs:

4>	when evaluating the one or more detected NR sidelink U2N Relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2N Relay UE ID and using the sl-FilterCoefficientRSRP in SIB12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the sl-FilterCoefficientRSRP in SidelinkPreconfigNR (out of coverage), before using the SD-RSRP measurement results;

4>	consider a candidate NR sidelink U2N Relay UE for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria;

3>	if the UE detects any suitable NR sidelink U2N Relay UE(s):

4>	consider one of the available suitable NR sidelink U2N relay UE(s) can be selected;

NOTE 2:	A candidate NR sidelink U2N Relay UE which meets all AS layer criteria defined in 5.8.15.3 and higher layer criteria defined in TS 23.304 [65] can be regarded as suitable NR sidelink U2N Relay UE by the NR sidelink U2N Remote UE. If multiple suitable NR sidelink U2N Relay UEs are available, it is up to Remote UE implementation to choose one NR sidelink U2N Relay UE. The details of the interaction with upper layers are up to UE implementation.

NOTE 3:	For L2 U2N Remote UEs in RRC_IDLE/INACTIVE and L3 U2N Remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE. Furthermore, L3 U2N Remote UE's selection on both cell and NR sidelink U2N Relay UE is also based on UE implementation.

NOTE 4:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on any information available in the discovery message including the RRC state information. The RRC state information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

3>	else:

4>	consider no NR sidelink U2N Relay UE to be selected.

When evaluating the currently selected NR sidelink U2N Relay UE, the U2N Remote UE should apply layer 3 filtering as specified in 5.5.3.2 using the sl-FilterCoefficientRSRP in SIB12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the sl-FilterCoefficientRSRP in SidelinkPreconfigNR (out of coverage), before using the SL-RSRP or SD-RSRP measurement results.

### 5.8.16	NR sidelink U2U Relay UE operation

#### 5.8.16.1	General

This procedure is used by a UE supporting NR sidelink U2U Relay UE operation configured by upper layers to forward NR sidelink integrated discovery messages or Model B Discovery messages to evaluate AS layer conditions. The procedure is also used to determine whether a NR sidelink UE is in proximity to NR sidelink U2U Relay UE in Model A Discovery messages.

#### 5.8.16.2	NR sidelink U2U Relay UE threshold conditions

A UE capable of NR sidelink U2U Relay UE operation shall:

1>	if the threshold conditions for sending Direct Communication Request message with integrated Discovery specified in this clause were previously not met:

2>	if the sd-RSRP-ThreshDiscConfig is not configured, or if the SL-RSRP of the Direct Communication Request message with integrated Discovery received from the Source NR sidelink U2U Remote UE is available and is above sd-RSRP-ThreshDiscConfig if configured:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the SL-RSRP of the Direct Communication Request message with integrated Discovery received from the Source NR sidelink U2U Remote UE is available and is below sd-RSRP-ThreshDiscConfig by sd-hystMaxRelay if configured:

3>	consider the threshold conditions not to be met (leave);

1>	if the threshold conditions for sending Relay Discovery Solicitation message with Model B Discovery specified in this clause were previously not met:

2>	if the sd-RSRP-ThreshDiscConfig is not configured, or if the SD-RSRP of the Model B Discovery message received from the Source NR sidelink U2U Remote UE is available and is above sd-RSRP-ThreshDiscConfig if configured:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the SD-RSRP of the Model B Discovery message received from the Source NR sidelink U2U Remote UE is available and is below sd-RSRP-ThreshDiscConfig by sd-hystMaxRelay if configured:

3>	consider the threshold conditions not to be met (leave);

When evaluating the Source NR sidelink U2U Remote UE, the U2U Relay UE shall apply layer 3 filtering as specified in 5.5.3.2 using the sd-FilterCoefficientU2U in SL-RelayUE-ConfigU2U if provided, before using the SL-RSRP or SD-RSRP measurement results.

#### 5.8.16.3	Neighbor UE(s) in proximity conditions

A UE capable of NR sidelink U2U Relay UE operation and is performing U2U Relay Discovery with Model A as specified in TS 23.304[65] shall:

1>	for each of potential neighbor UE(s):

2>	if the SL-RSRP of the UE is available and is above sl-RSRP-Thresh-DiscConfig if configured; or

2>	if the SD-RSRP of the UE is available and is above sd-RSRP-ThreshDiscConfig if configured:

3>	consider the UE as neighbor UE in discovery message to be transmitted as defined in TS 23.304 [65].

NOTE: The interaction with upper layers is left to UE implementation.

When evaluating the potential neighbor UE(s), the U2U Relay UE shall apply layer 3 filtering as specified in 5.5.3.2 using the sl-FilterCoefficientU2U or sd-FilterCoefficientU2U in SL-RelayUE-ConfigU2U if provided, before using the SL-RSRP or SD-RSRP measurement results respectively.

### 5.8.17	NR sidelink U2U Remote UE operation

#### 5.8.17.1	General

This procedure is used by a UE supporting NR sidelink U2U Remote UE operation configured by upper layers to transmit NR sidelink discovery messages to evaluate AS layer conditions. The procedure is also used to perform selection and reselection of NR sidelink U2U Relay UE.

#### 5.8.17.2	NR Sidelink U2U Remote UE threshold conditions

A UE capable of NR sidelink U2U Remote UE operation shall:

1>	if the threshold conditions for sending Relay Discovery Solicitation message with Model B or sending Direct Communication Request message with integrated Discovery specified in this clause were previously not met:

2>	if the SL-RSRP measurement of the peer NR sidelink U2U Remote UE is to be used, and if sl-RSRP-ThreshU2U is not configured, or the SL-RSRP measurement of the peer NR sidelink U2U Remote UE is below sl-RSRP-ThreshU2U by sl-HystMinU2U if configured; or

2>	if the SD-RSRP measurement of the peer NR sidelink U2U Remote UE is to be used, and if sd-RSRP-ThreshU2U is not configured, or the SD-RSRP measurement of the peer NR sidelink U2U Remote UE is below sd-RSRP-ThreshU2U by sd-HystMinU2U if configured; or

2>	if the peer NR sidelink U2U Remote UE is not reachable, i.e. SL-RSRP/SD-RSRP measurement of the peer sidelink U2U Remote UE is not available:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the SL-RSRP measurement of the peer NR sidelink U2U Remote UE is available and is above sl-RSRP-ThreshU2U if configured; or

2>	if the SD-RSRP measurement of the peer NR sidelink U2U Remote UE is available and is above sd-RSRP-ThreshU2U if configured:

3>	consider the threshold conditions not to be met (leave);

1>	if the threshold conditions for sending Relay Discovery Response message with Model B specified in this clause were previously not met:

2>	if the sd-RSRP-ThreshU2U is not configured, or if the SD-RSRP of the NR sidelink U2U Relay UE is available and is above sd-RSRP-ThreshU2U if configured:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the SD-RSRP of the NR sidelink U2U Relay UE is available and is below sd-RSRP-ThreshU2U by sd-HystMinU2U if configured:

3>	consider the threshold conditions not to be met (leave);

When evaluating the peer NR sidelink U2U Remote UE, the U2U Remote UE shall apply layer 3 filtering as specified in 5.5.3.2 using the sl-FilterCoefficientU2U or sd-FilterCoefficientU2U in SL-RemoteUE-ConfigU2U if provided, before using the SL-RSRP or SD-RSRP measurement results respectively.

#### 5.8.17.3	Conditions for selection and reselection of NR sidelink U2U Relay UE

A UE capable of NR sidelink U2U Remote UE operation initiates NR sidelink U2U Relay (re)selection procedure as specified in 5.8.17.4 when one of the following conditions is met:

1>	if the UE does not have a selected NR sidelink U2U Relay UE:

2>	if configured by upper layers to search for or select a NR sidelink U2U Relay UE; or

2>	if sl-RSRP-ThreshU2U is not configured, or if the SL-RSRP measurement of the peer NR sidelink U2U Remote UE is available and is below sl-RSRP-ThreshU2U by sl-HystMinU2U if configured; or

2>	if sd-RSRP-ThreshU2U is not configured, or if the SD-RSRP measurement of the peer NR sidelink U2U Remote UE is available and is below sd-RSRP-ThreshU2U by sd-HystMinU2U if configured;

1>	else if the UE has a selected NR sidelink U2U Relay UE:

2>	if the SL-RSRP of the currently selected NR sidelink U2U Relay UE is available and is below sl-RSRP-ThreshU2U by sl-HystMinU2U within sl-RemoteUE-ConfigU2U if configured; or

2>	if the SD-RSRP of the currently selected NR sidelink U2U Relay UE is available, and is below sd-RSRP-ThreshU2U by sd-HystMinU2U within sl-RemoteUE-ConfigU2U if configured; or

NOTE:	For relay selection, U2U Remote UE uses SL-RSRP measurements for relay selection trigger evaluation when there is data transmission from peer U2U Remote UE to U2U Remote UE. For relay reselection, U2U Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2U Relay UE to U2U Remote UE. And in both cases, it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

2>	if the upper layers indicate to (re)select another NR sidelink U2U Relay UE; or

2>	if the sidelink radio link failure is detected on the PC5-RRC connection with the current NR sidelink U2U Relay UE as specified in clause 5.8.9.3.

When evaluating the currently selected NR sidelink U2U Relay UE, the U2U Remote UE shall apply layer 3 filtering as specified in 5.5.3.2 using the sl-FilterCoefficientU2U or sd-FilterCoefficientU2U in SL-RemoteUE-ConfigU2U if provided, before using the SL-RSRP or SD-RSRP measurement results respectively.

#### 5.8.17.4	Actions related to selection and reselection of NR sidelink U2U Relay UE

Upon initiation of the NR sidelink U2U Relay (re)selection procedure, the UE shall:

1>	perform NR sidelink discovery procedure as specified in clause 5.8.13 or U2U Relay Communication with integrated Discovery as specified in clause 5.8.8, in order to search for candidate NR sidelink U2U Relay UEs:

2>	if the UE is performing NR sidelink discovery procedure as specified in clause 5.8.13:

3>	when evaluating the one or more detected NR sidelink U2U Relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2U Relay UE ID and using the sd-FilterCoefficientU2U in SIB12 (if in RRC_IDLE/INACTIVE), the sd-FilterCoefficientU2U in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the preconfigured sd-FilterCoefficientU2U as defined in 9.3 (out of coverage), before using the SD-RSRP measurement results;

3>	consider a candidate NR sidelink U2U Relay UE for which SD-RSRP exceeds sd-RSRP-ThreshU2U has met the AS criteria;

2>	if the UE is performing U2U Relay Communication with integrated Discovery as specified in TS 23.304 [65] and has received Direct Communication Request message(s) from one or multiple NR sidelink U2U Relay UEs:

3>	when evaluating the NR sidelink U2U Relay UE(s), apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2U Relay UE ID and using the sd-FilterCoefficientU2U in SIB12 (if in RRC_IDLE/INACTIVE), the sd-FilterCoefficientU2U in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the preconfigured sd-FilterCoefficientU2U as defined in 9.3 (out of coverage), before using the SL-RSRP measurement results;

3>	consider a candidate NR sidelink U2U Relay UE for which SL-RSRP exceeds sd-RSRP-ThreshU2U has met the AS criteria;

1>	if the UE detects any suitable NR sidelink U2U Relay UE(s):

2>	consider one of the available suitable NR sidelink U2U Relay UE(s) can be selected;

1>	else:

2>	consider no NR sidelink U2U Relay UE to be selected.

NOTE:	A candidate NR sidelink U2U Relay UE which meets all AS layer criteria defined in 5.8.17.4 and higher layer criteria defined in TS 23.304 [65] can be regarded as suitable NR sidelink U2U Relay UE by the NR sidelink U2U Remote UE. If multiple suitable NR sidelink U2U Relay UEs are available, it is up to Remote UE implementation to choose one NR sidelink U2U Relay UE. The details of the interaction with upper layers are up to UE implementation.

### 5.8.18	NR sidelink positioning

#### 5.8.18.1	General

The purpose of this procedure is to perform NR sidelink positioning as specified in TS 38.305 [73].

#### 5.8.18.2	NR sidelink positioning measurement

A UE capable of NR sidelink positioning that is configured by upper layers for performing SL-PRS measurement:

1>	if the conditions for NR sidelink positioning operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink positioning is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in RRCReconfiguration message or sl-FreqInfoList/sl-FreqInfoListSizeExt included in SIB12 and/or sl-PosFreqInfoList included in SIB23:

3>	if the UE is configured with sl-RxPool and/or sl-PRS-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

4>	configure lower layers to monitor sidelink control information and the corresponding SL-PRS using the pool(s) of resources indicated by sl-RxPool and/or sl-PRS-RxPool;

3>	else if the cell chosen for NR sidelink positioning provides SIB12 and/or SIB23:

4>	configure lower layers to monitor sidelink control information and the corresponding SL-PRS using the pool(s) of resources indicated by sl-RxPool in SIB12 and/or sl-PRS-RxPool in SIB23;

2>	else:

3>	configure lower layers to monitor sidelink control information and the corresponding SL-PRS using the pool(s) of resources that were preconfigured by sl-RxPool and/or sl-PRS-RxPool in SL-PreconfigurationNR, as defined in clause 9.3.

#### 5.8.18.3	NR sidelink positioning transmission

A UE capable of NR sidelink positioning that is configured by upper layers to transmit SL-PRS shall:

1>	if the conditions for NR sidelink positioning operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink positioning is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-PosConfigCommonNR within SIB23 or included in sl-ConfigCommonNR or sl-FreqInfoListSizeExt within SIB12:

3>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

4>	if the UE is configured with sl-ScheduledConfig:

5>	if T310 for MCG or T311 is running; and if sl-PRS-TxPoolExceptional or sl-TxPoolExceptional is included in sl-PosFreqInfoList or sl-FreqInfoList/sl-FreqInfoListSizeExt for the concerned frequency in SIB23 or SIB12 or included in sl-ConfigDedicatedNR in RRCReconfiguration; or

5>	if T301 is running and the cell on which the UE initiated RRC connection re-establishment provides SIB23 or SIB12 including sl-PRS-TxPoolExceptional or sl-TxPoolExceptional for the concerned frequency; or

5>	if T304 for MCG is running and the UE is configured with sl-PRS-TxPoolExceptional or sl-TxPoolExceptional included in sl-ConfigDedicatedNR for the concerned frequency in RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation scheme 2 based on random selection using the resource pool indicated by sl-PRS-TxPoolExceptional or sl-TxPoolExceptional as defined in TS 38.321 [3];

5>	else:

6>	configure lower layers to perform the sidelink resource allocation scheme 1 for NR sidelink positioning;

5>	if T311 is running, configure the lower layers to release the resources indicated by rrc-ConfiguredSidelinkGrant (if any);

4>	if the UE is configured with sl-UE-SelectedConfig:

5>	if a result of full sensing, if selected and is allowed by sl-PosAllowedResourceSelectionConfig, on the resources configured in sl-PRS-TxPoolSelectedNormal or by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency included in sl-ConfigDedicatedNR within RRCReconfiguration is not available in accordance with TS 38.214 [19];

6>	if sl-TxPoolExceptional or sl-PRS-TxPoolExceptional for the concerned frequency is included in RRCReconfiguration; or

6>	if the PCell provides SIB12 and/or SIB23 including sl-TxPoolExceptional or sl-PRS-TxPoolExceptional in sl-FreqInfoList/sl-FreqInfoListSizeExt or sl-PosFreqInfoList for the concerned frequency:

7>	configure lower layers to perform the sidelink resource allocation scheme 2 based on random selection using the pool of resources indicated by sl-TxPoolExceptional or sl-PRS-TxPoolExceptional as defined in TS 38.321 [3];

5>	else, if the sl-PRS-TxPoolSelectedNormal or sl-TxPoolSelectedNormal for the concerned frequency is included in the sl-ConfigDedicatedNR within RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation scheme 2 based on resource selection operation according to sl-PosAllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-PRS-TxPoolSelectedNormal for the concerned frequency, or based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency;

3>	else:

4>	if the cell chosen for NR sidelink positioning transmission provides SIB23 or SIB12:

5>	if SIB23 includes sl-PRS-TxPoolSelectedNormal for the concerned frequency, and a result of full sensing, if selected and is allowed by sl-PosAllowedResourceSelectionConfig, on the resources configured in the sl-PRS-TxPoolSelectedNormal is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-PosAllowedResourceSelectionConfig, is selected:

6>	configure lower layers to perform the sidelink resource allocation scheme 2 based on resource selection operation according to sl-PosAllowedResourceSelectionConfig using the pools of resources indicated by sl-PosTxPoolSelectedNormal for the concerned frequency as defined in TS 38.321 [3];

5>	if SIB12 includes sl-TxPoolSelectedNormal for the concerned frequency, and a result of full sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-TxPoolSelectedNormal is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

6>	configure lower layers to perform the sidelink resource allocation scheme 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency as defined in TS 38.321 [3];

5>	else if SIB23 includes sl-PRS-TxPoolExceptional or SIB12 includes sl-TxPoolExceptional for the concerned frequency:

6>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

6>	if a result of full sensing, if selected and is allowed by sl-PosAllowedResourceSelectionConfig, on the resources configured in sl-PRS-TxPoolSelectedNormal or if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency in SIB12 and/or SIB23 is not available in accordance with TS 38.214 [19]:

7>	configure lower layers to perform the sidelink resource allocation scheme 2 based on random selection (as defined in TS 38.321 [3]) using the pool of resources indicated by sl-PRS-TxPoolExceptional or sl-TxPoolExceptional for the concerned frequency;

2>	else:

3>	configure lower layers to perform the sidelink resource allocation scheme 2 based on resource selection operation according to sl-PosAllowedResourceSelectionConfig or sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-PRS-TxPoolSelectedNormal or sl-TxPoolSelectedNormal in SL-PreconfigurationNR for the concerned frequency.

NOTE:	Void.

NOTE 1:	The UE continues to use resources configured in rrc-ConfiguredSidelinkGrant (while T310 is running) until it is released (i.e. until T310 has expired). The UE does not use sidelink configured grant type 2 resources while T310 is running.

NOTE 2:	In case of RRC reconfiguration with sync, the UE uses resources configured in rrc-ConfiguredSidelinkGrant (while T304 on the MCG is running) if provided by the target cell.

NOTE 3:	It is up to UE implementation to determine, in accordance with TS 38.321[3], which resource pool to use if multiple resource pools are configured, and which resource allocation scheme is used in the AS based on UE capability (for a UE in RRC_IDLE/RRC_INACTIVE) and the allowed resource schemes sl-PosAllowedResourceSelectionConfig or sl-AllowedResourceSelectionConfig in the resource pool configuration.

### 5.8.19	NR sidelink multi-hop U2N Relay UE operation

#### 5.8.19.1	General

This procedure is used by a UE supporting NR sidelink U2N Relay UE operation in case of multi hop configured by upper layers to transmit NR sidelink discovery messages to evaluate AS layer conditions.

#### 5.8.19.2	NR sidelink U2N Relay UE threshold conditions

A UE capable of NR sidelink U2N Relay UE as an Last U2N Relay UE operation and is not having the PC5 connection with the Candidate Child UE shall:

1>	if the threshold conditions for sending the Discovery Solicitation Response message with Model B Discovery specified in this clause were previously not met:

2>	if sd-RSRP-ThreshDiscConfigMH is not configured; or the SD-RSRP of the Model B Discovery message received from the Candidate Child UE is available and is above the sd-RSRP-ThreshDiscConfigMH by sd-hystMaxRelayMH if configured;

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the SD-RSRP of the Model B Discovery message received from the Candidate Child UE is available and is below the sd-RSRP-ThreshDiscConfigMH by sd-hystMaxRelayMH if configured;

3>	consider the threshold conditions not to be met (leave);

A UE capable of NR sidelink U2N Relay UE as an Intermediate U2N Relay UE operation and has not established the PC5 connection with its Parent UE shall:

1>	if the threshold conditions for sending the Discovery Solicitation message with Model B Discovery specified in this clause were previously not met:

2>	if sd-RSRP-ThreshDiscConfigMH is not configured; or the SD-RSRP of the Model B Discovery message received from the Candidate Child UE is available and is above the sd-RSRP-ThreshDiscConfigMH by sd-hystMaxRelayMH if configured;

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the SD-RSRP of the Model B Discovery message received from the Candidate Child UE is available and is below the sd-RSRP-ThreshDiscConfigMH by sd-hystMaxRelayMH if configured;

3>	consider the threshold conditions not to be met (leave);

## 5.9	MBS Broadcast

### 5.9.1	Introduction

#### 5.9.1.1	General

UE receiving or interested to receive MBS broadcast service(s) applies MBS broadcast procedures described in this clause as well as the MBS Interest Indication procedure as specified in clause 5.9.4. The UE may acquire MBS broadcast only if the UE can acquire it without disrupting unicast, SDT or MBS multicast data reception.

MBS broadcast configuration information, except CFR configuration for MCCH/MTCH, is provided on MCCH logical channel. MCCH carries the MBSBroadcastConfiguration message which indicates the MBS broadcast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Optionally, the MBSBroadcastConfiguration message may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. The configuration information required by the UE to receive MCCH is provided in SIB1 and SIB20. Additionally, System Information may provide information related to service continuity of MBS broadcast in SIB21 and information related to the ISA(s) of MBS broadcast service(s) for NTN in SIB27.

#### 5.9.1.2	MCCH scheduling

The MCCH information (i.e. information transmitted in messages sent over MCCH) is transmitted periodically, using a configurable repetition period and within a configured transmission window. MCCH transmissions (and the associated radio resources and MCS) are indicated via the PDCCH addressed to MCCH-RNTI. PDCCH monitoring occasion(s) for MCCH transmission are determined according to the common search space indicated by searchspaceMCCH. If searchspaceMCCH is set to zero, PDCCH monitoring occasions for MCCH message reception in the MCCH transmission window are the same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchspaceMCCH is not set to zero, PDCCH monitoring occasions for MCCH message are determined based on search space indicated by searchspaceMCCH. PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the MCCH transmission window. The [x×N+K]th PDCCH monitoring occasion for MCCH message in MCCH transmission window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MCCH transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the MCCH transmission window, PDCCH for an MCCH message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception MCCH messages is up to UE implementation.

#### 5.9.1.3	MCCH information validity and notification of changes

Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period).

When the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH which schedules the MCCH in every repetition in that modification period.

Upon receiving a change notification, a UE receiving or interested to receive MBS services transmitted using MBS broadcast acquires the new MCCH information starting from the same slot. The UE applies the previously acquired MCCH information until the UE acquires the new MCCH information. The notification is transmitted with a 2-bit bitmap, see TS 38.212 [17] clause 7.3.1.5.1. The MSB in the 2-bit bitmap, when set to '1', indicates the start of new MBS service(s). The LSB in the 2-bit bitmap, when set to '1', indicates modification of MCCH information other than the change caused by start of new MBS service(s), e.g. modification of a configuration of an on-going MBS session(s), MBS session(s) stop or neighbouring cell information modification.

### 5.9.2	MCCH information acquisition

#### 5.9.2.1	General

Figure 5.9.2.1-1: MCCH information acquisition

The UE applies the MCCH information acquisition procedure to acquire the MBS broadcast configuration information broadcasted by the network. The procedure applies to MBS capable UEs interested to receive or that are receiving MBS broadcast services that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMCCH.

#### 5.9.2.2	Initiation

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

NOTE 1:	It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

NOTE 2:	It is up to UE implementation to use the cell/tracking area list and/or the Target Service Area in the USD or the ISA(s) in SIB27 to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

#### 5.9.2.3	MCCH information acquisition by the UE

An MBS capable UE interested to receive or receiving an MBS broadcast service shall:

1>	if the procedure is triggered by an MCCH information change notification:

2>	start acquiring the MBSBroadcastConfiguration message on MCCH in the concerned cell from the slot in which the change notification was received;

1>	if the UE enters a cell providing SIB20; or

1>	if the UE receives sCellSIB20:

2>	acquire the MBSBroadcastConfiguration message on MCCH in the concerned cell at the next repetition period.

#### 5.9.2.4	Actions upon reception of the MBSBroadcastConfiguration message

No UE requirements related to the contents of the MBSBroadcastConfiguration message apply other than those specified elsewhere e.g. within the corresponding field descriptions.

### 5.9.3	Broadcast MRB configuration

#### 5.9.3.1	General

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH.

NOTE:	How to perform a modification of a broadcast MRB which is already configured in the UE is left to UE implementation.

#### 5.9.3.2	Initiation

The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service which is not associated with any ISA and the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in, upon entering the ISA(s) associated with a MBS broadcast service the UE is interested in.

The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service, upon leaving the ISA(s) associated with an MBS broadcast service the UE is interested in.

#### 5.9.3.3	Broadcast MRB establishment

Upon a broadcast MRB establishment, the UE shall:

1>	establish a PDCP entity and an RLC entity in accordance with MRB-InfoBroadcast for this broadcast MRB included in the MBSBroadcastConfiguration message and the configuration specified in 9.1.1.7;

1>	configure the MAC layer in accordance with the mtch-SchedulingInfo (if included);

1>	configure the physical layer in accordance with the mbs-SessionInfoList, searchSpaceMTCH, and pdsch-ConfigMTCH, applicable for the broadcast MRB;

1>	if an SDAP entity with the received mbs-SessionId does not exist:

2>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

2>	indicate the establishment of the user plane resources for the mbs-SessionId to upper layers;

1>	receive DL-SCH on the cell where the MBSBroadcastConfiguration message was received for the established broadcast MRB using g-RNTI and mtch-SchedulingInfo (if included) in this message for this MBS broadcast service.

#### 5.9.3.4	Broadcast MRB release

Upon broadcast MRB release for MBS broadcast service, the UE shall:

1>	release the PDCP entity, RLC entity as well as the related MAC and physical layer configuration;

1>	if the SDAP entity associated with the corresponding mbs-SessionId has no associated MRB:

2>	release the SDAP entity, as specified in TS 37.324 [24] clause 5.1.2;

2>	indicate the release of the user plane resources for the mbs-SessionId to upper layers.

### 5.9.4	MBS Interest Indication

#### 5.9.4.1	General

Figure 5.9.4.1-1: MBS Interest Indication

The purpose of this procedure is to inform the network that the UE in RRC_CONNECTED is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation.

#### 5.9.4.2	Initiation

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

Upon initiating the procedure, the UE shall:

1>	if SIB21 is provided by the PCell; or

1>	if nonServingCellMII is provided in SIB1 by the PCell:

2>	ensure having a valid version of SIB21 for the PCell, if present;

2>	if the UE did not transmit MBS Interest Indication since last entering RRC_CONNECTED state; or

2>	if SIB21 is provided and since the last time the UE transmitted an MBS Interest Indication to a PCell providing SIB21, the UE connected to a PCell, not providing SIB21; or

2> if nonServingCellMII is provided in SIB1 and since the last time the UE transmitted an MBS Interest Indication to a PCell providing nonServingCellMII in SIB1, the UE connected to a PCell not providing nonServingCellMII in SIB1:

3>	if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is not empty:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

2>	else:

3>	if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is different from mbs-FreqList included in the last transmission of the MBS Interest Indication; or

3>	if the set of MBS broadcast frequencies of interest for MBS broadcast reception on non-serving cell, determined in accordance with 5.9.4.3, is different from freqInfoMBS included in the last transmission of the MBS Interest Indication; or

3>	if any of the subcarrier spacing and the CFR information for MBS broadcast reception on non-serving cell has changed since the last transmission of the MBS Interest Indication; or

3>	if the subcarrier spacing and the CFR information for MBS broadcast reception on non-serving cell have been acquired from the non-serving cell which were not reported in the previous MBS Interest Indication; or

3>	if the prioritisation of reception of all indicated MBS broadcast frequencies compared to reception of any of the established unicast bearers and multicast MRBs has changed since the last transmission of the MBS Interest Indication:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

NOTE:	The UE may send MBS Interest Indication even when it is able to receive the MBS services it is interested in, i.e. to avoid that the network allocates a configuration inhibiting MBS broadcast reception.

3>	else if SIB20 is provided for the PCell or for the SCell:

4>	if since the last time the UE transmitted the MBS Interest Indication, the UE connected to a PCell not providing SIB20 and the UE was not provided with SIB20 for an SCell; or

4>	if the set of MBS broadcast services of interest determined in accordance with 5.9.4.4 is different from mbs-ServiceList included in the last transmission of the MBS Interest Indication:

5>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate the transmission of MBSInterestIndication message.

#### 5.9.4.3	MBS frequencies of interest determination

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

2>	at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

2>	for at least one of these MBS sessions, SIB21 acquired from the PCell or a non-serving cell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH.

2>	the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency.

NOTE 3:	When evaluating which frequencies the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies.

#### 5.9.4.4	MBS services of interest determination

The UE shall:

1>	consider an MBS service to be part of the MBS services of interest if the following conditions are met:

2>	the UE is receiving or interested to receive this service via a broadcast MRB; and

2>	the session of this service is ongoing or about to start; and

2>	one or more MBS FSAIs in the USD for this service is included in SIB21 acquired from the PCell for a frequency belonging to the set of MBS frequencies of interest, determined according to 5.9.4.3 or SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service but that frequency is included in the USD for this service.

NOTE:	The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

#### 5.9.4.5	Setting of the contents of MBS Interest Indication

The UE shall set the contents of the MBS Interest Indication as follows:

1>	if the UE has a valid version of SIB21 for the PCell; and

1>	if the set of MBS frequencies of interest, determined in accordance with 5.9.4.3, is not empty:

2>	include mbs-FreqList and set it to include the MBS frequencies of interest sorted by decreasing order of interest, using the absoluteFrequencySSB for serving frequency, if applicable, and the ARFCN-ValueNR(s) as included in SIB21 or in USD (for neighbouring frequencies);

2>	include mbs-Priority if the UE prioritises reception of all indicated MBS frequencies above reception of any of the unicast bearers and multicast MRBs;

NOTE 1:	If the UE prioritises MBS broadcast reception and unicast/multicast data cannot be supported because of congestion on the MBS carrier(s), NG-RAN may for example initiate release of unicast bearers/multicast MRBs.

2>	if SIB20 is provided for the PCell or for the SCell:

3>	include mbs-ServiceList and set it to indicate the set of MBS services of interest sorted by decreasing order of interest determined in accordance with 5.9.4.4;

NOTE 2:	The mbs-ServiceList is not required to be used by the NG-RAN to determine the frequency on which to enable MBS broadcast reception for the UE.

1>	if nonServingCellMII is provided in SIB1 by the PCell; and

1>	if the set of MBS frequencies for MBS broadcast reception on non-serving cell, determined in accordance with 5.9.4.3, is not empty:

2>	include freqInfoMBS;

2>	if the UE has acquired cfr-InfoMBS and subcarrierSpacing for MBS broadcast reception on the non-serving cell:

3>	include cfr-InfoMBS and subcarrierSpacing.

## 5.10	MBS multicast reception in RRC_INACTIVE

### 5.10.1	Introduction

#### 5.10.1.1	General

A UE configured to receive MBS multicast service(s) in RRC_INACTIVE that the UE has joined applies MBS multicast procedures described in this clause.

The multicast MCCH information (i.e., the MBSMulticastConfiguration message) is provided in RRCRelease and on multicast MCCH logical channel.

When there is temporarily no data for an active multicast session or when the multicast session is deactivated, the network notifies the UE to stop monitoring the corresponding G-RNTI via the MBSMulticastConfiguration message. If the UE is notified to stop monitoring the G-RNTI(s) for all the joined multicast sessions, it stops monitoring the Multicast MCCH-RNTI for the cell where it received the notification.

Multicast MCCH carries the MBSMulticastConfiguration message which indicates the MBS multicast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Optionally, the MBSMulticastConfiguration message may also contain a list of neighbour cells providing the same MBS multicast service(s) for reception in RRC_INACTIVE as provided in the current cell. The configuration information required by the UE to receive multicast MCCH is provided in SIB24.

#### 5.10.1.2	Multicast MCCH scheduling

The multicast MCCH information (if sent on multicast MCCH logical channel) is transmitted periodically, using a configurable repetition period and within a configured transmission window. The multicast MCCH transmissions (and the associated radio resources and MCS) are indicated via the PDCCH addressed to Multicast MCCH-RNTI. PDCCH monitoring occasion(s) for the multicast MCCH transmission are determined according to the common search space indicated by searchSpaceMulticastMCCH. If searchSpaceMulticastMCCH is set to zero, PDCCH monitoring occasions for the multicast MCCH message reception in the multicast MCCH transmission window are the same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213 [13]. If searchSpaceMulticastMCCH is not set to zero, PDCCH monitoring occasions for the multicast MCCH message are determined based on search space indicated by searchSpaceMulticastMCCH. PDCCH monitoring occasions for the multicast MCCH message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the multicast MCCH transmission window. The [x×N+K]th PDCCH monitoring occasion for the multicast MCCH message in the multicast MCCH transmission window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in multicast MCCH transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the multicast MCCH transmission window, PDCCH for a multicast MCCH message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception of multicast MCCH messages is up to UE implementation.

#### 5.10.1.3	Multicast MCCH information validity and notification of changes

Change of multicast MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same multicast MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period).

When the network changes (some of) the multicast MCCH information, it notifies the UEs about the change starting from the beginning of the multicast MCCH modification period via PDCCH which schedules the multicast MCCH in every repetition in that modification period.

Upon receiving a change notification, a UE receiving MBS multicast service(s) in RRC_INACTIVE acquires the new multicast MCCH information starting from the same slot. The UE applies the previously acquired multicast MCCH information until the UE acquires the new multicast MCCH information. The notification is transmitted with a 2-bit bitmap, see TS 38.212 [17] clause 7.3.1.5.1. The MSB in the 2-bit bitmap is reserved. The LSB in the 2-bit bitmap, when set to '1', indicates modification of multicast MCCH information, e.g. modification of a configuration of an on-going MBS multicast session, stop of G-RNTI monitoring for the corresponding multicast session or neighbouring cell information modification.

### 5.10.2	Multicast MCCH information acquisition

#### 5.10.2.1	General

Figure 5.10.2.1-1: Multicast MCCH information acquisition

The UE applies the multicast MCCH information acquisition procedure to acquire the MBSMulticastConfiguration message from the network. The procedure applies to UEs configured to receive MBS multicast services in RRC_INACTIVE.

#### 5.10.2.2	Initiation

If configured to receive MBS multicast services in RRC_INACTIVE, a UE applies the multicast MCCH information acquisition procedure for PTM configuration update and upon selection to a new cell (i.e., different from the cell where the UE was configured to receive multicast in RRC_CONNECTED) providing SIB24 or reselection to a cell providing SIB24 (except in case the UE is aware that the multicast sessions that the UE has joined are not available for RRC_INACTIVE in the new cell) and may apply the multicast MCCH information acquisition procedure upon receiving GroupPaging in the paging message. A UE that is receiving MBS multicast data in RRC_INACTIVE shall apply the multicast MCCH information acquisition procedure upon receiving a notification that the multicast MCCH information has changed.

NOTE:	It is up to UE implementation how to address a possibility of the UE missing a multicast MCCH change notification.

Unless explicitly stated otherwise in the procedural specification, the multicast MCCH information acquisition procedure overwrites any stored multicast MCCH information as specified in 5.10.1.1, i.e. delta configuration is not applicable for multicast MCCH information and the UE discontinues using a field if it is absent in multicast MCCH information.

#### 5.10.2.3	Multicast MCCH information acquisition by the UE

A UE configured to receive an MBS multicast service in RRC_INACTIVE shall:

1>	if the procedure is triggered by a multicast MCCH information change notification:

2>	start acquiring the MBSMulticastConfiguration message on multicast MCCH in the concerned cell from the slot in which the change notification was received;

1>	if the UE selects a new cell (i.e., different from the cell where the UE was configured to receive multicast in RRC_CONNECTED) providing SIB24 or reselects a cell providing SIB24; or

1>	if the procedure is triggered by the reception of GroupPaging in the paging message in 5.3.2.3; or

1>	if the UE receives RRCRelease configuring the UE to receive MBS multicast in RRC_INACTIVE which does not include PTM configuration for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI:

2>	acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next repetition period.

#### 5.10.2.4	Actions upon reception of the MBSMulticastConfiguration message

No UE requirements related to the contents of the MBSMulticastConfiguration message apply other than those specified elsewhere, e.g., within the corresponding field descriptions.

### 5.10.3	MRB configuration

#### 5.10.3.1	General

The multicast MRB configuration procedure is used by the UE in RRC_INACTIVE state to configure PDCP, RLC, MAC entities and the physical layer upon PTM configuration update and moving to a cell providing SIB24. The UE may perform multicast MRB modification or release/establishment when PTM configuration is updated via multicast MCCH or when it moves to a cell where the PDCP COUNT of the corresponding multicast MRB is not synchronized within the RNA. The UE may perform multicast MRB modification when it moves to a cell where the PDCP COUNT of the corresponding multicast MRB is synchronized within the RNA. The UE resets MAC upon cell selection or reselection.

NOTE:	How to perform modification of a multicast MRB which is already configured in the UE is left to UE implementation.

Upon moving to a cell where the PDCP COUNT of a multicast MRB is not synchronized, an indication is sent to the lower layer to inform the PDCP COUNT non-synchronization of the corresponding multicast MRB. Upon transition from RRC_CONNECTED to RRC_INACTIVE in the same cell, the UE can continue using the multicast MRBs used in RRC_CONNECTED and in this case, the same LCIDs are used for the same multicast MRBs. Upon transition from RRC_INACTIVE to RRC_CONNECTED, the UE releases the MRBs configured in RRCRelease or multicast MCCH and performs delta configuration based on the stored MRB configuration from RRC_CONNECTED, if not indicated to perform full configuration.

#### 5.10.3.2	Multicast MRB establishment

Upon establishment of a multicast MRB, the UE shall:

1>	establish a PDCP entity and an RLC entity in accordance with mrb-ListMulticast for this multicast MRB included in the MBSMulticastConfiguration message;

1>	configure the MAC layer in accordance with the mtch-SchedulingInfo (if included);

1>	configure the physical layer in accordance with the mbs-SessionInfoListMulticast, searchSpaceMulticastMTCH, and pdsch-ConfigMTCH, applicable for the multicast MRB;

1>	if an SDAP entity with the received mbs-SessionId does not exist:

2>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

2>	indicate the establishment of the user plane resources for the mbs-SessionId to upper layers;

1>	receive DL-SCH for the established multicast MRB using g-RNTI (if not indicated to stop monitoring this G-RNTI) and mtch-SchedulingInfo (if included) in this message for this MBS multicast service.

#### 5.10.3.3	Multicast MRB release

Upon release of a multicast MRB, the UE shall:

1>	release the PDCP entity, RLC entity as well as the related MAC and physical layer configuration;

1>	if the SDAP entity associated with the corresponding mbs-SessionId has no associated MRB:

2>	release the SDAP entity, as specified in TS 37.324 [24] clause 5.1.2;

2>	indicate the release of the user plane resources for the mbs-SessionId to upper layers.

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

### 6.1.1	Introduction

The contents of each RRC message is specified in clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in clause 6.3.

Usage of the text "Network always configures the UE with a value for this field" in the field description indicates that the network has to provide a value for the field in this or in a previous message based on delta configuration (for an optional field with Need M). It does not imply a mandatory presence of the field.

### 6.1.2	Need codes and conditions for optional fields

The need for fields to be present in a message or an abstract type, i.e., the ASN.1 fields that are specified as OPTIONAL in the abstract notation (ASN.1), is specified by means of comment text tags attached to the OPTIONAL statement in the abstract syntax. All comment text tags are available for use in the downlink direction for RRC message and in the sidelink for PC5 RRC message. The meaning of each tag is specified in table 6.1.2-1.

If conditions are used, a conditional presence table is provided for the message or information element specifying the need of the field for each condition case. The table also specifies whether UE maintains or releases the value in case the field is absent. The conditions clarify what the UE may expect regarding the setting of the message by the network for the RRC message or by the peer UE in the sidelink RRC message. Violation of conditions is regarded as invalid network behaviour when transmitting downlink RRC message or invalid UE behavior when transmitting PC5 RRC message, which the UE is not required to cope with. Hence the general error handling defined in 10.4 does not apply in case a field is absent although it is mandatory according to the CondC or CondM condition.

For guidelines on the use of need codes and conditions, see Annex A.6 and A.7.

Table 6.1.2-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

NOTE:	In this version of the specification, the condition tags CondC and CondM are not used.

Any field with Need M or Need N in system information shall be interpreted as Need R.

The need code used within a CondX definition only applies for the case (part of the condition) where it is defined: A condition may have different need codes for different parts of the condition. In particular, the CondX definition may contain the following "otherwise the field is absent" parts:

-	"Otherwise, the field is absent": The field is not relevant or should not be configured when this part of the condition applies. In particular, the UE behaviour is not defined when the field is configured via another part of the condition and is reconfigured to this part of the condition. A need code is not provided when the transition from another part of the condition to this part of the condition is not supported, when the field clearly is a one-shot or there is no difference whether UE maintains or releases the value (e.g., in case the field is mandatory present according to the other part of the condition).

-	"Otherwise, the field is absent, Need R": The field is released if absent when this part of the condition applies. This handles UE behaviour in case the field is configured via another part of the condition and this part of the condition applies (which means that network when transmitting downlink RRC message or peer UE transmitting PC5 RRC message can assume UE releases the field if this part of the condition is valid).

-	"Otherwise, the field is absent, Need M": The UE retains the field if it was already configured when this part of the condition applies. This means the network when transmitting downlink RRC message or the peer UE when transmitting PC5 RRC message cannot release the field, but UE retains the previously configured value.

Use of different Need codes in different parts of a condition should be avoided.

For downlink RRC message and sidelink PC5 RRC messages, the need codes, conditions and ASN.1 defaults specified for a particular (child) field only apply in case the (parent) field including the particular field is present. Thus, if the parent is absent the UE shall not release the field unless the absence of the parent field implies that.

For (parent) fields without need codes in downlink RRC messages or sidelink PC5 RRC message, if the parent field is absent, UE shall follow the need codes of the child fields. Thus, if parent field is absent, the need code of each child field is followed (i.e. Need R child fields are released, Need M child fields are not modified and the actions for Need S child fields depend on the specified conditions of each field). Examples of (parent) fields in downlink RRC messages and sidelink PC5 RRC message without need codes where this rule applies are:

-	nonCriticalExtension fields at the end of a message using empty SEQUENCE extension mechanism,

-	groups of non-critical extensions using double brackets (referred to as extension groups), and

-	non-critical extensions at the end of a message or at the end of a structure, contained in a BIT STRING or OCTET STRING (referred to as parent extension fields).

The handling of need codes as specified in the previous is illustrated by means of an example, as shown in the following ASN.1.

-- /example/ ASN1START

RRCMessage-IEs ::=                SEQUENCE {

field1                            InformationElement1            OPTIONAL,  -- Need M

field2                            InformationElement2            OPTIONAL,  -- Need R

nonCriticalExtension              RRCMessage-v1570-IEs           OPTIONAL

}

RRCMessage-1570-IEs ::=           SEQUENCE {

field3                            InformationElement3            OPTIONAL,  -- Need M

nonCriticalExtension              RRCMessage-v1640-IEs           OPTIONAL

}

RRCMessage-v1640-IEs ::=          SEQUENCE {

field4                            InformationElement4            OPTIONAL,  -- Need R

nonCriticalExtension              SEQUENCE {}                    OPTIONAL

}

InformationElement1 ::=           SEQUENCE {

field10                           InformationElement10           OPTIONAL,  -- Need N

field11                           InformationElement11           OPTIONAL,  -- Need M

field12                           InformationElement12           OPTIONAL,  -- Need R

...,

[[

field13                           InformationElement13           OPTIONAL,  -- Need R

field14                           InformationElement14           OPTIONAL   -- Need M

]]

}

InformationElement2 ::=           SEQUENCE {

field21                           InformationElement11           OPTIONAL,  -- Need M

...

}

-- ASN1STOP

The handling of need codes as specified in the previous implies that:

-	if field1 in RRCMessage-IEs is absent, UE does not modify or take action on any child fields configured within field1 (regardless of their need codes);

-	if field2 in RRCMessage-IEs is absent, UE releases the field2 (and also its child field field21);

-	if field1 or field2 in RRCMessage-IEs is present, UE retains or releases their child fields according to the child field presence conditions;

-	if field1 in RRCMessage-IEs is present but the extension group containing field13 and field14 is absent, the UE releases field13 but does not modify field14;

-	if nonCriticalExtension defined by IE RRCMessage-v1570-IEs is absent, the UE does not modify field3 but releases field4;

### 6.1.3	General rules

In the ASN.1 of this specification, the first bit of a bit string refers to the leftmost bit, unless stated otherwise.

Upon reception of a list not using ToAddModList and ToReleaseList structure, the UE shall delete all entries of the list currently in the UE configuration before applying the received list and shall consider each entry as newly created. This applies also to lists whose size is extended (i.e. with a second list structure in the ASN.1 comprising additional entries), unless otherwise specified. This implies that Need M should not be used for fields in the entries of these lists; if used, UE will handle such fields equivalent to a Need R.

## 6.2	RRC messages

### 6.2.1	General message structure

#### –	NR-RRC-Definitions

This ASN.1 segment is the start of the NR RRC PDU definitions.

-- ASN1START

-- TAG-NR-RRC-DEFINITIONS-START

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-RRC-DEFINITIONS-STOP

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the network to the UE via BCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-BCH-MESSAGE-START

BCCH-BCH-Message ::=            SEQUENCE {

message                         BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::=        CHOICE {

mib                             MIB,

messageClassExtension           SEQUENCE {}

}

-- TAG-BCCH-BCH-MESSAGE-STOP

-- ASN1STOP

#### –	BCCH-DL-SCH-Message

The BCCH-DL-SCH-Message class is the set of RRC messages that may be sent from the network to the UE via DL-SCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-DL-SCH-MESSAGE-START

BCCH-DL-SCH-Message ::=         SEQUENCE {

message                         BCCH-DL-SCH-MessageType

}

BCCH-DL-SCH-MessageType ::=     CHOICE {

c1                              CHOICE {

systemInformation               SystemInformation,

systemInformationBlockType1     SIB1

},

messageClassExtension           SEQUENCE {}

}

-- TAG-BCCH-DL-SCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-CCCH-Message

The DL-CCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the downlink CCCH logical channel.

-- ASN1START

-- TAG-DL-CCCH-MESSAGE-START

DL-CCCH-Message ::=             SEQUENCE {

message                         DL-CCCH-MessageType

}

DL-CCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcReject                       RRCReject,

rrcSetup                        RRCSetup,

spare2                          NULL,

spare1                          NULL

},

messageClassExtension           SEQUENCE {}

}

-- TAG-DL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::=                  SEQUENCE {

message                             DL-DCCH-MessageType

}

DL-DCCH-MessageType ::=             CHOICE {

c1                                  CHOICE {

rrcReconfiguration                  RRCReconfiguration,

rrcResume                           RRCResume,

rrcRelease                          RRCRelease,

rrcReestablishment                  RRCReestablishment,

securityModeCommand                 SecurityModeCommand,

dlInformationTransfer               DLInformationTransfer,

ueCapabilityEnquiry                 UECapabilityEnquiry,

counterCheck                        CounterCheck,

mobilityFromNRCommand               MobilityFromNRCommand,

dlDedicatedMessageSegment-r16       DLDedicatedMessageSegment-r16,

ueInformationRequest-r16            UEInformationRequest-r16,

dlInformationTransferMRDC-r16       DLInformationTransferMRDC-r16,

loggedMeasurementConfiguration-r16  LoggedMeasurementConfiguration-r16,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension   SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

#### –	MCCH-Message

The MCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the MCCH logical channel.

-- ASN1START

-- TAG-MCCH-MESSAGE-START

MCCH-Message-r17 ::= SEQUENCE {

message              MCCH-MessageType-r17

}

MCCH-MessageType-r17 ::= CHOICE {

c1                       CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17,

spare1                            NULL

},

messageClassExtension   SEQUENCE {}

}

-- TAG-MCCH-MESSAGE-STOP

-- ASN1STOP

#### –	MulticastMCCH-Message

The MulticastMCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the multicast MCCH logical channel.

-- ASN1START

-- TAG-MULTICASTMCCH-MESSAGE-START

MulticastMCCH-Message-r18 ::= SEQUENCE {

message                       MulticastMCCH-MessageType-r18

}

MulticastMCCH-MessageType-r18 ::= CHOICE {

c1                                CHOICE {

mbsMulticastConfiguration-r18     MBSMulticastConfiguration-r18,

spare1                            NULL

},

messageClassExtension             SEQUENCE {}

}

-- TAG-MULTICASTMCCH-MESSAGE-STOP

-- ASN1STOP

#### –	PCCH-Message

The PCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the PCCH logical channel.

-- ASN1START

-- TAG-PCCH-PCH-MESSAGE-START

PCCH-Message ::=                SEQUENCE {

message                         PCCH-MessageType

}

PCCH-MessageType ::=            CHOICE {

c1                              CHOICE {

paging                          Paging,

spare1  NULL

},

messageClassExtension       SEQUENCE {}

}

-- TAG-PCCH-PCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH-Message

The UL-CCCH-Message class is the set of 48-bits RRC messages that may be sent from the UE to the Network on the uplink CCCH logical channel.

-- ASN1START

-- TAG-UL-CCCH-MESSAGE-START

UL-CCCH-Message ::=             SEQUENCE {

message                         UL-CCCH-MessageType

}

UL-CCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcSetupRequest                 RRCSetupRequest,

rrcResumeRequest                RRCResumeRequest,

rrcReestablishmentRequest       RRCReestablishmentRequest,

rrcSystemInfoRequest            RRCSystemInfoRequest

},

messageClassExtension           SEQUENCE {}

}

-- TAG-UL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH1-Message

The UL-CCCH1-Message class is the set of 64-bits RRC messages that may be sent from the UE to the Network on the uplink CCCH1 logical channel.

-- ASN1START

-- TAG-UL-CCCH1-MESSAGE-START

UL-CCCH1-Message ::=            SEQUENCE {

message                         UL-CCCH1-MessageType

}

UL-CCCH1-MessageType ::=        CHOICE {

c1                              CHOICE {

rrcResumeRequest1               RRCResumeRequest1,

spare3 NULL,

spare2 NULL,

spare1 NULL

},

messageClassExtension SEQUENCE {}

}

-- TAG-UL-CCCH1-MESSAGE-STOP

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::=             SEQUENCE {

message                         UL-DCCH-MessageType

}

UL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

measurementReport               MeasurementReport,

rrcReconfigurationComplete      RRCReconfigurationComplete,

rrcSetupComplete                RRCSetupComplete,

rrcReestablishmentComplete      RRCReestablishmentComplete,

rrcResumeComplete               RRCResumeComplete,

securityModeComplete            SecurityModeComplete,

securityModeFailure             SecurityModeFailure,

ulInformationTransfer           ULInformationTransfer,

locationMeasurementIndication   LocationMeasurementIndication,

ueCapabilityInformation         UECapabilityInformation,

counterCheckResponse            CounterCheckResponse,

ueAssistanceInformation         UEAssistanceInformation,

failureInformation              FailureInformation,

ulInformationTransferMRDC       ULInformationTransferMRDC,

scgFailureInformation           SCGFailureInformation,

scgFailureInformationEUTRA      SCGFailureInformationEUTRA

},

messageClassExtension           CHOICE {

c2                              CHOICE {

ulDedicatedMessageSegment-r16    ULDedicatedMessageSegment-r16,

dedicatedSIBRequest-r16         DedicatedSIBRequest-r16,

mcgFailureInformation-r16       MCGFailureInformation-r16,

ueInformationResponse-r16       UEInformationResponse-r16,

sidelinkUEInformationNR-r16     SidelinkUEInformationNR-r16,

ulInformationTransferIRAT-r16   ULInformationTransferIRAT-r16,

iabOtherInformation-r16         IABOtherInformation-r16,

mbsInterestIndication-r17       MBSInterestIndication-r17,

uePositioningAssistanceInfo-r17 UEPositioningAssistanceInfo-r17,

measurementReportAppLayer-r17   MeasurementReportAppLayer-r17,

indirectPathFailureInformation-r18 IndirectPathFailureInformation-r18, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtensionFuture-r16    SEQUENCE {}

}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

### 6.2.2	Message definitions

#### –	CounterCheck

The CounterCheck message is used by the network to indicate the current COUNT MSB values associated to each DRB and to request the UE to compare these to its COUNT MSB values and to report the comparison results to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

CounterCheck message

-- ASN1START

-- TAG-COUNTERCHECK-START

CounterCheck ::=                SEQUENCE {

rrc-TransactionIdentifier       RRC-TransactionIdentifier,

criticalExtensions              CHOICE {

counterCheck                    CounterCheck-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

CounterCheck-IEs ::=            SEQUENCE {

drb-CountMSB-InfoList           DRB-CountMSB-InfoList,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            SEQUENCE {}                         OPTIONAL

}

DRB-CountMSB-InfoList ::=       SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info

DRB-CountMSB-Info ::=           SEQUENCE {

drb-Identity                    DRB-Identity,

countMSB-Uplink                 INTEGER(0..33554431),

countMSB-Downlink               INTEGER(0..33554431)

}

-- TAG-COUNTERCHECK-STOP

-- ASN1STOP

CounterCheck-IEs field descriptions

drb-CountMSB-InfoList Indicates the MSBs of the COUNT values of the DRBs.

DRB-CountMSB-Info field descriptions

countMSB-Downlink Indicates the value of 25 MSBs from RX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

countMSB-Uplink Indicates the value of 25 MSBs from TX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

#### –	CounterCheckResponse

The CounterCheckResponse message is used by the UE to respond to a CounterCheck message.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

CounterCheckResponse message

-- ASN1START

-- TAG-COUNTERCHECKRESPONSE-START

CounterCheckResponse ::=        SEQUENCE {

rrc-TransactionIdentifier       RRC-TransactionIdentifier,

criticalExtensions              CHOICE {

counterCheckResponse            CounterCheckResponse-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

CounterCheckResponse-IEs ::=    SEQUENCE {

drb-CountInfoList               DRB-CountInfoList,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            SEQUENCE {}                         OPTIONAL

}

DRB-CountInfoList ::=           SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo

DRB-CountInfo ::=               SEQUENCE {

drb-Identity                    DRB-Identity,

count-Uplink                    INTEGER(0..4294967295),

count-Downlink                  INTEGER(0..4294967295)

}

-- TAG-COUNTERCHECKRESPONSE-STOP

-- ASN1STOP

CounterCheckResponse-IEs field descriptions

drb-CountInfoList Indicates the COUNT values of the DRBs.

DRB-CountInfo field descriptions

count-Downlink Indicates the value of RX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

count-Uplink Indicates the value of TX_NEXT – 1 (specified in TS 38.323 [5]) associated to this DRB.

#### –	DedicatedSIBRequest

The DedicatedSIBRequest message is used to request SIB(s) required by the UE in RRC_CONNECTED as specified in clause 5.2.2.3.5.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

DedicatedSIBRequest message

-- ASN1START

-- TAG-DEDICATEDSIBREQUEST-START

DedicatedSIBRequest-r16 ::=      SEQUENCE {

criticalExtensions               CHOICE {

dedicatedSIBRequest-r16          DedicatedSIBRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

DedicatedSIBRequest-r16-IEs ::=  SEQUENCE {

onDemandSIB-RequestList-r16       SEQUENCE {

requestedSIB-List-r16            SEQUENCE (SIZE (1..maxOnDemandSIB-r16)) OF SIB-ReqInfo-r16                OPTIONAL,

requestedPosSIB-List-r16         SEQUENCE (SIZE (1..maxOnDemandPosSIB-r16)) OF PosSIB-ReqInfo-r16          OPTIONAL

} OPTIONAL,

lateNonCriticalExtension         OCTET STRING                     OPTIONAL,

nonCriticalExtension             DedicatedSIBRequest-v1900-IEs    OPTIONAL

}

DedicatedSIBRequest-v1900-IEs ::=    SEQUENCE {

onDemandPosSIB-RequestList-r19       SEQUENCE {

requestedPeriodicAD-PosSIB-List-r19  SEQUENCE (SIZE (1..maxOnDemandPosSIB-r16)) OF PeriodicAD-PosSIB-ReqInfo-r19      OPTIONAL

}                                                                                                                         OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                                      OPTIONAL

}

SIB-ReqInfo-r16 ::=                   ENUMERATED { sib12, sib13, sib14, sib20-v1700, sib21-v1700, sib23-v1810, sib26-v1900, spare1 }

PosSIB-ReqInfo-r16 ::=       SEQUENCE {

gnss-id-r16                  GNSS-ID-r16                  OPTIONAL,

sbas-id-r16                  SBAS-ID-r16                  OPTIONAL,

posSibType-r16               ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6,

posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4,

posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10,

posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15,

posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20,

posSibType2-21, posSibType2-22, posSibType2-23, posSibType3-1, posSibType4-1,

posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3,..., posSibType1-9-v1710,

posSibType1-10-v1710, posSibType2-24-v1710, posSibType2-25-v1710,

posSibType6-4-v1710, posSibType6-5-v1710, posSibType6-6-v1710, posSibType2-17a-v1770,

posSibType2-18a-v1770, posSibType2-20a-v1770, posSibType1-11-v1800, posSibType1-12-v1800,

posSibType2-26-v1800, posSibType2-27-v1800, posSibType6-7-v1800, posSibType7-1-v1800,

posSibType7-2-v1800, posSibType7-3-v1800, posSibType7-4-v1800 }

}

PeriodicAD-PosSIB-ReqInfo-r19 ::=   SEQUENCE {

gnss-id-r19                         GNSS-ID-r16                                                                         OPTIONAL,

sbas-id-r19                         SBAS-ID-r16                                                                         OPTIONAL,

periodicAD-posSibType-r19           ENUMERATED { posSibType1-10, posSibType1-12, posSibType2-12, posSibType2-13, posSibType2-14,

posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-17a, posSibType2-18,

posSibType2-18a, posSibType2-19, posSibType2-20, posSibType2-20a, posSibType2-21,

posSibType2-22, posSibType2-23, posSibType6-7,... },

posSIB-ReqPeriodicControlParam-r19  PosSIB-ReqPeriodicControlParam-r19,

...

}

PosSIB-ReqPeriodicControlParam-r19 ::= SEQUENCE {

deliveryStatusReq-r19                            ENUMERATED {start, stop},

...

}

-- TAG-DEDICATEDSIBREQUEST-STOP

-- ASN1STOP

DedicatedSIBRequest field descriptions

requestedPeriodicAD-PosSIB-List Contains a list of posSIB(s) the UE may request with periodic control parameter while in RRC_CONNECTED.

requestedPosSIB-List Contains a list of posSIB(s) the UE requests while in RRC_CONNECTED.

requestedSIB-List Contains a list of SIB(s) the UE requests while in RRC_CONNECTED.

PeriodicAD-PosSIB-ReqInfo field descriptions

periodicAD-posSibType Types of posSIB that the UE may request with periodic control parameter while in RRC_CONNECTED.

PosSIB-ReqInfo field descriptions

gnss-id The presence of this field indicates that the request positioning SIB type is for a specific GNSS. Indicates a specific GNSS (see also TS 37.355 [49])

sbas-id The presence of this field indicates that the request positioning SIB type is for a specific SBAS. Indicates a specific SBAS (see also TS 37.355 [49]). If the UE includes this field it shall set gnss-id to sbas.

PosSIB-ReqPeriodicControlParam field descriptions

deliveryStatusReq This field specifies whether the periodic delivery of the posSIB is still needed or not. Value start requests the periodic delivery of the posSIB to start and value stop requests the periodic delivery of the posSIB to stop.

#### –	DLDedicatedMessageSegment

The DLDedicatedMessageSegment message is used to transfer one segment of the RRCResume or RRCReconfiguration messages.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLDedicatedMessageSegment message

-- ASN1START

-- TAG-DLDEDICATEDMESSAGESEGMENT-START

DLDedicatedMessageSegment-r16 ::=       SEQUENCE {

criticalExtensions                      CHOICE {

dlDedicatedMessageSegment-r16           DLDedicatedMessageSegment-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

DLDedicatedMessageSegment-r16-IEs ::=   SEQUENCE {

segmentNumber-r16                       INTEGER(0..4),

rrc-MessageSegmentContainer-r16         OCTET STRING,

rrc-MessageSegmentType-r16              ENUMERATED {notLastSegment, lastSegment},

lateNonCriticalExtension                OCTET STRING                                  OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                   OPTIONAL

}

-- TAG-DLDEDICATEDMESSAGESEGMENT-STOP

-- ASN1STOP

DLDedicatedMessageSegment field descriptions

segmentNumber Identifies the sequence number of a segment within the encoded DL DCCH message. The network transmits the segments with continuously increasing segmentNumber order so that the UE's RRC layer may expect to obtain them from lower layers in the correct order. Hence, the UE is not required to perform segment re-ordering on RRC level.

rrc-MessageSegmentContainer Includes a segment of the encoded DL DCCH message. The size of the included segment in this container should be small enough so the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit.

rrc-MessageSegmentType Indicates whether the included DL DCCH message segment is the last segment of the message or not.

#### –	DLInformationTransfer

The DLInformationTransfer message is used for the downlink transfer of NAS dedicated information, timing information for the 5G internal system clock, or IAB-DU specific F1-C related information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the network does not send this message until SRB2 is resumed. If only dedicatedInfoF1c is included, SRB2 is used.

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLInformationTransfer message

-- ASN1START

-- TAG-DLINFORMATIONTRANSFER-START

DLInformationTransfer ::=           SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

dlInformationTransfer           DLInformationTransfer-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

DLInformationTransfer-IEs ::=       SEQUENCE {

dedicatedNAS-Message                DedicatedNAS-Message                OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                DLInformationTransfer-v1610-IEs     OPTIONAL

}

DLInformationTransfer-v1610-IEs ::= SEQUENCE {

referenceTimeInfo-r16               ReferenceTimeInfo-r16               OPTIONAL,   -- Need N

nonCriticalExtension                DLInformationTransfer-v1700-IEs     OPTIONAL

}

DLInformationTransfer-v1700-IEs ::= SEQUENCE {

dedicatedInfoF1c-r17                DedicatedInfoF1c-r17                OPTIONAL,   -- Need N

rxTxTimeDiff-gNB-r17                RxTxTimeDiff-r17                    OPTIONAL,   -- Need N

ta-PDC-r17                          ENUMERATED {activate,deactivate}    OPTIONAL,   -- Need N

sib9Fallback-r17                    ENUMERATED {true}                   OPTIONAL,   -- Need N

nonCriticalExtension                DLInformationTransfer-v1800-IEs     OPTIONAL

}

DLInformationTransfer-v1800-IEs ::= SEQUENCE {

eventID-TSS-r18                     INTEGER(0..63)                      OPTIONAL,   -- Cond ClockQualityDetailsLevel

clockQualityDetailsLevel-r18        CHOICE {

clockQualityMetrics-r18             ClockQualityMetrics-r18,

clockQualityAcceptanceStatus-r18    ENUMERATED {acceptable, notAcceptable}

}                                                                       OPTIONAL,   -- Need N

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-DLINFORMATIONTRANSFER-STOP

-- ASN1STOP

DLInformationTransfer field descriptions

clockQualityDetailsLevel This field indicates the clock quality reporting control information as defined in TS 23.501 [32].

eventID-TSS This field indicates the status of the 5G access stratum time distribution parameter Clock Quality Reporting Control Information as defined in TS 23.501 [32].

rxTxTimeDiff-gNB Indicates the Rx-Tx time difference measurement at the gNB (see clause 5.2.3, TS 38.215 [9]). Upon receiving this field, the UE calculates the propagation delay based on the RTT-based PDC mechanism method as described in TS 38.300 [2]. The network does not configure this field, if the UE is configured with ta-PDC with value activate.

sib9Fallback Indicates that the UE fallbacks to receive referenceTimeInfo in SIB9.

ta-PDC Indicates whether the UE-side TA-based propagation delay compensation (PDC) is activated or de-activated. The network does not configure this field with activate, if the field rxTxTimeDiff-gNB is configured.

| Conditional Presence | Explanation |
| --- | --- |
| ClockQualityDetailsLevel | Field eventID-TSS is mandatory present if clockQualityDetailsLevel is present. Otherwise, the field is optionally present, Need M. |

#### –	DLInformationTransferMRDC

The DLInformationTransferMRDC message is used for the downlink transfer of RRC messages during fast MCG link recovery.

Signalling radio bearer: SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLInformationTransferMRDC message

-- ASN1START

-- TAG-DLINFORMATIONTRANSFERMRDC-START

DLInformationTransferMRDC-r16 ::=       SEQUENCE {

criticalExtensions                      CHOICE {

c1                                      CHOICE {

dlInformationTransferMRDC-r16           DLInformationTransferMRDC-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture                SEQUENCE {}

}

}

DLInformationTransferMRDC-r16-IEs::=    SEQUENCE {

dl-DCCH-MessageNR-r16                   OCTET STRING             OPTIONAL, -- Need N

dl-DCCH-MessageEUTRA-r16                OCTET STRING             OPTIONAL, -- Need N

lateNonCriticalExtension                OCTET STRING             OPTIONAL,

nonCriticalExtension                    SEQUENCE {}              OPTIONAL

}

-- TAG-DLINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

DLInformationTransferMRDC field descriptions

dl-DCCH-MessageNR Includes the DL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRCReconfiguration, RRCRelease, and MobilityFromNRCommand messages.

dl-DCCH-MessageEUTRA Includes the DL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRCConnectionReconfiguration, RRCConnectionRelease, and MobilityFromEUTRACommand messages as specified in TS 36.331 [10].

#### –	FailureInformation

The FailureInformation message is used to inform the network about a failure detected by the UE.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

FailureInformation message

-- ASN1START

-- TAG-FAILUREINFORMATION-START

FailureInformation ::=         SEQUENCE {

criticalExtensions             CHOICE {

failureInformation             FailureInformation-IEs,

criticalExtensionsFuture       SEQUENCE {}

}

}

FailureInformation-IEs ::=     SEQUENCE {

failureInfoRLC-Bearer          FailureInfoRLC-Bearer        OPTIONAL,

lateNonCriticalExtension       OCTET STRING                 OPTIONAL,

nonCriticalExtension           FailureInformation-v1610-IEs OPTIONAL

}

FailureInfoRLC-Bearer ::=      SEQUENCE {

cellGroupId                    CellGroupId,

logicalChannelIdentity         LogicalChannelIdentity,

failureType                    ENUMERATED {rlc-failure, spare3, spare2, spare1}

}

FailureInformation-v1610-IEs ::= SEQUENCE {

failureInfoDAPS-r16              FailureInfoDAPS-r16        OPTIONAL,

nonCriticalExtension             SEQUENCE {}                OPTIONAL

}

FailureInfoDAPS-r16 ::=          SEQUENCE {

failureType-r16                  ENUMERATED {daps-failure, spare3, spare2, spare1}

}

-- TAG-FAILUREINFORMATION-STOP

-- ASN1STOP

#### –	IABOtherInformation

The IABOtherInformation message is used by IAB-MT to request the network to allocate IP addresses for the collocated IAB-DU or inform the network about IP addresses allocated to the collocated IAB-DU.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: IAB-MT to Network

IABOtherInformation message

-- ASN1START

-- TAG-IABOTHERINFORMATION-START

IABOtherInformation-r16 ::=     SEQUENCE {

dummy                           RRC-TransactionIdentifier,

criticalExtensions              CHOICE {

iabOtherInformation-r16         IABOtherInformation-r16-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

IABOtherInformation-r16-IEs ::=         SEQUENCE {

ip-InfoType-r16                         CHOICE {

iab-IP-Request-r16                      SEQUENCE {

iab-IPv4-AddressNumReq-r16              IAB-IP-AddressNumReq-r16                OPTIONAL,

iab-IPv6-AddressReq-r16                 CHOICE {

iab-IPv6-AddressNumReq-r16              IAB-IP-AddressNumReq-r16,

iab-IPv6-AddressPrefixReq-r16           IAB-IP-AddressPrefixReq-r16,

...

}                                                                               OPTIONAL

},

iab-IP-Report-r16               SEQUENCE {

iab-IPv4-AddressReport-r16      IAB-IP-AddressAndTraffic-r16                    OPTIONAL,

iab-IPv6-Report-r16             CHOICE {

iab-IPv6-AddressReport-r16      IAB-IP-AddressAndTraffic-r16,

iab-IPv6-PrefixReport-r16       IAB-IP-PrefixAndTraffic-r16,

...

}                                                                               OPTIONAL

},

...

},

lateNonCriticalExtension        OCTET STRING                                            OPTIONAL,

nonCriticalExtension            SEQUENCE {}                                             OPTIONAL

}

IAB-IP-AddressNumReq-r16 ::=    SEQUENCE {

all-Traffic-NumReq-r16          INTEGER (1..8)                                  OPTIONAL,

f1-C-Traffic-NumReq-r16         INTEGER (1..8)                                  OPTIONAL,

f1-U-Traffic-NumReq-r16         INTEGER (1..8)                                  OPTIONAL,

non-F1-Traffic-NumReq-r16       INTEGER (1..8)                                  OPTIONAL,

...

}

IAB-IP-AddressPrefixReq-r16 ::= SEQUENCE {

all-Traffic-PrefixReq-r16       ENUMERATED {true}                               OPTIONAL,

f1-C-Traffic-PrefixReq-r16      ENUMERATED {true}                               OPTIONAL,

f1-U-Traffic-PrefixReq-r16      ENUMERATED {true}                               OPTIONAL,

non-F1-Traffic-PrefixReq-r16    ENUMERATED {true}                               OPTIONAL,

...

}

IAB-IP-AddressAndTraffic-r16 ::= SEQUENCE {

all-Traffic-IAB-IP-Address-r16  SEQUENCE (SIZE(1..8)) OF IAB-IP-Address-r16     OPTIONAL,

f1-C-Traffic-IP-Address-r16     SEQUENCE (SIZE(1..8)) OF IAB-IP-Address-r16     OPTIONAL,

f1-U-Traffic-IP-Address-r16     SEQUENCE (SIZE(1..8)) OF IAB-IP-Address-r16     OPTIONAL,

non-F1-Traffic-IP-Address-r16   SEQUENCE (SIZE(1..8)) OF IAB-IP-Address-r16     OPTIONAL

}

IAB-IP-PrefixAndTraffic-r16 ::= SEQUENCE {

all-Traffic-IAB-IP-Address-r16  IAB-IP-Address-r16                              OPTIONAL,

f1-C-Traffic-IP-Address-r16     IAB-IP-Address-r16                              OPTIONAL,

f1-U-Traffic-IP-Address-r16     IAB-IP-Address-r16                              OPTIONAL,

non-F1-Traffic-IP-Address-r16   IAB-IP-Address-r16                              OPTIONAL

}

-- TAG-IABOTHERINFORMATION-STOP

-- ASN1STOP

IABOtherInformation-IEs field descriptions

dummy This field is not used in the specification and network ignores the received value.

iab-IPv4-AddressNumReq This field is used to request the numbers of IPv4 address per specific usage. The specific usages include F1-C traffic, F1-U traffic, non-F1 traffic and all traffic.

iab-IPv4-AddressReport This field is used to report the IPv4 address per specific usage assigned by OAM for IAB-DU. The specific usages include F1-C traffic, F1-U traffic, non-F1 traffic and all traffic.

iab-IPv6-AddressNumReq This field is used to request the numbers of IPv6 address per specific usage. The specific usages include F1-C traffic, F1-U traffic, non-F1 traffic and all traffic.

iab-IPv6-AddressPrefixReq This field is used to request the prefix of IPv6 address per specific usage. The specific usages include F1-C traffic, F1-U traffic, non-F1 traffic and all traffic.

iab-IPv6-AddressReport This field is used to report the IPv6 address per specific usage assigned by OAM for IAB-DU. The specific usages include F1-C traffic, F1-U traffic, non-F1 traffic and all traffic.

iab-IPv6-PrefixReport This field is used to report the prefix of IPv6 address per specific usage assigned by OAM for IAB-DU. The specific usages include F1-C traffic, F1-U traffic, non-F1 traffic and all traffic.

IAB-IP-AddressNumReq-IEs field descriptions

all-Traffic-NumReq This field is used to request the numbers of IP address for all traffic.

f1-C-Traffic-NumReq This field is used to request the numbers of IP address for F1-C traffic.

f1-U-Traffic-NumReq This field is used to request the numbers of IP address for F1-U traffic.

non-F1-Traffic-NumReq This field is used to request the numbers of IP address for non-F1 traffic.

IAB-IP-AddressPrefixReq-IEs field descriptions

all-Traffic-PrefixReq This field is used to request the IPv6 address prefix for all traffic. The length of allocated IPv6 prefix is fixed to 64.

f1-C-Traffic-PrefixReq This field is used to request the IPv6 address prefix for F1-C traffic. The length of allocated IPv6 prefix is fixed to 64.

f1-U-Traffic-PrefixReq This field is used to request the IPv6 address prefix for F1-U traffic. The length of allocated IPv6 prefix is fixed to 64.

non-F1-Traffic-PrefixReq This field is used to request the IPv6 address prefix for non-F1 traffic. The length of allocated IPv6 prefix is fixed to 64.

IAB-IP-AddressAndTraffic-IEs field descriptions

all-Traffic-IAB-IP-Address This field is used to report to IAB-donor-CU the IP address(es) or IPv6 address prefix for all traffic.

f1-C-Traffic-IP-Address This field is used to report to IAB-donor-CU the IP address(es) or IPv6 address prefix for F1-C traffic.

f1-U-Traffic-IP-Address This field is used to report to IAB-donor-CU the IP address(es) or IPv6 address prefix for F1-U traffic.

non-F1-Traffic-IP-Address This field is used to report to IAB-donor-CU the IP address(es) or IPv6 address prefix for non-F1 traffic.

#### –	IndirectPathFailureInformation

The IndirectPathFailureInformation message is used to provide information regarding indirect path failure detected by the MP remote UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

IndirectPathFailureInformation message

-- ASN1START

-- TAG-INDIRECTPATHFAILUREINFORMATION-START

IndirectPathFailureInformation-r18 ::=        SEQUENCE {

criticalExtensions                            CHOICE {

indirectPathFailureInformation-r18            IndirectPathFailureInformation-r18-IEs,

criticalExtensionsFuture                      SEQUENCE {}

}

}

IndirectPathFailureInformation-r18-IEs ::=    SEQUENCE {

failureReportIndirectPath-r18                 FailureReportIndirectPath-r18                    OPTIONAL,

lateNonCriticalExtension                      OCTET STRING                                     OPTIONAL,

nonCriticalExtension                          SEQUENCE {}                                      OPTIONAL

}

FailureReportIndirectPath-r18 ::=             SEQUENCE {

failureTypeIndirectPath-r18                   ENUMERATED {t421-Expiry,sl-Failure,n3c-Failure, relayUE-Uu-RLF,

relayUE-Uu-RRC-Failure,

indirectPathAddChangeFailure, sl-PC5-Release, spare1} OPTIONAL,

sl-MeasResultServingRelay-r18                 OCTET STRING                                     OPTIONAL,

-- Contains PC5 SL-MeasResultRelay-r17

sl-MeasResultsCandRelay-r18                   OCTET STRING                                     OPTIONAL,

n3c-RelayUE-InfoList-r18                      SEQUENCE (SIZE (0..8)) OF N3C-RelayUE-Info-r18   OPTIONAL,

...

}

-- TAG-INDIRECTPATHFAILUREINFORMATION-STOP

-- ASN1STOP

IndirectPathFailureInformation field descriptions

failureTypeIndirectPath The field indicates the failure type of the indirect path failure.

n3c-RelayUE-InfoList Information of available N3C relay UE(s).

sl-MeasResultsCandRelay Measurement result(s) of candiate L2 U2N relay UE(s).

sl-MeasResultServingRelay Measurement result of serving L2 U2N relay UE.

#### –	LocationMeasurementIndication

The LocationMeasurementIndication message is used to indicate that the UE is going to either start or stop location related measurement which requires measurement gaps.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

LocationMeasurementIndication message

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINDICATION-START

LocationMeasurementIndication ::=           SEQUENCE {

criticalExtensions                          CHOICE {

locationMeasurementIndication               LocationMeasurementIndication-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

LocationMeasurementIndication-IEs ::=       SEQUENCE {

measurementIndication                       SetupRelease {LocationMeasurementInfo},

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        SEQUENCE{}                                                              OPTIONAL

}

-- TAG-LOCATIONMEASUREMENTINDICATION-STOP

-- ASN1STOP

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        LoggedMeasurementConfiguration-v1700-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1700-IEs ::= SEQUENCE {

sigLoggedMeasType-r17                       ENUMERATED {true}                        OPTIONAL, -- Need R

earlyMeasIndication-r17                     ENUMERATED {true}                        OPTIONAL, -- Need R

areaConfiguration-r17                       AreaConfiguration-r17                    OPTIONAL,  --Need R

nonCriticalExtension                        LoggedMeasurementConfiguration-v1800-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1800-IEs ::= SEQUENCE {

areaConfiguration-v1800                     AreaConfiguration-v1800                  OPTIONAL,  --Need R

nonCriticalExtension                        LoggedMeasurementConfiguration-v1900-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1900-IEs ::= SEQUENCE {

areaConfigurationNTN-List-r19                AreaConfigurationNTN-List-r19           OPTIONAL,  -- Cond logAreaNTN

nonCriticalExtension                         SEQUENCE {}                             OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

AreaConfigurationNTN-List-r19 ::= SEQUENCE (SIZE (1..maxNrofAreaNTN-r19)) OF AreaConfigurationNTN-r19

AreaConfigurationNTN-r19 ::=    SEQUENCE {

areaCoordinates-r19             CHOICE {

polygonArea                     OCTET STRING,

circleArea                      SEQUENCE {

referenceLocation-r19           ReferenceLocation-r17,

distanceRadius-r19              INTEGER (0..65535)

}

}

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting any of the included cell identities, the included tracking area codes/ frequencies, the included PNI-NPN identities or the SNPN identities. If areaConfiguration-r17 is present, the UE shall ignore areaConfiguration-r16. The areaConfiguration-v1800 is a non-critical extension of areaConfiguration-r17. See NOTE 1.

areaConfigurationNTN-List Used to restrict the geographic area in which the UE performs measurement logging for NTN deployment. The network does not configure areaConfiguration together with areaConfigurationNTN-List.

earlyMeasIndication If included, the field indicates the UE is allowed to log measurements on early measurement related frequencies in logged measurements.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs. The network does not include this field when the UE is configured with MDT configuration in SNPN access mode.

polygonArea Parameter type Polygon defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

sigLoggedMeasType If included, the field indicates a signalling based logged measurement configuration (See TS 37.320 [61]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT configuration or Event Triggerd MDT configuration.

| Conditional Presence | Explanation |
| --- | --- |
| logAreaNTN | The field is optionally present, Need R, for logging of measurements in NTN deployments, otherwise it is absent. |

NOTE 1:	The UE should perform measurement logging based on the following area configuration limitations:

-	If the areaConfiguration-r16/areaConfiguration-r17 is present, and the cag-ConfigList is absent, the UE should perform logging in both PN and PNI-NPN based on areaConfiguration-r16/areaConfiguration-r17, if any;

-	If the areaConfiguration-r17 and the cag-ConfigList are present simultaneously, the UE should perform logging in PN within the areaConfig-r16/areaConfig-r17 and perform logging in PNI-NPN within cag-ConfigList;

-	If the snpn-ConfigList is present, the UE should perform logging only in SNPN based on snpn-ConfigList. The snpn-ConfigList should not be configured together with PN or PNI-NPN area configurations.

#### –	MBSBroadcastConfiguration

The MBSBroadcastConfiguration message contains the control information applicable for MBS broadcast services transmitted via broadcast MRB.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: MCCH

Direction: Network to UE

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-v1900-IEs                                  OPTIONAL

}

MBSBroadcastConfiguration-v1900-IEs ::= SEQUENCE {

mbs-SessionInfoList-v1900             MBS-SessionInfoList-v1900                                            OPTIONAL,   -- Need R

nonCriticalExtension                  SEQUENCE {}                                                          OPTIONAL

}

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

MBSBroadcastConfiguration field descriptions

pdsch-ConfigMTCH Provides parameters for acquiring the PDSCH for MTCH. When this field is absent, the UE shall use parameters in pdsch-ConfigMCCH to acquire the PDSCH for MTCH.

mbs-SessionInfoList Provides the configuration of each MBS session provided by MBS broadcast in the current cell. If mbs-SessionInfoList-v1900 is present, it includes the same number of entries listed in the same order as in mbs-SessionInfoList-r17. The first element in mbs-SessionInfoList-v1900 corresponds to the first MBS broadcast service in mbs-SessionInfoList-r17.

mbs-NeighbourCellList List of neighbour cells providing one or more MBS broadcast services via broadcast MRB that are provided by the current cell. This field is used by the UE together with mtch-NeighbourCell field signalled for each MBS session in the corresponding MBS-SessionInfo. When an empty mbs-NeighbourCellList list is signalled, the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell. When a non-empty mbs-NeighbourCellList is signalled, the current serving cell does not provide information about MBS broadcast services of a neighbour cell that is not included in mbs-NeighbourCellList, i.e., the UE cannot determine the presence or absence of an MBS service of a neighbour cell that is absent. When the field mbs-NeighbourCellList is absent, the current serving cell does not provide information about MBS broadcast services in the neighbouring cells, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field.

#### –	MBSInterestIndication

The MBSInterestIndication message is used to inform network that the UE is receiving/ interested to receive or no longer receiving/ interested to receive MBS broadcast service(s) via a broadcast MRB.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MBSInterestIndication message

-- ASN1START

-- TAG-MBSINTERESTINDICATION-START

MBSInterestIndication-r17 ::= SEQUENCE {

criticalExtensions            CHOICE {

mbsInterestIndication-r17     MBSInterestIndication-r17-IEs,

criticalExtensionsFuture      SEQUENCE {}

}

}

MBSInterestIndication-r17-IEs ::= SEQUENCE {

mbs-FreqList-r17                  CarrierFreqListMBS-r17              OPTIONAL,

mbs-Priority-r17                  ENUMERATED {true}                   OPTIONAL,

mbs-ServiceList-r17               MBS-ServiceList-r17                 OPTIONAL,

lateNonCriticalExtension          OCTET STRING                        OPTIONAL,

nonCriticalExtension              MBSInterestIndication-v1800         OPTIONAL

}

MBSInterestIndication-v1800 ::= SEQUENCE {

mbs-NonServingInfoList-r18        MBS-NonServingInfoList-r18          OPTIONAL,

nonCriticalExtension              SEQUENCE {}                         OPTIONAL

}

-- TAG-MBSINTERESTINDICATION-STOP

-- ASN1STOP

MBSInterestIndication field descriptions

mbs-FreqList List of MBS frequencies on which the UE is receiving or interested to receive MBS broadcast service via a broadcast MRB.

mbs-NonServingInfoList Indicates information for MBS broadcast reception on the non-serving cell.

mbs-Priority Indicates whether the UE prioritises MBS broadcast reception above unicast and MBS multicast reception. The field is present (i.e. value true), if the UE prioritises reception of broadcast services, on frequencies indicated in mbs-FreqList, above a reception of any of the unicast bearers and multicast MRBs. Otherwise the field is absent.

mbs-ServiceList List of MBS broadcast services which the UE is receiving or interested to receive.

#### –	MBSMulticastConfiguration

The MBSMulticastConfiguration message contains the control information applicable for MBS multicast services transmitted via multicast MRBs for RRC_INACTIVE UEs.

Signalling radio bearer: N/A

RLC-SAP: UM

Logical channel: multicast MCCH

Direction: Network to UE

MBSMulticastConfiguration message

-- ASN1START

-- TAG-MBSMULTICASTCONFIGURATION-START

MBSMulticastConfiguration-r18 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsMulticastConfiguration-r18     MBSMulticastConfiguration-r18-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSMulticastConfiguration-r18-IEs ::= SEQUENCE {

mbs-SessionInfoListMulticast-r18      MBS-SessionInfoListMulticast-r18                                     OPTIONAL,   -- Need R

mbs-NeighbourCellList-r18             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r18                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r18                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r18        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

thresholdMBS-List-r18                 SEQUENCE (SIZE (1..maxNrofThresholdMBS-r18)) OF ThresholdMBS-r18     OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                                          OPTIONAL

}

ThresholdMBS-r18 ::=                  SEQUENCE {

rsrp-r18                              RSRP-Range                                                           OPTIONAL,   -- Need R

rsrq-r18                              RSRQ-Range                                                           OPTIONAL    -- Need R

}

-- TAG-MBSMULTICASTCONFIGURATION-STOP

-- ASN1STOP

MBSMulticastConfiguration field descriptions

mbs-NeighbourCellList List of neighbour cells providing one or more MBS multicast services for RRC_INACTIVE that are provided by the current cell. This field is used by the UE together with mtch-NeighbourCell field signalled for each MBS session in the corresponding MBS-SessionInfo. When an empty mbs-NeighbourCellList list is signalled, the UE shall assume that MBS multicast services signalled in mbs-SessionInfoListMulticast in the MBSMulticastConfiguration message are not provided in any neighbour cell. When a non-empty mbs-NeighbourCellList is signalled, the current serving cell does not provide information about MBS multicast services of a neighbour cell that is not included in mbs-NeighbourCellList, i.e., the UE cannot determine the presence or absence of an MBS multicast service of a neighbour cell that is absent. When the field mbs-NeighbourCellList is absent, the current serving cell does not provide information about MBS multicast services in the neighbouring cells, i.e. the UE cannot determine the presence or absence of an MBS multicast service in neighbouring cells based on the absence of this field.

mbs-SessionInfoListMulticast Provides the configuration of MBS multicast session(s) in the current cell.

pdsch-ConfigMTCH Provides parameters for acquiring the PDSCH for multicast MTCH. When this field is absent, the UE shall use parameters in pdsch-ConfigMCCH in SIB24 to acquire the PDSCH for multicast MTCH.

thresholdMBS-List List of reception quality thresholds for RRC connection resume for a UE receiving multicast in RRC_INACTIVE.

#### –	MCGFailureInformation

The MCGFailureInformation message is used to provide information regarding NR MCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MCGFailureInformation message

-- ASN1START

-- TAG-MCGFAILUREINFORMATION-START

MCGFailureInformation-r16 ::=    SEQUENCE {

criticalExtensions               CHOICE {

mcgFailureInformation-r16        MCGFailureInformation-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

MCGFailureInformation-r16-IEs ::= SEQUENCE {

failureReportMCG-r16              FailureReportMCG-r16                             OPTIONAL,

lateNonCriticalExtension          OCTET STRING                                     OPTIONAL,

nonCriticalExtension              SEQUENCE {}                                      OPTIONAL

}

FailureReportMCG-r16 ::=          SEQUENCE {

failureType-r16                   ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

t312-Expiry-r16, lbt-Failure-r16, beamFailureRecoveryFailure-r16,

bh-RLF-r16, spare1}                                                                            OPTIONAL,

measResultFreqList-r16            MeasResultList2NR                                                                     OPTIONAL,

measResultFreqListEUTRA-r16       MeasResultList2EUTRA                                                                  OPTIONAL,

measResultSCG-r16                 OCTET STRING (CONTAINING MeasResultSCG-Failure)                                       OPTIONAL,

measResultSCG-EUTRA-r16           OCTET STRING                                                                          OPTIONAL,

measResultFreqListUTRA-FDD-r16    MeasResultList2UTRA                                                                   OPTIONAL,

...

}

MeasResultList2UTRA ::=    SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2UTRA-FDD-r16

MeasResult2UTRA-FDD-r16 ::=       SEQUENCE {

carrierFreq-r16                   ARFCN-ValueUTRA-FDD-r16,

measResultNeighCellList-r16       MeasResultListUTRA-FDD-r16

}

MeasResultList2EUTRA ::=          SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16

-- TAG-MCGFAILUREINFORMATION-STOP

-- ASN1STOP

MCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by the measConfig associated with the MCG.

measResultFreqListEUTRA The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig associated with the MCG.

measResultFreqListUTRA-FDD The field contains available results of measurements on UTRA FDD frequencies the UE is configured to measure by measConfig associated with the MCG.

measResultSCG The field contains the MeasResultSCG-Failure IE which includes available measurement results on NR frequencies the UE is configured to measure by the measConfig associated with the SCG.

measResultSCG-EUTRA The field contains the EUTRA MeasResultSCG-FailureMRDC IE which includes available results of measurements on E-UTRA frequencies the UE is configured to measure by the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10].

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReport message

-- ASN1START

-- TAG-MEASUREMENTREPORT-START

MeasurementReport ::=               SEQUENCE {

criticalExtensions                  CHOICE {

measurementReport                   MeasurementReport-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

MeasurementReport-IEs ::=           SEQUENCE {

measResults                         MeasResults,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                              OPTIONAL

}

-- TAG-MEASUREMENTREPORT-STOP

-- ASN1STOP

#### –	MeasurementReportAppLayer

The MeasurementReportAppLayer message is used for sending application layer measurement report.

Signalling radio bearer: SRB4, SRB5

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReportAppLayer message

-- ASN1START

-- TAG-MEASUREMENTREPORTAPPLAYER-START

MeasurementReportAppLayer-r17 ::=     SEQUENCE {

criticalExtensions                    CHOICE {

measurementReportAppLayer-r17     MeasurementReportAppLayer-r17-IEs,

criticalExtensionsFuture              SEQUENCE {}

}

}

MeasurementReportAppLayer-r17-IEs ::=   SEQUENCE {

measurementReportAppLayerList-r17       MeasurementReportAppLayerList-r17,

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    MeasurementReportAppLayer-v1800-IEs                                    OPTIONAL

}

MeasurementReportAppLayer-v1800-IEs ::= SEQUENCE {

measurementReportAppLayerList-v1800     MeasurementReportAppLayerList-v1800                                    OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                             OPTIONAL

}

MeasurementReportAppLayerList-r17 ::= SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasReportAppLayer-r17

MeasurementReportAppLayerList-v1800 ::= SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasReportAppLayer-v1800

MeasReportAppLayer-r17 ::=            SEQUENCE {

measConfigAppLayerId-r17              MeasConfigAppLayerId-r17,

measReportAppLayerContainer-r17       OCTET STRING                                                             OPTIONAL,

appLayerSessionStatus-r17             ENUMERATED {start, stop}                                                 OPTIONAL,

ran-VisibleMeasurements-r17           RAN-VisibleMeasurements-r17                                              OPTIONAL

}

MeasReportAppLayer-v1800 ::=          SEQUENCE {

appLayerIdleInactiveConfig-r18        AppLayerIdleInactiveConfig-r18                                           OPTIONAL,

measReportAppLayerContainerList-r18   SEQUENCE (SIZE (1..maxNrofAppLayerReports-r18)) OF OCTET STRING          OPTIONAL,

...

}

RAN-VisibleMeasurements-r17 ::=       SEQUENCE {

appLayerBufferLevelList-r17           SEQUENCE (SIZE (1..8)) OF AppLayerBufferLevel-r17                        OPTIONAL,

playoutDelayForMediaStartup-r17       INTEGER (0..30000)                                                       OPTIONAL,

pdu-SessionIdList-r17                 SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionID            OPTIONAL,

...,

[[

pdu-SessionIdListExt-v1800            SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF QFI-List-r18             OPTIONAL

]]

}

AppLayerBufferLevel-r17 ::= INTEGER (0..30000)

QFI-List-r18 ::= SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI

-- TAG-MEASUREMENTREPORTAPPLAYER-STOP

-- ASN1STOP

MeasurementReportAppLayer field descriptions

measurementReportAppLayerList The field contains a list of application layer measurement reports. If measurementReportAppLayerList-v1800 is present, it contains the same number of entries, listed in the same order as in measurementReportAppLayerList-r17.

MeasReportAppLayer field descriptions

appLayerSessionStatus Indicates that an application layer measurement session in the application layer starts or ends. For application layer measurements applicable to RRC_IDLE or RRC_INACTIVE, the UE transmits appLayerSessionStatus upon transfer to RRC_CONNECTED if transmissionOfSessionStartStop is set to true for the application layer measurement configuration.

measReportAppLayerContainer The field contains the application layer measurement report container, see Annex L (normative) in TS 26.247 [68], clause 16.5 in TS 26.114 [69] and clause 9.4 in TS 26.118 [70].

measReportAppLayerContainerList The field contains a list of application layer measurement report containers for each measConfigAppLayerId, see Annex L (normative) in TS 26.247 [68], clause 16.5 in TS 26.114 [69] and clause 9.4 in TS 26.118 [70].

ran-VisibleMeasurements The field contains the RAN visible application layer measurement report.

RAN-VisibleMeasurements field descriptions

appLayerBufferLevelList The field indicates a list of application layer buffer levels, and each AppLayerBufferLevel indicates the application layer buffer level in ms. Value 0 corresponds to 0 ms, value 1 corresponds to 10 ms, value 2 corresponds to 20 ms and so on. If the buffer level is larger than the maximum value of 30000 (5 minutes), the UE reports 30000.

playoutDelayForMediaStartup Indicates the application layer playout delay for media start-up in ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1 ms, value 2 corresponds to 2 ms and so on. If the playout delay for media start-up is larger than the maximum value of 30000 ms, the UE reports 30000.

pdu-SessionIdList List of PDU session identities and QoS flow identities per PDU session associated with the application data flows subject to the RAN visible application layer measurements. If pdu-SessionIdListExt-v1800 is present, it contains the same number of entries, listed in the same order as in pdu-SessionIdList-r17.

#### –	MIB

The MIB includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: Network to UE

MIB

-- ASN1START

-- TAG-MIB-START

MIB ::=                             SEQUENCE {

systemFrameNumber                   BIT STRING (SIZE (6)),

subCarrierSpacingCommon             ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset                INTEGER (0..15),

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1,

cellBarred                          ENUMERATED {barred, notBarred},

intraFreqReselection                ENUMERATED {allowed, notAllowed},

spare                               BIT STRING (SIZE (1))

}

-- TAG-MIB-STOP

-- ASN1STOP

MIB field descriptions

cellBarred Value barred means that the cell is barred, as defined in TS 38.304 [20]. This field is ignored by IAB-MT and NCR-MT. This field is ignored for connectivity to NTN or ATG.

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.2) and uplink (see TS 38.211 [16], clause 6.4.1.1.3).

intraFreqReselection Controls cell selection/reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 38.304 [20]. This field is ignored by IAB-MT, NCR-MT and (e)RedCap UE.

pdcch-ConfigSIB1 Determines a common ControlResourceSet (CORESET), a common search space and necessary PDCCH parameters. If the field ssb-SubcarrierOffset indicates that SIB1 is absent, the field pdcch-ConfigSIB1 indicates the frequency positions where the UE may find SS/PBCH block with SIB1 or the frequency range where the network does not provide SS/PBCH block with SIB1 (see TS 38.213 [13], clause 13).

ssb-SubcarrierOffset Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See TS 38.211 [16], clause 7.4.3.1). For operation with shared spectrum channel access in FR1 (see 37.213 [48]), this field corresponds to kSSB, and kSSB is obtained from kSSB (see TS 38.211 [16], clause 7.4.3.1); the LSB of this field is used also for deriving the QCL relation between SS/PBCH blocks as specified in TS 38.213 [13], clause 4.1. The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [13]. This field may indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB (see TS 38.213 [13], clause 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see TS 38.213 [13], clause 13).

subCarrierSpacingCommon Subcarrier spacing for SIB1, Msg.2/4 and MsgB for initial access, paging and broadcast SI-messages. If the UE acquires this MIB on an FR1 carrier frequency, the value scs15or60 corresponds to 15 kHz and the value scs30or120 corresponds to 30 kHz. If the UE acquires this MIB on an FR2 carrier frequency, the value scs15or60 corresponds to 60 kHz and the value scs30or120 corresponds to 120 kHz. For operation with shared spectrum channel access in FR1 (see 37.213 [48]) and for operation in FR2-2, the subcarrier spacing for SIB1, Msg.2/4 and MsgB for initial access, paging and broadcast SI-messages is same as that for the corresponding SSB. For operation with shared spectrum channel access, this field instead is used for deriving the QCL relation between SS/PBCH blocks as specified in TS 38.213 [13], clause 4.1.

systemFrameNumber The 6 most significant bits (MSB) of the 10-bit System Frame Number (SFN). The 4 LSB of the SFN are conveyed in the PBCH transport block as part of channel coding (i.e. outside the MIB encoding), as defined in clause 7.1 in TS 38.212 [17].

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA/EPC, E-UTRA/5GC or UTRA-FDD.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

-- TAG-MOBILITYFROMNRCOMMAND-START

MobilityFromNRCommand ::=           SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

mobilityFromNRCommand           MobilityFromNRCommand-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=       SEQUENCE {

targetRAT-Type                      ENUMERATED { eutra, utra-fdd-v1610, spare2, spare1, ...},

targetRAT-MessageContainer          OCTET STRING,

nas-SecurityParamFromNR             OCTET STRING                                                OPTIONAL,   -- Cond HO-ToEPCUTRAN

lateNonCriticalExtension            OCTET STRING                                                OPTIONAL,

nonCriticalExtension                MobilityFromNRCommand-v1610-IEs                             OPTIONAL

}

MobilityFromNRCommand-v1610-IEs ::=     SEQUENCE {

voiceFallbackIndication-r16             ENUMERATED {true}                                       OPTIONAL,   -- Need N

nonCriticalExtension                    SEQUENCE {}                                             OPTIONAL

}

-- TAG-MOBILITYFROMNRCOMMAND-STOP

-- ASN1STOP

MobilityFromNRCommand-IEs field descriptions

nas-SecurityParamFromNR If targetRAT-Type is eutra, this field is used to deliver the key synchronisation and Key freshness for the NR to LTE/EPC handovers and a part of the downlink NAS COUNT as specified in TS 33.501 [11] and the content of the parameter is defined in TS 24.501 [23]. If targetRAT-Type is utra-fdd, this field is used to deliver the key synchronisation and Key freshness for the NR to FDD UTRAN handover and a part of the downlink NAS COUNT as specified in TS 33.501 [11] and the content of the parameter is defined in TS 24.501 [23].

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. A complete message is included, as specified in the other standard. See NOTE 1

targetRAT-Type Indicates the target RAT type.

voiceFallbackIndication Indicates the handover is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply, and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | TS 36.331 [10] (clause 5.4.2) | DL-DCCH-Message including the RRCConnectionReconfiguration |
| utra-fdd | TS 25.331 [45] (clause 10.2.16a) | Handover TO UTRAN command |

| Conditional Presence | Explanation |
| --- | --- |
| HO-ToEPCUTRAN | This field is mandatory present in case of inter system handover to "EPC" or "FDD UTRAN". Otherwise it is absent. |

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs                                                        OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700                                                  OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17                                                     OPTIONAL, -- Need N

nonCriticalExtension                Paging-v1800-IEs                                                        OPTIONAL

}

Paging-v1800-IEs ::=                SEQUENCE {

pagingRecordList-v1800              PagingRecordList-v1800                                                  OPTIONAL, -- Need N

pagingGroupList-v1800               PagingGroupList-v1800                                                   OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                                                             OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecordList-v1800 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1800

PagingGroupList-v1800 ::=           SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF GroupPaging-r18

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingRecord-v1800 ::=              SEQUENCE {

mt-SDT                              ENUMERATED {true}       OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

GroupPaging-r18 ::=                 SEQUENCE {

inactiveReceptionAllowed-r18        ENUMERATED {true}       OPTIONAL    -- Need N

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

inactiveReceptionAllowed Indicates whether the UE with a valid PTM configuration for a TMGI in the PagingGroupList stays in RRC_INACTIVE to receive the corresponding MBS multicast session.

mt-SDT Mobile Terminated SDT indication. The network includes mt-SDT indication in paging message only if the UE's I-RNTI is included in the paging message.

pagingRecordList If the network includes pagingRecordList-v1700, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix). If the network includes pagingRecordList-v1800, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix). The first element in pagingRecordList-v1700 corresponds to the first UE identity in pagingRecordList (i.e. without suffix). The second element in pagingRecordList-v1700 corresponds to the second UE identity in pagingRecordList (i.e. without suffix), and so on. The first element in pagingRecordList-v1800 corresponds to the first UE identity in pagingRecordList (i.e. without suffix). The second element in pagingRecordList-v1800 corresponds to the second UE identity in pagingRecordList (i.e. without suffix), and so on.

pagingCause Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If upper layers indicate the support of paging cause and if this field is not present but pagingRecordList-v1700 is present, it implies that the corresponding paging entry is for a service other than IMS voice. Otherwise, paging cause is undetermined.

pagingGroupList If the network includes pagingGroupList-v1800, it includes the same number of elements, and listed in the same order, as in pagingGroupList-r17. The first element corresponds to the first TMGI in pagingGroupList-r17. The second element corresponds to the second TMGI in pagingGroupList-r17, and so on.

#### –	RRCReestablishment

The RRCReestablishment message is used to re-establish SRB1.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReestablishment message

-- ASN1START

-- TAG-RRCREESTABLISHMENT-START

RRCReestablishment ::=              SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcReestablishment                  RRCReestablishment-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReestablishment-IEs ::=          SEQUENCE {

nextHopChainingCount                NextHopChainingCount,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                RRCReestablishment-v1700-IEs         OPTIONAL

}

RRCReestablishment-v1700-IEs ::=    SEQUENCE {

sl-L2RemoteUE-Config-r17            SetupRelease {SL-L2RemoteUE-Config-r17}    OPTIONAL, -- Cond L2RemoteUE

nonCriticalExtension                SEQUENCE {}                                OPTIONAL

}

-- TAG-RRCREESTABLISHMENT-STOP

-- ASN1STOP

RRCReestablishment-IEs field descriptions

sl-L2RemoteUE-Config Contains dedicated configurations used for single hop or multi hop L2 U2N relay related operation. The network configures only the SRAP configuration for local UE ID.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE; otherwise it is absent. |

#### –	RRCReestablishmentComplete

The RRCReestablishmentComplete message is used to confirm the successful completion of an RRC connection re-establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReestablishmentComplete message

-- ASN1START

-- TAG-RRCREESTABLISHMENTCOMPLETE-START

RRCReestablishmentComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReestablishmentComplete                  RRCReestablishmentComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

RRCReestablishmentComplete-IEs ::=          SEQUENCE {

lateNonCriticalExtension                    OCTET STRING                            OPTIONAL,

nonCriticalExtension                        RRCReestablishmentComplete-v1610-IEs    OPTIONAL

}

RRCReestablishmentComplete-v1610-IEs ::=    SEQUENCE {

ue-MeasurementsAvailable-r16                UE-MeasurementsAvailable-r16            OPTIONAL,

nonCriticalExtension                        RRCReestablishmentComplete-v1800-IEs    OPTIONAL

}

RRCReestablishmentComplete-v1800-IEs ::=    SEQUENCE {

flightPathInfoAvailable-r18                 ENUMERATED {true}                       OPTIONAL,

measConfigReportAppLayerAvailable-r18       ENUMERATED {true}                       OPTIONAL,

musim-CapRestrictionInd-r18                 ENUMERATED {true}                       OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                             OPTIONAL

}

-- TAG-RRCREESTABLISHMENTCOMPLETE-STOP

-- ASN1STOP

RRCReestablishmentComplete-IEs field descriptions

measConfigReportAppLayerAvailable Indication that the UE has at least one application layer measurement configuration with appLayerIdleInactiveConfig configured.

musim-CapRestrictionInd This field indicates the UE temporary capability restriction due to MUSIM operation.

#### –	RRCReestablishmentRequest

The RRCReestablishmentRequest message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCReestablishmentRequest message

-- ASN1START

-- TAG-RRCREESTABLISHMENTREQUEST-START

RRCReestablishmentRequest ::=       SEQUENCE {

rrcReestablishmentRequest           RRCReestablishmentRequest-IEs

}

RRCReestablishmentRequest-IEs ::=   SEQUENCE {

ue-Identity                         ReestabUE-Identity,

reestablishmentCause                ReestablishmentCause,

spare                               BIT STRING (SIZE (1))

}

ReestabUE-Identity ::=              SEQUENCE {

c-RNTI                              RNTI-Value,

physCellId                          PhysCellId,

shortMAC-I                          ShortMAC-I

}

ReestablishmentCause ::=            ENUMERATED {reconfigurationFailure, handoverFailure, otherFailure, spare1}

-- TAG-RRCREESTABLISHMENTREQUEST-STOP

-- ASN1STOP

ReestabUE-Identity field descriptions

physCellId The Physical Cell Identity of the PCell the UE was connected to prior to the failure.

RRCReestablishmentRequest-IEs field descriptions

reestablishmentCause Indicates the failure cause that triggered the re-establishment procedure. gNB is not expected to reject a RRCReestablishmentRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to retrieve UE context and to facilitate contention resolution by lower layers.

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING (CONTAINING RRCReconfiguration-v15t0-IEs)                 OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

-- Regular non-critical extensions:

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v1700-IEs                                         OPTIONAL

}

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need S

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1800-IEs                                   OPTIONAL

}

RRCReconfiguration-v1800-IEs ::=        SEQUENCE {

needForInterruptionConfigNR-r18         ENUMERATED { disabled, enabled }                                   OPTIONAL, -- Need M

aerial-Config-r18                           SetupRelease { Aerial-Config-r18 }                             OPTIONAL, -- Need M

sl-IndirectPathAddChange-r18                SetupRelease { SL-IndirectPathAddChange-r18 }                  OPTIONAL, -- Need M

n3c-IndirectPathAddChange-r18               SetupRelease { N3C-IndirectPathAddChange-r18 }                 OPTIONAL, -- Need M

n3c-IndirectPathConfigRelay-r18             SetupRelease { N3C-IndirectPathConfigRelay-r18 }               OPTIONAL, -- Need M

otherConfig-v1800                           OtherConfig-v1800                                              OPTIONAL, -- Need M

srs-PosResourceSetAggBW-CombinationList-r18 SetupRelease { SRS-PosResourceSetAggBW-CombinationList-r18 }   OPTIONAL, -- Need M

ltm-Config-r18                              SetupRelease {LTM-Config-r18}                                  OPTIONAL, -- Need M

nonCriticalExtension                        RRCReconfiguration-v1830-IEs                                   OPTIONAL

}

RRCReconfiguration-v1830-IEs ::=        SEQUENCE {

otherConfig-v1830                       OtherConfig-v1830                                                  OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1900-IEs                                       OPTIONAL

}

RRCReconfiguration-v1900-IEs ::=        SEQUENCE {

n3c-ExtIndirectPathAddChange-r19        SetupRelease { N3C-ExtIndirectPathAddChange-r19 }                  OPTIONAL, -- Need M

otherConfig-v1900                       OtherConfig-v1900                                                  OPTIONAL, -- Need M

onDemandPosSIB-RequestCtrlParam-r19     ENUMERATED { enabled }                                             OPTIONAL, -- Need R

retainLoggedMeasurements-r19            ENUMERATED {true}                                                  OPTIONAL, -- Need N

ltm-ConfigNRDC-r19                      SetupRelease {LTM-ConfigNRDC-r19}                                  OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                                        OPTIONAL

}

-- Late non-critical Rel-15 extensions:

RRCReconfiguration-v15t0-IEs ::=        SEQUENCE {

-- Following field is only to be used for late REL-15 extensions

lateNonCriticalExtension                OCTET STRING                                                       OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v16k0-IEs                                       OPTIONAL

}

RRCReconfiguration-v16k0-IEs ::=        SEQUENCE {

sl-ConfigDedicatedNR-v16k0              SetupRelease {SL-ConfigDedicatedNR-v16k0}                          OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE{}                                                         OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

UE-TxTEG-RequestUL-TDOA-Config-r17 ::=  CHOICE {

oneShot-r17                             NULL,

periodicReporting-r17                   ENUMERATED { ms160, ms320, ms1280, ms2560, ms61440, ms81920, ms368640, ms737280 }

}

SRS-PosResourceSetAggBW-CombinationList-r18 ::= SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResSetComb-r18)) OF SRS-PosResourceSetLinkedForAggBW-List-r18

SRS-PosResourceSetLinkedForAggBW-List-r18 ::= SEQUENCE (SIZE(2..maxNrOfLinkedSRS-PosResourceSet-r18)) OF SRS-PosResourceSetLinkedForAggBW-r18

N3C-ExtIndirectPathAddChange-r19 ::=            SEQUENCE (SIZE(1..maxNrofN3C-RelayUE-r19)) OF N3C-RelayUE-Info-r18

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

appLayerMeasConfig This field is used to configure application layer measurements. This field is absent when the UE is configured to operate with shared spectrum channel access or if sl-L2RemoteUE-Config-r17 is configured or not released.

bap-Config This field is used to configure the BAP entity for IAB nodes.

bap-Address Indicates the BAP address of an IAB-node. The BAP address of an IAB-node cannot be changed once configured for the cell group to the BAP entity.

conditionalReconfiguration Configuration of candidate target SpCell(s) and execution condition(s) for conditional handover, conditional PSCell addition or conditional PSCell change. The field is absent if any DAPS bearer is configured, if the sl-L2RemoteUE-Config or sl-L2RelayUE-Config is configured, or if the RRCReconfiguration message is contained within condRRCReconfig. When the masterCellGroup and/or secondaryCellGroup includes ReconfigurationWithSync, if this field is present, it only includes configurations/fields specific to subsequent CPAC. The RRCReconfiguration message contained in DLInformationTransferMRDC cannot contain the field conditionalReconfiguration for conditional PSCell change or for conditional PSCell addition. The network does not include this field in an RRCReconfiguration message contained within a LTM-Config IE.

daps-SourceRelease Indicates to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedPagingDelivery This field is used to transfer Paging message for the associated L2 U2N Remote UE or for the associated child UE to the L2 U2N Relay UE or to L2 Last U2N Relay UE in RRC_CONNECTED.

dedicatedPosSysInfoDelivery This field is used to transfer SIBPos to the UE in RRC_CONNECTED. This field may contain multiple posSIB segments of the same posSIB type. When the number of segments of posSIB of the same posSIB type exceeds the maximum number of posSIBs of this field (i.e., 32), the posSIB segments of the same posSIB type may be delivered in multiple RRCReconfiguration messages.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE (including L2 U2N Remote UE). The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8, SIB19, SIB20, SIB21, SIB25, SIB26 to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is also used to transfer the SIBs requested on-demand.

defaultUL-BAP-RoutingID This field is used for IAB-node to configure the default uplink Routing ID, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BAP-RoutingID can be (re-)configured when IAB-node IP address for F1-C related traffic changes. This field is mandatory only for IAB-node bootstrapping.

defaultUL-BH-RLC-Channel This field is used for IAB-nodes to configure the default uplink BH RLC channel, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BH-RLC-Channel can be (re-)configured when IAB-node IP address for F1-C related traffic changes, and the new IP address is anchored at a different IAB-donor-DU. This field is mandatory for IAB-node bootstrapping. If the IAB-MT is operating in EN-DC, the default uplink BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel either on the MCG or on the SCG depending on whether the MN or the SN configures this field.

flowControlFeedbackType This field is only used for IAB-node that support hop-by-hop flow control to configure the type of flow control feedback. Value perBH-RLC-Channel indicates that the IAB-node shall provide flow control feedback per BH RLC channel, value perRoutingID indicates that the IAB-node shall provide flow control feedback per routing ID, and value both indicates that the IAB-node shall provide flow control feedback both per BH RLC channel and per routing ID.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

iab-IP-Address This field is used to provide the IP address information for IAB-node.

iab-IP-AddressIndex This field is used to identify a configuration of an IP address.

iab-IP-AddressToAddModList List of IP addresses allocated for IAB-node to be added and modified.

iab-IP-AddressToReleaseList List of IP address allocated for IAB-node to be released.

iab-IP-Usage This field is used to indicate the usage of the assigned IP address. If this field is not configured, the assigned IP address is used for all traffic.

iab-donor-DU-BAP-Address This field is used to indicate the BAP address of the IAB-donor-DU where the IP address is anchored.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

ltm-Config The network does not configure this field in an RRCReconfiguration message within an LTM-Config IE and ConditionalReconfiguration IE.

ltm-ConfigNRDC This field contains LTM candidate configurations associated with the SCG and the MCG. The network does not configure this field in an RRCReconfiguration message contained in nr-SCG or transmitted on SRB3.

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time. The network always includes this field in an mrdc-SecondaryCellGroup set to setup which is included in an RRCReconfiguration message within an LTM-Config IE.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup, otherConfig, conditionalReconfiguration, ltm-Config, measConfig, bap-Config, IAB-IP-AddressConfigurationList and appLayerMeasConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

mrdc-SecondaryCellGroupConfig This field is used to configure and release an SCG in NR-DC and NE-DC.

musim-GapConfig Indicates the MUSIM gap configuration and controls setup/release of MUSIM gaps. In this version of the specification, the network does not configure MUSIM gap together preconfigured measurement gap for positioning. For the UE supporting musim-GapPriorityPreference, the network can configure MUSIM gap together with concurrent measurement gap. Otherwise, the network does not configure MUSIM gap together with concurrent measurement gap.

n3c-ExtIndirectPathAddChange This field indicates the configuration of one or more relay UEs via N3C link. The network does not configure this field together with n3c-IndirectPathAddChange.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForInterruptionConfigNR Indicates whether the UE shall report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. The network sets this field to enabled only if the needForGapsConfigNR is configured. The network sets this field to disabled if the needForGapsConfigNR is released.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

onDemandPosSIB-RequestCtrlParam Indicates whether the UE is enabled to request periodic delivery of posSIB(s) while in RRC_CONNECTED. This field can only be present when the field onDemandSIB-Request is present.

onDemandSIB-Request Indicates that the UE is allowed to request SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5.

onDemandSIB-RequestProhibitTimer Prohibit timer for requesting SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxBW-PreferenceConfigFR2-2, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfig, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, btNameList, wlanNameList, sensorNameList, obtainCommonLocation, idc-AssistanceConfig, multiRx-PreferenceReportingConfigFR2, ul-TrafficInfoReportingConfig, n3c-RelayUE-InfoReportConfig, successPSCell-Config, sn-InitiatedPSCellChange and gapOccasionCancelRatioReportConfig can be included.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs, multicast MRBs) including SDAP/PDCP. In (NG)EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3. SRB4 should not be configured if sl-L2RemoteUE-Config-r17 is configured or not released.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

retainLoggedMeasurements If present, it indicates that the UE shall retain the logged measurements available in VarCSI-LogMeasReport upon execution of this RRCReconfiguration message including the reconfigurationWithSync.

scg-State Indicates that the SCG is in deactivated state. This field is not used - in an RRCReconfiguration message received: - within mrdc-SecondaryCellGroup, or - in an E-UTRA RRCConnectionReconfiguration message, or - in an E-UTRA RRCConnectionResume message or - in an RRCReconfiguration message received via SRB3, except if the RRCReconfiguration message is included in DLInformationTransferMRDC. The field is absent if CPA, CPC, or subsequent CPAC is configured for the UE, or if the RRCReconfiguration message is contained in CondRRCReconfig, or PSCell is configured with tag2, or if the RRCReconfiguration message is included within an LTM-Config IE.

sl-L2RelayUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Relay UE or L2 U2U relay operation related configuration used by a UE acting as a L2 U2U Relay UE. In case of L2 U2N relay operation, the field is absent if conditionalReconfiguration is configured for CHO.

sl-L2RemoteUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Remote UE or L2 U2U relay operation related configuration used by a UE acting as a L2 U2U Remote UE. In case of L2 U2N relay operation, the field is absent if conditionalReconfiguration is configured for CHO, or if appLayerMeasConfig or SRB4 is configured/not released.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC).

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary, or if the RRCReconfiguration message is contained in condRRCReconfig for subsequent CPAC.

sl-ConfigDedicatedEUTRA-Info This field includes the E-UTRA RRCConnectionReconfiguration as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRCConnectionReconfiguration can only includes sidelink related fields for V2X sidelink communication, i.e. sl-V2X-ConfigDedicated, sl-V2X-SPS-Config, measConfig and/or otherConfig.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication/discovery/positioning.

sl-TimeOffsetEUTRA This field indicates the possible time offset to (de)activation of V2X sidelink transmission after receiving DCI format 3_1 used for scheduling V2X sidelink communication. Value ms0dpt75 corresponds to 0.75ms, ms1 corresponds to 1ms and so on. The network includes this field only when sl-ConfigDedicatedEUTRA is configured.

srs-PosResourceSetAggBW-CombinationList This field indicates the SRS resource sets across two or three carriers which are linked for SRS bandwidth aggregation in RRC_CONNECTED state as defined in clause 6.2.1.4 of TS 38.214 [19].

targetCellSMTC-SCG The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. When UE receives this field, UE applies the configuration based on the timing reference of NR PCell for PSCell addition and PSCell change for the case of no reconfiguration with sync of MCG, and UE applies the configuration based on the timing reference of target NR PCell for the case of reconfiguration with sync of MCG. If both this field and the smtc in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. This field can be configured only if the UE is configured with split SRB1 or SRB3.

ue-TxTEG-RequestUL-TDOA-Config Configures the periodicity of UE reporting for the association between Tx TEG and SRS Positioning resources. When configured with oneShot UE reports the association only one time. When configured with periodicReporting UE reports the association periodically and the periodicReporting indicates the periodicity. Value ms160 corresponds to 160ms, value ms320 corresponds to 320ms and so on.

ul-GapFR2-Config Indicates the FR2 UL gap configuration to UE. In EN-DC and NGEN-DC, the SN decides and configures the FR2 UL gap pattern. In NE-DC, the MN decides and configures the FR2 UL gap pattern. In NR-DC without FR2-FR2 band combination, the network entity which is configured with FR2 serving cell(s) decides and configures the FR2 UL gap pattern.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. If ReconfigurationWithSync is part of an RRCReconfiguration message within an LTM-Config IE associated with the MCG, the field is absent. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during a reconfiguration with sync which is not related to an LTM cell switch or subsequent CPAC, and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |
| SCG | The field is mandatory present in: - an RRCReconfiguration message contained in an RRCResume message (or in an RRCConnectionResume message, see TS 36.331 [10]), - an RRCReconfiguration message contained in an RRCConnectionReconfiguration message, see TS 36.331 [10], which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). The field is optional present, Need M, in: - an RRCReconfiguration message transmitted on SRB3, - an RRCReconfiguration message contained in another RRCReconfiguration message (or in an RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1 - an RRCReconfiguration message contained in another RRCReconfiguration message which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). Otherwise, the field is absent. |
| PagingRelay | For L2 U2N Relay UE, the field is optionally present, Need N. Otherwise, it is absent. |

#### –	RRCReconfigurationComplete

The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReconfigurationComplete                  RRCReconfigurationComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

RRCReconfigurationComplete-IEs ::=          SEQUENCE {

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1530-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1530-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentList                   UplinkTxDirectCurrentList                                               OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1560-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1560-IEs ::=    SEQUENCE {

scg-Response                                CHOICE {

nr-SCG-Response                             OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                          OCTET STRING

}                                                                                                                       OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1610-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1610-IEs ::=    SEQUENCE {

ue-MeasurementsAvailable-r16                UE-MeasurementsAvailable-r16                                            OPTIONAL,

needForGapsInfoNR-r16                       NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1640-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1640-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16     UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1700-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1700-IEs ::=    SEQUENCE {

needForGapNCSG-InfoNR-r17                   NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17                NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

selectedCondRRCReconfig-r17                 CondReconfigId-r16                                                      OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1720-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1720-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentMoreCarrierList-r17    UplinkTxDirectCurrentMoreCarrierList-r17                                OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1800-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1800-IEs ::=    SEQUENCE {

needForInterruptionInfoNR-r18               NeedForInterruptionInfoNR-r18                                           OPTIONAL,

flightPathInfoAvailable-r18                 ENUMERATED {true}                                                       OPTIONAL,

selectedPSCellForCHO-WithSCG-r18            SelectedPSCellForCHO-WithSCG-r18                                        OPTIONAL,

selectedSK-Counter-r18                      SK-Counter                                                              OPTIONAL,

measConfigReportAppLayerAvailable-r18       ENUMERATED {true}                                                       OPTIONAL,

appliedLTM-CandidateId-r18                  LTM-CandidateId-r18                                                     OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1900-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1900-IEs ::=    SEQUENCE {

applicabilityReportList-r19                 ApplicabilityReportList-r19                                             OPTIONAL,

csi-LogMeasAvailable-r19                    ENUMERATED {true}                                                       OPTIONAL,

referenceLocationReport-r19                 BIT STRING (SIZE (6))                                                   OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

RRCReconfigurationComplete-IEs field descriptions

applicabilityReportList The applicability reports related to prediction configurations and sets of parameters for prediction configurations.

csi-LogMeasAvailable Indicates that the UE has logged CSI radio measurements for network-side data collection to be reported to the network.

measConfigReportAppLayerAvailable Indication that the UE has at least one application layer measurement configuration with appLayerIdleInactiveConfig configured.

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForGapNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands.

needForGapNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands.

needForInterruptionInfoNR This field indicates whether interruption is needed while performing measurement on NR target bands without measurement gap.

referenceLocationReport This field indicates the reference locations that are closest to the UE for assisted SMTC configuration in RRC_CONNECTED state. The length of the bitmap is equal to the number of entries in the refLocList currently configured for the UE (either via OtherConfig or SIB2). The UE sets the bit at a given position to '1' if the reference location at the corresponding position in the refLocList is one of the closest locations, and to '0' otherwise. The first bit in the string corresponds to the first reference location in the list, and so on.

scg-Response In case of NR-DC (nr-SCG-Response), this field includes the RRCReconfigurationComplete message. In case of NE-DC (eutra-SCG-Response), this field includes the E-UTRA RRCConnectionReconfigurationComplete message as specified in TS 36.331 [10].

selectedCondRRCReconfig This field indicates the ID of the selected conditional reconfiguration the UE applied upon the execution of CPA or inter-SN CPC or subsequent CPAC.

selectedPSCellForCHO-WithSCG This field indicates the information of the selected target PSCell to target MN at execution of a conditional reconfiguration for CHO with candidate SCG(s).

selectedSK-Counter This field includes the selected sk-counter value for security key update upon the execution of subsequent CPAC.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentMoreCarrierList The Tx Direct Current locations for the configured intra-band CA requested by reportUplinkTxDirectCurrentMoreCarrier-r17.

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).

#### –	RRCReject

The RRCReject message is used to reject an RRC connection establishment or an RRC connection resumption.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCReject message

-- ASN1START

-- TAG-RRCREJECT-START

RRCReject ::=                       SEQUENCE {

criticalExtensions                  CHOICE {

rrcReject                           RRCReject-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReject-IEs ::=                   SEQUENCE {

waitTime                            RejectWaitTime                                                          OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCREJECT-STOP

-- ASN1STOP

RRCReject-IEs field descriptions

waitTime Wait time value in seconds. The field is always included.

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                       OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                                   OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                               OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                               OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}                             OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v1710-IEs                          OPTIONAL

}

RRCRelease-v1710-IEs ::=            SEQUENCE {

noLastCellUpdate-r17                ENUMERATED {true}                            OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                                  OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...,

nr-v1900                            CarrierInfoNR-v1900

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}                                             OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                                                             OPTIONAL,      -- Need S

...

}

CarrierInfoNR-v1900 ::=             SEQUENCE {

carrierFreq-r19                     ARFCN-ValueNR,

ssbSubcarrierSpacing-r19            SubcarrierSpacing,

smtc-r19                            SSB-MTC                                                             OPTIONAL,      -- Need S

satAssistanceInfoList-r19           SEQUENCE (SIZE (1..maxFFS)) OF NTN-Config-r17                       OPTIONAL,      -- Cond SIB19

...

}

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-UEIdentityRemote-r17             RNTI-Value                                                          OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }                                     OPTIONAL,   -- Need M

srs-PosRRC-Inactive-r17             SetupRelease { SRS-PosRRC-Inactive-r17 }                            OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17        ExtendedPagingCycle-r17                                             OPTIONAL    -- Cond RANPaging

]],

[[

ncd-SSB-RedCapInitialBWP-SDT-r17    SetupRelease {NonCellDefiningSSB-r17}                               OPTIONAL    -- Need M

]],

[[

resumeIndication-r18                ENUMERATED {true}                                                   OPTIONAL,   -- Need N

srs-PosRRC-InactiveEnhanced-r18     SetupRelease { SRS-PosRRC-InactiveEnhanced-r18 }                    OPTIONAL,   -- Need M

ran-ExtendedPagingCycleConfig-r18   ExtendedPagingCycleConfig-r18                                       OPTIONAL,  -- Cond RANPaging

multicastConfigInactive-r18         SetupRelease { MulticastConfigInactive-r18 }                        OPTIONAL   -- Need M

]]

}

PeriodicRNAU-TimerValue ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                                               OPTIONAL,       -- Need M

freqPriorityListNR                  FreqPriorityListNR                                                  OPTIONAL,       -- Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL,     -- Need R

...,

[[

freqPriorityListDedicatedSlicing-r17 FreqPriorityListDedicatedSlicing-r17                               OPTIONAL        -- Need M

]]

}

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode                    TrackingAreaCode,

ran-AreaCodeList                    SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode                            OPTIONAL    -- Need R

}

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                    OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need S

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL,   -- Need N

cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-ConfigInitialBWP-NUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-SUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-DL-r17        BWP-DownlinkDedicatedSDT-r17                                  OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17         TimeAlignmentTimer                                            OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17          RSRP-Range                                                    OPTIONAL,   -- Need M

cg-SDT-TA-ValidationConfig-r17        SetupRelease { CG-SDT-TA-ValidationConfig-r17 }               OPTIONAL,   -- Need M

cg-SDT-CS-RNTI-r17                    RNTI-Value                                                    OPTIONAL,   -- Need M

...,

[[

cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-RestrictionExt-v1800

OPTIONAL,   -- Need N

cg-MT-SDT-MaxDurationToNextCG-Occasion-r18 ENUMERATED {

ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600,

sec1200, sec1800, sec3600,

spare5, spare4, spare3, spare2, spare1}                 OPTIONAL    -- Need R

]]

}

CG-SDT-TA-ValidationConfig-r17 ::=  SEQUENCE {

cg-SDT-RSRP-ChangeThreshold-r17     ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22,

dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1}

}

BWP-DownlinkDedicatedSDT-r17 ::=    SEQUENCE {

pdcch-Config-r17                    SetupRelease { PDCCH-Config }                                       OPTIONAL,   -- Need M

pdsch-Config-r17                    SetupRelease { PDSCH-Config }                                       OPTIONAL,   -- Need M

...

}

BWP-UplinkDedicatedSDT-r17 ::=      SEQUENCE {

pusch-Config-r17                    SetupRelease { PUSCH-Config }                                       OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r17                 ConfiguredGrantConfigToAddModList-r16             OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r17                ConfiguredGrantConfigToReleaseList-r16            OPTIONAL,   -- Need N

...

}

CG-SDT-ConfigLCH-Restriction-r17 ::= SEQUENCE {

logicalChannelIdentity-r17          LogicalChannelIdentity,

configuredGrantType1Allowed-r17     ENUMERATED {true}                                                   OPTIONAL,   -- Need R

allowedCG-List-r17                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL    -- Need R

}

CG-SDT-ConfigLCH-RestrictionExt-v1800 ::= SEQUENCE {

cg-SDT-MaxDurationToNextCG-Occasion-r18 ENUMERATED {

ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600,

sec1200, sec1800, sec3600,

spare5, spare4, spare3, spare2, spare1}                    OPTIONAL    -- Need R

}

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17)

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

RSRP-ChangeThreshold-r17 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

SRS-PosConfig-r17 ::=               SEQUENCE {

srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16 OPTIONAL,-- Need N

srs-PosResourceSetToAddModList-r17  SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16  OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16      OPTIONAL,-- Need N

srs-PosResourceToAddModList-r17     SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16        OPTIONAL -- Need N

}

SRS-PosRRC-InactiveEnhanced-r18 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveEnhancedConfig-r18)

SRS-PosRRC-InactiveEnhancedConfig-r18 ::=    SEQUENCE {

srs-PosRRC-InactiveAggBW-ConfigList-r18      SetupRelease { SRS-PosRRC-InactiveAggBW-ConfigList-r18 }         OPTIONAL,  -- Need M

srs-PosTx-Hopping-r18                        SetupRelease { SRS-PosTx-Hopping-r18 }                           OPTIONAL,  -- Need M

srs-PosRRC-InactiveValidityAreaPreConfigList-r18 SetupRelease { SRS-PosRRC-InactiveValidityAreaPreConfigList-r18 } OPTIONAL, -- Need M

srs-PosRRC-InactiveValidityAreaNonPreConfig-r18  SetupRelease { SRS-PosRRC-InactiveValidityAreaConfig-r18 }        OPTIONAL, -- Need M

...,

[[

srs-PosRRC-InactiveAggBW-AdditionalCarriers-r18  SetupRelease {SRS-PosRRC-InactiveAggBW-AdditionalCarriers-r18 } OPTIONAL  -- Need M

]]

}

SRS-PosRRC-InactiveAggBW-AdditionalCarriers-r18 ::= SEQUENCE{

aggregatedPosSRS-CarrierList-r18       SEQUENCE (SIZE(1..maxNrOfLinkedSRS-CarriersInactive-1-r18)) OF SRS-PosConfigPerULCarrier-r18                     OPTIONAL, -- Need R

...

}

SRS-PosConfigPerULCarrier-r18 ::=         SEQUENCE{

freqInfo-r18                              ARFCN-ValueNR,

srs-PosConfig-r18                         SRS-PosConfig-r17,

scs-SpecificCarrier-r18                   SCS-SpecificCarrier                                            OPTIONAL,   -- Need R

bwp-r18                                   BWP                                                            OPTIONAL,   -- Need R

...

}

SRS-PosRRC-InactiveValidityAreaPreConfigList-r18  ::= SEQUENCE (SIZE(1..maxNrOfVA-r18)) OF SRS-PosRRC-InactiveValidityAreaConfig-r18

SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE {

srs-PosConfigValidityArea-r18                 SEQUENCE (SIZE(1..maxNrOfCellsInVA-r18)) OF CellIdentity,

srs-PosConfigNUL-r18                          SRS-PosConfig-r17                                              OPTIONAL,   -- Need R

srs-PosConfigSUL-r18                          SRS-PosConfig-r17                                              OPTIONAL,   -- Need R

bwp-NUL-r18                                   BWP                                                            OPTIONAL,   -- Need S

bwp-SUL-r18                                   BWP                                                            OPTIONAL,   -- Need S

areaValidityTA-Config-r18                     AreaValidityTA-Config-r18                                      OPTIONAL,   -- Need R

...,

[[

srs-PosConfigValidityAreaExt-v1830            SEQUENCE (SIZE(1..maxNrOfCellsInVA-Ext-r18)) OF CellIdentity   OPTIONAL    -- Need R

]],

[[

srs-PosRRC-InactiveAggBW-AdditionalCarriersPerVA-r18 SetupRelease {SRS-PosRRC-InactiveAggBW-AdditionalCarriers-r18 }

OPTIONAL,   -- Need M

srs-PosRRC-InactiveAggBW-ConfigListPerVA-r18     SetupRelease {SRS-PosRRC-InactiveAggBW-ConfigList-r18 }     OPTIONAL    -- Need M

]]

}

AreaValidityTA-Config-r18 ::=             SEQUENCE {

inactivePosSRS-ValidityAreaTAT-r18        ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity},

inactivePosSRS-ValidityAreaRSRP-r18       RSRP-ChangeThreshold-r17                                           OPTIONAL,   -- Need R

autonomousTA-AdjustmentEnabled-r18        ENUMERATED {true}                                                  OPTIONAL    -- Need R

}

SRS-PosRRC-InactiveAggBW-ConfigList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResSetCombInactive-r18)) OF

SRS-InactivePosResourceSetLinkedForAggBW-List-r18

SRS-InactivePosResourceSetLinkedForAggBW-List-r18 ::=  SEQUENCE (SIZE (2..maxNrOfLinkedSRS-PosResourceSet-r18)) OF

SRS-PosResourceSetLinkedForAggBW-r18

ExtendedPagingCycle-r17 ::=         ENUMERATED {rf256, rf512, rf1024, spare1}

ExtendedPagingCycleConfig-r18 ::=  SEQUENCE {

extendedPagingCycle-r18            ENUMERATED {hf2, hf4, hf8, hf16, hf32, hf64, hf128,hf256, hf512, hf1024,

spare6, spare5, spare4, spare3, spare2, spare1},

pagingPTWLength-r18                ENUMERATED {ms1280, ms2560, ms3840, ms5120, ms6400, ms7680, ms8960, ms10240, ms11520,

ms12800, ms14080, ms15360, ms16640, ms17920, ms19200, ms20480, ms21760,

ms23040, ms24320, ms25600, ms26880, ms28160, ms29440, ms30720, ms32000,

ms33280, ms34560, ms35840, ms37120, ms38400, ms39680, ms40960}

}

MulticastConfigInactive-r18::=         SEQUENCE {

inactivePTM-Config-r18                 OCTET STRING (CONTAINING MBSMulticastConfiguration-r18)     OPTIONAL, -- Need S

inactiveMCCH-Config-r18                OCTET STRING (CONTAINING SystemInformation)                 OPTIONAL  -- Need N

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

RRCRelease-IEs field descriptions

cellReselectionPriorities Dedicated priorities to be used for cell reselection as specified in TS 38.304 [20]. The maximum number of NR carrier frequencies that the network can configure through FreqPriorityListNR and FreqPriorityListDedicatedSlicing together is eight. If the same frequency is configured in both FreqPriorityListNR and FreqPriorityListDedicatedSlicing, the frequency is only counted once.

cnType Indicate that the UE is redirected to EPC or 5GC.

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised.

deprioritisationTimer Indicates the value for timer T325 (see clause 5.3.8.3 and TS 38.304 [20]). Value minN corresponds to N minutes.

srs-PosRRC-InactiveEnhanced Contains the SRS for positioning configuration in RRC_INACTIVE state that is applicable for a validity area. The field also contains bandwidth aggregation (see TS 38.214 [19], clause 6.2.1.4.2) and frequency hopping configurations (see TS 38.214 [19], clause 6.2.1.4.1) for SRS for positioning in RRC_INACTIVE state.

measIdleConfig Indicates measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

mpsPriorityIndication Indicates the UE can set the establishment cause to mps-PriorityAccess for a new connection following a redirect to NR or set the resume cause to mps-PriorityAccess for a resume following a redirect to NR. If the target RAT is E-UTRA, see TS 36.331 [10]. The gNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the gNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCRelease message.

multicastConfigInactive Indicates whether the UE is configured to receive MBS multicast in RRC_INACTIVE. The presence of this field indicates the UE is configured to receive MBS multicast in RRC_INACTIVE; otherwise, the UE is not configured to receive MBS multicast in RRC_INACTIVE.

noLastCellUpdate Presence of the field indicates that the last used cell for PEI shall not be updated. When the field is absent, the PEI-capable UE shall update its last used cell with the current cell. The UE shall not update its last used cell with the current cell if the AS security is not activated.

redirectedCarrierInfo Indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an NR or an inter-RAT carrier frequency, by means of cell selection at transition to RRC_IDLE or RRC_INACTIVE as specified in TS 38.304 [20]. Based on UE capability, the network may include redirectedCarrierInfo in RRCRelease message with suspendConfig if this message is sent in response to an RRCResumeRequest or an RRCResumeRequest1 which is triggered by the NAS layer (see 5.3.1.4 in TS 24.501 [23]). nr-v1900 is used to redirect a UE to an NTN.

srs-PosRRC-Inactive Contains the SRS for positioning configuration in RRC_INACTIVE state.

suspendConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure suspendConfig when the network redirect the UE to an inter-RAT carrier frequency or if the UE is configured with a DAPS bearer.

voiceFallbackIndication Indicates the RRC release is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency.

satAssistanceInfoList Satellite assistance information used to detect and perform measurements on the indicated frequency. If the field is not present, the UE uses the ntn-Config field(s) of elements in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt in SIB19 for which the carrierFreq matches.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

RAN-NotificationAreaInfo field descriptions

cellList A list of cells configured as RAN area.

ran-AreaConfigList A list of RAN area codes or RA code(s) as RAN area.

PLMN-RAN-AreaConfig field descriptions

plmn-Identity PLMN Identity to which the cells in ran-Area belong. If the field is absent the UE not in SNPN access mode uses the ID of the registered PLMN. This field is not included for UE in SNPN access mode (for UE in SNPN access mode the ran-Area always belongs to the registered SNPN).

ran-AreaCodeList The total number of RAN-AreaCodes of all PLMNs does not exceed 32.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or both TA code(s) and RAN area code(s) to configure a UE. The total number of TACs across all PLMNs does not exceed 16.

PLMN-RAN-AreaCell field descriptions

plmn-Identity PLMN Identity to which the cells in ran-AreaCells belong. If the field is absent the UE not in SNPN access mode uses the ID of the registered PLMN. This field is not included for UE in SNPN access mode (for UE in SNPN access mode the ran-AreaCells always belongs to the registered SNPN).

ran-AreaCells The total number of cells of all PLMNs does not exceed 32.

SDT-Config field descriptions

sdt-DRB-ContinueROHC Indicates whether the PDCP entity of the radio bearers configured for SDT continues or resets the ROHC header compression protocol during PDCP re-establishment during SDT procedure, as specified in TS 38.323 [5]. Value cell indicates that ROHC header compression continues when the UE resumes for SDT in the same cell as the PCell when the RRCRelease message was received. Value rna indicates that ROHC header compression continues when the UE resumes for SDT in a cell belonging to the same RNA as the PCell where the RRCRelease message was received. If the field is absent, the UE releases any stored value for this field and the PDCP entity of the radio bearers configured for SDT always resets the ROHC header compression protocol during PDCP re-establishment when SDT procedure is initiated, as specified in TS 38.323 [5].

sdt-DRB-List Indicates the ID(s) of the DRB(s) that are configured for SDT. If size of the sequence is zero, then the UE assumes that none of the DRBs are configured for SDT. The network only configures MN terminated MCG bearers for SDT.

sdt-SRB2-Indication Indicates whether SRB2 is configured for SDT or not.

SDT-MAC-PHY-CG-Config field descriptions

cg-MT-SDT-MaxDurationToNextCG-Occasion The maximum duration until the next CG-SDT occasion as specified in TS 38.321 [3] for MT-SDT. If configured, the CG-SDT resource can only be used for the initial CG-SDT transmission if the duration between the initiation of the CG-SDT procedure and the next CG-SDT occasion is less than the value configured by this field.

cg-SDT-ConfigInitialBWP-DL Downlink BWP configuration for CG-SDT. If a UE is an (e)RedCap UE and if the initialDownlinkBWP-RedCap is configured in downlinkConfigCommon in SIB1, this field is configured for initialDownlinkBWP-RedCap, otherwise it is configured for initialDownlinkBWP.

cg-SDT-ConfigInitialBWP-NUL UL BWP configuration for CG-SDT on NUL carrier. If a UE is an (e)RedCap UE and if the initialUplinkBWP-RedCap is configured in uplinkConfigCommon in SIB1, this field is configured for initialUplinkBWP-RedCap, otherwise it is configured for initialUplinkBWP for NUL.

cg-SDT-ConfigInitialBWP-SUL UL BWP configuration for CG-SDT on SUL carrier configured for the initialUplinkBWP for SUL.

cg-SDT-ConfigLCH-RestrictionToAddModList, cg-SDT-ConfigLCH-RestrictionToAddModListExt, cg-SDT-ConfigLCH-RestrictionToReleaseList Lists for adding and releasing logical channel mapping restrictions for CG-SDT. If the network includes cg-SDT-ConfigLCH-RestrictionToAddModListExt, it includes the same number of entries, and listed in the same order, as in cg-SDT-ConfigLCH-RestrictionToAddModList.

cg-SDT-CS-RNTI The CS-RNTI value for CG-SDT as specified in TS 38.321 [3].

cg-SDT-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for CG-SDT as specified in TS 38.321 [3].

cg-SDT-TA-ValidationConfig Configuration for the RSRP based TA validation. If this field is not configured, then the UE does not perform RSRP based TA validation.

cg-SDT-timeAlignmentTimer TAT value for CG-SDT as specified in TS 38.321 [3]. The network always configures this field when sdt-MAC-PHY-CG-Config is configured. This field is associated with the PTAG indicated by tag-Id.

CG-SDT-ConfigLCH-Restriction field descriptions

allowedCG-List This restriction applies only when the UL grant is a configured grant for CG-SDT. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated CG-SDT configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any CG-SDT configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any CG-SDT configured grant configurations. If the field configuredGrantType1Allowed is present, only those CG-SDT configured grant type 1 configurations indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any CG-SDT configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3].

cg-SDT-MaxDurationToNextCG-Occasion The maximum duration until the next CG-SDT occasion for the logical channel identified by the logicalChannelIdentity as specified in TS 38.321 [3]. If configured, the CG-SDT resource can only be used for the initial CG-SDT transmission if the duration between the initiation of the CG-SDT procedure and the next CG-SDT occasion is less than the value configured by this field as specified in TS 38.321 [3].

configuredGrantType1Allowed If present, or if the capability lcp-Restriction as specified in TS 38.306 [26] is not supported, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1 for CG-SDT. Otherwise, UL MAC SDUs from this logical channel cannot be transmitted on a configured grant type 1 for CG-SDT. Corresponds to "configuredGrantType1Allowed" in TS 38.321 [3].

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer associated with a servedRadioBearer configured for SDT.

CG-SDT-TA-ValidationConfig field descriptions

cg-SDT-RSRP-ChangeThreshold The RSRP threshold for TA validation for CG-SDT as specified in TS 38.321 [3]. Value dB2 corresponds to 2 dB, value dB4 corresponds to 4 dB and so on.

SRS-PosRRC-InactiveConfig field descriptions

bwp-NUL BWP configuration for SRS for Positioning during the RRC_INACTIVE state in Normal Uplink Carrier. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

bwp-SUL BWP configuration for SRS for Positioning during the RRC_INACTIVE state in Supplementary Uplink Carrier. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

inactivePosSRS-RSRP-ChangeThreshold RSRP threshold for the increase/decrease of RSRP for time alignment validation as specified in TS 38.321 [3].

inactivePosSRS-TimeAlignmentTimer TAT value for SRS for positioning transmission during RRC_INACTIVE state as specified in TS 38.321 [3]. The network always configures this field when srs-PosRRC-Inactive is configured.

srs-PosConfigNUL SRS for Positioning configuration in RRC_INACTIVE state in Normal Uplink Carrier.

srs-PosConfigSUL SRS for Positioning configuration in RRC_INACTIVE state in Supplementary Uplink Carrier.

SRS-PosRRC-InactiveEnhancedConfig field descriptions

srs-PosRRC-InactiveAggBW-ConfigList SRS for positioning configuration with additional one or two carrier(s) configuration where the primary carrier is provided by srs-PosRRC-Inactive-r17 for bandwidth aggregation and additional carriers are provided by srs-PosRRC-InactiveAggBW-AdditionalCarriers-r18 and to be used in RRC_INACTIVE state (see TS 38.214 [19], clause 6.2.1.4.2). This field is included only if srs-PosRRC-Inactive-r17 and srs-PosRRC-InactiveAggBW-AdditionalCarriers-r18 are configured.

srs-PosRRC-InactiveValidityAreaNonPreConfig Contains the SRS for positioning configuration to be applied immediately upon reception. The configuration is valid across a number of cells as indicated in srs-PosConfigValidityArea in RRC_INACTIVE state.

srs-PosRRC-InactiveValidityAreaPreConfigList Contains the SRS for positioning configurations to be applied when a trigger for an event is met and which is valid across a number of cells comprising a validity area during RRC_INACTIVE state. For each validity area, the UE is preconfigured with only one SRS for positioning configuration.

srs-PosTx-Hopping Contains configuration related to the SRS for Positioning with frequency hopping for RRC_INACTIVE state (see TS 38.214 [19], clause 6.2.1.4.1). When this field is present and SP positioning SRS is configured, other positioning SRS resource sets configured under RRCRelease message with suspendConfig cannot be configured as semi-persistent.

srs-PosRRC-InactiveAggBW-AdditionalCarriers Additional carriers of Positioning SRS resource for carrier agregation for positioning SRS transmission without validity area in RRC_INACTIVE.

SRS-PosRRC-InactiveValidityAreaConfig field descriptions

autonomousTA-AdjustmentEnabled This field indicates that UE may adjust the TA value and stored RSRP autonomously after cell reselection within a validity area, if configured.

bwp-NUL BWP configuration for SRS for Positioning during the RRC_INACTIVE state in Normal Uplink Carrier. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

bwp-SUL BWP configuration for SRS for Positioning during the RRC_INACTIVE state in Supplementary Uplink Carrier. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

inactivePosSRS-ValidityAreaRSRP RSRP threshold for the increase/decrease of RSRP for validity area time alignment validation as specified in TS 38.321 [3].

inactivePosSRS-ValidityAreaTAT Time alignment timer value for SRS for positioning transmission during RRC_INACTIVE state which is applicable in a validity area.

srs-PosConfigValidityArea, srs-PosConfigValidityAreaExt This field provides list of cells present in the validity area. The maximum number of cells in a validity area is 32 which can be provided by using these two fields srs-PosConfigValidityArea and srs-PosConfigValidityAreaExt.

srs-PosRRC-InactiveAggBW-AdditionalCarriersPerVA SRS resource configuration on additional one or two carriers in each validity area for positioning SRS transmission for carrier aggregation in RRC_INACTIVE.

srs-PosRRC-InactiveAggBW-ConfigListPerVA Linkage for positioning SRS transmission in RRC_INACTIVE in each validity area. The field is included only if srs-PosRRC-InactiveAggBW-AdditionalCarriersPerVA-r18 is configured.

srs-PosRRC-InactiveValidityArea Provides a list of cells where SRS Positioning Configuration in RRC_INACTIVE state is valid.

SuspendConfig field descriptions

ncd-SSB-RedCapInitialBWP-SDT Indicates that the UE uses the RedCap-specific initial DL BWP associated with the NCD-SSB for SDT. The network configures this field if an (e)RedCap UE is configured with SDT in the RedCap-specific initial DL BWP not associated with CD-SSB. If configured, the NCD-SSB indicated by this field can only be used during the SDT procedure for CG-SDT or RA-SDT. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

ran-ExtendedPagingCycle The extended DRX (eDRX) cycle for RAN-initiated paging to be applied by the UE as defined in TS 38.304 [20]. Value rf256 corresponds to 256 radio frames, value rf512 corresponds to 512 radio frames and so on. Value of the field indicates an eDRX cycle which is shorter or equal to the IDLE mode eDRX cycle configured for the UE.

ran-ExtendedPagingCycleConfig The extended DRX (eDRX) configuraiton for RAN-initiated paging to be applied by the UE when the eDRX cycle for RAN-initiated paging is longer than 10.24s.

ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo.

ran-PagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

resumeIndication Indicates that the UE shall trigger the RRC connection resume procedure after receiving this RRCRelease message, as specified in clause 5.3.8.3. The network only includes this field in the RRCRelease message used to terminate an ongoing SDT procedure.

sl-UEIdentityRemote Indicates the C-RNTI to the L2 U2N Remote UE.

t380 Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on.

MulticastConfigInactive field descriptions

inactivePTM-Config Indicates the multicast session(s) that can be received in RRC_INACTIVE and optionally the corresponding PTM configuration (which includes mrb-ListMulticast, pdsch-ConfigIndex, mtch-SSB-MappingWindowIndex, etc.) for the cell where the multicast session(s) was configured in RRC_CONNECTED. If absent, UE considers that all joined multicast sessions can be received in RRC_INACTIVE.

inactiveMCCH-Config Indicates multicast MCCH/MTCH configuration for MBS multicast reception in RRC_INACTIVE in the cell where the multicast session(s) was configured in RRC_CONNECTED. Only SIB24 is allowed to be included.

ExtendedPagingCycleConfig field descriptions

extendedPagingCycle The eDRX cycle longer than 10.24 s for RAN-initiated paging to be applied by the UE. Value hf2 corresponds to 2 hyper frames, value hf4 corresponds to 4 hyper frames and so on. Value of the field is shorter than or equal to the IDLE mode eDRX cycle configured for the UE.

pagingPTWLength The length of paging transmission window for RAN-initiated paging to be applied by the UE as defined in TS 38.304 [20]. Value ms1280 corresponds to 1280 milliseconds, value ms2560 corresponds to 2560 milliseconds and so on.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE's RNAU; otherwise it is absent. |
| RANPaging | This field is optionally present, Need R, if the UE is configured with IDLE eDRX, see TS 24.501 [23]; otherwise the field is not present. |
| Redirection2 | The field is optionally present, Need R, if redirectedCarrierInfo is included; otherwise the field is not present. |
| SIB19 | The field is mandatory present, Need R, if SIB19 is not configured in the cell; otherwise the field is optionally present. |

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=                       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcResume                           RRCResume-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCResume-IEs ::=                   SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                               OPTIONAL, -- Need M

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                       OPTIONAL, -- Need M

measConfig                          MeasConfig                                                      OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                RRCResume-v1560-IEs                                             OPTIONAL

}

RRCResume-v1560-IEs ::=             SEQUENCE {

radioBearerConfig2                  OCTET STRING (CONTAINING RadioBearerConfig)                     OPTIONAL, -- Need M

sk-Counter                          SK-Counter                                                      OPTIONAL, -- Need N

nonCriticalExtension                RRCResume-v1610-IEs                                             OPTIONAL

}

RRCResume-v1610-IEs ::=             SEQUENCE {

idleModeMeasurementReq-r16          ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreMCG-SCells-r16               ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreSCG-r16                      ENUMERATED {true}                                               OPTIONAL, -- Need N

mrdc-SecondaryCellGroup-r16         CHOICE {

nr-SCG-r16                          OCTET STRING (CONTAINING RRCReconfiguration),

eutra-SCG-r16                       OCTET STRING

}                                                                                                   OPTIONAL, -- Cond RestoreSCG

needForGapsConfigNR-r16             SetupRelease {NeedForGapsConfigNR-r16}                          OPTIONAL, -- Need M

nonCriticalExtension                RRCResume-v1700-IEs                                             OPTIONAL

}

RRCResume-v1700-IEs ::=             SEQUENCE {

sl-ConfigDedicatedNR-r17            SetupRelease {SL-ConfigDedicatedNR-r16}                         OPTIONAL, -- Cond L2RemoteUE

sl-L2RemoteUE-Config-r17            SetupRelease {SL-L2RemoteUE-Config-r17}                         OPTIONAL, -- Cond L2RemoteUE

needForGapNCSG-ConfigNR-r17         SetupRelease {NeedForGapNCSG-ConfigNR-r17}                      OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17      SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                   OPTIONAL, -- Need M

scg-State-r17                       ENUMERATED {deactivated}                                        OPTIONAL, -- Need S

appLayerMeasConfig-r17              AppLayerMeasConfig-r17                                          OPTIONAL, -- Need M

nonCriticalExtension                RRCResume-v1800-IEs                                             OPTIONAL

}

RRCResume-v1800-IEs ::=             SEQUENCE {

needForInterruptionConfigNR-r18     ENUMERATED { disabled, enabled }                                OPTIONAL, -- Need M

reselectionMeasurementReq-r18       ENUMERATED { true }                                             OPTIONAL, -- Need N

validatedMeasurementsReq-r18        ENUMERATED { true }                                             OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                                                     OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

RRCResume-IEs field descriptions

appLayerMeasConfig This field is used to configure application layer measurements. This field is absent when the UE is configured to operate with shared spectrum channel access.

idleModeMeasurementReq This field indicates that the UE shall report the idle/inactive measurements, if available, to the network in the RRCResumeComplete message

masterCellGroup Configuration of the master cell group.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup (with at least reconfigurationWithSync), otherConfig and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message only include the field scg-Configuration with at least mobilityControlInfoSCG.

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForInterruptionConfigNR Indicates whether the UE shall report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. The network sets this field to enabled only if the needForGapsConfigNR is configured. The network sets this field to disabled if the needForGapsConfigNR is released.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs, multicast MRBs) including SDAP/PDCP.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

reselectionMeasurementReq This field indicates that the UE shall report the reselection measurements, if available, to the network in the RRCResumeComplete message.

restoreMCG-SCells Indicates that the UE shall restore the MCG SCells from the UE Inactive AS Context, if stored.

restoreSCG Indicates that the UE shall restore the SCG configurations from the UE Inactive AS Context, if stored.

scg-State Indicates that the SCG is in deactivated state.

sk-Counter A counter used to derive S-KgNB or S-KeNB based on the newly derived KgNB during RRC Resume. The field is only included when there is one or more RB with keyToUse set to secondary or mrdc-SecondaryCellGroup is included.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication/discovery used by L2 U2N Remote UE.

sl-L2RemoteUE-Config Contains L2 U2N relay operation related configurations used by L2 U2N Remote UE. The field is absent if appLayerMeasConfig or SRB4 is configured/not released.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE; otherwise it is absent. |
| RestoreSCG | The field is mandatory present if restoreSCG is included. It is optionally present, Need M, otherwise. |

#### –	RRCResumeComplete

The RRCResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::=                   SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcResumeComplete                       RRCResumeComplete-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCResumeComplete-IEs ::=               SEQUENCE {

dedicatedNAS-Message                    DedicatedNAS-Message                                                    OPTIONAL,

selectedPLMN-Identity                   INTEGER (1..maxPLMN)                                                    OPTIONAL,

uplinkTxDirectCurrentList               UplinkTxDirectCurrentList                                               OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1610-IEs                                             OPTIONAL

}

RRCResumeComplete-v1610-IEs ::=         SEQUENCE {

idleMeasAvailable-r16                   ENUMERATED {true}                                                       OPTIONAL,

measResultIdleEUTRA-r16                 MeasResultIdleEUTRA-r16                                                 OPTIONAL,

measResultIdleNR-r16                    MeasResultIdleNR-r16                                                    OPTIONAL,

scg-Response-r16                        CHOICE {

nr-SCG-Response                         OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                      OCTET STRING

}                                                                                                               OPTIONAL,

ue-MeasurementsAvailable-r16            UE-MeasurementsAvailable-r16                                            OPTIONAL,

mobilityHistoryAvail-r16                ENUMERATED {true}                                                       OPTIONAL,

mobilityState-r16                       ENUMERATED {normal, medium, high, spare}                                OPTIONAL,

needForGapsInfoNR-r16                   NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1640-IEs                                             OPTIONAL

}

RRCResumeComplete-v1640-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16 UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1700-IEs                                             OPTIONAL

}

RRCResumeComplete-v1700-IEs ::=         SEQUENCE {

needForGapNCSG-InfoNR-r17               NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17            NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1720-IEs                                             OPTIONAL

}

RRCResumeComplete-v1720-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentMoreCarrierList-r17 UplinkTxDirectCurrentMoreCarrierList-r17                               OPTIONAL,

nonCriticalExtension                     RRCResumeComplete-v1800-IEs                                            OPTIONAL

}

RRCResumeComplete-v1800-IEs ::=         SEQUENCE {

needForInterruptionInfoNR-r18           NeedForInterruptionInfoNR-r18                                           OPTIONAL,

musim-CapRestrictionInd-r18             ENUMERATED {true}                                                       OPTIONAL,

flightPathInfoAvailable-r18             ENUMERATED {true}                                                       OPTIONAL,

measConfigReportAppLayerAvailable-r18   ENUMERATED {true}                                                       OPTIONAL,

measResultReselectionNR-r18             MeasResultIdleNR-r16                                                    OPTIONAL,

reselectionMeasAvailable-r18            ENUMERATED {true}                                                       OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1900-IEs                                             OPTIONAL

}

RRCResumeComplete-v1900-IEs ::=         SEQUENCE {

applicabilityReportList-r19             ApplicabilityReportList-r19                                             OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

RRCResumeComplete-IEs field descriptions

applicabilityReportList The applicability reports related to prediction configurations.

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

measConfigReportAppLayerAvailable Indication that the UE has at least one application layer measurement configuration with appLayerIdleInactiveConfig configured.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE.

musim-CapRestrictionInd This field indicates the UE temporary capability restriction due to MUSIM operation.

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForGapNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands

needForGapNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands

needForInterruptionInfoNR This field indicates whether interruption is needed while performing measurement on NR target bands without measurement gap.

reselectionMeasAvailable Indication that the UE has reselection measurement report available.

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentMoreCarrierList The Tx Direct Current locations for the configured intra-band CA requested by reportUplinkTxDirectCurrentMoreCarrier-r17.

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).

#### –	RRCResumeRequest

The RRCResumeRequest message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCResumeRequest message

-- ASN1START

-- TAG-RRCRESUMEREQUEST-START

RRCResumeRequest ::=            SEQUENCE {

rrcResumeRequest            RRCResumeRequest-IEs

}

RRCResumeRequest-IEs ::=        SEQUENCE {

resumeIdentity                  ShortI-RNTI-Value,

resumeMAC-I                     BIT STRING (SIZE (16)),

resumeCause                     ResumeCause,

spare                           BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST-STOP

-- ASN1STOP

RRCResumeRequest-IEs field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. The network is not expected to reject an RRCResumeRequest due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the AS security configuration as specified in 5.3.13.3.

#### –	RRCResumeRequest1

The RRCResumeRequest1 message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH1

Direction: UE to Network

RRCResumeRequest1 message

-- ASN1START

-- TAG-RRCRESUMEREQUEST1-START

RRCResumeRequest1 ::=         SEQUENCE {

rrcResumeRequest1          RRCResumeRequest1-IEs

}

RRCResumeRequest1-IEs ::=    SEQUENCE {

resumeIdentity               I-RNTI-Value,

resumeMAC-I                  BIT STRING (SIZE (16)),

resumeCause                  ResumeCause,

spare                        BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST1-STOP

-- ASN1STOP

RRCResumeRequest1-IEs field descriptions

resumeCause Provides the resume cause for the RRCResumeRequest1 as provided by the upper layers or RRC. A gNB is not expected to reject an RRCResumeRequest1 due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the AS security configuration as specified in 5.3.13.3.

#### –	RRCSetup

The RRCSetup message is used to establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCSetup message

-- ASN1START

-- TAG-RRCSETUP-START

RRCSetup ::=                        SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcSetup                            RRCSetup-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSetup-IEs ::=                    SEQUENCE {

radioBearerConfig                   RadioBearerConfig,

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig),

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                RRCSetup-v1700-IEs                                                      OPTIONAL

}

RRCSetup-v1700-IEs ::=              SEQUENCE {

sl-ConfigDedicatedNR-r17            SL-ConfigDedicatedNR-r16                           OPTIONAL, -- Cond L2RemoteUE

sl-L2RemoteUE-Config-r17            SL-L2RemoteUE-Config-r17                           OPTIONAL, -- Cond L2RemoteUE

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

-- TAG-RRCSETUP-STOP

-- ASN1STOP

RRCSetup-IEs field descriptions

masterCellGroup The network configures only the RLC bearer for the SRB1, mac-CellGroupConfig, physicalCellGroupConfig and spCellConfig.

radioBearerConfig Only SRB1 can be configured in RRC setup.

sl-ConfigDedicatedNR Contains dedicated configurations for NR sidelink communication. The network configures only the PC5 Relay RLC channel and sl-PHY-MAC-RLC-Config used for the SRB1.

sl-L2RemoteUE-Config Contains dedicated configurations used for L2 U2N relay related operation. The network configures only the SRAP configuration used for the SRB1 and local UE ID.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE; otherwise it is absent. |

#### –	RRCSetupComplete

The RRCSetupComplete message is used to confirm the successful completion of an RRC connection establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCSetupComplete message

-- ASN1START

-- TAG-RRCSETUPCOMPLETE-START

RRCSetupComplete ::=                SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcSetupComplete                    RRCSetupComplete-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSetupComplete-IEs ::=            SEQUENCE {

selectedPLMN-Identity               INTEGER (1..maxPLMN),

registeredAMF                       RegisteredAMF                                   OPTIONAL,

guami-Type                          ENUMERATED {native, mapped}                     OPTIONAL,

s-NSSAI-List                        SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI  OPTIONAL,

dedicatedNAS-Message                DedicatedNAS-Message,

ng-5G-S-TMSI-Value                  CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

ng-5G-S-TMSI-Part2                  BIT STRING (SIZE (9))

}                                                                                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING (CONTAINING RRCSetupComplete-v15s0-IEs)  OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1610-IEs                      OPTIONAL

}

-- Regular non-critical extensions:

RRCSetupComplete-v1610-IEs ::=      SEQUENCE {

iab-NodeIndication-r16              ENUMERATED {true}                               OPTIONAL,

idleMeasAvailable-r16               ENUMERATED {true}                               OPTIONAL,

ue-MeasurementsAvailable-r16        UE-MeasurementsAvailable-r16                    OPTIONAL,

mobilityHistoryAvail-r16            ENUMERATED {true}                               OPTIONAL,

mobilityState-r16                   ENUMERATED {normal, medium, high, spare}        OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1690-IEs                      OPTIONAL

}

RRCSetupComplete-v1690-IEs ::=      SEQUENCE {

ul-RRC-Segmentation-r16             ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1700-IEs                      OPTIONAL

}

RRCSetupComplete-v1700-IEs ::=      SEQUENCE {

onboardingRequest-r17               ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1800-IEs                      OPTIONAL

}

RRCSetupComplete-v1800-IEs ::=      SEQUENCE {

ncr-NodeIndication-r18              ENUMERATED {true}                               OPTIONAL,

musim-CapRestrictionInd-r18         ENUMERATED {true}                               OPTIONAL,

flightPathInfoAvailable-r18         ENUMERATED {true}                               OPTIONAL,

measConfigReportAppLayerAvailable-r18 ENUMERATED {true}                             OPTIONAL,

mobileIAB-NodeIndication-r18        ENUMERATED {true}                               OPTIONAL,

reselectionMeasAvailable-r18        ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                      OPTIONAL

}

-- Late non-critical extensions:

RRCSetupComplete-v15s0-IEs ::=      SEQUENCE {

-- Following field is only for REL-15 late non-critical extensions

lateNonCriticalExtension            OCTET STRING                                    OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v16j0-IEs                      OPTIONAL

}

RRCSetupComplete-v16j0-IEs ::=      SEQUENCE {

-- Following field is only for REL-16 late non-critical extensions

lateNonCriticalExtension            OCTET STRING                                    OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v17b0-IEs                      OPTIONAL

}

RRCSetupComplete-v17b0-IEs ::=      SEQUENCE {

ul-RRC-MaxCapaSegments-r17          ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                      OPTIONAL

}

RegisteredAMF ::=                   SEQUENCE {

plmn-Identity                       PLMN-Identity                                   OPTIONAL,

amf-Identifier                      AMF-Identifier

}

-- TAG-RRCSETUPCOMPLETE-STOP

-- ASN1STOP

RRCSetupComplete-IEs field descriptions

guami-Type This field is used to indicate whether the GUAMI included is native (derived from native 5G-GUTI) or mapped (from EPS, derived from EPS GUTI) as specified in TS 24.501 [23].

iab-NodeIndication This field is used to indicate that the connection is being established by an IAB-node as specified in TS 38.300 [2]. If this field is included, the UE shall not include the field mobileIAB-NodeIndication.

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

measConfigReportAppLayerAvailable Indication that the UE has at least one application layer measurement configuration with appLayerIdleInactiveConfig configured.

mobileIAB-NodeIndication This field is used to indicate that the connection is being established by a mobile IAB-node as specified in TS 38.300 [2]. If this field is included, the UE shall not include the field iab-NodeIndication.

mobilityState This field indicates the UE mobility state (as defined in TS 38.304 [20], clause 5.2.4.3) just prior to UE going into RRC_CONNECTED state. The UE indicates the value of medium and high when being in Medium-mobility and High-mobility states respectively. Otherwise the UE indicates the value normal.

musim-CapRestrictionInd This field indicates the UE temporary capability restriction due to MUSIM operation.

ncr-NodeIndication This field is used to indicate that the connection is being established by an NCR-node as specified in TS 38.300 [2].

ng-5G-S-TMSI-Part2 The leftmost 9 bits of 5G-S-TMSI.

onboardingRequest This field indicates that the connection is being established for UE onboarding in the selected onboarding SNPN, see TS 23.501 [32].

registeredAMF This field is used to transfer the GUAMI of the AMF where the UE is registered, as provided by upper layers, see TS 23.003 [21].

reselectionMeasAvailable Indication that the UE has reselection measurement report available.

selectedPLMN-Identity Index of the PLMN or SNPN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

ul-RRC-MaxCapaSegments This field indicates that the UE supports uplink RRC segmentation of UECapabilityInformation according to the network indication rrc-MaxCapaSegAllowed.

ul-RRC-Segmentation This field indicates the UE supports uplink RRC segmentation of UECapabilityInformation according to the network indication rrc-SegAllowed.

#### –	RRCSetupRequest

The RRCSetupRequest message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=                 SEQUENCE {

rrcSetupRequest                     RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=             SEQUENCE {

ue-Identity                         InitialUE-Identity,

establishmentCause                  EstablishmentCause,

spare                               BIT STRING (SIZE (1))

}

InitialUE-Identity ::=              CHOICE {

ng-5G-S-TMSI-Part1                  BIT STRING (SIZE (39)),

randomValue                         BIT STRING (SIZE (39))

}

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

RRCSetupRequest-IEs field descriptions

establishmentCause Provides the establishment cause for the RRCSetupRequest in accordance with the information received from upper layers. gNB is not expected to reject an RRCSetupRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to facilitate contention resolution by lower layers.

InitialUE-Identity field descriptions

ng-5G-S-TMSI-Part1 The rightmost 39 bits of 5G-S-TMSI.

randomValue Integer value in the range 0 to 239 – 1.

#### –	RRCSystemInfoRequest

The RRCSystemInfoRequest message is used to request SI message(s) required by the UE as specified in clause 5.2.2.3.3 and 5.2.2.3.3a.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSystemInfoRequest message

-- ASN1START

-- TAG-RRCSYSTEMINFOREQUEST-START

RRCSystemInfoRequest ::=            SEQUENCE {

criticalExtensions                  CHOICE {

rrcSystemInfoRequest                RRCSystemInfoRequest-IEs,

criticalExtensionsFuture-r16        CHOICE {

rrcPosSystemInfoRequest-r16         RRC-PosSystemInfoRequest-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

}

RRCSystemInfoRequest-IEs ::=    SEQUENCE {

requested-SI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                               BIT STRING (SIZE (12))

}

RRC-PosSystemInfoRequest-r16-IEs ::=  SEQUENCE {

requestedPosSI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                                 BIT STRING (SIZE (11))

}

-- TAG-RRCSYSTEMINFOREQUEST-STOP

-- ASN1STOP

RRCSystemInfoRequest-IEs field descriptions

requested-SI-List Contains a list of requested SI messages which are configured by schedulingInfoList in si-SchedulingInfo and schedulingInfoList2 in si-SchedulingInfo-v1700 (if present) in SIB1. If si-SchedulingInfo-v1700 is not present: - According to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on. If si-SchedulingInfo-v1700 is present: - The UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. - According to the order of entry in the list of concatenated SI messages, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.

requestedPosSI-List Contains a list of requested SI messages which are configured by posSchedulingInfoList in posSI-SchedulingInfo and schedulingInfoList2 in si-SchedulingInfo-v1700 (if present) in SIB1. If si-SchedulingInfo-v1700 is not present: - According to the order of entry in the list of SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on. If si-SchedulingInfo-v1700 is present: - The UE creates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. - According to the order of entry in the list of concatenated SI messages, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.

#### –	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                SEQUENCE {

criticalExtensions                       CHOICE {

scgFailureInformation                    SCGFailureInformation-IEs,

criticalExtensionsFuture                 SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=            SEQUENCE {

failureReportSCG                         FailureReportSCG                    OPTIONAL,

nonCriticalExtension                     SCGFailureInformation-v1590-IEs     OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=      SEQUENCE {

lateNonCriticalExtension                 OCTET STRING                        OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                         OPTIONAL

}

FailureReportSCG ::=                     SEQUENCE {

failureType                              ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList                       MeasResultFreqList                                    OPTIONAL,

measResultSCG-Failure                    OCTET STRING (CONTAINING MeasResultSCG-Failure)       OPTIONAL,

...,

[[

locationInfo-r16                         LocationInfo-r16                                      OPTIONAL,

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1} OPTIONAL

]],

[[

previousPSCellId-r17                 SEQUENCE {

physCellId-r17                       PhysCellId,

carrierFreq-r17                      ARFCN-ValueNR

}                                                             OPTIONAL,

failedPSCellId-r17                   SEQUENCE {

physCellId-r17                       PhysCellId,

carrierFreq-r17                      ARFCN-ValueNR

}                                                            OPTIONAL,

timeSCGFailure-r17                   INTEGER (0..1023)        OPTIONAL,

perRAInfoList-r17                    PerRAInfoList-r16          OPTIONAL

]],

[[

perRAInfoList-v17b0                  PerRAInfoList-v1660      OPTIONAL

]],

[[

perRAInfoList-v1840                  PerRAInfoList-v1800      OPTIONAL

]],

[[

cho-WithCandidateSCGInfoList-r19       Cho-WithCandidateSCGInfoList-r19        OPTIONAL

]]

}

MeasResultFreqList ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

previousPSCellId This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure, this field is absent. In case of subsequent CPC, this field indicates the physical cell identity and carrier frequency of the source PSCell associated to the last executed CPC.

failedPSCellId This field indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition.

timeSCGFailure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

#### –	SCGFailureInformationEUTRA

The SCGFailureInformationEUTRA message is used to provide information regarding E-UTRA SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformationEUTRA message

-- ASN1START

-- TAG-SCGFAILUREINFORMATIONEUTRA-START

SCGFailureInformationEUTRA ::=                SEQUENCE {

criticalExtensions                                CHOICE {

scgFailureInformationEUTRA                       SCGFailureInformationEUTRA-IEs,

criticalExtensionsFuture                          SEQUENCE {}

}

}

SCGFailureInformationEUTRA-IEs ::=           SEQUENCE {

failureReportSCG-EUTRA                           FailureReportSCG-EUTRA                      OPTIONAL,

nonCriticalExtension                              SCGFailureInformationEUTRA-v1590-IEs                                    OPTIONAL

}

SCGFailureInformationEUTRA-v1590-IEs ::=  SEQUENCE {

lateNonCriticalExtension                  OCTET STRING            OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                  OPTIONAL

}

FailureReportSCG-EUTRA ::=                       SEQUENCE {

failureType                                           ENUMERATED {

t313-Expiry, randomAccessProblem,rlc-MaxNumRetx,

scg-ChangeFailure, spare4,

spare3, spare2, spare1},

measResultFreqListMRDC                             MeasResultFreqListFailMRDC                                                   OPTIONAL,

measResultSCG-FailureMRDC                         OCTET STRING                                                         OPTIONAL,

...,

[[

locationInfo-r16                                    LocationInfo-r16                                                         OPTIONAL

]]

}

MeasResultFreqListFailMRDC ::=      SEQUENCE (SIZE (1.. maxFreq)) OF MeasResult2EUTRA

-- TAG-SCGFAILUREINFORMATIONEUTRA-STOP

-- ASN1STOP

SCGFailureInformationEUTRA field descriptions

measResultFreqListMRDC The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig.

measResultSCG-FailureMRDC Includes the E-UTRA MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by the E-UTRA RRCConnectionReconfiguration message.

#### –	SecurityModeCommand

The SecurityModeCommand message is used to command the activation of AS security.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

SecurityModeCommand message

-- ASN1START

-- TAG-SECURITYMODECOMMAND-START

SecurityModeCommand ::=             SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

securityModeCommand                 SecurityModeCommand-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SecurityModeCommand-IEs ::=         SEQUENCE {

securityConfigSMC                   SecurityConfigSMC,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

SecurityConfigSMC ::=               SEQUENCE {

securityAlgorithmConfig             SecurityAlgorithmConfig,

...

}

-- TAG-SECURITYMODECOMMAND-STOP

-- ASN1STOP

#### –	SecurityModeComplete

The SecurityModeComplete message is used to confirm the successful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeComplete message

-- ASN1START

-- TAG-SECURITYMODECOMPLETE-START

SecurityModeComplete ::=            SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

securityModeComplete                SecurityModeComplete-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SecurityModeComplete-IEs ::=        SEQUENCE {

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-SECURITYMODECOMPLETE-STOP

-- ASN1STOP

#### –	SecurityModeFailure

The SecurityModeFailure message is used to indicate an unsuccessful completion of a security mode command.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SecurityModeFailure message

-- ASN1START

-- TAG-SECURITYMODEFAILURE-START

SecurityModeFailure ::=             SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

securityModeFailure                 SecurityModeFailure-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SecurityModeFailure-IEs ::=         SEQUENCE {

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-SECURITYMODEFAILURE-STOP

-- ASN1STOP

#### –	SIB1

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                                              OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)                                              OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                                           OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo                                               OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB                                      OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}                                               OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                                               OPTIONAL,   -- Need R

ue-TimersAndConstants               UE-TimersAndConstants                                           OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                                           OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List                                         OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                                                   OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                SIB1-v1610-IEs                                                  OPTIONAL

}

SIB1-v1610-IEs ::=               SEQUENCE {

idleModeMeasurementsEUTRA-r16    ENUMERATED{true}                                                   OPTIONAL,  -- Need R

idleModeMeasurementsNR-r16       ENUMERATED{true}                                                   OPTIONAL,  -- Need R

posSI-SchedulingInfo-r16         PosSI-SchedulingInfo-r16                                           OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1630-IEs                                                     OPTIONAL

}

SIB1-v1630-IEs ::=               SEQUENCE {

uac-BarringInfo-v1630            SEQUENCE {

uac-AC1-SelectAssistInfo-r16     SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AC1-SelectAssistInfo-r16

}                                                                                                   OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1700-IEs                                                     OPTIONAL

}

SIB1-v1700-IEs ::=               SEQUENCE {

hsdn-Cell-r17                        ENUMERATED {true}                                              OPTIONAL,  -- Need R

uac-BarringInfo-v1700                SEQUENCE {

uac-BarringInfoSetList-v1700         UAC-BarringInfoSetList-v1700

}                                                                                                   OPTIONAL,  -- Cond MINT

sdt-ConfigCommon-r17                 SDT-ConfigCommonSIB-r17                                        OPTIONAL,  -- Need R

redCap-ConfigCommon-r17              RedCap-ConfigCommonSIB-r17                                     OPTIONAL,  -- Need R

featurePriorities-r17        SEQUENCE {

redCapPriority-r17           FeaturePriority-r17                                                OPTIONAL,  -- Need R

slicingPriority-r17          FeaturePriority-r17                                                OPTIONAL,  -- Need R

msg3-Repetitions-Priority-r17 FeaturePriority-r17                                               OPTIONAL,  -- Need R

sdt-Priority-r17             FeaturePriority-r17                                                OPTIONAL   -- Need R

}                                                                                                   OPTIONAL,  -- Need R

si-SchedulingInfo-v1700      SI-SchedulingInfo-v1700                                                OPTIONAL,  -- Need R

hyperSFN-r17                 BIT STRING (SIZE (10))                                                 OPTIONAL,  -- Need R

eDRX-AllowedIdle-r17         ENUMERATED {true}                                                      OPTIONAL,  -- Need R

eDRX-AllowedInactive-r17     ENUMERATED {true}                                                      OPTIONAL,  -- Cond EDRX-RC

intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed}                                     OPTIONAL,  -- Need S

cellBarredNTN-r17            ENUMERATED {barred, notBarred}                                         OPTIONAL,  -- Need S

nonCriticalExtension         SIB1-v1740-IEs                                                         OPTIONAL

}

SIB1-v1740-IEs ::=               SEQUENCE {

si-SchedulingInfo-v1740          SI-SchedulingInfo-v1740                                            OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1800-IEs                                                     OPTIONAL

}

SIB1-v1800-IEs ::=               SEQUENCE {

ncr-Support-r18                  ENUMERATED {true}                                                  OPTIONAL,  -- Need S

mt-SDT-ConfigCommonSIB-r18       MT-SDT-ConfigCommonSIB-r18                                         OPTIONAL,  -- Need R

musim-CapRestrictionAllowed-r18  ENUMERATED {true}                                                  OPTIONAL,  -- Need R

featurePriorities-v1800          SEQUENCE {

msg1-Repetitions-Priority-r18    FeaturePriority-r17                                            OPTIONAL,  -- Need R

eRedCapPriority-r18              FeaturePriority-r17                                            OPTIONAL   -- Need R

}                                                                                                   OPTIONAL,  -- Need R

si-SchedulingInfo-v1800          SI-SchedulingInfo-v1800                                            OPTIONAL,  -- Need R

cellBarredATG-r18                ENUMERATED {barred, notBarred}                                     OPTIONAL,  -- Need S

cellBarredNES-r18                ENUMERATED {notBarred}                                             OPTIONAL,  -- Need R

mobileIAB-Cell-r18               ENUMERATED {true}                                                  OPTIONAL,  -- Need R

eDRX-AllowedInactive-r18         ENUMERATED {true}                                                  OPTIONAL,  -- Cond EDRX-RC

intraFreqReselection-eRedCap-r18 ENUMERATED {allowed, notAllowed}                                   OPTIONAL,  -- Need S

nonServingCellMII-r18            ENUMERATED {true}                                                  OPTIONAL,  -- Need R

sdt-BeamFailureRecoveryProhibitTimer-r18  ENUMERATED {ms50, ms100, ms200, ms500, ms1000, ms1500, ms2000, ms3000}

OPTIONAL,  -- Need R

eRedCap-ConfigCommon-r18         ERedCap-ConfigCommonSIB-r18                                        OPTIONAL,  -- Need R

cellBarredFixedVSAT-r18          ENUMERATED {barred, notBarred}                                     OPTIONAL,  -- Cond NTN

cellBarredMobileVSAT-r18         ENUMERATED {barred, notBarred}                                     OPTIONAL,  -- Cond NTN

reselectionMeasurementsNR-r18    ENUMERATED{true}                                                   OPTIONAL,  -- Need R

cellBarred2RxXR-r18              ENUMERATED {barred}                                                OPTIONAL,  -- Need R

intraFreqReselection2RxXR-r18    ENUMERATED {allowed, notAllowed}                                   OPTIONAL,  -- Cond 2RxXR

barringExemptEmergencyCall-r18   ENUMERATED {true}                                                  OPTIONAL,  -- Cond EM-Barring

n3c-Support-r18                  ENUMERATED {true}                                                  OPTIONAL,  -- Need R

nonCriticalExtension             SEQUENCE {}                                                        OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16 ::=     ENUMERATED {a, b, c, notConfigured}

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range                                                            OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

RedCap-ConfigCommonSIB-r17 ::= SEQUENCE {

halfDuplexRedCapAllowed-r17    ENUMERATED {true}                                                    OPTIONAL,  -- Need R

cellBarredRedCap-r17           SEQUENCE {

cellBarredRedCap1Rx-r17        ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17        ENUMERATED {barred, notBarred}

}                                                                                                   OPTIONAL,  -- Need R

...

}

ERedCap-ConfigCommonSIB-r18 ::= SEQUENCE {

cellBarred-eRedCap-r18           SEQUENCE {

cellBarred-eRedCap1Rx-r18        ENUMERATED {barred, notBarred},

cellBarred-eRedCap2Rx-r18        ENUMERATED {barred, notBarred}

}

}

FeaturePriority-r17 ::= INTEGER (0..7)

MT-SDT-ConfigCommonSIB-r18 ::=       SEQUENCE {

mt-SDT-RSRP-Threshold-r18            RSRP-Range                                                            OPTIONAL, -- Need S

sdt-LogicalChannelSR-DelayTimer-r18  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Cond MT-SDT1

t319a-r18                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}                                  OPTIONAL  -- Cond MT-SDT2

}

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

barringExemptEmergencyCall Indicates whether the cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [20].

cellBarred2RxXR Value barred means that the cell is barred for a 2Rx XR UE indicating supportOf2RxXR for the selected frequency band as specified in clause 5.2.2.4.2. This field is ignored by all UEs not indicating supportOf2RxXR for the selected frequency band. This field may be configured only if the cell operates in a frequency band where 4Rx antenna ports are mandated as specified in TS 38.101-1 [15]. If this field is absent on a cell operating in a frequency band where 4RX antenna ports are mandated, a 2RX XR UE shall treat the cell as not barred, as specified in TS 38.304 [20].

cellBarredATG Value barred means that the cell is barred for connectivity to ATG, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to ATG. If not present, the UE considers the cell is not allowed for connectivity to ATG, as defined in TS 38.304 [20]. This field is only applicable to ATG-capable UEs.

cellBarred-eRedCap1Rx Value barred means that the cell is barred for an eRedCap UE supporting 1 Rx branch on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs. An eRedCap UE supporting 2 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarred-eRedCap2Rx is set to notBarred.

cellBarred-eRedCap2Rx Value barred means that the cell is barred for an eRedCap UE supporting 2 Rx branches on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs. An eRedCap UE supporting 1 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarred-eRedCap1Rx is set to notBarred.

cellBarredFixedVSAT Value barred means that the cell is barred for fixed VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for fixed VSAT UEs. This field is ignored by non-VSAT UEs.

cellBarredMobileVSAT Value barred means that the cell is barred for mobile VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for mobile VSAT UEs. This field is ignored by non-VSAT UEs.

cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX on the selected frequency band as described in 5.2.2.4.2.

cellBarredNTN Value barred means that the cell is barred for connectivity to NTN, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to NTN. If not present, the UE considers the cell is not allowed for connectivity to NTN, as defined in TS 38.304 [20]. This field is only applicable to NTN-capable UEs.

cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE supporting 1 Rx branch on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs. A RedCap UE supporting 2 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarredRedCap2Rx is set to notBarred.

cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE supporting 2 Rx branches on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs. A RedCap UE supporting 1 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarredRedCap1Rx is set to notBarred.

cellSelectionInfo Parameters for cell selection related to the serving cell.

eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

eDRX-AllowedIdle The presence of this field indicates that extended DRX for CN paging is allowed in the cell for UEs in RRC_IDLE or RRC_INACTIVE. The UE shall stop using extended DRX for CN paging in RRC_IDLE or RRC_INACTIVE if eDRX-AllowedIdle is not present.

eDRX-AllowedInactive The presence of eDRX-AllowedInactive-r17 indicates that extended DRX cycle equal to or shorter than 10.24 s for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX cycle equal to or shorter than 10.24 s for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive-r17 is not present. The presence of eDRX-AllowedInactive-r18 indicates that extended DRX cycle longer than 10.24 s for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX cycle longer than 10.24 s for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive-r18 is not present.

featurePriorities Indicates priorities for features, such as (e)RedCap, Slicing, SDT, MSG1-Repetitions and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

halfDuplexRedCap-Allowed The presence of this field indicates that the cell supports half-duplex FDD (e)RedCap UEs.

hsdn-Cell This field indicates this is a HSDN cell as specified in TS 38.304 [20].

hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. This field is excluded when determining changes in system information, i.e. changes of hyper SFN should not result in system information change notifications.

idleModeMeasurementsEUTRA This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements.

idleModeMeasurementsNR This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements.

ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode.

intraFreqReselection2RxXR This field controls cell selection/reselection to intra-frequency cells for 2Rx XR UEs when this cell is barred or treated as barred by the 2Rx XR UE, as specified in TS 38.304 [20]. This field is ignored by all UEs that are not 2Rx XR UEs. This field may be configured only if the cell operates in a frequency band where 4Rx antenna ports are mandated, as specified in TS 38.101-1 [15].

intraFreqReselection-eRedCap Controls cell selection/reselection to intra-frequency cells for eRedCap UEs when this cell is barred, or treated as barred by the eRedCap UE, as specified in TS 38.304 [20]. If not present, an eRedCap UE treats the cell as barred, i.e., the UE considers that the cell does not support eRedCap.

intraFreqReselectionRedCap Controls cell selection/reselection to intra-frequency cells for RedCap UEs when this cell is barred, or treated as barred by the RedCap UE, as specified in TS 38.304 [20]. If not present, a RedCap UE treats the cell as barred, i.e.,the UE considers that the cell does not support RedCap.

mobileIAB-Cell The presence of this field indicates that this is a mobile IAB cell.

mt-SDT-RSRP-Threshold RSRP threshold used to determine whether MT-SDT procedure can be initiated, as specified in TS 38.321 [3]. If the field is absent, and the field sdt-RSRP-Threshold is present, the UE applies the value in the field sdt-RSRP-Threshold.

musim-CapRestrictionAllowed Indicates the UE is allowed to send the musim-CapRestrictionInd in RRCSetupComplete, RRCResumeComplete and RRCReestablishmentComplete messages.

n3c-Support This field indicates the support of N3C MP. If the field is present, the UE can perform early detection of candidate N3C relay UEs. If absent, a UE is not required to perform early detection of candidate N3C relay UEs.

ncr-Support This field combines both the support of NCR and the cell status for NCR. If the field is present, the cell supports NCR and the cell is also considered as a candidate for cell (re)selection for NCR-node; if the field is absent, the cell does not support NCR and/or the cell is barred for NCR-node.

nonServingCellMII Indicates whether the MBSInterestIndication message for MBS broadcast reception on a non-serving cell is allowed to be transmitted to the serving gNB.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for serving cell. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffset Parameter "Qqualminoffset" in TS 38.304 [20]. Actual value Qqualminoffset = field value [dB]. If the field is absent, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

q-RxLevMinOffset Parameter "Qrxlevminoffset" in TS 38.304 [20]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

reselectionMeasurementsNR This field indicates that a UE that is configured for NR reselection measurements shall report availability of these measurements when establishing or resuming a connection in this cell.

sdt-BeamFailureRecoveryProhibitTimer The value of the prohibit timer used for RACH for beam failure indication during SDT as specified in TS 38.321 [3]. Value ms50 corresponds to 50 milliseconds, value ms100 corresponds to 100 milliseconds and so on.

sdt-DataVolumeThreshold Data volume threshold used to determine whether SDT can be initiated, as specified in TS 38.321 [3]. Value byte32 corresponds to 32 bytes, value byte100 corresponds to 100 bytes, and so on.

sdt-LogicalChannelSR-DelayTimer The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT, as specified in TS 38.321 [3]. Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on. If sdt-LogicalChannelSR-DelayTimer-r18 is absent and sdt-LogicalChannelSR-DelayTimer-r17 is present then, the UE applies the value configured in sdt-LogicalChannelSR-DelayTimer-r17 for this field. If this field is not configured, then logicalChannelSR-DelayTimer is not applied for SDT logical channels.

sdt-RSRP-Threshold RSRP threshold used to determine whether SDT procedure can be initiated, as specified in TS 38.321 [3].

servingCellConfigCommon Configuration of the serving cell.

t319a Initial value of the timer T319a used for detection of SDT failure. Value ms100 corresponds to 100 milliseconds, value ms200 corresponds to 200 milliseconds and so on. If t319a-r18 is absent, the UE applies the value configured in t319a-r17.

uac-AccessCategory1-SelectionAssistanceInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. If plmnCommon is chosen, the UAC-AccessCategory1-SelectionAssistanceInfo is applicable to all the PLMNs and SNPNs in plmn-IdentityInfoList and npn-IdentityInfoList. If individualPLMNList is chosen, the 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. If uac-AC1-SelectAssistInfo-r16 is present, the UE shall ignore the uac-AccessCategory1-SelectionAssistanceInfo.

uac-AC1-SelectAssistInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. The 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. Value notConfigured indicates that Access Category1 is not configured for the corresponding PLMN/SNPN.

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs/SNPNs, unless overwritten by the PLMN/SNPN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in clause 5.3.14.2.

ue-TimersAndConstants Timer and constant values to be used by the UE. The cell operating as PCell always provides this field.

useFullResumeID Indicates which resume identifier and Resume request message should be used. UE uses fullI-RNTI and RRCResumeRequest1 if the field is present, or shortI-RNTI and RRCResumeRequest if the field is absent.

| Conditional Presence | Explanation |
| --- | --- |
| 2RxXR | This field is mandatory present if cellBarred2RxXR is present, otherwise it is absent, Need R. |
| EDRX-RC | The field is optionally present, Need R, in a cell that enables eDRX-AllowedIdle, otherwise it is absent. |
| EM-Barring | The field is optionally present, Need R, in a cell that supports (e)RedCap or 2Rx XR UEs, otherwise it is absent. |
| MINT | The field is optionally present, Need R, in a cell that provides a configuration for disaster roaming, otherwise it is absent, Need R. |
| MT-SDT1 | This field is optionally present, Need S, in a cell that supports MT-SDT if sdt-ConfigCommon-r17 is not present, otherwise it is absent. |
| MT-SDT2 | This field is mandatory present in a cell that supports MT-SDT if sdt-ConfigCommon-r17 is not present, otherwise it is absent. |
| NTN | The field is optionally present, Need S, in a cell where cellBarredNTN is included with value notBarred, otherwise it is absent. |
| Standalone | The field is mandatory present in a cell that supports standalone operation, otherwise it is absent. |

#### –	SidelinkUEInformationNR

The SidelinkUEinformationNR message is used for the indication of NR sidelink UE information to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SidelinkUEInformationNR message

-- ASN1START

-- TAG-SIDELINKUEINFORMATIONNR-START

SidelinkUEInformationNR-r16::=         SEQUENCE {

criticalExtensions                  CHOICE {

sidelinkUEInformationNR-r16         SidelinkUEInformationNR-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SidelinkUEInformationNR-r16-IEs ::=    SEQUENCE {

sl-RxInterestedFreqList-r16            SL-InterestedFreqList-r16           OPTIONAL,

sl-TxResourceReqList-r16               SL-TxResourceReqList-r16            OPTIONAL,

sl-FailureList-r16                     SL-FailureList-r16                  OPTIONAL,

lateNonCriticalExtension               OCTET STRING                        OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1700-IEs   OPTIONAL

}

SidelinkUEInformationNR-v1700-IEs ::=  SEQUENCE {

sl-TxResourceReqList-v1700             SL-TxResourceReqList-v1700                                                 OPTIONAL,

sl-RxDRX-ReportList-v1700              SL-RxDRX-ReportList-v1700                                                  OPTIONAL,

sl-RxInterestedGC-BC-DestList-r17      SL-RxInterestedGC-BC-DestList-r17                                          OPTIONAL,

sl-RxInterestedFreqListDisc-r17        SL-InterestedFreqList-r16                                                  OPTIONAL,

sl-TxResourceReqListDisc-r17           SL-TxResourceReqListDisc-r17                                               OPTIONAL,

sl-TxResourceReqListCommRelay-r17      SL-TxResourceReqListCommRelay-r17                                          OPTIONAL,

ue-Type-r17                            ENUMERATED {relayUE, remoteUE}                                             OPTIONAL,

sl-SourceIdentityRemoteUE-r17          SL-SourceIdentity-r17                                                      OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1800-IEs                                          OPTIONAL

}

SidelinkUEInformationNR-v1800-IEs ::=  SEQUENCE {

sl-CarrierFailureList-r18              SL-CarrierFailureList-r18                                                  OPTIONAL,

sl-TxResourceReqListL2-U2U-r18         SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqL2-U2U-r18      OPTIONAL,

sl-PosRxInterestedFreqList-r18         SL-InterestedFreqList-r16                                                  OPTIONAL,

sl-PosTxResourceReqList-r18            SL-PosTxResourceReqList-r18                                                OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1840-IEs                                          OPTIONAL

}

SidelinkUEInformationNR-v1840-IEs ::=  SEQUENCE {

sl-PosRxInterestedFreqList2-r18        SL-InterestedFreqList-r16                                                  OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                                                                OPTIONAL

}

SL-InterestedFreqList-r16 ::=          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-TxResourceReqList-r16 ::=           SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16

SL-PosTxResourceReqList-r18 ::=        SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-PosTxResourceReq-r18

SL-TxResourceReq-r16 ::=               SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-CastType-r16                        ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-RLC-ModeIndicationList-r16          SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-ModeIndication-r16         OPTIONAL,

sl-QoS-InfoList-r16                    SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16          OPTIONAL,

sl-TypeTxSyncList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16                OPTIONAL,

sl-TxInterestedFreqList-r16            SL-TxInterestedFreqList-r16                                                OPTIONAL,

sl-CapabilityInformationSidelink-r16   OCTET STRING                                                               OPTIONAL

}

SL-TxResourceReqList-v1700 ::=         SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-v1700

SL-RxDRX-ReportList-v1700 ::=          SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxDRX-Report-v1700

SL-TxResourceReq-v1700 ::=             SEQUENCE {

sl-DRX-InfoFromRxList-r17              SEQUENCE (SIZE (1..maxNrofSL-RxInfoSet-r17)) OF SL-DRX-ConfigUC-SemiStatic-r17   OPTIONAL,

sl-DRX-Indication-r17                  ENUMERATED {on, off}                                                             OPTIONAL,

...,

[[

sl-QoS-InfoList-v1800                  SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-v1800              OPTIONAL

]]

}

SL-RxDRX-Report-v1700 ::=              SEQUENCE {

sl-DRX-ConfigFromTx-r17                SL-DRX-ConfigUC-SemiStatic-r17,

...

}

SL-RxInterestedGC-BC-DestList-r17 ::=  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxInterestedGC-BC-Dest-r17

SL-RxInterestedGC-BC-Dest-r17 ::=      SEQUENCE {

sl-RxInterestedQoS-InfoList-r17        SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16,

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16

}

SL-TxResourceReqListDisc-r17 ::=       SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqDisc-r17

SL-TxResourceReqDisc-r17 ::=           SEQUENCE {

sl-DestinationIdentityDisc-r17         SL-DestinationIdentity-r16,

sl-SourceIdentityRelayUE-r17           SL-SourceIdentity-r17                                                      OPTIONAL,

sl-CastTypeDisc-r17                    ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-TxInterestedFreqListDisc-r17        SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListDisc-r17              SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-DiscoveryType-r17                   ENUMERATED {relay, non-Relay},

...,

[[

ue-TypeU2U-r18                         ENUMERATED {relayUE, remoteUE}                                             OPTIONAL

]]

}

SL-TxResourceReqListCommRelay-r17 ::=  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqCommRelayInfo-r17

SL-TxResourceReqCommRelayInfo-r17 ::=  SEQUENCE {

sl-RelayDRXConfig-r17                 SL-TxResourceReq-v1700                                                       OPTIONAL,

sl-TxResourceReqCommRelay-r17         SL-TxResourceReqCommRelay-r17

}

SL-TxResourceReqCommRelay-r17 ::=      CHOICE {

sl-TxResourceReqL2U2N-Relay-r17        SL-TxResourceReqL2U2N-Relay-r17,

sl-TxResourceReqL3U2N-Relay-r17        SL-TxResourceReq-r16

}

SL-TxResourceReqL2U2N-Relay-r17 ::=    SEQUENCE {

sl-DestinationIdentityL2U2N-r17        SL-DestinationIdentity-r16                                                 OPTIONAL,

sl-TxInterestedFreqListL2U2N-r17       SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListL2U2N-r17             SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-LocalID-Request-r17                 ENUMERATED {true}                                                          OPTIONAL,

sl-PagingIdentityRemoteUE-r17          SL-PagingIdentityRemoteUE-r17                                              OPTIONAL,

sl-CapabilityInformationSidelink-r17   OCTET STRING                                                               OPTIONAL,

...

}

SL-TxResourceReqL2-U2U-r18 ::=         SEQUENCE {

sl-DestinationIdentityL2-U2U-r18       SL-DestinationIdentity-r16                                                 OPTIONAL,

sl-TxInterestedFreqListL2-U2U-r18      SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListL2-U2U-r18            SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-CapabilityInformationSidelink-r18   OCTET STRING                                                               OPTIONAL,

sl-U2U-InfoList-r18                    SEQUENCE (SIZE (1.. maxNrofRemoteUE-r17)) OF SL-U2U-Info-r18               OPTIONAL,

sl-RLC-ModeIndicationListL2-U2U-r18    SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-Mode-r18                   OPTIONAL,

...

}

SL-U2U-Info-r18 ::=                    SEQUENCE {

sl-U2U-Identity-r18                    CHOICE {

sl-TargetUE-Identity-r18               SL-DestinationIdentity-r16,

sl-SourceUE-Identity-r18               SL-SourceIdentity-r17

},

sl-E2E-QoS-InfoList-r18                 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16         OPTIONAL,

sl-PerHop-QoS-InfoList-r18              SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-SplitQoS-Info-r18    OPTIONAL,

sl-PerSLRB-QoS-InfoList-r18             SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-PerSLRB-QoS-Info-r18           OPTIONAL,

sl-CapabilityInformationTargetRemoteUE-r18  OCTET STRING                                                           OPTIONAL

}

SL-PosTxResourceReq-r18 ::=                SEQUENCE {

sl-PosDestinationIdentity-r18              SL-DestinationIdentity-r16,

sl-PosCastType-r18                         ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-PosTxInterestedFreqList-r18             SL-TxInterestedFreqList-r16                                            OPTIONAL,

sl-PosTypeTxSyncList-r18                   SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16            OPTIONAL,

sl-PosQoS-InfoList-r18                     SEQUENCE (SIZE (1..maxNrofSL-PRS-PerDest-r18)) OF SL-PRS-QoS-Info-r18  OPTIONAL,

sl-CapabilityInformationSidelink-r18       OCTET STRING                                                           OPTIONAL,

...,

[[

sl-PosTxInterestedFreqList2-r18            SL-TxInterestedFreqList-r16                                            OPTIONAL

]]

}

SL-TxInterestedFreqList-r16 ::=        SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-QoS-Info-r16 ::=                    SEQUENCE {

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16,

sl-QoS-Profile-r16                    SL-QoS-Profile-r16                                                          OPTIONAL

}

SL-QoS-Info-v1800 ::=                  SEQUENCE {

sl-TxInterestedFreqList-r18            SL-TxInterestedFreqList-r16,

sl-TxProfile-r18                       SL-TxProfile-r18                    OPTIONAL,

...

}

SL-TxProfile-r18 ::=                   ENUMERATED {backwardsCompatible, backwardsIncompatible}

SL-RLC-ModeIndication-r16 ::=          SEQUENCE {

sl-Mode-r16                            CHOICE  {

sl-AM-Mode-r16                         NULL,

sl-UM-Mode-r16                         NULL

},

sl-QoS-InfoList-r16                SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}

SL-FailureList-r16 ::=                 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-Failure-r16

SL-Failure-r16 ::=                     SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-Failure-r16                         ENUMERATED {rlf,configFailure, drxReject-v1710, spare5, spare4, spare3, spare2, spare1}

}

SL-CarrierFailureList-r18 ::=          SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-CarrierFailure-r18

SL-CarrierFailure-r18 ::=              SEQUENCE {

sl-DestinationIdentity-r18             SL-DestinationIdentity-r16,

sl-CarrierFailure-r18                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

}

SL-SplitQoS-Info-r18 ::=               SEQUENCE {

sl-QoS-FlowIdentity-r18                SL-QoS-FlowIdentity-r16,

sl-SplitPacketDelayBudget-r18          INTEGER (0..1023)                                                          OPTIONAL,

...

}

SL-PerSLRB-QoS-Info-r18 ::=            SEQUENCE {

sl-RemoteUE-SLRB-Identity-r18           SLRB-Uu-ConfigIndex-r16,

sl-QoS-ProfilePerSLRB-r18               SL-QoS-Profile-r16                                                        OPTIONAL

}

SL-PRS-QoS-Info-r18 ::=                SEQUENCE {

sl-PRS-Priority-r18                    INTEGER (1..8)                                                             OPTIONAL,

sl-PRS-DelayBudget-r18                 INTEGER (0..1023)                                                          OPTIONAL,

sl-PRS-Bandwidth-r18                   ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40,

mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400,

spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}         OPTIONAL,

...

}

SL-RLC-Mode-r18 ::=            CHOICE {

sl-AM-Mode-r18                 NULL,

sl-UM-Mode-r18                 NULL

}

-- TAG-SIDELINKUEINFORMATIONNR-STOP

-- ASN1STOP

SidelinkUEinformationNR field descriptions

sl-PosRxInterestedFreqList Indicates the index of frequency where dedicated SL-PRS resource pool(s) locates on which the UE is interested to perform SL-PRS measurement. The value 1 corresponds to the frequency of first entry in sl-PosFreqInfoList broadcast in SIB23, the value 2 corresponds to the frequency of second entry in sl-PosFreqInfoList broadcast in SIB23 and so on. In this release, only value 1 can be included in the interested frequency list.

sl-PosRxInterestedFreqList2 Indicates the index of frequency where shared SL-PRS resource pool(s) locates on which the UE is interested to perform SL-PRS measurement. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on. The list of interested frequencies indicated by this field should be a subset of the frequencies indicated by the field sl-RxInterestedFreqList.

sl-PosTxResourceReqList List of parameters to request the transmission resources for NR sidelink positioning for the associated destination.

sl-RxDRX-ReportList Indicates the accepted DRX configuration that is received from the peer UE and reported to the network for NR sidelink unicast communication.

sl-RxInterestedFreqList Indicates the index of frequency on which the UE is interested to receive NR sidelink communication. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

sl-RxInterestedGC-BC-DestList Indicates the reported QoS profile and associated destination for which UE is interested in reception to the network for NR sidelink groupcast and broadcast communication, or for NR sidelink discovery or ProSe Direct Link Establishment Request as described in TS 24.554 [72], or for Direct Link Establishment Request (TS 24.587 [57]).

sl-SourceIdentityRemoteUE This field is used to indicate the Source Layer-2 ID to be used to establish PC5 link with the target L2 U2N Relay UE for path switch.

sl-TxResourceReq Parameters to request the transmission resources for NR sidelink communication to the network in the Sidelink UE Information report.

sl-TxResourceReqList List of parameters to request the transmission resources for NR sidelink communication for the associated destination. If sl-TxResourceReqList-v1700 is present, it shall contain the same number of entries, listed in the same order as in sl-TxResourceReqList-r16.

ue-Type Indicates the UE is acting as U2N Relay UE or U2N Remote UE.

SL-TxResourceReq field descriptions

sl-CapabilityInformationSidelink Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from peer UE) received from the peer UE.

sl-CastType Indicates the cast type for the corresponding destination for which to request the resource.

sl-DestinationIdentity Indicates the destination for which the TX resource request and allocation from the network are concerned.

sl-DRX-Indication Indicates the sidelink DRX is applied (value on) or not applied (value off) for the associated destination. This field is only valid for NR sidelink groupcast communication.

sl-DRX-InfoFromRxList Indicates list of the sidelink DRX configurations as assistance information received from the peer UE for NR sidelink unicast communication.

sl-QoS-InfoList Includes the QoS profile of the sidelink QoS flow as specified in TS 23.287 [55]. If sl-QoS-InfoList-v1800 is included, shall include the same number of entries, and listed in the same order, as in sl-QoS-InfoList-r16.

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-RLC-ModeIndicationList Each entry of this field indicates the RLC mode and optionally the related QoS profiles for the sidelink radio bearer, which has not been configured by the network and is initiated by another UE in unicast. The RLC mode for one sidelink radio bearer is aligned between UE and NW by the sl-QoS-FlowIdentity.

sl-TxInterestedFreqList Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication, for each destination. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

sl-TypeTxSyncList A list of synchronization reference used by the UE. The UE shall include the same number of entries, listed in the same order, as in sl-TxInterestedFreqList, i.e. one for each carrier frequency included in sl-TxInterestedFreqList.

SL-Failure field descriptions

sl-DestinationIdentity Indicates the destination for which the SL failure is reporting for unicast.

sl-Failure Indicates the sidelink cause for the sidelink RLF (value rlf), sidelink AS configuration failure (value configFailure) and the rejection of sidelink DRX configuration (value drxReject-v1710) for the associated destination for unicast.

SL-RxDRX-Report field descriptions

sl-DRX-ConfigFromTx Indicates the sidelink DRX configuration received from the peer UE for NR sidelink unicast communication.

SL-RxInterestedGC-BC-Dest field descriptions

sl-RxInterestedQoS-InfoList Indicates the QoS profile for which UE reports its interested service to which SL DRX is applied to the network, for NR sidelink groupcast or broadcast reception.

SL-TxResourceReqDisc field descriptions

sl-CastTypeDisc Indicates the cast type for the NR sidelink discovery messages. Only value broadcast can be set in this release.

sl-DestinationIdentityDisc This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for relay discovery and non-relay discovery.

sl-SourceIdentityRelayUE This field is used to indicate the source L2 ID of relay-related discovery transmission by L2 U2N Relay UE.

sl-TxInterestedFreqListDisc Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink discovery. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

SL-PosTxResourceReq field descriptions

sl-CapabilityInformationSidelink Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from peer UE) received from the peer UE.

sl-PosCastType Indicates the cast type for the SL-PRS transmission.

sl-PosDestinationIdentity This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for SL-PRS transmission

sl-PosQoS-InfoList This field is used to indicate the QoS information for SL-PRS transmission.

sl-PosTxInterestedFreqList Each entry of this field indicates the index of frequency in SIB23 on which the UE is interested to transmit SL-PRS. The value 1 corresponds to the frequency of first entry in sl-PosFreqInfoList broadcast in SIB23, the value 2 corresponds to the frequency of second entry in sl-PosFreqInfoList broadcast in SIB23 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

sl-PosTxInterestedFreqList2 Each entry of this field indicates the index of frequency in SIB12 on which the UE is interested to transmit SL-PRS. The value 1 corresponds to the frequency of first entry in sl-PosFreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on. The list of interested frequencies indicated by this field should be a subset of the frequencies indicated by the field sl-TxInterestedFreqList.

sl-PosTypeTxSyncList A list of synchronization references used by the UE. The UE shall include the same number of entries, listed in the same order, as in sl-PosTxInterestedFreqList, i.e. one for each carrier frequency included in sl-PosTxInterestedFreqList.

sl-PRS-DelayBudget Indicates the SL-PRS delay budget provided by upper layers (see TS 38.355 [77]).

sl-PRS-Priority Indicates the priority of SL-PRS provided by upper layers (see TS 38.355 [77]). Value 1 is the highest priority whereas value 8 is the lowest priority.

sl-PRS-Bandwidth Indicates the desired bandwidth of the requested SL-PRS resources provided by upper layers (see TS 38.355 [77]) in the unit of MHz.

SL-TxResourceReqCommRelayInfo field descriptions

sl-RelayDRXConfig This field is used to indicate the applied sidelink DRX configuration for the relay related communication.

sl-DestinationIdentityL2U2N This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for the established PC5 link for relay by L2 U2N Relay UE.

sl-LocalID-Request This field is used to request local UE ID for the corresponding destination by the L2 U2N Relay UE.

sl-TxInterestedFreqListL2U2N Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication for established PC5 link for relay. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

sl-PagingIdentityRemoteUE This field is used to indicate the paging UE ID(s) for the corresponding destination(s) by the L2 U2N Relay UE.

SL-QoS-Info field descriptions

sl-TxInterestedFreqList Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication, for each QoS flow. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

sl-TxProfile Indicating Tx profile for each QoS flow, i.e., compatibility of supporting SL CA operation. The IE of SL-TxProfile is referred by upper layer signaling as specified TS 24.588 [78].

SL-CarrierFailure field descriptions

sl-CarrierFailure Indicate the carrier(s) where the Sidelink carrier failure has been indicated by lower layer as specified in TS 38.321 [3]. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

sl-DestinationIdentity This field is used to indicate the destination L2 ID for which the per-carrier failure report is concerned.

SL-TxResourceReqL2-U2U field descriptions

sl-CapabilityInformationSidelink Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from the L2 U2U Relay UE) received from the L2 U2U Relay UE by the L2 U2U Remote UE or includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from the target L2 U2U Remote UE) received from the target L2 U2U Remote UE by the L2 U2U Relay UE.

sl-DestinationIdentityL2-U2U This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for the established per-hop PC5 link between the L2 U2U Remote UE and L2 U2U Relay UE.

sl-TxInterestedFreqListL2-U2U Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication for established per-hop PC5 link. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

sl-U2U-InfoList This field indicates the information related to a list of end-to-end PC5 links.

SL-U2U-Info field descriptions

sl-CapabilityInformationTargetRemoteUE Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from the target L2 U2U Remote UE) received from the target L2 U2U Remote UE. In this version of the specification, only outOfOrderDeliverySidelink-r16 and accessStratumReleaseSidelink-r16 are included in the UECapabilityInformationSidelink message.

sl-E2E-QoS-InfoList This field is used by L2 U2U Remote UE to indicate a list of end-to-end QoS info.

sl-PerHop-QoS-InfoList This field is used by L2 U2U Remote UE to indicate a list of split QoS info for the first hop.

sl-PerSLRB-QoS-InfoList This field is used by L2 U2U Relay UE to indicate a list of split QoS info for the second hop in per-SLRB level, with each entry in accordance with an end-to-end SLRB.

sl-U2U-Identity This field is to identify an end-to-end PC5 link. For a L2 U2U Remote UE acting as source UE it includes sl-TargetUE-Identity to indicate the target L2 U2U Remote UE on the second hop, and for a L2 U2U Relay UE, it includes sl-SourceUE-Identity to indicate the source L2 U2U Remote UE on the first hop.

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks or Positioning System Information Blocks. All the SIBs or posSIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SystemInformation message

-- ASN1START

-- TAG-SYSTEMINFORMATION-START

SystemInformation ::=               SEQUENCE {

criticalExtensions                  CHOICE {

systemInformation                   SystemInformation-IEs,

criticalExtensionsFuture-r16    CHOICE {

posSystemInformation-r16        PosSystemInformation-r16-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

}

SystemInformation-IEs ::=           SEQUENCE {

sib-TypeAndInfo                     SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2                                SIB2,

sib3                                SIB3,

sib4                                SIB4,

sib5                                SIB5,

sib6                                SIB6,

sib7                                SIB7,

sib8                                SIB8,

sib9                                SIB9,

...,

sib10-v1610                         SIB10-r16,

sib11-v1610                         SIB11-r16,

sib12-v1610                         SIB12-r16,

sib13-v1610                         SIB13-r16,

sib14-v1610                         SIB14-r16,

sib15-v1700                         SIB15-r17,

sib16-v1700                         SIB16-r17,

sib17-v1700                         SIB17-r17,

sib18-v1700                         SIB18-r17,

sib19-v1700                         SIB19-r17,

sib20-v1700                         SIB20-r17,

sib21-v1700                         SIB21-r17,

sib22-v1800                         SIB22-r18,

sib23-v1800                         SIB23-r18,

sib24-v1800                         SIB24-r18,

sib25-v1800                         SIB25-r18,

sib17bis-v1820                      SIB17bis-r18,

sib26-v1900                         SIB26-r19,

sib27-v1900                         SIB27-r19

},

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-SYSTEMINFORMATION-STOP

-- ASN1STOP

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1610-IEs   OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       ReducedMaxCCs-r16                   OPTIONAL,

reducedMaxBW-FR1                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxBW-FR2                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

OverheatingAssistance-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17

} OPTIONAL,

reducedMaxMIMO-LayersFR2-2          SEQUENCE {

reducedMIMO-LayersFR2-2-DL          MIMO-LayersDL,

reducedMIMO-LayersFR2-2-UL          MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

ReducedAggregatedBandwidth-r17 ::= ENUMERATED {mhz0, mhz100, mhz200, mhz400, mhz800, mhz1200, mhz1600, mhz2000}

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

drx-Preference-r16                  DRX-Preference-r16                  OPTIONAL,

maxBW-Preference-r16                MaxBW-Preference-r16                OPTIONAL,

maxCC-Preference-r16                MaxCC-Preference-r16                OPTIONAL,

maxMIMO-LayerPreference-r16         MaxMIMO-LayerPreference-r16         OPTIONAL,

minSchedulingOffsetPreference-r16   MinSchedulingOffsetPreference-r16   OPTIONAL,

releasePreference-r16               ReleasePreference-r16               OPTIONAL,

sl-UE-AssistanceInformationNR-r16   SL-UE-AssistanceInformationNR-r16   OPTIONAL,

referenceTimeInfoPreference-r16     BOOLEAN                             OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1700-IEs   OPTIONAL

}

UEAssistanceInformation-v1700-IEs ::= SEQUENCE {

ul-GapFR2-Preference-r17              UL-GapFR2-Preference-r17              OPTIONAL,

musim-Assistance-r17                  MUSIM-Assistance-r17                  OPTIONAL,

overheatingAssistance-r17             OverheatingAssistance-r17             OPTIONAL,

maxBW-PreferenceFR2-2-r17             MaxBW-PreferenceFR2-2-r17             OPTIONAL,

maxMIMO-LayerPreferenceFR2-2-r17      MaxMIMO-LayerPreferenceFR2-2-r17      OPTIONAL,

minSchedulingOffsetPreferenceExt-r17  MinSchedulingOffsetPreferenceExt-r17  OPTIONAL,

rlm-MeasRelaxationState-r17           BOOLEAN                               OPTIONAL,

bfd-MeasRelaxationState-r17           BIT STRING (SIZE (1..maxNrofServingCells)) OPTIONAL,

nonSDT-DataIndication-r17             SEQUENCE {

resumeCause-r17                       ResumeCause                       OPTIONAL

}                                                                           OPTIONAL,

scg-DeactivationPreference-r17        ENUMERATED { scg-DeactivationPreferred, noPreference }    OPTIONAL,

uplinkData-r17                        ENUMERATED { true }                   OPTIONAL,

rrm-MeasRelaxationFulfilment-r17      BOOLEAN                               OPTIONAL,

propagationDelayDifference-r17        PropagationDelayDifference-r17        OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v1800-IEs     OPTIONAL

}

UEAssistanceInformation-v1800-IEs ::= SEQUENCE {

idc-FDM-Assistance-r18                IDC-FDM-Assistance-r18                          OPTIONAL,

idc-TDM-Assistance-r18                IDC-TDM-Assistance-r18                          OPTIONAL,

multiRx-PreferenceFR2-r18             ENUMERATED {single, multiple }                  OPTIONAL,

musim-Assistance-v1800                MUSIM-Assistance-v1800                          OPTIONAL,

flightPathInfoAvailable-r18           ENUMERATED {true}                               OPTIONAL,

ul-TrafficInfo-r18                    UL-TrafficInfo-r18                              OPTIONAL,

n3c-RelayUE-InfoList-r18              SEQUENCE (SIZE (0..8)) OF N3C-RelayUE-Info-r18  OPTIONAL,

sl-PRS-UE-AssistanceInformationNR-r18 SL-PRS-UE-AssistanceInformationNR-r18           OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v1900-IEs               OPTIONAL

}

UEAssistanceInformation-v1900-IEs ::= SEQUENCE {

gapOccasionCancelRatio-r19            GapOccasionCancelRatio-r19                      OPTIONAL,

lpwus-OffsetPreference-r19            LPWUS-OffsetPreference-r19                      OPTIONAL,

applicabilityReportList-r19           ApplicabilityReportList-r19                     OPTIONAL,

dataCollectionPreference-r19          DataCollectionPreference-r19                    OPTIONAL,

loggedDataCollectionAssistance-r19    LoggedDataCollectionAssistance-r19              OPTIONAL,

referenceLocationReport-r19           BIT STRING (SIZE (6))                           OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                     OPTIONAL

}

IDC-Assistance-r16 ::=                  SEQUENCE {

affectedCarrierFreqList-r16             AffectedCarrierFreqList-r16               OPTIONAL,

affectedCarrierFreqCombList-r16         AffectedCarrierFreqCombList-r16           OPTIONAL,

...

}

AffectedCarrierFreqList-r16 ::= SEQUENCE (SIZE (1.. maxFreqIDC-r16)) OF AffectedCarrierFreq-r16

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqCombList-r16 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqComb-r16

AffectedCarrierFreqComb-r16 ::=     SEQUENCE {

affectedCarrierFreqComb-r16         SEQUENCE (SIZE (2..maxNrofServingCells)) OF  ARFCN-ValueNR    OPTIONAL,

victimSystemType-r16                VictimSystemType-r16

}

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...,

[[

uwb-r18                     ENUMERATED {true}        OPTIONAL

]]

}

DRX-Preference-r16 ::=              SEQUENCE {

preferredDRX-InactivityTimer-r16    ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL,

preferredDRX-LongCycle-r16          ENUMERATED {

ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512,

ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycle-r16         ENUMERATED {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycleTimer-r16    INTEGER (1..16)    OPTIONAL

}

MaxBW-Preference-r16 ::=            SEQUENCE {

reducedMaxBW-FR1-r16                ReducedMaxBW-FRx-r16                     OPTIONAL,

reducedMaxBW-FR2-r16                ReducedMaxBW-FRx-r16                     OPTIONAL

}

MaxBW-PreferenceFR2-2-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL

} OPTIONAL

}

MaxCC-Preference-r16 ::=            SEQUENCE {

reducedMaxCCs-r16                   ReducedMaxCCs-r16                        OPTIONAL

}

MaxMIMO-LayerPreference-r16 ::=     SEQUENCE {

reducedMaxMIMO-LayersFR1-r16        SEQUENCE {

reducedMIMO-LayersFR1-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR1-UL-r16        INTEGER (1..4)

} OPTIONAL,

reducedMaxMIMO-LayersFR2-r16        SEQUENCE {

reducedMIMO-LayersFR2-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR2-UL-r16        INTEGER (1..4)

} OPTIONAL

}

MaxMIMO-LayerPreferenceFR2-2-r17 ::=    SEQUENCE {

reducedMaxMIMO-LayersFR2-2-r17          SEQUENCE {

reducedMIMO-LayersFR2-2-DL-r17          INTEGER (1..8),

reducedMIMO-LayersFR2-2-UL-r17          INTEGER (1..4)

} OPTIONAL

}

MinSchedulingOffsetPreference-r16 ::= SEQUENCE {

preferredK0-r16                       SEQUENCE {

preferredK0-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL,

preferredK0-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL

}                                                                                  OPTIONAL,

preferredK2-r16                       SEQUENCE {

preferredK2-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL,

preferredK2-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL

}                                                                                 OPTIONAL

}

MinSchedulingOffsetPreferenceExt-r17 ::=  SEQUENCE {

preferredK0-r17                           SEQUENCE {

preferredK0-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK0-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL,

preferredK2-r17                           SEQUENCE {

preferredK2-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK2-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL

}

MUSIM-Assistance-r17 ::=              SEQUENCE {

musim-PreferredRRC-State-r17          ENUMERATED {idle, inactive, outOfConnected}     OPTIONAL,

musim-GapPreferenceList-r17           MUSIM-GapPreferenceList-r17                     OPTIONAL

}

MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17

MUSIM-Assistance-v1800 ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18     MUSIM-GapPriorityPreferenceList-r18           OPTIONAL,

musim-GapKeepPreference-r18             ENUMERATED {true}                             OPTIONAL,

musim-CapRestriction-r18                MUSIM-CapRestriction-r18                      OPTIONAL,

musim-NeedForGapsInfoNR-r18             NeedForGapsInfoNR-r16                         OPTIONAL,

...

}

MUSIM-GapPriorityPreferenceList-r18 ::= SEQUENCE (SIZE (1..3)) OF GapPriority-r17

MUSIM-CapRestriction-r18 ::=            SEQUENCE {

musim-Cell-SCG-ToRelease-r18            MUSIM-Cell-SCG-ToRelease-r18                  OPTIONAL,

musim-CellToAffectList-r18              MUSIM-CellToAffectList-r18                    OPTIONAL,

musim-AffectedBandsList-r18             MUSIM-AffectedBandsList-r18                   OPTIONAL,

musim-AvoidedBandsList-r18              MUSIM-AvoidedBandsList-r18                    OPTIONAL,

musim-MaxCC-r18                         MUSIM-MaxCC-r18                               OPTIONAL

}

MUSIM-Cell-SCG-ToRelease-r18 ::=        SEQUENCE {

musim-CellToRelease-r18                 MUSIM-CellToRelease-r18                       OPTIONAL,

scg-ReleasePreference-r18               ENUMERATED {true}                             OPTIONAL

}

MUSIM-CellToRelease-r18 ::=             SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex

MUSIM-CellToAffectList-r18::=           SEQUENCE (SIZE (1..maxNrofServingCells)) OF MUSIM-CellToAffect-r18

MUSIM-CellToAffect-r18 ::=              SEQUENCE {

musim-ServCellIndex-r18                 ServCellIndex,

musim-MIMO-Layers-DL-r18                INTEGER (1..8)                                OPTIONAL,

musim-MIMO-Layers-UL-r18                INTEGER (1..4)                                OPTIONAL,

musim-SupportedBandwidth-DL-r18         SupportedBandwidth-v1700                      OPTIONAL,

musim-SupportedBandwidth-UL-r18         SupportedBandwidth-v1700                      OPTIONAL

}

MUSIM-AffectedBandsList-r18  ::=        SEQUENCE (SIZE (1..maxBandComb-MUSIM-r18)) OF MUSIM-AffectedBands-r18

MUSIM-AffectedBands-r18 ::=             SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF MUSIM-CapabilityRestrictedBandParameters-r18

MUSIM-CapabilityRestrictedBandParameters-r18 ::= SEQUENCE {

musim-bandEntryIndex-r18                MUSIM-BandEntryIndex-r18,

musim-CapabilityRestricted-r18          SEQUENCE {

musim-MIMO-Layers-DL-r18                INTEGER (1..8)                            OPTIONAL,

musim-MIMO-Layers-UL-r18                INTEGER (1..4)                            OPTIONAL,

musim-SupportedBandwidth-DL-r18         SupportedBandwidth-v1700                  OPTIONAL,

musim-SupportedBandwidth-UL-r18         SupportedBandwidth-v1700                  OPTIONAL

}

}

MUSIM-AvoidedBandsList-r18 ::=          SEQUENCE (SIZE (1..maxBandComb-MUSIM-r18)) OF MUSIM-AvoidedBands-r18

MUSIM-AvoidedBands-r18 ::=              SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF MUSIM-BandEntryIndex-r18

MUSIM-BandEntryIndex-r18 ::=            INTEGER(1.. maxCandidateBandIndex-r18)

MUSIM-MaxCC-r18 ::=                     SEQUENCE {

musim-MaxCC-TotalDL-r18                 INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-TotalUL-r18                 INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR1-DL-r18                  INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR1-UL-r18                  INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-1-DL-r18                INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-1-UL-r18                INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-2-DL-r18                INTEGER (1..32)                                OPTIONAL,

musim-MaxCC-FR2-2-UL-r18                INTEGER (1..32)                                OPTIONAL

}

LPWUS-OffsetPreference-r19 ::=          SEQUENCE {

timeOffset-r19                          ENUMERATED {ms5, ms13, ms37}                  OPTIONAL

}

ReleasePreference-r16 ::=           SEQUENCE {

preferredRRC-State-r16              ENUMERATED {idle, inactive, connected, outOfConnected}

}

ReducedMaxBW-FRx-r16 ::=            SEQUENCE {

reducedBW-DL-r16                    ReducedAggregatedBandwidth,

reducedBW-UL-r16                    ReducedAggregatedBandwidth

}

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

SL-UE-AssistanceInformationNR-r16 ::= SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16

SL-TrafficPatternInfo-r16::=          SEQUENCE {

trafficPeriodicity-r16                ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000},

timingOffset-r16                      INTEGER (0..10239),

messageSize-r16                       BIT STRING (SIZE (8)),

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16

}

UL-GapFR2-Preference-r17::=           SEQUENCE {

ul-GapFR2-PatternPreference-r17       INTEGER (0..3)                     OPTIONAL

}

PropagationDelayDifference-r17 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (-270..270)

IDC-FDM-Assistance-r18 ::=            SEQUENCE {

affectedCarrierFreqRangeList-r18      AffectedCarrierFreqRangeList-r18               OPTIONAL,

affectedCarrierFreqRangeCombList-r18  AffectedCarrierFreqRangeCombList-r18           OPTIONAL,

...

}

IDC-TDM-Assistance-r18 ::=            SEQUENCE {

cycleLength-r18                       ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30,

ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160,

ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240},

startOffset-r18                       INTEGER (0..10239),

slotOffset-r18                        INTEGER (0..31),

activeDuration-r18                    CHOICE {

subMilliSeconds-r18 INTEGER (1..31),

milliSeconds-r18    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

...

}

AffectedCarrierFreqRangeList-r18 ::=  SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18

AffectedCarrierFreqRange-r18 ::=      SEQUENCE {

affectedFreqRange-r18                 AffectedFreqRange-r18,

interferenceDirection-r18             ENUMERATED {nr, other, both, spare},

victimSystemType-r18                  VictimSystemType-r16                           OPTIONAL

}

AffectedCarrierFreqRangeCombList-r18 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqRangeComb-r18

AffectedCarrierFreqRangeComb-r18 ::=  SEQUENCE {

affectedCarrierFreqRangeComb-r18      SEQUENCE (SIZE (2..maxNrofServingCells)) OF AffectedFreqRange-r18,

interferenceDirection-r18             ENUMERATED {nr, other, both, spare},

victimSystemType-r18                  VictimSystemType-r16                           OPTIONAL

}

AffectedFreqRange-r18 ::=             SEQUENCE {

centerFreq-r18                        ARFCN-ValueNR,

affectedBandwidth-r18                 ENUMERATED {khz200, khz400, khz600, khz800, mhz1, mhz2, mhz3, mhz4, mhz5, mhz6,

mhz8, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200,

mhz300, mhz400, spare10, spare9, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

}

UL-TrafficInfo-r18 ::=                SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18

PDU-SessionUL-TrafficInfo-r18 ::=     SEQUENCE {

pdu-SessionID-r18                     PDU-SessionID,

qos-FlowUL-TrafficInfoList-r18        SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18

}

QOS-FlowUL-TrafficInfo-r18 ::=        SEQUENCE {

qfi-r18                               QFI,

jitterRange-r18                       SEQUENCE {

lowerBound-r18                        JitterBound-r18,

upperBound-r18                        JitterBound-r18

}                                                                                    OPTIONAL,

burstArrivalTime-r18                  CHOICE {

referenceTime                         ReferenceTime-r16,

referenceSFN-AndSlot                  ReferenceSFN-AndSlot-r18

}                                                                                    OPTIONAL,

trafficPeriodicity-r18                INTEGER (1..640000)                            OPTIONAL,

pdu-SetIdentification-r18             BOOLEAN                                        OPTIONAL,

psi-Identification-r18                BOOLEAN                                        OPTIONAL,

...

}

ReferenceSFN-AndSlot-r18 ::= SEQUENCE {

referenceSFN-r18                 INTEGER (0..1023),

referenceSlot-r18                INTEGER (0..639)

}

JitterBound-r18 ::= ENUMERATED {ms0, ms0dot5, ms1, ms1dot5, ms2, ms2dot5, ms3, ms3dot5, ms4, ms4dot5, ms5, ms5dot5, ms6, ms6dot5, ms7, beyondMs7}

SL-PRS-UE-AssistanceInformationNR-r18 ::= SEQUENCE (SIZE (1..maxNrofSL-PRS-TxConfig-r18)) OF SL-PRS-TxInfo-r18

SL-PRS-TxInfo-r18 ::=                 SEQUENCE {

sl-PRS-Periodicity-r18                ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000, spare6,

spare5, spare4, spare3, spare2, spare1},

sl-PRS-Priority-r18                   INTEGER (1..8)                                                            OPTIONAL,

sl-PRS-DelayBudget-r18                INTEGER (0..1023)                                                         OPTIONAL,

sl-PRS-Bandwidth-r18                  ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40,

mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400,

spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}       OPTIONAL,

...

}

GapOccasionCancelRatio-r19 ::= SEQUENCE {

gapOccasionCancelRatioPerFR-PerUE-r19   CHOICE {

perUE-r19                               GapOccasionRatio-r19,

perFR-r19                               SEQUENCE {

fr1-r19                                 GapOccasionRatio-r19                                                 OPTIONAL,

fr2-r19                                 GapOccasionRatio-r19                                                 OPTIONAL

}

}                                                                                                                    OPTIONAL,

gapConfigRatioList-r19                  SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapOccasionRatioPerGapConfig-r19    OPTIONAL

}

GapOccasionRatioPerGapConfig-r19 ::= SEQUENCE{

measGapId-r19                        MeasGapId-r17,

gapOccasionRatio-r19                 GapOccasionRatio-r19

}

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStart-r19                       ENUMERATED {start}                                                           OPTIONAL,

dataCollectionPreferredConfigurationList-r19  SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19  OPTIONAL,

dataCollectionStopConfigurationList-r19       SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19           OPTIONAL,

...

}

DataCollectionCandidateList-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19         ServCellIndex,

dataCollectionCandidateIdList-r19       SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19  OPTIONAL

}

DataCollectionList-r19 ::=                SEQUENCE {

dataCollectionStopServCellIndex-r19       ServCellIndex,

dataCollectionIdList-r19                  SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId    OPTIONAL

}

LoggedDataCollectionAssistance-r19 ::=    SEQUENCE {

lowPowerState-r19                         ENUMERATED {true}                                                             OPTIONAL,

bufferStatus-r19                          ENUMERATED {full, aboveThreshold}                                             OPTIONAL,

...

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

activeDuration Indicates the UE's preferred active duration to resolve the IDC problem. Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

affectedBandwidth Indicates the bandwidth around the center frequency of the carrier frequency range which is affected by the IDC problem. Value mhz5 corresponds to 5 MHz, value mhz10 corresponds to 10 MHz and so on. If candidateBandwidth is not configured, the UE is allowed to report the frequency range for any bandwidth as indicated by affectedBandwidth, within the frequency band limitation as defined in TS 38.101-1 [15], TS 38.101-2 [39], TS 38.101-3 [34] and TS 38.101-5 [75].

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqRangeList Indicates a list of NR carrier frequency ranges that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequency combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC.

affectedCarrierFreqRangeCombList Indicates a list of NR carrier frequency range combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC

bfd-MeasRelaxationState Indicates the relaxation state of BFD measurements. Each bit corresponds to a serving cell of the cell group. A serving cell is mapped to the (servCellIndex+1)-th bit, starting from MSB. A bit that is set to 1 indicates that the UE is performing BFD measurements relaxation on the serving cell mapped on the bit. A bit that is set to 0 indicates that the UE is not performing BFD measurements relaxation on the serving cell mapped on the bit. If a serving cell is not configured to the UE, the corresponding bit is set to 0.

bufferStatus Indicates the status of the buffer reserved for the logging of radio measurements for network-side data collection.

centerFreq Indicates the center frequency of the carrier frequency range which is affected by the IDC problem.

cycleLength Indicates the UE's preferred cycle length to resolve the IDC problem. Value in ms. Value ms2 corresponds to 2 ms, value ms3 corresponds to 3 ms, and so on.

dataCollectionCandidateIdList Indicates one or more IDs of candidate configurations preferred by the UE for UE-side data collection.

dataCollectionIdList Indicates one or more IDs of UE-side data collection configurations that the UE prefers to stop.

dataCollectionPreferredConfigurationList Indicates the UE's preferred radio resource configurations for UE-side data collection.

dataCollectionServCellIndex Index of the serving cell that the dataCollectionCandidateIdList refers to.

dataCollectionStart It indicates the UE's preference to be configured with radio resources for UE-side data collection.

dataCollectionStopConfigurationList Indicates the radio resource configurations for UE-side data collection that the UE prefers to stop.

dataCollectionStopServCellIndex Index of the serving cell that the dataCollectionIdList refers to.

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

loggedDataCollectionAssistance Indicates assistance information related to the logging of measurements for network-side data collection performed in accordance with CSI-LoggedMeasurementConfig.

lowPowerState It is set to 'true' if the UE has entered a low power state.

lpwus-OffsetPreference Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The reported preferred time offset value is equal to or longer than the minimum time gap reported by UE capability for a UE. If the field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring.

minSchedulingOffsetPreference Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving.

minSchedulingOffsetPreferenceExt Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving for SCS 480 kHz and/or 960 kHz.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2.

musim-AffectedBandsList Indicates the UE's preference on the band(s) and/or combination(s) of bands with restricted capability for MUSIM operation. If the MUSIM-CapabilityRestrictedBandParameters-r18 with same musim-bandEntryIndex appears more than once in the list of bands in a MUSIM-AffectedBands entry, the UE supports intra-band non-contiguous CA with restricted capability for MUSIM operation for this band. UE explicitly indicates each band and each combination of bands that are affected. The Network should respect these capability restrictions when configuring the UE with bands or band combinations that contain these bands and/or combination of bands. Fields musim-MIMO-Layers-DL/UL and musim-SupportedBandwidth-DL/UL indicate the max number of MIMO layers and max bandwidth on each CC of the band, respectively. The band(s) and/or combination(s) of bands are supported in UE capability, and the musim-MIMO-Layers-DL/UL and musim-SupportedBandwidth-DL/UL range up to the concerned maximum capability of band(s) and/or combination(s) of bands in UE capability. For each concerned band or band combination, the lowest value of each musim-MIMO-Layers-DL/UL/musim-SupportedBandwidth-DL/UL and the corresponding capabilities in the UE capability determines the supported maximum MIMO layers and maximum bandwidth for DL/UL, respectively.

musim-AvoidedBandsList Indicates the UE's preference on band(s) and/or combination(s) of bands to be avoided for MUSIM purpose. UE explicitly indicates each band and each combination of bands to be avoided. The list may include the band of the PCell. The Network should respect these capability restrictions for the band combinations that contain these bands and/or combination of bands. The band(s) and/or combination(s) of bands is a subset of the band combination(s) in UE capability.

musim-bandEntryIndex Indicates an NR band by referring to the position of a band entry in musim-CandidateBandList IE. Value 1 identifies the first band in the musim-CandidateBandList IE, value 2 identifies the second band in the musim-CandidateBandList IE, and so on.

musim-CapabilityRestricted Indicates the UE's preference on the temporary capability restriction on the band for MUSIM operation.

musim-CapRestriction Indicates the UE's preference on SCell(s) or PSCell to be released, serving cell(s) with restricted capability, band(s) or combination(s) of bands with restricted capability, or band(s) or band combination(s) to be avoided for UE temporary capabilities restriction.

musim-Cell-SCG-ToRelease Indicates the UE's preference on any serving cell(s), except for Pcell, and/or SCG to be released for MUSIM operation.

musim-CellToAffectList Indicates the UE's preference on the temporary capability restriction on the serving cell(s) for MUSIM operation.

musim-CellToRelease Indicates the UE's preference on the temporary capability restriction on the serving cell(s) to release, except PCell, for MUSIM operation.

musim-GapKeepPreference Indicates the UE's preference to keep all colliding gaps for requested MUSIM gap(s). If the field is absent, the colliding MUSIM gaps with lower priority shall be dropped as specified in TS 38.133 [14].

musim-GapPreferenceList Indicates the UE's MUSIM gap preference and related MUSIM gap configuration, as defined in TS 38.133 [14] clause 9.1.10.

musim-GapPriorityPreferenceList Indicates the UE's MUSIM gap priority preference for periodic MUSIM gaps as specified in TS 38.133[14]. If the UE includes musim-GapPriorityPreferenceList-r18, it includes the same number of entries, and listed in the same order for periodic gaps, as in musim-GapPreferenceList-r17.

musim-MaxCC Indicates the UE's preference on the temporary capability restriction on maximum number of CCs per DL/UL in total, and per FR1/FR2-1/F2-2.

musim-NeedForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands when in MUSIM operation while NR-DC or NE-DC is not configured.

musim-PreferredRRC-State Indicates the UE's preferred RRC state when leaving RRC_CONNECTED.

n3c-RelayUE-InfoList Information of available N3C relay UE(s).

nonSDT-DataIndication Informs the network about the arrival of data and/or signaling mapped to radio bearers not configured for SDT while SDT procedure is ongoing.

preferredDRX-InactivityTimer Indicates the UE's preferred DRX inactivity timer length for power saving. Value in ms (milliSecond). ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the DRX inactivity timer. If secondary DRX group is configured, the preferredDRX-InactivityTimer only applies to the default DRX group.

preferredDRX-LongCycle Indicates the UE's preferred long DRX cycle length for power saving. Value in ms. ms10 corresponds to 10ms, ms20 corresponds to 20 ms, ms32 corresponds to 32 ms, and so on. If preferredDRX-ShortCycle is provided, the value of preferredDRX-LongCycle shall be a multiple of the preferredDRX-ShortCycle value. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the long DRX cycle. This field may also indicate UE’s preference for cellDTX-DRX-Cycle.

preferredDRX-ShortCycle Indicates the UE's preferred short DRX cycle length for power saving. Value in ms. ms2 corresponds to 2ms, ms3 corresponds to 3 ms, ms4 corresponds to 4 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle.

preferredDRX-ShortCycleTimer Indicates the UE's preferred short DRX cycle timer for power saving. Value in multiples of preferredDRX-ShortCycle. A value of 1 corresponds to preferredDRX-ShortCycle, a value of 2 corresponds to 2 * preferredDRX-ShortCycle and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle timer. A preference for the short DRX cycle is indicated when a preference for the short DRX cycle timer is indicated.

preferredK0 Indicates the UE's preferred value of k0 (slot offset between DCI and its scheduled PDSCH - see TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k0 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k0 for cross-slot scheduling.

preferredK2 Indicates the UE's preferred value of k2 (slot offset between DCI and its scheduled PUSCH - see TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k2 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k2 for cross-slot scheduling.

preferredRRC-State Indicates the UE's preferred RRC state. The value idle is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_IDLE. The value inactive is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_INACTIVE. The value connected is indicated if the UE prefers to revert an earlier indication to leave RRC_CONNECTED state. The value outOfConnected is indicated if the UE prefers to be released from RRC_CONNECTED and has no preferred RRC state to transition to. The value connected can only be indicated if the UE is configured with connectedReporting.

propagationDelayDifference Indicates the one-way service link propagation delay difference between serving cell and each neighbour cell included in neighCellInfoList, defined as neighbour cell's service link propagation delay minus serving cell's service link propagation delay, in number of ms. First entry in propagationDelayDifference corresponds to first entry in neighCellInfoList, second entry in propagationDelayDifference corresponds to second entry in neighCellInfoList, and so on.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes SCells of the NR MCG, PSCell and SCells of the SCG. This maximum number only includes PSCell and SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell and SCells of the cell group that this UE assistance information is associated with. The maximum number of downlink SCells can only range up to the current active configuration when indicated to address power savings.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes SCells of the NR MCG, PSCell and SCells of the SCG. This maximum number only includes PSCell and SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell and SCells of the cell group that this UE assistance information is associated with. The maximum number of uplink SCells can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR1 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR1. The aggregated bandwidth across all downlink carrier(s) of FR1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR1. The aggregated bandwidth across all uplink carrier(s) of FR1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR1 of both the NR MCG and the SCG. This maximum aggregated bandwidth only includes carriers of FR1 of the SCG in (NG)EN-DC. Value mhz0 is not used when indicated to address overheating. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2-1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2-1. The aggregated bandwidth across all downlink carrier(s) of FR2-1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2-1. The aggregated bandwidth across all uplink carrier(s) of FR2-1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2-1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2-1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2-1 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2-1 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2-1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR2-2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2-2, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2-2. The aggregated bandwidth across all downlink carrier(s) of FR2-2 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2-2. The aggregated bandwidth across all uplink carrier(s) of FR2-2 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2-2. If the field is absent from the MaxBW-PreferenceFR2-2 IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2-2. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2-2 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2-2 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2-2 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMIMO-LayersFR1-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR1-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving (see NOTE 1). This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2-1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2-1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2-1 indicated by the field, to address overheating or power saving (see NOTE 1). This field is allowed to be reported only when UE is configured with serving cells operating on FR2-1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2-1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2-2 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-2. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2-2 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2-2 indicated by the field, to address overheating or power saving (see NOTE 1). This field is allowed to be reported only when UE is configured with serving cells operating on FR2-2. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2-2 in the cell group when indicated to address power savings.

referenceLocationReport This field indicates the reference locations that are closest to the UE for assisted SMTC configuration in RRC_CONNECTED state. The length of the bitmap is equal to the number of entries in the refLocList currently configured for the UE (either via OtherConfig or SIB2). The UE sets the bit at a given position to '1' if the reference location at the corresponding position in the refLocList is one of the closest locations, and to '0' otherwise. The first bit in the string corresponds to the first reference location in the list, and so on.

referenceTimeInfoPreference Indicates whether the UE prefers being provisioned with the timing information specified in the IE ReferenceTimeInfo.

resumeCause Provides the resume cause based on the information received from the upper layers.

rlm-MeasRelaxationState Indicates the relaxation state of RLM measurements. Value true indicates that the UE is performing relaxation of RLM measurements, and value false indicates that the UE is not performing relaxation of RLM measurements.

rrm-MeasRelaxationFulfilment Indicates whether the UE fulfils the relaxed measurement criterion for stationary UE in 5.7.4.4. Value true indicates that the UE fulfils the criterion, and value false indicates that the UE does not fulfil the criterion.

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-PRS-Bandwidth Indicates the desired bandwidth of the requested SL-PRS resources provided by upper layers (see TS 38.355 [77]) in the unit of MHz.

sl-PRS-DelayBudget Indicates the SL-PRS delay budget provided by upper layers (see TS 38.355 [77]).

sl-PRS-Periodicity Indicates the periodicity of SL-PRS transmission.

sl-PRS-Priority Indicates the priority of SL-PRS provided by upper layers (see TS 38.355 [77]). Value 1 is the highest priority whereas value 8 is the lowest priority.

sl-UE-AssistanceInformationNR Indicates the traffic characteristic of sidelink logical channel(s), specified in the IE SL-TrafficPatternInfo, that are setup for NR sidelink communication.

slotOffset Indicates the UE's preferred slot offset to resolve the IDC problem, in multiples of 1/32 ms.

startOffset Indicates the UE's preferred start offset to resolve the IDC problem, in multiples of 1 ms.

type1 Indicates the preferred amount of increment/decrement to the long DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

ul-GapFR2-PatternPreference Indicates the UE's preference on FR2 UL gap pattern as defined in TS 38.133 [14].

victimSystemType Indicate the list of victim system types to which IDC interference is caused from NR. Value gps, glonass, bds, galileo and navIC indicates the type of GNSS. Value wlan indicates WLAN and value bluetooth indicates Bluetooth. Value uwb indicates Ultra Wide Band.

NOTE 1:	The field may also indicate the UE's preference on reduced configuration corresponding to the maximum number of SRS ports (i.e. nrofSRS-Ports) of each serving cell operating on the associated frequency range.

SL-TrafficPatternInfo field descriptions

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 38.321 [3], table 6.1.3.1-2.

timingOffset This field indicates the estimated timing for a packet arrival in a sidelink logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

trafficPeriodicity This field indicates the estimated data arrival periodicity in a sidelink logical channel. Value ms20 corresponds to 20 ms, ms50 corresponds to 50 ms and so on.

UL-TrafficInfo field descriptions

burstArrivalTime Indicates the expected arrival time of the first packet of the Data Burst for the concerned QoS flow. If the UE provides both burstArrivalTime and jitterRange, burstArrivalTime is used as a reference time for the indicated jitter range. If burstArrivalTime is indicated as referenceTime, the indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If burstArrivalTime is indicated as referenceSFN-AndSlot, it refers to the UL timing of the closest SFN and slot of the PCell with the indicated number.

jitterRange Indicates the maximum deviation of the arrival time of the first packet of a Data Burst compared to the time indicated with burstArrivalTime and the periodicity of the Data Bursts. lowerBound indicates the negative deviation while upperBound indicates the positive deviation. This field shall only be reported together with the burstArrivalTime or after the burstArrivalTime has been already reported. Value ms0 corresponds to 0 ms, value 0dot5 to 0.5 ms, value ms1 to 1 ms and so on. Value beyondMs7 indicates the jitter bound is higher than 7 ms. Value 0 ms means there is no Data Burst arrival time deviation from the indicated burstArrivalTime.

pdu-SetIdentification Indicates whether the UE is able to identify PDU Set(s) for the QoS flow. If set to true, the UE is able to identify PDU Set(s) for the associated QoS flow, otherwise, the UE is not able to do so. Before receiving this indication, the network assumes the value is set to false.

psi-Identification Indicates whether the UE is able to identify PSI(s) for the QoS flow. This field shall only be set to true if pdu-SetIdentification is also set to true (or was set to true previously for the same QoS flow). If set to true, the UE is able to identify PSI(s) for the associated QoS flow, otherwise, the UE is not able to do so. Before receiving this indication, the network assumes the value is set to false.

qfi Identity of the QoS flow to which this UL traffic information refers.

trafficPeriodicity Indicates the average time period between the start times of two data bursts, expressed in the number of microseconds.

GapOccasionCancelRatio field descriptions

fr1, fr2 Includes the UE's preference for gap occasion cancellation ratio for the configured per FR measurement gap under measGapConfig.

gapConfigRatioList Includes the UE's preference for gap occasion cancellation ratio for the configured measurement gap with measGapId in gapToAddModList under measGapConfig.

perUE Includes the UE's perference for gap occasion cancellation ratio for the configured per UE measurement gap under measGapConfig.

#### –	UECapabilityEnquiry

The UECapabilityEnquiry message is used to request UE radio access capabilities for NR as well as for other RATs.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UECapabilityEnquiry message

-- ASN1START

-- TAG-UECAPABILITYENQUIRY-START

UECapabilityEnquiry ::=             SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityEnquiry                 UECapabilityEnquiry-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityEnquiry-IEs ::=         SEQUENCE {

ue-CapabilityRAT-RequestList        UE-CapabilityRAT-RequestList,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

ue-CapabilityEnquiryExt             OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                 OPTIONAL --  Need N

}

UECapabilityEnquiry-v1560-IEs ::=   SEQUENCE {

capabilityRequestFilterCommon       UE-CapabilityRequestFilterCommon                                        OPTIONAL, -- Need N

nonCriticalExtension                UECapabilityEnquiry-v1610-IEs                                           OPTIONAL

}

UECapabilityEnquiry-v1610-IEs ::=   SEQUENCE {

rrc-SegAllowed-r16                  ENUMERATED {enabled}           OPTIONAL, -- Need N

nonCriticalExtension                UECapabilityEnquiry-v17b0-IEs  OPTIONAL

}

UECapabilityEnquiry-v17b0-IEs ::=   SEQUENCE {

rrc-MaxCapaSegAllowed-r17           INTEGER (2..16)                OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                    OPTIONAL

}

-- TAG-UECAPABILITYENQUIRY-STOP

-- ASN1STOP

UECapabilityEnquiry-IEs field descriptions

rrc-MaxCapaSegAllowed This field is used to enable the UL message segment transfer for UECapabilityInformation message with the number of segments allowed by the network. The field is present only if rrc-SegAllowed is not present.

rrc-SegAllowed This field is used to enable the UL message segment transfer for UECapabilityInformation message. The field is present only if rrc-MaxCapaSegAllowed is not present.

#### –	UECapabilityInformation

The IE UECapabilityInformation message is used to transfer UE radio access capabilities requested by the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UECapabilityInformation message

-- ASN1START

-- TAG-UECAPABILITYINFORMATION-START

UECapabilityInformation ::=         SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityInformation             UECapabilityInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityInformation-IEs ::=     SEQUENCE {

ue-CapabilityRAT-ContainerList      UE-CapabilityRAT-ContainerList                                          OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYINFORMATION-STOP

-- ASN1STOP

#### –	UEInformationRequest

The UEInformationRequest message is used by the network to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UEInformationRequest message

-- ASN1START

-- TAG-UEINFORMATIONREQUEST-START

UEInformationRequest-r16 ::=     SEQUENCE {

rrc-TransactionIdentifier        RRC-TransactionIdentifier,

criticalExtensions               CHOICE {

ueInformationRequest-r16         UEInformationRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

UEInformationRequest-r16-IEs ::=   SEQUENCE {

idleModeMeasurementReq-r16         ENUMERATED{true}                         OPTIONAL, -- Need N

logMeasReportReq-r16               ENUMERATED {true}                        OPTIONAL, -- Need N

connEstFailReportReq-r16           ENUMERATED {true}                        OPTIONAL, -- Need N

ra-ReportReq-r16                   ENUMERATED {true}                        OPTIONAL, -- Need N

rlf-ReportReq-r16                  ENUMERATED {true}                        OPTIONAL, -- Need N

mobilityHistoryReportReq-r16       ENUMERATED {true}                        OPTIONAL, -- Need N

lateNonCriticalExtension           OCTET STRING                             OPTIONAL,

nonCriticalExtension               UEInformationRequest-v1700-IEs           OPTIONAL

}

UEInformationRequest-v1700-IEs ::= SEQUENCE {

successHO-ReportReq-r17            ENUMERATED {true}                        OPTIONAL, -- Need N

coarseLocationRequest-r17          ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension               UEInformationRequest-v1800-IEs           OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

flightPathInfoReq-r18              FlightPathInfoReportConfig-r18           OPTIONAL, -- Need N

successPSCell-ReportReq-r18        ENUMERATED {true}                        OPTIONAL, -- Need N

reselectionMeasurementReq-r18      ENUMERATED {true}                        OPTIONAL, -- Need N

validatedMeasurementsReq-r18       ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension               UEInformationRequest-v1900-IEs           OPTIONAL

}

UEInformationRequest-v1900-IEs ::= SEQUENCE {

csi-LogMeasReportReq-r19           ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension               SEQUENCE {}                              OPTIONAL

}

FlightPathInfoReportConfig-r18 ::= SEQUENCE {

maxWayPointNumber-r18              INTEGER (1..maxWayPoint-r18),

includeTimeStamp-r18               ENUMERATED {true}                        OPTIONAL  -- Need N

}

-- TAG-UEINFORMATIONREQUEST-STOP

-- ASN1STOP

UEInformationRequest-IEs field descriptions

coarseLocationRequest This field is used to request UE to report coarse location information.

connEstFailReportReq This field is used to indicate whether the UE shall report information about the connection failure.

csi-LogMeasReportReq This field is used to indicate whether the UE shall report information about CSI radio measurements logged in RRC_CONNECTED for network-side data collection.

flightPathInfoReq This field is used to indicate whether the UE shall report the flight path information, if available, and to specify the flight path information report configuration.

idleModeMeasurementReq This field indicates that the UE shall report the idle/inactive measurement information, if available, to the network in the UEInformationResponse message.

logMeasReportReq This field is used to indicate whether the UE shall report information about logged measurements.

mobilityHistoryReportReq This field is used to indicate whether the UE shall report information about mobility history information.

ra-ReportReq This field is used to indicate whether the UE shall report information about the random access procedure.

reselectionMeasurementReq This field indicates that the UE shall report the reselection measurement information, if available, to the network in the UEInformationResponse message.

rlf-ReportReq This field is used to indicate whether the UE shall report information about the radio link failure.

successHO-ReportReq This field is used to indicate whether the UE shall report information about the successful handover report.

successPSCell-ReportReq This field is used to indicate whether the UE shall report information about the successful PSCell change or addition report.

FlightPathInfoReportConfig field descriptions

includeTimeStamp Indicates whether time stamp of each way point can be reported in the flight path information report if time stamp information is available at the UE.

maxWayPointNumber Indicates the maximum number of way points UE can include in the flight path information report if this information is available at the UE.

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included) or SRB6 (when logged measurement information for network-side data collection is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1700-IEs     OPTIONAL

}

UEInformationResponse-v1700-IEs ::=  SEQUENCE {

successHO-Report-r17                 SuccessHO-Report-r17                OPTIONAL,

connEstFailReportList-r17            ConnEstFailReportList-r17           OPTIONAL,

coarseLocationInfo-r17               OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1800-IEs     OPTIONAL

}

UEInformationResponse-v1800-IEs ::=  SEQUENCE {

flightPathInfoReport-r18             FlightPathInfoReport-r18            OPTIONAL,

successPSCell-Report-r18             SuccessPSCell-Report-r18            OPTIONAL,

measResultReselectionNR-r18          MeasResultIdleNR-r16                OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1900-IEs     OPTIONAL

}

UEInformationResponse-v1900-IEs ::=  SEQUENCE {

csi-LogMeasReport-r19                CSI-LogMeasReport-r19               OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

FlightPathInfoReport-r18 ::=         SEQUENCE (SIZE (0..maxWayPoint-r18)) OF WayPoint-r18

WayPoint-r18 ::=                     SEQUENCE {

wayPointLocation-r18                 OCTET STRING,

timeStamp-r18                        AbsoluteTimeInfo-r16                OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]],

[[

nsag-ID-r19                          NSAG-ID-r17                         OPTIONAL,

reselectedCellId-r19                 CGI-Info-Logging-r16                OPTIONAL

]]

}

ConnEstFailReport-r16 ::=            SEQUENCE {

measResultFailedCell-r16             MeasResultFailedCell-r16,

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultList2NR-r16               OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

numberOfConnFail-r16                 INTEGER (1..8),

perRAInfoList-r16                            PerRAInfoList-r16,

timeSinceFailure-r16                 TimeSinceFailure-r16,

...

}

ConnEstFailReportList-r17 ::= SEQUENCE (SIZE (1..maxCEFReport-r17)) OF ConnEstFailReport-r16

MeasResultServingCell-r16 ::=        SEQUENCE {

resultsSSB-Cell                      MeasQuantityResults,

resultsSSB                           SEQUENCE{

best-ssb-Index                       SSB-Index,

best-ssb-Results                     MeasQuantityResults,

numberOfGoodSSB                      INTEGER (1..maxNrofSSBs-r16)

}                                                                        OPTIONAL

}

MeasResultFailedCell-r16 ::=         SEQUENCE {

cgi-Info                             CGI-Info-Logging-r16,

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList

}

}

}

RA-ReportList-r16 ::= SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16

RA-Report-r16 ::=                    SEQUENCE {

cellId-r16                           CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

ra-InformationCommon-r16             RA-InformationCommon-r16                         OPTIONAL,

raPurpose-r16                        ENUMERATED {accessRelated, beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized,

schedulingRequestFailure, noPUCCHResourceAvailable, requestForOtherSI,

msg3RequestForOtherSI-r17, lbt-Failure-r18, ltm-r19, spare6, spare5, spare4, spare3,

spare2, spare1},

...,

[[

spCellID-r17                         CGI-Info-Logging-r16                             OPTIONAL

]],

[[

sdt-Failed-r18                       ENUMERATED {true}                                OPTIONAL

]],

[[

sdt-FailureCause-r19                 ENUMERATED {t319a-expiry, maxRetxThreshold, preambleTransMax, configuredGrantTimer,

cg-SDT-TimeAlignmentTimer, cellReselection, spare2, spare1}       OPTIONAL,

sdt-DL-Rsrp-Info-r19                 RSRP-Range                                       OPTIONAL,

sdt-UL-DataVolume-r19                INTEGER (0..96000)                               OPTIONAL,

timeSinceSdt-Execution-r19           TimeSinceSdt-Execution-r19                       OPTIONAL

]]

}

RA-InformationCommon-r16 ::=         SEQUENCE {

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-FrequencyStartCFRA-r16          INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing                                OPTIONAL,

msg1-SubcarrierSpacingCFRA-r16       SubcarrierSpacing                                OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}               OPTIONAL,

msg1-FDMCFRA-r16                     ENUMERATED {one, two, four, eight}               OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16,

...,

[[

perRAInfoList-v1660                  PerRAInfoList-v1660                              OPTIONAL

]],

[[

msg1-SCS-From-prach-ConfigurationIndex-r16  ENUMERATED {kHz1dot25, kHz5, spare2, spare1}  OPTIONAL

]],

[[

msg1-SCS-From-prach-ConfigurationIndexCFRA-r16  ENUMERATED {kHz1dot25, kHz5, spare2, spare1} OPTIONAL

]],

[[

msgA-RO-FrequencyStart-r17           INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msgA-RO-FrequencyStartCFRA-r17       INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msgA-SubcarrierSpacing-r17           SubcarrierSpacing                                OPTIONAL,

msgA-RO-FDM-r17                      ENUMERATED {one, two, four, eight}               OPTIONAL,

msgA-RO-FDMCFRA-r17                  ENUMERATED {one, two, four, eight}               OPTIONAL,

msgA-SCS-From-prach-ConfigurationIndex-r17  ENUMERATED {kHz1dot25, kHz5, spare2, spare1}  OPTIONAL,

msgA-TransMax-r17                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}  OPTIONAL,

msgA-MCS-r17                         INTEGER (0..15)                                   OPTIONAL,

nrofPRBs-PerMsgA-PO-r17              INTEGER (1..32)                                  OPTIONAL,

msgA-PUSCH-TimeDomainAllocation-r17  INTEGER (1..maxNrofUL-Allocations)               OPTIONAL,

frequencyStartMsgA-PUSCH-r17         INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

nrofMsgA-PO-FDM-r17                  ENUMERATED {one, two, four, eight}               OPTIONAL,

dlPathlossRSRP-r17                   RSRP-Range                                       OPTIONAL,

intendedSIBs-r17                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-Type-r17      OPTIONAL,

ssbsForSI-Acquisition-r17            SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-Index    OPTIONAL,

msgA-PUSCH-PayloadSize-r17           BIT STRING (SIZE (5))                            OPTIONAL,

onDemandSISuccess-r17                ENUMERATED {true}                                OPTIONAL

]],

[[

usedFeatureCombination-r18           ReportedFeatureCombination-r18                   OPTIONAL,

triggeredFeatureCombination-r18      ReportedFeatureCombination-r18                   OPTIONAL,

startPreambleForThisPartition-r18    INTEGER (0..63)                                  OPTIONAL,

numberOfPreamblesPerSSB-ForThisPartition-r18  INTEGER (1..64)                         OPTIONAL,

attemptedBWP-InfoList-r18            SEQUENCE (SIZE (1..maxNrofBWPs)) OF AttemptedBWP-Info-r18  OPTIONAL,

numberOfLBT-Failures-r18             INTEGER (1..128)                                 OPTIONAL,

perRAInfoList-v1800                  PerRAInfoList-v1800                              OPTIONAL,

intendedSIBs-r18                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-Type-r18      OPTIONAL

]]

}

AttemptedBWP-Info-r18 ::=            SEQUENCE {

locationAndBandwidth-r18             INTEGER (0..37949),

subcarrierSpacing-r18                SubcarrierSpacing

}

ReportedFeatureCombination-r18 ::=   SEQUENCE {

redCap-r18                           ENUMERATED {true}                                OPTIONAL,

smallData-r18                        ENUMERATED {true}                                OPTIONAL,

nsag-r18                             NSAG-List-r17                                    OPTIONAL,

msg3-Repetitions-r18                 ENUMERATED {true}                                OPTIONAL,

msg1-Repetitions-r18                 ENUMERATED {true}                                OPTIONAL,

eRedCap-r18                          ENUMERATED {true}                                OPTIONAL,

triggered-S-NSSAI-List-r18           SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI   OPTIONAL

}

PerRAInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16

PerRAInfoList-v1660 ::= SEQUENCE (SIZE (1..200)) OF PerRACSI-RSInfo-v1660

PerRAInfo-r16 ::=                    CHOICE {

perRASSBInfoList-r16                 PerRASSBInfo-r16,

perRACSI-RSInfoList-r16              PerRACSI-RSInfo-r16

}

PerRAInfoList-v1800 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-v1800

PerRAInfo-v1800 ::=                  CHOICE {

perRASSBInfoList-v1800               PerRASSBInfo-v1800,

perRACSI-RSInfoList-v1800            PerRACSI-RSInfo-v1800

}

PerRASSBInfo-r16 ::=                 SEQUENCE {

ssb-Index-r16                        SSB-Index,

numberOfPreamblesSentOnSSB-r16       INTEGER (1..200),

perRAAttemptInfoList-r16             PerRAAttemptInfoList-r16

}

PerRASSBInfo-v1800 ::=               SEQUENCE {

allPreamblesBlocked                  ENUMERATED {true}                                OPTIONAL,

lbt-Detected-r18                     ENUMERATED {true}                                OPTIONAL,

...

}

PerRACSI-RSInfo-r16 ::=              SEQUENCE {

csi-RS-Index-r16                     CSI-RS-Index,

numberOfPreamblesSentOnCSI-RS-r16    INTEGER (1..200)

}

PerRACSI-RSInfo-v1660 ::=            SEQUENCE {

csi-RS-Index-v1660                   INTEGER (1..96)                                  OPTIONAL

}

PerRACSI-RSInfo-v1800 ::=            SEQUENCE {

allPreamblesBlocked                  ENUMERATED {true}                                OPTIONAL,

lbt-Detected-r18                     ENUMERATED {true}                                OPTIONAL,

...

}

PerRAAttemptInfoList-r16 ::=         SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16

PerRAAttemptInfo-r16 ::=             SEQUENCE {

contentionDetected-r16               BOOLEAN                OPTIONAL,

dlRSRPAboveThreshold-r16             BOOLEAN                OPTIONAL,

...,

[[

fallbackToFourStepRA-r17             ENUMERATED {true}      OPTIONAL

]]

}

SIB-Type-r17 ::= ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType9, sibType10, sibType11, sibType12,

sibType13, sibType14, posSIB-v1810, spare5, spare4, spare3, spare2, spare1}

SIB-Type-r18 ::= ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20,

sibType21, sibType22, sibType23, sibType24, sibType25, spare5, spare4,

spare3, spare2, spare1}

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

failedPCellId-r16                    CHOICE {

nrFailedPCellId-r16                  CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

eutraFailedPCellId-r16           CHOICE {

cellGlobalId-r16                 CGI-InfoEUTRALogging,

pci-arfcn-r16                    PCI-ARFCN-EUTRA-r16

}

},

reconnectCellId-r16                  CHOICE {

nrReconnectCellId-r16                CGI-Info-Logging-r16,

eutraReconnectCellId-r16             CGI-InfoEUTRALogging

}                                                                                        OPTIONAL,

timeUntilReconnection-r16            TimeUntilReconnection-r16                           OPTIONAL,

reestablishmentCellId-r16            CGI-Info-Logging-r16                                OPTIONAL,

timeConnFailure-r16                  INTEGER (0..1023)                                   OPTIONAL,

timeSinceFailure-r16                 TimeSinceFailure-r16,

connectionFailureType-r16            ENUMERATED {rlf, hof},

rlf-Cause-r16                        ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

beamFailureRecoveryFailure, lbtFailure-r16,

bh-rlfRecoveryFailure, t312-expiry-r17, spare1},

locationInfo-r16                     LocationInfo-r16                                    OPTIONAL,

noSuitableCellFound-r16              ENUMERATED {true}                                   OPTIONAL,

ra-InformationCommon-r16             RA-InformationCommon-r16                            OPTIONAL,

...,

[[

csi-rsRLMConfigBitmap-v1650          BIT STRING (SIZE (96))                              OPTIONAL

]],

[[

lastHO-Type-r17                      ENUMERATED {cho, daps, ltm-v1900, choWithCandidateSCG-v1900}              OPTIONAL,

timeConnSourceDAPS-Failure-r17       TimeConnSourceDAPS-Failure-r17                      OPTIONAL,

timeSinceCHO-Reconfig-r17            TimeSinceCHO-Reconfig-r17                           OPTIONAL,

choCellId-r17                        CHOICE {

cellGlobalId-r17                     CGI-Info-Logging-r16,

pci-arfcn-r17                        PCI-ARFCN-NR-r16

}                                                                                        OPTIONAL,

choCandidateCellList-r17             ChoCandidateCellList-r17                            OPTIONAL

]],

[[

pSCellId-r18                         CHOICE {

cellGlobalId-r18                     CGI-Info-Logging-r16,

pci-arfcn-r18                        PCI-ARFCN-NR-r16

}                                                                                        OPTIONAL,

mcg-RecoveryFailureCause-r18         ENUMERATED {t316-Expiry, scg-Deactivated, spare2, spare1}  OPTIONAL,

scg-FailureCause-r18                 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, scg-lbtFailure, beamFailureRecoveryFailure,

t312-Expiry, bh-RLF, beamFailure, spare5, spare4, spare3, spare2, spare1 }

OPTIONAL,

elapsedTimeSCG-Failure-r18           ElapsedTimeSCG-Failure-r18                          OPTIONAL,

voiceFallbackHO-r18                  ENUMERATED {true}                                   OPTIONAL,

measResultLastServCellRSSI-r18       RSSI-Range-r16                                      OPTIONAL,

measResultNeighFreqListRSSI-r18      MeasResultNeighFreqListRSSI-r18                     OPTIONAL,

bwp-Info-r18                         AttemptedBWP-Info-r18                               OPTIONAL,

elapsedTimeT316-r18                  ElapsedTimeT316-r18                                 OPTIONAL,

scg-FailedAfterMCG-r18               ENUMERATED {true}                                   OPTIONAL
        ]],

[[

measResultL1-LastServCell-r19        MeasResultL1-r19                                    OPTIONAL,

measResultL1-NeighCells-r19          MeasResultList3NR-r19                               OPTIONAL,

ltm-RecoveryCellId-r19               CHOICE {

cellGlobalId-r19                     CGI-Info-Logging-r16,

pci-arfcn-r19                        PCI-ARFCN-NR-r16

}                                                                                        OPTIONAL,

measResultLastServPSCell-r19         MeasResultRLFNR-r16                                 OPTIONAL,

cho-WithCandidateSCGInfoList-r19     Cho-WithCandidateSCGInfoList-r19                    OPTIONAL,

distanceFromReference1-r19           INTEGER(0.. 65535)                                  OPTIONAL

]]

},

eutra-RLF-Report-r16                 SEQUENCE {

failedPCellId-EUTRA                  CGI-InfoEUTRALogging,

measResult-RLF-Report-EUTRA-r16      OCTET STRING,

...,

[[

measResult-RLF-Report-EUTRA-v1690    OCTET STRING                                        OPTIONAL

]]

}

}

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                               OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

locationInfo-r17                         LocationInfo-r16                                    OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                           OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                                       OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                            OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                          OPTIONAL,

c-RNTI-r17                               RNTI-Value                                          OPTIONAL,

...,

[[

targetCell-PCI-ARFCN-r17                 PCI-ARFCN-NR-r16                                    OPTIONAL

]],

[[

eutra-TargetCellInfo-r18                 SEQUENCE {

targetPCellId-r18                        CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

targetCellMeas-r18                       MeasQuantityResultsEUTRA                       OPTIONAL

}                                                                                           OPTIONAL,

measResultServCellRSSI-r18               RSSI-Range-r16                                     OPTIONAL,

measResultNeighFreqListRSSI-r18          MeasResultNeighFreqListRSSI-r18                    OPTIONAL,

eutra-C-RNTI-r18                         EUTRA-C-RNTI                                       OPTIONAL,

timeSinceSHR-r18                         TimeSinceSHR-r18                                   OPTIONAL

]],

[[

sourceCellMeasL1-r19                     MeasResultL1-r19                                   OPTIONAL,

targetCellMeasL1-r19                     MeasResultL1-r19                                   OPTIONAL,

neighCellsMeasL1ListNR-r19               MeasResultList3NR-r19                              OPTIONAL,

rach-Less-r19                            ENUMERATED {true}                                  OPTIONAL,

sourcePSCellInfo-r19                     SEQUENCE {

sourcePSCellId-r19                       CGI-Info-Logging-r16,

sourcePSCellMeas-r19                     MeasResultSuccessHONR-r17                      OPTIONAL

}                                                                                           OPTIONAL,

cho-WithCandidateSCGInfoList-r19         Cho-WithCandidateSCGInfoList-r19                   OPTIONAL,

targetPSCellID-r19                       CHOICE {

cellGlobalId-r19                         CGI-Info-Logging-r16,

pci-arfcn-r19                            PCI-ARFCN-NR-r16

}                                                                                           OPTIONAL

]]

}

SuccessPSCell-Report-r18 ::=             SEQUENCE {

pCellId-r18                              CGI-Info-Logging-r16,

sourcePSCellInfo-r18                     SEQUENCE {

sourcePSCellId-r18                       CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

sourcePSCellMeas-r18                     MeasResultSuccessHONR-r17                       OPTIONAL

}                                                                                            OPTIONAL,

targetPSCellInfo-r18                     SEQUENCE {

targetPSCellId-r18                       CHOICE {

cellGlobalId-r18                     CGI-Info-Logging-r16,

pci-arfcn-r18                        PCI-ARFCN-NR-r16

},

targetPSCellMeas-r18                     MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r18                 SEQUENCE {

measResultListNR-r18                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r18                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

spr-Cause-r18                            SPR-Cause-r18                                       OPTIONAL,

timeSinceCPAC-Reconfig-r18               TimeSinceCPAC-Reconfig-r18                          OPTIONAL,

locationInfo-r18                         LocationInfo-r16                                    OPTIONAL,

ra-InformationCommon-r18                 RA-InformationCommon-r16                            OPTIONAL,

sn-InitiatedPSCellChange-r18             ENUMERATED {true}                                   OPTIONAL,

...,

[[

cho-WithCandidateSCGInfoList-r19         Cho-WithCandidateSCGInfoList-r19                    OPTIONAL,

targetPCellID-r19                        CHOICE {

cellGlobalId-r19                         CGI-Info-Logging-r16,

pci-arfcn-r19                            PCI-ARFCN-NR-r16

}                                                                                            OPTIONAL,

c-RNTI-r19                               RNTI-Value                                          OPTIONAL

]]

}

MeasResultNeighFreqListRSSI-r18 ::=      SEQUENCE(SIZE (1..maxFreq)) OF MeasResultNeighFreqRSSI-r18

MeasResultNeighFreqRSSI-r18 ::=          SEQUENCE {

ssbFrequency-r18                         ARFCN-ValueNR                                       OPTIONAL,

ssbSubcarrierSpacing-r18                 SubcarrierSpacing                                   OPTIONAL,

refFreqCSI-RS-r18                        ARFCN-ValueNR                                       OPTIONAL,

measResult-RSSI-r18                      RSSI-Range-r16                                      OPTIONAL

}

MeasResultList2NR-r16 ::=            SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2NR-r16

MeasResultList2EUTRA-r16 ::=         SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16

MeasResult2NR-r16 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                                           OPTIONAL,

refFreqCSI-RS-r16                    ARFCN-ValueNR                                           OPTIONAL,

measResultList-r16                   MeasResultListNR

}

MeasResultListLogging2NR-r16 ::=     SEQUENCE(SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16

MeasResultLogging2NR-r16 ::=         SEQUENCE {

carrierFreq-r16                      ARFCN-ValueNR,

measResultListLoggingNR-r16          MeasResultListLoggingNR-r16

}

MeasResultListLoggingNR-r16 ::=      SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

MeasResult2EUTRA-r16 ::=             SEQUENCE {

carrierFreq-r16                      ARFCN-ValueEUTRA,

measResultList-r16                   MeasResultListEUTRA

}

MeasResultRLFNR-r16 ::=              SEQUENCE {

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults                             OPTIONAL,

resultsCSI-RS-Cell-r16               MeasQuantityResults                             OPTIONAL

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList                         OPTIONAL,

ssbRLMConfigBitmap-r16               BIT STRING (SIZE (64))                          OPTIONAL,

resultsCSI-RS-Indexes-r16            ResultsPerCSI-RS-IndexList                      OPTIONAL,

csi-rsRLMConfigBitmap-r16            BIT STRING (SIZE (96))                          OPTIONAL

}                                                                                    OPTIONAL

}

}

MeasResultSuccessHONR-r17::=         SEQUENCE {

measResult-r17                       SEQUENCE {

cellResults-r17                      SEQUENCE{

resultsSSB-Cell-r17                  MeasQuantityResults                             OPTIONAL,

resultsCSI-RS-Cell-r17               MeasQuantityResults                             OPTIONAL

},

rsIndexResults-r17                   SEQUENCE{

resultsSSB-Indexes-r17               ResultsPerSSB-IndexList                         OPTIONAL,

resultsCSI-RS-Indexes-r17            ResultsPerCSI-RS-IndexList                      OPTIONAL

}

}

}

ChoCandidateCellList-r17 ::=         SEQUENCE(SIZE (1..maxNrofCondCells-r16)) OF ChoCandidateCell-r17

ChoCandidateCell-r17 ::=             CHOICE {

cellGlobalId-r17                     CGI-Info-Logging-r16,

pci-arfcn-r17                        PCI-ARFCN-NR-r16

}

SHR-Cause-r17 ::=                    SEQUENCE {

t304-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

t310-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

t312-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

sourceDAPS-Failure-r17               ENUMERATED {true}                                       OPTIONAL,

...

}

SPR-Cause-r18 ::=                    SEQUENCE {

t304-cause-r18                       ENUMERATED {true}                                       OPTIONAL,

t310-cause-r18                       ENUMERATED {true}                                       OPTIONAL,

t312-cause-r18                       ENUMERATED {true}                                       OPTIONAL,

...

}

CSI-LogMeasReport-r19 ::=            SEQUENCE {

csi-LogMeasInfoCellList-r19          CSI-LogMeasInfoCellList-r19,

csi-MoreLogMeasAvailable-r19         ENUMERATED {true}                                       OPTIONAL,

...

}

CSI-LogMeasInfoCellList-r19 ::=      SEQUENCE (SIZE (1..maxNrofServingCells)) OF CSI-LogMeasInfoCell-r19

CSI-LogMeasInfoCell-r19 ::=          SEQUENCE {

cellId-r19                           CHOICE {

cellGlobalId-r19                     CGI-Info-Logging-r16,

pci-arfcn-r19                        PCI-ARFCN-NR-r16

},

csi-LogMeasInfoList-r19              SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19,

...

}

CSI-LogMeasInfo-r19 ::=              SEQUENCE {

refCSI-LoggedMeasurementConfigId-r19 CSI-LoggedMeasurementConfigId-r19,

csi-RS-MeasResultList-r19            SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19   OPTIONAL,

ssb-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19                  OPTIONAL,

timeGap-r19                          ENUMERATED {true}                                                           OPTIONAL,    ...

}

CSI-RS-MeasResult-r19 ::=            SEQUENCE {

resourceId-r19                       NZP-CSI-RS-ResourceId,

l1-RSRP-r19                          RSRP-Range

}

SSB-MeasResult-r19 ::=               SEQUENCE {

ssb-Id-r19                           SSB-Index,

l1-RSRP-r19                          RSRP-Range

}

MeasResultL1-r19 ::=                 SEQUENCE {

resultsSSB-Indexes-r19               ResultsPerSSB-IndexList                                 OPTIONAL,

...

}

MeasResultList3NR-r19 ::=            SEQUENCE (SIZE (1..maxFreq)) OF MeasResult3NR-r19

MeasResult3NR-r19 ::=                SEQUENCE {

ssbFrequency-r19                     ARFCN-ValueNR                                           OPTIONAL,

l1-MeasResultList-r19                L1-MeasResultList-r19                                   OPTIONAL,

...

}

L1-MeasResultList-r19 ::=            SEQUENCE (SIZE (1..maxCellReport)) OF L1-MeasResultPerCell-r19

L1-MeasResultPerCell-r19 ::=         SEQUENCE {

physCellId-r19                       PhysCellId,

resultsSSB-Indexes-r19               ResultsPerSSB-IndexList                                 OPTIONAL,

...

}

Cho-WithCandidateSCGInfoList-r19 ::=             SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF Cho-WithCandidateSCGInfo-r19

TimeSinceFailure-r16 ::= INTEGER (0..172800)

MobilityHistoryReport-r16 ::= VisitedCellInfoList-r16

TimeUntilReconnection-r16 ::= INTEGER (0..172800)

TimeSinceCHO-Reconfig-r17 ::= INTEGER (0..1023)

TimeSinceCPAC-Reconfig-r18 ::= INTEGER (0.. 1023)

TimeConnSourceDAPS-Failure-r17 ::= INTEGER (0..1023)

UPInterruptionTimeAtHO-r17 ::= INTEGER (0..1023)

ElapsedTimeT316-r18 ::= INTEGER (0..2000)

ElapsedTimeSCG-Failure-r18 ::= INTEGER (0..1023)

TimeSinceSHR-r18 ::= INTEGER (0..172800)

TimeSinceSdt-Execution-r19 ::= INTEGER (0..172800)

-- TAG-UEINFORMATIONRESPONSE-STOP

-- ASN1STOP

UEInformationResponse-IEs field descriptions

coarseLocationInfo Parameter type Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement.

connEstFailReport This field is used to provide connection establishment failure or connection resume failure information.

connEstFailReportList This field is used to provide the list of connEstFailReport that are stored by the UE for the past up to maxCEFReport-r17.

csi-LogMeasReport This field is used to provide the logged measurement results for network-side data collection, stored by the UE in accordance with the CSI-LoggedMeasurementConfig.

flightPathInfoReport This field is used to provide the flight path information as list of waypoints and, if available, corresponding timestamps. List of size zero indicates the previously provided flight path information is no longer valid.

logMeasReport This field is used to provide the measurement results stored by the UE associated to logged MDT.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE or RRC_IDLE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE or RRC_IDLE.

ra-ReportList This field is used to provide the list of RA reports that is stored by the UE for up to maxRAReport-r16 number of random access procedures. If the UE is an eRedCap UE, this field is used to provide the list of RA reports that is stored by the UE for up to 2 number of random access procedures.

rlf-Report This field is used to indicate the RLF report related contents.

successHO-Report This field is used to provide the successful handover report if triggered based on the successful handover configuration.

successPSCell-Report This field is used to provide the successful PSCell change or addition report if triggered based on the successful PSCell change or addition report configuration.

LogMeasReport field descriptions

absoluteTimeStamp Indicates the absolute time when the logged measurement configuration logging is provided, as indicated by NR within absoluteTimeInfo.

anyCellSelectionDetected This field is used to indicate the detection of any cell selection state, as defined in TS 38.304 [20]. The UE sets this field when performing the logging of measurement results in RRC_IDLE or RRC_INACTIVE and there is no suitable cell or no acceptable cell.

inDeviceCoexDetected Indicates that measurement logging is suspended due to IDC problem detection.

measResultServingCell This field refers to the log measurement results taken in the Serving cell.

nsag-ID Indicates the NSAG ID with the highest priority, for the UE that was configured with slice-based cell reselection and was not able to perform a cell reselection to a cell asscoiated with the NSAG with highest priority (as specified in TS 38.304 [20]).

numberOfGoodSSB Indicates the number of good beams (beams that are above absThreshSS-BlocksConsolidation, if configured by the network) associated to the cells within the R value range (which is configured by network for cell reselection) of the highest ranked cell as part of the beam level measurements. If the UE has no SSB of a neighbour cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE does not include numberOfGoodSSB for the corresponding neighbour cell. If the UE has no SSB of the serving cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE shall set the numberOfGoodSSB for the serving cell to one.

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

reselectedCellId Indicates the cell that does not support the NSAG ID with highest priority, for the UE that was configured with slice-based cell reselection and was not able to perform a cell reselection to a cell asscoiated with NSAG with the highest priority (as specified in TS 38.304 [20]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure or connection resume failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure or connection resume failure happened.

numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures in the same cell independent of RRC state transition.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

RA-InformationCommon field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

allPreamblesBlocked This field is included when the all the preamble transmission attempts in the corresponding beam (SSB or CSI-RS) are blocked by failed LBT.

attemptedBWP-InfoList This field indicates locationAndBandwidth and subcarrierSpacing of all the bandwidth parts in which the consistent LBT failures are triggered at the moment of successful RA completion.

locationAndBandwidth Frequency domain location and bandwidth of the bandwidth part associated to the random-access resources used by the UE or of the bandwidth part in which the consistent LBT failures is triggered and not cancelled prior to successful completion of random access procedure (if this field is included in attemptedBWP-InfoList) or prior to RLF/HOF (if this field is included in attemptedBWP-InfoList or bwp-Info).

numberOfLBT-Failures This field is used to indicate the total number of preamble transmission attempts for which LBT failure indication is received in the RA procedure. If the number of LBT failure indications received from lower layers during the RA procedure exceeds or equals to 128, UE sets the field to 128.This field is optional present when there is at least one preamble transmission attempt for which LBT failure indication is received during the RA procedure, otherwise it is absent.

numberOfPreamblesPerSSB-ForThisPartition This field determines how many consecutive preambles are associated to the used feature or combination of features starting from the starting preamble(s) per SSB.

perRAInfoList, perRAInfoList-v1660 This field provides detailed information about each of the random access attempts in the chronological order of the random access attempts. If perRAInfoList-v1660 is present, it shall contain the same number of entries, listed in the same order as in perRAInfoList-r16.

startPreambleForThisPartition This field indicates the first preamble associated with the used feature or combination of features.

subcarrierSpacing Subcarrier spacing used in the bandwidth part associated to the random-access resources used by the UE or of the bandwidth part in which the consistent LBT failures is triggered and not cancelled prior to successful completion of random access procedure (if this field is included in attemptedBWP-InfoList) or prior to RLF/HOF (if this field is included in attemptedBWP-InfoList or bwp-Info).

triggeredFeatureCombination One or more features (e.g., RedCap, Slicing, SDT and MSG3 repetition) that triggers the random-access procedure. When triggered feature is Slicing, UE includes all the S-NSSAIs associated to the slices triggering the access attempt in the random-access procedure.

usedFeatureCombination The feature or combination of features (e.g., redCap, smallData, nsag and msg3-Repetitions) associated to the used random-access resources as specified in TS 38.321[3].

RA-Report field descriptions

cellID This field indicates the CGI of the cell in which the associated random access procedure was performed.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not. This field is not included when the UE performs random access attempt is using contention free random-access resources or when the raPurpose is set to requestForOtherSI or when the RA attempt is a 2-step RA attempt and fallback to 4-step RA did not occur (i.e. fallbackToFourStepRA is not included).

csi-RS-Index, csi-RS-Index-v1660 This field is used to indicate the CSI-RS index corresponding to the random access attempt. If the random access procedure is for beam failure recovery, the field indicates the NZP-CSI-RS-ResourceId. For CSI-RS index larger than maxNrofCSI-RS-ResourcesRRM-1, the index value is the sum of csi-RS-Index (without suffix) and csi-RS-Index-v1660.

dlPathlossRSRP Measeured RSRP of the DL pathloss reference obtained at the time of RA_Type selection stage of the RA procedure as captured in TS 38.321 [3].

dlRSRPAboveThreshold In 4 step random access procedure, this field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold rsrp-ThresholdSSB in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure initiated for beam failure recovery. Otherwise, if the UE has received rsrp-ThresholdSSB-r17 in FeatureCombinationPreambles used for the feature specific random access, the field is used to indicate whether DL beam (SSB) quality associated to the random access attempt was above or below this rsrp-ThresholdSSB-r17; else, if rsrp-ThresholdSSB-r17 is not included in FeatureCombinationPreambles used for the feature specific random access, this field is used to indicate whether DL beam (SSB) quality associated to the random access attempt was above or below rsrp-ThresholdSSB included in the RACH-ConfigCommon which includes this FeatureCombinationPreambles; else, this field is used to indicate whether DL beam (SSB) quality associated to the random access attempt was above or below rsrp-ThresholdSSB in rach-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure. In 2 step random access procedure, if the UE has received rsrp-ThresholdSSB-r17 in FeatureCombinationPreambles used for the feature specific random access, the field is used to indicate whether DL beam (SSB) quality associated to the random access attempt was above or below this rsrp-ThresholdSSB-r17; else, if rsrp-ThresholdSSB-r17 is not included in FeatureCombinationPreambles used for the feature specific random access, this field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold msgA-RSRP-ThresholdSSB included in the RACH-ConfigCommonTwoStepRA which includes this FeatureCombinationPreambles; else, this field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold msgA-RSRP-ThresholdSSB in rach-ConfigCommonTwoStepRA in UL BWP configuration of UL BWP selected for random access procedure.

fallbackToFourStepRA This field indicates if a fallback indication in MsgB is received (according to TS 38.321 [3]) for the 2-step random access attempt.

intendedSIBs This field indicates the SIB(s) the UE wanted to receive as a result of the on demand SI request (when the RA procedure is a used as a SI request) initiated by the UE. That is, it indicates the one(s) of the SIB(s) in the SI message(s) requested to be broadcast that the UE was interested in. Value posSIB indicates that the UE wanted to receive one or more positioning SIB(s).

lbt-Detected This field is included when there is at least one LBT failure indication received prior to change of beam for preamble transmission during RA procedure, otherwise this field is absent.

msg1-SCS-From-prach-ConfigurationIndex This field is set by the UE with the corresponding SCS for CBRA as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric when the msg1-SubcarrierSpacing is absent; otherwise, this field is absent.

msg1-SCS-From-prach-ConfigurationIndexCFRA This field is set by the UE with the corresponding SCS for CFRA as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric when the msg1-SubcarrierSpacing is absent; otherwise, this field is absent.

msgA-PUSCH-PayloadSize This field indicates the size of the overall payload available in the UE buffer at the time of initiating the 2 step RA procedure. The value refers to the index of TS 38.321 [3], table 6.1.3.1-1, corresponding to the UE buffer size.

msgA-RO-FDM This field indicates the number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one time instance for the PRACH resources configured for 2-step CBRA..

msgA-RO-FDMCFRA This field indicates the number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one time instance for the PRACH resources configured for 2-step CFRA.

msgA-RO-FrequencyStart This field indicates the lowest resource block of the contention based random-access resources for 2-step CBRA in the random-access procedure. The indication has the form of the offset of the lowest PRACH transmissions occasion with respect to PRB 0 in the frequency domain.

msgA-RO-FrequencyStartCFRA This field indicates the lowest resource block of the contention free random-access resources for the 2-step CFRA in the random-access procedure. The indication has the form of the offset of the lowest PRACH transmissions occasion with respect to PRB 0 in the frequency domain.

msgA-SCS-From-prach-ConfigurationIndex This field is set by the UE with the corresponding SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]) when the msgA-SubcarrierSpacing is absent and when only 2-step random-access resources are available in the UL BWP used in the random-access procedure; otherwise, this field is absent.

numberOfPreamblesSentOnCSI-RS This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding CSI-RS.

numberOfPreamblesSentOnSSB This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding SS/PBCH block.

onDemandSISuccess This field is set to true when the RA report entry is included because of either msg1 based on demand SI request or msg3 based on demand SI request and if the on-demand SI request is successful. Otherwise, the field is absent.

perRAAttemptInfoList This field provides detailed information about a random access attempt.

perRACSI-RSInfoList This field provides detailed information about the successive random access attempts associated to the same CSI-RS.

perRASSBInfoList This field provides detailed information about the successive random access attempts associated to the same SS/PBCH block.

ra-InformationCommon This field is used to provide information on random access attempts. This field is mandatory present.

raPurpose This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, RRC re-establishment procedure, transition from RRC-INACTIVE. The indicator beamFailureRecovery is used in case of successful beam failure recovery related RA procedure in the SpCell [3]. The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync. The indicator ltm is used if the UE executes a RACH-based LTM cell switch. The indicator ulUnSynchronized is used if the random access procedure is initiated in a SpCell by DL or UL data arrival during RRC_CONNECTED when the timeAlignmentTimer is not running in the PTAG or if the RA procedure is initiated in a serving cell by a PDCCH order [3]. The indicator schedulingRequestFailure is used in case of SR failures [3]. The indicator noPUCCHResourceAvailable is used when the UE has no valid SR PUCCH resources configured [3]. The indicator requestForOtherSI is used for MSG1 based on demand SI request. The indicator msg3RequestForOtherSI is used in case of MSG3 based SI request. The indication lbtFailure is used when the UE initiates RACH in SpCell due to consistent uplink LBT failures [3]. The field can also be used for the SCG-related RA-Report when the raPurpose is set to beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized, schedulingRequestFailure, noPUCCHResourceAvailable and lbtFailure.

sdt-DL-Rsrp-Info This field logs the RSRP value measured by UE during evaluation of SDT procedure. This field is included when the RA report entry is included because of SDT initiation failure. Otherwise, the field is absent.

sdt-Failed This field is included when the RA report entry is included because of SDT and if the SDT transmission failed. Otherwise, the field is absent.

sdt-FailureCause This field is included when the RA report entry is included because of SDT and if the SDT procedure failed. Otherwise, the field is absent. This field indicates the SDT failure cause. The field is set to t319a-expiry upon expiration of T319a timer. If the UE upper layers receive maxRetxThreshold reached indication from RLC while SDT procedure is ongoing, this field is set to maxRetxThreshold. It is set to preambleTransMax upon the UE upper layer receiving indication of reaching preambleTransMax from the MAC layer. Upon expiration of cg-SDT-TimeAlignmentTimer from the MAC, the field is set to cg-SDT-TimeAlignmentTimer. The field is set to configuredGrantTimer upon reception of indication that configuration grant timer has been expired from the MAC. The field is set to cellReselection upon SDT failure due to UE’s cell re-selection.

sdt-UL-DataVolume The field is set to t319a-expiry upon expiration of T319a timer. If the UE upper layers receive maxRetxThreshold reached indication from RLC while SDT procedure is ongoing, this field is set to maxRetxThreshold. It is set to preambleTransMax upon the UE upper layer receiving indication of reaching preambleTransMax from the MAC layer. Upon expiration of cg-SDT-TimeAlignmentTimer from the MAC, the field is set to cg-SDT-TimeAlignmentTimer. The field is set to configuredGrantTimer upon reception of indication that configuration grant timer has been expired from the MAC. The field is set to cellReselection upon SDT failure due to UE’s cell re-selection.

spCellID This field is used to indicate the CGI of the SpCell of the cell group associated to the SCell in which the associated random access procedure was performed. If the UE performs RA procedure on a SCell associated to the MCG, then this field is set to the CGI of the PCell and if the UE performs RA procedure on a SCell associated to the SCG, then this field is set to the CGI of the PSCell. If the CGI of the PSCell is not available at the UE for the RA procedure performed on a SCell associated to the SCG or for the RA procedure on the PSCell, this field is set to the CGI of the PCell. Otherwise, the field is absent.

ssb-Index This field is used to indicate the SS/PBCH index of the SS/PBCH block corresponding to the random access attempt.

ssbsForSI-Acquisition This field indicates the SSB(s) (in the form of SSB index(es)) that the UE used to receive the requested SI message(s). The field is present if the purpose of the random access procedure was to request on-demand SI (i.e. if the raPurpose is set to requestForOtherSI or msg3RequestForOtherSI). Otherwise, the field is absent.

timeSinceSdt-Execution This field logs the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

RLF-Report field descriptions

bwp-Info This field is used to indicate the BWP information in which the UE detected consistent uplink LBT failure. This field is set only when the detected consistent uplink LBT failure did not trigger the random access procedure.

choCandidateCellList This field is used to indicate the list of candidate target cells for conditional handover included in condRRCReconfig at the time of connection failure. The field does not include the candidate target cells included in measResultNeighCells.

choCellId This field is used to indicate the candidate target cell for conditional handover included in condRRCReconfig that the UE selected for CHO based recovery while T311 is running.

connectionFailureType This field is used to indicate whether the connection failure is due to radio link failure or handover failure.

csi-rsRLMConfigBitmap,csi-rsRLMConfigBitmap-v1650 These fields are used to indicate the CSI-RS indexes configured in the RLM configurations for the active BWP when the UE declares RLF or HOF. The UE first fills in the csi-rsRLMConfigBitmap-r16 to indicate the first 96 CSI-RS indexes and then csi-rsRLMConfigBitmap-v1650 to indicate the latter 96 CSI-RS indexes. The first/leftmost bit in csi-rsRLMConfigBitmap-r16 corresponds to CSI-RS index 0, the second bit corresponds to CSI-RS index 1. The first/leftmost bit in csi-rsRLMConfigBitmap-v1650 corresponds to CSI-RS index 96, the second bit corresponds to CSI-RS index 97. These fields are included only if the RadioLinkMonitoringConfig for the respective BWP is configured.

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

distanceFromReference1 This field indicates the measured distance between UE and the moving reference locations of the serving cell if the conditional handover is based on condEventD2. Each step represents 50m. The actual distance shall be rounded down to the nearest step value (i.e., FLOOR(actual distance[m] / 50)). The maximum value is 65535, which indicates a distance equal to or greater than 65535 multiplied by 50m.

elapsedTimeSCG-Failure This field is used to indicate the time elapsed between the SCG failure and the MCG failure. The maximum value 1023 means 1023ms or longer.

elapsedTimeT316 This field is used to indicate the value of the elapsed time of the timer T316. Value in milliseconds.

failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. For intra-NR handover nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

failedPCellId-EUTRA This field is used to indicate the PCell in which RLF is detected or the source PCell of the failed handover in an E-UTRA RLF report.

lastHO-Type This field is used to indicate the type of the last executed handover or LTM cell switch before the last detected connection failure. The field is set to cho if the last executed handover was initiated by a conditional reconfiguration execution. The field is set to daps if the last executed handover was a DAPS handover. The field is set to ltm if the last executed RRCReconfiguration message including reconfigurationWithSync was an LTM cell switch.This field is set to choWithCandidateSCG if the last executed RRCReconfiguraiton associated to both condExecutionCond and condExecutionCondPSCell.

tm-RecoveryCellId This field is used to indicate the candidate target cell for LTM cell switch included in ltm-Config associated with the MCG that the UE selected for LTM based recovery while T311 was running.

mcg-RecoveryFailureCause This field is used to indicate the cause of the fast MCG recovery failure.

measResultL1-LastServCell This field contains the last L1 measurement results collected in the PCell upon detecting radio link failure or the source PCell upon reconfiguration with sync failure if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell (in case HO failure) or PCell (in case RLF).

measResultL1-NeighCells This field contains the last L1 measurement results collected in neighbour MCG LTM candidate cells upon reconfiguration with sync failure or upon detecting radio link failure.

measResultListEUTRA This field refers to the last measurement results taken in the neighboring EUTRA Cells, when the radio link failure or handover failure happened.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells, when the radio link failure or handover failure happened.

measResultLastServCell This field refers to the log measurement results taken in the PCell upon detecting radio link failure or the source PCell upon handover failure.

measResultLastServCellRSSI This field refers to the log RSSI measurement results in dBm (see TS 38.215 [9]) taken for the frequency of the PCell upon detecting radio link failure or source PCell upon detecting handover failure.

measResultNeighFreqListRSSI This field is used to log the RSSI measurement results in dBm (see TS 38.215 [9]) taken for the neighbouring frequencies upon detecting radio link failure or handover failure, when UE operates in shared spectrum.

measResult-RLF-Report-EUTRA Includes the E-UTRA RLF-Report-r9 IE as specified in TS 36.331 [10].

measResult-RLF-Report-EUTRA-v1690 Includes the E-UTRA RLF-Report-v9e0 IE as specified in TS 36.331 [10].

noSuitableCellFound This field is set by the UE when the T311 expires.

previousPCellId This field is used to indicate the source PCell of the last handover (source PCell when the last executed RRCReconfiguration message including reconfigurationWithSync was received, or when the last MobilityFromNRCommand message concerned an inter-RAT handover from NR to EUTRA was received). For intra-NR handover or the handover from NR to EUTRA nrPreviousCell is included and for the handover from EUTRA to NR eutraPreviousCell is included.

pSCellId This field is used to indicate the PSCell in which the UE failed to perform fast MCG recovery procedure or the UE successfully performed fast MCG recovery procedure or the source PSCell of the CHO with candidate SCG procedure failure.

ra-InformationCommon This field is optionally included when connectionFailureType is set to 'hof' and a random access procedure is triggered for the failed reconfiguration with sync, or when connectionFailureType is set to 'rlf' and the rlf-Cause equals to 'randomAccessProblem' or 'beamRecoveryFailure'; otherwise this field is absent.

reconnectCellId This field is used to indicate the cell in which the UE comes back to connected after connection failure and after failing to perform reestablishment, or to indicate the suitable cell in which the UE reconnects after failure in performing MobilityFromNRCommand for voice fallback (without initiating re-establishment procedure). If the UE comes back to RRC CONNECTED in an NR cell then nrReconnectCellID is included and if the UE comes back to RRC CONNECTED in an LTE cell then eutraReconnectCellID is included.

reestablishmentCellId If the UE was not configured with conditionalReconfiguration at the time of re-establishment attempt, or if the cell selected for the re-establishment attempt is not a candidate target cell for conditional reconfiguration, this field is used to indicate the cell in which the re-establishment attempt was made after connection failure.

rlf-Cause This field is used to indicate the cause of the last radio link failure that was detected. In case of handover failure information reporting (i.e., the connectionFailureType is set to 'hof'), the UE is allowed to set this field to any value, except for the case in which a radio link failure was detected in the source PCell while performing a DAPS handover.

scg-FailedAfterMCG This field is set if for the SCG failure is detected after MCG failure while T316 is running.

ssbRLMConfigBitmap This field is used to indicate the SS/PBCH block indexes configured in the RLM configurations for the active BWP when the UE declares RLF or HOF.The first/leftmost bit corresponds to SSB index 0, the second bit corresponds to SSB index 1. This field is included only if the RadioLinkMonitoringConfig for the respective BWP is configured.

timeConnFailure This field is used to indicate the time elapsed since the last HO or LTM cell switch execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeConnSourceDAPS-Failure This field is used to indicate the time that elapsed between the last DAPS handover execution and the radio link failure detected in the source cell while T304 is running. Value in milliseconds. The maximum value 1023 means 1023ms or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link, handover or LTM cell switch) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link, handover or LTM cell switch) failure.

timeSinceCHO-Reconfig In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last handover execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeUntilReconnection This field is used to indicate the time that elapsed between the connection (radio link, handover or LTM cell switch) failure and the next time the UE comes to RRC CONNECTED in an NR or EUTRA cell, after failing to perform reestablishment. Value in seconds. The maximum value 172800 means 172800s or longer.

voiceFallbackHO This field is set if for the failed mobility from NR, the voiceFallbackIndication was included in the MobilityFromNRCommand message.

SuccessHO-Report field descriptions

c-RNTI This field indicates the C-RNTI assigned by the target PCell of the handover for which the successful HO report was generated.

eutra-TargetCellInfo This field is used to indicate the target EUTRA PCell and the last measurement results of the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

eutra-C-RNTI This field indicates the C-RNTI assigned by the E-UTRA target PCell of the mobility from NR command for which the successful HO report was generated.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells when a successful handover is executed.

measResultNeighFreqListRSSI This field is used to log the RSSI measurement results in dBm (see TS 38.215 [9]) taken for the neighbouring frequencies upon successful handover execution.

measResultServCellRSSI This field refers to the log RSSI measurement results in dBm (see TS 38.215 [9]) taken for the frequency of the source PCell upon successful handover execution.

neighCellsMeasL1ListNR This field refers to the last L1 measurement results taken in the neighboring MCG LTM candidate Cells when a successful reconfiguration with sync is executed if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell.

rach-Less This field is set if the successful HO report is trigged by RACH-less LTM cell switch.

rlf-InSourceDAPS This field indicates whether a radio link failure occurred at the source cell while T304 was running.

shr-Cause This field is used to indicate the cause of the successful HO report.

sourceCellMeas This field refers to the last measurement results taken in the source PCell of a handover in which the successful handover triggers the SuccessHO-Report.

sourceCellMeasL1 This field refers to the last L1 measurement results taken in the source PCell of a reconfiguration with sync if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell.

sourcePCellId This field is used to indicate the source PCell of a handover in which the successful handover triggers the SuccessHO-Report.

sourcePSCellId This field is used to indicate the source PSCell of a PSCell change associated with a CHO with candidate SCG in which the successful execution of CHO triggers the SuccessHO-Report.

sourcePSCellMeas This field refers to the last measurement results taken in the source PSCell of a PSCell change associated with a CHO with candidate SCG in which the successful execution of CHO triggers the SuccessHO-Report.

targetPCellId This field is used to indicate the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

targetPSCellId This field is used to indicate the target PSCell of a PSCell change/addition associated with a CHO with candidate SCG in which the successful execution of CHO triggers the SuccessHO-Report.

targetCellMeas This field refers to the last measurement results taken in the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

targetCellMeasL1 This field refers to the last L1 measurement results taken in the target PCell of a reconfiguration with sync if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell.

timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceSHR This field is used to indicate the time elapsed since the execution of the last MobilityFromNRCommand towards the target EUTRA cell. Value in seconds. The maximum value 172800 means 172800s or longer.

upInterruptionTimeAtHO This field is used to indicate the time elapsed between the time of arrival of the last PDCP PDU received from the source cell for any data radio bearer and the time of arrival of the first non-duplicate PDCP PDU received from the target cell for any data radio bearer, and it is measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell for any data radio bearer. The field is set only in case of DAPS handover. Value in milliseconds. The maximum value 1023 means 1023ms or longer.

FlightPathInfoReport field descriptions

timeStamp Time stamp that describes estimated time of arrival, if available, of the UE at the corresponding wayPointLocation.

wayPointLocation Location coordinates of the planned waypoint. Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

SuccessPSCell-Report field descriptions

c-RNTI This field indicates the C-RNTI assigned by the target PCell of the conditional handover successfully executed as part of CHO with candidate SCG in which the SuccessPSCell-Report was triggered.

measResultListNR This field refers to the last measurement results according to the initiating node configuration taken in the neighboring NR Cells when a successful PSCell change/addition is executed.

pCellId This field is used to indicate the PCell to which the UE was connected when the successful PSCell change or addition triggers the SuccessPSCell-Report. Alternatively this field indicates the source PCell to which the UE was connected to before executing CHO with candidate SCG in which the SuccessPSCell-Report was triggered.

sn-InitiatedPSCellChange This field indicates whether the PSCell change procedure for which the successful PSCell change report is logged is SN initiated or not.

spr-Cause This field is used to indicate the cause of the successful PSCell change or addition report.

sourcePSCellId This field is used to indicate the source PSCell of a PSCell change in which the successful PSCell change triggers the SuccessPSCell-Report.

sourcePSCellMeas This field refers to the last measurement results taken in the source PSCell of a PSCell change in which the successful PSCell change triggers the SuccessPSCell-Report.

targetPCellId This field is used to indicate the target PCell of a CHO with candidate SCG procedure in which the SuccessPSCell-Report was triggered.

targetPSCellId This field is used to indicate the target PSCell of a PSCell change/addition in which the successful PSCell change or addition triggers the SuccessPSCell-Report.

targetPSCellMeas This field refers to the last measurement results taken in the target PSCell of a PSCell change/addition in which the successful PSCell change or addition triggers the SuccessPSCell-Report.

timeSinceCPAC-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target PSCell and the reception of the latest conditional reconfiguration for this target PSCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

CSI-LogMeasReport field descriptions

cellId This field indicates the CGI of the cell in which the logging of the measurements included within csi-LogMeasInfoList was performed. If the CGI is not available, this field indicates the PCI-ARFCN-NR.

csi-MoreLogMeasAvailable This field is included if the UE has additional logged L1 radio measurements available for transmission.

csi-RS-MeasResultList List of logged L1 radio measurement results associated to CSI-RS resources.

l1-RSRP Indicates the measured L1 RSRP associated to the resourceId, if included within CSI-RS-MeasResult. Indicates the measured L1 RSRP associated to the ssb-Id, if included within SSB-MeasResult.

refCSI-LoggedMeasurementConfigId Reference to the instance of CSI-LoggedMeasurementConfig associated to the L1 radio measurement results reported in csi-RS-MeasResultList or ssb-MeasResultList.

resourceId Indicates the NZP-CSI-RS-ResourceId associated to which the UE has logged L1 radio measurement results.

ssb-Id Indicates the SSB-Index associated to which the UE has logged L1 radio measurement results.

ssb-MeasResultList List of logged L1 radio measurement results associated to SSBs.

timeGap Indicates that there was a time gap, longer than the logging periodicity, between the reported measurement results in this instance of CSI-LogMeasInfoList and the previous instance of CSI-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId for the same serving cell.

#### –	UEPositioningAssistanceInfo

The UEPositioningAssistanceInfo message is used to provide positioning assistance information as requested by the Network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEPositioningAssistanceInfo message

-- ASN1START

-- TAG-UEPOSITIONINGASSISTANCEINFO-START

UEPositioningAssistanceInfo-r17 ::= SEQUENCE {

criticalExtensions                  CHOICE {

uePositioningAssistanceInfo-r17     UEPositioningAssistanceInfo-r17-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEPositioningAssistanceInfo-r17-IEs ::= SEQUENCE {

ue-TxTEG-AssociationList-r17            UE-TxTEG-AssociationList-r17          OPTIONAL,

lateNonCriticalExtension                OCTET STRING                          OPTIONAL,

nonCriticalExtension                    UEPositioningAssistanceInfo-v1720-IEs OPTIONAL

}

UEPositioningAssistanceInfo-v1720-IEs::=    SEQUENCE {

ue-TxTEG-TimingErrorMarginValue-r17         ENUMERATED {tc0, tc2, tc4, tc6, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc56,

tc64, tc72, tc80}                                      OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                        OPTIONAL

}

UE-TxTEG-AssociationList-r17 ::= SEQUENCE (SIZE (1..maxNrOfTxTEGReport-r17)) OF UE-TxTEG-Association-r17

UE-TxTEG-Association-r17 ::=        SEQUENCE {

ue-TxTEG-ID-r17                     INTEGER (0..maxNrOfTxTEG-ID-1-r17),

nr-TimeStamp-r17                    NR-TimeStamp-r17,

associatedSRS-PosResourceIdList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16,

servCellId-r17                      ServCellIndex                            OPTIONAL

}

NR-TimeStamp-r17 ::= SEQUENCE {

nr-SFN-r17           INTEGER (0..1023),

nr-Slot-r17          CHOICE {

scs15-r17            INTEGER (0..9),

scs30-r17            INTEGER (0..19),

scs60-r17            INTEGER (0..39),

scs120-r17           INTEGER (0..79)

},

...

}

-- TAG-UEPOSITIONINGASSISTANCEINFO-STOP

-- ASN1STOP

UEPositioningAssistanceInfo field descriptions

associatedSRS-PosResourceIdList This field indicates list of SRS for Positioning Resources which are associated to the ue-TxTEG-ID.

nr-TimeStamp This field specifies the latest time instance at which the association is valid prior to the reporting.

servCellID This field indicates the serving cell information of SRS for positioning resources associated to the UE Tx TEG report.

ue-TxTEG-ID Identifies the ID of UE Tx TEG.

ue-TxTEG-TimingErrorMarginValue This field specifies the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo. Value tc0 corresponds to 0 Tc, tc2 corresponds to 2 Tc and so on (see TS 37.355 [49]).

#### –	ULDedicatedMessageSegment

The ULDedicatedMessageSegment message is used to transfer segments of the UECapabilityInformation or MeasurementReportAppLayer message. SRB1 is used at transfer of segments of UECapabilityInformation and SRB4 or SRB5 is used at transfer of segments of MeasurementReportAppLayer.

Signalling radio bearer: SRB1, SRB4 or SRB5

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

ULDedicatedMessageSegment message

-- ASN1START

-- TAG-ULDEDICATEDMESSAGESEGMENT-START

ULDedicatedMessageSegment-r16 ::=       SEQUENCE {

criticalExtensions                      CHOICE {

ulDedicatedMessageSegment-r16           ULDedicatedMessageSegment-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

ULDedicatedMessageSegment-r16-IEs ::=     SEQUENCE {

segmentNumber-r16                         INTEGER (0..15),

rrc-MessageSegmentContainer-r16           OCTET STRING,

rrc-MessageSegmentType-r16                ENUMERATED {notLastSegment, lastSegment},

lateNonCriticalExtension                  OCTET STRING                                  OPTIONAL,

nonCriticalExtension                      SEQUENCE {}                                   OPTIONAL

}

-- TAG-ULDEDICATEDMESSAGESEGMENT-STOP

-- ASN1STOP

ULDedicatedMessageSegment field descriptions

segmentNumber Identifies the sequence number of a segment within the encoded UL DCCH message.

rrc-MessageSegmentContainer Includes a segment of the encoded UL DCCH message. The size of the included segment in this container should be small enough that the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit.

rrc-MessageSegmentType Indicates whether the included UL DCCH message segment is the last segment or not.

#### –	ULInformationTransfer

The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information, or IAB-DU specific F1-C related information.

Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resumed. If only dedicatedInfoF1c is included, SRB2 is used.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

ULInformationTransfer message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFER-START

ULInformationTransfer ::=           SEQUENCE {

criticalExtensions                  CHOICE {

ulInformationTransfer               ULInformationTransfer-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransfer-IEs ::=       SEQUENCE {

dedicatedNAS-Message                DedicatedNAS-Message                OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                ULInformationTransfer-v1700-IEs     OPTIONAL

}

ULInformationTransfer-v1700-IEs ::=       SEQUENCE {

dedicatedInfoF1c-r17                      DedicatedInfoF1c-r17                OPTIONAL,

nonCriticalExtension                      SEQUENCE {}                         OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFER-STOP

-- ASN1STOP

#### –	ULInformationTransferIRAT

The ULInformationTransferIRAT message is used for the uplink transfer of information terminated at NR MCG but specified by another RAT. In this version of the specification, the message is used for V2X sidelink communication messages specified in TS 36.331 [10].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

ULInformationTransferIRAT message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERIRAT-START

ULInformationTransferIRAT-r16 ::=              SEQUENCE {

criticalExtensions                                CHOICE {

c1                                                   CHOICE {

ulInformationTransferIRAT-r16                    ULInformationTransferIRAT-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture                     SEQUENCE {}

}

}

ULInformationTransferIRAT-r16-IEs ::=        SEQUENCE {

ul-DCCH-MessageEUTRA-r16                           OCTET STRING             OPTIONAL,

lateNonCriticalExtension                           OCTET STRING              OPTIONAL,

nonCriticalExtension                                SEQUENCE {}               OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERIRAT-STOP

-- ASN1STOP

ULInformationTransferIRAT field descriptions

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message as defined in TS 36.331 [10]. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport, E-UTRA RRC SidelinkUEInformation and the E-UTRA RRC UEAssistanceInformation messages.

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR-DC dedicated information (e.g. for transferring the NR or E-UTRA RRC MeasurementReport message, the FailureInformation message, the UEAssistanceInformation message, the RRCReconfigurationComplete message, the IABOtherInformation message or the NR or E-UTRA RRC MCGFailureInformation message).

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

ULInformationTransferMRDC message

-- ASN1START

-- TAG-ULINFORMATIONTRANSFERMRDC-START

ULInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

ulInformationTransferMRDC                   ULInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ULInformationTransferMRDC-IEs::=           SEQUENCE {

ul-DCCH-MessageNR                           OCTET STRING                    OPTIONAL,

ul-DCCH-MessageEUTRA                        OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-ULINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, RRCReconfigurationComplete, UEAssistanceInformation, FailureInformation, and IABOtherInformation messages when sent via SRB1 and to transfer the NR MCGFailureInformation message when sent via SRB3.

ul-DCCH-MessageEUTRA Includes the UL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRC MeasurementReport message when sent via SRB1 and to transfer the E-UTRA MCGFailureInformation message when sent via SRB3.

## 6.3	RRC information elements

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUPRELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

-- TAG-SETUPRELEASE-STOP

-- ASN1STOP

### 6.3.1	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...,

[[

od-sib1-CellReselectionPriority-r19             CellReselectionPriority             OPTIONAL,       -- Need R

od-sib1-CellReselectionSubPriority-r19          CellReselectionSubPriority          OPTIONAL        -- Need R

]]

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need R

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]],

[[

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                    OPTIONAL         -- Cond SharedSpectrum2

]],

[[

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

]],

[[

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760              OPTIONAL,       -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760              OPTIONAL        -- Need R

]],

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18         OPTIONAL        -- Need S

]],

[[

refLocList-r19                      RefLocList-r19                                  OPTIONAL,        -- Need R

smtc5list-r19                       SEQUENCE (SIZE(1..6)) OF SSB-MTC5-r19           OPTIONAL         -- Need R

]]

},

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

cellEquivalentSize-r17                  INTEGER(2..16)                                  OPTIONAL,       -- Cond HSDN

relaxedMeasurement-r17                  SEQUENCE {

stationaryMobilityEvaluation-r17        SEQUENCE {

s-SearchDeltaP-Stationary-r17           ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17           ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

},

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

uav-PrioritizedFrequency-r19                ENUMERATED {true}                           OPTIONAL,       -- Need R

uav-PrioritizedFrequencyAltitudeRange-r19   SEQUENCE {

altitudeMin-r19                             Altitude-r18                            OPTIONAL,       -- Need S

altitudeMax-r19                             Altitude-r18                            OPTIONAL,       -- Need S

altitudeHyst-r19                            HysteresisAltitude-r18                  OPTIONAL        -- Need R

}                                                                                       OPTIONAL,       -- Need R

ssb-ToMeasureAltitudeBasedList-r19          SSB-ToMeasureAltitudeBasedList-r18          OPTIONAL,       -- Need R

relaxedMeasurementForServingAndNeighboringCell-r19 SEQUENCE {

cellEdgeEvaluationOnMR-ForLR-OnSSB-r19      SEQUENCE {

s-SearchThresholdP3-r19                     ReselectionThreshold,

s-SearchThresholdQ3-r19                     ReselectionThresholdQ               OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Cond SupportLR-OnSSB

cellEdgeEvaluationOnMR-ForLR-OnLPSS-r19     SEQUENCE {

s-SearchThresholdP4-r19                     ReselectionThreshold,

s-SearchThresholdQ4-r19                     ReselectionThresholdQ               OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Cond SupportLR-OnLPSS

cellEdgeEvaluationOnLR-ForLR-OnLPSS-r19     SEQUENCE {

rsrpThresholdLR-r19                         ThresholdP-LR-r19,

rsrqThresholdLR-r19                         ThresholdQ-LR-r19                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluationOnLR-ForLR-OnSSB-r19      SEQUENCE {

rsrpThresholdLR2-r19                        ThresholdP-LR-r19,

rsrqThresholdLR2-r19                        ThresholdQ-LR-r19                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL        -- Need R

}                                                                                       OPTIONAL,       -- Need R

offloadMeasurementForServingCell-r19        SEQUENCE {

cellEdgeEvaluationOnMR-ForLR-OnSSB-r19      SEQUENCE {

s-SearchThresholdP5-r19                     ReselectionThreshold,

s-SearchThresholdQ5-r19                     ReselectionThresholdQ               OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Cond SupportLR-OnSSB

cellEdgeEvaluationOnMR-ForLR-OnLPSS-r19 SEQUENCE {

s-SearchThresholdP6-r19                 ReselectionThreshold,

s-SearchThresholdQ6-r19                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Cond SupportLR-OnLPSS

cellEdgeEvaluationOnLR-ForLR-OnLPSS-r19 SEQUENCE {

rsrpThresholdLR3-r19                    ThresholdP-LR-r19,

rsrqThresholdLR3-r19                    ThresholdQ-LR-r19                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluationOnLR-ForLR-OnSSB-r19  SEQUENCE {

rsrpThresholdLR4-r19                    ThresholdP-LR-r19,

rsrqThresholdLR4-r19                    ThresholdQ-LR-r19                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluationOnLR-ForLR-OnLPSS-Exit-r19  SEQUENCE {

rsrpThresholdLR5-r19                    ThresholdP-LR-r19,

rsrqThresholdLR5-r19                    ThresholdQ-LR-r19                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Cond SupportLR-OnLPSS

cellEdgeEvaluationOnLR-ForLR-OnSSB-Exit-r19  SEQUENCE {

rsrpThresholdLR6-r19                    ThresholdP-LR-r19,

rsrqThresholdLR6-r19                    ThresholdQ-LR-r19                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL        -- Cond SupportLR-OnSSB

}                                                                                       OPTIONAL        -- Need R

]]

}

RangeToBestCell    ::= Q-OffsetRange

RefLocList-r19 ::= SEQUENCE (SIZE(1..6)) OF ReferenceLocation-r17

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

cellEdgeEvaluation Indicates the criteria for a UE to detect that it is not at cell edge, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.2).

cellEdgeEvaluationOnLR-ForLR-OnLPSS Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS, in order to relax serving cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.12.2), or to offload serving cell measurement to low power receiver (see TS 38.304 [20], clause 5.2.4.12.4).

cellEdgeEvaluationOnLR-ForLR-OnLPSS-Exit Indicates the exit criteria for serving cell measurement offloading for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. This field is optional present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS.

cellEdgeEvaluationOnLR-ForLR-OnSSB Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OFDM based LP-WUR measuring on SSB, in order to relax serving cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.12.2), or to offload serving cell measurement to low power receiver (see TS 38.304 [20], clause 5.2.4.12.4).

cellEdgeEvaluationOnLR-ForLR-OnSSB-Exit Indicates the exit criteria for serving cell measurement offloading for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OFDM based LP-WUR measuring on SSB.

cellEdgeEvaluationOnMR-ForLR-OnLPSS Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on main radio for OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS, in order to relax serving cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.12.2), or to offload serving cell measurement to low power receiver (see TS 38.304 [20], clause 5.2.4.12.4).

cellEdgeEvaluationOnMR-ForLR-OnSSB Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on main radio for OFDM based LP-WUR measuring on SSB, in order to relax serving cell cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.12.2), or to offload serving cell measurement to low power receiver (see TS 38.304 [20], clause 5.2.4.12.4).

cellEdgeEvaluationWhileStationary Indicates the criteria for a UE to detect that it is not at cell edge while stationary, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.4).

cellEquivalentSize The number of cell count used for mobility state estimation for this cell as specified in TS 38.304 [20].

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

combineRelaxedMeasCondition When both lowMobilityEvaluation and cellEdgeEvaluation criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when either or both of the criteria are met. (See TS 38.304 [20], clause 5.2.4.9.0)

combineRelaxedMeasCondition2 When both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when only the stationary criteria is met. (See TS 38.304 [20], clause 5.2.4.9.0)

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

highPriorityMeasRelax Indicates whether measurements can be relaxed on high priority frequencies. If the field is absent, the UE shall not relax measurements on high priority frequencies beyond "Thigher_priority_search" unless both low mobility and not at cell edge criteria are fulfilled (see TS 38.133 [14], clauses 4.2.2.7, 4.2.2.10 and 4.2.2.11).

intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells.

lowMobilityEvaluation Indicates the criteria for a UE to detect low mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.1).

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20].

od-sib1-CellReselectionPriority, od-sib1-CellReselectionSubPriority Cell reselection priorities to be considered by a UE supporting OD-SIB1 instead of cellReselectionPriority, cellReselectionSubPriority as specified in TS 38.304 [20].

offloadMeasurementForServingCell Configuration to allow offloading of serving cell RRM measurement on MR to serving cell RRM measurement on LP-WUR (see TS 38.304 [20], clause 5.2.4.12).

p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

q-Hyst Parameter "Qhyst" in TS 38.304 [20], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 38.304 [20]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 38.304 [20]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

rangeToBestCell Parameter "rangeToBestCell" in TS 38.304 [20]. The network configures only non-negative (in dB) values.

refLocList Indicates a reference location associated to an SMTC configuration in smtc5list. If present, it includes the same number of entries as smtc5list. The first entry in this list corresponds to the first entry in smtc5list, the second entry corresponds to the seccond entry in smtc5list, and so on.

relaxedMeasurement Configuration to allow relaxation of RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9). In NTN, this field is only applicable for GSO neighbour cells.

relaxedMeasurementForServingAndNeighboringCell Configuration to allow relaxation of serving cell and neighboring cell RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.12).

rsrpThresholdLR, rsrpThresholdLR2, rsrpThresholdLR3, rsrpThresholdLR4, rsrpThresholdLR5, rsrpThresholdLR6 Parameters "SRSRPThresholdLR", "SRSRPThresholdLR2", "SRSRPThresholdLR3", "SRSRPThresholdLR4", "SRSRPThresholdLR5", and "SRSRPThresholdLR6" in TS 38.304 [20]. The network configures rsrpThresholdLR3 and rsrpThresholdLR4 to be larger than or equal to rsrpThresholdLR and rsrpThresholdLR2, respectively, if there is such configuration(s).

rsrqThresholdLR, rsrqThresholdLR2, rsrThresholdLR3, rsrqThresholdLR4, rsrqThresholdLR5, rsrqThresholdLR6 Parameters "SRSRQThresholdLR", "SRSRQThresholdLR2", "SRSRQThresholdLR3", "SRSRQThresholdLR4", "SRSRQThresholdLR5", and "SRSRQThresholdLR6" in TS 38.304 [20]. The network configures rsrqThresholdLR3 and rsrqThresholdLR4 to be larger than or equal to rsrqThresholdLR and rsrqThresholdLR2, respectively, if there is such configuration(s).

s-IntraSearchP Parameter "SIntraSearchP" in TS 38.304 [20].

s-IntraSearchQ Parameter "SIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 38.304 [20]. If this field is absent, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 38.304 [20]. Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on.

s-SearchDeltaP-Stationary Parameter "SSearchDeltaP-Stationary" in TS 38.304 [20]. Value dB2 corresponds to 2 dB, dB3 corresponds to 3 dB and so on.

s-SearchThresholdP, s-SearchThresholdP2, s-SearchThresholdP3, s-SearchThresholdP4, s-SearchThresholdP5, s-SearchThresholdP6 Parameters "SSearchThresholdP", "SSearchThresholdP2", "SSearchThresholdP3", "SSearchThresholdP4", "SSearchThresholdP5", and "SSearchThresholdP6" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. The network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP, if there is such configuration(s). The network configures s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-SearchThresholdP3 and s-SearchThresholdP4, respectively, if there is such configuration(s).

s-SearchThresholdQ, s-SearchThresholdQ2, s-SearchThresholdQ3, s-SearchThresholdQ4, s-SearchThresholdQ5, s-SearchThresholdQ6 Parameters "SSearchThresholdQ", "SSearchThresholdQ2", "SSearchThresholdQ3", "SSearchThresholdQ4", "SSearchThresholdQ5", and "SSearchThresholdQ6" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ. The network configures both s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to s-IntraSearchQ and s-NonIntraSearchQ, if there is such configuration(s). The network configures s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to s-SearchThresholdQ3 and s-SearchThresholdQ4, respectively, if there is such configuration(s).

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc2-LP Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity. This field is not configured together with smtc4list.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

ssb-PositionQCL-Common Indicates the QCL relation between SS/PBCH blocks for intra-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. The UE behaviour is specified in TS 38.304 [20]. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

stationaryMobilityEvaluation Indicates the criteria for a UE to detect stationary mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.0).

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

threshServingLowP Parameter "ThreshServing, LowP" in TS 38.304 [20].

threshServingLowQ Parameter "ThreshServing, LowQ" in TS 38.304 [20].

t-SearchDeltaP Parameter "TSearchDeltaP" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

t-SearchDeltaP-Stationary Parameter "TSearchDeltaP-Stationary" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

uav-PrioritizedFrequency This field indicates this is a frequency prioritized by Aerial UE for cell reselection as specified in TS 38.304 [20]

uav-PrioritizedFrequencyAltitudeRange Indicates the altitude range where the configuration of uav-PrioritizedFrequency is valid. If absent, the uav-PrioritizedFrequency flag (if included), applies for all altitudes. For this altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For this altitude range, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

| Conditional Presence | Explanation |
| --- | --- |
| HSDN | The field is optionally present, Need R, if speedStateReselectionPars is present; otherwise the field is not present. |
| SharedSpectrum | This field is mandatory present if this intra-frequency operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this intra-frequency operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| SupportLR-OnLPSS | This field is mandatory present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. It is absent otherwise. |
| SupportLR-OnSSB | This field is mandatory present for the cell supporting OFDM based LP-WUR measuring on SSB. It is absent otherwise. |

#### –	SIB3

SIB3 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as exclude-listed cells.

SIB3 information element

-- ASN1START

-- TAG-SIB3-START

SIB3 ::=                            SEQUENCE {

intraFreqNeighCellList              IntraFreqNeighCellList                                          OPTIONAL,   -- Need R

intraFreqExcludedCellList           IntraFreqExcludedCellList                                       OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

...,

[[

intraFreqNeighCellList-v1610        IntraFreqNeighCellList-v1610                                    OPTIONAL,   -- Need R

intraFreqAllowedCellList-r16        IntraFreqAllowedCellList-r16                                    OPTIONAL,   -- Cond SharedSpectrum2

intraFreqCAG-CellList-r16           SEQUENCE (SIZE (1..maxPLMN)) OF IntraFreqCAG-CellListPerPLMN-r16    OPTIONAL    -- Need R

]],

[[

intraFreqNeighHSDN-CellList-r17     IntraFreqNeighHSDN-CellList-r17                                 OPTIONAL,    -- Need R

intraFreqNeighCellList-v1710        IntraFreqNeighCellList-v1710                                    OPTIONAL     -- Need R

]],

[[

channelAccessMode2-r17              ENUMERATED {enabled}                                            OPTIONAL     -- Need R

]],

[[

intraFreqOD-SIB1-ExcludedCellList-r19    CHOICE {

emptyList-r19                            NULL,

excludedCells-r19                        IntraFreqExcludedCellList

}                                                                                                   OPTIONAL     -- Need R

]]

}

IntraFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellList-v1610::=     SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo-v1610

IntraFreqNeighCellList-v1710 ::=     SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo-v1710

IntraFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                  OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                  OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                  OPTIONAL,   -- Need R

...

}

IntraFreqNeighCellInfo-v1610 ::=     SEQUENCE {

ssb-PositionQCL-r16                 SSB-PositionQCL-Relation-r16                    OPTIONAL   -- Cond SharedSpectrum2

}

IntraFreqNeighCellInfo-v1710 ::=     SEQUENCE {

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                    OPTIONAL   -- Cond SharedSpectrum2

}

IntraFreqExcludedCellList ::=       SEQUENCE (SIZE (1..maxCellExcluded)) OF PCI-Range

IntraFreqAllowedCellList-r16 ::=    SEQUENCE (SIZE (1..maxCellAllowed)) OF PCI-Range

IntraFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16               INTEGER (1..maxPLMN),

cag-CellList-r16                     SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

}

IntraFreqNeighHSDN-CellList-r17 ::= SEQUENCE (SIZE (1..maxCellIntra)) OF PCI-Range

-- TAG-SIB3-STOP

-- ASN1STOP

SIB3 field descriptions

channelAccessMode2 If present, this field indicates that intra-frequency neighbor cells apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the intra-frequency neighbor cells do not apply any channel access procedure.

intraFreqAllowedCellList List of allow-listed intra-frequency neighbouring cells, see TS 38.304 [20], clause 5.2.4.

intraFreqCAG-CellList List of intra-frequency neighbouring CAG cells (as defined in TS 38.304 [20]) per PLMN.

intraFreqExcludedCellList List of exclude-listed intra-frequency neighbouring cells.

intraFreqNeighCellList List of intra-frequency neighbouring cells with specific cell re-selection parameters. If intraFreqNeighCellList-v1610 is present, it shall contain the same number of entries, listed in the same order as in intraFreqNeighCellList (without suffix).

intraFreqNeighHSDN-CellList List of intra-frequency neighbouring HSDN cells as specified in TS 38.304 [20].

intraFreqOD-SIB1-ExcludedCellList List of exclude-listed intra-frequency neighbouring cells for a UE supporting OD-SIB1 as specified in TS 38.304 [20]. If intraFreqOD-SIB1-ExcludedCellList is present, the UE ignores intraFreqExcludedCellList (without suffix).

plmn-IdentityIndex Index of the PLMN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

q-OffsetCell Parameter "Qoffsets,n" in TS 38.304 [20].

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 38.304 [20]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 38.304 [20]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

q-RxLevMinOffsetCellSUL Parameter "QrxlevminoffsetcellSUL" in TS 38.304 [20]. Actual value QrxlevminoffsetcellSUL = field value * 2 [dB].

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific intra-frequency neighbor cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the value signalled by ssb-PositionQCL-Common in SIB2 for the indicated cell.

| Conditional Presence | Explanation |
| --- | --- |
| SharedSpectrum2 | The field is optional present, Need R, if this intra-frequency or neighbor cell operates with shared spectrum channel access. Otherwise, it is absent, Need R. |

#### –	SIB4

SIB4 contains information relevant for inter-frequency cell re-selection (i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1700      InterFreqCarrierFreqList-v1700              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1720      InterFreqCarrierFreqList-v1720              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1730      InterFreqCarrierFreqList-v1730              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1760      InterFreqCarrierFreqList-v1760              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1800      InterFreqCarrierFreqList-v1800              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1900      InterFreqCarrierFreqList-v1900              OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1610 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqList-v1700 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700

InterFreqCarrierFreqList-v1720 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1720

InterFreqCarrierFreqList-v1730 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1730

InterFreqCarrierFreqList-v1760 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1760

InterFreqCarrierFreqList-v1800 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800

InterFreqCarrierFreqList-v1900 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1900

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                         OPTIONAL,   -- Need R

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqExcludedCellList           InterFreqExcludedCellList                                   OPTIONAL,   -- Need R

...

}

InterFreqCarrierFreqInfo-v1610 ::=  SEQUENCE {

interFreqNeighCellList-v1610        InterFreqNeighCellList-v1610                                OPTIONAL,    -- Need R

smtc2-LP-r16                        SSB-MTC2-LP-r16                                             OPTIONAL,    -- Need R

interFreqAllowedCellList-r16        InterFreqAllowedCellList-r16                                OPTIONAL,    -- Cond SharedSpectrum2

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                                OPTIONAL,    -- Cond SharedSpectrum

interFreqCAG-CellList-r16           SEQUENCE (SIZE (1..maxPLMN)) OF InterFreqCAG-CellListPerPLMN-r16   OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1700 ::=  SEQUENCE {

interFreqNeighHSDN-CellList-r17     InterFreqNeighHSDN-CellList-r17                             OPTIONAL,    -- Need R

highSpeedMeasInterFreq-r17          ENUMERATED {true}                                           OPTIONAL,    -- Need R

redCapAccessAllowed-r17             ENUMERATED {true}                                           OPTIONAL,    -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,    -- Cond SharedSpectrum

interFreqNeighCellList-v1710        InterFreqNeighCellList-v1710                                OPTIONAL     -- Cond SharedSpectrum2

}

InterFreqCarrierFreqInfo-v1720 ::=  SEQUENCE {

smtc4list-r17                       SSB-MTC4List-r17                                            OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1730 ::=  SEQUENCE {

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1760 ::=  SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760                          OPTIONAL,    -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760                          OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1800 ::=  SEQUENCE {

dl-CarrierFreq-r18                  ARFCN-ValueNR                                               OPTIONAL,    -- Cond LessThan5MHz

frequencyBandList-r18               MultiFrequencyBandListNR-SIB                                OPTIONAL,    -- Cond LessThan5MHz

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                     OPTIONAL,    -- Need S

mobileIAB-CellList-r18              PCI-Range                                                   OPTIONAL,    -- Need R

mobileIAB-Freq-r18                  ENUMERATED {true}                                           OPTIONAL,    -- Need R

eRedCapAccessAllowed-r18            ENUMERATED {true}                                           OPTIONAL,    -- Need R

tn-AreaIdList-r18                   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF TN-AreaId-r18    OPTIONAL,    -- Need R

accessAllowed2RxXR-r18              ENUMERATED {true}                                           OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1900 ::=  SEQUENCE {

uav-PrioritizedFrequency-r19                   ENUMERATED {true}                                OPTIONAL,    -- Need R

uav-PrioritizedFrequencyAltitudeRange-r19      SEQUENCE {

altitudeMin-r19                                Altitude-r18                                 OPTIONAL,    -- Need S

altitudeMax-r19                                Altitude-r18                                 OPTIONAL,    -- Need S

altitudeHyst-r19                               HysteresisAltitude-r18                       OPTIONAL     -- Need R

}                                                                                               OPTIONAL,    -- Need R

ssb-ToMeasureAltitudeBasedList-r19                 SSB-ToMeasureAltitudeBasedList-r18           OPTIONAL,    -- Need R

interFreqOD-SIB1-ExcludedCellList-r19          CHOICE {

emptyList-r19                                   NULL,

excludedCells-r19                               InterFreqExcludedCellList

}                                                                                               OPTIONAL,    -- Need R

od-sib1-cellReselectionPriority-r19            CellReselectionPriority                          OPTIONAL,    -- Need R

od-sib1-cellReselectionSubPriority-r19         CellReselectionSubPriority                       OPTIONAL     -- Need R

}

InterFreqNeighHSDN-CellList-r17 ::= SEQUENCE (SIZE (1..maxCellInter)) OF PCI-Range

InterFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighCellList-v1710 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1710

InterFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                              OPTIONAL,   -- Need R

...

}

InterFreqNeighCellInfo-v1610 ::=    SEQUENCE {

ssb-PositionQCL-r16                 SSB-PositionQCL-Relation-r16                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqNeighCellInfo-v1710 ::=    SEQUENCE {

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqExcludedCellList ::=       SEQUENCE (SIZE (1..maxCellExcluded)) OF PCI-Range

InterFreqAllowedCellList-r16 ::=    SEQUENCE (SIZE (1..maxCellAllowed)) OF PCI-Range

InterFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16              INTEGER (1..maxPLMN),

cag-CellList-r16                    SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

}

-- TAG-SIB4-STOP

-- ASN1STOP

SIB4 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

accessAllowed2RxXR Indicates if the cells on the frequency support 2Rx XR UEs. If present, 2Rx XR UEs shall consider only these NR frequencies in cell reselection evaluation.

channelAccessMode2 If present, this field indicates that the neighbor cells on the inter-frequency apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the neighbor cells on the inter-frequency do not apply any channel access procedure.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

dl-CarrierFreq This field indicates center frequency of the SS block of the neighbour cells, where the frequency corresponds to a GSCN value as specified in TS 38.101-1 [15] or TS 38.101-5 [75]. For a neighbouring carrier frequency when dl-CarrierFreq-r18 is included, the network sets the corresponding value of dl-CarrierFreq (without suffix) to 250, and the UE applies dl-CarrierFreq-r18 instead of dl-CarrierFreq (without suffix). In such case, if the UE does not support the GSCN value corresponding to the dl-CarrierFreq-r18, it ignores the corresponding neighbour cell.

eRedCapAccessAllowed Indicates whether eRedCap UEs are allowed to access cells on the frequency.

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. For a neighbouring carrier frequency when frequencyBandList-r18 is included, the network sets the corresponding value of freqBandIndicatorNR in frequencyBandList (without suffix) to 200, and the UE applies frequencyBandList-r18 instead of frequencyBandList (without suffix).

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.5.

highSpeedMeasInterFreq If the field is set to true and UE supports high speed inter-frequency IDLE/INACTIVE measurements, the UE shall apply the enhanced inter-frequency RRM requirements on the inter-frequency carrier to support high speed up to 500 km/h in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [14].

interFreqAllowedCellList List of allow-listed inter-frequency neighbouring cells, see TS 38.304 [20], clause 5.2.4.

interFreqCAG-CellList List of inter-frequency neighbouring CAG cells (as defined in TS 38.304 [20] per PLMN.

interFreqCarrierFreqList List of neighbouring carrier frequencies and frequency specific cell re-selection information. If interFreqCarrierFreqList-v1610, interFreqCarrierFreqList-v1700, interFreqCarrierFreqList-v1720, interFreqCarrierFreqList-v1730, interFreqCarrierFreqList-v1760, interFreqCarrierFreqList-v1800 or interFreqCarrierFreqList-v1900 are present, they shall contain the same number of entries, listed in the same order as in interFreqCarrierFreqList (without suffix).

interFreqExcludedCellList List of exclude-listed inter-frequency neighbouring cells.

interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters. If interFreqNeighCellList-v1610 is present, it shall contain the same number of entries, listed in the same order as in interFreqNeighCellList (without suffix).

interFreqNeighHSDN-CellList List of inter-frequency neighbouring HSDN cells as specified in TS 38.304 [20].

interFreqOD-SIB1-ExcludedCellList List of exclude-listed inter-frequency neighbouring cells for a UE supporting OD-SIB1 as specified in TS 38.304 [20]. If interFreqOD-SIB1-ExcludedCellList is present, the UE ignores interFreqExcludedCellList (without suffix).

mobileIAB-CellList Contains a PCI range on which mobile IAB cells may be deployed.

mobileIAB-Freq If present, it indicates that a mobile IAB node may be deployed on the inter-frequency carrier.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

od-sib1-cellReselectionPriority, od-sib1-cellReselectionSubPriority Cell reselection priorities to be considered by UEs supporting OD-SIB1 instead of cellReselectionPriority, cellReselectionSubPriority.

plmn-IdentityIndex Index of the PLMN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

p-Max Value in dBm applicable for the neighbouring NR cells on this carrier frequency. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

q-OffsetCell Parameter "Qoffsets,n" in TS 38.304 [20].

q-OffsetFreq Parameter "Qoffsetfrequency" in TS 38.304 [20].

q-QualMin Parameter "Qqualmin" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 38.304 [20]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 38.304 [20]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

q-RxLevMinOffsetCellSUL Parameter "QrxlevminoffsetcellSUL" in TS 38.304 [20]. Actual value QrxlevminoffsetcellSUL = field value * 2 [dB].

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20].

redCapAccessAllowed Indicates whether RedCap UEs are allowed to access cells on the frequency.

smtc Measurement timing configuration for inter-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms in this frequency. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc2-LP Measurement timing configuration for inter-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in smtc in InterFreqCarrierFreqInfo. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in smtc in InterFreqCarrierFreqInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the inter-frequency neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no inter-frequency neighbour cells with a Long Periodicity. This field is not configured together with smtc4list.

smtc4list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific neighbor cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the common value signalled by ssb-PositionQCL-Common in SIB4 for the indicated cell.

ssb-PositionQCL-Common Indicates the QCL relation between SS/PBCH blocks for inter-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. The UE behaviour is specified in TS 38.304 [20]. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

threshX-HighP Parameter "ThreshX, HighP" in TS 38.304 [20].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 38.304 [20].

threshX-LowP Parameter "ThreshX, LowP" in TS 38.304 [20].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 38.304 [20].

tn-AreaIdList List of TN area identifiers. The associated coverage information is provided in SIB25.

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

uav-PrioritizedFrequency This field indicates this is a frequency prioritized by Aerial UE for cell reselection as specified in TS 38.304 [20].

uav-PrioritizedFrequencyAltitudeRange Indicates the altitude range where the configuration of uav-PrioritizedFrequency is valid. If absent, the uav-PrioritizedFrequency flag (if included), applies for all altitudes. For this altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For this altitude range, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

| Conditional Presence | Explanation |
| --- | --- |
| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB and the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.101-5 [75], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need R. |
| Mandatory | The field is mandatory present in SIB4. |
| RSRQ | The field is mandatory present if threshServingLowQ is present in SIB2; otherwise it is absent. |
| SharedSpectrum | This field is mandatory present if this inter-frequency operates with shared spectrum channel access. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | The field is optional present, Need R, if this inter-frequency or neighbor cell operates with shared spectrum channel access. Otherwise, it is absent, Need R. |

#### –	SIB5

SIB5 contains information relevant only for inter-RAT cell re-selection i.e. information about E-UTRA frequencies and E-UTRAs neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

SIB5 information element

-- ASN1START

-- TAG-SIB5-START

SIB5 ::=                            SEQUENCE {

carrierFreqListEUTRA                CarrierFreqListEUTRA                        OPTIONAL,       -- Need R

t-ReselectionEUTRA                  T-Reselection,

t-ReselectionEUTRA-SF               SpeedStateScaleFactors                      OPTIONAL,       -- Need S

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

carrierFreqListEUTRA-v1610      CarrierFreqListEUTRA-v1610                      OPTIONAL        -- Need R

]],

[[

carrierFreqListEUTRA-v1700      CarrierFreqListEUTRA-v1700                      OPTIONAL,       -- Need R

idleModeMeasVoiceFallback-r17   ENUMERATED{true}                                OPTIONAL        -- Need R

]],

[[

carrierFreqListEUTRA-v1800      CarrierFreqListEUTRA-v1800                      OPTIONAL        -- Need R

]]

}

CarrierFreqListEUTRA ::=            SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA

CarrierFreqListEUTRA-v1610 ::=      SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA-v1610

CarrierFreqListEUTRA-v1700 ::=      SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA-v1700

CarrierFreqListEUTRA-v1800 ::=      SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF CarrierFreqEUTRA-v1800

CarrierFreqEUTRA ::=                SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

eutra-multiBandInfoList             EUTRA-MultiBandInfoList                     OPTIONAL,       -- Need R

eutra-FreqNeighCellList             EUTRA-FreqNeighCellList                     OPTIONAL,       -- Need R

eutra-ExcludedCellList              EUTRA-FreqExcludedCellList                  OPTIONAL,       -- Need R

allowedMeasBandwidth                EUTRA-AllowedMeasBandwidth,

presenceAntennaPort1                EUTRA-PresenceAntennaPort1,

cellReselectionPriority             CellReselectionPriority                     OPTIONAL,       -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                  OPTIONAL,       -- Need R

threshX-High                        ReselectionThreshold,

threshX-Low                         ReselectionThreshold,

q-RxLevMin                          INTEGER (-70..-22),

q-QualMin                           INTEGER (-34..-3),

p-MaxEUTRA                          INTEGER (-30..33),

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                               OPTIONAL        -- Cond RSRQ

}

CarrierFreqEUTRA-v1610 ::= SEQUENCE {

highSpeedEUTRACarrier-r16       ENUMERATED {true}                               OPTIONAL        -- Need R

}

CarrierFreqEUTRA-v1700 ::=          SEQUENCE {

eutra-FreqNeighHSDN-CellList-r17    EUTRA-FreqNeighHSDN-CellList-r17            OPTIONAL        -- Need R

}

CarrierFreqEUTRA-v1800 ::=          SEQUENCE {

eutra-MultiBandInfoListAerial-r18   EUTRA-MultiBandInfoListAerial-r18           OPTIONAL,       -- Need R

tn-AreaIdList-r18                   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF TN-AreaId-r18  OPTIONAL   -- Need R

}

EUTRA-FreqNeighHSDN-CellList-r17 ::= SEQUENCE (SIZE (1..maxCellEUTRA)) OF EUTRA-PhysCellIdRange

EUTRA-FreqExcludedCellList ::=      SEQUENCE (SIZE (1..maxEUTRA-CellExcluded)) OF EUTRA-PhysCellIdRange

EUTRA-FreqNeighCellList ::=         SEQUENCE (SIZE (1..maxCellEUTRA)) OF EUTRA-FreqNeighCellInfo

EUTRA-FreqNeighCellInfo ::=         SEQUENCE {

physCellId                          EUTRA-PhysCellId,

dummy                               EUTRA-Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                              OPTIONAL,       -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                              OPTIONAL        -- Need R

}

-- TAG-SIB5-STOP

-- ASN1STOP

SIB5 field descriptions

carrierFreqListEUTRA List of carrier frequencies of E-UTRA. If the carrierFreqListEUTRA-v1610/ carrierFreqListEUTRA-v1700/ carrierFreqListEUTRA-v1800 is present, it shall contain the same number of entries, listed in the same order as in the carrierFreqListEUTRA (without suffix).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

eutra-ExcludedCellList List of exclude-listed E-UTRA neighbouring cells.

eutra-FreqNeighHSDN-CellList List of neighbouring EUTRA HSDN cells as specified in TS 36.304 [27].

eutra-multiBandInfoList Indicates the list of frequency bands in addition to the band represented by carrierFreq for which cell reselection parameters are common, and a list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [22], table 6.2.4-1, for the frequency bands in eutra-multiBandInfoList

highSpeedEUTRACarrier If the field is present, the UE shall apply the enhanced NR-EUTRA inter-RAT measurement requirements to support high speed up to 500 km/h as specified in TS 38.133 [14] to the E-UTRA carrier.

idleModeMeasVoiceFallback Indicates whether E-UTRA idle/inactive measurements and reporting for EPS fallback can be used.

p-MaxEUTRA The maximum allowed transmission power in dBm on the (uplink) carrier frequency, see TS 36.304 [27].

q-QualMin Parameter "Qqualmin" in TS 36.304 [27]. Actual value Qqualmin = field value [dB].

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 36.304 [27]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 36.304 [27]. Actual value Qrxlevmin = field value * 2 [dBm].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 36.304 [27]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

t-ReselectionEUTRA Parameter "TreselectionEUTRA" in TS 38.304 [20].

threshX-High Parameter "ThreshX, HighP" in TS 38.304 [20].

threshX-HighQ Parameter "ThreshX, HighQ" in TS 38.304 [20].

threshX-Low Parameter "ThreshX, LowP" in TS 38.304 [20].

threshX-LowQ Parameter "ThreshX, LowQ" in TS 38.304 [20].

tn-AreaIdList List of TN area identifiers. The associated coverage information is provided in SIB25.

t-ReselectionEUTRA-SF Parameter "Speed dependent ScalingFactor for TreselectionEUTRA" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

| Conditional Presence | Explanation |
| --- | --- |
| RSRQ | The field is mandatory present if the threshServingLowQ is present in SIB2; otherwise it is absent. |

#### –	SIB6

SIB6 contains an ETWS primary notification.

SIB6 information element

-- ASN1START

-- TAG-SIB6-START

SIB6 ::=                            SEQUENCE {

messageIdentifier                   BIT STRING (SIZE (16)),

serialNumber                        BIT STRING (SIZE (16)),

warningType                         OCTET STRING (SIZE (2)),

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

warningAreaCoordinates-r19          OCTET STRING                                OPTIONAL    -- Need R

]]

}

-- TAG-SIB6-STOP

-- ASN1STOP

SIB6 field descriptions

messageIdentifier Identifies the source and type of ETWS notification.

serialNumber Identifies variations of an ETWS notification.

warningAreaCoordinates Indicates the geographical area where the ETWS warning message is valid as defined in TS 23.041 [29].

warningType Identifies the warning type of the ETWS primary notification and provides information on emergency user alert and UE popup.

#### –	SIB7

SIB7 contains an ETWS secondary notification.

SIB7 information element

-- ASN1START

-- TAG-SIB7-START

SIB7 ::=                            SEQUENCE {

messageIdentifier                   BIT STRING (SIZE (16)),

serialNumber                        BIT STRING (SIZE (16)),

warningMessageSegmentType           ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber         INTEGER (0..63),

warningMessageSegment               OCTET STRING,

dataCodingScheme                    OCTET STRING (SIZE (1))                     OPTIONAL,   -- Cond Segment1

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

warningAreaCoordinatesSegment-r19   OCTET STRING                                OPTIONAL    -- Need R

]]

}

-- TAG-SIB7-STOP

-- ASN1STOP

SIB7 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of an ETWS notification.

messageIdentifier Identifies the source and type of ETWS notification.

serialNumber Identifies variations of an ETWS notification.

warningAreaCoordinatesSegment Carries a segment, with one or more octets, of the geographical area where the ETWS warning message is valid as defined in TS 23.041 [29]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [29] and so on.

warningMessageSegment Carries a segment of the Warning Message Contents IE.

warningMessageSegmentNumber Segment number of the ETWS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, A segment number of one corresponds to the second segment, and so on.

warningMessageSegmentType Indicates whether the included ETWS warning message segment is the last segment or not.

| Conditional Presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB7, otherwise it is absent. |

#### –	SIB8

SIB8 contains a CMAS notification.

SIB8 information element

-- ASN1START

-- TAG-SIB8-START

SIB8 ::=                        SEQUENCE {

messageIdentifier               BIT STRING (SIZE (16)),

serialNumber                    BIT STRING (SIZE (16)),

warningMessageSegmentType       ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber     INTEGER (0..63),

warningMessageSegment           OCTET STRING,

dataCodingScheme                OCTET STRING (SIZE (1))                         OPTIONAL,   -- Cond Segment1

warningAreaCoordinatesSegment   OCTET STRING                                    OPTIONAL,   -- Need R

lateNonCriticalExtension        OCTET STRING                                    OPTIONAL,

...

}

-- TAG-SIB8-STOP

-- ASN1STOP

SIB8 field descriptions

dataCodingScheme Identifies the alphabet/coding and the language applied variations of a CMAS notification.

messageIdentifier Identifies the source and type of CMAS notification.

serialNumber Identifies variations of a CMAS notification.

warningAreaCoordinatesSegment Carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [28]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [29] and so on.

warningMessageSegment Carries a segment, with one or more octets, of the Warning Message Contents IE defined in TS 38.413 [42]. The first octet of the Warning Message Contents IE is equivalent to the first octet of the CB data IE defined in and encoded according to TS 23.041 [29], clause 9.4.2.2.5, and so on.

warningMessageSegmentNumber Segment number of the CMAS warning message segment contained in the SIB. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

warningMessageSegmentType Indicates whether the included CMAS warning message segment is the last segment or not. If warning area coordinates are provided for the warning message, then this field applies to both warning message segment and warning area coordinates segment.

| Conditional Presence | Explanation |
| --- | --- |
| Segment1 | The field is mandatory present in the first segment of SIB8, otherwise it is absent. |

#### –	SIB9

SIB9 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

NOTE:	The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock.

SIB9 information element

-- ASN1START

-- TAG-SIB9-START

SIB9 ::=                            SEQUENCE {

timeInfo                            SEQUENCE {

timeInfoUTC                         INTEGER (0..549755813887),

dayLightSavingTime                  BIT STRING (SIZE (2))                   OPTIONAL,   -- Need R

leapSeconds                         INTEGER (-127..128)                     OPTIONAL,   -- Need R

localTimeOffset                     INTEGER (-63..64)                       OPTIONAL    -- Need R

}                                                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

referenceTimeInfo-r16               ReferenceTimeInfo-r16                       OPTIONAL    -- Need R

]],

[[

eventID-TSS-r18                     INTEGER (0..63)                             OPTIONAL    -- Need R

]]

}

-- TAG-SIB9-STOP

-- ASN1STOP

SIB9 field descriptions

dayLightSavingTime Indicates if and how daylight-saving time (DST) is applied to obtain the local time. The semantics are the same as the semantics of the Daylight Saving Time IE in TS 24.501 [23] and TS 24.008 [38]. The first/leftmost bit of the bit string contains the b2 of octet 3 and the second bit of the bit string contains b1 of octet 3 in the value part of the Daylight Saving Time IE in TS 24.008 [38].

eventID-TSS This field indicates the status of the 5G access stratum time distribution parameter Clock Quality Reporting Control Information as defined in TS 23.501 [32].

leapSeconds Number of leap seconds offset between GPS Time and UTC. UTC and GPS time are related i.e. GPS time -leapSeconds = UTC time.

localTimeOffset Offset between UTC and local time in units of 15 minutes. Actual value = field value * 15 minutes. Local time of the day is calculated as UTC time + localTimeOffset.

timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. In an NTN cell, the indicated time is referenced at the uplink time synchronization reference point (RP), i.e., UE should take into account the propagation delay between UE and RP when determining the UTC time at the UE. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). See NOTE 1. This field is excluded when determining changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of valueTag in SIB1.

NOTE 1:	The UE may use this field together with the leapSeconds field to obtain GPS time as follows: GPS Time (in seconds) = timeInfoUTC (in seconds) -  2,524,953,600 (seconds) + leapSeconds, where 2,524,953,600 is the number of seconds between 00:00:00 on Gregorian calendar date 1 January, 1900 and 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time).

#### –	SIB10

SIB10 contains the HRNNs of the NPNs listed in SIB1.

SIB10 information element

-- ASN1START

-- TAG-SIB10-START

SIB10-r16 ::=               SEQUENCE {

hrnn-List-r16               HRNN-List-r16                                   OPTIONAL,   -- Need R

lateNonCriticalExtension    OCTET STRING                                    OPTIONAL,

...

}

HRNN-List-r16 ::=           SEQUENCE (SIZE (1..maxNPN-r16)) OF HRNN-r16

HRNN-r16 ::=                SEQUENCE {

hrnn-r16                    OCTET STRING (SIZE(1.. maxHRNN-Len-r16))        OPTIONAL   -- Need R

}

-- TAG-SIB10-STOP

-- ASN1STOP

SIB10 field descriptions

HRNN-List The same amount of HRNN (see TS 23.003 [21]) elements as the number of NPNs in SIB 1 are included. The n-th entry of HRNN-List contains the human readable network name of the n-th NPN of SIB1. The hrnn in the corresponding entry in HRNN-List is absent if there is no HRNN associated with the given NPN.

#### –	SIB11

SIB11 contains information related to idle/inactive measurements.

SIB11 information element

-- ASN1START

-- TAG-SIB11-START

SIB11-r16 ::=                    SEQUENCE {

measIdleConfigSIB-r16            MeasIdleConfigSIB-r16                       OPTIONAL, -- Need S

lateNonCriticalExtension         OCTET STRING                                OPTIONAL,

...

}

-- TAG-SIB11-STOP

-- ASN1STOP

SIB11 field descriptions

measIdleConfigSIB Indicates measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

#### –	SIB12

SIB12 contains NR sidelink communication/discovery configuration.

SIB12 information element

-- ASN1START

-- TAG-SIB12-START

SIB12-r16 ::=                 SEQUENCE {

segmentNumber-r16             INTEGER (0..63),

segmentType-r16               ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r16          OCTET STRING

}

SIB12-IEs-r16 ::=             SEQUENCE {

sl-ConfigCommonNR-r16         SL-ConfigCommonNR-r16,

lateNonCriticalExtension      OCTET STRING (CONTAINING SIB12-IEs-v16k0)                                     OPTIONAL,

...,

[[

sl-DRX-ConfigCommonGC-BC-r17         SL-DRX-ConfigGC-BC-r17                                                 OPTIONAL,    -- Need R

sl-DiscConfigCommon-r17              SL-DiscConfigCommon-r17                                                OPTIONAL,    -- Need R

sl-L2U2N-Relay-r17                   ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-NonRelayDiscovery-r17             ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-L3U2N-RelayDiscovery-r17          ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-TimersAndConstantsRemoteUE-r17    UE-TimersAndConstantsRemoteUE-r17                                      OPTIONAL     -- Need R

]],

[[

sl-FreqInfoListSizeExt-v1800         SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-FreqConfigCommon-r16    OPTIONAL,    -- Need R

sl-RLC-BearerConfigListSizeExt-v1800 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16         OPTIONAL,    -- Need R

sl-SyncFreqList-r18                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-Freq-Id-r16               OPTIONAL,    -- Need R

sl-SyncTxMultiFreq-r18               ENUMERATED {true}                                                      OPTIONAL,    -- Need S

sl-MaxTransPowerCA-r18               P-Max                                                                  OPTIONAL,    -- Need R

sl-DiscConfigCommon-v1800            SL-DiscConfigCommon-v1800                                              OPTIONAL,    -- Need R

sl-L2-U2U-Relay-r18                  ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-L3-U2U-RelayDiscovery-r18         ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

t400-U2U-r18                         ENUMERATED {ms200, ms400, ms600, ms800, ms1200, ms2000, ms3000, ms4000} OPTIONAL    -- Need R

]],

[[

sl-DiscConfigCommon-v1840            SL-DiscConfigCommon-v1840                                              OPTIONAL     -- Need R

]],

[[

sl-L2U2N-MH-Relay-r19                ENUMERATED {enabled}                                                 OPTIONAL, -- Cond SH-Relay

sl-DiscConfigCommon-v1900            SL-DiscConfigCommon-v1900                                              OPTIONAL      -- Need R

]]

}

-- Late non-critical Rel-16 extensions:

SIB12-IEs-v16k0 ::=            SEQUENCE {

sl-ConfigCommonNR-v16k0             SL-ConfigCommonNR-v16k0                                                 OPTIONAL, -- Need R

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

SL-ConfigCommonNR-r16 ::=        SEQUENCE {

sl-FreqInfoList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16      OPTIONAL,    -- Need R

sl-UE-SelectedConfig-r16             SL-UE-SelectedConfig-r16                                               OPTIONAL,    -- Need R

sl-NR-AnchorCarrierFreqList-r16      SL-NR-AnchorCarrierFreqList-r16                                        OPTIONAL,    -- Need R

sl-EUTRA-AnchorCarrierFreqList-r16   SL-EUTRA-AnchorCarrierFreqList-r16                                     OPTIONAL,    -- Need R

sl-RadioBearerConfigList-r16         SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16       OPTIONAL,    -- Need R

sl-RLC-BearerConfigList-r16          SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16         OPTIONAL,    -- Need R

sl-MeasConfigCommon-r16              SL-MeasConfigCommon-r16                                                OPTIONAL,    -- Need R

sl-CSI-Acquisition-r16               ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-OffsetDFN-r16                     INTEGER (1..1000)                                                      OPTIONAL,    -- Need R

t400-r16                             ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL,    -- Need R

sl-MaxNumConsecutiveDTX-r16          ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32}                          OPTIONAL,    -- Need R

sl-SSB-PriorityNR-r16                INTEGER (1..8)                                                         OPTIONAL     -- Need R

}

SL-ConfigCommonNR-v16k0 ::=          SEQUENCE {

sl-FreqInfoListExt-v16k0             SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommonExt-v16k0 OPTIONAL    -- Need R

}

SL-NR-AnchorCarrierFreqList-r16 ::=  SEQUENCE (SIZE (1..maxFreqSL-NR-r16)) OF ARFCN-ValueNR

SL-EUTRA-AnchorCarrierFreqList-r16 ::= SEQUENCE (SIZE (1..maxFreqSL-EUTRA-r16)) OF ARFCN-ValueEUTRA

SL-DiscConfigCommon-r17 ::=   SEQUENCE {

sl-RelayUE-ConfigCommon-r17   SL-RelayUE-Config-r17,

sl-RemoteUE-ConfigCommon-r17  SL-RemoteUE-Config-r17

}

SL-DiscConfigCommon-v1800 ::=    SEQUENCE {

sl-RelayUE-ConfigCommonU2U-r18   SL-RelayUE-ConfigU2U-r18,

sl-RemoteUE-ConfigCommonU2U-r18  SL-RemoteUE-ConfigU2U-r18

}

SL-DiscConfigCommon-v1840 ::=     SEQUENCE {

sl-RelayUE-ConfigCommonU2U-v1840  SL-RelayUE-ConfigU2U-v1840,

sl-RemoteUE-ConfigCommonU2U-v1840 SL-RemoteUE-ConfigU2U-v1830

}

SL-DiscConfigCommon-v1900 ::=    SEQUENCE {

sl-RelayUE-ConfigCommonMH-r19    SL-RelayUE-ConfigMH-r19

}

-- TAG-SIB12-STOP

-- ASN1STOP

SIB12 field descriptions

segmentContainer This field includes a segment of the encoded SIB12-IEs. The size of the included segment in this container should be small enough that the SIB message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when SIB12 is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB12-IEs. A segment number of zero corresponds to the first segment, A segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

sl-CSI-Acquisition This field indicates whether CSI reporting is enabled in sidelink unicast. If not set, SL CSI reporting is disabled.

sl-DRX-ConfigCommonGC-BC This field indicates the sidelink DRX configuration for groupcast and broadcast communication, as specified in TS 38.321 [3]. This field, if present, also indicates the gNB is capable of sidelink DRX.

sl-EUTRA-AnchorCarrierFreqList This field indicates the EUTRA anchor carrier frequency list, which can provide the NR sidelink communication configurations.

sl-FreqInfoList, sl-FreqInfoListSizeExt, sl-FreqInfoListExt This field indicates the NR sidelink communication/discovery configuration on some carrier frequency (ies). In this release, only one entry can be configured in sl-FreqInfoList. More entries can be configured in sl-FreqInfoListSizeExt. If network includes sl-FreqInfoListExt, it includes the same number of entries, and listed in the same order, as in sl-FreqInfoList together with sl-FreqInfoListSizeExt. The first entry corresponds to the AdditionalSpectrumEmission of the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the second entry corresponds to the AdditionalSpectrumEmission of the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the third entry corresponds to the AdditionalSpectrumEmission of the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

sl-L2U2N-Relay This field indicates the support of NR sidelink Layer-2 single hop U2N relay operation.

sl-L2U2N-MH-Relay This field indicates the support of NR sidelink Layer-2 multi hop U2N relay operation.

sl-L2-U2U-Relay This field indicates the support of NR sidelink Layer-2 U2U relay operation.

sl-L3U2N-RelayDiscovery This field indicates the support of L3 U2N relay AS-layer capability, i.e. NR sidelink L3 U2N relay discovery.

sl-L3-U2U-RelayDiscovery This field indicates the support of L3 U2U relay AS-layer capability, i.e. NR sidelink L3 U2U relay discovery.

sl-MaxNumConsecutiveDTX This field indicates the maximum number of consecutive HARQ DTX before triggering sidelink RLF. Value n1 corresponds to 1, value n2 corresponds to 2, and so on.

sl-MaxTransPowerCA The maximum total transmit power to be used by the UE across all sidelink carriers.

sl-MeasConfigCommon This field indicates the measurement configurations (e.g. RSRP) for NR sidelink communication.

sl-NonRelayDiscovery This field indicates the support of NR sidelink non-relay discovery.

sl-NR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink communication/discovery configurations.

sl-OffsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

sl-RadioBearerConfigList This field indicates one or multiple sidelink radio bearer configurations.

sl-RLC-BearerConfigList, sl-RLC-BearerConfigListSizeExt This field indicates one or multiple sidelink RLC bearer configurations. For L2 U2U operation, sl-RLC-BearerConfigList also indicates the PC5 Relay RLC Channel configurations.

sl-SSB-PriorityNR This field indicates the priority of NR sidelink SSB transmission and reception.

sl-SyncFreqList Indicates a list of candidate carrier frequencies that can be used for the synchronisation of NR sidelink communication. For SL-Freq-Id-r16, the value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

sl-SyncTxMultiFreq Indicates that the UE transmits S-SSB on multiple carrier frequencies for NR sidelink communication. If this field is absent, the UE transmits S-SSB only on the synchronisation carrier frequency.

t400 Indicates the value for timer T400 as described in clause 7.1. Value ms100 corresponds to 100 ms, value ms200 corresponds to 200 ms and so on.

t400-U2U Indicates the value for timer T400 to be applied for end-to-end PC5 connection in sidelink U2U relay operation as described in clause 7.1. Value ms200 corresponds to 200 ms, value ms400 corresponds to 400 ms and so on.

| Conditional Presence | Explanation |
| --- | --- |
| SH-Relay | The field is optionally present, Need R, if sl-L2U2N-Relay is configured; otherwise it is absent, Need R. |

#### –	SIB13

SIB13 contains configurations of V2X sidelink communication defined in TS 36.331 [10].

SIB13 information element

-- ASN1START

-- TAG-SIB13-START

SIB13-r16 ::=                       SEQUENCE {

sl-V2X-ConfigCommon-r16             OCTET STRING,

dummy                               OCTET STRING,

tdd-Config-r16                      OCTET STRING,

lateNonCriticalExtension            OCTET STRING                          OPTIONAL,

...

}

-- TAG-SIB13-STOP

-- ASN1STOP

SIB13 field descriptions

dummy This field is not used in the specification and the UE ignores the received value.

sl-V2X-ConfigCommon This field includes the E-UTRA SystemInformationBlockType21 message as specified in TS 36.331 [10].

tdd-Config This field includes the tdd-Config in E-UTRA SystemInformationBlockType1 message as specified in TS 36.331 [10].

#### –	SIB14

SIB14 contains configurations of V2X sidelink communication defined in TS 36.331 [10], which can be used jointly with that included in SIB13.

SIB14 information element

-- ASN1START

-- TAG-SIB14-START

SIB14-r16 ::=                      SEQUENCE {

sl-V2X-ConfigCommonExt-r16         OCTET STRING,

lateNonCriticalExtension           OCTET STRING                          OPTIONAL,

...

}

-- TAG-SIB14-STOP

-- ASN1STOP

SIB14 field descriptions

sl-V2X-ConfigCommonExt This field includes the E-UTRA SystemInformationBlockType26 message as specified in TS 36.331 [10].

#### –	SIB15

SIB15 contains configurations of disaster roaming information.

SIB15 information element

-- ASN1START

-- TAG-SIB15-START

SIB15-r17 ::=                          SEQUENCE {

commonPLMNsWithDisasterCondition-r17   SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity                   OPTIONAL,  -- Need R

applicableDisasterInfoList-r17         SEQUENCE (SIZE (1..maxPLMN)) OF ApplicableDisasterInfo-r17      OPTIONAL,  -- Need R

lateNonCriticalExtension               OCTET STRING                                                    OPTIONAL,

...

}

ApplicableDisasterInfo-r17    ::= CHOICE {

noDisasterRoaming-r17             NULL,

disasterRelatedIndication-r17     NULL,

commonPLMNs-r17                   NULL,

dedicatedPLMNs-r17                SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity

}

-- TAG-SIB15-STOP

-- ASN1STOP

SIB15 field descriptions

commonPLMNsWithDisasterCondition A list of PLMN(s) for which disaster condition applies and that disaster inbound roaming is accepted, which can be commonly applicable to the PLMNs sharing the cell.

applicableDisasterInfoList A list indicating the applicable disaster roaming information for the networks indicated in plmn-IdentityInfoList and npn-IdentityInfoList-r16. The network indicates in this list one entry for each entry of plmn-IdentityInfoList, followed by one entry for each entry of npn-IdentityInfoList-r16, meaning that this list will have as many entries as the number of entries of the combination of plmn-IdentityInfoList and npn-IdentityInfoList-r16. The first entry in this list indicates the disaster roaming information applicable for the network(s) in the first entry of plmn-IdentityInfoList/npn-IdentityInfoList-r16, the second entry in this list indicates the disaster roaming information applicable for the network(s) in the second entry of plmn-IdentityInfoList/npn-IdentityInfoList-r16, and so on. Each entry in this list can either be having the value noDisasterRoaming, disasterRelatedIndication, commonPLMNs, or dedicatedPLMNs. If an entry in this list takes the value noDisasterRoaming, disaster inbound roaming is not allowed in this network(s). If an entry in this list takes the value disasterRelatedIndication, the meaning of this field for this network(s) is as specified for "disaster related indication" in TS 23.122 [74], clause 4.4.3.1.1. If an entry in this list takes the value commonPLMNs, the PLMN(s) with disaster conditions indicated in the field commonPLMNsWithDisasterCondition apply for this network(s). If an entry in this list contains the value dedicatedPLMNs, the listed PLMN(s) are the PLMN(s) with disaster conditions that the network(s) corresponding to this entry accepts disaster inbound roamers from. For SNPNs, the network indicates the value noDisasterRoaming.

#### –	SIB16

SIB16 contains configurations of slice-based cell reselection information.

SIB16 information element

-- ASN1START

-- TAG-SIB16-START

SIB16-r17 ::=                    SEQUENCE {

freqPriorityListSlicing-r17      FreqPriorityListSlicing-r17              OPTIONAL,  -- Need R

lateNonCriticalExtension         OCTET STRING                             OPTIONAL,

...

}

-- TAG-SIB16-STOP

-- ASN1STOP

SIB16 field descriptions

freqPriorityListSlicing This field indicates cell reselection priorities for slicing.

#### –	SIB17

SIB17 contains configurations of TRS resources for idle/inactive UEs.

SIB17 information element

-- ASN1START

-- TAG-SIB17-START

SIB17-r17 ::=               SEQUENCE {

segmentNumber-r17           INTEGER (0..63),

segmentType-r17             ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r17        OCTET STRING

}

SIB17-IEs-r17 ::=           SEQUENCE {

trs-ResourceSetConfig-r17   SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r17,

validityDuration-r17        ENUMERATED {t1, t2, t4, t8, t16, t32, t64, t128, t256, t512, infinity, spare5, spare4, spare3, spare2,

spare1}                                                            OPTIONAL,  -- Need S

lateNonCriticalExtension    OCTET STRING                                                                   OPTIONAL,

...

}

TRS-ResourceSet-r17 ::=                SEQUENCE {

powerControlOffsetSS-r17               ENUMERATED {db-3, db0, db3, db6},

scramblingID-Info-r17                  CHOICE {

scramblingIDforCommon-r17              ScramblingId,

scramblingIDperResourceListWith2-r17   SEQUENCE (SIZE (2)) OF ScramblingId,

scramblingIDperResourceListWith4-r17   SEQUENCE (SIZE (4)) OF ScramblingId,

...

},

firstOFDMSymbolInTimeDomain-r17            INTEGER (0..9),

startingRB-r17                             INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs-r17                                INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

ssb-Index-r17                              SSB-Index,

periodicityAndOffset-r17                   CHOICE {

slots10                                    INTEGER (0..9),

slots20                                    INTEGER (0..19),

slots40                                    INTEGER (0..39),

slots80                                    INTEGER (0..79)

},

frequencyDomainAllocation-r17              BIT STRING (SIZE (4)),

indBitID-r17                               INTEGER (0..5),

nrofResources-r17                          ENUMERATED {n2, n4}

}

-- TAG-SIB17-STOP

-- ASN1STOP

SIB17 field descriptions

segmentContainer This field includes a segment of the encoded SIB17-IEs. The size of the included segment in this container should be small enough that the SIB message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when SIB17 is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB17-IEs. A segment number of zero corresponds to the first segment, a segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

trs-ResourceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB17 with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable. If SIB scheduling indicates that SIB17 has changed, the UE considers its configured TRS(s) as unavailable until it receives the associated L1-based availability indication(s).

validityDuration The valid time duration for L1 availability indication, time unit is one default paging cycle. When the field is absent, UE assumes a default time duration to be 2 default paging cycles. The field is only valid while the UE has a valid SIB17.

TRS-ResourceSet field descriptions

firstOFDMSymbolInTimeDomain The index of the first OFDM symbol in the PRB used for TRS in a slot. The field indicates the first symbol in a slot for the first TRS resource within the slot, and the symbol for the second TRS resource in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4.

frequencyDomainAllocation Indicates the offset of the first RE to RE#0 in a RB in row1 in table 7.4.1.5.3-1 for frequency domain allocation within a physical resource block (TS 38.211 [16], clause 7.4.1.5.3).

indBitID The index of the associated bit in TRS availability indication field in DCI. Each TRS resource set is configured with an ID i for the association with (i+1)-th indication bit in TRS availability indication field in DCI.

nrofRBs Number of PRBs across which corresponding TRS resource spans.

nrofResources The number of TRS resources for a TRS resource set.

periodicityAndOffset The periodicity and slot offset (slot) for periodic TRS. It is used to determine the location of the first slot of TRS resource set. The periodicity value slots10 corresponds to 10 slots, value slots20 corresponds to 20 slots, and so on. Only the following values of the periodicity are used: 10 slots (SCS 15 kHz), 20 slots (SCS 15 and 30 kHz) and 40 slots (SCS 15, 30 and 60 kHz) and 80 slots (SCS 15, 30, 60, 120 kHz).

powerControlOffsetSS Power offset (dB) of NZP CSI-RS RE to SSS RE.

scramblingID-Info One or more scrambling IDs are configured for a TRS resource set. If a common scrambling ID is configured, it applies to all the TRS resources within the TRS resource set. Otherwise, each TRS resource within the TRS resource set is provided with a scrambling ID. If the number of TRS resources for the TRS resource set is 2, scramblingIDperResourceListWith2-r17 is configured, while scramblingIDperResourceListWith4-r17 is configured for the case that the number of TRS resources for the TRS resource set is 4.

ssb-Index The index of reference SSB with which quasi-collocation information is provided as specified in TS 38.214 [19] clause 5.1.5.

startingRB The PRB index where corresponding TRS resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

#### –	SIB17bis

SIB17bis contains configurations of TRS resources for idle/inactive UEs. SIB17bis is optionally scheduled if SIB17 is not scheduled.

SIB17bis information element

-- ASN1START

-- TAG-SIB17bis-START

SIB17bis-r18 ::=               SEQUENCE {

segmentNumber-r18           INTEGER (0..63),

segmentType-r18             ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r18        OCTET STRING

}

SIB17bis-IEs-r18 ::=           SEQUENCE {

trs-ResourceSetConfig-r18   SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r18        OPTIONAL,  -- Need R

validityDuration-r18        ENUMERATED {t1, t2, t4, t8, t16, t32, t64, t128, t256, t512, infinity, spare5, spare4, spare3, spare2,

spare1}                                                            OPTIONAL,  -- Need S

lateNonCriticalExtension    OCTET STRING                                                                   OPTIONAL,

...

}

TRS-ResourceSet-r18 ::=                SEQUENCE {

powerControlOffsetSS-r18               ENUMERATED {db-3, db0, db3, db6},

scramblingID-Info-r18                  CHOICE {

scramblingIDforCommon-r18              ScramblingId,

scramblingIDperResourceListWith2-r18   SEQUENCE (SIZE (2)) OF ScramblingId,

scramblingIDperResourceListWith4-r18   SEQUENCE (SIZE (4)) OF ScramblingId,

...

},

firstOFDMSymbolInTimeDomain-r18            INTEGER (0..9),

startingRB-r18                             INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs-r18                                INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

ssb-Index-r18                              SSB-Index,

periodicityAndOffset-r18                   CHOICE {

slots10                                    INTEGER (0..9),

slots20                                    INTEGER (0..19),

slots40                                    INTEGER (0..39),

slots80                                    INTEGER (0..79),

slots160                                   INTEGER (0..159),

slots320                                   INTEGER (0..319),

slots640                                   INTEGER (0..639)

},

frequencyDomainAllocation-r18              BIT STRING (SIZE (4)),

indBitID-r18                               INTEGER (0..5),

nrofResources-r18                          ENUMERATED {n2, n4}

}

-- TAG-SIB17bis-STOP

-- ASN1STOP

SIB17bis field descriptions

segmentContainer This field includes a segment of the encoded SIB17bis-IEs. The size of the included segment in this container should be small enough that the SIB message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when SIB17bis is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB17bis-IEs. A segment number of zero corresponds to the first segment, a segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

trs-ResourceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB17bis with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable. If SIB scheduling indicates that SIB17bis has changed, the UE considers its configured TRS(s) from SIB17bis as unavailable until it receives the associated L1-based availability indication(s).

validityDuration The valid time duration for L1 availability indication, time unit is one default paging cycle. When the field is absent, UE assumes a default time duration to be 2 default paging cycles. The field is only valid while the UE has a valid SIB17bis.

TRS-ResourceSet field descriptions

firstOFDMSymbolInTimeDomain The index of the first OFDM symbol in the PRB used for TRS in a slot. The field indicates the first symbol in a slot for the first TRS resource within the slot, and the symbol for the second TRS resource in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4.

frequencyDomainAllocation Indicates the offset of the first RE to RE#0 in a RB in row1 in table 7.4.1.5.3-1 for frequency domain allocation within a physical resource block (TS 38.211 [16], clause 7.4.1.5.3).

indBitID The index of the associated bit in TRS availability indication field in DCI. Each TRS resource set is configured with an ID i for the association with (i+1)-th indication bit in TRS availability indication field in DCI.

nrofRBs Number of PRBs across which corresponding TRS resource spans.

nrofResources The number of TRS resources for a TRS resource set.

periodicityAndOffset The periodicity and slot offset (slot) for periodic TRS. It is used to determine the location of the first slot of TRS resource set. The periodicity value slots10 corresponds to 10 slots, value slots20 corresponds to 20 slots, and so on. Only the following values of the periodicity are used: 10 slots (SCS 15 kHz), 20 slots (SCS 15 and 30 kHz), 40 slots (SCS 15, 30 and 60 kHz), 80 slots (SCS 15, 30, 60 and 120 kHz), 160 slots (SCS 30, 60 and 120 kHz), 320 slots (SCS 60 and 120 kHz), and 640 slots (SCS 120 kHz).

powerControlOffsetSS Power offset (dB) of NZP CSI-RS RE to SSS RE.

scramblingID-Info One or more scrambling IDs are configured for a TRS resource set. If a common scrambling ID is configured, it applies to all the TRS resources within the TRS resource set. Otherwise, each TRS resource within the TRS resource set is provided with a scrambling ID. If the number of TRS resources for the TRS resource set is 2, scramblingIDperResourceListWith2-r18 is configured, while scramblingIDperResourceListWith4-r18 is configured for the case that the number of TRS resources for the TRS resource set is 4.

ssb-Index The index of reference SSB with which quasi-collocation information is provided as specified in TS 38.214 [19] clause 5.1.5.

startingRB The PRB index where corresponding TRS resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

#### –	SIB18

SIB18 contains Group IDs for Network selection (GINs) to support access using credentials from a Credentials Holder or to support UE onboarding.

SIB18 information element

-- ASN1START

-- TAG-SIB18-START

SIB18-r17 ::=               SEQUENCE {

gin-ElementList-r17         SEQUENCE (SIZE (1..maxGIN-r17)) OF GIN-Element-r17            OPTIONAL,   -- Need R

gins-PerSNPN-List-r17       SEQUENCE (SIZE (1..maxNPN-r16)) OF GINs-PerSNPN-r17           OPTIONAL,   -- Need S

lateNonCriticalExtension    OCTET STRING                                                  OPTIONAL,

...

}

GIN-Element-r17 ::=         SEQUENCE {

plmn-Identity-r17           PLMN-Identity,

nid-List-r17                SEQUENCE (SIZE (1..maxGIN-r17)) OF NID-r16

}

GINs-PerSNPN-r17 ::=        SEQUENCE {

supportedGINs-r17           BIT STRING (SIZE (1..maxGIN-r17))                             OPTIONAL   -- Need R

}

-- TAG-SIB18-STOP

-- ASN1STOP

SIB18 field descriptions

gin-ElementList The gin-ElementList contains one or more GIN elements. Each GIN element contains either one GIN, which is identified by a PLMN ID and a NID, or multiple GINs that share the same PLMN ID. The total number of GINs indicated does not exceed maxGIN-r17. The GIN index m is defined as d1+d2+…+d(n-1)+i for the GIN included in the n-th entry of the gin-ElementList and the i-th entry of its corresponding GIN-Element, where d(k) is the number of GIN index values used in the k-th gin-ElementList entry.

gins-PerSNPN-List Indicates the supported GINs for each SNPN. The network includes the same number of entries as the number of SNPNs in snpn-AccessInfoList in provided in SIB1, and the n-th entry in this list corresponds to the n-th SNPN listed in snpn-AccessInfoList provided in SIB1. The network configures this field only if the cell broadcasts more than one SNPN in SIB1. If this field is absent, as in case of a single SNPN broadcasted in SIB1, the UE shall associate all GINs in gin-ElementList to that SNPN.

GINs-PerSNPN field descriptions

supportedGINs Indicates the GINs which are supported by the given SNPN. The first/leftmost bit corresponds to the GIN with GIN index 1, the second bit corresponds to the GIN with GIN index 2 and so on. A bit set to 1 indicates that the GIN is supported by the SNPN. If the field is not present, then the corresponding SNPN does not support any GINs.

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntn-CovEnh-r18                           NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

NTN-CovEnh-r18 ::=                       SEQUENCE {

numberOfMsg4HARQ-ACK-Repetitions-r18     BIT STRING (SIZE(4)),

rsrp-ThresholdMsg4HARQ-ACK-r18           RSRP-Range                                      OPTIONAL        -- Need R

}

SatSwitchWithReSync-r18 ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m. This field is only present in an NTN cell.

movingReferenceLocation Reference location of the serving cell of an NTN Earth-moving cell at a time reference. It is used in the evaluation of eventD2 and condEventD2 criteria for the serving cell in RRC_CONNECTED, and location-based measurement initiation in RRC_IDLE and RRC_INACTIVE when distanceThresh is also configured, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. This field is excluded when determining changes in system information, i.e., changes to movingReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch time. In a TN cell, this field is only present in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. Network provides ntn-Config for the first entry of ntn-NeighCellConfigList. If the ntn-Config is absent for any other entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies.

referenceLocation Reference location of the serving cell provided via NTN (quasi-)Earth fixed cell and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. This field is only present in an NTN cell.

satSwitchWithReSync Provides parameters for the target satellite required to perform satellite switch with resynchronization. This field is only present in an NTN cell and its presence indicates that satellite switch without PCI change is supported in the cell.

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-Service is the uplink time synchronization reference point of the cell. This field is only present in an NTN cell.

NTN-CovEnh field descriptions

numberOfMsg4HARQ-ACK-Repetitions The number of repetition slots for PUCCH transmission with HARQ-ACK information for Msg4, see clause 9.2.6 in TS 38.213 [13]. The first/leftmost bit corresponds to the repetition factor 1, the second bit corresponds to the repetition factor 2, the third bit corresponds to the repetition factor 4, and the last/rightmost bit corresponds to the repetition factor 8. The repetition factor 1 shall be indicated together with at least one other repetition factor.

rsrp-ThresholdMsg4HARQ-ACK This threshold is used by the UE for determining the configuration of the MAC entity for PUCCH repetition for Msg4 HARQ-ACK, as specified in clause 6.2.1 in TS 38.321 [3].

SatSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. The starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point. The value is given in number of subframes and can only be configured as 5 subframes or multiples of 5 subframes. This field is included only for soft satellite switch with re-synchronization.

t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.

#### –	SIB20

SIB20 contains the information required to acquire the MCCH/MTCH configuration for MBS broadcast.

SIB20 information element

-- ASN1START

-- TAG-SIB20-START

SIB20-r17 ::=	SEQUENCE {

mcch-Config-r17                MCCH-Config-r17,

cfr-ConfigMCCH-MTCH-r17        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...,

[[

cfr-ConfigMCCH-MTCH-RedCap-r18 CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

mcch-ConfigRedCap-r18          MCCH-Config-r17         OPTIONAL   -- Need S

]]

}

MCCH-Config-r17 ::= SEQUENCE {

mcch-RepetitionPeriodAndOffset-r17   MCCH-RepetitionPeriodAndOffset-r17,

mcch-WindowStartSlot-r17             INTEGER (0..79),

mcch-WindowDuration-r17              ENUMERATED {sl2, sl4, sl8, sl10, sl20, sl40,sl80, sl160}     OPTIONAL, -- Need S

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, rf2048, rf4096, rf8192, rf16384, rf32768, rf65536}

}

MCCH-RepetitionPeriodAndOffset-r17 ::= CHOICE {

rf1-r17                                INTEGER(0),

rf2-r17                                INTEGER(0..1),

rf4-r17                                INTEGER(0..3),

rf8-r17                                INTEGER(0..7),

rf16-r17                               INTEGER(0..15),

rf32-r17                               INTEGER(0..31),

rf64-r17                               INTEGER(0..63),

rf128-r17                              INTEGER(0..127),

rf256-r17                              INTEGER(0..255)

}

-- TAG-SIB20-STOP

-- ASN1STOP

SIB20 field descriptions

cfr-ConfigMCCH-MTCH Common frequency resource used for MCCH and MTCH reception. If the field is absent, the CFR for broadcast has the same location and size as CORESET#0 and PDSCH configuration of MCCH is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1.

cfr-ConfigMCCH-MTCH-RedCap Common frequency resource used for MCCH and MTCH reception for (e)RedCap UEs. If the field is absent, the (e)RedCap UE can use cfr-ConfigMCCH-MTCH if the UE supports the configured bandwidth.

mcch-ConfigRedcap Indicates MBS broadcast MCCH configuration for (e)Redcap UEs. If the field is absent, the (e)RedCap UE can use mcch-Config if the UE supports the bandwidth configured by cfr-ConfigMCCH-MTCH.

mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which SFN mod mcch-ModificationPeriod = 0. The contents of different transmissions of MCCH information can only be different if there is at least one such boundary in-between them. Value rf2 corresponds to two radio frames, value rf4 corresponds to four radio frames and so on.

mcch-RepetitionPeriodAndOffset Defines the length and the offset of the MCCH repetition period. rf1 corresponds to a repetition period length of one radio frame, rf2 corresponds to a repetition period length of two radio frames and so on. The corresponding integer value indicates the offset of the repetition period in the number of radio frames. MCCH is scheduled in the MCCH transmission window starting from each radio frame for which: SFN mod repetition period length = offset of the repetition period.

mcch-WindowDuration Indicates, starting from the slot indicated by mcch-WindowStartSlot, the duration in slots during which MCCH may be scheduled. Absence of this field means that MCCH is only scheduled in the slot indicated by mcch-WindowStartSlot. The network always configures mcch-WindowDuration to be shorter or equal to the length of MCCH repetition period.

mcch-WindowStartSlot Indicates the slot in which MCCH transmission window starts.

#### –	SIB21

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI).

SIB21 information element

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

...

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

-- TAG-SIB21-STOP

-- ASN1STOP

SIB21 field descriptions

mbs-FSAI-InterFreqList Contains a list of neighboring frequencies including additional bands, if any, that provide MBS services and the corresponding MBS FSAIs.

mbs-FSAI-IntraFreq Contains the list of MBS FSAIs for the current frequency. For MBS service continuity, the UE shall use all MBS FSAIs listed in mbs-FSAI-IntraFreq to derive the MBS frequencies of interest.

#### –	SIB22

SIB22 contains ATG assistance information for ATG access.

SIB22 information element

-- ASN1START

-- TAG-SIB22-START

SIB22-r18 ::=                         SEQUENCE {

atg-Config-r18                        ATG-Config-r18                                     OPTIONAL,     -- Need R

hs-ATG-CellReselectionSet-r18         ENUMERATED {true}                                  OPTIONAL,     -- Need R

atg-NeighCellConfigList-r18           ATG-NeighCellConfigList-r18                        OPTIONAL,     -- Need R

lateNonCriticalExtension              OCTET STRING                                       OPTIONAL,

...

}

ATG-NeighCellConfigList-r18 ::=       SEQUENCE (SIZE(1..maxCellATG-r18))  OF ATG-NeighCellConfig-r18

ATG-NeighCellConfig-r18 ::=           SEQUENCE {

atg-gNB-Location-r18                  ReferenceLocation-r17                              OPTIONAL,     -- Need R

height-gNB-r18                         INTEGER (-16384..16383)                            OPTIONAL,     -- Need R

carrierFreq-r18                       ARFCN-ValueNR                                      OPTIONAL,     -- Need R

physCellId-r18                        PhysCellId                                         OPTIONAL      -- Need R

}

-- TAG-SIB22-STOP

-- ASN1STOP

SIB22 field descriptions

atg-Config Provides parameters needed for ATG access such as ATG gNB location information, cell Specific Koffset, TA Report indication.

atg-NeighCellConfigList Provides ATG assistance information of ATG neighbour cells.

hs-ATG-CellReselectionSet Indicates whether the UE applies high speed inter-frequency measurements requirements for inter-frequency cell reselection in RRC_IDLE and RRC_INACTIVE states as specified in TS 38.133 [14]. If the field is absent UE applies only the NR cell reselection requirements as specified in TS 38.133 [14].

#### –	SIB23

SIB23 contains NR sidelink Positioning configuration for dedicated SL-PRS resource pool.

SIB23 information element

-- ASN1START

-- TAG-SIB23-START

SIB23-r18 ::=                         SEQUENCE {

segmentNumber-r18                     INTEGER (0..63),

segmentType-r18                       ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r18                  OCTET STRING

}

SIB23-IEs-r18 ::=                    SEQUENCE {

sl-PosConfigCommonNR-r18              SL-PosConfigCommonNR-r18,

lateNonCriticalExtension              OCTET STRING                                                           OPTIONAL,

...

}

SL-PosConfigCommonNR-r18 ::=          SEQUENCE {

sl-PosFreqInfoList-r18                SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16      OPTIONAL,    -- Need R

sl-PosUE-SelectedConfig-r18           SL-UE-SelectedConfig-r16                                               OPTIONAL,    -- Need R

sl-PosNR-AnchorCarrierFreqList-r18    SL-NR-AnchorCarrierFreqList-r16                                        OPTIONAL,    -- Need R

sl-PosMeasConfigCommon-r18            SL-MeasConfigCommon-r16                                                OPTIONAL,    -- Need R

sl-PosOffsetDFN-r18                   INTEGER (1..1000)                                                      OPTIONAL,    -- Need R

sl-PosSSB-PriorityNR-r18              INTEGER (1..8)                                                         OPTIONAL,    -- Need R

...

}

-- TAG-SIB23-STOP

-- ASN1STOP

SIB23 field descriptions

segmentContainer This field includes a segment of the encoded SIB23-IEs. The size of the included segment in this container should be small enough that the SI message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when SIB23 is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB23-IEs. A segment number of zero corresponds to the first segment, a segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

sl-PosConfigCommonNR This field indicates the NR sidelink positioning configuration.

SL-PosConfigCommonNR field descriptions

sl-PosFreqInfoList This field indicates the NR sidelink positioning carrier frequencies for SL-PRS transmission and reception. In this release, only one entry of SL-FreqConfigCommon is included in the list.

sl-PosMeasConfigCommon This field indicates the measurement configurations (e.g. RSRP) for NR sidelink positioning.

sl-PosNR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink positioning configurations.

sl-PosOffsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

sl-PosSSB-PriorityNR This field indicates the priority of NR sidelink SSB transmission and reception.

sl-PosUE-SelectedConfig Indicates the configuration used for UE autonomous resource selection.

#### –	SIB24

SIB24 contains the information required to acquire the multicast MCCH/MTCH configuration for MBS multicast reception in RRC_INACTIVE.

SIB24 information element

-- ASN1START

-- TAG-SIB24-START

SIB24-r18 ::= SEQUENCE {

multicastMCCH-Config-r18       MCCH-Config-r17         OPTIONAL,  -- Need S

cfr-ConfigMCCH-MTCH-r18        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...

}

-- TAG-SIB24-STOP

-- ASN1STOP

SIB24 field descriptions

cfr-ConfigMCCH-MTCH Common frequency resource used for multicast MCCH and/or multicast MTCH reception. If the field is absent, the CFR for multicast has the same location and size as CORESET#0 and PDSCH configuration of multicast MCCH, if present, is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1.

multicastMCCH-Config Indicates MCCH configuration for MBS multicast reception in RRC_INACTIVE. This field is always included if it is provided via broadcast signalling in SIB24.

#### –	SIB25

SIB25 contains TN coverage information to assist neighbour cell measurements for the UEs in an NTN cell.

SIB25 information element

-- ASN1START

-- TAG-SIB25-START

SIB25-r18 ::=                  SEQUENCE {

coverageAreaInfoList-r18       CoverageAreaInfoList-r18         OPTIONAL,  -- Need R

lateNonCriticalExtension       OCTET STRING                     OPTIONAL,

...

}

CoverageAreaInfoList-r18 ::=   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18

CoverageAreaInfo-r18 ::=       SEQUENCE {

tn-AreaId-r18                  TN-AreaId-r18,

tn-ReferenceLocation-r18       ReferenceLocation-r17,

tn-DistanceRadius-r18          INTEGER(0..65535)

}

-- TAG-SIB25-STOP

-- ASN1STOP

SIB25 field descriptions

coverageAreaInfoList Contains a list of TN coverage area's information to assist skipping TN measurements for NTN UEs in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

tn-DistanceRadius Distance from the TN coverage area reference location. It is used for skipping TN measurements in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 1m.

#### –	SIB26

SIB26 contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIB26 information element

-- ASN1START

-- TAG-SIB26-START

SIB26-r19 ::=                  SEQUENCE {

od-sib1-ConfigList-r19         SEQUENCE (SIZE(1..maxNrofOD-SIB1-r19))  OF OD-SIB1-Config-r19,

lateNonCriticalExtension       OCTET STRING                                                     OPTIONAL,

...

}

OD-SIB1-Config-r19  ::=       SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

validSIB26-Cells-r19                       SEQUENCE (SIZE(1.. maxPCI-OD-SIB1-r19)) OF PhysCellId,

sib1-TDD-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                OPTIONAL, -- Cond TDD

offsetToCarrier-r19                      INTEGER (0..2199),

carrierBandwidth-r19                     INTEGER (1..maxNrofPhysicalResourceBlocks),

offsetToPointA-r19                       INTEGER (0..2199)                                                     OPTIONAL, -- Cond TDD

frequencyBandList-r19                    MultiFrequencyBandListNR-SIB                                          OPTIONAL, -- Cond TDD_NES

ss-PBCH-BlockPower-r19                   INTEGER (-60..50),

ssb-PositionsInBurst-r19                 SEQUENCE {

inOneGroup                               BIT STRING (SIZE (8)),

groupPresence                            BIT STRING (SIZE (8))                                             OPTIONAL  -- Cond FR2-Only

},

ssb-Periodicity-r19                      ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1}       OPTIONAL, -- Need R

od-sib1-WindowDuration-r19               ENUMERATED {ms20, ms40, ms80, ms160, ms320, spare3, spare2, spare1},

od-sib1-windowStartOffset-r19            ENUMERATED {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

k-ssb-r19                                INTEGER (0..23),

controlResourceSetZero-r19               ControlResourceSetZero,

searchSpaceZero-r19                      SearchSpaceZero,

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19,

sib1-RequestConfigSUL-r19                SIB1-RequestConfig-r19                                                OPTIONAL  -- Need R

}

SIB1-RequestConfig-r19 ::=              SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                  SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

},

sib1-RequestPeriod-r19                  ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}          OPTIONAL, -- Need R

sib1-RequestResources-r19               SIB1-RequestResources-r19,

sib1-PDCCH-RestrictionToPRACH-r19       ENUMERATED {true}                                                      OPTIONAL, -- Need R

rach-ConfigSIB1-r19                     RACH-ConfigGeneric,

ra-SearchSpace-r19                      SearchSpace                                                            OPTIONAL, -- Need S

n-TimingAdvanceOffset-r19               ENUMERATED { n0, n25600, n39936, spare1 }                              OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19              ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare2, spare1} OPTIONAL, -- Need R

prach-RootSequenceIndex-r19             CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

sib1-rsrp-ThresholdSSB-r19               RSRP-Range                                                            OPTIONAL, -- Need R

rsrp-ThresholdSSB-SUL                    RSRP-Range                                                            OPTIONAL, -- Cond SUL

locationAndBandwidth-r19                 INTEGER (0..37949),

absoluteFrequencyPointA-r19              ARFCN-ValueNR                                                         OPTIONAL, -- Cond FDD

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                          OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480, kHz960, spare1}       OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                 OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, spare1}

}

SIB1-RequestResources-r19 ::=            SEQUENCE {

sib1-RA-PreambleStartIndex-r19           INTEGER (0..63),

sib1-RA-AssociationPeriodIndex-r19       INTEGER (0..15)                                                       OPTIONAL, -- Need R

sib1-RA-SSB-OccasionMaskIndex-r19        INTEGER (0..15)                                                       OPTIONAL  -- Need R

}

-- TAG-SIB26-STOP

-- ASN1STOP

SIB26 field descriptions

od-sib1-ConfigList Provides a configuration to request SIB1 for serving cell and neighbor cells.

OD-SIB1-Config field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2). For the case that 12PRB/20 PRB transmission bandwidth is used as specified in TS 38.101-1 [15], TS 38.211[16] and TS 38.213[13], the network shall configure the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively and the UE shall use 12PRB/20 PRB as the transmission bandwidth respectively.

carrierFreq Identifies the ARFCN for the SSB of the cell for which this configuration is valid.

controlResourceSetZero Determines a common ControlResourceSet (CORESET) with ID #0, see TS 38.213, clause 13.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs.

k-ssb Indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block N_CRB^SSB to the lowest-numbered subcarrier of the SS/PBCH block) of the cell indicated by physCellId in IE OD-SIB1-CellConfig for OD-SIB1 transmission, see TS 38.213 [13], clause 23. For FR2, network does not configure values larger than 11.

od-sib1-WindowDuration Indicate the duration of the time window of type 0 PDCCH monitoring occasions for SIB1 after transmission of SIB1 request.

od-sib1-windowStartOffset Indicate the time offset between the reference time point and the starting time for the time window of on-demand SIB1.

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

offsetToPointA Represents the offset to Point A as defined in TS 38.211 [16], clause 4.4.4.2.

searchSpaceZero Parameters of the common SearchSpace#0, see TS 38.213 [13], clause 13.

sib1-TDD-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity The SSB periodicity in ms for at least the purpose of RO validation determination for SIB1 request transmission.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1.

validSIB26-Cells Identifies the physical cell Id(s) for which this configuration is applied.

SIB1-RequestConfig field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2).

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by offsetToCarrier configured in IE OD-SIB1-Config and subcarrierSpacing of this BWP. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2) used for SIB1 request transmission.

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this cell. See TS 38.133, table 7.1.2-2.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1[57] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) to be used for SIB1 request transmission.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1).

sib1-PDCCH-RestrictionToPRACH Indicates whether the UE can assume that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for SIB1 request.

sib1-RequestPeriod Periodicity of the SIB1-Request configuration in number of association periods.

sib1-RequestResources Resources for SIB1 request

sib1-restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

sib1-RSRP-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt to transmit SIB1 request, see TS 38.321 [3].

ssb-perRACH-Occasion Number of SSBs per RACH occasion for OD-SIB1

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources configured in the corresponding OD-SIB1-Config, excluding preambles used for other purposes (e.g. for SI request, or SIB1 request).

ul-FrequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for UL operation as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the ul-FrequencyBandList field.

ul-SubCarrierSpacing Indicates the reference SCS for msg1-FrequencyStart-r19, offsetToCarrier-r19, carrierBandwidth-r19, and locationAndBandwidth-r19.

SIB1-RequestResources field descriptions

sib1-RA-AssociationPeriodIndex Index of the association period in the sib1-RequestPeriod in which the UE can send the SIB1 request for SIB using the preambles indicated by sib1-RA-PreambleStartIndex and rach occasions indicated by sib1-RA-SSB-OccasionMaskIndex.

sib1-RA-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = sib1-RA-PreambleStartIndex + i is used for SIB1 request; For N < 1, the preamble with preamble index = sib1-RA-PreambleStartIndex is used for SIB1 request.

sib1-RA-SSB-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| FDD | The field is optionally present, Need R, for FDD cells; otherwise it is absent. |
| SUL | The field is mandatory present, in IE sib1-RequestConfigSUL; otherwise it is absent. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |
| TDD_NES | The field is mandatory present for TDD cells, otherwise it is optionally present, Need R. |

#### –	SIB27

SIB27 contains the information of the intended service area(s) of an MBS broadcast service(s) in an NTN cell.

SIB27 information element

-- ASN1START

-- TAG-SIB27-START

SIB27-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea-r19                   OCTET STRING,

circleArea-r19                    SEQUENCE {

center-r19                      ReferenceLocation-r17,

radius-r19                      INTEGER(0..65535)

}

}

}

MBS-IntendedAreaID-r19  ::= INTEGER (1..maxNrofMBS-Area-r19)

-- TAG-SIB27-STOP

-- ASN1STOP

SIB27 field descriptions

intendedServiceAreaList Contains a list of intended service areas, each associated to one or more MBS broadcast services provided in an NTN cell.

polygonArea Parameter type Polygon defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

### 6.3.1a	Positioning System information blocks

#### –	PosSystemInformation-r16-IEs

-- ASN1START

-- TAG-POSSYSTEMINFORMATION-R16-IES-START

PosSystemInformation-r16-IEs ::= SEQUENCE {

posSIB-TypeAndInfo-r16           SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

posSib1-1-r16                    SIBpos-r16,

posSib1-2-r16                    SIBpos-r16,

posSib1-3-r16                    SIBpos-r16,

posSib1-4-r16                    SIBpos-r16,

posSib1-5-r16                    SIBpos-r16,

posSib1-6-r16                    SIBpos-r16,

posSib1-7-r16                    SIBpos-r16,

posSib1-8-r16                    SIBpos-r16,

posSib2-1-r16                    SIBpos-r16,

posSib2-2-r16                    SIBpos-r16,

posSib2-3-r16                    SIBpos-r16,

posSib2-4-r16                    SIBpos-r16,

posSib2-5-r16                    SIBpos-r16,

posSib2-6-r16                    SIBpos-r16,

posSib2-7-r16                    SIBpos-r16,

posSib2-8-r16                    SIBpos-r16,

posSib2-9-r16                    SIBpos-r16,

posSib2-10-r16                   SIBpos-r16,

posSib2-11-r16                   SIBpos-r16,

posSib2-12-r16                   SIBpos-r16,

posSib2-13-r16                   SIBpos-r16,

posSib2-14-r16                   SIBpos-r16,

posSib2-15-r16                   SIBpos-r16,

posSib2-16-r16                   SIBpos-r16,

posSib2-17-r16                   SIBpos-r16,

posSib2-18-r16                   SIBpos-r16,

posSib2-19-r16                   SIBpos-r16,

posSib2-20-r16                   SIBpos-r16,

posSib2-21-r16                   SIBpos-r16,

posSib2-22-r16                   SIBpos-r16,

posSib2-23-r16                   SIBpos-r16,

posSib3-1-r16                    SIBpos-r16,

posSib4-1-r16                    SIBpos-r16,

posSib5-1-r16                    SIBpos-r16,

posSib6-1-r16                    SIBpos-r16,

posSib6-2-r16                    SIBpos-r16,

posSib6-3-r16                    SIBpos-r16,

... ,

posSib1-9-v1700                  SIBpos-r16,

posSib1-10-v1700                 SIBpos-r16,

posSib2-24-v1700                 SIBpos-r16,

posSib2-25-v1700                 SIBpos-r16,

posSib6-4-v1700                  SIBpos-r16,

posSib6-5-v1700                  SIBpos-r16,

posSib6-6-v1700                  SIBpos-r16,

posSib2-17a-v1770                SIBpos-r16,

posSib2-18a-v1770                SIBpos-r16,

posSib2-20a-v1770                SIBpos-r16,

posSib1-11-v1800                 SIBpos-r16,

posSib1-12-v1800                 SIBpos-r16,

posSib2-26-v1800                 SIBpos-r16,

posSib2-27-v1800                 SIBpos-r16,

posSib6-7-v1800                  SIBpos-r16,

posSib7-1-v1800                  SIBpos-r16,

posSib7-2-v1800                  SIBpos-r16,

posSib7-3-v1800                  SIBpos-r16,

posSib7-4-v1800                  SIBpos-r16

},

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

-- TAG-POSSYSTEMINFORMATION-R16-IES-STOP

-- ASN1STOP

#### –	PosSI-SchedulingInfo

-- ASN1START

-- TAG-POSSI-SCHEDULINGINFO-START

PosSI-SchedulingInfo-r16 ::=               SEQUENCE {

posSchedulingInfoList-r16                  SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r16,

posSI-RequestConfig-r16                        SI-RequestConfig                                 OPTIONAL,  -- Cond MSG-1

posSI-RequestConfigSUL-r16                     SI-RequestConfig                                 OPTIONAL,  -- Cond SUL-MSG-1

...,

[[

posSI-RequestConfigRedCap-r17                  SI-RequestConfig                                 OPTIONAL   -- Cond REDCAP-MSG-1

]],

[[

posSI-RequestConfigMSG1-Repetition-r18         SI-RequestConfigRepetition-r18                   OPTIONAL,  -- Cond MSG-1

posSI-RequestConfigSUL-MSG1-Repetition-r18     SI-RequestConfigRepetition-r18                   OPTIONAL,  -- Cond SUL-MSG-1

posSI-RequestConfigRedCap-MSG1-Repetition-r18  SI-RequestConfigRepetition-r18                   OPTIONAL   -- Cond REDCAP-MSG-1

]]

}

PosSchedulingInfo-r16 ::= SEQUENCE {

offsetToSI-Used-r16          ENUMERATED {true}                                              OPTIONAL,  -- Need R

posSI-Periodicity-r16        ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

posSI-BroadcastStatus-r16    ENUMERATED {broadcasting, notBroadcasting},

posSIB-MappingInfo-r16       PosSIB-MappingInfo-r16,

...

}

PosSIB-MappingInfo-r16 ::=   SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r16

PosSIB-Type-r16 ::=          SEQUENCE {

encrypted-r16                ENUMERATED { true }                                            OPTIONAL,  -- Need R

gnss-id-r16                  GNSS-ID-r16                                                    OPTIONAL,  -- Need R

sbas-id-r16                  SBAS-ID-r16                                                    OPTIONAL,  -- Cond GNSS-ID-SBAS

posSibType-r16               ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6,

posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4,

posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10,

posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15,

posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20,

posSibType2-21, posSibType2-22, posSibType2-23, posSibType3-1, posSibType4-1,

posSibType5-1,posSibType6-1, posSibType6-2, posSibType6-3,... },

areaScope-r16                ENUMERATED {true}                                              OPTIONAL -- Need S

}

GNSS-ID-r16 ::= SEQUENCE {

gnss-id-r16              ENUMERATED{gps, sbas, qzss, galileo, glonass, bds, ..., navic-v1760},

...

}

SBAS-ID-r16 ::= SEQUENCE {

sbas-id-r16              ENUMERATED { waas, egnos, msas, gagan, ...},

...

}

-- TAG-POSSI-SCHEDULINGINFO-STOP

-- ASN1STOP

PosSI-SchedulingInfo field descriptions

areaScope Indicates that a posSIB is area specific. If the field is absent, the posSIB is cell specific.

encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 37.355 [49].

gnss-id The presence of this field indicates that the positioning SIB type is for a specific GNSS. Indicates a specific GNSS (see also TS 37.355 [49])

posSchedulingInfoList List of scheduling information for SI messages. If si-SchedulingInfo-v1700 is present, the network ensures that the total number of SI messages in the concatenated list of SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo and SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 does not exceed the limit of maxSI-Message.

posSI-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of posSI-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting.

posSI-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which posSI-BroadcastStatus or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting.

posSI-RequestConfigMSG1-Repetition Configuration of Msg1 repetition resources on NUL that the UE uses for requesting SI-messages for which posSI-BroadcastStatus or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

posSI-RequestConfigRedCap Configuration of Msg1 resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which posSI-BroadcastStatus or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting.

posSI-RequestConfigRedCap-MSG1-Repetition Configuration of Msg1 repetition resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which posSI-BroadcastStatus or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

posSI-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which posSI-BroadcastStatus or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting.

posSI-RequestConfigSUL-MSG1-Repetition Configuration of Msg1 repetition resources on SUL that the UE uses for requesting SI-messages for which posSI-BroadcastStatus or si-BroadcastStatus of the type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700, if present, is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

posSIB-MappingInfo List of the posSIBs mapped to this SystemInformation message.

posSibType The positioning SIB type is defined in TS 37.355 [49].

posSI-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. If the offsetToSI-Used is configured, the posSI-Periodicity of rf8 cannot be used.

offsetToSI-Used This field, if present, indicates that all the SI messages in posSchedulingInfoList are scheduled with an offset of 8 radio frames compared to SI messages in schedulingInfoList. offsetToSI-Used may be present only if the shortest configured SI message periodicity for SI messages in schedulingInfoList is 80ms. If SI offset is used, this field is present in each of the SI messages in the posSchedulingInfoList.

sbas-id The presence of this field indicates that the positioning SIB type is for a specific SBAS. Indicates a specific SBAS (see also TS 37.355 [49]).

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if gnss-id is set to sbas. It is absent otherwise. |
| MSG-1 | The field is optionally present, Need R, if posSI-BroadcastStatus is set to notBroadcasting for any SI-message included in posSchedulingInfoList or if si-BroadcastStatus is set to notBroadcasting for any SI-message containing type2 SIB included in schedulingInfoList2. It is absent otherwise. |
| SUL-MSG-1 | The field is optionally present, Need R, if supplementaryUplink is configured in ServingCellConfigCommonSIB, and if posSI-BroadcastStatus is set to notBroadcasting for any SI-message included in posSchedulingInfoList or if si-BroadcastStatus is set to notBroadcasting for any SI-message containing type2 SIB included in schedulingInfoList2. It is absent otherwise. |
| REDCAP-MSG-1 | The field is optionally present, Need R, if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB, and if posSI-BroadcastStatus is set to notBroadcasting for any SI-message included in posSchedulingInfoList or if si-BroadcastStatus is set to notBroadcasting for any SI-message containing type2 SIB included in schedulingInfoList2. It is absent otherwise. |

#### –	SIBpos

The IE SIBpos contains positioning assistance data as defined in TS 37.355 [49].

SIBpos information element

-- ASN1START

-- TAG-SIPOS-START

SIBpos-r16 ::= SEQUENCE {

assistanceDataSIB-Element-r16        OCTET STRING,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

...

}

-- TAG-SIPOS-STOP

-- ASN1STOP

SIBpos field descriptions

assistanceDataSIB-Element Parameter AssistanceDataSIBelement defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

### 6.3.2	Radio resource control information elements

#### –	AdditionalPCIIndex

The IE AdditionalPCIIndex identifies an additional physical cell identity (PCI) configured with additionalPCI-ToAddModList-r17 in ServingCellConfig.

AdditionalPCIIndex information element

-- ASN1START

-- TAG-ADDITIONALPCIINDEX-START

AdditionalPCIIndex-r17  ::=  INTEGER(1..maxNrofAdditionalPCI-r17)

-- TAG-ADDITIONALPCIINDEX-STOP

-- ASN1STOP

#### –	AdditionalSpectrumEmission

The IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by the UE (see TS 38.101-1 [15], clause 6.2.3/6.2A.3, TS 38.101-2 [39], clause 6.2.3/6.2A.3, and TS 38.101-5 [75], clause 6.2.3). If an extension is signalled using the extended value range (as defined by the IE AdditionalSpectrumEmission-v1760), the corresponding original field, using the value range as defined by the IE AdditionalSpectrumEmission (without suffix) shall be set to value 7.

AdditionalSpectrumEmission information element

-- ASN1START

-- TAG-ADDITIONALSPECTRUMEMISSION-START

AdditionalSpectrumEmission ::=              INTEGER (0..7)

AdditionalSpectrumEmission-v1760 ::=        INTEGER (8..39)

AdditionalSpectrumEmission-r18 ::=          INTEGER (0..39)

-- TAG-ADDITIONALSPECTRUMEMISSION-STOP

-- ASN1STOP

#### –	AdvancedReceiver-MU-MIMO

The IE AdvancedReceiver-MU-MIMO is used to provide a set of assistance information for R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions.

AdvancedReceiver-MU-MIMO information element

-- ASN1START

-- TAG-ADVANCEDRECEIVER-MU-MIMO-START

AdvancedReceiver-MU-MIMO-r18 ::=      SEQUENCE {

precodingAndResourceAllocation-r18    BOOLEAN                                                OPTIONAL,    -- Need M

pdsch-TimeDomainAllocation-r18        BOOLEAN                                                OPTIONAL,    -- Need M

mcs-Table-r18                         ENUMERATED {qam1024, qam256, qam64, spare1}            OPTIONAL,    -- Need R

advReceiver-MU-MIMO-DCI-1-1-r18       ENUMERATED {enabled}                                   OPTIONAL,    -- Need R

...

}

-- TAG-ADVANCEDRECEIVER-MU-MIMO-STOP

-- ASN1STOP

AdvancedReceiver-MU-MIMO field descriptions

advReceiver-MU-MIMO-DCI-1-1 Configure the presence of the co-scheduled UE information field in DCI format 1_1 (see TS 38.212 [17], clause 7.3.1.2.2).

mcs-Table Indicates the MCS table with the highest modulation order among all MCS tables configured to the co-scheduled UE(s), which has the same DMRS sequence as the target UE.

pdsch-TimeDomainAllocation If the field is set to true, the UE can assume the time domain resource assignment for PDSCH symbols of all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, is same as the target UE. If the field is set to false, the UE can not assume the time domain resource assignment for PDSCH symbols of all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, is same as the target UE.

precodingAndResourceAllocation If the field is set to true, the UE can assume the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE when PRG=2 or 4, when the target UE and any co-scheduled UEs are in different CDM groups and with the same DMRS sequence. If the field is set to false, the UE can not assume the precoding or resource allocation of the co-scheduled UE is the same in the PRG-level grid configured to the target UE when PRG=2 or 4, when the target UE and any co-scheduled UEs are in different CDM groups and with the same DMRS sequence.

NOTE:	the same DMRS sequence represents the same root DMRS sequence r(n), see TS 38.211 [16], clause 7.4.1.1.1.

#### –	Aerial-Config

The IE Aerial-Config provides configuration parameters for aerial UE.

Aerial-Config information element

-- ASN1START

-- TAG-AERIAL-CONFIG-START

Aerial-Config-r18 ::= SEQUENCE {

flightPathUpdateThrConfig-r18     SEQUENCE {

flightPathUpdateDistanceThr-r18   SetupRelease { FlightPathUpdateDistanceThr-r18 }        OPTIONAL, -- Need M

flightPathUpdateTimeThr-r18       SetupRelease { FlightPathUpdateTimeThr-r18 }            OPTIONAL  -- Need M

}                                                                                             OPTIONAL, -- Need M

...

}

FlightPathUpdateDistanceThr-r18 ::=   INTEGER (0..1023)

FlightPathUpdateTimeThr-r18 ::=       INTEGER (0..16383)

-- TAG-AERIAL-CONFIG-STOP

-- ASN1STOP

Aerial-Config field descriptions

flightPathUpdateDistanceThr Distance threshold for triggering flight path update indication. Actual value is 5 x field value in meters.

flightPathUpdateTimeThr Time threshold for triggering flight path update indication. Value in seconds.

#### –	Alpha

The IE Alpha defines possible values of a the pathloss compensation coefficient for uplink power control. Value alpha0 corresponds to the value 0, Value alpha04 corresponds to the value 0.4, Value alpha05 corresponds to the value 0.5 and so on. Value alpha1 corresponds to value 1. See also clause 7.1 of TS 38.213 [13].

-- ASN1START

-- TAG-ALPHA-START

Alpha ::=                       ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1}

-- TAG-ALPHA-STOP

-- ASN1STOP

#### –	Altitude

The IE Altitude is used to indicate altitude relative to sea level. The actual value is the field value in meters.

Altitude information element

-- ASN1START

-- TAG-ALTITUDE-START

Altitude-r18 ::=              INTEGER (minAltitude-r18..maxAltitude-r18)

-- TAG-ALTITUDE-STOP

-- ASN1STOP

#### –	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in TS 23.003 [21], clause 2.10.1.

AMF-Identifier information element

-- ASN1START

-- TAG-AMF-IDENTIFIER-START

AMF-Identifier ::=                      BIT STRING (SIZE (24))

-- TAG-AMF-IDENTIFIER-STOP

-- ASN1STOP

#### –	ApplicabilitySetConfigId

The IE ApplicabilitySetConfigId is used to identify an ApplicabilitySetConfig.

ApplicabilitySetConfigId information element

-- ASN1START

-- TAG-APPLICABILITYSETCONFIGID-START

ApplicabilitySetConfigId-r19 ::=            INTEGER (0..maxNrofApplicabilitySets-1-r19)

-- TAG-APPLICABILITYSETCONFIGID-STOP

-- ASN1STOP

#### –	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of configurations subject to the applicability determination procedure.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=      SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=          SEQUENCE {

applicabilityCellId-r19              ServCellIndex,

applicabilityInfoReportList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports-r19)) OF ApplicabilityInfoReport-r19  OPTIONAL,

...

}

ApplicabilityInfoReport-r19 ::=      SEQUENCE {

applicabilityInfoReportId-r19        CHOICE {

csi-ReportConfigId-r19               CSI-ReportConfigId,

applicabilitySetId-r19               ApplicabilitySetConfigId-r19,

spare2                               NULL,

spare1                               NULL

},

applicabilityStatus-r19              ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19   ENUMERATED {true}                                                                    OPTIONAL,

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

ApplicabilityReportList field descriptions

applicabilityCellId Index of the serving cell that the ApplicabilityReport refers to.

applicabilityInfoReportId Indicates the ID of a configuration subject to the applicability determination procedure.

applicabilityInfoReportList Indicates the list of applicability reports, each associatied with a configuration ID for a configuration subject to the applicability determination procedure.

applicabilityStatus Indicates whether the configuration associated to applicabilityInfoReportId is applicable or inapplicable.

releaseConfigurationPreference Indicates the UE's preference to release the configuration associated to applicabilityReportConfigId (e.g. due to model unavailability), if the applicabilityStatus is set to 'inapplicable'.

#### –	ARFCN-ValueEUTRA

The IE ARFCN-ValueEUTRA is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) E-UTRA carrier frequency, as defined in TS 36.101 [22].

ARFCN-ValueEUTRA information element

-- ASN1START

-- TAG-ARFCN-VALUEEUTRA-START

ARFCN-ValueEUTRA ::=                INTEGER (0..maxEARFCN)

-- TAG-ARFCN-VALUEEUTRA-STOP

-- ASN1STOP

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-1 [15], TS 38.101-2 [39] and TS 38.101-5 [75], clause 5.4.2.

-- ASN1START

-- TAG-ARFCN-VALUENR-START

ARFCN-ValueNR ::=               INTEGER (0..maxNARFCN)

-- TAG-ARFCN-VALUENR-STOP

-- ASN1STOP

#### –	ARFCN-ValueUTRA-FDD

The IE ARFCN-ValueUTRA-FDD is used to indicate the ARFCN applicable for a downlink (Nd, FDD) UTRA-FDD carrier frequency, as defined in TS 25.331 [45].

ARFCN-ValueUTRA-FDD information element

-- ASN1START

-- TAG-ARFCN-ValueUTRA-FDD-START

ARFCN-ValueUTRA-FDD-r16 ::=                INTEGER (0..16383)

-- TAG-ARFCN-ValueUTRA-FDD-STOP

-- ASN1STOP

#### –	AssociatedId

The IE AssociatedId indicates that the UE may assume similar properties of a DL Tx beam or beam set/list associated with the same value. The AssociatedID value is unique within a PLMN, i.e. it can only be associated with one same/similar beam deployment within the same PLMN.

AssociatedId information element

-- ASN1START

-- TAG-ASSOCIATEDID-START

AssociatedId-r19 ::=        BIT STRING (SIZE (24))

-- TAG-ASSOCIATEDID-STOP

-- ASN1STOP

#### –	ATG-Config

The IE ATG-Config provides parameters needed for the UE to access NR via ATG access.

ATG-Config information element

-- ASN1START

-- TAG-ATG-CONFIG-START

ATG-Config-r18 ::=          SEQUENCE {

atg-gNB-Location-r18         ReferenceLocation-r17                                 OPTIONAL,    -- Need R

height-gNB-r18                INTEGER (-16384..16383)                               OPTIONAL,    -- Need R

cellSpecificKoffset-r18      INTEGER(1..3)                                         OPTIONAL,    -- Need R

ta-ReportATG-r18             ENUMERATED {enabled}                                  OPTIONAL     -- Need R

}

-- TAG-ATG-CONFIG-STOP

-- ASN1STOP

ATG-Config field descriptions

atg-gNB-Location Indicates the BS location for ATG access.

cellSpecificKoffset Scheduling offset used for the timing relationships that are modified for ATG. Unit in slot.

height-gNB Indicates the height of the BS for ATG access relative to the sea level. Unit in meter.

ta-ReportATG When this field is included in SIB22, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

#### –	AvailabilityCombinationsPerCell

The IE AvailabilityCombinationsPerCell is used to configure the AvailabilityCombinations applicable for a cell of the IAB DU (see TS 38.213 [13], clause 14). Note that the IE AvailabilityCombinationsPerCellIndex can only be configured up to 511.

AvailabilityCombinationsPerCell information element

-- ASN1START

-- TAG-AVAILABILITYCOMBINATIONSPERCELL-START

AvailabilityCombinationsPerCell-r16 ::=     SEQUENCE {

availabilityCombinationsPerCellIndex-r16     AvailabilityCombinationsPerCellIndex-r16,

iab-DU-CellIdentity-r16                      CellIdentity,

positionInDCI-AI-r16                         INTEGER(0..maxAI-DCI-PayloadSize-1-r16)                              OPTIONAL, -- Need M

availabilityCombinations-r16                 SEQUENCE (SIZE (1..maxNrofAvailabilityCombinationsPerSet-r16)) OF AvailabilityCombination-r16,

...,

[[

availabilityCombinationsRB-Groups-r17   SEQUENCE (SIZE (1..maxNrofAvailabilityCombinationsPerSet-r16)) OF

AvailabilityCombinationRB-Groups-r17    OPTIONAL -- Need M

]],

[[

positionInDCI-AI-RBGroups-v1720              INTEGER(0..maxAI-DCI-PayloadSize-1-r16)                              OPTIONAL -- Need M

]]

}

AvailabilityCombinationsPerCellIndex-r16 ::= INTEGER(0..maxNrofDUCells-r16)

AvailabilityCombination-r16 ::=         SEQUENCE {

availabilityCombinationId-r16           AvailabilityCombinationId-r16,

resourceAvailability-r16                SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7)

}

AvailabilityCombinationId-r16 ::=       INTEGER (0..maxNrofAvailabilityCombinationsPerSet-1-r16)

AvailabilityCombinationRB-Groups-r17 ::= SEQUENCE {

availabilityCombinationId-r17    AvailabilityCombinationId-r16,

rb-SetGroups-r17                  SEQUENCE (SIZE (1..maxNrofRB-SetGroups-r17)) OF RB-SetGroup-r17                           OPTIONAL, -- Need R

resourceAvailability-r17         SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7)    OPTIONAL -- Need R

}

RB-SetGroup-r17 ::=       SEQUENCE {

resourceAvailability-r17  SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7) OPTIONAL, -- Need R

rb-Sets-r17                SEQUENCE (SIZE (1..maxNrofRB-Sets-r17)) OF INTEGER (0..7)                             OPTIONAL  -- Need R

}

-- TAG-AVAILABILITYCOMBINATIONSPERCELL-STOP

-- ASN1STOP

AvailabilityCombination field descriptions

availabilityCombinationId This ID is used in the DCI Format 2_5 payload to dynamically select this AvailabilityCombination, see TS 38.213 [13], clause 14.

resourceAvailability Indicates the resource availability of soft symbols for a set of consecutive slots in the time domain. The meaning of this field is described in TS 38.213 [13], Table 14.3. If included in RB-SetGroup within AvailabilityCombinationRB-Groups-r17, it indicates the availability of soft resources for an RB set group. If included in AvailabilityCombinationRB-Groups-r17 when the rb-SetGroups is not configured, it indicates the availability of soft resources in one or multiple slots for all RB sets of a DU cell.

AvailabilityCombinationsPerCell field descriptions

iab-DU-CellIdentity The ID of the IAB-DU cell for which the availabilityCombinations are applicable.

positionInDCI-AI The (starting) position (bit) of the availabilityCombinationId for the indicated IAB-DU cell (iab-DU-CellIdentity) within the DCI payload. If positionInDCI-AI-RBGroups is not configured, it applies to the availabilityCombinationId included in availabilityCombinations and in availabilityCombinationsRB-Groups. If positionInDCI-AI-RBGroups is configured, it applies to the availabilityCombinationId included in availabilityCombinations.

positionInDCI-AI-RBGroups The (starting) position (bit) of the availabilityCombinationId associated to the availabilityCombinationsRB-Groups for the indicated IAB-DU cell (iab-DU-CellIdentity) within the DCI payload.

AvailabilityCombinationRB-Groups field descriptions

rb-SetGroups Indicates the RB set groups configured for the availability combination. Each group includes consecutive RB sets.

rb-Sets Indicates the one or more RB set indexes associated to one or more RB sets configured for one RB set group.

#### –	AvailabilityIndicator

The IE AvailabilityIndicator is used to configure monitoring a PDCCH for Availability Indicators (AI).

AvailabilityIndicator information element

-- ASN1START

-- TAG-AVAILABILITYINDICATOR-START

AvailabilityIndicator-r16 ::=    SEQUENCE {

ai-RNTI-r16                      AI-RNTI-r16,

dci-PayloadSizeAI-r16            INTEGER (1..maxAI-DCI-PayloadSize-r16),

availableCombToAddModList-r16    SEQUENCE (SIZE(1..maxNrofDUCells-r16)) OF AvailabilityCombinationsPerCell-r16          OPTIONAL, -- Need N

availableCombToReleaseList-r16   SEQUENCE (SIZE(1..maxNrofDUCells-r16)) OF AvailabilityCombinationsPerCellIndex-r16     OPTIONAL, -- Need N

...

}

AI-RNTI-r16 ::=                      RNTI-Value

-- TAG-AVAILABILITYINDICATOR-STOP

-- ASN1STOP

AvailabilityIndicator field descriptions

ai-RNTI Used by an IAB-MT for detection of DCI format 2_5 indicating AvailabilityCombinationId for an IAB-DU's cells.

availableCombToAddModList A list of availabilityCombinations to add for the IAB-DU's cells. (see TS 38.213 [13], clause 14).

availableCombToReleaseList A list of availabilityCombinations to release for the IAB-DU's cells. (see TS 38.213 [13], clause 14).

dci-PayloadSizeAI Total length of the DCI payload scrambled with ai-RNTI (see TS 38.213 [13]).

#### –	BAP-RoutingID

The IE BAP-RoutingID is used for IAB-node to configure the BAP Routing ID.

BAP-RoutingID information element

-- ASN1START

-- TAG-BAPROUTINGID-START

BAP-RoutingID-r16::=        SEQUENCE{

bap-Address-r16              BIT STRING (SIZE (10)),

bap-PathId-r16               BIT STRING (SIZE (10))

}

-- TAG-BAPROUTINGID-STOP

-- ASN1STOP

BAP-RoutingID field descriptions

bap-Address The ID of a destination IAB-node or IAB-donor-DU used in the BAP header.

bap-PathId The ID of a path used in the BAP header.

#### –	BeamFailureRecoveryConfig

The IE BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYCONFIG-START

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                                                          OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                                                        OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                                                                OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}                                      OPTIONAL, -- Need M

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                           OPTIONAL, -- Need M

recoverySearchSpaceId               SearchSpaceId                                                             OPTIONAL, -- Need R

ra-Prioritization                   RA-Prioritization                                                         OPTIONAL, -- Need R

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}            OPTIONAL, -- Need M

...,

[[

msg1-SubcarrierSpacing              SubcarrierSpacing                                                         OPTIONAL  -- Need M

]],

[[

ra-PrioritizationTwoStep-r16        RA-Prioritization                                                         OPTIONAL, -- Need R

candidateBeamRSListExt-v1610        SetupRelease{ CandidateBeamRSListExt-r16 }                                OPTIONAL  -- Need M

]],

[[

spCell-BFR-CBRA-r16                 ENUMERATED {true}                                                         OPTIONAL  -- Need R

]],

[[

ra-OccasionType-r19                 ENUMERATED {sbfd}                                                         OPTIONAL  -- Need S

]]

}

PRACH-ResourceDedicatedBFR ::=      CHOICE {

ssb                                 BFR-SSB-Resource,

csi-RS                              BFR-CSIRS-Resource

}

BFR-SSB-Resource ::=                SEQUENCE {

ssb                                 SSB-Index,

ra-PreambleIndex                    INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=              SEQUENCE {

csi-RS                              NZP-CSI-RS-ResourceId,

ra-OccasionList                     SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)   OPTIONAL,   -- Need R

ra-PreambleIndex                    INTEGER (0..63)                                                                 OPTIONAL,   -- Need R

...

}

CandidateBeamRSListExt-r16::=       SEQUENCE (SIZE(1.. maxNrofCandidateBeamsExt-r16)) OF PRACH-ResourceDedicatedBFR

-- TAG-BEAMFAILURERECOVERYCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

candidateBeamRSList, candidateBeamRSListExt-v1610 Set of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. This set includes all elements of candidateBeamRSList (without suffix) and all elements of candidateBeamRSListExt-v1610. The UE maintains candidateBeamRSList and candidateBeamRSListExt-v1610 separately: Receiving candidateBeamRSListExt-v1610 set to release releases only the entries that were configured by candidateBeamRSListExt-v1610, and receiving candidateBeamRSListExt-v1610 set to setup replaces only the entries that were configured by candidateBeamRSListExt-v1610 with the newly signalled entries. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

msg1-SubcarrierSpacing Subcarrier spacing for contention free beam failure recovery (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free random access to recover from beam failure (see TS 38.213 [13], clause 6).

ra-OccasionType Indicates the second PRACH occasions of CFRA to be used by a SBFD aware UE (see TS 38.213 [13], clause 8). If absent, indicates the first PRACH occasions to be used.

ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access procedure for BFR (see TS 38.321 [3], clause 5.1.1).

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources.

rach-ConfigBFR Configuration of random access parameters for BFR.

recoverySearchSpaceId Search space to use for BFR RAR. The network configures this search space to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided. The CORESET associated with the recovery search space cannot be associated with another search space. Network always configures the UE with a value for this field when contention free random access resources for BFR are configured.

rootSequenceIndex-BFR PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) for beam failure recovery.

spCell-BFR-CBRA Indicates that UE is configured to send MAC CE for SpCell BFR as specified in TS38.321 [3].

ssb-perRACH-Occasion Number of SSBs per RACH occasion for CF-BFR, see TS 38.213 [13], clause 8.1.

BFR-CSIRS-Resource field descriptions

csi-RS The ID of a NZP-CSI-RS-Resource configured in the CSI-MeasConfig of this serving cell. This reference signal determines a candidate beam for beam failure recovery (BFR).

ra-OccasionList RA occasions that the UE shall use when performing BFR upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Per RO type, each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots. If the field is absent the UE uses the RA occasion associated with the SSB that is QCLed with this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS. If the field is absent, the UE uses the preamble index associated with the SSB that is QCLed with this CSI-RS.

BFR-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing BFR upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell. It determines a candidate beam for beam failure recovery (BFR).

#### –	BeamFailureRecoveryRSConfig

The IE BeamFailureRecoveryRSConfig is used to configure the UE with candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.17.

BeamFailureRecoveryRSConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYRSCONFIG-START

BeamFailureRecoveryRSConfig-r16 ::= SEQUENCE {

rsrp-ThresholdBFR-r16               RSRP-Range                                                               OPTIONAL, -- Need M

candidateBeamRS-List-r16            SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16     OPTIONAL, -- Need M

...,

[[

candidateBeamRS-List2-r17            SEQUENCE (SIZE(1..maxNrofCandidateBeams-r16)) OF CandidateBeamRS-r16     OPTIONAL  -- Need R

]]

}

-- TAG-BEAMFAILURERECOVERYRSCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryRSConfig field descriptions

candidateBeamRS-List A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery. The network always configures this parameter in every instance of this IE.

candidateBeamRS-List2 A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery.

rsrp-ThresholdBFR L1-RSRP threshold used for determining whether a candidate beam may be included by the UE in MAC CE for BFR (see TS 38.321 [3] and TS 38.213 [13], clause 6). The network always configures this parameter in every instance of this IE.

#### –	BetaOffsets

The IE BetaOffsets is used to configure beta-offset values, see TS 38.213 [13], clause 9.3.

BetaOffsets information element

-- ASN1START

-- TAG-BETAOFFSETS-START

BetaOffsets ::=                     SEQUENCE {

betaOffsetACK-Index1                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetACK-Index2                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetACK-Index3                INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1          INTEGER(0..31)                                                          OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2          INTEGER(0..31)                                                          OPTIONAL  -- Need S

}

-- TAG-BETAOFFSETS-STOP

-- ASN1STOP

BetaOffsets field descriptions

betaOffsetACK-Index1 Up to 2 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index2 Up to 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index3 Above 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetCSI-Part1-Index1 Up to 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part1-Index2 Above 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index1 Up to 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index2 Above 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

#### –	BetaOffsetsCrossPri

The IE BetaOffsetsCrossPri is used to configure beta-offset values for cross-priority HARQ-ACK multiplexing on PUSCH.

BetaOffsetsCrossPri information element

-- ASN1START

-- TAG-BETAOFFSETSCROSSPRI-START

BetaOffsetsCrossPri-r17 ::= SEQUENCE (SIZE(3)) OF INTEGER(0..31)

-- TAG-BETAOFFSETSCROSSPRI-STOP

-- ASN1STOP

#### –	BH-LogicalChannelIdentity

The IE BH-LogicalChannelIdentity is used to identify a logical channel between an IAB-node and its parent IAB-node or IAB-donor-DU.

BH-LogicalChannelIdentity information element

-- ASN1START

-- TAG-BHLOGICALCHANNELIDENTITY-START

BH-LogicalChannelIdentity-r16 ::=    CHOICE {

bh-LogicalChannelIdentity-r16        LogicalChannelIdentity,

bh-LogicalChannelIdentityExt-r16     BH-LogicalChannelIdentity-Ext-r16

}

-- TAG-BHLOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

BH-LogicalChannelIdentity field descriptions

bh-LogicalChannelIdentity ID used for the MAC logical channel.

bh-LogicalChannelIdentityExt ID used for the MAC logical channel.

#### –	BH-LogicalChannelIdentity-Ext

The IE BH-LogicalChannelIdentity-Ext is used to identify a logical channel between an IAB-node and its parent node.

BH-LogicalChannelIdentity-Ext information element

-- ASN1START

-- TAG-BHLOGICALCHANNELIDENTITYEXT-START

BH-LogicalChannelIdentity-Ext-r16 ::=   INTEGER (320.. maxLC-ID-Iab-r16)

-- TAG-BHLOGICALCHANNELIDENTITYEXT-STOP

-- ASN1STOP

#### –	BH-RLC-ChannelConfig

The IE BH-RLC-ChannelConfig is used to configure an RLC entity, a corresponding logical channel in MAC for BH RLC channel between IAB-node and its parent node.

BH-RLC-ChannelConfig information element

-- ASN1START

-- TAG-BHRLCCHANNELCONFIG-START

BH-RLC-ChannelConfig-r16::=      SEQUENCE {

bh-LogicalChannelIdentity-r16    BH-LogicalChannelIdentity-r16     OPTIONAL,   -- Cond LCH-SetupOnly

bh-RLC-ChannelID-r16             BH-RLC-ChannelID-r16,

reestablishRLC-r16               ENUMERATED {true}                 OPTIONAL,   -- Need N

rlc-Config-r16                   RLC-Config                        OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig-r16     LogicalChannelConfig              OPTIONAL,   -- Cond LCH-Setup

...

}

-- TAG-BHRLCCHANNELCONFIG-STOP

-- ASN1STOP

BH-RLC-ChannelConfig field descriptions

bh-LogicalChannelIdentity Indicates the logical channel id for BH RLC channel of the IAB-node.

bh-RLC-ChannelID Indicates the BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

reestablishRLC Indicates that RLC should be re-established.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a BH RLC channel. It is optionally present, Need M, otherwise. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a BH RLC channel. It is absent, Need M otherwise. |

#### –	BH-RLC-ChannelID

The IE BH-RLC-ChannelID is used to identify a BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

BH-RLC-ChannelID information element

-- ASN1START

-- TAG-BHRLCCHANNELID-START

BH-RLC-ChannelID-r16 ::=    BIT STRING (SIZE (16))

-- TAG-BHRLCCHANNELID-STOP

-- ASN1STOP

#### –	BSR-Config

The IE BSR-Config is used to configure buffer status reporting.

BSR-Config information element

-- ASN1START

-- TAG-BSR-CONFIG-START

BSR-Config ::=                      SEQUENCE {

periodicBSR-Timer                   ENUMERATED { sf1, sf5, sf10, sf16, sf20, sf32, sf40, sf64,

sf80, sf128, sf160, sf320, sf640, sf1280, sf2560, infinity },

retxBSR-Timer                       ENUMERATED { sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, sf2560,

sf5120, sf10240, spare5, spare4, spare3, spare2, spare1},

logicalChannelSR-DelayTimer         ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}                OPTIONAL, -- Need R

...

}

-- TAG-BSR-CONFIG-STOP

-- ASN1STOP

BSR-Config field descriptions

logicalChannelSR-DelayTimer Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on.

periodicBSR-Timer Value in number of subframes. Value sf1 corresponds to 1 subframe, value sf5 corresponds to 5 subframes and so on.

retxBSR-Timer Value in number of subframes. Value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes and so on.

#### –	BWP

The IE BWP is used to configure generic parameters of a bandwidth part as defined in TS 38.211 [16], clause 4.5, and TS 38.213 [13], clause 12.

For each serving cell the network configures at least an initial downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) initial uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The uplink and downlink bandwidth part configurations are divided into common and dedicated parameters.

BWP information element

-- ASN1START

-- TAG-BWP-START

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }                                                 OPTIONAL    -- Need R

}

-- TAG-BWP-STOP

-- ASN1STOP

BWP field descriptions

cyclicPrefix Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Normal CP is supported for all subcarrier spacings and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. (see TS 38.211 [16], clause 4.2). Except for SUL, the network ensures the same cyclic prefix length is used in active DL BWP and active UL BWP within a serving cell. If RedCap-specific initial DL BWP is configured, the network ensures the same cyclic prefix length is configured for initial DL BWP and RedCap-specific initial DL BWP.

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL / FrequencyInfoUL-SIB / FrequencyInfoDL-SIB within ServingCellConfigCommon / ServingCellConfigCommonSIB) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz For the initial DL BWP and operation in licensed spectrum this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell. Except for SUL, the network ensures the same subcarrier spacing is used in active DL BWP and active UL BWP within a serving cell. For the initial DL BWP and operation with shared spectrum channel access, the value of this field corresponds to the subcarrier spacing of the SSB associated to the initial DL BWP. If RedCap-specific initial DL BWP is configured, the network ensures the same subcarrier spacing is configured for initial DL BWP and RedCap-specific initial DL BWP.

#### –	BWP-Downlink

The IE BWP-Downlink is used to configure an additional downlink bandwidth part (not for the initial BWP).

BWP-Downlink information element

-- ASN1START

-- TAG-BWP-DOWNLINK-START

BWP-Downlink ::=                    SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-DownlinkCommon                                         OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-DownlinkDedicated                                      OPTIONAL,   -- Cond SetupOtherBWP

...

}

-- TAG-BWP-DOWNLINK-STOP

-- ASN1STOP

BWP-Downlink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The network configures the BWPs with consecutive IDs from 1. The Network does not include the value 0, since value 0 is reserved for the initial BWP.

| Conditional Presence | Explanation |
| --- | --- |
| SetupOtherBWP | The field is mandatory present upon configuration of a new DL BWP. The field is optionally present, Need M, otherwise. |

#### –	BWP-DownlinkCommon

The IE BWP-DownlinkCommon is used to configure the common parameters of a downlink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-DownlinkCommon information element

-- ASN1START

-- TAG-BWP-DOWNLINKCOMMON-START

BWP-DownlinkCommon ::=              SEQUENCE {

genericParameters                   BWP,

pdcch-ConfigCommon                  SetupRelease { PDCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pdsch-ConfigCommon                  SetupRelease { PDSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...

}

-- TAG-BWP-DOWNLINKCOMMON-STOP

-- ASN1STOP

BWP-DownlinkCommon field descriptions

pdcch-ConfigCommon Cell specific parameters for the PDCCH of this BWP. This field is absent for a dormant BWP.

pdsch-ConfigCommon Cell specific parameters for the PDSCH of this BWP.

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                     OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                       OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                        OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16                                        OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16                                       OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16                               OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoveryRSConfig-r16}                    OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                                     OPTIONAL    -- Need M

]],

[[

preConfGapStatus-r17                BIT STRING (SIZE (maxNrofGapId-r17))                              OPTIONAL,   -- Cond PreConfigMG

beamFailureRecoverySpCellConfig-r17 SetupRelease { BeamFailureRecoveryRSConfig-r16}                   OPTIONAL,   -- Cond SpCellOnly

harq-FeedbackEnablingforSPSactive-r17 BOOLEAN                                                         OPTIONAL,   -- Need R

cfr-ConfigMulticast-r17             SetupRelease { CFR-ConfigMulticast-r17 }                          OPTIONAL,   -- Need M

dl-PPW-PreConfigToAddModList-r17    DL-PPW-PreConfigToAddModList-r17                                  OPTIONAL,   -- Need N

dl-PPW-PreConfigToReleaseList-r17   DL-PPW-PreConfigToReleaseList-r17                                 OPTIONAL,   -- Need N

nonCellDefiningSSB-r17              NonCellDefiningSSB-r17                                            OPTIONAL,   -- Need R

servingCellMO-r17                   MeasObjectId                                                  OPTIONAL -- Cond MeasObject-NCD-SSB

]],

[[

tci-InDCI-r18                       SetupRelease {TCI-InDCI-r18}                                      OPTIONAL    -- Need M

]],

[[

sbfd-Config2-Reception-r19          ENUMERATED {enabled}                                              OPTIONAL,   -- Need S

pathlossOffsetPRACH-DCI-1-0-r19     ENUMERATED { enabled }                                            OPTIONAL,   -- Need R

prachAssociationDCI-1-0-r19         ENUMERATED { enabled }                                            OPTIONAL    -- Need R

]]

}

SPS-ConfigToAddModList-r16 ::=          SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config

SPS-ConfigToReleaseList-r16 ::=         SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationState-r16 ::=     SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationStateList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-DeactivationState)) OF SPS-ConfigDeactivationState-r16

DL-PPW-PreConfigToAddModList-r17 ::=    SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-PreConfig-r17

DL-PPW-PreConfigToReleaseList-r17 ::=   SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-ID-r17

TCI-InDCI-r18 ::=                   SEQUENCE {

tci-SelectionPresentInDCI-r18       ENUMERATED { enabled }                                            OPTIONAL,   -- Need R

applyIndicatedTCI-StateDCI-1-0-r18  ENUMERATED {first, second, both, spare1}                          OPTIONAL    -- Need R

}

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

BWP-DownlinkDedicated field descriptions

beamFailureRecoverySCellConfig Configuration of candidate RS for beam failure recovery on SCells.

beamFailureRecoverySpCellConfig Configuration of candidate RS for beam failure recovery on the SpCell. This field can only be configured when beamFailure-r17 is configured in RadioLinkMonitoringConfig.

cfr-ConfigMulticast UE specific common frequency resource configuration for MBS multicast for one dedicated BWP. This field can be configured within at most one serving cell.

dl-PPW-PreConfigToAddModList Indicates a list of DL-PRS processing window configurations to be added or modified for the dedicated DL BWP.

dl-PPW-PreConfigToReleaseList Indicates a list of DL-PRS processing window configurations to be released for the dedicated DL BWP.

harq-FeedbackEnablingforSPSactive If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled for the HARQ process corresponding to the first SPS PDSCH after activation. Otherwise, UE follows configuration of HARQ feedback enabled/disabled for the HARQ process corresponding to the first SPS PDSCH after activation.

nonCellDefiningSSB If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

pathlossOffsetPRACH-DCI-1-0 Enables the presence of a 1-bit DCI field in DCI format 1_0 for indicating the pathloss offset for PDCCH-ordered PRACH transmissions. This field can only be configured when at least one TCI state is configured with pathlossOffset (see TS 38.212 [17], clause 7.3.1).

pdcch-Config UE specific PDCCH configuration for one BWP.

pdsch-Config UE specific PDSCH configuration for one BWP.

prachAssociationDCI-1-0 Enables the presence of 1-bit DCI field "PRACH association indicator" in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1). This field can only be configured if twoTA-Without-MultiDCI-MultiTRP is configured.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated upon the switch to this BWP. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

sbfd-Config2-Reception Indicates that the PDSCH receptions can be in SBFD symbols and non-SBFD symbols in different slots in a given DL BWP, as specified in TS 38.214 [19], clause 5. If not enabled, the restriction that the receptions are restricted to SBFD symbols only or non-SBFD symbols only is applied for PDSCH receptions in the given DL BWP.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and nonCellDefiningSSB in BWP-DownlinkDedicated of the associated downlink BWP: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB in the nonCellDefiningSSB. If the field is present in a downlink BWP and the BWP is activated, the UE uses this measurement object for serving cell measurements (e.g., including those used in measurement report triggering events), otherwise, the UE uses the servingCellMO in ServingCellConfig IE.

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList. Network does not configure SPS in one BWP using this field and sps-ConfigMulticastToAddModList-r17 simultaneously.

sps-ConfigDeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple SPS configurations to be deactivated, see clause 10.2 in TS 38.213 [13]. If a state is mapped to multiple SPS configurations, each of these SPS configurations is configured with the same harq-CodebookID.

sps-ConfigToAddModList Indicates a list of one or more DL SPS configurations to be added or modified in one BWP. Except for reconfiguration with sync, the NW does not reconfigure a SPS configuration when it is active (see TS 38.321 [3]).

sps-ConfigToReleaseList Indicates a list of one or more DL SPS configurations to be released. The NW may release a SPS configuration at any time.

radioLinkMonitoringConfig UE specific configuration of radio link monitoring for detecting cell- and beam radio link failure occasions. The maximum number of failure detection resources should be limited up to 8 for both cell and beam radio link failure detection. For SCells, only periodic 1-port CSI-RS can be configured in IE RadioLinkMonitoringConfig.

sl-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving the SL grants (via SL-RNTI or SL-CS-RNTI) for NR sidelink communication/discovery.

sl-V2X-PDCCH-Config Indicates the UE specific PDCCH configurations for receiving SL grants (i.e. sidelink SPS) for V2X sidelink communication.

TCI-InDCI field descriptions

applyIndicatedTCI-StateDCI-1-0 This field indicates, for PDSCH reception scheduled or activated by DCI format 1_0, if the UE applies the first, the second or both "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

tci-SelectionPresentInDCI Used as specified in TS 38.214 [19] clause 5.1.5 and TS 38.212 [17] clauses 7.3.1.2.2 and 7.3.1.2.3.

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject-NCD-SSB | This field is optionally present, Need S, if nonCellDefiningSSB is configured in this DL BWP. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the BWP belongs to and configured with preConfigInd. It is absent, Need R, otherwise. |
| ScellOnly | The field is optionally present, Need M, in the BWP-DownlinkDedicated of an Scell. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need M, in the BWP-DownlinkDedicated of an Spcell. It is absent otherwise. |

#### –	BWP-Id

The IE BWP-Id is used to refer to Bandwidth Parts (BWP). The initial BWP (including RedCap-specific initial BWP, if configured) is referred to by BWP-Id 0. The other BWPs are referred to by BWP-Id 1 to maxNrofBWPs.

BWP-Id information element

-- ASN1START

-- TAG-BWP-ID-START

BWP-Id ::=                          INTEGER (0..maxNrofBWPs)

-- TAG-BWP-ID-STOP

-- ASN1STOP

#### –	BWP-Uplink

The IE BWP-Uplink is used to configure an additional uplink bandwidth part (not for the initial BWP).

BWP-Uplink information element

-- ASN1START

-- TAG-BWP-UPLINK-START

BWP-Uplink ::=                      SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-UplinkCommon                                            OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-UplinkDedicated                                         OPTIONAL,   -- Cond SetupOtherBWP

...

}

-- TAG-BWP-UPLINK-STOP

-- ASN1STOP

BWP-Uplink field descriptions

bwp-Id An identifier for this bandwidth part. Other parts of the RRC configuration use the BWP-Id to associate themselves with a particular bandwidth part. The network configures the BWPs with consecutive IDs from 1. The Network does not include the value 0, since value 0 is reserved for the initial BWP.

| Conditional Presence | Explanation |
| --- | --- |
| SetupOtherBWP | The field is mandatory present upon configuration of a new UL BWP. The field is optionally present, Need M, otherwise. |

#### –	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell, excluding additionalRACH-perPCI-ToAddModList and additionalRACH-perPCI-ToReleaseList, are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]],

[[

preambleTransMaxRO-Type-r19                  ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}  OPTIONAL,  -- Need R

sbfd-RO-Type-r19                             ENUMERATED {sbfd, non-sbfd}                                 OPTIONAL,  -- Need S

sbfd-RSRP-ThresholdRO-Type-r19               RSRP-Range                                                  OPTIONAL,  -- Need S

sbfd-RSRP-ThresholdRO-TypeUsage-r19          ENUMERATED {above,below}                                    OPTIONAL,  -- Need S

sbfd-RSRP-ThresholdMsg1-RepetitionNum2-r19   RSRP-Range                                                  OPTIONAL,  -- Need R

sbfd-RSRP-ThresholdMsg1-RepetitionNum4-r19   RSRP-Range                                                  OPTIONAL,  -- Need R

sbfd-RSRP-ThresholdMsg1-RepetitionNum8-r19   RSRP-Range                                                  OPTIONAL,  -- Need R

sbfd-RACH-Config-r19                         CHOICE {

sbfd-RACH-SingleConfig-r19                   NULL,

sbfd-RACH-DualConfig-r19                     SBFD-RACH-DualConfig-r19

}                                                                                                        OPTIONAL  -- Need R

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...,

[[

sbfd-RACH-DualConfig-r19            SBFD-RACH-DualConfig-r19                                     OPTIONAL  -- Cond NoSingleConfig

]]

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

SBFD-RACH-DualConfig-r19 ::=                          SEQUENCE {

sbfd-AdditionalRACH-Config-r19                        RACH-ConfigCommon                                  OPTIONAL,  -- Need R

sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes-r19    ENUMERATED {enabled}                               OPTIONAL   -- Need R

}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

BWP-UplinkCommon field descriptions

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

additionalRACH-perPCI-ToAddModList, additionalRACH-perPCI-ToReleaseList List of RACH configurations for the additional PCIs. The RACH configuration for an additional PCI is applied for Random Access procedure initiated by PDCCH order towards to the additional PCI, as specified in TS 38.321 clause 5.1.1b. This list includes the same number of elements like additionalPCI-ToAddModList for this serving cell and the n-th element of this list is for the PCI in the n-th element of additionalPCI-ToAddModList. Either the network does not configure any RACH configuration for any additional PCI, or the network configures a RACH configuration for each additional PCI. When the network releases an additional PCI of a serving cell, the network also explicitly releases the associated random access configuration in every UL BWP of the serving cell. This configuration may be different for different UEs.

enableRA-PrioritizationForSlicing Indicates whether or not the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep should override the ra-PrioritizationForAccessIdentity. The field is applicable only when the UE is configured by upper layers with both NSAG and Access Identity 1 or 2. If value TRUE is configured, the UE should only apply the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep. If value FALSE is configured, the UE should only apply ra-PrioritizationForAccessIdentity. If the field is absent, whether to use ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep or ra-PrioritizationForAccessIdentity is up to UE implementation.

mcs-Msg3-Repetitions Configuration of eight candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. Only the first 4 configured or default MCS indexes are used for PUSCH transmission scheduled by RAR UL grant. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {0, 1, 2, 3, 4, 5, 6, 7} (see TS 38.214 [19], clause 6.1.4).

msgA-ConfigCommon Configuration of the cell specific PRACH and PUSCH resource parameters for transmission of MsgA in 2-step random access type procedure. The NW can configure msgA-ConfigCommon only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP or DL BWPs associated with nonCellDefiningSSB or, for (e)RedCap UEs, the RedCap-specific initial downlink BWP. The network configures msgA-ConfigCommon (without suffix) and/or msgA-ConfigCommon-r17, whenever it configures contention free 2-step random access, the UE then applies the corresponding configuration depending on the RACH resource set selected upon RACH initialization, as specified in TS 38.321 [3].

numberOfMsg3-RepetitionsList The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. This field is only applicable when the UE selects Random Access resources indicating Msg3 repetition in this BWP. If this field is absent when the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon, the UE shall apply the values {n1, n2, n3, n4} (see TS 38.214 [19], clause 6.1.2.1).

preambleTransMax-Msg1-Repetition Max number of transmissions of MSG1 repetitions number (2, 4 and 8) performed before switching to higher repetition number (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when more than one repetition numbers are configured in shared RO. If the field is absent, switching from lower repetition number to higher repetition number is not allowed.

preambleTransMaxRO-Type Max number of RA preamble transmissions performed before switching to another RO type (i.e., from the second PRACH occasions to the first PRACH occasions and vice versa).

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP or DL BWPs associated with nonCellDefiningSSB or, for (e)RedCap UEs, the RedCap-specific initial downlink BWP. The network configures rach-ConfigCommon (without suffix) and/or rach-ConfigCommon-r17, whenever it configures contention free 4-step random access (e.g. for reconfiguration with sync or for beam failure recovery or PDCCH order), the UE then applies the corresponding configuration depending on the RACH resource set selected upon RACH initialization, as specified in TS 38.321 [3]. For RedCap-specific initial uplink BWP, rach-ConfigCommon is always configured when msgA-ConfigCommon is configured in this BWP.

rach-ConfigCommonIAB Configuration of cell specific random access parameters for the IAB-MT. The IAB specific IAB RACH configuration is used by IAB-MT, if configured.

rsrp-ThresholdMsg1-RepetitionNum2, rsrp-ThresholdMsg1-RepetitionNum4, rsrp-ThresholdMsg1-RepetitionNum8 Threshold used by the UE for determining whether to select resources indicating Msg1 repetition number 2, 4 or 8 in this BWP, as specified in TS 38.321 [3]. The value applies to all the BWPs and all RACH configurations. For a given MSG1 repetition number, this corresponding field is mandatory if both set(s) of Random Access resources with MSG1 repetition indication associated with this MSG1 repetition number and set(s) of Random Access resources without MSG1 repetition indication are configured in the BWP, or if the set(s) of Random Access resources with MSG1 repetition indication associated with this MSG1 repetition number and set(s) of Random Access resources with MSG1 repetition indication associated with a lower repetition number are configured in the BWP. It is absent otherwise.

rsrp-ThresholdMsg3 Threshold used by the UE for determining whether to select resources indicating Msg3 repetition in this BWP, as specified in TS 38.321 [3]. The field is mandatory if both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication are configured in the BWP. It is absent otherwise.

sbfd-RACH-Config Indicated SBFD RACH configuration type, see clause 5 in TS 38.211 [16] and clause 8 in TS 38.213 [13]. Value sbfd-RACH-SingleConfig indicates the single RACH configuration for SBFD random access operation is enabled, and value sbfd-RACH-DualConfig is used to configure dual RACH configurations and configure random access parameters in SBFD symbols by setting up one additional RACH configuration and can include all parameters in rach-ConfigCommon except rsrp-ThresholdSSB-SUL. If present and the UE is indicated to use SBFD random access operation for CFRA in the same BWP, the UE shall derive the SBFD RO locations based on this field.

sbfd-RO-Type Indicates which RO type to be used for a SBFD aware UE in the initial PRACH transmission. If absent, the RO type is determined based on sbfd-RSRP-ThresholdRO-Type and sbfd-RSRP-ThresholdRO-TypeUsage.

sbfd-RSRP-ThresholdMsg1-RepetitionNum2, sbfd-RSRP-ThresholdMsg1-RepetitionNum4, sbfd-RSRP-ThresholdMsg1-RepetitionNum8 Threshold used by the UE for determining whether to select resources indicating Msg1 repetition number 2, 4 or 8 within the second PRACH occasions (see TS 38.213 [13], clause 8).

sbfd-RSRP-ThresholdRO-Type Threshold used by the SBFD aware UE for choosing RACH occasion type for the initial PRACH preamble transmissions.

sbfd-RSRP-ThresholdRO-TypeUsage Indicate how the SBFD aware UE chooses RACH occasion type using sbfd-RSRP-ThresholdRO-Type for the initial PRACH preamble transmissions and is always configured together with sbfd-RSRP-ThresholdRO-Type. With value above, the SBFD aware UE chooses the second PRACH occasions if the measured downlink pathloss reference RSRP is above sbfd-RSRP-ThresholdRO-Type and chooses the first PRACH occasions otherwise. With value below, the SBFD aware UE chooses the second PRACH occasions if the measured downlink pathloss reference RSRP is below sbfd-RSRP-ThresholdRO-Type and chooses the first PRACH occasions otherwise. If all of sbfd-RO-Type, sbfd-RSRP-ThresholdRO-Type and sbfd-RSRP-ThresholdRO-TypeUsage are absent, it is up to UE implementation to determine the RO type.

useInterlacePUCCH-PUSCH If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for cell-specific PUSCH, e.g., PUSCH scheduled by RAR UL grant (see TS 38.213 [13] clause 8.3 and TS 38.214 [19], clause 6.1.2.2) and uses interlaced PUCCH Format 0 and 1 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1).

SBFD-RACH-DualConfig field descriptions

sbfd-AdditionalRACH-Config Indicates the configuration of dual RACH configurations by setting up one additional RACH configuration, see RACH configuration for SBFD random access operation in clause 5 in TS 38.211 [16] and clause 8 in TS 38.213 [13].

sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes Indicates whether a configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots is valid for RACH configuration Option 2.

| Conditional Presence | Explanation |
| --- | --- |
| Msg1Rep1 | This field is optionally present, Need R, if the set(s) of Random Access resources with MSG1 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise. |
| Msg3Rep | This field is optionally present, Need S, if the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise. |
| RA-PrioSliceAI | The field is optionally present in SIB1, Need R, if both parameters ra-PrioritizationForAccessIdentity and the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep are present in SIB1. It is absent otherwise. |
| SpCellOnly2 | The field is optionally present, Need M, in the BWP-UplinkCommon of an SpCell. It is absent otherwise. |
| 2TA-Only | The field is optionally present, Need N in the BWP-UplinkCommon if additionalPCI-ToAddModList is present in ServingCellConfig and if tag2 is present in ServingCellConfig. It is absent otherwise. |
| NoSingleConfig | This field is optionally present, Need R, if sbfd-RACH-Config in BWP-UplinkCommon is set to sbfd-RACH-DualConfig. It is absent otherwise. |

#### –	BWP-UplinkDedicated

The IE BWP-UplinkDedicated is used to configure the dedicated (UE specific) parameters of an uplink BWP.

BWP-UplinkDedicated information element

-- ASN1START

-- TAG-BWP-UPLINKDEDICATED-START

BWP-UplinkDedicated ::=             SEQUENCE {

pucch-Config                        SetupRelease { PUCCH-Config }                                           OPTIONAL,   -- Need M

pusch-Config                        SetupRelease { PUSCH-Config }                                           OPTIONAL,   -- Need M

configuredGrantConfig               SetupRelease { ConfiguredGrantConfig }                                  OPTIONAL,   -- Need M

srs-Config                          SetupRelease { SRS-Config }                                             OPTIONAL,   -- Need M

beamFailureRecoveryConfig           SetupRelease { BeamFailureRecoveryConfig }                              OPTIONAL,   -- Cond SpCellOnly

...,

[[

sl-PUCCH-Config-r16                 SetupRelease { PUCCH-Config }                                           OPTIONAL,   -- Need M

cp-ExtensionC2-r16                  INTEGER (1..28)                                                         OPTIONAL,   -- Need R

cp-ExtensionC3-r16                  INTEGER (1..28)                                                         OPTIONAL,   -- Need R

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pucch-ConfigurationList-r16         SetupRelease { PUCCH-ConfigurationList-r16 }                            OPTIONAL,   -- Need M

lbt-FailureRecoveryConfig-r16       SetupRelease { LBT-FailureRecoveryConfig-r16 }                          OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r16                 ConfiguredGrantConfigToAddModList-r16                 OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r16                ConfiguredGrantConfigToReleaseList-r16                OPTIONAL,   -- Need N

configuredGrantConfigType2DeactivationStateList-r16   ConfiguredGrantConfigType2DeactivationStateList-r16   OPTIONAL    -- Need R

]],

[[

ul-TCI-StateList-r17                CHOICE {

explicitlist                        SEQUENCE {

ul-TCI-ToAddModList-r17             SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-State-r17          OPTIONAL,   -- Need N

ul-TCI-ToReleaseList-r17            SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF TCI-UL-StateId-r17        OPTIONAL    -- Need N

},

unifiedTCI-StateRef-r17         ServingCellAndBWP-Id-r17

}                                                                                                           OPTIONAL,  -- Need R

ul-powerControl-r17                Uplink-powerControlId-r17                                                OPTIONAL,  -- Cond NoTCI-PC

pucch-ConfigurationListMulticast1-r17  SetupRelease { PUCCH-ConfigurationList-r16 }                         OPTIONAL,  -- Need M

pucch-ConfigurationListMulticast2-r17  SetupRelease { PUCCH-ConfigurationList-r16 }                         OPTIONAL   -- Need M

]],

[[

pucch-ConfigMulticast1-r17          SetupRelease { PUCCH-Config }                                           OPTIONAL,  -- Need M

pucch-ConfigMulticast2-r17          SetupRelease { PUCCH-Config }                                           OPTIONAL   -- Need M

]],

[[

pathlossReferenceRSToAddModList-r17     SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList-r17    SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL  -- Need N

]],

[[

sbfd-Config2-Transmission-r19       ENUMERATED {enabled}                                                    OPTIONAL,   -- Need S

sbfd-Config2-PUSCH-RB-Offset-r19    INTEGER(0..maxNrofPhysicalResourceBlocks)                               OPTIONAL,   -- Need R

ul-Muting-NonSBFD-Symbol-r19        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need S

twoTA-Without-MultiDCI-MultiTRP-r19  ENUMERATED {enabled}                                                   OPTIONAL   -- Need R

]]

}

ConfiguredGrantConfigToAddModList-r16    ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig

ConfiguredGrantConfigToReleaseList-r16   ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

ConfiguredGrantConfigType2DeactivationState-r16 ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

ConfiguredGrantConfigType2DeactivationStateList-r16  ::=

SEQUENCE (SIZE (1..maxNrofCG-Type2DeactivationState)) OF ConfiguredGrantConfigType2DeactivationState-r16

-- TAG-BWP-UPLINKDEDICATED-STOP

-- ASN1STOP

BWP-UplinkDedicated field descriptions

beamFailureRecoveryConfig Configuration of beam failure recovery. If supplementaryUplink is present, the field is present only in one of the uplink carriers, either UL or SUL.

configuredGrantConfig A Configured-Grant of type1 or type2. It may be configured for UL or SUL but in case of type1 not for both at a time. Except for reconfiguration with sync, the NW does not reconfigure configuredGrantConfig when there is an active configured uplink grant Type 2 (see TS 38.321 [3]). However, the NW may release the configuredGrantConfig at any time. Network can only configure configured grant in one BWP using either this field or configuredGrantConfigToAddModList.

configuredGrantConfigToAddModList Indicates a list of one or more configured grant configurations to be added or modified for one BWP. Except for reconfiguration with sync, the NW does not reconfigure a Type 2 configured grant configuration when it is active (see TS 38.321 [3]). The network configures multiple CG configurations for one BWP with either all configurations or no configuration configured with cg-RetransmissionTimer-r16.

configuredGrantConfigToReleaseList Indicates a list of one or more UL Configured Grant configurations to be released. The NW may release a configured grant configuration at any time.

configuredGrantConfigType2DeactivationStateList Indicates a list of the deactivation states in which each state can be mapped to a single or multiple Configured Grant type 2 configurations to be deactivated when the corresponding deactivation DCI is received, see clause 7.3.1 in TS 38.212 [17] and clause 10.2 in TS 38.213 [13].

cp-ExtensionC2, cp-ExtensionC3 Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

lbt-FailureRecoveryConfig Configures parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

pathlossReferenceRSToAddModList A list of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for path loss estimation for PUSCH, PUCCH and SRS for unified TCI state operation. If unifiedTCI-StateType is not configured for the serving cell, no element in this list is configured.

pucch-Config PUCCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL, the network configures PUCCH only on the BWPs of one of the uplinks (normal UL or SUL). The network configures PUCCH-Config at least on non-initial BWP(s) for SpCell and on all BWP(s) for PUCCH SCell. If supported by the UE, the network may configure at most one additional SCell of a cell group with PUCCH-Config (i.e. PUCCH SCell). If PUCCH cell switching is supported by the UE, the network may configure two TDD serving cells with PUCCH-Config within each PUCCH group. For supporting PUCCH cell switching in the PUCCH group with the SpCell, the TDD SpCell and one TDD SCell shall have PUCCH-Config on their normal UL. For supporting PUCCH cell switching in the PUCCH group with only SCells, two TDD SCells shall have PUCCH-Config on their normal UL. In (NG)EN-DC and NE-DC, the NW configures at most one serving cell per frequency range with PUCCH. In (NG)EN-DC and NE-DC, if two PUCCH groups are configured, the serving cells of the NR PUCCH group in FR2 use the same numerology. For NR-DC, the maximum number of PUCCH groups in each cell group is one, and only the same numerology is supported for the cell group with carriers only in FR2. The NW may configure PUCCH for a BWP when setting up the BWP. The network may also add/remove the pucch-Config in an RRCReconfiguration with reconfigurationWithSync (for SpCell or PUCCH SCell) or with SCell release and add (for PUCCH SCell) to move the PUCCH between the UL and SUL carrier of one serving cell. In other cases, only modifications of a previously configured pucch-Config are allowed. If one (S)UL BWP of a serving cell is configured with PUCCH, all other (S)UL BWPs must be configured with PUCCH, too.

pucch-ConfigurationList PUCCH configurations for two simultaneously constructed HARQ-ACK codebooks (see TS 38.213 [13], clause 9.1). Different PUCCH Resource IDs are configured in different PUCCH-Config within the pucch-ConfigurationList if configured.

pucch-ConfigurationListMulticast1 PUCCH configurations for two simultaneously constructed HARQ-ACK codebooks for MBS multicast (see TS 38.213, clause 9).

pucch-ConfigurationListMulticast2 PUCCH configurations for two simultaneously constructed NACK-only feedback for MBS multicast (see TS 38.213, clause 9).

pusch-Config PUSCH configuration for one BWP of the normal UL or SUL of a serving cell. If the UE is configured with SUL and if it has a PUSCH-Config for both UL and SUL, an UL/SUL indicator field in DCI indicates which of the two to use. See TS 38.212 [17], clause 7.3.1.

pucch-ConfigMulticast1 PUCCH configuration for the HARQ-ACK codebook for MBS multicast when multicast feedback is not configured with a priority value (see TS 38.213 [13], clause 9). If the field is not configured, pucch-Config applies.

pucch-ConfigMulticast2 PUCCH configuration for the NACK-only feedback for MBS multicast when multicast feedback is not configured with a priority value (see TS 38.213 [13], clause 9). If the field is not configured, pucch-Config applies.

sbfd-Config2-Transmission Indicates that the PUCCH and PUSCH transmissions can be in SBFD symbols and non-SBFD symbols in different slots in a given UL BWP (see TS 38.213 [13], clause 11 and TS 38.214 [19], clause 6).If not enabled, the restriction that the transmissions are restricted to SBFD symbols only or non-SBFD symbols only is applied for PUCCH and PUSCH transmissions in the given UL BWP.

sbfd-Config2-PUSCH-RB-Offset Indicates the RB offset to determine the starting PRB for Type 2 configured grant and dynamic grant PUSCH transmissions in SBFD symbols when the PUSCH transmissions can be in SBFD symbols and non-SBFD symbols in different slots in a given UL BWP (see TS 38.214 [19], clause 6).

sl-PUCCH-Config Indicates the UE specific PUCCH configurations used for the HARQ-ACK feedback reporting for NR sidelink communication.

srs-Config Uplink sounding reference signal configuration.

twoTA-Without-MultiDCI-MultiTRP Enables two TA for intra-cell without multi-DCI multi-TRP operation or inter-cell without multi-DCI multi-TRP operation.

ul-Muting-NonSBFD-Symbol Indicates whether a UE may apply UL resource muting in non-SBFD symbols if SBFD symbols are configured for the UE. If configured as ‘enabled’, UL muting is applicable in both SBFD symbols and non-SBFD symbols. Otherwise, UL resource muting is applicable only in SBFD symbols. This parameter does not apply for a UE configured with UL resource muting if SBFD symbols are not configured for the UE. In this case, UL resource muting is applicable in both flexible symbols and UL symbols.

ul-powerControl Configures power control parameters for PUCCH, PUSCH and SRS when UE is configured with unifiedTCI-StateType for this serving cell. For each serving cell, ul-powerControl is either configured in all BWP-UplinkDedicated or it is not configured in any BWP-UplinkDedicated. When unifiedTCI-StateRef in the BWP-UplinkDedicated or in the PDSCH-Config if the unifiedTCI-StateType is set to joint, of a serving cell refers to another serving cell, ul-powerControl is either configured in all BWP-UplinkDedicated of these two serving cells or it is not configured in any BWP-UplinkDedicated of these two serving cells.

ul-TCI-StateList Indicates the applicable UL TCI states for PUCCH, PUSCH and SRS.

ul-TCI-ToAddModList Indicates a list of UL TCI states.

unifiedTCI-StateRef Provides the serving cell and UL BWP where UL TCI states applicable to this UL BWP are defined. The value of unifiedTCI-StateType of current serving cell is the same in the serving cell indicated by unifiedTCI-StateRef.

useInterlacePUCCH-PUSCH If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for PUSCH (see TS 38.213 [13], clause 8.3 and TS 38.214 [19], clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for PUCCH (see TS 38.213 [13], clause 9.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| NoTCI-PC | The field is mandatory present if unifiedTCI-StateType is configured for this serving cell and ul-powerControl is not configured for any UL TCI state or joint TCI state of this serving cell. Otherwise it is absent, Need R |
| SpCellOnly | The field is optionally present, Need M, in the BWP-UplinkDedicated of an SpCell. It is absent otherwise. |

NOTE 1:	In case of RRCReconfiguration with reconfigurationWithSync, the UE performs a MAC reset, which involves releasing the PUCCH-CSI/SRS/SR configuration in accordance with clause 5.3.12 and TS 38.321 [3], clauses 5.12 and 5.2. Hence, for these parts of the dedicated radio resource configuration, delta signalling is not supported in the message when reconfigurationWithSync is included.

#### –	CandidateBeamRS

The IE CandidateBeamRS inlcudes candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.17.

CandidateBeamRS information element

-- ASN1START

-- TAG-CANDIDATEBEAMRS-START

CandidateBeamRS-r16 ::=                SEQUENCE {

candidateBeamConfig-r16                CHOICE {

ssb-r16                                SSB-Index,

csi-RS-r16                             NZP-CSI-RS-ResourceId

},

servingCellId                          ServCellIndex                                                            OPTIONAL  -- Need R

}

-- TAG-CANDIDATEBEAMRS-STOP

-- ASN1STOP

CandidateBeamRS field descriptions

candidateBeamConfig Indicates the resource (i.e. SSB or CSI-RS) defining this beam resource.

servingCellId If the field is absent, the RS belongs to the serving cell in which BeamFailureRecoveryRSConfig is configured.

#### –	CandidateTCI-State

The IE CandidateTCI-State defines a TCI states configuration which associate one or more reference signal with a corresponding quasi-colocation (QCL) type.

CandidateTCI-State information element

-- ASN1START

-- TAG-CANDIDATETCI-STATE-START

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                                                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17                                          OPTIONAL,   -- Cond Joint

tag-Id-ptr-r18                       ENUMERATED {n0,n1}                                                  OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                  Uplink-powerControlId-r17                                           OPTIONAL,   -- Cond Joint2

...

}

LTM-QCL-Info-r18 ::=                 SEQUENCE {

referenceSignal-r18                  CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

qcl-Type-r18                         ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-CANDIDATETCI-STATE-STOP

-- ASN1STOP

CandidateTCI-State field descriptions

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the candidate that includes this CandidateTCI-State and it refers to one of the PathlossReferenceRS configured within LTM-TCI-Info. In this version of the specification only SSB can be included as reference signal when PathlossReferenceRS is included within a CandidateTCI-State IE.

qcl-Type1, qcl-Type2 QCL information for the TCI state.

tci-StateId The ID number of the TCI state.

ul-PowerControl Indicates the UL power control parameters for PUSCH, PUCCH, and SRS of the candidate that includes this CandidateTCI-State. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the SpCellConfig of the ltm-CandidateConfig in the LTM-Candidate where the ltm-TCI-Info is configured.

LTM-QCL-Info field descriptions

referenceSignal The field csi-RS-Index refers to one of the NZP-CSI-RS-Resource configured within LTM-TCI-Info.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig and unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, Need R, otherwise. |
| Joint | This field is mandatory present, if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint. It is absent, Need R, otherwise. |
| Joint2 | This field is mandatory present if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to joint and ul-powerControl is not configured in any BWP-Uplink-Dedicated of the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |

#### –	CandidateTCI-UL-State

The IE CandidateTCI-UL-State defines an uplink TCI states configuration.

CandidateTCI-UL-State information element

-- ASN1START

-- TAG-CANDIDATETCI-UL-STATE-START

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

tag-Id-ptr-r18                          ENUMERATED {n0,n1}                                           OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                     Uplink-powerControlId-r17                                    OPTIONAL,   -- Cond separateTCI

...

}

-- TAG-CANDIDATETCI-UL-STATE-STOP

-- ASN1STOP

CandidateTCI-UL-State field descriptions

csi-RS-Index Indicates an NZP-CSI-RS-Resource of the LTM candidate that includes this CandidateTCI-UL-State.

pathlossReferenceRS-Id Indicates a PathlossReferenceRS of the LTM candidate that includes this CandidateTCI-UL-State and it refers to one of the PathlossReferenceRS configured within LTM-TCI-Info. In this version of the specification, only SSB can be included as reference signal when PathlossReferenceRS is included within a CandidateTCI-UL-State IE.

referenceSignal Reference signal with which spatial relation information is provided. The field csi-RS-Index refers to one of the NZP-CSI-RS-Resource configured within LTM-TCI-Info.

ssb-Index The index of a SSB/PBCH block as indicated in ltm-SSB-Config of the LTM candidate that includes this CandidateTCI-UL-State.

tci-UL-StateID The ID number of the uplink TCI state.

ul-PowerControl Indicates the UL power control parameters for PUSCH, PUCCH, and SRS of the candidate that includes this CandidateTCI-UL-State. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the SpCellConfig of the ltm-CandidateConfig in the LTM-Candidate where the ltm-TCI-Info is configured.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |
| separateTCI | This field is mandatory present if unifiedTCI-StateType in the ltm-TCI-Info within LTM-Candidate is set to separate and ul-powerControl is not configured in any BWP-Uplink-Dedicated of the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |

#### –	CellAccessRelatedInfo

The IE CellAccessRelatedInfo indicates cell access related information for this cell.

CellAccessRelatedInfo information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFO-START

CellAccessRelatedInfo   ::=         SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}             OPTIONAL,   -- Need R

...,

[[

cellReservedForFutureUse-r16        ENUMERATED {true}             OPTIONAL,   -- Need R

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16      OPTIONAL    -- Need R

]],

[[

snpn-AccessInfoList-r17             SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17    OPTIONAL    -- Need R

]]

}

SNPN-AccessInfo-r17 ::=         SEQUENCE {

extCH-Supported-r17                 ENUMERATED {true}             OPTIONAL,   -- Need R

extCH-WithoutConfigAllowed-r17      ENUMERATED {true}             OPTIONAL,   -- Need R

onboardingEnabled-r17               ENUMERATED {true}             OPTIONAL,   -- Need R

imsEmergencySupportForSNPN-r17      ENUMERATED {true}             OPTIONAL    -- Need R

}

-- TAG-CELLACCESSRELATEDINFO-STOP

-- ASN1STOP

CellAccessRelatedInfo field descriptions

cellReservedForFutureUse Indicates whether the cell is reserved, as defined in 38.304 [20] for future use. The field is applicable to all PLMNs and NPNs. This field is ignored by IAB-MT and NCR-MT.

cellReservedForOtherUse Indicates whether the cell is reserved, as defined in 38.304 [20]. The field is applicable to all PLMNs. This field is ignored by IAB-MT and NCR-MT for cell barring determination, but still considered by NPN capable IAB-MT and NPN capable NCR-MT for determination of an NPN-only cell.

npn-IdentityInfoList The npn-IdentityInfoList is used to configure a set of NPN-IdentityInfo elements. Each of those elements contains a list of one or more NPN Identities and additional information associated with those NPNs. The total number of PLMNs (identified by a PLMN identity in plmn -IdentityList), PNI-NPNs (identified by a PLMN identity and a CAG-ID), and SNPNs (identified by a PLMN identity and a NID) together in the PLMN-IdentityInfoList and NPN-IdentityInfoList does not exceed 12, except for the NPN-only cells. A PNI-NPN and SNPN can be included only once, and in only one entry of the NPN-IdentityInfoList. In case of NPN-only cells the PLMN-IdentityList contains a single element that does not count to the limit of 12 and the cellIdentity of the first entry of the PLMN-IdentityInfoList is set to the same value as the cellIdentity-r16 of the first entry of the NPN-IdentityInfoList. The NPN index is defined as B+c1+c2+…+c(n-1)+d1+d2+…+d(m-1)+e(i) for the NPN identity included in the n-th entry of NPN-IdentityInfoList and in the m-th entry of npn-Identitylist within that NPN-IdentityInfoList entry, and the i-th entry of its corresponding NPN-Identity, where - B is the index used for the last PLMN in the PLMN-IdentittyInfoList; in NPN-only cells B is considered 0; - c(j) is the number of NPN index values used in the j-th NPN-IdentityInfoList entry; - d(k) is the number of NPN index values used in the k-th npn-IdentityList entry within the n-th NPN-IdentityInfoList entry; - e(i) is - i if the n-th entry of NPN-IdentityInfoList entry is for SNPN(s); - 1 if the n-th entry of NPN-IdentityInfoList entry is for PNI-NPN(s).

plmn-IdentityInfoList The plmn-IdentityInfoList is used to configure a set of PLMN-IdentityInfo elements. Each of those elements contains a list of one or more PLMN Identities and additional information associated with those PLMNs. A PLMN-identity can be included only once, and in only one entry of the PLMN-IdentityInfoList. The PLMN index is defined as b1+b2+…+b(n-1)+i for the PLMN included at the n-th entry of PLMN-IdentityInfoList and the i-th entry of its corresponding PLMN-IdentityInfo, where b(j) is the number of PLMN-Identity entries in each PLMN-IdentityInfo, respectively.

snpn-AccessInfoList This list provides access related information for each SNPN in npn-IdentityInfoList, see TS 23.501 [32]. The n-th entry of the list contains the access related information of the n-th SNPN in npn-IdentityInfoList.

SNPN-AccessInfo field descriptions

extCH-Supported Indicates whether the SNPN supports access using credentials from a Credentials Holder as specified in TS 23.501 [32].

extCH-WithoutConfigAllowed Indicates whether the SNPN allows registration attempts with credentials from a Credentials Holder from UEs that are not explicitly configured to select the SNPN as specified in TS 23.501 [32].

imsEmergencySupportForSNPN Indicates whether the SNPN supports IMS emergency bearer services for UEs in limited service mode in the cell. If absent, IMS emergency call is not supported by the SNPN in the cell for UEs in limited service mode.

onboardingEnabled Indicates whether the onboarding SNPN allows registration for onboarding in the cell as specified in TS 23.501 [32].

#### –	CellAccessRelatedInfo-EUTRA-5GC

The IE CellAccessRelatedInfo-EUTRA-5GC indicates cell access related information for an LTE cell connected to 5GC.

CellAccessRelatedInfo-EUTRA-5GC information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFOEUTRA-5GC-START

CellAccessRelatedInfo-EUTRA-5GC  ::=    SEQUENCE {

plmn-IdentityList-eutra-5gc             PLMN-IdentityList-EUTRA-5GC,

trackingAreaCode-eutra-5gc              TrackingAreaCode,

ranac-5gc                               RAN-AreaCode                                OPTIONAL,

cellIdentity-eutra-5gc                  CellIdentity-EUTRA-5GC

}

PLMN-IdentityList-EUTRA-5GC::=          SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity-EUTRA-5GC

PLMN-Identity-EUTRA-5GC ::=             CHOICE {

plmn-Identity-EUTRA-5GC                 PLMN-Identity,

plmn-index                              INTEGER (1..maxPLMN)

}

CellIdentity-EUTRA-5GC ::=              CHOICE {

cellIdentity-EUTRA                      BIT STRING (SIZE (28)),

cellId-index                            INTEGER (1..maxPLMN)

}

-- TAG-CELLACCESSRELATEDINFOEUTRA-5GC-STOP

-- ASN1STOP

#### –	CellAccessRelatedInfo-EUTRA-EPC

The IE CellAccessRelatedInfo-EUTRA-EPC indicates cell access related information for an LTE cell connected to EPC.

CellAccessRelatedInfo-EUTRA-EPC information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFOEUTRA-EPC-START

CellAccessRelatedInfo-EUTRA-EPC  ::=    SEQUENCE {

plmn-IdentityList-eutra-epc             PLMN-IdentityList-EUTRA-EPC,

trackingAreaCode-eutra-epc              BIT STRING (SIZE (16)),

cellIdentity-eutra-epc                  BIT STRING (SIZE (28))

}

PLMN-IdentityList-EUTRA-EPC::=          SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity

-- TAG-CELLACCESSRELATEDINFOEUTRA-EPC-STOP

-- ASN1STOP

#### –	CellDTX-DRX-Config

The IE CellDTX-DRX-Config is used to configure cell DTX/DRX related parameters.

CellDTX-DRX-Config information element

-- ASN1START

-- TAG-CELLDTX-DRX-CONFIG-START

CellDTX-DRX-Config-r18 ::=             SEQUENCE {

cellDTX-DRX-onDurationTimer-r18        CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

cellDTX-DRX-CycleStartOffset-r18       CHOICE {

ms10                                   INTEGER(0..9),

ms20                                   INTEGER(0..19),

ms32                                   INTEGER(0..31),

ms40                                   INTEGER(0..39),

ms60                                   INTEGER(0..59),

ms64                                   INTEGER(0..63),

ms70                                   INTEGER(0..69),

ms80                                   INTEGER(0..79),

ms128                                  INTEGER(0..127),

ms160                                  INTEGER(0..159),

ms256                                  INTEGER(0..255),

ms320                                  INTEGER(0..319),

ms512                                  INTEGER(0..511),

ms640                                  INTEGER(0..639),

ms1024                                 INTEGER(0..1023),

ms1280                                 INTEGER(0..1279),

ms2048                                 INTEGER(0..2047),

ms2560                                 INTEGER(0..2559),

ms5120                                 INTEGER(0..5119),

ms10240                                INTEGER(0..10239)

},

cellDTX-DRX-SlotOffset-r18                 INTEGER (0..31),

cellDTX-DRX-ConfigType-r18                 ENUMERATED {dtx, drx, dtxdrx},

cellDTX-DRX-ActivationStatus-r18           ENUMERATED {activated, deactivated}           OPTIONAL   -- Need N

}

-- TAG-CELLDTX-DRX-CONFIG-STOP

-- ASN1STOP

CellDTX-DRX-Config field descriptions

cellDTX-DRX-CycleStartOffset Indicates cellDTX-DRX-Cycle in ms and cellDTX-DRX-StartOffset in multiples of 1 ms. The configured cellDTX-DRX-Cycle is an integer multiple of configured drx-longCycle or vice versa.

cellDTX-DRX-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

cellDTX-DRX-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

cellDTX-DRX-ActivationStatus Initial activation status of cell DTX/DRX indicating whether the UE shall activate the configuration according to the received parameters. This field is only used upon setup of a cell DTX/DRX configuration.

cellDTX-DRX-ConfigType Indicates whether the configuration is for cell DTX only, cell DRX only, or joint cell DTX/DRX configuration.

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]],

[[

f1c-TransferPathNRDC-r17                   ENUMERATED {mcg, scg, both}                                             OPTIONAL,   -- Need M

uplinkTxSwitching-2T-Mode-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Cond 2Tx

uplinkTxSwitching-DualUL-TxState-r17       ENUMERATED {oneT, twoT}                                                 OPTIONAL,   -- Cond 2Tx

uu-RelayRLC-ChannelToAddModList-r17        SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17

OPTIONAL,   -- Need N

uu-RelayRLC-ChannelToReleaseList-r17       SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelID-r17

OPTIONAL,   -- Need N

simultaneousU-TCI-UpdateList1-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList2-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList3-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList4-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

rlc-BearerToReleaseListExt-r17             SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentityExt-r17           OPTIONAL,   -- Need N

iab-ResourceConfigToAddModList-r17  SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfig-r17   OPTIONAL, -- Need N

iab-ResourceConfigToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfigID-r17 OPTIONAL  -- Need N

]],

[[

reportUplinkTxDirectCurrentMoreCarrier-r17 ReportUplinkTxDirectCurrentMoreCarrier-r17                            OPTIONAL  -- Need N

]],

[[

prioSCellPRACH-OverSP-PeriodicSRS-r17      ENUMERATED {enabled}                                                  OPTIONAL  -- Need R

]],

[[

ncr-FwdConfig-r18                          SetupRelease { NCR-FwdConfig-r18 }                                 OPTIONAL,  -- Cond NCR

autonomousDenialParameters-r18             SetupRelease {AutonomousDenialParameters-r18}                      OPTIONAL,   -- Need M

nonCollocatedTypeMRDC-r18                  ENUMERATED { true }                                                OPTIONAL,   -- Need R

nonCollocatedTypeNR-CA-r18                 ENUMERATED { true }                                                OPTIONAL,   -- Need R

uplinkTxSwitchingMoreBands-r18             SetupRelease { UplinkTxSwitchingMoreBands-r18 }                    OPTIONAL    -- Need M

]],

[[

uplinkTxSwitching3Tx-r19                   ENUMERATED { true }                                                OPTIONAL,  -- Cond 3Tx

lowBandCA-Switching-r19                    SetupRelease { LowBandCA-Switching-r19 }                           OPTIONAL,    -- Need M

nonCollocatedTypeMRDC-v1900                ENUMERATED { type1, type4 }                                        OPTIONAL,    -- Need R

nonCollocatedTypeNR-CA-v1900               ENUMERATED { type1, type4 }                                        OPTIONAL,    -- Need R

mprReductionExtensionRatio-r19    ENUMERATED {ratio1, ratio2, ratio3, ratio4, ratio5, ratio6, spare2, spare1} OPTIONAL     -- Need R

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                               OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

deactivatedSCG-Config-r17           SetupRelease { DeactivatedSCG-Config-r17 }                  OPTIONAL    -- Cond SCG-Opt

]]

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]],

[[

rach-LessHO-r18                 RACH-LessHO-r18                                                 OPTIONAL,   -- Need N

sl-IndirectPathMaintain-r18     ENUMERATED{true}                                                OPTIONAL     -- Cond MP

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]],

[[

od-ssb-r19                      SetupRelease {OD-SSB-r19}                                       OPTIONAL,   -- Need M

adapt-SSB-Config-r19            SetupRelease {Adapt-SSB-Config-r19}                             OPTIONAL    -- Need M

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

DeactivatedSCG-Config-r17 ::=       SEQUENCE {

bfd-and-RLM-r17                     BOOLEAN,

...

}

GoodServingCellEvaluation-r17 ::=       SEQUENCE {

offset-r17                              ENUMERATED {db2, db4, db6, db8}                         OPTIONAL   -- Need S

}

SL-PathSwitchConfig-r17 ::=         SEQUENCE {

targetRelayUE-Identity-r17          SL-SourceIdentity-r17,

t420-r17                            ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

...

}

IAB-ResourceConfig-r17 ::=          SEQUENCE {

iab-ResourceConfigID-r17            IAB-ResourceConfigID-r17,

slotList-r17                        SEQUENCE (SIZE (1..5120)) OF INTEGER (0..5119)                           OPTIONAL,    -- Need M

periodicitySlotList-r17             ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160}     OPTIONAL,    -- Need M

slotListSubcarrierSpacing-r17       SubcarrierSpacing                                                        OPTIONAL,    -- Need M

...

}

IAB-ResourceConfigID-r17 ::=        INTEGER(0..maxNrofIABResourceConfig-1-r17)

ReportUplinkTxDirectCurrentMoreCarrier-r17 ::= SEQUENCE (SIZE(1.. maxSimultaneousBands)) OF IntraBandCC-CombinationReqList-r17

IntraBandCC-CombinationReqList-r17::=   SEQUENCE {

servCellIndexList-r17                   SEQUENCE (SIZE(1.. maxNrofServingCells)) OF ServCellIndex,

cc-CombinationList-r17                  SEQUENCE (SIZE(1.. maxNrofReqComDC-Location-r17)) OF IntraBandCC-Combination-r17

}

IntraBandCC-Combination-r17::=      SEQUENCE (SIZE(1.. maxNrofServingCells)) OF CC-State-r17

CC-State-r17::=                     SEQUENCE {

dlCarrier-r17                       CarrierState-r17                             OPTIONAL, -- Need N

ulCarrier-r17                       CarrierState-r17                             OPTIONAL  -- Need N

}

CarrierState-r17::=                 CHOICE {

deActivated-r17                     NULL,

activeBWP-r17                       INTEGER (0..maxNrofBWPs)

}

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

}

UplinkTxSwitchingMoreBands-r18::=              SEQUENCE {

uplinkTxSwitchingBandList-r18                  SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR OPTIONAL,  -- Need M

uplinkTxSwitchingBandPairList-r18              UplinkTxSwitchingBandPairList-r18                OPTIONAL,   -- Need M

uplinkTxSwitchingAssociatedBandDualUL-List-r18 UplinkTxSwitchingAssociatedBandDualUL-List-r18   OPTIONAL,   -- Need M

...

}

UplinkTxSwitchingBandPairList-r18::=      SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18

UplinkTxSwitchingBandPairConfig-r18::=    SEQUENCE {

bandInfoUL1-r18                           UplinkTxSwitchingBandIndex-r18,

bandInfoUL2-r18                           UplinkTxSwitchingBandIndex-r18,

switchingOptionConfigForBandPair-r18      ENUMERATED {switchedUL, dualUL},

switching2T-Mode-r18                      ENUMERATED {enabled}                                             OPTIONAL,   -- Need S

switchingPeriodConfigForBandPair-r18      ENUMERATED {n35us, n140us}                                       OPTIONAL,   -- Need S

...

}

UplinkTxSwitchingAssociatedBandDualUL-List-r18::= SEQUENCE (SIZE (0..maxSimultaneousBands)) OF UplinkTxSwitchingAssociatedBandDualUL-r18

UplinkTxSwitchingAssociatedBandDualUL-r18::=  SEQUENCE {

transmitBand-r18                              UplinkTxSwitchingBandIndex-r18,

associatedBand-r18                            UplinkTxSwitchingBandIndex-r18

}

UplinkTxSwitchingBandIndex-r18::=  INTEGER (1..maxSimultaneousBands)

LowBandCA-Switching-r19::=                  SEQUENCE  {

switchingPattern-r19                        BIT STRING (SIZE (40))                      OPTIONAL,   -- Need M

gapDurationPCelltoSCell-r19                 INTEGER (1..3)                              OPTIONAL,   -- Need M

gapDurationSCelltoPCell-r19                 INTEGER (1..31)                             OPTIONAL,   -- Need M

...

}

OD-SSB-r19 ::=                      SEQUENCE {

od-ssb-absoluteFrequency-r19        ARFCN-ValueNR                                                     OPTIONAL,  -- Cond ODssbAOssb

od-ssb-SubcarrierSpacing-r19        SubcarrierSpacing                                                 OPTIONAL,  -- Cond ODssbOnly

od-ssb-PBCH-BlockPower-r19          INTEGER (-60..50)                                                 OPTIONAL,  -- Cond ODssbOnly

od-SSB-ConfigToAddModList-r19       SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-Config-r19      OPTIONAL,  -- Need N

od-SSB-ConfigToReleaseList-r19      SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-ConfigId-r19    OPTIONAL   -- Need N

}

Adapt-SSB-Config-r19 ::=                            SEQUENCE {

ssb-BurstPeriodicityList-r19                  SEQUENCE (SIZE (1..2)) OF Adapt-SSB-BurstPeriodicity-r19  OPTIONAL,     -- Need N

posInDCI-ssbPeriodicityIndicationForScell-r19 INTEGER (1..maxDCI-2-9-Size-r18)                         OPTIONAL      -- Need N

}

Adapt-SSB-BurstPeriodicity-r19 ::=  SEQUENCE {

ssb-Periodicity-r19           ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }  OPTIONAL,  -- Need S

ssb-Offset-r19                INTEGER (0..15)                                   OPTIONAL,  -- Need N

ssb-halfFrameIndex-r19        ENUMERATED { zero, one }                               OPTIONAL   -- Need N

}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

Adapt-SSB-Config field descriptions

posInDCI-ssbPeriodicityIndicationForScell The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212 [17], subclause 7.3.1.3.10).

ssb-halfFrameIndex Indicate whether SSB according to the ssb-Periodicity is in the first half or the second half of the frame.

ssb-Offset Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (ssb-Periodicity) = 0. The network configures this field according to ssb-Periodicity such that the indicated system frame does not exceed the corresponding adaptive SSB periodicity.

ssb-Periodicity Additional SSB burst periodicity for the Scell. If the field is absent, the UE applies the value ms5. If configured, the network ensures that SSB transmission occasions with larger periodicity are subset of SSB transmission occasions with shorter periodicity.

AutonomousDenialParamters field descriptions

autonomousDenialSlots Indicates the maximum number of the UL slots for which the UE is allowed to deny any UL transmission. Value n2 corresponds to 2 slots, value n5 to 5 slots and so on.

autonomousDenialValidity Indicates the validity period over which the UL autonomous denial slots shall be counted. Value n200 corresponds to 200 slots, value n500 corresponds to 500 slots and so on.

CC-State field descriptions

dlCarrier Indicates DL carrier activation state for this carrier and the related active BWP Index, if activated.

ulCarrier Indicates UL carrier activation state for this carrier and the related active BWP Index, if activated.

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

f1c-TransferPathNRDC The F1-C transfer path that an NR-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with mcg, IAB-MT can only use the MCG for F1-C transfer. If IAB-MT is configured with scg, IAB-MT can only use the SCG for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select the MCG or the SCG for F1-C transfer.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

mprReductionExtensionRatio Indicates the pair of lower-sided and higher-sided extension ratio as specified in TS 38.101-1 [15]. The ratio1 represents (1/2, 0), the ratio2 represents (0, 1/2), ratio3 represents (1/2, 1/2), ratio4 represents (1/4, 0), ratio5 represents (0, 1/4), ratio6 represents (1/4, 1/4). The first/leftmost value of the pair represents the lower-sided extension ratio, the second value of the pair represents the higher-sided extension ratio. The network can only configure this field in case of non-CA in NR SA.

ncr-FwdConfig Configuration of side control information for the NCR-Fwd access link.

nonCollocatedTypeMRDC The field nonCollocatedTypeMRDC (without suffix) is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If the field nonCollocatedTypeMRDC (without suffix) is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements (i.e. Type 1 UE requirement). If the field nonCollocatedTypeMRDC (without suffix) is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements (i.e. Type 2 UE requirement) if the UE is configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells and indicates the support of interBandMRDC-WithOverlapDL-Bands-r16. The field nonCollocatedTypeMRDC-v1900 is only present for a UE configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, or with maxMIMO-Layers with value equal to 2 for the serving cell(s) of the corresponding EUTRA band and 4 for the serving cell of the corresponding NR band. If nonCollocatedTypeMRDC-v1900 is present with type4, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS38.133 [14] and inter-band RF requirements including clause 7.10B in TS38.101-3 [34] (i.e. Type 4 UE requirement). If nonCollocatedTypeMRDC-v1900 is present with type1 or absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 of in TS38.133 [14] and inter-band RF requirements except clause 7.10B in TS38.101-3 [34] (i.e. Type 1 UE requirement) if the UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, or with maxMIMO-Layers with value equal to 2 for the serving cell(s) of the corresponding EUTRA band and 4 for the serving cell of the corresponding NR band.

nonCollocatedTypeNR-CA The field nonCollocatedTypeNR-CA (without suffix) is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If the field nonCollocatedTypeNR-CA (without suffix) is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15] (i.e. Type 1 UE requirement). If the field nonCollocatedTypeNR-CA (without suffix) is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] (i.e. Type 2 UE requirement) if the UE is configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells and indicates the support of intraBandNR-CA-non-collocated-r18. The field nonCollocatedTypeNR-CA-v1900 is only present for a UE configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells. If nonCollocatedTypeNR-CA-v1900 is present with type4, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/7.6.4-2 in TS38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS38.101-1 [15] (i.e. Type 4 UE requirement) when indicating support of intraBandNR-CA-non-collocated-r19. If nonCollocatedTypeNR-CA-v1900 is present with type1 or absent, the UE applies MRTD requirements according to Table 7.6.4-1 in TS38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS38.101-1 [15] (i.e. Type 1 UE requirement) if the UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells and indicates support of intraBandNR-CA-non-collocated-r19.

npn-IdentityInfoList This field is used to transfer npn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to SNPN Identity. If this field and plmn-IdentityInfoList are both absent, the UE uses the npn-IdentityInfoList in SIB1 of the PCell.

plmn-IdentityInfoList This field is used to transfer plmn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to PLMN Identity. If this field and npn-IdentityInfoList are both absent, the UE uses the plmn-IdentityInfoList in SIB1 of the PCell.

prioSCellPRACH-OverSP-PeriodicSRS When configured, the UE applies UL power control prioritization by prioritizing PRACH transmission on SCell over semi-persistent and/or periodic SRS transmission as defined in clause 7.5 of TS 38.213 [13].

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentMoreCarrier Enables reporting of uplink Direct Current location information when the UE is configured with intra-band CA. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. The UE only reports the uplink Direct Current location information that are related to the indicated cc-CombinationList. The network does not include carriers which locate in DL only spectrum described in TS 38.101-2 [39], clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3 in FR2 in the IntraBandCC-CombinationReqList. I.e. DL-only carrier in FR2 frequency spectrum is not used to calculate the default DC location.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlc-BearerToReleaseListExt List of the RLC entities and the corresponding MAC Logical Channels to be released for multicast MRBs.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14]. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellSIB20 This field is used to transfer SIB20 of the SCell in order to allow the UE for MBS broadcast reception on SCell. The network configures this field only for a single SCell at a time.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE, the Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States or the Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States apply simultaneously, as specified in TS 38.321 [3] clauses 6.1.3.47, 6.1.3.70 and 6.1.3.71, respectively. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitching3Tx Indicates Tx switching enhancement between 2 configured UL bands for 3Tx UEs is configured for inter-band UL CA, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx or 2Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19].

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching. This field is not applicable for a UE configured with uplinkTxSwitching3Tx.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

uplinkTxSwitching-2T-Mode Indicates 2Tx-2Tx switching mode is configured for inter-band UL CA or SUL, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19]. In this case, there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured. This field is not applicable for a UE configured with uplinkTxSwitching3Tx.

uplinkTxSwitching-DualUL-TxState Indicates the state of Tx chains if the state of Tx chains after the UL Tx switching is not unique (as specified in TS 38.214 [19]) in case of 2Tx-2Tx switching is configured and uplinkTxSwitchingOption is set to dualUL. Value oneT indicates 1Tx is assumed to be supported on the carriers on each band, value twoT indicates 2Tx is assumed to be supported on that carrier. This field applies for all band pairs if uplinkTxSwitchingMoreBands is configured. This field is not applicable for a UE configured with uplinkTxSwitching3Tx.

uplinkTxSwitchingMoreBands Indicates UL band list, band pair list and other configurations for ULTx switching. This field is not applicable for a UE configured with uplinkTxSwitching3Tx.

uu-RelayRLC-ChannelToAddModList List of the Uu RLC entities and the corresponding MAC Logical Channels to be added or modified.

uu-RelayRLC-ChannelToReleaseList List of the Uu RLC entities and the corresponding MAC Logical Channels to be released.

DeactivatedSCG-Config field descriptions

bfd-and-RLM If the field is set to true, the UE shall perform RLM and BFD on the PSCell when the SCG is deactivated and the network ensures that beamFailure-r17 is not configured in the radioLinkMonitoringConfig of the DL BWP of the PSCell in which the UE performs BFD. If set to false, the UE is not required to perform RLM and BFD on the PSCell when the SCG is deactivated.

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover.

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover.

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).

GoodServingCellEvaluation field descriptions

offset The parameter "X" (dB) for the good serving cell quality criterion in RRC_CONNECTED, for a cell operating in FR1 and FR2, respectively. If this field is absent, the UE applies the (default) value of 0 dB for "X".

IAB-ResourceConfig field descriptions

iab-ResourceConfigID This ID is used to indicate the specific resource configuration addressed by the MAC CEs specified in TS 38.321 [3].

periodicitySlotList Indicates the periodicity in ms of the list of slot indexes indicated in slotList.

slotList Indicates the list of slot indexes to which the information indicated in the specific MAC CE applies to, as specified in TS 38.321 [3]. The values of the entries in the slotList are strictly less than the value of the periodicitySlotList.

slotListSubcarrierSpacing Subcarrier spacing used as reference for the slotList configuration. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120 or 480 kHz

LowBandCA-Switching field descriptions

gapDurationPCelltoSCell Indicates the duration of the switching gap for the switching from PCell to the SCell, in the unit of 15KHz SCS symbol. NW ensures that the switching gap is enough to cover at least the switching period.

gapDurationSCelltoPCell Indicates the duration of switching gap for the switching from the SCell to PCell, in the unit of 15KHz SCS symbol. NW ensures that the switching gap is enough to cover at least the switching period and TA.

switchingPattern Indicates which carrier is configured for the corresponding slot for low NR band carrier aggregation via switching between FDD and supplementary downlink (SDL) carrier. ‘0’ indicates FDD carrier (PCell), and ‘1’ indicates SDL carrier (SCell). The pattern starts from the beginning of SFN 0 of PCell.

OD-SSB field descriptions

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB. If the field is absent, the UE applies absoluteFrequencySSB indicated in FrequencyInfoDL for this serving cell, if configured. Additional restrictions as described in subclause 4.4. of TS 38.213.

od-ssb-ConfigToAddModList List of OD-SSB configurations for this serving cell. Network configures maximum of one different OD-SSB frequency for the OD-SSB configurations which has than the serving cell SSB frequency.

od-ssb-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7. If the field is absent, UE applies the value PBCH-BlockPower configured in IE ServingCellConfigCommon.

od-ssb-SubcarrierSpacing Indicates subcarrier spacing of OD-SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz If the field is absent, the UE applies the value subcarrierSpacing configured in IE BWP.

RACH-LessHO field descriptions

ssb-Index This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission, see TS 38.321 [3]. The network configures this field when cg-RRC-Configuration is not configured for the initial uplink transmission in RACH-less handover in NTN or in case this cell is not a mobile IAB cell.

targetNTA This field refers to the timing adjustment, see TS 38.213 [13] and TS 38.321 [3], indicating the NTA value which the UE shall use for the target PTAG of handover. The value zero corresponds to NTA=0, while the value source corresponds to the NTA value of the source PTAG indicated by the tag-Id. Only value source is configured by the network in case source cell is a mobile IAB cell. In this version of the specification, the network shall always configure this field if rach-LessHO is part of an RRCReconfiguration message.

tci-StateID This field indicates a beam that the UE should use in the target cell to monitor PDCCH for initial uplink transmission and also indicates the TCI state information to be used in the target cell. The network configures this field in case this cell is not a NTN cell.

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

sl-IndirectPathMaintain Indicates that the L2 U2N Remote UE keeps the PC5 connection with its connected L2 U2N Relay UE.

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon or sets to the same periodicity as ssb-Periodicity-r17 in nonCellDefiningSSB-r17 if the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the first active DL BWP included in this RRC message is configured with nonCellDefiningSSB-r17, this field corresponds to the NCD-SSB indicated by nonCellDefiningSSB-r17, otherwise, this field corresponds to the CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL.

ReportUplinkTxDirectCurrentMoreCarrier field descriptions

IntraBandCC-Combination Indicates the state of the carriers and BWPs indexes of the carriers in a CC combination, each carrier in this combination corresponds to an entry in servCellIndexList with same order. This IE shall have the same size as servCellIndexList.

IntraBandCC-CombinationReqList Indicates the list of the requested carriers/BWPs combinations for an intra-band CA component.

servCellIndexList indicates the list of cell index for an intra-band CA component.

SCellConfig field descriptions

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED. This field is always configured when the network enables BFD relaxation for the UE in this SCell. This field is absent if failureDetectionSetN is present for the SCell.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated while this SCell is deactivated. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the default DRX group shall belong to another Frequency Range. If drx-ConfigSecondaryGroup is configured, the field is optionally present. The network always includes the field if the field was previously configured for this SCell and the SCell remains in the secondary DRX group. Removal of an individual SCell from the secondary DRX group is supported by using an SCell release and addition. Otherwise, if drx-ConfigSecondaryGroup is not configured, the field is absent and the UE shall release the field. The UE shall also release the field if drx-ConfigSecondaryGroup is released without including sCellToAddModList.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent and absoluteFrequencySSB is included, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the SCell is an SSB-less SCell (i.e., the IE absoluteFrequencySSB in ServingCellConfigCommon is absent), this field is absent.

SpCellConfig field descriptions

deactivatedSCG-Config Configuration applicable when the SCG is deactivated. The network always configures this field before or when indicating that the SCG is deactivated in an RRCReconfiguration, RRCResume, E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume message.

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables BFD relaxation for the UE in this SpCell. This field is absent if failureDetectionSetN is present for the SpCell.

goodServingCellEvaluationRLM Indicates the criterion for a UE to detect the good serving cell quality for RLM relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables RLM relaxation for the UE in this SpCell.

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. The t-SearchDeltaP-Connected is the parameter "TSearchDeltaP-Connected". Value s5 means 5 seconds, value s10 means 10 seconds and so on. Low mobility criterion is configured in NR PCell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC.

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

SL-PathSwitchConfig field descriptions

targetRelayUE-Identity Indicates the L2 source ID of the target L2 U2N Relay UE during path switch.

t420 Indicates the timer value of T420 to be used during path switch.

UplinkTxSwitchingMoreBands field descriptions

uplinkTxSwitchingBandList Indicates the NR frequency band number of the UL bands for UL Tx switching. If the UE needs to determine location of switching period as specified in TS 38.101-1 [15], the UE considers that the bands are listed in decreasing order of priority, i.e. the first/leftmost entry corresponds to the band with the highest priority, the next entry corresponds to the band with the second highest priority, and so on. The last entry corresponds the band with the lowest priority.

uplinkTxSwitchingBandPairList Indicates the band pairs involved in UL Tx switching, as well as the per band pair configurations.

uplinkTxSwitchingAssociatedBandDualUL-List Indicates the associated band for the transmitting band indicated by transmitBand which the transmitting carrier(s) is on as specified in TS 38.214 [19], clause 6.1.6. The network ensures that each band pair of a transmitting band and an associated band supports the dualUL switching option.

UplinkTxSwitchingBandIndex The value n indicates the band included at the n-th entry of uplinkTxSwitchingBandList.

UplinkTxSwitchingBandPairConfig field descriptions

bandInfoUL1, bandInfoUL2 Indicates the band index for a band pair. The value n indicates the band included at the n-th entry of uplinkTxSwitchingBandList.

switching2T-Mode Indicates 2Tx-2Tx switching mode is configured to the band pair. If this field is absent when uplink Tx switching is configured, it is interpreted that 1Tx-2Tx/1Tx-1Tx UL Tx switching is configured as specified in TS 38.214 [19].

switchingOptionConfigForBandPair Indicates the switching option for the band pair as specified in TS 38.214 [19], clause 6.1.6.

switchingPeriodConfigForBandPair Indicates the value of switching period for the band pair as specified in TS 38.214 [19], clause 6.1.6. Value n35us represents 35 us, n140us represents 140us. If the field is absent, 210 us is applied.

| Conditional Presence | Explanation |
| --- | --- |
| 2Tx | The field is optionally present, Need R, if uplinkTxSwitching is configured; otherwise it is absent, Need R. |
| 3Tx | The field is optionally present, Need R, if uplinkTxSwitching is configured; otherwise it is absent, Need R. |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DirectToIndirect-PathSwitch | The field is mandatory present for the L2 U2N remote UE at path switch to the target L2 U2N Relay UE (including direct to indirect path switch, single-hop indirect to single-hop indirect path switch, multi-hop indirect to single-hop indirect path switch and direct/single-hop indirect to multi-hop indirect path switch). It is absent otherwise. Note: the target L2 U2N Relay UE should not be the same as serving L2 U2N Relay UE for inter-gNB indirect to indirect path switch. |
| MP | This field is optionally present, Need N, if a L2 U2N remote UE is configured to perform MP direct path addition during indirect-to-direct path swith procedure, or to perform MP direct path release during direct-to-indirect path switch procedure. It is absent otherwise. |
| NCR | The field is optionally present, Need M, for NCR-MT. It is absent otherwise. |
| ODssbAOssb | The field is mandatory present, when absoluteFrequencySSB of the serving cell is absent. It is optionally present, Need R, otherwise. |
| ODssbOnly | The field is optionally present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is absent otherwise. |
| PreConfigMG | The field is optionally present, Need R, if there is at least one per UE gap configured with preConfigInd or there is at least one per FR gap of the same FR which the SCell belongs to and configured with preConfigInd. It is absent, Need R, otherwise. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - path switch of L2 U2N remote UE to the target PCell, - path switch of L2 U2N remote UE to the target L2 U2N Relay UE, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - SCG activation while the SCG was previously deactivated, - SCell addition, - reconfiguration with sync. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to master.

#### –	CellGroupId

The IE CellGroupId is used to identify a cell group. Value 0 identifies the master cell group. Other values identify secondary cell groups. In this version of the specification only values 0 and 1 are supported.

CellGroupId information element

-- ASN1START

-- TAG-CELLGROUPID-START

CellGroupId ::=                             INTEGER (0.. maxSecondaryCellGroups)

-- TAG-CELLGROUPID-STOP

-- ASN1STOP

#### –	CellIdentity

The IE CellIdentity is used to unambiguously identify a cell within a PLMN/SNPN.

CellIdentity information element

-- ASN1START

-- TAG-CELLIDENTITY-START

CellIdentity ::=                         BIT STRING (SIZE (36))

-- TAG-CELLIDENTITY-STOP

-- ASN1STOP

#### –	CellReselectionPriority

The IE CellReselectionPriority concerns the absolute priority of the concerned carrier frequency, as used by the cell reselection procedure. Corresponds to parameter "priority" in TS 38.304 [20]. Value 0 means lowest priority. The UE behaviour for the case the field is absent, if applicable, is specified in TS 38.304 [20].

CellReselectionPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONPRIORITY-START

CellReselectionPriority ::=             INTEGER (0..7)

-- TAG-CELLRESELECTIONPRIORITY-STOP

-- ASN1STOP

#### –	CellReselectionSubPriority

The IE CellReselectionSubPriority indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. Value oDot2 corresponds to 0.2, value oDot4 corresponds to 0.4 and so on.

CellReselectionSubPriority information element

-- ASN1START

-- TAG-CELLRESELECTIONSUBPRIORITY-START

CellReselectionSubPriority ::=          ENUMERATED {oDot2, oDot4, oDot6, oDot8}

-- TAG-CELLRESELECTIONSUBPRIORITY-STOP

-- ASN1STOP

#### –	CFR-ConfigMulticast

The IE CFR-ConfigMulticast indicates UE specific common frequency resource configuration for multicast for one dedicated BWP.

CFR-ConfigMulticast information element

-- ASN1START

-- TAG-CFR-CONFIGMULTICAST-START

CFR-ConfigMulticast-r17::= SEQUENCE {

locationAndBandwidthMulticast-r17              INTEGER (0..37949)                       OPTIONAL,    -- Need S

pdcch-ConfigMulticast-r17                      PDCCH-Config                             OPTIONAL,    -- Need M

pdsch-ConfigMulticast-r17                      PDSCH-Config                             OPTIONAL,    -- Need M

sps-ConfigMulticastToAddModList-r17            SPS-ConfigMulticastToAddModList-r17      OPTIONAL,    -- Need N

sps-ConfigMulticastToReleaseList-r17           SPS-ConfigMulticastToReleaseList-r17     OPTIONAL     -- Need N

}

SPS-ConfigMulticastToAddModList-r17 ::=	SEQUENCE (SIZE (1..8)) OF SPS-Config

SPS-ConfigMulticastToReleaseList-r17 ::= SEQUENCE (SIZE (1..8)) OF SPS-ConfigIndex-r16

-- TAG-CFR-CONFIGMULTICAST-STOP

-- ASN1STOP

CFR-ConfigMulticast field descriptions

locationAndBandwidthMulticast Frequency domain location and bandwidth for MBS multicast. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting N^size_BWP=275. The first PRB is a PRB determined by subcarrierSpacing of the associated BWP and offsetToCarrier corresponding to this subcarrier spacing. If not configured, the UE applies the value of locationAndBandwidth of the DL BWP in which the cfr-ConfigMulticast is configured.

pdcch-ConfigMulticast UE specific group-common PDCCH configuration for MBS multicast for one CFR.

pdsch-ConfigMulticast UE specific group-common PDSCH configuration for MBS multicast for one CFR.

sps-ConfigMulticastToAddModList Indicates a list of one or more DL SPS configurations for MBS multicast.

sps-ConfigMulticastToReleaseList Indicates a list of one or more DL SPS configurations to be released. The NW may release a SPS configuration at any time.

#### –	CGI-InfoEUTRA

The IE CGI-InfoEUTRA indicates EUTRA cell access related information, which is reported by the UE as part of E-UTRA report CGI procedure.

CGI-InfoEUTRA information element

-- ASN1START

-- TAG-CGI-INFOEUTRA-START

CGI-InfoEUTRA ::=                        SEQUENCE {

cgi-info-EPC                            SEQUENCE {

cgi-info-EPC-legacy                 CellAccessRelatedInfo-EUTRA-EPC,

cgi-info-EPC-list                   SEQUENCE (SIZE (1..maxPLMN)) OF CellAccessRelatedInfo-EUTRA-EPC             OPTIONAL

}                                                                                                                       OPTIONAL,

cgi-info-5GC                            SEQUENCE (SIZE (1..maxPLMN)) OF CellAccessRelatedInfo-EUTRA-5GC             OPTIONAL,

freqBandIndicator                       FreqBandIndicatorEUTRA,

multiBandInfoList                       MultiBandInfoListEUTRA                                                      OPTIONAL,

freqBandIndicatorPriority               ENUMERATED {true}                                                           OPTIONAL

}

-- TAG-CGI-INFOEUTRA-STOP

-- ASN1STOP

#### –	CGI-InfoEUTRALogging

The IE CGI-InfoEUTRALogging indicates EUTRA cell related information, which is reported by the UE as part of RLF reporting procedure.

CGI-InfoEUTRALogging information element

-- ASN1START

-- TAG-CGI-INFOEUTRALOGGING-START

CGI-InfoEUTRALogging ::=         SEQUENCE {

plmn-Identity-eutra-5gc          PLMN-Identity                                          OPTIONAL,

trackingAreaCode-eutra-5gc       TrackingAreaCode                                       OPTIONAL,

cellIdentity-eutra-5gc           BIT STRING (SIZE (28))                                 OPTIONAL,

plmn-Identity-eutra-epc          PLMN-Identity                                          OPTIONAL,

trackingAreaCode-eutra-epc       BIT STRING (SIZE (16))                                 OPTIONAL,

cellIdentity-eutra-epc           BIT STRING (SIZE (28))                                 OPTIONAL

}

-- TAG-CGI-INFOEUTRALOGGING-STOP

-- ASN1STOP

CGI-InfoEUTRALogging field descriptions

cellIdentity-eutra-epc, cellIdentity-eutra-5GC Unambiguously identify a cell within the context of the PLMN. It belongs the first PLMN entry of plmn-IdentityList (when connected to EPC) or of plmn-IdentityList-r15 (when connected to 5GC) in SystemInformationBlockType1.

plmn-Identity-eutra-epc, plmn-Identity-eutra-5GC Identifies the PLMN of the cell for the reported cellIdentity: the first PLMN entry of plmn-IdentityList (when connected to EPC) or of plmn-IdentityList-r15 (when connected to 5GC) in SystemInformationBlockType1 that contained the reported cellIdentity.

trackingAreaCode-eutra-epc, trackingAreaCode-eutra-5gc Indicates Tracking Area Code to which the cell indicated by cellIdentity-eutra-epc, cellIdentity-eutra-5GC belongs.

#### –	CGI-InfoNR

The IE CGI-InfoNR indicates cell access related information, which is reported by the UE as part of report CGI procedure.

CGI-InfoNR information element

-- ASN1START

-- TAG-CGI-INFO-NR-START

CGI-InfoNR ::=                    SEQUENCE {

plmn-IdentityInfoList               PLMN-IdentityInfoList               OPTIONAL,

frequencyBandList                   MultiFrequencyBandListNR            OPTIONAL,

noSIB1                              SEQUENCE {

ssb-SubcarrierOffset                INTEGER (0..15),

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1

}                                                                       OPTIONAL,

...,

[[

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16            OPTIONAL

]],

[[

cellReservedForOtherUse-r16         ENUMERATED {true}                   OPTIONAL

]],

[[

hsdn-Cell-r19                       ENUMERATED {true}                   OPTIONAL

]]

}

-- TAG-CGI-INFO-NR-STOP

-- ASN1STOP

CGI-InfoNR field descriptions

cellReservedForOtherUse Contains cellReservedForOtherUse field acquired by the UE that supports nr-CGI-Reporting-NPN from SIB1 of the cell for which report CGI procedure was requested by the network.

hsdn-Cell Contains hsdn-Cell field acquired by the UE that supports nr-CGI-Reporting-HSDN from SIB1 of the cell for which report CGI procedure was requested by the network.

noSIB1 Contains ssb-SubcarrierOffset and pdcch-ConfigSIB1 fields acquired by the UE from MIB of the cell for which report CGI procedure was requested by the network in case SIB1 was not broadcast by the cell.

#### –	CGI-Info-Logging

The IE CGI-Info-Logging indicates the NR Cell Global Identifier (NCGI) for logging purposes (e.g. RLF report), the globally unique identity, and the TAC information of a cell in NR.

CGI-Info-Logging information element

-- ASN1START

-- TAG-CGI-INFO-LOGGING-START

CGI-Info-Logging-r16 ::=     SEQUENCE {

plmn-Identity-r16                    PLMN-Identity,

cellIdentity-r16                     CellIdentity,

trackingAreaCode-r16                 TrackingAreaCode               OPTIONAL

}

-- TAG-CGI-INFO-LOGGING-STOP

-- ASN1STOP

CGI-Info-Logging field descriptions

cellIdentity Unambiguously identify a cell within the context of the PLMN. It belongs the first PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1.

plmn-Identity Identifies the PLMN of the cell for the reported cellIdentity: the first PLMN entry of plmn-IdentityList (in SIB1) in the instance of PLMN-IdentityInfoList that contained the reported cellIdentity.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The first entry from trackingAreaList is logged in case the cell is configured with multiple tracking areas, e.g., NTN cell.

#### –	Cho-WithCandidateSCGInfo

The IE Cho-WithCandidateSCGInfo contains information regarding events of CHO with Candidate SCG.

Cho-WithCandidateSCGInfo information element

-- ASN1START

-- TAG-CHO-WITHCANDIDATESCGINFO-START

Cho-WithCandidateSCGInfo-r19 ::=               SEQUENCE{

firstFulfilledConfig-r19                       ENUMERATED {cho, cpc}                             OPTIONAL,

timeBetweenFulfillment-r19                     TimeBetweenEvent-r17                              OPTIONAL,

timeBetweenLastFulfillmentAndEvent-r19         TimeBetweenEvent-r17                              OPTIONAL,

fulfilledConfigWhenChoOnly-r19                 ENUMERATED {cho, cpc, neither}                    OPTIONAL,

pCellId-r19              CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

psCellId-r19             CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,    ...

}

-- TAG-CHO-WITHCANDIDATESCGINFO-STOP

-- ASN1STOP

Cho-WithCandidateSCGInfo field descriptions

firstFulfilledConfig This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled first.

fulfilledConfigWhenChoOnly This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled at the time of receiving a complementary conditional reconfiguration i.e., a conditional reconfiguration for a candidate PCell for which atleast one CHO with conditional SCG is already configured.

timeBetweenFulfillment This field logs the time between fulfilment of conditional handover and conditional PSCell change or addition execution conditions.

timeBetweenLastFulfillmentAndEvent This field logs the time between fulfilment of either conditional handover or conditional PSCell change or addition execution conditions and failure for RLF and SCG failure.

#### –	CLI-RSSI-MeasResource

The IE CLI-RSSI-MeasResource is used to configure CLI-RSSI measurement resource which the UE may be configured to measure L1 CLI-RSSI.

CLI-RSSI-MeasResource information element

-- ASN1START

-- TAG-CLI-RSSI-MEASRESOURCE-START

CLI-RSSI-MeasResource-r19 ::=                 SEQUENCE {

cli-RSSI-MeasResourceId-r19                   CLI-RSSI-MeasResourceId-r19                                  OPTIONAL,   -- Need R

startSymbol-r19                               INTEGER (0..13)                                              OPTIONAL,   -- Need R

nrofSymbols-r19                               INTEGER (1..14)                                              OPTIONAL,   -- Need R

startPRB-r19                                  INTEGER (0..maxNrofPhysicalResourceBlocks-1)                 OPTIONAL,   -- Need R

nrofPRBs-r19                                  INTEGER (1..maxNrofPhysicalResourceBlocks)                   OPTIONAL,   -- Need R

cli-RSSI-PeriodicityAndOffset-r19             CSI-ReportPeriodicityAndOffset                               OPTIONAL,   -- Need R

qcl-InfoPeriodic-CLI-RSSI-MeasResource-r19    TCI-StateId                                                  OPTIONAL,   -- Need R

...

}

-- TAG-CLI-RSSI-MEASRESOURCE-STOP

-- ASN1STOP

CLI-RSSI-MeasResource field descriptions

cli-RSSI-MeasResourceId Identifies a CLI-RSSI measurement resource.

cli-RSSI-PeriodicityAndOffset Indicates the periodicity and slot offset for this CLI-RSSI-MeasResource, with the same value range as CSI-ResourcePeriodicityAndOffset.

nrofPRBs Indicates the allowed size of the BW for L1 CLI-RSSI measurement.

nrofSymbols Indicates the number of symbols for L1 CLI-RSSI measurement within a slot.

qcl-InfoPeriodic-CLI-RSSI-MeasResource Indicates a reference to one TCI-State in TCI-States for providing the QCL source and QCL type for a target periodic CLI-RSSI-MeasResource

startPRB Indicates the starting PRB index of the measurement bandwidth.

startSymbol Indicates the starting symbol of the CLI-RSSI-MeasurementResource within a slot

#### –	CLI-RSSI-MeasResourceId

The IE CLI-RSSI-MeasResourceId is used to identify a CLI-RSSI-MeasResourceSet.

CLI-RSSI-MeasResourceId information element

-- ASN1START

-- TAG-CLI-RSSI-MEASRESOURCEID-START

CLI-RSSI-MeasResourceId-r19 ::=      INTEGER(0..maxNrofCLI-RSSI-MeasResources-1-r19)

-- TAG-CLI-RSSI-MEASRESOURCEID-STOP

-- ASN1STOP

#### –	CLI-RSSI-MeasResourceSet

The IE CLI-RSSI-MeasResourceSet specifies a set of CLI-RSSI measurement resources (their IDs) for L1 CLI-RSSI measurement and set-specific parameters.

CLI-RSSI-MeasResourceSet information element

-- ASN1START

-- TAG-CLI-RSSI-MEASRESOURCESET-START

CLI-RSSI-MeasResourceSet-r19 ::=    SEQUENCE {

aperiodicTriggeringOffset-r19       INTEGER (0..31)                                                       OPTIONAL,   -- Need R

cli-RSSI-MeasResourceSetId-r19      CLI-RSSI-MeasResourceSetId-r19                                        OPTIONAL,   -- Need R

cli-RSSI-MeasResourceIdList-r19     SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourcesPerSet-r19) ) OF CLI-RSSI-MeasResourceId-r19

OPTIONAL,   -- Need R

...

}

-- TAG-CLI-RSSI-MEASRESOURCESET-STOP

-- ASN1STOP

CLI-RSSI-MeasResourceSet field descriptions

aperiodicTriggeringOffset Indicates a slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured.

cli-RSSI-MeasResourceIdList Indicates the CLI-RSSI measurement resources associated with this CLI-RSSI measurement resource set.

cli-RSSI-MeasResourceSetId Indicates ID of CLI-RSSI measurement resource set.

#### –	CLI-RSSI-MeasResourceSetId

The IE CLI-RSSI-MeasResourceSetId is used to identify a CLI-RSSI-MeasResourceSet.

CLI-RSSI-MeasResourceSetId information element

-- ASN1START

-- TAG-CLI-RSSI-MEASRESOURCESETID-START

CLI-RSSI-MeasResourceSetId-r19 ::=      INTEGER(0..maxNrofCLI-RSSI-MeasResourceSets-1-r19)

-- TAG-CLI-RSSI-MEASRESOURCESETID-STOP

-- ASN1STOP

#### –	CLI-RSSI-Range

The IE CLI-RSSI-Range specifies the value range used in CLI-RSSI measurements and thresholds. The integer value for CLI-RSSI measurements is according to Table 10.1.22.2.2-1 in TS 38.133 [14].

CLI-RSSI-Range information element

-- ASN1START

-- TAG-CLI-RSSI-RANGE-START

CLI-RSSI-Range-r16 ::=                      INTEGER(0..76)

-- TAG-CLI-RSSI-RANGE-STOP

-- ASN1STOP

#### –	ClockQualityMetrics

The IE ClockQualityMetrics is used to configure RAN timing synchronisation status information as specified in TS 38.473 [36]

ClockQualityMetrics information element

-- ASN1START

-- TAG-CLOCKQUALITYMETRICS-START

ClockQualityMetrics-r18 ::= SEQUENCE {

synchronisationState-r18        ENUMERATED {locked, holdover, freerun, spare1}                        OPTIONAL,   -- Need N

tracebilityToUTC-r18            BOOLEAN                                                               OPTIONAL,   -- Need N

tracebilityToGNSS-r18           BOOLEAN                                                               OPTIONAL,   -- Need N

clockFrequencyStability-r18     BIT STRING (SIZE(16))                                                 OPTIONAL,   -- Need N

clockAccuracy-r18               CHOICE {

value INTEGER (1..40000000),

index INTEGER (32..47)

}                                                                                                     OPTIONAL,   -- Need N

parentTimeSource-r18            ENUMERATED {syncE, pTP, gNSS,atomicClock, terrestialRadio,

serialTimeCode, nTP, handset, other, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}           OPTIONAL,   -- Need N

...

}

-- TAG-CLOCKQUALITYMETRICS-STOP

-- ASN1STOP

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see TS 38.214 [19], clause 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::=                                  SEQUENCE {

codebookType                                        CHOICE {

type1                                               SEQUENCE {

subType                                             CHOICE {

typeI-SinglePanel                                   SEQUENCE {

nrOfAntennaPorts                                    CHOICE {

two                                                 SEQUENCE {

twoTX-CodebookSubsetRestriction                     BIT STRING (SIZE (6))

},

moreThanTwo                                         SEQUENCE {

n1-n2                                               CHOICE {

two-one-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction              BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction              BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction            BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction               BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction            BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction             BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction           BIT STRING (SIZE (64))

},

typeI-SinglePanel-codebookSubsetRestriction-i2      BIT STRING (SIZE (16))        OPTIONAL    -- Need R

}

},

typeI-SinglePanel-ri-Restriction                    BIT STRING (SIZE (8))

},

typeI-MultiPanel                                    SEQUENCE {

ng-n1-n2                                                CHOICE {

two-two-one-TypeI-MultiPanel-Restriction                BIT STRING (SIZE (8)),

two-four-one-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (16)),

four-two-one-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (8)),

two-two-two-TypeI-MultiPanel-Restriction                BIT STRING (SIZE (64)),

two-eight-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (32)),

four-four-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (16)),

two-four-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (128)),

four-two-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (64))

},

ri-Restriction                          BIT STRING (SIZE (4))

}

},

codebookMode                                        INTEGER (1..2)

},

type2                                   SEQUENCE {

subType                                 CHOICE {

typeII                                  SEQUENCE {

n1-n2-codebookSubsetRestriction         CHOICE {

two-one                                 BIT STRING (SIZE (16)),

two-two                                 BIT STRING (SIZE (43)),

four-one                                BIT STRING (SIZE (32)),

three-two                               BIT STRING (SIZE (59)),

six-one                                 BIT STRING (SIZE (48)),

four-two                                BIT STRING (SIZE (75)),

eight-one                               BIT STRING (SIZE (64)),

four-three                              BIT STRING (SIZE (107)),

six-two                                 BIT STRING (SIZE (107)),

twelve-one                              BIT STRING (SIZE (96)),

four-four                               BIT STRING (SIZE (139)),

eight-two                               BIT STRING (SIZE (139)),

sixteen-one                             BIT STRING (SIZE (128))

},

typeII-RI-Restriction                   BIT STRING (SIZE (2))

},

typeII-PortSelection                    SEQUENCE {

portSelectionSamplingSize               ENUMERATED {n1, n2, n3, n4}                   OPTIONAL,       -- Need R

typeII-PortSelectionRI-Restriction      BIT STRING (SIZE (2))

}

},

phaseAlphabetSize                       ENUMERATED {n4, n8},

subbandAmplitude                        BOOLEAN,

numberOfBeams                           ENUMERATED {two, three, four}

}

}

}

CodebookConfig-r16  ::=                SEQUENCE  {

codebookType                           CHOICE {

type2                                  SEQUENCE {

subType                                CHOICE {

typeII-r16                             SEQUENCE  {

n1-n2-codebookSubsetRestriction-r16    CHOICE {

two-one                                BIT STRING (SIZE (16)),

two-two                                BIT STRING (SIZE (43)),

four-one                               BIT STRING (SIZE (32)),

three-two                              BIT STRING (SIZE (59)),

six-one                                BIT STRING (SIZE (48)),

four-two                               BIT STRING (SIZE (75)),

eight-one                              BIT STRING (SIZE (64)),

four-three                             BIT STRING (SIZE (107)),

six-two                                BIT STRING (SIZE (107)),

twelve-one                             BIT STRING (SIZE (96)),

four-four                              BIT STRING (SIZE (139)),

eight-two                              BIT STRING (SIZE (139)),

sixteen-one                            BIT STRING (SIZE (128))

},

typeII-RI-Restriction-r16              BIT STRING (SIZE(4))

},

typeII-PortSelection-r16  SEQUENCE {

portSelectionSamplingSize-r16          ENUMERATED {n1, n2, n3, n4},

typeII-PortSelectionRI-Restriction-r16 BIT STRING (SIZE (4))

}

},

numberOfPMI-SubbandsPerCQI-Subband-r16 INTEGER (1..2),

paramCombination-r16                   INTEGER (1..8)

}

}

}

CodebookConfig-r17  ::=               SEQUENCE  {

codebookType                          CHOICE   {

type1                                 SEQUENCE  {

typeI-SinglePanel-Group1-r17          SEQUENCE {

nrOfAntennaPorts                      CHOICE {

two                                   SEQUENCE {

twoTX-CodebookSubsetRestriction1-r17  BIT STRING (SIZE (6))

},

moreThanTwo                            SEQUENCE {

n1-n2                                        CHOICE {

two-one-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction1-r17      BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction1-r17      BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction1-r17    BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction1-r17       BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction1-r17    BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction1-r17     BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction1-r17   BIT STRING (SIZE (64))

}

}

}

}                                                                                       OPTIONAL,  -- Need R

typeI-SinglePanel-Group2-r17           SEQUENCE {

nrOfAntennaPorts                       CHOICE {

two                                    SEQUENCE {

twoTX-CodebookSubsetRestriction2-r17   BIT STRING (SIZE (6))

},

moreThanTwo                            SEQUENCE {

n1-n2                                        CHOICE {

two-one-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction2-r17      BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction2-r17      BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction2-r17    BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction2-r17       BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction2-r17    BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction2-r17     BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction2-r17   BIT STRING (SIZE (64))

}

}

}

}                                                                                       OPTIONAL,  -- Need R

typeI-SinglePanel-ri-RestrictionSTRP-r17                    BIT STRING (SIZE (8))       OPTIONAL,  -- Need R

typeI-SinglePanel-ri-RestrictionSDM-r17                     BIT STRING (SIZE (4))       OPTIONAL   -- Need R

},

type2                                 SEQUENCE {

typeII-PortSelection-r17              SEQUENCE {

paramCombination-r17                   INTEGER (1..8),

valueOfN-r17                           ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r17 INTEGER(1..2)                                OPTIONAL,  -- Need R

typeII-PortSelectionRI-Restriction-r17 BIT STRING (SIZE (4))

}

}

}

}

CodebookConfig-v1730  ::=             SEQUENCE  {

codebookType                          CHOICE   {

type1                                 SEQUENCE  {

codebookMode                          INTEGER (1..2)                                    OPTIONAL   -- Need R

}

}

}

CodebookConfig-r18  ::=                   SEQUENCE {

codebookType                              CHOICE {

type2                                     CHOICE {

typeII-CJT-r18                            SEQUENCE {

n1-n2-codebookSubsetRestrictionList-r18   N1-N2-CBSR-List-r18,

paramCombination-CJT-r18                  INTEGER (1..7),

paramCombination-CJT-L-r18                SEQUENCE (SIZE (1..4)) OF INTEGER (1..5),

restrictedCMR-Selection-r18               ENUMERATED {enable},

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)

},

typeII-CJT-PortSelection-r18              SEQUENCE {

paramCombination-CJT-PS-r18               INTEGER (1..5),

paramCombination-CJT-PS-alpha-r18         SEQUENCE (SIZE (1..4)) OF INTEGER (1..8),

restrictedCMR-Selection-r18               ENUMERATED {enable},

valueOfO3-r18                             ENUMERATED {n1, n4}                          OPTIONAL,  -- Need R

valueOfN-CJT-r18                          ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

typeII-PortSelectionRI-Restriction-r18    BIT STRING (SIZE (4)),

codebookMode-r18                          INTEGER (1..2)

},

typeII-Doppler-r18                        SEQUENCE {

n1-n2-codebookSubsetRestriction-r18       N1-N2-CBSR-r18,

paramCombination-Doppler-r18              INTEGER (1..9),

td-dd-config-r18                          TD-DD-Config-r18,

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

predictionDelay-r18                       ENUMERATED {m0,n0,n1,n2 },

typeII-RI-Restriction-r18                 BIT STRING (SIZE (4))

},

typeII-DopplerPortSelection-r18           SEQUENCE {

paramCombinationDoppler-PS-r18            INTEGER (1..8),

td-dd-config-r18                          TD-DD-Config-r18,

valueOfN-Doppler-r18                      ENUMERATED {n2, n4}                          OPTIONAL,  -- Need R

numberOfPMI-SubbandsPerCQI-Subband-r18    INTEGER(1..2),

predictionDelay-r18                       ENUMERATED {m0,n0,n1,n2 },

typeII-PortSelectionRI-Restriction-r18    BIT STRING (SIZE (4))

}

}

}

}

CodebookConfig-r19  ::=                  SEQUENCE {

codebookType                             CHOICE {

type1                                    CHOICE {

typeI-SinglePanel-r19                    SEQUENCE {

codebookMode-r19                         ENUMERATED {modeA,modeB},

typeI-SinglePanelRI-Restriction-r19      BIT STRING (SIZE (8)),

n1-n2-r19                                ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four,

eight-eight},

typeI-CodebookSubsetRestriction-r19      TypeI-X1-X2-CBSR-r19                                    OPTIONAL,   -- Need R

typeI-SoftScalingRank-r19                TypeI-X1-X2-SoftScalingRank-r19                         OPTIONAL,   -- Need R

cri-TypeI-SinglePanelRI-Restriction-r19  SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8))         OPTIONAL,   -- Need R

cri-TypeI-SinglePanelN1-N2-CBSR-r19      CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19                OPTIONAL    -- Need R

},

typeI-MultiPanel-r19                     SEQUENCE {

ri-Restriction-r19                       BIT STRING (SIZE (4)),

ng-n1-n2-r19                             CHOICE {

two-four-three-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (192)),

two-six-two-TypeI-MultiPanel-Restriction-r19                 BIT STRING (SIZE (192)),

two-eight-two-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (256)),

four-four-two-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (128)),

two-four-four-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (256)),

four-four-four-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256)),

four-eight-two-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256))

}

}

},

type2                                    CHOICE {

etypeII-r19                              SEQUENCE {

typeII-RI-Restriction-r19                BIT STRING (SIZE (4)),

numberOfPMI-SubbandsPerCQI-Subband-r19   INTEGER(1..2),

paramCombination-r19                     INTEGER (1..8),

n1-n2-r19                                ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four,

eight-eight},

typeII-CodebookSubsetRestriction-r19     TypeII-X1-X2-CBSR-r19                                   OPTIONAL,   -- Need R

cri-TypeII-RI-Restriction-r19            SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (4))         OPTIONAL,   -- Need R

cri-TypeII-N1-N2-CBSR-r19                CRI-TypeII-N1-N2-CBSR-List-r19                          OPTIONAL    -- Need R

},

typeII-FePortSelection-r19              SEQUENCE {

typeII-RI-Restriction-r19                 BIT STRING (SIZE (4)),

numberOfPMI-SubbandsPerCQI-Subband-r19    INTEGER(1..2),

paramCombination-r19                      INTEGER (1..7),

valueOfN-r19                              ENUMERATED {n2, n4}                          OPTIONAL  -- Need R

},

typeII-Doppler-r19                       SEQUENCE {

typeII-RI-Restriction-r19                 BIT STRING (SIZE (4)),

numberOfPMI-SubbandsPerCQI-Subband-r19    INTEGER(1..2),

paramCombination-Doppler-r19              INTEGER (1..9),

td-dd-config-r19                          TD-DD-Config-r18,

predictionDelay-r19                       ENUMERATED {m0,n0,n1,n2},

n1-n2-r19                                 ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four,

eight-eight},

typeII-CodebookSubsetRestriction-r19      TypeII-X1-X2-CBSR-r19                                  OPTIONAL    -- Need R

}

}

}

}

CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 ::= CHOICE {

two-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8))},

two-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (64))},

four-one-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (16))},

three-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (96))},

six-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (24))},

four-two-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (128))},

eight-one-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (32))},

four-three-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (192))},

six-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (192))},

twelve-one-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (48))},

four-four-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (256))},

eight-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (256))},

sixteen-one-r19        CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (64))}

}

CRI-TypeII-N1-N2-CBSR-List-r19 ::= CHOICE {

two-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (16))},

two-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (43))},

four-one-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (32))},

three-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (59))},

six-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (48))},

four-two-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (75))},

eight-one-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (64))},

four-three-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (107))},

six-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (107))},

twelve-one-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (96))},

four-four-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (139))},

eight-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (139))},

sixteen-one-r19        CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (128))}

}

N1-N2-CBSR-List-r18 ::= CHOICE {

two-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (8))},

two-two-r18            CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (27))},

four-one-r18           CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (16))},

three-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (35))},

six-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (24))},

four-two-r18           CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (43))},

eight-one-r18          CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (32))},

four-three-r18         CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (59))},

twelve-one-r18         CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (48))},

four-four-r18          CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (75))},

eight-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (75))},

sixteen-one-r18        CHOICE {no-cbsr-r18 NULL, cbsr-list-r18 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (64))}

}

N1-N2-CBSR-r18 ::=     CHOICE {

two-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (8))},

two-two-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (27))},

four-one-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (16))},

three-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (35))},

six-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (24))},

four-two-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (43))},

eight-one-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (32))},

four-three-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (59))},

twelve-one-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (48))},

four-four-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

eight-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

sixteen-one-r18        CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (64))}

}

TD-DD-Config-r18 ::=        SEQUENCE {

vectorLengthDD-r18           ENUMERATED {n1, n2, n4, n8},

unitDurationDD-r18           ENUMERATED {m1, m2}                                   OPTIONAL,  -- Need R

aperiodicResourceOffset-r18  INTEGER(1..2)                                         OPTIONAL,  -- Need R

tdCQI-r18                    ENUMERATED {n11, n12, n2, spare1}                     OPTIONAL   -- Need R

}

TypeI-X1-X2-CBSR-r19 ::= CHOICE {

one-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (384)), thirtyTwo-r19 BIT STRING (SIZE (512)),

sixtyFour-r19 BIT STRING (SIZE (1024))},

two-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (192)), thirtyTwo-r19 BIT STRING (SIZE (256)),

sixtyFour-r19 BIT STRING (SIZE (512))},

two-two-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (96)), thirtyTwo-r19 BIT STRING (SIZE (128)),

sixtyFour-r19 BIT STRING (SIZE (256))},

four-one-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (96)), thirtyTwo-r19 BIT STRING (SIZE (128)),

sixtyFour-r19 BIT STRING (SIZE (256))},

four-two-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (48)), thirtyTwo-r19 BIT STRING (SIZE (64)),

sixtyFour-r19 BIT STRING (SIZE (128))},

four-four-r19  CHOICE {twentyFour-r19 BIT STRING (SIZE (24)), thirtyTwo-r19 BIT STRING (SIZE (32)),

sixtyFour-r19 BIT STRING (SIZE (64))}

}

TypeI-X1-X2-SoftScalingRank-r19 ::= CHOICE {

two-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (576)), thirtyTwo-r19 BIT STRING (SIZE (768)),

sixtyFour-r19 BIT STRING (SIZE (1536))},

two-two-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (288)), thirtyTwo-r19 BIT STRING (SIZE (384)),

sixtyFour-r19 BIT STRING (SIZE (768))},

four-one-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (288)), thirtyTwo-r19 BIT STRING (SIZE (384)),

sixtyFour-r19 BIT STRING (SIZE (768))},

four-two-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (144)), thirtyTwo-r19 BIT STRING (SIZE (192)),

sixtyFour-r19 BIT STRING (SIZE (384))},

four-four-r19  CHOICE {twentyFour-r19 BIT STRING (SIZE (72)), thirtyTwo-r19 BIT STRING (SIZE (96)),

sixtyFour-r19 BIT STRING (SIZE (192))},

eight-one-r19  CHOICE {twentyFour-r19 BIT STRING (SIZE (144)), thirtyTwo-r19 BIT STRING (SIZE (192)),

sixtyFour-r19 BIT STRING (SIZE (384))}

}

TypeII-X1-X2-CBSR-r19 ::= CHOICE {

one-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (24)), thirtyTwo-r19 BIT STRING (SIZE (32)),

sixtyFour-r19 BIT STRING (SIZE (64))},

two-one-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (12)), thirtyTwo-r19 BIT STRING (SIZE (16)),

sixtyFour-r19 BIT STRING (SIZE (32))},

two-two-r19    CHOICE {twentyFour-r19 BIT STRING (SIZE (6)), thirtyTwo-r19 BIT STRING (SIZE (8)),

sixtyFour-r19 BIT STRING (SIZE (16))},

four-one-r19   CHOICE {twentyFour-r19 BIT STRING (SIZE (6)), thirtyTwo-r19 BIT STRING (SIZE (8)),

sixtyFour-r19 BIT STRING (SIZE (16))},

four-two-r19   CHOICE {thirtyTwo-r19 BIT STRING (SIZE (4)), sixtyFour-r19 BIT STRING (SIZE (8))}

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

CodebookConfig field descriptions

codebookMode CodebookMode as specified in TS 38.214 [19], clause 5.2.2.2 8 and 5.2.2.9.

codebookType CodebookType including possibly sub-types and the corresponding parameters for each (see TS 38.214 [19], clause 5.2.2.2).

cri-TypeI-SinglePanelN1-N2-CBSR Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.1.4.2). Value no-cbsr means no codebook subset restriction is configured for the n1-n2 pair. If a codebook subset restriction is configured for the n1-n2 pair, the number of elements in cbsr-list is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(s) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. An element in the list corresponds to the element at the same position in nzp-CSI-RS-Resources.

cri-TypeI-SinglePanelRI-Restriction Restriction for RI for CRI-TypeI-SinglePanelRI-Restriction (see TS 38.214 [19], clause 5.2.1.4.2).

cri-TypeII-N1-N2-CBSR Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.1.4.2). Value no-cbsr means no codebook subset restriction is configured for the n1-n2 pair. If a codebook subset restriction is configured for the n1-n2 pair, the number of elements in cbsr-list is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(s) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. An element in the list corresponds to the element at the same position in nzp-CSI-RS-Resources.

cri-TypeII-ri-Restriction Restriction for RI for CRI-TypeII-RI-Restriction (see TS 38.214 [19], clause 5.2.1.4.2).

n1-n2-codebookSubsetRestriction, n1-n2-codebookSubsetRestrictionList Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.3). Value no-cbsr means no codebook subset restriction is configured for the n1-n2 pair. If a codebook subset restriction is configured for the n1-n2 pair, the number of elements in cbsr-list in n1-n2-codebookSubSetRestrictionList is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(s) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. An element in the list corresponds to the element at the same position in nzp-CSI-RS-Resources.

n1-n2 Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.1).

ng-n1-n2 Codebook subset restriction for Type I Multi-panel codebook (see TS 38.214 [19], clause 5.2.2.2.2).

numberOfBeams Number of beams, L, used for linear combination.

numberOfPMI-SubbandsPerCQI-Subband Field indicates how PMI subbands are defined per CQI subband according to TS 38.214 [19], clause 5.2.2.2.5, 5.2.2.2.8 and 5.2.2.9.

paramCombination, paramCombination-CJT-r18, paramCombination-CJT-L-r18, paramCombination-CJT-PS-r18, paramCombination-CJT-PS-alpha, paramCombinationDoppler-r18, paramCombinationDoppler-PS-r18, paramCombination-r19, paramCombinationDoppler-r19 Field describes supported parameter combination (M, , ) as specified in TS 38.214 [19] Clause 5.2.2. For fields paramCombination-CJT-L-r18 or paramCombination-CJT-PS-alpha the number of elements is 1, 2 or 4, and the values are configured according to the number of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet elements from Tables 5.2.2.2.8-1 and 5.2.2.2.9-1 in TS 38.214 [19], respectively. The allowed configuration combinations for paramCombination-CJT-r18 and paramCombination-CJT-L-r18 are given in Table 5.2.2.2.8-3 in TS 38.214 [19]. The allowed configuration combinations for paramCombination-CJT-PS-r18 and paramCombination-CJT-PS-alpha-r18 are given in Table 5.2.2.2.9-3 in TS 38.214 [19]. The allowed configuration combinations for paramCombination-r19 and paramCombinationDoppler-r19 are specified in clause 5.2.2 of TS 38.214 [19].

phaseAlphabetSize The size of the PSK alphabet, QPSK or 8-PSK.

portSelectionSamplingSize The size of the port selection codebook (parameter d), see TS 38.214 [19] clause 5.2.2.2.6.

predictionDelay Prediction delay for Doppler and Doppler port selection codebooks see TS 38.214 [19], Clause 5.2.1.4. The first value m0 means that the first slot for which the CSI corresponds to is the slot where the CSI reference resource is located at. For the other three candidate values (n0, n1, n2), then the first slot for which the CSI corresponds to is given by l= n+delta, where delta can take on values of 0, 1, 2 and n the slot in which CSI is reported.

restrictedCMR-Selection Enabling CSI-RS resource (CMR) restriction, (see TS 38.214 [19], clause 5.2.2.2.8)

ri-Restriction Restriction for RI for TypeI-MultiPanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.2).

subbandAmplitude If subband amplitude reporting is activated (true).

twoTX-CodebookSubsetRestriction Codebook subset restriction for 2TX codebook (see TS 38.214 [19] clause 5.2.2.2.1).

typeI-CodebookSubsetRestriction Codebook subset restriction for codebook typeI-SinglePanel-r19 where the size of the bit string is related to the values configured in n1-n2 and (X1, X2) values as defined in TS 38.214 [19] clause 5.2.2.2.1a. Value twentyFour is used when n1-n2 is set to eight-three or six-four, value thirtyTwo is used when n1-n2 is set to sixteen-two or eight-four and value sixtyFour is used when n1-n2 is set to sixteen-four or eight-eight.

typeI-SinglePanel-codebookSubsetRestriction-i2 i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI (see TS 38.214 [19] clause 5.2.2.2.1).

typeI-SinglePanel-ri-Restriction Restriction for RI for TypeI-SinglePanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.1).

typeI-SinglePanel-Group1, typeI-SinglePanel-Group2 Configures codebooks for CSI calculation when UE is configured with two CMR Groups with CMRGroupingAndPairing in the NZP-CSI-RS-ResourceSet associated with the CSI-ReportConfig. Network configures the same number of ports for both codebooks.

typeI-SinglePanel-ri-RestrictionSDM, typeI-SinglePanel-ri-RestrictionSTRP Restriction for RI for N Resource Pairs when two CMR Groups are configured with CMRGroupingAndPairing in the NZP-CSI-RS-ResourceSet associated with the CSI-ReportConfig (see TS 38.214 [19], clause 5.2.1.4.2).

typeI-SoftScalingRank Power scaling for codebook typeI-SinglePanel-r19 where the size of the bit string is related to the values configured in n1-n2 and (X1, X2) values as defined in TS 38.214 [19] clause 5.2.2.2.1a. Value twentyFour is used when n1-n2 is set to eight-three or six-four, value thirtyTwo is used when n1-n2 is set to sixteen-two or eight-four and value sixtyFour is used when n1-n2 is set to sixteen-four or eight-eight.

typeII-CodebookSubsetRestriction Codebook subset restriction for codebook eTypeII-r19 and typeII-Doppler-r19 where the size of the bit string is related to the values configured in n1-n2 and (X1, X2) values as defined in TS 38.214 [19] clause 5.2.2.2.5. Value twentyFour is used when n1-n2 is set to eight-three or six-four, value thirtyTwo is used when n1-n2 is set to sixteen-two or eight-four and value sixtyFour is used when n1-n2 is set to sixteen-four or eight-eight.

typeII-PortSelectionRI-Restriction Restriction for RI for TypeII-PortSelection-RI-Restriction (see TS 38.214 [19], clauses 5.2.2.2.4, 5.2.2.2.6 and 5.2.2.2.7).

typeII-RI-Restriction Restriction for RI for TypeII-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2).

valueOfN Field provides the value of parameter N as specified in TS 38.214 [19], clause 5.2.2.2.7. The field is present only when M=2 set by paramCombination, see TS 38.214 [19].

valueOfO3 O3: oversampling factor for frequency-domain basis selection offset Only applicable when CodebookMode = 'Mode1'.

valueOfN-CJT Field describes the size of the window of FD basis for FeType-II port selection codebook for multi-TRP CJT. For M=2, N=2,4 (see TS 38.214 [19], clause 5.2.2.2.9)

valueOfN-Doppler Field describes the size of the window of FD basis for FeType-II port selection codebook for Doppler CSI. For M=2, N=2,4 (see TS 38.214 [19], clause 5.2.2.2.9)

TD-DD-Config field descriptions

aperiodicResourceOffset Offset m (in slots) between two consecutive aperiodic CSI-RS resources. See TS 38.214 [19], clause 5.2.1.4.

tdCQI Number of TD CQIs (X) in each SB and ref slot(s) + ref W2(s). 1-1: 1 TD CQI, ref = 1st slot and 1st W2 1-2: 1 TD CQI, ref = 1st and last slots, and 1st and last W2 2: 2 TD CQIs (see TS 38.214 [19], clause 5.2.1.4).

unitDurationDD DD/TD unit duration (in slots), for AP-CSI-RS only. Note: For P/SP-CSI-RS, d is set to CSI-RS periodicity (see TS 38.214 [19], clause 5.2.1.4). If the value of aperiodicResourceOffset is '1', network configures value 'm1' for this field. Value m1 corresponds to 1 slot and value m2 corresponds to 2 slots.

vectorLengthDD Doppler-/time-domain (DD/TD) basis vector length; See TS 38.214 [19], clause 5.2.1.4.

#### –	CommonLocationInfo

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo information element

-- ASN1START

-- TAG-COMMONLOCATIONINFO-START

CommonLocationInfo-r16 ::= SEQUENCE {

gnss-TOD-msec-r16          OCTET STRING     OPTIONAL,

locationTimestamp-r16      OCTET STRING     OPTIONAL,

locationCoordinate-r16     OCTET STRING     OPTIONAL,

locationError-r16          OCTET STRING     OPTIONAL,

locationSource-r16         OCTET STRING     OPTIONAL,

velocityEstimate-r16       OCTET STRING     OPTIONAL

}

-- TAG-COMMONLOCATIONINFO-STOP

-- ASN1STOP

CommonLocationInfo field descriptions

gnss-TOD-msec Parameter type gnss-TOD-msec defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

#### –	CondReconfigId

The IE CondReconfigId is used to identify a CHO, CPA, CPC, subsequent CPAC, or CHO with candidate SCG(s) configuration.

CondReconfigId information element

-- ASN1START

-- TAG-CONDRECONFIGID-START

CondReconfigId-r16 ::=                    INTEGER (1.. maxNrofCondCells-r16)

-- TAG-CONDRECONFIGID-STOP

-- ASN1STOP

#### –	CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated fields.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondPSCell-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigCHO-WithSCG

subsequentCondReconfig-r18       SubsequentCondReconfig-r18                            OPTIONAL,    -- Need M

securityCellSetId-r18            SecurityCellSetId-r18                                 OPTIONAL,    -- Need M

scpac-ConfigComplete-r18         ENUMERATED {true}                                     OPTIONAL     -- Cond CPAC

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

SubsequentCondReconfig-r18 ::=   SEQUENCE {

condExecutionCondToReleaseList-r18   CondExecutionCondToReleaseList-r18                OPTIONAL,   -- Need N

condExecutionCondToAddModList-r18    CondExecutionCondToAddModList-r18                 OPTIONAL,   -- Need N

...

}

CondExecutionCondToAddModList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondExecutionCondToAddMod-r18

CondExecutionCondToAddMod-r18 ::=    SEQUENCE {

subsequentCondReconfigId-r18         CondReconfigId-r16,

subsequentCondExecutionCond-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

subsequentCondExecutionCondSCG-r18   OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL,    -- Need M

...

}

CondExecutionCondToReleaseList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for CHO, CPA, intra-SN CPC without MN involvement, MN initiated inter-SN CPC, MN initiated subsequent CPAC, or SN initiated intra-SN subsequent CPAC without MN involvement. When configuring 2 triggering events (Meas Ids) for a candidate cell, the network ensures that both refer to the same measObject. Except for CHO with only event condEventD1, or condEventD2, or condEventT1, if the network configures condEventD1 or condEventD2 or condEventT1 for a candidate cell, the network configures a second triggering event condEventA3, condEventA4 or condEventA5 for the same candidate cell. The network configures at most one from condEventD1, condEventD2 or condEventT1 for the same candidate cell. For CPA, MN-initiated inter-SN CPC, and for MN initiated subsequent CPAC, the network only indicates MeasId(s) associated with condEventA4. For intra-SN CPC and for SN initiated intra-SN subsequent CPAC without MN involvement, the network only indicates MeasId(s) associated with condEventA3 or condEventA5.

condExecutionCondPSCell The execution condition that needs to be fulfilled for the associated PSCell in order to trigger the execution of a conditional reconfiguration for CHO with candidate SCG(s). The Meas Ids refer to the measConfig associated with the MCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. The network only indicates MeasId(s) associated with condEventA4.

condExecutionCondSCG Contains execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for SN initiated inter-SN CPC, SN initiated inter-SN subsequent CPAC, or SN initiated intra-SN subsequent CPAC with MN involvement. The Meas Ids refer to the measConfig associated with the SCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For each condReconfigId, the network always configures either condExecutionCond or condExecutionCondSCG (not both). The network only indicates MeasId(s) associated with condEventA3 or condEventA5.

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config.

securityCellSetId This field is used to determine whether the UE should perform security update when conditional reconfiguration containing subsequentCondReconfig is executed. If the field servingSecurityCellSetId is configured in conditionalReconfiguration, this field is configured for all the candidate configurations for subsequent CPAC.

subsequentCondReconfig Contains the execution conditions that need to be fulfilled in order to trigger the execution of a subsequent CPAC. If the field is configured, the configuration of candidate PSCells for subsequent CPAC is supported. The subsequent execution condition is used for conditional reconfiguration evaluation for other candidate cells when the RRCReconfiguration message contained in condRRCReconfig has been applied.

CondExecutionCondToAddMod field descriptions

subsequentCondExecutionCond The execution condition that needs to be fulfilled in order to trigger the subsequent execution of a conditional reconfiguration for SN initiated intra-SN subsequent CPAC without MN involvement. When configuring 2 triggering events (Meas Ids) for a candidate cell, the network ensures that both refer to the same measObject. The network only indicates MeasId(s) associated with condEventA3 or condEventA5.

subsequentCondExecutionCondSCG Contains execution condition that needs to be fulfilled in order to trigger the subsequent execution of a conditional reconfiguration for SN initiated inter-SN subsequent CPAC, SN initiated intra-SN subsequent CPAC with MN involvement, or MN initiated subsequent CPAC. The Meas Ids refer to the measConfig associated with the SCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. The network only indicates MeasId(s) associated with condEventA3 or condEventA5.

| Conditional Presence | Explanation |
| --- | --- |
| condReconfigAdd | The field is mandatory present when a condReconfigId is being added. Otherwise the field is optional, need M. |
| condReconfigCHO-WithSCG | This field is optional present, need M, if the RRCReconfiguration message contained in corresponding condRRCReconfig includes the nr-SCG and condExecutionCond is configured. Otherwise, it is absent. |
| CPAC | The field is optionally present, need M, when the conditional reconfiguration includes at least one candidate PSCell supporting subsequent CPAC. Otherwise, the field is absent, need R. |

#### –	ConditionalReconfiguration

The IE ConditionalReconfiguration is used to add, modify and release the configuration of conditional reconfiguration.

ConditionalReconfiguration information element

-- ASN1START

-- TAG-CONDITIONALRECONFIGURATION-START

ConditionalReconfiguration-r16 ::=   SEQUENCE {

attemptCondReconfig-r16              ENUMERATED {true}              OPTIONAL,   -- Cond CHO

condReconfigToRemoveList-r16         CondReconfigToRemoveList-r16   OPTIONAL,   -- Need N

condReconfigToAddModList-r16         CondReconfigToAddModList-r16   OPTIONAL,   -- Need N

...,

[[

scpac-ReferenceConfiguration-r18     SetupRelease {ReferenceConfiguration-r18}          OPTIONAL,   -- Need M

servingSecurityCellSetId-r18         SecurityCellSetId-r18                              OPTIONAL,   -- Need M

sk-CounterConfiguration-r18          SK-CounterConfiguration-r18                        OPTIONAL    -- Need M

]]

}

CondReconfigToRemoveList-r16 ::=     SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

SK-CounterConfiguration-r18      ::= SEQUENCE {

sk-CounterConfigToReleaseList-r18    SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SecurityCellSetId-r18    OPTIONAL,    -- Need N

sk-CounterConfigToAddModList-r18     SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfig-r18     OPTIONAL     -- Need N

}

SK-CounterConfig-r18 ::=             SEQUENCE {

securityCellSetId-r18                SecurityCellSetId-r18,

sk-CounterList-r18                   SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

SecurityCellSetId-r18 ::= INTEGER (1.. maxSecurityCellSet-r18)

-- TAG-CONDITIONALRECONFIGURATION-STOP

-- ASN1STOP

ConditionalReconfiguration field descriptions

attemptCondReconfig If present, the UE shall perform conditional reconfiguration if selected cell is a target candidate cell and it is the first cell selection after failure as described in clause 5.3.7.3.

condReconfigToAddModList List of the configuration of candidate SpCells to be added or modified for CHO, CPA or CPC.

condReconfigToRemoveList List of the configuration of candidate SpCells to be removed.

scpac-ReferenceConfiguration Includes the reference configuration for the candidate supporting subsequent CPAC.

servingSecurityCellSetId This field identifies the security cell set for serving PSCell. The network does not provide this field for the conditional reconfiguration(s) generated by the SN.

sk-counterConfiguration Includes a list of sk-Counter from which the UE should select the sk-counter used to derive S-KgNB for inter-SN subsequent CPAC. The network does not provide this field for the conditional reconfiguration(s) generated by the SN.

| Conditional Presence | Explanation |
| --- | --- |
| CHO | The field is optional present, Need R, if the UE is configured with at least a candidate SpCell for CHO. Otherwise the field is not present. |

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2). Multiple Configured Grant configurations may be configured in one BWP of a serving cell.

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                INTEGER (1..64)                                                         OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant           SEQUENCE {

timeDomainOffset                    INTEGER (0..5119),

timeDomainAllocation                INTEGER (0..15),

frequencyDomainAllocation           BIT STRING (SIZE(18)),

antennaPort                         INTEGER (0..31),

dmrs-SeqInitialization              INTEGER (0..1)                                                         OPTIONAL,   -- Need R

precodingAndNumberOfLayers          INTEGER (0..63),

srs-ResourceIndicator               INTEGER (0..15)                                                        OPTIONAL,   -- Need R

mcsAndTBS                           INTEGER (0..31),

frequencyHoppingOffset              INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                          OPTIONAL,   -- Need R

pathlossReferenceIndex              INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...,

[[

pusch-RepTypeIndicator-r16          ENUMERATED {pusch-RepTypeA,pusch-RepTypeB}                             OPTIONAL,   -- Need M

frequencyHoppingPUSCH-RepTypeB-r16  ENUMERATED {interRepetition, interSlot}                                OPTIONAL,   -- Cond RepTypeB

timeReferenceSFN-r16                ENUMERATED {sfn512}                                                    OPTIONAL    -- Need S

]],

[[

pathlossReferenceIndex2-r17         INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)                       OPTIONAL,   -- Need R

srs-ResourceIndicator2-r17          INTEGER (0..15)                                                        OPTIONAL,   -- Need R

precodingAndNumberOfLayers2-r17     INTEGER (0..63)                                                        OPTIONAL,   -- Need R

timeDomainAllocation-v1710          INTEGER (16..63)                                                       OPTIONAL,   -- Need M

timeDomainOffset-r17                INTEGER (0..40959)                                                     OPTIONAL,   -- Need R

cg-SDT-Configuration-r17            CG-SDT-Configuration-r17                                               OPTIONAL    -- Need M

]],

[[

srs-ResourceSetId-r18               SRS-ResourceSetId                                                      OPTIONAL,   -- Need R

cg-LTM-Configuration-r18            CG-RRC-Configuration-r18                                               OPTIONAL, -- Cond LTM

cg-SDT-PeriodicityExt-r18          ENUMERATED {

sym1x14x1280, sym2x14x1280, sym4x14x1280 , sym8x14x1280, sym16x14x1280,

sym32x14x1280, sym48x14x1280, sym64x14x1280, sym96x14x1280, sym128x14x1280,

sym192x14x1280, sym240x14x1280, sym256x14x1280, sym384x14x1280, sym472x14x1280,

sym480x14x1280, sym512x14x1280, sym768x14x1280, sym944x14x1280, sym960x14x1280,

sym1408x14x1280, sym1536x14x1280, sym1888x14x1280, sym1920x14x1280,

sym2816x14x1280, sym3072x14x1280, sym3776x14x1280, sym5632x14x1280,

sym6144x14x1280, sym7552x14x1280, sym7680x14x1280, sym11264x14x1280,

sym15104x14x1280, sym15360x14x1280, sym22528x14x1280, sym30208x14x1280,

sym45056x14x1280, sym60416x14x1280, sym90112x14x1280, sym180224x14x1280,

sym4x12x1280, sym8x12x1280, sym16x12x1280, sym32x12x1280, sym192x12x1280,

sym384x12x1280, sym960x12x1280, sym1888x12x1280, sym3776x12x1280,

sym5632x12x1280, sym11264x12x1280, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1

}                                                                OPTIONAL,   -- Cond CG-SDT1

timeReferenceHyperSFN-r18           INTEGER (0..1023)                                                OPTIONAL,   -- Cond CG-SDT2

cg-RRC-Configuration-r18            CG-RRC-Configuration-r18                                       OPTIONAL, -- Cond RACH-LessHO

applyIndicatedTCI-State-r18         ENUMERATED {first, second, both, spare1}                               OPTIONAL    -- Need R

]],

[[

sbfd-Config2-PUSCH-RB-Offset-r19    INTEGER (0..maxNrofPhysicalResourceBlocks)                             OPTIONAL,   -- Need R

symbolType-r19                      ENUMERATED {sbfd, non-sbfd}                                            OPTIONAL,   -- Need R

frequencyHoppingOffset-SBFD-r19     INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                          OPTIONAL,   -- Need R

pusch-MutingResources-r19           PUSCH-MutingResources-r19                                              OPTIONAL,   -- Need R

occ-LengthAndSequenceIndex-r19      CHOICE {

length2   INTEGER(0..1),

length4   INTEGER(0..3)

}                                                                      OPTIONAL    -- Need R

]]

}                                                                                                              OPTIONAL,   -- Need R

...,

[[

cg-RetransmissionTimer-r16          INTEGER (1..64)                                                            OPTIONAL,   -- Need R

cg-minDFI-Delay-r16                 ENUMERATED

{sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14,

sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14,sym15x14, sym16x14

}                                                       OPTIONAL,   -- Need R

cg-nrofPUSCH-InSlot-r16             INTEGER (1..7)                                                  OPTIONAL,   -- Need R

cg-nrofSlots-r16                    INTEGER (1..40)                                                 OPTIONAL,   -- Need R

cg-StartingOffsets-r16              CG-StartingOffsets-r16                                          OPTIONAL,   -- Need R

cg-UCI-Multiplexing-r16             ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

cg-COT-SharingOffset-r16            INTEGER (1..39)                                                 OPTIONAL,   -- Need R

betaOffsetCG-UCI-r16                INTEGER (0..31)                                                 OPTIONAL,   -- Need R

cg-COT-SharingList-r16              SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16                 OPTIONAL,   -- Need R

harq-ProcID-Offset-r16              INTEGER (0..15)                                                 OPTIONAL,   -- Need M

harq-ProcID-Offset2-r16             INTEGER (0..15)                                                 OPTIONAL,   -- Need M

configuredGrantConfigIndex-r16      ConfiguredGrantConfigIndex-r16                                  OPTIONAL,   -- Cond CG-List

configuredGrantConfigIndexMAC-r16   ConfiguredGrantConfigIndexMAC-r16                               OPTIONAL,   -- Cond CG-IndexMAC

periodicityExt-r16                  INTEGER (1..5120)                                               OPTIONAL,   -- Need R

startingFromRV0-r16                 ENUMERATED {on, off}                                            OPTIONAL,   -- Need R

phy-PriorityIndex-r16               ENUMERATED {p0, p1}                                             OPTIONAL,   -- Need R

autonomousTx-r16                    ENUMERATED {enabled}                                OPTIONAL    -- Cond LCH-BasedPrioritization

]],

[[

cg-betaOffsetsCrossPri0-r17         SetupRelease { BetaOffsetsCrossPriSelCG-r17 }                   OPTIONAL,   -- Need M

cg-betaOffsetsCrossPri1-r17         SetupRelease { BetaOffsetsCrossPriSelCG-r17 }                   OPTIONAL,   -- Need M

mappingPattern-r17                  ENUMERATED {cyclicMapping, sequentialMapping}                   OPTIONAL,   -- Cond SRSsets

sequenceOffsetForRV-r17             INTEGER (0..3)                                                  OPTIONAL,   -- Need R

p0-PUSCH-Alpha2-r17                 P0-PUSCH-AlphaSetId                                             OPTIONAL,   -- Need R

powerControlLoopToUse2-r17          ENUMERATED {n0, n1}                                             OPTIONAL,   -- Need R

cg-COT-SharingList-r17              SEQUENCE (SIZE (1..50722)) OF CG-COT-Sharing-r17                OPTIONAL,   -- Need R

periodicityExt-r17                  INTEGER (1..40960)                                              OPTIONAL,   -- Need R

repK-v1710                          ENUMERATED {n12, n16, n24, n32}                                 OPTIONAL,   -- Need R

nrofHARQ-Processes-v1700            INTEGER(17..32)                                                 OPTIONAL,   -- Need M

harq-ProcID-Offset2-v1700           INTEGER (16..31)                                                OPTIONAL,   -- Need R

configuredGrantTimer-v1700          INTEGER(33..288)                                                OPTIONAL,   -- Need R

cg-minDFI-Delay-v1710               INTEGER (238..3584)                                             OPTIONAL    -- Need R

]],

[[

harq-ProcID-Offset-v1730            INTEGER (16..31)                                                OPTIONAL,   -- Need R

cg-nrofSlots-r17                    INTEGER (1..320)                                                OPTIONAL    -- Need R

]],

[[

disableCG-RetransmissionMonitoring-r18  ENUMERATED {true}                                           OPTIONAL,   -- Need R

nrofSlotsInCG-Period-r18            INTEGER (2..32)                                                 OPTIONAL,   -- Need R

uto-UCI-Config-r18                      SEQUENCE {

nrofBitsInUTO-UCI-r18               INTEGER (3..8),

betaOffsetUTO-UCI-r18               INTEGER (0..31),

...

}                                                                                                   OPTIONAL    -- Need R

]],

[[

precodingAndNumberOfLayers-v1850    INTEGER (64..1023)                                              OPTIONAL,   -- Need R

srs-ResourceIndicator-v1850         INTEGER (16..255)                                               OPTIONAL    -- Need R

]]

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

CG-COT-Sharing-r16 ::= CHOICE {

noCOT-Sharing-r16                   NULL,

cot-Sharing-r16                     SEQUENCE {

duration-r16                       INTEGER (1..39),

offset-r16                         INTEGER (1..39),

channelAccessPriority-r16          INTEGER (1..4)

}

}

CG-COT-Sharing-r17 ::=  CHOICE {

noCOT-Sharing-r17                   NULL,

cot-Sharing-r17                     SEQUENCE {

duration-r17                       INTEGER (1..319),

offset-r17                         INTEGER (1..319)

}

}

CG-StartingOffsets-r16 ::= SEQUENCE {

cg-StartingFullBW-InsideCOT-r16         SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingFullBW-OutsideCOT-r16        SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)             OPTIONAL,   -- Need R

cg-StartingPartialBW-InsideCOT-r16      INTEGER (0..6)                                       OPTIONAL,   -- Need R

cg-StartingPartialBW-OutsideCOT-r16     INTEGER (0..6)                                       OPTIONAL    -- Need R

}

BetaOffsetsCrossPriSelCG-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (1..4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17      BetaOffsetsCrossPri-r17

}

CG-SDT-Configuration-r17 ::= SEQUENCE {

cg-SDT-RetransmissionTimer   INTEGER (1..64)                                                 OPTIONAL,   -- Need R

sdt-SSB-Subset-r17       CHOICE {

shortBitmap-r17          BIT STRING (SIZE (4)),

mediumBitmap-r17         BIT STRING (SIZE (8)),

longBitmap-r17           BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

sdt-SSB-PerCG-PUSCH-r17   ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

sdt-P0-PUSCH-r17         INTEGER (-16..15)                                                   OPTIONAL, -- Need M

sdt-Alpha-r17            ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

sdt-DMRS-Ports-r17       CHOICE {

dmrsType1-r17            BIT STRING (SIZE (8)),

dmrsType2-r17            BIT STRING (SIZE (12))

}                                                                                            OPTIONAL,  -- Need M

sdt-NrofDMRS-Sequences-r17  INTEGER (1..2)                                                   OPTIONAL   -- Need M

}

CG-RRC-Configuration-r18 ::=   SEQUENCE {

cg-RRC-RetransmissionTimer-r18 INTEGER (1..288)                                               OPTIONAL,   -- Need R

cg-RRC-RSRP-ThresholdSSB-r18   RSRP-Range                                                    OPTIONAL,   -- Need R

rrc-SSB-Subset-r18             CHOICE {

shortBitmap-r18                BIT STRING (SIZE (4)),

mediumBitmap-r18               BIT STRING (SIZE (8)),

longBitmap-r18                 BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

rrc-SSB-PerCG-PUSCH-r18        ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

rrc-P0-PUSCH-r18               INTEGER (-16..15)                                             OPTIONAL,   -- Need M

rrc-Alpha-r18                  ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M

rrc-DMRS-Ports-r18             CHOICE {

dmrsType1-r18                  BIT STRING (SIZE (8)),

dmrsType2-r18                  BIT STRING (SIZE (12))

}                                                                                             OPTIONAL,  -- Need M

rrc-NrofDMRS-Sequences-r18  INTEGER (1..2)                                                    OPTIONAL,  -- Need M

...

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

ConfiguredGrantConfig field descriptions

antennaPort Indicates the antenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1. The UE ignores this field in case of CG-SDT or if cg-RRC-Configuration is configured.

applyIndicatedTCI-State This field indicates, for PUSCH transmission(s) corresponding a Type1-CG configuration, if UE applies the first, the second or both "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.1. Network does not configure this field if cg-RRC-Configuration is configured.

autonomousTx If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3].

betaOffsetCG-UCI Beta offset for CG-UCI in CG-PUSCH, see TS 38.213 [13], clause 9.3

betaOffsetUTO-UCI Beta offset value for UTO-UCI multiplexing on CG PUSCH, see TS 38.213 [13], clause 9.3.

cg-betaOffsetsCrossPri0, cg-betaOffsetsCrossPri1 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK in CG-PUSCH with different priorities. The field cg-betaOffsetsCrossPri0 indicates multiplexing LP HARQ-ACK in HP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p1. The field cg-betaOffsetsCrossPri1 indicates multiplexing HP HARQ-ACK in LP CG-PUSCH. This field is configured only if phy-PriorityIndex-r16 is configured with value p0.

cg-COT-SharingList Indicates a table for COT sharing combinations (see 37.213 [48], clause 4.1.3). One row of the table can be set to noCOT-Sharing to indicate that there is no channel occupancy sharing. If the cg-RetransmissionTimer-r16 is configured and the UE operates as an initiating device in semi-static channel access mode (see TS 37.213 [48], clause 4.3), then cg-COT-SharingList-r16 is configured.

cg-COT-SharingOffset Indicates the offset from the end of the slot where the COT sharing indication in UCI is enabled where the offset in symbols is equal to 14*n, where n is the signaled value for cg-COT-SharingOffset. Applicable when ul-toDL-COT-SharingED-Threshold-r16 is not configured (see 37.213 [48], clause 4.1.3).

cg-DMRS-Configuration DMRS configuration (see TS 38.214 [19], clause 6.1.2.3).

cg-minDFI-Delay Indicates the minimum duration (in unit of symbols) from the ending symbol of the PUSCH to the starting symbol of the PDCCH containing the downlink feedback indication (DFI) carrying HARQ-ACK for this PUSCH. The HARQ-ACK received before this minimum duration is not considered as valid for this PUSCH (see TS 38.213 [13], clause 10.5). The following minimum duration values are supported, depending on the configured subcarrier spacing [symbols]: 15 kHz: 7, m*14, where m = {1, 2, 3, 4} 30 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} 120 kHz: 7, m*14, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32} 480 kHz: m*14, where m = {2, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100, 104, 108, 112, 116, 120, 124, 128} 960 kHz: m*14, where m = {4, 8, 16, 24, 32, 40, 48, 56, 64, 72, 80, 88, 96, 104, 112, 120, 128, 136, 144, 152, 160, 168, 176, 184, 192, 200, 208, 216, 224, 232, 240, 248, 256}

cg-nrofPUSCH-InSlot Indicates the number of consecutive PUSCH configured to CG within a slot where the SLIV indicating the first PUSCH and additional PUSCH appended with the same length (see TS 38.214 [19], clause 6.1.2.3). The network can only configure this field if cg-RetransmissionTimer is configured.

cg-nrofSlots Indicates the number of allocated slots in a configured grant periodicity following the time instance of configured grant offset (see TS 38.214 [19], clause 6.1.2.3). cg-nrofSlots-r17 is only applicable for operation with shared spectrum channel access in FR2-2. When cg-nrofSlots-r17 is configured, the UE shall ignore cg-nrofSlots-r16. The network can only configure this field if cg-RetransmissionTimer is configured.

cg-RetransmissionTimer Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than or equal to the value of configuredGrantTimer. This field is always configured together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2. The network does not configure this field for CG-SDT.

cg-SDT-PeriodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 (see TS 38.321 [3], clause 5.8.2) for extended CG-SDT periodicities. If this field is present, the fields periodicity and periodicityExt are ignored. The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: n*14*1280, where n={1, 2, 4, 8, 48, 96, 240, 472, 944, 1408, 2816} 30 kHz: n*14*1280, where n={2, 4, 8, 16, 96, 192, 480, 944, 1888, 2816, 5632} 60 kHz with normal CP n*14*1280, where n={4, 8, 16, 32, 192, 384, 960, 1888, 3776, 5632,11264} 60 kHz with ECP: n*12*1280, where n={4, 8, 16, 32, 192, 384, 960, 1888, 3776, 5632,11264} 120 kHz: n*14*1280, where n={8, 16, 32, 64, 384, 768, 1920, 3776, 7552, 11264, 22528} 480 kHz: n*14*1280, where n={32, 64, 128, 256, 1536, 3072, 7680, 15104, 30208, 45056, 90112} 960 kHz: n*14*1280, where n={64, 128, 256, 512, 3072, 6144, 15360, 30208, 60416, 90112, 180224}

cg-StartingOffsets This field is not applicable for a UE which is allowed to operate as an initiating device in semi-static channel access mode, i.e., not applicable for a UE configured with UE FFP parameters (e.g. period, offset) regardless whether the UE would initiate its own COT or would share gNB's COT.

cg-UCI-Multiplexing If present, this field indicates that in the case of PUCCH overlapping with CG-PUSCH(s) including CG-UCI within a PUCCH group, HARQ-ACK is multiplexed on the CG-PUSCH including CG-UCI (see TS 38.213 [13], clause 9).

configuredGrantConfigIndex Indicates the index of the Configured Grant configurations within the BWP.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

disableCG-RetransmissionMonitoring When this field is configured, the UE does not start the drx-HARQ-RTT-TimerUL for PUSCH transmissions using configured uplink grants corresponding to this ConfiguredGrantConfig. See TS 38.321 [3], clause 5.7.

configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity. When cg-RetransmissonTimer is configured, if HARQ processes are shared among different configured grants on the same BWP, configuredGrantTimer * periodicity is set to the same value for the configurations that share HARQ processes on this BWP. The value of the extension configuredGrantTimer is 2 times the configured value.

dmrs-SeqInitialization The network configures this field if transformPrecoder is disabled or when the value of sdt-NrofDMRS-Sequences is set to 1. Otherwise, the field is absent.

frequencyDomainAllocation Indicates the frequency domain resource allocation, see TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1).

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured. The field frequencyHopping applies to configured grant for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3.1).

frequencyHoppingOffset Frequency hopping offset used when frequency hopping is enabled (see TS 38.214 [19], clause 6.1.2 and clause 6.3).

frequencyHoppingOffset-SBFD Configures the frequency hopping offset for Type 1 configured grant PUSCH in SBFD symbols (see TS 38.214 [19], 6).

frequencyHoppingPUSCH-RepTypeB Indicates the frequency hopping scheme for Type 1 CG when pusch-RepTypeIndicator is set to 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1). The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, the frequency hopping is not enabled for Type 1 CG.

harq-ProcID-Offset For operation with shared spectrum channel access configured with cg-RetransmissionTimer-r16, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)]. harq-ProcID-Offset-v1730 is only applicable for operation with shared spectrum channel access in FR2-2. If the field harq-ProcID-Offset-v1730 is present, the UE shall ignore the harq-ProcID-Offset-r16. The network does not configure this field for CG-SDT.

harq-ProcID-Offset2 Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured together with cg-RetransmissionTimer-r16. If the field harq-ProcID-Offset2-v1700 is present, the UE shall ignore the harq-ProcID-Offset2-r16.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' for PUSCH transmission with a Type 1 configured grant and/or a Type 2 configured grant as described in clause 6.1.2.3 of TS 38.214 [19]

mcs-Table Indicates the MCS table the UE shall use for PUSCH without transform precoding. If the field is absent the UE applies the value qam64.

mcs-TableTransformPrecoder Indicates the MCS table the UE shall use for PUSCH with transform precoding. If the field is absent the UE applies the value qam64.

mcsAndTBS The modulation order, target code rate and TB size (see TS 38.214 [19], clause 6.1.2). The NW does not configure the values 28~31 in this version of the specification.

nrofBitsInUTO-UCI Indicates the number of bits in the UTO-UCI bitmap (see TS 38.212 [17], clause 6.2.7, 6.3.2, TS 38.213 [13], clause 9.3.1, TS 38.214 [19], clause 5.2.3). When this field is configured, UTO-UCI is enabled for the UE.

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix). The network sets the value of this field to 1 when cg-LTM-Configuration is configured.

nrofSlotsInCG-Period Number of consecutive slots for CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, see TS 38.214 [19], clause 6.1. The network does not configure this field for operation on shared spectrum.

occ-LengthAndSequenceIndex Indicates the inter-slot OCC length and sequence index for a Type 1 CG configuration, see TS 38.214 [19].

pathlossReferenceIndex Indicates the reference signal index used as PUSCH pathloss reference (see TS 38.213 [13], clause 7.1.1). In case of CG-SDT or if cg-RRC-Configuration is configured or if enablePL-RS-UpdateForType1CG-PUSCH is configured or if unifiedTCI-StateType is configured, the UE ignores this field.

pathlossReferenceIndex2 Indicates the reference signal used as PUSCH pathloss reference for the second SRS resource set. When this field is present, pathlossReferenceIndex indicates the reference signal used as PUSCH pathloss reference for the first SRS resource set. Network does not configure this field if cg-RRC-Configuration is configured.

p0-PUSCH-Alpha Index of the P0-PUSCH-AlphaSet to be used for this configuration. If unifiedTCI-StateType is configured, the UE ignores this field.

p0-PUSCH-Alpha2 Index of the P0-PUSCH-AlphaSet to be used for second SRS resource set. If this field is present, the p0-PUSCH-Alpha provides index for the P0-PUSCH-AlphaSet to be used for first SRS resource set.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} 480 and 960 kHz: n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} In case of SDT, the network does not configure periodicity values less than 5ms.

periodicityExt This field is used to calculate the periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). If this field is present, the UE shall ignore field periodicity (without suffix). Network does not configure periodicityExt-r17 together with periodicityExt-r16. The following periodicites are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 640. 30 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP: periodicityExt*14, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: periodicityExt*12, where periodicityExt has a value between 1 and 2560. 120 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 5120. 480 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 20480. 960 kHz: periodicityExt*14, where periodicityExt has a value between 1 and 40960. In case of SDT, the network does not configure periodicity values less than 5ms.

phy-PriorityIndex Indicates the PHY priority of CG PUSCH at least for PHY-layer collision handling. Value p0 indicates low priority and value p1 indicates high priority. The network does not configure this for CG-SDT.

powerControlLoopToUse Closed control loop to apply (see TS 38.213 [13], clause 7.1.1). If unifiedTCI-StateType is configured, the UE ignores this field.

powerControlLoopToUse2 Closed control loop to apply to second SRS resource set (see TS 38.213 [13], clause 7.1.1). If this field is present, the powerControlLoopToUse applies to the first SRS resource set.

precodingAndNumberOfLayers, precodingAndNumberOfLayers-v1850 Indicates the precoding and number of layers (see TS 38.212 [17], clause 7.3.1.1.2, and TS 38.214 [19], clause 6.1.2.3). In case of CG-SDT or if cg-RRC-Configuration is configured, network sets this field to 1. Field precodingAndNumberOfLayers-v1850 is only configured when nrofSRS-Ports-n8 is configured. If the network configures precodingAndNumberOfLayers-v1850, the UE shall ignore precodingAndNumberOfLayers (without suffix).

precodingAndNumberOfLayers2 Indicates the precoding and number of layers for the second SRS resource set. When this field is present, precodingAndNumberOfLayers indicated the precoding and number of layers for the first SRS resource set. Network does not configure this field if cg-RRC-Configuration is configured.

pusch-MutingResources Used to configure the time location and frequency location of UL muting resources for Type 1 configured grant PUSCH transmission, see clause 6 in TS 38.211 [16] and clause 6 in TS 38.214 [19].

pusch-RepTypeIndicator Indicates whether UE follows the behavior for PUSCH repetition type A or the behavior for PUSCH repetition type B for each Type 1 configured grant configuration. The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B' (see TS 38.214 [19], clause 6.1.2.3). The value pusch-RepTypeB is not configured simultaneously with nrofSlotsInCG-Period-r18. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation with shared spectrum channel access.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent. Note: rbg-Size is used when the transformPrecoder parameter is disabled.

repK-RV The redundancy version (RV) sequence to use. See TS 38.214 [19], clause 6.1.2. The network configures this field if repetitions are used, i.e., if repK is set to n2, n4 or n8. This field is not configured when cg-RetransmissionTimer is configured. Otherwise, the field is absent.

repK Number of repetitions K, see TS 38.214 [19]. If the field repK-v1710 is present, the UE shall ignore the repK (without suffix).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1. For Type 1 UL data transmission without grant, resourceAllocation should be resourceAllocationType0 or resourceAllocationType1.

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2).

sbfd-Config2-PUSCH-RB-Offset Indicates the RB offset to determine the starting PRB for Type1 configured grant PUSCH transmissions in SBFD symbols when the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 6).

sequenceOffsetForRV Configures the RV offset for the starting RV for the first repetition (first actual repetition in PUSCH repetition Type B) towards the second 'SRS resource set' for PUSCH configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

srs-ResourceSetId Indicates the associated SRS resource set for PUSCH+PUSCH simultaneous uplink transmsision for CG-type 1 PUSCH. Network does not configure this field if cg-RRC-Configuration is configured.

srs-ResourceIndicator, srs-ResourceIndicator-v1850 Indicates the SRS resource to be used (see TS 38.212 [17], clause 7.3.1.1.2, and TS 38.214 [19], clause 6.1.2.3). The network does not configure this for CG-SDT or if cg-RRC-Configuration is configured. Field srs-ResourceIndicator-v1850 is only configured when 8 antenna ports are configured (see TS 38.214 [19], clause 6.1.1.2). The network does not configure both srs-ResourceIndicator and srs-ResourceIndicator-v1850.

srs-ResourceIndicator2 Indicates the SRS resource to be used for the second SRS resource set. When this field is present, the srs-ResourceIndicator is used for the first SRS resource set. Network does not configure this field if cg-RRC-Configuration is configured.

startingFromRV0 This field is used to determine the initial transmission occasion of a transport block for a given RV sequence, see TS 38.214 [19], clause 6.1.2.3.1. The network does not configure this field if cg-RetransmissionTimer-r16 is configured for CG operation.

symbolType Configures the valid symbol type for Type 1 CG PUSCH when the transmissions are restricted to SBFD symbols only or non-SBFD symbols only. The network does not configure this field if the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 6)

timeDomainAllocation, timeDomainAllocation-v1710 Indicates a combination of start symbol and length and PUSCH mapping type, see TS 38.214 [19], clause 6.1.2 and TS 38.212 [17], clause 7.3.1. If the field timeDomainAllocation-v1710 is present, the UE shall ignore timeDomainAllocation field (without suffix).

timeDomainOffset Offset related to the reference SFN indicated by timeReferenceSFN, see TS 38.321 [3], clause 5.8.2. timeDomainOffset-r17 is only applicable to 480 kHz and 960 kHz. If timeDomainOffset-r17 is present, the UE shall ignore timeDomainOffset (without suffix).

timeReferenceHyperSFN Indicates H-SFN used for determination of the offset of a resource in time domain. The UE uses the closest H-SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceHyperSFN is not present, the reference hyper SFN is 0.

timeReferenceSFN Indicates SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceSFN is not present, the reference SFN is 0.

transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList), see TS 38.214 [19], clause 6.1.3.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic. The network does not configure this for CG-SDT.

CG-COT-Sharing field descriptions

channelAccessPriority Indicates the Channel Access Priority Class that the gNB can assume when sharing the UE initiated COT (see 37.213 [48], clause 4.1.3).

duration Indicates the number of DL transmission slots within UE initiated COT (see 37.213 [48], clause 4.1.3).

offset Indicates the number of DL transmission slots from the end of the slot where CG-UCI is detected after which COT sharing can be used (see 37.213 [48], clause 4.1.3).

CG-StartingOffsets field descriptions

cg-StartingFullBW-InsideCOT A set of configured grant PUSCH transmission starting offsets (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingFullBW-OutsideCOT A set of configured grant PUSCH transmission starting offset indices (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation includes all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-InsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is inside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

cg-StartingPartialBW-OutsideCOT A set of configured grant PUSCH transmission starting offset index (see TS 38.211[16], Table 5.3.1-2) which indicates the length of a CP extension of the first symbol that is located before the configured resource when frequency domain resource allocation does not include all interlaces in the allocated RB set(s) and the CG PUSCH resource is outside gNB COT (see TS 38.214 [19], clause 6.1.2.3).

CG-SDT-Configuration and CG-RRC-Configuration field descriptions

cg-RRC-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for the CG as specified in TS 38.321 [3]. This field is absent in cg-LTM-Configuration.

cg-SDT-RetransmissionTimer, cg-RRC-RetransmissionTimer Indicates the initial value of the configured grant retransmission timer used for the initial transmission of CG with CCCH (for CG-SDT) or DCCH message (see TS 38.321 [3]) in multiples of periodicity. The field cg-RRC-RetransmissionTimer is not configured together with the field harq-ProcID-Offset for operations in unlicensed spectrum.

sdt-DMRS-Ports, rrc-DMRS-Ports Indicates the set of DMRS ports for SSB to PUSCH mapping (see TS 38.213 [13]). The first (left-most / most significant) bit corresponds to DMRS port 0, the second most significant bit corresponds to DMRS port 1, and so on. A bit set to 1 indicates that this DMRS port is used for mapping. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-NrofDMRS-Sequences, rrc-NrofDMRS-Sequences Indicates the number of DMRS sequences for SSB to PUSCH mapping (see TS 38.213 [13]). In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-SSB-Subset, rrc-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not included in the SSB subset for SSB to CG PUSCH mapping while value 1 indicates that the corresponding SS/PBCH block is included in SSB subset for SSB to CG PUSCH mapping. If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-SSB-PerCG-PUSCH, rrc-SSB-PerCG-PUSCH The number of SSBs per CG PUSCH (see TS 38.213 [13]). Value one corresponds to 1 SSBs per CG PUSCH, value two corresponds to 2 SSBs per CG PUSCH and so on. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

sdt-P0-PUSCH, rrc-P0-PUSCH Indicates P0 value for PUSCH in steps of 1dB (see TS 38.213 [13]). When this field is configured, the UE ignores the p0-PUSCH-Alpha. This field is absent in cg-LTM-Configuration.

sdt-Alpha, rrc-Alpha Indicates alpha value for PUSCH. alpha0 indicates value 0 is used, alpha04 indicates value 4 is used and so on (see TS 38.213 [13]). When this field is configured, the UE ignores the p0-PUSCH-Alpha. This field is absent in cg-LTM-Configuration.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-BasedPrioritization | This field is optionally present, Need R, if lch-BasedPrioritization is configured in the MAC entity. It is absent otherwise. |
| RACH-LessHO | The field is optionally present, Need N, if rach-LessHO is present in reconfigurationWithSync. It is absent otherwise. |
| RepTypeB | The field is optionally present if pusch-RepTypeIndicator is set to pusch-RepTypeB, Need S, and absent otherwise. |
| CG-List | The field is mandatory present when included in configuredGrantConfigToAddModList-r16, otherwise the field is absent. |
| CG-IndexMAC | The field is mandatory present if at least one configured grant is configured by configuredGrantConfigToAddModList-r16 in any BWP of this MAC entity, otherwise it is optionally present, need R. |
| CG-SDT1 | This field is optionally present, Need R, if cg-SDT-Configuration is configured, otherwise it is absent. |
| CG-SDT2 | This field is optionally present, Need S, if cg-SDT-PeriodicityExt is configured, otherwise it is absent. |
| LTM | The field is optionally present, Need R, in an RRCReconfiguration message within the LTM-Config IE. Otherwise, the field is absent. |
| SRSsets | This field is mandatory present when UE is configured with two SRS sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage codebook or non-codebook and none of multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel is configured. Otherwise it is absent, Need R |

#### –	ConfiguredGrantConfigIndex

The IE ConfiguredGrantConfigIndex is used to indicate the index of one of multiple UL Configured Grant configurations in one BWP.

ConfiguredGrantConfigIndex information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIGINDEX-START

ConfiguredGrantConfigIndex-r16 ::= INTEGER (0.. maxNrofConfiguredGrantConfig-1-r16)

-- TAG-CONFIGUREDGRANTCONFIGINDEX-STOP

-- ASN1STOP

#### –	ConfiguredGrantConfigIndexMAC

The IE ConfiguredGrantConfigIndexMAC is used to indicate the unique Configured Grant configurations index per MAC entity.

ConfiguredGrantConfigIndexMAC information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIGINDEXMAC-START

ConfiguredGrantConfigIndexMAC-r16 ::= INTEGER (0.. maxNrofConfiguredGrantConfigMAC-1-r16)

-- TAG-CONFIGUREDGRANTCONFIGINDEXMAC-STOP

-- ASN1STOP

#### –	ConnEstFailureControl

The IE ConnEstFailureControl is used to configure parameters for connection establishment failure control.

ConnEstFailureControl information element

-- ASN1START

-- TAG-CONNESTFAILURECONTROL-START

ConnEstFailureControl ::=   SEQUENCE {

connEstFailCount                    ENUMERATED {n1, n2, n3, n4},

connEstFailOffsetValidity           ENUMERATED {s30, s60, s120, s240, s300, s420, s600, s900},

connEstFailOffset                   INTEGER (0..15)                                                         OPTIONAL    -- Need S

}

-- TAG-CONNESTFAILURECONTROL-STOP

-- ASN1STOP

ConnEstFailureControl field descriptions

connEstFailCount Number of times that the UE detects T300 expiry on the same cell before applying connEstFailOffset.

connEstFailOffset Parameter "Qoffsettemp" in TS 38.304 [20]. If the field is absent, the value of infinity shall be used for "Qoffsettemp".

connEstFailOffsetValidity Amount of time that the UE applies connEstFailOffset before removing the offset from evaluation of the cell. Value s30 corresponds to 30 seconds, value s60 corresponds to 60 seconds, and so on.

#### –	ControlResourceSet

The IE ControlResourceSet is used to configure a time/frequency control resource set (CORESET) in which to search for downlink control information (see TS 38.213 [13], clause 10.1). For the UE not supporting multipleCORESET in FR1, in order to receive MBS multicast in CFR within the UE's active BWP, if a CORESET is not configured within the PDCCH-ConfigMulticast, the CORESET other than CORESET#0 configured within the UE's active BWP for scheduling unicast can be used for scheduling MBS multicast, and the CORESET is expected to be included completely within the CFR and the parameters configured in the CORESET are expected to be supported by the UE for MBS multicast.

ControlResourceSet information element

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::=              SEQUENCE {

controlResourceSetId                ControlResourceSetId,

frequencyDomainResources            BIT STRING (SIZE (45)),

duration                            INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType                 CHOICE {

interleaved                         SEQUENCE {

reg-BundleSize                      ENUMERATED {n2, n3, n6},

interleaverSize                     ENUMERATED {n2, n3, n6},

shiftIndex                          INTEGER(0..maxNrofPhysicalResourceBlocks-1)       OPTIONAL -- Need S

},

nonInterleaved                      NULL

},

precoderGranularity                 ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList           SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP

tci-StatesPDCCH-ToReleaseList       SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP

tci-PresentInDCI                        ENUMERATED {enabled}                                  OPTIONAL, -- Need S

pdcch-DMRS-ScramblingID                 INTEGER (0..65535)                                    OPTIONAL, -- Need S

...,

[[

rb-Offset-r16                       INTEGER (0..5)                                            OPTIONAL, -- Need S

tci-PresentDCI-1-2-r16              INTEGER (1..3)                                            OPTIONAL, -- Need S

coresetPoolIndex-r16                INTEGER (0..1)                                            OPTIONAL, -- Need S

controlResourceSetId-v1610          ControlResourceSetId-v1610                                OPTIONAL  -- Need S

]],

[[

followUnifiedTCI-State-r17          ENUMERATED {enabled}                                      OPTIONAL  -- Need R

]],

[[

applyIndicatedTCI-State-r18         ENUMERATED {first, second, both,  none}                   OPTIONAL  -- Cond FollowUTCI

]]

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

ControlResourceSet field descriptions

applyIndicatedTCI-State This field indicates, for PDCCH reception on this CORESET, if UE applies the first, the second, both or none "indicated" DL only TCI or joint TCI as specified in TS 38.213 [13], clause 10.1. Value both can be configured only if sfnSchemePDCCH is configured in the serving cell.

cce-REG-MappingType Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG) (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

controlResourceSetId Identifies the instance of the ControlResourceSet IE. Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon (controlResourceSetZero) and is hence not used here in the ControlResourceSet IE. Other values identify CORESETs configured by dedicated signalling or in SIB1 or SIB20. The controlResourceSetId is unique among the BWPs of a serving cell. If the field controlResourceSetId-v1610 is present, the UE shall ignore the controlResourceSetId field (without suffix).

coresetPoolIndex The index of the CORESET pool for this CORESET as specified in TS 38.213 [13] (clauses 9 and 10) and TS 38.214 [19] (clauses 5.1 and 6.1). If the field is absent, the UE applies the value 0.

duration Contiguous time duration of the CORESET in number of symbols (see TS 38.211 [16], clause 7.3.2.2).

followUnifiedTCI-State When set to enabled, for PDCCH reception on this CORESET, the UE applies the "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from the first RB group in the BWP or MBS CFR where the CORESET is configured. When at least one search space is configured with freqMonitorLocation-r16, only the first NRBG,set0size bits are valid (see TS 38.213 [13], clause 10.1). The first (left-most / most significant) bit corresponds to the first RB group in the BWP or MBS CFR where the CORESET is configured, and so on. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero (see TS 38.211 [16], clause 7.3.2.2).

interleaverSize Interleaver-size (see TS 38.211 [16], clause 7.3.2.2).

pdcch-DMRS-ScramblingID PDCCH DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.3.1). When the field is absent the UE applies the value of the physCellId configured for this serving cell.

precoderGranularity Precoder granularity in frequency domain (see TS 38.211 [16], clauses 7.3.2.2 and 7.4.1.3.2).

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1).

reg-BundleSize Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles (see TS 38.211 [16], clause 7.3.2.2).

shiftIndex When the field is absent the UE applies the value of the physCellIdconfigured for this serving cell (see TS 38.211 [16], clause 7.3.2.2).

tci-PresentInDCI This field indicates if TCI field is present or absent in DCI format 1_1, DCI format 1_3 and DCI format 4_2. When the field is absent the UE considers the TCI to be absent/disabled. In case of cross carrier scheduling, the network sets this field to enabled for the ControlResourceSet used for cross carrier scheduling in DCI format 1_1 in the scheduling cell if enableDefaultBeamForCCS is not configured (see TS 38.214 [19], clause 5.1.5).

tci-PresentDCI-1-2 Configures the number of bits for "Transmission configuration indicator" in DCI format 1_2. When the field is absent the UE applies the value of 0 bit for the "Transmission configuration indicator" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.5). In case of cross carrier scheduling, the network configures this field for the ControlResourceSet used for cross carrier scheduling in DCI format 1_2 in the scheduling cell if enableDefaultBeamForCCS is not configured (see TS 38.214 [19], clause 5.1.5).

tci-StatesPDCCH-ToAddList A subset of the TCI states defined in pdsch-Config, either with tci-StatesToAddModList or dl-OrJointTCI-StateList, included in the BWP-DownlinkDedicated corresponding to the serving cell and to the DL BWP to which the ControlResourceSet belong to. They are used for providing QCL relationships between the DL RS(s) in one RS Set (TCI-State) and the PDCCH DMRS ports (see TS 38.213 [13], clause 6.). The network configures at most maxNrofTCI-StatesPDCCH entries. The QCL relationships defined herein do not apply to MBS broadcast.

| Conditional Presence | Explanation |
| --- | --- |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present or if more than one value for the field coresetPoolIndex is configured in controlResourceSet for the same bandwidthpart. Otherwise, it is optionally present, Need R. |
| NotSIB-initialBWP | The field is absent in SIB1/SIB20 and in the PDCCH-ConfigCommon of the initial BWP in ServingCellConfigCommon, if SIB1/SIB20 is broadcasted. Otherwise, it is optionally present, Need N. |

#### –	ControlResourceSetId

The ControlResourceSetId IE concerns a short identity, used to identify a control resource set within a serving cell. The ControlResourceSetId = 0 identifies the ControlResourceSet#0 configured via PBCH (MIB) and in controlResourceSetZero (ServingCellConfigCommon). The ID space is used across the BWPs and MBS CFRs of a Serving Cell.

ControlResourceSetId information element

-- ASN1START

-- TAG-CONTROLRESOURCESETID-START

ControlResourceSetId ::=                INTEGER (0..maxNrofControlResourceSets-1)

ControlResourceSetId-r16 ::=            INTEGER (0..maxNrofControlResourceSets-1-r16)

ControlResourceSetId-v1610 ::=          INTEGER (maxNrofControlResourceSets..maxNrofControlResourceSets-1-r16)

-- TAG-CONTROLRESOURCESETID-STOP

-- ASN1STOP

#### –	ControlResourceSetZero

The IE ControlResourceSetZero is used to configure CORESET#0 of the initial BWP (see TS 38.213 [13], clause 13).

ControlResourceSetZero information element

-- ASN1START

-- TAG-CONTROLRESOURCESETZERO-START

ControlResourceSetZero ::=                  INTEGER (0..15)

-- TAG-CONTROLRESOURCESETZERO-STOP

-- ASN1STOP

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CROSSCARRIERSCHEDULINGCONFIG-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...,

[[

carrierIndicatorSize-r16            SEQUENCE {

carrierIndicatorSizeDCI-1-2-r16        INTEGER (0..3),

carrierIndicatorSizeDCI-0-2-r16        INTEGER (0..3)

}                                                                                       OPTIONAL,  -- Cond CIF-PRESENCE

enableDefaultBeamForCCS-r16         ENUMERATED {enabled}                                OPTIONAL  -- Need S

]],

[[

ccs-BlindDetectionSplit-r17         ENUMERATED {oneSeventh, threeFourteenth, twoSeventh, threeSeventh,

oneHalf, fourSeventh, fiveSeventh, spare1}      OPTIONAL  -- Need R

]]

}

-- TAG-CROSSCARRIERSCHEDULINGCONFIG-STOP

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

carrierIndicatorSizeDCI-0-2, carrierIndicatorSizeDCI-1-2 Configures the number of bits for the field of carrier indicator in PDCCH DCI format 0_2/1_2. The field carrierIndicatorSizeDCI-0-2 refers to DCI format 0_2 and the field carrierIndicatorSizeDCI-1-2 refers to DCI format 1_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).

ccs-BlindDetectionSplit Indicates the share of blind detection candidates and non-overlapping CCEs for PDCCH monitoring on an SpCell and an SCell when cross-carrier scheduling is configured from the SCell for the SpCell (see TS 38.213 [13], clause 10.1.1). The network only configures this field when it sets the field other for an SpCell, i.e., when it configures cross-carrier scheduling of the SpCell by a PDCCH on an Scell.

cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13]. If configured for an SpCell, the non-fallback DCI formats on the SpCell include same number of CIF bits as the corresponding non-fallback DCI formats on the scheduling cell, and the CIF bits are considered reserved.

enableDefaultBeamForCCS This field indicates whether default beam selection for cross-carrier scheduled PDSCH or aperiodic CSI-RS is enabled, see TS 38.214 [19]. If not present, the default beam selection behaviour is not applied, i.e. Rel-15 behaviour is applied. This field can only be configured in the cross-scheduled SCell or SpCell.

other Parameters for cross-carrier scheduling. If configured for an SpCell, the SpCell can be scheduled by the PDCCH on another SCell as well as by the PDCCH on the SpCell. If configured for an SCell, the SCell is scheduled by a PDCCH on another cell.

own Parameters for self-scheduling, i.e., a serving cell is scheduled by its own PDCCH.

schedulingCellId If configured for an SpCell, this field indicates which SCell, in addition to the SpCell, signals the downlink allocations and uplink grants, if applicable, for the concerned SpCell. If configured for an Scell, this field indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell. In case the UE is configured with two PUCCH groups, the scheduling cell and the scheduled cell are within the same PUCCH group. If drx-ConfigSecondaryGroup is configured in the MAC-CellGroupConfig associated with this serving cell, the scheduling cell and the scheduled cell belong to the same Frequency Range. In addition, the serving cell with an aperiodic CSI trigger and the PUSCH resource scheduled for the report are on the same carrier and serving cell, but the cell for which CSI is reported may belong to the same or a different Frequency Range. The network should not trigger a CSI request for a serving cell in the other Frequency Range when that serving cell is outside Active Time.

| Conditional Presence | Explanation |
| --- | --- |
| CIF-PRESENCE | The field is mandatory present if the cif-Presence is set to true. The field is absent otherwise. |

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of aperiodic trigger states. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

CSI-AperiodicTriggerStateList information element

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]],

[[

delayOffsetCompensation-r19         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]]

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)               OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)          OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...,

[[

resourcesForChannel2-r17        CHOICE {

nzp-CSI-RS2-r17                 SEQUENCE {

resourceSet2-r17                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info2-r17                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Cond Aperiodic

},

csi-SSB-ResourceSet2-r17        INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)

}                                                                                             OPTIONAL,  -- Need R

csi-SSB-ResourceSetExt          INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)          OPTIONAL   -- Need R

]],

[[

resourcesForChannelTDCP-r18     SEQUENCE {

resourceSet2TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

resourceSet3TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)              OPTIONAL  -- Need R

}                                                                                                     OPTIONAL,  -- Cond TDCP

applyIndicatedTCI-State-r18     CHOICE {

perSet-r18                      ENUMERATED {first, second},

perResource-r18                 SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF ENUMERATED {first, second}

}                                                                                                OPTIONAL, -- Cond ApplyIndicatedTCI

applyIndicatedTCI-State2-r18    CHOICE {

perSet-r18                     ENUMERATED {first, second},

perResource-r18                SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF  ENUMERATED {first, second}

}                                                                                                     OPTIONAL, -- Cond SecondCSICMR

csi-ReportSubConfigTriggerList-r18  CSI-ReportSubConfigTriggerList-r18                                OPTIONAL   -- Need R

]],

[[

resourcesForChannelCLI-r19      SEQUENCE {

resourceSetCLI-r19              INTEGER (1..maxNrofCLI-MeasResourceSetsPerConfig-r19)                       OPTIONAL,  -- Need R

qcl-infoCLI-r19                 SEQUENCE (SIZE(1..maxNrofCLI-MeasResourceSetsPerConfig-r19)) OF TCI-StateId OPTIONAL   -- Need R

},

resourcesForChannelCJTC-r19     SEQUENCE {

resourceSet2CJTC-r19        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

resourceSet3CJTC-r19        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)              OPTIONAL,  -- Need R

resourceSet4CJTC-r19        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)              OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Cond CJTC

mr-SelectedResources-r19         SEQUENCE {

firstSelectedResource-r19    INTEGER (1..8),

secondSelectedResource-r19   INTEGER (1..8)                                                   OPTIONAL  -- Need R

}                                                                                                     OPTIONAL  -- Need R

]]

}

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

CSI-AperiodicTriggerState field descriptions

delayOffsetCompensation Indicates whether the UE should perform delay offset compensation based on the latest linked CJTC-Dd report when codebook type is set to typeII-CJT as specified in TS 38.214 [19] clause 5.2.1.4.2.

ltm-AssociatedReportConfigInfo This field configures the aperiodic CSI reports of LTM candidate cells. If ltm-associatedReportConfigInfo is configured the UE shall ignore the field associatedReportConfigInfoList.

CSI-AssociatedReportConfigInfo field descriptions

ap-CSI-MultiplexingMode Indicates if the behavior of transmitting aperiodic CSI on the first PUSCH repetitions corresponding to two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' is enabled or not.

applyIndicatedTCI-State, applyIndicatedTCI-State2 This field indicates, for an aperiodic CSI-RS resource set (perSet) or for CSI-RS resource (perResource), if UE applies the first or the second "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.2.1.5.1. The applyIndicatedTCI-State is for ResourcesForChannel, and applyIndicatedTCI-State2 is for ResourcesForChannels2. When applyIndicatedTCI-State and applyIndicatedTCI-State2 are absent, the UE shall use qcl-info for ResourcesForChannel and use qcl-info2 for ResourcesForChannel2.

csi-IM-ResourcesForInterference CSI-IM-ResourceSet for interference measurement. Entry number in csi-IM-ResourceSetList in the CSI-ResourceConfig indicated by csi-IM-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on). The indicated CSI-IM-ResourceSet should have exactly the same number of resources like the NZP-CSI-RS-ResourceSet indicated in resourceSet within nzp-CSI-RS.

csi-ReportSubConfigTriggerList A list of sub-configuration ID(s) of N sub-configurations out of L configured sub-configurations within a CSI-ReportConfig associated with a triggering state for aperiodic CSI reporting on PUSCH.

csi-SSB-ResourceSet, csi-SSB-ResourceSet2 CSI-SSB-ResourceSet for channel measurements. Entry number in csi-SSB-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

mr-SelectedResources Indicates one or two resources to be selected among eight CSI-RS resources. This field is only configured for codebookType set to typeI-SinglePanel-r19 or etypeII-r19. This field is used in clause 5.2.1.4.2 in TS 38.214 [19].

nzp-CSI-RS-ResourcesForInterference NZP-CSI-RS-ResourceSet for interference measurement. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by nzp-CSI-RS-ResourcesForInterference in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

qcl-info, qcl-info2 Fields qcl-Info and qcl-Info2 are lists of references to TCI-States for providing the QCL source and QCL type for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet, indicated by resourceSet within nzp-CSI-RS and by resourceSet2 within nzp-CSI-RS2, respectively. Each TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resourcesForChannelMeasurement (in the CSI-ReportConfig indicated by reportConfigId above) belong to. First entry in qcl-info/qcl-info2 corresponds to first entry in nzp-CSI-RS-Resources of that NZP-CSI-RS-ResourceSet, second entry in qcl-info/qcl-info2 corresponds to second entry in nzp-CSI-RS-Resources, and so on (see TS 38.214 [19], clause 5.2.1.5.1). When this field is absent for aperiodic CSI RS, and applyIndicatedTCI-State or applyIndicatedTCI-State2 is not configured, the UE shall use QCL information included in the "indicated" DL only/Joint TCI state as specified in TS 38.214 [19].

qcl-infoCLI List of references to TCI-States for providing the QCL source and QCL type for each CLI measurement resources listed in CLI measurement resource set indicated by resourceSetCLI. Each TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resourcesForChannelMeasurement (in the CSI-ReportConfig indicated by reportConfigId above) belong to. First entry in qcl-infoCLI corresponds to first entry in CLI measurement resources of that CLI measurement resource set, second entry in qcl-infoCLI corresponds to second entry in CLI measurement resources, and so on. The length of this list is the same as the number of resources in the CLI measurement resource set.

reportConfigId The reportConfigId of one of the CSI-ReportConfigToAddMod configured in CSI-MeasConfig.

resourcesForChannel2 Configures reference signals for channel measurement corresponding to the second resource set for L1-RSRP measurement as configured in IE CSI-ResourceConfig when nrofReportedGroups-r17 is configured in IE CSI-ReportConfig. If this is present, network configures csi-SSB-ResourceSetExt instead of csi-SSB-ResourceSet and the UE ignores csi-SSB-ResourceSet in resourcesForChannel, and the resourcesForChannel configures the reference signals for channel measurement corresponding to the first resource set for L1-RSRP measurement (see TS 38.214 [19], clause 5.2.1.4).

resourcesForChannelCJTC Configures reference signals for channel measurement corresponding to the second resource set, the third resource set and the fourth resource set as specified in clause 5.2.1.4.1 in TS 38.214 [19].

resourcesForChannelCLI Configures CLI measurement resource set and the list of corresponding references to TCI-States. If present, the following fields should be ignored by the UE: resourcesForChannel, csi-IM-ResourcesForInterference, nzp-CSI-RS-ResourcesForInterference, resourcesForChannel2-r17, resourcesForChannelTDCP-r18, applyIndicatedTCI-State-r18, csi-ReportSubConfigTriggerList-r18.

resourcesForChannelTDCP Configures reference signals for channel measurement corresponding to the second resource set and third resource set for TDCP reporting. All CSI resources of these two resource sets always share the same QCL-info with the resource sets indicated by resourcesForChannel as spcified in TS 38.214 [19].

resourceSet, resourceSet2 NZP-CSI-RS-ResourceSet for channel measurements. Entry number in nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

resourceSetCLI CLI measurement resource set for L1 UE-to-UE CLI measurements. Entry number in cli-MeasurementResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on).

| Conditional Presence | Explanation |
| --- | --- |
| Aperiodic | The field is mandatory present if the NZP-CSI-RS-Resources in the associated resourceSet have the resourceType aperiodic and unifiedTCI-StateType is not configured. The field is optionally present, Need R, if the NZP-CSI-RS-Resources in the associated resourceSet have the resourceType aperiodic and unifiedTCI-StateType is configured. The field is absent otherwise. |
| ApplyIndicatedTCI | The field is optionally present, need R, if resourceForChannel has aperiodic CSI-RS and unifiedTCI-StateType is configured. This field is absent otherwise. |
| CJTC | This field is absent if resourcesForChannel2 or resourcesForChannelTDCP is configured. It is optionally present, Need R, otherwise. |
| CSI-IM-ForInterference | This field is mandatory present if the CSI-ReportConfig identified by reportConfigId is configured with csi-IM-ResourcesForInterference; otherwise it is absent. |
| NZP-CSI-RS-ForInterference | This field is mandatory present if the CSI-ReportConfig identified by reportConfigId is configured with nzp-CSI-RS-ResourcesForInterference; otherwise it is absent. |
| SecondCSICMR | This field is mandatory present if resourcesForChannel2 is configured with aperiodic CSI-RS and applyIndicatedTCI-State is configured. It is absent otherwise. |
| TDCP | This field is absent if resourcesForChannel2 or resourcesForChannelCJTC is configured. It is optionally present, Need R, otherwise. |

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

CSI-FrequencyOccupation field descriptions

nrofRBs Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 24 and the width of the associated BWP. If the configured value is larger than the width of the corresponding BWP, the UE shall assume that the actual CSI-RS bandwidth is equal to the width of the BWP.

startingRB PRB where this CSI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Only multiples of 4 are allowed (0, 4, ...)

#### –	CSI-IM-Resource

The IE CSI-IM-Resource is used to configure one CSI Interference Management (IM) resource.

CSI-IM-Resource information element

-- ASN1START

-- TAG-CSI-IM-RESOURCE-START

CSI-IM-Resource ::=                 SEQUENCE {

csi-IM-ResourceId                   CSI-IM-ResourceId,

csi-IM-ResourceElementPattern           CHOICE {

pattern0                                SEQUENCE {

subcarrierLocation-p0                   ENUMERATED { s0, s2, s4, s6, s8, s10 },

symbolLocation-p0                       INTEGER (0..12)

},

pattern1                                SEQUENCE {

subcarrierLocation-p1                   ENUMERATED { s0, s4, s8 },

symbolLocation-p1                       INTEGER (0..13)

}

}                                                                                   OPTIONAL,   -- Need M

freqBand                            CSI-FrequencyOccupation                         OPTIONAL,   -- Need M

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

...

}

-- TAG-CSI-IM-RESOURCE-STOP

-- ASN1STOP

CSI-IM-Resource field descriptions

csi-IM-ResourceElementPattern The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters (see TS 38.214 [19], clause 5.2.2.4)

freqBand Frequency-occupancy of CSI-IM (see TS 38.214 [19], clause 5.2.2.4)

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent CSI-IM. Network always configures the UE with a value for this field for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). A change of configuration between periodic or semi-persistent and aperiodic for a CSI-IM-Resource is not supported without a release and add.

subcarrierLocation-p0 OFDM subcarrier occupancy of the CSI-IM resource for Pattern0 (see TS 38.214 [19], clause 5.2.2.4)

subcarrierLocation-p1 OFDM subcarrier occupancy of the CSI-IM resource for Pattern1 (see TS 38.214 [19], clause 5.2.2.4)

symbolLocation-p0 OFDM symbol location of the CSI-IM resource for Pattern0 (see TS 38.214 [19], clause 5.2.2.4)

symbolLocation-p1 OFDM symbol location of the CSI-IM resource for Pattern1 (see TS 38.214 [19], clause 5.2.2.4)

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent CSI-IM-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::=               INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSet

The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSet information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESET-START

CSI-IM-ResourceSet ::=              SEQUENCE {

csi-IM-ResourceSetId                CSI-IM-ResourceSetId,

csi-IM-Resources                    SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId,

...

}

-- TAG-CSI-IM-RESOURCESET-STOP

-- ASN1STOP

CSI-IM-ResourceSet field descriptions

csi-IM-Resources CSI-IM-Resources associated with this CSI-IM-ResourceSet (see TS 38.214 [19], clause 5.2).

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=            INTEGER (0..maxNrofCSI-IM-ResourceSets-1)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-LoggedMeasurementConfig

The IE CSI-LoggedMeasurementConfig is used to configure a CSI logged measurement configuration. It defines a group of one or more CSI resources for which the UE logs the associated L1 radio measurements.

CSI-LoggedMeasurementConfig information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-START

CSI-LoggedMeasurementConfig-r19 ::=             SEQUENCE {

csi-LoggedMeasurementConfigId-r19               CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19                    CSI-ResourceConfigId,

loggingPeriodicity-r19                          ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}    OPTIONAL,  -- Need M

csi-LoggedMeasurementEventTriggerConfig-r19     CSI-LoggedMeasurementEventTriggerConfig-r19                    OPTIONAL,  -- Need R

...

}

CSI-LoggedMeasurementEventTriggerConfig-r19 ::= SEQUENCE {

threshold-r19                                   CHOICE {

aboveThreshold-r19                              MeasTriggerQuantity,

belowThreshold-r19                              MeasTriggerQuantity

},

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

...

}

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-STOP

-- ASN1STOP

CSI-LoggedMeasurementConfig field descriptions

csi-LoggedMeasurementConfigId This field indicates the instance of CSI-LoggedMeasurementConfig.

csi-LoggedMeasurementEventTriggerConfig This field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception.

csi-LoggedResourceConfig Resources in which the UE performs channel measurement whose associated measurement results are logged by the UE. The csi-LoggedResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources.

loggingPeriodicity The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

#### –	CSI-LoggedMeasurementConfigId

The IE CSI-LoggedMeasurementConfigId is used to identify a CSI-LoggedMeasurementConfig.

CSI-LoggedMeasurementConfigId information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIGID-START

CSI-LoggedMeasurementConfigId-r19 ::=            INTEGER (0..maxNrofLoggedMeasurementConfigurations-1-r19)

-- TAG-CSI-LOGGEDMEASUREMENTCONFIGID-STOP

-- ASN1STOP

#### –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]],

[[

cli-RSSI-MeasResourceToAddModList-r19        SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResources-r19)) OF CLI-RSSI-MeasResource-r19

OPTIONAL, -- Need N

cli-RSSI-MeasResourceToReleaseList-r19       SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResources-r19)) OF CLI-RSSI-MeasResourceId-r19

OPTIONAL, -- Need N

cli-RSSI-MeasResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourceSets-r19)) OF CLI-RSSI-MeasResourceSet-r19

OPTIONAL, -- Need N

cli-RSSI-MeasResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourceSets-r19)) OF CLI-RSSI-MeasResourceSetId-r19

OPTIONAL, -- Need N

srs-RSRP-MeasResourceToAddModList-r19        SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResources-r19)) OF SRS-RSRP-MeasResource-r19

OPTIONAL, -- Need N

srs-RSRP-MeasResourceToReleaseList-r19       SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResources-r19)) OF SRS-RSRP-MeasResourceId-r19

OPTIONAL, -- Need N

srs-RSRP-MeasResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourceSets-r19)) OF SRS-RSRP-MeasResourceSet-r19

OPTIONAL, -- Need N

srs-RSRP-MeasResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourceSets-r19)) OF SRS-RSRP-MeasResourceSetId-r19

OPTIONAL, -- Need N

csi-LoggedMeasurementConfigToAddModList-r19  SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LoggedMeasurementConfig-r19

OPTIONAL, -- Need N

csi-LoggedMeasurementConfigToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LoggedMeasurementConfigId-r19

OPTIONAL  -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

CSI-MeasConfig field descriptions

aperiodicTriggerStateList Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement (see TS 38.214 [19], clause 5.2.1).

cli-RSSI-MeasResourceToAddModList Pool of CLI-RSSI-MeasResource for CLI RSSI measurement.

cli-RSSI-MeasResourceSetToAddModList Pool of CLI-RSSI-MeasResourceSet for CLI RSSI measurement.

csi-IM-ResourceSetToAddModList Pool of CSI-IM-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

csi-IM-ResourceToAddModList Pool of CSI-IM-Resource which can be referred to from CSI-IM-ResourceSet.

csi-LoggedMeasurementConfigToAddModList Configured CSI logged measurements for network-side data collection.

csi-ReportConfigToAddModList Configured CSI report settings as specified in TS 38.214 [19] clause 5.2.1.1.

csi-ResourceConfigToAddModList Configured CSI resource settings as specified in TS 38.214 [19] clause 5.2.1.2.

csi-SSB-ResourceSetToAddModList Pool of CSI-SSB-ResourceSet which can be referred to from CSI-ResourceConfig.

ltm-CSI-ReportConfigToAddModList Configured CSI report settings for LTM as specified in TS 38.214 [19].

nzp-CSI-RS-ResourceSetToAddModList Pool of NZP-CSI-RS-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

nzp-CSI-RS-ResourceToAddModList Pool of NZP-CSI-RS-Resource which can be referred to from NZP-CSI-RS-ResourceSet.

reportTriggerSize, reportTriggerSizeDCI-0-2 Size of CSI request field in DCI (bits) (see TS 38.214 [19], clause 5.2.1.5.1). The field reportTriggerSize applies to DCI format 0_1 and the field reportTriggerSizeDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 5.2.1.5.1).

scellActivationRS-ConfigToAddModList Configured RS for fast SCell activation as specified in TS 38.214 [19] clause 5.2.1.5.3.

srs-RSRP-MeasResourceSetToAddModList Pool of SRS-RSRP-MeasResourceSet for SRS RSRP measurement.

srs-RSRP-MeasResourceToAddModList Pool of SRS-RSRP-MeasResource for SRS RSRP measurement.

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). The IE CSI-ReportConfig is also used to configure UE initiated CSI reporting. See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

cqi-Table                   ENUMERATED {table1, table2, table3, table4-r17}                                     OPTIONAL,   -- Need R

subbandSize                 ENUMERATED {value1, value2},

non-PMI-PortIndication      SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL,   -- Need R

...,

[[

semiPersistentOnPUSCH-v1530         SEQUENCE {

reportSlotConfig-v1530              ENUMERATED {sl4, sl8, sl16}

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1610         SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

aperiodic-v1610                     SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

reportQuantity-r16                  CHOICE {

cri-SINR-r16                         NULL,

ssb-Index-SINR-r16                   NULL

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r16                          CodebookConfig-r16                                              OPTIONAL    -- Need R

]],

[[

cqi-BitsPerSubband-r17              ENUMERATED {bits4}                                                      OPTIONAL,   -- Need R

groupBasedBeamReporting-v1710       SEQUENCE {

nrofReportedGroups-r17              ENUMERATED {n1, n2, n3, n4}

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r17                  CodebookConfig-r17                                                      OPTIONAL,   -- Need R

sharedCMR-r17                       ENUMERATED {enable}                                                     OPTIONAL,   -- Need R

csi-ReportMode-r17                  ENUMERATED {mode1, mode2}                                               OPTIONAL,   -- Need R

numberOfSingleTRP-CSI-Mode1-r17     ENUMERATED {n0, n1, n2}                                                 OPTIONAL,   -- Need R

reportQuantity-r17                  CHOICE {

cri-RSRP-Index-r17                  NULL,

ssb-Index-RSRP-Index-r17            NULL,

cri-SINR-Index-r17                  NULL,

ssb-Index-SINR-Index-r17            NULL

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1720         SEQUENCE {

reportSlotOffsetList-r17            SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-2-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-1-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL    -- Need R

}                                                                                                           OPTIONAL,   -- Need R

aperiodic-v1720                     SEQUENCE {

reportSlotOffsetList-r17            SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-2-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-1-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL    -- Need R

}                                                                                                           OPTIONAL    -- Need R

]],

[[

codebookConfig-v1730                CodebookConfig-v1730                                                    OPTIONAL    -- Need R

]],

[[

groupBasedBeamReporting-v1800       SEQUENCE {

reportingMode-r18                   ENUMERATED {jointULDL, onlyUL}

}                                                                                                           OPTIONAL,   -- Need R

reportQuantity-r18                  TDCP-r18                                                                OPTIONAL,   -- Need R

codebookConfig-r18                  CodebookConfig-r18                                                      OPTIONAL,   -- Need R

csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18

OPTIONAL,   -- Need N

csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18

OPTIONAL    -- Need N

]],

[[

nrofReportedCLImeasResources-r19       ENUMERATED {n1, n2, n3, n4}                                          OPTIONAL,    -- Need R

pucch-CSI-ResourceListExt-r19          SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-ResourceExt-v1900      OPTIONAL,    -- Need R

symbolType-r19                         ENUMERATED {sbfd, non-sbfd}                                          OPTIONAL,    -- Need R

nrofReportedRS-v1900                   ENUMERATED {n6, n8}                                                  OPTIONAL,    -- Need R

predictionConfiguration-r19            CHOICE {

csi-InferencePrediction-r19            ENUMERATED {true},

configurationForChannelPrediction-r19  SEQUENCE {

resourcesForChannelPrediction-r19      CSI-ResourceConfigId                                         OPTIONAL,    -- Need R

associatedIdForChannelPrediction-r19   AssociatedId-r19                                             OPTIONAL,    -- Need R

associatedIdForChannelMeasurement-r19  AssociatedId-r19                                             OPTIONAL,    -- Need R

nrofReportedPredicted-RS-r19           ENUMERATED {n1, n2, n3, n4}                                  OPTIONAL,    -- Need R

nrofTimeInstance-r19                   ENUMERATED {n1, n2, n4, n8}                                  OPTIONAL,    -- Need R

timeGap-r19                            ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1}

OPTIONAL,    -- Need R

...

},

configurationForChannelMonitoring-r19  SEQUENCE {

refToPredictionConfig-r19              CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19          ENUMERATED {n1, n2}                                          OPTIONAL,    -- Need R

nrofTransmissionOccasion-r19           ENUMERATED {n1, n3, n7, n15}                                 OPTIONAL,    -- Need R

timeInstanceFor-RS-PAI-r19             ENUMERATED {n1, n2, n8, spare1}                              OPTIONAL,    -- Need R

mappingToResourcesForChannelPrediction-r19

BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))     OPTIONAL,    -- Need R

timeInstanceFor-SGCS-r19               ENUMERATED {n1, spare3, spare2, spare1}                      OPTIONAL,    -- Need R

...

}

}                                                                                                           OPTIONAL,    -- Need R

codebookConfig-r19                     CodebookConfig-r19                                                   OPTIONAL,    -- Need R

portMappingMethod-r19                  ENUMERATED {method1, method2}                                 OPTIONAL, -- Cond codebookBased

valueOfM-r19                           INTEGER(1..4)                                                        OPTIONAL,    -- Need R

reportQuantity-r19                     CHOICE {

none-BM-r19                            NULL,

none-CSI-r19                           NULL,

p-CRI-r19                              NULL,

p-SSB-Index-r19                        NULL,

p-CRI-RSRP-r19                         NULL,

p-SSB-Index-RSRP-r19                   NULL,

rs-PAI-r19                             NULL,

sgcs-r19                               NULL,

cjtc-Dd-r19                            NULL,

cjtc-F-r19                             NULL,

cjtc-P-r19                             NULL,

cjtc-Dd-F-r19                          NULL,

cli-RSSI-r19                           NULL,

cli-SRS-RSRP-r19                       NULL

}                                                                                                           OPTIONAL,    -- Need R

csi-ReportCJTC-r19                     CSI-ReportCJTC-r19                                                   OPTIONAL,    -- Need R

csi-ReportSubConfigToAddModListExt-r19 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18))

OF CSI-ReportSubConfig-v1900                               OPTIONAL,    -- Need N

csi-ReportUE-IBR-r19                   CSI-ReportUE-IBR-r19                                                 OPTIONAL,    -- Need R

linkedCJTC-Report-r19                  CSI-ReportConfigId                                                   OPTIONAL     -- Need R

]]

}

PortIndexFor8Ranks ::=              CHOICE {

portIndex8                          SEQUENCE{

rank1-8                             PortIndex8                                                      OPTIONAL,   -- Need R

rank2-8                             SEQUENCE(SIZE(2)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank3-8                             SEQUENCE(SIZE(3)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank4-8                             SEQUENCE(SIZE(4)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank5-8                             SEQUENCE(SIZE(5)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank6-8                             SEQUENCE(SIZE(6)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank7-8                             SEQUENCE(SIZE(7)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank8-8                             SEQUENCE(SIZE(8)) OF PortIndex8                                 OPTIONAL    -- Need R

},

portIndex4                          SEQUENCE{

rank1-4                             PortIndex4                                                      OPTIONAL,   -- Need R

rank2-4                             SEQUENCE(SIZE(2)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank3-4                             SEQUENCE(SIZE(3)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank4-4                             SEQUENCE(SIZE(4)) OF PortIndex4                                 OPTIONAL    -- Need R

},

portIndex2                          SEQUENCE{

rank1-2                             PortIndex2                                                      OPTIONAL,   -- Need R

rank2-2                             SEQUENCE(SIZE(2)) OF PortIndex2                                 OPTIONAL    -- Need R

},

portIndex1                          NULL

}

PortIndex8::=                       INTEGER (0..7)

PortIndex4::=                       INTEGER (0..3)

PortIndex2::=                       INTEGER (0..1)

TDCP-r18 ::=                        SEQUENCE {

delayDSetofLengthY-r18              SEQUENCE (SIZE (1.. maxNrofdelayD-r18)) OF DelayD,

phaseReporting-r18                  ENUMERATED {enable}                                                 OPTIONAL    -- Need R

}

DelayD ::=                          ENUMERATED { symb4, slot1, slot2, slot3, slot4, slot5, slot6, slot10 }

CSI-ReportSubConfig-r18 ::=         SEQUENCE {

reportSubConfigId-r18               CSI-ReportSubConfigId-r18,

reportSubConfigParams-r18           CHOICE {

a1-parameters                       SEQUENCE {

codebookSubConfig-r18               CodebookConfig                                              OPTIONAL,   -- Need R

portSubsetIndicator-r18             CHOICE {

p2                                  BIT STRING (SIZE (2)),

p4                                  BIT STRING (SIZE (4)),

p8                                  BIT STRING (SIZE (8)),

p12                                 BIT STRING (SIZE (12)),

p16                                 BIT STRING (SIZE (16)),

p24                                 BIT STRING (SIZE (24)),

p32                                 BIT STRING (SIZE (32))

}                                                                                               OPTIONAL,   -- Need R

non-PMI-PortIndication-r18          SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks

OPTIONAL   --  Need R

},

a2-parameters                       SEQUENCE {

nzp-CSI-RS-ResourceList-r18         SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18

}

}                                                                                                       OPTIONAL,   -- Need R

powerOffset-r18                     INTEGER(0..23)                                                      OPTIONAL    -- Need R

}

NZP-CSI-RS-ResourceIndex-r18 ::=    INTEGER (0..maxNrofNZP-CSI-RS-ResourcesPerSet-1-r18)

CSI-ReportCJTC-r19 ::=              SEQUENCE {

--Editor’s note: associatedSRS-ResourceSet can be updated based on further RAN1 discussion.

associatedSRS-ResourceSet-r19       SEQUENCE {

srs-ResourceSetId-r19               SRS-ResourceSetId,

srs-ResourceId-r19                  SRS-ResourceId,

referenceAntennaPort-r19            INTEGER (1..8)                                                   OPTIONAL     -- Need R

}                                                                                                        OPTIONAL,    -- Need R

valueOfAD-r19                           ENUMERATED {dot5, one}                                           OPTIONAL,    -- Need R

valueOfMD-r19                           ENUMERATED {n32, n64, n128, n256}                                OPTIONAL,    -- Need R

valueOfAFO-r19                          ENUMERATED {zeroDot1, zeroDot2}                                  OPTIONAL,    -- Need R

valueOfMFO-r19                          ENUMERATED {n16, n32, n256 }                                     OPTIONAL,    -- Need R

valueOfMPhi-r19                         ENUMERATED {n16, n32}                                            OPTIONAL,    -- Need R

subbandSizeCJTC-r19                     ENUMERATED {n1, n2, n4, n8, n16, wideband}                       OPTIONAL,    -- Need R

nrofSubbandsPO-r19                      SEQUENCE (SIZE (1..16)) OF INTEGER (1..275)                      OPTIONAL     -- Need R

}

CSI-ReportSubConfig-v1900 ::=         SEQUENCE {

reportSubConfigParams-v1900           SEQUENCE {

a1-Parameters-v1900                   SEQUENCE {

portSubsetIndicator-v1900             CHOICE {

p48                                   BIT STRING (SIZE (48)),

p64                                   BIT STRING (SIZE (64)),

p128                                  BIT STRING (SIZE (128))

}                                                                                                OPTIONAL     -- Need R

}

},

...

}

CSI-ReportUE-IBR-r19 ::=               SEQUENCE {

eventTypeUE-IBR-r19                    CHOICE {

event1-r19                             SEQUENCE {

eventThreshold-r19                     RSRP-Range

},

event2-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31)

},

event7-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31),

valueOfQ-r19                           INTEGER (1..8)

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                NULL,

modeB-r19                                SEQUENCE {

pusch-ResourceOfModeB-r19                SEQUENCE {

configuredGrantConfigIndex-r19           ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                            BWP-Id,

servCellIndex-r19                        ServCellIndex

},

minimumPucch-PuschOffset-r19 ENUMERATED {symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256,

symb512}

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion.

}

},

nrofReportedRS-UE-IBR-r19                ENUMERATED {n1, n2, n3, n4},    tci-ServCellIndex-r19                    ServCellIndex                                                        OPTIONAL,    -- Need R

currentBeamReport-r19                    ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

conditionFulfillmentIndicator-r19        ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

eventCountWindow-r19                     SEQUENCE {

eventInstanceCount-r19                   INTEGER (2..16),

eventDetectionTimeWindow-r19             ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280}

}                                                                                                             OPTIONAL,    -- Need R

pucch-Resource-r19                       SEQUENCE {

periodicityAndOffset                     CHOICE {

sym2                                     NULL,

sym6or7                                  NULL,

sl1                                      NULL,                       -- Recurs in every slot

sl2                                      INTEGER (0..1),

sl4                                      INTEGER (0..3),

sl5                                      INTEGER (0..4),

sl8                                      INTEGER (0..7),

sl10                                     INTEGER (0..9),

sl16                                     INTEGER (0..15),

sl20                                     INTEGER (0..19),

sl40                                     INTEGER (0..39),

sl80                                     INTEGER (0..79),

sl160                                    INTEGER (0..159),

sl320                                    INTEGER (0..319),

sl640                                    INTEGER (0..639)

},

resource                                 PUCCH-ResourceId,

ul-BWP-Id-r19                            BWP-Id,

pucch-Cell-r19                           ENUMERATED {spCell, pucch-Scell}

}

}

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

Editor's Note:	FFS the value range of the fields nrofTimeInstance-r19, timeGap-r19, timeInstanceFor-RS-PAI-r19, and timeInstanceFor-SGCS-r19, based on RAN1 progress.

Editor's Note:	FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases.

CSI-ReportConfig field descriptions

associatedIdForChannelMeasurement Indicates that the UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associatedIdForChannelMeasurement or with the same associatedIdForChannelPrediction. This field is absent if resourcesForChannelPrediction is not configured or if resourcesForChannelMeasurement is equal to or a subset of resourcesForChannelPrediction.

associatedIdForChannelPrediction Indicates that the UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associatedIdForChannelMeasurement or with the same associatedIdForChannelPrediction. This field is absent if resourcesForChannelPrediction is not configured.

carrier Indicates in which serving cell the CSI-ResourceConfig indicated below are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

codebookConfig Codebook configuration for Type-1 or Type-2 including codebook subset restriction. Network can only configure one of codebookConfig, codebookConfig-r16, codebookConfig-r17, codebookConfig-r18 or codebookConfig-r19 in a CSI-ReportConfig. The network includes codebookConfig-v1730 only if codebookConfig-r17 is configured.

cqi-BitsPerSubband This field can only be present if cqi-FormatIndicator is set to subbandCQI. If the field is configured with bits4, the UE uses 4-bit sub-band CQI. If the field is not present and cqi-FormatIndicator is set to subbandCQI, the UE uses 2-bit sub-band differential CQI.

cqi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI (see TS 38.214 [19], clause 5.2.1.4).

cqi-Table Which CQI table to use for CQI calculation (see TS 38.214 [19], clause 5.2.2.1). For an (e)RedCap UE, CQI table 2 is only supported if the UE indicates support of 256QAM for PDSCH.

csi-IM-ResourcesForInterference CSI IM resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only CSI-IM resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

csi-InferencePrediction Indicates whether the UE reports predicted CSI based on inference.

csi-ReportCJTC Configures parameters used for CJT calibration.

csi-ReportingBand Indicates a contiguous or non-contiguous subset of subbands in the bandwidth part which CSI shall be reported for. Each bit in the bit-string represents one subband in order of frequency position in the BWP. The right-most bit in the bit string represents the lowest subband with the lowest frequency position in the BWP. The choice determines the number of subbands (subbands3 for 3 subbands, subbands4 for 4 subbands, and so on) (see TS 38.214 [19], clause 5.2.1.4). This field is absent if there are less than 24 PRBs (no sub band) and present otherwise (see TS 38.214 [19], clause 5.2.1.4). NOTE: In TS 38.212 [17] clause 6.3.1.1.2 and TS 38.214 [19] clause 5.2.1.4, only subbands to be reported are numbered, e.g. subband #0 is the subband corresponding to the right-most bit set to 1.

csi-ReportMode Configures the CSI report modes Mode1 or Mode 2 (see TS 38.214 [19], clause 5.2.1.4.2)

csi-ReportSubConfigToAddModList, csi-ReportSubConfigToAddModListExt List of CSI-ReportSubConfiguration(s) in a CSI report configuration to add or modify. No simultaneous configuration of portSubsetIndicator and a list of nzp-CSI-RS-resources in a same CSI report sub-configuration. The number of elements in a list is at least 2. If the network includes csi-ReportSubConfigToAddModListExt, it includes the same number of entries, and listed in the same order, as in csi-ReportSubConfigToAddModList.

csi-ReportSubConfigToReleaseList List of CSI-ReportSubConfiguration(s) in a CSI report configuration to release.

csi-ReportUE-IBR Configures parameters used for the UE initiated CSI reporting. When this field is configured, the UE ignores reportConfigType. When this field is set to event1, eventThreshold can only be configured with values from 14 to 113.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

groupBasedBeamReporting Turning on/off group beam based reporting (see TS 38.214 [19], clause 5.2.1.4). If groupBasedBeamReporting (without suffix) is set to disabled, groupBasedBeamReporting-v1710 and groupBasedBeamReporting-v1800 is absent.

linkedCJTC-Report This field is used in clause 5.2.1.4 in TS 38.214 [19].

mappingToResourcesForChannelPrediction If configured, this field indicates the resources included in resourcesForChannelMeasurement to be used for monitoring the channel predictions in the resources resourcesForChannelPrediction included within the linked prediction report configuration indicated by refToPredictionConfig. This field indicates Y non-zero bits, where Y is the size of the resource set for monitoring in resourcesForChannelMeasurement. The x-th MSB of the bitmap corresponds to x-th resource in resourcesForChannelPrediction in the linked prediction report configuration indicated by refToPredictionConfig. The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the resourcesForChannelMeasurement set for monitoring, 1≤y≤Y. This field is mandatory present only if the size of resourcesForChannelMeasurement is smaller than the size of resourcesForChannelPrediction in the linked prediction report configuration indicated by refToPredictionConfig. This field is present only if reportQuantity-r19 is set to 'rs-PAI-r19'.

non-PMI-PortIndication Port indication for RI/CQI calculation. For each CSI-RS resource in the linked ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback (see TS 38.214 [19], clause 5.2.1.4.2). The first entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the CSI-ResourceConfig whose CSI-ResourceConfigId is indicated in a CSI-MeasId together with the above CSI-ReportConfigId; the second entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the second entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig, and so on until the NZP-CSI-RS-Resource indicated by the last entry in nzp-CSI-RS-Resources in the in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig. Then the next entry corresponds to the NZP-CSI-RS-Resource indicated by the first entry in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the second entry of nzp-CSI-RS-ResourceSetList of the same CSI-ResourceConfig and so on.

nrofBestBeamForMonitoring Indicates the number of best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring for performance metric calculation. This field is present only if the field reportQuantity-r19 is set to 'rs-PAI-r19'.

nrofReportedGroups Number of reported resource groups per CSI-report. Value n1 means one resource group, n2 means 2 resource groups, and so on. If nrofReportedGroups is configured, the UE ignores groupBasedBeamReporting (without suffix).

nrofReportedPredictedRS Indicates the number (K) of predicted RS resources to be reported per report setting, if nrofTimeInstance is not configured. Indicates the number (K) of predicted RS resources per time instance to be reported per report setting, if nrofTimeInstance is configured. This field is present only if reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index’-r19, 'p-CRI-RSRP-r19' or 'p-SSB-Index-RSRP-r19'.

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. (see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1. Network does not configure nrofReportedRS-v1900 at the same time as nrofReportedRS (without suffix).

nrofTimeInstance Indicates the number of future time instance(s) N for prediction to be reported per report setting. This field is present only if reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index’-r19, 'p-CRI-RSRP-r19', 'p-SSB-Index-RSRP-r19' or 'none-BM-r19' and if timeGap is configured.

nrofTransmissionOccasion Indicates the number of (N) latest transmission occasion(s) of monitoring resources for performance metric calculation. This field is present only if the field reportQuantity-r19 is set to 'rs-PAI-r19'.

numberOfSingleTRP-CSI-Mode1 Configures the number of reported X CSIs when csi-ReportMode is set to 'Mode 1' as described in TS 38.214 [19], clause 5.2.1.4.2. The field is present only if csi-ReportMode configures Mode 1.

nzp-CSI-RS-ResourcesForInterference NZP CSI RS resources for interference measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources. The bwp-Id in that CSI-ResourceConfig is the same value as the bwp-Id in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement.

p0alpha Index of the p0-alpha set determining the power control for this CSI report transmission (see TS 38.214 [19], clause 6.2.1.2).

pdsch-BundleSizeForCSI PRB bundling size to assume for CQI calculation when reportQuantity is CRI/RI/i1/CQI. If the field is absent, the UE assumes that no PRB bundling is applied (see TS 38.214 [19], clause 5.2.1.4.2).

pmi-FormatIndicator Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see TS 38.214 [19], clause 5.2.1.4).

portMappingMethod Indicates the mapping from CSI-RS resource index/port index per resource to port index for CSI/PMI calculation (see TS 38.214 [19], clause 5.2.1.4).

pucch-CSI-ResourceList Indicates which PUCCH resource to use for reporting on PUCCH.

refToPredictionConfig Indicates the linked CSI-ReportConfigId corresponding to a prediction report configuration.

reportConfigType Time domain behavior of reporting configuration.

reportFreqConfiguration Reporting configuration in the frequency domain. (see TS 38.214 [19], clause 5.2.1.4).

reportQuantity The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16, reportQuantity-r17, reportQuantity-r18 or reportQuantity-r19 is present, UE shall ignore reportQuantity (without suffix). Network does not configure reportQuantity-r16, reportQuantity-r17, reportQuantity-r18 or reportQuantity-r19. Network does not configure reportQuantity-r16, reportQuantity-r17, reportQuantity-r18 or reportQuantity-r19 simultaneously.

reportingMode Configures the UE with reporting mode for group based reporting.(see TS 38.214 [19] clause 5.2.1.4).

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). If the field reportSlotConfig-v1530 is present, the UE shall ignore the value provided in reportSlotConfig (without suffix).

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on. The first report is transmitted in slot n+Y, second report in n+Y+P, where P is the configured periodicity. Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. This list must have the same number of entries as the pusch-TimeDomainAllocationList in PUSCH-Config. A particular value is indicated in DCI. The network indicates in the DCI field of the UL grant, which of the configured report slot offsets the UE shall apply. The DCI value 0 corresponds to the first report slot offset in this list, the DCI value 1 corresponds to the second report slot offset in this list, and so on (see TS 38.214 [19], clause 6.1.2.1). The field reportSlotOffsetListDCI-0-1 applies to DCI format 0_1 and the field reportSlotOffsetListDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1). The fields reportSlotOffsetList-r17, reportSlotOffsetListDCI-0-1-r17 and reportSlotOffsetListDCI-0-2-r17 are only applicable for SCS 480 kHz and 960 kHz and if they are configured, the UE shall ignore the fields reportSlotOffsetList (without suffix), reportSlotOffsetListDCI-0-1 (without suffix) and reportSlotOffsetListDCI-0-2 (without suffix) for SCS 480 kHz and 960 kHz.

resourcesForChannelMeasurement Resources for channel measurement. csi-ResourceConfigId of a CSI-ResourceConfig included in the configuration of the serving cell indicated with the field "carrier" above. The CSI-ResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources. This CSI-ReportConfig is associated with the DL BWP indicated by bwp-Id in that CSI-ResourceConfig.

resourcesForChannelPrediction Indicates resources to be predicted based on measurements performed on resourcesForChannelMeasurement. The UE is not expected to measure the resources to be predicted, unless the reportQuantity-r19 is set to 'none-BM-r19'. This field is present only if reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index’-r19, 'p-CRI-RSRP-r19', 'p-SSB-Index-RSRP-r19' or 'none-BM-r19'.

sharedCMR Enables sharing of channel measurement resources between different CSI measurement hypotheses when (1) csi-ReportMode is set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 is set to 1 or 2; or (2) csi-ReportMode is set to 'Mode2' (see TS 38.214 [19], clause 5.2.1.4.2).

subbandSize Indicates one out of two possible BWP-dependent values for the subband size as indicated in TS 38.214 [19], table 5.2.1.4-2 . If csi-ReportingBand is absent, the UE shall ignore this field.

symbolType Configures the symbol type of periodic/semi-persistent CSI-RS resources for CSI derivation for a CSI report associated with periodic/semi-persistent CSI-RS (see TS 38.214 [19], clause 5).

timeGap Indicates the time gap between the reference time and the first future time instance for prediction, if nrofTimeInstance-r19 is set to 1. Indicates the time gap between two consecutive future time instances for prediction, if nrofTimeInstance-r19 is set to >1. This field is present only if resourcesForChannelPrediction-r19 and nrofTimeInstance-r19 are configured.

timeInstanceFor-RS-PAI Indicates the f-th time instance is used for the performance metric calculation. This field is present only if reportQuantity-r19 is set to 'rs-PAI-r19'.

timeInstanceFor-SGCS Indicates the f-th doppler domain unit is used for the performance metric calculation for N4>1. This field is present only if reportQuantity-r19 is set to 'sgcs-r19'.

timeRestrictionForChannelMeasurements Time domain measurement restriction for the channel (signal) measurements (see TS 38.214 [19], clause 5.2.1.1).

timeRestrictionForInterferenceMeasurements Time domain measurement restriction for interference measurements (see TS 38.214 [19], clause 5.2.1.1).

valueOfM This field is used in clause 5.1, 5.2 and 5.4 in TS 38.214 [19]. This field is configured up to value 4 if codebookType is set to typeI-SinglePanel and up to value 2 if codebookType is set to typeII-r16.

CSI-ReportCJTC field descriptions

associatedSRS-ResourceSet This field is used in clause 5.2.1.4 in TS 38.214 [19].

nrOfSubbandsPO This field is used in clause 5.2.1.4 in TS 38.214 [19].

referenceAntennaPort This field is used in clause 5.2.1.4 in TS 38.214 [19].

subbandSizeCJTC This field is used in clause 5.2.1.4 in TS 38.214 [19].

valueOfAD This field is used in clause 5.2.1.4 in TS 38.214 [19]. Value dot5 corresponds to 0.5 CP and value one corresponds to 1 CP.

valueOfAFO This field is used in clause 5.2.1.4 in TS 38.214 [19]. Value zeroDot1 corresponds to 0.1 ppm and value zeroDot2 corresponds to 0.2 ppm.

valueOfMD This field is used in clause 5.2.1.4 in TS 38.214 [19].

valueOfMFO This field is used in clause 5.2.1.4 in TS 38.214 [19].

valueOfMPhi This field is used in clause 5.2.1.4 in TS 38.214 [19].

CSI-ReportSubConfig field descriptions

codebookSubConfig Applicable codebook parameters for the ports indicated by portSubsetIndicator. Applicable value ranges for codebook subset restriction, rank restriction, N1, N2, and Ng and twoTX-CodebookSubsetRestriction follow existing specification according to the codebookConfig configured within the CSI-ReportConfig, and apply for the number of ports determined by portSubsetIndicator (see TS 38.214 [19], clause 5.2.1.4.2). In this field, the network always sets the codebookType to type1. When reportQuantity is set to 'cri-RI-i1-CQI', the parameter typeISinglePanel-codebookSubsetRestriction-i2 is mandatory to be configured in the codebookSubConfig for each sub-configuration that includes portSubsetIndicator.

non-PMI-PortIndication Port indication for RI/CQI calculation applicable only for non-PMI feedback. The field shall be configured only if the portSubsetIndicator-r18 is configured. For each CSI-RS resource in the linked CSI-ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports out of P ports to use within the sub-configuration. P corresponds to the number of bits with value 1 in the bitmap portSubsetIndicator-r18.

nzp-CSI-RS-ResourceList List of NZP CSI RS resources for the sub-configuration that is a (sub)set of NZP CSI-RS resource(s) of the CSI-RS resource set for channel measurement associated with the sub-configuration in the CSI report configuration. Value 0 refers to the first NZP CSI RS resource of the CSI-RS resource set, value 1 refers to the second NZP CSI RS resource of the CSI-RS resource set, and so on.

portSubsetIndicator, portSubsetIndicator-v1900 Indicates the (sub)set of CSI-RS antenna ports used for CSI calculation of the sub-configuration. In the bit string, each bit corresponds to an antenna port. When a bit is set to 1, the corresponding port is enabled for CSI calculation corresponding to the sub-configuration. When the bit is set to zero, the corresponding port is not enabled for CSI calculation corresponding to the sub-configuration. The size of the bit string equals P bits, where P=2/4/8/12/16/24/32/48/64/128 represents the number of ports of the NZP CSI-RS resource(s) in the resource set for channel measurement associated with the CSI-ReportConfig. The network does not configure portSubsetIndicator and portSubsetIndicator-v1900 simultaneously.

powerOffset When powerControlOffset is configured in NZP CSI-RS resources in the resource set for channel measurement associated with the CSI-ReportConfig, the power offset of PDSCH EPRE to NZP CSI-RS EPRE is equal to powerControlOffset - powerOffset.

reportSubConfigParams, reportSubConfigParams-v1900 Indicates the parameters to be used for the CSI report sub-configuration, which includes either a1-parameters that contain the antenna port subset and the associated parameters relevant to the sub-configuration or a2-parameters that contain the list of NZP CSI-RS resources for the sub-configuration (see TS 38.214 [19], clause 5.2.1.4.2).

CSI-ReportUE-IBR field descriptions

conditionFulfillmentIndicator If configured, the UE includes an indication whether or not each reported RS fulfilled the event condition. This parameter is only configured if eventDetectionTimeWindow is configured and nrofReportedRS is not set to n1.

currentBeamReport If configured, the UE includes measurements of the current beam in the UE initiated CSI reporting.

eventDetectionTimeWindow Indicates the time window length for triggering event determination (see TS 38.214 [19], clause 5.2). Value ms4 corresponds to 4 milliseconds, value ms5 corresponds to 5 milliseconds and so on.

eventInstanceCount Indicates the minimum number of event instances for one same new beam within a configured time window that the UE can initiate UEIBM report (see TS 38.214 [19], clause 5.2.1.5.4.1). This field is only configured if eventDetectionTimeWindow is configured.

eventTypeUE-IBR Indicates the event type for UE initiated CSI reporting and associated fields as specified in clause 5.2.1.5.4 of TS 38.214 [19].

minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH in Mode-B from the PUCCH. Value symb0 corresponds to 0, value symb1 corresponds to 1 and so on.

nrofReportedRS-UE-IBR The number of reported RS in the UE initiated CSI reporting. Value n1 corresponds to 1 reported RS, value n2 corresponds to 2 reported RSs and so on.

pucch-Resource Indicates the periodic PUCCH resource for the UE initiated CSI reporting indicator for both mode-A and mode-B UE initated CSI reporting: - to request dynamically scheduled PUSCH to carry UE-initiated/event-driven beam report for mode-A; - to notify the network of a Type-1 CG PUSCH to carry UE-initiated/event-driven beam report for mode-B.

pusch-ResourceOfModeB Indicates Type-1 CG PUSCH resource for the UE initiated CSI reporting in mode-B.

reportTransmissionMode Indicates the transmission mode for UE initiated CSI reporting. Value modeA indicates transmission of UE initiated CSI reporting in a dynamically scheduled uplink grant and value modeB indicates transmission of UE initiated CSI reporting in a pre-configured type-1 configured uplink grant.

tci-ServCellIndex Indicates the serving cell on which the indicated TCI state used to determine the current beam RS is applied (see TS 38.214 [19], clause 5.2.1.5.4).

PortIndexFor8Ranks field descriptions

portIndex8 Port-Index configuration for up to rank 8. If present, the network configures port indexes for at least one of the ranks.

portIndex4 Port-Index configuration for up to rank 4. If present, the network configures port indexes for at least one of the ranks.

portIndex2 Port-Index configuration for up to rank 2. If present, the network configures port indexes for at least one of the ranks.

portIndex1 Port-Index configuration for rank 1.

TDCP field descriptions

delayDSetofLengthY Configures a set of Y delay values for TDCP reporting, see reference TS 38.214 clause 5.2.1.4. The symb4 denotes 4 symbols, the slot1 denotes 1 slot, the slot2 denotes 2 slots and so on. The value slot10 is applicable only to SCS >=30kHz. The parameter Y, see reference TS38.214 clause 5.2.1.4, is given by the length of the set of D values.

phaseReporting Configures the UE for phase reporting for TDCP reporting see reference TS 38.214 clause 5.2.1.4

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | This field is optionally present, Need R, if codebookConfig-r19 is configured. It is absent otherwise. |

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::=              INTEGER (0..maxNrofCSI-ReportConfigurations-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ReportPeriodicityAndOffset

The IE CSI-ReportPeriodicityAndOffset is used to configure a periodicity and a corresponding offset. Both the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on.

CSI-ReportPeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-REPORTPERIODICITYANDOFFSET-START

CSI-ReportPeriodicityAndOffset ::=  CHOICE {

slots4                              INTEGER(0..3),

slots5                              INTEGER(0..4),

slots8                              INTEGER(0..7),

slots10                             INTEGER(0..9),

slots16                             INTEGER(0..15),

slots20                             INTEGER(0..19),

slots40                             INTEGER(0..39),

slots80                             INTEGER(0..79),

slots160                            INTEGER(0..159),

slots320                            INTEGER(0..319)

}

-- TAG-CSI-REPORTPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CSI-ReportSubConfigId

The IE CSI-ReportSubConfigId is used to indicate the index of one CSI-ReportSubConfig within a CSI report configuration.

CSI-ReportSubConfigId information element

-- ASN1START

-- TAG-CSI-REPORTSUBCONFIGID-START

CSI-ReportSubConfigId-r18 ::=              INTEGER (0..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-1-r18)

-- TAG-CSI-REPORTSUBCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ReportSubConfigTriggerList

The IE CSI-ReportSubConfigTriggerList is used to configure a list of sub-configuration ID(s) of N sub-configurations out of L configured sub-configurations within a CSI-ReportConfig associated with a triggering state for semi-persistent CSI reporting on PUSCH and aperiodic CSI reporting.

CSI-ReportSubConfigTriggerList information element

-- ASN1START

-- TAG-CSI-REPORTSUBCONFIGTRIGGERLIST-START

CSI-ReportSubConfigTriggerList-r18 ::= SEQUENCE (SIZE(1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18

-- TAG-CSI-REPORTSUBCONFIGTRIGGERLIST-STOP

-- ASN1STOP

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet, CSI-SSB-ResourceSet, CLI-RSSI-MeasResourceSet and/or SRS-RSRP-MeasResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]],

[[

cli-MeasResourceSetList-r19       CHOICE {

cli-RSSI-MeasResourceSetList-r19  SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourceSetsPerConfig-r19)) OF CLI-RSSI-MeasResourceSetId-r19,

srs-RSRP-MeasResourceSetList-r19  SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourceSetsPerConfig-r19)) OF SRS-RSRP-MeasResourceSetId-r19,

...

}                                                                                                          OPTIONAL -- Need R

]]

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS or CLI measurement resources associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.

cli-MeasResourceSetList List of references to SRS-RSRP measurement resource set(s) or CLI-RSSI measurement resource set(s) used for L1 SRS-RSRP measurement and reporting or L1 CLI-RSSI measurement and reporting (see TS 38.214 [19], clause 5). Value cli-RSSI-MeasResourceSetList contains up to maxNrofCLI-RSSI-MeasResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise. Value srs-RSRP-MeasResourceSetList contains up to maxNrofSRS-RSRP-MeasResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise.

csi-IM-ResourceSetList List of references to CSI-IM resources used for CSI measurement and reporting in a CSI-RS resource set. Contains up to maxNrofCSI-IM-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.

csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). The csi-SSB-ResourceSetListExt provides additional references and can only be configured if csi-SSB-ResourceSetList is configured and groupBasedBeamReporting-v1710 is configured in the CSI-ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement. If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement, the network configures 2 resource sets, which may be two CSI SSB resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

nzp-CSI-RS-ResourceSetList List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. If resourceType is set to 'aperiodic', the network configures up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets. If resourceType is is set to 'periodic' or 'semiPersistent' and groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig, the network configures 1 resource set. If resourceType is set to 'periodic' or 'semiPersistent' and groupBasedBeamReporting-v1710 is configured, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1. If resourceType is set to 'periodic' and reportQuantity is set to 'TDCP', the network configures up to 3 resource sets, see TS 38.214 [19] clause 5.2.1.2.

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). It does not apply to resources provided in the csi-SSB-ResourceSetList. If the associated csi-ResourceConfigId is included in CSI-LoggedMeasurementConfig, it is set to 'periodic'.

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=    CHOICE {

slots4                                  INTEGER (0..3),

slots5                                  INTEGER (0..4),

slots8                                  INTEGER (0..7),

slots10                                 INTEGER (0..9),

slots16                                 INTEGER (0..15),

slots20                                 INTEGER (0..19),

slots32                                 INTEGER (0..31),

slots40                                 INTEGER (0..39),

slots64                                 INTEGER (0..63),

slots80                                 INTEGER (0..79),

slots160                                INTEGER (0..159),

slots320                                INTEGER (0..319),

slots640                                INTEGER (0..639)

}

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CSI-RS-ResourceConfigMobility

The IE CSI-RS-ResourceConfigMobility is used to configure CSI-RS based RRM measurements.

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}                                                                                                           OPTIONAL, -- Need R

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

sequenceGenerationConfig            INTEGER (0..1023),

...,

[[

slotConfig-r17                      CHOICE {

ms4                                 INTEGER (0..255),

ms5                                 INTEGER (0..319),

ms10                                INTEGER (0..639),

ms20                                INTEGER (0..1279),

ms40                                INTEGER (0..2559)

}                                                                                                           OPTIONAL  -- Need R

]]

}

CSI-RS-Index ::=                    INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB and the support of increasedNumberofCSIRSPerMO capability (see TS 38.214 [19], clause 5.1.6.1.3).

density Frequency domain density for the 1-port CSI-RS for L3 mobility. See TS 38.211 [16], clause 7.4.1.

nrofPRBs Allowed size of the measurement BW in PRBs. See TS 38.211 [16], clause 7.4.1.

startPRB Starting PRB index of the measurement bandwidth. See TS 38.211 [16], clause 7.4.1.

CSI-RS-ResourceConfigMobility field descriptions

csi-RS-CellList-Mobility List of cells for CSI-RS based RRM measurements.

refServCellIndex Indicates the serving cell providing the timing reference for CSI-RS resources without associatedSSB. The field may be present only if there is at least one CSI-RS resource configured without associatedSSB. If this field is absent, the UE shall use the timing of the PCell for measurements on the CSI-RS resources without associatedSSB. The CSI-RS resources and the serving cell indicated by refServCellIndex for timing reference should be located in the same band.

subcarrierSpacing Subcarrier spacing of CSI-RS. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

CSI-RS-Resource-Mobility field descriptions

associatedSSB If this field is present, the UE may base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the cell indicated by the cellId in the CSI-RS-CellMobility. In this case, the UE is not required to monitor that CSI-RS resource if the UE cannot detect the SS/PBCH block indicated by this associatedSSB and cellId. If this field is absent, the UE shall base the timing of the CSI-RS resource indicated in CSI-RS-Resource-Mobility on the timing of the serving cell indicated by refServCellIndex. In this case, the UE is required to measure the CSI-RS resource even if SS/PBCH block(s) with cellId in the CSI-RS-CellMobility are not detected. CSI-RS resources with and without associatedSSB may be configured in accordance with the rules in TS 38.214 [19], clause 5.1.6.1.3.

csi-RS-Index CSI-RS resource index associated to the CSI-RS resource to be measured (and used for reporting).

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS, see TS 38.211 [16], clause 7.4.1.5.3.

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3 including table 7.4.1.5.2-1. The number of bits that may be set to one depend on the chosen row in that table.

isQuasiColocated Indicates that the CSI-RS resource is quasi co-located with the associated SS/PBCH block, see TS 38.214 [19], clause 5.1.6.1.3.

sequenceGenerationConfig Scrambling ID for CSI-RS (see TS 38.211 [16], clause 7.4.1.5.2).

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacing is set to kHz15, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 3/4/9/19/39 slots. When subcarrierSpacing is set to kHz30, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 7/9/19/39/79 slots. When subcarrierSpacing is set to kHz60, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 15/19/39/79/159 slots. When subcarrierSpacing is set kHz120, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 31/39/79/159/319 slots. When subcarrierSpacing is set to kHz480, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 127/159/319/639/1279 slots. When subcarrierSpacing is set to kHz960, the maximum offset values for periodicities ms4/ms5/ms10/ms20/ms40 are 255/319/639/1279/2559 slots. If slotConfig-r17 is present, UE shall ignore the slotConfig (without suffix).

#### –	CSI-RS-ResourceMapping

The IE CSI-RS-ResourceMapping is used to configure the resource element mapping of a CSI-RS resource in time- and frequency domain.

CSI-RS-ResourceMapping information element

-- ASN1START

-- TAG-CSI-RS-RESOURCEMAPPING-START

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

-- TAG-CSI-RS-RESOURCEMAPPING-STOP

-- ASN1STOP

CSI-RS-ResourceMapping field descriptions

cdm-Type CDM type (see TS 38.214 [19], clause 5.2.2.3.1).

density Density of CSI-RS resource measured in RE/port/PRB (see TS 38.211 [16], clause 7.4.1.5.3). Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, value 1 (one) is allowed for X=4, 8, 12. For density = 1/2, includes 1-bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS.

firstOFDMSymbolInTimeDomain2 Time domain allocation within a physical resource block. See TS 38.211 [16], clause 7.4.1.5.3.

firstOFDMSymbolInTimeDomain Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. See TS 38.211 [16], clause 7.4.1.5.3.

freqBand Wideband or partial band CSI-RS, (see TS 38.214 [19], clause 5.2.2.3.1).

frequencyDomainAllocation Frequency domain allocation within a physical resource block in accordance with TS 38.211 [16], clause 7.4.1.5.3. The applicable row number in table 7.4.1.5.3-1 is determined by the frequencyDomainAllocation for rows 1, 2 and 4, and for other rows by matching the values in the column Ports, Density and CDMtype in table 7.4.1.5.3-1 with the values of nrofPorts, cdm-Type and density below and, when more than one row has the 3 values matching, by selecting the row where the column (k bar, l bar) in table 7.4.1.5.3-1 has indexes for k ranging from 0 to 2*n-1 where n is the number of bits set to 1 in frequencyDomainAllocation.

nrofPorts Number of ports (see TS 38.214 [19], clause 5.2.2.3.1).

#### –	CSI-SemiPersistentOnPUSCH-TriggerStateList

The CSI-SemiPersistentOnPUSCH-TriggerStateList IE is used to configure the UE with list of trigger states for semi-persistent reporting of channel state information on L1. See also TS 38.214 [19], clause 5.2.

CSI-SemiPersistentOnPUSCH-TriggerStateList information element

-- ASN1START

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-START

CSI-SemiPersistentOnPUSCH-TriggerStateList ::= SEQUENCE(SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-SemiPersistentOnPUSCH-TriggerState

CSI-SemiPersistentOnPUSCH-TriggerState ::=     SEQUENCE {

associatedReportConfigInfo                     CSI-ReportConfigId,

...,

[[

sp-CSI-MultiplexingMode-r17                    ENUMERATED {enabled}                                           OPTIONAL   -- Need R

]],

[[

csi-ReportSubConfigTriggerList-r18             CSI-ReportSubConfigTriggerList-r18                             OPTIONAL,  -- Need R

ltm-AssociatedReportConfigInfo-r18             LTM-CSI-ReportConfigId-r18                                     OPTIONAL   -- Need R

]]

}

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-STOP

-- ASN1STOP

CSI-SemiPersistentOnPUSCH-TriggerStateList field descriptions

csi-ReportSubConfigTriggerList A list of sub-configuration ID(s) of N sub-configurations out of L configured sub-configurations within a CSI-ReportConfig associated with a triggering state for semi-persistent CSI reporting on PUSCH.

ltm-AssociatedReportConfigInfo This field configures semi-persistent CSI reports on PUSCH of LTM candidate cells. If ltm-associatedReportConfigInfo is configured the UE shall ignore the field associatedReportConfigInfo.

sp-CSI-MultiplexingMode Indicates if the behavior of transmitting SP-CSI on the first PUSCH repetitions corresponding to two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' is enabled or not.

#### –	CSI-SSB-ResourceSet

The IE CSI-SSB-ResourceSet is used to configure one SS/PBCH block resource set which refers to SS/PBCH as indicated in ServingCellConfigCommon and ServingCellConfig.

CSI-SSB-ResourceSet information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESET-START

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17        SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF ServingAdditionalPCIIndex-r17  OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

-- TAG-CSI-SSB-RESOURCESET-STOP

-- ASN1STOP

CSI-SSB-ResourceSet field descriptions

servingAdditionalPCIList Indicates the physical cell IDs (PCI) of the SSBs in the csi-SSB-ResourceList. If present, the list has the same number of entries as csi-SSB-ResourceList. The first entry of the list indicates the value of the PCI for the first entry of csi-SSB-ResourceList, the second entry of this list indicates the value of the PCI for the second entry of csi-SSB-ResourceList, and so on. For each entry, the following applies: - If the value is zero, the PCI is the PCI of the serving cell in which this CSI-SSB-ResourceSet is defined; - otherwise, the value is additionalPCIIndex-r17 of an SSB-MTC-AdditionalPCI-r17 configured using the additionalPCI-ToAddModList-r17 in ServingCellConfig, and the PCI is the additionalPCI-r17 in this SSB-MTC-AdditionalPCI-r17.

#### –	CSI-SSB-ResourceSetId

The IE CSI-SSB-ResourceSetId is used to identify one SS/PBCH block resource set.

CSI-SSB-ResourceId information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESETID-START

CSI-SSB-ResourceSetId ::=           INTEGER (0..maxNrofCSI-SSB-ResourceSets-1)

-- TAG-CSI-SSB-RESOURCESETID-STOP

-- ASN1STOP

#### –	DataCollectionCandidateConfigId

The IE DataCollectionCandidateConfigId is used to identify a DataCollectionCandidateConfigParameters.

DataCollectionCandidateConfigId information element

-- ASN1START

-- TAG-DATACOLLECTIONCANDIDATECONFIGID-START

DataCollectionCandidateConfigId-r19 ::=            INTEGER (0..maxCandidateConfig-1-r19)

-- TAG-DATACOLLECTIONCANDIDATECONFIGID-STOP

-- ASN1STOP

#### –	DedicatedNAS-Message

The IE DedicatedNAS-Message is used to transfer UE specific NAS layer information between the 5GC CN and the UE. The RRC layer is transparent for this information.

DedicatedNAS-Message information element

-- ASN1START

-- TAG-DEDICATED-NAS-MESSAGE-START

DedicatedNAS-Message ::=        OCTET STRING

-- TAG-DEDICATED-NAS-MESSAGE-STOP

-- ASN1STOP

#### –	DL-PPW-PreConfig

The IE DL-PPW-PreConfig provides configuration for a measurement window where a UE is expected to measure the DL PRS, if it is inside the active DL BWP and with the same numerology as the active DL BWP. Based upon the indication received in the configuration, the UE identifies whether the DL PRS priority is higher than that of the other DL signals or channels and accordingly determines, for example, the UE is expected to measure the DL PRS and is not expected to receive other DL signals and channels.

DL-PPW-PreConfig information element

-- ASN1START

-- TAG-DL-PPW-PRECONFIG-START

DL-PPW-PreConfig-r17 ::= SEQUENCE {

dl-PPW-ID-r17            DL-PPW-ID-r17,

dl-PPW-PeriodicityAndStartSlot-r17     DL-PPW-PeriodicityAndStartSlot-r17,

length-r17                               INTEGER (1..160),

type-r17                                 ENUMERATED {type1A, type1B, type2}                            OPTIONAL, -- Cond MultiType

priority-r17                             ENUMERATED {st1, st2, st3}                                    OPTIONAL  -- Cond MultiState

}

DL-PPW-ID-r17 ::= INTEGER  (0..maxNrofPPW-ID-1-r17)

DL-PPW-PeriodicityAndStartSlot-r17 ::= CHOICE {

scs15                                    CHOICE {

n4                         INTEGER (0..3),

n5                         INTEGER (0..4),

n8                         INTEGER (0..7),

n10                        INTEGER (0..9),

n16                        INTEGER (0..15),

n20                        INTEGER (0..19),

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n160                       INTEGER (0..159),

n320                       INTEGER (0..319),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

...

},

scs30                                    CHOICE {

n8                         INTEGER (0..7),

n10                        INTEGER (0..9),

n16                        INTEGER (0..15),

n20                        INTEGER (0..19),

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n128                       INTEGER (0..127),

n160                       INTEGER (0..159),

n320                       INTEGER (0..319),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

n20480                     INTEGER (0..20479),

...

},

scs60                                    CHOICE {

n16                        INTEGER (0..15),

n20                        INTEGER (0..19),

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n128                       INTEGER (0..127),

n160                       INTEGER (0..159),

n256                       INTEGER (0..255),

n320                       INTEGER (0..319),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

n20480                     INTEGER (0..20479),

n40960                     INTEGER (0..40959),

...

},

scs120                                   CHOICE {

n32                        INTEGER (0..31),

n40                        INTEGER (0..39),

n64                        INTEGER (0..63),

n80                        INTEGER (0..79),

n128                       INTEGER (0..127),

n160                       INTEGER (0..159),

n256                       INTEGER (0..255),

n320                       INTEGER (0..319),

n512                       INTEGER (0..511),

n640                       INTEGER (0..639),

n1280                      INTEGER (0..1279),

n2560                      INTEGER (0..2559),

n5120                      INTEGER (0..5119),

n10240                     INTEGER (0..10239),

n20480                     INTEGER (0..20479),

n40960                     INTEGER (0..40959),

n81920                     INTEGER (0..81919),

...

},

...

}

-- TAG-DL-PPW-PRECONFIG-STOP

-- ASN1STOP

DL-PPW-PreConfig field descriptions

dl-PPW-ID Indicates the pre-configured ID for DL-PRS processing window configuration.

dl-PPW-PeriodicityAndStartSlot Indicates the periodicity in slots and the offset of the starting slot with respect to SFN #0 slot #0 of the serving cell where the DL-PRS processing window is configured.

length Indicates the length of DL-PRS processing window in slots. Value 1 indicates length of one slot, value 2 indicates length of two slots and so on.

priority Indicates the priority between PDCCH/PDSCH/CSI-RS and PRS as specified in TS 38.214 [19].

type Indicates the DL-PRS processing window type as specified in TS 38.214 [19].

| Conditional Presence | Explanation |
| --- | --- |
| MultiType | The field is mandatory present when the UE reports its capability on supporting multiple processing types, otherwise it is absent. |
| MultiState | The field is mandatory present when the UE reports its capability on supporting option 1 or option 2 for the configured type, otherwise it is absent. |

#### –	DMRS-BundlingPUCCH-Config

The IE DMRS-BundlingPUCCH-Config-r17 is used to configure DMRS bundling for PUCCH.

DMRS-BundlingPUCCH-Config information element

-- ASN1START

-- TAG-DMRS-BUNDLINGPUCCH-CONFIG-START

DMRS-BundlingPUCCH-Config-r17 ::=         SEQUENCE {

pucch-DMRS-Bundling-r17                   ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pucch-TimeDomainWindowLength-r17          INTEGER (2..8)                                                  OPTIONAL,   -- Need S

pucch-WindowRestart-r17                   ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pucch-FrequencyHoppingInterval-r17        ENUMERATED {s2, s4, s5, s10}                                    OPTIONAL,   -- Need S

...

}

-- TAG-DMRS-BUNDLINGPUCCH-CONFIG-STOP

-- ASN1STOP

DMRS-BundlingPUCCH-Config field descriptions

pucch-DMRS-Bundling Indicates whether DMRS bundling and time domain window for PUCCH are jointly enabled.

pucch-FrequencyHoppingInterval Configures the number of consecutive slots for the UE to perform inter-slot frequency hopping with inter-slot bundling for PUCCH. When both inter-frequency hopping and DMRS bundling are enabled for PUCCH repetitions, the UE is expected to be configured with at least one pucch-FrequencyHoppingInterval-r17 and pucch-TimeDomainWindowLength-r17. When DMRS bundling for PUCCH is enabled by pucch-DMRS-Bundling-r17, PUCCH frequency hopping interval is only determined by the configuration of PUCCH hopping interval if PUCCH hopping interval is configured. If the field is absent, the number of consecutive slots for the UE to perform inter-slot PUCCH frequency hopping is indicated by pucch-TimeDomainWindowLength-r17.

pucch-TimeDomainWindowLength Configures the length of a nominal time domain window in slots for DMRS bundling for PUCCH. The value shall not exceed the maximum duration for DMRS bundling for PUCCH as specified in TS 38.306 [26]. If this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the time duration for the transmission of all PUCCH repetitions and the maximum duration for DMRS bundling for PUCCH as specified in TS 38.306 [26].

pucch-WindowRestart Indicates whether UE bundles PUCCH DMRS remaining in a nominal time domain window after event(s) triggered by DCI or MAC CE that violate power consistency and phase continuity requirements is enabled (see TS 38.214 [19], clause 6.1.7). NOTE: Events, which are triggered by DCI or MAC CE, but do not require UE capability to resume maintaining power consistency and/or phase continuity as specified in clause 6.1.7 of TS 38.214 [19], are excluded.

#### –	DMRS-BundlingPUSCH-Config

The IE DMRS-BundlingPUSCH-Config-r17 is used to configure DMRS bundling for PUSCH.

DMRS-BundlingPUSCH-Config information element

-- ASN1START

-- TAG-DMRS-BUNDLINGPUSCH-CONFIG-START

DMRS-BundlingPUSCH-Config-r17 ::=          SEQUENCE {

pusch-DMRS-Bundling-r17                    ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pusch-TimeDomainWindowLength-r17           INTEGER (2..32)                                                 OPTIONAL,   -- Need S

pusch-WindowRestart-r17                    ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pusch-FrequencyHoppingInterval-r17         ENUMERATED {s2, s4, s5, s6, s8, s10, s12, s14, s16, s20}        OPTIONAL,   -- Need S

...

}

-- TAG-DMRS-BUNDLINGPUSCH-CONFIG-STOP

-- ASN1STOP

DMRS-BundlingPUSCH-Config field descriptions

pusch-DMRS-Bundling Indicates whether DMRS bundling and time domain window for PUSCH are jointly enabled.

pusch-FrequencyHoppingInterval Configures the number of consecutive slots for the UE to perform inter-slot frequency hopping with inter-slot bundling for PUSCH. When both inter-frequency hopping and DMRS bundling are enabled for PUSCH repetitions, the UE is expected to be configured with at least one pusch-FrequencyHoppingInterval-r17 and pusch-TimeDomainWindowLength-r17. This parameter is shared for both DG-PUSCH and CG-PUSCH. When DMRS bundling for PUSCH is enabled by pusch-DMRS-Bundling-r17, PUSCH frequency hopping interval is only determined by the configuration of PUSCH hopping interval if PUSCH hopping interval is configured. If the field is absent, the number of consecutive slots for the UE to perform inter-slot PUSCH frequency hopping is indicated by pusch-TimeDomainWindowLength-r17. Note: For unpaired spectrum, the UE is not expected to be configured the value of s6, s8, s12, s14 and s16.

pusch-TimeDomainWindowLength Configures the length of a nominal time domain window in number of consecutive slots for DMRS bundling for PUSCH. The value shall not exceed the maximum duration for DMRS bundling for PUSCH as specified in TS 38.306 [26]. For PUSCH repetition type A/B, if this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the time duration for the transmission of all PUSCH repetitions and the maximum duration for DMRS bundling for PUSCH as specified in TS 38.306 [26]. For TBoMS, if this field is absent, the UE shall apply the default value that is the minimum value in the unit of consecutive slots of the duration of TBoMS transmission (including repetition of TBoMS) and the maximum duration for DMRS bundling for PUSCH as specified in TS 38.306 [26].

pusch-WindowRestart Indicates whether UE bundles PUSCH DMRS remaining in a nominal time domain window after event(s) triggered by DCI or MAC CE that violate power consistency and phase continuity requirements is enabled (see TS 38.214 [19], clause 6.1.7). NOTE: Events, which are triggered by DCI or MAC CE, but do not require UE capability to resume maintaining power consistency and/or phase continuity as specified in clause 6.1.7 of TS 38.214 [19], are excluded.

#### –	DMRS-DownlinkConfig

The IE DMRS-DownlinkConfig is used to configure downlink demodulation reference signals for PDSCH.

DMRS-DownlinkConfig information element

-- ASN1START

-- TAG-DMRS-DOWNLINKCONFIG-START

DMRS-DownlinkConfig ::=             SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                      OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                           OPTIONAL,   -- Need S

maxLength                           ENUMERATED {len2}                                                       OPTIONAL,   -- Need S

scramblingID0                       INTEGER (0..65535)                                                      OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                                      OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-DownlinkConfig  }                                   OPTIONAL,   -- Need M

...,

[[

dmrs-Downlink-r16               ENUMERATED {enabled}                                                        OPTIONAL    -- Need R

]],

[[

dmrs-TypeEnh-r18                ENUMERATED {enabled}                                                        OPTIONAL    -- Need R

]]

}

-- TAG-DMRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

DMRS-DownlinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in DL, see Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 in TS 38.211 [16]. If the field is absent, the UE applies the value pos2. See also clause 7.4.1.1.2 for additional constraints on how the network may set this field depending on the setting of other fields.

dmrs-Downlink This field indicates whether low PAPR DMRS is used, as specified in TS38.211 [16], clause 7.4.1.1.1.

dmrs-Type Selection of the DMRS type to be used for DL (see TS 38.211 [16], clause 7.4.1.1.1). If the field is absent, the UE uses DMRS type 1.

dmrs-TypeEnh This field is used in TS 38.211 [16], clause 7.4.1.1.2.

maxLength The maximum number of OFDM symbols for DL front loaded DMRS. len1 corresponds to value 1. len2 corresponds to value 2. If the field is absent, the UE applies value len1. If set to len2, the UE determines the actual number of DM-RS symbols by the associated DCI. (see TS 38.211 [16], clause 7.4.1.1.2).

phaseTrackingRS Configures downlink PTRS. If the field is not configured, the UE assumes that downlink PTRS are absent. See TS 38.214 [19] clause 5.1.6.3.

scramblingID0 DL DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.1.1). When the field is absent the UE applies the value physCellId configured for this serving cell.

scramblingID1 DL DMRS scrambling initialization (see TS 38.211 [16], clause 7.4.1.1.1). When the field is absent the UE applies the value physCellId configured for this serving cell.

#### –	DMRS-UplinkConfig

The IE DMRS-UplinkConfig is used to configure uplink demodulation reference signals for PUSCH.

DMRS-UplinkConfig information element

-- ASN1START

-- TAG-DMRS-UPLINKCONFIG-START

DMRS-UplinkConfig ::=               SEQUENCE {

dmrs-Type                           ENUMERATED {type2}                                                  OPTIONAL,   -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}                                       OPTIONAL,   -- Need S

phaseTrackingRS                     SetupRelease { PTRS-UplinkConfig }                                  OPTIONAL,   -- Need M

maxLength                           ENUMERATED {len2}                                                   OPTIONAL,   -- Need S

transformPrecodingDisabled          SEQUENCE {

scramblingID0                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

scramblingID1                       INTEGER (0..65535)                                              OPTIONAL,   -- Need S

...,

[[

dmrs-Uplink-r16                     ENUMERATED {enabled}                                            OPTIONAL    -- Need R

]]

}                                                                                                       OPTIONAL,   -- Need R

transformPrecodingEnabled           SEQUENCE {

nPUSCH-Identity                     INTEGER(0..1007)                                                OPTIONAL,   -- Need S

sequenceGroupHopping                ENUMERATED {disabled}                                           OPTIONAL,   -- Need S

sequenceHopping                     ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

...,

[[

dmrs-UplinkTransformPrecoding-r16   SetupRelease {DMRS-UplinkTransformPrecoding-r16}                OPTIONAL    -- Need M

]]

}                                                                                                       OPTIONAL,   -- Need R

...,

[[

dmrs-TypeEnh-r18                    ENUMERATED {enabled}                                                OPTIONAL    -- Need R

]]

}

DMRS-UplinkTransformPrecoding-r16  ::=  SEQUENCE {

pi2BPSK-ScramblingID0                   INTEGER(0..65535)                                               OPTIONAL,   -- Need S

pi2BPSK-ScramblingID1                   INTEGER(0..65535)                                               OPTIONAL    -- Need S

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

DMRS-UplinkConfig field descriptions

dmrs-AdditionalPosition Position for additional DM-RS in UL (see TS 38.211 [16], clause 6.4.1.1.3). If the field is absent, the UE applies the value pos2. See also clause 6.4.1.1.3 for additional constraints on how the network may set this field depending on the setting of other fields.

dmrs-Type Selection of the DMRS type to be used for UL (see TS 38.211 [16], clause 6.4.1.1.3) If the field is absent, the UE uses DMRS type 1.

dmrs-TypeEnh This field is used in TS 38.211 [16], clause 6.4.1.1.3.

dmrs-Uplink This field indicates whether low PAPR DMRS is used, as specified in TS38.211 [16], clause 6.4.1.1.1.1.

dmrs-UplinkTransformPrecoding This field indicates whether low PAPR DMRS is used for PUSCH with pi/2 BPSK modulation, as specified in TS38.211 [16], clause 6.4.1.1.1.2. The network configures this field only if tp-pi2BPSK is configured in PUSCH-Config.

maxLength The maximum number of OFDM symbols for UL front loaded DMRS. len1 corresponds to value 1. len2 corresponds to value 2. If the field is absent, the UE applies value len1. If set to len2, the UE determines the actual number of DM-RS symbols by the associated DCI. (see TS 38.211 [16], clause 6.4.1.1.3).

nPUSCH-Identity Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS. If the value is absent or released, the UE uses the value Physical cell ID (physCellId). See TS 38.211 [16].

phaseTrackingRS Configures uplink PTRS (see TS 38.211 [16]).

pi2BPSK-ScramblingID0, pi2BPSK-ScramblingID1 UL DMRS scrambling initialization for pi/2 BPSK DMRS for PUSCH (see TS 38.211 [16], Clause 6.4.1.1.2). When the field is absent the UE applies the value Physical cell ID (physCellId) of the serving cell.

scramblingID0 UL DMRS scrambling initialization for CP-OFDM (see TS 38.211 [16], clause 6.4.1.1.1.1). When the field is absent the UE applies the value Physical cell ID (physCellId).

scramblingID1 UL DMRS scrambling initialization for CP-OFDM. (see TS 38.211 [16], clause 6.4.1.1.1.1). When the field is absent the UE applies the value Physical cell ID (physCellId).

sequenceGroupHopping For DMRS transmission with transform precoder the NW may configure group hopping by the cell-specific parameter groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon. In this case, the NW may include this UE specific field to disable group hopping for PUSCH transmission except for Msg3, i.e., to override the configuration in PUSCH-ConfigCommon (see TS 38.211 [16]). If the field is absent, the UE uses the same hopping mode as for Msg3.

sequenceHopping Determines if sequence hopping is enabled for DMRS transmission with transform precoder for PUSCH transmission other than Msg3 (sequence hopping is always disabled for Msg3). If the field is absent, the UE uses the same hopping mode as for msg3. The network does not configure simultaneous group hopping and sequence hopping. See TS 38.211 [16], clause 6.4.1.1.1.2.

transformPrecodingDisabled DMRS related parameters for Cyclic Prefix OFDM.

transformPrecodingEnabled DMRS related parameters for DFT-s-OFDM (Transform Precoding).

#### –	DownlinkConfigCommon

The IE DownlinkConfigCommon provides common downlink parameters of a cell.

DownlinkConfigCommon information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMON-START

DownlinkConfigCommon ::=        SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL                                 OPTIONAL,   -- Cond InterFreqHOAndServCellAdd

initialDownlinkBWP              BWP-DownlinkCommon                              OPTIONAL,   -- Cond ServCellAdd

...,

[[

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                              OPTIONAL    -- Need R

]]

}

-- TAG-DOWNLINKCONFIGCOMMON-STOP

-- ASN1STOP

DownlinkConfigCommon field descriptions

frequencyInfoDL Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a serving cell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover, and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell) and upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

#### –	DownlinkConfigCommonSIB

The IE DownlinkConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMONSIB-START

DownlinkConfigCommonSIB ::=     SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL-SIB,

initialDownlinkBWP              BWP-DownlinkCommon,

bcch-Config                     BCCH-Config,

pcch-Config                     PCCH-Config,

...,

[[

pei-Config-r17                  PEI-Config-r17                         OPTIONAL,     -- Need R

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                     OPTIONAL      -- Need R

]],

[[

frequencyInfoDL-v1800           FrequencyInfoDL-SIB-v1800              OPTIONAL      -- Need R

]],

[[

lowPowerConfig-r19              LowPowerConfig-r19                     OPTIONAL,     -- Need R

pagingAdaptPEI-Config-r19       PEI-Config-r17                         OPTIONAL      -- Need R

]]

}

DownlinkConfigCommonSIB-v1760 ::=   SEQUENCE {

frequencyInfoDL-v1760               FrequencyInfoDL-SIB-v1760

}

BCCH-Config ::=                 SEQUENCE {

modificationPeriodCoeff         ENUMERATED {n2, n4, n8, n16},

...

}

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16        INTEGER (2..4)               OPTIONAL  -- Cond SharedSpectrum2

]],

[[

ranPagingInIdlePO-r17                              ENUMERATED {true}            OPTIONAL,  -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                               OPTIONAL  -- Need R

]],

[[

pagingAdaptation-r19                  SEQUENCE {

pagingAdapt-NS-r19                    ENUMERATED {eight,four, two, one},

pagingAdaptNAndPagingFrameOffset-r19  CHOICE {

oneT                                  NULL,

halfT                                 INTEGER (0..1),

quarterT                              INTEGER (0..3),

oneEighthT                            INTEGER (0..7),

oneSixteenthT                         INTEGER (0..15),

oneThirtySecondT                      INTEGER (0..31)

},

pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19  CHOICE {

sCS15KHZoneT                          SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT            SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..71679),

sCS480KHZoneThirtySecondT             SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..143359)

}                                                                              OPTIONAL           -- Need R

}                                                                                  OPTIONAL           -- Need R

]]

}

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...,

[[

po-NumPerPEI-r19                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r19                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r19                       INTEGER (0..32)

]]

}

SubgroupConfig-r17 ::=     SEQUENCE {

subgroupsNumPerPO-r17      INTEGER (1.. maxNrofPagingSubgroups-r17),

subgroupsNumForUEID-r17    INTEGER (1.. maxNrofPagingSubgroups-r17)                                        OPTIONAL,  -- Need S

...

}

LowPowerConfig-r19 ::=                    SEQUENCE {

lpwus-MvalueAndSeqConfigFR1-r19           CHOICE {

nOne                                      SEQUENCE {

lpwus-OverlaidSeqRoots                    SEQUENCE {

root1-r19                                 INTEGER (1..131),

root2-r19                                 INTEGER (1..131)                       OPTIONAL    -- Need R

}                                                                                    OPTIONAL,   -- Cond FFS[OFDM-only]

lpwus-OverlaidSeqNum-r19             ENUMERATED {n1, n2, n4, n8, n16}                OPTIONAL    -- Need R

},

nTwo                                      SEQUENCE {

lpwus-OverlaidSeqRoots                    SEQUENCE {

root1-r19                                 INTEGER (1..61),

root2-r19                                 INTEGER (1..61)                        OPTIONAL    -- Need R

}                                                                                    OPTIONAL,   -- Cond FFS[OFDM-only]

lpwus-OverlaidSeqNum-r19                  ENUMERATED {n1, n2, n4, n8}                OPTIONAL    -- Need R

},

nFour                                     SEQUENCE {

lpwus-OverlaidSeqRoots                    SEQUENCE {

root1-r19                                 INTEGER (1..31),

root2-r19                                 INTEGER (1..31)                        OPTIONAL    -- Need R

}                                                                                    OPTIONAL,   -- Cond FFS[OFDM-only]

lpwus-OverlaidSeqNum-r19                  ENUMERATED {n1, n2, n4}                    OPTIONAL    -- Need R

}

}                                                                                            OPTIONAL,   -- Cond FR1-Only

lpwus-MvalueAndSeqConfigFR2-r19           CHOICE {

nOne                                      SEQUENCE {

lpwus-OverlaidSeqRoots                    SEQUENCE {

root1-r19                                 INTEGER (1..131),

root2-r19                                 INTEGER (1..131)                       OPTIONAL    -- Need R

}                                                                                    OPTIONAL,   -- Cond FFS[OFDM-only]

lpwus-OverlaidSeqNum-SCS-120kHz-r19       ENUMERATED {n1, n2}                        OPTIONAL    -- Need R

}

}                                                                                            OPTIONAL,   -- Cond FR2-Only

lpwus-LoFrameOffsetList-r19               SEQUENCE {

offsetForLongerWakeUpDelay-r19            SEQUENCE (SIZE (1..4)) OF INTEGER (8..200)     OPTIONAL,   -- Need R

offsetForShorterWakeUpDelay-r19           SEQUENCE (SIZE (1..4)) OF INTEGER (8..200)     OPTIONAL    -- Need R

},

lpwus-MoNumPerLo-r19                      ENUMERATED {n1, n2, n3, n4},

lpwus-PoNumPerLo-r19                      ENUMERATED {po1, po2, po4}                         OPTIONAL,   -- Need R

lpwus-EPRE-Ratio-r19                      ENUMERATED {dBminus3, dB0, dB3, dB6}               OPTIONAL,   -- Need R

lpwus-AvailableSlot-r19                   CHOICE {

n10                                       BIT STRING (SIZE (10)),

n20                                       BIT STRING (SIZE (20)),

n40                                       BIT STRING (SIZE (40))

}                                                                                            OPTIONAL,   -- Need R

lpwus-AvailableSymbol-r19                 CHOICE {

oneSlot                                   BIT STRING (SIZE (14)),

twoSlots                                  BIT STRING (SIZE (28))

}                                                                                            OPTIONAL,   -- Need R

lpwus-OffsetFirstMoWithinLo-r19           ENUMERATED {ffs}                                   OPTIONAL,   -- Need R

lpwus-NominalMoDuration-r19               ENUMERATED {ffs}                                   OPTIONAL,   -- Need R

lpwus-ActualDuration-r19                  ENUMERATED {ffs}                                   OPTIONAL,   -- Need R

lpwus-LPSS-StartRB-r19                    INTEGER (0..263)                                   OPTIONAL,   -- Need R

lpwus-LPSS-BeamSubset-r19                 CHOICE {

shortBitmap                               BIT STRING (SIZE (4)),

mediumBitmap                              BIT STRING (SIZE (8)),

longBitmap                                BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

lpss-EPRE-Ratio-r19                       ENUMERATED {dBminus3, dB0, dB3, dB6}               OPTIONAL,   -- Need R

lpss-BinarySeqIndex-r19                   INTEGER (1..4)                                     OPTIONAL,   -- Cond FFS[OOK-only]

lpss-MvalueAndSeqConfig-r19               CHOICE {

nOne                                      SEQUENCE {

lpss-BinarySeqLen-r19                     ENUMERATED {n6, n8}                        OPTIONAL,   -- Cond FFS[OOK-only]

lpss-OverlaidSeqRoot-r19                  INTEGER (1..131)                           OPTIONAL    -- Cond OOK4-Only

},

nTwo                                      SEQUENCE {

lpss-BinarySeqLen-r19                     ENUMERATED {n12, n16}                      OPTIONAL,   -- Cond FFS[OOK-only]

lpss-OverlaidSeqRoot-r19                  INTEGER (1..61)                            OPTIONAL    -- Cond OOK4-Only

},

nFour                                     SEQUENCE {

lpss-BinarySeqLen-r19                     ENUMERATED {n16, n32}                      OPTIONAL,   -- Cond FFS[OOK-only]

lpss-OverlaidSeqRoot-r19                  INTEGER (1..31)                            OPTIONAL    -- Cond OOK4-Only

}

}                                                                                            OPTIONAL,   -- Need R

lpss-PeriodicityAndOffset-r19             CHOICE {

ms160                                     INTEGER (0..159),

ms320                                     INTEGER (0..319)

}                                                                                            OPTIONAL,   -- Cond FFS[OOK-only]

lpss-StartSymbol-r19                      SEQUENCE {

startSymbol1-r19                          INTEGER (0..10),

startSymbol2-r19                          INTEGER (0..10)                                OPTIONAL

}                                                                                            OPTIONAL,   -- Need R

lp-SubgroupConfig-r19                     LP-SubgroupConfig-r19,

entryCondition-r19                        EntryCondition-r19,

exitCondition-r19                         ExitCondition-r19,

...

}

LP-SubgroupConfig-r19 ::=        SEQUENCE {

lp-SubgroupsNumPerPO-r19         INTEGER (1.. maxNrofPagingSubgroupsLP-r19),

lp-SubgroupsNumForUEID-r19       INTEGER (1.. maxNrofPagingSubgroupsLP-r19)                  OPTIONAL,   -- Need S

...

}

EntryCondition-r19 ::=           SEQUENCE {

entryEvaluationOnMR-ForLR-OnLPSS-r19  SEQUENCE {

thresholdP1-r19                       ReselectionThreshold,

thresholdQ1-r19                       ReselectionThresholdQ                              OPTIONAL    -- Need R

}                                                                                            OPTIONAL,   -- Cond SupportLR-OnLPSS

entryEvaluationOnMR-ForLR-OnSSB-r19   SEQUENCE {

thresholdP2-r19                       ReselectionThreshold,

thresholdQ2-r19                       ReselectionThresholdQ                              OPTIONAL    -- Need R

}                                                                                            OPTIONAL,   -- Cond SupportLR-OnSSB

entryEvaluationOnLR-ForLR-OnSSB-r19   SEQUENCE {

thresholdP1-LR-r19                    ThresholdP-LR-r19,

thresholdQ1-LR-r19                    ThresholdQ-LR-r19                                  OPTIONAL    -- Need R

}                                                                                            OPTIONAL,   -- Need R

entryEvaluationOnLR-ForLR-OnLPSS-r19  SEQUENCE {

thresholdP3-LR-r19                    ThresholdP-LR-r19,

thresholdQ3-LR-r19                    ThresholdQ-LR-r19                                  OPTIONAL    -- Need R

}                                                                                            OPTIONAL,   -- Need R

...

}

ExitCondition-r19 ::=             SEQUENCE {

exitEvaluationOnLR-ForLR-OnLPSS-r19   SEQUENCE {

thresholdP2-LR-r19                    ThresholdP-LR-r19,

thresholdQ2-LR-r19                    ThresholdQ-LR-r19                                  OPTIONAL    -- Need R

}                                                                                            OPTIONAL,   -- Cond SupportLR-OnLPSS

exitEvaluationOnLR-ForLR-OnSSB-r19   SEQUENCE {

thresholdP4-LR-r19                    ThresholdP-LR-r19,

thresholdQ4-LR-r19                    ThresholdQ-LR-r19                                  OPTIONAL    -- Need R

}                                                                                            OPTIONAL,   -- Cond SupportLR-OnSSB

...

}

-- TAG-DOWNLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

Editor’s NOTE: On the RRC parameters with value of ENUMERATED {ffs}, to be updated according to further RAN1 inputs.

DownlinkConfigCommonSIB field descriptions

bcch-Config The modification period related configuration.

entryCondition The entry condition for LP-WUS monitoring related configuration.

exitCondition The exit condition for LP-WUS monitoring related configuration.

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a PCell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment.

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

lastUsedCellOnly When present, the field indicates that the UE monitors PEI only if the latest received RRCRelease without noLastCellUpdate is from this cell. A PEI-capable UE stores its last used cell information.

lowPowerConfig The LP-WUS and LP-SS related configuration.

lp-SubgroupConfig The LP-WUS subgroup related configuration.

pagingAdaptPEI-Config The PEI related configuration for paging adaptation. If the UE supports paging adaptation for PEI, it ignores pei-Config-r17, if configured.

pcch-Config The paging related configuration.

pei-Config The PEI related configuration.

subgroupConfig The paging subgroup related configuration.

BCCH-Config field descriptions

modificationPeriodCoeff Actual modification period, expressed in number of radio frames m = modificationPeriodCoeff * defaultPagingCycle, see clause 5.2.2.2.2. n2 corresponds to value 2, n4 corresponds to value 4, and so on.

PCCH-Config field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

firstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion for paging of each PO of the PF, see TS 38.304 [20].

pagingAdapt-NS Number of paging occasions per paging frame for paging adaptation.

pagingAdaptFirstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion for paging of each PO of the PF for paging adaptation, see TS 38.304 [20].

pagingAdaptNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]) for paging adaptation. A value of oneThirtySecondT corresponds to T/32, a value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 160 ms, N can be set to {oneSixteenthT, oneThirtySecondT} If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

nAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 160 ms, N can be set to oneSixteenthT If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT}

nrofPDCCH-MonitoringOccasionPerSSB-InPO The number of PDCCH monitoring occasions corresponding to an SSB within a Paging Occasion, see TS 38.304 [20], clause 7.1.

ns Number of paging occasions per paging frame.

ranPagingInIdlePO Indicates that the network supports to send RAN paging in PO that corresponds to the i_s as determined by UE in RRC_IDLE state, see TS38.304 [20].

PEI-Config field descriptions

payloadSizeDCI-2-7 Payload size of PEI DCI, i.e., DCI format 2_7. The size is no larger than the payload size of paging DCI which has maximum of 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

SubgroupConfig field descriptions

subgroupsNumPerPO Total number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from PEI signaling. The field represents the sum of CN-assigned and UEID-based subgroups supported by for PEI the network. When PEI-Config is configured, there is always at least one subgroup (UEID-based subgroup or CN-assigned subgroup) configured.

subgroupsNumForUEID Number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from PEI signaling, for UEID-based subgrouping method for PEI. When present, the field is set to an integer smaller than or equal to subgroupsNumPerPO. subgroupsNumPerPO equals to subgroupsNumForUEID when the network does not configure CN-assigned subgrouping for PEI. When pei-Config is configured, the field is absent when the network only configures CN-assigned subgrouping. Both this field and subgroupsNumPerPO are equal to 1 when the network does not configure subgrouping for PEI. When pei-Config is configured, if the field is absent, the UE uses subgrouping according to TS 38.304 [20], clause 7.3.0.

LowPowerConfig field descriptions

lpss-BinarySeqIndex Indicates the LP-SS binary sequence index in the cell for RRC IDLE/INACTIVE (see TS 38.211 [16], clause 7.4.5.1.1).

lpss-BinarySeqLen Indicates the LP-SS binary sequence length in the cell for RRC IDLE/INACTIVE. The value n6 means BinarySeqLen (see TS 38.211 [16], clause 7.4.5.1.1) is set to 6, value n8 means BinarySeqLen is set to 8, and so on.

lpss-EPRE-Ratio Indicates the EPRE ratio between LP-SS and SSB in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). A value of dBminus3 means EPRE ratio is set to -3dB, a value of dB0 means EPRE ratio is set to 0dB, and so on. If the M value = 1 for both LP-WUS and LP-SS, or the M value > 1 for both LP-WUS and LP-SS, the difference between lpwus-EPRE-Ratio and lpss-EPRE-Ratio shall be no larger than 3 dB. If M value =1 for LP-WUS and the M value >1 for LP-SS, the ERPE ratio for LP-SS minus the EPRE ratio for LP-WUS should be within the range of -6 dB to 0 dB. Note: EPRE refers to EPRE in one OFDM symbol with non-zero power (from baseband perspective) LP-SS transmission.

lpss-MvalueAndSeqConfig Indicates the number of OOK symbols in an OFDM symbol for LP-SS in FR1 or FR2 for RRC IDLE/INACTIVE (corresponding to parameter MLPSS, see TS 38.211 [16], clause 7.4.5.1.1) and the LP-SS sequence configuration in the cell for RRC_IDLE/INACTIVE. A value of nOne means M value is set to 1, a value of nTwo means M value is set to 2, and so on.

lpss-OverlaidSeqRoots Indicates the configuration of single overlaid sequence root for LP-SS in the cell for RRC IDLE/INACTIVE (see TS 38.211 [16], clause 7.4.2.5.1.2).

lpss-PeriodicityAndOffset Indicates the configuration of LP-SS periodicity and time offset (see TS 38.213 [13]). The time offset is configured for the first LP-SS occasion with reference to SFN#0. A value of ms160 means the LP-SS periodicity (see TS 38.213 [13], clause 10.4C) is set to 160ms, a value of ms320 means the LP-SS periodicity is set to 320ms. For each LP-SS periodicity value, the time offset (see TS 38.213 [13], clause 10.4C) is configured with the unit of ms.

lpss-StartSymbol Indicates start symbol(s) within a slot for LP-SS (see TS 38.213 [13], clause 10.4C), where the slot is determined from the periodicity/offset configuration, i.e. lpss-PeriodicityAndOffset, for LP-SS. If only startSymbol1 is configured, there is one LP-SS occasion in a slot. If both startSymbol1 and startSymbol2 are configured, there are two LP-SS occasions in a slot.

lpwus-ActualDuration Indicates the actual duration for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C), in number of OFDM symbols.

lpwus-AvailableSlot Indicates the available slot(s) for LP-WUS using a unit level bitmap with a periodicity 10, 20, or 40 units (see TS 38.213 [13], clause 10.4C) in RRC IDLE/INACTIVE, where the unit is one slot if 14-bit symbol level bitmap is used, or the unit is two slots if 28-bits symbol level bitmap is used, as configured by lpwus-AvailableSymbol. The most significant bit of the bit string represents the first unit and the second most significant bit represents the second unit and so on. Value 1 in the bitmap indicates that the corresponding unit is available, value 0 indicates that the corresponding unit is not available. This slot pattern repeats itself continuously.

lpwus-AvailableSymbol Indicates the available symbol(s) for LP-WUS within the slot(s) that are indicated as available for LP-WUS configured by lpwus-AvailableSlot (see TS 38.213 [13], clause 10.4C) in RRC IDLE/INACTIVE. A 14-bit or 28-bit symbol level bitmap that covers one or two slots can be configured. For oneSlot, the 14 bits represent the symbols within the slot. For twoSlots, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on. Value 1 in the bitmap indicates that the corresponding symbol is available, value 0 indicates that the corresponding symbol is not available.

lpwus-EPRE-Ratio Indicates the EPRE ratio between LP-WUS and SSB in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). A value of dBminus3 means EPRE ratio is set to -3dB, a value of dB0 means EPRE ratio is set to 0dB, and so on. If the M value = 1 for both LP-WUS and LP-SS, or the M value > 1 for both LP-WUS and LP-SS, the difference between lpwus-EPRE-Ratio and lpss-EPRE-Ratio shall be no larger than 3 dB. If M value =1 for LP-WUS and the M value >1 for LP-SS, the ERPE ratio for LP-SS minus the EPRE ratio for LP-WUS should be within the range of -6 dB to 0 dB. Note: EPRE refers to EPRE in one OFDM symbol with non-zero power (from baseband perspective) LP-WUS transmission.

lpwus-LoFrameOffsetList Indicates the frame-level offset between an LO and a reference PO/PF for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). The entry number of offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay is equal to CEIL (Ns / lpwus-PoNumPerLo), where Ns is the number of paging occasions per paging frame configured in SIB1. At least one frame-level offset, i.e., offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay is provided.

lpwus-LPSS-BeamSubset Indicates the subset of beams for LP-WUS/LP-SS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C), where the full set of beams refers to the actual transmitted SSBs determined according to ssb-PositionsInBurst configured in SIB1. The most significant bit of the bit string corresponds to beam index 0, the second most significant bit of the bit string corresponds to beam index 1, and so on. Value 0 in the bitmap indicates that the corresponding beam is not transmitted while value 1 indicates that the corresponding beam is transmitted. One bit could be set to value 1 only if the corresponding bit in ssb-PositionsInBurst is set to value 1. If this field is not configured, UE assumes the LP-WUS/LP-SS is transmitted for all the actual transmitted beams determined according to ssb-PositionsInBurst configured in SIB1.

lpwus-LPSS-StartRB Indicates the starting RB of LP-WUS and LP-SS for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). The starting RB index reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the DL initial BWP is configured by gNB. The starting PRB index configured for LP-WUS is that same as that of LP-SS.

lpwus-MoNumPerLo Indicates the number of LP-WUS MO(s) per beam within an LO in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). Value n1 means 1 MO per bean within an LO, value n2 means 2 MO per beam within an LO, and so on.

lpwus-MvalueAndSeqConfigFR1 Indicates the number of OOK symbols in an OFDM symbol for LP-WUS in FR1 for RRC IDLE/INACTIVE (corresponding to parameter MWUS, see TS 38.211 [16], clause 7.4.4.1.1in TS 38.211 [16]) and the LP-WUS sequence configuration in the cell for RRC_IDLE/INACTIVE. A value of nOne means M value is set to 1, a value of nTwo means M value is set to 2, and so on.

lpwus-MvalueAndSeqConfigFR2 Indicates the number of OOK symbols in an OFDM symbol for LP-WUS in FR2 for RRC IDLE/INACTIVE (corresponding to parameter MWUS, see TS 38.211 [16], clause 7.4.4.1.1 in TS 38.211 [16]) and the LP-WUS sequence configuration in the cell for RRC_IDLE/INACTIVE. A value of nOne means M value is set to 1.

lpwus-NominalMoDuration Indicates the nominal MO duration for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C), in number of OFDM symbols.

lpwus-OffsetFirstMoWithinLo Indicates the symbol leve offset for the starting time location of the first LP-WUS MO in an LO w.r.t. the reference point (see TS 38.213 [13], clause 10.4C). The reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. Common symbol level offset is shared across all LOs.

lpwus-OverlaidSeqNum Indicates the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for LP-WUS in the cell in FR1 for RRC IDLE/INACTIVE (corresponding to parameter Nseq, see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-OverlaidSeqNum-SCS-120kHz Indicates the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for LP-WUS in the cell for RRC IDLE/INACITVE for SCS of 120KHz in FR2 (see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-OverlaidSeqRoots Indicates the configuration of overlaid sequence root for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-PoNumPerLo Indicates the number of PO(s) associated with one LO for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.213 [13], clause 10.4C). Value po1 means 1 PO associated with one LO, value po2 means 2 POs associated with one LO, and so on.

offsetForLongerWakeUpDelay Indicates the frame-level offset value for LP-WUS correposnding to longer UE wake-up delay (see TS 38.213 [13], clause 10.4C). The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. Each entry in offsetForLongerWakeUpDelay is configured with the unit of frames.

offsetForShorterWakeUpDelay Indicates the frame-level offset value for LP-WUS correposnding to shorter UE wake-up delay (see TS 38.213 [13], clause 10.4C). The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. Each entry in offsetForShorterWakeUpDelay is configured with the unit of frames.

root1 Indicates the first overlaid sequence root for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.211 [16], clause 7.4.4.1.1).

root2 Indicates the second overlaid sequence root for LP-WUS in the cell for RRC IDLE/INACTIVE (see TS 38.211 [16], clause 7.4.4.1.1). For FR1, this field shall be present for the case when the value of parameter lpwus-OverlaidSeqNum is configured as 16/8/4 for M value=1/2/4, respectively; otherwise, it is optional.

LP-SubgroupConfig field descriptions

lp-SubgroupsNumForUEID Number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from LP-WUS signaling, for UEID-based subgrouping method for LP-WUS. When present, the field is set to an integer smaller than or equal to lp-SubgroupsNumPerPO. lp-SubgroupsNumPerPO equals to lp-SubgroupsNumForUEID when the network does not configure CN-assigned subgrouping for LP-WUS. When lowPowerConfig is configured, the field is absent when the network only configures CN-assigned subgrouping for LP-WUS. Both this field and lp-SubgroupsNumPerPO are equal to 1 when the network does not configure subgrouping for LP-WUS. When lowPowerConfig is configured, if the field is absent, the UE uses subgrouping according to TS 38.304 [20], clause 7.y.2.

lp-SubgroupsNumPerPO Total number of subgroups per Paging Occasion (PO) for UE to monitor codepoint from LP-WUS signaling. The field represents the sum of CN-assigned and UEID-based subgroups supported for LP-WUS by the network. When lowPowerConfig is configured, there is always at least one subgroup (UEID-based subgroup or CN-assigned subgroup) configured. The maximum number of subgroups per PO is 31, 15, and 7 when the number of PO(s) associated with one LO for LP-WUS, i.e., lpwus-PoNumPerLo, is configured as 1, 2, and 4, respectively.

EntryCondition, ExitCondition field descriptions

entryEvaluationOnLR-ForLR-OnLPSS Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on LP-WUR (see TS 38.304 [20], clause 7.5.1). This field is optional present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. It is absent otherwise.

entryEvaluationOnLR-ForLR-OnSSB Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on LP-WUR (see TS 38.304 [20], clause 7.5.1). This field is optional present for the cell supporting OFDM based LP-WUR measuring on SSB. It is absent otherwise.

entryEvaluationOnMR-ForLR-OnLPSS Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on MR (see TS 38.304 [20], clause 7.5.1).

entryEvaluationOnMR-ForLR-OnSSB Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine whether the entry condition for using LP-WUS is fulfilled or not based on the serving cell measurement on MR (see TS 38.304 [20], clause 7.5.1).

exitEvaluationOnLR-ForLR-OnLPSS Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine whether the exit condition for using LP-WUS is fulfilled or not based on the serving cell measurement on LP-WUR (see TS 38.304 [20], clause 7.5.1). This field is optional present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. It is absent otherwise.

exitEvaluationOnLR-ForLR-OnSSB Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine whether the exit condition for using LP-WUS is fulfilled or not based on the serving cell measurement on LP-WUR (see TS 38.304 [20], clause 7.5.1). This field is optional present for the cell supporting OFDM based LP-WUR measuring on SSB. It is absent otherwise.

thresholdP1, thresholdP2 Parameters "SThresholdP1", and "SThresholdP2" in TS 38.304 [20].

thresholdQ1, thresholdQ2 Parameters "SThresholdQ1", and "SThresholdQ2"in TS 38.304 [20].

thresholdP1-LR, thresholdP2-LR, thresholdP3-LR, thresholdP4-LR Parameters "SThresholdP1-LR", "SThresholdP2-LR", "SThresholdP3-LR", and "SThresholdP4-LR" in TS 38.304 [20].

thresholdQ1-LR, thresholdQ2-LR, thresholdQ3-LR, thresholdQ4-LR Parameters "SThresholdQ1-LR", "SThresholdQ2-LR", "SThresholdQ3-LR", and "SThresholdQ4-LR" in TS 38.304 [20].

| Conditional Presence | Explanation |
| --- | --- |
| SharedSpectrum2 | The field is optional present, Need R, if this cell operates with shared spectrum channel access. Otherwise, it is absent, Need R. |
| FR1-Only | This field is mandatory present for an FR1 carrier frequency. It is absent otherwise. |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise. |
| SupportLR-OnLPSS | This field is mandatory present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. It is absent otherwise. |
| SupportLR-OnSSB | This field is mandatory present for the cell supporting OFDM based LP-WUR measuring on SSB. It is absent otherwise. |
| OOK4-Only | This field is mandatory present for OOK-4 based LP-SS with M value >1, and optional present for OOK-4 based LP-SS with M value =1. Otherwise, it is absent. |
| OFDM-Only | FFS This field is mandatory present for OFDM based LP-WUR for LP-WUS operation in RRC IDLE/INACTIVE. Otherwise, it is absent. |

#### –	DownlinkPreemption

The IE DownlinkPreemption is used to configure the UE to monitor PDCCH for the INT-RNTI (interruption).

DownlinkPreemption information element

-- ASN1START

-- TAG-DOWNLINKPREEMPTION-START

DownlinkPreemption ::=              SEQUENCE {

int-RNTI                            RNTI-Value,

timeFrequencySet                    ENUMERATED {set0, set1},

dci-PayloadSize                     INTEGER (0..maxINT-DCI-PayloadSize),

int-ConfigurationPerServingCell     SEQUENCE (SIZE (1..maxNrofServingCells)) OF INT-ConfigurationPerServingCell,

...

}

INT-ConfigurationPerServingCell ::= SEQUENCE {

servingCellId                       ServCellIndex,

positionInDCI                       INTEGER (0..maxINT-DCI-PayloadSize-1)

}

-- TAG-DOWNLINKPREEMPTION-STOP

-- ASN1STOP

DownlinkPreemption field descriptions

dci-PayloadSize Total length of the DCI payload scrambled with INT-RNTI (see TS 38.213 [13], clause 11.2).

int-ConfigurationPerServingCell Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload (see TS 38.213 [13], clause 11.2).

int-RNTI RNTI used for indication pre-emption in DL (see TS 38.213 [13], clause 10).

timeFrequencySet Set selection for DL-preemption indication (see TS 38.213 [13], clause 11.2) The set determines how the UE interprets the DL preemption DCI payload.

INT-ConfigurationPerServingCell field descriptions

positionInDCI Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2). Must be multiples of 14 (bit).

#### –	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

DRB-Identity information element

-- ASN1START

-- TAG-DRB-IDENTITY-START

DRB-Identity ::=                    INTEGER (1..32)

-- TAG-DRB-IDENTITY-STOP

-- ASN1STOP

#### –	DRX-Config

The IE DRX-Config is used to configure DRX related parameters.

DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

drx-RetransmissionTimerDL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

DRX-ConfigExt-v1700 ::=                 SEQUENCE {

drx-HARQ-RTT-TimerDL-r17                INTEGER (0..448),

drx-HARQ-RTT-TimerUL-r17                INTEGER (0..448)

}

DRX-ConfigExt2-v1800 ::=                SEQUENCE {

drx-NonIntegerLongCycleStartOffset-r18  CHOICE {

ms1001over240                           INTEGER(0..3),

ms25over6                               INTEGER(0..3),

ms25over3                               INTEGER(0..7),

ms1001over120                           INTEGER(0..7),

ms100over9                              INTEGER(0..10),

ms25over2                               INTEGER(0..11),

ms40over3                               INTEGER(0..12),

ms125over9                              INTEGER(0..12),

ms50over3                               INTEGER(0..15),

ms1001over60                            INTEGER(0..15),

ms125over6                              INTEGER(0..19),

ms200over9                              INTEGER(0..21),

ms250over9                              INTEGER(0..26),

ms100over3                              INTEGER(0..32),

ms1001over30                            INTEGER(0..32),

ms75over2                               INTEGER(0..36),

ms125over3                              INTEGER(0..40),

ms1001over24                            INTEGER(0..40),

ms200over3                              INTEGER(0..65),

ms1001over15                            INTEGER(0..65),

ms250over3                              INTEGER(0..82),

ms1001over12                            INTEGER(0..82),

ms400over3                              INTEGER(0..132),

...

},

shortDRX-r18                            SEQUENCE {

drx-NonIntegerShortCycle-r18            ENUMERATED {ms1001over240, ms25over6, ms25over3, ms1001over120, ms100over9, ms25over2,

ms40over3, ms125over9, ms50over3, ms1001over60, ms125over6, ms200over9,

ms100over3, ms1001over30, ms125over3, ms1001over24, ms200over3, spare15,

spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6,

spare5, spare4, spare3, spare2, spare1},

drx-ShortCycleTimer-r18                 INTEGER (1..16)

}                                                                                                            OPTIONAL,   -- Need R

drx-TimeReferenceSFN-r18                    ENUMERATED {sfn512}                                              OPTIONAL    -- Need N

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received. drx-HARQ-RTT-TimerDL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerDL (without suffix) for SCS 480 kHz and 960 kHz.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted. drx-HARQ-RTT-TimerUL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerUL (without suffix) for SCS 480 kHz and 960 kHz.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value.

drx-NonIntegerLongCycleStartOffset drx-NonIntegerLongCycle in non-integer number of ms (e.g. ms1001over240 corresponds to 1001/240 ms, ms25over6 corresponds to 25/6 ms and so on) and drx-StartOffset in multiples of 1 ms. If drx-NonIntegerShortCycle is configured, the value of drx-NonIntegerLongCycle shall be a multiple of the drx-NonIntegerShortCycle value. If drx-NonIntegerLongCycleStartOffset-r18 is configured, the UE shall ignore drx-LongCycleStartOffset.

drx-NonIntegerShortCycle Value in non-integer number of ms, e.g. ms1001over240 corresponds to 1001/240 ms, ms25over6 corresponds to 25/6 ms and so on.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimerDL Value in number of slot lengths of the BWP where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle or drx-NonIntegerShortCycle (depending on which one is configured). A value of 1 corresponds to drx-ShortCycle or drx-NonIntegerShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle or 2 * drx-NonIntegerShortCycle and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

drx-TimeReferenceSFN Indicates how the UE initializes the counter DRX_SFN_COUNTER, see TS 38.321 [3], clause 5.7.

shortDRX Configuration of a short DRX cycle. The network configures only one of shortDRX (without a suffix) or shortDRX-r18.

#### –	DRX-ConfigSecondaryGroup

The IE DRX-ConfigSecondaryGroup is used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3].

DRX-ConfigSecondaryGroup information element

-- ASN1START

-- TAG-DRX-CONFIGSECONDARYGROUP-START

DRX-ConfigSecondaryGroup-r16 ::=   SEQUENCE {

drx-onDurationTimer-r16            CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer-r16            ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

-- TAG-DRX-CONFIGSECONDARYGROUP-STOP

-- ASN1STOP

DRX-ConfigSecondaryGroup field descriptions

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0 ms, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on, as specified in TS 38.321 [3]. The network configures a drx-InactivityTimer value for the second DRX group that is smaller than the drx-InactivityTimer configured for the default DRX group in IE DRX-Config.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSeconds). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on, as specified in TS 38.321 [3]. The network configures a drx-onDurationTimer value for the second DRX group that is smaller than the drx-onDurationTimer configured for the default DRX group in IE DRX-Config.

#### –	DRX-ConfigSL

The IE DRX-ConfigSL is used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3].

DRX-ConfigSL information element

-- ASN1START

-- TAG-DRX-CONFIGSL-START

DRX-ConfigSL-r17 ::=            SEQUENCE {

drx-HARQ-RTT-TimerSL-r17        INTEGER (0..56),

drx-RetransmissionTimerSL-r17   ENUMERATED {sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

-- TAG-DRX-CONFIGSL-STOP

-- ASN1STOP

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL For sidelink configured grant Type 1, value in number of symbols of the activated DL BWP of PCell. For other cases, value in number of symbols of the BWP where the PDCCH was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL For sidelink configured grant Type 1, value in number of slot lengths of the activated DL BWP of PCell. For other cases, value in number of slot lengths of the BWP where the PDCCH was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

#### –	EarlyUL-SyncConfig

The IE EarlyUL-SyncConfig is used to configure random access resources for the early UL synchronization procedure.

EarlyUL-SyncConfig information element

-- ASN1START

-- TAG-EARLYUL-SYNCCONFIG-START

EarlyUL-SyncConfig-r18 ::=         SEQUENCE {

frequencyInfoUL-r18                FrequencyInfoUL,

rach-ConfigGeneric-r18             RACH-ConfigGeneric,

bwp-GenericParameters-r18          BWP,

ssb-PerRACH-Occasion-r18           ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need R

prach-RootSequenceIndex-r18        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                             OPTIONAL, -- Need R

ltm-PRACH-SubcarrierSpacing-r18    SubcarrierSpacing                                                          OPTIONAL, -- Cond L139

n-TimingAdvanceOffset-r18          ENUMERATED { n0, n25600, n39936, spare1 }                                  OPTIONAL, -- Need R

...,

[[

ltm-tdd-UL-DL-ConfigurationCommon-r18 TDD-UL-DL-ConfigCommon                                                  OPTIONAL, -- Cond TDD

ltm-restrictedSetConfig-r18        ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL  -- Need R

]],

[[

ltm-TimeAlignmentTimer-r19         TimeAlignmentTimer                                                         OPTIONAL, -- Need R

ltm-TimeAlignmentTimerTag2-r19     TimeAlignmentTimer                                                         OPTIONAL  -- Cond 2TA

]]

}

-- TAG-EARLYUL-SYNCCONFIG-STOP

-- ASN1STOP

EarlyUL-SyncConfig field descriptions

frequencyInfoUL This field provides basic parameters of an uplink carrier for PRACH transmission on a candidate cell.

ltm-PRACH-SubcarrierSpacing Indicates subcarrier spacing of PRACH for LTM (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]).

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on a candidate cell. If the field is absent, the UE applies the value as defined in TS 38.133 [14], table 7.1.2-2.

rach-ConfigGeneric RACH parameters for performing a random access procedure on a candidate cell.

ssb-PerRACH-Occasion This field indicated the number of SSBs for RACH occasion.

ltm-TimeAlignmentTimerTag2 The TAT value for TAG with ID tag2-Id, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |
| TDD | This field is optionally present, Need R, for TDD LTM candidate cells. It is absent otherwise. |

#### –	EphemerisInfo

The IE EphemerisInfo provides satellite ephemeris. Ephemeris may be expressed either in format of position and velocity state vector in ECEF or in format of orbital parameters in ECI. Note: The ECI and ECEF coincide at epochTime, i.e., x,y,z axis in ECEF are aligned with x,y,z axis in ECI at epochTime.

EphemerisInfo information element

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

EphemerisInfo field descriptions

eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [71]. Unit is radian. Step of 1.431 * 10-8. Actual value = field value * (1.431 * 10-8).

inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8) for positive values. Actual value = field value * (2.341* 10-8) + pi for negative values.

longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

meanAnomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [71]. Unit is meter. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3).

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

#### –	EpochTime

The IE EpochTime is used to indicate the epoch time for the NTN assistance information, and it is defined as the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for EpochTime of the serving or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell when this field is provided in an NTN cell and the gNB when this field is provided in a TN cell. In case of handover or conditional handover, the reference point for EpochTime of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell. In case of handover or conditional handover, EpochTime is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the target cell. Otherwise, EpochTime is used based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the serving cell.

EpochTime information element

-- ASN1START

-- TAG-EPOCHTIME-START

EpochTime-r17 ::=              SEQUENCE {

sfn-r17                        INTEGER(0..1023),

subFrameNR-r17                 INTEGER(0..9)

}

-- TAG-EPOCHTIME-STOP

-- ASN1STOP

EpochTime field descriptions

sfn For serving cell, it indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. For neighbour or target cell, it indicates the SFN nearest to the frame where the message indicating the epochTime is received.

#### –	EUTRA-C-RNTI

The IE EUTRA-C-RNTI identifies a UE having a RRC connection within an E-UTRA cell.

EUTRA-C-RNTI information element

-- ASN1START

-- TAG-EUTRACRNTI-START

EUTRA-C-RNTI ::=                      BIT STRING (SIZE (16))

-- TAG-EUTRACRNTI-STOP

-- ASN1STOP

#### –	FeatureCombination

The IE FeatureCombination indicates a feature or a combination of features to be associated with a set of Random Access resources (i.e. an instance of FeatureCombinationPreambles).

FeatureCombination information element

-- ASN1START

-- TAG-FEATURECOMBINATION-START

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                    OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                                    OPTIONAL,  -- Need R

eRedCap-r18                ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                    OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

-- TAG-FEATURECOMBINATION-STOP

-- ASN1STOP

FeatureCombination field descriptions

eRedCap If present, this field indicates that eRedCap is part of this feature combination. The fields redCap and eRedCap shall not be both set to true. If the UE is an eRedCap UE and there is no set of configured RA resources with eRedCap set to true among all sets of configured RA resources, the UE considers redCap to be applicable for random access procedure. This field is not configured in a set of preambles that is configured with 2-step random-access type.

msg1-Repetitions If present, this field indicates that signalling of msg1 repetition is part of this feature combination. This field is not configured in a set of preambles that is configured with 2-step random-access type.

msg3-Repetitions If present, this field indicates that signalling of msg3 repetition is part of this feature combination. This field is not configured in a set of preambles that is configured with 2-step random-access type.

nsag If present, this field indicates NSAG(s) that are part of this feature combination.

redCap If present, this field indicates that RedCap is part of this feature combination.

smallData If present, this field indicates that Small Data is part of this feature combination.

#### –	FeatureCombinationPreambles

The IE FeatureCombinationPreambles associates a set of preambles with a feature combination. For parameters which can be provided in this IE, the UE applies this field value when performing Random Access using a preamble in this featureCombinationPreambles, otherwise the UE applies the corresponding value as determined by applicable Need Code, e.g. Need S. On a specific BWP, there can be at most one set of preambles associated with a given feature combination per RA Type (i.e. 4-step RACH or 2-step RACH) per MSG1 repetition number.

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)                                           OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                                                               OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16                                     OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range                                                OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range                                                OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)                                           OPTIONAL, -- Need R

...,

[[

msg1-RepetitionNum-r18                ENUMERATED {n2, n4, n8, spare1}                                   OPTIONAL, -- Cond Msg1Rep2

msg1-RepetitionTimeOffsetROGroup-r18  ENUMERATED {n4, n8, n16, spare1}                             OPTIONAL  -- Cond Msg1Rep3

]]

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOP

FeatureCombinationPreambles field descriptions

deltaPreamble Power offset between msg3 or msgA-PUSCH and RACH preamble transmission. If configured, this parameter overrides msg3-DeltaPreamble or msgA-DeltaPreamble, Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1). If msgA-DeltaPreamble is configured in separateMsgA-PUSCH-Config-r17, this field is absent. This field is set to the same value for all FeatureCombinationPreambles for MSG1 repetitions.

featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. Network ensures that at least one field within the featureCombination is configured. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2).

msg1-RepetitionNum Indicates which MSG1-repetition number that this FeatureCombinationPreambles is associated with.

msg1-RepetitionTimeOffsetROGroup Indicates a time offset of the starting ROs between two successive RO groups for a given repetition number (2, 4 or 8) associated with this FeatureCombinationPreambles for each frequency resource index within a time period (see TS 38.213 [13]). If this field is absent, the time offset is implicitly determined (see TS 38.213 [13]). For each MSG1 repetition number, the following values are applicable. • {n16}, for RO groups for MSG1 repetition number 8 • {n8, n16}, for RO groups for MSG1 repetition number 4 • {n4, n8, n16}, for RO groups for MSG1 repetition number 2

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the concerned feature combination in the BWP. If configured, this parameter overrides msgA-RSRP-Threshold-r16. If absent, the UE applies msgA-RSRP-Threshold-r16, if configured

numberOfPreamblesPerSSB-ForThisPartition It determines how many consecutive preambles are associated to the Feature Combination starting from the starting preamble(s) per SSB.

numberOfRA-PreamblesGroupA It determines how many consecutive preambles per SSB are associated to Group A starting from the starting preamble(s). The remaining preambles associated to the Feature Combination are associated to Group B

ra-SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2). If this feature combination preambles are associated to a RACH-ConfigCommon-twostepRA, this field correspond to ra-MsgA-SizeGroupA, otherwise it corresponds to ra-Msg3SizeGroupA.

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]). If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, it corresponds to msgA-RSRP-ThresholdSSB, as defined in TS 38.321 [3]. If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommon, it it corresponds to rsrp-ThresholdSSB, as defined in TS 38.321 [3]. If this parameter is not included in FeatureCombinationPreambles which is included in RACH-ConfigCommon, the UE applies rsrp-ThresholdSSB included in the RACH-ConfigCommon which includes this FeatureCombinationPreambles. If this parameter is not included in FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, the UE applies msgA-RSRP-ThresholdSSB included in the RACH-ConfigCommonTwoStepRA which includes this FeatureCombinationPreambles.

separateMsgA-PUSCH-Config If present, it specifies how the 2-step RACH preambles identified by this FeatureCombinationPreambles are mapped to a PUSCH slot separate from the one defined in MsgA-ConfigCommon-r16. If the field is absent, the UE should apply the corresponding parameter in the RACH-ConfigCommonTwoStepRA of the BWP which includes the FeatureCombinationPreambles IE.

ssb-SharedRO-MaskIndex Mask index (see TS 38.321 [3]). Indicates a subset of ROs where preambles are allocated for this feature combination. If this field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA: - in case of separate ROs are configured for 4-step and 2-step random access, this field indicates a subset of ROs configured within this RACH-ConfigCommonTwoStepRA; - in case shared ROs are used for 4-step and 2-step random access, it indicates the subset of ROs configured within RACH-ConfigCommon, which are the subset of ROs configured for 2-step random access. This field is configured when there is more than one RO per SSB. If the field is absent, all ROs configured in RACH-ConfigCommon or RACH-ConfigCommonTwoStepRA containing this FeatureCombinationPreambles are shared. The network does not configure this field, if the field msg1-RepetitionNum is configured.

startPreambleForThisPartition It defines the first preamble associated with the Feature Combination. If the UE is provided with a number N of SSB block indexes associated with one PRACH occasion, and N<1, the first preamble in each PRACH occasion is the one having the same index as indicated by this field. If N>=1, N blocks of preambles associated with the Feature Combination are defined, each having start index + startPreambleForThisPartition, where n refers to SSB block index (see TS 38.213 [13], clause 8.1).

| Conditional Presence | Explanation |
| --- | --- |
| MsgAConfigCommon | The field is optionally present, Need S, if FeatureCombinationPreambles is included in RACH-ConfigCommonTwoStepRA. Otherwise, it is absent. If the field is absent in FeatureCombinationPreambles included in RACH-ConfigCommonTwoStepRA, the UE applies MsgA-PUSCH-Config included in the corresponding MsgA-ConfigCommon. |
| Msg1Rep2 | The field is mandatory present, if msg1-Repetitions is included in FeatureCombination for this concerned FeatureCombinationPreambles. Otherwise, it is absent. |
| Msg1Rep3 | The field is optionally present, Need S, if msg1-Repetitions is included in FeatureCombination for this concerned FeatureCombinationPreambles. Otherwise, it is absent. |

#### –	FilterCoefficient

The IE FilterCoefficient specifies the measurement filtering coefficient. Value fc0 corresponds to k = 0, fc1 corresponds to k = 1, and so on.

FilterCoefficient information element

-- ASN1START

-- TAG-FILTERCOEFFICIENT-START

FilterCoefficient ::=       ENUMERATED { fc0, fc1, fc2, fc3, fc4, fc5, fc6, fc7, fc8, fc9, fc11, fc13, fc15, fc17, fc19, spare1, ...}

-- TAG-FILTERCOEFFICIENT-STOP

-- ASN1STOP

#### –	FreqBandIndicatorNR

The IE FreqBandIndicatorNR is used to convey an NR frequency band number as defined in TS 38.101-1 [15], TS 38.101-2 [39] and TS 38.101-5 [75].

FreqBandIndicatorNR information element

-- ASN1START

-- TAG-FREQBANDINDICATORNR-START

FreqBandIndicatorNR ::=             INTEGER (1..1024)

-- TAG-FREQBANDINDICATORNR-STOP

-- ASN1STOP

#### –	FreqPriorityListDedicatedSlicing

The IE FreqPriorityListDedicatedSlicing provides dedicated cell reselection priorities for slicing in RRCRelease.

FreqPriorityListDedicatedSlicing information element

-- ASN1START

-- TAG-FREQPRIORITYLISTDEDICATEDSLICING-START

FreqPriorityListDedicatedSlicing-r17 ::= SEQUENCE (SIZE (1.. maxFreq)) OF FreqPriorityDedicatedSlicing-r17

FreqPriorityDedicatedSlicing-r17 ::=     SEQUENCE {

dl-ExplicitCarrierFreq-r17               ARFCN-ValueNR,

sliceInfoListDedicated-r17               SliceInfoListDedicated-r17                                    OPTIONAL  -- Cond Mandatory

}

SliceInfoListDedicated-r17 ::=           SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfoDedicated-r17

SliceInfoDedicated-r17 ::=               SEQUENCE {

nsag-IdentityInfo-r17                    NSAG-IdentityInfo-r17,

nsag-CellReselectionPriority-r17         CellReselectionPriority                                       OPTIONAL,  -- Need R

nsag-CellReselectionSubPriority-r17      CellReselectionSubPriority                                    OPTIONAL   -- Need R

}

-- TAG-FREQPRIORITYLISTDEDICATEDSLICING-STOP

-- ASN1STOP

FreqPriorityDedicatedSlicing field descriptions

dl-ExplicitCarrierFreq Indicates the downlink carrier frequency to which SliceInfoListDedicated is associated.

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |

#### –	FreqPriorityListSlicing

The IE FreqPriorityListSlicing indicates cell reselection priorities for slicing in SIB16.

FreqPriorityListSlicing information element

-- ASN1START

-- TAG-FREQPRIORITYLISTSLICING-START

FreqPriorityListSlicing-r17 ::= SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17

FreqPrioritySlicing-r17 ::=     SEQUENCE {

dl-ImplicitCarrierFreq-r17            INTEGER (0..maxFreq),

sliceInfoList-r17                     SliceInfoList-r17                                               OPTIONAL  -- Cond Mandatory

}

SliceInfoList-r17 ::=             SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17

SliceInfo-r17 ::=                 SEQUENCE {

nsag-IdentityInfo-r17             NSAG-IdentityInfo-r17,

nsag-CellReselectionPriority-r17  CellReselectionPriority                                             OPTIONAL,  -- Need R

nsag-CellReselectionSubPriority-r17 CellReselectionSubPriority                                        OPTIONAL,  -- Need R

sliceCellListNR-r17               CHOICE {

sliceAllowedCellListNR-r17        SliceCellListNR-r17,

sliceExcludedCellListNR-r17       SliceCellListNR-r17

}                                                                                                     OPTIONAL  -- Need R

}

SliceCellListNR-r17 ::=           SEQUENCE (SIZE (1..maxCellSlice-r17)) OF PCI-Range

-- TAG-FREQPRIORITYLISTSLICING-STOP

-- ASN1STOP

FreqPriorityListSlicing field descriptions

dl-ImplicitCarrierFreq Indicates the downlink carrier frequency to which sliceInfoList is associated with. The frequency is signalled implicitly, value 0 corresponds to the serving frequency, value 1 corresponds to the first frequency indicated by the InterFreqCarrierFreqList in SIB4, and value 2 corresponds to the second frequency indicated by the InterFreqCarrierFreqList in SIB4, and so on.

SliceInfo field descriptions

sliceAllowedCellListNR List of allow-listed cells for slicing. If present, the cells listed in this list support the corresponding nsag-frequency pair, and the cells not listed in this list do not support the corresponding nsag-frequency pair, according to TS 38.304 [20], clause 5.2.4.11.

sliceCellListNR Contains either the list of allow-listed or exclude-listed cells for slicing. If absent, it implies all the cells support the corresponding nsag-frequency pair, according to 38.304 [20], clause 5.2.4.11.

sliceExcludedCellListNR List of exclude-listed cells for slicing. If present, the cells listed in this list do not support the corresponding nsag-frequency pair, and the cells not listed in this list support the corresponding nsag-frequency pair, according to TS 38.304 [20], clause 5.2.4.11.

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |

#### –	FrequencyInfoDL

The IE FrequencyInfoDL provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCYINFODL-START

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...,

[[

referenceCell-r18                   ServCellIndex                                                   OPTIONAL    -- Cond SSBlessSCell

]]

}

-- TAG-FREQUENCYINFODL-STOP

-- ASN1STOP

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. The SSB of the SCell supporting 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13] is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent for an inter-band SSB-less SCell, the reference serving cell is defined in TS 38.133 [14].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need S. |
| SSBlessSCell | The field is optionally present, Need S, if the inter-band SSB-less SCell is configured for this cell and absoluteFrequencySSB is absent. It is absent otherwise, Need R. |

#### –	FrequencyInfoDL-SIB

The IE FrequencyInfoDL-SIB provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFODL-SIB-START

FrequencyInfoDL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB,

offsetToPointA                      INTEGER (0..2199),

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier

}

FrequencyInfoDL-SIB-v1760 ::=       SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760

}

FrequencyInfoDL-SIB-v1800 ::=       SEQUENCE {

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18

}

-- TAG-FREQUENCYINFODL-SIB-STOP

-- ASN1STOP

FrequencyInfoDL-SIB field descriptions

offsetToPointA Represents the offset to Point A as defined in TS 38.211 [16], clause 4.4.4.2.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A (see TS 38.211 [16], clause 5.3). The network configures this for all SCSs that are used in DL BWPs in this serving cell.

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL-OrEarlyUL-Sync

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL-OrEarlyUL-Sync

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

additionalSpectrumEmission-v1760    AdditionalSpectrumEmission-v1760                        OPTIONAL    -- Need S

]],

[[

additionalSpectrumEmissionAerial-r18   AdditionalSpectrumEmission-r18                       OPTIONAL   -- Need S

]]

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If both additionalSpectrumEmission (without suffix) and additionalSpectrumEmission-v1760 are absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], tables 6.2.3.1-1A, 6.2A.3.1.1-2 and 6.2A.3.1.2-2, TS 38.101-2 [39], tables 6.2.3.1-2 and 6.2A.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured and if signalled, the same vaue in additionalSpectrumEmission-v1760 for all uplink carrier(s) of the same band with UL configured, except for additionalSpectrumEmission value corresponding to NS_55/NS_57. If NS_55/NS_57 (see TS 38.101-1 [15], table 6.2.3.1-1) is applicable for at least one uplink carrier, the network may configure either NS_55/NS_57 or NS_01 for these uplink carriers, and NS_01 for the remaining uplink carrier(s) of band n77. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.

additionalSpectrumEmissionAerial The additional spectrum emission requirements to be applied by the aerial UE on this uplink (see TS 38.101-1 [15], clause 6.2K). If the field is absent, the aerial UE uses value indicated by the field additionalSpectrumEmission/ additionalSpectrumEmission-v1760.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. Value in dBm. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emission requirements as specified in TS 38.106 [79].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL-OrEarlyUL-Sync | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), if this FrequencyInfoUL is for a supplementary uplink (SUL), or if this FrequencyInfoUL is configured within EarlyUL-SyncConfig. It is absent, Need R, otherwise. |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |

#### –	FrequencyInfoUL-SIB

The IE FrequencyInfoUL-SIB provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-SIB-START

FrequencyInfoUL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB                            OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                 OPTIONAL    -- Need S

]]

}

FrequencyInfoUL-SIB-v1760 ::=       SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760

}

-- TAG-FREQUENCYINFOUL-SIB-STOP

-- ASN1STOP

FrequencyInfoUL-SIB field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

frequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

frequencyBandListAerial Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for aerial UE as defined in TS 38.101-1 [15], clause 6.2K. If the field is present, the aerial UE shall apply the first listed band which it supports in the frequencyBandListAerial field. If the field is absent, frequencyBandList applies.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emission requirements as specified in TS 38.106 [79].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A (see TS 38.211 [16], clause 5.3). The network configures this for all SCSs that are used in UL BWPs configured in this serving cell.

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL-SIB is for the paired UL for a DL (defined in a FrequencyInfoDL-SIB) or if this FrequencyInfoUL-SIB is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL-SIB is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL-SIB is for the paired UL for a DL (defined in a FrequencyInfoDL-SIB), or if this FrequencyInfoUL-SIB is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL-SIB is for a supplementary uplink (SUL). It is absent otherwise. |

#### –	GapOccasionRatio

Ratio of gap occasions that is recommended for cancellation during a time period of 1 second. Value 'pc0' corresponds to 0 percent, 'pc20' corresponds to 20 percent and so on.

GapOccasionRatio information element

-- ASN1START

-- TAG-GAPOCCASIONRATIO-START

GapOccasionRatio-r19 ::= ENUMERATED {pc0, pc20, pc40, pc60}

-- TAG-GAPOCCASIONRATIO-STOP

-- ASN1STOP

#### –	GapPriority

The IE GapPriority is used to identify the priority of a gap configuration.

GapPriority information element

-- ASN1START

-- TAG-GAPPRIORITY-START

GapPriority-r17 ::=                       INTEGER (1..maxNrOfGapPri-r17)

-- TAG-GAPPRIORITY-STOP

-- ASN1STOP

#### –	HighSpeedConfig

The IE HighSpeedConfig is used to configure parameters for high speed scenarios.

HighSpeedConfig information element

-- ASN1START

-- TAG-HIGHSPEEDCONFIG-START

HighSpeedConfig-r16 ::=  SEQUENCE {

highSpeedMeasFlag-r16    ENUMERATED {true}        OPTIONAL,   -- Cond SpCellOnly

highSpeedDemodFlag-r16   ENUMERATED {true}        OPTIONAL,   -- Need R

...

}

HighSpeedConfig-v1700 ::=  SEQUENCE {

highSpeedMeasCA-Scell-r17    ENUMERATED {true}        OPTIONAL,   -- Cond SCellOnly

highSpeedMeasInterFreq-r17   ENUMERATED {true}        OPTIONAL,   -- Cond SpCellOnly2

highSpeedDemodCA-Scell-r17   ENUMERATED {true}        OPTIONAL,   -- Need R

...

}

HighSpeedConfigFR2-r17 ::=  SEQUENCE {

highSpeedMeasFlagFR2-r17                    ENUMERATED {set1, set2}                       OPTIONAL,   -- Need R

highSpeedDeploymentTypeFR2-r17              ENUMERATED {unidirectional, bidirectional}    OPTIONAL,   -- Need R

highSpeedLargeOneStepUL-TimingFR2-r17       ENUMERATED {true}                             OPTIONAL,   -- Need R

...

}

-- TAG-HIGHSPEEDCONFIG-STOP

-- ASN1STOP

HighSpeedConfig field descriptions

HighSpeedDemodCA-Scell If the field is present and UE supports demodulationEnhancementCA-r17, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SCell.

highSpeedDemodFlag If the field is present and UE supports demodulationEnhancement-r16, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SpCell.

highSpeedDeploymentTypeFR2 If the field is present, and field value is unidirectional, the UE shall assume uni-directional deployment or if field value is birectional the UE shall assume bidirectional deployment for FR2 up to 350km/h as specified in TS 38.133 [14].

highSpeedLargeOneStepUL-TimingFR2 If the field is present, large one step UE autonomous uplink transmit timing adjustment for FR2 up to 350km/h as specified in TS 38.133 [14] is enabled.

highSpeedMeasCA-Scell If the field is present and UE supports measurementEnhancementCA-r17, the UE shall apply the enhanced RRM requirements to the serving frequency of SCell for carrier aggregation to support high speed up to 500 km/h as specified in TS 38.133 [14].

highSpeedMeasFlag If the field is present and UE supports measurementEnhancement-r16, the UE shall apply the enhanced intra-NR and inter-RAT EUTRAN RRM requirements to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports intraNR-MeasurementEnhancement-r16, the UE shall apply enhanced intra-NR RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. If the field is present and UE supports interRAT-MeasurementEnhancement-r16, the UE shall apply enhanced inter-RAT EUTRAN RRM requirement to support high speed up to 500 km/h as specified in TS 38.133 [14]. This parameter only applies to the serving frequency of SpCell.

highSpeedMeasFlagFR2 If the field is present and UE supports ue-PowerClass-v1700 set to pc6, the UE shall apply enhanced intra-frequency RRM requirement to the serving frequency of SpCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14]. If the field is present and the UE supports enhanced inter-frequency RRM requirements for FR2 HST in RRC_IDLE and RRC_INACTIVE, the UE shall apply enhanced inter-frequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_IDLE and RRC_INACTIVE. If the field is present for SpCell and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced inter-frequency RRM requirement to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. If the field is present for SCell(s) and and the UE supports measEnhCAInterFreqFR2-r18, the UE shall apply enhanced intra-frequency RRM requirements to the serving frequency of the corresponding SCell to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14] in RRC_CONNECTED. If the UE is configured with intra-band CA and the field highSpeedMeasFlagFR2 is present, the network configures the same value for all serving cells. The field value, set1 or set2, is applied as specified in TS38.133 [14].

highSpeedMeasInterFreq If the field is present and UE supports measurementEnhancementInterFreq-r17, the UE shall apply the enhanced RRM requirements for inter-frequency measurement in RRC_CONNECTED to support high speed up to 500 km/h as specified in TS 38.133 [14].

| Conditional Presence | Explanation |
| --- | --- |
| SCellOnly | The field is optionally present, Need R, in ServingCellConfigCommon of an SCell. It is absent otherwise. |
| SpCellOnly | The field is optionally present, Need R, in ServingCellConfigCommonSIB or in the ServingCellConfigCommon of an SpCell. It is absent otherwise. |
| SpCellOnly2 | The field is optionally present, Need R, in ServingCellConfigCommon of an SpCell. It is absent otherwise. |

#### –	Hysteresis

The IE Hysteresis is a parameter used within the entry and leave condition of an event triggered reporting condition. The actual value is field value * 0.5 dB.

Hysteresis information element

-- ASN1START

-- TAG-HYSTERESIS-START

Hysteresis ::=                      INTEGER (0..30)

-- TAG-HYSTERESIS-STOP

-- ASN1STOP

#### –	HysteresisAltitude

The IE HysteresisAltitude is a parameter used within the entry and leave condition of an altitude-based event triggered reporting condition and within altitude range based parameter configuration. The actual value is field value in meters.

HysteresisAltitude information element

-- ASN1START

-- TAG-HYSTERESISALTITUDE-START

HysteresisAltitude-r18 ::=                      INTEGER (0..64)

-- TAG-HYSTERESISALTITUDE-STOP

-- ASN1STOP

#### –	HysteresisLocation

The IE HysteresisLocation is a parameter used within entry and leave condition of a location based event triggered reporting condition. The actual value is field value * 10 meters.

HysteresisLocation information element

-- ASN1START

-- TAG-HYSTERESISLOCATION-START

HysteresisLocation-r17 ::=          INTEGER (0..32768)

-- TAG-HYSTERESISLOCATION-STOP

-- ASN1STOP

#### –	InvalidSymbolPattern

The IE InvalidSymbolPattern is used to configure one invalid symbol pattern for PUSCH transmission repetition type B applicable for both DCI format 0_1 and 0_2, see TS 38.214 [19], clause 6.1.

InvalidSymbolPattern information element

-- ASN1START

-- TAG-INVALIDSYMBOLPATTERN-START

InvalidSymbolPattern-r16 ::=     SEQUENCE {

symbols-r16                      CHOICE {

oneSlot                          BIT STRING (SIZE (14)),

twoSlots                         BIT STRING (SIZE (28))

},

periodicityAndPattern-r16        CHOICE {

n2                               BIT STRING (SIZE (2)),

n4                               BIT STRING (SIZE (4)),

n5                               BIT STRING (SIZE (5)),

n8                               BIT STRING (SIZE (8)),

n10                              BIT STRING (SIZE (10)),

n20                              BIT STRING (SIZE (20)),

n40                              BIT STRING (SIZE (40))

}                                                                OPTIONAL,   -- Need M

...

}

-- TAG-INVALIDSYMBOLPATTERN-STOP

-- ASN1STOP

InvalidSymbolPattern field descriptions

periodicityAndPattern A time domain repetition pattern at which the pattern defined by symbols recurs. This slot pattern repeats itself continuously. When the field is not configured, the UE uses the value n1 (see TS 38.214 [19], clause 6.1).

symbols A symbol level bitmap in time domain (see TS 38.214[19], clause 6.1). For oneSlot, if ECP is configured, the first 12 bits represent the symbols within the slot and the last two bits within the bitstring are ignored by the UE; Otherwise, the 14 bits represent the symbols within the slot. For twoSlots, if ECP is configured, the first 12 bits represent the symbols within the first slot and the next 12 bits represent the symbols in the second slot and the last four bits within the bit string are ignored by the UE; Otherwise, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on. This pattern recurs (in time domain) with the configured periodicityAndPattern.

#### –	I-RNTI-Value

The IE I-RNTI-Value is used to identify the suspended UE context of a UE in RRC_INACTIVE.

I-RNTI-Value information element

-- ASN1START

-- TAG-I-RNTI-VALUE-START

I-RNTI-Value ::=                        BIT STRING (SIZE(40))

-- TAG-I-RNTI-VALUE-STOP

-- ASN1STOP

#### –	LBT-FailureRecoveryConfig

The IE LBT-FailureRecoveryConfig-r16 is used to configure the parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

LBT-FailureRecoveryConfig information element

-- ASN1START

-- TAG-LBT-FAILURERECOVERYCONFIG-START

LBT-FailureRecoveryConfig-r16 ::=    SEQUENCE {

lbt-FailureInstanceMaxCount-r16      ENUMERATED {n4, n8, n16, n32, n64, n128},

lbt-FailureDetectionTimer-r16        ENUMERATED {ms10, ms20, ms40, ms80, ms160, ms320},

...

}

-- TAG-LBT-FAILURERECOVERYCONFIG-STOP

-- ASN1STOP

LBT-FailureRecoveryConfig field descriptions

lbt-FailureDetectionTimer Timer for consistent uplink LBT failure detection (see TS 38.321 [3]). Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

lbt-FailureInstanceMaxCount This field determines after how many LBT failure indications received from the physical layer the UE triggers uplink LBT failure recovery (see TS 38.321 [3]). Value n4 corresponds to 4, value n8 corresponds to 8, and so on.

#### –	LocationInfo

The IE LocationInfo is used to transfer available detailed location information, Bluetooth, WLAN and sensor available measurement results at the UE.

LocationInfo information element

-- ASN1START

-- TAG-LOCATIONINFO-START

LocationInfo-r16 ::=      SEQUENCE {

commonLocationInfo-r16    CommonLocationInfo-r16          OPTIONAL,

bt-LocationInfo-r16       LogMeasResultListBT-r16         OPTIONAL,

wlan-LocationInfo-r16     LogMeasResultListWLAN-r16       OPTIONAL,

sensor-LocationInfo-r16   Sensor-LocationInfo-r16         OPTIONAL,

...

}

-- TAG-LOCATIONINFO-STOP

-- ASN1STOP

#### –	LocationMeasurementInfo

The IE LocationMeasurementInfo defines the information sent by the UE to the network to assist with the configuration of measurement gaps for location related measurements.

LocationMeasurementInfo information element

-- ASN1START

-- TAG-LOCATIONMEASUREMENTINFO-START

LocationMeasurementInfo ::=     CHOICE {

eutra-RSTD                  EUTRA-RSTD-InfoList,

...,

eutra-FineTimingDetection   NULL,

nr-PRS-Measurement-r16      NR-PRS-MeasurementInfoList-r16

}

EUTRA-RSTD-InfoList ::= SEQUENCE (SIZE (1..maxInterRAT-RSTD-Freq)) OF EUTRA-RSTD-Info

EUTRA-RSTD-Info ::= SEQUENCE {

carrierFreq                 ARFCN-ValueEUTRA,

measPRS-Offset              INTEGER (0..39),

...

}

NR-PRS-MeasurementInfoList-r16 ::= SEQUENCE (SIZE (1..maxFreqLayers)) OF NR-PRS-MeasurementInfo-r16

NR-PRS-MeasurementInfo-r16 ::=      SEQUENCE {

dl-PRS-PointA-r16                   ARFCN-ValueNR,

nr-MeasPRS-RepetitionAndOffset-r16  CHOICE {

ms20-r16                            INTEGER (0..19),

ms40-r16                            INTEGER (0..39),

ms80-r16                            INTEGER (0..79),

ms160-r16                           INTEGER (0..159),

...

},

nr-MeasPRS-length-r16               ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

...

}

-- TAG-LOCATIONMEASUREMENTINFO-STOP

-- ASN1STOP

LocationMeasurementInfo field descriptions

carrierFreq The EARFCN value of the carrier received from upper layers for which the UE needs to perform the inter-RAT RSTD measurements.

measPRS-Offset Indicates the requested gap offset for performing RSTD measurements towards E-UTRA. It is the smallest subframe offset from the beginning of subframe 0 of SFN=0 of the serving cell of the requested gap for measuring PRS positioning occasions in the carrier frequency carrierFreq for which the UE needs to perform the inter-RAT RSTD measurements. The PRS positioning occasion information is received from upper layers. The value of measPRS-Offset is obtained by mapping the starting subframe of the PRS positioning occasion in the measured cell onto the corresponding subframe in the serving cell and is calculated as the serving cell's number of subframes from SFN=0 mod 40. The UE shall take into account any additional time required by the UE to start PRS measurements on the other carrier when it does this mapping for determining the measPRS-Offset. NOTE: Figure 6.2.2-1 in TS 36.331[10] illustrates the measPRS-Offset field.

dl-PRS-PointA The ARFCN value of the carrier received from upper layers for which the UE needs to perform the NR DL-PRS measurements.

nr-MeasPRS-RepetitionAndOffset Indicates the gap periodicity in ms and offset in number of subframes of the requested measurement gap for performing NR DL-PRS measurements.

nr-MeasPRS-length Indicates measurement gap length in ms of the requested measurement gap for performing NR DL-PRS measurements. The measurement gap length is according to in Table 9.1.2-1 in TS 38.133 [14].

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- Cond PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                   OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, ms0p01-v1700, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                              OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                                 OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}               OPTIONAL,    -- Need R

[[

allowedCG-List-r16                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL,   -- Need S

allowedPHY-PriorityIndex-r16        ENUMERATED {p0, p1}                                                 OPTIONAL    -- Need S

]],

[[

logicalChannelGroupIAB-Ext-r17      INTEGER (0..maxLCG-ID-IAB-r17)                                      OPTIONAL,   -- Need R

allowedHARQ-mode-r17                ENUMERATED {harqModeA, harqModeB}                                   OPTIONAL    -- Need R

]],

[[

enhancedLCP-r19                 SEQUENCE{

priorityAdjustmentThreshold-r19 INTEGER (1..64),

additionalPriority-r19          INTEGER (1..16),

...

}                                                                                                       OPTIONAL    -- Need R

]]

}                                                                                                       OPTIONAL,   -- Cond UL

...,

[[

channelAccessPriority-r16           INTEGER (1..4)                                                      OPTIONAL,   -- Need R

bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}                                   OPTIONAL    -- Need R

]]

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

additionalPriority The additional logical channel priority that overrides the logical channel priority configured by the field priority when the logical channel priority adjustment condition is satisfied as specified in TS 38.321 [3]. For the same logical channel configuration, the value of the field shall be smaller than that of the field priority.

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4 and DRBs.

allowedPHY-PriorityIndex This restriction applies only when the UL grant is a dynamic grant. If the field is present and the dynamic grant has a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to the dynamic grants indicating PHY-priority index equal to the values configured by this field. If the field is present and the dynamic grant does not have a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to this dynamic grant if the value of the field is p0, see TS 38.213 [13], clause 9. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any dynamic grants. Corresponds to "allowedPHY-PriorityIndex" as specified in TS 38.321 [3].

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Corresponds to 'allowedSCS-List' as specified in TS 38.321 [3]. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3].

bitRateMultiplier Bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [3]. Value x40 indicates bit rate multiplier 40, value x70 indicates bit rate multiplier 70 and so on.

bitRateQueryProhibitTimer The timer is used for bit rate recommendation query in TS 38.321 [3], in seconds. Value s0 means 0 s, s0dot4 means 0.4 s and so on.

bucketSizeDuration Value in ms. ms5 corresponds to 5 ms, value ms10 corresponds to 10 ms, and so on.

channelAccessPriority Indicates the Channel Access Priority Class (CAPC), as specified in TS 38.300 [2], to be used on uplink transmissions for operation with shared spectrum channel access in FR1. The network configures this field only for SRB2 and DRBs.

configuredGrantType1Allowed If present, or if the capability lcp-Restriction as specified in TS 38.306 [26] is not supported, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Otherwise, UL MAC SDUs from this logical channel cannot be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

logicalChannelGroup, logicalChannelGroupIAB-Ext ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to. The logicalChannelGroupIAB-Ext is only applicable to the IAB-MT. When logicalChannelGroupIAB-Ext is configured, logicalChannelGroup shall be ignored.

logicalChannelSR-Mask Controls SR triggering when a configured uplink grant of type1 or type2 is configured. true indicates that SR masking is configured for this logical channel as specified in TS 38.321 [3].

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to false if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration" in TS 38.321 [3]. The PUSCH duration is calculated based on the same length of all symbols, and the shortest length applies if the symbol lengths are different.

prioritisedBitRate Value in kiloBytes/s. Value kBps0 corresponds to 0 kiloBytes/s, value kBps8 corresponds to 8 kiloBytes/s, value kBps16 corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

priority Logical channel priority, as specified in TS 38.321 [3].

priorityAdjustmentThreshold Remaining time threshold for determining whether the additional logical channel priority configured by additionalPriority is applied for the logical channel, as specified in TS 38.321 [3]. Value in number of milliseconds.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| PDCP-CADuplication | The field is mandatory present if the DRB/SRB associated with this logical channel is configured with PDCP CA duplication in UL in the cell group in which this IE is included (i.e. the PDCP entity is associated with multiple RLC entities belonging to this cell group). Otherwise the field is optionally present, need R. |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB or multicast MRB. It is optionally present, Need R, for a logical channel with uplink if it serves an SRB. Otherwise it is absent. |

#### –	LogicalChannelIdentity

The IE LogicalChannelIdentity is used to identify one logical channel (LogicalChannelConfig) and the corresponding RLC bearer (RLC-BearerConfig) or BH RLC channel (BH-RLC-ChannelConfig) or Uu Relay RLC channel (Uu-RelayRLC-ChannelConfig) or PC5 Relay RLC channel (SL-RLC-ChannelConfigPC5).

LogicalChannelIdentity information element

-- ASN1START

-- TAG-LOGICALCHANNELIDENTITY-START

LogicalChannelIdentity ::=          INTEGER (1..maxLC-ID)

-- TAG-LOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

#### –	LTE-NeighCellsCRS-AssistInfoList

The IE LTE-NeighCellsCRS-AssistInfoList-r17 is used to provide configuration information of neighbour LTE cells to assist the UE to perform CRS interference mitigation (CRS-IM) in scenarios with overlapping spectrum for LTE and NR.

LTE-NeighCellsCRS-AssistInfoList information element

-- ASN1START

-- TAG-LTE-NEIGHCELLSCRS-ASSISTINFOLIST-START

LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17

LTE-NeighCellsCRS-AssistInfo-r17 ::=     SEQUENCE {

neighCarrierBandwidthDL-r17              ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1}   OPTIONAL,   -- Cond CRS-IM

neighCarrierFreqDL-r17                   INTEGER (0..16383)                                          OPTIONAL,   -- Need S

neighCellId-r17                          EUTRA-PhysCellId                                            OPTIONAL,   -- Need S

neighCRS-muting-r17                      ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

neighMBSFN-SubframeConfigList-r17        EUTRA-MBSFN-SubframeConfigList                              OPTIONAL,   -- Need S

neighNrofCRS-Ports-r17                   ENUMERATED {n1, n2, n4}                                     OPTIONAL,   -- Need S

neighV-Shift-r17                         ENUMERATED {n0, n1, n2, n3, n4, n5}                         OPTIONAL    -- Cond NotCellID

}

-- TAG-LTE-NEIGHCELLSCRS-ASSISTINFOLIST-STOP

-- ASN1STOP

LTE-NeighCellsCRS-AssistInfo field descriptions

neighCarrierBandwidthDL Indicates the channel bandwidth of the neighbour LTE cell in number of PRBs. If the field is absent, the UE applies the value of carrierBandwidthDL indicated in RateMatchPatternLTE-CRS for this serving cell, if configured.

neighCarrierFreqDL Indicates the downlink centre frequency of the neighbour LTE cell. If the field is absent, the UE applies the value of carrierFreqDL indicated in RateMatchPatternLTE-CRS for this serving cell, if configured.

neighCellId Indicates the physical cell ID of the neighbour LTE cell for which the other fields within the same LTE-NeighCellsCRS-AssistInfo apply. If the IE LTE-NeighCellsCRS-AssistInfoList contains multiple list entries, either this field or neighV-Shift is included in each instance. If the IE LTE-NeighCellsCRS-AssistInfoList contains multiple list entries, the entry with neighV-Shift is only used for neighbour LTE cells for which neighCellId is not provided (i.e. the entry with neighCellId takes precedence over the entry with neighV-Shift, if provided). If the IE LTE-NeighCellsCRS-AssistInfoList contains one list entry with neither this field nor neighV-Shift, the information within the entry applies to all neighbour LTE cells.

neighCRS-muting Indicates whether the CRS interference mitigation is enabled in the neighbour LTE cell, as specified in TS 36.133 [40], clause 3.6.1.1.

neighMBSFN-SubframeConfigList Indicates the MBSFN subframe configuration of the neighbour LTE cell. If RateMatchPatternLTE-CRS is configured for this serving cell and the field is absent, the UE applies the value of mbsfn-SubframeConfigList indicated in RateMatchPatternLTE-CRS for this serving cell if configured; otherwise, if the field is absent, the UE assumes MBSFN is not configured in the neighbour LTE cell.

neighNrofCRS-Ports Indicates the CRS antenna ports number of the neighbour LTE cell. If the field is absent, the UE applies the value of nrofCRS-Ports indicated in RateMatchPatternLTE-CRS for this serving cell, if configured. If RateMatchPatternLTE-CRS is not configured for this serving cell and the field is absent, the UE applies the default value n4.

neighV-Shift Indicates the shifting value v-shift of neighbour LTE cells for which the other fields within the same LTE-NeighCellsCRS-AssistInfo apply.

| Conditional Presence | Explanation |
| --- | --- |
| CRS-IM | For the serving cell with 15kHz SCS, this field is mandatory present for the UE supporting the capability of crs-IM-nonDSS-NWA-15kHzSCS-r17, but not supporting crs-IM-nonDSS-15kHzSCS-r17, if RateMatchPatternLTE-CRS is not configured for this serving cell. Otherwise it is optionally present, Need S if RateMatchPatternLTE-CRS is configured for this serving cell; Need M otherwise. For the serving cell with 30kHz SCS, this field is mandatory present for the UE supporting the capability of crs-IM-nonDSS-NWA-30kHzSCS-r17, but not supporting crs-IM-nonDSS-30kHzSCS-r17, if RateMatchPatternLTE-CRS is not configured for this serving cell. Otherwise it is optionally present, Need S if RateMatchPatternLTE-CRS is configured for this serving cell; Need M otherwise. |
| NotCellID | If the field neighCellId is present, this field shall be absent; otherwise, it is optionally present if the IE LTE-NeighCellsCRS-AssistInfoList contains one list entry, Need S, or it is mandatory present if the IE LTE-NeighCellsCRS-AssistInfoList contains multiple list entries. |

#### –	LTM-CandidateId

The IE LTM-CandidateId is used to identify an LTM candidate configuration.

LTM-CandidateId information element

-- ASN1START

-- TAG-LTM-CANDIDATEID-START

LTM-CandidateId-r18 ::=                             INTEGER (1..maxNrofLTM-Configs-r18)

-- TAG-LTM-CANDIDATEID-STOP

-- ASN1STOP

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     LTM-NoSecurityChangeId-r19                            OPTIONAL,    -- Need M

ltm-ExecutionCondition-r19                     SetupRelease {LTM-ExecutionConditionList-r19}         OPTIONAL,    -- Need M

ltm-NZP-CSI-RS-ResourceToAddModList-r19        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r19       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

ltm-CSI-ReportConfig-r19                       LTM-CSI-ReportConfig-r18                              OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceToAddModList-r19            SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceToReleaseList-r19           SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToAddModList-r19         SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToReleaseList-r19        SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId

OPTIONAL     -- Need N

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need S

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

LTM-NoSecurityChangeId-r19 ::= INTEGER (1..maxNrofLTM-Configs-plus1-r18)

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-ExecutionCondition This field can only be included in an ltm-Config associated with the MCG.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NoSecurityChangeID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

LTM-SSB-Config field descriptions

ssb-Periodicity The SSB periodicity in ms. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1).

ssb-PositionsInBurst For operation in licensed spectrum, indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network always includes this field if ltm-SSB-Config is configured.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7. The network always includes this field if ltm-SSB-Config is configured.

#### –	LTM-Config

The IE LTM-Config is used to provide LTM configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Need M

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19   LTM-NoSecurityChangeId-r19                                          OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19   SetupRelease {LTM-ExecutionConditionList-r19}                       OPTIONAL    -- Need M

]]

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

ltm-ServingCellExecutionCondition This field can can only be included in an ltm-Config associated with the MCG.

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |

#### –	LTM-ConfigNRDC

The IE LTM-ConfigNRDC is used to provide LTM configurations in NR-DC.

LTM-ConfigNRDC information element

-- ASN1START

-- TAG-LTM-CONFIGNRDC-START

LTM-ConfigNRDC-r19 ::=                 SEQUENCE {

ltm-ConfigurationSCG-r19               LTM-Config-r18                                                            OPTIONAL, -- Need M

ltm-SK-CounterConfigToAddModList-r19   SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19    OPTIONAL, -- Need N

ltm-SK-CounterConfigToReleaseList-r19  SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF LTM-NoSecurityChangeId-r19 OPTIONAL, -- Need N

...

}

-- TAG-LTM-CONFIGNRDC-STOP

-- ASN1STOP

LTM-ConfigNRDC field descriptions

ltm-ConfigurationSCG The network does not configure this field in an RRCReconfiguration message within an LTM-Config IE and ConditionalReconfiguration IE.

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure gNB-scheduled measurement report on the cell in which the LTM-CSI-ReportConfig is included when the field ltm-ReportConfigType is configured as periodic, semi-persistentOnPUCCH, or semi-persistentOnPUSCH/aperiodic.

The IE LTM-CSI-ReportConfig is also used to configure events for the conditional execution of an LTM cell switch procedure.

The IE LTM-CSI-ReportConfig is also used to configure event-triggered measurement report when the field ltm-ReportConfigType is configured as eventTriggered, within which the criteria for triggering measurement report by MAC CE as in TS 38.321 [3] is specified.

Event LTM2:	Beam of SpCell becomes worse than absolute threshold;

Event LTM3:	Beam of candidate cell becomes amount of offset better than the beam of SpCell;

Event LTM4:	Beam of candidate cell becomes better than absolute threshold;

Event LTM5:	Beam of SpCell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

candidateSpecificOffsetS-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL  -- Need R

]]

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

LTM-EventTriggeredPeriodicReport-r19 ::= SEQUENCE {

reportInterval-r19                             ReportInterval-v1900,

reportAmount-r19                               ENUMERATED {r2, r4, r8, r16, r32, r64, infinity, spare1},

...

}

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                                       OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                                       OPTIONAL, -- Cond LTM2

...

}

LTM-CandidateReportConfig-r19 ::=            SEQUENCE {

ltm-CandidateReportConfigId-r19                LTM-CandidateId-r18,

candidateSpecificOffset-r19                    MeasTriggerQuantityOffset                              OPTIONAL, -- Cond notEventLTM2

...

}

LTM-ReportContent-v1900  ::=                SEQUENCE {

reportQuantity-r19                             ENUMERATED {cri-RSRP, ssb-index-RSRP, cri-RI-PMI-CQI, value1},

...

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

LTM-CandidateReportConfig field descriptions

ltm-CandidateReportConfigId LTM candidate cell ID for which the UE is required to measure reference signal and perform LTM event evaluation as specified in TS 38.321 [3].

candidateSpecificOffset Offset for event condition that is applicable for all the reference signals belonging to the candidate cell with the candidate cell ID ltm-CandidateReportConfigId. If the field is absent, the value '0dB' is applied.

candidateSpecificOffsetS Offset for event condition that is applicable for all the reference signals belonging to serving cell. If the field is absent, the value '0dB' is applied.

LTM-CSI-ReportConfig field descriptions

eventId Type of LTM event for triggering event-triggered measurement report as specified in TS 38.321 [3].

hysteresis Hysteresis when evaluating the entering/leaving conditions for an LTM event.

ltm-CandidateReportConfigList List of report configurations for LTM candidate IDs. If the field is absent the UE shall measure all the LTM candidate cells associated to the field ltm-ResourcesForChannelMeasurement.

ltm-EventTriggeredPeriodicReport This field indicates when an LTM event is triggered, whether the event-triggered measurement report is sent periodically. If the field is absent, the event-triggered measurement report is sent once, as specified in TS 38.321 [3].

ltm-EventTriggeredReportContent This field indicates what to include in a measurement report when an LTM event is triggered. When this field is absent, the field ltm-ReportConfigType is set to eventTriggered, and the corresponding LTM-CSI-ReportConfigId is part of an LTM-ExecutionConditionList IE, when the associated LTM event is fulfilled, the UE triggers an LTM cell switch procedure instead of an event-triggered measurement report, as specified in TS 38.321 [3].

ltm-ReportConfigType This field specifies how the UE shall report the measurement results for LTM either by gNB-scheduled measurement report or by event-triggered measurement report by MAC CE. The UE shall ignore this field if LTM-CSI-ReportConfig is configured in a LTM-Candidate IE.

ltm-ReportContent This field defines the content of the LTM L1 measurement report. The UE shall ignore this field if the field ltm-ReportConfigType is set to eventTriggered.

ltm-ResourcesForChannelMeasurement, ltm-ResourceForInterferenceMeasurements This field indicates the index of SSB or CSI-RS in the field LTM-CSI-ResourceConfig.

ltm2-Threshold, ltm4-Threshold, ltm5-Threshold1, ltm5-Threshold2 Thresholds defined in the entering/leaving conditions for different LTM events.

ltm3-Offset Offset for the entering/leaving condition for event LTM3. The actual value is field value * 0.5 dB.

reportOnLeave Indicates whether the event-triggered measurement report by MAC CE shall be triggered when leaving condition is satisfied, as specified in TS 38.321 [3].

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). The UE shall ignore the offset provided by this field in case semiPersistentOnPUSCH is configured.

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH and aperiodic reporting.

LTM-ReportContent field descriptions

nrOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

nrOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

reportQuantity Indicates the report quantity for the CSI report.

LTM-EventTriggeredReportContent field descriptions

allowReportAnyBeam Indicates whether the UE can report the measurement results for the beams not satisfying the conditions of the events at least for the time duration configured by timeToTrigger as specified in TS 38.321 [3].

maxNumberOfReportedBeams This field defines maximum number of beams whose measurements results can be reported in the event-triggered measurement report by MAC CE as specified in TS 38.321 [3].

reportCurrentBeam Indicates whether the UE is required to report the measurement result of the current beam as specified in TS 38.321 [3].

LTM-EventTriggeredPeriodicReport field descriptions

reportInterval This field defines the periodicity of the event-triggered periodic measurement report as specified in TS 38.321 [3].

reportAmount Number of measurement reports needs to be transmitted after the event is triggered as specified in TS 38.321 [3]. Value 'r2' means the report is sent twice, ’r3’ means the report is sent three times, and so on.

| Conditional Presence | Explanation |
| --- | --- |
| LTM2 | This field is mandatory in case the eventId is configured as eventLTM2. Otherwise, it is optionally present, need R. |
| notEventLTM2 | This field is not present when the eventId is configured as eventLTM2. Otherwise, it is optionally present, need S. |
| onlyLTM3 | This fiels is optionally present, need S, when eventId is configured as eventLTM3. Otherwise, it is absent. |

#### –	LTM-CSI-ReportConfigId

The IE LTM-CSI-ReportConfigId is used to identify an LTM-CSI-ReportConfig.

LTM-CSI-ReportConfigId information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIGID-START

LTM-CSI-ReportConfigId-r18 ::=            INTEGER (0..maxNrofLTM-CSI-ReportConfigurations-1-r18)

-- TAG-LTM-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18             LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19      LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19          LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                    ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond NoSSB

]]

}

LTM-SSB-ResourceSet-r18 ::=         SEQUENCE {

ltm-SSB-ResourceList-r18            SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=  SEQUENCE {

ltm-CSI-RS-ResourceList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-CSI-IM-ResourceSet-r19 ::=      SEQUENCE {

ltm-CSI-IM-ResourceSetId-r19        CSI-IM-ResourceSetId,

ltm-CandidateId-r19                 LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

LTM-CSI-ResourceConfig field descriptions

ltm-CSI-IM-ResourceSet This field indicates the resource set for LTM interference measurements. This field is absent if ltm-NZP-CSI-RS-ResourceSet is configured. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet.

ltm-NZP-CSI-RS-ResourceSet This field indicates the resource set for LTM measuremenet based on CSI-RS. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet.

ltm-SSB-ResourceSet This field indicates the resource set for LTM measurement based on SSB.

resourceType Time domain behavior of LTM CSI resource (see TS 38.214 [19], clause 5.2.1.2). .

LTM-SSB-ResourceSet field descriptions

ltm-CandidateIdList This field indicates the LTM candidate configuration IDs related to the SSBs in the ltm-SSB-ResourceList. The list has the same number of entries as ltm-SSB-ResourceList. The first entry in this list shall be associated to the first entry in ltm-SSB-ResourceList, the second entry of this list shall be associated to the second entry in ltm-SSB-ResourceList, and so on.

ltm-SSB-ResourceList This field is used to indicate on SS/PBCH block resources from one or more LTM candidate cells.

LTM-NZP-CSI-RS-ResourceSet field descriptions

ltm-NZP-CSI-RS-ResourceList This field is used to indicate on NZP CSI-RS resources from one or more LTM candidate cells. If the resource type of the NZP-CSI-RS resource is semi-persistent, the ltm-ReportConfigType within LTM-CSI-ReportConfig that this resource set is associated with cannot be configured as eventTriggered.

LTM-CSI-IM-ResourceSet field descriptions

ltm-CSI-IM-ResourceList This field is used to indicate on CSI-IM resources from one or more LTM candidate cells.

| Conditional Presence | Explanation |
| --- | --- |
| NoSSB | This field is mandatory present, Need R, if the field ltm-NZP-CSI-RS-ResourceSet or ltm-CSI-IM-ResourceSet is configured. Otherwise, it is absent. |

#### –	LTM-CSI-ResourceConfigId

The IE LTM-CSI-ResourceConfigId is used to identify an LTM-CSI-ResourceConfig.

LTM-CSI-ResourceConfigId information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIGID-START

LTM-CSI-ResourceConfigId-r18 ::=            INTEGER (0..maxNrofLTM-CSI-ResourceConfigurations-1-r18)

-- TAG-LTM-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	LTM-ExecutionConditionList

The IE LTM-ExecutionConditionList is used to configure LTM cell switch conditions.

LTM-ExecutionConditionList information element

-- ASN1START

-- TAG-LTM-EXECUTIONCONDITIONLIST-START

LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19

LTM-ExecutionCondition-r19 ::=     SEQUENCE {

ltm-CandidateId-r19                LTM-CandidateId-r18,

executionCondition-r19             CHOICE {

l1-Conditions-r19                  LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                  SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

...

}

-- TAG-LTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

#### –	LTM-TCI-Info

The IE LTM-TCI-Info is used to configure TCI related information for an LTM candidate configuration to be used during activation of TCI state(s) and/or upon the reception of the LTM Cell Switch procedure.

LTM-TCI-Info information element

-- ASN1START

-- TAG-LTM-TCI-INFO-START

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...

}

-- TAG-LTM-TCI-INFO-STOP

-- ASN1STOP

LTM-TCI-Info field descriptions

ltm-NZP-CSI-RS-ResourceSetToAddModList When the field ltm-NZP-CSI-RS-ResourceSetToAddModList is configured within an LTM-Candidate IE, this field is absent

ltm-NZP-CSI-RS-ResourceSetToReleaseList When the field ltm-NZP-CSI-RS-ResourceSetToReleaseList is configured within an LTM-Candidate IE, this field is absent

ltm-NZP-CSI-RS-ResourceToAddModList When the field ltm-NZP-CSI-RS-ResourceToAddModList is configured within an LTM-Candidate IE, this field is absent

ltm-NZP-CSI-RS-ResourceToReleaseList When the field ltm-NZP-CSI-RS-ResourceToReleaseList is configured within an LTM-Candidate IE, this field is absent

unifiedTCI-StateType Indicates the unified TCI states type the UE is configured for this LTM candidate configuration. The value separate means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StateToAddModList for UL TCI states. The value joint means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation. The network always configures this field.

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup-r16 }                   OPTIONAL    -- Need M

]],

[[

enhancedSkipUplinkTxDynamic-r16     ENUMERATED {true}                                               OPTIONAL,   -- Need R

enhancedSkipUplinkTxConfigured-r16  ENUMERATED {true}                                               OPTIONAL    -- Need R

]],

[[

intraCG-Prioritization-r17          ENUMERATED {enabled}                        OPTIONAL,    -- Cond LCH-PrioWithReTxTimer

drx-ConfigSL-r17                    SetupRelease { DRX-ConfigSL-r17 }           OPTIONAL,    -- Need M

drx-ConfigExt-v1700                 SetupRelease { DRX-ConfigExt-v1700 }        OPTIONAL,    -- Need M

schedulingRequestID-BFR-r17         SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestID-BFR2-r17        SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestConfig-v1700       SchedulingRequestConfig-v1700               OPTIONAL,    -- Need M

tar-Config-r17                      SetupRelease { TAR-Config-r17  }                                OPTIONAL,    -- Need M

g-RNTI-ConfigToAddModList-r17       SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17       OPTIONAL,    -- Need N

g-RNTI-ConfigToReleaseList-r17      SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17     OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToAddModList-r17    SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17    OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToReleaseList-r17   SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17  OPTIONAL,    -- Need N

allowCSI-SRS-Tx-MulticastDRX-Active-r17   BOOLEAN                                                           OPTIONAL     -- Need M

]],

[[

schedulingRequestID-PosMG-Request-r17 SchedulingRequestId                                                   OPTIONAL,    -- Need R

drx-LastTransmissionUL-r17          ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

posMG-Request-r17                   ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

drx-ConfigExt2-v1800                SetupRelease { DRX-ConfigExt2-v1800 }                                   OPTIONAL,    -- Need M

additionalBS-TableAllowed-r18       BIT STRING (SIZE (maxNrofLCGs-r18))                                     OPTIONAL,    -- Need R

dsr-ConfigToAddModList-r18          SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-DSR-Config-r18              OPTIONAL,    -- Need N

dsr-ConfigToReleaseList-r18         SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-Id-r18                      OPTIONAL,    -- Need N

tar-Config-r18                      SetupRelease { TAR-Config-r18  }                                        OPTIONAL     -- Need M

]],

[[

ul-RateQuery-r19                    SEQUENCE {

ul-RateQueryConfigList-r19          SEQUENCE (SIZE (1..maxNrofRateQueryQFIs-r19)) OF QoS-FlowIdentity-r19,

ul-RateQueryProhibitTimer-r19       ENUMERATED {s0, s0dot1, s0dot2, s0dot5, s1, s2, s5, s10, s20, s30, s60,

s90, s120, s300, s600, spare1},

...

}                                                                                                       OPTIONAL,    -- Need R

ul-RateControlConfigList-r19        SEQUENCE (SIZE (1..maxNrofRateCtrlQFIs-r19)) OF QoS-FlowIdentity-r19    OPTIONAL,    -- Need R

schedulingRequestID-LTM-r19         SchedulingRequestId                                                     OPTIONAL     -- Need R

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

MBS-RNTI-SpecificConfig-r17 ::=        SEQUENCE {

mbs-RNTI-SpecificConfigId-r17          MBS-RNTI-SpecificConfigId-r17,

groupCommon-RNTI-r17                   CHOICE {

g-RNTI                                 RNTI-Value,

g-CS-RNTI                              RNTI-Value

},

drx-ConfigPTM-r17                      SetupRelease { DRX-ConfigPTM-r17 }                          OPTIONAL,   -- Need M

harq-FeedbackEnablerMulticast-r17      ENUMERATED {dci-enabler, enabled}                           OPTIONAL,   -- Need S

harq-FeedbackOptionMulticast-r17       ENUMERATED {ack-nack, nack-only}                            OPTIONAL,   -- Cond HARQFeedback

pdsch-AggregationFactor-r17            ENUMERATED {n2, n4, n8}                                     OPTIONAL    -- Cond G-RNTI

}

MBS-RNTI-SpecificConfigId-r17 ::= INTEGER (0..maxG-RNTI-1-r17)

LCG-DSR-Config-r18 ::= SEQUENCE {

lcg-Id-r18                      LCG-Id-r18,

remainingTimeThreshold-r18      INTEGER (1..64),

...,

[[

dsr-ReportingThresList-r19      SEQUENCE (SIZE (1..maxDSR-ReportingThres-r19)) OF DSR-ReportingThreshold-r19  OPTIONAL,    -- Need R

dsr-ReportNonDelayCriticalData-r19  ENUMERATED {enabled}                                        OPTIONAL    -- Cond ReportThresList

]]

}

LCG-Id-r18 ::= INTEGER (0..maxLCG-ID)

DSR-ReportingThreshold-r19 ::= INTEGER (1..64)

QoS-FlowIdentity-r19 ::= SEQUENCE {

pdu-SessionID-r19                          PDU-SessionID,

qfi-r19                                    QFI

}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

additionalBS-TableAllowed Indicates whether a UE is allowed to utilize the refined buffer size levels, as specified in TS 38.321 [3], for a certain Logical Channel Group. The leftmost bit corresponds to LCG ID=0, second leftmost bit to LCG ID=1 and so on. The UE is allowed to utilize the refined buffer size levels for a Logical Channel Group only when the corresponding bit is set to 1.

allowCSI-SRS-Tx-MulticastDRX-Active Used to control the CSI/SRS transmission during MBS multicast DRX ActiveTime, see TS 38.321 [3].

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config, drx-ConfigExt, drx-ConfigExt2 Used to configure DRX as specified in TS 38.321 [3]. Network only configures drx-ConfigExt or drx-ConfigExt2 when drx-Config is configured.

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

drx-ConfigSL Used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3]. Network only configures this field if sl-ScheduledConfig is configured and drx-Config is configured.

drx-LastTransmissionUL If this field is present, the start of the drx-HARQ-RTT-TimerUL is after the last transmission within a bundle, see TS 38.321 [3].

dsr-ConfigToAddModList List of LCG-specific DSR configurations to add or modify.

dsr-ConfigToReleaseList List of LCG-specific DSR configurations to release.

g-RNTI-ConfigToAddModList List of G-RNTI configurations to add or modify. Up to 8 G-RNTIs can be configured in total in this release based on the UE capability.

g-RNTI-ConfigToReleaseList List of G-RNTI configurations to release.

g-CS-RNTI-ConfigToAddModList List of G-CS-RNTI configurations to add or modify. Up to 8 G-CS-RNTIs can be configured in total in this release based on the UE capability.

g-CS-RNTI-ConfigToReleaseList List of G-CS-RNTI configurations to release.

intraCG-Prioritization Used to enable HARQ process ID selection based on LCH-priority for one CG as specified in TS 38.321 [3].

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3]. The network does not configure lch-BasedPrioritization with enhancedSkipUplinkTxDynamic simultaneously nor lch-BasedPrioritization with enhancedSkipUplinkTxConfigured simultaneously.

posMG-Request Indicates whether UE is configured to send UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-BFR Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet1 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet2 of the same serving cell.

schedulingRequestID-BFR2 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet2 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet1 of the same serving cell.

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

schedulingRequestID-LTM Indicates the scheduling request configuration applicable for event-triggered measurement report by MAC CE, as specified in TS 38.321 [3].

schedulingRequestID-PosMG-Request Indicates the scheduling request configuration applicable for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

skipUplinkTxDynamic, enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured If set to true, the UE skips UL transmissions as described in TS 38.321 [3]. If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, REPETITION_NUMBER (as specified in TS 38.321 [3], clause 5.4.2.1) of the corresponding PUSCH transmission of the uplink grant shall be equal to 1. The network does not configure enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true together with numberOfSlotsTBoMS-r17. The network does not configure skipUplinkTxDynamic with value true if eventTypeUE-IBR is configured for a serving cell in the corresponding MAC entity.

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

ul-RateControlConfigList Includes the list of QoS flows for which the UL rate control is enabled.

ul-RateQueryConfigList Includes the list of QoS flows for which the UL rate query is enabled.

ul-RateQueryProhibitTimer This timer is used for uplink date rate query MAC CE as specified in TS 38.321 [3]. Value s0 means 0 s, s0dot1 means 0.1 s and so on.

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR, see TS 38.321 [3].

MBS-RNTI-SpecificConfig field descriptions

drx-ConfigPTM Used to configure DRX for PTM transmission as specified in TS 38.321 [3].

g-CS-RNTI Used to scramble the SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH for one or more MBS multicast services.

g-RNTI Used to scramble the scheduling and transmission of PTM for one or more MBS multicast services.

groupCommon-RNTI Used to configure g-RNTI or g-CS-RNTI.

harq-FeedbackEnablerMulticast Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI as specified in TS 38.213 [13]. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent, the UE behavior is specified in TS 38.213 [13].

harq-FeedbackOptionMulticast Indicates the feedback mode for MBS multicast dynamically scheduled PDSCH or SPS PDSCH.

mbs-RNTI-SpecificConfigId An identifier of the RNTI specific configuration for MBS multicast.

pdsch-AggregationFactor Number of repetitions for dynamically scheduled MBS multicast data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent and groupCommon-RNTI is set to g-RNTI, the UE applies the value 1.

LCG-DSR-Config field descriptions

dsr-ReportingThresList List of remaining time thresholds configured in ascending order for reporting delay status information in the Multiple Entry DSR MAC CE, as specified in TS 38.321 [3]. At least one configured DSR reporting threshold should be no lower than the remainingTimeThreshold. If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with remainingTimeThreshold should be configured with dsr-ReportingThresList. The IE DSR-ReportingThreshold in number of milliseconds.

dsr-ReportNonDelayCriticalData Indicates whether the UE should consider the non-delay reporting data ahead of delay reporting data in the delay status reporting data volume calculation for the Logical Channel Group as specified in TS 38.323 [5].

lcg-Id Identifier of the Logical Channel Group which the DSR configuration refers to.

remainingTimeThreshold Remaining time threshold used for triggering Single Entry and Multiple Entry DSR for the logical channels belonging to this Logical Channel Group, as specified in TS 38.321 [3]. Value in number of milliseconds.

QoS-FlowIdentity field descriptions

qfi Identifier of the QoS flow for which bit rate query or bit rate control is enabled.

pdu-SessionID Identifier of the PDU session to which the QoS flow idenfitied by the field qfi belongs.

| Conditional Presence | Explanation |
| --- | --- |
| G-RNTI | This field is optionally present, Need S, if groupCommon-RNTI is set to g-RNTI. The field is absent when groupCommon-RNTI is set to g-CS-RNTI. |
| HARQFeedback | The field is mandatory present when harq-FeedbackEnablerMulticast is present. It is absent otherwise. |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |
| LCH-PrioWithReTxTimer | This field is optionally present, Need R, if lch-BasedPrioritization-r16 is configured in this MAC entity and cg-RetransmissionTimer-r16 is configured for any configured grant configuration associated with this MAC entity. It is absent otherwise, Need R. |
| ReportThresList | This field is optionally present, Need R, if the field dsr-ReportingThresList is present. It is absent otherwise. |

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]],

[[

effectiveMeasWindowConfig-r18       SetupRelease {MeasWindowConfig-r18}                                 OPTIONAL    -- Need M

]],

[[

cssf-Config-r19                     SEQUENCE {

cssf-MeasMO-List-r19                CSSF-MeasMO-List-r19                                            OPTIONAL    -- Need R

}                                                                                                       OPTIONAL    -- Need R

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

CSSF-MeasMO-List-r19 ::=                SEQUENCE (SIZE (1..maxNrofMeasMO-r19)) OF MeasObjectId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

cssf-Config Used to enable or disable the CSSF optimization feature.

cssf-MeasMO-List Used to indicate which measurement object associated to SCell (i.e. SCC MO) is measured as defined in TS 38.133 [14]. At most one SCC MO per band is configured in this list. The SCC MO for a given band is indicated only if there are more than one SCC MO configured with SSB based measurement in the band, and neither PCell nor PSCell is configured in the band.

effectiveMeasWindowConfig Used to setup and release effective measurement window in NR for E-UTRA measurements.

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps. In NR-DC, the field can only be configured in the measConfig associated with MCG, and when configured, it applies to all the inter-frequency measurements configured by MN and SN.

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS. This field is also applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18. This field is not configured to a UE configured with event(s) related to the L2 U2N Relay UE.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEASGAPCONFIG-START

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                                              OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                                              OPTIONAL    -- Need M

]],

[[

gapToAddModList-r17           SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17                    OPTIONAL,   -- Need N

gapToReleaseList-r17          SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17                    OPTIONAL,   -- Need N

posMeasGapPreConfigToAddModList-r17      PosMeasGapPreConfigToAddModList-r17                                OPTIONAL,   -- Need N

posMeasGapPreConfigToReleaseList-r17     PosMeasGapPreConfigToReleaseList-r17                               OPTIONAL    -- Need N

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16           ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

mgl-r16                             ENUMERATED {ms10, ms20}                                             OPTIONAL    -- Cond PRS

]]

}

GapConfig-r17 ::=                   SEQUENCE {

measGapId-r17                       MeasGapId-r17,

gapType-r17                         ENUMERATED {perUE, perFR1, perFR2},

gapOffset-r17                       INTEGER (0..159),

mgl-r17                             ENUMERATED {ms1, ms1dot5, ms2, ms3, ms3dot5, ms4, ms5, ms5dot5, ms6, ms10, ms20},

mgrp-r17                            ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17                            ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot75},

refServCellIndicator-r17            ENUMERATED {pCell, pSCell, mcg-FR2}                                 OPTIONAL,   -- Cond NEDCorNRDC

refFR2-ServCellAsyncCA-r17          ServCellIndex                                                       OPTIONAL,   -- Cond AsyncCA

preConfigInd-r17                    ENUMERATED {true}                                                   OPTIONAL,   -- Need R

ncsgInd-r17                         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

gapAssociationPRS-r17               ENUMERATED {true}                                                   OPTIONAL,   -- Need R

gapSharing-r17                      MeasGapSharingScheme                                                OPTIONAL,   -- Need R

gapPriority-r17                     GapPriority-r17                                                     OPTIONAL,   -- Need R

...

}

PosMeasGapPreConfigToAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF PosGapConfig-r17

PosMeasGapPreConfigToReleaseList-r17 ::= SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF MeasPosPreConfigGapId-r17

PosGapConfig-r17 ::=                SEQUENCE {

measPosPreConfigGapId-r17           MeasPosPreConfigGapId-r17,

gapOffset-r17                       INTEGER (0..159),

mgl-r17                             ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

mgrp-r17                            ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17                            ENUMERATED {ms0, ms0dot25, ms0dot5},

gapType-r17                         ENUMERATED {perUE, perFR1, perFR2},

...

}

MeasPosPreConfigGapId-r17 ::= INTEGER (1..maxNrofPreConfigPosGapId-r17)

-- TAG-MEASGAPCONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapAssociationPRS Indicates that PRS measurement is associated with this measurement gap. The network only includes this field for one per-UE gap or for one per-FR gap. If concurrent gap (i.e. one of the gap combination as defined in Table 9.1.8-1 in TS 38.133 [14]) is configured and no gap is configured with this field, the PRS measurement is associated with the gap configured via GapConfig (without suffix), if available. If both per-UE gap and per-FR gap are configured via GapConfig and/or GapConfig-r17, the PRS measurement is always associated with the per-UE gap.

gapFR1 Indicates measurement gap configuration that applies to FR1 only. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 measurement gap). In NE-DC, gapFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap). In NR-DC, gapFR1 can only be set up in the measConfig associated with MCG. gapFR1 can not be configured together with gapUE. The applicability of the FR1 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapFR2 Indicates measurement gap configuration applies to FR2 only. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap). In NR-DC, gapFR2 can only be set up in the measConfig associated with MCG. gapFR2 cannot be configured together with gapUE. The applicability of the FR2 measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range is from 0 to mgrp-1. If ncsgInd-r17 is present, this offset value refers to the starting point of VIL1 (the visible interruption length before the ML).

gapPriority Indicates the priority of this measurement gap (see TS 38.133 [14]). Value 1 indicates highest priority, value 2 indicates second level priority, and so on.

gapSharing Indicates the measurement gap sharing scheme that applies to this GapConfig. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapToAddModList A list of of measurement gap configuration to be added or modified. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8 for TN or clause 9.1C.8 for NTN), the maximum number of configured measurement gap is limited by the gap combinations defined in Table 9.1.8-1 in TS 38.133 [14] for TN.For NTN the maximum number of configured measurement gap is limited by UE capability indicated in parallelMeasurementGap. In this version of the specification, the network configures this field only in NR standalone. This field is used only for a UE that supports pre-configured measurement gap, concurrent measurement gap, or NCSG. In this version of the specification, the network does not configure concurrent measurement gap together with preconfigured measurement gap for positioning. For the UE supporting the capability of musim-GapPriorityPreference, the network can configure concurrent measurement gap together with MUSIM gap. Otherwise, the network does not configure concurrent measurement gap together with MUSIM gap.

gapToReleaseList A list of measurement gap configuration to be released.

gapType Indicates the type of this measurement gap. Value perUE indicates that it is a per UE measurement gap, value perFR1 indicates that it is an FR1 measurement gap, and value perFR2 indicates that it is an FR2 measurement gap.

gapUE Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). In (NG)EN-DC, gapUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE measurement gap). In NE-DC, gapUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap). In NR-DC, gapUE can only be set up in the measConfig associated with MCG. If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. For TN, the applicability of the per UE measurement gap is according to Table 9.1.2-2 and Table 9.1.2-3 for non-RedCap UEs, and Table 9.1A.2-2 for RedCap UEs in TS 38.133 [14]. For NTN, the applicability of per UE measurement gap is according to subclause 9.1C.2 in TS 38.133 [14].

measGapId The ID of this measurement gap configuration.

mgl Value mgl is the measurement gap length in ms of the measurement gap. If ncsgInd-r17 is not present, the measurement gap length is according to Table 9.1.2-1 for non-RedCap UEs in TN, Table 9.1A.2-1 for RedCap UEs in TN, and Table 9.1C.2-1 for NTN in TS 38.133 [14]. If ncsgInd-r17 is present, this field indicates the measurement length (ML) in NCSG pattern and is configured according to Table 9.1.9.3-1 in TS 38.133 [14]. Value ms1dot5 corresponds to 1.5 ms, ms3 corresponds to 3 ms and so on. If mgl-r16 is present, UE shall ignore the mgl (without suffix). Value ms1, ms2, and ms5 can only be configured if ncsgInd is present.

mgrp If ncsgInd-r17 is not present, the mgrp field indicates the measurement gap repetition period in (ms) of the measurement gap according to Table 9.1.2-1 for non-RedCap UEs in TN, Table 9.1A.2-1 for RedCap UEs in TN, and Table 9.1C.2-1 for NTN in TS 38.133 [14]. If ncsgInd-r17 is present, the mgrp field indicates the Visible Interruption Repetition Period (VIRP) of NCSG pattern and is configured according to Table 9.1.9.3-1 in TS 38.133 [14].

mgta Value mgta is the measurement gap timing advance in ms. The applicability of the measurement gap timing advance is according to clause 9.1.2 for non-RedCap UEs in TN, clause 9.1A.2 for RedCap UEs in TN, and clause 9.1C.2 for NTN of TS 38.133 [14], or according to clause 9.1.9 of TS 38.133 [14] if ncsgInd is present. Value ms0 corresponds to 0 ms, ms0dot25 corresponds to 0.25 ms, ms0dot5 corresponds to 0.5 ms and ms0dot75 corresponds to 0.75 ms. For FR2, the network only configures 0 ms and 0.25 ms if ncsgInd is not present. If ncsgInd is present, the network only configures 0ms for per-UE NCSG and FR1 NCSG and only configures 0ms or 0.75ms for FR2 NCSG. Value ms0dot75 can only be configured if ncsgInd is present.

ncsgInd Indicates that the measurement gap is a NCSG as specified in 38.133 [14].

posMeasGapPreConfigToAddModList List of preconfigured measurement gap for positioning to add and/or modify. All the gaps configured are associated with the measurement of PRS for RSTD, UE-RxTx Time Difference, PRS-RSRP and PRS-RSRPP as defined in TS 38.215 [9]. In this version of the specification, the network does not configure preconfigured measurement gap for positioning together with concurrent measurement gap or MUSIM gap.

posMeasGapPreConfigToReleaseList List of preconfigured measurement gap for positioning to release.

preConfigInd Indicates whether the measurement gap is a pre-configured measurement gap.

refFR2ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

refServCellIndicator Indicates the serving cell whose SFN and subframe are used for gap calculation for this gap pattern. Value pCell corresponds to the PCell, pSCell corresponds to the PSCell, and mcg-FR2 corresponds to a serving cell on FR2 frequency in MCG.

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring and reconfiguring FR2 gap pattern to UE in: - (NG)EN-DC or NR SA with asynchronous CA involving FR2 carrier(s); - NE-DC or NR-DC with asynchronous CA involving FR2 carrier(s), if the field refServCellIndicator is set to mcg-FR2. Otherwise, it is absent, Need R. |
| NEDCorNRDC | This field is mandatory present when configuring and reconfiguring gap pattern to UE in NE-DC or NR-DC. Otherwise, it is absent, Need R. |
| PRS | This field is optionally present, Need R, when configuring gap pattern to UE for measurements of DL-PRS configured via LPP (TS 37.355 [49]). Otherwise, it is absent. |

#### –	MeasGapId

The IE MeasGapId used to identify a per UE or per FR measurement gap configuration.

MeasGapId information element

-- ASN1START

-- TAG-MEASGAPID-START

MeasGapId-r17 ::=                       INTEGER (1..maxNrofGapId-r17)

-- TAG-MEASGAPID-STOP

-- ASN1STOP

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

-- TAG-MEASGAPSHARINGCONFIG-START

MeasGapSharingConfig ::=        SEQUENCE {

gapSharingFR2                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   -- Need M

...,

[[

gapSharingFR1                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   --Need M

gapSharingUE                    SetupRelease { MeasGapSharingScheme }       OPTIONAL    --Need M

]]

}

MeasGapSharingScheme::=         ENUMERATED {scheme00, scheme01, scheme10, scheme11}

-- TAG-MEASGAPSHARINGCONFIG-STOP

-- ASN1STOP

MeasGapSharingConfig field descriptions

gapSharingFR1 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR1. In (NG)EN-DC, gapSharingFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap sharing). In NE-DC, gapSharingFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap sharing). In NR-DC, gapSharingFR1 can only be set up in the measConfig associated with MCG. gapSharingFR1 can not be configured together with gapSharingUE. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingFR2 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR2. In (NG)EN-DC or NE-DC, gapSharingFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap sharing). In NR-DC, gapSharingFR2 can only be set up by MCG in the measConfig associated with MCG. gapSharingFR2 cannot be configured together with gapSharingUE. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingUE Indicates the measurement gap sharing scheme that applies to the gap set via gapUE. In (NG)EN-DC, gapSharingUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap sharing). In NE-DC, gapSharingUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap sharing). In NR-DC, gapSharingUE can only be set up in the measConfig associated with MCG. If gapSharingUE is configured, then neither gapSharingFR1 nor gapSharingFR2 can be configured. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEASID-START

MeasId ::=                          INTEGER (1..maxNrofMeasId)

-- TAG-MEASID-STOP

-- ASN1STOP

#### –	MeasIdleConfig

The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE.

MeasIdleConfig information element

-- ASN1START

-- TAG-MEASIDLECONFIG-START

MeasIdleConfigSIB-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need S

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need S

...,

[[

measIdleCarrierListNR-LessThan5MHz-r18   SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16       OPTIONAL,    -- Cond LessThan5MHz

measReselectionCarrierListNR-r18     SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasReselectionCarrierNR-r18    OPTIONAL,    -- Need S

measReselectionCarrierListNR-LessThan5MHz-r18   SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasReselectionCarrierNR-r18  OPTIONAL,    -- Cond LessThan5MHz

measIdleValidityDuration-r18         MeasurementValidityDuration-r18                                         OPTIONAL,    -- Need S

measReselectionValidityDuration-r18  MeasurementValidityDuration-r18                                         OPTIONAL     -- Need S

]]

}

MeasIdleConfigDedicated-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need N

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need N

measIdleDuration-r16            ENUMERATED{sec10, sec30, sec60, sec120, sec180, sec240, sec300, spare},

validityAreaList-r16            ValidityAreaList-r16                                                   OPTIONAL,     -- Need N

...,

[[

measReselectionCarrierListNR-r18     SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasReselectionCarrierNR-r18    OPTIONAL,    -- Need S

measIdleValidityDuration-r18         MeasurementValidityDuration-r18                                         OPTIONAL,    -- Need S

measReselectionValidityDuration-r18  MeasurementValidityDuration-r18                                         OPTIONAL     -- Need S

]]

}

ValidityAreaList-r16 ::= SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF ValidityArea-r16

ValidityArea-r16 ::=             SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

validityCellList-r16             ValidityCellList                                                     OPTIONAL   -- Need N

}

ValidityCellList ::= SEQUENCE (SIZE (1.. maxCellMeasIdle-r16)) OF PCI-Range

MeasIdleCarrierNR-r16 ::=        SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

ssbSubcarrierSpacing-r16         SubcarrierSpacing,

frequencyBandList                MultiFrequencyBandListNR                                             OPTIONAL,  -- Need R

measCellListNR-r16               CellListNR-r16                                                       OPTIONAL,  -- Need R

reportQuantities-r16             ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r16             SEQUENCE {

idleRSRP-Threshold-NR-r16        RSRP-Range                                                           OPTIONAL,  -- Need R

idleRSRQ-Threshold-NR-r16        RSRQ-Range                                                           OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Need R

ssb-MeasConfig-r16               SEQUENCE {

nrofSS-BlocksToAverage-r16          INTEGER (2..maxNrofSS-BlocksToAverage)                            OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation-r16 ThresholdNR                                                       OPTIONAL,   -- Need S

smtc-r16                            SSB-MTC                                                           OPTIONAL,   -- Need S

ssb-ToMeasure-r16                   SSB-ToMeasure                                                     OPTIONAL,   -- Need S

deriveSSB-IndexFromCell-r16         BOOLEAN,

ss-RSSI-Measurement-r16             SS-RSSI-Measurement                                               OPTIONAL    -- Need S

}                                                                                                     OPTIONAL,  -- Need S

beamMeasConfigIdle-r16           BeamMeasConfigIdle-NR-r16                                            OPTIONAL,  -- Need R

...

}

MeasIdleCarrierEUTRA-r16 ::=     SEQUENCE {

carrierFreqEUTRA-r16             ARFCN-ValueEUTRA,

allowedMeasBandwidth-r16         EUTRA-AllowedMeasBandwidth,

measCellListEUTRA-r16            CellListEUTRA-r16                                                    OPTIONAL,  -- Need R

reportQuantitiesEUTRA-r16        ENUMERATED {rsrp, rsrq, both},

qualityThresholdEUTRA-r16        SEQUENCE {

idleRSRP-Threshold-EUTRA-r16     RSRP-RangeEUTRA                                                      OPTIONAL,  -- Need R

idleRSRQ-Threshold-EUTRA-r16     RSRQ-RangeEUTRA-r16                                                  OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Need S

...

}

MeasReselectionCarrierNR-r18 ::= SEQUENCE {

carrierFreq-r18                  ARFCN-ValueNR,

...

}

CellListNR-r16  ::=       SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF PCI-Range

CellListEUTRA-r16  ::=    SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange

BeamMeasConfigIdle-NR-r16  ::=   SEQUENCE {

reportQuantityRS-Indexes-r16     ENUMERATED {rsrp, rsrq, both},

maxNrofRS-IndexesToReport-r16    INTEGER (1.. maxNrofIndexesToReport),

includeBeamMeasurements-r16      BOOLEAN

}

RSRQ-RangeEUTRA-r16 ::=   INTEGER (-30..46)

-- TAG-MEASIDLECONFIG-STOP

-- ASN1STOP

MeasIdleConfig field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

beamMeasConfigIdle Indicates the beam level measurement configuration.

carrierFreq Indicates the NR carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

carrierFreqEUTRA Indicates the E-UTRA carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR idle/inactive measurement parameters apply. The UE shall select the first listed band which it supports in the frequencyBandList field to represent the NR neighbour carrier frequency.

includeBeamMeasurements Indicates whether or not the UE shall include beam measurements in the NR idle/inactive measurement results.

maxNrofRS-IndexesToReport Max number of beam indices to include in the idle/inactive measurement result.

measCellListEUTRA Indicates the list of E-UTRA cells which the UE is requested to measure and report for idle/inactive measurements.

measCellListNR Indicates the list of NR cells which the UE is requested to measure and report for idle/inactive measurements.

measIdleCarrierListEUTRA Indicates the E-UTRA carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleCarrierListNR Indicates the NR carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleCarrierListNR-LessThan5MHz Indicates the NR carriers to be measured during RRC_IDLE or RRC_INACTIVE for the cell(s) supporting 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Total number of MeasIdleCarrierNR included in measIdleCarrierListNR and measIdleCarrierListNR-LessThan5MHz does not exceed maxFreqIdle-r16.

measIdleDuration Indicates the duration for performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. Value sec10 correspond to 10 seconds, value sec30 to 30 seconds and so on.

measIdleValidityDuration, measReselectionValidityDuration Indicates time values for UE to determine validity of reported idle/inactive and reselection measurements as defined in TS 38.133[14]. Value s5 correspond to 5 seconds, value s10 correspond to 10 seconds and so on.

measReselectionCarrierListNR Indicates the NR carriers for reselection measurement reporting.

measReselectionCarrierListNR-LessThan5MHz Indicates the NR carriers for reselection measurement reporting for the cell(s) supporting 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Total number of MeasReselectionCarrierNR included in measReselectionCarrierListNR and measReselectionCarrierListNR-LessThan5MHz does not exceed maxFreqIdle-r16.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

qualityThreshold Indicates the quality thresholds for reporting the measured cells for idle/inactive NR measurements.

qualityThresholdEUTRA Indicates the quality thresholds for reporting the measured cells for idle/inactive E-UTRA measurements.

reportQuantities Indicates which measurement quantities UE is requested to report in the idle/inactive measurement report.

reportQuantitiesEUTRA Indicates which E-UTRA measurement quantities the UE is requested to report in the idle/inactive measurement report.

reportQuantityRS-Indexes Indicates which measurement information per beam index the UE shall include in the NR idle/inactive measurement results.

smtc Indicates the measurement timing configuration for inter-frequency measurement. If this field is absent in VarMeasIdleConfig, the UE assumes that SSB periodicity is 5 ms in this frequency.

ssbSubcarrierSpacing Indicates subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent in VarMeasIdleConfig, the UE measures on all SS-blocks.

ss-RSSI-Measurement Indicates the SSB-based RSSI measurement configuration. If the field is absent in VarMeasIdleConfig, the UE behaviour is defined in TS 38.215 [89], clause 5.1.3.

validityAreaList Indicates the list of frequencies and optionally, for each frequency, a list of cells within which the UE is required to perform measurements while in RRC_IDLE and RRC_INACTIVE.

| Conditional Presence | Explanation |
| --- | --- |
| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB and the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need R. |

#### –	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEASIDTOADDMODLIST-START

MeasIdToAddModList ::=              SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=                  SEQUENCE {

measId                              MeasId,

measObjectId                        MeasObjectId,

reportConfigId                      ReportConfigId

}

-- TAG-MEASIDTOADDMODLIST-STOP

-- ASN1STOP

#### –	MeasObjectCLI

The IE MeasObjectCLI specifies information applicable for SRS-RSRP measurements and/or CLI-RSSI measurements.

MeasObjectCLI information element

-- ASN1START

-- TAG-MEASOBJECTCLI-START

MeasObjectCLI-r16 ::=                  SEQUENCE {

cli-ResourceConfig-r16               CLI-ResourceConfig-r16,

...

}

CLI-ResourceConfig-r16 ::=          SEQUENCE {

srs-ResourceConfig-r16              SetupRelease { SRS-ResourceListConfigCLI-r16 }                 OPTIONAL,   -- Need M

rssi-ResourceConfig-r16             SetupRelease { RSSI-ResourceListConfigCLI-r16 }                OPTIONAL    -- Need M

}

SRS-ResourceListConfigCLI-r16 ::=   SEQUENCE (SIZE (1.. maxNrofCLI-SRS-Resources-r16)) OF SRS-ResourceConfigCLI-r16

RSSI-ResourceListConfigCLI-r16 ::=  SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-Resources-r16)) OF RSSI-ResourceConfigCLI-r16

SRS-ResourceConfigCLI-r16 ::=       SEQUENCE {

srs-Resource-r16                    SRS-Resource,

srs-SCS-r16                         SubcarrierSpacing,

refServCellIndex-r16                ServCellIndex                                                  OPTIONAL,   -- Need S

refBWP-r16                          BWP-Id,

...

}

RSSI-ResourceConfigCLI-r16 ::=      SEQUENCE {

rssi-ResourceId-r16                 RSSI-ResourceId-r16,

rssi-SCS-r16                        SubcarrierSpacing,

startPRB-r16                        INTEGER (0..2169),

nrofPRBs-r16                        INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startPosition-r16                   INTEGER (0..13),

nrofSymbols-r16                     INTEGER (1..14),

rssi-PeriodicityAndOffset-r16       RSSI-PeriodicityAndOffset-r16,

refServCellIndex-r16                ServCellIndex                                                  OPTIONAL,   -- Need S

...

}

RSSI-ResourceId-r16 ::=             INTEGER (0.. maxNrofCLI-RSSI-Resources-1-r16)

RSSI-PeriodicityAndOffset-r16 ::=   CHOICE {

sl10                                INTEGER(0..9),

sl20                                INTEGER(0..19),

sl40                                INTEGER(0..39),

sl80                                INTEGER(0..79),

sl160                               INTEGER(0..159),

sl320                               INTEGER(0..319),

s1640                               INTEGER(0..639),

...

}

-- TAG-MEASOBJECTCLI-STOP

-- ASN1STOP

CLI-ResourceConfig field descriptions

srs-ResourceConfig SRS resources to be used for CLI measurements.

rssi-ResourceConfig CLI-RSSI resources to be used for CLI measurements.

MeasObjectCLI field descriptions

cli-ResourceConfig SRS and/or CLI-RSSI resource configuration for CLI measurement.

SRS-ResourceConfigCLI field descriptions

refBWP DL BWP id that is used to derive the reference point of the SRS resource (see TS 38.211[16], clause 6.4.1.4.3)

refServCellIndex The index of the reference serving cell that the refBWP belongs to. If this field is absent, the reference serving cell is PCell.

srs-SCS Subcarrier spacing for SRS. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

RSSI-ResourceConfigCLI field descriptions

nrofPRBs Allowed size of the measurement BW. Only multiples of 4 are allowed. The smallest configurable number is the minimum of 4 and the width of the active DL BWP. If the configured value is larger than the width of the active DL BWP, the UE shall assume that the actual CLI-RSSI resource bandwidth is within the active DL BWP.

nrofSymbols Within a slot that is configured for CLI-RSSI measurement (see slotConfiguration), the UE measures the RSSI from startPosition to startPosition + nrofSymbols - 1. The configured CLI-RSSI resource does not exceed the slot boundary of the reference SCS. If the SCS of configured DL BWP(s) is larger than the reference SCS, network configures startPosition and nrofSymbols such that the configured CLI-RSSI resource not to exceed the slot boundary corresponding to the configured BWP SCS. If the reference SCS is larger than SCS of configured DL BWP(s), network ensures startPosition and nrofSymbols are integer multiple of reference SCS divided by configured BWP SCS.

refServCellIndex The index of the reference serving cell. Frequency reference point of the RSSI resource is subcarrier 0 of CRB0 of the reference serving cell. If this field is absent, the reference serving cell is PCell.

rssi-PeriodicityAndOffset Periodicity and slot offset for this CLI-RSSI resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots.

rssi-SCS Reference subcarrier spacing for CLI-RSSI measurement. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

startPosition OFDM symbol location of the CLI-RSSI resource within a slot.

startPRB Starting PRB index of the measurement bandwidth. For the case where the reference subcarrier spacing is smaller than subcarrier spacing of active DL BWP(s), network configures startPRB and nrofPRBs are as a multiple of active BW SCS divided by reference SCS.

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for E-UTRA cells.

MeasObjectEUTRA information element

-- ASN1START

-- TAG-MEASOBJECTEUTRA-START

MeasObjectEUTRA::=                          SEQUENCE {

carrierFreq                                 ARFCN-ValueEUTRA,

allowedMeasBandwidth                        EUTRA-AllowedMeasBandwidth,

cellsToRemoveListEUTRAN                     EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

cellsToAddModListEUTRAN                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-Cell         OPTIONAL,    -- Need N

excludedCellsToRemoveListEUTRAN             EUTRA-CellIndexList                                         OPTIONAL,    -- Need N

excludedCellsToAddModListEUTRAN             SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-ExcludedCell OPTIONAL,    -- Need N

eutra-PresenceAntennaPort1                  EUTRA-PresenceAntennaPort1,

eutra-Q-OffsetRange                         EUTRA-Q-OffsetRange                                         OPTIONAL,    -- Need R

widebandRSRQ-Meas                           BOOLEAN,

...,

[[

associatedMeasGap-r17                       MeasGapId-r17                                               OPTIONAL     -- Need R

]],

[[

measSequence-r18                            MeasSequence-r18                                            OPTIONAL     -- Need R

]]

}

EUTRA-CellIndexList ::=                     SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF EUTRA-CellIndex

EUTRA-CellIndex ::=                         INTEGER (1..maxCellMeasEUTRA)

EUTRA-Cell ::=                              SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellId                                  EUTRA-PhysCellId,

cellIndividualOffset                        EUTRA-Q-OffsetRange

}

EUTRA-ExcludedCell ::=                      SEQUENCE {

cellIndexEUTRA                              EUTRA-CellIndex,

physCellIdRange                             EUTRA-PhysCellIdRange

}

-- TAG-MEASOBJECTEUTRA-STOP

-- ASN1STOP

EUTRAN-ExcludedCell field descriptions

cellIndexEUTRA Entry index in the cell list.

physicalCellIdRange Physical cell identity or a range of physical cell identities.

EUTRAN-Cell field descriptions

physicalCellId Physical cell identity of a cell in the cell list.

cellIndividualOffset Cell individual offset applicable to a specific cell. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

MeasObjectEUTRA field descriptions

allowedMeasBandwidth The maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [33].

associatedMeasGap Indicates the associated measurement gap for measuring this EUTRA frequency. If this field is absent, the associated meaurment gap is the gap configured via gapFR1 or gapUE.

carrierFreq Identifies E-UTRA carrier frequency for which this configuration is valid. Network does not configure more than one MeasObjectEUTRA for the same physical frequency, regardless of the E-ARFCN used to indicate this.

cellsToAddModListEUTRAN List of cells to add/ modify in the cell list.

cellsToRemoveListEUTRAN List of cells to remove from the cell list.

eutra-PresenceAntennaPort1 When set to true, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

eutra-Q-OffsetRange Used to indicate a cell, or frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

excludedCellsToAddModListEUTRAN List of cells to add/ modify in the exclude-list of cells.

excludedCellsToRemoveListEUTRAN List of cells to remove from the exclude-list of cells.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not configured, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

widebandRSRQ-Meas If set to true, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [40]. The network may set the field to true if the measurement bandwidth indicated by allowedMeasBandwidth is 50 resource blocks or larger; otherwise the network sets this field to false.

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEASOBJECTID-START

MeasObjectId ::=                    INTEGER (1..maxNrofObjectId)

-- TAG-MEASOBJECTID-STOP

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                       SSB-MTC3List-r16                                                OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL,   -- Need R

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

measCyclePSCell-r17                 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1}

OPTIONAL,   -- Cond SCG

cellsToAddModListExt-v1710          CellsToAddModListExt-v1710                                      OPTIONAL    -- Need N

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]],

[[

smtc5list-r19                       SSB-MTC5List-r19                                                OPTIONAL,   -- Need R

smtc6list-r19                       SSB-MTC6List-r19                                                OPTIONAL,   -- Need R

smtc7-SSBAdapt-r19                  SSB-MTC-SSBAdapt-r19                                      OPTIONAL,   -- Cond IntraFreqConnected

neighSCellMeasSkipping-r19          ENUMERATED { enabled }                                          OPTIONAL    -- Cond SCellMO

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC5List-r19::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC5-r19

SSB-MTC6List-r19::=                 SEQUENCE (SIZE(1..6)) OF SSB-MTC

SSB-MTC-SSBAdapt-r19 ::=            SEQUENCE (SIZE(1..2)) OF SSB-MTC

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCellInter-r17    ServCellIndex                                                   OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,   -- Cond SharedSpectrum2

ssb-PositionQCL-Cells-r17           SetupRelease {SSB-PositionQCL-CellList-r17}                     OPTIONAL    -- Need M

]],

[[

cca-CellsToAddModList-r17           PCI-List                                                        OPTIONAL,   -- Need N

cca-CellsToRemoveList-r17           PCI-List                                                        OPTIONAL    -- Need N

]],

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }             OPTIONAL    -- Need M

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddModListExt-v1710 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1710

CellsToAddModListExt-v1800 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1800

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need S

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Cond NeighbourCell

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...,

[[

rmtc-Bandwidth-r17                  ENUMERATED {mhz100, mhz400, mhz800, mhz1600, mhz2000}           OPTIONAL,   -- Need R

measDurationSymbols-v1700           ENUMERATED {sym140, sym560, sym1120}                            OPTIONAL,   -- Need R

ref-SCS-CP-v1700                    ENUMERATED {kHz120, kHz480, kHz960}                             OPTIONAL,   -- Need R

tci-StateInfo-r17               SEQUENCE {

tci-StateId-r17                  TCI-StateId,

ref-ServCellId-r17               ServCellIndex                                                  OPTIONAL   -- Need R

} OPTIONAL -- Need R

]],

[[

ref-BWPId-r17                   BWP-Id                                                              OPTIONAL   -- Need R

]]

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

SSB-PositionQCL-CellList-r17 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-Cell-r17

SSB-PositionQCL-Cell-r17         ::= SEQUENCE {

physCellId-r17                        PhysCellId,

ssb-PositionQCL-r17                   SSB-PositionQCL-Relation-r17

}

SSB-ToMeasureAltitudeBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofAltitudeRanges-r18)) OF SSB-ToMeasureAltitudeBased-r18

SSB-ToMeasureAltitudeBased-r18 ::=     SEQUENCE {

altitudeRange-r18                      SEQUENCE {

altitudeMin-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeMax-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeHyst-r18                       HysteresisAltitude-r18                                   OPTIONAL   -- Need R

},

ssb-ToMeasure-r18                      SSB-ToMeasure                                                OPTIONAL   -- Need S

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17,

referenceLocation-r18                  ReferenceLocation-r17                                        OPTIONAL   -- Need R

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

ntn-NeighbourCellInfo Includes satellite assistance information of an NTN neighbour cell.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL In this version of the specification, the network does not include this field.

physCellId Physical cell identity of a cell in the cell list.

referenceLocation Reference location of a neighbor NTN Earth-moving cell for the evaluation of the trigger criteria of an associated ReportConfig which contains EventD2 or condEventD2.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

allowedCellsToAddModList List of cells to add/modify in the allow-list of cells. It applies only to SSB resources.

allowedCellsToRemoveList List of cells to remove from the allow-list of cells.

associatedMeasGapSSB Indicates the associated measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapSSB2 Indicates the associated additional measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object for NTN deployments. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapSSB.

associatedMeasGapCSIRS Indicates the associated measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapCSIRS2 Indicates the associated additional measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object for NTN deployments. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapCSIRS. In this release of the specification, this field is not configured for NTN deployments.

cellsToAddModList List of cells to add/modify in the cell list. If the network includes cellsToAddModListExt-v1710 and/or cellsToAddModListExt-v1800, it contains the same number of entries listed in the same order as in cellsToAddModList (i.e without suffix).

cellsToRemoveList List of cells to remove from the cell list.

excludedCellsToAddModList List of cells to add/modify in the exclude-list of cells. It applies only to SSB resources.

excludedCellsToRemoveList List of cells to remove from the exclude-list of cells.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCyclePSCell The parameter is used only when the PSCell is configured on the frequency indicated by the measObjectNR and the SCG is deactivated, see TS 38.133 [14]. The field may also be configured when the PSCell is not configured on that frequency. The network always configures measCyclePSCell for the measObjectNR associated with the PSCell if bfd-and-RLM is set to true and the SCG is deactivated. Value ms160 corresponds to 160 ms, value ms256 corresponds to 256 ms and so on.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on. This field is ignored for on-demand SSB measurements during fast measurement window.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not provided, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

neighSCellMeasSkipping If configured, the UE is not required to perform neighbour cell measurements on the frequency of this MeasObjectNR. This field is only applicable to UE supporting antennaArrayType-r18 and CA.

nrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured). This field is not configured together with smtc4list.

smtc3list Measurement timing configuration list for SS corresponding to IAB-MT. This is used for the IAB-node's discovery of other IAB-nodes and the IAB-Donor-DUs.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. smtc5list may include an additional SMTC periodicity to the one indicated in smtc1.

smtc6list Measurement timing configuration list for OD-SSB, see clause 5.5.2.10.

smtc7-SSBAdapt Measurement timing configuration for adapted SSB transmission, see clause 5.5.2.10.

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for all measured cells as specified in TS 38.213 [13], clause 4.1.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

t312 The value of timer T312. Value ms0 represents 0 ms, ms50 represents 50 ms and so on.

ReferenceSignalConfig field descriptions

csi-rs-ResourceConfigMobility CSI-RS resources to be used for CSI-RS based RRM measurements.

ssb-ConfigMobility SSB configuration for mobility (nominal SSBs, timing configuration).

RMTC-Config field descriptions

measDurationSymbols Number of consecutive symbols for which the Physical Layer reports samples of RSSI (see TS 38.215 [9], clause 5.1.21). Value sym1 corresponds to one symbol, sym14or12 corresponds to 14 symbols of the reference numerology for NCP and 12 symbols for ECP, and so on. If measDurationSymbols-v1700 is signalled, the UE ignores measDurationSymbols-r16.

ref-BWPId Indicates the reference BWP for the TCI state indicated in tci-StateInfo. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

ref-SCS-CP Indicates a reference subcarrier spacing and cyclic prefix to be used for RSSI measurements (see TS 38.215 [9]). Value kHz15 corresponds to 15kHz, kHz30 corresponds to 30 kHz, value kHz60-NCP corresponds to 60 kHz using normal cyclic prefix (NCP), and kHz60-ECP corresponds to 60 kHz using extended cyclic prefix (ECP). If ref-SCS-CP-v1700 is signalled, the UE ignores ref-SCS-CP-r16.

ref-ServCellId Indicates the FR2-2 reference serving cell index for the TCI state. Network includes this field if tci-StateInfo is present. This field is only applicable for operation with shared spectrum channel access in FR2-2 and network does not configure this if the UE does not have any serving cells in FR2-2.

rmtc-Bandwidth Indicates the bandwidth for the RSSI measurement (see TS 38. 215 [9], clause 5.1.21).

rmtc-Frequency Indicates the center frequency of the measured bandwidth for a frequency which operates with shared spectrum channel access (see TS 38. 215 [9], clause 5.1.21).

rmtc-Periodicity Indicates the RSSI measurement timing configuration (RMTC) periodicity (see TS 38.215 [9], clause 5.1.21).

rmtc-SubframeOffset Indicates the RSSI measurement timing configuration (RMTC) subframe offset for this frequency (see TS 38.215 [9], clause 5.1.21). For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDurationSymbols which shall be selected to be between 0 and the configured rmtc-Periodicity with equal probability.

tci-StateId Indicates the TCI state to be used for RSSI measurements. This field is only applicable for shared spectrum channel access in FR2-2. Network does not configure this if the UE does not have any serving cells in FR2-2 and in such a case, it is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

SSB-ConfigMobility field descriptions

cca-CellsToAddModList, cca-CellsToRemoveList Lists of cells to be added or removed from the list of neighbor cells that apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2.

deriveSSB-IndexFromCell If this field is set to true, UE assumes SFN and frame boundary alignment across cells on the same frequency carrier as specified in TS 38.133 [14]. Hence, if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, this field indicates whether the UE can utilize the timing of this serving cell to derive the index of SS block transmitted by neighbour cell. Otherwise, this field indicates whether the UE may use the timing of any detected cell on that target frequency to derive the SSB index of all neighbour cells on that frequency.

deriveSSB-IndexFromCellInter If this field is present, UE assumes SFN and frame boundary alignment between the reference serving cell indicated by ServCellIndex and all neighbour cells in this MeasObjectNR as specified in TS 38.133 [14]. This field also indicates that the UE can utilize the timing of the reference serving cell indicated by ServCellIndex to derive the index of SS block transmitted by all inter-frequency neighbour cells on the frequency indicated by the MeasObjectNR. When this field is included, the network should set deriveSSB-IndexFromCell to true.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. When the UE is within an altitude range indicated by altitudeRange, it ignores the ssb-ToMeasure (without suffix), and applies the corresponding ssb-ToMeasure-r18 if present, otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

SSB-PositionQCL-CellsToAddMod field descriptions

physCellId Physical cell identity of a cell in the cell list.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the value signalled by ssb-PositionQCL-Common.

| Conditional Presence | Explanation |
| --- | --- |
| AssociatedGapCSIRS | This field is optionally present, Need R if associatedMeasGapCSIRS is configured, otherwise, it is absent. |
| AssociatedGapSSB | This field is optionally present, Need R if associatedMeasGapSSB is configured, otherwise, it is absent. |
| CSI-RS | This field is mandatory present if csi-rs-ResourceConfigMobility is configured, otherwise, it is absent. |
| IntraFreqConnected | This field is optionally present, Need R if the UE is configured with a serving cell for which (absoluteFrequencySSB, subcarrierSpacing) in ServingCellConfigCommon is equal to (ssbFrequency, ssbSubcarrierSpacing) in this MeasObjectNR, otherwise, it is absent. |
| SCellMO | This field is optionallly present, Need R, in the MeasObjectNR associated with an SCell. Otherwise the field is absent. |
| SCG | This field is optionallly present, Need R, in the measConfig associated with the SCG. It is absent in the measConfig associated with the MCG. |
| NeighbourCell | This field is mandatory present if this MeasObject is configured by the serving cell for a neighbour cell served by a NTN Earth-moving cell and is associated with a ReportConfig which contains EventD2 or condEventD2. Otherwise, it is optional, Need R. |
| SharedSpectrum | This field is mandatory present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this MeasObject is for a frequency which operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| SSBorAssociatedSSB | This field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. Otherwise, it is absent, Need R. |

#### –	MeasObjectNR-SL

The IE MeasObjectNR-SL concerns a measurement object including a list of transmission resource pool(s) for which CBR measurement is performed for NR sidelink communication/discovery.

MeasObjectNR-SL information element

-- ASN1START

-- TAG-MEASOBJECTNR-SL-START

MeasObjectNR-SL-r16 ::=      SEQUENCE {

tx-PoolMeasToRemoveList-r16  Tx-PoolMeasList-r16                           OPTIONAL,       -- Need N

tx-PoolMeasToAddModList-r16  Tx-PoolMeasList-r16                           OPTIONAL        -- Need N

}

MeasObjectNR-SL-r18 ::=      SEQUENCE {

sl-Frequency-r18             INTEGER (1..maxNrofFreqSL-r16),

tx-PoolMeasToRemoveList-r18  Tx-PoolMeasList-r16                           OPTIONAL,       -- Need N

tx-PoolMeasToAddModList-r18  Tx-PoolMeasList-r16                           OPTIONAL        -- Need N

}

Tx-PoolMeasList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-PoolToMeasureNR-r16)) OF SL-ResourcePoolID-r16

-- TAG-MEASOBJECTNR-SL-STOP

-- ASN1STOP

MeasObjectNR-SL field descriptions

sl-Frequency Indicates the sidelink frequency associated to this MeasObjectNR-SL. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of first entry in sl-FreqInfoListSizeExt broadcast in SIB12, the value 3 corresponds to the frequency of second entry in sl-FreqInfoListSizeExt broadcast in SIB12 and so on.

#### –	MeasObjectRxTxDiff

The IE MeasObjectRxTxDiff is used to configure the measurement object for UE Rx-Tx time difference measurement.

MeasObjectRxTxDiff information element

-- ASN1START

-- TAG-MEASOBJECTRXTXDIFF-START

MeasObjectRxTxDiff-r17 ::=      SEQUENCE {

dl-Ref-r17      CHOICE {

prs-Ref-r17             NULL,

csi-RS-Ref-r17          NULL,

...

}                               OPTIONAL,   -- Need R

...

}

-- TAG-MEASOBJECTRXTXDIFF-STOP

-- ASN1STOP

MeasObjectRxTxDiff field descriptions

dl-Ref configures the DL references signals to measure Rx-Tx time difference. prs-Ref-r17 indicates PRS is chosen, and csi-RS-Ref-r17 indicates that CSI-RS for tracking is chosen. Only one PRS resource set is configured by the network. Only one NZP-CSI-RS-ResourceSet can be configured with pdc-Info-r17 set to true and it is used for UE Rx-Tx time difference measurement. Only reference signals from the PCell of the MCG can be configured by the network.

#### –	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEASOBJECTTOADDMODLIST-START

MeasObjectToAddModList ::=                  SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=                      SEQUENCE {

measObjectId                                MeasObjectId,

measObject                                  CHOICE {

measObjectNR                                MeasObjectNR,

...,

measObjectEUTRA                             MeasObjectEUTRA,

measObjectUTRA-FDD-r16                      MeasObjectUTRA-FDD-r16,

measObjectNR-SL-r16                         MeasObjectNR-SL-r16,

measObjectCLI-r16                           MeasObjectCLI-r16,

measObjectRxTxDiff-r17                      MeasObjectRxTxDiff-r17,

measObjectRelay-r17                         SL-MeasObject-r16,

measObjectNR-SL-r18                         MeasObjectNR-SL-r18

}

}

-- TAG-MEASOBJECTTOADDMODLIST-STOP

-- ASN1STOP

#### –	MeasObjectUTRA-FDD

The IE MeasObjectUTRA-FDD specifies information applicable for inter-RAT UTRA-FDD neighbouring cells.

MeasObjectUTRA-FDD information element

-- ASN1START

-- TAG-MEASOBJECTUTRA-FDD-START

MeasObjectUTRA-FDD-r16 ::=                  SEQUENCE {

carrierFreq-r16                             ARFCN-ValueUTRA-FDD-r16,

utra-FDD-Q-OffsetRange-r16                  UTRA-FDD-Q-OffsetRange-r16              OPTIONAL,         -- Need R

cellsToRemoveList-r16                       UTRA-FDD-CellIndexList-r16              OPTIONAL,         -- Need N

cellsToAddModList-r16                       CellsToAddModListUTRA-FDD-r16           OPTIONAL,         -- Need N

...

}

CellsToAddModListUTRA-FDD-r16 ::=           SEQUENCE (SIZE (1..maxCellMeasUTRA-FDD-r16)) OF CellsToAddModUTRA-FDD-r16

CellsToAddModUTRA-FDD-r16 ::=               SEQUENCE {

cellIndexUTRA-FDD-r16                       UTRA-FDD-CellIndex-r16,

physCellId-r16                              PhysCellIdUTRA-FDD-r16

}

UTRA-FDD-CellIndexList-r16 ::=              SEQUENCE (SIZE (1..maxCellMeasUTRA-FDD-r16)) OF UTRA-FDD-CellIndex-r16

UTRA-FDD-CellIndex-r16 ::=                  INTEGER (1..maxCellMeasUTRA-FDD-r16)

-- TAG-MEASOBJECTUTRA-FDD-STOP

-- ASN1STOP

MeasObjectUTRA-FDD field descriptions

carrierFreq Identifies UTRA-FDD carrier frequency for which this configuration is valid. NR does not configure more than one measurement object for the same physical frequency regardless of the ARFCN used to indicate this.

cellIndexUTRA-FDD Entry index in the neighbouring cell list.

cellsToAddModList List of UTRA-FDD cells to add/modify in the neighbouring cell list.

cellsToRemoveList List of cells to remove from the neighbouring cell list.

utra-FDD-Q-OffsetRange Used to indicate a frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB.

#### –	MeasResultCellListSFTD-NR

The IE MeasResultCellListSFTD-NR consists of SFN and radio frame boundary difference between the PCell and an NR cell as specified in TS 38.215 [9] and TS 38.133 [14].

MeasResultCellListSFTD-NR information element

-- ASN1START

-- TAG-MEASRESULTCELLLISTSFTD-NR-START

MeasResultCellListSFTD-NR ::=          SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-NR

MeasResultCellSFTD-NR ::=              SEQUENCE {

physCellId                            PhysCellId,

sfn-OffsetResult                      INTEGER (0..1023),

frameBoundaryOffsetResult             INTEGER (-30720..30719),

rsrp-Result                           RSRP-Range                      OPTIONAL

}

-- TAG-MEASRESULTCELLLISTSFTD-NR-STOP

-- ASN1STOP

MeasResultCellSFTD-NR field descriptions

sfn-OffsetResult Indicates the SFN difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the NR cell as an integer value according to TS 38.215 [9].

#### –	MeasResultCellListSFTD-EUTRA

The IE MeasResultCellListSFTD-EUTRA consists of SFN and radio frame boundary difference between the PCell and an E-UTRA PSCell.

MeasResultCellListSFTD-EUTRA information element

-- ASN1START

-- TAG-MEASRESULTCELLLISTSFTD-EUTRA-START

MeasResultCellListSFTD-EUTRA ::=          SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultSFTD-EUTRA

MeasResultSFTD-EUTRA ::=           SEQUENCE {

eutra-PhysCellId                    EUTRA-PhysCellId,

sfn-OffsetResult                    INTEGER (0..1023),

frameBoundaryOffsetResult           INTEGER (-30720..30719),

rsrp-Result                         RSRP-Range                      OPTIONAL

}

-- TAG-MEASRESULTCELLLISTSFTD-EUTRA-STOP

-- ASN1STOP

MeasResultSFTD-EUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed.

sfn-OffsetResult Indicates the SFN difference between the PCell and the E-UTRA cell as an integer value according to TS 38.215 [9].

frameBoundaryOffsetResult Indicates the frame boundary difference between the PCell and the E-UTRA cell as an integer value according to TS 38.215 [9].

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, inter-RAT mobility and measured results for NR sidelink communication/discovery/positioning.

MeasResults information element

-- ASN1START

-- TAG-MEASRESULTS-START

MeasResults ::=                         SEQUENCE {

measId                                  MeasId,

measResultServingMOList                 MeasResultServMOList,

measResultNeighCells                    CHOICE {

measResultListNR                        MeasResultListNR,

...,

measResultListEUTRA                     MeasResultListEUTRA,

measResultListUTRA-FDD-r16              MeasResultListUTRA-FDD-r16,

sl-MeasResultsCandRelay-r17             OCTET STRING        -- Contains PC5 SL-MeasResultListRelay-r17

}                                                                                                                   OPTIONAL,

...,

[[

measResultServFreqListEUTRA-SCG         MeasResultServFreqListEUTRA-SCG                                             OPTIONAL,

measResultServFreqListNR-SCG            MeasResultServFreqListNR-SCG                                                OPTIONAL,

measResultSFTD-EUTRA                    MeasResultSFTD-EUTRA                                                        OPTIONAL,

measResultSFTD-NR                       MeasResultCellSFTD-NR                                                       OPTIONAL

]],

[[

measResultCellListSFTD-NR               MeasResultCellListSFTD-NR                                                   OPTIONAL

]],

[[

measResultForRSSI-r16                   MeasResultForRSSI-r16                                                       OPTIONAL,

locationInfo-r16                        LocationInfo-r16                                                            OPTIONAL,

ul-PDCP-DelayValueResultList-r16        UL-PDCP-DelayValueResultList-r16                                            OPTIONAL,

measResultsSL-r16                       MeasResultsSL-r16                                                           OPTIONAL,

measResultCLI-r16                       MeasResultCLI-r16                                                           OPTIONAL

]],

[[

measResultRxTxTimeDiff-r17              MeasResultRxTxTimeDiff-r17                                                  OPTIONAL,

sl-MeasResultServingRelay-r17           OCTET STRING                                                                OPTIONAL,

-- Contains PC5 SL-MeasResultRelay-r17

ul-PDCP-ExcessDelayResultList-r17       UL-PDCP-ExcessDelayResultList-r17                                           OPTIONAL,

coarseLocationInfo-r17                  OCTET STRING                                                                OPTIONAL

]],

[[

altitudeUE-r18                          Altitude-r18                                                                OPTIONAL,

cellsMetReportOnLeaveList-r18           SEQUENCE (SIZE (1..maxCellReport)) OF PhysCellId                            OPTIONAL

]]

}

MeasResultServMOList ::=                SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=                    SEQUENCE {

servCellId                              ServCellIndex,

measResultServingCell                   MeasResultNR,

measResultBestNeighCell                 MeasResultNR                                                                OPTIONAL,

...

}

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]],

[[

choCandidate-r17                        ENUMERATED {true}                                                           OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                             OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                                    OPTIONAL,

firstTriggeredEvent-r17                 ENUMERATED {condFirstEvent, condSecondEvent}                            OPTIONAL

}                                                                                                               OPTIONAL

]],

[[

entering-r18                            ENUMERATED {true}                                                           OPTIONAL

]],

[[

distanceFromReference2-r19              INTEGER (0.. 65535)                                                         OPTIONAL

]]

}

MeasResultListEUTRA ::=                 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,

...,

[[

hsdn-Cell-r19                       ENUMERATED {true}                                                               OPTIONAL

]]

}

MultiBandInfoListEUTRA ::=              SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicatorEUTRA

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

MeasQuantityResultsEUTRA ::=            SEQUENCE {

rsrp                                    RSRP-RangeEUTRA                                                             OPTIONAL,

rsrq                                    RSRQ-RangeEUTRA                                                             OPTIONAL,

sinr                                    SINR-RangeEUTRA                                                             OPTIONAL

}

ResultsPerSSB-IndexList::=              SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::=                 SEQUENCE {

ssb-Index                               SSB-Index,

ssb-Results                             MeasQuantityResults                                                         OPTIONAL

}

ResultsPerCSI-RS-IndexList::=           SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::=              SEQUENCE {

csi-RS-Index                            CSI-RS-Index,

csi-RS-Results                          MeasQuantityResults                                                         OPTIONAL

}

MeasResultServFreqListEUTRA-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA

MeasResultServFreqListNR-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResult2NR

MeasResultListUTRA-FDD-r16 ::=          SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA-FDD-r16

MeasResultUTRA-FDD-r16 ::=              SEQUENCE {

physCellId-r16                          PhysCellIdUTRA-FDD-r16,

measResult-r16                          SEQUENCE {

utra-FDD-RSCP-r16                       INTEGER (-5..91)          OPTIONAL,

utra-FDD-EcN0-r16                       INTEGER (0..49)           OPTIONAL

}

}

MeasResultForRSSI-r16 ::=        SEQUENCE {

rssi-Result-r16                  RSSI-Range-r16,

channelOccupancy-r16             INTEGER (0..100)

}

MeasResultCLI-r16 ::=            SEQUENCE {

measResultListSRS-RSRP-r16       MeasResultListSRS-RSRP-r16                                                         OPTIONAL,

measResultListCLI-RSSI-r16       MeasResultListCLI-RSSI-r16                                                         OPTIONAL

}

MeasResultListSRS-RSRP-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultSRS-RSRP-r16

MeasResultSRS-RSRP-r16 ::=       SEQUENCE {

srs-ResourceId-r16               SRS-ResourceId,

srs-RSRP-Result-r16              SRS-RSRP-Range-r16

}

MeasResultListCLI-RSSI-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultCLI-RSSI-r16

MeasResultCLI-RSSI-r16 ::=       SEQUENCE {

rssi-ResourceId-r16              RSSI-ResourceId-r16,

cli-RSSI-Result-r16              CLI-RSSI-Range-r16

}

UL-PDCP-DelayValueResultList-r16 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16

UL-PDCP-DelayValueResult-r16 ::= SEQUENCE {

drb-Id-r16                       DRB-Identity,

averageDelay-r16                 INTEGER (0..10000),

...

}

UL-PDCP-ExcessDelayResultList-r17 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-ExcessDelayResult-r17

UL-PDCP-ExcessDelayResult-r17 ::= SEQUENCE {

drb-Id-r17                        DRB-Identity,

excessDelay-r17                   INTEGER (0..31),

...

}

TimeBetweenEvent-r17 ::= INTEGER (0..1023)

-- TAG-MEASRESULTS-STOP

-- ASN1STOP

MeasResultEUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed. The UE reports a value in the range 0..503, other values are reserved.

hsdn-Cell Contains hsdn-Cell field acquired by the UE that supports eutra-CGI-Reporting-HSDN from SIB1 of the cell for which report CGI procedure was requested by the network.

MeasResultNR field descriptions

averageDelay Indicates average delay for the packets during the reporting period, as specified in TS 38.314 [53]. Value 0 corresponds to 0 millisecond, value 1 corresponds to 0.1 millisecond, value 2 corresponds to 0.2 millisecond, and so on.

cellResults Cell level measurement results.

cellsMetReportOnLeaveList This field indicates the list of cells which met the event leaving condition.

choCandidate This field indicates whether the associated cell is a candidate target cell for conditional handover or conditional PSCell change or addition. This field may be included only in the SuccessHO-Report or SuccessPSCell-Report within UEInformationResponse message.

choConfig If the associated cell is a candidate target cell for conditional handover, this field indicates the conditional handover execution condition for each measId within condTriggerConfig associated to the cell. This field may be included only in the rlf-report within UEInformationResponse message.

distanceFromReference2 This field indicates the measured distance between UE and the moving reference locations of associated neighbour cell if the conditional handover is based on condEventD2. Each step represents 50m. The actual distance shall be rounded down to the nearest step value (i.e., FLOOR(actual distance[m] / 50)). The maximum value is 65535, which indicates a distance equal to or greater than 65535 multiplied by 50m.

drb-Id Indicates DRB value for which uplink PDCP delay ratio or value is provided, according to TS 38.314 [53].

entering This field indicates if the event entering condition for the cell is satisfied and the cell has been just added within cellsTriggeredList.

firstTriggeredEvent This field is set to condFirstEvent if the execution condition associated to the first entry of choConfig was fulfilled first in time. This field is set to condSecondEvent if the execution condition associated to the second entry of choConfig was fulfilled first in time. This field may be included in rlf-report within UEInformationResponse message or in SCGFailureInformation message.

locationInfo Positioning related information and measurements.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.

timeBetweenEvents Indicates the time elapsed between fulfilling the conditional execution conditions included in choConfig. Value in milliseconds. The maximum value 1023 means 1023ms or longer. This field may be included in the reports associated to UEInformationResponse message, e.g., rlf-Report or in the SCGFailureInformation message.

MeasResultUTRA-FDD field descriptions

physCellId The physical cell identity of the UTRA-FDD cell for which the reporting is being performed.

utra-FDD-EcN0 According to CPICH_Ec/No in TS 25.133 [46] for FDD.

utra-FDD-RSCP According to CPICH_RSCP in TS 25.133 [46] for FDD.

MeasResults field descriptions

coarseLocationInfo This field indicates the coarse location information reported by the UE. This field is coded as the Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement

excessDelay Indicates the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs, according to the UL PDCP Excess Packet Delay per DRB mapping table, as defined in TS 38.314 [53], Table 4.3.1.e-1.

measId Identifies the measurement identity for which the reporting is being performed.

measQuantityResults The value sinr is not included when it is used for LogMeasReport-r16.

measResultCellListSFTD-NR SFTD measurement results between the PCell and the NR neighbour cell(s) in NR standalone.

measResultCLI CLI measurement results.

measResultEUTRA Measured results of an E-UTRA cell.

measResultForRSSI Includes measured RSSI result in dBm (see TS 38.215 [9]) and channelOccupancy which is the percentage of samples when the RSSI was above the configured channelOccupancyThreshold for the associated reportConfig.

measResultListEUTRA List of measured results for the maximum number of reported best cells for an E-UTRA measurement identity.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultListUTRA-FDD List of measured results for the maximum number of reported best cells for a UTRA-FDD measurement identity.

measResultNR Measured results of an NR cell.

measResultServFreqListEUTRA-SCG Measured results of the E-UTRA SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each E-UTRA SCG serving frequency.

measResultServFreqListNR-SCG Measured results of the NR SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each NR SCG serving frequency.

measResultServingMOList Measured results of measured cells with reference signals indicated in the serving cell measurement objects including measurement results of SpCell, configured SCell(s) and best neighbouring cell within measured cells with reference signals indicated in on each serving cell measurement object. If the sending of the MeasurementReport message is triggered by a measurement configured by the field sl-ConfigDedicatedForNR received within an E-UTRA RRCConnectionReconfiguration message (i.e. CBR measurements), this field is not applicable and its contents is ignored by the network.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC.

measResultSFTD-NR SFTD measurement results between the PCell and the NR PScell in NR-DC.

measResultsSL CBR measurements results for NR sidelink communication/discovery/positioning.

measResultUTRA-FDD Measured result of a UTRA-FDD cell.

sl-MeasResultsCandRelay Measurement result(s) of candiate L2 U2N relay UE(s).

sl-MeasResultServingRelay Measurement result of serving L2 U2N relay UE.

#### –	MeasResult2EUTRA

The IE MeasResult2EUTRA contains measurements on E-UTRA frequencies.

MeasResult2EUTRA information element

-- ASN1START

-- TAG-MEASRESULT2EUTRA-START

MeasResult2EUTRA ::=       SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

measResultServingCell               MeasResultEUTRA                 OPTIONAL,

measResultBestNeighCell             MeasResultEUTRA                 OPTIONAL,

...

}

-- TAG-MEASRESULT2EUTRA-STOP

-- ASN1STOP

#### –	MeasResult2NR

The IE MeasResult2NR contains measurements on NR frequencies.

MeasResult2NR information element

-- ASN1START

-- TAG-MEASRESULT2NR-START

MeasResult2NR ::=                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                           OPTIONAL,

refFreqCSI-RS                       ARFCN-ValueNR                           OPTIONAL,

measResultServingCell               MeasResultNR                            OPTIONAL,

measResultNeighCellListNR           MeasResultListNR                        OPTIONAL,

...

}

-- TAG-MEASRESULT2NR-STOP

-- ASN1STOP

#### –	MeasResultIdleEUTRA

The IE MeasResultIdleEUTRA covers the E-UTRA measurement results performed in RRC_IDLE and RRC_INACTIVE.

MeasResultIdleEUTRA information element

-- ASN1START

-- TAG-MEASRESULTIDLEEUTRA-START

MeasResultIdleEUTRA-r16 ::= SEQUENCE {

measResultsPerCarrierListIdleEUTRA-r16   SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleEUTRA-r16,

...

}

MeasResultsPerCarrierIdleEUTRA-r16 ::=  SEQUENCE {

carrierFreqEUTRA-r16                    ARFCN-ValueEUTRA,

measResultsPerCellListIdleEUTRA-r16     SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleEUTRA-r16,

...

}

MeasResultsPerCellIdleEUTRA-r16 ::=     SEQUENCE {

eutra-PhysCellId-r16                    EUTRA-PhysCellId,

measIdleResultEUTRA-r16                 SEQUENCE {

rsrp-ResultEUTRA-r16                     RSRP-RangeEUTRA                                                     OPTIONAL,

rsrq-ResultEUTRA-r16                     RSRQ-RangeEUTRA-r16                                                 OPTIONAL

},

...

}

-- TAG-MEASRESULTIDLEEUTRA-STOP

-- ASN1STOP

MeasResultIdleEUTRA field descriptions

carrierFreqEUTRA Indicates the E-UTRA carrier frequency.

eutra-PhysCellId Indicates the physical cell identity of an E-UTRA cell.

measIdleResultEUTRA Idle/inactive measurement results for an E-UTRA cell.

measResultsPerCarrierListIdleEUTRA List of idle/inactive measured results for the maximum number of reported E-UTRA carriers.

measResultsPerCellListIdleEUTRA List of idle/inactive measured results for the maximum number of reported best cells for a given E-UTRA carrier.

#### –	MeasResultIdleNR

The IE MeasResultIdleNR covers the NR measurement results performed in RRC_IDLE and RRC_INACTIVE.

MeasResultIdleNR information element

-- ASN1START

-- TAG-MEASRESULTIDLENR-START

MeasResultIdleNR-r16 ::=  SEQUENCE {

measResultServingCell-r16 SEQUENCE {

rsrp-Result-r16           RSRP-Range                                                                        OPTIONAL,

rsrq-Result-r16           RSRQ-Range                                                                        OPTIONAL,

resultsSSB-Indexes-r16    ResultsPerSSB-IndexList-r16                                                       OPTIONAL

},

measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16    OPTIONAL,

...

}

MeasResultsPerCarrierIdleNR-r16 ::=   SEQUENCE {

carrierFreq-r16                       ARFCN-ValueNR,

measResultsPerCellListIdleNR-r16      SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16,

...

}

MeasResultsPerCellIdleNR-r16 ::=  SEQUENCE {

physCellId-r16                    PhysCellId,

measIdleResultNR-r16              SEQUENCE {

rsrp-Result-r16                   RSRP-Range                                                              OPTIONAL,

rsrq-Result-r16                   RSRQ-Range                                                              OPTIONAL,

resultsSSB-Indexes-r16            ResultsPerSSB-IndexList-r16                                             OPTIONAL

},

...,

[[

validityStatus-r18                MeasurementValidityDuration-r18                                             OPTIONAL

]]

}

ResultsPerSSB-IndexList-r16 ::=   SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16

ResultsPerSSB-IndexIdle-r16 ::=   SEQUENCE {

ssb-Index-r16                     SSB-Index,

ssb-Results-r16                   SEQUENCE {

ssb-RSRP-Result-r16               RSRP-Range                                                              OPTIONAL,

ssb-RSRQ-Result-r16               RSRQ-Range                                                              OPTIONAL

}                                                                                                     OPTIONAL

}

-- TAG-MEASRESULTIDLENR-STOP

-- ASN1STOP

MeasResultIdleNR field descriptions

carrierFreq Indicates the NR carrier frequency.

measIdleResultNR Idle/inactive measurement results for an NR cell (optionally including beam level measurements).

measResultServingCell Measured results of the serving cell (i.e., PCell) from idle/inactive measurements.

measResultsPerCellListIdleNR List of idle/inactive measured results for the maximum number of reported best cells for a given NR carrier.

resultsSSB-Indexes Beam level measurement results (indexes and optionally, beam measurements).

#### –	MeasResultRxTxTimeDiff

The IE MeasResultRxTxTimeDiff is used to provide Rx-Tx time difference measurement result.

MeasResultRxTxTimeDiff information element

-- ASN1START

-- TAG-MEASRESULTRXTXTIMEDIFF-START

MeasResultRxTxTimeDiff-r17 ::= SEQUENCE {

rxTxTimeDiff-ue-r17            RxTxTimeDiff-r17      OPTIONAL,

...

}

-- TAG-MEASRESULTRXTXTIMEDIFF-STOP

-- ASN1STOP

MeasResultRxTxTimeDiff field descriptions

rxTxTimeDiff-ue indicates the Rx-Tx Time difference measurement at the UE (see clause 5.1.30, TS 38.215 [9]).

#### –	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in (NG)EN-DC and NR-DC.

MeasResultSCG-Failure information element

-- ASN1START

-- TAG-MEASRESULTSCG-FAILURE-START

MeasResultSCG-Failure ::=           SEQUENCE {

measResultPerMOList                 MeasResultList2NR,

...,

[[

locationInfo-r16                    LocationInfo-r16            OPTIONAL

]]

}

MeasResultList2NR ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-MEASRESULTSCG-FAILURE-STOP

-- ASN1STOP

#### –	MeasResultsSL

The IE MeasResultsSL covers measured results for NR sidelink communication/discovery/positioning.

MeasResultsSL information element

-- ASN1START

-- TAG-MEASRESULTSSL-START

MeasResultsSL-r16 ::=         SEQUENCE {

measResultsListSL-r16         CHOICE {

measResultNR-SL-r16           MeasResultNR-SL-r16,

...,

[[

measResultNR-SL-r18           MeasResultNR-SL-r18

]]

},

...

}

MeasResultNR-SL-r16 ::=       SEQUENCE {

measResultListCBR-NR-r16      SEQUENCE (SIZE (1.. maxNrofSL-PoolToMeasureNR-r16)) OF MeasResultCBR-NR-r16,

...

}

MeasResultCBR-NR-r16 ::=      SEQUENCE {

sl-poolReportIdentity-r16     SL-ResourcePoolID-r16,

sl-CBR-ResultsNR-r16          SL-CBR-r16,

...

}

MeasResultNR-SL-r18 ::=       SEQUENCE {

measResultListCBR-DedicatedSL-PRS-r18 SEQUENCE (SIZE (1..maxNrofDedicatedSL-PRS-PoolToMeas-r18)) OF MeasResultCBR-DedicatedSL-PRS-r18,

...

}

MeasResultCBR-DedicatedSL-PRS-r18 ::= SEQUENCE {

sl-PRS-ResourcePoolID-r18             SL-PRS-ResourcePoolID-r18,

sL-CBR-ResultsDedicatedSL-PRS-RP-r18  SL-CBR-r16,

...

}

-- TAG-MEASRESULTSSL-STOP

-- ASN1STOP

MeasResultsSL field descriptions

measResultNR-SL Include the measured results for NR sidelink communication/discovery/positioning.

MeasResultNR-SL field descriptions

measResultListCBR-DedicatedSL-PRS CBR measurement results for NR sidelink positioning for dedicated SL PRS resource pool.

measResultListCBR-NR CBR measurement results for NR sidelink communication/discovery.

sl-poolReportIdentity The identity of the transmission resource pool which is corresponding to the sl-ResourcePoolID configured in a resource pool for NR sidelink communication/discovery.

sl-PRS-ResourcePoolID The ID of the transmission resource pool which is corresponding to the sl-PRS-ResourcePoolID configured in a resource pool for NR sidelink positioning.

#### –	MeasSequence

The IE MeasSequence is used to configure a recommended sequence for intra/inter-RAT intra/inter-frequency measurement.

MeasSequence information element

-- ASN1START

-- TAG-MEASSEQUENCE-START

MeasSequence-r18 ::=             INTEGER (1..maxMeasSequence-r18)

-- TAG-MEASSEQUENCE-STOP

-- ASN1STOP

#### –	MeasTriggerQuantity

Measurement quantity defined in the measurement report configurations (e.g., events defined in ReportConfig-NR, or LTM-CSI-ReportConfig) for triggering measurement reports (e.g., by RRC message MeasurementReport or by MAC CE in TS 38.321 [3]). When the IE is configured under LTM-CSI-ReportConfig, only the field rsrp is applicable.

MeasTriggerQuantity information element

-- ASN1START

-- TAG-MEASTRIGGERQUANTITY-START

MeasTriggerQuantity ::=                     CHOICE {

rsrp                                        RSRP-Range,

rsrq                                        RSRQ-Range,

sinr                                        SINR-Range

}

-- TAG-MEASTRIGGERQUANTITY-STOP

-- ASN1STOP

#### –	MeasTriggerQuantityEUTRA

The IE MeasTriggerQuantityEUTRA is used to configure the trigger quantity and reporting range for E-UTRA measurements. The RSRP, RSRQ and SINR ranges correspond to RSRP-Range, RSRQ-Range and RS-SINR-Range in TS 36.331 [10], respectively.

MeasTriggerQuantityEUTRA information element

-- ASN1START

-- TAG-MEASTRIGGERQUANTITYEUTRA-START

MeasTriggerQuantityEUTRA::=                 CHOICE {

rsrp                                        RSRP-RangeEUTRA,

rsrq                                        RSRQ-RangeEUTRA,

sinr                                        SINR-RangeEUTRA

}

RSRP-RangeEUTRA ::=                 INTEGER (0..97)

RSRQ-RangeEUTRA ::=                 INTEGER (0..34)

SINR-RangeEUTRA ::=                 INTEGER (0..127)

-- TAG-MEASTRIGGERQUANTITYEUTRA-STOP

-- ASN1STOP

#### –	MeasTriggerQuantityOffset

Offsets defined in measurement report configurations (e.g., events defined in ReportConfig-NR, or LTM-CSI-ReportConfig) for triggering measurement reports (e.g., by RRC message MeasurementReport or by MAC CE in TS 38.321 [3]). Values in the unit of 'dB'. When the IE is configured under LTM-CSI-ReportConfig, only the field rsrp is applicable.

MeasTriggerQuantityOffset information element

-- ASN1START

-- TAG-MEASTRIGGERQUANTITYOFFSET-START

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

-- TAG-MEASTRIGGERQUANTITYOFFSET-STOP

-- ASN1STOP

#### –	MeasurementValidityDuration

The IE MeasurementValidityDuration indicates time values for validation of reported idle/inactive and reselection measurements as defined in TS 38.133[14]. Value s5 correspond to 5 seconds, value s10 correspond to 10 seconds and so on.

MeasurementValidityDuration information element

-- ASN1START

-- TAG-MEASUREMENTVALIDITYDURATION-START

MeasurementValidityDuration-r18 ::= ENUMERATED {s5, s10, s20, s50, s100, spare3, spare2, spare1}

-- TAG-MEASUREMENTVALIDITYDURATION-STOP

-- ASN1STOP

#### –	MeasWindowConfig

The IE MeasWindowConfig specifies the effective measurement window configuration for inter-RAT E-UTRA measurement.

MeasWindowConfig information element

-- ASN1START

-- TAG-MEASWINDOWCONFIG-START

MeasWindowConfig-r18 ::=    SEQUENCE {

windowOffsetPeriodicity-r18 CHOICE {

periodicityMs40-r18     INTEGER (0..39),

periodicityMs80-r18     INTEGER (0..79),

...

},

windowDuration-r18              ENUMERATED {ms2, ms5, ms5dot5, spare1},

...

}

-- TAG-MEASWINDOWCONFIG-STOP

-- ASN1STOP

MeasWindowConfig field descriptions

windowDuration Value windowDuration is the measurement window length in ms of effective measurement window. Value ms2 corresponds to 2 ms, value ms5 corresponds to 5 ms, and Value ms5dot5 corresponds to 5.5 ms.

windowOffsetPeriodicity Indicates the periodicity and offset of effective measurement window. The choice determines the periodicity (periodicityMs40 for 40ms and periodicityMs80 for 80 ms). The field value indicates the offset in ms.

#### –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

-- TAG-MOBILITYSTATEPARAMETERS-START

MobilityStateParameters ::=         SEQUENCE{

t-Evaluation                        ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal                        ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium                  INTEGER (1..16),

n-CellChangeHigh                    INTEGER (1..16)

}

-- TAG-MOBILITYSTATEPARAMETERS-STOP

-- ASN1STOP

MobilityStateParameters field descriptions

n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in TS 38.304 [20].

n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in TS 38.304 [20].

t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in TS 38.304 [20]. Value in seconds, s30 corresponds to 30 s and so on.

t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in TS 38.304 [20]. Value in seconds, value s30 corresponds to 30 seconds and so on.

#### –	MRB-Identity

The IE MRB-Identity is used to identify a multicast MRB used by a UE.

MRB-Identity information element

-- ASN1START

-- TAG-MRB-IDENTITY-START

MRB-Identity-r17 ::=                    INTEGER (1..512)

-- TAG-MRB-IDENTITY-STOP

-- ASN1STOP

#### –	MsgA-ConfigCommon

The IE MsgA-ConfigCommon is used to configure the PRACH and PUSCH resource for transmission of MsgA in 2-step random access type procedure.

-- ASN1START

-- TAG-MSGACONFIGCOMMON-START

MsgA-ConfigCommon-r16 ::=           SEQUENCE {

rach-ConfigCommonTwoStepRA-r16      RACH-ConfigCommonTwoStepRA-r16,

msgA-PUSCH-Config-r16               MsgA-PUSCH-Config-r16                                      OPTIONAL --Cond InitialBWPConfig

}

-- TAG-MSGACONFIGCOMMON-STOP

-- ASN1STOP

MsgA-ConfigCommon field descriptions

msgA-PUSCH-Config Configuration of cell-specific MsgA PUSCH parameters which the UE uses for contention-based MsgA PUSCH transmission of this BWP. If the field is not configured for the selected UL BWP, the UE shall use the MsgA PUSCH configuration of initial UL BWP.

rach-ConfigCommonTwoStepRA Configuration of cell specific random access parameters which the UE uses for contention based and contention free 2-step random access type procedure as well as for 2-step RA type contention based beam failure recovery in this BWP.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWPConfig | The field is mandatory present when MsgA-ConfigCommon is configured for the initial uplink BWP, or when MsgA-ConfigCommon is configured for a non-initial uplink BWP and MsgA-ConfigCommon is not configured for the initial uplink BWP, otherwise the field is optionally present, Need S. |

#### –	MsgA-PUSCH-Config

The IE MsgA-PUSCH-Config is used to specify the PUSCH allocation for MsgA in 2-step random access type procedure.

MsgA-PUSCH-Config information element

-- ASN1START

-- TAG-MSGA-PUSCH-CONFIG-START

MsgA-PUSCH-Config-r16 ::=                      SEQUENCE {

msgA-PUSCH-ResourceGroupA-r16                  MsgA-PUSCH-Resource-r16                                       OPTIONAL, -- Cond InitialBWPConfig

msgA-PUSCH-ResourceGroupB-r16                  MsgA-PUSCH-Resource-r16                                       OPTIONAL, -- Cond GroupBConfigured

msgA-TransformPrecoder-r16                    ENUMERATED {enabled, disabled}                                 OPTIONAL, -- Need S

msgA-DataScramblingIndex-r16                   INTEGER (0..1023)                                             OPTIONAL, -- Need S

msgA-DeltaPreamble-r16                         INTEGER (-1..6)                                               OPTIONAL  -- Need R

}

MsgA-PUSCH-Resource-r16 ::=                    SEQUENCE {

msgA-MCS-r16                                   INTEGER (0..15),

nrofSlotsMsgA-PUSCH-r16                        INTEGER (1..4),

nrofMsgA-PO-PerSlot-r16                        ENUMERATED {one, two, three, six},

msgA-PUSCH-TimeDomainOffset-r16                INTEGER (1..32),

msgA-PUSCH-TimeDomainAllocation-r16            INTEGER (1..maxNrofUL-Allocations)                            OPTIONAL, -- Need S

startSymbolAndLengthMsgA-PO-r16                INTEGER (0..127)                                              OPTIONAL, -- Need S

mappingTypeMsgA-PUSCH-r16                      ENUMERATED {typeA, typeB}                                     OPTIONAL, -- Need S

guardPeriodMsgA-PUSCH-r16                      INTEGER (0..3)                                                OPTIONAL, -- Need R

guardBandMsgA-PUSCH-r16                        INTEGER (0..1),

frequencyStartMsgA-PUSCH-r16                   INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofPRBs-PerMsgA-PO-r16                        INTEGER (1..32),

nrofMsgA-PO-FDM-r16                            ENUMERATED {one, two, four, eight},

msgA-IntraSlotFrequencyHopping-r16             ENUMERATED {enabled}                                          OPTIONAL, -- Need R

msgA-HoppingBits-r16                           BIT STRING (SIZE(2))                                          OPTIONAL, -- Cond FreqHopConfigured

msgA-DMRS-Config-r16                           MsgA-DMRS-Config-r16,

nrofDMRS-Sequences-r16                         INTEGER (1..2),

msgA-Alpha-r16                                 ENUMERATED {alpha0, alpha04, alpha05, alpha06,

alpha07, alpha08, alpha09, alpha1}                OPTIONAL, -- Need S

interlaceIndexFirstPO-MsgA-PUSCH-r16           INTEGER (1..10)                                               OPTIONAL, -- Need R

nrofInterlacesPerMsgA-PO-r16                   INTEGER (1..10)                                               OPTIONAL, -- Need R

...

}

MsgA-DMRS-Config-r16 ::=                       SEQUENCE {

msgA-DMRS-AdditionalPosition-r16               ENUMERATED {pos0, pos1, pos3}                                 OPTIONAL, -- Need S

msgA-MaxLength-r16                             ENUMERATED {len2}                                             OPTIONAL, -- Need S

msgA-PUSCH-DMRS-CDM-Group-r16                  INTEGER (0..1)                                                OPTIONAL, -- Need S

msgA-PUSCH-NrofPorts-r16                       INTEGER (0..1)                                                OPTIONAL, -- Need S

msgA-ScramblingID0-r16                         INTEGER (0..65535)                                            OPTIONAL, -- Need S

msgA-ScramblingID1-r16                         INTEGER (0..65535)                                            OPTIONAL  -- Need S

}

-- TAG-MSGA-PUSCH-CONFIG-STOP

-- ASN1STOP

MsgA-PUSCH-Config field descriptions

msgA-DataScramblingIndex Identifier used to initiate data scrambling (c_init) for msgA PUSCH. If the field is absent the UE applies the value Physical cell ID (physCellID).

msgA-DeltaPreamble Power offset of msgA PUSCH relative to the preamble received target power. Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1).

msgA-PUSCH-ResourceGroupA MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group A. If field is not configured for the selected UL BWP, the UE shall use the MsgA PUSCH configuration for group A of initial UL BWP or RedCap-specific initial UL BWP (if configured) for (e)RedCap UEs.

msgA-PUSCH-ResourceGroupB MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group B.

msgA-TransformPrecoder Enables or disables the transform precoder for MsgA transmission (see clause 6.1.3 of TS 38.214 [19]). If the field is absent in MsgA-ConfigCommon which is configured directly within a BWP (i.e., not within AdditionalRACH-Config), the UE determines whether to enable the transform precoding according to the msg3-transformPrecoder in RACH-ConfigCommon in the configured BWP. If the field is absent in MsgA-ConfigCommon in AdditionalRACH-Config, the UE determines whether to enable the transform precoding according to the msg3-transformPrecoder in RACH-ConfigCommon in the same AdditionalRACH-Config.

MsgA-PUSCH-Resource field descriptions

guardBandMsgA-PUSCH PRB-level guard band between FDMed PUSCH occasions (see TS 38.213 [13], clause 8.1A). If interlaced PUSCH is configured, value 0 is applied.

guardPeriodMsgA-PUSCH Guard period between PUSCH occasions in the unit of symbols (see TS 38.213 [13], clause 8.1A).

frequencyStartMsgA-PUSCH Offset of lowest PUSCH occasion in frequency domain with respect to PRB 0 (see TS 38.213 [13], clause 8.1A).

interlaceIndexFirstPO-MsgA-PUSCH Interlace index of the first PUSCH occasion in frequency domain if interlaced PUSCH is configured. For 30kHz SCS only the integers 1, 2, 3, 4, 5 are applicable (see TS 38.213 [13], clause 8.1A).

mappingTypeMsgA-PUSCH PUSCH mapping type A or B. If the field is absent, the UE shall use the parameter msgA-PUSCH-TimeDomainAllocation (see TS 38.213 [13], clause 8.1A).

msgA-Alpha Dedicated alpha value for MsgA PUSCH. If the field is absent, the UE shall use the value of msg3-Alpha if configured, else UE applies value 1 (see TS 38.213 [13], clause 7.1.1).

msgA-DMRS-Config DMRS configuration for msgA PUSCH (see TS 38.213 [13], clause 8.1A and TS 38.214 [19] clause 6.2.2).

msgA-HoppingBits Value of hopping bits to indicate which frequency offset to be used for second hop. See Table 8.3-1 in TS 38.213 [13].

msgA-IntraSlotFrequencyHopping Intra-slot frequency hopping per PUSCH occasion (see TS 38.213 [13], clause 8.1A).

msgA-MCS Indicates the MCS index for msgA PUSCH from the Table 6.1.4.1-1 for DFT-s-OFDM and Table 5.1.3.1-1 for CP-OFDM in TS 38.214 [19].

msgA-PUSCH-TimeDomainAllocation Indicates a combination of start symbol and length and PUSCH mapping type from the TDRA table (PUSCH-TimeDomainResourceAllocationList if provided in PUSCH-ConfigCommon, or else the default Table 6.1.2.1.1-2 in 38.214 [19] is used if pusch-TimeDomainAllocationList is not provided in PUSCH-ConfigCommon). The parameter K2 in the table is not used for msgA PUSCH. The network configures one of msgA-PUSCH-TimeDomainAllocation and startSymbolAndLengthMsgA-PO, but not both. If the field is absent, the UE shall use the value of startSymbolAndLenghtMsgA-PO.

msgA-PUSCH-TimeDomainOffset A single time offset with respect to the start of each PRACH slot (with at least one valid RO), counted as the number of slots (based on the numerology of active UL BWP). See TS 38.213 [13], clause 8.1A.

nrofDMRS-Sequences Number of DMRS sequences for MsgA PUSCH for CP-OFDM. In case of single PUSCH configuration or if the DMRS symbols of multiple configurations are not overlapped, if the DMRS resources configured in one PUSCH occasion is no larger than 8 (for len2) or 4 (for len1), then only DMRS port is configured.

nrofInterlacesPerMsgA-PO Number of consecutive interlaces per PUSCH occasion if interlaced PUSCH is configured. For 30kHz SCS only the integers 1, 2, 3, 4, 5 are applicable (see TS 38.213 [13], clause 8.1A).

nrofMsgA-PO-FDM The number of msgA PUSCH occasions FDMed in one time instance (see TS 38.213 [13], clause 8.1A).

nrofMsgA-PO-PerSlot Number of time domain PUSCH occasions in each slot. PUSCH occasions including guard period are contiguous in time domain within a slot (see TS 38.213 [13], clause 8.1A).

nrofPRBs-PerMsgA-PO Number of PRBs per PUSCH occasion (see TS 38.213 [13], clause 8.1A). An eRedcap UE ignores the PRACH and PUSCH resource parameters for transmission of MsgA in 2-step random access type procedure if configured number of PRBs per PUSCH occasion for Preambles group A is larger than the maximum number of PRBs that can be configured per slot or per hop. If configured number of PRBs per PUSCH occasion for Preambles group B is larger than the maximum number of PRBs that can be configured per slot or per hop, an eRedCap UE ignores the PRACH and PUSCH resource parameters for transmission of MsgA using Preambles group B.

nrofSlotsMsgA-PUSCH Number of slots (in active UL BWP numerology) containing one or multiple PUSCH occasions, each slot has the same time domain resource allocation (see TS 38.213 [13], clause 8.1A).

startSymbolAndLengthMsgA-PO An index giving valid combinations of start symbol, length and mapping type as start and length indicator (SLIV) for the first msgA PUSCH occasion, for RRC_CONNECTED UEs in non-initial BWP as described in TS 38.214 [19] clause 6.1.2. The network configures the field so that the allocation does not cross the slot boundary. The number of occupied symbols excludes the guard period. If the field is absent, the UE shall use the value in msgA-PUSCH-TimeDomainAllocation (see TS 38.213 [13], clause 8.1A). The network configures one of msgA-PUSCH-TimeDomainAllocation and startSymbolAndLengthMsgA-PO, but not both. If the field is absent, the UE shall use the value of msgA-PUSCH-TimeDomainAllocation.

MsgA-DMRS-Config field descriptions

msgA-DMRS-AdditionalPosition Indicates the position for additional DM-RS. If the field is absent, the UE applies value pos2.

msgA-MaxLength indicates single-symbol or double-symbol DMRS. If the field is absent, the UE applies value len1.

msgA-PUSCH-DMRS-CDM-Group 1-bit indication of indices of CDM group(s). If the field is absent, then both CDM groups are used.

msgA-PUSCH-NrofPorts 0 indicates 1 port per CDM group, 1 indicates 2 ports per CDM group. If the field is absent then 4 ports per CDM group are used (see TS 38.213 [13], clause 8.1A).

msgA-ScramblingID0 UL DMRS scrambling initialization for CP-OFDM. If the field is absent the UE applies the value Physical cell ID (physCellID).

msgA-ScramblingID1 UL DMRS scrambling initialization for CP-OFDM. If the field is absent the UE applies the value Physical cell ID (physCellID).

| Conditional Presence | Explanation |
| --- | --- |
| FreqHopConfigured | This field is mandatory present when the field msgA-IntraSlotFrequencyHopping is configured. Otherwise, the field is absent. |
| GroupBConfigured | The field is mandatory present if groupB-ConfiguredTwoStepRA is configured in RACH-ConfigCommonTwoStepRA, otherwise the field is absent. |
| InitialBWPConfig | The field is mandatory present when MsgA-ConfigCommon is configured for the initial uplink BWP, or when MsgA-ConfigCommon is configured for a non-initial uplink BWP and MsgA-ConfigCommon is not configured for the initial uplink BWP, otherwise the field is optionally present, Need S. |

#### –	MultiFrequencyBandListNR

The IE MultiFrequencyBandListNR is used to configure a list of one or multiple NR frequency bands.

MultiFrequencyBandListNR information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-START

MultiFrequencyBandListNR ::=        SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR

-- TAG-MULTIFREQUENCYBANDLISTNR-STOP

-- ASN1STOP

#### –	MultiFrequencyBandListNR-SIB

The IE MultiFrequencyBandListNR-SIB indicates the list of frequency bands, for which cell (re-)selection parameters are common, and a list of additionalPmax and additionalSpectrumEmission.

MultiFrequencyBandListNR-SIB information element

-- ASN1START

-- TAG-MULTIFREQUENCYBANDLISTNR-SIB-START

MultiFrequencyBandListNR-SIB ::=            SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo

NR-MultiBandInfo ::=                        SEQUENCE {

freqBandIndicatorNR                         FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxList                              NR-NS-PmaxList              OPTIONAL    -- Need S

}

MultiFrequencyBandListNR-SIB-v1760 ::=      SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo-v1760

NR-MultiBandInfo-v1760 ::=                  SEQUENCE {

nr-NS-PmaxList-v1760                        NR-NS-PmaxList-v1760        OPTIONAL    -- Need S

}

MultiFrequencyBandListNR-Aerial-SIB-r18 ::= SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfoAerial-r18

NR-MultiBandInfoAerial-r18 ::=              SEQUENCE {

freqBandIndicatorNR-Aerial-r18              FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxListAerial-r18                    NR-NS-PmaxListAerial-r18    OPTIONAL    -- Need S

}

-- TAG-MULTIFREQUENCYBANDLISTNR-SIB-STOP

-- ASN1STOP

NR-MultiBandInfo field descriptions

freqBandIndicatorNR Provides an NR frequency band number as defined in TS 38.101-1 [15], TS 38.101-2 [39], table 5.2-1, and TS 38.101-5 [75], table 5.2.2-1.

freqBandIndicatorNR-Aerial Provides an NR frequency band number for aerial UE(s), as defined in TS 38.101-1 [15], TS 38.101-2 [39], table 5.2-1, and TS 38.101-5 [75], table 5.2.2-1.

nr-NS-PmaxList Provides a list of additionalPmax and additionalSpectrumEmission values. If the field is absent the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15] table 6.2.3.1-1A, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). This field is ignored by IAB-MT and wide area NCR-MT (see TS 38.106 [79]). The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The wide area NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79]. If nr-NS-PmaxList-v1760 is present, it shall contain the same number of entries, listed in the same order as in nr-NS-PmaxList (without suffix).

nr-NS-PmaxListAerial Provides a list of additionalPmax and additionalSpectrumEmission values for aerial UE(s) as defined in TS 38.101-1 [15], clause 6.2K. If the field is absent, the value indicated by the corresponding field within frequencyBandList for the same NR frequency band number applies, if present. Otherwise (i.e. the field is not present for the same NR frequency band number in any of the nr-NS-PmaxList, nr-NS-PmaxList-v1760 or nr-NS-PmaxListAerial), the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15] table 6.2.3.1-1A, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). This field is ignored by IAB-MT and wide area NCR-MT (see TS 38.106 [79]). The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The wide area NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

| Conditional Presence | Explanation |
| --- | --- |
| OptULNotSIB2 | The field is absent for SIB2 and is mandatory present if the parent IE is included in SIB4 or frequencyInfoDL-SIB. Otherwise, if the field is absent in frequencyInfoUL-SIB in UplinkConfigCommonSIB, the UE will use the frequency band indicated in frequencyInfoDL-SIB in DownlinkConfigCommonSIB. |

#### –	MUSIM-GapConfig

The IE MUSIM-GapConfig specifies the MUSIM gap configuration and controls setup/release of MUSIM gaps.

MUSIM-GapConfig information element

-- ASN1START

-- TAG-MUSIM-GAPCONFIG-START

MUSIM-GapConfig-r17 ::=                  SEQUENCE {

musim-GapToReleaseList-r17               SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17               OPTIONAL, -- Need N

musim-GapToAddModList-r17                SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17                 OPTIONAL, -- Need N

musim-AperiodicGap-r17                   MUSIM-GapInfo-r17                                       OPTIONAL, -- Need N

...,

[[

musim-GapToAddModListExt-v1820           SEQUENCE (SIZE (1..3)) OF MUSIM-GapExt-v1820            OPTIONAL, -- Need N

musim-GapKeep-r18                        ENUMERATED {true}                                       OPTIONAL  -- Need N

]]

}

MUSIM-Gap-r17 ::=                        SEQUENCE {

musim-GapId-r17                          MUSIM-GapId-r17,

musim-GapInfo-r17                        MUSIM-GapInfo-r17

}

MUSIM-GapExt-v1820 ::=                   SEQUENCE {

gapPriority-r18                          GapPriority-r17

}

-- TAG-MUSIM-GAPCONFIG-STOP

-- ASN1STOP

MUSIM-GapConfig field descriptions

musim-AperiodicGap Indicates the MUSIM aperiodic gap as specified in TS 38.133 [14] clause 9.1.10. If UE indicates the musim-Starting-SFN-AndSubframe when requesting aperiodic gap the network can only configure the aperiodic gap with the same start point or no aperiodic gap. If the field musim-Starting-SFN-AndSubframe is absent for aperiodic gap, network can configure any timing as the starting point for aperiodic gap or configure no aperiodic gap.

musim-GapInfo Indicates the values for musim-GapLength and musim-GapRepetitionAndOffset. When network provides periodic gap, network always signals the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred MUSIM gap configuration.

musim-GapKeep Indicates the UE is allowed to use "keep solution" for colliding MUSIM periodic/aperiodic gaps. If "keep solution" is not granted, collisions between MUSIM periodic gaps are resolved based on the assigned MUSIM gap priorities as specified in TS 38.133[14].

musim-GapToAddModListExt Indicates the priority of MUSIM periodic gap(s). If the network includes musim-GapToAddModListExt-v1820, it includes the same number of entries, and listed in the same order, as in musim-GapToAddModList-r17. For the priority of MUSIM aperiodic gap, the MUSIM aperiodic gap is always kept (not dropped) from UE perspective in case of collisions with other gaps (i.e. all gaps including MUSIM gaps, etc).

musim-GapToAddModList List of MUSIM periodic gap patterns to add or modify.

musim-GapToReleaseList List of MUSIM periodic gap patterns to release.

#### –	MUSIM-GapId

The IE MUSIM-GapId is used to identify UE periodic MUSIM gap(s) to add, modify or release.

MUSIM-GapId information element

-- ASN1START

-- TAG-MUSIM-GAPID-START

MUSIM-GapId-r17 ::=                  INTEGER (0..2)

-- TAG-MUSIM-GAPID-STOP

-- ASN1STOP

#### –	MUSIM-GapInfo

The IE MUSIM-GapInfo is used to indicate MUSIM gap parameters.

MUSIM-GapInfo information element

-- ASN1START

-- TAG-MUSIM-GAPINFO-START

MUSIM-GapInfo-r17 ::=               SEQUENCE {

musim-Starting-SFN-AndSubframe-r17  MUSIM-Starting-SFN-AndSubframe-r17             OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Cond gapSetup

musim-GapRepetitionAndOffset-r17    CHOICE {

ms20-r17                            INTEGER (0..19),

ms40-r17                            INTEGER (0..39),

ms80-r17                            INTEGER (0..79),

ms160-r17                           INTEGER (0..159),

ms320-r17                           INTEGER (0..319),

ms640-r17                           INTEGER (0..639),

ms1280-r17                          INTEGER (0..1279),

ms2560-r17                          INTEGER (0..2559),

ms5120-r17                          INTEGER (0..5119),

...

}                                                                                  OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE {

starting-SFN-r17                       INTEGER (0..1023),

startingSubframe-r17                   INTEGER (0..9)

}

-- TAG-MUSIM-GAPINFO-STOP

-- ASN1STOP

MUSIM-GapInfo field descriptions

musim-GapLength Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for both periodic gap and aperiodic gap preference indication.

musim-GapRepetitionAndOffset Indicates the gap repetition period in ms and gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for the periodic MUSIM gap preference indication.

musim-Starting-SFN-AndSubframe Indicates gap starting position for the aperiodic MUSIM gap. This field is optionally present for the aperiodic MUSIM gap preference indication.

starting-SFN Indicates gap starting SFN number for the aperiodic MUSIM gap.

startingSubframe Indicates gap starting subframe number for the aperiodic MUSIM gap.

| Conditional Presence | Explanation |
| --- | --- |
| aperiodic | This field is mandatory present in case of aperiodic MUSIM gap configuration. Otherwise it is absent. |
| gapSetup | The field is mandatory present upon configuration of a new MUSIM gap. The field is optionally present, Need M, otherwise. |
| periodic | This field is mandatory present in case of periodic MUSIM gap configuration. Otherwise it is absent. |

#### –	N3C-IndirectPathConfigRelay

The IE N3C-IndirectPathConfigRelay indicates the N3C indirect path related configuration used by N3C relay UE in MP.

N3C-IndirectPathConfigRelay information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHCONFIGRELAY-START

N3C-IndirectPathConfigRelay-r18 ::=    SEQUENCE {

n3c-MappingToReleaseList-r18           SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17        OPTIONAL, -- Need N

n3c-MappingToAddModList-r18            SEQUENCE (SIZE (1..maxLC-ID)) OF N3C-MappingConfig-r18              OPTIONAL, -- Need N

...

}

N3C-MappingConfig-r18 ::=              SEQUENCE {

n3c-RemoteUE-RB-Identity-r18           SL-RemoteUE-RB-Identity-r17,

n3c-RLC-ChannelUu-r18                  Uu-RelayRLC-ChannelID-r17,

...

}

-- TAG-N3C-INDIRECTPATHCONFIGRELAY-STOP

-- ASN1STOP

N3C-IndirectPathConfigRelay field descriptions

n3c-MappingToAddModList Indicates the list of mappings between the bearer identity of the N3C remote UE and the Uu Relay RLC channel to be added or modified. The Uu Relay RLC channel is associated to only one N3C remote UE's end-to-end bearer.

n3c-MappingToReleaseList Indicates the list of mappings between the bearer identity of the N3C remote UE and the Uu Relay RLC channel to be released.

#### –	N3C-IndirectPathAddChange

The IE N3C-IndirectPathAddChange indicates the N3C indirect path related configuration used by N3C remote UE.

N3C-IndirectPathAddChange information element

-- ASN1START

-- TAG-N3C-INDIRECTPATHADDCHANGE-START

N3C-IndirectPathAddChange-r18 ::=  SEQUENCE {

n3c-RelayIdentification-r18        N3C-RelayUE-Info-r18,

...

}

-- TAG-N3C-INDIRECTPATHADDCHANGE-STOP

-- ASN1STOP

N3C-IndirectPathAddChange field descriptions

n3c-RelayIdentification Indicates the NCGI and C-RNTI of N3C relay UE.

#### –	N3C-RelayUE-Info

The IE N3C-RelayUE-Info includes the information of N3C relay UE.

N3C-RelayUE-Info information element

-- ASN1START

-- TAG-N3CRELAYUEINFO-START

N3C-RelayUE-Info-r18 ::=      SEQUENCE {

n3c-CellGlobalId-r18          SEQUENCE {

n3c-PLMN-Id-r18               PLMN-Identity,

n3c-CellIdentity-r18          CellIdentity

},

n3c-C-RNTI-r18                RNTI-Value

}

-- TAG-N3CRELAYUEINFO-STOP

-- ASN1STOP

#### –	NCR-AperiodicFwdConfig

The IE NCR-AperiodicFwdConfig is used to configure a list of aperiodic forwarding time resources for NCR-Fwd access link (see TS 38.212 [17], clause 7.3.1.3.9 and TS 38.213 [13], clause 20).

NCR-AperiodicFwdConfig information element

-- ASN1START

-- TAG-NCR-APERIODICFWDCONFIG-START

NCR-AperiodicFwdConfig-r18 ::= SEQUENCE {

aperiodicFwdTimeRsrcToAddModList-r18  SEQUENCE (SIZE (1..maxNrofAperiodicFwdTimeResource-r18)) OF NCR-AperiodicFwdTimeResource-r18

OPTIONAL, -- Need N

aperiodicFwdTimeRsrcToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofAperiodicFwdTimeResource-r18)) OF NCR-AperiodicFwdTimeResourceId-r18

OPTIONAL, -- Need N

referenceSCS-r18                      SubcarrierSpacing                                             OPTIONAL, -- Need M

aperiodicBeamFieldWidth-r18           INTEGER (1..6)                                                OPTIONAL, -- Need M

numberOfFields-r18                    INTEGER (1..32)                                               OPTIONAL, -- Need M

...

}

NCR-AperiodicFwdTimeResource-r18 ::= SEQUENCE {

aperiodicFwdTimeRsrcId-r18           NCR-AperiodicFwdTimeResourceId-r18,

slotOffsetAperiodic-r18              INTEGER (0..14),

symbolOffset-r18                     INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18                INTEGER (1..28),

...

}

NCR-AperiodicFwdTimeResourceId-r18 ::= INTEGER (0..maxNrofAperiodicFwdTimeResource-1-r18)

-- TAG-NCR-APERIODICFWDCONFIG-STOP

-- ASN1STOP

NCR-AperiodicFwdConfig field descriptions

aperiodicBeamFieldWidth Indicates the bitwidth of each beam index field in DCI carrying aperiodic beam indication.

aperiodicFwdTimeRsrcToAddModList List of aperiodic forwarding time resources to be added or modified.

durationInSymbols Indicates the time duration in number of symbols.

numberOfFields Indicates the number of time resource fields in DCI carrying aperiodic beam indication.

referenceSCS Indicates the reference subcarrier spacing for all the time resources in the list. Only values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

slotOffsetAperiodic Indicates the slot offset used to define the start slot of aperiodic time resource.

symbolOffset Indicates the symbol offset in one slot.

#### –	NCR-FwdConfig

The IE NCR-FwdConfig contains configuration related to periodic, aperiodic and semi-persistent beam indication for NCR-Fwd access link.

NCR-FwdConfig information element

-- ASN1START

-- TAG-NCR-FWDCONFIG-START

NCR-FwdConfig-r18 ::=                SEQUENCE {

periodicFwdRsrcSetToAddModList-r18   SEQUENCE (SIZE (1..maxNrofPeriodicFwdResourceSet-r18)) OF NCR-PeriodicFwdResourceSet-r18

OPTIONAL,  -- Need N

periodicFwdRsrcSetToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofPeriodicFwdResourceSet-r18)) OF NCR-PeriodicFwdResourceSetId-r18

OPTIONAL,  -- Need N

aperiodicFwdConfig-r18                      SetupRelease {NCR-AperiodicFwdConfig-r18}                   OPTIONAL,   -- Need M

semiPersistentFwdRsrcSetToAddModList-r18    SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResourceSet-r18)) OF

NCR-SemiPersistentFwdResourceSet-r18    OPTIONAL,  -- Need N

semiPersistentFwdRsrcSetToReleaseList-r18   SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResourceSet-r18)) OF

NCR-SemiPersistentFwdResourceSetId-r18  OPTIONAL,  -- Need N

...

}

-- TAG-NCR-FWDCONFIG-STOP

-- ASN1STOP

NCR-FwdConfig field descriptions

aperiodicFwdConfig Aperiodic time resource configuration for beam indication for NCR. The configuration includes a list of time domain resources that can be selected for aperiodic forwarding.

periodicFwdRsrcSetToAddModList List of periodic forwarding resource configurations to be added or modified.

periodicFwdRsrcSetToReleaseList List of periodic forwarding resource configurations to be released.

semiPersistentFwdRsrcSetToAddModList List of semi-persistent forwarding resource configurations to be added or modified.

#### –	NCR-PeriodicityAndOffset

The IE NCR-PeriodicityAndOffset is used to indicate the periodicity (in slot or ms) and offset (in slot) for periodic forwarding resource and semi-persistent forwarding resource.

NCR-PeriodicityAndOffset information element

-- ASN1START

-- TAG-NCR-PERIODICITYANDOFFSET-START

NCR-PeriodicityAndOffset-r18 ::= CHOICE {

slot                             NCR-SlotPeriodicityAndSlotOffset-r18,

ms                               NCR-MsPeriodicityAndSlotOffset-r18

}

NCR-SlotPeriodicityAndSlotOffset-r18 ::=  CHOICE {

sl1                                       NULL,

sl2                                       INTEGER(0..1),

sl4                                       INTEGER(0..3),

sl5                                       INTEGER(0..4),

sl8                                       INTEGER(0..7),

sl10                                      INTEGER(0..9),

sl16                                      INTEGER(0..15),

sl20                                      INTEGER(0..19),

sl32                                      INTEGER(0..31),

sl40                                      INTEGER(0..39),

sl64                                      INTEGER(0..63),

sl80                                      INTEGER(0..79),

sl128                                     INTEGER(0..127),

sl160                                     INTEGER(0..159),

sl256                                     INTEGER(0..255),

sl320                                     INTEGER(0..319),

sl512                                     INTEGER(0..511),

sl640                                     INTEGER(0..639),

sl1024                                    INTEGER(0..1023),

sl1280                                    INTEGER(0..1279),

sl2560                                    INTEGER(0..2559),

sl5120                                    INTEGER(0..5119),

sl10240                                   INTEGER(0..10239),

...

}

NCR-MsPeriodicityAndSlotOffset-r18 ::=    CHOICE {

ms1                                       INTEGER(0..15),

ms2                                       INTEGER(0..31),

ms4                                       INTEGER(0..63),

ms5                                       INTEGER(0..79),

ms8                                       INTEGER(0..127),

ms10                                      INTEGER(0..159),

ms16                                      INTEGER(0..255),

ms20                                      INTEGER(0..319),

ms32                                      INTEGER(0..511),

ms40                                      INTEGER(0..639),

ms64                                      INTEGER(0..1023),

ms80                                      INTEGER(0..1279),

ms128                                     INTEGER(0..2047),

ms160                                     INTEGER(0..2559),

ms256                                     INTEGER(0..4095),

ms320                                     INTEGER(0..5119),

ms512                                     INTEGER(0..8191),

ms640                                     INTEGER(0..10239),

ms1024                                    INTEGER(0..16383),

ms1280                                    INTEGER(0..20479),

ms2560                                    INTEGER(0..40959),

ms5120                                    INTEGER(0..81919),

ms10240                                   INTEGER(0..163839),

...

}

-- TAG-NCR-PERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	NCR-PeriodicFwdResourceSet

The IE NCR-PeriodicFwdResourceSet is used to configure a list of periodic forwarding resources for NCR-Fwd access link (see TS 38.213 [13], clause 20). Each periodic forwarding configuration includes a list of periodic forwarding resources, a common periodicity and a common reference SCS.

NCR-PeriodicFwdResourceSet information element

-- ASN1START

-- TAG-NCR-PERIODICFWDRESOURCESET-START

NCR-PeriodicFwdResourceSet-r18 ::=  SEQUENCE {

periodicFwdRsrcSetId-r18            NCR-PeriodicFwdResourceSetId-r18,

periodicFwdRsrcToAddModList-r18     SEQUENCE (SIZE (1..maxNrofPeriodicFwdResource-r18)) OF NCR-PeriodicFwdResource-r18

OPTIONAL,  -- Need N

periodicFwdRsrcToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofPeriodicFwdResource-r18)) OF NCR-PeriodicFwdResourceId-r18

OPTIONAL,  -- Need N

referenceSCS-r18                    SubcarrierSpacing                                                   OPTIONAL,  -- Need M

priorityFlag-r18                    ENUMERATED {true}                                                   OPTIONAL,  -- Need R

...

}

NCR-PeriodicFwdResource-r18 ::=  SEQUENCE {

periodicFwdRsrcId-r18            NCR-PeriodicFwdResourceId-r18,

beamIndex-r18                    INTEGER (0..63),

periodicTimeRsrc-r18             SEQUENCE {

periodicityAndOffset-r18     NCR-PeriodicityAndOffset-r18,

symbolOffset-r18             INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18        INTEGER (1..112)

},

...

}

NCR-PeriodicFwdResourceId-r18 ::= INTEGER (0..maxNrofPeriodicFwdResource-1-r18)

-- TAG-NCR-PERIODICFWDRESOURCESET-STOP

-- ASN1STOP

NCR-PeriodicFwdResourceSet field descriptions

beamIndex Indicates the logical beam index for NCR-Fwd access link. NCR-Fwd is assumed to be ON over the indicated time domain resource if there is beam indication.

durationInSymbols Indicates the time duration in number of symbols.

periodicFwdRsrcToAddModList List of periodic forwarding resources to be added or modified.

periodicityAndOffset Indicates the periodicity and slot offset for the periodic forwarding resource. All the periodic forwarding resources configured within the same resource set should have the same periodicity. If the periodicity is expressed in [ms], the value range of slot offset is from 0 to Periodicity[ms] * referenceSCS[kHz]/15 -1.

priorityFlag Indicates the priority for the list of periodic forwarding resources, as specified in TS 38.213 [13], clause 20.

referenceSCS Indicates the reference subcarrier spacing for all the time resources in the list. Only values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

symbolOffset Indicates the symbol offset in one slot.

#### –	NCR-PeriodicFwdResourceSetId

The IE NCR-PeriodicFwdResourceSetId is used to identify one NCR-PeriodicFwdResourceSet.

NCR-PeriodicFwdResourceSetId information element

-- ASN1START

-- TAG-NCR-PERIODICFWDRESOURCESETID-START

NCR-PeriodicFwdResourceSetId-r18 ::= INTEGER (0..maxNrofPeriodicFwdResourceSet-1-r18)

-- TAG-NCR-PERIODICFWDRESOURCESETID-STOP

-- ASN1STOP

#### –	NCR-SemiPersistentFwdResourceSet

The IE NCR-SemiPersistentFwdResourceSet is used to configure a list of semi-persistent forwarding resources for NCR-Fwd access link (see TS 38.213 [13], clause 20). Each semi-persistent forwarding resource configuration includes a list of semi-persistent forwarding resources, a common periodicity and a common reference SCS.

NCR-SemiPersistentFwdResourceSet information element

-- ASN1START

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESET-START

NCR-SemiPersistentFwdResourceSet-r18 ::= SEQUENCE {

semiPersistentFwdRsrcSetId-r18           NCR-SemiPersistentFwdResourceSetId-r18,

semiPersistentFwdRsrcToAddModList-r18    SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResource-r18)) OF

NCR-SemiPersistentFwdResource-r18                       OPTIONAL,   -- Need N

semiPersistentFwdRsrcToReleaseList-r18   SEQUENCE (SIZE (1..maxNrofSemiPersistentFwdResource-r18)) OF

NCR-SemiPersistentFwdResourceId-r18                     OPTIONAL,  -- Need N

referenceSCS-r18                         SubcarrierSpacing                                               OPTIONAL,  -- Need M

priorityFlag-r18                         ENUMERATED {true}                                               OPTIONAL,   -- Need R

...

}

NCR-SemiPersistentFwdResource-r18 ::= SEQUENCE {

semiPersistentFwdRsrcId-r18           NCR-SemiPersistentFwdResourceId-r18,

beamIndex-r18                         INTEGER (0..63),

semiPersistentTimeRsrc-r18            SEQUENCE {

periodicityAndOffset-r18              NCR-PeriodicityAndOffset-r18,

symbolOffset-r18                      INTEGER (0..maxNrofSymbols-1),

durationInSymbols-r18                 INTEGER (1..112)

},

...

}

NCR-SemiPersistentFwdResourceId-r18 ::= INTEGER (0..maxNrofSemiPersistentFwdResource-1-r18)

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESET-STOP

-- ASN1STOP

NCR-SemiPersistentFwdResourceSet field descriptions

beamIndex Indicates logical beam index for NCR-Fwd access link. NCR-Fwd is assumed to be ON over the indicated time domain resource if there is beam indication.

durationInSymbols Indicates the time duration in number of symbols.

periodicityAndOffset Indicates the periodicity and slot offset for the semi-persistent forwarding resource. All the semi-persistent forwarding resources configured within the same resource set should have the same periodicity. If the periodicity is expressed in [ms], the value range of slot offset is from 0 to Periodicity[ms] * referenceSCS[kHz]/15 -1.

priorityFlag Indicates the priority for the list of semi-persistent forwarding resources, as specified in TS 38.213 [13], clause 20.

referenceSCS Indicates the reference subcarrier spacing for all the time resources in the list. Only values kHz15, kHz30, kHz60, kHz120 and kHz240 are applicable.

semiPersistentFwdRsrcToAddModList List of semi-persistent forwarding resources to be added or modified.

symbolOffset Indicates the symbol offset in one slot.

#### –	NCR-SemiPersistentFwdResourceSetId

The IE NCR-SemiPersistentFwdResourceSetId is used to identify one NCR-SemiPersistentFwdResourceSet.

NCR-SemiPersistentFwdResourceSetId information element

-- ASN1START

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESETID-START

NCR-SemiPersistentFwdResourceSetId-r18 ::= INTEGER (0..maxNrofSemiPersistentFwdResourceSet-1-r18)

-- TAG-NCR-SEMIPERSISTENTFWDRESOURCESETID-STOP

-- ASN1STOP

#### –	NeedForGapsConfigNR

The IE NeedForGapsConfigNR contains configuration related to the reporting of measurement gap requirement information.

NeedForGapsConfigNR information element

-- ASN1START

-- TAG-NeedForGapsConfigNR-START

NeedForGapsConfigNR-r16 ::=        SEQUENCE {

requestedTargetBandFilterNR-r16       SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR               OPTIONAL          -- Need R

}

-- TAG-NeedForGapsConfigNR-STOP

-- ASN1STOP

NeedForGapsConfigNR field descriptions

requestedTargetBandFilterNR Indicates the target NR bands that the UE is requested to report the gap requirement information.

#### –	NeedForGapsInfoNR

The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapsInfoNR information element

-- ASN1START

-- TAG-NeedForGapsInfoNR-START

NeedForGapsInfoNR-r16 ::=        SEQUENCE {

intraFreq-needForGap-r16      NeedForGapsIntraFreqList-r16,

interFreq-needForGap-r16      NeedForGapsBandListNR-r16

}

NeedForGapsIntraFreqList-r16 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16

NeedForGapsBandListNR-r16 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16

NeedForGapsIntraFreq-r16  ::=                 SEQUENCE {

servCellId-r16                               ServCellIndex,

gapIndicationIntra-r16                       ENUMERATED {gap, no-gap}

}

NeedForGapsNR-r16  ::=                        SEQUENCE {

bandNR-r16                                   FreqBandIndicatorNR,

gapIndication-r16                            ENUMERATED {gap, no-gap}

}

-- TAG-NeedForGapsInfoNR-STOP

-- ASN1STOP

NeedForGapsInfoNR field descriptions

intraFreq-needForGap Indicates the measurement gap requirement information for NR intra-frequency measurement.

interFreq-needForGap Indicates the measurement gap requirement information for NR inter-frequency measurement.

NeedForGapsIntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB) do not contain the frequency domain resources of the SSB associated to the initial DL BWP (CD-SSB). Value no-gap indicates a measurement gap is not needed to measure the SSB associated to the initial DL BWP (CD-SSB) for all configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB), no matter the SSB is within the configured BWP or not. This field shall be set to 'no-gap' for the serving cell(s) belonging to the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE.

NeedForGapsNR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed.

#### –	NeedForGapNCSG-ConfigEUTRA

The IE NeedForGapNCSG-ConfigEUTRA contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForGapNCSG-ConfigEUTRA information element

-- ASN1START

-- TAG-NeedForGapNCSG-ConfigEUTRA-START

NeedForGapNCSG-ConfigEUTRA-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-EUTRA-r17   SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA     OPTIONAL    -- Need R

}

-- TAG-NeedForGapNCSG-ConfigEUTRA-STOP

-- ASN1STOP

NeedForGapNCSG-ConfigEUTRA field descriptions

requestedTargetBandFilterNCSG-EUTRA Indicates the target E-UTRA bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForGapNCSG-ConfigNR

The IE NeedForGapNCSG-ConfigNR contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForGapNCSG-ConfigNR information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-CONFIGNR-START

NeedForGapNCSG-ConfigNR-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-NR-r17   SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR          OPTIONAL          -- Need R

}

-- TAG-NEEDFORGAPNCSG-CONFIGNR-STOP

-- ASN1STOP

NeedForGapNCSG-ConfigNR field descriptions

requestedTargetBandFilterNCSG-NR Indicates the target NR bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForGapNCSG-InfoEUTRA

The IE NeedForGapNCSG-InfoEUTRA indicates whether measurement gap or NCSG is required for the UE to perform measurements on an E-UTRA target band while NR-DC or NE-DC is not configured.

NeedForGapNCSG-InfoEUTRA information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-INFOEUTRA-START

NeedForGapNCSG-InfoEUTRA-r17 ::=   SEQUENCE {

needForNCSG-EUTRA-r17              SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17

}

NeedForNCSG-EUTRA-r17  ::=         SEQUENCE {

bandEUTRA-r17                      FreqBandIndicatorEUTRA,

gapIndication-r17                  ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NEEDFORGAPNCSG-INFOEUTRA-STOP

-- ASN1STOP

NeedForGapNCSG-InfoEUTRA field descriptions

needForNCSG-EUTRA Indicates the measurement gap and NCSG requirement information for E-UTRA measurement.

NeedForNCSG-EUTRA field descriptions

bandEUTRA Indicates the E-UTRA target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform measurements on the concerned E-UTRA target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration message or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that NCSG is needed, value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed. Value nogap-noncsg also indicates interruption is not needed.

#### –	NeedForGapNCSG-InfoNR

The IE NeedForGapNCSG-InfoNR indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapNCSG-InfoNR information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-INFONR-START

NeedForGapNCSG-InfoNR-r17 ::=     SEQUENCE {

intraFreq-needForNCSG-r17         NeedForNCSG-IntraFreqList-r17,

interFreq-needForNCSG-r17         NeedForNCSG-BandListNR-r17

}

NeedForNCSG-IntraFreqList-r17 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17

NeedForNCSG-BandListNR-r17 ::=    SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17

NeedForNCSG-IntraFreq-r17  ::=    SEQUENCE {

servCellId-r17                    ServCellIndex,

gapIndicationIntra-r17            ENUMERATED {gap, ncsg, nogap-noncsg}

}

NeedForNCSG-NR-r17  ::=           SEQUENCE {

bandNR-r17                        FreqBandIndicatorNR,

gapIndication-r17                 ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NEEDFORGAPNCSG-INFONR-STOP

-- ASN1STOP

NeedForGapNCSG-InfoNR field descriptions

intraFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR intra-frequency measurement.

interFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR inter-frequency measurement.

NeedForNCSG-IntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap or NCSG is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB) do not contain the frequency domain resources of the SSB associated to the initial DL BWP (CD-SSB). Value ncsg indicates that a NCSG is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value nogap-noncsg indicates that neither a measurement gap nor a NCSG is needed to measure the SSB associated to the initial DL BWP (CD-SSB) for all configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB), no matter the SSB is within the configured BWP or not. This field shall be set to 'nogap-noncsg' for the serving cell cell(s) belonging to the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE. Value nogap-noncsg also indicates interruption is not needed.

NeedForNCSG-NR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that a NCSG is needed, and value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed. Value nogap-noncsg also indicates interruption is not needed.

#### –	NeedForInterruptionInfoNR

The IE NeedForInterruptionInfoNR indicates whether interruption is needed for the UE to perform SSB based measurements on an NR target band without measurement gap while NR-DC or NE-DC is not configured.

NeedForInterruptionInfoNR information element

-- ASN1START

-- TAG-NeedForInterruptionInfoNR-START

NeedForInterruptionInfoNR-r18 ::=    SEQUENCE {

intraFreq-needForInterruption-r18    NeedForInterruptionIntraFreqList-r18,

interFreq-needForInterruption-r18    NeedForInterruptionBandListNR-r18,

...

}

NeedForInterruptionIntraFreqList-r18 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForInterruptionNR-r18

NeedForInterruptionBandListNR-r18 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18

NeedForInterruptionNR-r18  ::=       SEQUENCE {

interruptionIndication-r18           ENUMERATED {no-gap-with-interruption, no-gap-no-interruption}                     OPTIONAL

}

-- TAG-NeedForInterruptionInfoNR-STOP

-- ASN1STOP

NeedForInterruptionInfoNR field descriptions

intraFreq-needForInterruption Indicates the interruption requirement information for NR intra-frequency measurement. Each entry in the list is associated to the entry in list intraFreq-needForGap-r16 with the same index. This field shall be set to no-gap-no-interruption for the serving cell(s) belonging to the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE.

interFreq-needForInterruption Indicates the interruption requirement information for NR inter-frequency measurement. Each entry in the list is associated to the entry in list interFreq-needForGap-r16 with the same index.

NeedForInterruptionNR field descriptions

interruptionIndication Indicates whether interruption is needed for the UE to perform SSB based measurements without measurement gap. Value no-gap-with-interruption indicates that interruption is needed. Value no-gap-no-interruption indicates interruption is not needed.

#### –	NextHopChainingCount

The IE NextHopChainingCount is used to update the KgNB key and corresponds to parameter NCC: See TS 33.501 [11].

NextHopChainingCount information element

-- ASN1START

-- TAG-NEXTHOPCHAININGCOUNT-START

NextHopChainingCount ::=                    INTEGER (0..7)

-- TAG-NEXTHOPCHAININGCOUNT-STOP

-- ASN1STOP

#### –	NG-5G-S-TMSI

The IE NG-5G-S-TMSI contains a 5G S-Temporary Mobile Subscription Identifier (5G-S-TMSI), a temporary UE identity provided by the 5GC which uniquely identifies the UE within the tracking area, see TS 23.003 [21].

NG-5G-S-TMSI information element

-- ASN1START

-- TAG-NG-5G-S-TMSI-START

NG-5G-S-TMSI ::=                         BIT STRING (SIZE (48))

-- TAG-NG-5G-S-TMSI-STOP

-- ASN1STOP

#### –	NonCellDefiningSSB

The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

NonCellDefiningSSB field descriptions

absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.

ssb-Periodicity The periodicity of this NCD-SSB. The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB. If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB).

ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB.

#### –	NPN-Identity

The IE NPN-Identity includes either a list of CAG-IDs or a list of NIDs per PLMN Identity. Further information regarding how to set the IE is specified in TS 23.003 [21].

NPN-Identity information element

-- ASN1START

-- TAG-NPN-IDENTITY-START

NPN-Identity-r16 ::=             CHOICE {

pni-npn-r16                      SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

cag-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16

},

snpn-r16                         SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

nid-List-r16                     SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

}

CAG-IdentityInfo-r16 ::=         SEQUENCE {

cag-Identity-r16                 BIT STRING (SIZE (32)),

manualCAGselectionAllowed-r16    ENUMERATED {true}                         OPTIONAL   -- Need R

}

NID-r16 ::=                      BIT STRING (SIZE (44))

-- TAG-NPN-IDENTITY-STOP

-- ASN1STOP

NPN-Identity field descriptions

cag-Identity A CAG-ID as specified in TS 23.003 [21]. The PLMN ID and a CAG ID in the NPN-Identity identifies a PNI-NPN.

cag-IdentityList The cag-IdentityList contains one or more CAG IDs. All CAG IDs associated to the same PLMN ID are listed in the same cag-IdentityList entry.

manualCAGselectionAllowed The manualCAGselectionAllowed indicates that the CAG ID can be selected manually even if it is outside the UE's allowed CAG list.

NID A NID as specified in TS 23.003 [21]. The PLMN ID and a NID in the NPN-Identity identifies a SNPN.

nid-List The nid-List contains one or more NID.

#### –	NPN-IdentityInfoList

The IE NPN-IdentityInfoList includes a list of NPN identity information.

NPN-IdentityInfoList information element

-- ASN1START

-- TAG-NPN-IDENTITYINFOLIST-START

NPN-IdentityInfoList-r16 ::=     SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16

NPN-IdentityInfo-r16 ::=         SEQUENCE {

npn-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16,

trackingAreaCode-r16             TrackingAreaCode,

ranac-r16                        RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity-r16                 CellIdentity,

cellReservedForOperatorUse-r16   ENUMERATED {reserved, notReserved},

iab-Support-r16                  ENUMERATED {true}                                           OPTIONAL,       -- Need S

...,

[[

gNB-ID-Length-r17                INTEGER (22..32)                                            OPTIONAL        -- Need R

]],

[[

mobileIAB-Support-r18            ENUMERATED {true}                                           OPTIONAL        -- Need S

]]

}

-- TAG-NPN-IDENTITYINFOLIST-STOP

-- ASN1STOP

NPN-IdentityInfoList field descriptions

iab-Support This field combines both the support of IAB and the cell status for IAB. If the field is present, the cell supports IAB and the cell is also considered as a candidate for cell (re)selection for IAB-nodes; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell.

gNB-ID-Length Indicates the length of the gNB ID out of the 36-bit long cellIdentity. This field is always present if the mobileIAB-Support is broadcasted.

mobileIAB-Support This field indicates the support of mobile IAB and whether the cell can be considered as a candidate for cell (re)selection for mobile IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell. If the field is absent, the cell is barred for mobile IAB-node.

NPN-IdentityInfo The NPN-IdentityInfo contains one or more NPN identities and additional information associated with those NPNs. Only the same type of NPNs (either SNPNs or PNI-NPNs) can be listed in a NPN-IdentityInfo element.

npn-IdentityList The npn-IdentityList contains one or more NPN Identity elements.

trackingAreaCode Indicates the Tracking Area Code to which the cell indicated by cellIdentity field belongs.

ranac Indicates the RAN Area Code to which the cell indicated by cellIdentity field belongs.

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (for the NPN(s) identified in the npn-IdentityList) as defined in TS 38.304 [20]. This field is ignored by NPN capable IAB-MT and NPN capable NCR-MT.

#### –	NR-DL-PRS-PDC-Info

The IE NR-DL-PRS-PDC-Info defines downlink PRS configuration for PDC.

NR-DL-PRS-PDC-Info information element

-- ASN1START

-- TAG-NR-DL-PRS-PDC-INFO-START

NR-DL-PRS-PDC-Info-r17 ::=    SEQUENCE {

nr-DL-PRS-PDC-ResourceSet-r17          NR-DL-PRS-PDC-ResourceSet-r17                    OPTIONAL, -- Need R

...

}

NR-DL-PRS-PDC-ResourceSet-r17 ::=    SEQUENCE {

periodicityAndOffset-r17     NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17,

numSymbols-r17               ENUMERATED {n2, n4, n6, n12, n1-v1800, spare3, spare2, spare1},

dl-PRS-ResourceBandwidth-r17        INTEGER (1..63),

dl-PRS-StartPRB-r17                 INTEGER (0..2176),

resourceList-r17             SEQUENCE (SIZE (1..maxNrofPRS-ResourcesPerSet-r17)) OF NR-DL-PRS-Resource-r17,

repFactorAndTimeGap-r17      RepFactorAndTimeGap-r17                                    OPTIONAL,   -- Need S

...

}

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17 ::= CHOICE {

scs15-r17       CHOICE {

n4-r17                  INTEGER (0..3),

n5-r17                  INTEGER (0..4),

n8-r17                  INTEGER (0..7),

n10-r17                 INTEGER (0..9),

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n160-r17                INTEGER (0..159),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

...

},

scs30-r17       CHOICE {

n8-r17                  INTEGER (0..7),

n10-r17                 INTEGER (0..9),

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

...

},

scs60-r17       CHOICE {

n16-r17                 INTEGER (0..15),

n20-r17                 INTEGER (0..19),

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n256-r17                INTEGER (0..255),

n320-r17                INTEGER (0..319),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

n40960-r17              INTEGER (0..40959),

...

},

scs120-r17      CHOICE {

n32-r17                 INTEGER (0..31),

n40-r17                 INTEGER (0..39),

n64-r17                 INTEGER (0..63),

n80-r17                 INTEGER (0..79),

n128-r17                INTEGER (0..127),

n160-r17                INTEGER (0..159),

n256-r17                INTEGER (0..255),

n320-r17                INTEGER (0..319),

n512-r17                INTEGER (0..511),

n640-r17                INTEGER (0..639),

n1280-r17               INTEGER (0..1279),

n2560-r17               INTEGER (0..2559),

n5120-r17               INTEGER (0..5119),

n10240-r17              INTEGER (0..10239),

n20480-r17              INTEGER (0..20479),

n40960-r17              INTEGER (0..40959),

n81920-r17              INTEGER (0..81919),

...

},

...

}

NR-DL-PRS-Resource-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17            NR-DL-PRS-ResourceID-r17,

dl-PRS-SequenceID-r17               INTEGER (0..4095),

dl-PRS-CombSizeN-AndReOffset-r17    CHOICE {

n2-r17                          INTEGER (0..1),

n4-r17                          INTEGER (0..3),

n6-r17                          INTEGER (0..5),

n12-r17                         INTEGER (0..11),

...

},

dl-PRS-ResourceSlotOffset-r17       INTEGER (0..maxNrofPRS-ResourceOffsetValue-1-r17),

dl-PRS-ResourceSymbolOffset-r17     INTEGER (0..12),

dl-PRS-QCL-Info-r17                 DL-PRS-QCL-Info-r17                         OPTIONAL, -- Need N

...,

[[

dl-PRS-ResourceSymbolOffset-v1800   INTEGER (13)                                OPTIONAL  -- Need R

]]

}

DL-PRS-QCL-Info-r17 ::= CHOICE {

ssb-r17                     SEQUENCE {

ssb-Index-r17                   INTEGER (0..63),

rs-Type-r17                     ENUMERATED {typeC, typeD, typeC-plus-typeD},

...

},

dl-PRS-r17                  SEQUENCE {

qcl-DL-PRS-ResourceID-r17       NR-DL-PRS-ResourceID-r17,

...

},

...

}

NR-DL-PRS-ResourceID-r17 ::= INTEGER (0..maxNrofPRS-ResourcesPerSet-1-r17)

RepFactorAndTimeGap-r17 ::=  SEQUENCE {

repetitionFactor-r17         ENUMERATED {n2, n4, n6, n8, n16, n32, spare2, spare1},

timeGap-r17                  ENUMERATED {s1, s2, s4, s8, s16, s32, spare2, spare1}

}

-- TAG-NR-DL-PRS-PDC-INFO-STOP

-- ASN1STOP

NR-DL-PRS-PDC-ResourceSet field descriptions

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for all the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs in this resource set. All DL-PRS Resources of the DL-PRS-PDC Resource Set have the same bandwidth. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to subcarrier 0 in common resource block 0 for the DL-PRS Resource. All DL-PRS Resources of the DL-PRS-PDC Resource Set have the same value of dl-PRS-StartPRB.

numSymbols This field specifies the number of symbols per DL-PRS Resource within a slot. The UE does not expect to be configured for PDC with a PRS with numSymbols equals to n1 unless an SSB index is provided as a typeC or typeC-plus-typeD QCL source, or another PRS resource with numSymbols more than n1 is provided as QCL source.

periodicityAndOffset This field specifies the periodicity of DL-PRS allocation in slots and the slot offset with respect to SFN #0 slot #0 in the PCell where the DL-PRS-PDC Resource Set is configured (i.e., slot where the first DL-PRS Resource of DL-PRS-PDC Resource Set occurs).

repFactorAndTimeGap If this field is absent, the value for repetitionFactor is 1 (i.e., no resource repetition).

RepFactorAndTimeGap field descriptions

repetitionFactor This field specifies how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of the DL-PRS Resource Set. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

timeGap This field specifies the offset in units of slots between two repeated instances of a DL-PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL-PRS Resource Set. The time duration spanned by one DL-PRS Resource Set containing repeated DL-PRS Resources should not exceed the periodicity configured by periodicityAndOffset.

NR-DL-PRS-Resource field description

dl-PRS-ResourceSymbolOffset This field specifies the starting symbol of the DL-PRS Resource within a slot. If dl-PRS-ResourceSymbolOffset-v1800 is present, the UE shall ignore dl-PRS-ResourceSymbolOffset-r17.

#### –	NR-NS-PmaxList

The IE NR-NS-PmaxList is used to configure a list of additionalPmax and additionalSpectrumEmission, as defined in TS 38.101-1 [15], table 6.2.3.1-1A, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A for a given frequency band.

NR-NS-PmaxList information element

-- ASN1START

-- TAG-NR-NS-PMAXLIST-START

NR-NS-PmaxList ::=                      SEQUENCE (SIZE (1..maxNR-NS-Pmax)) OF NR-NS-PmaxValue

NR-NS-PmaxValue ::=                     SEQUENCE {

additionalPmax                          P-Max                               OPTIONAL,   -- Need N

additionalSpectrumEmission              AdditionalSpectrumEmission

}

NR-NS-PmaxList-v1760 ::=                SEQUENCE (SIZE (1.. maxNR-NS-Pmax)) OF NR-NS-PmaxValue-v1760

NR-NS-PmaxValue-v1760 ::=               SEQUENCE {

additionalSpectrumEmission-v1760        AdditionalSpectrumEmission-v1760    OPTIONAL   -- Need N

}

NR-NS-PmaxListAerial-r18 ::=            SEQUENCE (SIZE (1..maxNR-NS-Pmax)) OF NR-NS-PmaxValueAerial-r18

NR-NS-PmaxValueAerial-r18 ::=           SEQUENCE {

additionalPmax-r18                      P-Max                               OPTIONAL,   -- Need N

additionalSpectrumEmission-r18          AdditionalSpectrumEmission-r18

}

-- TAG-NR-NS-PMAXLIST-STOP

-- ASN1STOP

#### –	NSAG-ID

The IE NSAG-ID is used to identify an NSAG (TS 23.501 [32]) for slice-based cell reselection or slice-based random access.

NSAG-ID information element

-- ASN1START

-- TAG-NSAG-ID-START

NSAG-ID-r17 ::=                      BIT STRING (SIZE (8))

-- TAG-NSAG-ID-STOP

-- ASN1STOP

#### –	NSAG-IdentityInfo

The IE NSAG-IdentityInfo is used to identify an NSAG (TS 23.501 [32]) for slice-based cell reselection.

NSAG-IdentityInfo information element

-- ASN1START

-- TAG-NSAG-IDENTITYINFO-START

NSAG-IdentityInfo-r17 ::=            SEQUENCE {

nsag-ID-r17                          NSAG-ID-r17,

trackingAreaCode-r17                 TrackingAreaCode               OPTIONAL      -- Need R

}

-- TAG-NSAG-IDENTITYINFO-STOP

-- ASN1STOP

NSAG-IdentityInfo field descriptions

trackingAreaCode If absent, UE assumes the trackingAreaCode of the serving cell.

#### –	NTN-Config

The IE NTN-Config provides parameters needed for the UE to access NR via NTN access.

NTN-Config information element

-- ASN1START

-- TAG-NTN-CONFIG-START

NTN-Config-r17 ::=             SEQUENCE {

epochTime-r17                  EpochTime-r17                                                            OPTIONAL,  -- Need R

ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35,

s40, s45, s50, s55, s60, s120, s180, s240, s900}              OPTIONAL,  -- Cond SIB19

cellSpecificKoffset-r17        INTEGER(1..1023)                                                         OPTIONAL,  -- Need R

kmac-r17                       INTEGER(1..512)                                                          OPTIONAL,  -- Need R

ta-Info-r17                    TA-Info-r17                                                              OPTIONAL,  -- Need R

ntn-PolarizationDL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need R

ntn-PolarizationUL-r17         ENUMERATED {rhcp,lhcp,linear}                                            OPTIONAL,  -- Need S

ephemerisInfo-r17              EphemerisInfo-r17                                                        OPTIONAL,  -- Need R

ta-Report-r17                  ENUMERATED {enabled}                                                     OPTIONAL,  -- Need R

...

}

TA-Info-r17 ::=                 SEQUENCE  {

ta-Common-r17                  INTEGER(0..66485757),

ta-CommonDrift-r17             INTEGER(-257303..257303)                                                 OPTIONAL,  -- Need R

ta-CommonDriftVariant-r17      INTEGER(0..28949)                                                        OPTIONAL   -- Need R

}

-- TAG-NTN-CONFIG-STOP

-- ASN1STOP

NTN-Config field descriptions

ephemerisInfo This field provides satellite ephemeris either in format of position and velocity state vector or in format of orbital parameters. This field is excluded when determining changes in system information, i.e. changes to ephemerisInfo should neither result in system information change notifications nor in a modification of valueTag in SIB1.

epochTime If this field is absent for the NTN serving cell, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when ntn-Config is provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync in an NTN cell, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. If this field is absent in ntn-Config provided via NTN-NeighCellConfig in a TN cell, the epoch time is the end of SI window where this SIB19 is scheduled. In case of satellite switch with resynchronization, this field is based on the timing of the cell served by the source satellite. This field is excluded when determining changes in system information, i.e. changes to epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

cellSpecificKoffset Scheduling offset used for the timing relationships that are modified for NTN (see TS 38.213 [13]). The unit of the field K_offset is number of slots for a given subcarrier spacing of 15 kHz. If the field is absent UE assumes value 0.

kmac Scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. If the field is absent UE assumes value 0. The unit of kmac is number of slots for a given subcarrier spacing of 15 kHz.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates Polarization information for uplink service link. If not present and ntn-PolarizationDL is present, UE assumes the same polarization for UL and DL. If both ntn-PolarizationUL and ntn-PolarizationDL are not present, the UE releases any stored value.

ntn-UlSyncValidityDuration A validity duration configured by the network for assistance information (i.e. Serving and/or neighbour satellite ephemeris and Common TA parameters) which indicates the maximum time duration (from epochTime) during which the UE can apply assistance information without having acquired new assistance information. The unit of ntn-UlSyncValidityDuration is second. Value s5 corresponds to 5 s, value s10 indicate 10 s and so on. If this field is absent in ntn-Config provided via NTN-NeighCellConfig or SatSwitchWithReSync in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in ntn-Config provided via NTN-NeighCellConfig in a TN cell, how the UE sets the validity duration is left to UE implementation. This field is excluded when determining changes in system information, i.e. changes of ntn-UlSyncValidityDuration should neither result in system information change notifications nor in a modification of valueTag in SIB1. ntn-UlSyncValidityDuration is only updated when at least one of epochTime, ta-Info, ephemerisInfo is updated.

ta-Common Network-controlled common timing advanced value and it may include any timing offset considered necessary by the network. ta-Common with value of 0 is supported. The granularity of ta-Common is 4.072 × 10^(-3) μs. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-Common should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDrift Indicate drift rate of the common TA. The granularity of ta-CommonDrift is 0.2 × 10^(-3) μs⁄s. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDrift should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-CommonDriftVariant Indicate drift rate variation of the common TA. The granularity of ta-CommonDriftVariant is 0.2×10^(-4) μs⁄s^2. Values are given in unit of corresponding granularity. This field is excluded when determining changes in system information, i.e. changes of ta-CommonDriftVariant should neither result in system information change notifications nor in a modification of valueTag in SIB1.

ta-Report When this field is included in SIB19, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

| Conditional Presence | Explanation |
| --- | --- |
| SIB19 | The field is mandatory present for the serving cell in SIB19. The field is optionally present, Need R, otherwise. |

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS transmitted in the cell where the IE is included, which the UE may be configured to measure on (see TS 38.214 [19], clause 5.2.2.3.1). A change of configuration between periodic, semi-persistent or aperiodic for an NZP-CSI-RS-Resource is not supported without a release and add.

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]],

[[

additionalOneSlotOffset-r19         ENUMERATED{enabled}                             OPTIONAL,   -- Need R

additionalSlotOffset-r19            INTEGER(0..7)                                   OPTIONAL    -- Need R

]]

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

NZP-CSI-RS-Resource field descriptions

additionalOneSlotOffset See TS 38.214 [19], clause 5.2.2.3.1. This field is only configured for codebook typeI-SinglePanel-r19, typeI-MultiPanel-r19, etypeII-r19 and typeII-FePortSelection-r19.

additionalSlotOffset See TS 38.214 [19], clause 5.2.2.3.1. The value 0 corresponds to 0 slots, value 1 corresponds to 1 slot and so on.

periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots (see TS 38.214 [19], clause 5.2.2.3.1). Network always configures the UE with a value for this field for periodic and semi-persistent NZP-CSI-RS-Resource (as indicated in CSI-ResourceConfig).

powerControlOffset Power offset of PDSCH EPRE to NZP CSI-RS EPRE. Value in dB (see TS 38.214 [19], clauses 5.2.2.3.1 and 4.1). The UE shall ignore this field in case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE.

powerControlOffsetSS Power offset of NZP CSI-RS EPRE to SS/PBCH block EPRE. Value in dB (see TS 38.214 [19], clause 5.2.2.3.1).

qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State or dl-OrJoint-TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1). In case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE, it refers to the TCI state identifier in CandidateTCI-State and is defined in ltm-DL-OrJointTCI-StateToAddModList within the LTM-Candidate IE.

resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource.

scramblingID Scrambling ID (see TS 38.214 [19], clause 5.2.2.3.1).

| Conditional Presence | Explanation |
| --- | --- |
| LTM | The field is optionally present, Need R, in an LTM-Candidate IE. Otherwise, the field is absent. |
| Periodic | The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::=           INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                  OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                           OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}                                                       OPTIONAL,   -- Need R

...,

[[

aperiodicTriggeringOffset-r16       INTEGER(0..31)                                                          OPTIONAL   -- Need S

]],

[[

pdc-Info-r17                        ENUMERATED {true}                                                       OPTIONAL,  -- Need R

cmrGroupingAndPairing-r17           CMRGroupingAndPairing-r17                                               OPTIONAL,  -- Need R

aperiodicTriggeringOffset-r17       INTEGER (0..124)                                                        OPTIONAL,  -- Need S

aperiodicTriggeringOffsetL2-r17     INTEGER(0..31)                                                          OPTIONAL   -- Need R

]],

[[

resourceType-r18                    ENUMERATED {periodic}                                                   OPTIONAL   -- Cond LTM

]],

[[

kdopp-r19                           SEQUENCE {

numberOfResourceGroups-r19          ENUMERATED {n4, n8, n12},

numberOfResourcesPerGroup-r19       ENUMERATED {n2, n3, n4}

}                                                                                                           OPTIONAL,  -- Need R

additionalOneSlotOffsetDoppler-r19  CHOICE {

resourceGroup4                      BIT STRING(SIZE(4)),

resourceGroup8                      BIT STRING(SIZE(8)),

resourceGroup12                     BIT STRING(SIZE(12))

}                                                                                                           OPTIONAL   -- Need R

]]

}

CMRGroupingAndPairing-r17 ::=        SEQUENCE {

nrofResourcesGroup1-r17              INTEGER (1..7),

pair1OfNZP-CSI-RS-r17                NZP-CSI-RS-Pairing-r17                                                 OPTIONAL,  -- Need R

pair2OfNZP-CSI-RS-r17                NZP-CSI-RS-Pairing-r17                                                 OPTIONAL   -- Need R

}

NZP-CSI-RS-Pairing-r17  ::=          SEQUENCE {

nzp-CSI-RS-ResourceId1-r17           INTEGER (1..7),

nzp-CSI-RS-ResourceId2-r17           INTEGER (1..7)

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

NZP-CSI-RS-ResourceSet field descriptions

additionalOneSlotOffsetDoppler See TS 38.214 [19], clause 5.2.2.3.1. This field is only configured for codebook typeII-Doppler-r19.

aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16, aperiodicTriggeringOffset-r17 Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. For aperiodicTriggeringOffset, the value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. For aperiodicTriggeringOffset-r16 and aperiodicTriggeringOffset-r17, the value indicates the number of slots. aperiodicTriggeringOffset-r17 is applicable to SCS 480 kHz and 960 kHz, and only the values of integer multiples of 4 are valid, i.e. 0, 4, 8, and so on. The network configures only one of the fields. When neither field is included, the UE applies the value 0. This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

aperiodicTriggeringOffsetL2 Indicates triggering offset of aperiodic NZP CSI-RS resources used for fast activation of the SCell (see clause 5.2.1.5.3 of TS 38.214 [19]), when the NZP CSI-RS resources are activated by the MAC CE (see clause 5.9 of TS 38.321 [3]). The value indicates the number of slots. This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

cmrGroupingAndPairing Configures CMR groups and pairs. The first nrofResourcesGroup1 resources in the NZP-CSI-RS resource set belong to Group 1 and the remaining resources in the NZP-CSI-RS resource set belong to Group 2. nrofResourcesGroup1 is K1 and the number of remaining resources in the NZP-CSI-RS resource set belonging to Group 2 is K2 as specified in TS 38.214 clause 5.2.1.4.1. Maximum total number in Group 1 and Group 2 is 8 (see TS 38.214 [19], clauses 5.2.1.4.1 and 5.2.1.4.2). This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

kdopp The number of configured resource groups and number of NZP CSI-RS resources in each group. For numberOfResourceGroups, value n4 corresponds to 4 resource groups, value n8 corresponds to 8 resource groups and value n12 corresponds to 12 resource groups. For numberOfResourcesPerGroup, value n2 corresponds to 2 NZP CSI-RS resources per group, value n3 corresponds to 3 NZP CSI-RS resources per group and value n4 corresponds to 4 NZP CSI-RS resources per group. This field is only configured for codebook typeII-Doppler-r19.

pair1OfNZP-CSI-RS, pair2OfNZP-CSI-RS A pair of NZP CSI-RS resources. In one pair, one resource shall belong to group 1 and the other resource shall belong to group 2 (see TS 38.214 [19], clause 5.2.1.4.1).

nzp-CSI-RS-Resources NZP-CSI-RS-Resources associated with this NZP-CSI-RS resource set (see TS 38.214 [19], clause 5.2). If the NZP-CSI-RS-ResourceSet is indicated in a CSI-ResourceConfig indicated by resourcesForChannelMeasurement in a CSI-ReportConfig, if the codebookType in the codebookConfig in the CSI-ReportConfig is typeII-Doppler-r18 or typeII-DopplerPortSelection-r18, there are at most 1 periodic or semi-persistent NZP CSI-RS resource and 4, 8 or 12 aperiodic NZP CSI-RS resources and if the codebookType is typeII-CJT-r18 or typeII-CJT-PortSelection-r18, there are 1, 2, 3, or 4 aperiodic, periodic or semi-persistent NZP-CSI-RS resources, see TS 38.214 [19], clause 5.2.1.4.

nzp-CSI-RS-ResourceId1, nzp-CSI-RS-ResourceId2 The nzp-CSI-RS-ResourceId1-r17 represents the index of the NZP CSI-RS resource in Resource Group 1, and nzp-CSI-RS-ResourceId2-r17 represents the index of the NZP CSI-RS resource in Resource Group 2.

pdc-Info Indicates that this NZP-CSI-RS-ResourceSet, if configured also with trs-Info, is used for propagation delay compensation. The field can be present only if trs-info is present. The field can be present in only one NZP-CSI-RS-ResourceSet. If network configures this field for an NZP-CSI-RS-ResourceSet, the UE measures the UE Rx-Tx time difference based on resources configured in this resource set. This field is not present in case NZP-CSI-RS-ResourcesSet is received as part of an LTM-Candidate IE.

repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter (see TS 38.214 [19], clauses 5.2.2.3.1 and 5.1.6.1.2). It can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP, L1 SINR or "no report". If NZP-CSI-RS-ResourceSet if configured in LTM-Candidate, the field can only be configured as off, if present.

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2).

trs-Info Indicates that the antenna port for all NZP-CSI-RS resources in the CSI-RS resource set is same. If the field is absent or released the UE applies the value false (see TS 38.214 [19], clause 5.2.2.3.1).

| Conditional Presence | Explanation |
| --- | --- |
| LTM | The field is optionally present, Need R, in an LTM-TCI-Info IE. Otherwise, the field is absent. |

#### –	NZP-CSI-RS-ResourceSetId

The IE NZP-CSI-RS-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESETID-START

NZP-CSI-RS-ResourceSetId ::=        INTEGER (0..maxNrofNZP-CSI-RS-ResourceSets-1)

-- TAG-NZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by od-ssb-ActivationStatus or by a MAC CE see TS 38.321 [3], clause 6.1.3.88.

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19           OD-SSB-ConfigId-r19,

od-ssb-SFN-Offset-r19         INTEGER (0..15)                                                           OPTIONAL, -- Need R

od-ssb-halfFrameIndex-r19     ENUMERATED {zero, one}                                                    OPTIONAL, -- Need R

od-ssb-ActivationStatus-r19   ENUMERATED {activated}                                                    OPTIONAL, -- Need S

od-ssb-Periodicity-r19        ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

od-ssb-PositionsInBurst-r19   CHOICE {

shortBitmap                   BIT STRING (SIZE (4)),

mediumBitmap                  BIT STRING (SIZE (8)),

longBitmap                    BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond ODssbAOssb

od-ssb-NrofBursts-r19         ENUMERATED {n5, n10, n15, n20, n25, n30, n40, n50, n75, n100, n150, n200} OPTIONAL, -- Cond MACCE

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

OD-SSB-Config field descriptions

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated at one point of time.

od-ssb-halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

od-ssb-NrofBursts Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated. Network only configures this field when od-ssb-ActivationStatus is absent. For FR1, the value range is {5, 10, 15, 20, 25, 30, 40, 50}. For FR2, the value range is {25, 30, 40, 50, 75, 100, 150, 200}.

od-ssb-Periodicity The SSB periodicity in ms.

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, UE applies the value ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ssb-SFN-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0, where SFN refers to the SFN of this serving cell. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

| Conditional Presence | Explanation |
| --- | --- |
| ODssbAOssb | The field is mandatory present, when absoluteFrequencySSB of the serving cell is absent. The field is absent when absoluteFrequencySSB of the serving cell is present and od-ssb-absoluteFrequency is absent.It is optionally present, Need R, otherwise. |
| MACCE | The field is optionally present, Need R, when od-ssb-ActivationStatus is absent. It is absent otherwise. |

#### –	P-Max

The IE P-Max is used to limit the UE's uplink transmission power on a carrier frequency, in TS 38.101-1 [15] and in TS 38.101-5 [75], and is used to calculate the parameter Pcompensation defined in TS 38.304 [20]. In ATG cell, actual value of P-Max = 9 + field value [dBm].

P-Max information element

-- ASN1START

-- TAG-P-MAX-START

P-Max ::=                           INTEGER (-30..33)

-- TAG-P-MAX-STOP

-- ASN1STOP

#### –	PathlossReferenceRS

The IE PathlossReferenceRS is used to configure a Reference Signal (e.g. a CSI-RS config or a SS block) to be used for path loss estimation for PUSCH, PUCCH and SRS for unified TCI state operation.

PathlossReferenceRS information element

-- ASN1START

-- TAG-PATHLOSSREFERENCERS-START

PathlossReferenceRS-r17 ::=   SEQUENCE {

pathlossReferenceRS-Id-r17          PathlossReferenceRS-Id-r17,

referenceSignal-r17                 CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

additionalPCI-r17                   AdditionalPCIIndex-r17                                         OPTIONAL  -- Cond RS-SSB

}

-- TAG-PATHLOSSREFERENCERS-STOP

-- ASN1STOP

PathlossReferenceRS field descriptions

additionalPCI Indicates the physical cell ID (PCI) of the SSB for the referenceSignal.

| Conditional Presence | Explanation |
| --- | --- |
| RS-SSB | The field is optionally present, Need R, if ssb-Index is configured for referenceSignal. Otherwise it is absent, Need R. |

#### –	PathlossReferenceRS-Id

The IE PathlossReferenceRS-Id is an ID for a reference signal (RS) configured as PUSCH, PUCCH and SRS pathloss reference RS for unified TCI state operation.

PathlossReferenceRS-Id information element

-- ASN1START

-- TAG-PATHLOSSREFERENCERS-ID-START

PathlossReferenceRS-Id-r17 ::= INTEGER (0..maxNrofPathlossReferenceRSs-1-r17)

-- TAG-PATHLOSSREFERENCERS-ID-STOP

-- ASN1STOP

#### –	PCI-ARFCN-EUTRA

The IE PCI-ARFCN-EUTRA is used to encode EUTRA PCI and ARFCN.

PCI-ARFCN-EUTRA information element

-- ASN1START

-- TAG-PCIARFCNEUTRA-START

PCI-ARFCN-EUTRA-r16 ::=             SEQUENCE {

physCellId-r16                      EUTRA-PhysCellId,

carrierFreq-r16                     ARFCN-ValueEUTRA

}

-- TAG-PCIARFCNEUTRA-STOP

-- ASN1STOP

#### –	PCI-ARFCN-NR

The IE PCI-ARFCN-NR is used to encode NR PCI and ARFCN.

PCI-ARFCN-NR information element

-- ASN1START

-- TAG-PCIARFCNNR-START

PCI-ARFCN-NR-r16 ::=                SEQUENCE {

physCellId-r16                      PhysCellId,

carrierFreq-r16                     ARFCN-ValueNR

}

-- TAG-PCIARFCNNR-STOP

-- ASN1STOP

#### –	PCI-List

The IE PCI-List concerns a list of physical cell identities, which may be used for different purposes.

PCI-List information element

-- ASN1START

-- TAG-PCI-LIST-START

PCI-List ::=                        SEQUENCE (SIZE (1..maxNrofCellMeas)) OF PhysCellId

-- TAG-PCI-LIST-STOP

-- ASN1STOP

#### –	PCI-Range

The IE PCI-Range is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PCI-Range, the Network may configure overlapping ranges of physical cell identities.

PCI-Range information element

-- ASN1START

-- TAG-PCI-RANGE-START

PCI-Range ::=                       SEQUENCE {

start                               PhysCellId,

range                               ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008,spare1}                  OPTIONAL    -- Need S

}

-- TAG-PCI-RANGE-STOP

-- ASN1STOP

PCI-Range field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, value n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

#### –	PCI-RangeElement

The IE PCI-RangeElement is used to define a PCI-Range as part of a list (e.g. AddMod list).

PCI-RangeElement information element

-- ASN1START

-- TAG-PCI-RANGEELEMENT-START

PCI-RangeElement ::=                SEQUENCE {

pci-RangeIndex                      PCI-RangeIndex,

pci-Range                           PCI-Range

}

-- TAG-PCI-RANGEELEMENT-STOP

-- ASN1STOP

PCI-RangeElement field descriptions

pci-Range Physical cell identity or a range of physical cell identities.

#### –	PCI-RangeIndex

The IE PCI-RangeIndex identifies a physical cell id range, which may be used for different purposes.

PCI-RangeIndex information element

-- ASN1START

-- TAG-PCI-RANGEINDEX-START

PCI-RangeIndex ::=                  INTEGER (1..maxNrofPCI-Ranges)

-- TAG-PCI-RANGEINDEX-STOP

-- ASN1STOP

#### –	PCI-RangeIndexList

The IE PCI-RangeIndexList concerns a list of indexes of physical cell id ranges, which may be used for different purposes.

PCI-RangeIndexList information element

-- ASN1START

-- TAG-PCI-RANGEINDEXLIST-START

PCI-RangeIndexList ::=              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeIndex

-- TAG-PCI-RANGEINDEXLIST-STOP

-- ASN1STOP

#### –	PDCCH-Config

The IE PDCCH-Config is used to configure UE specific PDCCH parameters or MBS multicast PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH. If this IE is used for the scheduled SCell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpacesToReleaseList are absent. If the IE is used for a dormant BWP, the fields other than controlResourceSetToAddModList and controlResourceSetToReleaseList are absent. If this IE is used for MBS CFR, the field downlinkPreemptiom,tpc-PUSCH, tpc-SRS, uplinkCancellation, monitoringCapabilityConfig, and searchSpaceSwitchConfig are absent.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                      OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId                    OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                            OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                          OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                              OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                            OPTIONAL,   -- Need M

...,

[[

controlResourceSetToAddModListSizeExt-v1610 SEQUENCE (SIZE (1..2)) OF ControlResourceSet             OPTIONAL,   -- Need N

controlResourceSetToReleaseListSizeExt-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16        OPTIONAL,   -- Need N

searchSpacesToAddModListExt-r16     SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16                     OPTIONAL,   -- Need N

uplinkCancellation-r16              SetupRelease { UplinkCancellation-r16 }                          OPTIONAL,   -- Need M

monitoringCapabilityConfig-r16      ENUMERATED { r15monitoringcapability,r16monitoringcapability }   OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r16         SearchSpaceSwitchConfig-r16                                      OPTIONAL    -- Need R

]],

[[

searchSpacesToAddModListExt-v1700   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700                   OPTIONAL,   -- Need N

monitoringCapabilityConfig-v1710    ENUMERATED { r17monitoringcapability }                           OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r17         SearchSpaceSwitchConfig-r17                                      OPTIONAL,   -- Need R

pdcch-SkippingDurationList-r17      SEQUENCE(SIZE (1..3)) OF SCS-SpecificDuration-r17                OPTIONAL    -- Need R

]],

[[

pdcch-MonitoringResumptionAfterNack-r18 ENUMERATED {true}                                            OPTIONAL,   -- Need R

searchSpacesToAddModListExt-v1800   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1800                   OPTIONAL    -- Need N

]],

[[

searchSpaceSwitchConfig-r19         SearchSpaceSwitchConfig-r19                                      OPTIONAL    -- Need R

]]

}

SearchSpaceSwitchConfig-r16 ::=     SEQUENCE {

cellGroupsForSwitchList-r16         SEQUENCE(SIZE (1..4)) OF CellGroupForSwitch-r16                  OPTIONAL,   -- Need R

searchSpaceSwitchDelay-r16          INTEGER (10..52)                                                 OPTIONAL    -- Need R

}

SearchSpaceSwitchConfig-r17 ::=     SEQUENCE {

searchSpaceSwitchTimer-r17          SCS-SpecificDuration-r17                                         OPTIONAL,   -- Need R

searchSpaceSwitchDelay-r17          INTEGER (10..52)                                                 OPTIONAL    -- Need R

}

SearchSpaceSwitchConfig-r19 ::=     SEQUENCE {

sr-TriggeredSearchSpaceSwitchGroupId-r19 INTEGER (0.. maxNrofSearchSpaceGroups-1-r17)                OPTIONAL    -- Need R

}

CellGroupForSwitch-r16 ::=          SEQUENCE(SIZE (1..16)) OF ServCellIndex

SCS-SpecificDuration-r17   ::=      INTEGER (1..166)

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

PDCCH-Config field descriptions

controlResourceSetToAddModList, controlResourceSetToAddModListSizeExt List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE. The network restrictions on configuration of CORESETs per DL BWP are specified in TS 38.213 [13], clause 10.1 and TS 38.306 [26]. The UE shall consider entries in controlResourceSetToAddModList and in controlResourceSetToAddModListSizeExt as a single list, i.e. an entry created using controlResourceSetToAddModList can be modified using controlResourceSetToAddModListSizeExt (or deleted using controlResourceSetToReleaseListSizeExt) and vice-versa. In case network reconfigures control resource set with the same ControlResourceSetId as used for commonControlResourceSet or commonControlResourceSetExt configured via PDCCH-ConfigCommon or via SIB20, the configuration from PDCCH-Config always takes precedence and should not be updated by the UE based on servingCellConfigCommon or based on SIB20.

controlResourceSetToReleaseList, controlResourceSetToReleaseListSizeExt List of UE specifically configured Control Resource Sets (CORESETs) to be released by the UE. This field only applies to CORESETs configured by controlResourceSetToAddModList or controlResourceSetToAddModListSizeExt and does not release the field commonControlResourceSet configured by PDCCH-ConfigCommon and commonControlResourceSetExt configured by SIB20.

downlinkPreemption Configuration of downlink preemption indications to be monitored in this cell (see TS 38.213 [13], clause 11.2).

monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability, Rel-16 PDCCH monitoring capability or Rel-17 PDCCH monitoring capability for PDCCH monitoring on a serving cell (see TS 38.213 [13], clause 10.1). Value r15monitoringcapability enables the Rel-15 monitoring capability, and value r16monitoringcapability enables the Rel-16 PDCCH monitoring capability. r17monitoringcapability enables the Rel-17 PDCCH multi-slot monitoring capability. For 480 and 960 kHz SCS, only value r17monitoringcapability is applicable.

pdcch-MonitoringResumptionAfterNack This field is used to enable UE PDCCH monitoring resumption after PDCCH skipping has started, if the UE transmits a NACK due to incorrectly decoding a PDSCH scheduled by a DCI format received from the serving cell (see TS 38.213 [13], clause 10.4).

pdcch-SkippingDurationList Provides one or more values to derive the skipping duration in unit of slots, as specified in TS 38.213 [13], clause 10.4. The DCI which schedules data indicates which of the values is to be applied (see TS 38.213 [13], clause 10.4). For the 15kHz SCS, for each entry, only the first 26 values are valid and correspond to {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100}. For the 30kHz SCS, for each entry, only the first 46 values are valid and correspond to {1, 2, 3, …, 40, 60, 80, 100, 120, 160, 200}. For the 60kHz SCS, for each entry, only the first 86 values are valid and correspond to {1, 2, 3, …, 80, 120, 160, 200, 240, 320, 400}. For the 120kHz SCS, for each entry, the 166 values correspond to {1, 2, 3, …, 160, 240, 320, 400, 480, 640, 800}. For the 480kHz SCS, for each entry, the 166 values correspond to {4, 8, 12, …, 640, 960, 1280, 1600, 1920, 2560, 3200}. For the 960kHz SCS, for each entry, the 166 values correspond to {8, 16, 24, …, 1280, 1920, 2560, 3200, 3840, 5120, 6400}.

searchSpacesToAddModList, searchSpacesToAddModListExt List of UE specifically configured Search Spaces or MBS multicast Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces). If the network includes searchSpacesToAddModListExt, it includes the same number of entries, and listed in the same order, as in searchSpacesToAddModList in each of them.

searchSpaceSwitchConfig Configuration to control the UE behavior to switch from search space group X back to search space group 0, as specified in clause 10 of TS 38.213 [13]. The network only configures either searchSpaceSwitchConfig-r16 or searchSpaceSwitchConfig-r17 for a UE.

tpc-PUCCH Enable and configure reception of group TPC commands for PUCCH.

tpc-PUSCH Enable and configure reception of group TPC commands for PUSCH.

tpc-SRS Enable and configure reception of group TPC commands for SRS.

uplinkCancellation Configuration of uplink cancellation indications to be monitored in this cell (see TS 38.213 [13], clause 11.2A).

SearchSpaceSwitchConfig field descriptions

cellGroupsForSwitchList The list of serving cells which are bundled for the search space group switching purpose (see TS 38.213 [13], clause 10.4). A serving cell can belong to only one CellGroupForSwitch. The network configures the same list for all BWPs of serving cells in the same CellGroupForSwitch.

searchSpaceSwitchDelay Indicates the value to be applied by a UE for Search Space Set Group switching; corresponds to the P value in TS 38.213 [13], clause 10.4. The network configures the same value for all BWPs of serving cells in the same CellGroupForSwitch. For 120/480/960 kHz SCS, only values 40,41, ... 52 are valid and the actual value = field value * SCS/120 kHz i.e. field value 40 corresponds to 40 with 120 kHz SCS, 160 with 480 kHz SCS and 320 with 960 kHz SCS, and so on.

searchSpaceSwitchTimer Timer (in unit of slots) to control the UE behavior to switch from search space group X back to search space group 0, as specified in clause 10 of TS 38.213 [13]. For the 15kHz SCS, only the first 26 values are valid and correspond to {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100}. For the 30kHz SCS, only the first 46 values are valid and correspond to {1, 2, 3, …, 40, 60, 80, 100, 120, 160, 200}. For the 60kHz SCS, only the first 86 values are valid and correspond to {1, 2, 3, …, 80, 120, 160, 200, 240, 320, 400}. For the 120kHz SCS, the 166 values correspond to {1, 2, 3, …, 160, 240, 320, 400, 480, 640, 800}. For the 480kHz SCS, the 166 values correspond to {4, 8, 12, …, 640, 960, 1280, 1600, 1920, 2560, 3200}. For the 960kHz SCS, the 166 values correspond to {8, 16, 24, …, 1280, 1920, 2560, 3200, 3840, 5120, 6400}.

sr-TriggeredSearchSpaceSwitchGroupId Identifies the target search space group for SR-triggered search space group switching as described in clause 10.4 of TS 38.213 [13].

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as in dedicated signalling.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

[[

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT          SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}                                                                                           OPTIONAL    -- Cond OtherBWP

]],

[[

commonSearchSpaceListExt-r16                                             SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-r16     OPTIONAL  -- Need R

]],

[[

sdt-SearchSpace-r17                 CHOICE {

newSearchSpace                      SearchSpace,

existingSearchSpace                 SearchSpaceId

}                                                                                           OPTIONAL,   -- Need R

searchSpaceMCCH-r17                 SearchSpaceId                                           OPTIONAL,   -- Need R

searchSpaceMTCH-r17                 SearchSpaceId                                           OPTIONAL,   -- Need S

commonSearchSpaceListExt2-r17       SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1700           OPTIONAL,   -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE {

sCS480KHZoneEighthT                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                           OPTIONAL,   -- Need R

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

]],

[[

followUnifiedTCI-State-v1720           ENUMERATED {enabled}                                  OPTIONAL     -- Need R

]],

[[

applyIndicatedTCI-State-r18            ENUMERATED {first, second, both, none}                OPTIONAL,   -- Cond FollowUTCI

commonSearchSpaceListExt-r18           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800         OPTIONAL,   -- Need R

searchSpaceMulticastMCCH-r18           SearchSpaceId                                         OPTIONAL,   -- Need R

searchSpaceMulticastMTCH-r18           SearchSpaceId                                         OPTIONAL    -- Need S

]],

[[

commonSearchSpaceListExt-r19           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800                  OPTIONAL,   -- Need R

pagingAdaptFirstPDCCH-MonitoringOccasionOfPO-r19 CHOICE {

sCS15KHZoneT                                     SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                       SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT      SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                           SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..71679),

sCS480KHZoneThirtySecondT                        SEQUENCE (SIZE (1..maxNrofPO-PerPF-r19)) OF INTEGER (0..143359)

}                                                                                                     OPTIONAL,  -- Cond OtherBWP

pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O-r19   CHOICE {

sCS15KHZoneT                                     SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                       SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT      SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT

SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT

SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT

SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                           SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..71679),

sCS480KHZoneThirtySecondT                        SEQUENCE (SIZE (1..maxPEI-PerPF-r19)) OF INTEGER (0..143359)

}                                                                                                OPTIONAL, -- Cond InitialBWP-Paging

pagingAdaptPagingSearchSpace           SearchSpaceId                                             OPTIONAL  -- Need S

]]

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

applyIndicatedTCI-State This field indicates, for PDCCH reception in CORESET #0, if UE applies the first, the second, both or none of the "indicated" DL only TCI or joint TCI as specified in TS 38.213 [13], clause 10.1. Value both is not configured if the CORESET is associated with searchSpaceZero for Type 0/0A/2 common search space and can be configured only if sfnSchemePDCCH is configured in the serving cell.

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the RedCap-specific initial downlink BWP in SIB1 for (e)RedCap such that it does not have to be contained in the bandwidth of CORESET#0.

commonSearchSpaceList, commonSearchSpaceListExt, commonSearchSpaceListExt2 A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0. If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the SearchSpace entries is considered to be newly created and the conditions and Need codes for setup of the entry apply. If the network includes commonSearchSpaceListExt/commonSearchSpaceListExt2, it includes the same number of entries, and listed in the same order, as in commonSearchSpaceList.

controlResourceSetZero Parameters of the common CORESET#0 which can be used in any common or UE-specific search spaces. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) controlResourceSetZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions defined in TS 38.213 [13], clause 10 are satisfied.

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

firstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP, see TS 38.304 [20]. The field sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS120KHZoneSixteenthT can be applied for SCS 480kHz, corresponding to sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS480KHZquarterT-SCS120KHZoneSixteenthT in IE DownlinkConfigCommonSIB respectively.

followUnifiedTCI-State When set to enabled, for PDCCH reception in CORESET #0, the UE applies the "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP for paging adaptation, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in pagingAdaptFirstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. If pagingAdaptPEI-Config is configured in DownlinkConfigCommonSIB, this field is configured.

pagingAdaptFirstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP for paging adaptation, see TS 38.304 [20].

pagingAdaptPagingSearchSpace ID of the search space for paging adaptation (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_INACTIVE while SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), or an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_INACTIVE while SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), or an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

pei-ConfigBWP Provides the configuration for PEI reception in this BWP. If the field is absent, the UE does not receive PEI in this BWP. For the initialDownlinkBWP-RedCap not including CD-SSB and the entire CORESET#0, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing monitors PEI in the initialDownlinkBWP that includes CORESET#0, if the initialDownlinkBWP is configured with pei-ConfigBWP.

pei-SearchSpace ID of dedicated search space for PEI. It can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0. The CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level follows Table 10.1-1 of TS38.213 [13]. SearchSpaceId = 0 can be configured for the case of SS/PBCH block and CORESET multiplexing pattern 2 or 3.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], clause 5.15 are met.

sdt-SearchSpace Common search space for CG-SDT and RA-SDT (see TS 38.213 [13]). If an existingSearchSpace is used, the network only signals the search space ID of the ra-SearchSpace.

searchSpaceMCCH ID of the search space for MCCH. If the field is absent, the UE does not receive MCCH in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, the (e)RedCap UE in RRC_IDLE or RRC_INACTIVE may receive MCCH using searchSpaceMCCH in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceMTCH ID of the search space for MTCH of MBS broadcast. If the field is absent, the UE applies searchSpaceMCCH also for MTCH, (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, the (e)RedCap UE in RRC_IDLE or RRC_INACTIVE may receive MTCH using searchSpaceMTCH in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceMulticastMCCH ID of the search space for multicast MCCH. If the field is absent, the UE does not receive multicast MCCH in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0.

searchSpaceMulticastMTCH ID of the search space for multicast MTCH. If the field is absent, the UE applies searchSpaceMulticastMCCH also for multicast MTCH, (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive other system information using searchSpaceOtherSystemInformation in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive SIB1 using searchSpaceSIB1 in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0), searchSpaceZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions described in TS 38.213 [13], clause 10, are satisfied.

| Conditional Presence | Explanation |
| --- | --- |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present or if more than one value for the field coresetPoolIndex is configured in controlResourceSet for the same bandwidthpart. Otherwise, it is optionally present, Need R. |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon except it is the RedCap-specific initial BWP not including CD-SSB and the entire CORESET#0 in which case the field is absent, Need R; it is absent in other BWPs and when sent in system information. If SIB1 is not broadcast and there is an SSB associated to the cell, the field is optionally present, Need M, in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon (still with the same setting for all UEs). In other cases, the field is absent. |
| InitialBWP-Paging | This field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap including CD-SSB and the entire CORESET#0, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise, this field is absent. |
| OtherBWP | This field is optionally present, Need R, if this BWP is not the initialDownlinkBWP and pagingSearchSpace is configured in this BWP. Otherwise this field is absent. |

#### –	PDCCH-ConfigSIB1

The IE PDCCH-ConfigSIB1 is used to configure CORESET#0 and search space#0.

PDCCH-ConfigSIB1 information element

-- ASN1START

-- TAG-PDCCH-CONFIGSIB1-START

PDCCH-ConfigSIB1 ::=                SEQUENCE {

controlResourceSetZero              ControlResourceSetZero,

searchSpaceZero                     SearchSpaceZero

}

-- TAG-PDCCH-CONFIGSIB1-STOP

-- ASN1STOP

PDCCH-ConfigSIB1 field descriptions

controlResourceSetZero Determines a common ControlResourceSet (CORESET) with ID #0, see TS 38.213 [13], clause 13.

searchSpaceZero Determines a common search space with ID #0, see TS 38.213 [13], clause 13.

#### –	PDCCH-ServingCellConfig

The IE PDCCH-ServingCellConfig is used to configure UE specific PDCCH parameters applicable across all bandwidth parts of a serving cell.

PDCCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDCCH-SERVINGCELLCONFIG-START

PDCCH-ServingCellConfig ::=         SEQUENCE {

slotFormatIndicator                 SetupRelease { SlotFormatIndicator }                                OPTIONAL,   -- Need M

...,

[[

availabilityIndicator-r16           SetupRelease {AvailabilityIndicator-r16}                            OPTIONAL,   -- Need M

searchSpaceSwitchTimer-r16          INTEGER (1..80)                                                     OPTIONAL    -- Need R

]],

[[

searchSpaceSwitchTimer-v1710        INTEGER (81..1280)                                                  OPTIONAL    -- Need R

]]

}

-- TAG-PDCCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDCCH-ServingCellConfig field descriptions

availabilityIndicator Use to configure monitoring a PDCCH for Availability Indicators (AI).

searchSpaceSwitchTimer The value of the timer in slots for monitoring PDCCH in the active DL BWP of the serving cell before moving to the default search space group (see TS 38.213 [13], clause 10.4). For 15 kHz SCS, {1..20} are valid. For 30 kHz SCS, {1..40} are valid. For 60kHz SCS, {1..80} are valid. For 120 kHz SCS, {1..160} are valid. For 480 kHz SCS, {1..640} are valid. For 960 kHz SCS, {1..1280} are valid. The network configures the same value for all serving cells in the same CellGroupForSwitch.

slotFormatIndicator Configuration of Slot-Format-Indicators to be monitored in the correspondingly configured PDCCHs of this serving cell.

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling, MBS multicast and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup1

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM-UM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL,   -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]],

[[

discardTimerExt-r16     SetupRelease { DiscardTimerExt-r16 }                                OPTIONAL,    -- Cond DRB2

moreThanTwoRLC-DRB-r16  SEQUENCE {

splitSecondaryPath-r16  LogicalChannelIdentity                                          OPTIONAL,   -- Cond SplitBearer2

duplicationState-r16    SEQUENCE (SIZE (3)) OF BOOLEAN                                  OPTIONAL    -- Need S

}                                                                                           OPTIONAL,   -- Cond MoreThanTwoRLC-DRB

ethernetHeaderCompression-r16  SetupRelease { EthernetHeaderCompression-r16 }               OPTIONAL    -- Need M

]],

[[

survivalTimeStateSupport-r17   ENUMERATED {true}                                            OPTIONAL,   -- Cond Drb-Duplication

uplinkDataCompression-r17      SetupRelease { UplinkDataCompression-r17 }                   OPTIONAL,   -- Cond Rlc-AM

discardTimerExt2-r17           SetupRelease { DiscardTimerExt2-r17 }                        OPTIONAL,   -- Need M

initialRX-DELIV-r17            BIT STRING (SIZE (32))                                       OPTIONAL    -- Cond MRB-Initialization

]],

[[

pdu-SetDiscard-r18             ENUMERATED {true}                                            OPTIONAL,   -- Need R

discardTimerForLowImportance-r18   SetupRelease { DiscardTimerForLowImportance-r18 }        OPTIONAL,   -- Cond DRB2

primaryPathOnIndirectPath-r18  ENUMERATED {true}                                            OPTIONAL,   -- Cond SplitBearerMP

sn-GapReport-r18               ENUMERATED {true}                                            OPTIONAL    -- Need R

]],

[[

remainingTimeThresholdRLC-ReTx-r19              RLC-AM-RemainingTimeThreshold-r19           OPTIONAL,   -- Cond RLC-AM

remainingTimeThresholdRLC-Polling-r19           RLC-AM-RemainingTimeThreshold-r19           OPTIONAL    -- Cond RLC-AM

]]

}

EthernetHeaderCompression-r16 ::=  SEQUENCE {

ehc-Common-r16                     SEQUENCE {

ehc-CID-Length-r16                 ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink-r16               SEQUENCE {

drb-ContinueEHC-DL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL,   -- Need M

ehc-Uplink-r16                 SEQUENCE {

maxCID-EHC-UL-r16              INTEGER (1..32767),

drb-ContinueEHC-UL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL    -- Need M

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

DiscardTimerExt2-r17 ::= ENUMERATED {ms2000, spare3, spare2, spare1}

UplinkDataCompression-r17 ::= CHOICE {

newSetup                      SEQUENCE {

bufferSize-r17                ENUMERATED {kbyte2, kbyte4, kbyte8, spare1},

dictionary-r17                ENUMERATED {sip-SDP, operator}                            OPTIONAL    -- Need N

},

drb-ContinueUDC           NULL

}

DiscardTimerForLowImportance-r18 ::= ENUMERATED {ms0, ms2, ms4, ms6, ms8, ms10, ms12, ms14, ms18, ms22, ms26, ms30, ms40, ms50, ms75, ms100}

RLC-AM-RemainingTimeThreshold-r19 ::= INTEGER (1..64)

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

discardTimerExt Value in ms of discardTimer specified in TS 38.323 [5]. Value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1ms and so on. If this field is present, the field discardTimer is ignored and discardTimerExt is used instead.

discardTimerExt2 Value in ms of discardTimerExt specified in TS 38.323 [5]. Value ms2000 corresponds to 2000 ms. If this field is present, the field discardTimer and discardTimerExt are ignored and discardTimerExt2 is used instead.

discardTimerForLowImportance Value in ms of discardTimerForLowImportance specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms2 corresponds to 2 ms and so on. The value of this timer for a PDCP entity is always configured shorter than discardTimer, discardTimerExt or discardTimerExt2, whichever is used for the PDCP entity. The presence of this field indicates that PSI-based SDU discard is configured for a DRB. The network configures at most 8 DRBs with discardTimerForLowImportance.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer. This field can be configured for both DRB and multicast MRB.

duplicationState This field indicates the uplink PDCP duplication state for the associated RLC entities at the time of receiving this IE. If set to true, the PDCP duplication state is activated for the associated RLC entity. The index for the indication is determined by ascending order of logical channel ID of all RLC entities other than the primary RLC entity indicated by primaryPath in the order of MCG and SCG, as in clause 6.1.3.32 of TS 38.321 [3]. For MP, the index for the indication is determined by ascending order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path, as in clause 6.1.3.32 of TS 38.321 [3]. If the number of associated RLC entities other than the primary RLC entity is two, UE ignores the value in the largest index of this field. If the field is absent, the PDCP duplication states are deactivated for all associated RLC entities.

ethernetHeaderCompression This fields configures Ethernet Header Compression. This field can only be configured for a bi-directional DRB or a bi-directional multicast MRB. The network reconfigures ethernetHeaderCompression only upon reconfiguration involving PDCP re-establishment and with neither drb-ContinueEHC-DL nor drb-ContinueEHC-UL configured. Network only configures this field when uplinkDataCompression is not configured.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB or a multicast MRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure DAPS bearer(s), and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured. Network only configures this field when uplinkDataCompression is not configured.

initialRX-DELIV Indicates the initial value of RX_DELIV during PDCP window initialization for multicast MRB as specified in TS 38.323 [5]. The first/leftmost bit of the bit string contains the most significant bit.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity. This field is not present if the bearer is configured as DAPS bearer.

moreThanTwoRLC-DRB This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. For PDCP entity with more than two associated RLC entities for UL transmission, this field is always present. If the field moreThanTwoRLC-DRB is present, the value of this field is ignored and the state of the duplication is indicated by duplicationState. For PDCP entity with more than two associated RLC entities, only NR RLC bearer is supported.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdu-SetDiscard If set to true, the UE shall perform PDU Set based discarding for this PDCP entity, as specified in TS 38.323 [5]. This field is only configured for a DRB.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs, except for the split SRB2 of the IAB-MT, and except when the UE is required to set the primaryPath to refer to the SCG or indirect path as specified in clause 5.7.3b.4. In this last case, if the network sends an RRCReconfiguration message (in NR-DC) or an EUTRA RRCConnectionReconfiguration message (in (NG)EN-DC) keeping SRB1 as split SRB, the network explicitly configures the primaryPath for the PDCP entity of SRB1 to refer to the MCG. In this version of the specification, only cell group ID corresponding to MCG is supported for DRBs when the SCG is deactivated. In MR-DC, the NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field. This field is also used for configuring Multi-path Primary Path defined in [5] for MP operation. In MP, when the primary path is indirect path, the field cellGroup and logicalChannel are absent, and the field primaryPathOnIndirectPath is set to true.

primaryPathOnIndirectPath Indicates that the primary path is SL indirect path, or primary path is associated with the N3C indirect path, for DRB when MP is configured.

remainingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.

remainingTimeThresholdRLC-ReTx Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based retransmission as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.

sn-GapReport Indicates whether the PDCP entity is configured to send a PDCP SN gap report in the uplink, as specified in TS 38.323 [5]. This field is only configured for DRBs. This field can be included only if outOfOrderDelivery is not configured.

splitSecondaryPath Indicates the LCID of the split secondary RLC entity as specified in TS 38.323 [5] for fallback to split bearer operation when UL data transmission with more than two RLC entities is associated with the PDCP entity. This RLC entity belongs to a cell group that is different from the cell group indicated by cellGroup in the field primaryPath. This RLC entity belongs to the cell group of the direct path if primaryPathOnIndirectPath is set to true in MP case.

statusReportRequired For AM DRBs, AM MRBs and DAPS UM DRBs, indicates whether the DRB or the multicast MRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For DAPS AM DRBs, it also indicates whether the DRB is configured to send a second PDCP status report in the uplink, as specified in TS 38.323 [5].

survivalTimeStateSupport Indicates whether the DRB associated with this PDCP entity has survival time state support. If this field is configured to be true, all associated RLC entities are activated for PDCP duplication upon reception of a retransmission grant addressed to CS-RNTI, as specified in TS 38.321 [3].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG and when the SCG is deactivated or for multi-path U2N Remote UEs not supporting splitDRB-WithUL-BothDirectIndirect. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two. The network reconfigures uplinkDataCompression only upon reconfiguration involving PDCP re-establishment. If the field is set to drb-ContinueUDC, the PDCP entity continues the uplink data compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is set to drb-ContinueUDC only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

EthernetHeaderCompression field descriptions

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

ehc-CID-Length Indicates the length of the CID field for EHC packet. The value bits7 indicates the length is 7 bits, and the value bits15 indicates the length is 15 bits. Once the field ethernetHeaderCompression-r16 is configured for a DRB or a multicast MRB, the value of the field ehc-CID-Length for this DRB or multicast MRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplnik. Otherwise, it is not configured for uplink.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 38.323 [5]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

UplinkDataCompression field descriptions

bufferSize This field indicates the buffer size applied for UDC as specified in TS 38.323 [5]. Value kbyte2 means 2048 bytes, kbyte4 means 4096 bytes and so on.

dictionary This field indicates which pre-defined dictionary is used for UDC as specified in TS 38.323 [5]. The value sip-SDP means that UE shall prefill the buffer with standard dictionary for SIP and SDP defined in TS 38.323 [5], and the value operator means that UE shall prefill the buffer with operator-defined dictionary.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB/multicast MRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs and MRBs. |
| Drb-Duplication | For SRBs, this field is absent. For DRBs, this field is absent if duplication is not configured. Otherwise, this field is optional, need R. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer (except for multicast MRB) with more than one associated logical channel and upon RRC reconfiguration with the association of additional logical channels to the PDCP entity. The field is also mandatory present in case the field moreThanTwoRLC-DRB is included in PDCP-Config. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| MoreThanTwoRLC-DRB | For SRBs, this field is absent. For DRBs, this field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than two associated logical channels and upon RRC reconfiguration with the association of one or more additional logical channel(s) to the PDCP entity so that the PDCP entity has more than two associated logical channels. Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present, Need M. Otherwise, the field is absent, Need R. |
| Rlc-AM | For RLC AM, the field is optionally present, need M. Otherwise, the field is absent. |
| Rlc-AM-UM | In case of DRB, for RLC UM (if the UE supports DAPS handover) or RLC AM, the field is optionally present, need R. In case of multicast MRB, if multicast MRB is associated with at least one RLC AM entity, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of DRB setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| SplitBearer2 | The field is mandatory present, in case of a split bearer except MP split bearer with primary path on direct path. Otherwise the field is absent. |
| SplitBearerMP | The field is absent for SRBs. Otherwise, the field is optionally present, need R, when MP is configured. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC or if the UE supports user plane integrity protection when connected to E-UTRA/EPC (as specified in TS 33.401 [30]). Otherwise the field is absent. |
| Setup1 | This field is mandatory present in case of DRB setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| MRB-Initialization | This field is mandatory present in case of multicast MRB setup or in case UE configured with multicast reception in RRC_INACTIVE resumes the RRC connection. In case of PDCP re-establishment for multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |
| RLC-AM | For RLC AM, this field is optionally present, need R; Otherwise, the field is absent. |

#### –	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters. If this IE is used for MBS CFR, the following fields shall be absent: tci-StatesToAddModList, tci-StatesToReleaseList, zp-CSI-RS-ResourceToAddModList, minimumSchedulingOffsetK0, antennaPortsFieldPresenceDCI-1-2, aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2, aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, dmrs-SequenceInitializationDCI-1-2, harq-ProcessNumberSizeDCI-1-2, mcs-TableDCI-1-2, numberOfBitsForRV-DCI-1-2, pdsch-AggregationFactor, pdsch-TimeDomainAllocationListDCI-1-2, prb-BundlingTypeDCI-1-2, priorityIndicatorDCI-1-2, rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2DCI-1-2, resourceAllocationType1GranularityDCI-1-2, vrb-ToPRB-InterleaverDCI-1-2, referenceOfSLIVDCI-1-2, resourceAllocationDCI-1-2, dataScramblingIdentityPDSCH2-r16, repetitionSchemeConfig, pdsch-ConfigDCI-1-3, pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }           OPTIONAL,   -- Need S

bundleSizeSet2                      ENUMERATED { n4, wideband }                                     OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...,

[[

maxMIMO-Layers-r16                      SetupRelease { MaxMIMO-LayersDL-r16 }                               OPTIONAL,   -- Need M

minimumSchedulingOffsetK0-r16           SetupRelease { MinSchedulingOffsetK0-Values-r16 }                   OPTIONAL,   -- Need M

-- Start of the parameters for DCI format 1_2 introduced in V16.1.0

antennaPortsFieldPresenceDCI-1-2-r16            ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-SequenceInitializationDCI-1-2-r16          ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

harq-ProcessNumberSizeDCI-1-2-r16               INTEGER (0..4)                                              OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r16                            ENUMERATED {qam256, qam64LowSE}                             OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-2-r16                   INTEGER (0..2)                                              OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationListDCI-1-2-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

prb-BundlingTypeDCI-1-2-r16             CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2-r16                      ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

}                                                                                                           OPTIONAL,   -- Need R

priorityIndicatorDCI-1-2-r16                ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

rateMatchPatternGroup1DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

rateMatchPatternGroup2DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

resourceAllocationType1GranularityDCI-1-2-r16  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

vrb-ToPRB-InterleaverDCI-1-2-r16            ENUMERATED {n2, n4}                                             OPTIONAL,   -- Need S

referenceOfSLIVDCI-1-2-r16                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

resourceAllocationDCI-1-2-r16               ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 1_2 introduced in V16.1.0

priorityIndicatorDCI-1-1-r16             ENUMERATED {enabled}                                               OPTIONAL,   -- Need S

dataScramblingIdentityPDSCH2-r16         INTEGER (0..1023)                                                  OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationList-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }        OPTIONAL,   -- Need M

repetitionSchemeConfig-r16               SetupRelease { RepetitionSchemeConfig-r16}                         OPTIONAL    -- Need M

]],

[[

repetitionSchemeConfig-v1630             SetupRelease { RepetitionSchemeConfig-v1630}                       OPTIONAL    -- Need M

]],

[[

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2-r17    ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-1-2-r17           ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2-r17     ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxDCI-1-2-r17               ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-2-r17                   ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

dl-OrJointTCI-StateList-r17                  CHOICE {

explicitlist                                 SEQUENCE {

dl-OrJointTCI-StateToAddModList-r17        SEQUENCE (SIZE (1..maxNrofTCI-States)) OF TCI-State

OPTIONAL,   -- Need N

dl-OrJointTCI-StateToReleaseList-r17       SEQUENCE (SIZE (1..maxNrofTCI-States)) OF TCI-StateId

OPTIONAL    -- Need N

},

unifiedTCI-StateRef-r17                  ServingCellAndBWP-Id-r17

}                                                                                                           OPTIONAL,   -- Need R

beamAppTime-r17                              ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336, spare2,

spare1}                                             OPTIONAL,   -- Need R

dummy                                        SetupRelease { Dummy-TDRA-List }                                    OPTIONAL,   -- Need M

dmrs-FD-OCC-DisabledForRank1-PDSCH-r17          ENUMERATED {true}                                           OPTIONAL,   -- Need R

minimumSchedulingOffsetK0-r17                   SetupRelease { MinSchedulingOffsetK0-Values-r17 }           OPTIONAL,   -- Need M

harq-ProcessNumberSizeDCI-1-2-v1700          INTEGER (0..5)                                                 OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-1-r17            INTEGER (5)                                                    OPTIONAL,   -- Need R

mcs-Table-r17                                ENUMERATED {qam1024}                                           OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r17                         ENUMERATED {qam1024}                                           OPTIONAL,   -- Need R

xOverheadMulticast-r17                       ENUMERATED {xOh6, xOh12, xOh18}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-4-2-r17                 ENUMERATED {enabled}                                           OPTIONAL,   -- Need S

sizeDCI-4-2-r17                              INTEGER (20..maxDCI-4-2-Size-r17)                              OPTIONAL    -- Need R

]],

[[

pdsch-TimeDomainAllocationListForMultiPDSCH-r17 SetupRelease { MultiPDSCH-TDRA-List-r17 }                   OPTIONAL    -- Need M

]],

[[

advancedReceiver-MU-MIMO-r18                 SetupRelease { AdvancedReceiver-MU-MIMO-r18 }                  OPTIONAL,   -- Need M

pdsch-ConfigDCI-1-3-r18                      SetupRelease { PDSCH-ConfigDCI-1-3-r18 }                       OPTIONAL    -- Need M

]],

[[

pdsch-ConfigDCI-1-3-v1860                    SetupRelease { PDSCH-ConfigDCI-1-3-v1860 }                     OPTIONAL    -- Need M

]],

[[

mg-CancellationDCI-1-1-r19                   ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

mg-CancellationDCI-1-2-r19                   ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

mg-CancellationDCI-1-3-r19                   ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3-r19  SetupRelease { MultiPDSCH-TDRA-List-r17 }          OPTIONAL,   -- Need M

harq-ProcessNumberSizeDCI-1-1-Ext-r19        INTEGER (5)                                                    OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-2-Ext-r19        INTEGER (0..5)                                                 OPTIONAL,   -- Need R

pdsch-ConfigDCI-1-3-v1900                    SetupRelease { PDSCH-ConfigDCI-1-3-v1900 }                     OPTIONAL    -- Need M

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

MinSchedulingOffsetK0-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r16)

MinSchedulingOffsetK0-Values-r17 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r17)

MaxMIMO-LayersDL-r16 ::=                INTEGER (1..8)

PDSCH-ConfigDCI-1-3-r18 ::=                    SEQUENCE {

resourceAllocationDCI-1-3-r18                  ENUMERATED {resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

rbg-SizeDCI-1-3-r18                            ENUMERATED {config1, config2, config3, spare1}               OPTIONAL, -- Cond DCI-1-3

resourceAllocationType1GranularityDCI-1-3-r18  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-3-r18                  INTEGER (0..2)                                               OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-3-r18              INTEGER (0..5)                                               OPTIONAL    -- Need R

}

PDSCH-ConfigDCI-1-3-v1860 ::=                  SEQUENCE {

enabledDefaultBeamForMultiCellScheduling-r18   ENUMERATED {enabled}

}

PDSCH-ConfigDCI-1-3-v1900 ::=                  SEQUENCE {

harq-ProcessNumberSizeDCI-1-3-Ext-r19          INTEGER (0..5)                                               OPTIONAL    -- Need R

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

advancedReceiver-MU-MIMO A set of assistance information for R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions.

antennaPortsFieldPresenceDCI-1-2 Configure the presence of "Antenna ports" field in DCI format 1_2. When the field is configured, then the "Antenna ports" field is present in DCI format 1_2. Otherwise, the field size is set to 0 for DCI format 1_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 nor dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 is configured, this field is absent.

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList, aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2). The field aperiodic-ZP-CSI-RS-ResourceSetsToAddModList applies to DCI format 1_1 and the field aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.2 and TS 38.212 [17] clause 7.3.1).

beamAppTime Indicates the first slot to apply the unified TCI indicated by DCI as specified in TS 38.214 Clause 5.1.5. The value n1 means 1 symbol, n2 two symbols and so on. The first slot is at least Y symbols indicated by beamAppTime parameter after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. The same value shall be configured for all serving cells in any one of the simultaneousU-TCI-UpdateListN configured in IE CellGroupConfig based on the smallest SCS of the active BWP.

dataScramblingIdentityPDSCH, dataScramblingIdentityPDSCH2 Identifier(s) used to initialize data scrambling (c_init) for PDSCH as specified in TS 38.211 [16], clause 7.3.1.1. The dataScramblingIdentityPDSCH2 is configured if coresetPoolIndex is configured with 1 for at least one CORESET in the same BWP.

dl-OrJointTCI-StateToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports, PDCCH DMRS ports, and CSI-RS, and in case of joint mode, also the PUSCH, PUCCH and SRS (see TS 38.214 [19], clause 5.1.5).

dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeA applies to DCI formats 1_1 and 1_3, and the field dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeB applies to DCI formats 1_1 and 1_3, and the field dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-FD-OCC-DisabledForRank1-PDSCH If configured, the UE may assume that the set of remaining orthogonal antenna ports, which are within the same code division multiplexing (CDM) group and have different frequency domain orthogonal cover codes (FD-OCC), are not associated with the PDSCH of another UE (see TS 38.214 [19], clause 5.1.6.2). It is applicable for PDSCH SCS of 480 and 960 kHz when rank 1 PDSCH with type-1 or type-2 DMRS is scheduled. If dmrs-TypeEnh-r18 is configured, this field is not configured.

dmrs-SequenceInitializationDCI-1_2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 1_2 If the field is absent, then the UE applies the value of 0 bit for the field "DMRS Sequence Initialization" in DCI format 1_2. If the field is present, then the UE applies the value of 1 bit as in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

harq-ProcessNumberSizeDCI-1-1-Ext Configure the number of bits for the field "HARQ process number" in DCI format 1_1 in TN (see TS 38.212 [17], clause 7.3.1). For FR1, this field can only be configured when the maximum number of layers configured for PDSCH is no more than 4.

harq-ProcessNumberSizeDCI-1-2 Configure the number of bits for the field "HARQ process number" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1). Only one of this field and harq-ProcessNumberSizeDCI-1-2-Ext is configured by network.

harq-ProcessNumberSizeDCI-1-2-Ext Configure the number of bits for the field "HARQ process number" in DCI format 1_2 in TN (see TS 38.212 [17], clause 7.3.1). For FR1, this field can only be configured when the maximum number of layers configured for PDSCH is no more than 4. Only one of this field and harq-ProcessNumberSizeDCI-1-2 is configured by network.

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in this DL BWP. If not configured, the UE uses the maxMIMO-Layers configuration in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs, when the UE operates in this BWP. The value of maxMIMO-Layers for a DL BWP shall be smaller than or equal to the value of maxMIMO-Layers configured in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs. For MBS multicast, indicates the maximum number of MIMO layers to be used for group-common PDSCH of MBS multicast in this CFR. If not configured for CFR, the UE applies value 1. The value of maxMIMO-Layers for a CFR shall be smaller than or equal to the value of maxMIMO-Layers configured in PDSCH-ServingCellConfig IE of the serving cell to which this CFR belongs.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH for DCI formats 1_0, 1_1 and 1_3 (see TS 38.214 [19], clause 5.1.3.1). If all fields are absent the UE applies the value 64QAM. If the field mcs-Table-r17 is present for DCI formats 1_1 and 1_3, the network does not configure the field mcs-Table (without suffix). For an (e)RedCap UE, the 256QAM MCS table for PDSCH is only supported if the UE indicates support of 256QAM for PDSCH.

mcs-TableDCI-1-2 Indicates which MCS table the UE shall use for PDSCH for DCI format 1_2 (see TS 38.214 [19], clause 5.1.3.1). If all fields are absent the UE applies the value 64QAM. If the field mcs-TableDCI-1-2-r17 is present, the network does not configure the field mcs-TableDCI-1-2-r16. For an (e)RedCap UE, the 256QAM MCS table for PDSCH is only supported if the UE indicates support of 256QAM for PDSCH.

mg-CancellationDCI-1-1 Indicates the presence of Measurement gap cancellation in DCI format 1_1 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-1-2 Indicates the presence of Measurement gap cancellation in DCI format 1_2 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-1-3 Indicates the presence of Measurement gap cancellation in DCI format 1_3 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17]. The field could only be configured if the co-scheduled cells are intra-band and have the same sub-carrier spacing.

minimumSchedulingOffsetK0 List of minimum K0 values. Minimum K0 parameter denotes minimum applicable value(s) for the TDRA table for PDSCH and for A-CSI RS triggering Offset(s) (see TS 38.214 [19], clause 5.3.1).

numberOfBitsForRV-DCI-1-2 Configures the number of bits for "Redundancy version" in the DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent in PDSCH-Config which is not used for MBS CFR, the UE applies the value 1.

pdsch-HARQ-ACK-EnhType3DCI-1-2 When configured, enhanced Type 3 HARQ-ACK codebook triggering by DCI format 1_2 is enabled.

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2 Enables the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in DCI format 1_2 if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH cell group.

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 When configured, DCI format 1_2 can request the UE to report A/N for all HARQ processes and all component carriers configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-RetxDCI-1-2 When configured, DCI format 1_2 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource (see TS 38.213 [13], clause 9.1.5).

pdsch-TimeDomainAllocationList, pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH, pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 List of time-domain configurations for timing of DL assignment to DL data. The field pdsch-TimeDomainAllocationList (with or without suffix) applies to DCI format 1_0, DCI format 1_1 and DCI format 1_3 (see table 5.1.2.1.1-1 in TS 38.214 [19]), and if the field pdsch-TimeDomainAllocationListDCI-1-2 is not configured, to DCI format 1_2. If the field pdsch-TimeDomainAllocationListDCI-1-2 is configured, it applies to DCI format 1_2 (see table 5.1.2.1.1-1A in TS 38.214 [19]). The field pdsch-TimeDomainAllocationListForMultiPDSCH applies to DCI format 1_1. The field pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 applies to DCI format 1_3. The network does not configure the pdsch-TimeDomainAllocationList-r16 simultaneously with the pdsch-TimeDomainAllocationList (without suffix) in the same PDSCH-Config.

prb-BundlingType, prb-BundlingTypeDCI-1-2 Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2. The field prb-BundlingType applies to DCI formats 1_1 and 1_3, and the field prb-BundlingTypeDCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.3).

priorityIndicatorDCI-1-1, priorityIndicatorDCI-1-2, priorityIndicatorDCI-4-2 Configure the presence of "priority indicator" in DCI format 1_1/1_2/4_2. When the field is absent in the IE, then 0 bit for "priority indicator" in DCI format 1_1/1_2/4_2. The field priorityIndicatorDCI-1-1 applies to DCI format 1_1, the field priorityIndicatorDCI-1-2 applies to DCI format 1_2 and the field priorityIndicatorDCI-4-2 applies to DCI format 4_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9).

pucch-sSCellDynDCI-1-2 When configured, PUCCH cell switching based on dynamic indication in DCI format 1_2 is enabled (see TS 38.213 [13], clause 9.A).

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set. If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config for MBS CFR and PDSCH-Config for the assoicated BWP, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for MBS CFR can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for the assoicated BWP.

rateMatchPatternGroup1, rateMatchPatternGroup1DCI-1-2 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup1 applies to DCI formats 1_1 and 1_3, and the field rateMatchPatternGroup1DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2, rateMatchPatternGroup2DCI-1-2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup2 applies to DCI formats 1_1 and 1_3, and the field rateMatchPatternGroup2DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns (see TS 38.214 [19], clause 5.1.4.1). If a RateMatchPattern with the same RateMatchPatternId is configured in both MBS CFR and its associated BWP, the entire RateMatchPattern configuration, including the set of RBs/REs indicated by the patterns for the rate matching around, shall be the same and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH except PDSCH scheduled by DCI format 1_3. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

referenceOfSLIVDCI-1-2 Enable using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV for DCI format 1_2. When the RRC parameter enables the utilization of the new reference, the new reference is applied for TDRA entries with K0=0. For other entries (if any) in the same TDRA table, the reference is slot boundary as in Rel-15. PDSCH mapping type A is not supported with the new reference. The new reference of SLIV is not configured for a serving cell configured to be scheduled by cross-carrier scheduling on a scheduling cell with different numerology (see TS 38.212 [17] clause 7.3.1 and TS 38.214 [19] clause 5.1.2.1).

repetitionSchemeConfig Configure the UE with repetition schemes. The network does not configure repetitionSchemeConfig-r16 and repetitionSchemeConfig-v1630 simultaneously to setup in the same PDSCH-Config. The network does not configure this parameter and sfnSchemePDSCH in MIMOParam-r17 simultaneously in the same serving cell.

resourceAllocation, resourceAllocationDCI-1-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2). The field resourceAllocation applies to DCI format 1_1, and the field resourceAllocationDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-2 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

sizeDCI-4-2 Indicates the size of DCI format 4-2 (see TS 38.213 [13], clause 10.1).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5). If unifiedTCI-StateType is configured for the serving cell, no element in this list is configured.

unifiedTCI-StateRef Provides the serving cell and BWP where the configuration for dl-OrJointTCI-StateToAddModList-r17 are defined. When this field is present, dl-OrJointTCI-StateToAddModList and dl-OrJointTCI-StateToReleaseList are not present. The value of unifiedTCI-StateType of current serving cell is the same in the serving cell indicated by unifiedTCI-StateRef.

vrb-ToPRB-Interleaver, vrb-ToPRB-InterleaverDCI-1-2 Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

xOverheadMulticast Accounts for an overhead from CSI-RS, CORESET etc. If the field is absent, the UE applies value xOh0 (see TS 38.214 [19]).

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).

PDSCH-ConfigDCI-1-3 field descriptions

enabledDefaultBeamForMultiCellScheduling This field indicates whether default beam selection for DCI format 1_3 scheduled PDSCH or aperiodic CSI-RS is enabled (see TS 38.214 [19], clause 5.1.5 and clause 5.2.1.5).

harq-ProcessNumberSizeDCI-1-3 Configure the number of bits for the field "HARQ process number" in DCI format 1_3 (see TS 38.212 [17], clause 7.3.1). Only one of this field and harq-ProcessNumberSizeDCI-1-3-Ext is configured by network.

harq-ProcessNumberSizeDCI-1-3-Ext Configure the number of bits for the field "HARQ process number" in DCI format 1_3 in TN (see TS 38.212 [17], clause 7.3.1). This field can only be configured when the maximum number of layers configured for PDSCH is no more than 4. Only one of this field and harq-ProcessNumberSizeDCI-1-3 is configured by network.

numberOfBitsForRV-DCI-1-3 Configures the number of bits for "Redundancy version" in the DCI format 1_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

rbg-SizeDCI-1-3 Selection among config 1, config 2 and config 3 for RBG size for PDSCH scheduled by DCI format 1_3. The UE ignores this field if resourceAllocationDCI-1-3 is set to resourceAllocationType1. (see TS 38.214 [19], clause 5.1.2.2.1).

resourceAllocationDCI-1-3 Configuration of resource allocation type 0 and resource allocation type 1 for DCI format 1_3 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-3 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_3. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

| Conditional Presence | Explanation |
| --- | --- |
| DCI-1-3 | This field is mandatory present when ScheduledCellListDCI-1-3 is configured to the serving cell. Otherwise, it is absent, Need R. |

#### –	PDSCH-ConfigCommon

The IE PDSCH-ConfigCommon is used to configure cell specific PDSCH parameters.

PDSCH-ConfigCommon information element

-- ASN1START

-- TAG-PDSCH-CONFIGCOMMON-START

PDSCH-ConfigCommon ::=                  SEQUENCE {

pdsch-TimeDomainAllocationList                  PDSCH-TimeDomainResourceAllocationList          OPTIONAL,   -- Need R

...,

[[

pdsch-AggregationFactor-r19                     ENUMERATED {n2,n4}                              OPTIONAL    -- Need R

]]

}

-- TAG-PDSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDSCH-ConfigCommon field descriptions

pdsch-AggregationFactor Indicates the number of consecutive repetition slots for Msg4 PDSCH, see TS 38.214 [19], clause 5.1.2.1.

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data (see table 5.1.2.1.1-1 in TS 38.214 [19]).

#### –	PDSCH-ServingCellConfig

The IE PDSCH-ServingCellConfig is used to configure UE specific PDSCH parameters that are common across the UE's BWPs of one serving cell.

PDSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PDSCH-SERVINGCELLCONFIG-START

PDSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PDSCH-CodeBlockGroupTransmission }              OPTIONAL,   -- Need M

xOverhead                               ENUMERATED { xOh6, xOh12, xOh18 }                              OPTIONAL,   -- Need S

nrofHARQ-ProcessesForPDSCH              ENUMERATED {n2, n4, n6, n10, n12, n16}                         OPTIONAL,   -- Need S

pucch-Cell                              ServCellIndex                                                  OPTIONAL,   -- Cond SCellAddOnly

...,

[[

maxMIMO-Layers                          INTEGER (1..8)                                                 OPTIONAL,   -- Need M

processingType2Enabled                  BOOLEAN                                                        OPTIONAL    -- Need M

]],

[[

pdsch-CodeBlockGroupTransmissionList-r16 SetupRelease { PDSCH-CodeBlockGroupTransmissionList-r16 }     OPTIONAL    -- Need M

]],

[[

downlinkHARQ-FeedbackDisabled-r17       SetupRelease { DownlinkHARQ-FeedbackDisabled-r17 }              OPTIONAL,   -- Need M

nrofHARQ-ProcessesForPDSCH-v1700        ENUMERATED {n32}                                               OPTIONAL    -- Need R

]]

}

PDSCH-CodeBlockGroupTransmission ::=    SEQUENCE {

maxCodeBlockGroupsPerTransportBlock     ENUMERATED {n2, n4, n6, n8},

codeBlockGroupFlushIndicator            BOOLEAN,

...

}

PDSCH-CodeBlockGroupTransmissionList-r16 ::=    SEQUENCE (SIZE (1..2)) OF PDSCH-CodeBlockGroupTransmission

DownlinkHARQ-FeedbackDisabled-r17 ::= BIT STRING (SIZE (32))

-- TAG-PDSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PDSCH-CodeBlockGroupTransmission field descriptions

codeBlockGroupFlushIndicator Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see TS 38.212 [17], clause 7.3.1.2.2).

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW, the maximum CBG is 4 (see TS 38.213 [13], clause 9.1.1).

PDSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.213 [13], clause 9.1.1). Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group. The network does not configure this field if - the SCS of at least one DL BWP configured in the cell is 480 or 960 kHz - Type-1 HARQ-ACK codebook is configured and pdsch-TimeDomainAllocationListForMultiPDSCH-r17 for this serving cell contains pdsch-AllocationList with multiple entries (multiple PDSCH) - Type-2 HARQ-ACK codebook is configured and pdsch-TimeDomainAllocationListForMultiPDSCH-r17 for any cell in the same PUCCH cell group associated with this serving cell contains pdsch-AllocationList with multiple entries (multiple PDSCH)

downlinkHARQ-FeedbackDisabled Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID. The first/leftmost bit corresponds to HARQ process ID 0, the next bit to HARQ process ID 1 and so on. Bits corresponding to HARQ process IDs that are not configured shall be ignored. The bit(s) set to one identify HARQ processes with disabled DL HARQ feedback and the bit(s) set to zero identify HARQ processes with enabled DL HARQ feedback.

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in all BWPs of this serving cell. (see TS 38.212 [17], clause 5.4.2.1).

nrofHARQ-ProcessesForPDSCH The number of HARQ processes to be used on the PDSCH of a serving cell. Value n2 corresponds to 2 HARQ processes, value n4 to 4 HARQ processes, and so on. If both nrofHARQ-ProcessesForPDSCH and nrofHARQ-ProcessesForPDSCH-v1700 are absent, the UE uses 8 HARQ processes (see TS 38.214 [19], clause 5.1).

pdsch-CodeBlockGroupTransmissionList A list of configurations for up to two simultaneously constructed HARQ-ACK codebooks (see TS 38.213 [13], clause 9.3).

processingType2Enabled Enables configuration of advanced processing time capability 2 for PDSCH (see 38.214 [19], clause 5.3).

pucch-Cell The ID of the serving cell (of the same cell group) to use for PUCCH. If the field is absent, the UE sends the HARQ feedback on the PUCCH of the SpCell of this cell group, or on this serving cell if it is a PUCCH SCell.

xOverhead Accounts for overhead from CSI-RS, CORESET, etc. If the field is absent, the UE applies value xOh0 (see TS 38.214 [19], clause 5.1.3.2).

| Conditional Presence | Explanation |
| --- | --- |
| SCellAddOnly | It is optionally present, Need S, for (non-PUCCH) SCells when adding a new SCell. The field is absent, Need M, when reconfiguring SCells. The field is also absent for the SpCells as well as for a PUCCH SCell. |

#### –	PDSCH-TimeDomainResourceAllocationList

The IE PDSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PDSCH. The PDSCH-TimeDomainResourceAllocationList contains one or more of such PDSCH-TimeDomainResourceAllocations. The network indicates in the DL assignment which of the configured time domain allocations the UE shall apply for that DL assignment. The UE determines the bit width of the DCI field based on the number of entries in the PDSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PDSCH-TimeDomainResourceAllocationList information element

-- ASN1START

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PDSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::=   SEQUENCE {

k0                                      INTEGER(0..32)                                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PDSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation-r16

PDSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k0-r16                                     INTEGER(0..32)                                              OPTIONAL,   -- Need S

mappingType-r16                            ENUMERATED {typeA, typeB},

startSymbolAndLength-r16                   INTEGER (0..127),

repetitionNumber-r16                       ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16} OPTIONAL, -- Cond Formats1-0_1-1_4-0_4-1_4-2

...,

[[

k0-v1710                                INTEGER(33..128)                                               OPTIONAL    -- Need S

]],

[[

repetitionNumber-v1730                  ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                   OPTIONAL   -- Cond Format1-2

]]

}

Dummy-TDRA-List ::= SEQUENCE (SIZE(1.. maxNrofDL-Allocations)) OF MultiPDSCH-TDRA-r17

MultiPDSCH-TDRA-List-r17 ::= SEQUENCE (SIZE(1.. maxNrofDL-AllocationsExt-r17)) OF MultiPDSCH-TDRA-r17

MultiPDSCH-TDRA-r17 ::= SEQUENCE {

pdsch-TDRA-List-r17                 SEQUENCE (SIZE(1..maxNrofMultiplePDSCHs-r17)) OF PDSCH-TimeDomainResourceAllocation-r16,

...

}

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1). k0-v1710 is only applicable for PDSCH SCS of 480 kHz and 960 kHz. If multiple PDSCHs are configured per PDCCH, the network always configures this field. Otherwise, when the field is absent and only one PDSCH is configured per PDCCH, the UE applies the value 0.

mappingType PDSCH mapping type (see TS 38.214 [19], clause 5.3).

repetitionNumber Indicates the number of PDSCH transmission occasions for slot-based repetition scheme in IE RepetitionSchemeConfig. The parameter is used as specified in 38.214 [19].

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

MultiPDSCH-TimeDomainResourceAllocation field descriptions

pdsch-TDRA-List One or multiple PDSCHs which can be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 5.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Format1-2 | In pdsch-TimeDomainAllocationListDCI-1-2, this field is optionally present, Need R. It is absent, Need R, otherwise. |
| Formats1-0_1-1_4-0_4-1_4-2 | In pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH, and SIB20, this field is absent. Otherwise, in pdsch-TimeDomainResourceAllocationList-r16, this field is optionally present, Need R. |

#### –	PDU-SessionID

The IE PDU-SessionID identifies the PDU Session.

PDU-SessionID information element

-- ASN1START

-- TAG-PDU-SESSIONID-START

PDU-SessionID ::=   INTEGER (0..255)

-- TAG-PDU-SESSIONID-STOP

-- ASN1STOP

#### –	PHR-Config

The IE PHR-Config is used to configure parameters for power headroom reporting.

PHR-Config information element

-- ASN1START

-- TAG-PHR-CONFIG-START

PHR-Config ::=                      SEQUENCE {

phr-PeriodicTimer                   ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity},

phr-ProhibitTimer                   ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

phr-Tx-PowerFactorChange            ENUMERATED {dB1, dB3, dB6, infinity},

multiplePHR                         BOOLEAN,

dummy                               BOOLEAN,

phr-Type2OtherCell                  BOOLEAN,

phr-ModeOtherCG                     ENUMERATED {real, virtual},

...,

[[

mpe-Reporting-FR2-r16               SetupRelease { MPE-Config-FR2-r16 }                     OPTIONAL     -- Need M

]],

[[

mpe-Reporting-FR2-r17               SetupRelease { MPE-Config-FR2-r17 }                     OPTIONAL,    -- Need M

twoPHRMode-r17                      ENUMERATED {enabled}                                    OPTIONAL     -- Need R

]],

[[

phr-AssumedPUSCH-Reporting-r18      ENUMERATED {enabled}                                    OPTIONAL,    -- Need R

dpc-Reporting-FR1-r18               ENUMERATED {enabled}                                    OPTIONAL     -- Need R

]]

}

MPE-Config-FR2-r16 ::=              SEQUENCE {

mpe-ProhibitTimer-r16               ENUMERATED {sf0, sf10, sf20, sf50, sf100, sf200, sf500, sf1000},

mpe-Threshold-r16                   ENUMERATED {dB3, dB6, dB9, dB12}

}

MPE-Config-FR2-r17 ::=              SEQUENCE {

mpe-ProhibitTimer-r17               ENUMERATED {sf0, sf10, sf20, sf50, sf100, sf200, sf500, sf1000},

mpe-Threshold-r17                   ENUMERATED {dB3, dB6, dB9, dB12},

numberOfN-r17                       INTEGER(1..4),

...

}

-- TAG-PHR-CONFIG-STOP

-- ASN1STOP

PHR-Config field descriptions

dpc-Reporting-FR1 Indicates if the delta power class (DPC) is reported, as specified in TS 38.321 [3].

dummy This field is not used in this version of the specification and the UE ignores the received value.

mpe-ProhibitTimer Value in number of subframes for MPE reporting, as specified in TS 38.321 [3]. Value sf10 corresponds to 10 subframes, and so on.

mpe-Reporting-FR2 Indicates whether the UE shall report MPE P-MPR in the PHR MAC control element, as specified in TS 38.321 [3].

mpe-Threshold Value of the P-MPR threshold in dB for reporting MPE P-MPR when FR2 is configured, as specified in TS 38.321 [3]. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

multiplePHR Indicates if power headroom shall be reported using the Single Entry PHR MAC control element or Multiple Entry PHR MAC control element defined in TS 38.321 [3]. True means to use Multiple Entry PHR MAC control element and False means to use the Single Entry PHR MAC control element defined in TS 38.321 [3]. The network configures this field to true for MR-DC and UL CA for NR, and to false in all other cases.

numberOfN Number of reported P-MPR values in a PHR MAC CE.

phr-AssumedPUSCH-Reporting Indicates if the PHR with an assumed PUSCH is reported, as specified in TS 38.321 [3]. The network ensures phr-AssumedPUSCH-Reporting-r18 and twoPHRMode-r17 are not configured at the same time for a UE.

phr-ModeOtherCG Indicates the mode (i.e. real or virtual) used for the PHR of the activated cells that are part of the other Cell Group (i.e. MCG or SCG), when DC is configured. If the UE is configured with only one cell group (no DC), it ignores the field.

phr-PeriodicTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. Value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

phr-ProhibitTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. Value sf0 corresponds to 0 subframe, value sf10 corresponds to 10 subframes, value sf20 corresponds to 20 subframes, and so on.

phr-Tx-PowerFactorChange Value in dB for PHR reporting as specified in TS 38.321 [3]. Value dB1 corresponds to 1 dB, dB3 corresponds to 3 dB and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

phr-Type2OtherCell If set to true, the UE shall report a PHR type 2 for the SpCell of the other MAC entity. See TS 38.321 [3], clause 5.4.6. Network sets this field to false if the UE is not configured with an E-UTRA MAC entity.

twoPHRMode Indicates if the power headroom shall be reported as two PHRs (each PHR associated with a SRS resource set) is enabled or not. The network ensures phr-AssumedPUSCH-Reporting-r18 and twoPHRMode-r17 are not configured at the same time for a UE.

#### –	PhysCellId

The PhysCellId identifies the physical cell identity (PCI).

PhysCellId information element

-- ASN1START

-- TAG-PHYSCELLID-START

PhysCellId ::=                      INTEGER (0..1007)

-- TAG-PHYSCELLID-STOP

-- ASN1STOP

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }                                 OPTIONAL,   -- Need M

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16          ENUMERATED {semiStatic, dynamic}           OPTIONAL,   -- Cond twoPUCCHgroup

p-NR-FR2-r16                                              P-Max                                     OPTIONAL,   -- Need R

p-UE-FR2-r16                                              P-Max                                     OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR1-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR2-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

pdsch-HARQ-ACK-Codebook-r16            ENUMERATED {enhancedDynamic}                                 OPTIONAL,   -- Need R

nfi-TotalDAI-Included-r16              ENUMERATED {true}                                            OPTIONAL,   -- Need R

ul-TotalDAI-Included-r16               ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedback-r16     ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

downlinkAssignmentIndexDCI-0-2-r16     ENUMERATED { enabled }                                       OPTIONAL,   -- Need S

downlinkAssignmentIndexDCI-1-2-r16     ENUMERATED {n1, n2, n4}                                      OPTIONAL,   -- Need S

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}               OPTIONAL,   -- Need M

ackNackFeedbackMode-r16                ENUMERATED {joint, separate}                                 OPTIONAL,   -- Need R

pdcch-BlindDetectionCA-CombIndicator-r16 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r16 }  OPTIONAL,   -- Need M

pdcch-BlindDetection2-r16                SetupRelease { PDCCH-BlindDetection2-r16 }                 OPTIONAL,   -- Need M

pdcch-BlindDetection3-r16                SetupRelease { PDCCH-BlindDetection3-r16 }                 OPTIONAL,   -- Need M

bdFactorR-r16                          ENUMERATED {n1}                                              OPTIONAL    -- Need R

]],

[[

-- start of enhanced Type3 feedback

pdsch-HARQ-ACK-EnhType3ToAddModList-r17   SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17

OPTIONAL,   -- Need N

pdsch-HARQ-ACK-EnhType3ToReleaseList-r17  SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3SecondaryToAddModList-r17   SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3SecondaryToReleaseList-r17  SEQUENCE (SIZE(1..maxNrofEnhType3HARQ-ACK-r17)) OF PDSCH-HARQ-ACK-EnhType3Index-r17

OPTIONAL,    -- Need N

pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup-r17 ENUMERATED {enabled}                        OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-EnhType3DCI-Field-r17                ENUMERATED {enabled}                            OPTIONAL,   -- Need R

-- end of enhanced Type3 feedback

-- start of triggering of HARQ-ACK re-transmission on a PUCCH resource

pdsch-HARQ-ACK-Retx-r17                   ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup-r17  ENUMERATED {enabled}                                    OPTIONAL,   -- Cond twoPUCCHgroup

-- end of triggering of HARQ-ACK re-transmission on a PUCCH resource

-- start of PUCCH Cell switching

pucch-sSCell-r17                         SCellIndex                                                    OPTIONAL,   -- Need R

pucch-sSCellSecondaryPUCCHgroup-r17      SCellIndex                                                    OPTIONAL,   -- Cond twoPUCCHgroup

pucch-sSCellDyn-r17                      ENUMERATED {enabled}                                       OPTIONAL,   -- Need R

pucch-sSCellDynSecondaryPUCCHgroup-r17   ENUMERATED {enabled}                                       OPTIONAL,   -- Cond twoPUCCHgroup

pucch-sSCellPattern-r17                      SEQUENCE (SIZE(1..maxNrofSlots)) OF INTEGER (0..1)        OPTIONAL,   -- Need R

pucch-sSCellPatternSecondaryPUCCHgroup-r17   SEQUENCE (SIZE(1..maxNrofSlots)) OF INTEGER (0..1)        OPTIONAL,   -- Cond twoPUCCHgroup

-- end of PUCCH Cell switching

uci-MuxWithDiffPrio-r17           ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

uci-MuxWithDiffPrioSecondaryPUCCHgroup-r17     ENUMERATED {enabled}                         OPTIONAL,   -- Cond twoPUCCHgroup

simultaneousPUCCH-PUSCH-r17       ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup-r17       ENUMERATED {enabled}                  OPTIONAL,   -- Cond twoPUCCHgroup

prioLowDG-HighCG-r17              ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

prioHighDG-LowCG-r17              ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

twoQCLTypeDforPDCCHRepetition-r17 ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

multicastConfig-r17               SetupRelease { MulticastConfig-r17 }                      OPTIONAL,   -- Need M

pdcch-BlindDetectionCA-CombIndicator-r17 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r17 }  OPTIONAL   -- Need M

]],

[[

simultaneousSR-PUSCH-diffPUCCH-Groups-r17      ENUMERATED {enabled}                         OPTIONAL    -- Cond twoPUCCHgroup

]],

[[

intraBandNC-PRACH-simulTx-r17     ENUMERATED {enabled}                                      OPTIONAL    -- Need R

]],

[[

pdcch-BlindDetection4-r17         SetupRelease { PDCCH-BlindDetection4-r17 }                OPTIONAL    -- Need M

]],

[[

simultaneousPUCCH-PUSCH-SamePriority-r17       ENUMERATED {enabled}                         OPTIONAL,   -- Need R

simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup-r17       ENUMERATED {enabled}     OPTIONAL    -- Cond twoPUCCHgroup

]],

[[

ncr-RNTI-r18                      RNTI-Value                                                OPTIONAL,   -- Cond NCR

cellDTRX-DCI-config-r18           SetupRelease { CellDTRX-DCI-config-r18 }                  OPTIONAL,   -- Need M

twoQCL-TypeD-ForMultiDCI-r18      ENUMERATED {enabled}                                      OPTIONAL,   -- Need R

enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18           ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableDiffPUCCH-Resource-r18                                       ENUMERATED {enabled}     OPTIONAL,   -- Need R

enableDiffCB-Size-r18                                              ENUMERATED {enabled}     OPTIONAL    -- Need R

]],

[[

lpwus-Config-r19                           SetupRelease { LPWUS-Config-r19 }                OPTIONAL,   -- Need M

adap-SSBPeriodicityIndication-RNTI-r19     SetupRelease {RNTI-Value}                        OPTIONAL,   -- Need M

adap-SSB-SizeDCI-2-9-r19                   SetupRelease {SizeDCI-2-9-r19}                   OPTIONAL    -- Need M

]]

}

PDSCH-HARQ-ACK-EnhType3-r17 ::=         SEQUENCE {

pdsch-HARQ-ACK-EnhType3Index-r17    PDSCH-HARQ-ACK-EnhType3Index-r17,

applicable-r17   CHOICE {

perCC                            SEQUENCE (SIZE (1..maxNrofServingCells)) OF INTEGER (0..1),

perHARQ                          SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (16))

},

pdsch-HARQ-ACK-EnhType3NDI-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3CBG-r17         ENUMERATED {true}                                            OPTIONAL,   -- Need S

...,

[[

perHARQ-Ext-r17                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF BIT STRING (SIZE (32)) OPTIONAL -- Need R

]]

}

PDSCH-HARQ-ACK-EnhType3Index-r17 ::=    INTEGER (0..maxNrofEnhType3HARQ-ACK-1-r17)

PDCCH-BlindDetection ::=                INTEGER (1..15)

DCP-Config-r16 ::=                  SEQUENCE {

ps-RNTI-r16                         RNTI-Value,

ps-Offset-r16                       INTEGER (1..120),

sizeDCI-2-6-r16                     INTEGER (1..maxDCI-2-6-Size-r16),

ps-PositionDCI-2-6-r16              INTEGER (0..maxDCI-2-6-Size-1-r16),

ps-WakeUp-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitPeriodicL1-RSRP-r16      ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitOtherPeriodicCSI-r16     ENUMERATED {true}                                               OPTIONAL    -- Need S

}

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

PDCCH-BlindDetectionCA-CombIndicator-r16 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r16                  INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                  INTEGER (1..15)

}

PDCCH-BlindDetection2-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection3-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection4-r17 ::=                INTEGER (1..15)

MulticastConfig-r17 ::=                 SEQUENCE {

pdsch-HARQ-ACK-CodebookListMulticast-r17    SetupRelease { PDSCH-HARQ-ACK-CodebookList-r16}         OPTIONAL,   -- Need M

type1CodebookGenerationMode-r17             ENUMERATED { mode1, mode2}                              OPTIONAL    -- Need M

}

PDCCH-BlindDetectionCA-CombIndicator-r17 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r17                  INTEGER (1..15)                                        OPTIONAL,   -- Need R

pdcch-BlindDetectionCA2-r17                  INTEGER (1..15)                                        OPTIONAL,   -- Need R

pdcch-BlindDetectionCA3-r17                  INTEGER (1..15)

}

CellDTRX-DCI-config-r18 ::=         SEQUENCE {

cellDTRX-RNTI-r18                   RNTI-Value,

sizeDCI-2-9-r18                     INTEGER (1..maxDCI-2-9-Size-r18)

}

LPWUS-Config-r19 ::=                  SEQUENCE {

lpwus-MvalueAndSeqConfigFR1-r19       CHOICE {

nOne                                  SEQUENCE {

lpwus-OverlaidSeqRoots                SEQUENCE {

root1-r19                             INTEGER (1..131),

root2-r19                             INTEGER (1..131)                                  OPTIONAL    -- Need R

},

lpwus-OverlaidSeqNum-r19              ENUMERATED {n1, n2, n4, n8, n16}                      OPTIONAL    -- Need R

},

nTwo                                  SEQUENCE {

lpwus-OverlaidSeqRoots                SEQUENCE {

root1-r19                             INTEGER (1..61),

root2-r19                             INTEGER (1..61)                                   OPTIONAL    -- Need R

},

lpwus-OverlaidSeqNum-r19              ENUMERATED {n1, n2, n4, n8}                           OPTIONAL    -- Need R

},

nFour                                 SEQUENCE {

lpwus-OverlaidSeqRoots                SEQUENCE {

root1-r19                             INTEGER (1..31),

root2-r19                             INTEGER (1..31)                                   OPTIONAL    -- Need R

},

lpwus-OverlaidSeqNum-r19              ENUMERATED {n1, n2, n4}                               OPTIONAL    -- Need R

}

}                                                                                                   OPTIONAL,   -- Cond FR1-Only

lpwus-MvalueAndSeqConfigFR2-r19       CHOICE {

nOne                                  SEQUENCE {

lpwus-OverlaidSeqRoots                SEQUENCE {

root1-r19                             INTEGER (1..131),

root2-r19                             INTEGER (1..131)                                  OPTIONAL    -- Need R

},

lpwus-OverlaidSeqNum-SCS-120kHz-r19   ENUMERATED {n1, n2}                                   OPTIONAL,   -- Need R

lpwus-OverlaidSeqNum-SCS-60kHz-r19    ENUMERATED {n1, n2, n4}                               OPTIONAL    -- Need R

},

nTwo                                  SEQUENCE {

lpwus-OverlaidSeqRoots                SEQUENCE {

root1-r19                             INTEGER (1..61),

root2-r19                             INTEGER (1..61)                                   OPTIONAL    -- Need R

},

lpwus-OverlaidSeqNum-SCS-60kHz-r19    ENUMERATED {n1, n2}                                   OPTIONAL    -- Need R

}

}                                                                                                   OPTIONAL,   -- Cond FR2-Only

lpwus-StartRB-r19                     INTEGER (0..263)                                              OPTIONAL,   -- Need R

lpwus-TimeOffset11-r19                ENUMERATED {ffs}                                              OPTIONAL,   -- Cond Option11

lpwus-TimeOffset12-r19                ENUMERATED {ffs}                                              OPTIONAL,   -- Cond Option12

lpwus-Mo11-r19                        ENUMERATED {ffs}                                              OPTIONAL,   -- Cond Option11

lpwus-Mo12-r19                        ENUMERATED {ffs}                                              OPTIONAL,   -- Cond Option12

lpwus-NumOfMo11-r19                   ENUMERATED {ffs}                                              OPTIONAL,   -- Cond Option11

lpwus-NumOfMo12-r19                   ENUMERATED {ffs}                                              OPTIONAL,   -- Cond Option12

lpwus-TCI-States-r19                  ENUMERATED {n1, n2, n3}                                       OPTIONAL,   -- Need R

lpwus-NominalMoDuration-r19           ENUMERATED {ffs}                                              OPTIONAL,   -- Need R

lpwus-ActualDuration-r19              ENUMERATED {ffs}                                              OPTIONAL,   -- Need R

lpwus-AvailableSlot-r19               CHOICE {

n10                                   BIT STRING (SIZE (10)),

n20                                   BIT STRING (SIZE (20)),

n40                                   BIT STRING (SIZE (40))

}                                                                                                   OPTIONAL,   -- Need R

lpwus-AvailableSymbol-r19             CHOICE {

oneSlot                               BIT STRING (SIZE (14)),

twoSlots                              BIT STRING (SIZE (28))

}                                                                                                   OPTIONAL,   -- Need R

lpwus-TransmitOtherPeriodicCSI-r19    ENUMERATED {true}                                             OPTIONAL,   -- Need S

lpwus-TransmitPeriodicL1-RSRP-r19     ENUMERATED {true}                                             OPTIONAL,   -- Need S

lpwus-Codepoint-r19                   SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (1..5)),

lpwus-PDCCH-MonitoringTimer-r19       ENUMERATED {ffs}                                              OPTIONAL    -- Cond Option12

}

SizeDCI-2-9-r19 ::=                       INTEGER (1..maxDCI-2-9-Size-r18)

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

Editor’s NOTE: On the RRC parameters with value of ENUMERATED {ffs}, to be updated according to further RAN1 inputs.

PhysicalCellGroupConfig field descriptions

ackNackFeedbackMode Indicates which among the joint and separate ACK/NACK feedback modes to use within a slot as specified in TS 38.213 [13] (clause 9).

adap-SSB-SizeDCI-2-9 The size of DCI format 2_9 for SSB burst periodicity switching(see TS 38.212, subclause 7.3.1.3.10).

adap-SSBPeriodicityIndication-RNTI The RNTI value for scrambling CRC of DCI format 2_9 for SSB burst periodicity switching for Scell(s).

bdFactorR Parameter for determining and distributing the maximum numbers of BD/CCE for mPDCCH based mPDSCH transmission as specified in TS 38.213 [13] Clause 10.1.

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

downlinkAssignmentIndexDCI-0-2 Indicates if "Downlink assignment index" is present or absent in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is absent, then 0 bit for "Downlink assignment index" in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is present, then the bitwidth of "Downlink assignment index" in DCI format 0_2 is defined in the same was as that in DCI format 0_1 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

downlinkAssignmentIndexDCI-1-2 Configures the number of bits for "Downlink assignment index" in DCI format 1_2. If the field is absent, then 0 bit is applied for "Downlink assignment index" in DCI format 1_2. Note that 1 bit and 2 bits are applied if only one serving cell is configured in the DL and pdsch-HARQ-ACK-Codebook is set to dynamic. 4 bits is applied if more than one serving cell are configured in the DL and pdsch-HARQ-ACK-Codebook is set to dynamic (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

enableDiffCB-Size This field indicates whether a different codebook size from the size determined based on HARQ-ACK information associated with PDSCH reception(s) scheduled before a UL grant, is determined or not to include HARQ-ACK information associated with PDSCH reception(s) scheduled after a UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition. If enabled, a different HARQ codebook size might be determined. See clause 9 in TS 38.213 [13].

enableDiffPUCCH-Resource This field indicates whether a different PUCCH resource in time domain in a slot from the PUCCH resource indicated by the last DCI format before a UL grant in the slot, is determined or not to include HARQ-ACK information associated with PDSCH reception(s) scheduled after a UL grant scheduling a PUSCH transmission with repetitions and the HARQ-ACK information are multiplexed on a repetition of the PUSCH transmission other than a first repetition in the same slot. If enabled, a different PUCCH resource in time domain might be determined. See clause 9 in TS 38.213 [13].

enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant If enabled, UE multiplexes Type-1 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. See clause 9 in TS 38.213 [13]. This feature is not simultaneously enabled with PUCCH cell switching.

enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant If enabled, UE multiplexes Type-2 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. See clause 9 in TS 38.213 [13]. This feature is not simultaneously enabled with PUCCH cell switching.

enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant If enabled, UE multiplexes Type-3 HARQ-ACK codebook on a repetition of a PUSCH transmission other than a first repetition, where the HARQ-ACK codebook includes HARQ-ACK information associated with PDSCH reception(s) scheduled after the UL grant scheduling the PUSCH transmission. See clause 9 in TS 38.213 [13]. This feature is not simultaneously enabled with PUCCH cell switching.

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUCCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clause 9.1.2.1). If the field harq-ACK SpatialBundlingPUCCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUCCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clause 9.1.2.1). When the field is absent, the use of spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUCCH. See TS 38.213 [13], clause 9.1.2.1. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUSCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). If the field harq-ACK SpatialBundlingPUSCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUSCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). When the field is absent, the use of spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUSCH. See TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

intraBandNC-PRACH-simulTx Enables parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA (see TS 38.213 [13], clause 8.1 and TS 38.214 [19], clause 6.2.1). This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

lpwus-Config Configuration for UE to use LP-WUS on PCell and/or PSCell to control the PDCCH monitoring as specified in TS 38.321 [3] Clause 5.7. The network will not configure lpwus-Config and dcp-Config for a UE simultaneously.

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the mcs-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

ncr-RNTI RNTI value for NCR-MT, used to scramble the PDCCHs carrying side control information (see TS 38.213 [13], clause 10.1).

nfi-TotalDAI-Included Indicates whether the NFI and total DAI fields of the non-scheduled PDSCH group is included in the non-fallback DL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

nrdc-PCmode-FR1 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 1 (FR1) (see TS 38.213 [13], clause 7.6).

nrdc-PCmode-FR2 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 2 (FR2) (see TS 38.213 [13], clause 7.6).

pdcch-BlindDetection, pdcch-BlindDetection2, pdcch-BlindDetection3, pdcch-BlindDetection4 Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC. The network configures pdcch-BlindDetection2 only if the UE is in NR-DC with at least one downlink cell using Rel-16 PDCCH monitoring capability. The network configures pdcch-BlindDetection3 only if the UE is in NR-DC with at least one downlink cell using Rel-15 PDCCH monitoring capability. The network configures pdcch-BlindDetection4 only if the UE is in NR-DC with at least one downlink cell using Rel-17 PDCCH monitoring capability.

pdcch-BlindDetectionCA-CombIndicator Configure one combination of pdcch-BlindDetectionCA1 (for R15) and pdcch-BlindDetectionCA2 (for R16) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 as UE capability. The combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 configured by pdcch-BlindDetectionCA-CombIndicator is from the more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 reported by UE (see TS 38.213 [13], clause 10). pdcch-BlindDetectionCA-CombIndicator-r17 is used to configure one combination of pdcch-BlindDetectionCA1 (for R15), pdcch-BlindDetectionCA2 (for R16) and pdcch-BlindDetectionCA3 (for R17) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2 and pdcch-BlindDetectionCA3 as UE capability. The combination of pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2 and pdcch-BlindDetectionCA3 configured by pdcch-BlindDetectionCA-CombIndicator-r17 is from the more than one combination of pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2 and pdcch-BlindDetectionCA3 reported by UE (see TS 38.213 [13], clause 10). pdcch-BlindDetectionCA-CombIndicator-r16 and pdcch-BlindDetectionCA-CombIndicator-r17 are not configured simultaneously.

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and non-CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). If pdsch-HARQ-ACK-Codebook-r16 is signalled, UE shall ignore the pdsch-HARQ-ACK-Codebook (without suffix). For the HARQ-ACK for sidelink, if pdsch-HARQ-ACK-Codebook-r16 is signalled, the UE uses pdsch-HARQ-ACK-Codebook (without suffix) and ignores pdsch-HARQ-ACK-Codebook-r16. If the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary PUCCH group. Otherwise, this field is applied to the cell group (i.e. for all the cells within the cell group). For the HARQ-ACK for sidelink, if the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary and secondary PUCCH group and the UE ignores pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup.

pdsch-HARQ-ACK-CodebookList A list of configurations for one or two HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured). For the HARQ-ACK for sidelink, the UE uses pdsch-HARQ-ACK-Codebook and ignores pdsch-HARQ-ACK-CodebookList if this field is present.

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). It is configured for secondary PUCCH group.

pdsch-HARQ-ACK-EnhType3DCI-Field, pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup Indicates the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the primary PUCCH group if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH group, or in the secondary PUCCH group if the more than one enhanced Type 3 HARQ-ACK code is configured for the secondary PUCCH group, respectively.

pdsch-HARQ-ACK-EnhType3ToAddModList, pdsch-HARQ-ACK-EnhType3SecondaryToAddModList Configure the list of enhanced Type 3 HARQ-ACK codebooks for the primary PUCCH group and the secondary PUCCH group, respectively. When configured, DCI format 1_1 can request the UE to report A/N for one of the configured enhanced Type 3 HARQ-ACK codebooks in the corresponding PUCCH group (see TS 38.213 [13], clause 9.1.4). The network can configure pdsch-HARQ-ACK-EnhType3SecondaryToAddModList only if secondary PUCCH group is configured.

pdsch-HARQ-ACK-OneShotFeedback When configured, the DCI format 1_1 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-OneShotFeedbackCBG When configured, the DCI format 1_1 can request the UE to include CBG level A/N for each CC with CBG level transmission configured. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-OneShotFeedbackNDI When configured, the DCI format 1_1 can request the UE to include NDI for each A/N reported. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-Retx, pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup When configured, the DCI format 1_1 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource in the primary PUCCH group and the secondary PUCCH group, respectively (see TS 38.213 [13], clause 9.1.5).

prioLowDG-HighCG Enable PHY prioritization for the case where low-priority dynamic grant-PUSCH collides with high-priority configured grant-PUSCH on a BWP of a serving cell (see TS 38.213 [13], clause 9), when the UE has generated transport blocks for both DG-PUSCH and CG-PUSCH as described in TS 38.321 [3].

prioHighDG-LowCG Enable PHY prioritization for the case where high-priority dynamic grant PUSCH collides with low-priority configured grant PUSCH on a BWP of a serving cell (see TS 38.213 [13], clause 9), when the UE has generated transport blocks for both DG-PUSCH and CG-PUSCH as described in TS 38.321 [3].

ps-RNTI RNTI value for scrambling CRC of DCI format 2_6 used for power saving (see TS 38.213 [13], clause 10.1).

ps-Offset The start of the search-time of DCI format 2_6 with CRC scrambled by PS-RNTI relative to the start of the drx-onDurationTimer of Long DRX (see TS 38.213 [13], clause 10.3). Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.

ps-WakeUp Indicates the UE to wake-up if DCI format 2_6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

ps-PositionDCI-2-6 Starting position of UE wakeup and SCell dormancy indication in DCI format 2_6 (see TS 38.213 [13], clause 10.3).

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start.

ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

pucch-sSCell, pucch-sSCellSecondaryPUCCHgroup indictates the alternative PUCCH cells for PUCCH cell switching in the primary and the secondary PUCCH group, respectively. For the primary PUCCH group, it is configured for cells on top of SpCell. For the secondary PUCCH group, it is configured for cell on top of the PUCCH SCell.

pucch-sSCellDyn, pucch-sSCellDynsecondaryPUCCHgroup When configured, PUCCH cell switching based on dynamic indication in DCI format 1_1 is enabled (see TS 38.213 [13], clause 9.A, clause 9.1.5), respectively for the primary PUCCH group and the secondary PUCCH group.

pucch-sSCellPattern, pucch-sSCellPatternSecondaryPUCCHgroup When configured, the UE applies the semi-static PUCCH cell switching (see TS 38.213 [13], clause 9.A) using the time domain pattern of applicable PUCCH cells indicated by this field, respectively for the primary PUCCH group and the secondary PUCCH group.

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC. A UE does not expect to be configured with this parameter in this release of the specification.

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

p-UE-FR2 The maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR2 (configured for the cell group). A UE does not expect to be configured with this parameter in this release of the specification.

simultaneousPUCCH-PUSCH, simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup Enables simultaneous PUCCH and PUSCH transmissions with different priorities for the primary PUCCH group and the secondary PUCCH group, respectively.

simultaneousPUCCH-PUSCH-SamePriority, simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup Enables simultaneous PUCCH and PUSCH transmissions on different cells in different bands with same priority for the primary PUCCH group and the secondary PUCCH group, respectively, as specified in clause 9 of TS 38.213 [13].

simultaneousSR-PUSCH-diffPUCCH-Groups Enables simultaneous SR and PUSCH transmissions in different PUCCH groups (see TS 38.321 [3], clause 5.4.1, clause 5.4.4).

sizeDCI-2-6 Size of DCI format 2_6 (see TS 38.213 [13], clause 10.3).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

twoQCL-TypeD-ForMultiDCI Indicates whether a UE is expected to identify and monitor two QCL-TypeD properties for multiple overlapping CORESETs, where the first QCL-TypeD is associated with coresetPoolIndex value 0, and the second QCL-TypeD is associated with coresetPoolIndex value 1. (See TS 38,213 [13], clause 10).

twoQCLTypeDforPDCCHRepetition Indicates whether a UE is expected UE to identify and monitor two QCL-TypeD properties for multiple overlapping CORESETs in the case of PDCCH repetition.

uci-MuxWithDiffPrio, uci-MuxWithDiffPrio-secondaryPUCCHgroup When configured, enables multiplexing a high-priority (HP) HARQ-ACK UCI and a low-priority (LP) HARQ-ACK UCI into a PUCCH or PUSCH for the primary PUCCH group and the secondary PUCCH group, respectively.

ul-TotalDAI-Included Indicates whether the total DAI fields of the additional PDSCH group is included in the non-fallback UL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.213 [13]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB.

MulticastConfig field descriptions

pdsch-HARQ-ACK-CodebookListMulticast A list of configurations for one or two HARQ-ACK codebooks for MBS multicast. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured).

type1CodebookGenerationMode Indicates the mode of Type-1 HARQ-ACK codebook generation, as specified in TS 38.213 [13]. Mode 1 is based on the k1 values that are in the intersection of K1 set for unicast and K1 set for multicast. Mode 2 is based on the k1 values that are in the union of K1 set for unicast and K1 set for multicast.

PDSCH-HARQ-ACK-EnhType3 field descriptions

pdsch-HARQ-ACK-EnhType3CBG When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include CBG level A/N for each CC with CBG level transmission configured of the enhanced Type 3 HARQ-ACK codebook. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC.

pdsch-HARQ-ACK-EnhType3NDI When configured, the DCI format 1_1 or DCI format 1_2 can request the UE to include NDI for each A/N reported of the enhanced Type 3 HARQ-ACK codebook.

perCC Configures enhanced Type 3 HARQ-ACK codebook using per CC configuration.

perHARQ, perHARQ-Ext Configures enhanced Type 3 HARQ-ACK codebook using per HARQ process and CC configuration. perHARQ-Ext is present only when nrofHARQ-ProcessesForPDSCH-v1700 is present in pdsch-ServingCellConfig of at least one serving cell in the PUCCH group. If perHARQ-Ext is present, the UE ignores perHARQ.

CellDTRX-DCI-config field descriptions

cellDTRX-RNTI The RNTI value for scrambling CRC of DCI format 2_9 for activating and/or deactivating Cell DTX and/or Cell DRX and/or NES mode for CHO indication.

sizeDCI-2-9 The size of DCI format 2_9.

LPWUS-Config field descriptions

lpwus-ActualDuration Indicates the actual duration for LP-WUS in the cell for RRC CONNECTED (see TS 38.213 [13], clause 10.4D), in number of OFDM symbols.

lpwus-AvailableSlot Indicates the available slot(s) for LP-WUS using a unit level bitmap with a periodicity 10, 20, or 40 units (see TS 38.213 [13], clause 10.4D) in RRC CONNECTED, where the unit is one slot if 14-bit symbol level bitmap is used, or the unit is two slots if 28-bits symbol level bitmap is used, as configured by lpwus-AvailableSymbol. The most significant bit of the bit string represents the first unit and the second most significant bit represents the second unit and so on. Value 1 in the bitmap indicates that the corresponding unit is available, value 0 indicates that the corresponding unit is not available. This slot pattern repeats itself continuously.

lpwus-AvailableSymbol Indicates the available symbol(s) for LP-WUS within the slot(s) that are indicated as available for LP-WUS configured by lpwus-AvailableSlot (see TS 38.213 [13], clause 10.4D) in RRC CONNECTED. A 14-bit or 28-bit symbol level bitmap that covers one or two slots can be configured. For oneSlot, the 14 bits represent the symbols within the slot. For twoSlots, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on. Value 1 in the bitmap indicates that the corresponding symbol is available, value 0 indicates that the corresponding symbol is not available.

lpwus-Codepoint Indicates the codepoints that the UE monitors per MO for LP-WUS (see TS 38.213 [13], clause 10.4D).

lpwus-Mo11 Indicates the periodicity and time offset relative to the start of SFN#0 for LP-WUS MO for LP-WUS operation option 1-1 (see TS 38.213 [13], clause 10.4D). [To be updated based on further progress.]

lpwus-Mo12 Indicates the periodicity and time offset relative to the start of SFN#0 for LP-WUS MO for LP-WUS operation option 1-2 (see TS 38.213 [13], clause 10.4D). [To be updated based on further progress.]

lpwus-MvalueAndSeqConfigFR1 Indicates the number of OOK symbols in an OFDM symbol for LP-WUS in FR1 for RRC CONNECTED (corresponding to parameter MWUS, see TS 38.211 [16], clause 7.4.4.1.1) and the LP-WUS sequence configuration in the cell for RRC_IDLE/INACTIVE. A value of nOne means M value is set to 1, a value of nTwo means M value is set to 2, and so on.

lpwus-MvalueAndSeqConfigFR2 Indicates the number of OOK symbols in an OFDM symbol for LP-WUS in FR2 for RRC CONNECTED (corresponding to parameter MWUS, see TS 38.211 [16], clause 7.4.4.1.1) and the LP-WUS sequence configuration in the cell for RRC_IDLE/INACTIVE. A value of nOne means M value is set to 1 only for SCS of 120KHz and 60KHz, a value of nTwo means M value is set to 2 only for SCS of 60KHz

lpwus-NominalMoDuration Indicates the nominal MO duration for LP-WUS in the cell for RRC CONNECTED (see TS 38.213 [13], clause 10.4D), in number of OFDM symbols.

lpwus-NumOfMo11 Indicates the number of the earliest LP-WUS MOs to be monitored by UE from time offset prior to a slot where the drx-onDurationTimer of Long DRX would start for LP-WUS operation option 1-1 (see TS 38.213 [13], clause 10.4D).

lpwus-NumOfMo12 Indicates the number of LP-WUS MOs to be monitored by UE per periodicity for LP-WUS operation option 1-2 (see TS 38.213 [13], clause 10.4D).

lpwus-OverlaidSeqNum Indicates the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for LP-WUS in the cell for RRC CONNECTED (corresponding to parameter Nseq, see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-OverlaidSeqNum-SCS-60kHz Indicates the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for LP-WUS in the cell for RRC CONNECTED for SCS of 60KHz in FR2 (see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-OverlaidSeqNum-SCS-120kHz Indicates the number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for LP-WUS in the cell for RRC CONNECTED for SCS of 120KHz in FR2 (see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-OverlaidSeqRoots Indicates the configuration of overlaid sequence root for LP-WUS in the cell for RRC CONNECTED (see TS 38.211 [16], clause 7.4.4.1.1).

lpwus-PDCCH-MonitoringTimer Indicates the length of the timer for UE to monitor PDCCH after LP-WUS is detected for LP-WUS operation option 1-2 (see TS 38.321 [3], clause 5.7).

lpwus-StartRB Indicates the starting RB of LP-WUS for RRC CONNECTED (see TS 38.213 [13], clause 10.4D). The starting RB index reference to carrier boundary determined by offsetToCarrier corresponding to SCS of the active BWP is configured by gNB.

lpwus-TCI-States Indicates the configuration for UE to derive the active TCI state for LP-WUS in RRC CONNECTED (see TS 38.213 [13], clause 10.4D). Value n1 means the CORESET#1, and value n2 means the CORESET#2, and so on. This field is configured if the UE is not configured with dl-OrJointTCI-StateList for unified TCI state.

lpwus-TimeOffset11 Indicates the time offset determines the start of LP-WUS monitoring relative to the start of the drx-onDurationTimer of Long DRX for LP-WUS operation option 1-1 (see TS 38.213 [13], clause 10.4D). [ffs Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.] The network will not configure lpwus-TimeOffset11 and lpwus-TimeOffset12 for a UE in one cell simultaneously.

lpwus-TimeOffset12 Indicates the time offset determines the start of UE PDCCH monitoring via the start of lpwus-PDCCH-MonitoringTimer after LP-WUS is detected for LP-WUS operation option 1-2 (see TS 38.213 [13], clause 10.4D). [ffs Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.] The network will not configure lpwus-TimeOffset11 and lpwus-TimeOffset12 for a UE in one cell simultaneously.

lpwus-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports during the time given by the configured drx-onDurationTimer if the UE is not indicated to wake-up (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports during the time given by the configured drx-onDurationTimer if the UE is not indicated to wake-up.

lpwus-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) during the time given by the configured drx-onDurationTimer if the UE is not indicated to wake-up (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) during the time given by the configured drx-onDurationTimer if the UE is not indicated to wake-up.

root1 Indicates the first overlaid sequence root for LP-WUS in the cell for RRC CONNECTED (see TS 38.211 [16], clause 7.4.4.1.1).

root2 Indicates the second overlaid sequence root for LP-WUS in the cell for RRC CONNECTED (see TS 38.211 [16], clause 7.4.4.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| NCR | This field is optionally present, Need M for NCR-MT. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in (NG)EN-DC as defined in TS 38.213 [13]. It is absent otherwise. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise, Need R. |
| FR1-Only | This field is mandatory present for an FR1 carrier frequency. It is absent otherwise and UE releases any configured value. |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| Option11 | This field is mandatory present for LP-WUS operation option 1-1. It is absent otherwise and UE releases any configured value. |
| Option12 | This field is mandatory present for LP-WUS operation option 1-2. It is absent otherwise and UE releases any configured value. |

#### –	PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE is specified in TS 23.003 [21].

PLMN-Identity information element

-- ASN1START

-- TAG-PLMN-IDENTITY-START

PLMN-Identity ::=                   SEQUENCE {

mcc                                 MCC                 OPTIONAL,                   -- Cond MCC

mnc                                 MNC

}

MCC ::=                             SEQUENCE (SIZE (3)) OF MCC-MNC-Digit

MNC ::=                             SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit

MCC-MNC-Digit ::=                   INTEGER (0..9)

-- TAG-PLMN-IDENTITY-STOP

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [21].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [21].

| Conditional Presence | Explanation |
| --- | --- |
| MCC | This field is mandatory present when PLMN-Identity is not used in a list or if it is the first entry of PLMN-Identity in a list. Otherwise it is optionally present, Need S. |

#### –	PLMN-IdentityInfoList

The IE PLMN-IdentityInfoList includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...,

[[

iab-Support-r16                     ENUMERATED {true}                                               OPTIONAL       -- Need S

]],

[[

trackingAreaList-r17                SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode             OPTIONAL,      -- Need R

gNB-ID-Length-r17                   INTEGER (22..32)                                                OPTIONAL    -- Cond eventID-TSS

]],

[[

mobileIAB-Support-r18               ENUMERATED {true}                                               OPTIONAL       -- Need S

]]

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

PLMN-IdentityInfo field descriptions

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (per PLMN), as defined in TS 38.304 [20]. This field is ignored by IAB-MT and NCR-MT.

gNB-ID-Length Indicates the length of the gNB ID out of the 36-bit long cellIdentity. This field is always present if the mobileIAB-Support is broadcasted.

iab-Support This field combines both the support of IAB and the cell status for IAB. If the field is present, the cell supports IAB and the cell is also considered as a candidate for cell (re)selection for IAB-node; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell.

mobileIAB-Support This field indicates the support of mobile IAB and whether the cell can be considered as a candidate for cell (re)selection for mobile IAB-node. This field is absent if mobileIAB-Cell is broadcasted in the cell. If the field is absent, the cell is barred for mobile IAB-node.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell.

trackingAreaList List of Tracking Areas to which the cell indicated by cellIdentity field belongs. If this field is present, network does not configure trackingAreaCode. Total number of different TACs across different PLMN-IdentityInfos shall not exceed maxTAC. This field is only present in an NTN cell.

| Conditional Presence | Explanation |
| --- | --- |
| eventID-TSS | If eventID-TSS is present in SIB9, this field is mandatory present in the PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1. Otherwise, the field is optionally present, Need R. |

#### –	PLMN-IdentityList2

Includes a list of PLMN identities.

PLMN-IdentityList2 information element

-- ASN1START

-- TAG-PLMNIDENTITYLIST2-START

PLMN-IdentityList2-r16 ::= SEQUENCE (SIZE (1..16)) OF PLMN-Identity

-- TAG-PLMNIDENTITYLIST2-STOP

-- ASN1STOP

#### –	PRB-Id

The IE PRB-Id identifies a Physical Resource Block (PRB) position within a carrier.

PRB-Id information element

-- ASN1START

-- TAG-PRB-ID-START

PRB-Id ::=                          INTEGER (0..maxNrofPhysicalResourceBlocks-1)

-- TAG-PRB-ID-STOP

-- ASN1STOP

#### –	PTRS-DownlinkConfig

The IE PTRS-DownlinkConfig is used to configure downlink phase tracking reference signals (PTRS) (see TS 38.214 [19] clause 5.1.6.3)

PTRS-DownlinkConfig information element

-- ASN1START

-- TAG-PTRS-DOWNLINKCONFIG-START

PTRS-DownlinkConfig ::=             SEQUENCE {

frequencyDensity                    SEQUENCE (SIZE (2)) OF INTEGER (1..276)                                 OPTIONAL,   -- Need S

timeDensity                         SEQUENCE (SIZE (3)) OF INTEGER (0..29)                                  OPTIONAL,   -- Need S

epre-Ratio                          INTEGER (0..3)                                                          OPTIONAL,   -- Need S

resourceElementOffset               ENUMERATED { offset01, offset10, offset11 }                             OPTIONAL,   -- Need S

...,

[[

maxNrofPorts-r16                    ENUMERATED {n1, n2}                                                     OPTIONAL    -- Need R

]]

}

-- TAG-PTRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

PTRS-DownlinkConfig field descriptions

epre-Ratio EPRE ratio between PTRS and PDSCH. Value 0 corresponds to the codepoint "00" in tables 4.1-2 and 4.1-2A. Value 1 corresponds to codepoint "01", and so on. If the field is not provided, the UE applies value 0 (see TS 38.214 [19], clause 4.1).

frequencyDensity Presence and frequency density of DL PT-RS as a function of Scheduled BW. If the field is absent, the UE uses K_PT-RS = 2 (see TS 38.214 [19], clause 5.1.6.3, table 5.1.6.3-2).

maxNrofPorts The maximum number of DL PTRS ports specified in TS 38.214 [19] (clause 5.1.6.3). 2 PT-RS ports can only be configured for a DL BWP that is configured, as specified in TS 38.214 [19] clause 5.1, with a mode where a single PDSCH has association between the DM-RS ports and the TCI states as defined in TS 38.214 [19] clause 5.1.6.2.

resourceElementOffset Indicates the subcarrier offset for DL PTRS. If the field is absent, the UE applies the value offset00 (see TS 38.211 [16], clause 7.4.1.2.2).

timeDensity Presence and time density of DL PT-RS as a function of MCS. The value 29 is only applicable for MCS Table 5.1.3.1-1 (TS 38.214 [19]). If the field is absent, the UE uses L_PT-RS = 1 (see TS 38.214 [19], clause 5.1.6.3, table 5.1.6.3-1).

#### –	PTRS-UplinkConfig

The IE PTRS-UplinkConfig is used to configure uplink Phase-Tracking-Reference-Signals (PTRS).

PTRS-UplinkConfig information element

-- ASN1START

-- TAG-PTRS-UPLINKCONFIG-START

PTRS-UplinkConfig ::=                   SEQUENCE {

transformPrecoderDisabled               SEQUENCE {

frequencyDensity                    SEQUENCE (SIZE (2)) OF INTEGER (1..276)                 OPTIONAL,   -- Need S

timeDensity                         SEQUENCE (SIZE (3)) OF INTEGER (0..29)                  OPTIONAL,   -- Need S

maxNrofPorts                        ENUMERATED {n1, n2},

resourceElementOffset               ENUMERATED {offset01, offset10, offset11 }              OPTIONAL,   -- Need S

ptrs-Power                          ENUMERATED {p00, p01, p10, p11}

}                                                                                               OPTIONAL,   -- Need R

transformPrecoderEnabled                SEQUENCE {

sampleDensity                           SEQUENCE (SIZE (5)) OF INTEGER (1..276),

timeDensityTransformPrecoding           ENUMERATED {d2}                                     OPTIONAL    -- Need S

}                                                                                               OPTIONAL,   -- Need R

...,

[[

maxNrofPorts-SDM-r18                     ENUMERATED {n1, n2}                                     OPTIONAL    -- Need R

]]

}

-- TAG-PTRS-UPLINKCONFIG-STOP

-- ASN1STOP

PTRS-UplinkConfig field descriptions

frequencyDensity Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW If the field is absent, the UE uses K_PT-RS = 2 (see TS 38.214 [19], clause 6.1).

maxNrofPorts, maxNrofPorts-SDM The maximum number of UL PTRS ports for CP-OFDM (see TS 38.214 [19], clause 6.2.3.1).

ptrs-Power UL PTRS power boosting factor per PTRS port (see TS 38.214 [19], clause 6.1, table 6.2.3.1.3).

resourceElementOffset Indicates the subcarrier offset for UL PTRS for CP-OFDM. If the field is absent, the UE applies the value offset00 (see TS 38.211 [16], clause 6.4.1.2.2).

sampleDensity Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn, n=0,1,2,3,4}, that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should use depending on the scheduled BW, see TS 38.214 [19], clause 6.1, table 6.2.3.2-1.

timeDensity Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS If the field is absent, the UE uses L_PT-RS = 1 (see TS 38.214 [19], clause 6.1).

timeDensityTransformPrecoding Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM. If the field is absent, the UE applies value d1 (see TS 38.214 [19], clause 6.1).

transformPrecoderDisabled Configuration of UL PTRS without transform precoder (with CP-OFDM).

transformPrecoderEnabled Configuration of UL PTRS with transform precoder (DFT-S-OFDM).

#### –	PUCCH-Config

The IE PUCCH-Config is used to configure UE specific PUCCH parameters (per BWP).

PUCCH-Config information element

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-Config ::=                        SEQUENCE {

resourceSetToAddModList                 SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet   OPTIONAL, -- Need N

resourceSetToReleaseList                SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N

resourceToAddModList                    SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource         OPTIONAL, -- Need N

resourceToReleaseList                   SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId       OPTIONAL, -- Need N

format1                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format3                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format4                                 SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

schedulingRequestResourceToAddModList   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig

OPTIONAL, -- Need N

schedulingRequestResourceToReleaseList  SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId

OPTIONAL, -- Need N

multi-CSI-PUCCH-ResourceList            SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId                            OPTIONAL, -- Need M

dl-DataToUL-ACK                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)                             OPTIONAL, -- Need M

spatialRelationInfoToAddModList         SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseList        SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

pucch-PowerControl                      PUCCH-PowerControl                                                    OPTIONAL, -- Need M

...,

[[

resourceToAddModListExt-v1610           SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceExt-v1610  OPTIONAL, -- Need N

dl-DataToUL-ACK-r16                     SetupRelease { DL-DataToUL-ACK-r16 }                                  OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-1-r16          SetupRelease { UL-AccessConfigListDCI-1-1-r16 }                       OPTIONAL, -- Need M

subslotLengthForPUCCH-r16               CHOICE {

normalCP-r16                        ENUMERATED {n2,n7},

extendedCP-r16                      ENUMERATED {n2,n6}

}                                                                                                             OPTIONAL, -- Need R

dl-DataToUL-ACK-DCI-1-2-r16             SetupRelease { DL-DataToUL-ACK-DCI-1-2-r16}                           OPTIONAL, -- Need M

numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2-r16  INTEGER (0..3)                                             OPTIONAL, -- Need R

dmrs-UplinkTransformPrecodingPUCCH-r16  ENUMERATED {enabled}                                                  OPTIONAL,  -- Cond PI2-BPSK

spatialRelationInfoToAddModListSizeExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfo

OPTIONAL, -- Need N

spatialRelationInfoToReleaseListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofSpatialRelationInfosDiff-r16)) OF PUCCH-SpatialRelationInfoId

OPTIONAL, -- Need N

spatialRelationInfoToAddModListExt-v1610  SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF PUCCH-SpatialRelationInfoExt-r16

OPTIONAL, -- Need N

spatialRelationInfoToReleaseListExt-v1610    SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos-r16)) OF

PUCCH-SpatialRelationInfoId-r16       OPTIONAL, -- Need N

resourceGroupToAddModList-r16           SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroup-r16

OPTIONAL, -- Need N

resourceGroupToReleaseList-r16          SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceGroups-r16)) OF PUCCH-ResourceGroupId-r16

OPTIONAL, -- Need N

sps-PUCCH-AN-List-r16                   SetupRelease { SPS-PUCCH-AN-List-r16 }                                OPTIONAL,  -- Need M

schedulingRequestResourceToAddModListExt-v1610   SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1610

OPTIONAL -- Need N

]],

[[

format0-r17                             SetupRelease { PUCCH-FormatConfig }                                   OPTIONAL, -- Need M

format2Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

format3Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

format4Ext-r17                          SetupRelease { PUCCH-FormatConfigExt-r17 }                            OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-2-r17          SetupRelease { UL-AccessConfigListDCI-1-2-r17 }                       OPTIONAL, -- Need M

mappingPattern-r17                      ENUMERATED {cyclicMapping, sequentialMapping}                         OPTIONAL, -- Need R

powerControlSetInfoToAddModList-r17     SEQUENCE (SIZE (1..maxNrofPowerControlSetInfos-r17)) OF PUCCH-PowerControlSetInfo-r17

OPTIONAL, -- Need N

powerControlSetInfoToReleaseList-r17    SEQUENCE (SIZE (1..maxNrofPowerControlSetInfos-r17)) OF PUCCH-PowerControlSetInfoId-r17

OPTIONAL, -- Need N

secondTPCFieldDCI-1-1-r17               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

secondTPCFieldDCI-1-2-r17               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

dl-DataToUL-ACK-r17                     SetupRelease { DL-DataToUL-ACK-r17 }                                  OPTIONAL, -- Need M

dl-DataToUL-ACK-DCI-1-2-r17             SetupRelease { DL-DataToUL-ACK-DCI-1-2-r17}                           OPTIONAL, -- Need M

ul-AccessConfigListDCI-1-1-r17          SetupRelease { UL-AccessConfigListDCI-1-1-r17 }                       OPTIONAL, -- Need M

schedulingRequestResourceToAddModListExt-v1700 SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1700

OPTIONAL, -- Need N

dmrs-BundlingPUCCH-Config-r17           SetupRelease { DMRS-BundlingPUCCH-Config-r17 }                        OPTIONAL, -- Need M

dl-DataToUL-ACK-v1700                   SetupRelease { DL-DataToUL-ACK-v1700 }                                OPTIONAL, -- Need M

dl-DataToUL-ACK-MulticastDCI-Format4-1-r17 SetupRelease { DL-DataToUL-ACK-MulticastDCI-Format4-1-r17 }        OPTIONAL, -- Need M

sps-PUCCH-AN-ListMulticast-r17          SetupRelease { SPS-PUCCH-AN-List-r16 }                                OPTIONAL  -- Need M

]],

[[

schedulingRequestResourceToAddModListExt-v1900 SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfigExt-v1900

OPTIONAL, -- Need N

resourceToAddModListExt-v1900                  SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceExt-v1900

OPTIONAL  -- Need N

]]

}

PUCCH-FormatConfig ::=                  SEQUENCE {

interslotFrequencyHopping               ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

additionalDMRS                          ENUMERATED {true}                                                     OPTIONAL, -- Need R

maxCodeRate                             PUCCH-MaxCodeRate                                                     OPTIONAL, -- Need R

nrofSlots                               ENUMERATED {n2,n4,n8}                                                 OPTIONAL, -- Need S

pi2BPSK                                 ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

simultaneousHARQ-ACK-CSI                ENUMERATED {true}                                                     OPTIONAL  -- Need R

}

PUCCH-FormatConfigExt-r17 ::=           SEQUENCE {

maxCodeRateLP-r17                       PUCCH-MaxCodeRate                                                     OPTIONAL, -- Need R

...

}

PUCCH-MaxCodeRate ::=                   ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}

-- A set with one or more PUCCH resources

PUCCH-ResourceSet ::=                   SEQUENCE {

pucch-ResourceSetId                     PUCCH-ResourceSetId,

resourceList                            SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-ResourceId,

maxPayloadSize                          INTEGER (4..256)                                                      OPTIONAL  -- Need R

}

PUCCH-ResourceSetId ::=                 INTEGER (0..maxNrofPUCCH-ResourceSets-1)

PUCCH-Resource ::=                      SEQUENCE {

pucch-ResourceId                        PUCCH-ResourceId,

startingPRB                             PRB-Id,

intraSlotFrequencyHopping               ENUMERATED { enabled }                                                OPTIONAL, -- Need R

secondHopPRB                            PRB-Id                                                                OPTIONAL, -- Need R

format                                  CHOICE {

format0                                 PUCCH-format0,

format1                                 PUCCH-format1,

format2                                 PUCCH-format2,

format3                                 PUCCH-format3,

format4                                 PUCCH-format4

}

}

PUCCH-ResourceExt-v1610 ::=             SEQUENCE {

interlaceAllocation-r16                 SEQUENCE {

rb-SetIndex-r16                         INTEGER (0..4),

interlace0-r16                          CHOICE {

scs15                                   INTEGER (0..9),

scs30                                   INTEGER (0..4)

}

}                                                                                                             OPTIONAL,  --Need R

format-v1610                            CHOICE {

interlace1-v1610                        INTEGER (0..9),

occ-v1610                               SEQUENCE {

occ-Length-v1610                        ENUMERATED {n2,n4}                                            OPTIONAL, -- Need M

occ-Index-v1610                         ENUMERATED {n0,n1,n2,n3}                                      OPTIONAL  -- Need M

}

}                                                                                                             OPTIONAL,  -- Need R

...,

[[

format-v1700                            SEQUENCE {

nrofPRBs-r17                            INTEGER (1..16)

}                                                                                                             OPTIONAL,  -- Need R

pucch-RepetitionNrofSlots-r17           ENUMERATED { n1,n2,n4,n8 }                                            OPTIONAL   -- Need R

]],

[[

applyIndicatedTCI-State-r18             ENUMERATED {first, second, both, spare1}                              OPTIONAL, -- Need R

multipanelSFN-Scheme-r18                ENUMERATED {enabled}                                                  OPTIONAL, -- Need R

dl-DataToUL-ACK-r18                     SetupRelease { DL-DataToUL-ACK-r18 }                                  OPTIONAL, -- Need M

dl-DataToUL-ACK-DCI-1-2-r18             SetupRelease { DL-DataToUL-ACK-DCI-1-2-r18}                           OPTIONAL   -- Need M

]]

}

PUCCH-ResourceExt-v1900 ::=    SEQUENCE {

startingPRB-SBFD-r19           PRB-Id                                                                         OPTIONAL, -- Need S

secondHopPRB-SBFD-r19          PRB-Id                                                                         OPTIONAL  -- Need S

}

PUCCH-ResourceId ::=                    INTEGER (0..maxNrofPUCCH-Resources-1)

PUCCH-format0 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format1 ::=                               SEQUENCE {

initialCyclicShift                              INTEGER(0..11),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10),

timeDomainOCC                                   INTEGER(0..6)

}

PUCCH-format2 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (1..2),

startingSymbolIndex                             INTEGER(0..13)

}

PUCCH-format3 ::=                               SEQUENCE {

nrofPRBs                                        INTEGER (1..16),

nrofSymbols                                     INTEGER (4..14),

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-format4 ::=                               SEQUENCE {

nrofSymbols                                     INTEGER (4..14),

occ-Length                                      ENUMERATED {n2,n4},

occ-Index                                       ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex                             INTEGER(0..10)

}

PUCCH-ResourceGroup-r16 ::=                SEQUENCE {

pucch-ResourceGroupId-r16                  PUCCH-ResourceGroupId-r16,

resourcePerGroupList-r16                   SEQUENCE (SIZE (1..maxNrofPUCCH-ResourcesPerGroup-r16)) OF PUCCH-ResourceId

}

PUCCH-ResourceGroupId-r16 ::=              INTEGER (0..maxNrofPUCCH-ResourceGroups-1-r16)

DL-DataToUL-ACK-r16 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)

DL-DataToUL-ACK-r17 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (-1..127)

DL-DataToUL-ACK-v1700 ::=                  SEQUENCE (SIZE (1..8)) OF INTEGER (16..31)

DL-DataToUL-ACK-r18 ::=                    SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)

DL-DataToUL-ACK-DCI-1-2-r16 ::=            SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)

DL-DataToUL-ACK-DCI-1-2-r17 ::=            SEQUENCE (SIZE (1..8)) OF INTEGER (0..127)

DL-DataToUL-ACK-DCI-1-2-r18 ::=            SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)

UL-AccessConfigListDCI-1-1-r16 ::=         SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)

UL-AccessConfigListDCI-1-2-r17 ::=         SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)

UL-AccessConfigListDCI-1-1-r17 ::=         SEQUENCE (SIZE (1..3)) OF INTEGER (0..2)

DL-DataToUL-ACK-MulticastDCI-Format4-1-r17 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

PUCCH-Config field descriptions

dl-DataToUL-ACK, dl-DataToUL-ACK-DCI-1-2 List of timing for given PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). The dl-DataToUL-ACK-v1700 is applicable for NTN and dl-DataToUL-ACK-r17 is applicable for FR2-2. The dl-DataToUL-ACK-r18 is applicable for ATG. If dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-v1700 or dl-DataToUL-ACK-r18 is signalled, UE shall ignore the dl-DataToUL-ACK (without suffix). The value -1 corresponds to "inapplicable value" for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH. The fields dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2-r17 are only applicable for SCS of 480 kHz or 960 kHz. The field dl-DataToUL-ACK-r18 applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2-r18 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

dl-DataToUL-ACK-MulticastDCI-Format4-1 List of timing for given group-common PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK-MulticastDciFormat4-1 applies to DCI format 4_1 for MBS multicast (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

dmrs-BundlingPUCCH-Config Configuration of the parameters for DMRS bundling for PUCCH (see TS 38.214 [19], clause 6.1.7). DMRS bundling for PUCCH is not supported for PUCCH format 0/2. In this release, this is not applicable to FR2-2.

dmrs-UplinkTransformPrecodingPUCCH This field is used for PUCCH formats 3 and 4 according to TS 38.211, Clause 6.4.1.3.3.1.

format0 Parameters that are common for all PUCCH resources of format 0.

format1 Parameters that are common for all PUCCH resources of format 1.

format2 Parameters that are common for all PUCCH resources of format 2.

format3 Parameters that are common for all PUCCH resources of format 3.

format4 Parameters that are common for all PUCCH resources of format 4.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern for when a PUCCH resource used for repetitions of a PUCCH transmission includes first and second spatial settings for FR2, or first and second sets of power control parameters for FR1 (see TS 38.213 [13], clause 9.2.6).

numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2 Configuration of the number of bits for "PUCCH resource indicator" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

powerControlSetInfoToAddModList Configures power control sets for repetition of a PUCCH transmission in FR1. This field is not configured if ul-powerControl is configured in the BWP-UplinkDedicated in which the PUCCH-Config is included.

pucch-PowerControl Configures power control parameters PUCCH transmission.

resourceGroupToAddModList, resourceGroupToReleaseList Lists for adding and releasing groups of PUCCH resources that can be updated simultaneously for spatial relations with a MAC CE.

resourceSetToAddModList, resourceSetToReleaseList Lists for adding and releasing PUCCH resource sets (see TS 38.213 [13], clause 9.2).

resourceToAddModList, resourceToAddModListExt, resourceToReleaseList Lists for adding and releasing PUCCH resources applicable for the UL BWP and serving cell in which the PUCCH-Config is defined. The resources defined herein are referred to from other parts of the configuration to determine which resource the UE shall use for which report. If the network includes of resourceToAddModListExt, it includes the same number of entries, and listed in the same order, as in resourceToAddModList.

secondTPCFieldDCI-1-1, secondTPCFieldDCI-1-2 A second TPC field can be configured via RRC for DCI-1-1 and DCI-1-2. Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to "closedLoopIndex" value = 0 and 1.

spatialRelationInfoToAddModList, spatialRelationInfoToAddModListSizeExt , spatialRelationInfoToAddModListExt Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS. If the list has more than one element, MAC-CE selects a single element (see TS 38.321 [3], clause 5.18.8 and TS 38.213 [13], clause 9.2.2). The UE shall consider entries in spatialRelationInfoToAddModList and in spatialRelationInfoToAddModListSizeExt as a single list, i.e. an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt (or deleted using spatialRelationInfoToReleaseListSizeExt) and vice-versa. If the network includes spatialRelationInfoToAddModListExt, it includes the same number of entries, and listed in the same order, as in the concatenation of spatialRelationInfoToAddModList and of spatialRelationInfoToAddModListSizeExt. If unifiedTCI-StateType is configured for the serving cell, no element in this list is configured.

spatialRelationInfoToReleaseList, spatialRelationInfoToReleaseListSizeExt, spatialRelationInfoToReleaseListExt Lists of spatial relation configurations between a reference RS and PUCCH to be released by the UE.

sps-PUCCH-AN-List Indicates a list of PUCCH resources for DL SPS HARQ ACK. The field maxPayloadSize is absent for the first and the last SPS-PUCCH-AN in the list. If configured, this overrides n1PUCCH-AN in SPS-config.

sps-PUCCH-AN-ListMulticast The field is used to configure the list of PUCCH resources per HARQ ACK codebook for MBS multicast.

subslotLengthForPUCCH Indicates the sub-slot length for sub-slot based PUCCH feedback in number of symbols (see TS 38.213 [13], clause 9). Value n2 corresponds to 2 symbols, value n6 corresponds to 6 symbols, value n7 corresponds to 7 symbols. For normal CP, the value is either n2 or n7. For extended CP, the value is either n2 or n6.

ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 List of the combinations of cyclic prefix extension and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable, respectively, to DCI format 1_1 and DCI format 1_2. The fields ul-AccessConfigListDCI-1-1-r16 and ul-AccessConfigListDCI-1-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.2.2-6). The field ul-AccessConfigListDCI-1-1-r17 indicates a list which only contains UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.2.2-6A).

PUCCH-format3 field descriptions

nrofPRBs The supported values are 1,2,3,4,5,6,8,9,10,12,15 and 16. The UE shall ignore this field when format-v1610 is configured.

PUCCH-FormatConfig, PUCCH-FormatConfigExt field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS symbols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 0, 1, 2, 3 or 4 is repeated over multiple slots. For a PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213 [13], clause 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. See TS 38.213 [13], clause 9.2.5.

maxCodeRateLP Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. This field configures additional max code rate in the second entry of PUCCH-ConfigurationList-r16 for multiplexing low-priority (LP) HARQ-ACK and high-priority (HP) UCI in a PUCCH as described Clause 9.2.5.3 of TS 38.213 [13]. The field is absent for the first entry of PUCCH-ConfigurationList-r16.

nrofSlots Number of slots with the same PUCCH. When the field is absent the UE applies the value n1. See TS 38.213 [13], clause 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.5.

rb-SetIndex Indicates the RB set where PUCCH resource is allocated.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213 [13], clause 9.2.5. When the field is absent the UE applies the value off. The field is not applicable for format 0 and 1.

PUCCH-Resource, PUCCH-ResourceExt field descriptions

applyIndicatedTCI-State This field indicates, for PUCCH transmission(s) corresponding to this PUCCH resource, if UE applies the first, the second or both "indicated" UL only TCI or joint TCI as specified in TS 38.213 9.2.2. For PUCCH resources belonging to a PUCCH group, network configures same value.

format Selection of the PUCCH format (format 0 – 4) and format-specific parameters, see TS 38.213 [13], clause 9.2. format0 and format1 are only allowed for a resource in a first PUCCH resource set. format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. The network can only configure format-v1610 when format is set to format2 or format3. The network only configures format-v1700 when format is set to format0, format1 or format4.

interlace0 This is the only interlace of interlaced PUCCH Format 0 and 1 and the first interlace for interlaced PUCCH Format 2 and 3.

interlace1 A second interlace, in addition to interlace 0, as specified in TS 38.213 [13], clause 9.2.1. For 15kHz SCS, values {0..9} are applicable; for 30kHz SCS, values {0..4} are applicable. For 15kHz SCS, the values of interlace1 shall satisfy interlace1=mod(interlace0+X,10) where X=1, -1, or 5.

intraSlotFrequencyHopping Enabling intra-slot frequency hopping, applicable for all types of PUCCH formats. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. See TS 38.213 [13], clause 9.2.1.

nrofPRBs Indicates the number of PRBs used per PUCCH resource for the PUCCH format, see TS 38.213 [13], clause 9.2.1. This field is applicable for PUCCH format0, format1, and format4 in FR2-2. The supported values for format4 are 1,2,3,4,5,6,8,9,10,12,15 and 16.

multipanelSFN-Scheme Parameter to configure multiple panel simultaneous uplink transmission SFN scheme for PUCCH resources, see TS 38.214 [19] clause 6.1.1.

occ-Index Indicates the orthogonal cover code index (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

occ-Length Indicates the orthogonal cover code length (see TS 38.213 [13], clause 9.2.1). This field is applicable when useInterlacePUCCH-PUSCH-16 is configured.

pucch-RepetitionNrofSlots Configuration of PUCCH repetition factor per PUCCH resource with associated scheduling DCI corresponding to Rel-17 dynamic PUCCH repetition. For a PUCCH resource, if both the field pucch-RepetitionNrofSlots and the field nrofSlots are present, the field nrofSlots is ignored and apply the value of pucch-RepetitionNrofSlots corresponding to Rel-17 dynamic PUCCH repetition. If this field is absent in a PUCCH resource with associated scheduling DCI, the UE applies the value of field nrofSlots.

pucch-ResourceId Identifier of the PUCCH resource.

secondHopPRB Index of first PRB after frequency hopping of PUCCH. This value is applicable for intra-slot frequency hopping (see TS 38.213 [13], clause 9.2.1) or inter-slot frequency hopping (see TS 38.213 [13], clause 9.2.6).

secondHopPRB-SBFD Indicates the second hop PRB of the PUCCH resource in SBFD symbols. If not configured, the second hop PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId.

startingPRB-SBFD Indicates the starting PRB of the PUCCH resource in SBFD symbols. If not configured, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId.

PUCCH-ResourceSet field descriptions

maxPayloadSize Maximum number of UCI information bits that the UE may transmit using this PUCCH resource set (see TS 38.213 [13], clause 9.2.1). In a PUCCH occurrence, the UE chooses the first of its PUCCH-ResourceSet which supports the number of bits that the UE wants to transmit. The field is absent in the first set (Set0) and in the last configured set since the UE derives the maximum number of UCI information bits as specified in TS 38.213 [13], clause 9.2.1. This field can take integer values that are multiples of 4.

resourceList PUCCH resources of format0 and format1 are only allowed in the first PUCCH resource set, i.e., in a PUCCH-ResourceSet with pucch-ResourceSetId = 0. This set may contain between 1 and 32 resources. PUCCH resources of format2, format3 and format4 are only allowed in a PUCCH-ResourceSet with pucch-ResourceSetId > 0. If present, these sets contain between 1 and 8 resources each. The UE chooses a PUCCH-Resource from this list as specified in TS 38.213 [13], clause 9.2.3. Note that this list contains only a list of resource IDs. The actual resources are configured in PUCCH-Config.

| Conditional Presence | Explanation |
| --- | --- |
| PI2-BPSK | The field is optionally present, Need R, if format3 and/or format4 are configured and pi2BPSK is configured in each of them. It is absent, Need R otherwise. |

#### –	PUCCH-ConfigCommon

The IE PUCCH-ConfigCommon is used to configure the cell specific PUCCH parameters.

PUCCH-ConfigCommon information element

-- ASN1START

-- TAG-PUCCH-CONFIGCOMMON-START

PUCCH-ConfigCommon ::=              SEQUENCE {

pucch-ResourceCommon                INTEGER (0..15)                                      OPTIONAL,   -- Cond InitialBWP-Only

pucch-GroupHopping                  ENUMERATED { neither, enable, disable },

hoppingId                           INTEGER (0..1023)                                    OPTIONAL,   -- Need R

p0-nominal                          INTEGER (-202..24)                                   OPTIONAL,   -- Need R

...,

[[

nrofPRBs                            INTEGER (1..16)                                      OPTIONAL,   -- Need R

intra-SlotFH-r17                    ENUMERATED {fromLowerEdge, fromUpperEdge}            OPTIONAL,   -- Cond InitialBWP-RedCapOnly

pucch-ResourceCommonRedCap-r17      INTEGER (0..15)                                      OPTIONAL,   -- Cond InitialBWP-RedCap

additionalPRBOffset-r17             ENUMERATED {n2, n3, n4, n6, n8, n9, n10, n12}        OPTIONAL    -- Cond InitialBWP-RedCapOnly

]],

[[

p0-nominal-SBFD-r19                 INTEGER (-202..24)                                   OPTIONAL    -- Need R

]]

}

-- TAG-PUCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUCCH-ConfigCommon field descriptions

additionalPRBOffset When intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is disabled, each common PUCCH resource is mapped to a single PRB on one side of the UL BWP. This parameter determines an additional PRB offset in the PRB mapping for the PUCCH resource. If the field is not configured, the UE shall assume an additional PRB offset of zero.

hoppingId Cell-specific scrambling ID for group hopping and sequence hopping if enabled, see TS 38.211 [16], clause 6.3.2.2.

intra-SlotFH-r17 In case a separate initial UL BWP is configured for (e)RedCap UEs, the presence of this parameter indicates whether intra-slot PUCCH frequency hopping within the separate initial UL BWP in the common PUCCH resource is enabled for (e)RedCap UEs. If this field is absent, intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is enabled. If this field is present, intra-slot PUCCH frequency hopping within RedCap-specific initial UL BWP is disabled and each PUCCH resource is mapped to a single PRB on one side of the UL BWP and this parameter determines whether the PRB index in the PRB mapping is counted in increasing order from the lower edge or in decreasing order from the upper edge of the UL BWP.

nrofPRBs Indicates the number of PRBs used per PUCCH resource for PUCCH format 0 and format 1 in FR2-2, see TS 38.213 [13], clause 9.2.1.

p0-nominal Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.2).

p0-nominal-SBFD Configures separate power control parameter P0 for PUCCH transmissions in SBFD symbols.

pucch-GroupHopping Configuration of group- and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. Value neither implies neither group or sequence hopping is enabled. Value enable enables group hopping and disables sequence hopping. Value disable disables group hopping and enables sequence hopping (see TS 38.211 [16], clause 6.3.2.2).

pucch-ResourceCommon An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

pucch-ResourceCommonRedCap An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters for (e)RedCap UEs. The UE uses those PUCCH resources until it is provided with a dedicated PUCCH-Config (e.g. during initial access) on the initial uplink BWP. Once the network provides a dedicated PUCCH-Config for that bandwidth part the UE applies that one instead of the one provided in this field (see TS 38.213 [13], clause 9.2).

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | The field is mandatory present in the PUCCH-ConfigCommon of the initial BWP (BWP#0) in SIB1. It is absent in other BWPs including the RedCap-specific initial uplink BWP, if configured. |
| InitialBWP-RedCap | The field is mandatory present in the PUCCH-ConfigCommon of the RedCap-specific initial BWP. It is optional present, Need R, in the PUCCH-ConfigCommon of the initial BWP configured by initialUplinkBWP. It is absent in other BWPs. |
| InitialBWP-RedCapOnly | The field is optional present, Need S, in the PUCCH-ConfigCommon of the RedCap-specific initial BWP. It is absent in other BWPs. |

#### –	PUCCH-ConfigurationList

The IE PUCCH-ConfigurationList is used to configure UE specific PUCCH parameters (per BWP) for two simultaneously constructed HARQ-ACK codebooks. See TS 38.213 [13], clause 9.1.

PUCCH-ConfigurationList information element

-- ASN1START

-- TAG-PUCCH-CONFIGURATIONLIST-START

PUCCH-ConfigurationList-r16  ::=     SEQUENCE (SIZE (1..2)) OF PUCCH-Config

-- TAG-PUCCH-CONFIGURATIONLIST-STOP

-- ASN1STOP

#### –	PUCCH-CSI-Resource

The IE PUCCH-CSI-Resource is used to indicate a PUCCH resource to use for reporting on PUCCH.

PUCCH-CSI-Resource information element

-- ASN1START

-- TAG-PUCCH-CSI-RESOURCE-START

PUCCH-CSI-Resource ::=              SEQUENCE {

uplinkBandwidthPartId               BWP-Id,

pucch-Resource                      PUCCH-ResourceId

}

PUCCH-CSI-ResourceExt-v1900 ::=     SEQUENCE {

symbolType                          ENUMERATED {sbfd, non-sbfd}                             OPTIONAL,   -- Need R

...

}

-- TAG-PUCCH-CSI-RESOURCE-STOP

-- ASN1STOP

PUCCH-CSI-Resource field descriptions

pucch-Resource PUCCH resource for the associated uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. When two PUCCH-Config are configured within PUCCH-ConfigurationList, PUCCH-ResourceId in a PUCCH-CSI-Resource refers to a PUCCH-Resource in the PUCCH-Config used for HARQ-ACK with low priority.

symbolType Configures the valid symbol type for PUCCH carrying P-CSI or SP-CSI when the transmissions are restricted to SBFD symbols only or non-SBFD symbols only for the UL BWP. The network does not configure this field if the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 7.2).

#### –	PUCCH-PathlossReferenceRS-Id

The IE PUCCH-PathlossReferenceRS-Id is an ID for a reference signal (RS) configured as PUCCH pathloss reference (see TS 38.213 [13], clause 7.2).

PUCCH-PathlossReferenceRS-Id information element

-- ASN1START

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-START

PUCCH-PathlossReferenceRS-Id ::=            INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1)

PUCCH-PathlossReferenceRS-Id-v1610 ::=      INTEGER (maxNrofPUCCH-PathlossReferenceRSs..maxNrofPUCCH-PathlossReferenceRSs-1-r16)

PUCCH-PathlossReferenceRS-Id-r17 ::=        INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1-r17)

-- TAG-PUCCH-PATHLOSSREFERENCERS-ID-STOP

-- ASN1STOP

#### –	PUCCH-PowerControl

The IE PUCCH-PowerControl is used to configure UE-specific parameters for the power control of PUCCH.

PUCCH-PowerControl information element

-- ASN1START

-- TAG-PUCCH-POWERCONTROL-START

PUCCH-PowerControl ::=              SEQUENCE {

deltaF-PUCCH-f0                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f1                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f2                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f3                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

deltaF-PUCCH-f4                     INTEGER (-16..15)                                                       OPTIONAL, -- Need R

p0-Set                              SEQUENCE (SIZE (1..maxNrofPUCCH-P0-PerSet)) OF P0-PUCCH                 OPTIONAL, -- Need M

pathlossReferenceRSs                SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS

OPTIONAL, -- Need M

twoPUCCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSs-v1610          SetupRelease { PathlossReferenceRSs-v1610 }                             OPTIONAL -- Need M

]]

}

P0-PUCCH ::=                            SEQUENCE {

p0-PUCCH-Id                             P0-PUCCH-Id,

p0-PUCCH-Value                          INTEGER (-16..15)

}

P0-PUCCH-Id ::=                         INTEGER (1..8)

PathlossReferenceRSs-v1610 ::=          SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSsDiff-r16)) OF PUCCH-PathlossReferenceRS-r16

PUCCH-PathlossReferenceRS ::=                   SEQUENCE {

pucch-PathlossReferenceRS-Id                PUCCH-PathlossReferenceRS-Id,

referenceSignal                             CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

}

PUCCH-PathlossReferenceRS-r16 ::=                   SEQUENCE {

pucch-PathlossReferenceRS-Id-r16                    PUCCH-PathlossReferenceRS-Id-v1610,

referenceSignal-r16                                 CHOICE {

ssb-Index-r16                                       SSB-Index,

csi-RS-Index-r16                                    NZP-CSI-RS-ResourceId

}

}

PUCCH-PowerControlSetInfo-r17 ::=       SEQUENCE {

pucch-PowerControlSetInfoId-r17         PUCCH-PowerControlSetInfoId-r17,

p0-PUCCH-Id-r17                         P0-PUCCH-Id,

pucch-ClosedLoopIndex-r17               ENUMERATED { i0, i1 },

pucch-PathlossReferenceRS-Id-r17        PUCCH-PathlossReferenceRS-Id-r17

}

PUCCH-PowerControlSetInfoId-r17 ::=     INTEGER (1.. maxNrofPowerControlSetInfos-r17)

-- TAG-PUCCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUCCH field descriptions

p0-PUCCH-Value P0 value for PUCCH with 1dB step size.

PUCCH-PowerControl field descriptions

deltaF-PUCCH-f0 deltaF for PUCCH format 0 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f1 deltaF for PUCCH format 1 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f2 deltaF for PUCCH format 2 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f3 deltaF for PUCCH format 3 with 1dB step size (see TS 38.213 [13], clause 7.2).

deltaF-PUCCH-f4 deltaF for PUCCH format 4 with 1dB step size (see TS 38.213 [13], clause 7.2).

p0-Set A set with dedicated P0 values for PUCCH, i.e., {P01, P02,... } (see TS 38.213 [13], clause 7.2). This field is not configured if unifiedTCI-StateType is configured for the serving cell.

pathlossReferenceRSs, pathlossReferenceRSs-v1610 A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured. If the field is not configured, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2). The set includes Reference Signals indicated in pathlossReferenceRSs (without suffix) and in pathlossReferenceRSs-v1610. The UE maintains pathlossReferenceRSs and pathlossReferenceRSs-v1610 separately: Receiving pathlossReferenceRSs-v1610 set to release releases only the entries that were configured by pathlossReferenceRSs-v1610, and receiving pathlossReferenceRSs-v1610 set to setup replaces only the entries that were configured by pathlossReferenceRSs-v1610 with the newly signalled entries. Neither the field pathlossReferenceRSs (without suffix) nor pathlossReferenceRSs-v1610 are configured if unifiedTCI-StateType is configured for the serving cell.

twoPUCCH-PC-AdjustmentStates Number of PUCCH power control adjustment states maintained by the UE (i.e., g(i)). If the field is present (n2) the UE maintains two power control states (i.e., g(i,0) and g(i,1)). If the field is absent, it maintains one power control state (i.e., g(i,0)) (see TS 38.213 [13], clause 7.2).

#### –	PUCCH-SpatialRelationInfo

The IE PUCCH-SpatialRelationInfo is used to configure the spatial setting for PUCCH transmission and the parameters for PUCCH power control, see TS 38.213, [13], clause 9.2.2.

PUCCH-SpatialRelationInfo information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfo ::=           SEQUENCE {

pucch-SpatialRelationInfoId         PUCCH-SpatialRelationInfoId,

servingCellId                           ServCellIndex                                                    OPTIONAL,   -- Need S

referenceSignal                         CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId,

srs                                     PUCCH-SRS

},

pucch-PathlossReferenceRS-Id            PUCCH-PathlossReferenceRS-Id,

p0-PUCCH-Id                             P0-PUCCH-Id,

closedLoopIndex                         ENUMERATED { i0, i1 }

}

PUCCH-SpatialRelationInfoExt-r16 ::=       SEQUENCE {

pucch-SpatialRelationInfoId-v1610         PUCCH-SpatialRelationInfoId-v1610                              OPTIONAL,   -- Need S

pucch-PathlossReferenceRS-Id-v1610        PUCCH-PathlossReferenceRS-Id-v1610                             OPTIONAL,    --Need R

...

}

PUCCH-SRS ::=                       SEQUENCE {

resource                            SRS-ResourceId,

uplinkBWP                           BWP-Id

}

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

PUCCH-SpatialRelationInfo field descriptions

pucch-PathLossReferenceRS-Id When pucch-PathLossReferenceRS-Id-v1610 is configured, the UE shall ignore pucch-PathLossReferenceRS-Id (without suffix).

pucch-SpatialRelationInfoId When pucch-SpatialRelationInfoId-v1610 is configured, the UE shall ignore pucch-SpatialRelationInfoId (without suffix). If pucch-SpatialRelationInfoId-v1610 is absent, the UE shall use the pucch-SpatialRelationInfoId (without suffix).

servingCellId If the field is absent, the UE applies the ServCellId of the serving cell in which this PUCCH-SpatialRelationInfo is configured

#### –	PUCCH-SpatialRelationInfo-Id

The IE PUCCH-SpatialRelationInfo-Id is used to identify a PUCCH-SpatialRelationInfo

PUCCH-SpatialRelationInfo-Id information element

-- ASN1START

-- TAG-PUCCH-SPATIALRELATIONINFO-START

PUCCH-SpatialRelationInfoId ::=         INTEGER (1..maxNrofSpatialRelationInfos)

PUCCH-SpatialRelationInfoId-r16 ::=     INTEGER (1..maxNrofSpatialRelationInfos-r16)

PUCCH-SpatialRelationInfoId-v1610::=    INTEGER (maxNrofSpatialRelationInfos-plus-1..maxNrofSpatialRelationInfos-r16)

-- TAG-PUCCH-SPATIALRELATIONINFO-STOP

-- ASN1STOP

#### –	PUCCH-TPC-CommandConfig

The IE PUCCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUCCH from a group-TPC messages on DCI.

PUCCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUCCH-TPC-COMMANDCONFIG-START

PUCCH-TPC-CommandConfig ::=             SEQUENCE {

tpc-IndexPCell                          INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-OfSpcell

tpc-IndexPUCCH-SCell                    INTEGER (1..15)                         OPTIONAL,   -- Cond PDCCH-ofSpCellOrPUCCH-SCell

...,

[[

tpc-IndexPUCCH-sSCell-r17                             INTEGER (1..15)                         OPTIONAL,  -- Need R

tpc-IndexPUCCH-sScellSecondaryPUCCHgroup-r17          INTEGER (1..15)                         OPTIONAL   -- Cond twoPUCCHgroup

]]

}

-- TAG-PUCCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUCCH-TPC-CommandConfig field descriptions

tpc-IndexPCell An index determining the position of the first bit of TPC command (applicable to the SpCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-SCell An index determining the position of the first bit of TPC command (applicable to the PUCCH SCell) inside the DCI format 2-2 payload.

tpc-IndexPUCCH-sSCell, tpc-IndexPUCCH-sSCellSecondaryPUCCHgroup An index determining the position of the first bit of TPC command (applicable to the alternative PUCCH cell for PUCCH cell switching) inside the DCI format 2-2 payload, for the primary PUCCH group and the secondary PUCCH group respectively.

| Conditional Presence | Explanation |
| --- | --- |
| PDCCH-OfSpcell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| PDCCH-ofSpCellOrPUCCH-SCell | The field is mandatory present if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the PUCCH-SCell. The field is optionally present, need R, if the UE is configured with a PUCCH SCell in this cell group and if the PUCCH-TPC-CommandConfig is provided in the PDCCH-Config for the SpCell. Otherwise, the field is absent, Need R. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise. |

#### –	PUSCH-Config

The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

pusch-PowerControl                      PUSCH-PowerControl                                                  OPTIONAL,   -- Need M

frequencyHopping                        ENUMERATED {intraSlot, interSlot}                                   OPTIONAL,   -- Need S

frequencyHoppingOffsetLists             SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList          SetupRelease { PUSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

transformPrecoder                       ENUMERATED {enabled, disabled}                                      OPTIONAL,   -- Need S

codebookSubset                          ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL, -- Cond codebookBased

maxRank                                 INTEGER (1..4)                                                OPTIONAL, -- Cond codebookBased

rbg-Size                                ENUMERATED { config2}                                         OPTIONAL, -- Need S

uci-OnPUSCH                             SetupRelease { UCI-OnPUSCH}                                   OPTIONAL, -- Need M

tp-pi2BPSK                              ENUMERATED {enabled}                                          OPTIONAL, -- Need S

...,

[[

minimumSchedulingOffsetK2-r16           SetupRelease { MinSchedulingOffsetK2-Values-r16 }             OPTIONAL,  -- Need M

ul-AccessConfigListDCI-0-1-r16          SetupRelease { UL-AccessConfigListDCI-0-1-r16 }               OPTIONAL,  -- Need M

-- Start of the parameters for DCI format 0_2 introduced in V16.1.0

harq-ProcessNumberSizeDCI-0-2-r16                       INTEGER (0..4)                                OPTIONAL,   -- Need R

dmrs-SequenceInitializationDCI-0-2-r16                  ENUMERATED {enabled}                          OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-2-r16                           INTEGER (0..2)                                OPTIONAL,   -- Need R

antennaPortsFieldPresenceDCI-0-2-r16                    ENUMERATED {enabled}                          OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

frequencyHoppingDCI-0-2-r16                             CHOICE {

pusch-RepTypeA                                          ENUMERATED {intraSlot, interSlot},

pusch-RepTypeB                                          ENUMERATED {interRepetition, interSlot}

}                                                                                                     OPTIONAL,   -- Need S

frequencyHoppingOffsetListsDCI-0-2-r16  SetupRelease { FrequencyHoppingOffsetListsDCI-0-2-r16}        OPTIONAL,  -- Need M

codebookSubsetDCI-0-2-r16               ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL,   -- Cond codebookBased

invalidSymbolPatternIndicatorDCI-0-2-r16                ENUMERATED {enabled}                          OPTIONAL,   -- Need S

maxRankDCI-0-2-r16                                      INTEGER (1..4)                                OPTIONAL,   -- Cond codebookBased

mcs-TableDCI-0-2-r16                                    ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

mcs-TableTransformPrecoderDCI-0-2-r16                   ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

priorityIndicatorDCI-0-2-r16                            ENUMERATED {enabled}                          OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-2-r16                       ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}  OPTIONAL,  -- Need R

resourceAllocationDCI-0-2-r16                           ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

resourceAllocationType1GranularityDCI-0-2-r16           ENUMERATED { n2,n4,n8,n16 }                   OPTIONAL,   -- Need S

uci-OnPUSCH-ListDCI-0-2-r16                             SetupRelease { UCI-OnPUSCH-ListDCI-0-2-r16}   OPTIONAL,   -- Need M

pusch-TimeDomainAllocationListDCI-0-2-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 0_2 introduced in V16.1.0

-- Start of the parameters for DCI format 0_1 introduced in V16.1.0

pusch-TimeDomainAllocationListDCI-0-1-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

invalidSymbolPatternIndicatorDCI-0-1-r16          ENUMERATED {enabled}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-0-1-r16                      ENUMERATED {enabled}                                OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-1-r16                 ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}        OPTIONAL,   -- Need R

frequencyHoppingDCI-0-1-r16                 ENUMERATED {interRepetition, interSlot}                   OPTIONAL,   -- Cond RepTypeB

uci-OnPUSCH-ListDCI-0-1-r16                 SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }             OPTIONAL,  -- Need M

-- End of the parameters for DCI format 0_1 introduced in V16.1.0

invalidSymbolPattern-r16                    InvalidSymbolPattern-r16                                  OPTIONAL,   -- Need S

pusch-PowerControl-v1610                SetupRelease {PUSCH-PowerControl-v1610}                       OPTIONAL,   -- Need M

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpowerMode2}         OPTIONAL,   -- Need R

pusch-TimeDomainAllocationListForMultiPUSCH-r16  SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,  --  Need M

numberOfInvalidSymbolsForDL-UL-Switching-r16        INTEGER (1..4)                                    OPTIONAL    -- Cond RepTypeB2

]],

[[

ul-AccessConfigListDCI-0-2-r17          SetupRelease { UL-AccessConfigListDCI-0-2-r17 }               OPTIONAL,  -- Need M

betaOffsetsCrossPri0-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                   OPTIONAL,  -- Need M

betaOffsetsCrossPri1-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                   OPTIONAL,  -- Need M

betaOffsetsCrossPri0DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }            OPTIONAL,  -- Need M

betaOffsetsCrossPri1DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }            OPTIONAL,  -- Need M

mappingPattern-r17                      ENUMERATED {cyclicMapping, sequentialMapping}                 OPTIONAL,  -- Cond SRSsets

secondTPCFieldDCI-0-1-r17               ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

secondTPCFieldDCI-0-2-r17               ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

sequenceOffsetForRV-r17                 INTEGER (0..3)                                                OPTIONAL,  -- Need R

ul-AccessConfigListDCI-0-1-r17          SetupRelease { UL-AccessConfigListDCI-0-1-r17 }                OPTIONAL,  -- Need M

minimumSchedulingOffsetK2-r17           SetupRelease { MinSchedulingOffsetK2-Values-r17 }              OPTIONAL,  -- Need M

availableSlotCounting-r17               ENUMERATED { enabled }                                         OPTIONAL,  -- Need S

dmrs-BundlingPUSCH-Config-r17           SetupRelease { DMRS-BundlingPUSCH-Config-r17 }                 OPTIONAL,  -- Need M

harq-ProcessNumberSizeDCI-0-2-v1700     INTEGER (5)                                                    OPTIONAL,  -- Need R

harq-ProcessNumberSizeDCI-0-1-r17       INTEGER (5)                                                    OPTIONAL,  -- Need R

mpe-ResourcePoolToAddModList-r17       SEQUENCE (SIZE(1..maxMPE-Resources-r17)) OF MPE-Resource-r17    OPTIONAL,  -- Need N

mpe-ResourcePoolToReleaseList-r17      SEQUENCE (SIZE(1..maxMPE-Resources-r17)) OF MPE-ResourceId-r17  OPTIONAL   -- Need N

]],

[[

maxRank-v1810                           INTEGER (5..8)                                                 OPTIONAL,  -- Need R

sTx-2Panel-r18                          ENUMERATED {enabled}                                           OPTIONAL,  -- Need R

multipanelSchemeSDM-r18                 SDM-Scheme-r18                                                 OPTIONAL,  -- Need R

multipanelSchemeSFN-r18                 SFN-Scheme-r18                                                 OPTIONAL,  -- Need R

codebookTypeUL-r18                      SetupRelease { CodebookTypeUL-r18 }                            OPTIONAL,  -- Need M

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL,  -- Need R

dynamicTransformPrecoderFieldPresenceDCI-0-1-r18    ENUMERATED {enabled}                               OPTIONAL,  -- Need R

dynamicTransformPrecoderFieldPresenceDCI-0-2-r18    ENUMERATED {enabled}                               OPTIONAL,  -- Need R

pusch-ConfigDCI-0-3-r18                 SetupRelease { PUSCH-ConfigDCI-0-3-r18 }                       OPTIONAL   -- Need M

]],

[[

mg-CancellationDCI-0-1-r19              ENUMERATED {enabled}                                           OPTIONAL,  -- Need R

mg-CancellationDCI-0-2-r19              ENUMERATED {enabled}                                           OPTIONAL,  -- Need R

mg-CancellationDCI-0-3-r19              ENUMERATED {enabled}                                           OPTIONAL,  -- Need R

pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3-r19  SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL, --  Need M

harq-ProcessNumberSizeDCI-0-1-Ext-r19   INTEGER (5)                                                    OPTIONAL,  -- Need R

harq-ProcessNumberSizeDCI-0-2-Ext-r19   INTEGER (0..5)                                                 OPTIONAL,  -- Need R

pusch-ConfigDCI-0-3-v1900               SetupRelease { PUSCH-ConfigDCI-0-3-v1900 }                     OPTIONAL,  -- Need M

frequencyHoppingOffsetLists-SBFD-r19        SEQUENCE (SIZE (1..4)) OF INTEGER (1..maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

frequencyHoppingOffsetListsDCI-0-2-SBFD-r19 SEQUENCE (SIZE (1..4)) OF INTEGER (1..maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

pusch-MutingResources-r19                   PUSCH-MutingResources-r19                                  OPTIONAL    -- Need R

]]

}

UCI-OnPUSCH ::=                         SEQUENCE {

betaOffsets                             CHOICE {

dynamic                             SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic                          BetaOffsets

}                                                                                                 OPTIONAL, -- Need M

scaling                                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

MinSchedulingOffsetK2-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r16)

MinSchedulingOffsetK2-Values-r17 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r17)

UCI-OnPUSCH-DCI-0-2-r16 ::=             SEQUENCE {

betaOffsetsDCI-0-2-r16                  CHOICE {

dynamicDCI-0-2-r16                      CHOICE {

oneBit-r16                              SEQUENCE (SIZE (2)) OF BetaOffsets,

twoBits-r16                             SEQUENCE (SIZE (4)) OF BetaOffsets

},

semiStaticDCI-0-2-r16          BetaOffsets

}                                                                                                 OPTIONAL,   -- Need M

scalingDCI-0-2-r16                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

FrequencyHoppingOffsetListsDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

UCI-OnPUSCH-ListDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH-DCI-0-2-r16

UCI-OnPUSCH-ListDCI-0-1-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH

UL-AccessConfigListDCI-0-1-r16 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

UL-AccessConfigListDCI-0-1-r17 ::= SEQUENCE (SIZE (1..3)) OF INTEGER (0..2)

UL-AccessConfigListDCI-0-2-r17 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

BetaOffsetsCrossPriSel-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17          BetaOffsetsCrossPri-r17

}

BetaOffsetsCrossPriSelDCI-0-2-r17 ::= CHOICE {

dynamicDCI-0-2-r17      CHOICE {

oneBit-r17              SEQUENCE (SIZE (2)) OF BetaOffsetsCrossPri-r17,

twoBits-r17             SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17

},

semiStaticDCI-0-2-r17   BetaOffsetsCrossPri-r17

}

MPE-Resource-r17 ::=        SEQUENCE {

mpe-ResourceId-r17          MPE-ResourceId-r17,

cell-r17                    ServCellIndex                                                         OPTIONAL,    -- Need R

additionalPCI-r17           AdditionalPCIIndex-r17                                                OPTIONAL,    -- Need R

mpe-ReferenceSignal-r17     CHOICE {

csi-RS-Resource-r17         NZP-CSI-RS-ResourceId,

ssb-Resource-r17            SSB-Index

}

}

MPE-ResourceId-r17 ::=      INTEGER (1..maxMPE-Resources-r17)

SDM-Scheme-r18   ::=        SEQUENCE {

maxRankSDM-r18              INTEGER (1..2)                                                        OPTIONAL,    -- Need R

maxRankSDM-DCI-0-2-r18      INTEGER (1..2)                                                        OPTIONAL     -- Need R

}

SFN-Scheme-r18   ::=        SEQUENCE {

maxRankSFN-r18              INTEGER (1..2)                                                        OPTIONAL,    -- Need R

maxRankSFN-DCI-0-2-r18      INTEGER (1..2)                                                        OPTIONAL     -- Need R

}

CodebookTypeUL-r18 ::=      CHOICE {

codebook1-r18               ENUMERATED {ng1n4n1, ng1n2n2},

codebook2-r18               ENUMERATED {ng2},

codebook3-r18               ENUMERATED {ng4},

codebook4-r18               ENUMERATED {ng8}

}

PUSCH-ConfigDCI-0-3-r18 ::=                   SEQUENCE {

resourceAllocationDCI-0-3-r18                 ENUMERATED {resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

rbg-SizeDCI-0-3-r18                           ENUMERATED {config2, config3}                                  OPTIONAL,   -- Need S

resourceAllocationType1GranularityDCI-0-3-r18 ENUMERATED {n2,n4,n8,n16}                                      OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-3-r18                 INTEGER (0..2)                                                 OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-0-3-r18               INTEGER (0..5)                                                 OPTIONAL,   -- Need R

uci-OnPUSCH-ListDCI-0-3-r18                   SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }                  OPTIONAL    -- Need M

}

PUSCH-ConfigDCI-0-3-v1900 ::=                 SEQUENCE {

harq-ProcessNumberSizeDCI-0-3-Ext-r19         INTEGER (0..5)                                                 OPTIONAL    -- Need R

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

PUSCH-Config field descriptions

antennaPortsFieldPresenceDCI-0-2 Configure the presence of "Antenna ports" field in DCI format 0_2. When the field is configured, then the "Antenna ports" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 nor dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 is configured, this field is absent.

applyIndicatedTCI-State This field indicates, for a PUSCH transmission, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.1.

availableSlotCounting Indicate whether PUSCH repetitions counted on the basis of available slots is enabled. If the field is absent, PUSCH repetitions counted on the basis of available slots is disabled.

betaOffsetsCrossPri0, betaOffsetsCrossPri1, betaOffsetsCrossPri0DCI-0-2, betaOffsetsCrossPri1DCI-0-2 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK on dynamically scheduled PUSCH with different priorities, see TS 38.213 [13], clause 9.3. The field betaOffsetsCrossPrio0 indicates multiplexing low priority (LP) HARQ-ACK on dynamically scheduled high priority (HP) PUSCH. The field betaOffsetsCrossPrio1 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH. The field betaOffsetsCrossPrio0DCI-0-2 indicates multiplexing LP HARQ-ACK on dynamically scheduled HP PUSCH by DCI format 0_2. The field betaOffsetsCrossPrio1DCI-0-2 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH by DCI format 0_2.

codebookSubset, codebookSubsetDCI-0-2 Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities (see TS 38.214 [19], clause 6.1.1.1). The field codebookSubset applies to DCI formats 0_1 and 0_3, and the field codebookSubsetDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1).

codebookTypeUL Configures a codebook and the corresponding number of antenna port groups for codebook-based transmission of PUSCH with 8 antenna ports, see TS 38.211 [16], tables 6.3.1.5-9 to 6.3.1.5-47, and table 6.3.1.5-8 respectively). The values ng1n4n1 and ng1n2n2 correspond to codebooks with one antenna port group (Ng=1), while ng2, ng4, and ng8 correspond to codebooks with Ng=2, 4, and 8 antenna port groups, respectively.

dataScramblingIdentityPUSCH Identifier used to initialise data scrambling (c_init) for PUSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 6.3.1.1).

dmrs-BundlingPUSCH-Config Configure the parameters for DMRS bundling for PUSCH (see TS 38.214 [19], clause 6.1.7). In this release, this is not applicable to FR2-2.

dmrs-SequenceInitializationDCI-0-2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 0_2. If the field is absent, then 0 bit for the field "DMRS Sequence Initialization" in DCI format 0_2. If the field is present, then the number of bits is determined in the same way as DCI format 0_1 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeA, dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type A (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeA applies to DCI formats 0_1 and 0_3, and the field dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeB, dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type B (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeB applies to DCI formats 0_1 and 0_3, and the field dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dynamicTransformPrecoderFieldPresenceDCI-0-1 Configure the presence of "Dynamic Transform Precoder" field in DCI format 0_1. When the field is configured, then the "Dynamic Transform Precoder" field is present in DCI format 0_1. Otherwise, the field size is set to 0 for DCI format 0_1 (See TS 38.212 [17]). The network ensures dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 and twoPHRMode-r17 cannot be configured at the same time for a UE.

dynamicTransformPrecoderFieldPresenceDCI-0-2 Configure the presence of "Dynamic Transform Precoder" field in DCI format 0_2. When the field is configured, then the "Dynamic Transform Precoder" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17]). The network ensures dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 and twoPHRMode-r17 cannot be configured at the same time for a UE.

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3). The field frequencyHopping applies to DCI formats 0_0, 0_1 and 0_3 for 'pusch-RepTypeA'.

frequencyHoppingDCI-0-1 Indicates the frequency hopping scheme for DCI format 0_1 when pusch-RepTypeIndicatorDCI-0-1 is set to 'pusch-RepTypeB', The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for DCI format 0_1 for 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1).

frequencyHoppingDCI-0-2 Indicate the frequency hopping scheme for DCI format 0_2. The value intraSlot enables 'intra-slot frequency hopping', and the value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. When pusch-RepTypeIndicatorDCI-0-2 is not set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'intra-slot frequency hopping and 'inter-slot frequency hopping' if enabled. When pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'inter-repetition frequency hopping' and 'inter-slot frequency hopping' if enabled. If the field is absent, frequency hopping is not configured for DCI format 0_2 (see TS 38.214 [19], clause 6.3).

frequencyHoppingOffsetLists, frequencyHoppingOffsetListsDCI-0-2 Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 configured grant activation (see TS 38.214 [19], clause 6.3). The field frequencyHoppingOffsetLists applies to DCI formats 0_0, 0_1 and 0_3, and the field frequencyHoppingOffsetListsDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.3).

frequencyHoppingOffsetLists-SBFD, frequencyHoppingOffsetListsDCI-0-2-SBFD Configure the frequency hopping offset lists for PUSCH scheduled by DCI and Type 2 configured grant PUSCH in SBFD symbols, and the field frequencyHoppingOffsetListsDCI-0-2-SBFD applies to DCI format 0_2 (see TS 38.214 [19], clause 6.3.3).

harq-ProcessNumberSizeDCI-0-1-Ext Configure the number of bits for the field "HARQ process number" in DCI format 0_1 in TN (see TS 38.212 [17], clause 7.3.1). This field can only be configured when the maximum number of layers configured for PUSCH is no more than 4.

harq-ProcessNumberSizeDCI-0-2 Configure the number of bits for the field "HARQ process number" in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1). Only one of this field and harq-ProcessNumberSizeDCI-0-2-Ext is configured by network.

harq-ProcessNumberSizeDCI-0-2-Ext Configure the number of bits for the field "HARQ process number" in DCI format 0_2 for TN (see TS 38.212 [17], clause 7.3.1). This field can only be configured when the maximum number of layers configured for PUSCH is no more than 4. Only one of this field and harq-ProcessNumberSizeDCI-0-2 is configured by network.

invalidSymbolPattern Indicates one pattern for invalid symbols for PUSCH transmission repetition type B applicable to both DCI format 0_1 and 0_2. If InvalidSymbolPattern is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols (see TS 38.214 [19] clause 6.1).

invalidSymbolPatternIndicatorDCI-0-1, invalidSymbolPatternIndicatorDCI-0-2 Indicates the presence of an additional bit in the DCI format 0_1/0_2. If invalidSymbolPattern is absent, then both invalidSymbolPatternIndicatorDCI-0-1 and invalidSymbolPatternIndicatorDCI-0-2 are absent. The field invalidSymbolPatternIndicatorDCI-0-1 applies to the DCI format 0_1 and the field invalidSymbolPatternIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19] clause 6.1). If the field is absent, the UE behaviour is specified in TS 38.214 [19], clause 6.1.2.1.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern for when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' for PUSCH transmission and the PUSCH transmission occasions are associated with both SRS resource sets.

maxRank, maxRankDCI-0-2 Subset of PMIs addressed by TRIs from 1 to ULmaxRank (see TS 38.214 [19], clause 6.1.1.1). The field maxRank applies to DCI formats 0_1 and 0_3, and the field maxRankDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1). If network configures maxRank-v1810 UE ignores maxRank (without suffix).

mcs-Table, mcs-TableFormat0-2 Indicates which MCS table the UE shall use for PUSCH without transform precoder (see TS 38.214 [19], clause 6.1.4.1). If the field is absent the UE applies the value 64QAM. The field mcs-Table applies to DCI formats 0_0, 0_1 and 0_3, and the field mcs-TableDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

mcs-TableTransformPrecoder, mcs-TableTransformPrecoderDCI-0-2 Indicates which MCS table the UE shall use for PUSCH with transform precoding (see TS 38.214 [19], clause 6.1.4.1) If the field is absent the UE applies the value 64QAM. The field mcs-TableTransformPrecoder applies to DCI formats 0_0, 0_1 and 0_3, and the field mcs-TableTransformPrecoderDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

mg-CancellationDCI-0-1 Indicates the presence of Measurement gap cancellation in DCI format 0_1 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-0-2 Indicates the presence of Measurement gap cancellation in DCI format 0_2 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-0-3 Indicates the presence of Measurement gap cancellation in DCI format 0_3 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17]. The field could only be configured if the co-scheduled cells are intra-band and have the same sub-carrier spacing.

minimumSchedulingOffsetK2 List of minimum K2 values. Minimum K2 parameter denotes minimum applicable value(s) for the Time domain resource assignment table for PUSCH (see TS 38.214 [19], clause 6.1.2.1).

mpe-ResourcePoolToAddModList List of SSB/CSI-RS resources for P-MPR reporting. Each resource is configured with serving cell index where the resource is configured for the UE. The additionalPCI is configured only if the resource is SSB. For each resource, if neither cell nor additionalPCI is present, the SSB/CSI-RS resource is from the serving cell where the PUSCH-Config is configured.

multipanelSchemeSDM Configures UE with a multiple panel simultaneous uplink transmission SDM scheme for PUSCH, as specified in TS 38.214 [19], clause 6.1. Network does not configure multipanelSchemeSDM with multipanelSchemeSFN. When this parameter is configured, two SRS resource sets with usage for codebook or noncodebook are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2.

multipanelSchemeSFN Configures UE with a multiple panel simultaneous uplink transmission SFN scheme for PUSCH, as specified in TS 38.214 [19], clause 6.1. Network does not configure multipanelSchemeSFN with multipanelSchemeSDM. When this parameter is configured, two SRS resource sets with usage for codebook or noncodebook are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2.

numberOfBitsForRV-DCI-0-2 Configures the number of bits for "Redundancy version" in the DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

numberOfInvalidSymbolsForDL-UL-Switching Indicates the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B. If it is absent, no symbol is explicitly defined for DL-to-UL switching (see TS 38.214 [19], clause 6.1).

priorityIndicatorDCI-0-1, priorityIndicatorDCI-0-2 Configures the presence of "priority indicator" in DCI format 0_1/0_2. When the field is absent in the IE, then the UE shall apply 0 bit for "Priority indicator" in DCI format 0_1/0_2. The field priorityIndicatorDCI-0-1 applies to DCI format 0_1 and the field priorityIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17] clause 7.3.1 and TS 38.213 [13] clause 9).

pusch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 6.1.2.1). If the field is absent the UE applies the value 1.

pusch-MutingResources Configures the time location and frequency location of UL muting resources for dynamic grant PUSCH transmission and Type 2 configured grant PUSCH transmission.

pusch-PowerControl Configures power control parameters PUSCH transmission.

pusch-RepTypeIndicatorDCI-0-1, pusch-RepTypeIndicatorDCI-0-2 Indicates whether UE follows the behavior for "PUSCH repetition type A" or the behavior for "PUSCH repetition type B" for the PUSCH scheduled by DCI format 0_1/0_2 and for Type 2 CG associated with the activating DCI format 0_1/0_2.The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B'. The field pusch-RepTypeIndicatorDCI-0-1 applies to DCI format 0_1 and the field pusch-RepTypeIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1).

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1). The field pusch-TimeDomainAllocationList applies to DCI format 0_0, or DCI formats 0_1 and 0_3 when the field pusch-TimeDomainAllocationListDCI-0-1 is not configured (see TS 38.214 [19], table 6.1.2.1.1-1 and tables 6.1.2.1.1-1A and 6.1.2.1.1-1C). The network does not configure the pusch-TimeDomainAllocationList (without suffix) simultaneously with the pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16 or pusch-TimeDomainAllocationListForMultiPUSCH-r16.

pusch-TimeDomainAllocationListDCI-0-1 Configuration of the time domain resource allocation (TDRA) table for DCI formats 0_1 and 0_3 (see TS 38.214 [19], clause 6.1, tables 6.1.2.1.1-1A and 6.1.2.1.1-1C).

pusch-TimeDomainAllocationListDCI-0-2 Configuration of the time domain resource allocation (TDRA) table for DCI format 0_2 (see TS 38.214 [19], clause 6.1.2, table 6.1.2.1.1-1B).

pusch-TimeDomainAllocationListForMultiPUSCH, pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor if extendedK2 is not configured. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17. For a cell within a cell set configured to be schedulable by a DCI format 0_3, the network does not configure pusch-TimeDomainAllocationListForMultiPUSCH-DCI-0-3 simultaneously with the numberOfSlotsTBoMS-r17 and/or pusch-AggregationFactor.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH except PUSCH scheduled by DCI format 0_3. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocation, resourceAllocationDCI-0-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 6.1.2). The field resourceAllocation applies to DCI format 0_1 and the field resourceAllocationDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-2 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

secondTPCFieldDCI-0-1, secondTPCFieldDCI-0-2 A second TPC field can be configured via RRC for DCI-0-1 and DCI-0-2. Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to "closedLoopIndex" value = 0 and 1,

sequenceOffsetForRV Configures the RV offset for the starting RV for the first repetition (first actual repetition in PUSCH repetition Type B) towards the second 'SRS resource set' for PUSCH configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

sTx-2Panel Parameter to enable PUSCH+PUSCH multiple panel simultaneous uplink transmission, as specified in TS 38.214 [19], clause 6.1.

tp-pi2BPSK Enables pi/2-BPSK modulation with transform precoding if the field is present and disables it otherwise.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList).

txConfig Whether UE uses codebook based or non-codebook based transmission (see TS 38.214 [19], clause 6.1.1). If the field is absent, the UE transmits PUSCH on one antenna port, see TS 38.214 [19], clause 6.1.1.

uci-OnPUSCH-ListDCI-0-1, uci-OnPUSCH-ListDCI-0-2 Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_1/0_2. The field uci-OnPUSCH-ListDCI-0-1 applies to DCI format 0_1 and the field uci-OnPUSCH-ListDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 List of the combinations of cyclic prefix extension, channel access priority class (CAPC), and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable for DCI format 0_1 and DCI format 0_2, respectively. The fields ul-AccessConfigListDCI-0-1-r16 and ul-AccessConfigListDCI-0-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.1.2-35). The field ul-AccessConfigListDCI-0-1-r17 only contains a list of UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.1.2-35A).

ul-FullPowerTransmission Configures the UE with UL full power transmission mode as specified in TS 38.213 [13]. This field is not configured if ul-powerControl is configured in the BWP-UplinkDedicated in which the PUCCH-Config is included.

PUSCH-ConfigDCI-0-3 field descriptions

harq-ProcessNumberSizeDCI-0-3 Configure the number of bits for the field "HARQ process number" in DCI format 0_3 (see TS 38.212 [17], clause 7.3.1). Only one of this field and harq-ProcessNumberSizeDCI-0-3-Ext is configured by network.

harq-ProcessNumberSizeDCI-0-3-Ext Configure the number of bits for the field "HARQ process number" in DCI format 0_3 in TN (see TS 38.212 [17], clause 7.3.1). This field can only be configured when the maximum number of layers configured for PUSCH is no more than 4. Only one of this field and harq-ProcessNumberSizeDCI-0-3 is configured by network.

numberOfBitsForRV-DCI-0-3 Configures the number of bits for "Redundancy version" in the DCI format 0_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

rbg-SizeDCI-0-3 Selection among configuration 1, configuration 2 and configuration 3 for RBG size for PUSCH scheduled by DCI format 0_3. The UE does not apply this field if resourceAllocationDCI-0-3 is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocationDCI-0-3 Configuration of resource allocation type 0 and resource allocation type 1 for DCI format 0_3 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-3 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_3. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

uci-OnPUSCH-ListDCI-0-3 Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

SDM-Scheme field descriptions

maxRankSDM, maxRankSDM-DCI-0-2 configure maximal number of MIMO layers of each panel in SDM scheme for codebook based PUSCH or for DCI 0_2 for codebook based PUSCH.

SFN-Scheme field descriptions

maxRankSFN, maxRankSFN-DCI-0-2 configure maximal number of MIMO layers of each panel in SFN scheme for codebook based PUSCH or for DCI 0_2 for codebook based PUSCH.

UCI-OnPUSCH field descriptions

betaOffsets Selection between and configuration of dynamic and semi-static beta-offset for DCI formats other than DCI format 0_2. If the field is not configured, the UE applies the value 'semiStatic' (see TS 38.213 [13], clause 9.3).

scaling Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI formats other than DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on. The value configured herein is applicable for PUSCH with configured grant (see TS 38.212 [17], clause 6.3).

UCI-OnPUSCH-DCI-0-2 field descriptions

betaOffsetsDCI-0-2 Configuration of beta-offset for DCI format 0_2. If semiStaticDCI-0-2 is chosen, the UE shall apply the value of 0 bit for the field of beta offset indicator in DCI format 0_2. If dynamicDCI-0-2 is chosen, the UE shall apply the value of 1 bit or 2 bits for the field of beta offset indicator in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

dynamicDCI-0-2 Indicates the UE applies the value 'dynamic' for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.3).

semiStaticDCI-0-2 Indicates the UE applies the value 'semiStatic' for DCI format 0_2. (see TS 38.212 [17], clause 7.3.1 and see TS 38.213 [13], clause 9.3).

scalingDCI-0-2 Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on (see TS 38.212 [17], clause 6.3).

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | The field is mandatory present if txConfig is set to codebook and absent otherwise. |
| RepTypeB | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise. |
| RepTypeB2 | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 or pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise. |
| SRSsets | This field is mandatory present when UE is configured with two SRS sets in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage codebook or non-codebook and none of multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel is configured. It is absent otherwise. |

#### –	PUSCH-ConfigCommon

The IE PUSCH-ConfigCommon is used to configure the cell specific PUSCH parameters.

PUSCH-ConfigCommon information element

-- ASN1START

-- TAG-PUSCH-CONFIGCOMMON-START

PUSCH-ConfigCommon ::=                  SEQUENCE {

groupHoppingEnabledTransformPrecoding   ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

pusch-TimeDomainAllocationList          PUSCH-TimeDomainResourceAllocationList                              OPTIONAL,   -- Need R

msg3-DeltaPreamble                      INTEGER (-1..6)                                                     OPTIONAL,   -- Need R

p0-NominalWithGrant                     INTEGER (-202..24)                                                  OPTIONAL,   -- Need R

...

}

-- TAG-PUSCH-CONFIGCOMMON-STOP

-- ASN1STOP

PUSCH-ConfigCommon field descriptions

groupHoppingEnabledTransformPrecoding For DMRS transmission with transform precoder, the NW may configure group hopping by this cell-specific parameter, see TS 38.211 [16], clause 6.4.1.1.1.2.

msg3-DeltaPreamble Power offset between msg3 and RACH preamble transmission. Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1)

p0-NominalWithGrant P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1) This field is cell specific

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1).

#### –	PUSCH-MutingResources

The IE PUSCH-MutingResources is used to configure the time location and frequency location of UL muting resources.

PUSCH-MutingResources information element

-- ASN1START

-- TAG-PUSCH-MUTINGRESOURCES-START

PUSCH-MutingResources-r19 ::= SEQUENCE {

symbolPos-r19                 SEQUENCE (SIZE (1..2)) OF INTEGER (0.. 13)                                    OPTIONAL,   -- Need R

combOffset-r19                INTEGER (0..1)                                                                OPTIONAL,   -- Need R

...

}

-- TAG-PUSCH-MUTINGRESOURCES-STOP

-- ASN1STOP

PUSCH-MutingResources field descriptions

symbolPos Configures the symbol location of one or two UL muting symbol(s) within a slot for PUSCH.

combOffset Configures the comb offset of UL muting symbol(s) for PUSCH.

#### –	PUSCH-PowerControl

The IE PUSCH-PowerControl is used to configure UE specific power control parameter for PUSCH.

PUSCH-PowerControl information element

-- ASN1START

-- TAG-PUSCH-POWERCONTROL-START

PUSCH-PowerControl ::=              SEQUENCE {

tpc-Accumulation                    ENUMERATED { disabled }                                                 OPTIONAL, -- Need S

msg3-Alpha                          Alpha                                                                   OPTIONAL, -- Need S

p0-NominalWithoutGrant              INTEGER (-202..24)                                                      OPTIONAL, -- Need M

p0-AlphaSets                        SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet     OPTIONAL, -- Need M

pathlossReferenceRSToAddModList     SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList    SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id

OPTIONAL,  -- Need N

twoPUSCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

deltaMCS                            ENUMERATED {enabled}                                                    OPTIONAL, -- Need S

sri-PUSCH-MappingToAddModList       SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList      SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId

OPTIONAL  -- Need N

}

P0-PUSCH-AlphaSet ::=               SEQUENCE {

p0-PUSCH-AlphaSetId                 P0-PUSCH-AlphaSetId,

p0                                  INTEGER (-16..15)                                                       OPTIONAL, -- Need S

alpha                               Alpha                                                                   OPTIONAL  -- Need S

}

P0-PUSCH-AlphaSetId ::=             INTEGER (0..maxNrofP0-PUSCH-AlphaSets-1)

PUSCH-PathlossReferenceRS ::=       SEQUENCE {

pusch-PathlossReferenceRS-Id        PUSCH-PathlossReferenceRS-Id,

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

}

}

PUSCH-PathlossReferenceRS-r16 ::=   SEQUENCE {

pusch-PathlossReferenceRS-Id-r16    PUSCH-PathlossReferenceRS-Id-v1610,

referenceSignal-r16                 CHOICE {

ssb-Index-r16                       SSB-Index,

csi-RS-Index-r16                    NZP-CSI-RS-ResourceId

}

}

DummyPathlossReferenceRS-v1710 ::= SEQUENCE {

pusch-PathlossReferenceRS-Id-r17    PUSCH-PathlossReferenceRS-Id-r17,

additionalPCI-r17                   AdditionalPCIIndex-r17                                         OPTIONAL  -- Need R

}

PUSCH-PathlossReferenceRS-Id ::=    INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)

PUSCH-PathlossReferenceRS-Id-v1610 ::= INTEGER (maxNrofPUSCH-PathlossReferenceRSs..maxNrofPUSCH-PathlossReferenceRSs-1-r16)

PUSCH-PathlossReferenceRS-Id-r17 ::= INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1-r16)

SRI-PUSCH-PowerControl ::=          SEQUENCE {

sri-PUSCH-PowerControlId            SRI-PUSCH-PowerControlId,

sri-PUSCH-PathlossReferenceRS-Id    PUSCH-PathlossReferenceRS-Id,

sri-P0-PUSCH-AlphaSetId             P0-PUSCH-AlphaSetId,

sri-PUSCH-ClosedLoopIndex           ENUMERATED { i0, i1 }

}

SRI-PUSCH-PowerControlId ::=        INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

PUSCH-PowerControl-v1610 ::=        SEQUENCE {

pathlossReferenceRSToAddModListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSsDiff-r16)) OF PUSCH-PathlossReferenceRS-r16

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseListSizeExt-v1610  SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSsDiff-r16)) OF PUSCH-PathlossReferenceRS-Id-v1610

OPTIONAL, -- Need N

p0-PUSCH-SetList-r16                SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF P0-PUSCH-Set-r16      OPTIONAL, -- Need R

olpc-ParameterSet                   SEQUENCE {

olpc-ParameterSetDCI-0-1-r16        INTEGER (1..2)                                                      OPTIONAL, -- Need R

olpc-ParameterSetDCI-0-2-r16        INTEGER (1..2)                                                      OPTIONAL  -- Need R

}                                                                                                           OPTIONAL, -- Need M

...,

[[

sri-PUSCH-MappingToAddModList2-r17  SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList2-r17 SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId OPTIONAL, -- Need N

p0-PUSCH-SetList2-r17               SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF P0-PUSCH-Set-r16      OPTIONAL, -- Need R

dummy SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs-r16)) OF DummyPathlossReferenceRS-v1710          OPTIONAL  -- Need N

]],

[[

msg3-Alpha-SBFD-r19                 Alpha                                                                   OPTIONAL -- Need S

]]

}

P0-PUSCH-Set-r16 ::=                SEQUENCE {

p0-PUSCH-SetId-r16                  P0-PUSCH-SetId-r16,

p0-List-r16                         SEQUENCE (SIZE (1..maxNrofP0-PUSCH-Set-r16)) OF P0-PUSCH-r16            OPTIONAL, -- Need R

...

}

P0-PUSCH-SetId-r16 ::=              INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

P0-PUSCH-r16 ::=                    INTEGER (-16..15)

-- TAG-PUSCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUSCH-AlphaSet field descriptions

alpha alpha value for PUSCH with grant (except msg3) (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

p0 P0 value for PUSCH with grant (except msg3) in steps of 1dB (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 0.

P0-PUSCH-Set field descriptions

p0-List Configuration of {p0-PUSCH, p0-PUSCH} sets for PUSCH. If SRI is present in the DCI, then one p0-PUSCH can be configured in P0-PUSCH-Set. If SRI is not present in the DCI, and both olpc-ParameterSetDCI-0-1 and olpc-ParameterSetDCI-0-2 are configured to be 1 bit, then one p0-PUSCH can be configured in P0-PUSCH-Set. If SRI is not present in the DCI, and if any of olpc-ParameterSetDCI-0-1 and olpc-ParameterSetDCI-0-2 is configured to be 2 bits, then two p0-PUSCH values can be configured in P0-PUSCH-Set (see TS 38.213 [13] clause 7 and TS 38.212 [17] clause 7.3.1).

p0-PUSCH-SetId Configure the index of a p0-PUSCH-Set (see TS 38.213 [13] clause 7 and TS 38.212 [17] clause 7.3.1).

PUSCH-PowerControl field descriptions

deltaMCS Indicates whether to apply delta MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH (see TS 38.213 [13], clause 7.1).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

msg3-Alpha Dedicated alpha value for msg3 PUSCH (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

msg3-Alpha-SBFD Configures separate dedicated alpha value for msg3 PUSCH in SBFD symbols. When the field is absent, msg3-Alpha configured for msg3 PUSCH transmission in non-SBFD symbols is used if msg3 PUSCH transmission is transmitted in SBFD symbols.

olpc-ParameterSetDCI-0-1, olpc-ParameterSetDCI-0-2 Configures the number of bits for Open-loop power control parameter set indication for DCI format 0_1/0_2 in case SRI is not configured in the DCI. 2 bits is applicable only if SRI is not present in the DCI format 0_1. The field olpc-ParameterSetDCI-0-1 applies to DCI format 0_1 and the field olpc-ParameterSetDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11).

p0-AlphaSets Configuration {p0-pusch, alpha} sets for PUSCH (except msg3 and msgA PUSCH), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3/msgA PUSCH, P0-UE is set to 0 and alpha is set according to either msg3-Alpha or msgA-Alpha (see TS 38.213 [13], clause 7.1). This field is not configured if unifiedTCI-StateType is configured for the serving cell.

p0-NominalWithoutGrant P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1).

p0-PUSCH-SetList Configure one additional P0-PUSCH-Set per SRI. If present, the one bit or 2 bits in the DCI is used to dynamically indicate among the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value(s) from the P0-PUSCH-Set (See TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 17).

p0-PUSCH-SetList2 For indicating per-TRP OLPC set in DCI format 0_1/0_2 with the open-loop power control parameter set indication field, a second p0-PUSCH-SetList-r16 is used. When this field is present the p0-PUSCH-SetList-r16 corresponds to the first SRS resource set (see TS 38.213 [13]).

pathlossReferenceRSToAddModList, pathlossReferenceRSToAddModListSizeExt A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUSCH path loss estimation. The set consists of Reference Signals configured using pathLossReferenceRSToAddModList and Reference Signals configured using pathlossReferenceRSToAddModListSizeExt. Up to maxNrofPUSCH-PathlossReferenceRSs may be configured (see TS 38.213 [13], clause 7.1). No elements in the list pathLossReferenceRSToAddModList nor pathlossReferenceRSToAddModListSizeExt are configured if unifiedTCI-StateType is configured for the serving cell.

pathlossReferenceRSToReleaseList, pathlossReferenceRSToReleaseListSizeExt Lists of reference signals for PUSCH path loss estimation to be released by the UE.

sri-PUSCH-MappingToAddModList A list of SRI-PUSCH-PowerControl elements among which one is selected by the SRI field in DCI (see TS 38.213 [13], clause 7.1). No elements in this list are configured if unifiedTCI-StateType is configured for the serving cell.

sri-PUSCH-MappingToAddModList2 A list of SRI-PUSCH-PowerControl elements for second SRS-resource set, among which one is selected by the SRI field in DCI (see TS 38.213 [13], clause 7.1). When this field is present the sri-PUSCH-MappingToAddModList corresponds to the first SRS resource set for PUSCH. No elements in this list are configured if unifiedTCI-StateType is configured for the serving cell.

tpc-Accumulation If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation. If the field is absent, TPC accumulation is enabled (see TS 38.213 [13], clause 7.1).

twoPUSCH-PC-AdjustmentStates Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains two power control states (i.e., fc(i,0) and fc(i,1)). If the field is absent, it maintains one power control state (i.e., fc(i,0)) (see TS 38.213 [13], clause 7.1).

SRI-PUSCH-PowerControl field descriptions

sri-P0-PUSCH-AlphaSetId The ID of a P0-PUSCH-AlphaSet as configured in p0-AlphaSets in PUSCH-PowerControl.

sri-PUSCH-ClosedLoopIndex The index of the closed power control loop associated with this SRI-PUSCH-PowerControl.

sri-PUSCH-PathlossReferenceRS-Id The ID of PUSCH-PathlossReferenceRS as configured in the pathlossReferenceRSToAddModList in PUSCH-PowerControl.

sri-PUSCH-PowerControlId The ID of this SRI-PUSCH-PowerControl configuration. It is used as the codepoint (payload) in the SRI DCI field.

#### –	PUSCH-ServingCellConfig

The IE PUSCH-ServingCellConfig is used to configure UE specific PUSCH parameters that are common across the UE's BWPs of one serving cell.

PUSCH-ServingCellConfig information element

-- ASN1START

-- TAG-PUSCH-SERVINGCELLCONFIG-START

PUSCH-ServingCellConfig ::=             SEQUENCE {

codeBlockGroupTransmission              SetupRelease { PUSCH-CodeBlockGroupTransmission }       OPTIONAL,   -- Need M

rateMatching                            ENUMERATED {limitedBufferRM}                            OPTIONAL,   -- Need S

xOverhead                               ENUMERATED {xoh6, xoh12, xoh18}                         OPTIONAL,   -- Need S

...,

[[

maxMIMO-Layers                          INTEGER (1..4)                                          OPTIONAL,   -- Need M

processingType2Enabled                  BOOLEAN                                                 OPTIONAL    -- Need M

]],

[[

maxMIMO-LayersDCI-0-2-r16               SetupRelease { MaxMIMO-LayersDCI-0-2-r16}               OPTIONAL    -- Need M

]],

[[

nrofHARQ-ProcessesForPUSCH-r17          ENUMERATED {n32}                                        OPTIONAL,   -- Need R

uplinkHARQ-mode-r17                     SetupRelease { UplinkHARQ-mode-r17}                     OPTIONAL    -- Need M

]],

[[

maxMIMO-Layers-v1810                    INTEGER (5..8)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSDM-r18                INTEGER (1..2)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSDM-DCI-0-2-r18        INTEGER (1..2)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSFN-r18                INTEGER (1..2)                                          OPTIONAL,   -- Need R

maxMIMO-LayersforSFN-DCI-0-2-r18        INTEGER (1..2)                                          OPTIONAL    -- Need R

]]

}

PUSCH-CodeBlockGroupTransmission ::=    SEQUENCE {

maxCodeBlockGroupsPerTransportBlock     ENUMERATED {n2, n4, n6, n8},

...

}

MaxMIMO-LayersDCI-0-2-r16 ::=           INTEGER (1..4)

UplinkHARQ-mode-r17 ::=                 BIT STRING (SIZE (32))

-- TAG-PUSCH-SERVINGCELLCONFIG-STOP

-- ASN1STOP

PUSCH-CodeBlockGroupTransmission field descriptions

maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB (see TS 38.213 [13], clause 9.1).

PUSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.214 [19], clause 6.1.5). The network does not configure this field if the SCS of at least one UL BWP configured in the cell is 480 or 960 kHz.

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PUSCH in all BWPs of the corresponding UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRank to the same value. The field maxMIMO-Layers refers to DCI format 0_1. If network configures maxMIMO-Layers-v1810 the UE ignores maxMIMO-Layers (without suffix).

maxMIMO-LayersforSDM Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SDM scheme for NCB PUSCH

maxMIMO-LayersforSDM-DCI-0-2 Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SDM scheme for DCI format 0_2 for NCB PUSCH

maxMIMO-LayersforSFN Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SFN scheme for NCB PUSCH

maxMIMO-LayersforSFN-DCI-0-2 Parameter to indicate maximal number of MIMO layers of each panel for PUSCH with SFN scheme for DCI format 0_2 for NCB PUSCH

nrofHARQ-ProcessesForPUSCH The number of HARQ processes to be used on the PUSCH of a serving cell. Value n32 corresponds to 32 HARQ processes. If the field is absent, the UE uses 16 HARQ processes (see TS 38.214 [19], clause 6.1).

processingType2Enabled Enables configuration of advanced processing time capability 2 for PUSCH (see 38.214 [19], clause 6.4).

rateMatching Enables LBRM (Limited buffer rate-matching). When the field is absent the UE applies FBRM (Full buffer rate-matchingLBRM) (see TS 38.212 [17], clause 5.4.2).

xOverhead If the field is absent, the UE applies the value 0 (see TS 38.214 [19], clause 6.1.4.2).

maxMIMO-LayersDCI-0-2 Indicates the maximum MIMO layer to be used for PUSCH for DCI format 0_2 in all BWPs of the corresponding UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRankDCI-0-2 to the same value.

uplinkHARQ-mode Used to set the HARQ mode per HARQ process ID, see TS 38.321 [3]. The first/leftmost bit corresponds to HARQ process ID 0, the next bit to HARQ process ID 1 and so on. Bits corresponding to HARQ process IDs that are not configured shall be ignored. A bit set to one identifies a HARQ process with HARQmodeA and a bit set to zero identifies a HARQ process with HARQ modeB. This field applies for SRBs and DRBs.

#### –	PUSCH-TimeDomainResourceAllocationList

The IE PUSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PUSCH. PUSCH-TimeDomainResourceAllocationList contains one or more of such PUSCH-TimeDomainResourceAllocations. The network indicates in the UL grant which of the configured time domain allocations the UE shall apply for that UL grant. The UE determines the bit width of the DCI field based on the number of entries in the PUSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PUSCH-TimeDomainResourceAllocation information element

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

k2                                      INTEGER(0..32)                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                     INTEGER(0..32)          OPTIONAL,   -- Need S

puschAllocationList-r16                    SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16,

...

}

PUSCH-Allocation-r16 ::=  SEQUENCE {

mappingType-r16                           ENUMERATED {typeA, typeB}                     OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbolAndLength-r16                  INTEGER (0..127)                              OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbol-r16                           INTEGER (0..13)                               OPTIONAL,   -- Cond RepTypeB

length-r16                                INTEGER (1..14)                               OPTIONAL,   -- Cond RepTypeB

numberOfRepetitions-r16                   ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL,   -- Cond Format01-02

...,

[[

numberOfRepetitionsExt-r17                ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2,

spare1}                           OPTIONAL,   -- Cond Format01-02-For-TypeA

numberOfSlotsTBoMS-r17                    ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1}  OPTIONAL,   -- Need R

extendedK2-r17                            INTEGER (0..128)                              OPTIONAL    -- Cond MultiPUSCH

]],

[[

ul-MutingIndicator-r19                   ENUMERATED {enabled}                           OPTIONAL,   -- Need S

occ-Length-r19                           ENUMERATED {n2, n4}                            OPTIONAL    -- Need R

]]

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PUSCH-TimeDomainResourceAllocationList field descriptions

extendedK2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) configurable per PUSCH allocation. Only values {0..32} are applicable for PUSCH SCS of 120 kHz and for FR1. When the field is absent for the first PUSCH if multiple PUSCH are configured per PDCCH and k2-r16 is absent, or when the field is absent and only one PUSCH is configured per PDCCH and k2-r16 is absent, the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, the value 3 when PUSCH SCS is 120 kHz, the value 11 when PUSCH SCS is 480 kHz, and the value 21 when PUSCH SCS is 960 kHz. If multiple contiguous PUSCHs are configured per PDCCH, when the field extendedK2(n) corresponding to k2 of the PUSCH(s) in the n-th slot (n>1), or of the PUSCH(s) except the first PUSCH in the first slot (n=1), is absent, the UE applies k2 of the first PUSCH plus n-1.

k2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1). When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, and the value 3 when PUSCH SCS is 120 kHz. k2 is absent/ignored if extendedK2 is present.

length Indicates the length allocated for PUSCH for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1).

mappingType Mapping type (see TS 38.214 [19], clause 6.1.2.1).

numberOfRepetitions Number of repetitions for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1). When numberOfSlotsTBoMS-r17 is set to 2, 4 or 8 (i.e. TB processing over multi-slot (TBoMS) PUSCH is enabled), it indicates the number of repetitions of a single TBoMS.

numberOfRepetitionsExt Number of repetitions for DCI format 0_1/0_2 if pusch-RepTypeIndicatorDCI-0-1/pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB (see TS 38.214 [19], clause 6.1.2.1). If this field is present, the field numberOfRepeitions-r16 is ignored for PUSCH repetition Type A.

numberOfSlotsTBoMS Number of slots allocated for TB processing over multi-slot PUSCH for DCI format 0_1/0_2. If a number of repetitions K is configured by numberOfRepetitions or numberOfRepetitionsExt, the network configures numberOfSlotsTBoMS (N) and K such that N*K ≤ 32 (see TS 38.214 [19], clause 6.1.2.1). The network does not configure the numberOfSlotsTBoMS-r17 simultaneously with the pusch-TimeDomainAllocationListForMultiPUSCH-r16. The network does not configure numberOfSlotsTBoMS-r17 together with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true.

occ-Length Indicates the inter-slot OCC length for a dynamic grant or a Type 2 CG configuration, see TS 38.214 [19].

puschAllocationList The field puschAllocationList-r16 indicates one or multiple PUSCH continuous in time domain which share a common k2 (see TS 38.214 [19], clause 6.1.2.1). In this release, this field configures one or multiple PUSCH that may be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 6.1.2.1). The puschAllocationList-r16 only has one element in pusch-TimeDomainAllocationListDCI-0-1-r16 and in pusch-TimeDomainAllocationListDCI-0-2-r16.

startSymbol Indicates the index of start symbol for PUSCH for DCI format 0_1/0_2 (see TS 38.214 [19], clause 6.1.2.1).

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary. (see TS 38.214 [19], clause 6.1.2.1).

ul-MutingIndicator Indicates whether the semi-statically configured UL muting symbol(s) is enabled for a given PUSCH in the PUSCH TDRA table. If this parameter is absent, the UL muting symbol(s) are disabled.

| Conditional Presence | Explanation |
| --- | --- |
| Format01-02 | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1 and in pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present. |
| Format01-02-For-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is optionally present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is optionally present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB, Need R. It is absent otherwise, Need R. |
| NotFormat01-02-Or-TypeA | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is mandatory present. In pusch-TimeDomainAllocationListDCI-0-1, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-1 is not set to pusch-RepTypeB. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-2 is not set to pusch-RepTypeB. It is absent otherwise, Need R. |
| RepTypeB | In pusch-TimeDomainAllocationListForMultiPUSCH-r16, the field is absent. In pusch-TimeDomainAllocationListDCI-0-1, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise, Need R. In pusch-TimeDomainAllocationListDCI-0-2, the field is mandatory present if pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise, Need R. |
| MultiPUSCH | In case size of puschAllocationList is higher than 1, the field extendedK2(n) corresponding to k2 of the n-th PUSCH, n>1, is mandatory present for all n, if any two consecutive PUSCHs are non-contiguous. Otherwise, it is optionally present, Need S. |

#### –	PUSCH-TPC-CommandConfig

The IE PUSCH-TPC-CommandConfig is used to configure the UE for extracting TPC commands for PUSCH from a group-TPC messages on DCI.

PUSCH-TPC-CommandConfig information element

-- ASN1START

-- TAG-PUSCH-TPC-COMMANDCONFIG-START

PUSCH-TPC-CommandConfig ::=         SEQUENCE {

tpc-Index                           INTEGER (1..15)                                                 OPTIONAL,   -- Cond SUL

tpc-IndexSUL                        INTEGER (1..15)                                                 OPTIONAL,   -- Cond SUL-Only

targetCell                          ServCellIndex                                                   OPTIONAL,   -- Need S

...

}

-- TAG-PUSCH-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

PUSCH-TPC-CommandConfig field descriptions

targetCell The serving cell to which the acquired power control commands are applicable. If the value is absent, the UE applies the TPC commands to the serving cell on which the command has been received.

tpc-Index An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

tpc-IndexSUL An index determining the position of the first bit of TPC command inside the DCI format 2-2 payload.

| Conditional Presence | Explanation |
| --- | --- |
| SUL-Only | The field is optionally present, Need R, if supplementaryUplink is configured within ServingCellConfig. It is absent otherwise. |
| SUL | The field is optionally present, Need R, if supplementaryUplink is configured within ServingCellConfig. It is mandatory present otherwise. |

#### –	QFI

The IE QFI is used to indicate the QoS Flow Identifier.

QFI information element

-- ASN1START

-- TAG-QFI-START

QFI ::=                             INTEGER (0..maxQFI)

-- TAG-QFI-STOP

-- ASN1STOP

#### –	Q-OffsetRange

The IE Q-OffsetRange is used to indicate a cell, beam or measurement object specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

Q-OffsetRange information element

-- ASN1START

-- TAG-Q-OFFSETRANGE-START

Q-OffsetRange ::=                   ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	Q-QualMin

The IE Q-QualMin is used to indicate for cell selection/ re-selection the required minimum received RSRQ level in the (NR) cell. Corresponds to parameter Qqualmin in TS 38.304 [20]. Actual value Qqualmin = field value [dB].

Q-QualMin information element

-- ASN1START

-- TAG-Q-QUALMIN-START

Q-QualMin ::=                       INTEGER (-43..-12)

-- TAG-Q-QUALMIN-STOP

-- ASN1STOP

#### –	Q-RxLevMin

The IE Q-RxLevMin is used to indicate for cell selection/ re-selection the required minimum received RSRP level in the (NR) cell. Corresponds to parameter Qrxlevmin in TS 38.304 [20]. Actual value Qrxlevmin = field value * 2 [dBm].

Q-RxLevMin information element

-- ASN1START

-- TAG-Q-RXLEVMIN-START

Q-RxLevMin ::=                      INTEGER (-70..-22)

-- TAG-Q-RXLEVMIN-STOP

-- ASN1STOP

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITYCONFIG-START

QuantityConfig ::=                  SEQUENCE {

quantityConfigNR-List               SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR          OPTIONAL,   -- Need M

...,

[[

quantityConfigEUTRA                 FilterConfig                                                            OPTIONAL    -- Need M

]],

[[

quantityConfigUTRA-FDD-r16          QuantityConfigUTRA-FDD-r16                                              OPTIONAL,   -- Need M

quantityConfigCLI-r16               FilterConfigCLI-r16                                                     OPTIONAL    -- Need M

]]

}

QuantityConfigNR::=                 SEQUENCE {

quantityConfigCell                  QuantityConfigRS,

quantityConfigRS-Index              QuantityConfigRS                                                        OPTIONAL    -- Need M

}

QuantityConfigRS ::=                SEQUENCE {

ssb-FilterConfig                    FilterConfig,

csi-RS-FilterConfig                 FilterConfig

}

FilterConfig ::=                    SEQUENCE {

filterCoefficientRSRP               FilterCoefficient                                       DEFAULT fc4,

filterCoefficientRSRQ               FilterCoefficient                                       DEFAULT fc4,

filterCoefficientRS-SINR            FilterCoefficient                                       DEFAULT fc4

}

FilterConfigCLI-r16 ::=             SEQUENCE {

filterCoefficientSRS-RSRP-r16       FilterCoefficient                                       DEFAULT fc4,

filterCoefficientCLI-RSSI-r16       FilterCoefficient                                       DEFAULT fc4

}

QuantityConfigUTRA-FDD-r16 ::=      SEQUENCE {

filterCoefficientRSCP-r16           FilterCoefficient                                       DEFAULT fc4,

filterCoefficientEcNO-r16           FilterCoefficient                                       DEFAULT fc4

}

-- TAG-QUANTITYCONFIG-STOP

-- ASN1STOP

QuantityConfigNR field descriptions

quantityConfigCell Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

quantityConfigRS-Index Specifies L3 filter configurations for measurement results per RS index for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

QuantityConfigRS field descriptions

csi-RS-FilterConfig CSI-RS based L3 filter configurations: Specifies L3 filter configurations for CSI-RSRP, CSI-RSRQ and CSI-SINR measurement results from the L1 filter(s), as defined in TS 38.215 [9].

ssb-FilterConfig SS Block based L3 filter configurations: Specifies L3 filter configurations for SS-RSRP, SS-RSRQ and SS-SINR measurement results from the L1 filter(s), as defined in TS 38.215 [9].

QuantityConfigUTRA-FDD field descriptions

filterCoefficientRSCP Specifies L3 filter coefficient for FDD UTRAN CPICH_RSCP measuement results from L1 filter.

filterCoefficientEcN0 Specifies L3 filter coefficient for FDD UTRAN CPICH_EcN0 measuement results from L1 filter.

#### –	RACH-ConfigCommon

The IE RACH-ConfigCommon is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

groupBconfigured                    SEQUENCE {

ra-Msg3SizeGroupA                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB            ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA          INTEGER (1..64)

}                                                                                                       OPTIONAL,   -- Need R

ra-ContentionResolutionTimer            ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB                       RSRP-Range                                                      OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL                   RSRP-Range                                                      OPTIONAL,   -- Cond SUL

prach-RootSequenceIndex                 CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

msg1-SubcarrierSpacing                  SubcarrierSpacing                                               OPTIONAL,   -- Cond L139

restrictedSetConfig                     ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

...,

[[

ra-PrioritizationForAccessIdentity-r16  SEQUENCE {

ra-Prioritization-r16                   RA-Prioritization,

ra-PrioritizationForAI-r16              BIT STRING (SIZE (2))

}                                                                                                       OPTIONAL,   -- Cond InitialBWP-Only

prach-RootSequenceIndex-r16             CHOICE {

l571                                    INTEGER (0..569),

l1151                                   INTEGER (0..1149)

}   OPTIONAL   -- Need R

]],

[[

ra-PrioritizationForSlicing-r17         RA-PrioritizationForSlicing-r17                          OPTIONAL,   -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL -- Cond AdditionalRACH

]],

[[

rach-Config-Adapt-r19               RA-AdaptConfig-r19                              OPTIONAL           -- Need R

]]

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP

RACH-ConfigCommon field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 4-step RA. The network does not configure this list to have more than 16 entries.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).

msg3-transformPrecoder Enables the transform precoder for Msg3 transmission according to clause 6.1.3 of TS 38.214 [19]. If the field is absent, the UE disables the transformer precoder (see TS 38.213 [13], clause 8.3).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A. This determines implicitly the number of CB preambles per SSB available in group B. (see TS 38.321 [3], clause 5.1.1). The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839 or L=139 or L=571 or L=1151. The length of the root sequence corresponding with the index indicated in this IE should be consistent with the one indicated in prach-ConfigurationIndex in the RACH-ConfigDedicated (if configured). If prach-RootSequenceIndex-r16 is signalled, UE shall ignore the prach-RootSequenceIndex (without suffix). For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

ra-ContentionResolutionTimer The initial value for the contention resolution timer (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on.

ra-Msg3SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A (see TS 38.321 [3], clause 5.1.2). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

ra-Prioritization Parameters which apply for prioritized random access procedure on any UL BWP of SpCell for specific Access Identities (see TS 38.321 [3], clause 5.1.1a).

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 indicates that the field ra-Prioritization-r16 applies otherwise the field does not apply (see TS 23.501 [32]).

ra-PrioritizationForSlicing Parameters which apply to configure prioritized CBRA 4-step random access type for slicing.

rach-Config-Adapt Parameters which apply to configure adaptive random access occasions.

rach-ConfigGeneric RACH parameters for both regular random access and beam failure recovery.

restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

rsrp-ThresholdSSB-SUL The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). The value applies to all the BWPs and all RACH configurations.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEighth corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). See TS 38.213 [13].

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources defined in RACH-ConfigCommon, excluding preambles used for other purposes (e.g. for SI request). If the field is absent, all 64 preambles are available for RA. The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, i.e. it should be a multiple of the number of SSBs per RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| AdditionalRACH | The field is mandatory present if the RACH-ConfigCommon is included in an AdditionalRACH-Config. When included in initialUplinkBWP-RedCap to indicate other feature(s) than redcap and eRedCap, this field is mandatory present with at least FeatureCombinationPreambles list entries: the list entry/entries indicating only redcap or eRedCap and the other(s) indicating both redcap or eRedCap and one or multiple other feature(s) (e.g., smallData, nsag or msg3-Repetitions). When included in initialUplinkBWP-RedCap to indicate eRedCap and RedCap separately, this field is mandatory present with at least two FeatureCombinationPreambles list entries: one list entry indicating only redcap and the other list entry indicating only eRedCap. Otherwise, it is optional, Need R. |
| InitialBWP-Only | This field is optionally present, Need R, if this BWP is the initial BWP of SpCell. Otherwise, the field is absent. |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, or if L=571 for FR2-2, otherwise the field is absent, Need S. |
| SUL | The field is mandatory present in rach-ConfigCommon in initialUplinkBWP if supplementaryUplink is configured in ServingCellConfigCommonSIB or if supplementaryUplinkConfig is configured in ServingCellConfigCommon; otherwise, the field is absent. This field is neither configured in additionalRACH-Config nor in sbfd-RACH-DualConfig. |

#### –	RACH-ConfigCommonTwoStepRA

The IE RACH-ConfigCommonTwoStepRA is used to specify cell specific 2-step random-access type parameters.

RACH-ConfigCommonTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-START

RACH-ConfigCommonTwoStepRA-r16 ::=                   SEQUENCE {

rach-ConfigGenericTwoStepRA-r16                      RACH-ConfigGenericTwoStepRA-r16,

msgA-TotalNumberOfRA-Preambles-r16                   INTEGER (1..63)                                    OPTIONAL, -- Need S

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16   CHOICE {

oneEighth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                              ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                                 INTEGER (1..16),

eight                                                INTEGER (1..8),

sixteen                                              INTEGER (1..4)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-CB-PreamblesPerSSB-PerSharedRO-r16              INTEGER (1..60)                                                OPTIONAL, -- Cond SharedRO

msgA-SSB-SharedRO-MaskIndex-r16                      INTEGER (1..15)                                                OPTIONAL, -- Need S

groupB-ConfiguredTwoStepRA-r16                       GroupB-ConfiguredTwoStepRA-r16                                 OPTIONAL, -- Need S

msgA-PRACH-RootSequenceIndex-r16                     CHOICE {

l839                                                 INTEGER (0..837),

l139                                                 INTEGER (0..137),

l571                                                 INTEGER (0..569),

l1151                                                INTEGER (0..1149)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-TransMax-r16                                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}     OPTIONAL, -- Need R

msgA-RSRP-Threshold-r16                              RSRP-Range                                                     OPTIONAL, -- Cond 2Step4Step

msgA-RSRP-ThresholdSSB-r16                           RSRP-Range                                                     OPTIONAL, -- Need R

msgA-SubcarrierSpacing-r16                           SubcarrierSpacing                                              OPTIONAL, -- Cond 2StepOnlyL139

msgA-RestrictedSetConfig-r16                         ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB}                                OPTIONAL, -- Cond 2StepOnly

ra-PrioritizationForAccessIdentityTwoStep-r16        SEQUENCE {

ra-Prioritization-r16                                RA-Prioritization,

ra-PrioritizationForAI-r16                           BIT STRING (SIZE (2))

}                                                                                                                   OPTIONAL, -- Cond InitialBWP-Only

ra-ContentionResolutionTimer-r16                     ENUMERATED {sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}     OPTIONAL, -- Cond 2StepOnly

...,

[[

ra-PrioritizationForSlicingTwoStep-r17               RA-PrioritizationForSlicing-r17              OPTIONAL, -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL  -- Cond AdditionalRACH

]]

}

GroupB-ConfiguredTwoStepRA-r16 ::=                       SEQUENCE {

ra-MsgA-SizeGroupA-r16                               ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800,

b1000, b72, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r16                         ENUMERATED {minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r16                       INTEGER (1..64)

}

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-STOP

-- ASN1STOP

RACH-ConfigCommonTwoStepRA field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 2-step RA. The network does not configure this list to have more than 16 entries.

groupB-ConfiguredTwoStepRA Preamble grouping for 2-step random access type. If the field is absent then there is only one preamble group configured and only one msgA PUSCH configuration.

msgA-CB-PreamblesPerSSB-PerSharedRO Number of contention-based preambles used for 2-step RA type from the non-CBRA 4-step type preambles associated with each SSB for RO shared with 4-step type RA. The number of preambles for 2-step RA type shall not exceed the number of preambles per SSB minus the number of contention-based preambles per SSB for 4-step type RA. The possible value range for this parameter needs to be aligned with value range for the configured SSBs per RACH occasion in ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. The field is only applicable for the case of shared ROs with 4-step type random access.

msgA-PRACH-RootSequenceIndex PRACH root sequence index. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e., not within AdditionalRACH-Config), the UE applies the value in field prach-RootSequenceIndex in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the corresponding value of prach-RootSequenceIndex in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access. For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

msgA-RestrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets for 2-step random access type preamble. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE applies the value in field restrictedSetConfig in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the value of restrictedSetConfig in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access.

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-RSRP-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config) and both 2-step and 4-step are configured for the BWP, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. If the field is not configured in AdditionalRACH-Config and both 2-step and 4-step are configured in AdditionalRACH-Config, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon in the same AdditionalRACH-Config. The field is not present when RACH occasions are shared between 2-step and 4-step type random access in the BWP.

msgA-SSB-SharedRO-MaskIndex Indicates the subset of 4-step type ROs shared with 2-step random access type for each SSB. This field is configured when there is more than one RO per SSB. If the field is absent, and 4-step and 2-step has shared ROs, then all ROs are shared.

msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. If the field is absent, the UE applies the SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]) in case of 2-step only BWP, otherwise the UE applies the same SCS as Msg1 derived from RACH-ConfigCommon. The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).

msgA-TotalNumberOfRA-Preambles Indicates the total number of preambles used for contention-based and contention-free 2-step random access type when ROs for 2-step are not shared with 4-step. If the field is absent, and 2-step and 4-step does not have shared ROs, all 64 preambles are available for 2-step random access type.

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent, switching from 2-step RA type to 4-step RA type is not allowed.

ra-ContentionResolutionTimer The initial value for the contention resolution timer for fallback RAR in case no 4-step random access type is configured (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on. If both 2-step and 4-step random access type resources are configured on the BWP, then this field is absent. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config.

ra-Prioritization Parameters which apply for prioritized random access procedure on any UL BWP of SpCell for specific Access Identities (see TS 38.321 [3], clause 5.1.1a).

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 for an Access Identity indicates that the field ra-Prioritization-r16 applies, otherwise the field does not apply.

ra-PrioritizationForSlicingTwoStep Parameters which apply to configure prioritized CBRA 2-step random access type for slicing.

rach-ConfigGenericTwoStepRA 2-step random access type parameters for both regular random access and beam failure recovery.

GroupB-ConfiguredTwoStepRA field descriptions

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see TS 38.321 [3], clause 5.1.1).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A for idle/inactive or connected mode. The setting of the number of preambles for each group should be consistent with msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB or msgA-CB-PreamblesPerSSB-PerSharedRO if configured.

ra-MsgA-SizeGroupA Transport block size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| 2Step4Step | The field is mandatory present if both 2-step random access type and 4-step random access type are configured in the BWP, otherwise the field is not present. The field is mandatory present in msgA-ConfigCommon field in AdditionalRACH-Config if both 2-step random access type and 4-step random access type are configured for the same feature combination in the BWP. |
| 2StepOnlyL139 | The field is mandatory present if msgA-PRACH-RootSequenceIndex L=139 and no 4-step random access type is configured, or if L=571 for FR2-2 and no 4-step random access type is configured, otherwise the field is absent, Need S. |
| 2StepOnly | The field is mandatory present in msgA-ConfigCommon field in BWP-UplinkCommon if rach-ConfigCommon field is absent in this BWP-UplinkCommon, otherwise the field is optionally present in msgA-ConfigCommon field in BWP-UplinkCommon, Need S. The field is mandatory present in msgA-ConfigCommon field in AdditionalRACH-Config if rach-ConfigCommon field is absent in this AdditionalRACH-Config, otherwise the field is optionally present in msgA-ConfigCommon field in AdditionalRACH-Config, Need S. |
| AdditionalRACH | The field is mandatory present if the msgA-ConfigCommon is included in an AdditionalRACH-Config. When included in initialUplinkBWP-RedCap to indicate other feature(s) than redcap, this field is mandatory present with at least two FeatureCombinationPreambles list entries: one list entry indicating only redcap and the other(s) indicating both redcap and one or multiple other feature(s) (e.g. smallData, nsag or msg3-Repetitions). Otherwise, it is optional, Need R. |
| InitialBWP-Only | This field is optionally present, Need R, if this BWP is the initial BWP of SpCell. Otherwise, the field is absent. |
| SharedRO | The field is mandatory present if the 2-step random access type occasions are shared with 4-step random access type, otherwise the field is not present. |

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization                                                       OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16                                                        OPTIONAL  -- Need S

]],

[[

ra-OccasionType-r19             ENUMERATED {sbfd}                                                       OPTIONAL  -- Need S

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                             OPTIONAL -- Cond Occasions

]],

[[

msg1-RepetitionNum-r18          ENUMERATED {n2, n4, n8, spare1}                                               OPTIONAL -- Cond 4StepCFRArep

]]

}

CFRA-TwoStep-r16 ::=                    SEQUENCE {

occasionsTwoStepRA-r16                  SEQUENCE {

rach-ConfigGenericTwoStepRA-r16         RACH-ConfigGenericTwoStepRA-r16,

ssb-PerRACH-OccasionTwoStepRA-r16       ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}

}                                                                                                     OPTIONAL, -- Need S

msgA-CFRA-PUSCH-r16                     MsgA-PUSCH-Resource-r16,

msgA-TransMax-r16                       ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}    OPTIONAL, -- Need S

resourcesTwoStep-r16                    SEQUENCE {

ssb-ResourceList                        SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex                INTEGER (0..15)

},

...

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...,

[[

msgA-PUSCH-Resource-Index-r16   INTEGER (0..3071)     OPTIONAL  -- Cond 2StepCFRA

]]

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Per RO type, each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

msg1-RepetitionNum Indicates the MSG1 repetition number used for contention free 4-step random access type in TS 38.321 [3]. If this field is absent, the UE performs contention free 4-step random access without MSG1-Repetitions.

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList. The UE shall ignore this field if the field msg1-RepetitionNum included in CFRA is configured.

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

msgA-PUSCH-Resource-Index Identifies the index of the PUSCH resource used for MSGA CFRA. The PUSCH resource index indicates a valid PUSCH occasion (as specified in TS 38.213 [13], clause 8.1A) and the associated DMRS resources corresponding to a PRACH slot. The PUSCH resource indexes are sequentially numbered and are mapped to valid PUSCH occasions corresponding to a PRACH slot which are ordered, first, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions; second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRSid resource index is determined first in an ascending order of a DMRS port index and then in an ascending order of a DMRS sequence index, third in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot and fourth, in increasing order of indexes for PUSCH slots. For the case of contention free 2-step random access type, if this field is absent, the UE shall use the value 0.

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

CFRA-TwoStep field descriptions

msgA-CFRA-PUSCH PUSCH resource configuration(s) for msgA CFRA.

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step type random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent in cfra-TwoStep, switching from 2-step RA type to 4-step RA type is not allowed.

occasionsTwoStepRA RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommonTwoStepRA in the first active UL BWP.

ra-SSB-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGenericTwoStepRA Configuration of contention free random access occasions for CFRA 2-step random access type.

ssb-PerRACH-OccasionTwoStep Number of SSBs per RACH occasion for 2-step random access type.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If this field and cfra-TwoStep are absent, the UE performs contention based random access.

cfra-TwoStep Parameters for contention free 2-step random access type to a given target cell. Network ensures that cfra and cfra-TwoStep are not configured at the same time. If this field and cfra are absent, the UE performs contention based random access.

ra-OccasionType Indicates the second PRACH occasions for CFRA to be used by a SBFD aware UE. If absent, indicates the first PRACH occasions to be used.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access type procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |
| Occasions | The field is optionally present, Need S, if the field occasions is present, otherwise it is absent. |
| 2StepCFRA | The field is optionally present for the case of 2-step RA type contention free random access, Need S, otherwise it is absent. |
| 4StepCFRArep | For non-(e)RedCap UEs, the field is optionally present, Need S, if resources is set to ssb and there is one FeatureCombinationPreambles entry indicating only msg1-Repetitions which is associated with the same Msg1 repetition number. For RedCap UEs or if RedCap is considered to be applicable for this Random Access procedure for eRedCap UEs, the field is optionally present, Need S, if resources is set to ssb and there is one FeatureCombinationPreambles entry indicating only redCap and msg1-Repetitions which is associated with the same Msg1 repetition number. For eRedCap UEs, if eRedCap is considered to be applicable for this Random Access procedure, the field is optional present, Need S, if resource is set to ssb and there is one FeatureCombinationPreambles entry indicating only eRedCap and msg1-Repetitions which is associated with the same Msg1 repetition number. Otherwise, it is absent. |

#### –	RACH-ConfigGeneric

The IE RACH-ConfigGeneric is used to specify the random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigGeneric information element

-- ASN1START

-- TAG-RACH-CONFIGGENERIC-START

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}              OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                 OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                 OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                       OPTIONAL,   -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                              OPTIONAL    -- Need R

]],

[[

ra-ResponseWindow-v1700                     ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL    -- Need R

]],

[[

sbfd-RACH-SingleConfig-preambleReceivedTargetPower-r19     INTEGER (-202..-60)                              OPTIONAL    -- Need R

]]

}

-- TAG-RACH-CONFIGGENERIC-STOP

-- ASN1STOP

RACH-ConfigGeneric field descriptions

msg1-FDM The number of PRACH transmission occasions FDMed in one time instance. (see TS 38.211 [16], clause 6.3.3.2).

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. This field is used to configure the offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 for the first PRACH occasions if sbfd-RACH-SingleConfig is configured and for both the first PRACH occasions and second PRACH occasions if sbfd-RACH-DualConfig is configured (see TS 38.213 [13], clause 8), or, the lowest PRB in PRBs that are both in the active UL BWP and in the UL sub-band for the second PRACH occasions if sbfd-RACH-SingleConfig is configured (see TS 38.213 [13], clause 8). The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2).

powerRampingStep Power ramping steps for PRACH (see TS 38.321 [3],5.1.3). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

prach-ConfigurationFrameOffset-IAB Frame offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. (see TS 38.211 [16], clause 6.3.3.2).

prach-ConfigurationIndex PRACH configuration index. For prach-ConfigurationIndex configured under beamFailureRecoveryConfig, the prach-ConfigurationIndex can only correspond to the short preamble format, (see TS 38.211 [16], clause 6.3.3.2). If the field prach-ConfigurationIndex-v1610 is present, the UE shall ignore the value provided in prach-ConfigurationIndex (without suffix).

prach-ConfigurationPeriodScaling-IAB Scaling factor to extend the periodicity of the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. Value scf1 corresponds to scaling factor of 1 and so on. (see TS 38.211 [16], clause 6.3.3.2).

prach-ConfigurationSOffset-IAB Subframe/Slot offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex and is used only by the IAB-MT. (see TS 38.211 [16], clause 6.3.3.2).

preambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.4, TS 38.321 [3], clauses 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5). The UE shall ignore this field in case rach-ConfigGeneric is included within an EarlyUL-SyncConfig IE.

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. The UE shall ignore this field in case rach-ConfigGeneric is included within an EarlyUL-SyncConfig IE.

sbfd-RACH-SingleConfig-preambleReceivedTargetPower Configures preambleReceivedTargetPower for PRACH transmission in the second PRACH occasions or for Msg3 PUSCH transmission in SBFD symbols if sbfd-RACH-SingleConfig is configured, see clause 5 in TS 38.211 [16] and clause 8 in TS 38.213 [13].

zeroCorrelationZoneConfig N-CS configuration, see Table 6.3.3.1-5 in TS 38.211 [16].

#### –	RACH-ConfigGenericTwoStepRA

The IE RACH-ConfigGenericTwoStepRA is used to specify the 2-step random access type parameters.

RACH-ConfigGenericTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGGENERICTWOSTEPRA-START

RACH-ConfigGenericTwoStepRA-r16 ::=     SEQUENCE {

msgA-PRACH-ConfigurationIndex-r16       INTEGER (0..262)                                                OPTIONAL, -- Cond 2StepOnly

msgA-RO-FDM-r16                         ENUMERATED {one, two, four, eight}                              OPTIONAL, -- Cond 2StepOnly

msgA-RO-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)                    OPTIONAL, -- Cond 2StepOnly

msgA-ZeroCorrelationZoneConfig-r16      INTEGER (0..15)                                                 OPTIONAL, -- Cond 2StepOnly

msgA-PreamblePowerRampingStep-r16       ENUMERATED {dB0, dB2, dB4, dB6}                                 OPTIONAL, -- Cond 2StepOnlyNoCFRA

msgA-PreambleReceivedTargetPower-r16    INTEGER (-202..-60)                                             OPTIONAL, -- Cond 2StepOnlyNoCFRA

msgB-ResponseWindow-r16                 ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80, sl160, sl320}

OPTIONAL, -- Cond NoCFRA

preambleTransMax-r16                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}  OPTIONAL, -- Cond 2StepOnlyNoCFRA

...,

[[

msgB-ResponseWindow-v1700               ENUMERATED {sl240, sl640, sl960, sl1280, sl1920, sl2560}        OPTIONAL  -- Cond NoCFRA2

]]

}

-- TAG-RACH-CONFIGGENERICTWOSTEPRA-STOP

-- ASN1STOP

RACH-ConfigGenericTwoStepRA field descriptions

msgA-PreamblePowerRampingStep Power ramping steps for msgA PRACH. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the field is absent in other cases, UE shall use the value of powerRampingStep in RACH-ConfigGeneric in the configured BWP (see TS 38.321 [3], 5.1.3). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA. The field is absent if RACH-ConfigGenericTwoStepRA is included in CFRA-TwoStep in RACH-ConfigDedicated and then the UE uses the value of msgA-PreamblePowerRampingStep in RACH-ConfigGenericTwoStepRA configured for CBRA.

msgA-PreambleReceivedTargetPower The target power level at the network receiver side (see TS 38.213 [13], clause 7.1.1 and TS 38.321 [3], clause 5.1.1). Only multiples of 2 dBm may be chosen (e.g -202, -200, -198, …). If the field is absent, UE shall use the value of preambleReceivedTargetPower in RACH-ConfigGeneric in the configured BWP. This field may only be present if no 4-step type RA is configured in the BWP. The field is absent if RACH-ConfigGenericTwoStepRA is included in CFRA-TwoStep in RACH-ConfigDedicated and then the UE uses the value of msgA-PreambleReceivedTargetPower in RACH-ConfigGenericTwoStepRA configured for CBRA.

msgA-PRACH-ConfigurationIndex Cell-specific PRACH configuration index for 2-step RA type. If the field is absent in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE shall use the value of corresponding 4-step random access parameter in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the value is in the range of 256 to 262, the field prach-ConfigurationIndex-v1610 should be considered configured (see TS 38.211 [16], clause 6.3.3.2). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgA-RO-FDM The number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one time instance. If the field is absent in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), UE shall use value of msg1-FDM in RACH-ConfigGeneric in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the value of msg1-FDM in RACH-ConfigCommon in the same AdditionalRACH-Config (see TS 38.211 [16], clause 6.3.3.2). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgA-RO-FrequencyStart Offset of lowest PRACH transmissions occasion in frequency domain with respect to PRB 0. If the field is absent in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), UE shall use value of msg1-FrequencyStart in RACH-ConfigGeneric in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the value of msg1-FrequencyStart in RACH-ConfigCommon in the same AdditionalRACH-Config (see TS 38.211 [16], clauses 5.3.2 and 6.3.3.2). This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgA-ZeroCorrelationZoneConfig N-CS configuration for msgA preamble, see Table 6.3.3.1-5 in TS 38.211 [16]. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the field is absent in other cases, UE shall use value zeroCorrelationZoneConfig in RACH-ConfigGeneric in the configured BWP. This field may only be present if no 4-step type RA is configured in the BWP or in the case of separate ROs with 4-step type RA.

msgB-ResponseWindow MsgB monitoring window length in number of slots. The network configures a value lower than or equal to 40ms (see TS 38.321 [3], clause 5.1.1). The network does not configure msgB-ResponseWindow-r16 simultaneously with msgB-ResponseWindow-v1700, and if both fields are absent, the UE uses the value of msgB-ResponseWindow in RACH-ConfigGenericTwoStepRA configured for CBRA.

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5). If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE shall apply the corresponding value in RACH-ConfigCommon in the same AdditionalRACH-Config. If the field is absent in other cases, UE shall use the value of preambleTransMax in RACH-ConfigGeneric in the configured BWP. The field is absent if RACH-ConfigGenericTwoStepRA is included in CFRA-TwoStep in RACH-ConfigDedicated and then the UE uses the value of preambleTransMax in RACH-ConfigGenericTwoStepRA configured for CBRA.

| Conditional Presence | Explanation |
| --- | --- |
| 2StepOnly | The field is mandatory present in msgA-ConfigCommon field in BWP-UplinkCommon if rach-ConfigCommon field is absent in this BWP-UplinkCommon, otherwise the field is optionally present in msgA-ConfigCommon field in BWP-UplinkCommon, Need S. The field is mandatory present in msgA-ConfigCommon in AdditionalRACH-Config if rach-ConfigCommon field is absent in this AdditionalRACH-Config, otherwise the field is optionally present in msgA-ConfigCommon field in AdditionalRACH-Config, Need S. |
| 2StepOnlyNoCFRA | The field is mandatory present if RACH-ConfigGenericTwoStepRA is included in the RACH-ConfigCommonTwoStepRA and there are no 4-step random access configurations configured in the BWP (i.e only 2-step random access type configured in the BWP), otherwise (i.e. 4-step random access configuration also exists in the BWP) the field is optionally present, Need S. When RACH-ConfigGenericTwoStepRA is included in the RACH-ConfigDedicated, this field is absent. |
| NoCFRA | The field is mandatory present if msgB-ResponseWindow-r17 is absent and RACH-ConfigGenericTwoStepRA is not included in CFRA-TwoStep in RACH-ConfigDedicated, otherwise the field is absent, Need S. |
| NoCFRA2 | The field is mandatory present if msgB-ResponseWindow-r16 is absent and RACH-ConfigGenericTwoStepRA is not included in CFRA-TwoStep in RACH-ConfigDedicated, otherwise the field is absent, Need S. |

#### –	RACH-ConfigTwoTA

The IE RACH-ConfigTwoTA is used to specify random access parameters for each additional PCI configured for the serving cell.

RACH-ConfigTwoTA information element

-- ASN1START

-- TAG-RACH-CONFIGTWOTA-START

RACH-ConfigTwoTA-r18 ::=     SEQUENCE {

additionalPCI-andRACH-Index-r18  AdditionalPCIIndex-r17,

rach-ConfigGeneric-r18       RACH-ConfigGeneric,

ssb-perRACH-Occasion-r18     ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}   OPTIONAL,   -- Need M

prach-RootSequenceIndex-r18  CHOICE {

l839                         INTEGER (0..837),

l139                         INTEGER (0..137),

l571                         INTEGER (0..569),

l1151                        INTEGER (0..1149)

},

msg1-SubcarrierSpacing-r18   SubcarrierSpacing                                                            OPTIONAL,   -- Cond L139

...,

[[

twoTA-restrictedSetConfig-r18    ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}     OPTIONAL  -- Need R

]]

}

-- TAG-RACH-CONFIGTWOTA-STOP

-- ASN1STOP

RACH-ConfigTwoTA field descriptions

additionalPCI-andRACH-Index Indicates the associated PCI to this random access configuration.

msg1-SubcarrierSpacing Subcarrier spacing of PRACH when prach-RootSequenceIndex has value set to l139 (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]).

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839, L=139, L=571 or L=1151. For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

rach-ConfigGeneric RACH parameters for contention free random access occasions for CFRA.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

twoTA-restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets associated with additional PCI, see TS 38.211 [16], clause 6.3.3.1.

| Conditional Presence | Explanation |
| --- | --- |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, or if L=571 for FR2-2, otherwise the field is absent, Need S. |

#### –	RA-AdaptConfig

The IE RA-AdaptConfig is used to configure adaptive random access occasions.

RA-AdaptConfig information element

-- ASN1START

-- TAG-RA-ADAPTCONFIG-START

RA-AdaptConfig-r19 ::=                      SEQUENCE {

prach-ConfigurationIndex-r19                INTEGER (0..255),

msg1-FDM-r19                                ENUMERATED {one, two, four, eight},

msg1-FrequencyStart-r19                     INTEGER (0..maxNrofPhysicalResourceBlocks-1),

prach-SubsetMask-Index-Adaptation-r19       ENUMERATED {one, two, three, four}                               OPTIONAL,   -- Need R

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                             OPTIONAL,   -- Need R

validityDurationForRACH-Adaptation-r19                       ENUMERATED {n2, n4, n8, n16},

valueKforAssociationPatternPeriodsForPRACH-subsetMask-r19        ENUMERATED {n2, n4, n8, n16}                 OPTIONAL,   -- Need S

...

}

-- TAG-RA-ADAPTCONFIG-STOP

-- ASN1STOP

RA-AdaptConfig field descriptions

validityDurationForRACH-Adapt Indicates the validity time duration for availability indication of additional PRACH resources indicated by DCI 1_0 with P-RNTI, time unit is one SI modification period (see TS 38.213, subclause 8.1).

valueKforAssociationPatternPeriodsForPRACH The value of Kmask used for mapping of mask index to association periods per Kmask association pattern periods (See TS 38.213, subclause 8.1). Absence of this field indicates the value 1.

#### –	RA-Prioritization

The IE RA-Prioritization is used to configure prioritized random access.

RA-Prioritization information element

-- ASN1START

-- TAG-RA-PRIORITIZATION-START

RA-Prioritization ::=           SEQUENCE {

powerRampingStepHighPriority    ENUMERATED {dB0, dB2, dB4, dB6},

scalingFactorBI                 ENUMERATED {zero, dot25, dot5, dot75}                               OPTIONAL,   -- Need R

...

}

-- TAG-RA-PRIORITIZATION-STOP

-- ASN1STOP

RA-Prioritization field descriptions

powerRampingStepHighPriority Power ramping step applied for prioritized random access procedure.

scalingFactorBI Scaling factor for the backoff indicator (BI) for the prioritized random access procedure. (see TS 38.321 [3], clause 5.1.4). Value zero corresponds to 0, value dot25 corresponds to 0.25 and so on.

#### –	RA-PrioritizationForSlicing

The IE RA-PrioritizationForSlicing is used to configure prioritized random access for slicing.

RA-PrioritizationForSlicing information element

-- ASN1START

-- TAG-RA-PRIORITIZATIONFORSLICING-START

RA-PrioritizationForSlicing-r17 ::=    SEQUENCE {

ra-PrioritizationSliceInfoList-r17     RA-PrioritizationSliceInfoList-r17,

...

}

RA-PrioritizationSliceInfoList-r17 ::= SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF RA-PrioritizationSliceInfo-r17

RA-PrioritizationSliceInfo-r17 ::=     SEQUENCE {

nsag-ID-List-r17                       SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF NSAG-ID-r17,

ra-Prioritization-r17                  RA-Prioritization,

...

}

-- TAG-RA-PRIORITIZATIONFORSLICING-STOP

-- ASN1STOP

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling, multicast MRBs and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIOBEARERCONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...,

[[

mrb-ToAddModList-r17                    MRB-ToAddModList-r17                                    OPTIONAL,   -- Need N

mrb-ToReleaseList-r17                   MRB-ToReleaseList-r17                                   OPTIONAL,   -- Need N

srb4-ToAddMod-r17                       SRB-ToAddMod                                            OPTIONAL,   -- Need N

srb4-ToRelease-r17                      ENUMERATED{true}                                        OPTIONAL    -- Need N

]],

[[

srb5-ToAddMod-r18                       SRB-ToAddMod                                            OPTIONAL,   -- Need N

srb5-ToRelease-r18                      ENUMERATED{true}                                        OPTIONAL    -- Need N

]],

[[

srb6-ToAddMod-r19                       SRB-ToAddMod                                            OPTIONAL,   -- Need N

srb6-ToRelease-r19                      ENUMERATED{true}                                        OPTIONAL    -- Need N

]]

}

SRB-ToAddModList ::=                    SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=                        SEQUENCE {

srb-Identity                            SRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

discardOnPDCP                           ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

srb-Identity-v1700                      SRB-Identity-v1700                                      OPTIONAL    -- Need M

]],

[[

srb-Identity-v1800                      SRB-Identity-v1800                                      OPTIONAL,   -- Need M

n3c-BearerAssociated-r18                ENUMERATED{true}                                        OPTIONAL    -- Cond N3C MP

]],

[[

srb-Identity-v1900                      SRB-Identity-v1900                                      OPTIONAL    -- Need M

]]

}

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),

sdap-Config                             SDAP-Config

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

[[

daps-Config-r16                         ENUMERATED{true}                                        OPTIONAL    -- Cond DAPS

]],

[[

n3c-BearerAssociated-r18                ENUMERATED{true}                                        OPTIONAL    -- Cond N3C MP

]]

}

DRB-ToReleaseList ::=                   SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=                      SEQUENCE {

securityAlgorithmConfig                 SecurityAlgorithmConfig                                 OPTIONAL,   -- Cond RBTermChange1

keyToUse                                ENUMERATED{master, secondary}                           OPTIONAL,   -- Cond RBTermChange

...

}

MRB-ToAddModList-r17 ::=                SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-ToAddMod-r17

MRB-ToAddMod-r17 ::=                    SEQUENCE {

mbs-SessionId-r17                       TMGI-r17                                                OPTIONAL,   -- Cond MRBSetup

mrb-Identity-r17                        MRB-Identity-r17,

mrb-IdentityNew-r17                     MRB-Identity-r17                                        OPTIONAL,   -- Need N

reestablishPDCP-r17                     ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP-r17                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config-r17                         PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

MRB-ToReleaseList-r17 ::=               SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-Identity-r17

-- TAG-RADIOBEARERCONFIG-STOP

-- ASN1STOP

DRB-ToAddMod and MRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

daps-Config Indicates that the bearer is configured as DAPS bearer. The network does not include this field in an RRCReconfiguration message contained within a LTM-Config IE.

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts/indirect path of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer.

mbs-SessionId Indicates which multicast MBS session the bearer is associated with.

mrb-Identity Identification of the multicast MRB.

mrb-IdentityNew New identity of the multicast MRB when mrb-Identity needs to be changed, e.g. as a result of a handover.

n3c-BearerAssociated Indicates that the radio bearer is associated with the N3C indirect path.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer, if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE, or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE

recoverPDCP Indicates that PDCP should perform recovery according to TS 38.323 [5]. Network doesn't include this field if the bearer is configured as DAPS bearer, if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE, or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE.

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR or E-UTRA connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included after AS security has been activated, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig. The field is not included when configuring SRB1 before AS security is activated.

srb3-ToRelease Release SRB3. SRB3 release can only be done over SRB1 and only at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this IE RadioBearerConfig are using the master key or the secondary key for deriving ciphering and/or integrity protection keys. For MR-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig.

SRB-ToAddMod field descriptions

discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS 38.323 [5]. Network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE which is received within a MCG RRCReconfiguration message via SRB1, or if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE which includes the field ltm-NoSecurityChangeID or ltm-NoResetID.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to reconfiguration with sync, for SRB2 when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR, the network does not set this field to true. For LTE SRBs using NR PDCP, it could be for handover, RRC connection reestablishment or resume. Network doesn't include this field if any DAPS bearer is configured or if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE. For SRB3, network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE.

srb-Identity, srb-Identity-v1700, srb-Identity-v1800 Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only. Value 4 is applicable for SRB4 only. Value 5 is applicable for SRB5 only. Value 6 is applicable for SRB6 only. If srb-Identity-v1700, srb-Identity-v1800 or srb-Identity-v1900 is received for an SRB, the UE shall ignore srb-Identity (i.e. without suffix) for this SRB.

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| RBTermChange1 | The field is mandatory present in case of: - set up of signalling and data radio bearer, - change of termination point for the radio bearer between MN and SN, - handover from E-UTRA/EPC or E-UTRA/5GC to NR, - handover from NR or E-UTRA/EPC to E-UTRA/5GC if the UE supports NGEN-DC. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB/multicast MRB is being setup or corresponding DRB/multicast MRB is reconfigured with NR PDCP or corresponding SRB associated with two RLC entities is being setup or if the number of RLC bearers associated with the DRB/multicast MRB or SRB is changed. The field is optionally present, Need S, if the corresponding SRB associated with one RLC entity is being setup or corresponding SRB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| HO-Conn | The field is mandatory present - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured, - or in case of RRCSetup. Otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | If mrb-ToAddModList is not included, the field is mandatory present for UEs other than NCR-MT - in case of inter-system handover from E-UTRA/EPC to E-UTRA/5GC or NR, - or when the fullConfig is included in the RRCReconfiguration message and NE-DC/NR-DC is not configured. In case of RRCSetup, the field is absent; otherwise the field is optionally present, need N. |
| DAPS | The field is optionally present, need N, in case masterCellGroup includes ReconfigurationWithSync, SCell(s) and SCG are not configured, multi-DCI/single-DCI based multi-TRP are not configured in any DL BWP, supplementaryUplink is not configured, ethernetHeaderCompression is not configured for the DRB, conditionalReconfiguration is not configured, and NR sidelink and V2X sidelink are not configured. Otherwise the field is absent. |
| MRBSetup | The field is mandatory present if the corresponding multicast MRB is being setup; otherwise the field is optionally present, need M. |
| N3C MP | The field is optionally present if the corresponding radio bearer is being setup for MP with N3C indirect path, need R. It is absent otherwise. |

#### –	RadioLinkMonitoringConfig

The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1.

RadioLinkMonitoringConfig information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGCONFIG-START

RadioLinkMonitoringConfig ::=       SEQUENCE {

failureDetectionResourcesToAddModList   SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS

OPTIONAL, -- Need N

failureDetectionResourcesToReleaseList  SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF RadioLinkMonitoringRS-Id

OPTIONAL, -- Need N

beamFailureInstanceMaxCount             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                          OPTIONAL, -- Need R

beamFailureDetectionTimer               ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}  OPTIONAL, -- Need R

...,

[[

beamFailure-r17                         BeamFailureDetection-r17                                              OPTIONAL  -- Need R

]]

}

BeamFailureDetection-r17 ::=        SEQUENCE {

failureDetectionSet1-r17            BeamFailureDetectionSet-r17                                               OPTIONAL, -- Need R

failureDetectionSet2-r17            BeamFailureDetectionSet-r17                                               OPTIONAL, -- Need R

additionalPCI-r17                   AdditionalPCIIndex-r17                                                    OPTIONAL  -- Need R

}

RadioLinkMonitoringRS ::=           SEQUENCE {

radioLinkMonitoringRS-Id            RadioLinkMonitoringRS-Id,

purpose                             ENUMERATED {beamFailure, rlf, both},

detectionResource                   CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

...

}

BeamFailureDetectionSet-r17  ::=    SEQUENCE {

bfdResourcesToAddModList-r17        SEQUENCE (SIZE(1..maxNrofBFDResourcePerSet-r17)) OF BeamLinkMonitoringRS-r17

OPTIONAL, -- Need N

bfdResourcesToReleaseList-r17       SEQUENCE (SIZE(1..maxNrofBFDResourcePerSet-r17)) OF BeamLinkMonitoringRS-Id-r17

OPTIONAL, -- Need N

beamFailureInstanceMaxCount-r17     ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10}                              OPTIONAL, -- Need R

beamFailureDetectionTimer-r17       ENUMERATED {pbfd1, pbfd2, pbfd3, pbfd4, pbfd5, pbfd6, pbfd8, pbfd10}      OPTIONAL, -- Need R

...

}

BeamLinkMonitoringRS-r17 ::=        SEQUENCE {

beamLinkMonitoringRS-Id-r17         BeamLinkMonitoringRS-Id-r17,

detectionResource-r17               CHOICE {

ssb-Index                       SSB-Index,

csi-RS-Index                    NZP-CSI-RS-ResourceId

},

...

}

BeamLinkMonitoringRS-Id-r17 ::=     INTEGER (0..maxNrofFailureDetectionResources-1-r17)

-- TAG-RADIOLINKMONITORINGCONFIG-STOP

-- ASN1STOP

RadioLinkMonitoringConfig field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs in the failureDetectionSet2. If candidateBeamRS-List2 is configured in IE BeamFailureRecoveryRSConfig the field indicates the physical cell IDs (PCI) of the SSBs in the candidateBeamRS-List2.

beamFailureDetectionTimer Timer for beam failure detection (see TS 38.321 [3], clause 5.17). See also the BeamFailureRecoveryConfig IE. Value in number of "Qout,LR reporting periods of Beam Failure Detection" Reference Signal (see TS 38.213 [13], clause 6). Value pbfd1 corresponds to 1 Qout,LR reporting period of Beam Failure Detection Reference Signal, value pbfd2 corresponds to 2 Qout,LR reporting periods of Beam Failure Detection Reference Signal and so on.

beamFailureInstanceMaxCount This field determines after how many beam failure events the UE triggers beam failure recovery (see TS 38.321 [3], clause 5.17). Value n1 corresponds to 1 beam failure instance, value n2 corresponds to 2 beam failure instances and so on.

failureDetectionResourcesToAddModList A list of reference signals for detecting beam failure and/or cell level radio link failure (RLF). The limits of the reference signals that the network can configure are specified in TS 38.213 [13], table 5-1. The network configures at most two detectionResources per BWP for the purpose beamFailure or both. If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5. The network ensures that the UE has a suitable set of reference signals for performing cell-RLM. If failureDetectionSet1-r17 and failureDetectionSet2-r17 are present, the purpose of RadioLinkMonitoringRS in failureDetectionResourcesToAddModList only can be set to rlf.

failureDetectionSet1, failureDetectionSet2 Configures parameters for beamfailure detection towards beam failure detection resources configured in the set. If additional PCIs are configured using additionalPCI-ToAddModList for the serving cell, each RS in one set can be associated only with one PCI. Network always configures the failureDetectionSet1 and failureDetectionSet2 together. failureDetectionSetN is present if and only if candidateBeamRS-List2-r17 is configured. When a failureDetectionSetN is present, after the reconfiguration, the UE shall consider all the reference signals for this failure detection set as activated if at most maxBFD-RS-resourcesPerSetPerBWP-r17 reference signals are configured for each failure detection set, otherwise the UE shall consider all the reference signals in this failure detection set as deactivated. If bfdResourcesToAddModList-r17 in failureDetectionSetN is not present, the UE determines the RS(es) in each failureDetectionSetN as described in TS 38.213 [13], clause 6.

RadioLinkMonitoringRS field descriptions

detectionResource A reference signal that the UE shall use for radio link monitoring or beam failure detection (depending on the indicated purpose). Only periodic 1-port CSI-RS can be configured on SCell for beam failure detection purpose.

purpose Determines whether the UE shall monitor the associated reference signal for the purpose of cell- and/or beam failure detection. For SCell, network only configures the value to beamFailure.

#### –	RadioLinkMonitoringRS-Id

The IE RadioLinkMonitoringRS-Id is used to identify one RadioLinkMonitoringRS.

RadioLinkMonitoringRS-Id information element

-- ASN1START

-- TAG-RADIOLINKMONITORINGRS-ID-START

RadioLinkMonitoringRS-Id ::=            INTEGER (0..maxNrofFailureDetectionResources-1)

-- TAG-RADIOLINKMONITORINGRS-ID-STOP

-- ASN1STOP

#### –	RAN-AreaCode

The IE RAN-AreaCode is used to identify a RAN area within the scope of a tracking area.

RAN-AreaCode information element

-- ASN1START

-- TAG-RAN-AREACODE-START

RAN-AreaCode ::=                INTEGER (0..255)

-- TAG-RAN-AREACODE-STOP

-- ASN1STOP

#### –	RateMatchPattern

The IE RateMatchPattern is used to configure one rate matching pattern for PDSCH, see TS 38.214 [19], clause 5.1.4.1.

RateMatchPattern information element

-- ASN1START

-- TAG-RATEMATCHPATTERN-START

RateMatchPattern ::=                SEQUENCE {

rateMatchPatternId                  RateMatchPatternId,

patternType                         CHOICE {

bitmaps                             SEQUENCE {

resourceBlocks                      BIT STRING (SIZE (275)),

symbolsInResourceBlock              CHOICE {

oneSlot                             BIT STRING (SIZE (14)),

twoSlots                            BIT STRING (SIZE (28))

},

periodicityAndPattern               CHOICE {

n2                                  BIT STRING (SIZE (2)),

n4                                  BIT STRING (SIZE (4)),

n5                                  BIT STRING (SIZE (5)),

n8                                  BIT STRING (SIZE (8)),

n10                                 BIT STRING (SIZE (10)),

n20                                 BIT STRING (SIZE (20)),

n40                                 BIT STRING (SIZE (40))

}                                                                                           OPTIONAL,   -- Need S

...

},

controlResourceSet                  ControlResourceSetId

},

subcarrierSpacing                   SubcarrierSpacing                                               OPTIONAL,   -- Cond CellLevel

dummy                               ENUMERATED { dynamic, semiStatic },

...,

[[

controlResourceSet-r16              ControlResourceSetId-r16                                        OPTIONAL    -- Need R

]]

}

-- TAG-RATEMATCHPATTERN-STOP

-- ASN1STOP

RateMatchPattern field descriptions

bitmaps Indicates rate matching pattern by a pair of bitmaps resourceBlocks and symbolsInResourceBlock to define the rate match pattern within one or two slots, and a third bitmap periodicityAndPattern to define the repetition pattern with which the pattern defined by the above bitmap pair occurs.

controlResourceSet This ControlResourceSet is used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it. In frequency domain, the resource is determined by the frequency domain resource of the CORESET with the corresponding CORESET ID. Time domain resource is determined by the parameters of the associated search space of the CORESET. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

periodicityAndPattern A time domain repetition pattern at which the pattern defined by symbolsInResourceBlock and resourceBlocks recurs. This slot pattern repeats itself continuously. Absence of this field indicates the value n1 (see TS 38.214 [19], clause 5.1.4.1).

resourceBlocks A resource block level bitmap in the frequency domain. A bit in the bitmap set to 1 indicates that the UE shall apply rate matching in the corresponding resource block in accordance with the symbolsInResourceBlock bitmap. If used as cell-level rate matching pattern, the bitmap identifies "common resource blocks (CRB)". If used for MBS broadcast CFR, the bitmap identifies "physical resource blocks" inside the MBS broadcast CFR. If used as BWP-level rate matching pattern, the bitmap identifies "physical resource blocks" inside the BWP or MBS multicast CFR. The first/ leftmost bit corresponds to resource block 0, and so on (see TS 38.214 [19], clause 5.1.4.1).

subcarrierSpacing The SubcarrierSpacing for this resource pattern. If the field is absent, the UE applies the SCS of the associated BWP. The value kHz15 corresponds to µ=0, the value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency (see TS 38.214 [19], clause 5.1.4.1): FR1: 15, 30 or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

symbolsInResourceBlock A symbol level bitmap in time domain. It indicates with a bit set to true that the UE shall rate match around the corresponding symbol. This pattern recurs (in time domain) with the configured periodicityAndPattern (see TS 38.214 [19], clause 5.1.4.1). For oneSlot, if ECP is configured, the first 12 bits represent the symbols within the slot and the last two bits within the bitstring are ignored by the UE; Otherwise, the 14 bits represent the symbols within the slot. For twoSlots, if ECP is configured, the first 12 bits represent the symbols within the first slot and the next 12 bits represent the symbols in the second slot and the last four bits within the bit string are ignored by the UE; Otherwise, the first 14 bits represent the symbols within the first slot and the next 14 bits represent the symbols in the second slot. For the bits representing symbols in a slot, the most significant bit of the bit string represents the first symbol in the slot and the second most significant bit represents the second symbol in the slot and so on.

| Conditional Presence | Explanation |
| --- | --- |
| CellLevel | The field is mandatory present if the RateMatchPattern is defined on cell level. The field is absent when the RateMatchPattern is defined on BWP level or defined for MBS broadcast CFR. If the RateMatchPattern is defined on BWP level, the UE applies the SCS of the BWP and if RateMatchPattern is defined for MBS broadcast CFR, the UE applies the SCS of the initial BWP or RedCap-specific initial BWP (if configured) for (e)RedCap UEs. |

#### –	RateMatchPatternId

The IE RateMatchPatternId identifies one RateMatchPattern (see TS 38.214 [19], clause 5.1.4.1).

RateMatchPatternId information element

-- ASN1START

-- TAG-RATEMATCHPATTERNID-START

RateMatchPatternId ::=              INTEGER (0..maxNrofRateMatchPatterns-1)

-- TAG-RATEMATCHPATTERNID-STOP

-- ASN1STOP

#### –	RateMatchPatternLTE-CRS

The IE RateMatchPatternLTE-CRS is used to configure a pattern to rate match around LTE CRS. See TS 38.214 [19], clause 5.1.4.2.

RateMatchPatternLTE-CRS information element

-- ASN1START

-- TAG-RATEMATCHPATTERNLTE-CRS-START

RateMatchPatternLTE-CRS ::=         SEQUENCE {

carrierFreqDL                       INTEGER (0..16383),

carrierBandwidthDL                  ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1},

mbsfn-SubframeConfigList            EUTRA-MBSFN-SubframeConfigList                                          OPTIONAL,   -- Need M

nrofCRS-Ports                       ENUMERATED {n1, n2, n4},

v-Shift                             ENUMERATED {n0, n1, n2, n3, n4, n5}

}

LTE-CRS-PatternList-r16 ::=         SEQUENCE (SIZE (1..maxLTE-CRS-Patterns-r16)) OF RateMatchPatternLTE-CRS

-- TAG-RATEMATCHPATTERNLTE-CRS-STOP

-- ASN1STOP

RateMatchPatternLTE-CRS field descriptions

carrierBandwidthDL BW of the LTE carrier in number of PRBs (see TS 38.214 [19], clause 5.1.4.2).

carrierFreqDL Center of the LTE carrier (see TS 38.214 [19], clause 5.1.4.2).

mbsfn-SubframeConfigList LTE MBSFN subframe configuration (see TS 38.214 [19], clause 5.1.4.2).

nrofCRS-Ports Number of LTE CRS antenna port to rate-match around (see TS 38.214 [19], clause 5.1.4.2).

v-Shift Shifting value v-shift in LTE to rate match around LTE CRS (see TS 38.214 [19], clause 5.1.4.2).

#### –	ReferenceConfiguration

The IE ReferenceConfiguration is used provide a configuration that is common, within the same cell group, to all configured non-complete candidate configurations.

ReferenceConfiguration information element

-- ASN1START

-- TAG-REFERENCECONFIGURATION-START

ReferenceConfiguration-r18 ::= OCTET STRING (CONTAINING RRCReconfiguration)

-- TAG-REFERENCECONFIGURATION-STOP

-- ASN1STOP

#### –	ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

#### –	ReferenceTimeInfo

The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping, see TS 23.501 [32], clause 5.27.1.2.

ReferenceTimeInfo information element

-- ASN1START

-- TAG-REFERENCETIMEINFO-START

ReferenceTimeInfo-r16 ::= SEQUENCE {

time-r16                            ReferenceTime-r16,

uncertainty-r16                     INTEGER (0..32767)          OPTIONAL,   -- Need S

timeInfoType-r16                    ENUMERATED {localClock}     OPTIONAL,   -- Need S

referenceSFN-r16                    INTEGER (0..1023)           OPTIONAL    -- Cond RefTime

}

ReferenceTime-r16 ::=           SEQUENCE {

refDays-r16                         INTEGER (0..72999),

refSeconds-r16                      INTEGER (0..86399),

refMilliSeconds-r16                 INTEGER (0..999),

refTenNanoSeconds-r16               INTEGER (0..99999)

}

-- TAG-REFERENCETIMEINFO-STOP

-- ASN1STOP

ReferenceTimeInfo field descriptions

referenceSFN This field indicates the reference SFN corresponding to the reference time information. If referenceTimeInfo field is received in DLInformationTransfer message, this field indicates the SFN of PCell.

time This field indicates time reference with 10ns granularity. If included in DLInformationTransfer and if UE-side TA PDC is de-activated, the indicated time may not be referenced at the network, i.e., gNB may pre-compensate for RF propagation delay. If included in DLInformationTransfer and if UE is requested to transmit UE Rx-Tx time difference measurement, the indicated time may not be referenced at the network, i.e., gNB may pre-compensate for RF propagation delay. Otherwise, the indicated time is referenced at the network, i.e., without compensating for RF propagation delay. In an NTN cell, the indicated time is referenced at the uplink time synchronization reference point (RP), i.e., UE should take into account the propagation delay between UE and RP when determining the UTC time at the UE. The indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If the referenceTimeInfo field is received in DLInformationTransfer message, the time field indicates the time at the ending boundary of the system frame indicated by referenceSFN. The UE considers this frame (indicated by referenceSFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future). If the referenceTimeInfo field is received in SIB9, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SIB9 is transmitted. If referenceTimeInfo field is received in SIB9, this field is excluded when determining changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of valueTag in SIB1. NOTE: The estimated time in an NTN-cell may be less accurate than the estimated time in a TN-cell.

timeInfoType If timeInfoType is not included, the time indicates the GPS time and the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the origin of the time is unspecified.

uncertainty This field indicates the uncertainty of the reference time information provided by the time field. The uncertainty is 25ns multiplied by this field. If this field is absent, the uncertainty is unspecified.

| Conditional Presence | Explanation |
| --- | --- |
| RefTime | The field is mandatory present if referenceTimeInfo is included in DLInformationTransfer message; otherwise the field is absent. |

#### –	RejectWaitTime

The IE RejectWaitTime is used to provide the value in seconds for timer T302.

RejectWaitTime information element

-- ASN1START

-- TAG-REJECTWAITTIME-START

RejectWaitTime ::=                  INTEGER (1..16)

-- TAG-REJECTWAITTIME-STOP

-- ASN1STOP

#### –	RepetitionSchemeConfig

The IE RepetitionSchemeConfig is used to configure the UE with repetition schemes as specified in TS 38.214 [19] clause 5.1.

RepetitionSchemeConfig information element

-- ASN1START

-- TAG-REPETITIONSCHEMECONFIG-START

RepetitionSchemeConfig-r16 ::= CHOICE {

fdm-TDM-r16                        SetupRelease { FDM-TDM-r16 },

slotBased-r16                      SetupRelease { SlotBased-r16 }

}

RepetitionSchemeConfig-v1630 ::=   SEQUENCE {

slotBased-v1630                    SetupRelease { SlotBased-v1630 }

}

FDM-TDM-r16 ::=                SEQUENCE {

repetitionScheme-r16           ENUMERATED {fdmSchemeA, fdmSchemeB,tdmSchemeA },

startingSymbolOffsetK-r16      INTEGER (0..7)                                    OPTIONAL  -- Need R

}

SlotBased-r16 ::=              SEQUENCE {

tciMapping-r16                 ENUMERATED {cyclicMapping, sequentialMapping},

sequenceOffsetForRV-r16        INTEGER (1..3)

}

SlotBased-v1630 ::=            SEQUENCE {

tciMapping-r16                 ENUMERATED {cyclicMapping, sequentialMapping},

sequenceOffsetForRV-r16        INTEGER (0)

}

-- TAG-REPETITIONSCHEMECONFIG-STOP

-- ASN1STOP

RepetitionSchemeConfig field descriptions

fdm-TDM Configures UE with a repetition scheme among fdmSchemeA, fdmSchemeB and tdmSchemeA as specified in clause 5.1 of TS 38.214 [19]. The network does not set this field to release. Upon reception of this field in RepetitionSchemeConfig-r16, the UE shall release slotBased if previously configured in the same instance of RepetitionSchemeConfig-r16.

sequenceOffsetForRV For slot-based repetition scheme, selected RV sequence is applied to transmission occasions associated to the first TCI state. The RV sequence associated to the second TCI state is determined by a RV offset from that selected RV sequence.

slotBased Configures UE with slot-based repetition scheme. Network always configures this field when the parameter repetitionNumber is present in IE PDSCH-TimeDomainResourceAllocationList. The network does not set this field to release. Upon reception of this field in RepetitionSchemeConfig-r16, the UE shall release fdm-TDM if previously configured in the same instance of RepetitionSchemeConfig-r16.

startingSymbolOffsetK The starting symbol of the second transmission occasion has K symbol offset relative to the last symbol of the first transmission occasion. When UE is configured with tdmSchemeA, the parameter startingSymbolOffsetK is present, otherwise absent.

tciMapping Enables TCI state mapping method to PDSCH transmission occasions.

#### –	ReportConfigId

The IE ReportConfigId is used to identify a measurement reporting configuration.

ReportConfigId information element

-- ASN1START

-- TAG-REPORTCONFIGID-START

ReportConfigId ::=                          INTEGER (1..maxReportConfigId)

-- TAG-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	ReportConfigInterRAT

The IE ReportConfigInterRAT specifies criteria for triggering of an inter-RAT measurement reporting event, or an L2 U2N relay measurement reporting event. The inter-RAT measurement reporting events for E-UTRA and UTRA-FDD are labelled BN with N equal to 1, 2 and so on. The measurement reporting events for L2 U2N relay UE are labelled YN with N equal to 1, 2 and so on, and Z1.

Event B1:	Neighbour becomes better than absolute threshold;

Event B2:	PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2;

Event Y1: PCell becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

Event Y2: Candidate L2 U2N Relay UE becomes better than absolute threshold;

Event Z1: Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND candidate L2 U2N Relay UE becomes better than another absolute threshold2;

ReportConfigInterRAT information element

-- ASN1START

-- TAG-REPORTCONFIGINTERRAT-START

ReportConfigInterRAT ::=                    SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfigInterRAT,

eventTriggered                              EventTriggerConfigInterRAT,

reportCGI                                   ReportCGI-EUTRA,

...,

reportSFTD                                  ReportSFTD-EUTRA

}

}

ReportCGI-EUTRA ::=                         SEQUENCE {

cellForWhichToReportCGI         EUTRA-PhysCellId,

...,

[[

useAutonomousGaps-r16           ENUMERATED {setup}                OPTIONAL     -- Need R

]]

}

ReportSFTD-EUTRA ::=                     SEQUENCE {

reportSFTD-Meas                            BOOLEAN,

reportRSRP                                 BOOLEAN,

...

}

EventTriggerConfigInterRAT ::=              SEQUENCE {

eventId                                     CHOICE {

eventB1                                     SEQUENCE {

b1-ThresholdEUTRA                           MeasTriggerQuantityEUTRA,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

...

},

eventB2                                     SEQUENCE {

b2-Threshold1                               MeasTriggerQuantity,

b2-Threshold2EUTRA                          MeasTriggerQuantityEUTRA,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

...

},

...,

[[

eventB1-UTRA-FDD-r16                         SEQUENCE {

b1-ThresholdUTRA-FDD-r16                    MeasTriggerQuantityUTRA-FDD-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger,

...

},

eventB2-UTRA-FDD-r16                         SEQUENCE {

b2-Threshold1-r16                           MeasTriggerQuantity,

b2-Threshold2UTRA-FDD-r16                   MeasTriggerQuantityUTRA-FDD-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger,

...

}

]],

[[

eventY1-Relay-r17                            SEQUENCE {

y1-Threshold1-r17                            MeasTriggerQuantity,

y1-Threshold2-Relay-r17                      SL-MeasTriggerQuantity-r16,

reportOnLeave-r17                            BOOLEAN,

hysteresis-r17                               Hysteresis,

timeToTrigger-r17                            TimeToTrigger,

...

},

eventY2-Relay-r17                            SEQUENCE {

y2-Threshold-Relay-r17                       SL-MeasTriggerQuantity-r16,

reportOnLeave-r17                            BOOLEAN,

hysteresis-r17                               Hysteresis,

timeToTrigger-r17                            TimeToTrigger,

...

}

]],

[[

eventZ1-Relay-r18                            SEQUENCE {

z1-Threshold1-Relay-r18                      SEQUENCE {

sl-RSRP-r18                                  SL-MeasTriggerQuantity-r16,

sd-RSRP-r18                                  SL-MeasTriggerQuantity-r16                OPTIONAL    -- Need S

},

z1-Threshold2-Relay-r18                      SL-MeasTriggerQuantity-r16,

reportOnLeave-r18                            BOOLEAN,

hysteresis-r18                               Hysteresis,

timeToTrigger-r18                            TimeToTrigger,

...

}

]]

},

rsType                              NR-RS-Type,

reportInterval                      ReportInterval,

reportAmount                        ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity                      MeasReportQuantity,

maxReportCells                      INTEGER (1..maxCellReport),

...,

[[

reportQuantityUTRA-FDD-r16          MeasReportQuantityUTRA-FDD-r16         OPTIONAL   -- Need R

]],

[[

includeCommonLocationInfo-r16       ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                  SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16              SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

reportQuantityRelay-r17             SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]],

[[

cellIndividualOffsetList-r18        SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF CellIndividualOffsetList-EUTRA-r18 OPTIONAL -- Need R

]]

}

PeriodicalReportConfigInterRAT ::=              SEQUENCE {

reportInterval                                  ReportInterval,

reportAmount                                    ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity                                  MeasReportQuantity,

maxReportCells                                  INTEGER (1..maxCellReport),

...,

[[

reportQuantityUTRA-FDD-r16                      MeasReportQuantityUTRA-FDD-r16         OPTIONAL   -- Need R

]],

[[

includeCommonLocationInfo-r16       ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                  SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16              SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

reportQuantityRelay-r17             SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]]

}

MeasTriggerQuantityUTRA-FDD-r16 ::=          CHOICE{

utra-FDD-RSCP-r16                            INTEGER (-5..91),

utra-FDD-EcN0-r16                            INTEGER (0..49)

}

MeasReportQuantityUTRA-FDD-r16 ::=        SEQUENCE {

cpich-RSCP                                BOOLEAN,

cpich-EcN0                                BOOLEAN

}

CellIndividualOffsetList-EUTRA-r18 ::=   SEQUENCE {

physCellId-r18                            EUTRA-PhysCellId,

cellIndividualOffset-r18                  EUTRA-Q-OffsetRange,

carrierFreq-r18                           ARFCN-ValueEUTRA            OPTIONAL    -- Need R

}

-- TAG-REPORTCONFIGINTERRAT-STOP

-- ASN1STOP

ReportConfigInterRAT field descriptions

reportType Type of the configured measurement report. In (NG)EN-DC, and NR-DC, network does not configure report of type ReportCGI-EUTRA for SCG.

ReportCGI-EUTRA field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the E-UTRAN neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfigInterRAT field descriptions

b2-Threshold1 NR threshold to be used in inter RAT measurement report triggering condition for event B2.

bN-ThresholdEUTRA E-UTRA threshold value associated with the selected trigger quantity (RSRP, RSRQ, SINR) to be used in inter RAT measurement report triggering condition for event number bN. In the same eventB2, the network configures the same CHOICE name (rsrp, rsrq or sinr) for the MeasTriggerQuantity of the b2-Threshold1 and for the MeasTriggerQuantityEUTRA of the b2-Threshold2EUTRA.

eventId Choice of inter RAT event triggered reporting criteria.

maxReportCells Max number of non-serving cells/candidate L2 U2N Relay UEs to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList or for a L2 U2N Relay UE in relaysTriggeredList, as specified in 5.5.4.1.

reportQuantity, reportQuantityUTRA-FDD The cell measurement quantities to be included in the measurement report. If the field eventB1-UTRA-FDD or eventB2-UTRA-FDD is present, the UE shall ignore the value(s) provided in reportQuantity.

reportQuantityRelay The L2 U2N Relay UE measurement quantity to be included in measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

bN-ThresholdUTRA-FDD UTRA-FDD threshold value associated with the selected trigger quantity (RSCP, EcN0) to be used in inter RAT measurement report triggering condition for event number bN. utra-FDD-RSCP corresponds to CPICH_RSCP in TS 25.133 [46] for FDD. utra-FDD-EcN0 corresponds to CPICH_Ec/No in TS 25.133 [46] for FDD. For utra-FDD-RSCP: The actual value is field value – 115 dBm. For utra-FDD-EcN0: The actual value is (field value – 49)/2 dB.

y1-Threshold1 NR threshold to be used in measurement report triggering condition for event Y1.

y1-Threshold2-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. RSRP) to be used in measurement report triggering condition for event Y1.

y2-Threshold-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. RSRP) to be used in measurement report triggering condition for event Y2.

z1-Threshold1-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. SL-RSRP and/or SD-RSRP) to be used in measurement report triggering condition for serving Relay UE in event Z1. If the field sd-RSRP is not included, the UE considers it to be equal to sl-RSRP.

z1-Threshold2-Relay L2 U2N Relay threshold value associated with the selected trigger quantity (i.e. SD-RSRP) to be used in measurement report triggering condition for candidate Relay UE in event Z1.

PeriodicalReportConfigInterRAT field descriptions

maxReportCells Max number of non-serving cells/candidate L2 U2N Relay UEs to include in the measurement report.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantity, reportQuantityUTRA-FDD The cell measurement quantities to be included in the measurement report. If the field reportQuantityUTRA-FDD is present, the UE shall ignore the value(s) provided in reportQuantity.

CellIndividualOffsetList-EUTRA field descriptions

carrierFreq Indicates the EUTRA frequency for which cellIndividualOffset is applicable. If the field is not configured, the EUTRA frequency indicated by carrierFreq within the MeasObjectEUTRA of the measID associated with this ReportConfigInterRAT applies.

cellIndividualOffset Cell individual offsets applicable to a specific measurement event. If this field is present, the UE, for the same cell, shall ignore the cell individual offset configured within the MeasObjectEUTRA of the measID associated with this ReportConfigInterRAT.

physCellId Physical cell identity of a E-UTRAN cell in the cell list.

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO, CPA or CPC event or of an L2 U2N relay measurement reporting event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO, CPA or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell;

Event D1:	Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 becomes shorter than configured threshold distanceThreshFromReference2;

Event D2:	Distance between UE and the serving cell moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation and its corresponding satellite ephemeris and epoch time for the neighbor cell provided in the associated MeasObjectNR becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent A3: Conditional reconfiguration candidate or LTM candidate cell becomes amount of offset better than PCell/PSCell;

CondEvent A4: Conditional reconfiguration candidate becomes better than absolute threshold where condEventA4 can also be used for current PSCell (i.e., in case it is configured as candidate PSCell for CondEvent A4 evaluation) for CHO with candidate SCG(s) case;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate or LTM candidate cell becomes better than another absolute threshold2;

CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent D2: Distance between UE and the serving cell moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation and its corresponding satellite ephemeris and epoch time for the conditional reconfiguration candidate provided in the associated MeasObjectNR becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration;

CondEvent A3H1: Conditional reconfiguration candidate becomes offset better than SpCell and the Aerial UE altitude becomes higher than a threshold;

CondEvent A3H2: Conditional reconfiguration candidate becomes offset better than SpCell and the Aerial UE altitude becomes lower than a threshold;

CondEvent A5H1: SpCell becomes worse than threshold1 and conditional reconfiguration candidate becomes better than threshold2 and the Aerial UE altitude becomes higher than a threshold3;

CondEvent A5H2: SpCell becomes worse than threshold1 and conditional reconfiguration candidate becomes better than threshold2 and the Aerial UE altitude becomes lower than a threshold3.

Event X1:	Serving L2 U2N Relay UE becomes worse than absolute threshold1 AND NR Cell becomes better than another absolute threshold2;

Event X2:	Serving L2 U2N Relay UE becomes worse than absolute threshold;

For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI.

Event I1:	Interference becomes higher than absolute threshold;

The reporting events concerning Aerial UE altitude are labelled HN with N equal to 1 and 2. Additionally, the reporting events concerning Aerial UE altitude and the neighboring cell measurements simultaneously are labelled AMHN with M equal to 3, 4, 5 and N equal to 1, 2.

Event H1:	Aerial UE altitude becomes higher than a threshold;

Event H2:	Aerial UE altitude becomes lower than a threshold;

Event A3H1:	Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes higher than a threshold;

Event A3H2:	Neighbour becomes offset better than SpCell and the Aerial UE altitude becomes lower than a threshold;

Event A4H1:	Neighbour becomes better than threshold1 and the Aerial UE altitude becomes higher than a threshold2;

Event A4H2:	Neighbour becomes better than threshold1 and the Aerial UE altitude becomes lower than a threshold2;

Event A5H1:	SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes higher than a threshold3;

Event A5H2:	SpCell becomes worse than threshold1 and neighbour becomes better than threshold2 and the Aerial UE altitude becomes lower than a threshold3.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...,

condEventA4-r17                  SEQUENCE {

a4-Threshold-r17                 MeasTriggerQuantity,

hysteresis-r17                   Hysteresis,

timeToTrigger-r17                TimeToTrigger

},

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

},

condEventD2-r18                  SEQUENCE {

distanceThreshFromReference1-r18 INTEGER(0.. 65535),

distanceThreshFromReference2-r18 INTEGER(0.. 65535),

hysteresisLocation-r18           HysteresisLocation-r17,

timeToTrigger-r18                TimeToTrigger

},

condEventA3H1-r19                SEQUENCE {

a3-Offset-r19                    MeasTriggerQuantityOffset,

hysteresis-r19                   Hysteresis,

timeToTrigger-r19                TimeToTrigger,

h1-Threshold-r19                 Altitude-r18,

h1-Hysteresis-r19                HysteresisAltitude-r18

},

condEventA3H2-r19                SEQUENCE {

a3-Offset-r19                    MeasTriggerQuantityOffset,

hysteresis-r19                   Hysteresis,

timeToTrigger-r19                TimeToTrigger,

h2-Threshold-r19                 Altitude-r18,

h2-Hysteresis-r19                HysteresisAltitude-r18

},

condEventA5H1-r19                SEQUENCE {

a5-Threshold1-r19                MeasTriggerQuantity,

a5-Threshold2-r19                MeasTriggerQuantity,

hysteresis-r19                   Hysteresis,

timeToTrigger-r19                TimeToTrigger,

h1-Threshold-r19                 Altitude-r18,

h1-Hysteresis-r19                HysteresisAltitude-r18

},

condEventA5H2-r19                SEQUENCE {

a5-Threshold1-r19                MeasTriggerQuantity,

a5-Threshold2-r19                MeasTriggerQuantity,

hysteresis-r19                   Hysteresis,

timeToTrigger-r19                TimeToTrigger,

h2-Threshold-r19                 Altitude-r18,

h2-Hysteresis-r19                HysteresisAltitude-r18

}

},

rsType-r16                       NR-RS-Type,

...,

[[

nesEvent-r18                     ENUMERATED {true}                                OPTIONAL    -- Need R

]]

}

EventTriggerConfig ::=                      SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

...,

[[

eventX1-r17                                 SEQUENCE {

x1-Threshold1-Relay-r17                     SL-MeasTriggerQuantity-r16,

x1-Threshold2-r17                           MeasTriggerQuantity,

reportOnLeave-r17                           BOOLEAN,

hysteresis-r17                              Hysteresis,

timeToTrigger-r17                           TimeToTrigger,

useAllowedCellList-r17                      BOOLEAN

},

eventX2-r17                                 SEQUENCE {

x2-Threshold-Relay-r17                      SL-MeasTriggerQuantity-r16,

reportOnLeave-r17                           BOOLEAN,

hysteresis-r17                              Hysteresis,

timeToTrigger-r17                           TimeToTrigger

},

eventD1-r17                                 SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(1.. 65525),

distanceThreshFromReference2-r17            INTEGER(1.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

reportOnLeave-r17                           BOOLEAN,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                           TimeToTrigger

}

]],

[[

eventH1-r18                                SEQUENCE {

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

reportOnLeave-r18                           BOOLEAN,

timeToTrigger-r18                           TimeToTrigger,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventH2-r18                                SEQUENCE {

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

reportOnLeave-r18                           BOOLEAN,

timeToTrigger-r18                           TimeToTrigger,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA3H1-r18                              SEQUENCE {

a3-Offset-r18                               MeasTriggerQuantityOffset,

reportOnLeave-r18                           BOOLEAN,

a3-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA3H2-r18                              SEQUENCE {

a3-Offset-r18                               MeasTriggerQuantityOffset,

reportOnLeave-r18                           BOOLEAN,

a3-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA4H1-r18                              SEQUENCE {

a4-Threshold-r18                            MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a4-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA4H2-r18                              SEQUENCE {

a4-Threshold-r18                            MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a4-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA5H1-r18                              SEQUENCE {

a5-Threshold1-r18                           MeasTriggerQuantity,

a5-Threshold2-r18                           MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a5-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h1-Threshold-r18                            Altitude-r18,

h1-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventA5H2-r18                             SEQUENCE {

a5-Threshold1-r18                           MeasTriggerQuantity,

a5-Threshold2-r18                           MeasTriggerQuantity,

reportOnLeave-r18                           BOOLEAN,

a5-Hysteresis-r18                           Hysteresis,

timeToTrigger-r18                           TimeToTrigger,

useAllowedCellList-r18                      BOOLEAN,

h2-Threshold-r18                            Altitude-r18,

h2-Hysteresis-r18                           HysteresisAltitude-r18,

includeAltitudeUE-r18                       BOOLEAN,

simulMultiTriggerSingleMeasReport-r18       BOOLEAN

},

eventD2-r18                                 SEQUENCE {

distanceThreshFromReference1-r18            INTEGER(1.. 65535),

distanceThreshFromReference2-r18            INTEGER(1.. 65535),

reportOnLeave-r18                           BOOLEAN,

hysteresisLocation-r18                      HysteresisLocation-r17,

timeToTrigger-r18                           TimeToTrigger

}

]]

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

coarseLocationRequest-r17                   ENUMERATED {true}                                              OPTIONAL,   -- Need R

reportQuantityRelay-r17                     SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]],

[[

numberOfTriggeringCells-r18                 INTEGER (2..maxCellReport)                                     OPTIONAL,   -- Need R

cellIndividualOffsetList-r18                SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellIndividualOffsetList-r18 OPTIONAL, -- Need R

eventX1-SD-Threshold1-r18                   SL-MeasTriggerQuantity-r16                                     OPTIONAL,   -- Need S

eventX2-SD-Threshold-r18                    SL-MeasTriggerQuantity-r16                                     OPTIONAL,   -- Need S

reportOnBestCellChange-r18                  ENUMERATED {n1, n2}                                            OPTIONAL,    -- Need R

enteringLeavingReport-r18                   ENUMERATED {true}                                              OPTIONAL     -- Need R

]]

}

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useAllowedCellList                          BOOLEAN,

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL,   -- Need M

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL,   -- Need M

reportAddNeighMeas-r16                      ENUMERATED {setup}                                             OPTIONAL    -- Need R

]],

[[

ul-ExcessDelayConfig-r17                    SetupRelease { UL-ExcessDelayConfig-r17 }                      OPTIONAL,   -- Need M

coarseLocationRequest-r17                   ENUMERATED {true}                                              OPTIONAL,   -- Need R

reportQuantityRelay-r17                     SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]]

}

NR-RS-Type ::=                              ENUMERATED {ssb, csi-rs}

MeasReportQuantity ::=                      SEQUENCE {

rsrp                                        BOOLEAN,

rsrq                                        BOOLEAN,

sinr                                        BOOLEAN

}

MeasRSSI-ReportConfig-r16 ::=               SEQUENCE {

channelOccupancyThreshold-r16               RSSI-Range-r16         OPTIONAL   -- Need R

}

CLI-EventTriggerConfig-r16 ::=              SEQUENCE {

eventId-r16                                 CHOICE {

eventI1-r16                                 SEQUENCE {

i1-Threshold-r16                            MeasTriggerQuantityCLI-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger

},

...

},

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

CLI-PeriodicalReportConfig-r16 ::=          SEQUENCE {

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCLI-r16                       MeasReportQuantityCLI-r16,

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

RxTxPeriodical-r17  ::=                     SEQUENCE {

rxTxReportInterval-r17                      RxTxReportInterval-r17                             OPTIONAL,   -- Need R

reportAmount-r17                            ENUMERATED {r1, infinity, spare6, spare5, spare4, spare3, spare2, spare1},

...

}

RxTxReportInterval-r17 ::= ENUMERATED {ms80,ms120,ms160,ms240,ms320,ms480,ms640,ms1024,ms1280,ms2048,ms2560,ms5120,spare4,spare3,spare2,spare1}

MeasTriggerQuantityCLI-r16 ::=              CHOICE {

srs-RSRP-r16                                SRS-RSRP-Range-r16,

cli-RSSI-r16                                CLI-RSSI-Range-r16

}

MeasReportQuantityCLI-r16 ::=               ENUMERATED {srs-rsrp, cli-rssi}

ReportOnScellActivation-r18 ::=             SEQUENCE {

rsType-r18                                  NR-RS-Type,

reportQuantityRS-Indexes-r18                MeasReportQuantity,

maxNrofRS-IndexesToReport-r18               INTEGER (1..maxNrofIndexesToReport),

includeBeamMeasurements-r18                 BOOLEAN

}

CellIndividualOffsetList-r18 ::=    SEQUENCE {

physCellId-r18                      PhysCellId,

cellIndividualOffset-r18            Q-OffsetRangeList,

ssbFrequency-r18                    ARFCN-ValueNR              OPTIONAL    -- Need R

}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

a4-Threshold Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a4.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m.

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

nesEvent Indicates the event is a NES-specific CHO event and the event is only considered to be satisfied if indication from lower layers is received indicating the applicability of NES-specific CHO event and the related entry condition(s) is fulfilled. This field can only be configured for condEventA3, condEventA4 or condEventA5. This field cannot be configured for CPAC.

referenceLocation1, referenceLocation2 The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

ReportConfigNR field descriptions

reportType Type of the configured measurement report. In MR-DC, network does not configure report of type reportCGI using SRB3. The condTriggerConfig is used for CHO, CPA or CPC configuration.

ReportCGI field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the NR neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. In the same eventA5, eventA5H1, eventA5H2, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation.

coarseLocationRequest This field is used to request UE to report coarse location information.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for eventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for eventD2. Each step represents 50m.

eventId Choice of NR event triggered reporting criteria.

eventXN-SD-Threshold Indicates the SD-RSRP threshold value for the serving L2 U2N Relay UE in event XN (N equals 1 or 2). If this field is not included, the UE considers the SD-RSRP threshold value equals to the one indicated by x1-Threshold1-Relay/ x2-Threshold-Relay.

includeAltitudeUE This field is used to request UE to report altitude information.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

numberOfTriggeringCells Indicates the number of cells detected that are required to fulfill an event for a measurement report to be triggered. This field is applicable only for the events concerning neighbor cells, i.e. eventA3, eventA4, eventA5, eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, eventA5H2.

referenceLocation1, referenceLocation2 The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to neighbour cell.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnBestCellChange Indicates whether the UE shall only send measurement report if the measured best cell (when configured to n1) or two best cells (when configured to n2) have changed. In this release of the specification, this field is applicable only for the events concerning neighbor cells. This field can only be configured when the value of the field reportAmount is set to any other value than r1.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList or for a L2 U2N Relay UE in relaysTriggeredList, as specified in 5.5.4.1. Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met if configured in eventD1, eventD2, eventH1, eventH2 as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

simulMultiTriggerSingleMeasReport Indicates when multiple events with the same eventID satisfy the measurement report triggering condition(s), whether to consider only the event with the smallest value between the altitude of the UE and the configured altitude threshold.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useAllowedCellList Indicates whether only the cells included in the allow-list of the associated measObject are applicable as specified in 5.5.4.1.

useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObjectNR. If value FALSE is configured, the timer T312 is considered as disabled. Network configures value TRUE only if reportType is set to eventTriggered.

xN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event xN. If multiple thresholds are defined for event number xN, the thresholds are differentiated by M. x1-Threshold1 and x2-Threshold indicates the threshold value for the serving L2 U2N Relay UE, x1-Threshold2 indicates the threshold value for the NR Cells.

CLI-EventTriggerConfig field descriptions

i1-Threshold Threshold value associated to the selected trigger quantity (e.g. SRS-RSRP, CLI-RSSI) to be used in CLI measurement report triggering condition for event i1.

eventId Choice of CLI event triggered reporting criteria.

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CLI measurement resource in srsTriggeredList or rssiTriggeredList, as specified in 5.5.4.1.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

CLI-PeriodicalReportConfig field descriptions

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportQuantityCLI The CLI measurement quantities to be included in the measurement report.

PeriodicalReportConfig field descriptions

coarseLocationRequest This field is used to request UE to report coarse location information.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

ul-DelayValueConfig Indicates that the UE shall perform the actual UL PDCP Packet Average Delay measurement per DRB as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Packet Average Delay per DRB measurement as specified in TS 38.314 [53].

ul-ExcessDelayConfig Indicates that the UE shall perform the actual UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53].

useAllowedCellList Indicates whether only the cells included in the allow-list of the associated measObject are applicable as specified in 5.5.4.1.

ReportSFTD-NR field descriptions

cellForWhichToReportSFTD Indicates the target NR neighbour cells for SFTD measurement between PCell and NR neighbour cells.

drx-SFTD-NeighMeas Indicates that the UE shall use available idle periods (i.e. DRX off periods) for the SFTD measurement in NR standalone. The network only includes drx-SFTD-NeighMeas field when reprtSFTD-NeighMeas is set to true.

reportSFTD-Meas Indicates whether UE is required to perform SFTD measurement between PCell and NR PSCell in NR-DC.

reportSFTD-NeighMeas Indicates whether UE is required to perform SFTD measurement between PCell and NR neighbour cells in NR standalone. The network does not include this field if reportSFTD-Meas is set to true.

reportRSRP Indicates whether UE is required to include RSRP result of NR PSCell or NR neighbour cells in SFTD measurement result, derived based on SSB. If it is set to true, the network should ensure that ssb-ConfigMobility is included in the measurement object for NR PSCell or NR neighbour cells.

RxTxPeriodical field descriptions

reportAmount This field indicates the number of UE Rx-Tx time difference measurement reports. If configured to r1, the network does not configure rxTxReportInterval and only one measurement is reported. If configured to infinity, UE periodically reports measurements according to the periodicity configured by rxTxReportInterval.

rxTxReportInterval This field indicates the measurement reporting periodicity of UE Rx-Tx time difference.

other field descriptions

MeasTriggerQuantity SINR is applicable only for CONNECTED mode events.

ReportOnScellActivation field descriptions

rsType Indicates which RS is used to provide the measurement result. Only value ssb can be set in this release.

reportQuantityRS-Indexes Indicates which measurement information per RS index is used to sort the reported measurement results and is included in the measurement report.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report.

includeBeamMeasurements Indicates whether to include the measurement result per RS index in the measurement report.

CellIndividualOffsetList field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific measurement event. If this field is present, the UE, for the same cell, shall ignore the cell individual offset configured within the MeasObjectNR of the measID associated with this ReportConfigNR.

physCellId Physical cell identity of a cell in the cell list.

ssbFrequency Indicates the NR frequency of SS for which cellIndividualOffset is applicable. If the field is not configured, the NR frequency of SS indicated by ssbFrequency indicated within the MeasObjectNR of the measID associated with this ReportConfigNR applies.

#### –	ReportConfigNR-SL

The IE ReportConfigNR-SL specifies criteria for triggering of a CBR measurement reporting event for NR sidelink communication/discovery. Measurement reporting events are based on CBR measurement results on the corresponding transmission resource pools. These events are labelled CN with N equal to 1 and 2.

Event C1:	CBR of NR sidelink communication/discovery is above a threshold;

Event C2:	CBR of NR sidelink communication/discovery is below a threshold;

ReportConfigNR-SL information element

-- ASN1START

-- TAG-REPORTCONFIGNR-SL-START

ReportConfigNR-SL-r16 ::=            SEQUENCE {

reportType-r16                       CHOICE {

periodical-r16                       PeriodicalReportConfigNR-SL-r16,

eventTriggered-r16                   EventTriggerConfigNR-SL-r16

}

}

EventTriggerConfigNR-SL-r16::=       SEQUENCE {

eventId-r16                          CHOICE {

eventC1                              SEQUENCE {

c1-Threshold-r16                     SL-CBR-r16,

hysteresis-r16                       Hysteresis,

timeToTrigger-r16                    TimeToTrigger

},

eventC2-r16                  SEQUENCE {

c2-Threshold-r16             SL-CBR-r16,

hysteresis-r16               Hysteresis,

timeToTrigger-r16            TimeToTrigger

},

...

},

reportInterval-r16               ReportInterval,

reportAmount-r16                 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity-r16               MeasReportQuantity-r16,

...

}

PeriodicalReportConfigNR-SL-r16 ::=  SEQUENCE {

reportInterval-r16                   ReportInterval,

reportAmount-r16                     ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantity-r16                   MeasReportQuantity-r16,

...

}

MeasReportQuantity-r16 ::=           SEQUENCE {

cbr-r16                              BOOLEAN,

...

}

-- TAG-REPORTCONFIGNR-SL-STOP

-- ASN1STOP

ReportConfigNR-SL field descriptions

reportType Type of the configured CBR measurement report for NR sidelink communication/discovery.

EventTriggerConfigNR-SL field descriptions

cN-Threshold Threshold used for events C1 and C2 specified in clauses 5.5.4.11 and 5.5.4.12, respectively.

eventId Choice of NR event triggered reporting criteria.

reportAmoun Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportQuantity The sidelink measurement quantities to be included in the measurement report. In this release, this is set as the CBR measurement result.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

PeriodicalReportConfigNR-SL field descriptions

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportQuantity The sidelink measurement quantities to be included in the measurement report. In this release, this is set as the CBR measurement result.

#### –	ReportConfigToAddModList

The IE ReportConfigToAddModList concerns a list of reporting configurations to add or modify.

ReportConfigToAddModList information element

-- ASN1START

-- TAG-REPORTCONFIGTOADDMODLIST-START

ReportConfigToAddModList ::=        SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod

ReportConfigToAddMod ::=            SEQUENCE {

reportConfigId                      ReportConfigId,

reportConfig                        CHOICE {

reportConfigNR                      ReportConfigNR,

...,

reportConfigInterRAT                ReportConfigInterRAT,

reportConfigNR-SL-r16               ReportConfigNR-SL-r16

}

}

-- TAG-REPORTCONFIGTOADDMODLIST-STOP

-- ASN1STOP

#### –	ReportInterval

The IE ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1) when reportType is set to either eventTriggered, periodical, cli-EventTriggered or cli-Periodical. Value ms120 corresponds to 120 ms, value ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=                  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

ReportInterval-v1900 ::=            ENUMERATED {ms20, ms60, ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240,

ms20480, ms40960, min1,min6, min12, min30 }

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

#### –	ReselectionThreshold

The IE ReselectionThreshold is used to indicate an Rx level threshold for cell reselection. Actual value of threshold = field value * 2 [dB].

ReselectionThreshold information element

-- ASN1START

-- TAG-RESELECTIONTHRESHOLD-START

ReselectionThreshold ::=                INTEGER (0..31)

-- TAG-RESELECTIONTHRESHOLD-STOP

-- ASN1STOP

#### –	ReselectionThresholdQ

The IE ReselectionThresholdQ is used to indicate a quality level threshold for cell reselection. Actual value of threshold = field value [dB].

ReselectionThresholdQ information element

-- ASN1START

-- TAG-RESELECTIONTHRESHOLDQ-START

ReselectionThresholdQ ::=           INTEGER (0..31)

-- TAG-RESELECTIONTHRESHOLDQ-STOP

-- ASN1STOP

#### –	ResumeCause

The IE ResumeCause is used to indicate the resume cause in RRCResumeRequest, RRCResumeRequest1 and UEAssistanceInformation.

ResumeCause information element

-- ASN1START

-- TAG-RESUMECAUSE-START

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, mt-SDT-v1810, srs-PosConfigOrActivationReq-v1800, spare3, spare2, spare1 }

-- TAG-RESUMECAUSE-STOP

-- ASN1STOP

#### –	RLC-BearerConfig

The IE RLC-BearerConfig is used to configure an RLC entity, a corresponding logical channel in MAC and the linking to a PDCP entity (served radio bearer).

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v1610                            RLC-Config-v1610                                    OPTIONAL    -- Need R

]],

[[

rlc-Config-v1700                            RLC-Config-v1700                                    OPTIONAL,   -- Need R

logicalChannelIdentityExt-r17               LogicalChannelIdentityExt-r17                       OPTIONAL,   -- Cond LCH-SetupModMRB

multicastRLC-BearerConfig-r17               MulticastRLC-BearerConfig-r17                       OPTIONAL,   -- Cond LCH-SetupOnlyMRB

servedRadioBearerSRB4-r17                   SRB-Identity-v1700                                  OPTIONAL    -- Cond LCH-SetupOnlySRB4

]],

[[

servedRadioBearerSRB5-r18                   SRB-Identity-v1800                                  OPTIONAL    -- Cond LCH-SetupOnlySRB5

]],

[[

rlc-Config-v1900                            RLC-Config-v1900                                    OPTIONAL,   -- Need R

servedRadioBearerSRB6-r19                   SRB-Identity-v1900                                  OPTIONAL  -- Cond LCH-SetupOnlySRB6

]]

}

MulticastRLC-BearerConfig-r17 ::=           SEQUENCE {

servedMBS-RadioBearer-r17                   MRB-Identity-r17,

isPTM-Entity-r17                            ENUMERATED {true}                                   OPTIONAL    -- Need S

}

LogicalChannelIdentityExt-r17 ::=           INTEGER (320..65855)

-- TAG-RLC-BEARERCONFIG-STOP

-- ASN1STOP

RLC-BearerConfig field descriptions

isPTM-Entity If configured, indicates that the RLC entity is used for PTM reception. When the field is absent the RLC entity is used for PTP transmission/reception.

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer.

logicalChannelIdentityExt Extended logical channel ID used commonly for the MAC logical channel and for the RLC bearer for PTM reception. If this field is configured, the UE shall ignore logicalChannelIdentity.

reestablishRLC Indicates that RLC should be re-established. Network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2, multicast MRBs and DRBs, unless full configuration is used, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment, the network does not set this field to true. The network does not include this field if servedRadioBearer is set to drb-Identity and the RLC-BearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE. For DRBs, network doesn't include this field if the RLC-BearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE. Network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE which is received within a MCG RRCReconfiguration message via SRB1.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters. RLC mode reconfiguration can only be performed by DRB/multicast MRB release/addition or full configuration. The network may configure rlc-Config-v1610 only when rlc-Config (without suffix) is set to am.

servedMBS-RadioBearer Associates the RLC Bearer with a multicast MRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedMBS-RadioBearer.

servedRadioBearer, servedRadioBearerSRB4, servedRadioBearerSRB5, servedRadioBearerSRB6 Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling restrictions (moreThanOneRLC in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a DRB or a multicast MRB or SRB4 or SRB5. This field is optionally present, Need S, upon creation of a new logical channel for an SRB except SRB4 and SRB5. It is optionally present, Need M, otherwise. |
| LCH-SetupModMRB | This field is optionally present upon creation of a new logical channel for PTM reception for a multicast MRB. If this field is included upon creation of a new logical channel for PTM reception for a multicast MRB, it shall be present when modifying this logical channel. The field is absent for logical channels configured for an SRB and a DRB. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a DRB or an SRB (servedRadioBearer). It is absent, Need M otherwise. |
| LCH-SetupOnlyMRB | This field is mandatory present upon creation of a new logical channel for a multicast MRB and upon modification of MRB-Identity of the served MRB. It is absent, Need M otherwise. |
| LCH-SetupOnlySRB4 | This field is mandatory present upon creation of a new logical channel for SRB4 (servedRadioBearerSRB4). It is absent, Need M otherwise. |
| LCH-SetupOnlySRB5 | This field is mandatory present upon creation of a new logical channel for SRB5 (servedRadioBearerSRB5). It is absent, Need M otherwise. |
| LCH-SetupOnlySRB6 | This field is mandatory present upon creation of a new logical channel for SRB6 (servedRadioBearerSRB6). It is absent, Need M otherwise. |

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs, multicast MRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

UL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-PollRetransmit                    T-PollRetransmit,

pollPDU                             PollPDU,

pollByte                            PollByte,

maxRetxThreshold                    ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly,

t-StatusProhibit                    T-StatusProhibit

}

UL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL    -- Cond Reestab

}

DL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly

}

T-PollRetransmit ::=                ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610,

ms4-v1610, spare1}

PollPDU ::=                         ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=                        ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=                    ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=                ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=                ENUMERATED {size6, size12}

SN-FieldLengthAM ::=                ENUMERATED {size12, size18}

RLC-Config-v1610 ::=                SEQUENCE {

dl-AM-RLC-v1610                     DL-AM-RLC-v1610

}

RLC-Config-v1700 ::=                SEQUENCE {

dl-AM-RLC-v1700                     DL-AM-RLC-v1700,

dl-UM-RLC-v1700                     DL-UM-RLC-v1700

}

RLC-Config-v1900 ::=                SEQUENCE {

dl-AM-RLC-v1900                     DL-AM-RLC-v1900,

ul-AM-RLC-v1900                     UL-AM-RLC-v1900

}

DL-AM-RLC-v1610 ::=                 SEQUENCE {

t-StatusProhibit-v1610              T-StatusProhibit-v1610                               OPTIONAL,   -- Need R

...

}

DL-AM-RLC-v1700 ::=                 SEQUENCE {

t-ReassemblyExt-r17                 T-ReassemblyExt-r17                                  OPTIONAL    -- Need R

}

DL-AM-RLC-v1900 ::=                 SEQUENCE {

t-RxDiscard-r19                     T-RxDiscard-r19                                      OPTIONAL    -- Need R

}

UL-AM-RLC-v1900 ::=                 SEQUENCE {

stopReTxDiscardedSDU-r19            ENUMERATED {enabled}                                 OPTIONAL    -- Need R

}

DL-UM-RLC-v1700 ::=                 SEQUENCE {

t-ReassemblyExt-r17                 T-ReassemblyExt-r17                                  OPTIONAL    -- Need R

}

T-StatusProhibit-v1610 ::=          ENUMERATED { ms1, ms2, ms3, ms4, spare4, spare3, spare2, spare1}

T-ReassemblyExt-r17 ::=             ENUMERATED {ms210, ms220, ms340, ms350, ms550, ms1100, ms1650, ms2200}

T-RxDiscard-r19 ::=                 ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300,

ms400, ms500, ms750, ms1000, ms1500, ms2000, ms3000, spare13, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-RLC-CONFIG-STOP

-- ASN1STOP

RLC-Config field descriptions

maxRetxThreshold Parameter for RLC AM in TS 38.322 [4]. Value t1 corresponds to 1 retransmission, value t2 corresponds to 2 retransmissions and so on.

pollByte Parameter for RLC AM in TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, value kB50 corresponds to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.

pollPDU Parameter for RLC AM in TS 38.322 [4]. Value p4 corresponds to 4 PDUs, value p8 corresponds to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.

sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The value of sn-FieldLength of an RLC entity for the DRB/multicast MRB shall be changed only using reconfiguration with sync. The network configures only value size12 in SN-FieldLengthAM for SRB.

stopReTxDiscardedSDU Indicates whether the Tx side of the RLC entity should stop transmission and retransmission of the RLC SDUs or its segments when discard indication of the SDUs is received from the PDCP layer as specified in TS 38.323 [5].

t-PollRetransmit Timer for RLC AM in TS 38.322 [4], in milliseconds. Value ms5 means 5 ms, value ms10 means 10 ms and so on.

t-Reassembly, t-ReassemblyExt Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-ReassemblyExt-r17 is configured, the UE shall ignore t-Reassembly (without suffix).

t-RxDiscard Timer for the AMD RLC PDU(s) discard at the Rx side of the RLC entity, see TS 38.322 [4]. For the value of the IE T-RxDiscard, value ms10 means 10 milliseconds, value ms20 means 20 milliseconds, and so on. The value of the field should not be lower than that configured by the field t-Reassembly or t-ReassemblyExt.

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-StatusProhibit-v1610 is present, the UE shall ignore t-StatusProhibit (without suffix).

| Conditional Presence | Explanation |
| --- | --- |
| Reestab | The field is mandatory present at RLC bearer setup. It is optionally present, need M, at RLC re-establishment. Otherwise it is absent. Need M. |

#### –	RLF-TimersAndConstants

The IE RLF-TimersAndConstants is used to configure UE specific timers and constants.

RLF-TimersAndConstants information element

-- ASN1START

-- TAG-RLF-TIMERSANDCONSTANTS-START

RLF-TimersAndConstants ::=          SEQUENCE {

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}

]]

}

-- TAG-RLF-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

RLF-TimersAndConstants field descriptions

n3xy Constants are described in clause 7.3. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

t3xy Timers are described in clause 7.1. Value ms0 corresponds to 0 ms, value ms50 corresponds to 50 ms and so on.

#### –	RNTI-Value

The IE RNTI-Value represents a Radio Network Temporary Identity.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=                      INTEGER (0..65535)

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

#### –	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 156) dBm, except for the IE value 127, in which case the actual value is infinity.

RSRP-Range information element

-- ASN1START

-- TAG-RSRP-RANGE-START

RSRP-Range ::=                      INTEGER(0..127)

-- TAG-RSRP-RANGE-STOP

-- ASN1STOP

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. For measurements, integer value for RSRQ measurements is according to Table 10.1.11.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 87) / 2 dB.

RSRQ-Range information element

-- ASN1START

-- TAG-RSRQ-RANGE-START

RSRQ-Range ::=                      INTEGER(0..127)

-- TAG-RSRQ-RANGE-STOP

-- ASN1STOP

#### –	RSSI-Range

The IE RSSI-Range specifies the value range used in RSSI measurements and thresholds for NR operation with shared spectrum channel access. The integer value for RSSI measurements is according to Table 10.1.34.3-1 in TS 38.133 [14].

RSSI-Range information element

-- ASN1START

-- TAG-RSSI-RANGE-START

RSSI-Range-r16 ::=                  INTEGER(0..76)

-- TAG-RSSI-RANGE-STOP

-- ASN1STOP

#### –	RxTxTimeDiff

The IE RxTxTimeDiff contains the Rx-Tx time difference measurement at either the UE or the gNB.

RxTxTimeDiff information element

-- ASN1START

-- TAG-RXTXTIMEDIFF-START

RxTxTimeDiff-r17  ::= SEQUENCE {

result-k5-r17         INTEGER (0..61565)                 OPTIONAL, -- Need N

...

}

-- TAG-RXTXTIMEDIFF-STOP

-- ASN1STOP

RxTxTimeDiff field descriptions

result-k5 This field indicates the Rx-Tx time difference measurement, see TS 38.215 [9], clause 10.1.25.3.1 of TS 38.133 [14] for UE Rx-Tx time difference and clause 13.2.1 of TS 38.133 [14] for gNB Rx-Tx time difference.

#### –	SCellActivationRS-Config

The IE SCellActivationRS-Config is used to configure a Reference Signal for fast activation of the SCell where the IE is included (see TS 38.214 [19], clause 5.2.1.5.3. Usage of an SCellActivationRS-Config is indicated by including its scellActivationRS-Id in the Enhanced SCell activation MAC CE (see TS 38.321 [3] clause 6.1.3.55).

SCellActivationRS-Config information element

-- ASN1START

-- TAG-SCELLACTIVATIONRS-CONFIG-START

SCellActivationRS-Config-r17 ::= SEQUENCE {

scellActivationRS-Id-r17         SCellActivationRS-ConfigId-r17,

resourceSet-r17                  NZP-CSI-RS-ResourceSetId,

gapBetweenBursts-r17             INTEGER (2..31)                                                            OPTIONAL, -- Need R

qcl-Info-r17                     TCI-StateId,

...

}

-- TAG-SCELLACTIVATIONRS-CONFIG-STOP

-- ASN1STOP

SCellActivationRS-Config field descriptions

gapBetweenBursts When this field is present, there are two bursts and it indicates the gap between the two bursts in number of slots. When this field is absent, there is a single burst.

qcl-Info Reference to TCI-State for providing the QCL source and QCL type for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet indicated by resourceSet (see TS 38.214 [19], clause 5.1.6.1.1.1). TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink of this serving cell indicated by firstActiveDownlinkBWP-Id in the ServingCellConfig in which this IE is included.

resourceSet nzp-CSI-ResourceSetId of the NZP-CSI-RS-ResourceSet of this serving cell used as resource configuration for one or two bursts for SCell activation. This NZP-CSI-RS-ResourceSet consists of four NZP CSI-RS resources in two consecutive slots with two NZP CSI-RS resources in each slot (see TS 38.214 [19], clause 5.1.6.1.1.1). The CSI-RS associated with this NZP-CSI-RS-ResourceSet are located in the BWP addressed by firstActiveDownlinkBWP-Id.

#### –	SCellActivationRS-ConfigId

The IE SCellActivationRS-ConfigId is used to identify one SCellActivationRS-Config.

SCellActivationRS-ConfigId information element

-- ASN1START

-- TAG-SCELLACTIVATIONRS-CONFIGID-START

SCellActivationRS-ConfigId-r17 ::=        INTEGER (1.. maxNrofSCellActRS-r17)

-- TAG-SCELLACTIVATIONRS-CONFIGID-STOP

-- ASN1STOP

#### –	SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell. The value range is shared across the Cell Groups.

SCellIndex information element

-- ASN1START

-- TAG-SCELLINDEX-START

SCellIndex ::=                      INTEGER (1..31)

-- TAG-SCELLINDEX-STOP

-- ASN1STOP

#### –	SchedulingRequestConfig

The IE SchedulingRequestConfig is used to configure the parameters, for the dedicated scheduling request (SR) resources.

SchedulingRequestConfig information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTCONFIG-START

SchedulingRequestConfig ::=         SEQUENCE {

schedulingRequestToAddModList       SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod

OPTIONAL, -- Need N

schedulingRequestToReleaseList      SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestId

OPTIONAL  -- Need N

}

SchedulingRequestToAddMod ::=       SEQUENCE {

schedulingRequestId                 SchedulingRequestId,

sr-ProhibitTimer                    ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128}          OPTIONAL, -- Need S

sr-TransMax                         ENUMERATED { n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

SchedulingRequestConfig-v1700 ::=       SEQUENCE {

schedulingRequestToAddModListExt-v1700  SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddModExt-v1700

OPTIONAL  -- Need N

}

SchedulingRequestToAddModExt-v1700 ::=  SEQUENCE {

sr-ProhibitTimer-v1700                  ENUMERATED { ms192, ms256, ms320, ms384, ms448, ms512, ms576, ms640, ms1082, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

OPTIONAL  -- Need R

}

-- TAG-SCHEDULINGREQUESTCONFIG-STOP

-- ASN1STOP

SchedulingRequestConfig field descriptions

schedulingRequestToAddModList, schedulingRequestToAddModListExt List of Scheduling Request configurations to add or modify. If schedulingRequestToAddModListExt is configured, it contains the same number of entries, and in the same order, as schedulingRequestToAddModList.

schedulingRequestToReleaseList List of Scheduling Request configurations to release.

SchedulingRequestToAddMod field descriptions

schedulingRequestId Used to modify a SR configuration and to indicate, in LogicalChannelConfig, the SR configuration to which a logical channel is mapped and to indicate, in SchedulingRequestresourceConfig, the SR configuration for which a scheduling request resource is used.

sr-ProhibitTimer Timer for SR transmission on PUCCH in TS 38.321 [3]. Value is in ms. Value ms1 corresponds to 1ms, value ms2 corresponds to 2ms, and so on. If sr-ProhibitTimer-v1700 is configured, UE shall ignore sr-ProhibitTimer (without suffix). If both sr-ProhibitTimer (without suffix) and sr-ProhibitTimer-v1700 are absent, the UE applies the value 0.

sr-TransMax Maximum number of SR transmissions as described in TS 38.321 [3]. Value n4 corresponds to 4, value n8 corresponds to 8, and so on.

#### –	SchedulingRequestId

The IE SchedulingRequestId is used to identify a Scheduling Request instance in the MAC layer.

SchedulingRequestId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTID-START

SchedulingRequestId ::=             INTEGER (0..7)

-- TAG-SCHEDULINGREQUESTID-STOP

-- ASN1STOP

#### –	SchedulingRequestResourceConfig

The IE SchedulingRequestResourceConfig determines physical layer resources on PUCCH where the UE may send the dedicated scheduling request (D-SR) (see TS 38.213 [13], clause 9.2.4).

SchedulingRequestResourceConfig information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-START

SchedulingRequestResourceConfig ::=     SEQUENCE {

schedulingRequestResourceId             SchedulingRequestResourceId,

schedulingRequestID                     SchedulingRequestId,

periodicityAndOffset                    CHOICE {

sym2                                    NULL,

sym6or7                                 NULL,

sl1                                     NULL,                       -- Recurs in every slot

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639)

}                                                                                                       OPTIONAL,   -- Need M

resource                                PUCCH-ResourceId                                                OPTIONAL    -- Need M

}

SchedulingRequestResourceConfigExt-v1610 ::=   SEQUENCE {

phy-PriorityIndex-r16                       ENUMERATED {p0, p1}                                         OPTIONAL,   -- Need M

...

}

SchedulingRequestResourceConfigExt-v1700 ::=    SEQUENCE {

periodicityAndOffset-r17                        CHOICE {

sl1280                                          INTEGER (0..1279),

sl2560                                          INTEGER (0..2559),

sl5120                                          INTEGER (0..5119)

}                                                                                                       OPTIONAL   -- Need M

}

SchedulingRequestResourceConfigExt-v1900 ::=    SEQUENCE {

symbolType-r19                                  ENUMERATED {sbfd, non-sbfd}                             OPTIONAL   -- Need R

}

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-STOP

-- ASN1STOP

SchedulingRequestResourceConfig field descriptions

periodicityAndOffset SR periodicity and offset in number of symbols or slots (see TS 38.213 [13], clause 9.2.4) The following periodicities may be configured depending on the chosen subcarrier spacing: SCS = 15 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl SCS = 30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 20sl, 40sl, 80sl, 160sl SCS = 60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 16sl, 20sl, 40sl, 80sl, 160sl, 320sl SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 5sl, 8sl, 10sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl SCS = 480 kHz: 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl, 1280sl, 2560sl SCS = 960 kHz: 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, 640sl, 1280sl, 2560sl, 5120sl sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols. For periodicities 2sym, 7sym and sl1 the UE assumes an offset of 0 slots. If periodicityAndOffset-r17 is present, any previously configured periodicityAndOffset (without suffix) is released, and vice versa. UE indicates support of the following SR periodicities in additionalSR-Periodicities (see TS 38.306, clause 4.2.6): SCS = 30 kHz: 5sl SCS = 120 kHz: 5sl, 10sl

phy-PriorityIndex Indicates whether this scheduling request resource is high or low priority in PHY prioritization/multiplexing handling (see TS 38.213 [13], clause 9.2.4). Value p0 indicates low priority and value p1 indicates high priority.

resource ID of the PUCCH resource in which the UE shall send the scheduling request. The actual PUCCH-Resource is configured in PUCCH-Config of the same UL BWP and serving cell as this SchedulingRequestResourceConfig. The network configures a PUCCH-Resource of PUCCH-format0 or PUCCH-format1 (other formats not supported) (see TS 38.213 [13], clause 9.2.4)

schedulingRequestID The ID of the SchedulingRequestConfig that uses this scheduling request resource.

symbolType Configures the valid symbol type for PUCCH configured for SR when the transmissions are restricted to SBFD symbols only or non-SBFD symbols only for the UL BWP. The network does not configure this field if the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 6)

#### –	SchedulingRequestResourceId

The IE SchedulingRequestResourceId is used to identify scheduling request resources on PUCCH.

SchedulingRequestResourceId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCEID-START

SchedulingRequestResourceId ::=     INTEGER (1..maxNrofSR-Resources)

-- TAG-SCHEDULINGREQUESTRESOURCEID-STOP

-- ASN1STOP

#### –	ScramblingId

The IE ScramblingID is used for scrambling channels and reference signals.

ScramblingId information element

-- ASN1START

-- TAG-SCRAMBLINGID-START

ScramblingId ::=                    INTEGER(0..1023)

-- TAG-SCRAMBLINGID-STOP

-- ASN1STOP

#### –	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]],

[[

sbfd-Subband-Allocation-r19     SBFD-Subband-Allocation-r19                             OPTIONAL            -- Need R

]]

}

SBFD-Subband-Allocation-r19 ::=             SEQUENCE {

ul-subbandlocationAndBandwidth-r19          INTEGER (0..37949)                          OPTIONAL,           -- Need R

firstDL-subbandlocationAndBandwidth-r19     INTEGER (0..37949)                          OPTIONAL,           -- Need R

secondDL-subbandlocationAndBandwidth-r19    INTEGER (0..37949)                          OPTIONAL,           -- Need R

...

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2). For the case that 12PRB/20 PRB transmission bandwidth is used as specified in TS 38.101-1 [15], TS 38.211[16] and TS 38.213[13], the network shall configure the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively and the UE shall use 12PRB/20 PRB as the transmission bandwidth respectively.

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

txDirectCurrentLocation Indicates the downlink Tx Direct Current location for the carrier. A value in the range 0..3299 indicates the subcarrier index within the carrier. The values in the value range 3301..4095 are reserved and ignored by the UE. If this field is absent for downlink within ServingCellConfigCommon and ServingCellConfigCommonSIB, the UE assumes the default value of 3300 (i.e. "Outside the carrier"). (see TS 38.211 [16], clause 4.4.2). Network does not configure this field via ServingCellConfig or for uplink carriers.

subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

SBFD-Subband-Allocation field descriptions

firstDL-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of the first DL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for UL carriers.

secondDL-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of the second DL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for UL carriers.

ul-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of UL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for DL carriers.

#### –	SDAP-Config

The IE SDAP-Config is used to set the configurable SDAP parameters for a data radio bearer. All configured instances of SDAP-Config with the same value of pdu-Session correspond to the same SDAP entity as specified in TS 37.324 [24].

SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

...

}

-- TAG-SDAP-CONFIG-STOP

-- ASN1STOP

SDAP-Config field descriptions

defaultDRB Indicates whether or not this is the default DRB for this PDU session. Among all configured instances of SDAP-Config with the same value of pdu-Session, this field shall be set to true in at most one instance of SDAP-Config and to false in all other instances.

mappedQoS-FlowsToAdd Indicates the list of QFIs of UL QoS flows of the PDU session to be additionally mapped to this DRB. A QFI value can be included at most once in all configured instances of SDAP-Config with the same value of pdu-Session. For QoS flow remapping, the QFI value of the remapped QoS flow is only included in mappedQoS-FlowsToAdd in sdap-Config corresponding to the new DRB and not included in mappedQoS-FlowsToRelease in sdap-Config corresponding to the old DRB.

mappedQoS-FlowsToRelease Indicates the list of QFIs of QoS flows of the PDU session to be released from existing QoS flow to DRB mapping of this DRB.

pdu-Session Identity of the PDU session whose QoS flows are mapped to the DRB.

sdap-HeaderUL Indicates whether or not a SDAP header is present for UL data on this DRB. The field cannot be changed after a DRB is established. The network sets this field to present if the field defaultDRB is set to true.

sdap-HeaderDL Indicates whether or not a SDAP header is present for DL data on this DRB. The field cannot be changed after a DRB is established.

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled SCell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions). For a scheduled SpCell in the case of the cross carrier scheduling, if the search space is linked to another search space in the scheduling SCell, all the optional fields of this search space in the scheduled SpCell are absent (regardless of their presence conditions) except for nrofCandidates.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup4

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need S

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]],

[[

dci-FormatsNCR-r18                   ENUMERATED {formats2-8}                                OPTIONAL,    -- Need R

dci-FormatsSL-PRS-r18                ENUMERATED {formats3-2, formats3-0-And-3-2, formats3-0-And-3-1-And-3-2,

formats3-1-And-3-2}                        OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceExt-r16 ::=                   SEQUENCE {

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly2

searchSpaceType-r16                     SEQUENCE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-5-r16                      SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL,    -- Cond Setup3

searchSpaceGroupIdList-r16                      SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)           OPTIONAL,    -- Need R

freqMonitorLocations-r16                        BIT STRING (SIZE (5))                               OPTIONAL     -- Need R

}

SearchSpaceExt-v1700 ::=            SEQUENCE {

monitoringSlotPeriodicityAndOffset-v1710 CHOICE {

sl32                                     INTEGER (0..31),

sl64                                     INTEGER (0..63),

sl128                                    INTEGER (0..127),

sl5120                                   INTEGER (0..5119),

sl10240                                  INTEGER (0..10239),

sl20480                                  INTEGER (0..20479)

}                                                                                                   OPTIONAL,   -- Cond Setup5

monitoringSlotsWithinSlotGroup-r17       CHOICE {

slotGroupLength4-r17                     BIT STRING (SIZE (4)),

slotGroupLength8-r17                     BIT STRING (SIZE (8))

}                                                                                                   OPTIONAL,   -- Need R

duration-r17                             INTEGER (4..20476)                                         OPTIONAL,   -- Need R

searchSpaceType-r17             SEQUENCE{

common-r17                      SEQUENCE {

dci-Format4-0-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-2-r17               SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format4-1-AndFormat4-2-r17  SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-7-r17               SEQUENCE {

nrofCandidates-PEI-r17          SEQUENCE {

aggregationLevel4-r17       ENUMERATED {n0, n1, n2, n3, n4}                         OPTIONAL,   -- Need R

aggregationLevel8-r17       ENUMERATED {n0, n1, n2}                                 OPTIONAL,   -- Need R

aggregationLevel16-r17      ENUMERATED {n0, n1}                                     OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL    -- Need R

}

}                                                                                                   OPTIONAL,   -- Need R

searchSpaceGroupIdList-r17          SEQUENCE (SIZE (1.. 3)) OF INTEGER (0.. maxNrofSearchSpaceGroups-1-r17)  OPTIONAL,  -- Cond DedicatedOnly

searchSpaceLinkingId-r17            INTEGER (0..maxNrofSearchSpacesLinks-1-r17)                     OPTIONAL    -- Cond DedicatedOnly

}

SearchSpaceExt-v1800 ::=            SEQUENCE {

searchSpaceType-r18                 CHOICE {

common-r18                          SEQUENCE {

dci-Format2-9-r18                   SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

},

ue-Specific-r18                     SEQUENCE {

dci-FormatsMC-r18                   ENUMERATED {formats0-3, formats1-3, formats0-3-And-1-3} OPTIONAL,   -- Need R

...

}

}                                                                                                   OPTIONAL    -- Need R

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controlResourceSetId are configured in the same BWP as this SearchSpace except commonControlResourceSetExt which is configured by SIB20. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Format2-4 If configured, UE monitors the DCI format 2_4 according to TS 38.213 [13], clause 11.2A.

dci-Format2-5 If configured, IAB-MT monitors the DCI format 2_5 according to TS 38.213 [13], clause 14.

dci-Format2-6 If configured, UE monitors the DCI format 2_6 according to TS 38.213 [13], clause 10.1, 10.3. DCI format 2_6 can only be configured on the SpCell.

dci-Format2-7 If configured, UE monitors the DCI format 2_7 according to TS 38.213 [13], clause 10.1, 10.4A.

dci-Format2-9 If configured, UE monitors the DCI format 2_9 according to TS 38.213 [13], clause 10.1, 11.5. DCI format 2_9 can be configured on only one cell in the cell group.

dci-Format4-0 If configured, the UE monitors the DCI format 4_0 with CRC scrambled by MCCH-RNTI/G-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-1-AndFormat4-2 If configured, the UE monitors the DCI format 4_1 and 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [11.1].

dci-Format4-1 If configured, the UE monitors the DCI format 4_1 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Format4-2 If configured, the UE monitors the DCI format 4_2 with CRC scrambled by G-RNTI/G-CS-RNTI according to TS 38.213 [13], clause [10.1].

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1). This field is not configured for operation with shared spectrum channel access in this release.

dci-FormatsMC Indicates whether the UE monitors in this USS for DCI format 0_3 or for format 1_3 or for formats 0_3 and 1_3. Separate search space sets for DCI formats in this field and the DCI formats in ue-Specific (without suffix) are independently configured. NOTE: This parameter is used only for SearchSpace configured to the scheduling cell, while another SearchSpace configured to the reference scheduled cell (if any) configures only nrofCandidates (i.e., all other optional fields are absent) with same searchSpaceId with that for scheduling cell.

dci-Formats-MT Indicates whether the IAB-MT monitors the DCI formats 2-5 according to TS 38.213 [13], clause 14.

dci-FormatsNCR Indicates whether the NCR-MT monitors the DCI formats 2-8 according to TS 38.213 [13], clause 20.

dci-FormatsSL Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1 or for format 3-0 or for format 3-1 or for formats 3-0 and 3-1. If this field is present, the field dci-Formats is ignored and dci-FormatsSL is used.

dci-FormatsSL-PRS Indicates whether the UE monitors in this USS for DCI formats 3-2 or for formats 3-0 and 3-2 or for formats 3-0 and 3-1 and 3-2 or for formats3-1 and 3-2. If this field is present, the field dci-Formats is ignored and dci-FormatsSL-PRS is used.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). For SCS 480 kHz and SCS 960 kHz, duration-r17 is used, and the configured duration is restricted to be an integer multiple of L slots and smaller than periodicity, where L is the configured length of the bitmap monitoringSlotsWithinSlotGroup-r17. If duration-r17 is absent, the UE assumes the duration in slots is equal to L. The maximum valid duration is periodicity-L. For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set, see TS 38.213, clause 10.1. Each bit in the bitmap corresponds to one RB set, and the leftmost (most significant) bit corresponds to RB set 0 in the BWP. A bit set to 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set.

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. For SCS 15, 30, 60, and 120 kHz and if the UE is configured to monitor: - DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. - DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10). - DCI format 2_4, only the values 'sl1', 'sl2', 'sl4', 'sl5', 'sl8' and 'sl10' are applicable. For SCS 480 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40', 'sl64', and 'sl80' are applicable. - DCI format 2_1, only the values ′sl4′, ′sl8′, and ′sl16′ are applicable. - DCI format 2_4, only the values 'sl4', 'sl8', 'sl16', 'sl20', 'sl32', 'sl40' are applicable. For SCS 960 kHz and if the UE is configured to monitor: - DCI format 2_0, only the values 'sl8', 'sl16', 'sl32', 'sl40', 'sl64', 'sl80', 'sl128', and 'sl160' are applicable. - DCI format 2_1, only the values ′sl8′, ′sl16′, and 'sl32' are applicable. - DCI format 2_4, only the values 'sl8', 'sl16', 'sl32', 'sl40', 'sl64', 'sl80' are applicable. For SCS 480 kHz and SCS 960 kHz, and the configured periodicity and offset are restricted to be an integer multiple of L slots, where L is the configured length of the bitmap provided by monitoringSlotsWithinSlotGroup-r17, i.e. for a given periodicity, the offset has a range of {0, L, 2*L, …, L*FLOOR(1/L*(periodicity-1))}. For IAB-MT, If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_5, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10). If monitoringSlotPeriodicityAndOffset-v1710 is present, any previously configured monitoringSlotPeriodicityAndOffset is released, and if monitoringSlotPeriodicityAndOffset is present, any previously configured monitoringSlotPeriodicityAndOffset-v1710 is released.

monitoringSlotsWithinSlotGroup Indicates which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. The first (leftmost, most significant) bit represents the first slot in the slot group, the second bit represents the second slot in the slot group, and so on. A bit set to '1' indicates that the corresponding slot is configured for multi-slot PDCCH monitoring (see TS 38.213 [13], clause 10). The number of slots for multi-slot PDCCH monitoring is configured according to clause 10 in TS 38.213 [13].

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for (multi-slot) PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE or IAB-MT. For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10. For IAB-MT: For DCI format 2_0 or DCI format 2_5, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-CI The number of PDCCH candidates specifically for format 2-4 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 10.1). In this release of the specification, only value n1 is valid.

nrofCandidates-PEI The number of PDCCH candidates specifically for format 2-7 for the configured aggregation level.

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1). For a search space configured with freqMonitorLocations-r16, only value ′n1′ is valid.

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceGroupIdList-r16, searchSpaceGroupIdList-r17 List of search space group IDs which the search space is associated with. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1 if searchSpaceGroupIdList-r16 is included. The network configures at most 3 search space groups per BWP where the group ID is either 0, 1 or 2 if searchSpaceGroupIdList-r17 is included. And if searchSpaceGroupIdList-r17 is included, searchSpaceGroupIdList-r16 is ignored.

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active. For an IAB-MT, the search space defines how/where to search for PDCCH candidates for an IAB-MT where each search space is associated with one ControlResearchSet and for a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpaceLinkingId This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same SearchSpaceLinkingId UE assumes these search spaces are linked to PDCCH repetition REF. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same SS set type (USS/CSS). The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set. The following SS sets cannot be linked with another SS set for PDCCH repetition: SS set 0, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, searchSpaceMCCH, searchSpaceMTCH, peiSearchSpace, and sdt-SearchSpace. SS set configured by recoverySearchSpaceId cannot be linked to another SS set for PDCCH repetition. When a scheduled serving cell is configured to be cross-carrier scheduled by a scheduling serving cell, two PDCCH candidates (with the same AL and candidate index associated with the scheduled serving cell) are linked only if the corresponding two SS sets in the scheduling serving cell are linked and two SS sets in the scheduled serving cell with the same SS set IDs are also linked. This parameter is not applicable to search space configured with dci-FormatsSL for monitoring format 3-0 or format 3-1 or for monitoring formats 3-0 and format 3-1. This parameter is not applicable to search space configured with dci-FormatsSL-PRS for monitoring format 3-2 or for monitoring formats 3-0 and 3-2 or for monitoring formats 3-0 and 3-1 and 3-2 or for monitoring formats 3-1 and 3-2.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| DedicatedOnly | In PDCCH-Config, the field is optionally present, Need R. Otherwise it is absent, Need R. |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| Setup2 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt-r16/searchSpacesToAddModListExt-v1700/searchSpacesToAddModListExt-v1800 of the parent IE with the field searchSpaceType-r16 or searchSpaceType-r17 or searchSpaceType-r18 included. This field is absent if the same SearchSpace ID is included in searchSpacesToAddModListExt-v1800 of the parent IE with the field searchSpaceType-r18 included and set to dci-FormatsMC-r18. Otherwise it is optionally present, Need M. |
| Setup3 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt (without suffix) of the parent IE with the field searchSpaceType (without suffix) included. Otherwise it is optionally present, Need M. |
| Setup4 | This field is mandatory present upon creation of a new SearchSpace if monitoringSlotPeriodicityAndOffset-v1710 is not included. It is optionally present, Need M, otherwise. |
| Setup5 | This field is mandatory present upon creation of a new SearchSpace if monitoringSlotPeriodicityAndOffset (without suffix) is not included. It is optionally present, Need M, otherwise. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent, Need M, otherwise. |
| SetupOnly2 | In PDCCH-Config, the field is optionally present upon creation of a new SearchSpace and absent, Need M upon reconfiguration of an existing SearchSpace. In PDCCH-ConfigCommon, the field is absent. |

#### –	SearchSpaceId

The IE SearchSpaceId is used to identify Search Spaces. The ID space is used across the BWPs of a Serving Cell. The search space with the SearchSpaceId = 0 identifies the search space configured via PBCH (MIB) and in ServingCellConfigCommon (searchSpaceZero). The number of Search Spaces per BWP is limited to 10 including the common and UE specific Search Spaces.

SearchSpaceId information element

-- ASN1START

-- TAG-SEARCHSPACEID-START

SearchSpaceId ::=                   INTEGER (0..maxNrofSearchSpaces-1)

-- TAG-SEARCHSPACEID-STOP

-- ASN1STOP

#### –	SearchSpaceZero

The IE SearchSpaceZero is used to configure SearchSpace#0 of the initial BWP (see TS 38.213 [13], clause 13).

SearchSpaceZero information element

-- ASN1START

-- TAG-SEARCHSPACEZERO-START

SearchSpaceZero ::=                 INTEGER (0..15)

-- TAG-SEARCHSPACEZERO-STOP

-- ASN1STOP

#### –	SecurityAlgorithmConfig

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm and AS ciphering algorithm for SRBs and DRBs.

SecurityAlgorithmConfig information element

-- ASN1START

-- TAG-SECURITYALGORITHMCONFIG-START

SecurityAlgorithmConfig ::=         SEQUENCE {

cipheringAlgorithm                  CipheringAlgorithm,

integrityProtAlgorithm              IntegrityProtAlgorithm          OPTIONAL,   -- Need R

...

}

IntegrityProtAlgorithm ::=          ENUMERATED {

nia0, nia1, nia2, nia3, spare4, spare3,

spare2, spare1, ...}

CipheringAlgorithm ::=              ENUMERATED {

nea0, nea1, nea2, nea3, spare4, spare3,

spare2, spare1, ...}

-- TAG-SECURITYALGORITHMCONFIG-STOP

-- ASN1STOP

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nea0-nea3 are identical to the LTE algorithms eea0-3. The algorithms configured for all bearers using master key shall be the same, and the algorithms configured for all bearers using secondary key, if any, shall be the same. If UE is connected to E-UTRA/EPC, this field indicates the ciphering algorithm to be used for RBs configured with NR PDCP, as specified in TS 33.501 [11].

integrityProtAlgorithm Indicates the integrity protection algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nia0-nia3 are identical to the E-UTRA algorithms eia0-3. The algorithms configured for all bearers using master key shall be the same and the algorithms configured for all bearers using secondary key, if any, shall be the same. The network does not configure nia0 except for unauthenticated emergency sessions for unauthenticated UEs in LSM (limited service mode). If UE is connected to E-UTRA/EPC, this field indicates the integrity protection algorithm to be used for SRBs configured with NR PDCP as specified in TS 33.501 [11], and DRBs configured with integrity protection as specified in TS 33.401 [30]. The network does not configure nia0 for SRB3.

#### –	SelectedPSCellForCHO-WithSCG

The IE SelectedPSCellForCHO-WithSCG is used to indicate the information of the selected target PSCell to target MN at execution of a conditional reconfiguration for CHO with candidate SCG(s).

SelectedPSCellForCHO-WithSCG information element

-- ASN1START

-- TAG-SELECTEDPSCELLFORCHO-WITHSCG-START

SelectedPSCellForCHO-WithSCG-r18 ::= SEQUENCE {

ssbFrequency-r18                     ARFCN-ValueNR,

physCellId-r18                       PhysCellId

}

-- TAG-SELECTEDPSCELLFORCHO-WITHSCG-STOP

-- ASN1STOP

#### –	SemiStaticChannelAccessConfig

The IE SemiStaticChannelAccessConfig is used to configure channel access parameters when the network is operating in semi-static channel access mode (see clause 4.3 in TS 37.213 [48].

SemiStaticChannelAccessConfig information element

-- ASN1START

-- TAG-SEMISTATICCHANNELACCESSCONFIG-START

SemiStaticChannelAccessConfig-r16 ::=    SEQUENCE {

period-r16                               ENUMERATED {ms1, ms2, ms2dot5, ms4, ms5, ms10}

}

-- TAG-SEMISTATICCHANNELACCESSCONFIG-STOP

-- ASN1STOP

SemiStaticChannelAccessConfig field descriptions

period Indicates the periodicity of the semi-static channel access mode (see TS 37.213 [48], clause 4.3). Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on.

#### –	SemiStaticChannelAccessConfigUE

The IE SemiStaticChannelAccessConfigUE is used to configure channel access parameters for UE initiated semi-static channel access.

SemiStaticChannelAccessConfigUE information element

-- ASN1START

-- TAG-SEMISTATICCHANNELACCESSCONFIGUE-START

SemiStaticChannelAccessConfigUE-r17 ::=    SEQUENCE {

periodUE-r17                               ENUMERATED {ms1, ms2, ms2dot5, ms4, ms5, ms10, spare2, spare1},

offsetUE-r17                               INTEGER (0..559)

}

-- TAG-SEMISTATICCHANNELACCESSCONFIGUE-STOP

-- ASN1STOP

SemiStaticChannelAccessConfigUE field descriptions

periodUE Indicates the period of a channel occupancy that the UE can initiate as described in TS 37.213 [48], clause 4.3. Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on.

offsetUE Indicates the number of symbols from the beginning of the even indexed radio frame to the start of the first period within that radio frame that the UE can initiate a channel occupancy (see TS 37.213 [48], clause 4.3), based on the smallest SCS among the configured SCSs in the serving cell. The offset duration indicated by this field is less than the period duration indicated by periodUE. The maximum value is 139, 279 and 559 for 15, 30 and 60 kHz subcarrier spacing, respectively.

#### –	Sensor-LocationInfo

The IE Sensor-LocationInfo is used by the UE to provide sensor information.

Sensor-LocationInfo information element

-- ASN1START

-- TAG-SENSORLOCATIONINFO-START

Sensor-LocationInfo-r16 ::= SEQUENCE {

sensor-MeasurementInformation-r16    OCTET STRING    OPTIONAL,

sensor-MotionInformation-r16         OCTET STRING    OPTIONAL,

...

}

-- TAG-SENSORLOCATIONINFO-STOP

-- ASN1STOP

Sensor-LocationInfo field descriptions

sensor-MeasurementInformation This field provides barometric pressure measurements as Sensor-MeasurementInformation defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

sensor-MotionInformation This field provides motion sensor measurements as Sensor-MotionInformation defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

#### –	ServingCellAndBWP-Id

The IE ServingCellAndBWP-Id is used to indicate a serving cell and an uplink or a downlink BWP.

ServingCellAndBWP-Id information element

-- ASN1START

-- TAG-SERVINGCELLANDBWP-ID-START

ServingCellAndBWP-Id-r17 ::= SEQUENCE {

servingcell-r17              ServCellIndex,

bwp-r17                      BWP-Id

}

-- TAG-SERVINGCELLANDBWP-ID-STOP

-- ASN1STOP

#### –	ServCellIndex

The IE ServCellIndex concerns a short identity, used to uniquely identify a serving cell (i.e. the PCell, the PSCell or an SCell) across the cell groups. Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

-- TAG-SERVCELLINDEX-START

ServCellIndex ::=                   INTEGER (0..maxNrofServingCells-1)

-- TAG-SERVCELLINDEX-STOP

-- ASN1STOP

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

supplementaryUplinkRelease-r16      ENUMERATED {true}                                                       OPTIONAL,   -- Need N

tdd-UL-DL-ConfigurationDedicated-IAB-MT-r16    TDD-UL-DL-ConfigDedicated-IAB-MT-r16                         OPTIONAL,   -- Cond TDD_IAB

dormantBWP-Config-r16               SetupRelease { DormantBWP-Config-r16 }                                  OPTIONAL,   -- Need M

ca-SlotOffset-r16                   CHOICE {

refSCS15kHz                         INTEGER (-2..2),

refSCS30KHz                         INTEGER (-5..5),

refSCS60KHz                         INTEGER (-10..10),

refSCS120KHz                        INTEGER (-20..20)

}                                                                                                           OPTIONAL,   -- Cond AsyncCA

dummy2                              SetupRelease { DummyJ }                                                 OPTIONAL,   -- Need M

intraCellGuardBandsDL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

intraCellGuardBandsUL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

csi-RS-ValidationWithDCI-r16        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-CRS-PatternList1-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList2-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

crs-RateMatch-PerCORESETPoolIndex-r16  ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

enableTwoDefaultTCI-States-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultTCI-StatePerCoresetPoolIndex-r16 ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

enableBeamSwitchTiming-r16          ENUMERATED {true}                                                       OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType1-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType2-r16    ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

directionalCollisionHandling-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

channelAccessConfig-r16             SetupRelease { ChannelAccessConfig-r16 }                                OPTIONAL    -- Need M

]],

[[

nr-dl-PRS-PDC-Info-r17                 SetupRelease {NR-DL-PRS-PDC-Info-r17}                                OPTIONAL,   -- Need M

semiStaticChannelAccessConfigUE-r17    SetupRelease {SemiStaticChannelAccessConfigUE-r17}                   OPTIONAL,   -- Need M

mimoParam-r17                       SetupRelease {MIMOParam-r17}                                            OPTIONAL,   -- Need M

channelAccessMode2-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

timeDomainHARQ-BundlingType1-r17    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

nrofHARQ-BundlingGroups-r17         ENUMERATED {n1, n2, n4}                                                 OPTIONAL,   -- Need R

fdmed-ReceptionMulticast-r17        ENUMERATED {true}                                                       OPTIONAL,   -- Need R

moreThanOneNackOnlyMode-r17         ENUMERATED {mode2}                                                      OPTIONAL,   -- Need S

tci-ActivatedConfig-r17             TCI-ActivatedConfig-r17                                                 OPTIONAL,   -- Cond TCI_ActivatedConfig

directionalCollisionHandling-DC-r17 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-NeighCellsCRS-AssistInfoList-r17  SetupRelease { LTE-NeighCellsCRS-AssistInfoList-r17 }                 OPTIONAL    -- Need M

]],

[[

lte-NeighCellsCRS-Assumptions-r17   ENUMERATED {false}                                                      OPTIONAL    -- Need R

]],

[[

crossCarrierSchedulingConfigRelease-r17 ENUMERATED {true}                                                   OPTIONAL    -- Need N

]],

[[

multiPDSCH-PerSlotType1-CB-r17      ENUMERATED {enabled, disabled}                                          OPTIONAL    -- Need R

]],

[[

lte-CRS-PatternList3-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList4-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

pdcch-CandidateReceptionWithCRS-Overlap-r18  ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

cjt-Scheme-PDSCH-r18                ENUMERATED {cjtSchemeA, cjtSchemeB}                                     OPTIONAL,   -- Need R

tag2-r18                            Tag2-r18                                                                OPTIONAL,   -- Need R

cellDTX-DRX-Config-r18              SetupRelease { CellDTX-DRX-Config-r18 }                                 OPTIONAL,   -- Need M

positionInDCI-cellDTRX-r18          INTEGER (0..maxDCI-2-9-Size-1-r18)                                      OPTIONAL,   -- Need R

cellDTX-DRX-L1activation-r18        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

mc-DCI-SetOfCellsToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18    OPTIONAL,   -- Need N

mc-DCI-SetOfCellsToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF SetOfCellsId-r18         OPTIONAL    -- Need N

]],

[[

mimoParam-v1850                     SetupRelease {MIMOParam-v1850}                                          OPTIONAL    -- Need M

]],

[[

mc-DCI-SetOfCellsToAddModListExt-v1900 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCellsExt-v1900

OPTIONAL,   -- Need N

mimoParam-v1900                     SetupRelease {MIMOParam-v1900}                                          OPTIONAL,   -- Need M

servingCellMO-OD-r19                MeasObjectId                                                         OPTIONAL, -- Cond Diff_Freq

cjt-Scheme-PDSCH-v1900              ENUMERATED {cjt-SchemeC, cjt-SchemeD, cjt-SchemeE}                      OPTIONAL,   -- Need R

ntn-RedcapPrioritizeUL-Dynamic-r19  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

ntn-RedcapPrioritizeUL-Semistatic-r19 ENUMERATED {enabled}                                                  OPTIONAL    -- Need R

]]

}

Tag2-r18 ::=                        SEQUENCE {

tag2-Id-r18                         TAG-Id,

tag2-flag-r18                       BOOLEAN,

n-TimingAdvanceOffset2-r18          ENUMERATED { n0, n25600, n39936, spare1 }                           OPTIONAL    -- Need S

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                                     OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                              OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink                          OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                                  OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                                   OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                                 OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

enablePL-RS-UpdateForPUSCH-SRS-r16  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

uplinkTxSwitching-r16               SetupRelease { UplinkTxSwitching-r16 }                                  OPTIONAL,   -- Need M

mpr-PowerBoost-FR2-r16              ENUMERATED {true}                                                       OPTIONAL    -- Need R

]],

[[

srs-PosTx-Hopping-r18               SetupRelease { SRS-PosTx-Hopping-r18 }                                  OPTIONAL,   -- Need M

enablePL-RS-UpdateForType1CG-PUSCH-r18  ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

powerBoostPi2BPSK-r18               BOOLEAN                                                                 OPTIONAL,   -- Need R

powerBoostQPSK-r18                  BOOLEAN                                                                 OPTIONAL    -- Need R

]]

}

DummyJ ::=                          SEQUENCE {

maxEnergyDetectionThreshold-r16         INTEGER(-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-20..-13),

ul-toDL-COT-SharingED-Threshold-r16     INTEGER (-85..-52)                                                  OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16         ENUMERATED {true}                                                   OPTIONAL    -- Need R

}

ChannelAccessConfig-r16 ::=         SEQUENCE {

energyDetectionConfig-r16           CHOICE {

maxEnergyDetectionThreshold-r16         INTEGER (-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-13..20)

}                                                                                                           OPTIONAL,   -- Need R

ul-toDL-COT-SharingED-Threshold-r16         INTEGER (-85..-52)                                              OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16             ENUMERATED {true}                                               OPTIONAL    -- Need R

}

IntraCellGuardBandsPerSCS-r16 ::=      SEQUENCE {

guardBandSCS-r16                       SubcarrierSpacing,

intraCellGuardBands-r16                SEQUENCE (SIZE (1..4)) OF GuardBand-r16

}

GuardBand-r16 ::=                      SEQUENCE {

startCRB-r16                          INTEGER (0..274),

nrofCRBs-r16                          INTEGER (0..15)

}

DormancyGroupID-r16 ::=         INTEGER (0..4)

DormantBWP-Config-r16::=               SEQUENCE {

dormantBWP-Id-r16                      BWP-Id                                                           OPTIONAL,   -- Need M

withinActiveTimeConfig-r16             SetupRelease { WithinActiveTimeConfig-r16 }                      OPTIONAL,   -- Need M

outsideActiveTimeConfig-r16            SetupRelease { OutsideActiveTimeConfig-r16 }                     OPTIONAL    -- Need M

}

WithinActiveTimeConfig-r16 ::=         SEQUENCE {

firstWithinActiveTimeBWP-Id-r16         BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupWithinActiveTime-r16       DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

OutsideActiveTimeConfig-r16 ::=        SEQUENCE {

firstOutsideActiveTimeBWP-Id-r16        BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupOutsideActiveTime-r16      DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

UplinkTxSwitching-r16 ::=              SEQUENCE {

uplinkTxSwitchingPeriodLocation-r16    BOOLEAN,

uplinkTxSwitchingCarrier-r16           ENUMERATED {carrier1, carrier2}

}

MIMOParam-r17 ::= SEQUENCE {

additionalPCI-ToAddModList-r17     SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF SSB-MTC-AdditionalPCI-r17  OPTIONAL,   -- Need N

additionalPCI-ToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17     OPTIONAL,   -- Need N

unifiedTCI-StateType-r17           ENUMERATED {separate, joint}                                         OPTIONAL,   -- Need R

uplink-PowerControlToAddModList-r17  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControl-r17      OPTIONAL,   -- Need N

uplink-PowerControlToReleaseList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17    OPTIONAL,   -- Need N

sfnSchemePDCCH-r17                 ENUMERATED {sfnSchemeA,sfnSchemeB}                                   OPTIONAL,   -- Need R

sfnSchemePDSCH-r17                 ENUMERATED {sfnSchemeA,sfnSchemeB}                                   OPTIONAL    -- Need R

}

MIMOParam-v1850 ::= SEQUENCE {

additionalTDDConfig-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofAdditionalPCI-r17)) OF  AdditionalTDDConfig-perPCI-ToAddMod-r18

OPTIONAL, -- Cond 2TA-TDD-Only

additionalTDDConfig-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17

OPTIONAL  -- Need N

}

MIMOParam-v1900 ::=                           SEQUENCE {

additionalUplinkPowerControlToAddModList-r19  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlExt-v1900 OPTIONAL, -- Need N

additionalUplinkPowerControlToReleaseList-r19 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17    OPTIONAL, -- Need N

...

}

AdditionalTDDConfig-perPCI-ToAddMod-r18 ::=       SEQUENCE {

additionalTDDConfig-Index-r18                     AdditionalPCIIndex-r17,

tdd-UL-DL-ConfigurationCommon-r18                 TDD-UL-DL-ConfigCommon

}

MC-DCI-SetOfCells-r18 ::=          SEQUENCE {

setOfCellsId-r18                   SetOfCellsId-r18,

nCI-Value-r18                      INTEGER (0..7),

scheduledCellListDCI-1-3-r18       SEQUENCE (SIZE (2..maxNrofCellsInSet-r18)) OF ServCellIndex          OPTIONAL,   -- Need R

scheduledCellListDCI-0-3-r18       SEQUENCE (SIZE (2..maxNrofCellsInSet-r18)) OF ServCellIndex          OPTIONAL,   -- Need R

scheduledCellComboListDCI-1-3-r18  SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 OPTIONAL,   -- Need R

scheduledCellComboListDCI-0-3-r18  SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 OPTIONAL,   -- Need R

antennaPortsDCI1-3-r18             ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI1-3

antennaPortsDCI0-3-r18             ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

tpmi-DCI0-3-r18                    ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

sri-DCI0-3-r18                     ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

priorityIndicatorDCI-1-3-r18       ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

priorityIndicatorDCI-0-3-r18       ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

dormancyDCI-1-3-r18                ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

dormancyDCI-0-3-r18                ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pdcchMonAdaptDCI-1-3-r18           ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pdcchMonAdaptDCI-0-3-r18           ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

minimumSchedulingOffsetK0DCI-1-3-r18        ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

minimumSchedulingOffsetK0DCI-0-3-r18        ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3-r18   ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-enhType3DCI-1-3-r18          ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3-r18  ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-retxDCI-1-3-r18     ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-3-r18         ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-1-3-r18     SEQUENCE (SIZE (1..32)) OF TDRA-FieldIndexDCI-1-3-r18                OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-0-3-r18     SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-0-3-r18                OPTIONAL,   -- Need R

rateMatchListDCI-1-3-r18           SEQUENCE (SIZE (1..16)) OF RateMatchDCI-1-3-r18                      OPTIONAL,   -- Need R

zp-CSI-RSListDCI-1-3-r18           SEQUENCE (SIZE (1..8)) OF ZP-CSI-DCI-1-3-r18                         OPTIONAL,   -- Need R

tci-ListDCI-1-3-r18                SEQUENCE (SIZE (1..16)) OF TCI-DCI-1-3-r18                           OPTIONAL,   -- Need R

srs-RequestListDCI-1-3-r18         SEQUENCE (SIZE (1..16)) OF SRS-RequestCombo-r18                      OPTIONAL,   -- Need R

srs-OffsetListDCI-1-3-r18          SEQUENCE (SIZE (1..8)) OF SRS-OffsetCombo-r18                        OPTIONAL,   -- Need R

srs-RequestListDCI-0-3-r18         SEQUENCE (SIZE (1..16)) OF SRS-RequestCombo-r18                      OPTIONAL,   -- Need R

srs-OffsetListDCI-0-3-r18          SEQUENCE (SIZE (1..8)) OF SRS-OffsetCombo-r18                        OPTIONAL    -- Need R

}

MC-DCI-SetOfCellsExt-v1900 ::=     SEQUENCE {

tdra-FieldIndexListDCI-1-3-r19     SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-1-3-r19                OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-0-3-r19     SEQUENCE (SIZE (1..128)) OF TDRA-FieldIndexDCI-0-3-r19               OPTIONAL    -- Need R

}

SetOfCellsId-r18 ::=                   INTEGER (0..maxNrofSetsOfCells-1-r18)

ScheduledCellCombo-r18 ::=             SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER (0..maxNrofCellsInSet-1-r18)

RateMatchDCI-1-3-r18 ::=               SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (1..2))

ZP-CSI-DCI-1-3-r18 ::=                 SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (1..2))

TCI-DCI-1-3-r18 ::=                    SEQUENCE (SIZE (2.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (3))

SRS-RequestCombo-r18 ::=               SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (2..3))

SRS-OffsetCombo-r18 ::=                SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF INTEGER (0..3)

TDRA-FieldIndexDCI-1-3-r18 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofDL-Allocations-1-r18)

TDRA-FieldIndexDCI-1-3-r19 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0.. maxNrofDL-AllocationsExt-1-r19)

TDRA-FieldIndexDCI-0-3-r18 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18)

TDRA-FieldIndexDCI-0-3-r19 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18)

-- TAG-SERVINGCELLCONFIG-STOP

-- ASN1STOP

ChannelAccessConfig field descriptions

absenceOfAnyOtherTechnology Presence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier; absence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 37.213 [48] clauses 4.2.1 and 4.2.3.

energyDetectionConfig Indicates whether to use the maxEnergyDetectionThreshold or the energyDetectionThresholdOffset (see TS 37.213 [48], clause 4.2.3).

energyDetectionThresholdOffset Indicates the offset to the default maximum energy detection threshold value. Unit in dB. Value -13 corresponds to -13dB, value -12 corresponds to -12dB, and so on (i.e. in steps of 1dB) as specified in TS 37.213 [48], clause 4.2.3.

maxEnergyDetectionThreshold Indicates the absolute maximum energy detection threshold value. Unit in dBm. Value -85 corresponds to -85 dBm, value -84 corresponds to -84 dBm, and so on (i.e. in steps of 1dBm) as specified in TS 37.213 [48], clause 4.2.3.

ul-toDL-COT-SharingED-Threshold Maximum energy detection threshold that the UE should use to share channel occupancy with gNB for DL transmission as specified in TS 37.213 [48], clause 4.1.3 for downlink channel access and clause 4.2.3 for uplink channel access. This field is not applicable in semi-static channel access mode.

ServingCellConfig field descriptions

additionalPCI-ToAddModList List of information for the additional SSB with different PCI than the serving cell PCI. The additional SSBs with different PCIs are not used for serving cell quality derivation.

additionalTDDConfig-perPCI-ToAddModList List of TDD-UL-DL configurations for the additional PCIs. When the network releases an additional PCI of a serving cell, the network also explicitly releases the associated TDD-UL-DL configuration for the additional PCI.

bwp-InactivityTimer The duration in ms after which the UE falls back to the default Bandwidth Part (see TS 38.321 [3], clause 5.15). When the network releases the timer configuration, the UE stops the timer without switching to the default BWP.

ca-SlotOffset Slot offset between the primary cell (PCell/PSCell) and the SCell in unaligned frame boundary with slot alignment and partial SFN alignment inter-band CA. Based on this field, the UE determines the time offset of the SCell as specified in clause 4.5 of TS 38.211 [16]. The granularity of this field is determined by the reference SCS for the slot offset (i.e. the maximum of PCell/PSCell lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfigCommon or ServingCellConfigCommonSIB and this serving cell's lowest SCS among all the configured SCSs in DL/UL SCS-SpecificCarrierList in ServingCellConfigCommon or ServingCellConfigCommonSIB). The Network configures at most single non-zero offset duration in ms (independent on SCS) among CCs in the unaligned CA configuration. If the field is absent, the UE applies the value of 0. The slot offset value can only be changed with SCell release and add.

cbg-TxDiffTBsProcessingType1, cbg-TxDiffTBsProcessingType2 Indicates whether processing types 1 and 2 based CBG based operation is enabled according to Rel-16 UE capabilities.

cellDTX-DRX-Config Used to configure cell DTX/DRX for the serving cell, as specified in TS 38.321 [3]. A maximum of two cell DTX/DRX patterns can be configured per MAC entity for different serving cells. The two configured patterns are aligned, that the start and slot offset are common and the periodicity of one pattern is an integer multiple of the other. Cell DTX is configured only when connected mode DRX is configured.

cellDTX-DRX-L1activation Indicates whether this serving cell has enabled L1 signaling based on DCI 2_9 for dynamic activation/deactivation of cell DTX/DRX configuration.

cjt-Scheme-PDSCH, cjt-Scheme-PDSCH-v1900 This field is used to configure CJT Tx scheme cjtSchemeA or cjtSchemeB, cjt-SchemeC, cjt-SchemeD or cjt-SchemeE for PDSCH reception, see TS 38.214 [19] clause 5.1.5. The network does not configure cjt-Scheme-PDSCH and cjt-Scheme-PDSCH-v1900 simultaneously.

channelAccessConfig List of parameters used for access procedures of operation with shared spectrum channel access (see TS 37.213 [48).

channelAccessMode2 If present, this field indicates that the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE does not apply these channel access procedures. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations. Overwrites the corresponding field in ServingCellConfigCommon or ServingCellConfigCommonSIB for this serving cell.

crossCarrierSchedulingConfig Indicates whether this serving cell is cross-carrier scheduled by another serving cell or whether it cross-carrier schedules another serving cell. If the field other is configured for an SpCell (i.e., the SpCell is cross-carrier scheduled by another serving cell), the SpCell can be additionally scheduled by the PDCCH on the SpCell.

crossCarrierSchedulingConfigRelease If this field is included, the UE shall release the cross carrier scheduling configuration configured by crossCarrierSchedulingConfig. The network may only include either crossCarrierSchedulingConfigRelease or crossCarrierSchedulingConfig at a time.

crs-RateMatch-PerCORESETPoolIndex Indicates how UE performs rate matching when both lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 are configured or when both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are configured as specified in TS 38.214 [19], clause 5.1.4.2.

csi-RS-ValidationWithDCI Indicates how the UE performs periodic and semi-persistent CSI-RS reception in a slot. The presence of this field indicates that the UE uses DCI detection to validate whether to receive CSI-RS (see TS 38.213 [13], clause 11.1).

defaultDownlinkBWP-Id The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of the BWP inactivity timer. This field is UE specific. When the field is absent the UE uses the initial BWP as default BWP. (see TS 38.213 [13], clause 12 and TS 38.321 [3], clause 5.15).

directionalCollisionHandling Indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD CA with same SCS as specified in TS 38.213 [13], clause 11.1. The half-duplex operation only applies within the same frequency range and cell group. The network only configures this field for TDD serving cells that are using the same SCS.

directionalCollisionHandling-DC For the IAB-MT, it indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD NR-DC with same SCS within same cell group or cross different cell groups.

dormantBWP-Config The dormant BWP configuration for an SCell. This field can be configured only for a (non-PUCCH) SCell.

downlinkBWP-ToAddModList List of additional downlink bandwidth parts to be added or modified. (see TS 38.213 [13], clause 12).

downlinkBWP-ToReleaseList List of additional downlink bandwidth parts to be released. (see TS 38.213 [13], clause 12).

downlinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in DownlinkConfigCommon / DownlinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]. If the UE is an (e)RedCap UE and needs to autonomously switch to its initial downlink bandwidth part to perform a random access procedure but its current UE specific channel bandwidth does not cover the initial downlink bandwidth part, the UE autonomously changes its UE specific channel bandwidth to cover the initial downlink bandwidth part. In that case, after completion of the random access procedure, the network ensures that the UE specific channel bandwidth fully covers the UE's active downlink bandwidth part in subsequent bandwidth part switch operations.

dummy1, dummy 2 This field is not used in the specification. If received it shall be ignored by the UE.

enableBeamSwitchTiming Indicates the aperiodic CSI-RS triggering with beam switching triggering behaviour as defined in clause 5.2.1.5.1 of TS 38.214 [19].

enableDefaultTCI-StatePerCoresetPoolIndex Presence of this field indicates the UE shall follow the release 16 behavior of default TCI state per CORESETPoolindex when the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet is enabled.

enableTwoDefaultTCI-States Presence of this field indicates the UE shall follow the release 16 behavior of two default TCI states for PDSCH when at least one TCI codepoint is mapped to two TCI states is enabled

fdmed-ReceptionMulticast Indicates the Type-1 HARQ codebook generation as specified in TS 38.213 [13], clause 9.1.2.1.

firstActiveDownlinkBWP-Id If configured for an SpCell, this field contains the ID of the DL BWP to be activated or to be used for RLM, BFD and measurements if included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated, upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If the field is absent for the PSCell at SCG deactivation, the UE considers the previously activated DL BWP as the BWP to be used for RLM, BFD and measurements. If the field is absent for the PSCell at SCG activation, the DL BWP to be activated is the DL BWP previously to be used for RLM, BFD and measurements. If configured for an SCell, this field contains the ID of the downlink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BWP-Id = 0. Upon reconfiguration with reconfigurationWithSync, the network sets the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id to the same value.

initialDownlinkBWP The dedicated (UE-specific) configuration for the initial downlink bandwidth-part (i.e., DL BWP#0). If any of the optional IEs are configured within this IE, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

intraCellGuardBandsDL-List, intraCellGuardBandsUL-List List of intra-cell guard bands in a serving cell for operation with shared spectrum channel access in FR1. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required.

lte-CRS-PatternList1 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround simultaneously.

lte-CRS-PatternList2 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH scheduled with a DCI detected on a CORESET with CORESETPoolIndex configured with 1. This list is configured only if CORESETPoolIndex configured with 1. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList1, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList1, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1.

lte-CRS-PatternList3 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously.

lte-CRS-PatternList4 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

lte-NeighCellsCRS-AssistInfoList A list of LTE neighbour cells configuration information which is used to assist the UE to perform CRS interference mitigation (CRS-IM) in scenarios with overlapping spectrum for LTE and NR (see TS 38.101-4 [59]). If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the LTE-NeighCellsCRS-AssistInfo entries is considered to be newly created and the conditions and Need codes for setup of the entry apply.

lte-NeighCellsCRS-Assumptions If the field is not configured, the following default network configuration assumptions are valid for all LTE neighbour cells for the purpose of CRS interference mitigation (CRS-IM) in scenarios with overlapping spectrum for LTE and NR (see TS 38.101-4 [59]). - The CRS port number is the same as the one indicated in RateMatchPatternLTE-CRS if configured for the serving cell. - The CRS port number is 4 if RateMatchPatternLTE-CRS is not configured for the serving cell. - The channel bandwidth and centre frequency are the same as the ones indicated in RateMatchPatternLTE-CRS if configured for the serving cell. - The MBSFN configuration is the same as the one indicated in RateMatchPatternLTE-CRS if configured for the serving cell. If RateMatchPatternLTE-CRS is not configured for the serving cell, MBSFN subframe is not configured. - Network-based CRS interference mitigation (i.e., CRS muting), as in crs-IntfMitigConfig specified in TS 36.331 [10], is not enabled. If the field is configured (i.e. false) and LTE-NeighCellsCRS-AssistInfoList is configured, the configuration provided in LTE-NeighCellsCRS-AssistInfoList overrides the default network configuration assumptions. If the field is configured (i.e. false) and LTE-NeighCellsCRS-AssistInfoList is not configured, it is up to the UE implementation whether to apply CRS-IM operation.

mc-DCI-SetOfCellsToAddModList, mc-DCI-SetOfCellsToAddModListExt List of up to N (N<=4) configurations of set(s) of cells for multi-cell PDSCH/PUSCH scheduling from the serving cell, where N is reported as UE capability and up to 4 sets of cells can be configured per PUCCH group. When this field is configured to a SCell, PCell cannot be included in either ScheduledCellListDCI-1-3 or ScheduledCellListDCI-0-3. If mc-DCI-SetOfCellsToAddModListExt is present, it contains the same number of entries, and listed in the same order, as in mc-DCI-SetOfCellsToAddModList.

mc-DCI-SetOfCellsToReleaseList List of cell set configurations to release.

multiPDSCH-PerSlotType1-CB Configures the UE behaviour for Type1 codebook HARQ ACK generation regarding the number of PDSCHs per slot on a serving cell as specified in TS 38.213 [13], clause 9.1.2.1. When this parameter is configured and set to disabled for a serving cell, the network does not schedule UE with more than one PDSCH in a slot on the serving cell if HARQ-ACKs of any two PDSCHs in the slot on the serving cell are supposed to be reported on one PUCCH resource in the same PUCCH slot. If two coresetPoolIndex values are configured, the number of received PDSCHs is per coresetPoolIndex value per slot for a serving cell. If the UE generates two HARQ-ACK codebooks for two priorities, the number of received PDSCHs is per priority per slot for a serving cell. If fdmed-ReceptionMulticast is configured, the number of received PDSCHs is per traffic type (unicast / multicast) per slot for a serving cell.

nr-dl-PRS-PDC-Info Configures the DL PRS for propagation delay compensation. When configured, the UE measures the UE Rx-Tx time difference based on the reference signals configured in this field.

nrofHARQ-BundlingGroups Indicates the number of HARQ bundling groups for type2 HARQ-ACK codebook.

ntn-RedcapPrioritizeUL-Dynamic For HD-FDD (e)RedCap NTN UEs, this field indicates that UL is prioritized for DL-UL collisions when dynamically scheduled DL reception collides with dynamically scheduled UL transmission other than collisions with PDSCH scheduled by Type 0/0A/1/2 PDCCH in CSS or PDCCH ordered PRACH (see TS 38.213 [13], clause 17.2).

ntn-RedcapPrioritizeUL-Semistatic For HD-FDD (e)RedCap NTN UEs, this field indicates that UL is prioritized for DL-UL collisions when semi-static DL reception collides with semi-static UL transmission) other than collisions with Type 0/0A/1/2 PDCCH in CSS other than collisions with PDSCH scheduled by Type 0/0A/1/2 PDCCH in CSS (see TS 38.213 [13], clause 17.2).

pathlossReferenceLinking Indicates whether UE shall apply as pathloss reference either the downlink of SpCell (PCell for MCG or PSCell for SCG) or of SCell that corresponds with this uplink (see TS 38.213 [13], clause 7).

pdcch-CandidateReceptionWithCRS-Overlap Presence of this field indicates the UE shall monitor PDCCH candidates that overlap with LTE CRS RE(s).

pdsch-ServingCellConfig PDSCH related parameters that are not BWP-specific.

positionInDCI-cellDTRX The starting bit position of an information block of DCI format 2_9 for this serving cell (see TS 38.212 [17], clause 7.3.1.3.10).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology. See TS 38.214 [19], clause 5.1.4.1. If a RateMatchPattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, the entire RateMatchPattern configuration shall be the same, including the set of RBs/REs indicated by the patterns for the rate matching around, and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

sCellDeactivationTimer SCell deactivation timer in TS 38.321 [3]. If the field is absent, the UE applies the value infinity.

sfnSchemePDCCH This parameter is used to configure single frequency network scheme for PDCCH: sfnSchemeA or sfnSchemeB as specified (see TS 38.214 [19], clause 5.1). If network includes both sfnSchemePDCCH and sfnSchemePDSCH, same value shall be configured.

sfnSchemePDSCH This parameter is used to configure single frequency network scheme for PDSCH: sfnSchemeA or sfnSchemeB as specified (see TS 38.214 [19], clause 5.1). If network includes both sfnSchemePDCCH and sfnSchemePDSCH, same value shall be configured. The network does not configure this parameter and repetitionSchemeConfig in PDSCH-Config simultaneously in the same serving cell.

semiStaticChannelAccessConfigUE When this field is configured and when channelAccessMode-r16 (see IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB) is configured to semiStatic, the UE operates in semi-static channel access mode and can initiate a channel occupancy periodically (see TS 37.213 [48], Clause 4.3). The period can be configured independently from period configured in SemiStaticChannelAccessConfig-r16 if the UE indicates the corresponding capability. Otherwise, the periodicity configured by periodUE-r17 is an integer multiple of or an integer factor of the periodicity indicated by period in SemiStaticChannelAccessConfig-r16.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. If the serving cell is associated with SSB, the following relationship applies between the corresponding MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList. If the serving cell is not associated with SSB (i.e. SSB-less SCell), the carrier frequency indicated by ssbFrequency of the corresponding MeasObjectNR, if configured, is within the frequency range indicated by any entry of the scs-SpecificCarrierList.

servingCellMO-OD measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell instead of servingCellMO in IE ServingCellConfig.

supplementaryUplink Network may configure this field only when supplementaryUplinkConfig is configured in ServingCellConfigCommon or supplementaryUplink is configured in ServingCellConfigCommonSIB.

supplementaryUplinkRelease If this field is included, the UE shall release the uplink configuration configured by supplementaryUplink. The network only includes either supplementaryUplinkRelease or supplementaryUplink at a time.

tag-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell or set of TCI-States of this cell are associated with.

tag2 This field is used to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if coresetPoolIndex for a BWP is configured with more than one value or if twoTA-Without-MultiDCI-MultiTRP is configured for a BWP.

tci-ActivatedConfig If configured for an SCell, or if configured for the PSCell when the SCG is being activated upon the reception of the containing message, the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell. If configured for the PSCell when the SCG is indicated as deactivated in the containing message: - the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD. When this field is absent for the PSCell and the SCG is being deactivated: - the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

tdd-UL-DL-ConfigurationDedicated-IAB-MT Resource configuration per IAB-MT D/U/F overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL ConfigurationCommon.

unifiedTCI-StateType Indicates the unified TCI state type the UE is configured for this serving cell. The value separate means this serving cell is configured with dl-OrJointTCI-StateList for DL TCI state and ul-TCI-StateList for UL TCI state. The value joint means this serving cell is configured with dl-OrJointTCI-StateList for joint TCI state for UL and DL operation.

uplinkConfig Network may configure this field only when uplinkConfigCommon is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB. Addition or release of this field can only be done upon SCell addition or release (respectively).

uplink-PowerControlToAddModList Configures UL power control parameters for PUSCH, PUCCH and SRS when field unifiedTCI-StateType is configured for this serving cell.

Tag2 field descriptions

n-TimingAdvanceOffset2 The N_TA-Offset2 to be applied for PDCCH order CFRA towards the active additionalPCI as specified in TS 38.133 [14] clause 7.1.1 and for all uplink transmissions on this serving cell associated to tag2 as specified in TS 38.213 [13] clause 4.2. This field is always present if SSB-MTC-AdditionalPCI is configured. It is absent otherwise. If absent, the N_TA-Offset is applied for all uplink transmissions on this serving cell associated to tag2.

tag2-flag If this field is set to true, the tag2-Id is associated to value 0 and tag-Id is associated to value 1 of field TI bit in RAR, fallbackRAR and in the absolute TAC MAC CE, see TS 38.321 [3]. Otherwise, the tag2-Id is associated to value 1 and tag-Id is associated to value 0 of field TI bit in RAR, fallbackRAR and in the absolute TAC MAC CE, see TS 38.321 [3].

tag2-Id Timing Advance Group ID, as specified in TS 38.321 [3], which this cell or set of TCI-States of this cell are associated with.

UplinkConfig field descriptions

carrierSwitching Includes parameters for configuration of carrier based SRS switching (see TS 38.214 [19], clause 6.2.1.3.

enableDefaultBeamPL-ForPUSCH0-0, enableDefaultBeamPL-ForPUCCH, enableDefaultBeamPL-ForSRS When the parameter is present, UE derives the spatial relation and the corresponding pathloss reference Rs as specified in 38.213, clauses 7.1.1, 7.2.1, 7.3.1 and 9.2.2. The network only configures these parameters for FR2.

enablePL-RS-UpdateForPUSCH-SRS When this parameter is present, the Rel-16 feature of MAC CE based pathloss RS updates for PUSCH/SRS is enabled. Network only configures this parameter when the UE is configured with sri-PUSCH-PowerControl. If this field is not configured, network configures at most 4 pathloss RS resources for PUSCH/PUCCH/SRS transmissions per BWP, not including pathloss RS resources for SRS transmissions for positioning. (See TS 38.213 [13], clause 7).

enablePL-RS-UpdateForType1CG-PUSCH When this parameter is present, the Rel-18 feature of MAC CE based pathloss RS updates for Type 1 CG-PUSCH is enabled. The network only configures this parameter, when the parameter enablePL-RS-UpdateForPUSCH-SRS is configured. (See TS 38.213 [13], clause 7).

firstActiveUplinkBWP-Id If configured for an SpCell, this field contains the ID of the UL BWP to be activated upon performing the RRC (re-)configuration. If the field is absent, the RRC (re-)configuration does not impose a BWP switch. If configured for an SCell, this field contains the ID of the uplink bandwidth part to be used upon activation of an SCell. The initial bandwidth part is referred to by BandiwdthPartId = 0.

initialUplinkBWP The dedicated (UE-specific) configuration for the initial uplink bandwidth-part (i.e. UL BWP#0). If any of the optional IEs are configured within this IE as part of the IE uplinkConfig, the UE considers the BWP#0 to be an RRC configured BWP (from UE capability viewpoint). Otherwise, the UE does not consider the BWP#0 as an RRC configured BWP (from UE capability viewpoint). Network always configures the UE with a value for this field if no other BWPs are configured. NOTE1

moreThanOneNackOnlyMode Indicates the mode of NACK-only feedback in the PUCCH transmission, as specified in TS 38.213 [13], clause 18. If multicast CFR is not configured, this field is not included. Otherwise, if the field is absent, UE uses mode 1 for multicast CFR.

mpr-PowerBoost-FR2 Indicates whether UE is allowed to boost uplink transmission power by suspending in-band emission (IBE) requirements as specified in TS 38.101-2 [39]. Network only configures this field for FR2 serving cells.

powerBoostPi2BPSK If this field is set to true, the UE determines the maximum output power for PUCCH/PUSCH transmissions that use pi/2 BPSK modulation according to TS 38.101-1 [15] /TS 38.101-5 [75], clause 6.2.4. The network ensures that powerBoostPi2BPSK and powerBoostPi2BPSK-r18 are not configured at the same time for a UE.

powerBoostQPSK If this field is set to true, the UE determines the maximum output power for PUSCH transmissions that use QPSK modulation according to TS 38.101-1 [15], clause 6.2.4.

pusch-ServingCellConfig PUSCH related parameters that are not BWP-specific.

srs-PosTx-Hopping Contains configuration related to the SRS for Positioning with frequency hopping for RRC_CONNECTED state. When this field is present for a certain uplink carrier, and SP positioning SRS is configured, other positioning SRS resource sets configured in the uplink BWP configurations (i.e., BWP-UplinkDedicated) on this uplink carrier cannot be configured as semi-persistent.

uplinkBWP-ToAddModList The additional bandwidth parts for uplink to be added or modified. In case of TDD uplink- and downlink BWP with the same bandwidthPartId are considered as a BWP pair and must have the same center frequency.

uplinkBWP-ToReleaseList The additional bandwidth parts for uplink to be released.

uplinkChannelBW-PerSCS-List A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A. The UE uses the configuration provided in this field only for the purpose of channel bandwidth and location determination. If absent, UE uses the configuration indicated in scs-SpecificCarrierList in UplinkConfigCommon / UplinkConfigCommonSIB. Network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]. If the UE is an (e)RedCap UE and needs to autonomously switch to its initial uplink bandwidth part to perform a random access procedure but its current UE specific channel bandwidth does not cover the initial uplink bandwidth part, the UE autonomously changes its UE specific channel bandwidth to cover the initial uplink bandwidth part. In that case, after completion of the random access procedure, the network ensures that the UE specific channel bandwidth fully covers the UE's active uplink bandwidth part in subsequent bandwidth part switch operations.

uplinkTxSwitchingPeriodLocation Indicates whether the location of UL Tx switching period is configured in this uplink carrier in case of inter-band UL CA, SUL, or (NG)EN-DC, as specified in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of (NG)EN-DC, network always configures this field to TRUE for NR carrier (i.e. with (NG)EN-DC, the UL switching period always occurs on the NR carrier). In case of inter-band UL CA or SUL, for dynamic uplink Tx switching between 2 bands with 2 uplink carriers or 3 uplink carriers as defined in TS 38.101-1 [15], network configures this field to TRUE for the uplink carrier(s) on one band and configures this field to FALSE for the uplink carrier(s) on the other band. This field is set to the same value for the carriers on the same band.

uplinkTxSwitchingCarrier Indicates that the configured carrier is carrier1 or carrier2 for dynamic uplink Tx switching, as defined in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of (NG)EN-DC, network always configures the NR carrier as carrier 2. In case of inter-band UL CA or SUL, for dynamic uplink Tx switching between 2 bands with 2 uplink carriers or 3 uplink carriers as defined in TS 38.101-1 [15], network configures the uplink carrier(s) on one band as carrier1 and the uplink carrier(s) on the other band as carrier2. This field is set to the same value for the carriers on the same band.

DormantBWP-Config field descriptions

dormancyGroupWithinActiveTime This field contains the ID of an SCell group for Dormancy within active time, to which this SCell belongs. The use of the Dormancy within active time for SCell groups is specified in TS 38.213 [13].

dormancyGroupOutsideActiveTime This field contains the ID of an SCell group for Dormancy outside active time, to which this SCell belongs. The use of the Dormancy outside active time for SCell groups is specified in TS 38.213 [13].

dormantBWP-Id This field contains the ID of the downlink bandwidth part to be used as dormant BWP. If this field is configured, its value is different from defaultDownlinkBWP-Id, and at least one of the withinActiveTimeConfig and outsideActiveTimeConfig should be configured.

firstOutsideActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy outside active time.

firstWithinActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy within active time.

outsideActiveTimeConfig This field contains the configuration to be used for SCell dormancy outside active time, as specified in TS 38.213 [13]. The field can only be configured when the cell group the SCell belongs to is configured with dcp-Config.

withinActiveTimeConfig This field contains the configuration to be used for SCell dormancy within active time, as specified in TS 38.213 [13].

GuardBand field descriptions

startCRB Indicates the starting RB of the guard band.

nrofCRB Indicates the length of the guard band in RBs. When set to 0, zero-size guard band is used.

MC-DCI-SetOfCells field descriptions

antennaPortsDCI1-3, antennaPortsDCI0-3 Configure the indication type for antenna port(s) field in DCI format 1_3 and DCI format 0_3, respectively (see TS 38.212, clauses 7.3.1.2.4 and 7.3.1.1.4).

dormancyDCI-1-3, dormancyDCI-0-3 Configure the presence of Scell dormancy indication field in DCI format 1_3 and DCI format 0_3, respectively.

minimumSchedulingOffsetK0DCI-1-3, minimumSchedulingOffsetK0DCI-0-3 Configure the presence of minimum applicable scheduling offset indicator field in DCI format 1_3 and DCI format 0_3, respectively.

nCI-Value Configure n_CI value used for the set of cells, where unique n_CI value is configured for each set of cells.

pdcchMonAdaptDCI-1-3, pdcchMonAdaptDCI-0-3 Configure the presence of PDCCH monitoring adaptation indication field in DCI format 1_3 and DCI format 0_3, respectively.

pdsch-HARQ-ACK-enhType3DCI-1-3 Enable the enhanced Type 3 HARQ-ACK codebook triggering using DCI format 1_3.

pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3 Enables the enhanced Type 3 CB through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in DCI format 1_3 if the more than one enhanced Type HARQ-ACK codebook is configured for the primary PUCCH cell group.

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3 When configured, the DCI format 1_3 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group.

pdsch-HARQ-ACK-retxDCI-1-3 When configured, the DCI format 1_3 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource (see TS 38.213 [13], clause 9.1.5).

priorityIndicatorDCI-1-3, priorityIndicatorDCI-0-3 Configure the presence of priority indicator field in DCI format 1_3 and DCI format 0_3, respectively (see TS 38.212 [17], clauses 7.3.1.2.4 and 7.3.1.1.4 and TS 38.213 [13] clause 9).

pucch-sSCellDynDCI-1-3 Configure the UE with PUCCH cell switching based on dynamic indication in DCI format 1_3 (see TS 38.213 [13], clause 9.A).

RateMatchDCI-1-3 Configure each row of the joint rate matching indication table for DL scheduling via DCI format 1_3, where bitmap for a cell points to a corresponding rate matching indication applicable for DCI format 1_1 (i.e., MSB and LSB of bitmap refer rateMatchPatternGroup1 and rateMatchPatternGroup2 for a cell, respectively), the order of rate matching indication bitmap in each row refers the order of cells in ScheduledCellListDCI-1-3, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP (i.e., first bitmap is for the first cell in ScheduledCellListDCI-1-X, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP and so on), the number of entries in a row of rateMatchDCI-1-3 should be the same as the number of cells, that are configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP, included in ScheduledCellListDCI-1-3, and entries for co-scheduled cells in a row of rateMatchDCI-1-3 are interpreted based on the BWPs of co-scheduled cells on which the UE operates based on the BWP indicator field of DCI format 1_3 (see TS 38.212 [17], clause 7.3.1.2.4 and TS 38.213 [13], clause 12).

rateMatchListDCI-1-3 Configure joint rate matching indication table for DL scheduling via DCI format 1_3.

ScheduledCellCombo Configure each row of the table for combinations of co-scheduled cells for DL scheduling via DCI format 1_3 and for UL scheduling via DCI format 0_3, where index with value INTEGER (0...3) of co-scheduled cell refers to scheduledCellListDCI-1-3 for DL and scheduledCellListDCI-0-3 for UL.

scheduledCellComboListDCI-1-3, scheduledCellComboListDCI-0-3 Configure the table for combinations of co-scheduled cells for DL scheduling via DCI format 1_3 and UL scheduling via DCI format 0_3, respectively.

scheduledCellListDCI-1-3, scheduledCellListDCI-0-3 Configure the list of possible co-scheduled cells in the set for DL scheduling via DCI format 1_3 and UL scheduling via DCI format 0_3 respectively, where the serving cells in the list are in ascending order of serving cell indices and are mapped to index {0, 1, 2, 3} in the set. Total number of cells within the same set of cells i.e., in scheduledCellListDCI-1-3 and scheduledCellListDCI-0-3, is up to 4. When a cell is included in either or both of scheduledCellListDCI-1-3 or scheduledCellListDCI-0-3 for one set of cells MC-DCI-SetofCells, the cell cannot be included in any of scheduledCellListDCI-1-3 or scheduledCellListDCI-0-3 for any other set of cells.

setOfCellsId Configure index of the set of cells to be indicated in DCI format 0_3/1_3.

sri-DCI0-3 Configure the indication type for SRS resource indicator field in DCI format 0_3 (See TS 38.212, clause 7.3.1.1.4).

SRS-OffsetCombo Configure each row of the joint SRS offset indicator table for DL scheduling via DCI format 1_3 and for UL scheduling via DCI format 0_3, where index for a cell points to a corresponding SRS offset indicator applicable for DCI formats 1_1 and 0_1, and the order of SRS offset indicator index in each row refers the order of cells in scheduledCell-ListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3, that are configured with more than one entry in availableSlotOffsetList for at least one aperiodic SRS resource set on at least one UL BWP and so on) for DL and scheduledCellListDCI-0-3 for UL, included in scheduledCellListDCI-1-3 for srs-OffsetListDCI-1-3 and scheduledCellListDCI-0-3 for srs-OffsetListDCI-0-3, and entries for co-scheduled cells in a row of SRS-OffsetCombo are interpreted based on the BWPs of co-scheduled cells on which the UE operates based on the BWP indicator field of DCI format 1_3 and 0_3 (see TS 38.212 [17], clause 7.3.1.2.4 and clause 7.3.1.1.4, and TS 38.213 [13], clause 12).

srs-OffsetListDCI-1-3, srs-OffsetListDCI-0-3 Configure joint SRS offset indicator table for DL scheduling via DCI format 1_3 and UL scheduling via DCI format 0_3, respectively.

SRS-RequestCombo Configure each row of the joint SRS request table for DL scheduling via DCI format 1_3 and for UL scheduling via DCI format 0_3, where index for a cell points to a corresponding SRS request applicable for DCI formats 1_1 and 0_1, and the order of SRS request index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3 and so on) for DL and scheduledCellListDCI-0-3 for UL. The number of entries in a row of SRS-RequestCombo should be the same as the number of cells included in scheduledCellListDCI-1-3 for srs-RequestListDCI-1-3 and scheduledCellListDCI-0-3 for srs-RequestListDCI-0-3, and entries for co-scheduled cells in a row of SRS-RequestCombo are interpreted based on the BWPs of co-scheduled cells on which the UE operates based on the BWP indicator field of DCI format 1_3 and 0_3 (see TS 38.212 [17], clause 7.3.1.2.4 and clause 7.3.1.1.4, and TS 38.213 [13], clause 12).

srs-RequestListDCI-1-3, srs-RequestListDCI-0-3 Configure joint SRS request table for DL scheduling via DCI format 1_3 and UL scheduling via DCI format 0_3, respectively.

TCI-DCI-1-3 Configure each row of the joint TCI table for DL scheduling via DCI format 1_3, where index for a cell points to a corresponding TCI applicable for DCI format 1_1, and the order of TCI index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3 that configured with tci-StatesToAddModList or dl-OrJointTCI-StateList and so on), the number of entries in a row of TCI-DCI-1-3 should be the same as the number of cells that configured with tci-StatesToAddModList or dl-OrJointTCI-StateList on at least one DL BWP, included in scheduledCellListDCI-1-3, and entries for cells in a row of TCI-DCI-1-3 are interpreted based on the BWPs of cells in scheduledCellListDCI-1-3 on which the UE operates based on the BWP indicator field of DCI format 1_3 (see TS 38.212 [17], clause 7.3.1.2.4, and TS 38.213 [13], clause 12).

tci-ListDCI-1-3 Configure joint TCI table for DL scheduling via DCI format 1_3

TDRA-FieldIndexDCI-0-3 Configure each row of the joint TDRA field table for UL scheduling via DCI format 0_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 0_1 when TDRA-FieldIndexDCI-0-3-r18 is configured or for DCI format 0_3 when TDRA-FieldIndexDCI-0-3-r19 is configured, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-0-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], of the first cell in scheduledCellListDCI-0-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], of the first cell and so on), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-0-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 0_3, as specified in TS 38.212 [17], across cells included in scheduledCellListDCI-0-3.

TDRA-FieldIndexDCI-1-3 Configure each row of the joint TDRA field table for DL scheduling via DCI format 1_3 containing the applicable TDRA field indexes for multiple BWPs/cells, where the TDRA index for a BWP of a cell points to a corresponding TDRA in the TDRA table applicable for DCI format 1_1 when TDRA-FieldIndexDCI-1-3-r18 is configured or for DCI format 1_3 when TDRA-FieldIndexDCI-1-3-r19 is configured, the order of TDRA index in each row refers the BWP-Id for a cell and the order of cells in scheduledCellListDCI-1-3 (i.e., first TDRA index in a row is for the smallest BWP-Id that can be scheduled by the DCI format 1_3, as specified in TS 38.212 [17], of the first cell in scheduledCellListDCI-1-3, second TDRA index in a row is for the second smallest BWP-Id that can be scheduled by the DCI format 1_3, as specified in TS 38.212 [17], of the first cell and so on ), and the number of TDRA indices in a row of TDRA-FieldIndexDCI-1-3 should be the same as the total number of BWPs that can be scheduled by the DCI format 1_3, as specified in TS 38.212 [17], across cells included in scheduledCellListDCI-1-3.

tdra-FieldIndexListDCI-1-3, tdra-FieldIndexListDCI-0-3 Configure joint TDRA table for DL scheduling via DCI format 1_3 and UL scheduling via DCI format 0_3, respectively.

tpmi-DCI0-3 Configure the indication type for precoding information and number of layers field in DCI format 0_3 (See TS 38.212 [17], clause 7.3.1.1.4).

ZP-CSI-DCI-1-3 Configure each row of the joint ZP-CSI-RS trigger table for DL scheduling via DCI format 1_3, where index for a cell points to a corresponding ZP-CSI-RS trigger applicable for DCI format 1_1, and the order of ZP-CSI-RS trigger index in each row refers the order of cells in scheduledCellListDCI-1-3 (i.e., first index is for the first cell in scheduledCellListDCI-1-3, that are configured with aperiodic-ZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP and so on), the number of entries in a row of ZP-CSI-DCI-1-3 should be the same as the number of cells, that are configured with aperiodic-ZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP, included in scheduledCellListDCI-1-3, and entries for co-scheduled cells in a row of ZP-CSI-DCI-1-3 are interpreted based on the BWPs of co-scheduled cells on which the UE operates based on the BWP indicator field of DCI format 1_3 (see TS 38.212 [17], clause 7.3.1.2.4 and TS 38.213 [13], clause 12).

zp-CSI-RSListDCI-1-3 Configure joint ZP-CSI-RS trigger table for DL scheduling via DCI format 1_3.

NOTE 1:	If the dedicated part of initial UL/DL BWP configuration is absent, the initial BWP can be used but with some limitations. For example, changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA-TDD-Only | The field is optionally present, Need N, for a TDD cell, in the mimoParam-v1850 if additionalPCI-ToAddModList is present in ServingCellConfig and if tag2 is present in ServingCellConfig. It is absent otherwise. |
| AsyncCA | This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S. |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. For SSB-less SCell(s), this field is not present if intraF-NeighMeasForSCellWithoutSSB is not supported by the UE, otherwise this field is optionally present, Need M. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M. |
| TCI_ActivatedConfig | This field is optional Need N for SCells if sCellState is configured, otherwise it is absent. This field is optional Need S for the PSCell when the SCG is indicated as deactivated or is being activated, otherwise it is absent. This field is absent for the PCell. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TDD_IAB | For IAB-MT, this field is optionally present, Need R, for TDD cells. It is absent otherwise. |
| TypeDCI0-3 | This field is mandatory present if ScheduledCellListDCI-0-3 is configured, otherwise it is absent, Need R. |
| TypeDCI1-3 | This field is mandatory present if ScheduledCellListDCI-1-3 is configured, otherwise it is absent, Need R. |
| Diff_Freq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. |

#### –	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMON-START

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                                       OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16          ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}                   OPTIONAL, -- Need R

ssb-PositionQCL-r16                     SSB-PositionQCL-Relation-r16                                    OPTIONAL, -- Cond SharedSpectrum

highSpeedConfig-r16                     HighSpeedConfig-r16                                             OPTIONAL  -- Need R

]],

[[

highSpeedConfig-v1700               HighSpeedConfig-v1700                                               OPTIONAL, -- Need R

channelAccessMode2-r17              ENUMERATED {enabled}                                                OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}  OPTIONAL, -- Need R

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                        OPTIONAL, -- Cond SharedSpectrum2

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                              OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommon-v1700                                            OPTIONAL, -- Need R

ntn-Config-r17                      NTN-Config-r17                                                      OPTIONAL  -- Need R

]],

[[

featurePriorities-r17               SEQUENCE {

redCapPriority-r17                  FeaturePriority-r17                                             OPTIONAL, -- Need R

slicingPriority-r17                 FeaturePriority-r17                                             OPTIONAL, -- Need R

msg3-Repetitions-Priority-r17       FeaturePriority-r17                                             OPTIONAL, -- Need R

sdt-Priority-r17                    FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17            ENUMERATED {enabled}                                                OPTIONAL -- Cond SharedSpectrum2

]],

[[

featurePriorities-v1800             SEQUENCE {

msg1-Repetitions-Priority-r18       FeaturePriority-r17                                             OPTIONAL, -- Need R

eRedCapPriority-r18                 FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL, -- Need R

atg-Config-r18                      ATG-Config-r18                                                      OPTIONAL   -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMON-STOP

-- ASN1STOP

ServingCellConfigCommon field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.1) and uplink (TS 38.211 [16], clause 6.4.1.1.3).

downlinkConfigCommon The common downlink configuration of the serving cell, including the frequency information configuration and the initial downlink BWP common configuration. The parameters provided herein should match the parameters configured by MIB and SIB1 (if provided) of the serving cell, with the exception of controlResourceSetZero and searchSpaceZero which can be configured in ServingCellConfigCommon even if MIB indicates that they are absent.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-r17 is applicable to SCS 480 kHz and SCS 960 kHz.

featurePriorities Indicates priorities for features, such as (e)RedCap, Slicing, SDT, MSG1-Repetitions, and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1.

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on this serving cell if n-TimingAdvanceOffset2 is not configured. If tag2 is configured for this serving cell, this field is to be applied to all uplink transmissions associated to tag-id configured for this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2/table 7.1C.2-4.

ra-ChannelAccess If present, this field indicates that the UE shall apply channel access procedures before msg1/msgA transmission for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4.5 for FR2-2.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology (see TS 38.214 [19], clause 5.1.4.1). If a RateMatchPattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, the entire RateMatchPattern configuration, including the set of RBs/REs indicated by the patterns for the rate matching around, shall be the same and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-periodicityServingCell The SSB periodicity in ms for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssb-PositionQCL Indicates the QCL relation between SSB positions for this serving cell as specified in TS 38.213 [13], clause 4.1.

ssb-PositionsInBurst For operation in licensed spectrum, indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access, the UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst (see TS 38.213 [13], clause 4.1). If the k-th bit of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access in FR1, only mediumBitmap is used, and for FR2-2, longBitmap is used.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

supplementaryUplinkConfig The network configures this field only if uplinkConfigCommon is configured. If this field is absent, the UE shall release the supplementaryUplinkConfig and the supplementaryUplink configured in ServingCellConfig of this serving cell, if configured.

tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

| Conditional Presence | Explanation |
| --- | --- |
| AbsFreqSSB | The field is absent when absoluteFrequencySSB in frequencyInfoDL is absent, otherwise the field is mandatory present. |
| HOAndServCellAdd | This field is mandatory present upon SpCell change (including path switch between a serving cell and a L2 U2N Relay UE) and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent. |
| HOAndServCellWithSSB | This field is mandatory present upon SpCell change and upon serving cell (SCell with SSB or PSCell) addition. Otherwise, the field is absent. |
| SharedSpectrum | This field is mandatory present if this cell operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this cell operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

#### –	ServingCellConfigCommonSIB

The IE ServingCellConfigCommonSIB is used to configure cell specific parameters of a UE's serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                      OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                               OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16      ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}               OPTIONAL, -- Need R

highSpeedConfig-r16                 HighSpeedConfig-r16                                         OPTIONAL  -- Need R

]],

[[

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-v1700    ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25} OPTIONAL,  -- Need R

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                      OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommonSIB-v1700                                 OPTIONAL  -- Need R

]],

[[

enhancedMeasurementNGSO-r17         ENUMERATED {true}                                           OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17                ENUMERATED {enabled}                                        OPTIONAL  -- Cond SharedSpectrum2

]],

[[

downlinkConfigCommon-v1760          DownlinkConfigCommonSIB-v1760                               OPTIONAL, -- Need R

uplinkConfigCommon-v1760            UplinkConfigCommonSIB-v1760                                 OPTIONAL  -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMONSIB-STOP

-- ASN1STOP

ServingCellConfigCommonSIB field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, this field indicates that the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-v1700 is applicable to SCS 480 kHz and SCS 960 kHz.

enhancedMeasurementNGSO If the field is present and UE supports the enhanced cell reselection requirements for NTN NGSO in RRC_IDLE/RRC_INACTIVE, the UE shall apply the enhanced cell reselection requirements for NTN NGSO as specified in TS 38.133 [14], clauses 4.2C.2.3 and 4.2C.2.4.

groupPresence This field is present when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to the SS/PBCH index 0-7, the second bit corresponds to SS/PBCH block 8-15, and so on. Value 0 in the bitmap indicates that the SSBs according to inOneGroup are absent. Value 1 indicates that the SS/PBCH blocks are transmitted in accordance with inOneGroup.

inOneGroup When maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1, only the 4 leftmost bits are valid; the UE ignores the 4 rightmost bits. When maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1, all 8 bits are valid. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. When maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1, all 8 bit are valid; The first/ leftmost bit corresponds to the first SS/PBCH block index in the group (i.e., to SSB index 0, 8, and so on); the second bit corresponds to the second SS/PBCH block index in the group (i.e., to SSB index 1, 9, and so on), and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2/table 7.1C.2-4.

ra-ChannelAccess If present, this field indicates that the UE shall apply channel access procedures before msg1/msgA transmission for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4.5 for FR2-2.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR1, only inOneGroup is used and the UE interprets this field same as mediumBitmap in ServingCellConfigCommon. The UE assumes that a bit in inOneGroup at position k > NSSBQCL is 0, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR2-2, the m-th bit in groupPresence is set to 0 for m > NSSBQCL/8, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| SharedSpectrum | This field is mandatory present if this cell operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this cell operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

#### –	ShortI-RNTI-Value

The IE ShortI-RNTI-Value is used to identify the suspended UE context of a UE in RRC_INACTIVE using fewer bits compared to I-RNTI-Value.

ShortI-RNTI-Value information element

-- ASN1START

-- TAG-SHORTI-RNTI-VALUE-START

ShortI-RNTI-Value ::=   BIT STRING (SIZE(24))

-- TAG-SHORTI-RNTI-VALUE-STOP

-- ASN1STOP

#### –	ShortMAC-I

The IE ShortMAC-I is used to identify and verify the UE at RRC connection re-establishment. The 16 least significant bits of the MAC-I calculated using the AS security configuration of the source PCell, as specified in 5.3.7.4.

ShortMAC-I information element

-- ASN1START

-- TAG-SHORTMAC-I-START

ShortMAC-I ::=                      BIT STRING (SIZE (16))

-- TAG-SHORTMAC-I-STOP

-- ASN1STOP

#### –	SINR-Range

The IE SINR-Range specifies the value range used in SINR measurements and thresholds. For measurements, integer value for SINR measurements is according to Table 10.1.16.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value – 46) / 2 dB.

SINR-Range information element

-- ASN1START

-- TAG-SINR-RANGE-START

SINR-Range ::=                      INTEGER(0..127)

-- TAG-SINR-RANGE-STOP

-- ASN1STOP

#### –	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request without Msg1 repetition. For the random access parameters (e.g. prach-RootSequenceIndex, msg1-SubcarrierSpacing, etc.) not configured in SI-RequestConfig, the UE applies the parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP unless otherwise specified.

SI-RequestConfig information element

-- ASN1START

-- TAG-SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SI-REQUESTCONFIG-STOP

-- ASN1STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting. Otherwise: - If si-SchedulingInfo-v1700 is not present and the SI-RequestConfig is used for on-demand SI request in SI-SchedulingInfo or PosSI-SchedulingInfo, the 1st entry in the list corresponds to the first SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfig is configured in SI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which si-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and SI-RequestConfig is configured in PosSI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

#### –	SI-RequestConfigRepetition

The IE SI-RequestConfigRepetition contains configuration for Msg1 based SI request with Msg1 repetition. For the random access parameters (e.g. prach-RootSequenceIndex, msg1-SubcarrierSpacing and msg1-RepetitionTimeOffsetROGroup, etc.) not configured in SI-RequestConfigRepetition, the UE applies the parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP unless otherwise specified.

SI-RequestConfigRepetition information element

-- ASN1START

-- TAG-SI-REQUESTCONFIGREPETITION-START

SI-RequestConfigRepetition-r18 ::=     SEQUENCE {

rach-OccasionsSI-r18                   SEQUENCE {

rach-ConfigSI-r18                      RACH-ConfigGeneric,

ssb-perRACH-Occasion-r18               ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                               OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum2-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum4-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum8-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

...

}

SI-RequestResourcesRepetition-r18 ::=  SEQUENCE {

ra-PreambleStartIndex-r18              INTEGER (0..63)

}

-- TAG-SI-REQUESTCONFIGREPETITION-STOP

-- ASN1STOP

SI-RequestConfigRepetition field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP.

si-RequestResourcesRepetitionNum2, si-RequestResourcesRepetitionNum4, si-RequestResourcesRepetitionNum8 Indicates the configurations for Msg1-based SI request with Msg1 repetition for repetition number 2, 4 and 8, respectively. If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting. Otherwise: - If si-SchedulingInfo-v1700 is not present and the si-RequestResourcesRepetitionNum2/si-RequestResourcesRepetitionNum4/si-RequestResourcesRepetitionNum8 is used for on-demand SI request in SI-SchedulingInfo or PosSI-SchedulingInfo, the 1st entry in the list corresponds to the first SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList or posSchedulingInfoList for which si-BroadcastStatus or posSI-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and si-RequestResourcesRepetitionNum2/si-RequestResourcesRepetitionNum4/si-RequestResourcesRepetitionNum8 is configured in SI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which si-BroadcastStatus is set to notBroadcasting and so on. - If si-SchedulingInfo-v1700 is present and si-RequestResourcesRepetitionNum2/si-RequestResourcesRepetitionNum4/si-RequestResourcesRepetitionNum8 is configured in PosSI-SchedulingInfo for on-demand SI request, the UE generates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. The 1st entry in the list corresponds to the first SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message for which posSI-BroadcastStatus or si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResourcesRepetitionNum2/si-RequestResourcesRepetitionNum4/si-RequestResourcesRepetitionNum8 should not result in system information change notification.

SI-RequestResourcesRepetition field descriptions

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request with Msg1 repetition; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request with Msg1 repetition.

#### –	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG-SI-SCHEDULINGINFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 },

si-RequestConfig                    SI-RequestConfig                                                OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                          OPTIONAL,  -- Need R

...

}

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

SI-SchedulingInfo-v1700 ::=         SEQUENCE {

schedulingInfoList2-r17             SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17,

dummy                              SI-RequestConfig                                                 OPTIONAL

}

SI-SchedulingInfo-v1740 ::=         SEQUENCE {

si-RequestConfigRedCap-r17          SI-RequestConfig                                                OPTIONAL   -- Cond REDCAP-MSG-1

}

SI-SchedulingInfo-v1800 ::=         SEQUENCE {

si-RequestConfigMSG1-Repetition-r18           SI-RequestConfigRepetition-r18                        OPTIONAL,  -- Cond MSG-1

si-RequestConfigRedCap-MSG1-Repetition-r18    SI-RequestConfigRepetition-r18                        OPTIONAL,  -- Cond REDCAP-MSG-1

si-RequestConfigSUL-MSG1-Repetition-r18       SI-RequestConfigRepetition-r18                        OPTIONAL   -- Cond SUL-MSG-1

}

SchedulingInfo2-r17 ::=             SEQUENCE {

si-BroadcastStatus-r17              ENUMERATED {broadcasting, notBroadcasting},

si-WindowPosition-r17               INTEGER (1..256),

si-Periodicity-r17                  ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo-r17                 SIB-Mapping-v1700

}

SIB-Mapping ::=                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo

SIB-Mapping-v1700  ::=              SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610,

sibType14-v1610, spare3, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                 OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                               OPTIONAL -- Need S

}

SIB-TypeInfo-v1700 ::=              SEQUENCE {

sibType-r17                         CHOICE {

type1-r17                           ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20, sibType21,

sibType22-v1800, sibType23-v1800 ,sibType24-v1800, sibType25-v1800,

sibType17bis-v1820, sibType26-v1900, spare3, spare2, spare1,...},

type2-r17                           SEQUENCE {

posSibType-r17                      ENUMERATED {posSibType1-9, posSibType1-10, posSibType2-24, posSibType2-25,

posSibType6-4, posSibType6-5, posSibType6-6, posSibType2-17a-v1770,

posSibType2-18a-v1770, posSibType2-20a-v1770, posSibType1-11-v1800,

posSibType1-12-v1800, posSibType2-26-v1800, posSibType2-27-v1800,

posSibType6-7-v1800, posSibType7-1-v1800,...,

posSibType7-2-v1800, posSibType7-3-v1800, posSibType7-4-v1800},

encrypted-r17                       ENUMERATED { true }                                     OPTIONAL, -- Need R

gnss-id-r17                         GNSS-ID-r16                                             OPTIONAL, -- Need R

sbas-id-r17                         SBAS-ID-r16                                             OPTIONAL  -- Cond GNSS-ID-SBAS

}

},

valueTag-r17                            INTEGER (0..31)                                             OPTIONAL, -- Cond NonPosSIB

areaScope-r17                           ENUMERATED {true}                                           OPTIONAL  -- Need S

}

-- TAG-SI-SCHEDULINGINFO-STOP

-- ASN1STOP

SchedulingInfo field descriptions

areaScope Indicates that a SIB is area specific. If the field is absent, the SIB is cell specific.

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting. The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1 if ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell and if the UE has a stored valid version of od-sib1-Config for this cell.

si-Periodicity Periodicity of the SI-message in radio frames. Value rf8 corresponds to 8 radio frames, value rf16 corresponds to 16 radio frames, and so on.

SI-SchedulingInfo field descriptions

dummy This field is not used in this specification. If received, it is ignored by the UE.

schedulingInfoList, schedulingInfoList2 List of scheduling information for SI messages. If si-SchedulingInfo-v1700 is present, the network ensures that the total number of SI messages in the list of concatenated SI messages configured by schedulingInfoList in si-SchedulingInfo and SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 does not exceed the limit of maxSI-Message.

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigMSG1-Repetition Configuration of Msg1 repetition resources on NUL that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

si-RequestConfigRedCap Configuration of Msg1 resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigRedCap-MSG1-Repetition Configuration of Msg1 repetition resources for initialUplinkBWP-RedCap that the (e)RedCap UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigSUL-MSG1-Repetition Configuration of Msg1 repetition resources on SUL that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting. This field is only applicable when Msg1 repetition resources can be used for requesting SI-messages.

si-WindowLength The length of the SI scheduling window. Value s5 corresponds to 5 slots, value s10 corresponds to 10 slots and so on. The network always configures si-WindowLength to be shorter than or equal to the si-Periodicity. The values s2560-v1710 and s5120-v1710 are only applicable for SCS 480 kHz.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

SchedulingInfo2 field descriptions

encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 37.355 [49].

gnss-id The presence of this field indicates that the positioning SIB type is for a specific GNSS. Indicates a specific GNSS (see also TS 37.355 [49])

posSibType The posSIBs as defined in TS 37.355 [49] mapped to SI for scheduling using schedulingInfoList2.

sbas-id The presence of this field indicates that the positioning SIB type is for a specific SBAS. Indicates a specific SBAS (see also TS 37.355 [49]).

si-WindowPosition This field indicates the SI window position of the associated SI-message. The network provides si-WindowPosition in an ascending order, i.e. si-WindowPosition in the subsequent entry in schedulingInfoList2 has always value higher than in the previous entry of schedulingInfoList2. The network configures this field in a way that ensures that SI messages scheduled by schedulingInfoList and/or posSchedulingInfoList do not overlap with SI messages scheduled by schedulingInfoList2.

sib-MappingInfo Indicates which SIBs or posSIBs are contained in the SI message.

sibType The type of SIB(s) mapped to SI for scheduling using schedulingInfoList2. Value type1 indicates SIBs and value type2 indicates posSIBs.

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if gnss-id is set to sbas. It is absent otherwise. |
| MSG-1 | The field is optionally present, Need R, if si-BroadcastStatus is set to notBroadcasting for any SI-message included in schedulingInfoList or any SI-message containing type1 SIB included in schedulingInfoList2. It is absent otherwise. |
| SIB-TYPE | The field is mandatory present if the SIB type is different from SIB6, SIB7 or SIB8. For SIB6, SIB7 and SIB8 it is absent. |
| NonPosSIB | The field is mandatory present if the SIB type is type1. For type2 it is absent. |
| SUL-MSG-1 | The field is optionally present, Need R, if supplementaryUplink is configured in ServingCellConfigCommonSIB and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in schedulingInfoList or any SI-message containing type1 SIB included in schedulingInfoList2. It is absent otherwise. |
| REDCAP-MSG-1 | The field is optionally present, Need R, if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in schedulingInfoList or any SI-message containing type1 SIB included in schedulingInfoList2. It is absent otherwise. |

#### –	SK-Counter

The IE SK-Counter is a counter used upon initial configuration of SN security for NR-DC and NE-DC, as well as upon refresh of S-KgNB or S-KeNB based on the current or newly derived KgNB during RRC Resume or RRC Reconfiguration, as defined in TS 33.501 [11].

-- ASN1START

-- TAG-SKCOUNTER-START

SK-Counter ::=  INTEGER (0..65535)

-- TAG-SKCOUNTER-STOP

-- ASN1STOP

#### –	SK-CounterConfigLTM

The IE SK-CounterConfigLTM is used to configure a list of sk-Counter values used to derive the S-KgNB.

SK-CounterConfigLTM information element

-- ASN1START

-- TAG-SK-COUNTERCONFIGLTM-START

SK-CounterConfigLTM-r19 ::=          SEQUENCE {

ltm-NoSecurityChangeID-r19           LTM-NoSecurityChangeId-r19,

sk-CounterList-r19                   SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter

}

-- TAG-SK-COUNTERCONFIGLTM-STOP

-- ASN1STOP

#### –	SlotFormatCombinationsPerCell

The IE SlotFormatCombinationsPerCell is used to configure the SlotFormatCombinations applicable for one serving cell (see TS 38.213 [13], clause 11.1.1).

SlotFormatCombinationsPerCell information element

-- ASN1START

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-START

SlotFormatCombinationsPerCell ::=   SEQUENCE {

servingCellId                       ServCellIndex,

subcarrierSpacing                   SubcarrierSpacing,

subcarrierSpacing2                  SubcarrierSpacing                                                         OPTIONAL, -- Need R

slotFormatCombinations              SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination

OPTIONAL, -- Need M

positionInDCI                       INTEGER(0..maxSFI-DCI-PayloadSize-1)                                      OPTIONAL, -- Need M

...,

[[

enableConfiguredUL-r16              ENUMERATED {enabled}                                                      OPTIONAL  -- Need R

]]

}

SlotFormatCombination ::=           SEQUENCE {

slotFormatCombinationId             SlotFormatCombinationId,

slotFormats                         SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255)

}

SlotFormatCombinationId ::=         INTEGER (0..maxNrofSlotFormatCombinationsPerSet-1)

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-STOP

-- ASN1STOP

SlotFormatCombination field descriptions

slotFormatCombinationId This ID is used in the DCI payload to dynamically select this SlotFormatCombination (see TS 38.213 [13], clause 11.1.1).

slotFormats Slot formats that occur in consecutive slots in time domain order as listed here (see TS 38.213 [13], clause 11.1.1 and TS 38.213 [13], clause 14 for IAB-MT).

SlotFormatCombinationsPerCell field descriptions

enableConfiguredUL If configured, the UE is allowed to transmit uplink signals/channels (SRS, PUCCH, CG-PUSCH) in the set of symbols of the slot when the UE does not detect a DCI format 2_0 providing a slot format for the set of symbols (see TS 38.213 [13], 11.1.1). This field is applicable only if cg-RetransmissionTimer-r16 is configured.

positionInDCI The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.1.1).

servingCellId The ID of the serving cell for which the slotFormatCombinations are applicable.

slotFormatCombinations A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see TS 38.211 [16], clause 4.3.2). The total number of slotFormats in the slotFormatCombinations list does not exceed 512.

subcarrierSpacing2 Reference subcarrier spacing for a Slot Format Combination on an FDD or SUL cell (see TS 38.213 [13], clause 11.1.1). For FDD, subcarrierSpacing (SFI-scs) is the reference SCS for DL BWP and subcarrierSpacing2 (SFI-scs2) is the reference SCS for UL BWP. For SUL, subcarrierSpacing (SFI-scs) is the reference SCS for non-SUL carrier and subcarrierSpacing2 (SFI-scs2) is the reference SCS for SUL carrier. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications.

subcarrierSpacing Reference subcarrier spacing for this Slot Format Combination. The network configures a value that is smaller than or equal to any SCS of configured BWPs of the serving cell that the command applies to. And the network configures a value that is smaller than or equal to the SCS of the serving cell which the UE monitors for SFI indications (see TS 38.213 [13], clause 11.1.1).

#### –	SlotFormatIndicator

The IE SlotFormatIndicator is used to configure monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI).

SlotFormatIndicator information element

-- ASN1START

-- TAG-SLOTFORMATINDICATOR-START

SlotFormatIndicator ::=     SEQUENCE {

sfi-RNTI                    RNTI-Value,

dci-PayloadSize             INTEGER (1..maxSFI-DCI-PayloadSize),

slotFormatCombToAddModList  SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell

OPTIONAL, -- Need N

slotFormatCombToReleaseList SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex                       OPTIONAL, -- Need N

...,

[[

availableRB-SetsToAddModList-r16  SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF AvailableRB-SetsPerCell-r16   OPTIONAL, -- Need N

availableRB-SetsToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex                 OPTIONAL, -- Need N

switchTriggerToAddModList-r16     SEQUENCE (SIZE(1..4)) OF SearchSpaceSwitchTrigger-r16                                   OPTIONAL, -- Need N

switchTriggerToReleaseList-r16    SEQUENCE (SIZE(1..4)) OF ServCellIndex                                                  OPTIONAL, -- Need N

co-DurationsPerCellToAddModList-r16 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF CO-DurationsPerCell-r16     OPTIONAL, -- Need N

co-DurationsPerCellToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF ServCellIndex              OPTIONAL  -- Need N

]],

[[

switchTriggerToAddModListSizeExt-r16   SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroupMinus4-r16)) OF

SearchSpaceSwitchTrigger-r16  OPTIONAL, -- Need N

switchTriggerToReleaseListSizeExt-r16  SEQUENCE (SIZE(1.. maxNrofAggregatedCellsPerCellGroupMinus4-r16)) OF

ServCellIndex                 OPTIONAL  -- Need N

]],

[[

co-DurationsPerCellToAddModList-r17 SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF CO-DurationsPerCell-r17 OPTIONAL  -- Need N

]]

}

CO-DurationsPerCell-r16 ::=   SEQUENCE {

servingCellId-r16             ServCellIndex,

positionInDCI-r16             INTEGER(0..maxSFI-DCI-PayloadSize-1),

subcarrierSpacing-r16         SubcarrierSpacing,

co-DurationList-r16           SEQUENCE (SIZE(1..64)) OF CO-Duration-r16

}

CO-DurationsPerCell-r17 ::=   SEQUENCE {

servingCellId-r17             ServCellIndex,

positionInDCI-r17             INTEGER(0..maxSFI-DCI-PayloadSize-1),

subcarrierSpacing-r17         SubcarrierSpacing,

co-DurationList-r17           SEQUENCE (SIZE(1..64)) OF CO-Duration-r17

}

CO-Duration-r16 ::=    INTEGER (0..1120)

CO-Duration-r17 ::=    INTEGER (0..4480)

AvailableRB-SetsPerCell-r16 ::=   SEQUENCE {

servingCellId-r16                 ServCellIndex,

positionInDCI-r16                 INTEGER(0..maxSFI-DCI-PayloadSize-1)

}

SearchSpaceSwitchTrigger-r16 ::=   SEQUENCE {

servingCellId-r16                  ServCellIndex,

positionInDCI-r16                  INTEGER(0..maxSFI-DCI-PayloadSize-1)

}

-- TAG-SLOTFORMATINDICATOR-STOP

-- ASN1STOP

SlotFormatIndicator field descriptions

availableRB-SetsToAddModList A list of AvailableRB-SetsPerCell objects (see TS 38.213 [13], clause 11.1.1).

co-DurationsPerCellToAddModList A list of CO-DurationsPerCell objects. If not configured, the UE uses the slot format indicator (SFI), if available, to determine the channel occupancy duration (see TS 38.213 [13], clause 11.1.1).

co-DurationsPerCellToReleaseList A list of CO-DurationsPerCell objects to be released. An entry created using co-DurationsPerCellToAddModList-r16 or co-DurationsPerCellToAddModList-r17 can be deleted using co-DurationsPerCellToReleaseList.

dci-PayloadSize Total length of the DCI payload scrambled with SFI-RNTI (see TS 38.213 [13], clause 11.1.1).

sfi-RNTI RNTI used for SFI on the given cell (see TS 38.213 [13], clause 11.1.1).

slotFormatCombToAddModList A list of SlotFormatCombinations for the UE's serving cells (see TS 38.213 [13], clause 11.1.1).

switchTriggerToAddModList, switchTriggerToAddModListSizeExt A list of SearchSpaceSwitchTrigger objects. Each SearchSpaceSwitchTrigger object provides position in DCI of the bit field indicating search space switching flag for a serving cell or, if cellGroupsForSwitchList is configured, group of serving cells (see TS 38.213 [13], clause 10.4). If cellGroupsForSwitchList is configured, only one of the cells belonging to the same cell group is added/modified, and the configuration applies to all cells belonging to the cellGroupsForSwitchList (see TS 38.213 [13], clause 10.4). The network configures more than 4 SearchSpaceSwitchTrigger objects only if cellGroupsForSwitchList is not configured. The UE shall consider entries in switchTriggerToAddModList and in switchTriggerToAddModListSizeExt as a single list, i.e. an entry created using switchTriggerToAddModList can be modifed using switchTriggerToAddModListSizeExt and vice-versa.

switchTriggerToReleaseModList, switchTriggerToReleaseListSizeExt A list of SearchSpaceSwitchTriggers to be released. If cellGroupsForSwitchList is configured, the SearchSpaceSwitchTrigger is released for all serving cells belonging to the same CellGroupForSwitch. The UE shall consider entries in switchTriggerToReleaseList and in switchTriggerToReleaseListSizeExt as a single list, i.e. an entry created using switchTriggerToAddModList or switchTriggerToAddModListSizeExt can be deleted using switchTriggerToReleaseList or switchTriggerToReleaseListSizeExt.

AvailableRB-SetsPerCell field descriptions

positionInDCI The (starting) position of the bits within DCI payload indicating the availability of the RB sets of a serving cell (see TS 38.213 [13], clause 11.1.1).

servingCelIId The ID of the serving cell for which the configuration is applicable.

CO-DurationsPerCell field descriptions

co-DurationList A list of Channel Occupancy duration in symbols. The maximum duration that can be configured for the following SCS: - 15 kHz: 280. - 30 kHz: 560. - 60 kHz: 1120. - 120 kHz: 560. - 480 kHz: 2240. - 960 kHz: 4480.

positionInDCI Position in DCI of the bit field indicating Channel Occupancy duration for UE's serving cells (see TS 38.213 [13], clause 11.1.1).

servingCelIId The ID of the serving cell for which the configuration is applicable.

subcarrierSpacing Reference subcarrier spacing for the list of Channel Occupancy durations (see TS 38.213 [13], clause 11.1.1). Only the following values are applicable depending on the used frequency range: FR1: 15, 30, or 60 kHz FR2-2: 120, 480, or 960 kHz

SearchSpaceSwitchTrigger field descriptions

positionInDCI The position of the bit within DCI payload containing a search space switching flag (see TS 38.213 [13], clause 11.1.1).

servingCellId The ID of the serving cell for which the configuration is applicable or the group of serving cells as indicated by CellGroupsForSwitch-r16 containing this servingCellId.

#### –	S-NSSAI

The IE S-NSSAI (Single Network Slice Selection Assistance Information) identifies a Network Slice end to end and comprises a slice/service type and a slice differentiator, see TS 23.003 [21].

S-NSSAI information element

-- ASN1START

-- TAG-S-NSSAI-START

S-NSSAI  ::=                        CHOICE{

sst                                 BIT STRING (SIZE (8)),

sst-SD                              BIT STRING (SIZE (32))

}

-- TAG-S-NSSAI-STOP

-- ASN1STOP

S-NSSAI field descriptions

sst Indicates the S-NSSAI consisting of Slice/Service Type, see TS 23.003 [21].

sst-SD Indicates the S-NSSAI consisting of Slice/Service Type and Slice Differentiator, see TS 23.003 [21].

#### –	SpeedStateScaleFactors

The IE SpeedStateScaleFactors concerns factors, to be applied when the UE is in medium or high speed state, used for scaling a mobility control related parameter.

SpeedStateScaleFactors information element

-- ASN1START

-- TAG-SPEEDSTATESCALEFACTORS-START

SpeedStateScaleFactors ::=          SEQUENCE {

sf-Medium                           ENUMERATED {oDot25, oDot5, oDot75, lDot0},

sf-High                             ENUMERATED {oDot25, oDot5, oDot75, lDot0}

}

-- TAG-SPEEDSTATESCALEFACTORS-STOP

-- ASN1STOP

SpeedStateScaleFactors field descriptions

sf-High The concerned mobility control related parameter is multiplied with this factor if the UE is in High Mobility state as defined in TS 38.304 [20]. Value oDot25 corresponds to 0.25, value oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

sf-Medium The concerned mobility control related parameter is multiplied with this factor if the UE is in Medium Mobility state as defined in TS 38.304 [20]. Value oDot25 corresponds to 0.25, value oDot5 corresponds to 0.5, value oDot75 corresponds to 0.75, and so on.

#### –	SPS-Config

The IE SPS-Config is used to configure downlink semi-persistent transmission. Multiple Downlink SPS configurations may be configured in one BWP of a serving cell.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

SPS-Config ::=                  SEQUENCE {

periodicity                     ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1},

nrofHARQ-Processes              INTEGER (1..8),

n1PUCCH-AN                      PUCCH-ResourceId                                                            OPTIONAL,   -- Need M

mcs-Table                       ENUMERATED {qam64LowSE}                                                     OPTIONAL,   -- Need S

...,

[[

sps-ConfigIndex-r16             SPS-ConfigIndex-r16                                                         OPTIONAL,   -- Cond SPS-List

harq-ProcID-Offset-r16          INTEGER (0..15)                                                             OPTIONAL,   -- Need R

periodicityExt-r16              INTEGER (1..5120)                                                           OPTIONAL,   -- Need R

harq-CodebookID-r16             INTEGER (1..2)                                                              OPTIONAL,   -- Need R

pdsch-AggregationFactor-r16     ENUMERATED {n1, n2, n4, n8 }                                                OPTIONAL    -- Need S

]],

[[

sps-HARQ-Deferral-r17           INTEGER (1..32)                                                             OPTIONAL,   -- Need R

n1PUCCH-AN-PUCCHsSCell-r17      PUCCH-ResourceId                                                            OPTIONAL,   -- Need R

periodicityExt-r17              INTEGER (1..40960)                                                          OPTIONAL,   -- Need R

nrofHARQ-Processes-v1710        INTEGER(9..32)                                                              OPTIONAL,   -- Need R

harq-ProcID-Offset-v1700        INTEGER (16..31)                                                            OPTIONAL    -- Need R

]]

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

SPS-Config field descriptions

harq-CodebookID Indicates the HARQ-ACK codebook index for the corresponding HARQ-ACK codebook for SPS PDSCH and ACK for SPS PDSCH release.

harq-ProcID-Offset Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.3.1.

mcs-Table Indicates the MCS table the UE shall use for DL SPS (see TS 38.214 [19],clause 5.1.3.1. If present, the UE shall use the MCS table of low-SE 64QAM table indicated in Table 5.1.3.1-3 of TS 38.214 [19]. If this field is absent and field mcs-table in PDSCH-Config is set to 'qam256' and the activating DCI is of format 1_1, the UE applies the 256QAM table indicated in Table 5.1.3.1-2 of TS 38.214 [19]. If this field is absent and the field mcs-Table-r17 in PDSCH-Config is set to 'qam1024' and the activating DCI is format 1_1, the UE applies the 1024QAM table indicated in Table 5.1.3.1-4 of TS 38.214 [19]. Otherwise, the UE applies the non-low-SE 64QAM table indicated in Table 5.1.3.1-1 of TS 38.214 [19].

n1PUCCH-AN HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. The actual PUCCH-Resource is configured in PUCCH-Config and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

n1PUCCH-AN-PUCCHsSCell HARQ resource for PUCCH on PUCCH switching SCell (sSCell) for DL SPS. The network configures the resource either as format 0 or format 1. The actual PUCCH-Resource is configured in PUCCH-Config of the PUCCH sSCell and referred to by its ID. See TS 38.213 [13], clause 9.2.3.

nrofHARQ-Processes Number of configured HARQ processes for SPS DL (see TS 38.321 [3], clause 5.8.1). If UE is configured with nrofHARQ-Processes-v1710 UE shall ignore nrofHARQ-Processes (without suffix).

pdsch-AggregationFactor Number of repetitions for SPS PDSCH (see TS 38.214 [19], clause 5.1.2.1). When the field is absent, the UE applies the value 1 for MBS multicast data and the pdsch-AggregationFactor in pdsch-Config for other data.

periodicity Periodicity for DL SPS (see TS 38.214 [19] and TS 38.321 [3], clause 5.8.1).

periodicityExt This field is used to calculate the periodicity for DL SPS (see TS 38.214 [19] and see TS 38.321 [3], clause 5.8.1). If this field is present, the field periodicity is ignored. The following periodicities are supported depending on the configured subcarrier spacing [ms]: 15 kHz: periodicityExt, where periodicityExt has a value between 1 and 640. 30 kHz: 0.5 x periodicityExt, where periodicityExt has a value between 1 and 1280. 60 kHz with normal CP. 0.25 x periodicityExt, where periodicityExt has a value between 1 and 2560. 60 kHz with ECP: 0.25 x periodicityExt, where periodicityExt has a value between 1 and 2560. 120 kHz: 0.125 x periodicityExt, where periodicityExt has a value between 1 and 5120. 480 kHz: 0.03125 x periodicityExt, where periodicityExt has a value between 1 and 20480. 960 kHz: 0.015625 x periodicityExt, where periodicityExt has a value between 1 and 40960. periodicityExt-r17 is only applicable for SCS 480 kHz and 960 kHz.

sps-ConfigIndex Indicates the index of one of multiple SPS configurations.

sps-HARQ-Deferral Indicates the maximum number of slots or subslots the transmission of DL SPS HARQ-ACK in a slot or subslot can be deferred (see TS 38.213 [13], clause 9.2.5.4).

| Conditional Presence | Explanation |
| --- | --- |
| SPS-List | The field is mandatory present when included in sps-ConfigToAddModList-r16 or sps-ConfigMulticastToAddModList-r17, otherwise the field is absent. |

#### –	SPS-ConfigIndex

The IE SPS-ConfigIndex is used to indicate the index of one of multiple DL SPS configurations in one BWP.

SPS-ConfigIndex information element

-- ASN1START

-- TAG-SPS-CONFIGINDEX-START

SPS-ConfigIndex-r16             ::= INTEGER (0.. maxNrofSPS-Config-1-r16)

-- TAG-SPS-CONFIGINDEX-STOP

-- ASN1STOP

#### –	SPS-PUCCH-AN

The IE SPS-PUCCH-AN is used to indicate a PUCCH resource for HARQ ACK and configure the corresponding maximum payload size for the PUCCH resource.

SPS-PUCCH-AN information element

-- ASN1START

-- TAG-SPS-PUCCH-AN-START

SPS-PUCCH-AN-r16  ::=           SEQUENCE {

sps-PUCCH-AN-ResourceID-r16     PUCCH-ResourceId,

maxPayloadSize-r16              INTEGER (4..256)                     OPTIONAL    -- Need R

}

-- TAG-SPS-PUCCH-AN-STOP

-- ASN1STOP

SPS-PUCCH-AN field descriptions

maxPayloadSize Indicates the maximum payload size for the corresponding PUCCH resource ID.

sps-PUCCH-AN-ResourceID Indicates the PUCCH resource ID

#### –	SPS-PUCCH-AN-List

The IE SPS-PUCCH-AN-List is used to configure the list of PUCCH resources per HARQ ACK codebook

SPS-PUCCH-AN-List information element

-- ASN1START

-- TAG-SPS-PUCCH-AN-LIST-START

SPS-PUCCH-AN-List-r16 ::=      SEQUENCE (SIZE(1..4)) OF SPS-PUCCH-AN-r16

-- TAG-SPS-PUCCH-AN-LIST-STOP

-- ASN1STOP

#### –	SRB-Identity

The IE SRB-Identity is used to identify a Signalling Radio Bearer (SRB) used by a UE.

SRB-Identity information element

-- ASN1START

-- TAG-SRB-IDENTITY-START

SRB-Identity ::=                    INTEGER (1..3)

SRB-Identity-v1700 ::=              INTEGER (4)

SRB-Identity-v1800 ::=              INTEGER (5)

SRB-Identity-v1900 ::=              INTEGER (6)

-- TAG-SRB-IDENTITY-STOP

-- ASN1STOP

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure SRS carrier switching when PUSCH is not configured or to configure independent SRS power control from that of PUSCH.

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=            SEQUENCE {

srs-SwitchFromServCellIndex         INTEGER (0..31)                                                         OPTIONAL,   -- Need M

srs-SwitchFromCarrier               ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group                 CHOICE {

typeA                               SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB                               SRS-TPC-PDCCH-Config

}                                                                                                           OPTIONAL,   -- Need M

monitoringCells                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex               OPTIONAL,   -- Need M

...

}

SRS-TPC-PDCCH-Config ::=            SEQUENCE {

srs-CC-SetIndexlist                 SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex                                OPTIONAL    -- Need M

}

SRS-CC-SetIndex ::=                 SEQUENCE {

cc-SetIndex                         INTEGER (0..3)                                                          OPTIONAL,   -- Need M

cc-IndexInOneCC-Set                 INTEGER (0..7)                                                          OPTIONAL    -- Need M

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

SRS-CC-SetIndex field descriptions

cc-IndexInOneCC-Set Indicates the CC index in one CC set for Type A (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA.

cc-SetIndex Indicates the CC set index for Type A associated (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network always includes this field when the srs-TPC-PDCCH-Group is set to typeA. The network does not configure this field to 3 in this release of specification.

SRS-CarrierSwitching field descriptions

monitoringCells A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4).

srs-SwitchFromCarrier The UE ignores this field in case of independent SRS power control from that of PUSCH.

srs-SwitchFromServCellIndex Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less SCell. During SRS transmission on a PUSCH-less SCell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less SCell to transmit SRS. (see TS 38.214 [19], clause 6.2.1.3).

srs-TPC-PDCCH-Group Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.

typeA Type A trigger configuration for SRS transmission on a PUSCH-less SCell, or for SRS transmission on a serving cell with independent SRS power control from that of PUSCH (see TS 38.213 [13], clause 11.4). In this release, the network configures at most one entry (the first entry) of typeA, and the first entry corresponds to the serving cell in which the SRS-CarrierSwitching field is configured. SRS carrier switching to SUL carrier is not supported in this version of the specification.

typeB Type B trigger configuration for SRS transmission on a PUSCH-less SCell, or for SRS transmission on a serving cell with independent SRS power control from that of PUSCH (see TS 38.213 [13], clause 11.4).

SRS-TPC-PDCCH-Config field descriptions

srs-CC-SetIndexlist A list of pairs of [cc-SetIndex; cc-IndexInOneCC-Set] (see TS 38.212 [17], TS 38.213 [13], clause 7.3.1, 11.4). The network does not configure this field for typeB.

#### –	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI). The network does not configure SRS specific power control parameters alpha (without suffix) or pathlossReferenceRS if unifiedTCI-StateType is configured for the serving cell.

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId              OPTIONAL, -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource                OPTIONAL, -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                                   OPTIONAL, -- Need S

...,

[[

srs-RequestDCI-1-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16  OPTIONAL  -- Need N

]],

[[

dci-TriggeringPosResourceSetLink-r18    ENUMERATED { enabled }                                                  OPTIONAL  -- Need R

]],

[[

srs-TwoSeparatePowerControlAdjustmentStates-r19   ENUMERATED {enabled}                                          OPTIONAL, -- Need R

tpc-OfSRS-ClosedLoopIndexInDCI-1-1-r19            ENUMERATED {enabled}                                          OPTIONAL, -- Need R

srs-ClosedLoopIndexIndicatorInDCI-1-1-r19         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]]

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]],

[[

usagePDC-r17                            ENUMERATED {true}                                              OPTIONAL, -- Need R

availableSlotOffsetList-r17             SEQUENCE (SIZE(1..4)) OF AvailableSlotOffset-r17               OPTIONAL, -- Need R

followUnifiedTCI-StateSRS-r17           ENUMERATED {enabled}                                           OPTIONAL  -- Need R

]],

[[

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL  -- Cond FollowUTCI

]],

[[

symbolType-r19                          ENUMERATED {sbfd, non-sbfd}                                    OPTIONAL, -- Need R

associatedCSI-RS-Set-r19                NZP-CSI-RS-ResourceSetId                                       OPTIONAL, -- Need R

srs-PortGrouping-r19                    ENUMERATED {enabled}                                           OPTIONAL, -- Need R

fourPortSRS-3Tx-r19                     ENUMERATED {enabled}                                           OPTIONAL  -- Need R

]]

}

AvailableSlotOffset-r17 ::=   INTEGER (0..7)

PathlossReferenceRS-Config ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

PathlossReferenceRSList-r16 ::=             SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRS-r16)) OF PathlossReferenceRS-r16

PathlossReferenceRS-r16 ::=                 SEQUENCE {

srs-PathlossReferenceRS-Id-r16              SRS-PathlossReferenceRS-Id-r16,

pathlossReferenceRS-r16                     PathlossReferenceRS-Config

}

SRS-PathlossReferenceRS-Id-r16 ::=          INTEGER (0..maxNrofSRS-PathlossReferenceRS-1-r16)

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-PosResourceSetId-r16 ::=            INTEGER (0..maxNrofSRS-PosResourceSets-1-r16)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                           OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                        OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                                      OPTIONAL    -- Need R

]],

[[

spatialRelationInfo-PDC-r17             SetupRelease { SpatialRelationInfo-PDC-r17 }                   OPTIONAL,   -- Need M

resourceMapping-r17                     SEQUENCE {

startPosition-r17                       INTEGER (0..13),

nrofSymbols-r17                         ENUMERATED {n1, n2, n4, n8, n10, n12, n14},

repetitionFactor-r17                    ENUMERATED {n1, n2, n4, n5, n6, n7, n8, n10, n12, n14}

}                                                                                                      OPTIONAL,   -- Need R

partialFreqSounding-r17                 SEQUENCE {

startRBIndexFScaling-r17                CHOICE{

startRBIndexAndFreqScalingFactor2-r17   INTEGER (0..1),

startRBIndexAndFreqScalingFactor4-r17   INTEGER (0..3)

},

enableStartRBHopping-r17                ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

transmissionComb-n8-r17                 SEQUENCE {

combOffset-n8-r17                       INTEGER (0..7),

cyclicShift-n8-r17                      INTEGER (0..5)

}                                                                                                      OPTIONAL,   -- Need R

srs-TCI-State-r17                       CHOICE {

srs-UL-TCI-State                        TCI-UL-StateId-r17,

srs-DLorJointTCI-State                  TCI-StateId

}                                                                                                      OPTIONAL    -- Need R

]],

[[

repetitionFactor-v1730                  ENUMERATED {n3}                                                OPTIONAL,   -- Need R

srs-DLorJointTCI-State-v1730            SEQUENCE {

cellAndBWP-r17                          ServingCellAndBWP-Id-r17

}                                                                                                 OPTIONAL -- Cond DLorJointTCI-SRS

]],

[[

nrofSRS-Ports-n8-r18                    ENUMERATED {ports8, ports8tdm}                                 OPTIONAL,   -- Need R

combOffsetHopping-r18                   SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n4                       BIT STRING (SIZE (4)),

transmissionComb-n8                       BIT STRING (SIZE (8))

}                                                                                                  OPTIONAL,   -- Need R

hoppingWithRepetition-r18               ENUMERATED {symbol, repetition}                            OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

cyclicShiftHopping-r18                  SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n2                       BIT STRING (SIZE (8)),

transmissionComb-n4                       BIT STRING (SIZE (12)),

transmissionComb-n8                       BIT STRING (SIZE (6))

}                                                                                                  OPTIONAL,   -- Need R

hoppingFinerGranularity-r18             ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL    -- Need R

]]

}

SRS-PosResource-r16::=                  SEQUENCE {

srs-PosResourceId-r16                   SRS-PosResourceId-r16,

transmissionComb-r16                    CHOICE {

n2-r16                                  SEQUENCE {

combOffset-n2-r16                       INTEGER (0..1),

cyclicShift-n2-r16                      INTEGER (0..7)

},

n4-r16                                  SEQUENCE {

combOffset-n4-r16                        INTEGER (0..3),

cyclicShift-n4-r16                      INTEGER (0..11)

},

n8-r16                                  SEQUENCE {

combOffset-n8-r16                       INTEGER (0..7),

cyclicShift-n8-r16                      INTEGER (0..5)

},

...

},

resourceMapping-r16                       SEQUENCE {

startPosition-r16                           INTEGER (0..13),

nrofSymbols-r16                             ENUMERATED {n1, n2, n4, n8, n12}

},

freqDomainShift-r16                       INTEGER (0..268),

freqHopping-r16                           SEQUENCE {

c-SRS-r16                                 INTEGER (0..63),

...

},

groupOrSequenceHopping-r16                ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType-r16                          CHOICE {

aperiodic-r16                             SEQUENCE {

slotOffset-r16                            INTEGER (1..32)                                      OPTIONAL,   -- Need S

...

},

semi-persistent-r16                       SEQUENCE {

periodicityAndOffset-sp-r16               SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-sp-Ext-r16           SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]],

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                             OPTIONAL     --Need R

]]

},

periodic-r16                              SEQUENCE {

periodicityAndOffset-p-r16                SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-p-Ext-r16            SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]],

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                             OPTIONAL     --Need R

]]

}

},

sequenceId-r16                            INTEGER (0..65535),

spatialRelationInfoPos-r16                SRS-SpatialRelationInfoPos-r16                               OPTIONAL,   -- Need R

...,

[[

txHoppingConfig-r18                       TxHoppingConfig-r18                                          OPTIONAL    --Need R

]]

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                                      OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PosResourceId-r16 ::=               INTEGER (0..maxNrofSRS-PosResources-1-r16)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}

SRS-PeriodicityAndOffsetExt-r16 ::=     CHOICE {

sl128                                   INTEGER(0..127),

sl256                                   INTEGER(0..255),

sl512                                   INTEGER(0..511),

sl20480                                 INTEGER(0..20479)

}

SpatialRelationInfo-PDC-r17 ::=   SEQUENCE {

referenceSignal                   CHOICE {

ssb-Index                         SSB-Index,

csi-RS-Index                      NZP-CSI-RS-ResourceId,

dl-PRS-PDC                        NR-DL-PRS-ResourceID-r17,

srs                               SEQUENCE {

resourceId                        SRS-ResourceId,

uplinkBWP                         BWP-Id

},

...

},

...

}

TxHoppingConfig-r18 ::=             SEQUENCE {

overlapValue-r18                    ENUMERATED {zeroRB, oneRB, twoRB, fourRB},

numberOfHops-r18                    INTEGER(1..6),

slotOffsetForRemainingHopsList-r18  SEQUENCE (SIZE (1..maxNrofHops-1-r18) ) OF SlotOffsetForRemainingHops-r18,

...

}

SlotOffsetForRemainingHops-r18 ::=  SEQUENCE {

slotOffsetRemainingHops-r18         CHOICE {

aperiodic-r18                       SEQUENCE {

slotOffset-r18                      INTEGER (1..32)                                            OPTIONAL,   -- Need S

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

semi-persistent-r18                 SEQUENCE {

periodicityAndOffset-sp-r18         SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-sp-Ext-r18     SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

periodic-r18                        SEQUENCE {

periodicityAndOffset-p-r18          SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-p-Ext-r18      SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need S

...

},

...

}

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

dci-TriggeringPosResourceSetLink Indicates whether the single DCI-triggering SRS positioning resource sets across the linked carriers is enabled or not for bandwidth aggregation.

srs-ClosedLoopIndexIndicatorInDCI-1-1 Enables the presence of 1-bit SRS closed loop index indicator in DCI format 1_1 (see TS 38.212 [17], clause 7.3.1). This field is only present if srs-TwoSeparatePowerControlAdjustmentStates is configured.

srs-TwoSeparatePowerControlAdjustmentStates Indicates that two separate SRS power control adjustment states are configured (see TS 38.213 [13], clause 7.3.1).

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

tpc-OfSRS-ClosedLoopIndexInDCI-1-1 Enables the presence of 2-bit TPC command for separate SRS closed loop adjustment state(s) in DCI format 1_1 (see TS 38.212 [17], clause 7.3.1).

SRS-Resource, SRS-PosResource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n8 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

combOffsetHopping Configures UE with comb offset hopping. The hoppingId is used to initialize pseudo random comb offset hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingWithRepetition configures time-domain hopping behavior for repetition factor R>1. The hoppingSubset indicates a set of comb offset by a bit string (see clause 6.4.1.4.3 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scohSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

cyclicShiftHopping Configures UE with cyclic shift hopping. The hoppingId is used to initialize pseudo random cyclic shift hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingFinerGranularity enables finer granular hopping, see TS 38.211 [16], clause 6.4.1.4.2. If hoppingSubset is configured, hoppingFinerGranularity is not configured. The hoppingSubset indicates a set of cyclic shift by a bit string (see clause 6.4.1.4.2 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scshSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

enableStartRBHopping When this RRC parameter is configured, start RB location hopping is enabled for partial frequency sounding in different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS as described in clause 6.4.1.4 in TS 38.211.

fourPortSRS-3Tx Indicates whether port 1003 is disabled for all SRS resources in the SRS resource set with usage set to codebook or antennaSwitching, or whether 3Tx transmission is enabled for a configured SRS resource set with usage set to nonCodebook. This field can only be configured if nrofSRS-Ports for each SRS resources in the SRS resource set is set to ports4.

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area. c-SRS Indicates the maximum bandwidth per hop. When TxHoppingConfig is configured for RedCap UE, the valid values for c-SRS are such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively. The same value for c-SRS is configured for all the hops when TxHoppingConfig is configured.

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). For CLI SRS-RSRP measurement, the network always configures this parameter to 'neither'. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area.

nrofSRS-Ports Number of ports. For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.

nrofSRS-Ports-n8 Number of ports if the number of antenna ports is 8. The value 'ports8' configures UE with 8 antenna ports and the value 'ports8tdm' configures the UE with 8 antenna ports which are partitioned into 2 subsets with each subset having 4 different ports, and the subsets are mapped to different OFDM symbols, see TS 38.211 [16], clause 6.4.1.4.2. If combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured, this field is not set to ports8tdm. If this field is present UE ignores the field nrofSRS-Ports.

periodicityAndOffset-p, periodicityAndOffset-p-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, sl1280 and sl2560 cannot be configured. For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz except when periodicity of 20480ms is configured. When periodicityAndOffset-p-Ext is present, periodicityAndOffset-p shall be ignored by the UE.

periodicityAndOffset-sp, periodicityAndOffset-sp-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz. When periodicityAndOffset-sp-Ext is present, periodicityAndOffset-sp shall be ignored by the UE.

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). This parameter is not applicable to CLI SRS-RSRP measurement.

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). If resourceMapping-r17 is signalled, resourceMapping-r16 is not signalled and the UE shall ignore the resourceMapping (without suffix) and only the values of nrofSymbols which are integer multiples of the configured repetitionFactor can be configured. The network can only signal repetitionFactor-v1730 if resourceMapping-r17 is signalled. When repetitionFactor-v1730 is signalled, the UE shall ignore repetitionFactor-r17. For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area. nrofSymbols is same for all the hops when TxHoppingConfig is configured.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource, or slot offset for aperiodic SRS resource for positioning (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, only 'periodic' is applicable for resourceType. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-PosResource. If the field is absent the UE applies no offset (value 0).

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1). This parameter is not applicable to CLI SRS-RSRP measurement. This field is not configured if unifiedTCI-StateType is configured for the serving cell.

spatialRelationInfo-PDC Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS-PDC (see TS 38.214 [19], clause 6.2.1). The field is present in case of resourceType=periodic and usagePDC-r17=true in the SRS-ResourceSet, otherwise the field is absent.

spatialRelationInfoPos Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS (see TS 38.214 [19], clause 6.2.1). If the IE srs-ResourceId-Ext is present, the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 63. Otherwise the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 31. If the SRS for positioning is configured for RRC_INACTIVE state, the srs-ResourceId and csi-RS-IndexServing are not included in this field.

srs-PosPeriodicConfigHyperSFN-Index Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480m. If this field is not configured, the UE assumes that SRS for positioning periodictity longer than one Hyper SFN is not configured.

srs-RequestDCI-0-2 Indicate the number of bits for "SRS request" in DCI format 0_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 0_2 is applied. If the parameter srs-RequestDCI-0-2 is configured to value 1, 1 bit is used to indicate one of the first two rows of Table 7.3.1.1.2-24 in TS 38.212 [17] for triggered aperiodic SRS resource set. If the value 2 is configured, 2 bits are used to indicate one of the rows of Table 7.3.1.1.2-24 in TS 38.212 [17]. When UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication.

srs-RequestDCI-1-2 Indicate the number of bits for "SRS request" in DCI format 1_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 1_2 is applied. When the UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication (see TS 38.214 [19], clause 6.1.1.2).

srs-ResourceSetToAddModListDCI-0-2 List of SRS resource set to be added or modified for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-ResourceSetToReleaseListDCI-0-2 List of SRS resource set to be released for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-TCI-State Configuration of either a UL TCI state or a joint TCI state for the SRS resource. In case of UL TCI-State, refers to the TCI state defined in ul-TCI-StateList in the BWP-UplinkDedicated where the SRS-Config is configured. In case of joint TCI state, refers to a TCI state defined in dl-OrJointTCI-StateList in pdsch-Config of the BWP-DownlinkDedicated and serving cell indicated by cellAndBWP. This field is absent when the SRS resource is in an SRS-ResourceSet configured with followUnifiedTCI-StateSRS-r17 or applyIndicatedTCI-State, or when the field unifiedTCI-StateType is not configured to the serving cell which the SRS resource is located in.

startRBIndexAndFreqScalingFactor Configures the UE with the startRBIndex and freqScalingFactor for partial frequency sounding as described in Clause 6.4.1.4 in TS 38.211. The startRBIndexForFScaling2 gives the startRBIndex when freqScalingFactor is 2 and the startRBIndexForFScaling4 gives the startRBIndex when FreqScalingFactor is 4

transmissionComb, transmissionComb-n2, transmissionComb-n4, transmissionComb-n8 Comb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1). If network configures field transmissionComb-n8, the UE ignores transmissionComb. If srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

SRS-ResourceSet, SRS-PosResourceSet field descriptions

alpha alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6).

applyIndicatedTCI-State This field indicates, for an SRS-ResourceSet, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.2.1.

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

associatedCSI-RS-Set ID of CSI-RS resource set associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

availableSlotOffsetList Indicates a list of up to four different available slot offset values from slot n+k to the slot where the aperiodic SRS resource set is transmitted, where slot n is the slot with the triggering DCI, and k is the slotOffset (without suffix) as described in clause 6.2.1 of TS 38.214 [19].

csi-RS ID of CSI-RS resource associated with this SRS resource set (see TS 38.214 [19], clause 6.1.1.2).

dl-PRS This field indicates a PRS configuration.

followUnifiedTCI-StateSRS When set to enabled, for SRS resource Set, the UE applies the "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. This parameter may be configured for aperiodic SRS for BM or SRS of any time-domain behavior for codebook, non-codebook, and antenna switching.

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS-Pos A reference signal (e.g. a SS block or a DL-PRS config) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRSList Multiple candidate pathloss reference RS(s) for SRS power control, where one candidate RS can be mapped to SRS Resource Set via MAC CE (clause 6.1.3.27 in TS 38.321 [3]). The network can only configure this field if pathlossReferenceRS is not configured in the same SRS-ResourceSet.

resourceType Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS. The aperiodic SRS is not applicable for the UE in RRC_INACTIVE. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0).

srs-PortGrouping If configured, it indicates that SRS port grouping is enabled. This field can be configured only if reportQuantity is set to cri-RI-CQI and the usage of the SRS resource set is set to antennaSwitching.

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3).

srs-ResourceIdList, srs-PosResourceIdList The IDs of the SRS-Resources/SRS-PosResource used in this SRS-ResourceSet/SRS-PosResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceIdList is commonly configured across cells within the validity area.

srs-ResourceSetId, srs-PosResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceSetId is commonly configured across cells within the validity area.

ssb-IndexServing Indicates SSB index belonging to a serving cell where the SRS is configured.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

symbolType Configures the valid symbol type for SRS resources in the SRS resource set. The network does not configure this field if usage set to 'antennaSwitching’ (see TS 38.214, clause 6).

usage Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported.

usagePDC If configured, it indicates that this SRS resource set is used for propagation delay compensation. The field can be present in only one SRS-ResourceSet.

SRS-SpatialRelationInfoPos field descriptions

csi-RS-IndexServing Indicates CSI-RS index belonging to a serving cell.

dl-PRS This field indicates a PRS configuration.

resourceSelection Indicates whether the configured SRS spatial relation resource is a SRS-Resource or SRS-PosResource.

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

ssb-IndexServing Indicates SSB index belonging to a serving cell.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

SSB-InfoNCell field descriptions

physicalCellId This field specifies the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided.

ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell or NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.

ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId.

DL-PRS-Info field descriptions

dl-PRS-ID This field specifies the UE specific TRP ID (see TS 37.355 [49]) for which PRS configuration is provided.

dl-PRS-ResourceSetId This field specifies the PRS-ResourceSet ID of a PRS resourceSet.

dl-PRS-ResourceId This field specifies the PRS-Resource ID of a PRS resource. If this field is absent, the UE determines the dl-PRS-ResourceID based on its PRS measurement from the TRP (see TS 37.355 [49]) and DL-PRS Resource Set.

SSB-Configuration field descriptions

halfFrameIndex Indicates whether SSB is in the first half or the second half of the frame. Value zero indicates the first half and value 1 indicates the second half.

integerSubframeOffset Indicates the subframe boundary offset of the cell in which SSB is transmited.

sfn0-Offset Indiactes the time offset of the SFN0 slot 0 for the cell with respect to SFN0 slot 0 of serving cell.

sfn-Offset Specifies the SFN offset between the cell in which SSB is transmited and serving cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of serving cell to the beginning of the closest subsequent radio frame #0 of the cell in which SSB is transmitted.

sfn-SSB-Offset Indicates the SFN offset of the transmitted SSB relative to the start of the SSB period. Value 0 indicates that the SSB is transmitted in the first system frame, value 1 indicates that SSB is transmitted in the second system frame and so on. The network configures this field according to the field ssb-Periodicity such that the indicated system frame does not exceed the configured SSB periodicity.

ssb-Freq Indicates the frequency of the SSB.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity Indicates the periodicity of the SSB. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

TxHoppingConfig field descriptions

numberOfHops This field specifies the number of hops. Value 1 indicates one hop, value 2 indicates two hops and so on.

overlapValue This field specifies the overlap during SRS transmission in terms of number of resource blocks. Value zeroRB implies 0 RB, value oneRB corresponds to 1 RB, value twoRB corresponds to 2 RBs and so on. The same value for the overlapValue is configured to all the hops.

slotOffsetForRemainingHopsList This field specifies the starting slot offset and starting symbol for the SRS resource with tx hopping for different resource types (aperiodic, semi-persistent or periodic SRS transmission). Each hop is configured with the same periodicity.

| Conditional Presence | Explanation |
| --- | --- |
| DLorJointTCI-SRS | The field is mandatory present if srs-DLorJointTCI-State is configured, otherwise it is absent Need R. |
| FollowUTCI | The field is optionally present, Need R, if the field followUnifiedTCI-StateSRS is present. Otherwise, it is absent. |
| NonCodebook | This field is optionally present, Need M, in case of non-codebook based transmission, otherwise the field is absent. |
| Pathloss | The field is mandatory present if the IE SSB-InfoNcell is included in pathlossReferenceRS-Pos; otherwise it is optionally present, Need R |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optionally present, Need M, otherwise. |

#### –	SRS-PosTx-Hopping

The IE SRS-PosTx-Hopping specifies the uplink Tx frequency hopping configuration for SRS for positioning transmission.

SRS-PosTx-Hopping information element

-- ASN1START

-- TAG-SRS-PosTx-Hopping-START

SRS-PosTx-Hopping-r18 ::=                       SEQUENCE {

srs-PosConfig-r18                               SRS-PosConfig-r17,

bwp-r18                                         BWP                                                              OPTIONAL, -- Need R

inactivePosSRS-TimeAlignmentTimer-r18           TimeAlignmentTimer                                               OPTIONAL, -- Need M

inactivePosSRS-RSRP-ChangeThreshold-r18         RSRP-ChangeThreshold-r17                                         OPTIONAL, -- Need M

srs-PosUplinkTransmissionWindowConfig-r18       SetupRelease { SRS-PosUplinkTransmissionWindowConfig-r18 }       OPTIONAL, -- Need M

...

}

SRS-PosUplinkTransmissionWindowConfig-r18 ::=   SEQUENCE {

dummy                             INTEGER(0..1023),

windowPeriodicityAndOffset-r18                  CHOICE {

periodicityAndOffset-r18                        SRS-PeriodicityAndOffset-r16,

periodicityAndOffsetExt-r18                     SRS-PeriodicityAndOffsetExt-r16

},

duration-r18                                    ENUMERATED {sl1, sl2, sl4, sl6},

...

}

-- TAG-SRS-PosTx-Hopping-STOP

-- ASN1STOP

SRS-PosTx-Hopping field descriptions

bwp For RRC_CONNECTED state, it indicates the frequency region outside of active BWP for SRS for positioning frequency hopping. For RRC_INACTIVE state, it indicates the BWP configuration for SRS for positioning frequency hopping.

inactivePosSRS-RSRP-ChangeThreshold RSRP threshold for the increase/decrease of RSRP for time alignment validation as specified in TS 38.321 [3].

inactivePosSRS-TimeAlignmentTimer TAT value for SRS for positioning transmission during RRC_INACTIVE state as specified in TS 38.321 [3]. The network always configures this field when srs-PosRRC-Inactive is configured.

srs-PosConfig Provides the SRS configuration to be used for frequency hopping.

srs-PosUplinkTransmissionWindowConfig Used to setup and release the configuration of the periodic uplink transmission window for SRS for positioning with Tx frequency hopping in RRC_CONNECTED state.

SRS-PosUplinkTransmissionWindowConfig field descriptions

dummy This field is not used in the specification and the UE ignores the received value.

duration Indicates the duration of the uplink transmission window for SRS for positioning with Tx frequency hopping. Value sl1 indicates 1 slot, sl2 indicates 2 slots and so on.

periodicityAndOffset, periodicityAndOffsetExt Indicates the periodicity and slot offset with respect to SFN#0 slot#0 for uplink transmission window for SRS for positioning with Tx frequency hopping. The UL transmission window starts in the first symbol of the slot for which the following condition is satisfied: CURRENT_slot modulo (periodicity) = slot offset where CURRENT_slot = current SFN × number of slots per frame + slot number in the frame, and number of slots per frame refers to the number of consecutive slots per frame as specified in TS 38.211[16].

#### –	SRS-PosResourceSetLinkedForAggBW

The IE SRS-PosResourceSetLinkedForAggBW provides the SRS Positioning Resource Sets that are linked for bandwidth aggregation across two or three component carriers.

SRS-PosResourceSetLinkedForAggBW information element

-- ASN1START

-- TAG-SRS-POSRESOURCESETLINKEDFORAGGBW-START

SRS-PosResourceSetLinkedForAggBW-r18	::= SEQUENCE {

srs-PosResourceSetLinked-r18             SRS-PosResourceSetId-r16,

srs-LinkedConfig-r18                     CHOICE {

rrc-connectedState-r18                   SEQUENCE {

servingCellAndBWP-Id-r18                  ServingCellAndBWP-Id-r17,

...

},

rrc-inactiveState-r18                    SEQUENCE {

scs-SpecificCarrier-r18                   SCS-SpecificCarrier,

freqInfo-r18                             ARFCN-ValueNR,

...

}

},

...

}

-- TAG-SRS-POSRESOURCESETLINKEDFORAGGBW-STOP

-- ASN1STOP

SRS-PosResourceSetLinkedForAggBW field descriptions

freqInfo Indicates the SRS Positioning Resource set carrier frequency that is linked for bandwidth aggregation.

scs-SpecificCarrier A set of UE specific channel bandwidth and location configurations for different subcarrier spacings (numerologies). Defined in relation to Point A and to be used for SRS for positioning bandwidth aggregation.

servingCellAndBWP-Id Provides the serving cell and BWP config for SRS for positioning transmission using aggregated bandwidth in RRC_CONNECTED state.

srs-LinkedConfig Provides the linked bandwidth aggregation configuration for RRC_CONNECTED or RRC_INACTIVE state for the SRS for positioning transmission.

srs-PosResourceSetLinked Indicates the SRS Positioning Resource set that is linked for bandwidth aggregation.

#### –	SRS-RSRP-MeasResource

The IE SRS-RSRP-MeasResource specifies SRS-RSRP measurement resource based on SRS-Resource in SRS-Config, which the UE may be configured to measure L1 SRS-RSRP.

SRS-RSRP-MeasResource information element

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCE-START

SRS-RSRP-MeasResource-r19 ::=            SEQUENCE {

srs-RSRP-MeasResourceId-r19              SRS-RSRP-MeasResourceId-r19                                  OPTIONAL,   -- Need R

srs-Resource-r19                         SRS-Resource                                                 OPTIONAL,   -- Need R

qcl-InfoPeriodicSRS-RSRP-MeasResource    TCI-StateId                                                  OPTIONAL,   -- Need R

...

}

-- TAG-SRS-RSRP-MEASRESOURCE-STOP

-- ASN1STOP

SRS-RSRP-MeasResource field descriptions

qcl-InfoPeriodicSRS-RSRP-MeasResource Indicates a reference to one TCI-State in TCI-States for providing the QCL source and QCL type for a target periodic SRS-RSRP-MeasResource.

srs-Resource Indicates the SRS-RSRP measurement resources based on SRS-Resource configuration in SRS-Config.

srs-RSRP-MeasResourceId Identifies SRS-RSRP measurement resource.

#### –	SRS-RSRP-MeasResourceId

The IE SRS-RSRP-MeasResourceId is used to identify a SRS-RSRP-MeasResource.

SRS-RSRP-MeasResourceId information element

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCEID-START

SRS-RSRP-MeasResourceId-r19 ::=      INTEGER(0..maxNrofSRS-RSRP-MeasResources-1-r19)

-- TAG-SRS-RSRP-MEASRESOURCEID-STOP

-- ASN1STOP

#### –	SRS-RSRP-MeasResourceSet

The IE SRS-RSRP-MeasResourceSet specifies a set of SRS-RSRP measurement resources (their IDs) for L1 SRS-RSRP measurement and set-specific parameters.

SRS-RSRP-MeasResourceSet information element

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCESET-START

SRS-RSRP-MeasResourceSet-r19 ::=      SEQUENCE {

aperiodicTriggeringOffset-r19         INTEGER (1..31)                                                      OPTIONAL,   -- Need R

srs-RSRP-MeasResourceSetId-r19        SRS-RSRP-MeasResourceSetId-r19                                       OPTIONAL,   -- Need R

srs-RSRP-MeasResourceIdList-r19       SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourcesPerSet-r19) ) OF SRS-RSRP-MeasResourceId-r19

OPTIONAL,   -- Need R

...

}

-- TAG-SRS-RSRP-MEASRESOURCESET-STOP

-- ASN1STOP

SRS-RSRP-MeasResourceSet field descriptions

aperiodicTriggeringOffset Indicates a slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured.

srs-RSRP-MeasResourceIdList Indicates the SRS-RSRP measurement resources associated with this SRS-RSRP measurement resource set.

srs-RSRP-MeasResourceSetId Indicates ID of SRS-RSRP measurement resource set.

#### –	SRS-RSRP-MeasResourceSetId

The IE SRS-RSRP-MeasResourceSetId is used to identify a SRS-RSRP-MeasResourceSet.

SRS-RSRP-MeasResourceSetId information element

-- ASN1START

-- TAG-SRS-RSRP-MEASRESOURCESETID-START

SRS-RSRP-MeasResourceSetId-r19 ::=      INTEGER(0..maxNrofSRS-RSRP-MeasResourceSets-1-r19)

-- TAG-SRS-RSRP-MEASRESOURCESETID-STOP

-- ASN1STOP

#### –	SRS-RSRP-Range

The IE SRS-RSRP-Range specifies the value range used in SRS-RSRP measurements and thresholds. The integer value for SRS-RSRP measurements is according to Table 10.1.22.1.2-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value –140) dBm, except for the IE value 98, in which case the actual value is infinity.

SRS-RSRP-Range information element

-- ASN1START

-- TAG-SRS-RSRP-RANGE-START

SRS-RSRP-Range-r16 ::=                      INTEGER(0..98)

-- TAG-SRS-RSRP-RANGE-STOP

-- ASN1STOP

#### –	SRS-TPC-CommandConfig

The IE SRS-TPC-CommandConfig is used to configure the UE for extracting TPC commands for SRS from a group-TPC messages on DCI.

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL               INTEGER (1..31)                                                     OPTIONAL    -- Need R

]],

[[

startingBitOfFormat2-3-r1900            INTEGER (1..45)                                                     OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

SRS-TPC-CommandConfig field descriptions

fieldTypeFormat2-3 The type of a field within the group DCI with SRS request fields (optional), which indicates how many bits in the field are for SRS request (0 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 38.212 [17] clause 7.3.1 and , TS 38.213 [13], clause 11.4.

startingBitOfFormat2-3, startingBitOfFormat2-3-r1900 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4). The network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-v1900 simultaneously.

startingBitOfFormat2-3SUL The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for SUL carrier (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4).

#### –	SSB-Index

The IE SSB-Index identifies an SS-Block within an SS-Burst. See TS 38.213 [13], clause 4.1.

SSB-Index information element

-- ASN1START

-- TAG-SSB-INDEX-START

SSB-Index ::=                       INTEGER (0..maxNrofSSBs-1)

-- TAG-SSB-INDEX-STOP

-- ASN1STOP

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC5-r19 ::=             SEQUENCE {

pci-List-r19                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

periodicity-r19              ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}             OPTIONAL,  -- Need M

offset-r19                   INTEGER (0..159)                                                               OPTIONAL   -- Need M

}

SSB-MTC-AdditionalPCI-r17 ::=       SEQUENCE {

additionalPCIIndex-r17              AdditionalPCIIndex-r17,

additionalPCI-r17                   PhysCellId,

periodicity-r17                     ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

ssb-PositionsInBurst-r17            CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

},

ss-PBCH-BlockPower-r17              INTEGER (-60..50)

}

-- TAG-SSB-MTC-STOP

-- ASN1STOP

SSB-MTC field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

SSB-MTC2 field descriptions

pci-List PCIs that follow this SMTC.

SSB-MTC3 field descriptions

duration Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (see TS 38.213 [13], clause 4.1).

pci-List PCIs that follow this SMTC, used for IAB-node discovery.

periodicityAndOffset Periodicity and offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Periodicity and offset are given in number of subframes.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS block index 0, the second bit corresponds to SS block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS block is not to be measured while value 1 indicates that the corresponding SS block is to be measured (see TS 38.215 [9]). When the field is not configured the IAB-MT measures on all SS blocks. Regardless of the value of this field, SS blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

SSB-MTC4 field descriptions

pci-List PCIs that follow this SMTC.

offset Offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Offset is given in number of subframes.

SSB-MTC5 field descriptions

offset Offset of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10. Offset is given in number of subframes.

pci-List PCIs that follow this SMTC.

periodicity Periodicity of the measurement window in which to receive SS/PBCH blocks, see 5.5.2.10.

SSB-MTC-AdditionalPCI field descriptions

additionalPCI PCI of the additional SSB different from serving cell PCI.

periodicity Periodicity of the SS/PBCH blocks, see 5.5.2.10. Periodicity is given in number of subframes.

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

#### –	SSB-PositionQCL-Relation

The IE SSB-PositionQCL-Relation is used to indicate the QCL relationship between SSB positions on the frequency indicated by ssbFrequency (see TS 38.213 [13], clause 4.1) for operation with shared spectrum channel access. Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

SSB-PositionQCL-Relation information element

-- ASN1START

-- TAG-SSB-POSITIONQCL-RELATION-START

SSB-PositionQCL-Relation-r16 ::=  ENUMERATED {n1,n2,n4,n8}

SSB-PositionQCL-Relation-r17 ::=  ENUMERATED {n32, n64}

-- TAG-SSB-POSITIONQCL-RELATION-STOP

-- ASN1STOP

#### –	SSB-ToMeasure

The IE SSB-ToMeasure is used to configure a pattern of SSBs. For operation with shared spectrum channel access in FR1, only mediumBitmap is used, and for FR2-2, longBitmap is used.

SSB-ToMeasure information element

-- ASN1START

-- TAG-SSB-TOMEASURE-START

SSB-ToMeasure ::=                   CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}

-- TAG-SSB-TOMEASURE-STOP

-- ASN1STOP

SSB-ToMeasure field descriptions

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR2-2, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to (k– 1) may be transmitted; if the k-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, if the k-th bit is set to 1, the UE assumes that one or more SS/PBCH blocks within the SMTC measurement duration with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL-Common and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap. If ssb-PositionQCL is configured with a value smaller than ssb-PositionQCL-Common, only the leftmost K bits (K = ssb-PositionQCL) are applicable for the corresponding cell.

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

#### –	SS-RSSI-Measurement

The IE SS-RSSI-Measurement is used to configure RSSI measurements based on synchronization reference signals.

SS-RSSI-Measurement information element

-- ASN1START

-- TAG-SS-RSSI-MEASUREMENT-START

SS-RSSI-Measurement ::=             SEQUENCE {

measurementSlots                    BIT STRING (SIZE (1..80)),

endSymbol                           INTEGER(0..3)

}

-- TAG-SS-RSSI-MEASUREMENT-STOP

-- ASN1STOP

SS-RSSI-Measurement field descriptions

endSymbol Within a slot that is configured for RSSI measurements (see measurementSlots) the UE measures the RSSI from symbol 0 to symbol endSymbol. This field identifies the entry in Table 5.1.3-1 in TS 38.215 [9], which determines the actual end symbol.

measurementSlots Indicates the slots in which the UE can perform RSSI measurements. The length of the BIT STRING is equal to the number of slots in the configured SMTC window (determined by the duration and by the subcarrierSpacing). The first (left-most / most significant) bit in the bitmap corresponds to the first slot in the SMTC window, the second bit in the bitmap corresponds to the second slot in the SMTC window, and so on. The UE measures in slots for which the corresponding bit in the bitmap is set to 1. In case this field is configured for a SCell with ca-SlotOffset-r16, the bits in the bitmap corresponds to the slots that are fully contained in the SMTC window.

#### –	SubcarrierSpacing

The IE SubcarrierSpacing determines the subcarrier spacing. Restrictions applicable for certain frequencies, channels or signals are clarified in the fields that use this IE.

SubcarrierSpacing information element

-- ASN1START

-- TAG-SUBCARRIERSPACING-START

SubcarrierSpacing ::=               ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, kHz480-v1700, kHz960-v1700, spare1}

-- TAG-SUBCARRIERSPACING-STOP

-- ASN1STOP

#### –	T-Reselection

The IE T-Reselection concerns the cell reselection timer TreselectionRAT for NR and E-UTRA. Value in seconds. For value 0, behaviour as specified in 7.1.2 applies.

T-Reselection information element

-- ASN1START

-- TAG-TRESELECTION-START

T-Reselection ::=                   INTEGER (0..7)

-- TAG-TRESELECTION-STOP

-- ASN1STOP

#### –	TAG-Config

The IE TAG-Config is used to configure parameters for a time-alignment group.

TAG-Config information element

-- ASN1START

-- TAG-TAG-CONFIG-START

TAG-Config ::=                      SEQUENCE {

tag-ToReleaseList                   SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-Id                          OPTIONAL,   -- Need N

tag-ToAddModList                    SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG                             OPTIONAL    -- Need N

}

TAG ::=                             SEQUENCE {

tag-Id                              TAG-Id,

timeAlignmentTimer                  TimeAlignmentTimer,

...

}

TAG-Id ::=                          INTEGER (0..maxNrofTAGs-1)

-- TAG-TAG-CONFIG-STOP

-- ASN1STOP

TAG field descriptions

tag-Id Indicates the TAG of the SpCell or an SCell, see TS 38.321 [3]. Uniquely identifies the TAG within the scope of a Cell Group (i.e. MCG or SCG).

timeAlignmentTimer The timeAlignmentTimer for TAG with ID tag-Id, as specified in TS 38.321 [3].

#### –	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks and ATG network.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config-r17 ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15, spare13, spare12, spare11, spare10, spare9, spare8, spare7,

spare6, spare5, spare4, spare3, spare2, spare1}          OPTIONAL,    -- Need R

timingAdvanceSR-r17                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

TAR-Config-r18 ::=                      SEQUENCE {

offsetThresholdTA-r18               INTEGER (1..56)                                                     OPTIONAL,    -- Need R

timingAdvanceSR-r18                 ENUMERATED {enabled}                                                OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For offsetThresholdTA-r17, value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1 ms, and so on. Network only configures offsetThresholdTA-r18 for ATG, which is in unit of symbols based on the subcarrier spacing of the BWP indicated by firstActiveDownlinkBWP-Id if received in the same RRC message, otherwise, it is based on the subcarrier spacing of the active DL BWP of the PCell when offsetThresholdTA-r18 is received. Once configured, the value of TA offset threshold does not change autonomously by the UE with BWP switching.

timingAdvanceSR Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

#### –	TCI-ActivatedConfig

The IE TCI-ActivatedConfig is used to provide activated TCI states for PDSCH and/or PDCCH of the PSCell or of an SCell.

TCI-ActivatedConfig information element

-- ASN1START

-- TAG-TCI-ACTIVATEDCONFIG-START

TCI-ActivatedConfig-r17 ::= SEQUENCE {

pdcch-TCI-r17               SEQUENCE (SIZE (1..5)) OF TCI-StateId,

pdsch-TCI-r17               BIT STRING (SIZE (1..maxNrofTCI-States))

}

-- TAG-TCI-ACTIVATEDCONFIG-STOP

-- ASN1STOP

TCI-ActivatedConfig field descriptions

pdcch-TCI Indicates the TCI state for PDCCH for each configured CORESET of the DL BWP to be activated at SCell activation, to be activated for the PSCell at SCG activation and/or to be used for BFD, RLM and measurements while the SCG is deactivated. The list includes exactly as many entries as CORESETs configured in this BWP, ordered by increasing values of ControlResourceSet-Id, i.e. the first entry indicates the TCI state for the configured CORESET with the lowest ControlResourceset-Id value, the second value indicates the TCI states for the configured CORESET with the second lowest ControlResourceset-Id value, and so on.

pdsch-TCI Indicates TCI states for PDSCH reception at SCell addition/activation or of the PSCell at SCG activation. This field indicates activated TCI state(s) for this BWP ordered by increasing values of TCI-StateId, i.e. the first bit indicates the activation state of the TCI state with the lowest TCI-StateId value, the second value indicates the activation status of the TCI state with the second lowest TCI-State-Id value, and so on. A bit set to 0 indicates that the corresponding TCI state is deactivated, a bit set to 1 indicates that the TCI state is activated.

#### –	TCI-State

The IE TCI-State associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...,

[[

additionalPCI-r17                   AdditionalPCIIndex-r17                                      OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17          PathlossReferenceRS-Id-r17                                  OPTIONAL,   -- Cond JointTCI1

ul-powerControl-r17                 Uplink-powerControlId-r17                                   OPTIONAL    -- Cond JointTCI

]],

[[

tag-Id-ptr-r18                      ENUMERATED {n0,n1}                                          OPTIONAL    -- Cond 2TA

]],

[[

pathlossOffset-r19                  ENUMERATED {

dB-12, dB-8, dB-4, dB0, dB4, dB8,

dB12, dB16, dB20, dB24, dB28, dB32, dB36,

dB40, dB44, dB48, dB52, dB56, dB60}                     OPTIONAL    -- Need R

]]

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                                               OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                                                      OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

QCL-Info field descriptions

bwp-Id The DL BWP which the RS is located in. If the field is absent, the RS is located in the DL BWP in which the TCI-State is applied by the UE.

cell The UE's serving cell in which the referenceSignal is configured. If the field is absent, the referenceSignal is configured in the serving cell in which the TCI-State is applied by the UE. The RS can be located on a serving cell other than the serving cell for which the TCI-State is applied by the UE only if the qcl-Type is configured as typeC or typeD. If the referenceSignal is set to csi-rs and unifiedTCI-StateType is configured, either both cell and bwp-Id are present or both cell and bwp-Id are absent. See TS 38.214 [19] clause 5.1.5.

referenceSignal Reference signal with which quasi-collocation information is provided as specified in TS 38.214 [19] clause 5.1.5.

qcl-Type QCL type as specified in TS 38.214 [19] clause 5.1.5.

TCI-State field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs when referenceSignal is configured as SSB for both QCL-Type1 and QCL-Type2. In case the cell is present, the additionalPCI refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell indicated by the field cell. Otherwise, it refers to a PCI value configured in a list additionalPCI-ToAddModList configured in the serving cell where the TCI-State is applied by the UE. When this field is present the cell for qcl-Type1 and qcl-Type2 is configured with same value, if present.

pathlossOffset Indicates the pathloss offset applied to the UL only TCI or joint TCI state. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

pathlossReferenceRS-Id The ID of the reference signal (e.g. a CSI-RS or an SS block) used for PUSCH, PUCCH and SRS path loss estimation. This field refers to an element in the list configured using pathlossReferenceRSToAddModList in the serving cell and UL BWP where the TCI State is applied by the UE.

qcl-Type1, qcl-Type2 QCL information for the TCI state as specified in TS 38.214 [19] clause 5.1.5.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

tci-StateId ID number of the TCI state.

ul-PowerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the serving cell where the dl-OrJointTCI-StateToAddModList is configured. For each serving cell, ul-powerControl is either configured in each TCI-State configured or referred to in dl-OrJointTCI-StateList in each DL BWP, or it is not configured in any TCI-State configured or referred to in dl-OrJointTCI-StateList in any DL BWP.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present for the serving cell and the serving cell is configured with unifiedTCI-StateType set to joint. It is absent, Need R, otherwise. |
| CSI-RS-Indicated | This field is mandatory present if csi-rs is included and unifiedTCI-StateType is not configured. This field is optionally present, Need R, if csi-rs is included and unifiedTCI-StateType is configured. Otherwise, it is absent, Need R. |
| JointTCI | This field is optionally present, Need R, if this serving cell is configured with unifiedTCI-StateType set to 'joint'. It is absent, Need R, otherwise. |
| JointTCI1 | This field is mandatory present, if this serving cell is configured with unifiedTCI-StateType set to 'joint'. It is absent, Need R, otherwise. |

#### –	TCI-StateId

The IE TCI-StateId is used to identify one TCI-State configuration.

TCI-StateId information element

-- ASN1START

-- TAG-TCI-STATEID-START

TCI-StateId ::=                     INTEGER (0..maxNrofTCI-States-1)

-- TAG-TCI-STATEID-STOP

-- ASN1STOP

#### –	TCI-UL-State

The IE TCI-UL-State indicates the TCI state information for UL transmission.

TCI-UL-State information element

-- ASN1START

-- TAG-TCI-UL-STATE-START

TCI-UL-State-r17 ::=             SEQUENCE {

tci-UL-StateId-r17              TCI-UL-StateId-r17,

servingCellId-r17                ServCellIndex                                         OPTIONAL,   -- Need R

bwp-Id-r17                       BWP-Id                                                OPTIONAL,   -- Cond CSI-RSorSRS-Indicated

referenceSignal-r17              CHOICE {

ssb-Index-r17                    SSB-Index,

csi-RS-Index-r17                 NZP-CSI-RS-ResourceId,

srs-r17                          SRS-ResourceId

},

additionalPCI-r17                AdditionalPCIIndex-r17                                OPTIONAL,   -- Need R

ul-powerControl-r17              Uplink-powerControlId-r17                             OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17       PathlossReferenceRS-Id-r17                            OPTIONAL,   -- Cond Mandatory

...,

[[

tag-Id-ptr-r18                   ENUMERATED {n0,n1}                                    OPTIONAL    -- Cond 2TA

]],

[[

pathlossOffset-r19               ENUMERATED {

dB-12, dB-8, dB-4, dB0, dB4, dB8,

dB12, dB16, dB20, dB24, dB28, dB32, dB36,

dB40, dB44, dB48, dB52, dB56, dB60}               OPTIONAL    -- Need R

]]

}

-- TAG-TCI-UL-STATE-STOP

-- ASN1STOP

TCI-UL-State field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs when referenceSignal is configured as SSB. In case the servingCellId is present, the additionalPCI refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell indicated by the field servingCellId. Otherwise, it refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell where the ul-TCI-StateList is applied by the UE.

bwp-Id The DL BWP which the CSI-RS is located in or UL BWP where the SRS is located in.

pathlossOffset Indicates the pathloss offset applied to the UL only TCI or joint TCI state. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

pathlossReferenceRS-Id The ID of the reference Signal (e.g. a CSI-RS or a SS block) used for PUSCH, PUCCH and SRS path loss estimation. This field refers to an element in the list configured using pathlossReferenceRSToAddModList in the serving cell and UL BWP where the UL TCI State is applied by the UE.

servingCellId The UE's serving cell in which the referenceSignal is configured. If the field is absent, the referenceSignal is configured in the serving cell in which the TCI-UL-State is applied by the UE.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

ul-powerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the serving cell where the ul-TCI-ToAddModList is configured. For each serving cell, ul-powerControl is either configured in each TCI-UL-State configured or referred to in ul-OrJointTCI-StateList in each UL BWP, or it is not configured in any TCI-UL-State configured or referred to in ul-OrJointTCI-StateList in any UL BWP.

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatory present if tag2 is present for the serving cell. It is absent, Need R, otherwise. |
| CSI-RSorSRS-Indicated | This field is mandatory present if referenceSignal is set to csi-RS-index or to srs, absent otherwise |
| Mandatory | The field is mandatory present. |

#### –	TCI-UL-StateId

The IE TCI-UL-StateId is used to identify one TCI-UL-State configuration.

TCI-UL-StateId information element

-- ASN1START

-- TAG-TCI-UL-STATEID-START

TCI-UL-StateId-r17 ::=              INTEGER (0..maxUL-TCI-1-r17)

-- TAG-TCI-UL-STATEID-STOP

-- ASN1STOP

#### –	TDD-UL-DL-ConfigCommon

The IE TDD-UL-DL-ConfigCommon determines the cell specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigCommon information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGCOMMON-START

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                                               OPTIONAL -- Need R

]],

[[

sbfd-StartingSlotIndex-r19          INTEGER (0..maxNrofSlots-1)                                             OPTIONAL, -- Need R

sbfd-StartingSymbolIndex-r19        INTEGER (0..maxNrofSymbols-1)                                           OPTIONAL, -- Need R

sbfd-EndingSlotIndex-r19            INTEGER (0..maxNrofSlots-1)                                             OPTIONAL, -- Need R

sbfd-EndingSymbolIndex-r19          INTEGER (0..maxNrofSymbols-1)                                           OPTIONAL  -- Need R

]]

}

-- TAG-TDD-UL-DL-CONFIGCOMMON-STOP

-- ASN1STOP

TDD-UL-DL-ConfigCommon field descriptions

referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz The network configures a not larger than any SCS of configured BWPs for the serving cell. The network or SL-PreconfigGeneral configures a not larger than the SCS of (pre-)configured SL BWP.See TS 38.213 [13], clause 11.1.

TDD-UL-DL-Pattern field descriptions

dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see TS 38.213 [13], clause 11.1. If the dl-UL-TransmissionPeriodicity-v1530 is signalled, UE shall ignore the dl-UL-TransmissionPeriodicity (without suffix).

nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 320.

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see TS 38.213 [13], clause 11.1).

nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 320.

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see TS 38.213 [13], clause 11.1).

sbfd-StartingSlotIndex, sbfd-EndingSlotIndex Configures the starting slot index and the ending slot index of SBFD subbands within a TDD-UL-DL period.

sbfd-StartingSymbolIndex, sbfd-EndingSymbolIndex Configures the starting symbol index within the starting slot, the ending symbol index within the ending slot of SBFD subbands within a TDD-UL-DL period.

#### –	TDD-UL-DL-ConfigDedicated

The IE TDD-UL-DL-ConfigDedicated determines the UE-specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigDedicated information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGDEDICATED-START

TDD-UL-DL-ConfigDedicated ::=       SEQUENCE {

slotSpecificConfigurationsToAddModList      SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig       OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList     SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex        OPTIONAL, -- Need N

...

}

TDD-UL-DL-ConfigDedicated-IAB-MT-r16::=         SEQUENCE {

slotSpecificConfigurationsToAddModList-IAB-MT-r16  SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig-IAB-MT-r16      OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList-IAB-MT-r16 SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex                  OPTIONAL, -- Need N

...

}

TDD-UL-DL-SlotConfig ::=            SEQUENCE {

slotIndex                           TDD-UL-DL-SlotIndex,

symbols                             CHOICE {

allDownlink                         NULL,

allUplink                           NULL,

explicit                            SEQUENCE {

nrofDownlinkSymbols                 INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL, -- Need S

nrofUplinkSymbols                   INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotConfig-IAB-MT-r16::=    SEQUENCE {

slotIndex-r16                           TDD-UL-DL-SlotIndex,

symbols-IAB-MT-r16                      CHOICE {

allDownlink-r16                         NULL,

allUplink-r16                           NULL,

explicit-r16                            SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)                               OPTIONAL, -- Need S

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)                               OPTIONAL  -- Need S

},

explicit-IAB-MT-r16                     SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)                               OPTIONAL, -- Need S

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)                               OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotIndex ::=             INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIGDEDICATED-STOP

-- ASN1STOP

TDD-UL-DL-ConfigDedicated field descriptions

slotSpecificConfigurationsToAddModList The slotSpecificConfigurationToAddModList allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

TDD-UL-DL-ConfigDedicated-IAB-MT field descriptions

slotSpecificConfigurationsToAddModList-IAB-MT The slotSpecificConfigurationToAddModList-IAB-MT allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon with a limitation that effectively only flexible symbols can be overwritten in Rel-16.

slotSpecificConfigurationsToReleaseList-IAB-MT The slotSpecificConfigurationsToReleaseList-IAB-MT allows release of a set of slot configuration previously add with slotSpecificConfigurationToAddModList-IAB-MT.

TDD-UL-DL-SlotConfig field descriptions

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see TS 38.213 [13], clause 11.1).

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see TS 38.213 [13], clause 11.1).

slotIndex Identifies a slot within a slot configuration period given in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

symbols The direction (downlink or uplink) for the symbols in this slot. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively.

TDD-UL-DL-SlotConfig-IAB-MT field descriptions

symbols-IAB-MT The symbols-IAB-MT is used to configure an IAB-MT with the SlotConfig applicable for one serving cell. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively; value explicit-IAB-MT indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink, respectively.

#### –	ThresholdP-LR

The IE ThresholdLR is used to indicate a measured RSRP threshold for LP-WUS. Actual value of threshold = field value * 2 [dBm].

ThresholdP-LR information element

-- ASN1START

-- TAG-THRESHOLDP-LR-START

ThresholdP-LR-r19 ::=               INTEGER (-80..0)

-- TAG-THRESHOLDP-LR-STOP

-- ASN1STOP

#### –	ThresholdQ-LR

The IE ThresholdQ-LR is used to indicate a measured RSRQ threshold for LP-WUS. Actual value of threshold = field value [dB].

ThresholdQ-LR information element

-- ASN1START

-- TAG-THRESHOLDQ-LR-START

ThresholdQ-LR-r19 ::=               INTEGER (-34..0)

-- TAG-THRESHOLDQ-LR-STOP

-- ASN1STOP

#### –	TimeAlignmentTimer

The IE TimeAlignmentTimer is used to configure the time alignment timer as specified in TS 38.321 [3]. The values are in ms.

TimeAlignmentTimer information element

-- ASN1START

-- TAG-TIMEALIGNMENTTIMER-START

TimeAlignmentTimer ::=              ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

-- TAG-TIMEALIGNMENTTIMER-STOP

-- ASN1STOP

#### –	TimeToTrigger

The IE TimeToTrigger specifies the value range used for time to trigger parameter, which concerns the time during which specific criteria for the event needs to be met in order to trigger a measurement report either by the RRC message MeasurementReport or by MAC CE in TS 38.321 [3] or start logging of measurements for network-side data collection. Value ms0 corresponds to 0 ms and behaviour as specified in 7.1.2 applies, value ms40 corresponds to 40 ms, and so on.

TimeToTrigger information element

-- ASN1START

-- TAG-TIMETOTRIGGER-START

TimeToTrigger ::=                   ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

-- TAG-TIMETOTRIGGER-STOP

-- ASN1STOP

#### –	TN-AreaId

The IE TN-AreaId is used to identify a TN coverage area in an NTN system.

TN-AreaId information element

-- ASN1START

-- TAG-TN-AREAID-START

TN-AreaId-r18 ::=                  INTEGER (1..maxTN-AreaInfo-r18)

-- TAG-TN-AREAID-STOP

-- ASN1STOP

#### –	TrackingAreaCode

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN/SNPN, see TS 24.501 [23].

TrackingAreaCode information element

-- ASN1START

-- TAG-TRACKINGAREACODE-START

TrackingAreaCode ::= BIT STRING (SIZE (24))

-- TAG-TRACKINGAREACODE-STOP

-- ASN1STOP

#### –	UAC-BarringInfoSetIndex

The IE UAC-BarringInfoSetIndex provides the index of the entry in uac-BarringInfoSetList. Value 1 corresponds to the first entry in uac-BarringInfoSetList, value 2 corresponds to the second entry in this list and so on. An index value referring to an entry not included in uac-BarringInfoSetList indicates no barring.

UAC-BarringInfoSetIndex information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETINDEX-START

UAC-BarringInfoSetIndex ::=                INTEGER (1..maxBarringInfoSet)

-- TAG-UAC-BARRINGINFOSETINDEX-STOP

-- ASN1STOP

#### –	UAC-BarringInfoSetList

The IE UAC-BarringInfoSetList provides a list of access control parameter sets. An access category can be configured with access parameters according to one of the sets.

UAC-BarringInfoSetList information element

-- ASN1START

-- TAG-UAC-BARRINGINFOSETLIST-START

UAC-BarringInfoSetList ::=          SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet

UAC-BarringInfoSetList-v1700 ::= SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet-v1700

UAC-BarringInfoSet ::=              SEQUENCE {

uac-BarringFactor                   ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime                     ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity        BIT STRING (SIZE(7))

}

UAC-BarringInfoSet-v1700 ::= SEQUENCE {

uac-BarringFactorForAI3-r17  ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40, p50, p60, p70, p75, p80, p85, p90, p95}

OPTIONAL  -- Need S

}

-- TAG-UAC-BARRINGINFOSETLIST-STOP

-- ASN1STOP

UAC-BarringInfoSetList field descriptions

uac-BarringInfoSetList List of access control parameter sets. Each access category can be configured with access parameters corresponding to a particular set by uac-barringInfoSetIndex. Association of an access category with an index that has no corresponding entry in the uac-BarringInfoSetList is valid configuration and indicates no barring.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, bit 2 in the bit string corresponds to Access Identity 11, bit 3 in the bit string corresponds to Access Identity 12, bit 4 in the bit string corresponds to Access Identity 13, bit 5 in the bit string corresponds to Access Identity 14, and bit 6 in the bit string corresponds to Access Identity 15. Value 0 means that access attempt is allowed for the corresponding access identity.

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringFactorForAI3 Barring factor applicable for Access Identity 3. Represents the probability that access attempt would be allowed during access barring check. If absent, the UE considers the access attempt as allowed.

uac-BarringTime The average time in seconds before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category, see 5.3.14.5.

#### –	UAC-BarringPerCatList

The IE UAC-BarringPerCatList provides access control parameters for a list of access categories.

UAC-BarringPerCatList information element

-- ASN1START

-- TAG-UAC-BARRINGPERCATLIST-START

UAC-BarringPerCatList ::=           SEQUENCE (SIZE (1..maxAccessCat-1)) OF UAC-BarringPerCat

UAC-BarringPerCat ::=               SEQUENCE {

accessCategory                       INTEGER (1..maxAccessCat-1),

uac-barringInfoSetIndex              UAC-BarringInfoSetIndex

}

-- TAG-UAC-BARRINGPERCATLIST-STOP

-- ASN1STOP

UAC-BarringPerCatList field descriptions

accessCategory The Access Category according to TS 22.261 [25].

#### –	UAC-BarringPerPLMN-List

The IE UAC-BarringPerPLMN-List provides access category specific access control parameters, which are configured per PLMN/SNPN.

UAC-BarringPerPLMN-List information element

-- ASN1START

-- TAG-UAC-BARRINGPERPLMN-LIST-START

UAC-BarringPerPLMN-List ::=         SEQUENCE (SIZE (1.. maxPLMN)) OF UAC-BarringPerPLMN

UAC-BarringPerPLMN ::=              SEQUENCE {

plmn-IdentityIndex                  INTEGER (1..maxPLMN),

uac-ACBarringListType               CHOICE{

uac-ImplicitACBarringList           SEQUENCE (SIZE(maxAccessCat-1)) OF UAC-BarringInfoSetIndex,

uac-ExplicitACBarringList           UAC-BarringPerCatList

}                                                                                                     OPTIONAL     -- Need S

}

-- TAG-UAC-BARRINGPERPLMN-LIST-STOP

-- ASN1STOP

UAC-BarringPerPLMN-List field descriptions

uac-ACBarringListType Access control parameters for each access category valid only for a specific PLMN or SNPN. UE behaviour upon absence of this field is specified in clause 5.3.14.2.

plmn-IdentityIndex Index of the PLMN or SNPN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

#### –	UE-TimersAndConstants

The IE UE-TimersAndConstants contains timers and constants used by the UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstants information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTS-START

UE-TimersAndConstants ::=           SEQUENCE {

t300                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t301                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

t319                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

...

}

-- TAG-UE-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

#### –	UE-TimersAndConstantsRemoteUE

The IE UE-TimersAndConstantsRemoteUE contains timers and constants used by the L2 U2N Remote UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstantsRemoteUE information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-START

UE-TimersAndConstantsRemoteUE-r17 ::= SEQUENCE {

t300-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t301-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t319-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

...

}

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-STOP

-- ASN1STOP

UE-TimersAndConstantsRemoteUE field descriptions

t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE. The effective T300 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T300 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively.

t301-RemoteUE Indicates the timer value of T301 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t301 applies to L2 U2N Remote UE. The effective T301 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T301 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively.

t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE. The effective T319 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T319 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively.

#### –	UL-DelayValueConfig

The IE UL-DelayValueConfig specifies the configuration of the UL PDCP Packet Delay value per DRB measurement specified in TS 38.314 [53].

UL-DelayValueConfig information element

-- ASN1START

-- TAG-ULDELAYVALUECONFIG-START

UL-DelayValueConfig-r16 ::=  SEQUENCE {

delay-DRBlist-r16            SEQUENCE (SIZE(1..maxDRB)) OF DRB-Identity

}

-- TAG-ULDELAYVALUECONFIG-STOP

-- ASN1STOP

UL-DelayValueConfig field descriptions

delay-DRBlist Indicates the DRB IDs used by UE to provide results of UL PDCP Packet Delay value per DRB measurement as specified in TS 38.314 [53].

#### –	UL-ExcessDelayConfig

The IE UL-ExcessDelayConfig IE specifies the configuration of the UL PDCP Excess Packet Delay per DRB measurement specified in TS 38.314 [53].

UL-ExcessDelayConfig information element

-- ASN1START

-- TAG-ULEXCESSDELAYCONFIG-START

UL-ExcessDelayConfig-r17 ::=  SEQUENCE {

excessDelay-DRBlist-r17       SEQUENCE (SIZE(1..maxDRB)) OF ExcessDelay-DRB-IdentityInfo-r17

}

ExcessDelay-DRB-IdentityInfo-r17 ::=  SEQUENCE {

drb-IdentityList                      SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity,

delayThreshold                        ENUMERATED {ms0dot25, ms0dot5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70,

ms80, ms90, ms100, ms150, ms300, ms500}

}

-- TAG-ULEXCESSDELAYCONFIG-STOP

-- ASN1STOP

UL-ExcessDelayConfig field descriptions

drb-IdentityList Indicates the DRB IDs used by UE to provide results of UL PDCP Excess Packet Delay per DRB measurement as specified in TS 38.314 [53].

delayThreshold Indicates the delay threshold for the DRB IDs indicated in DRB-IdentityList. Value ms0dot25 corresponds to 0.25ms, ms0dot5 corresponds to 0.5ms, ms1 corresponds to 1ms and so on.

#### –	UL-GapFR2-Config

The IE UL-GapFR2-Config specifies the FR2 uplink gap configuration.

UL-GapFR2-Config information element

-- ASN1START

-- TAG-UL-GAPFR2-CONFIG-START

UL-GapFR2-Config-r17 ::=      SEQUENCE {

gapOffset-r17                 INTEGER (0..159),

ugl-r17                       ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms1},

ugrp-r17                      ENUMERATED {ms5, ms20, ms40, ms160},

refFR2-ServCellAsyncCA-r17    ServCellIndex                                                      OPTIONAL -- Cond AsyncCA

}

-- TAG-UL-GAPFR2-CONFIG-STOP

-- ASN1STOP

UL-GapFR2-Config field descriptions

gapOffset Value gapOffset is the gap offset of the FR2 UL gap pattern with UGRP indicated in the field ugrp. The value range is from 0 to ugrp-1.

refFR2-ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 UL gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

ugl Value ugl is the gap length in ms of the FR2 UL gap. The FR2 UL gap length is according to Table 9.1.11-1 in TS 38.133 [14]. Value ms0dot125 corresponds to 0.125 ms, ms0dot25 corresponds to 0.25 ms and so on.

ugrp Value ugrp is the gap repetition period in (ms) of the FR2 UL gap. The FR2 UL gap repetition period is according to Table 9.1.11-1 in TS 38.133 [14].

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 UL gap pattern to UE in: - (NG)EN-DC, NR SA, NE-DC or NR-DC without FR2-FR2 band combination, with asynchronous CA involving FR2 carriers. Otherwise, it is absent. |

#### –	UplinkCancellation

The IE UplinkCancellation is used to configure the UE to monitor PDCCH for the CI-RNTI.

UplinkCancellation information element

-- ASN1START

-- TAG-UPLINKCANCELLATION-START

UplinkCancellation-r16 ::=           SEQUENCE {

ci-RNTI-r16                          RNTI-Value,

dci-PayloadSizeForCI-r16             INTEGER (0..maxCI-DCI-PayloadSize-r16),

ci-ConfigurationPerServingCell-r16   SEQUENCE (SIZE (1..maxNrofServingCells)) OF CI-ConfigurationPerServingCell-r16,

...

}

CI-ConfigurationPerServingCell-r16 ::=   SEQUENCE {

servingCellId                            ServCellIndex,

positionInDCI-r16                        INTEGER (0..maxCI-DCI-PayloadSize-1-r16),

positionInDCI-ForSUL-r16                 INTEGER (0..maxCI-DCI-PayloadSize-1-r16)                             OPTIONAL,   -- Cond SUL-Only

ci-PayloadSize-r16                       ENUMERATED {n1, n2, n4, n5, n7, n8, n10, n14, n16, n20, n28, n32, n35, n42, n56, n112},

timeFrequencyRegion-r16                  SEQUENCE {

timeDurationForCI-r16                    ENUMERATED {n2, n4, n7, n14}                                      OPTIONAL,   -- Cond SymbolPeriodicity

timeGranularityForCI-r16                 ENUMERATED {n1, n2, n4, n7, n14, n28},

frequencyRegionForCI-r16                 INTEGER (0..37949),

deltaOffset-r16                          INTEGER (0..2),

...

},

uplinkCancellationPriority-v1610         ENUMERATED {enabled}                                                  OPTIONAL    -- Need S

}

-- TAG-UPLINKCANCELLATION-STOP

-- ASN1STOP

UplinkCancellation field descriptions

ci-ConfigurationPerServingCell Indicates (per serving cell) the position of the ci-PaylaodSize bit CI values inside the DCI payload (see TS 38.213 [13], clause 11.2A).

ci-RNTI RNTI used for indication cancellation in UL (see TS 38.212 [17] clause 7.3.1 and TS 38.213 [13], clause 11.2A).

dci-PayloadSizeForCI Total length of the DCI payload scrambled with CI-RNTI (see TS 38.213 [13], clause 11.2A).

CI-ConfigurationPerServingCell field descriptions

ci-PayloadSize Configures the field size for each UL cancelation indicator of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A).

deltaOffset Configures the additional offset from the end of a PDCCH reception where the UE detects the DCI format 2_4 and the first symbol of the T_"CI" symbols, in the unit of OFDM symbols (see TS 38.213 [13], clause 11.2A).

frequencyRegionForCI Configures the reference frequency region where a detected UL CI is applicable (see TS 38.213 [13], clause 11.2A). It is defined in the same way as locationAndBandwidth.

positionInDCI Starting position (in number of bit) of the ci-PayloadSize bit CI value applicable for this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2A).

positionInDCI-ForSUL Starting position (in number of bit) of the ci-PayloadSize bit CI value applicable for SUL of this serving cell (servingCellId) within the DCI payload (see TS 38.213 [13], clause 11.2A).

timeDurationForCI Configures the duration of the reference time region in symbols where a detected UL CI is applicable of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A). If the field is absent, i.e., the configured UL CI monitoring periodicity indicated by monitoringSlotPeriodicityAndOffset for DCI format 2_4 is larger than 1 slot or 1 slot with only one monitoring occasion, the UE applies the value of the configured UL CI monitoring periodicity,

timeFrequencyRegion Configures the reference time and frequency region where a detected UL CI is applicable of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A).

timeGranularityForCI Configures the number of partitions within the time region of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.2A).

uplinkCancellationPriority Configures uplink cancellation behavior if both UL CI and intra-UE priority indicator are configured for a given UE. If the field is present, then UL CI is only applicable to the UL transmissions indicated/configured as low priority level. If the field is absent, UL CI is applicable to UL transmission irrespective of its priority level (see TS 38.213 [13], clause 11.2A).

| Conditional Presence | Explanation |
| --- | --- |
| SUL-Only | The field is optionally present, Need R, if supplementaryUplink is configured in ServingCellConfig. It is absent otherwise. |
| SymbolPeriodicity | This field is mandatory present if the configured UL CI monitoring periodicity indicated by monitoringSlotPeriodicityAndOffset for DCI format 2_4 is 1 slot with more than one monitoring occasion, otherwise absent. |

#### –	UplinkConfigCommon

The IE UplinkConfigCommon provides common uplink parameters of a cell.

UplinkConfigCommon information element

-- ASN1START

-- TAG-UPLINKCONFIGCOMMON-START

UplinkConfigCommon ::=              SEQUENCE {

frequencyInfoUL                     FrequencyInfoUL                                 OPTIONAL,   -- Cond InterFreqHOAndServCellAdd

initialUplinkBWP                    BWP-UplinkCommon                                OPTIONAL,   -- Cond ServCellAdd

dummy                               TimeAlignmentTimer

}

UplinkConfigCommon-v1700 ::=        SEQUENCE {

initialUplinkBWP-RedCap-r17         BWP-UplinkCommon                                OPTIONAL    -- Need R

}

-- TAG-UPLINKCONFIGCOMMON-STOP

-- ASN1STOP

UplinkConfigCommon field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

initialUplinkBWP The initial uplink BWP configuration for a serving cell (see TS 38.213 [13], clause 12).

initialUplinkBWP-RedCap If present, (e)RedCap UEs use this UL BWP instead of initialUplinkBWP. If absent, (e)RedCap UEs use initialUplinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is optionally present, Need M. |
| ServCellAdd | This field is mandatory present upon serving cell addition (for PSCell and SCell) and upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

#### –	UplinkConfigCommonSIB

The IE UplinkConfigCommonSIB provides common uplink parameters of a cell.

UplinkConfigCommonSIB information element

-- ASN1START

-- TAG-UPLINKCONFIGCOMMONSIB-START

UplinkConfigCommonSIB ::=               SEQUENCE {

frequencyInfoUL                         FrequencyInfoUL-SIB,

initialUplinkBWP                        BWP-UplinkCommon,

timeAlignmentTimerCommon                TimeAlignmentTimer

}

UplinkConfigCommonSIB-v1700 ::=         SEQUENCE {

initialUplinkBWP-RedCap-r17             BWP-UplinkCommon                                OPTIONAL   -- Need R

}

UplinkConfigCommonSIB-v1760 ::=         SEQUENCE {

frequencyInfoUL-v1760                   FrequencyInfoUL-SIB-v1760

}

-- TAG-UPLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

UplinkConfigCommonSIB field descriptions

frequencyInfoUL Absolute uplink frequency configuration and subcarrier specific virtual carriers.

InitialUplinkBWP The initial uplink BWP configuration for a PCell (see TS 38.213 [13], clause 12).

initialUplinkBWP-RedCap If present, (e)RedCap UEs use this UL BWP instead of initialUplinkBWP. If absent, (e)RedCap UEs use initialUplinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

#### –	Uplink-PowerControl

The IE Uplink-PowerControl is used to configure UE specific power control parameter for PUSCH, PUCCH and SRS.

Uplink-PowerControl information element

-- ASN1START

-- TAG-UPLINK-POWERCONTROL-START

Uplink-powerControl-r17  ::= SEQUENCE {

ul-powercontrolId-r17        Uplink-powerControlId-r17,

p0AlphaSetforPUSCH-r17       P0AlphaSet-r17                                                               OPTIONAL, -- Need R

p0AlphaSetforPUCCH-r17       P0AlphaSet-r17                                                               OPTIONAL, -- Need R

p0AlphaSetforSRS-r17         P0AlphaSet-r17                                                               OPTIONAL  -- Need R

}

P0AlphaSet-r17 ::=           SEQUENCE {

p0-r17                       INTEGER (-16..15)                                                            OPTIONAL, -- Need R

alpha-r17                    Alpha                                                                        OPTIONAL, -- Need S

closedLoopIndex-r17          ENUMERATED { i0, i1 }

}

Uplink-powerControlId-r17 ::= INTEGER(1.. maxUL-TCI-r17)

Uplink-powerControlExt-v1900  ::=    SEQUENCE {

p0AlphaSetforPUSCH-SBFD-r19          P0AlphaSet-r17                                                       OPTIONAL, -- Need R

p0AlphaSetforPUCCH-SBFD-r19          P0AlphaSet-r17                                                       OPTIONAL, -- Need R

p0AlphaSetforSRS-SBFD-r19            P0AlphaSet-r17                                                       OPTIONAL  -- Need R

}

-- TAG-UPLINK-POWERCONTROL-STOP

-- ASN1STOP

Uplink-PowerControl, Uplink-powerControlExt field descriptions

p0AlphaSetforPUSCH, p0AlphaSetforPUCCH, p0AlphaSetforSRS Configures power control parameters for PUSCH, PUCCH and SRS (see TS 38.213 [13], clause 7.2). When the field alpha is absent in p0AlphaSetforPUSCH, the UE applies the value 1 for PUSCH power control. When the field alpha is absent in p0AlphaSetforSRS, the UE applies the value 1 for SRS power control. In p0AlphaSetForPUCCH, the field alpha is absent (not used). If srs-TwoSeparatePowerControlAdjustmentStates is configured and srs-PowerControlAdjustmentStates is set to separateClosedLoop, the field closedLoopIndex in p0AlphaSetforSRS indicates one of the separate SRS power control adjustment states i.e. value of i0 and i1 refer to the first and the second power control adjustment state separate from PUSCH, respectively.

p0AlphaSetforPUSCH-SBFD, p0AlphaSetforPUCCH-SBFD, p0AlphaSetforSRS-SBFD Configures separate UL power control parameters for PUSCH, PUCCH and SRS transmissions in SBFD symbols (see TS 38.213 [13], clause 6 and clause 7).

#### –	Uu-RelayRLC-ChannelConfig

The IE Uu-RelayRLC-ChannelConfig is used to configure an RLC entity, a corresponding logical channel in MAC for Uu Relay RLC channel between L2 U2N Relay UE and network, or between a N3C relay UE and network in case of MP.

Uu-RelayRLC-ChannelConfig information element

-- ASN1START

-- TAG-UU-RELAYRLC-CHANNELCONFIG-START

Uu-RelayRLC-ChannelConfig-r17::= SEQUENCE {

uu-LogicalChannelIdentity-r17    LogicalChannelIdentity                    OPTIONAL,   -- Cond RelayLCH-SetupOnly

uu-RelayRLC-ChannelID-r17        Uu-RelayRLC-ChannelID-r17,

reestablishRLC-r17               ENUMERATED {true}                         OPTIONAL,   -- Need N

rlc-Config-r17                   RLC-Config                                OPTIONAL,   -- Cond RelayLCH-Setup

mac-LogicalChannelConfig-r17     LogicalChannelConfig                      OPTIONAL,   -- Cond RelayLCH-Setup

...

}

-- TAG-UU-RELAYRLC-CHANNELCONFIG-STOP

-- ASN1STOP

Uu-RelayRLC-ChannelConfig field descriptions

uu-LogicalChannelIdentity Indicates the logical channel id for Uu Relay RLC channel of the L2 U2N Relay UE or of the L2 Last U2N Relay UE.

uu-RelayRLC-ChannelID Indicates the Uu Relay RLC channel in the link between L2 U2N Relay UE and network or between L2 Last U2N Relay UE and network.

reestablishRLC Indicates that RLC should be re-established.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters.

| Conditional Presence | Explanation |
| --- | --- |
| RelayLCH-Setup | This field is mandatory present upon creation of a new logical channel for a Uu Relay RLC channel. It is optionally present, Need M, otherwise. |
| RelayLCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a Uu Relay RLC channel. It is absent otherwise. |

#### –	Uu-RelayRLC-ChannelID

The IE Uu-RelayRLC-ChannelID is used to identify a Uu Relay RLC channel in the link between L2 U2N Relay UE and network or between L2 Last U2N Relay UE and network.

Uu-RelayRLC-ChannelID information element

-- ASN1START

-- TAG-UU-RELAYRLC-CHANNELID-START

Uu-RelayRLC-ChannelID-r17 ::= INTEGER (1..maxLC-ID)

-- TAG-UU-RELAYRLC-CHANNELID-STOP

-- ASN1STOP

#### –	UplinkTxDirectCurrentList

The IE UplinkTxDirectCurrentList indicates the Tx Direct Current locations per serving cell for each configured UL BWP in the serving cell, based on the BWP numerology and the associated carrier bandwidth.

UplinkTxDirectCurrentList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTLIST-START

UplinkTxDirectCurrentList ::=           SEQUENCE (SIZE (1..maxNrofServingCells)) OF UplinkTxDirectCurrentCell

UplinkTxDirectCurrentCell ::=           SEQUENCE {

servCellIndex                           ServCellIndex,

uplinkDirectCurrentBWP                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP,

...,

[[

uplinkDirectCurrentBWP-SUL              SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkTxDirectCurrentBWP               OPTIONAL

]]

}

UplinkTxDirectCurrentBWP ::=            SEQUENCE {

bwp-Id                                  BWP-Id,

shift7dot5kHz                           BOOLEAN,

txDirectCurrentLocation                 INTEGER (0..3301)

}

-- TAG-UPLINKTXDIRECTCURRENTLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentBWP field descriptions

bwp-Id The BWP-Id of the corresponding uplink BWP.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true. Otherwise 7.5 kHz shift is not applied.

txDirectCurrentLocation The uplink Tx Direct Current location for the carrier. Only values in the value range of this field between 0 and 3299, which indicate the subcarrier index within the carrier corresponding to the numerology of the corresponding uplink BWP and value 3300, which indicates "Outside the carrier" and value 3301, which indicates "Undetermined position within the carrier" are used in this version of the specification.

UplinkTxDirectCurrentCell field descriptions

servCellIndex The serving cell ID of the serving cell corresponding to the uplinkDirectCurrentBWP.

uplinkDirectCurrentBWP The Tx Direct Current locations for all the uplink BWPs configured at the corresponding serving cell.

uplinkDirectCurrentBWP-SUL The Tx Direct Current locations for all the supplementary uplink BWPs configured at the corresponding serving cell.

#### –	UplinkTxDirectCurrentMoreCarrierList

The IE UplinkTxDirectCurrentMoreCarrierList indicates the Tx Direct Current locations for intra-band CA including one, two or more uplink carriers. The UE does not report the uplink Direct Current location information for SUL carrier(s).

UplinkTxDirectCurrentMoreCarrierList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTMORECARRIERLIST-START

UplinkTxDirectCurrentMoreCarrierList-r17 ::=   SEQUENCE (SIZE (1..maxNrofCC-Group-r17)) OF CC-Group-r17

CC-Group-r17 ::=           SEQUENCE {

servCellIndexLower-r17     ServCellIndex,

servCellIndexHigher-r17    ServCellIndex              OPTIONAL,

defaultDC-Location-r17     DefaultDC-Location-r17,

offsetToDefault-r17        CHOICE{

offsetValue                OffsetValue-r17,

offsetlist                 SEQUENCE (SIZE(1..maxNrofReqComDC-Location-r17)) OF OffsetValue-r17

}                                                     OPTIONAL

}

OffsetValue-r17::=         SEQUENCE {

offsetValue-r17            INTEGER (-20000.. 20000),

shift7dot5kHz-r17          BOOLEAN

}

DefaultDC-Location-r17 ::= CHOICE {

ul                         FrequencyComponent-r17,

dl                         FrequencyComponent-r17,

ulAndDL                    FrequencyComponent-r17

}

FrequencyComponent-r17 ::=  ENUMERATED {activeCarrier,configuredCarrier,activeBWP,configuredBWP}

-- TAG-UPLINKTXDIRECTCURRENTMORECARRIERLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentMoreCarrierList and CC-Group field descriptions

CC-Group The contiguous carriers sharing the same PA in an intra-band UL CA configuration. The UE shall report only one DC location for an intra-band CC combination with one active uplink carrier in case DefaultDC-Location is set to activeCarrier or activeBWP.

defaultDC-Location Indicates the default DC location derivation option. The default Tx Direct Current is located at the mathematical center of the UE bandwidth, i.e. between the lower edge of the lowest subcarrier of the lowest frequency component and the upper edge of the highest subcarrier of the highest frequency component, rounded to the subcarrier grid of the lowest SCS defined for the component carrier on which the default Direct Current is located. The lowest and highest frequency components used for derivation of mathematical center are indicated by FrequencyComponent in the associated CC-Group, where the lowest frequency component and the highest frequency component may be the same. If the mathematical center of the UE bandwidth lands on frequencies where there is no subcarrier grid defined, the subcarrier grid of the lowest SCS of the nearest lower frequency component carrier shall be extended to cover the frequency of the mathematical default Direct Current location.

offsetToDefault Indicates the DC location offset to the default DC location derived from defaultDC-Location. The lowest SCS in the CC group is used as the offset granularity. Value 0 respresents no offset. offsetValue is used in case DefaultDC-Location is set to configuredCarrier or configuredBWP. offsetlist is used in case DefaultDC-Location is set to activeCarrier or activeBWP. Each entity in this list corresponds to the entry in carriers combination in IntraBandCC-CombinationReqList of the intra-band CA component. For each CC group, the UE shall include the same number of entries, and listed in the same order as in CC-CombinationList. If DefaultDC-Location is set the activeCarrier, same offsetValue is signalled for all requested carriers combinations with same active carriers states(regardless of the active BWP index).

servCellIndexHigher Indicates the serving cell index of the highest edge of the CC-Group. If asbsent, there is only one carrier in this group indicated by servCellIndexLower.

servCellIndexLower Indicates the serving cell index of the lowest edge of the CC-Group.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true, otherwise 7.5 kHz shift is not applied.

DefaultDC-Location field descriptions

dl Indicates that the default DC location is derived based on the DL frequencies of the frequency component.

ul Indicates that the default DC location is derived based on the UL frequencies of the frequency component.

ulAndDL Indicates that the default DC location is derived based on the edge most frequencies among any DL and UL frequency components.

#### –	UplinkTxDirectCurrentTwoCarrierList

The IE UplinkTxDirectCurrentTwoCarrierList indicates the Tx Direct Current locations when uplink intra-band CA with two carriers is configured, based on the configured carriers and BWP numerology and the associated carrier bandwidth of the carriers. The UE does not report the uplink Direct Current location information for SUL carrier(s).

UplinkTxDirectCurrentTwoCarrierList information element

-- ASN1START

-- TAG-UPLINKTXDIRECTCURRENTTWOCARRIERLIST-START

UplinkTxDirectCurrentTwoCarrierList-r16 ::=   SEQUENCE (SIZE (1..maxNrofTxDC-TwoCarrier-r16)) OF UplinkTxDirectCurrentTwoCarrier-r16

UplinkTxDirectCurrentTwoCarrier-r16 ::=       SEQUENCE {

carrierOneInfo-r16                            UplinkTxDirectCurrentCarrierInfo-r16,

carrierTwoInfo-r16                            UplinkTxDirectCurrentCarrierInfo-r16,

singlePA-TxDirectCurrent-r16                  UplinkTxDirectCurrentTwoCarrierInfo-r16,

secondPA-TxDirectCurrent-r16                  UplinkTxDirectCurrentTwoCarrierInfo-r16            OPTIONAL

}

UplinkTxDirectCurrentCarrierInfo-r16 ::=      SEQUENCE {

servCellIndex-r16                             ServCellIndex,

servCellInfo-r16                              CHOICE {

bwp-Id-r16                                    BWP-Id,

deactivatedCarrier-r16                        ENUMERATED {deactivated}

}

}

UplinkTxDirectCurrentTwoCarrierInfo-r16 ::=   SEQUENCE {

referenceCarrierIndex-r16                     ServCellIndex,

shift7dot5kHz-r16                             BOOLEAN,

txDirectCurrentLocation-r16                   INTEGER (0..3301)

}

-- TAG-UPLINKTXDIRECTCURRENTTWOCARRIERLIST-STOP

-- ASN1STOP

UplinkTxDirectCurrentTwoCarrierInfo field descriptions

referenceCarrierIndex The serving cell ID of the carrier which is to be used as the reference for interpreting the Tx Direction Current location as reported using txDirectCurrentLocation-r16. The numerology of the uplink BWP ID reported with bwp-Id-r16 for this serving cell is the numerology used for interpreting the reported subcarrier location.

shift7dot5kHz Indicates whether there is 7.5 kHz shift or not. 7.5 kHz shift is applied if the field is set to true. Otherwise 7.5 kHz shift is not applied.

txDirectCurrentLocation The uplink Tx Direct Current location for the two carrier uplink CA with the serving cells reported using carrierOneInfo-r16 and carrierTwoInfo-r16. Values in the range of this field between 0 and 3299 indicate the subcarrier index of the uplink Tx Direct Current location with the subcarrier taken from the serving cell with ID referenceCarrierIndex and the numerology of the corresponding uplink BWP reported for this serving cell. Value 3300 indicates "Outside the carrier" and value 3301 indicates "Undetermined position within the carrier".

UplinkTxDirectCurrentCarrierInfo field descriptions

bwp-Id The BWP ID of the serving cell which is part of the two carrier uplink carrier aggregation. The UE shall not report this field if the serving cell is reported as deactivated using deactivatedCarrier-r16.

deactivatedCarrier For the reported uplink Tx Direct Current location(s) corresponding to singlePA-TxDirectCurrent-r16, indicates whether the carrier is deactivated or not for this serving cell. If the carrier refers to the PCell, the UE shall not set this field to deactivated.

servCellIndex The serving cell ID of the serving cell which is part of the two carrier uplink carrier aggregation.

UplinkTxDirectCurrentTwoCarrier field descriptions

carrierOneInfo The serving cell ID and BWP ID of the first carrier of the uplink carrier aggregation for which the uplink Tx Direct Current location(s) are being reported.

carrierTwoInfo The serving cell ID and BWP ID of the second carrier of the uplink carrier aggregation for which the uplink Tx Direct Current location(s) are being reported.

singlePA-TxDirectCurrent The uplink Tx Direct Current location for the UE which support single PA for this uplink carrier aggregation. For the UEs which support dual PA for this uplink carrier aggregation, this field is for reporting the uplink Tx Direct Current location of the first PA.

secondPA-TxDirectCurrent The uplink Tx Direct Current location used by the UE with the second PA for the UEs which support dual PA for this uplink carrier aggregation. This field shall be absent for the UplinkTxDirectCurrentTwoCarrier entity where deactivatedCarrier of carrierOneInfo or carrierTwoInfo is set to deactivated.

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource (see TS 38.214 [19], clause 5.1.4.2). Reconfiguration of a ZP-CSI-RS-Resource between periodic or semi-persistent and aperiodic is not supported.

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=              SEQUENCE {

zp-CSI-RS-ResourceId                ZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=            INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

ZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS (see TS 38.214 [19], clause 5.1.4.2). Network always configures the UE with a value for this field for periodic and semi-persistent ZP-CSI-RS resource (as indicated in PDSCH-Config).

resourceMapping OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot.

zp-CSI-RS-ResourceId ZP CSI-RS resource configuration ID (see TS 38.214 [19], clause 5.1.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent ZP-CSI-RS-Resources (as indicated in PDSCH-Config). The field is absent otherwise. |

#### –	ZP-CSI-RS-ResourceSet

The IE ZP-CSI-RS-ResourceSet refers to a set of ZP-CSI-RS-Resources using their ZP-CSI-RS-ResourceIds.

ZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESET-START

ZP-CSI-RS-ResourceSet ::=           SEQUENCE {

zp-CSI-RS-ResourceSetId             ZP-CSI-RS-ResourceSetId,

zp-CSI-RS-ResourceIdList            SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId,

...

}

-- TAG-ZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

ZP-CSI-RS-ResourceSet field descriptions

zp-CSI-RS-ResourceIdList The list of ZP-CSI-RS-ResourceId identifying the ZP-CSI-RS-Resource elements belonging to this set.

#### –	ZP-CSI-RS-ResourceSetId

The IE ZP-CSI-RS-ResourceSetId identifies a ZP-CSI-RS-ResourceSet.

ZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESETID-START

ZP-CSI-RS-ResourceSetId ::=                     INTEGER (0..maxNrofZP-CSI-RS-ResourceSets-1)

-- TAG-ZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

### 6.3.3	UE capability information elements

#### –	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, rel16, rel17, rel18, rel19, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

#### –	AerialParameters

The IE AerialParameters is used to convey the capabilities supported by the UE for aerial operation.

AerialParameters information element

-- ASN1START

-- TAG-AERIALPARAMETERS-START

AerialParameters-r18 ::=                      SEQUENCE {

-- Support of Aerial UE features

aerialUE-Capability-r18                       ENUMERATED {supported}                               OPTIONAL,

-- Support of altitude measurement and event H1/H2-triggered reporting

altitudeMeas-r18                              ENUMERATED {supported}                               OPTIONAL,

-- Support of altitude based measurement configuration of SSB-ToMeasure

altitudeBasedSSB-ToMeasure-r18                ENUMERATED {supported}                               OPTIONAL,

-- Support of events A3H1, A3H2, A4H1, A4H2, A5H1, A5H2

eventAxHy-r18                                 ENUMERATED {supported}                               OPTIONAL,

-- Support of flight path reporting

flightPathReporting-r18                       ENUMERATED {supported}                               OPTIONAL,

-- Support of flight path availability indication via UAI

flightPathAvailabilityIndicationUAI-r18       ENUMERATED {supported}                               OPTIONAL,

-- Support of numberOfTriggeringCells for eventA3, eventA4, and eventA5, and additionally, if the UE supports eventAxHy-r18,

-- support of numberOfTriggeringCells for eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, and eventA5H2

multipleCellsMeasExtension-r18                ENUMERATED {supported}                               OPTIONAL,

-- Support of handling aerial-specific Ns value(s) and Pmax list broadcasted by the cell

nr-NS-PmaxListAerial-r18                      ENUMERATED {supported}                               OPTIONAL,

-- Support of reporting only the measurement report corresponding to the event with the smallest value between the

-- altitude of the UAV and the altitude threshold for which the altitude-related entering condition e.g. A3H1-2 is satisfied, when

-- multiple events of the same type (Hx or AxHy) for the same MO (for AxHy) are triggered simultaneously.

simulMultiTriggerSingleMeasReport-r18         ENUMERATED {supported}                               OPTIONAL,

-- Support of A2X service(s) using PC5 Sidelink and dedicated resource pool for A2X service(s)

sl-A2X-Service-r18                                 ENUMERATED {brid, daa, bridAndDAA}                   OPTIONAL,

...,

[[

-- Support of condEvents A3H1, A3H2, A5H1, A5H2

condEventAxHy-r19                             ENUMERATED {supported}                               OPTIONAL

]]

}

-- TAG-AERIALPARAMETERS-STOP

-- ASN1STOP

#### –	AIML-Parameters

The IE AIML-Parameters is used to convey the capabilities supported by the UE for AI/ML beam management and AI/ML CSI prediction.

AIML-Parameters information element

-- ASN1START

-- TAG-AIML-PARAMETERS-START

AIML-Parameters-r19 ::=            SEQUENCE {

applicabilityReportingCSI-r19                  ENUMERATED {supported}                                      OPTIONAL,

applicabilityReportingOther-r19                ENUMERATED {supported}                                      OPTIONAL,

loggedDataCollection-r19                       ENUMERATED {supported}                                      OPTIONAL,

eventBasedLoggedDataCollection-r19             ENUMERATED {supported}                                      OPTIONAL,

dataThresholdAvailabilityIndication-r19        ENUMERATED {supported}                                      OPTIONAL

}

-- TAG-AIML-PARAMETERS-STOP

-- ASN1STOP

#### –	AppLayerMeasParameters

The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements.

AppLayerMeasParameters information element

-- ASN1START

-- TAG-APPLAYERMEASPARAMETERS-START

AppLayerMeasParameters-r17 ::=            SEQUENCE {

qoe-Streaming-MeasReport-r17              ENUMERATED {supported}                                             OPTIONAL,

qoe-MTSI-MeasReport-r17                   ENUMERATED {supported}                                             OPTIONAL,

qoe-VR-MeasReport-r17                     ENUMERATED {supported}                                             OPTIONAL,

ran-VisibleQoE-Streaming-MeasReport-r17   ENUMERATED {supported}                                             OPTIONAL,

ran-VisibleQoE-VR-MeasReport-r17          ENUMERATED {supported}                                             OPTIONAL,

ul-MeasurementReportAppLayer-Seg-r17      ENUMERATED {supported}                                             OPTIONAL,

...,

[[

qoe-IdleInactiveMeasReport-r18            ENUMERATED {supported}                                             OPTIONAL,

qoe-NRDC-MeasReport-r18                   ENUMERATED {supported}                                             OPTIONAL,

qoe-AdditionalMemoryMeasReport-r18        ENUMERATED {kB128, kB256, kB512, kB1024}                           OPTIONAL,

qoe-PriorityBasedDiscarding-r18           ENUMERATED {supported}                                             OPTIONAL,

srb5-r18                                  ENUMERATED {supported}                                             OPTIONAL

]]

}

-- TAG-APPLAYERMEASPARAMETERS-STOP

-- ASN1STOP

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA, NR non-CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-v1540 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540

BandCombinationList-v1550 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1550

BandCombinationList-v1560 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1560

BandCombinationList-v1570 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1570

BandCombinationList-v1580 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1580

BandCombinationList-v1590 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1590

BandCombinationList-v15g0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v15g0

BandCombinationList-v15n0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v15n0

BandCombinationList-v1610 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610

BandCombinationList-v1630 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1630

BandCombinationList-v1640 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1640

BandCombinationList-v1650 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1650

BandCombinationList-v1680 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1680

BandCombinationList-v1690 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1690

BandCombinationList-v16a0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v16a0

BandCombinationList-v16j0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v16j0

BandCombinationList-v1700 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1700

BandCombinationList-v1720 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1720

BandCombinationList-v1730 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1730

BandCombinationList-v1740 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1740

BandCombinationList-v1760 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1760

BandCombinationList-v1770 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1770

BandCombinationList-v1780 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1780

BandCombinationList-v1790 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1790

BandCombinationList-v17b0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v17b0

BandCombinationList-v1800 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1800

BandCombinationList-v1830 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1830

BandCombinationList-v1840 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1840

BandCombinationList-v1860 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1860

BandCombinationList-v1900 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1900

BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16

BandCombinationList-UplinkTxSwitch-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1630

BandCombinationList-UplinkTxSwitch-v1640 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1640

BandCombinationList-UplinkTxSwitch-v1650 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1650

BandCombinationList-UplinkTxSwitch-v1670 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1670

BandCombinationList-UplinkTxSwitch-v1690 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1690

BandCombinationList-UplinkTxSwitch-v16a0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v16a0

BandCombinationList-UplinkTxSwitch-v16e0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v16e0

BandCombinationList-UplinkTxSwitch-v16j0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v16j0

BandCombinationList-UplinkTxSwitch-v1700 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1700

BandCombinationList-UplinkTxSwitch-v1720 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1720

BandCombinationList-UplinkTxSwitch-v1730 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1730

BandCombinationList-UplinkTxSwitch-v1740 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1740

BandCombinationList-UplinkTxSwitch-v1760 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1760

BandCombinationList-UplinkTxSwitch-v1770 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1770

BandCombinationList-UplinkTxSwitch-v1780 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1780

BandCombinationList-UplinkTxSwitch-v1790 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1790

BandCombinationList-UplinkTxSwitch-v17b0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v17b0

BandCombinationList-UplinkTxSwitch-v1800 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1800

BandCombinationList-UplinkTxSwitch-v1830 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1830

BandCombinationList-UplinkTxSwitch-v1840 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1840

BandCombinationList-UplinkTxSwitch-v1860 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1860

BandCombinationList-UplinkTxSwitch-v1900 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1900

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-v1540::=            SEQUENCE {

bandList-v1540                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540,

ca-ParametersNR-v1540               CA-ParametersNR-v1540                       OPTIONAL

}

BandCombination-v1550 ::=           SEQUENCE {

ca-ParametersNR-v1550               CA-ParametersNR-v1550

}

BandCombination-v1560::=            SEQUENCE {

ne-DC-BC                                ENUMERATED {supported}                 OPTIONAL,

ca-ParametersNRDC                       CA-ParametersNRDC                      OPTIONAL,

ca-ParametersEUTRA-v1560                CA-ParametersEUTRA-v1560               OPTIONAL,

ca-ParametersNR-v1560                   CA-ParametersNR-v1560                  OPTIONAL

}

BandCombination-v1570 ::=           SEQUENCE {

ca-ParametersEUTRA-v1570            CA-ParametersEUTRA-v1570

}

BandCombination-v1580 ::=           SEQUENCE {

mrdc-Parameters-v1580               MRDC-Parameters-v1580

}

BandCombination-v1590::=            SEQUENCE {

supportedBandwidthCombinationSetIntraENDC  BIT STRING (SIZE (1..32))           OPTIONAL,

mrdc-Parameters-v1590                      MRDC-Parameters-v1590

}

BandCombination-v15g0::=            SEQUENCE {

ca-ParametersNR-v15g0               CA-ParametersNR-v15g0                      OPTIONAL,

ca-ParametersNRDC-v15g0             CA-ParametersNRDC-v15g0                    OPTIONAL,

mrdc-Parameters-v15g0               MRDC-Parameters-v15g0                      OPTIONAL

}

BandCombination-v15n0::=            SEQUENCE {

mrdc-Parameters-v15n0               MRDC-Parameters-v15n0

}

BandCombination-v1610 ::=           SEQUENCE {

bandList-v1610                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610  OPTIONAL,

ca-ParametersNR-v1610               CA-ParametersNR-v1610                  OPTIONAL,

ca-ParametersNRDC-v1610             CA-ParametersNRDC-v1610                OPTIONAL,

powerClass-v1610                    ENUMERATED {pc1dot5}                   OPTIONAL,

powerClassNRPart-r16                ENUMERATED {pc1, pc2, pc3, pc5}        OPTIONAL,

featureSetCombinationDAPS-r16       FeatureSetCombinationId                OPTIONAL,

mrdc-Parameters-v1620               MRDC-Parameters-v1620                  OPTIONAL

}

BandCombination-v1630 ::=                   SEQUENCE {

ca-ParametersNR-v1630                       CA-ParametersNR-v1630                                             OPTIONAL,

ca-ParametersNRDC-v1630                     CA-ParametersNRDC-v1630                                           OPTIONAL,

mrdc-Parameters-v1630                       MRDC-Parameters-v1630                                             OPTIONAL,

supportedTxBandCombListPerBC-Sidelink-r16   BIT STRING (SIZE (1..maxBandComb))                                OPTIONAL,

supportedRxBandCombListPerBC-Sidelink-r16   BIT STRING (SIZE (1..maxBandComb))                                OPTIONAL,

scalingFactorTxSidelink-r16                 SEQUENCE (SIZE (1..maxBandComb)) OF ScalingFactorSidelink-r16     OPTIONAL,

scalingFactorRxSidelink-r16                 SEQUENCE (SIZE (1..maxBandComb)) OF ScalingFactorSidelink-r16     OPTIONAL

}

BandCombination-v1640 ::=                   SEQUENCE {

ca-ParametersNR-v1640                       CA-ParametersNR-v1640                                             OPTIONAL,

ca-ParametersNRDC-v1640                     CA-ParametersNRDC-v1640                                           OPTIONAL

}

BandCombination-v1650 ::=          SEQUENCE {

ca-ParametersNRDC-v1650             CA-ParametersNRDC-v1650                 OPTIONAL

}

BandCombination-v1680 ::=          SEQUENCE {

intrabandConcurrentOperationPowerClass-r16 SEQUENCE (SIZE (1..maxBandComb)) OF IntraBandPowerClass-r16     OPTIONAL

}

BandCombination-v1690 ::=          SEQUENCE {

dummy                              CA-ParametersNR-v1690                 OPTIONAL

}

BandCombination-v16a0 ::=          SEQUENCE {

ca-ParametersNR-v16a0              CA-ParametersNR-v16a0                    OPTIONAL,

ca-ParametersNRDC-v16a0            CA-ParametersNRDC-v16a0                  OPTIONAL

}

BandCombination-v16j0::=           SEQUENCE {

ca-ParametersNR-v16j0              CA-ParametersNR-v1690                    OPTIONAL,

ca-ParametersNRDC-v16j0            CA-ParametersNRDC-v16j0                  OPTIONAL

}

BandCombination-v1700 ::=          SEQUENCE {

ca-ParametersNR-v1700              CA-ParametersNR-v1700                    OPTIONAL,

ca-ParametersNRDC-v1700            CA-ParametersNRDC-v1700                  OPTIONAL,

mrdc-Parameters-v1700              MRDC-Parameters-v1700                    OPTIONAL,

bandList-v1710                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1710  OPTIONAL,

supportedBandCombListPerBC-SL-RelayDiscovery-r17      BIT STRING (SIZE (1..maxBandComb))               OPTIONAL,

supportedBandCombListPerBC-SL-NonRelayDiscovery-r17   BIT STRING (SIZE (1..maxBandComb))               OPTIONAL

}

BandCombination-v1720 ::=          SEQUENCE {

ca-ParametersNR-v1720              CA-ParametersNR-v1720                    OPTIONAL,

ca-ParametersNRDC-v1720            CA-ParametersNRDC-v1720                  OPTIONAL

}

BandCombination-v1730 ::=          SEQUENCE {

ca-ParametersNR-v1730              CA-ParametersNR-v1730                    OPTIONAL,

ca-ParametersNRDC-v1730            CA-ParametersNRDC-v1730                  OPTIONAL,

bandList-v1730                     SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1730  OPTIONAL

}

BandCombination-v1740 ::=          SEQUENCE {

dummy                              CA-ParametersNR-v1740                    OPTIONAL

}

BandCombination-v1760 ::=          SEQUENCE {

ca-ParametersNR-v1760              CA-ParametersNR-v1760,

ca-ParametersNRDC-v1760            CA-ParametersNRDC-v1760

}

BandCombination-v1770::=            SEQUENCE {

bandList-v1770                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1770,

mrdc-Parameters-v1770               MRDC-Parameters-v1770                      OPTIONAL,

ca-ParametersNR-v1770               CA-ParametersNR-v1770                      OPTIONAL

}

BandCombination-v1780 ::=          SEQUENCE {

ca-ParametersNR-v1780               CA-ParametersNR-v1780                                              OPTIONAL,

ca-ParametersNRDC-v1780             CA-ParametersNRDC-v1780                                            OPTIONAL,

bandList-v1780                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1780  OPTIONAL,

mrdc-Parameters-v1780               MRDC-Parameters-v1770                                              OPTIONAL

}

BandCombination-v1790 ::=                    SEQUENCE {

supportedIntraENDC-BandCombinationList-r17   SEQUENCE (SIZE (1..maxNrofIntraEndc-Components-r17)) OF SupportedIntraENDC-BandCombination-r17           OPTIONAL

}

BandCombination-v17b0::=           SEQUENCE {

ca-ParametersNR-v17b0              CA-ParametersNR-v1740                       OPTIONAL,

ca-ParametersNRDC-v17b0            CA-ParametersNRDC-v17b0                     OPTIONAL

}

BandCombination-v1800 ::=          SEQUENCE {

ca-ParametersNR-v1800               CA-ParametersNR-v1800                                                  OPTIONAL,

ca-ParametersNRDC-v1800             CA-ParametersNRDC-v1800                                                OPTIONAL,

supportedBandCombListPerBC-SL-U2U-RelayDiscovery-r18   BIT STRING (SIZE (1..maxBandComb))                  OPTIONAL,

bandList-v1810                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1810      OPTIONAL

}

BandCombination-v1830 ::=          SEQUENCE {

ca-ParametersNR-v1830               CA-ParametersNR-v1830                                                  OPTIONAL,

ca-ParametersNRDC-v1830             CA-ParametersNRDC-v1830                                                OPTIONAL

}

BandCombination-v1840 ::=          SEQUENCE {

mrdc-Parameters-v1840               MRDC-Parameters-v1840                                                  OPTIONAL

}

BandCombination-v1860 ::=          SEQUENCE {

ca-ParametersNR-v1860              CA-ParametersNR-v1860                                                   OPTIONAL

}

BandCombination-v1900 ::=          SEQUENCE {

ca-ParametersNR-v1900               CA-ParametersNR-v1900                                                  OPTIONAL,

ca-ParametersNRDC-v1900             CA-ParametersNRDC-v1900                                                OPTIONAL,

mrdc-Parameters-v1900               MRDC-Parameters-v1900                                                  OPTIONAL,

bandList-v1900                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1900      OPTIONAL

}

BandCombination-UplinkTxSwitch-r16 ::= SEQUENCE {

bandCombination-r16                 BandCombination,

bandCombination-v1540               BandCombination-v1540                      OPTIONAL,

bandCombination-v1560               BandCombination-v1560                      OPTIONAL,

bandCombination-v1570               BandCombination-v1570                      OPTIONAL,

bandCombination-v1580               BandCombination-v1580                      OPTIONAL,

bandCombination-v1590               BandCombination-v1590                      OPTIONAL,

bandCombination-v1610               BandCombination-v1610                      OPTIONAL,

supportedBandPairListNR-r16         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r16,

uplinkTxSwitching-OptionSupport-r16 ENUMERATED {switchedUL, dualUL, both}      OPTIONAL,

uplinkTxSwitching-PowerBoosting-r16 ENUMERATED {supported}                     OPTIONAL,

...,

[[

-- R4 16-5 UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching

uplinkTxSwitching-PUSCH-TransCoherence-r16     ENUMERATED {nonCoherent, fullCoherent}   OPTIONAL

]]

}

BandCombination-UplinkTxSwitch-v1630 ::=    SEQUENCE {

bandCombination-v1630                       BandCombination-v1630              OPTIONAL

}

BandCombination-UplinkTxSwitch-v1640 ::=    SEQUENCE {

bandCombination-v1640                       BandCombination-v1640              OPTIONAL

}

BandCombination-UplinkTxSwitch-v1650 ::= SEQUENCE {

bandCombination-v1650               BandCombination-v1650                      OPTIONAL

}

BandCombination-UplinkTxSwitch-v1670 ::= SEQUENCE {

bandCombination-v15g0                    BandCombination-v15g0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1690 ::=  SEQUENCE {

bandCombination-v1690                     BandCombination-v1690                OPTIONAL

}

BandCombination-UplinkTxSwitch-v16a0 ::= SEQUENCE {

bandCombination-v16a0                    BandCombination-v16a0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v16e0 ::= SEQUENCE {

bandCombination-v15n0                    BandCombination-v15n0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v16j0 ::= SEQUENCE {

bandCombination-v16j0                    BandCombination-v16j0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1700 ::= SEQUENCE {

bandCombination-v1700                    BandCombination-v1700                      OPTIONAL,

-- R4 16-1/16-2/16-3 Dynamic Tx switching between 2CC/3CC 2Tx-2Tx/1Tx-2Tx switching

supportedBandPairListNR-v1700            SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v1700  OPTIONAL,

-- R4 16-6: UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching

uplinkTxSwitchingBandParametersList-v1700 SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF UplinkTxSwitchingBandParameters-v1700  OPTIONAL

}

BandCombination-UplinkTxSwitch-v1720 ::= SEQUENCE {

bandCombination-v1720                    BandCombination-v1720                 OPTIONAL,

uplinkTxSwitching-OptionSupport2T2T-r17  ENUMERATED {switchedUL, dualUL, both} OPTIONAL

}

BandCombination-UplinkTxSwitch-v1730 ::= SEQUENCE {

bandCombination-v1730                    BandCombination-v1730                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1740 ::= SEQUENCE {

bandCombination-v1740                    BandCombination-v1740                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1760 ::= SEQUENCE {

bandCombination-v1760                    BandCombination-v1760                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1770 ::= SEQUENCE {

bandCombination-v1770                    BandCombination-v1770                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1780 ::= SEQUENCE {

bandCombination-v1780                    BandCombination-v1780                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1790 ::= SEQUENCE {

bandCombination-v1790                    BandCombination-v1790                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v17b0 ::= SEQUENCE {

bandCombination-v17b0                    BandCombination-v17b0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1800 ::=     SEQUENCE {

bandCombination-v1800                        BandCombination-v1800                                                         OPTIONAL,

supportedBandPairListNR-r18                  SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r18   OPTIONAL,

-- R1 49-Y: Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots

uplinkTxSwitchingMinimumSeparationTime-r18   ENUMERATED {n0us, n500us}                                                     OPTIONAL,

-- R4 38-4: Switching Period for unaffected Band for Dual UL

uplinkTxSwitchingAdditionalPeriodDualUL-List-r18 SEQUENCE (SIZE (1..maxULTxSwitchingBetweenBandPairs-r18)) OF

UplinkTxSwitchingAdditionalPeriodDualUL-r18                     OPTIONAL,

-- R4 38-6: Switching period restriction for fallback band combination

switchingPeriodRestriction-r18               ENUMERATED {true}                                                             OPTIONAL

}

BandCombination-UplinkTxSwitch-v1830 ::=     SEQUENCE {

bandCombination-v1830                        BandCombination-v1830                                                         OPTIONAL

}

BandCombination-UplinkTxSwitch-v1840 ::= SEQUENCE {

bandCombination-v1840                    BandCombination-v1840                                                          OPTIONAL,

supportedBandPairListNR-v1840            SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v1840  OPTIONAL

}

BandCombination-UplinkTxSwitch-v1860 ::= SEQUENCE {

bandCombination-v1860                    BandCombination-v1860                                                          OPTIONAL

}

BandCombination-UplinkTxSwitch-v1900 ::= SEQUENCE {

bandCombination-v1900                    BandCombination-v1900                                                          OPTIONAL,

-- R1 67-5: Enhanced handling of simultaneous SRS carrier switching and uplink Tx switching

simultaneousSRS-UplinkTxSwitch-r19       ENUMERATED {max, sum}                                                          OPTIONAL,

supportedBandPairListNR-v1900            SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v1900  OPTIONAL,

uplinkTxSwitchingBandParametersList-v1900 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF UplinkTxSwitchingBandParameters-v1900

OPTIONAL

}

ULTxSwitchingBandPair-r16 ::=       SEQUENCE {

bandIndexUL1-r16                    INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r16                    INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingPeriod-r16         ENUMERATED {n35us, n140us, n210us},

uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL

}

ULTxSwitchingBandPair-v1700 ::=     SEQUENCE {

uplinkTxSwitchingPeriod2T2T-r17     ENUMERATED {n35us, n140us, n210us}     OPTIONAL

}

ULTxSwitchingBandPair-r18 ::=                             SEQUENCE {

bandIndexUL1-r18                                           INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r18                                           INTEGER(1..maxSimultaneousBands),

-- R1 49-X: Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands

uplinkTxSwitchingOptionForBandPair-r18                     ENUMERATED {switchedUL, dualUL, both},

-- R4 38-1: Switching period for dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL up to two TAGs

uplinkTxSwitchingPeriodForBandPair-r18                     SEQUENCE {

switchingPeriodFor2T-r18                                 ENUMERATED {n35us, n140us, n210us}                       OPTIONAL,

switchingPeriodFor1T-r18                                 ENUMERATED {n35us, n140us, n210us}

},

-- R4 38-2: Application of DL interruptions due to dynamic UL Tx switching

uplinkTxSwitching-DL-Interruption-r18                      BIT STRING (SIZE(1..maxSimultaneousBands))                   OPTIONAL,

-- R4 38-3: Switching Period for unaffected Band for Dual UL

uplinkTxSwitchingPeriodUnaffectedBandDualUL-List-r18       SEQUENCE (SIZE (1..maxSimultaneousBands-2-r18)) OF

SwitchingPeriodUnaffectedBandDualUL-r18            OPTIONAL

}

ULTxSwitchingBandPair-v1840 ::=     SEQUENCE {

-- R1 49-Z: Support of 2-band configuration of 1T-1T UL Tx switching by using Rel-18 UL Tx switching configurations

configured1T1T-OnTwoBands-r18       ENUMERATED {supported}                                                              OPTIONAL

}

ULTxSwitchingBandPair-v1900 ::=     SEQUENCE {

-- R4 57-1: Switch period for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band

uplink3TxSwitchingPeriodUpTo2TPerBandDualUL-v1900         ENUMERATED {n35us, n140us, n210us},

-- R4 57-2: Application of DL interruptions due to dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band

uplinkTxSwitching-DL-Interruption-DualUL-v1900            BIT STRING (SIZE(1..maxSimultaneousBands))                    OPTIONAL

}

UplinkTxSwitchingBandParameters-v1700 ::=                 SEQUENCE {

bandIndex-r17                                              INTEGER(1..maxSimultaneousBands),

-- R4 38-5: UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among up to 4 bands

uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17             ENUMERATED {nonCoherent, fullCoherent}                       OPTIONAL

}

UplinkTxSwitchingBandParameters-v1900 ::=                 SEQUENCE {

bandIndex-r19                                             INTEGER(1..maxSimultaneousBands),

-- R4 57-3: UL-MIMO coherence capability for dynamic UL Tx switching between 2 bands for 3Tx UE with up to 2Tx per band

uplinkTxSwitching3Tx-PUSCH-TransCoherence-DualUL-v1900    ENUMERATED {nonCoherent, fullCoherent}                        OPTIONAL

}

UplinkTxSwitchingAdditionalPeriodDualUL-r18::=            SEQUENCE {

uplinkTxSwitchingBetweenBandPairs-r18                     SEQUENCE {

bandPairIndex1-r18                                        INTEGER(1.. maxULTxSwitchingBandPairs),

anotherBandPairOrBand-r18                                 CHOICE {

bandPairIndex2-r18                                        INTEGER(1.. maxULTxSwitchingBandPairs),

bandIndex-r18                                             INTEGER(1..maxSimultaneousBands)

}

},

-- R4 38-4: Additional switching Period for switching case across three or four bands for Dual UL

switchingAdditionalPeriodDualUL-r18                   ENUMERATED {n35us, n140us, n210us}

}

SwitchingPeriodUnaffectedBandDualUL-r18::=                SEQUENCE {

bandIndexUnaffected-r18                                   INTEGER(1..maxSimultaneousBands),

periodUnaffectedBandDualUL-r18                            CHOICE {

maintainedUL-Trans-r18                                    NULL,

periodOnULBands-r18                                       ENUMERATED {n35us, n140us, n210us}

}

}

BandParameters ::=                      CHOICE {

eutra                               SEQUENCE {

bandEUTRA                           FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL,

ca-BandwidthClassUL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL

},

nr                                  SEQUENCE {

bandNR                              FreqBandIndicatorNR,

ca-BandwidthClassDL-NR              CA-BandwidthClassNR                    OPTIONAL,

ca-BandwidthClassUL-NR              CA-BandwidthClassNR                    OPTIONAL

}

}

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1610 ::=         SEQUENCE {

srs-TxSwitch-v1610               SEQUENCE {

supportedSRS-TxPortSwitch-v1610  ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL

}

BandParameters-v1710 ::=         SEQUENCE {

-- R1 23-8-3	SRS Antenna switching for >4Rx

srs-AntennaSwitchingBeyond4RX-r17                     SEQUENCE {

-- 1. Support of SRS antenna switching xTyR with y>4

supportedSRS-TxPortSwitchBeyond4Rx-r17                BIT STRING (SIZE (11)),

-- 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL

entryNumberAffectBeyond4Rx-r17                        INTEGER (1..32)      OPTIONAL,

-- 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL

entryNumberSwitchBeyond4Rx-r17                        INTEGER (1..32)      OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1730 ::= SEQUENCE {

-- R1 39-3-2	Affected bands for inter-band CA during SRS carrier switching

srs-SwitchingAffectedBandsListNR-r17    SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingAffectedBandsNR-r17

}

BandParameters-v1770 ::=         SEQUENCE {

ca-BandwidthClassDL-NR-r17       CA-BandwidthClassNR-r17                    OPTIONAL,

ca-BandwidthClassUL-NR-r17       CA-BandwidthClassNR-r17                    OPTIONAL

}

BandParameters-v1780 ::=         SEQUENCE {

ca-BandwidthClassDL-NR-r17       CA-BandwidthClassNR-r17                    OPTIONAL,

ca-BandwidthClassUL-NR-r17       CA-BandwidthClassNR-r17                    OPTIONAL,

supportedAggBW-FR2-r17           SEQUENCE {

supportedAggBW-DL-r17            SupportedAggBandwidth-r17               OPTIONAL,

supportedAggBW-UL-r17            SupportedAggBandwidth-r17               OPTIONAL

}    OPTIONAL

}

BandParameters-v1810 ::=         SEQUENCE {

-- R1 40-5-4: SRS 8 Tx ports-antenna switching

srs-AntennaSwitching8T8R-r18     SEQUENCE {

antennaSwitch8T8R-r18            ENUMERATED {noTdm, tdmAndNoTdm}        OPTIONAL,

downgradeConfig-r18          CHOICE {

empty-r18                  NULL,

downgrade-r18              BIT STRING (SIZE (11))

}                                                                       OPTIONAL,

entryNumberAffect-r18        INTEGER (1..32)                            OPTIONAL,

entryNumberSwitch-r18        INTEGER (1..32)                            OPTIONAL

}                                                                           OPTIONAL

}

BandParameters-v1900 ::=         SEQUENCE {

-- R1 59-3-3: 3T6R Antenna switching

srs-AntennaSwitching3T6R-r19     SEQUENCE {

entryNumberAffect-r19            INTEGER (1..32)                        OPTIONAL,

entryNumberSwitch-r19            INTEGER (1..32)                        OPTIONAL

}                                                                           OPTIONAL,

-- R1 59-3-3a: 3T3R Antenna switching

srs-AntennaSwitching3T3R-r19     SEQUENCE {

entryNumberAffect-r19            INTEGER (1..32)                        OPTIONAL,

entryNumberSwitch-r19            INTEGER (1..32)                        OPTIONAL

}                                                                           OPTIONAL

}

ScalingFactorSidelink-r16 ::=       ENUMERATED {f0p4, f0p75, f0p8, f1}

IntraBandPowerClass-r16 ::=         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

SRS-SwitchingAffectedBandsNR-r17 ::= BIT STRING (SIZE (1..maxSimultaneousBands))

SupportedIntraENDC-BandCombination-r17 ::=       SEQUENCE {

supportedBandwidthCombinationSetIntraENDC-v1790  BIT STRING (SIZE (1..32))           OPTIONAL,

mrdc-Parameters-v1790                            MRDC-Parameters-v1790               OPTIONAL

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

BandCombination field descriptions

BandCombinationList-v1540, BandCombinationList-v1550, BandCombinationList-v1560, BandCombinationList-v1570, BandCombinationList-v1580, BandCombinationList-v1590, BandCombinationList-v15g0, BandCombinationList-v15n0, BandCombinationList-v1610, BandCombinationList-v1630, BandCombinationList-v1640, BandCombinationList-v1650, BandCombinationList-v1680, BandCombinationList-v1690, BandCombinationList-v16a0, BandCombinationList-v16j0, BandCombinationList-v1700, BandCombinationList-v1720, BandCombinationList-v1730, BandCombinationList-v1760, BandCombinationList-v1780, BandCombinationList-v1790, BandCombinationList-v17b0, BandCombinationList-v1800, BandCombinationList-v1830, BandCombinationList-v1840, BandCombinationList-v1860, BandCombinationList-v1900 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix). If the field is included in supportedBandCombinationListNEDC-Only-v1610, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList of supportedBandCombinationListNEDC-Only (without suffix) field. If the field is included in supportedBandCombinationListNEDC-Only-v15a0, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix) of supportedBandCombinationListNEDC-Only (without suffix) field.

BandCombinationList-UplinkTxSwitch-r16, BandCombinationList-UplinkTxSwitch-v1630, BandCombinationList-UplinkTxSwitch-v1640, BandCombinationList-UplinkTxSwitch-v1650, BandCombinationList-UplinkTxSwitch-v1690, BandCombinationList-UplinkTxSwitch-v16a0, BandCombinationList-UplinkTxSwitch-v16e0, BandCombinationList-UplinkTxSwitch-v16j0, BandCombinationList-UplinkTxSwitch-v1700, BandCombinationList-UplinkTxSwitch-v1720, BandCombinationList-UplinkTxSwitch-v1730, BandCombinationList-UplinkTxSwitch-v1760, BandCombinationList-UplinkTxSwitch-v1780, BandCombinationList-UplinkTxSwitch-v1790, BandCombinationList-UplinkTxSwitch-v17b0, BandCombinationList-UplinkTxSwitch-v1800, BandCombinationList-UplinkTxSwitch-v1830, BandCombinationList-UplinkTxSwitch-v1840, BandCombinationList-UplinkTxSwitch-v1860, BandCombinationList-UplinkTxSwitch-v1900 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList-UplinkTxSwitch-r16. For the field of supportedBandCombinationList-UplinkTxSwitch-v1700, if the UE does not support 2Tx-2Tx switching for a given band combination, the field of supportedBandPairListNR-v1700 in the corresponding entry is absent.

ca-ParametersNRDC If the field (without suffix) is included for a band combination in the NR capability container, the field (without suffix) indicates support of NR-DC. Otherwise, the field is absent. If a version of the field (with suffix) is absent for a band combination, ca-ParametersNR field version in BandCombination corresponding to the ca-ParametersNR-ForDC field version in the field (with suffix) is applicable to the UE configured with NR-DC for the band combination.

featureSetCombinationDAPS If this field is present for a band combination, it reports the feature set combination supported for the band combination when any DAPS bearer is configured.

ne-DC-BC If the field is included for a band combination in the MR-DC capability container, the field indicates support of NE-DC. Otherwise, the field is absent.

supportedBandPairListNR-r16, supportedBandPairListNR-v1700, supportedBandPairListNR-v1900 Indicates a list of band pair supporting UL Tx switching as defined in TS 38.101-1 [15] for a given band combination. A UE supporting 2Tx-2Tx switching should include both of supportedBandPairListNR-r16 and supportedBandPairListNR-v1700. And the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r16. If the UE does not support 2Tx-2Tx switching for a given band pair, the field of uplinkTxSwitchingPeriod2T2T in the corresponding entry is absent. A 3Tx UE supporting Tx switching between 2 UL bands with up to 2Tx per band should indicate both of supportedBandPairListNR-r16 and supportedBandPairListNR-v1900. And the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r16.

supportedBandPairListNR-r18, supportedBandPairListNR-v1840 Indicates a list of band pair supporting UL Tx switching up to 4 bands as defined in TS 38.101-1 [15] for a given band combination. The UE shall include all the possible band pairs. If supportedBandPairListNR-v1840 is included, the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r18. For a band pair only supporting 1Tx-1Tx switching, the UE should include switchingPeriodFor1T in ULTxSwitchingBandPair-r18. For a band pair supporting 1Tx-2Tx switching, the UE always supports 1Tx-1Tx switching, and the UE should include switchingPeriodFor1T in ULTxSwitchingBandPair-r18. For a band pair supporting 2Tx-2Tx switching, the UE always supports 1Tx-2Tx switching and 1Tx-1Tx switching, the UE should include switchingPeriodFor2T as well as switchingPeriodFor1T in ULTxSwitchingBandPair-r18.

srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on

srs-SwitchingTimesListEUTRA Indicates, for a particular pair of E-UTRA bands, the RF retuning time when switching between an E-UTRA carrier corresponding to this band entry and another (PUSCH-less) E-UTRA carrier corresponding to the band entry in the order indicated below: - For the first E-UTRA band, the UE shall include the same number of entries for E-UTRA bands as in bandList, i.e. first entry corresponds to first E-UTRA band in bandList and so on, - For the second E-UTRA band, the UE shall include one entry less, i.e. first entry corresponds to the second E-UTRA band in bandList and so on - And so on

srs-TxSwitch Indicates supported SRS antenna switch capability for the associated band. If the UE indicates support of SRS-SwitchingTimeNR, the UE is allowed to set this field for a band with associated FeatureSetUplinkId set to 0 for SRS carrier switching.

supportedIntraENDC-BandCombinationList Indicates BCS and/or spectrum contiguity capability for each entry in a list of intra-band (NG)EN-DC components in an inter-band (NG)EN-DC band combination. The UE shall include the entries in the order corresponding to the order of NR band entries of the intra-band (NG)EN-DC components in the bandList in the inter-band (NG)EN-DC band combination (i.e., BandCombination without suffix).

uplinkTxSwitchingBandParametersList-v1700, uplinkTxSwitchingBandParametersList-v1900 Indicates a list of per band per band combination capabilities for UL Tx switching.

#### –	BandCombinationListSidelinkEUTRA-NR

The IE BandCombinationListSidelinkEUTRA-NR contains a list of V2X sidelink and NR sidelink band combinations.

BandCombinationListSidelinkEUTRA-NR information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-START

BandCombinationListSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-r16

BandCombinationListSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1630

BandCombinationListSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1710

BandCombinationParametersSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-r16

BandCombinationParametersSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1630

BandCombinationParametersSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1710

BandParametersSidelinkEUTRA-NR-r16 ::= CHOICE {

eutra                                  SEQUENCE {

bandParametersSidelinkEUTRA1-r16       OCTET STRING                         OPTIONAL,

bandParametersSidelinkEUTRA2-r16       OCTET STRING                         OPTIONAL

},

nr                                     SEQUENCE {

bandParametersSidelinkNR-r16           BandParametersSidelink-r16

}

}

BandParametersSidelinkEUTRA-NR-v1630 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

tx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

rx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

sl-CrossCarrierScheduling-r16            ENUMERATED {supported}                          OPTIONAL

}

}

BandParametersSidelinkEUTRA-NR-v1710 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

--32-4

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--32-2a:  Receiving NR sidelink of PSFCH

rx-sidelinkPSFCH-r17                     ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL,

--32-5b-1

tx-IUC-Scheme2-Mode2Sidelink-r17         ENUMERATED {n4, n8, n16}                          OPTIONAL

}

}

BandParametersSidelink-r16 ::= SEQUENCE {

freqBandSidelink-r16           FreqBandIndicatorNR

}

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-STOP

-- ASN1STOP

BandParametersSidelinkEUTRA-NR field descriptions

bandParametersSidelinkEUTRA1, bandParametersSidelinkEUTRA2 This field includes the V2X-BandParameters-r14 and V2X-BandParameters-v1530 IE as specified in 36.331 [10]. It is used for reporting the per-band capability for V2X sidelink communication.

#### –	BandCombinationListSL-Discovery

The IE BandCombinationListSL-Discovery contains a list of NR Sidelink discovery band combinations.

BandCombinationListSidelinkSL-Discovery information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSLDISCOVERY-START

BandCombinationListSL-Discovery-r17 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkDiscovery-r17

BandParametersSidelinkDiscovery-r17 ::= SEQUENCE {

sl-CrossCarrierScheduling-r17            ENUMERATED {supported}                            OPTIONAL,

--R1 32-4: Transmitting NR sidelink mode 2 with partial sensing

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--R1 32-5a-1: Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL

}

-- TAG-BANDCOMBINATIONLISTSLDISCOVERY-STOP

-- ASN1STOP

#### –	CA-BandwidthClassEUTRA

The IE CA-BandwidthClassEUTRA indicates the E-UTRA CA bandwidth class as defined in TS 36.101 [22], table 5.6A-1.

CA-BandwidthClassEUTRA information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=          ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

#### –	CA-BandwidthClassNR

The IE CA-BandwidthClassNR indicates the NR CA bandwidth class as defined in TS 38.101-1 [15], table 5.3A.5-1 and TS 38.101-2 [39], table 5.3A.4-1.

CA-BandwidthClassNR information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

-- R4 17-6: new CA BW Classes R2-R12

-- R4 17-7: new CA BW Classes V, W

CA-BandwidthClassNR ::=             ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...,r2-v1730, r3-v1730, r4-v1730, r5-v1730, r6-v1730, r7-v1730, r8-v1730, r9-v1730, r10-v1730, r11-v1730, r12-v1730,v-v1770, w-v1770 }

CA-BandwidthClassNR-r17 ::=         ENUMERATED {r, s, t, u, ...}

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

#### –	CA-ParametersEUTRA

The IE CA-ParametersEUTRA contains the E-UTRA part of band combination parameters for a given MR-DC band combination.

NOTE:	If additional E-UTRA band combination parameters are defined in TS 36.331 [10], which are supported for MR-DC, they will be defined here as well.

CA-ParametersEUTRA information element

-- ASN1START

-- TAG-CA-PARAMETERSEUTRA-START

CA-ParametersEUTRA ::=                          SEQUENCE {

multipleTimingAdvance                           ENUMERATED {supported}                          OPTIONAL,

simultaneousRx-Tx                               ENUMERATED {supported}                          OPTIONAL,

supportedNAICS-2CRS-AP                          BIT STRING (SIZE (1..8))                        OPTIONAL,

additionalRx-Tx-PerformanceReq                  ENUMERATED {supported}                          OPTIONAL,

ue-CA-PowerClass-N                              ENUMERATED {class2}                             OPTIONAL,

supportedBandwidthCombinationSetEUTRA-v1530     BIT STRING (SIZE (1..32))                       OPTIONAL,

...

}

CA-ParametersEUTRA-v1560 ::=                    SEQUENCE {

fd-MIMO-TotalWeightedLayers                     INTEGER (2..128)                                OPTIONAL

}

CA-ParametersEUTRA-v1570 ::=                    SEQUENCE {

dl-1024QAM-TotalWeightedLayers                  INTEGER (0..10)                                 OPTIONAL

}

-- TAG-CA-PARAMETERSEUTRA-STOP

-- ASN1STOP

#### –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)     OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)    OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

CA-ParametersNR-v1550 ::=           SEQUENCE {

dummy                               ENUMERATED {supported}                      OPTIONAL

}

CA-ParametersNR-v1560 ::=           SEQUENCE {

diffNumerologyWithinPUCCH-GroupLargerSCS      ENUMERATED {supported}            OPTIONAL

}

CA-ParametersNR-v15g0 ::=           SEQUENCE {

simultaneousRxTxInterBandCAPerBandPair        SimultaneousRxTxPerBandPair       OPTIONAL,

simultaneousRxTxSULPerBandPair                SimultaneousRxTxPerBandPair       OPTIONAL

}

CA-ParametersNR-v1610 ::=           SEQUENCE {

-- R1 9-3: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA

parallelTxMsgA-SRS-PUCCH-PUSCH-r16                ENUMERATED {supported}        OPTIONAL,

-- R1 9-4: MsgA operation in a band combination including SUL

msgA-SUL-r16                                      ENUMERATED {supported}        OPTIONAL,

-- R1 10-9c: Joint search space group switching across multiple cells

jointSearchSpaceSwitchAcrossCells-r16             ENUMERATED {supported}        OPTIONAL,

-- R1 14-5: Half-duplex UE behaviour in TDD CA for same SCS

half-DuplexTDD-CA-SameSCS-r16                     ENUMERATED {supported}        OPTIONAL,

-- R1 18-4: SCell dormancy within active time

scellDormancyWithinActiveTime-r16                 ENUMERATED {supported}        OPTIONAL,

-- R1 18-4a: SCell dormancy outside active time

scellDormancyOutsideActiveTime-r16                ENUMERATED {supported}        OPTIONAL,

-- R1 18-6: Cross-carrier A-CSI RS triggering with different SCS

crossCarrierA-CSI-trigDiffSCS-r16                 ENUMERATED {higherA-CSI-SCS,lowerA-CSI-SCS,both}   OPTIONAL,

-- R1 18-6a: Default QCL assumption for cross-carrier A-CSI-RS triggering

defaultQCL-CrossCarrierA-CSI-Trig-r16             ENUMERATED {diffOnly, both}   OPTIONAL,

-- R1 18-7: CA with non-aligned frame boundaries for inter-band CA

interCA-NonAlignedFrame-r16                       ENUMERATED {supported}        OPTIONAL,

simul-SRS-Trans-BC-r16                            ENUMERATED {n2}               OPTIONAL,

interFreqDAPS-r16                                 SEQUENCE {

interFreqAsyncDAPS-r16                            ENUMERATED {supported}    OPTIONAL,

interFreqDiffSCS-DAPS-r16                         ENUMERATED {supported}    OPTIONAL,

interFreqMultiUL-TransmissionDAPS-r16             ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode1-r16     ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode2-r16     ENUMERATED {supported}    OPTIONAL,

interFreqDynamicPowerSharingDAPS-r16              ENUMERATED {short, long}  OPTIONAL,

interFreqUL-TransCancellationDAPS-r16             ENUMERATED {supported}    OPTIONAL

}                                                                               OPTIONAL,

codebookParametersPerBC-r16                       CodebookParameters-v1610      OPTIONAL,

-- R1 16-2a-10 Value of R for BD/CCE

blindDetectFactor-r16                             INTEGER (1..2)                OPTIONAL,

-- R1 11-2a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r16                            SEQUENCE {

maxNumberOfMonitoringCC-r16                       INTEGER (2..16),

supportedSpanArrangement-r16                      ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                               OPTIONAL,

-- R1 11-2c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers

pdcch-BlindDetectionCA-Mixed-r16                  SEQUENCE {

pdcch-BlindDetectionCA1-r16                       INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                       INTEGER (1..15),

supportedSpanArrangement-r16                      ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                               OPTIONAL,

-- R1 11-2d: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for

-- SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-BlindDetectionMCG-UE-r16                    INTEGER (1..14)               OPTIONAL,

pdcch-BlindDetectionSCG-UE-r16                    INTEGER (1..14)               OPTIONAL,

-- R1 11-2e: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and

-- Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMCG-UE-Mixed-r16              SEQUENCE {

pdcch-BlindDetectionMCG-UE1-r16                   INTEGER (0..15),

pdcch-BlindDetectionMCG-UE2-r16                   INTEGER (0..15)

}                                                                               OPTIONAL,

pdcch-BlindDetectionSCG-UE-Mixed-r16              SEQUENCE {

pdcch-BlindDetectionSCG-UE1-r16                   INTEGER (0..15),

pdcch-BlindDetectionSCG-UE2-r16                   INTEGER (0..15)

}                                                                               OPTIONAL,

-- R1 18-5 cross-carrier scheduling with different SCS in DL CA

crossCarrierSchedulingDL-DiffSCS-r16              ENUMERATED {low-to-high, high-to-low, both} OPTIONAL,

-- R1 18-5a Default QCL assumption for cross-carrier scheduling

crossCarrierSchedulingDefaultQCL-r16              ENUMERATED {diff-only, both}  OPTIONAL,

-- R1 18-5b cross-carrier scheduling with different SCS in UL CA

crossCarrierSchedulingUL-DiffSCS-r16              ENUMERATED {low-to-high, high-to-low, both} OPTIONAL,

-- R1 13.19a Simultaneous positioning SRS and MIMO SRS transmission for a given BC

simul-SRS-MIMO-Trans-BC-r16                       ENUMERATED {n2}               OPTIONAL,

-- R1 16-3a, 16-3a-1, 16-3b, 16-3b-1: New Individual Codebook

codebookParametersAdditionPerBC-r16               CodebookParametersAdditionPerBC-r16         OPTIONAL,

-- R1 16-8: Mixed codebook

codebookComboParametersAdditionPerBC-r16          CodebookComboParametersAdditionPerBC-r16    OPTIONAL

}

CA-ParametersNR-v1630 ::= SEQUENCE {

-- R1 22-5b: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for inter-band UL CA

-- R1 22-5d: Simultaneous transmission of SRS for antenna switching for inter-band UL CA

simulTX-SRS-AntSwitchingInterBandUL-CA-r16        SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R4 8-5: supported beam management type for inter-band CA

beamManagementType-r16                            ENUMERATED {ibm, dummy}                       OPTIONAL,

-- R4 7-3a: UL frequency separation class with aggregate BW and Gap BW

intraBandFreqSeparationUL-AggBW-GapBW-r16         ENUMERATED {classI, classII, classIII}      OPTIONAL,

-- RAN 89: Case B in case of Inter-band CA with non-aligned frame boundaries

interCA-NonAlignedFrame-B-r16                     ENUMERATED {supported}                      OPTIONAL

}

CA-ParametersNR-v1640 ::= SEQUENCE {

-- R4 7-5: Support of reporting UL Tx DC locations for uplink intra-band CA.

uplinkTxDC-TwoCarrierReport-r16                               ENUMERATED {supported}          OPTIONAL,

-- RAN 22-6: Support of up to 3 different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA

-- where UE is not configured with two NR PUCCH groups

maxUpTo3Diff-NumerologiesConfigSinglePUCCH-grp-r16            PUCCH-Grp-CarrierTypes-r16      OPTIONAL,

-- RAN 22-6a: Support of up to 4 different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA

-- where UE is not configured with two NR PUCCH groups

maxUpTo4Diff-NumerologiesConfigSinglePUCCH-grp-r16            PUCCH-Grp-CarrierTypes-r16      OPTIONAL,

-- RAN 22-7: Support two PUCCH groups for NR-CA with 3 or more bands with at least two carrier types

twoPUCCH-Grp-ConfigurationsList-r16 SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r16)) OF TwoPUCCH-Grp-Configurations-r16 OPTIONAL,

-- R1 22-7a: Different numerology across NR PUCCH groups

diffNumerologyAcrossPUCCH-Group-CarrierTypes-r16              ENUMERATED {supported}          OPTIONAL,

-- R1 22-7b: Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of smaller SCS

diffNumerologyWithinPUCCH-GroupSmallerSCS-CarrierTypes-r16    ENUMERATED {supported}          OPTIONAL,

-- R1 22-7c: Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of larger SCS

diffNumerologyWithinPUCCH-GroupLargerSCS-CarrierTypes-r16     ENUMERATED {supported}          OPTIONAL,

-- R1 11-2f: add the replicated FGs of 11-2a/c with restriction for non-aligned span case

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-NonAlignedSpan-r16                         INTEGER (2..16)                 OPTIONAL,

-- R1 11-2g: add the replicated FGs of 11-2a/c with restriction for non-aligned span case

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16               SEQUENCE {

pdcch-BlindDetectionCA1-r16                                   INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                                   INTEGER (1..15)

}                                                                                             OPTIONAL

}

CA-ParametersNR-v1690 ::= SEQUENCE {

csi-ReportingCrossPUCCH-Grp-r16          SEQUENCE {

computationTimeForA-CSI-r16              ENUMERATED {sameAsNoCross, relaxed},

additionalSymbols-r16                    SEQUENCE {

scs-15kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-30kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-60kHz-additionalSymbols-r16          ENUMERATED {s14, s28, s56}       OPTIONAL,

scs-120kHz-additionalSymbols-r16         ENUMERATED {s14, s28, s56}       OPTIONAL

}                                                                             OPTIONAL,

sp-CSI-ReportingOnPUCCH-r16              ENUMERATED {supported}               OPTIONAL,

sp-CSI-ReportingOnPUSCH-r16              ENUMERATED {supported}               OPTIONAL,

carrierTypePairList-r16                  SEQUENCE (SIZE (1..maxCarrierTypePairList-r16)) OF CarrierTypePair-r16

}                                                                                 OPTIONAL

}

CA-ParametersNR-v16a0 ::= SEQUENCE {

pdcch-BlindDetectionMixedList-r16    SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF PDCCH-BlindDetectionMixedList-r16

}

CA-ParametersNR-v1700 ::= SEQUENCE {

-- R1 23-9-1: Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) per band combination information

codebookParametersfetype2PerBC-r17               CodebookParametersfetype2PerBC-r17           OPTIONAL,

-- R4 18-4: Support of enhanced Demodulation requirements for CA in HST SFN FR1

demodulationEnhancementCA-r17                    ENUMERATED {supported}                       OPTIONAL,

-- R4 20-1: Maximum uplink duty cycle for NR inter-band CA power class 2

maxUplinkDutyCycle-interBandCA-PC2-r17           ENUMERATED {n50, n60, n70, n80, n90, n100}   OPTIONAL,

-- R4 20-2: Maximum uplink duty cycle for NR SUL combination power class 2

maxUplinkDutyCycle-SULcombination-PC2-r17        ENUMERATED {n50, n60, n70, n80, n90, n100}   OPTIONAL,

beamManagementType-CBM-r17                       ENUMERATED {supported}                       OPTIONAL,

-- R1 25-18: Parallel PUCCH and PUSCH transmission across CCs in inter-band CA

parallelTxPUCCH-PUSCH-r17                        ENUMERATED {supported}      OPTIONAL,

-- R1 23-9-5	Active CSI-RS resources and ports for mixed codebook types in any slot per band combination

codebookComboParameterMixedTypePerBC-r17         CodebookComboParameterMixedTypePerBC-r17     OPTIONAL,

-- R1 23-7-1	Basic Features of CSI Enhancement for Multi-TRP

mTRP-CSI-EnhancementPerBC-r17                    SEQUENCE {

maxNumNZP-CSI-RS-r17                             INTEGER (2..8),

cSI-Report-mode-r17                              ENUMERATED {mode1, mode2, both},

supportedComboAcrossCCs-r17                      SEQUENCE (SIZE (1..16)) OF CSI-MultiTRP-SupportedCombinations-r17,

codebookMode-NCJT-r17	ENUMERATED{mode1,mode1And2}

}                                                                                             OPTIONAL,

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

codebookComboParameterMultiTRP-PerBC-r17         CodebookComboParameterMultiTRP-PerBC-r17     OPTIONAL,

-- R1 24-8b: 32 DL HARQ processes for FR 2-2 - maximum number of component carriers

maxCC-32-DL-HARQ-ProcessFR2-2-r17                ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32} OPTIONAL,

-- R1 24-9b: 32 UL HARQ processes for FR 2-2 - maximum number of component carriers

maxCC-32-UL-HARQ-ProcessFR2-2-r17                ENUMERATED {n1, n2, n3, n4, n5, n8, n16, n32}  OPTIONAL,

-- R1 34-2: Cross-carrier scheduling from SCell to PCell/PSCell (Type B)

crossCarrierSchedulingSCell-SpCellTypeB-r17      CrossCarrierSchedulingSCell-SpCell-r17       OPTIONAL,

-- R1 34-1: Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A)

crossCarrierSchedulingSCell-SpCellTypeA-r17      CrossCarrierSchedulingSCell-SpCell-r17       OPTIONAL,

-- R1 34-1a: DCI formats on PCell/PSCell USS set(s) support

dci-FormatsPCellPSCellUSS-Sets-r17               ENUMERATED {supported}                       OPTIONAL,

-- R1 34-3: Disabling scaling factor alpha when sSCell is deactivated

disablingScalingFactorDeactSCell-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 34-4: Disabling scaling factor alpha when sSCell is deactivated

disablingScalingFactorDormantSCell-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 34-5: Non-aligned frame boundaries between PCell/PSCell and sSCell

non-AlignedFrameBoundaries-r17 SEQUENCE {

scs15kHz-15kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs15kHz-30kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs15kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs30kHz-30kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs30kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs60kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL

}                                                                                             OPTIONAL

}

CA-ParametersNR-v1720 ::= SEQUENCE {

-- R1 39-1: Parallel SRS and PUCCH/PUSCH transmission across CCs in intra-band non-contiguous CA

parallelTxSRS-PUCCH-PUSCH-intraBand-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 39-2: Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA

parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 25-9: Semi-static PUCCH cell switching for a single PUCCH group only

semiStaticPUCCH-CellSwitchSingleGroup-r17        SEQUENCE {

pucch-Group-r17                                ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                           PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-9a: Semi-static PUCCH cell switching for two PUCCH groups

semiStaticPUCCH-CellSwitchTwoGroups-r17    SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17 OPTIONAL,

-- R1 25-10: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots for a single

-- PUCCH group only

dynamicPUCCH-CellSwitchSameLengthSingleGroup-r17 SEQUENCE {

pucch-Group-r17                                  ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                       PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-10a: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots

-- for a single PUCCH group only

dynamicPUCCH-CellSwitchDiffLengthSingleGroup-r17 SEQUENCE {

pucch-Group-r17                                  ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                           PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-10b: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots for two PUCCH

-- groups

dynamicPUCCH-CellSwitchSameLengthTwoGroups-r17   SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17

OPTIONAL,

-- R1 25-10c: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots for two

-- PUCCH groups

dynamicPUCCH-CellSwitchDiffLengthTwoGroups-r17   SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17

OPTIONAL,

-- R1 33-2a: ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based

-- feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticast-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 33-2d: PTP retransmission for multicast dynamic scheduling

ptp-Retx-Multicast-r17                           ENUMERATED {supported}                       OPTIONAL,

-- R1 33-4: NACK-only based HARQ-ACK feedback for RRC-based enabling/disabling multicast with ACK/NACK transforming

nack-OnlyFeedbackForMulticast-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 33-4a: NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission

nack-OnlyFeedbackSpecificResourceForMulticast-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 33-5-1a: ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback

-- for SPS group-common PDSCH for multicast

ack-NACK-FeedbackForSPS-Multicast-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 33-5-1d: PTP retransmission for SPS group-common PDSCH for multicast

ptp-Retx-SPS-Multicast-r17                       ENUMERATED {supported}                       OPTIONAL,

-- R4 26-1: Higher Power Limit CA DC

higherPowerLimit-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 39-4: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA

parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17     ENUMERATED {supported}                       OPTIONAL,

-- R1 24-11a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when

-- configured with DL CA with Rel-17 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r17                           INTEGER (4..16)                              OPTIONAL,

-- R1 24-11f: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs for MCG and for SCG

-- when configured for NR-DC operation with Rel-17 PDCCH monitoring capability on all the serving cells

pdcch-BlindDetectionMCG-SCG-List-r17             SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMCG-SCG-r17

OPTIONAL,

-- R1 24-11c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 15 PDCCH monitoring capabilities on

-- different Carriers

-- R1 24-11g: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and

-- Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList1-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed-r17

OPTIONAL,

-- R1 24-11d: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 16 PDCCH monitoring capabilities on

-- different Carriers

-- R1 24-11h: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and

-- Rel. 16 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList2-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed-r17

OPTIONAL,

-- R1 24-11e: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17, Rel. 16 and Rel. 15 PDCCH monitoring

-- capabilities on different carriers

-- R1 24-11i: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17,

-- Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList3-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed1-r17

OPTIONAL

}

CA-ParametersNR-v1730 ::= SEQUENCE {

-- R1 30-4a: DM-RS bundling for PUSCH repetition type A (per BC)

dmrs-BundlingPUSCH-RepTypeAPerBC-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4b: DM-RS bundling for PUSCH repetition type B(per BC)

dmrs-BundlingPUSCH-RepTypeBPerBC-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4c: DM-RS bundling for TB processing over multi-slot PUSCH(per BC)

dmrs-BundlingPUSCH-multiSlotPerBC-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4d: DMRS bundling for PUCCH repetitions(per BC)

dmrs-BundlingPUCCH-RepPerBC-r17                        ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4g: Restart DM-RS bundling (per BC)

dmrs-BundlingRestartPerBC-r17                          ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4h: DM-RS bundling for non-back-to-back transmission (per BC)

dmrs-BundlingNonBackToBackTX-PerBC-r17                 ENUMERATED {supported}                         OPTIONAL,

-- R1 39-3-1: Stay on the target CC for SRS carrier switching

stayOnTargetCC-SRS-CarrierSwitch-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-3a: FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

fdm-CodebookForMux-UnicastMulticastHARQ-ACK-r17        ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-3b: Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

mode2-TDM-CodebookForMux-UnicastMulticastHARQ-ACK-r17  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-4: Mode 1 for type1 codebook generation

mode1-ForType1-CodebookGeneration-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-5-1j: NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission

-- for SPS group-commmon PDSCH for multicast

nack-OnlyFeedbackSpecificResourceForSPS-Multicast-r17  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-2: Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast

multiPUCCH-ConfigForMulticast-r17                      ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-3: PUCCH resource configuration for multicast feedback for SPS GC-PDSCH

pucch-ConfigForSPS-Multicast-r17                       ENUMERATED {supported}                         OPTIONAL,

-- The following parameter is associated with R1 33-2a, R1 33-3-3a, and R1 33-3-3b, and is not a RAN1 FG.

maxNumberG-RNTI-HARQ-ACK-Codebook-r17                  INTEGER (1..4)                                 OPTIONAL,

-- R1 33-3-5: Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook

-- type

mux-HARQ-ACK-UnicastMulticast-r17                      ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1740 ::= SEQUENCE {

-- R1 33-5-1f: NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback

-- for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-Multicast-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-1: PUCCH resource configuration for multicast feedback for dynamically scheduled multicast

singlePUCCH-ConfigForMulticast-r17                     ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1760 ::= SEQUENCE {

prioSCellPRACH-OverSP-PeriodicSRS-Support-r17          ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1770 ::= SEQUENCE {

parallelTxPUCCH-PUSCH-SamePriority-r17                 ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1780 ::= SEQUENCE {

parallelTxPUCCH-PUSCH-SamePriority-r17      ENUMERATED {supported}                    OPTIONAL,

supportedAggBW-FR1-r17      SEQUENCE {

scalingFactorSCS-r17                    ENUMERATED {true}                         OPTIONAL,

supportedAggBW-FDD-DL-r17               SupportedAggBandwidth-r17                 OPTIONAL,

supportedAggBW-FDD-UL-r17               SupportedAggBandwidth-r17                 OPTIONAL,

supportedAggBW-TDD-DL-r17               SupportedAggBandwidth-r17                 OPTIONAL,

supportedAggBW-TDD-UL-r17               SupportedAggBandwidth-r17                 OPTIONAL,

supportedAggBW-TotalDL-r17              SupportedAggBandwidth-r17                 OPTIONAL,

supportedAggBW-TotalUL-r17              SupportedAggBandwidth-r17                 OPTIONAL

}    OPTIONAL

}

CA-ParametersNR-v1800 ::= SEQUENCE {

codebookParametersetype2DopplerCSI-PerBC-r18  CodebookParametersetype2DopplerCSI-r18                  OPTIONAL,

codebookParametersfetype2DopplerCSI-PerBC-r18 CodebookParametersfetype2DopplerCSI-r18                 OPTIONAL,

codebookParametersetype2CJT-PerBC-r18         CodebookParametersetype2CJT-r18                         OPTIONAL,

codebookParametersfetype2CJT-PerBC-r18        CodebookParametersfetype2CJT-r18                        OPTIONAL,

codebookComboParametersCJT-PerBC-r18          CodebookComboParametersCJT-r18                          OPTIONAL,

codebookParametersHARQ-ACK-PUSCH-PerBC-r18    CodebookParametersHARQ-ACK-PUSCH-r18                    OPTIONAL,

-- R1 40-2-8: Maximum number of TAGs across all CCs

maxNumberTAG-AcrossCC-r18                     INTEGER (2..4)                                          OPTIONAL,

-- R1 40-3-3-1: TDCP (Time Domain Channel Properties) report

tdcp-ReportPerBC-r18                          SEQUENCE {

valueX-r18                                    INTEGER (1..2),

maxNumberActiveResource-r18                   INTEGER (2..32)

}                                                                                                     OPTIONAL,

-- R1 40-3-3-5: Number of CSI-RS resources for TDCP

tdcp-ResourcePerBC-r18                        SEQUENCE {

maxNumberConfigPerCC-r18                      ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberConfigAcrossCC-r18                   INTEGER (1..32),

maxNumberSimultaneousPerCC-r18                ENUMERATED {n2, n4, n6, n8, n12, n16, n20, n24, n28, n32}

}                                                                                                     OPTIONAL,

-- R1 40-3-1-24: Timeline for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI

timelineRelax-CJT-CSI-CA-r18                  ENUMERATED {n0,n2}                                      OPTIONAL,

-- R1 42-1: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting

spatialAdaptation-CSI-FeedbackPerBC-r18       SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             SEQUENCE {

sdType1-Resource-r18                          ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22,

n24, n26, n28, n30, n32, n34, n36, n38, n40, n42, n44,

n46, n48, n50, n52, n54, n56, n58, n60, n62, n64},

sdType2-Resource-r18                          ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22,

n24, n26, n28, n30, n32, n34, n36, n38, n40, n42, n44,

n46, n48, n50, n52, n54, n56, n58, n60, n62, n64}

},

maxNumberPortsAcrossCC-r18                    SEQUENCE {

sdType1-Resource-r18                          INTEGER (1..32),

sdType2-Resource-r18                          INTEGER (1..32)

}

}                                                                                                   OPTIONAL,

-- R1 40-7-2a: Association between CSI-RS and SRS for non-codebook case

nonCodebook-CSI-RS-SRS-PerBC-r18    SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource   OPTIONAL,

-- R1 42-1a: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting on

-- PUSCH

spatialAdaptation-CSI-FeedbackPUSCH-PerBC-r18 SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26, n28,

n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52, n54,

n56, n58, n60, n62, n64},

maxNumberPortsAcrossCC-r18                    INTEGER (1..32)

}                                                                                                   OPTIONAL,

-- R1 42-1b: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting

spatialAdaptation-CSI-FeedbackAperiodicPerBC-r18 SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18                SEQUENCE {

sdType1-Resource-r18                             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22,

n24, n26, n28, n30, n32, n34, n36, n38, n40, n42, n44,

n46, n48, n50, n52, n54, n56, n58, n60, n62, n64},

sdType2-Resource-r18                             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22,

n24, n26, n28, n30, n32, n34, n36, n38, n40, n42, n44,

n46, n48, n50, n52, n54, n56, n58, n60, n62, n64}

},

maxNumberPortsAcrossCC-r18                       SEQUENCE {

sdType1-Resource-r18                             INTEGER (1..32),

sdType2-Resource-r18                             INTEGER (1..32)        }

}                                                                                                   OPTIONAL,

-- R1 42-1c: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI

-- reporting on PUCCH

spatialAdaptation-CSI-FeedbackPUCCH-PerBC-r18 SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26, n28,

n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52, n54,

n56, n58, n60, n62, n64},

maxNumberPortsAcrossCC-r18                    INTEGER (1..32)

}                                                                                                   OPTIONAL,

-- R1 42-2: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting

powerAdaptation-CSI-FeedbackPerBC-r18         SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26, n28,

n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52, n54,

n56, n58, n60, n62, n64},

maxNumberPortsAcrossCC-r18                    INTEGER (1..32)

}                                                                                                   OPTIONAL,

-- R1 42-2a: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting on PUSCH

powerAdaptation-CSI-FeedbackPUSCH-PerBC-r18   SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26, n28,

n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52, n54,

n56, n58, n60, n62, n64},

maxNumberPortsAcrossCC-r18                   INTEGER (1..32)

}                                                                                                   OPTIONAL,

-- R1 42-2b: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting

powerAdaptation-CSI-FeedbackAperiodicPerBC-r18 SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26, n28,

n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52, n54,

n56, n58, n60, n62, n64},

maxNumberPortsAcrossCC-r18                    INTEGER (1..32)

}                                                                                                   OPTIONAL,

-- R1 42-2c: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI

-- reporting on PUCCH

powerAdaptation-CSI-FeedbackPUCCH-PerBC-r18   SEQUENCE {

maxNumberCSI-ResourceAcrossCC-r18             ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26, n28,

n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52, n54,

n56, n58, n60, n62, n64},

maxNumberPortsAcrossCC-r18                    INTEGER (1..32)

}                                                                                                   OPTIONAL,

-- R1 42-7: Mixed codebook combination for spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s),

-- each containing one port subset configuration

mixCodeBookSpatialAdaptationPerBC-r18         SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource       OPTIONAL,

-- R1 42-9: Indicates whether the UE supports CSI report framework and the number of CSI report(s) which the UE can

-- simultaneously process across all CCs, and across MCG and SCG in case of NR-DC.

simultaneousCSI-SubReportsAllCC-r18           INTEGER (5..32)                                       OPTIONAL,

-- R1 49-1: Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling

-- cell and cells in the set

multiCell-PDSCH-DCI-1-3-SameSCS-r18           SEQUENCE {

coScheduledCellSCS-r18                        SEQUENCE {

nonSharedSpectrum-fdd-fr1                     ENUMERATED {supported}                        OPTIONAL,

nonSharedSpectrum-tdd-fr1                     ENUMERATED {supported}                        OPTIONAL,

sharedSpectrum-tdd-fr1                        ENUMERATED {supported}                        OPTIONAL,

fr2-1                                         ENUMERATED {supported}                        OPTIONAL,

fr2-2                                         ENUMERATED {supported}                        OPTIONAL

},

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

harqFeedbackType-r18                          ENUMERATED {type1, type2, type1And2},

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both},

supportOfSearchSpace-r18                      ENUMERATED {supported}                            OPTIONAL,

licensed-fdd-tdd-fr1-r18                      ENUMERATED {supported}                            OPTIONAL

}                                                                                                   OPTIONAL,

-- R1 49-1b: Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different

-- SCS/carrier type between scheduling cell and cells in the set

multiCell-PDSCH-DCI-1-3-DiffSCS-r18           SEQUENCE {

coScheduledCellSCS-r18                        ENUMERATED {lowScheduling-highScheduled, highScheduling-lowScheduled, both},

combinationCarrierType-r18                    SEQUENCE (SIZE(1..maxSchedulingBandCombination-r18)) OF

CombinationCarrierType-r18,

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

harqFeedbackType-r18                          ENUMERATED {type1, type2, type1And2},

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both}

}                                                                                                   OPTIONAL,

-- R1 49-2: Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell

-- and cells in the set

multiCell-PUSCH-DCI-0-3-SameSCS-r18           SEQUENCE {

coScheduledCellSCS-r18                        SEQUENCE {

nonSharedSpectrum-fdd-fr1                     ENUMERATED {supported}                        OPTIONAL,

nonSharedSpectrum-tdd-fr1                     ENUMERATED {supported}                        OPTIONAL,

sharedSpectrum-tdd-fr1                        ENUMERATED {supported}                        OPTIONAL,

fr2-1                                         ENUMERATED {supported}                        OPTIONAL,

fr2-2                                         ENUMERATED {supported}                        OPTIONAL

},

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both},

supportOfSearchSpace-r18                      ENUMERATED {supported}                            OPTIONAL,

licensed-fdd-tdd-fr1-r18                      ENUMERATED {supported}                            OPTIONAL

}                                                                                                   OPTIONAL,

-- R1 49-2b: Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with

-- different SCS/carrier type between scheduling cell and cells in the set

multiCell-PUSCH-DCI-0-3-DiffSCS-r18           SEQUENCE {

coScheduledCellSCS-r18                        ENUMERATED {lowScheduling-highScheduled, highScheduling-lowScheduled, both},

combinationCarrierType-r18                    SEQUENCE (SIZE(1..maxSchedulingBandCombination-r18)) OF

CombinationCarrierType-r18,

maxNumberCoScheduledCell-r18                  INTEGER (2..4),

maxNumberSetsOfCellAcrossPUCCH-Group-r18      INTEGER (1..8),

maxNumberSetsOfCellScheduling-r18             INTEGER (1..4),

coScheduledCellIndicationScheme-r18           ENUMERATED {fdra,cellInd, both}

}                                                                                                    OPTIONAL,

-- R1 49-3x: Advanced UE capability for larger number of unicast DL DCI

advUnicastDCI-DL-r18                          SEQUENCE {

scs-15kHz-120kHz-r18                         ENUMERATED {n2, n4}                               OPTIONAL,

scs-15kHz-60kHz-r18                          ENUMERATED {n2, n4}                               OPTIONAL,

scs-30kHz-120kHz-r18                         ENUMERATED {n2, n4}                               OPTIONAL,

scs-15kHz-30kHz-r18                          ENUMERATED {n2}                                   OPTIONAL,

scs-30kHz-60kHz-r18                          ENUMERATED {n2}                                   OPTIONAL,

scs-60kHz-120kHz-r18                         ENUMERATED {n2}                                   OPTIONAL

}                                                                                                   OPTIONAL,

-- R1 49-3y: Advanced UE capability for larger number of unicast UL DCI

advUnicastDCI-UL-r18                          SEQUENCE {

scs-15kHz-120kHz-r18                         ENUMERATED {n2, n4}                               OPTIONAL,

scs-15kHz-60kHz-r18                          ENUMERATED {n2, n4}                               OPTIONAL,

scs-30kHz-120kHz-r18                         ENUMERATED {n2, n4}                               OPTIONAL,

scs-15kHz-30kHz-r18                          ENUMERATED {n2}                                   OPTIONAL,

scs-30kHz-60kHz-r18                          ENUMERATED {n2}                                   OPTIONAL,

scs-60kHz-120kHz-r18                         ENUMERATED {n2}                                   OPTIONAL

}                                                                                                   OPTIONAL,

-- R1 49-5a: Trigger Type 3 HARQ CB based feedback using DCI format 1_3

type3HARQ-CB-DCI-1-3-r18                          ENUMERATED {supported}                            OPTIONAL,

-- R1 49-5b: Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3

type3EnhHARQ-CB-DCI-1-3-r18                   SEQUENCE {

numberOfCodebook-r18                          ENUMERATED {n1, n2, n4, n8},

maxNumberPUCCH-Trans-r18                      INTEGER (1..7)

}                                                                                                   OPTIONAL,

-- R1 49-9: SCell dormancy indication within active time in DCI format 0_3/1_3

scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 ENUMERATED {supported}                            OPTIONAL,

pdcch-MonitoringCA-Ext-r18                    CHOICE {

-- R1 55-6a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when

-- configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r18                    SEQUENCE {

maxNumberOfMonitoringCC-r18               INTEGER (2..16),

supportedSpanArrangement-r18              ENUMERATED {alignedOnly, alignedAndNonAligned}

},

-- R1 55-6f: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when

-- configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells with restriction for non-aligned

-- span case

pdcch-MonitoringCA-NonAlignedSpan-r18         INTEGER (2..16)

}                                                                                                   OPTIONAL,

pdcch-BlindDetectionCA-MixedExt-r18           CHOICE {

-- R1 55-6c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers

pdcch-BlindDetectionCA-Mixed-r18              SEQUENCE {

blindDetectionCA-Mixed-r18                    SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF

PDCCH-BlindDetectionCA-MixedExt-r16,

supportedSpanArrangement-r18                  ENUMERATED{ alignedOnly, alignedAndNonAligned }

},

-- R1 55-6g: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers with restriction for non-aligned span case

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r18  SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF

PDCCH-BlindDetectionCA-MixedExt-r16

}                                                                                                   OPTIONAL,

-- R1 55-6e: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16

-- and Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMCG-SCG-List-r18          SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF

PDCCH-BlindDetectionMixed2-r18     OPTIONAL,

-- R4 33-1: Support of intra-band non-collocated NR CA operation

intraBandNR-CA-non-collocated-r18             ENUMERATED {supported}                                OPTIONAL

}

CA-ParametersNR-v1830 ::= SEQUENCE {

-- R1 45-1: Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure

intraFreqL1-MeasConfig-r18                            SEQUENCE {

supportedMaxIntraFreqCellsConfig-r18                   INTEGER (1..8),

supportedMaxIntraFreqCellsPerReport-r18                INTEGER (1..4),

supportedMaxReportBeamsPerReportedCell-r18             INTEGER (1..4),

supportedMaxReportBeamsReports-r18                     ENUMERATED {n1,n2,n3,n4,n6,n8,n9,n12,n16},

supportedMaxAperiodic-LTM-CSI-ReportConfig-r18         INTEGER (0..4),

supportedMaxPeriodic-LTM-CSI-ReportConfig-r18          INTEGER (1..4),

supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r18    INTEGER (0..4)

}                                                                                                   OPTIONAL,

-- R1 45-1a: Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure

interFreqL1-MeasConfig-r18                            SEQUENCE {

supportedMaxIntraInterFreqCellsConfig-r18              INTEGER (1..8),

supportedMaxIntraInterFreqCellsPerReport-r18           INTEGER (1..4),

supportedMaxIntraInterFreqBeamsPerCellReports-r18      INTEGER (1..4),

supportedMaxIntraInterFreqBeamsReports-r18             ENUMERATED {n1,n2,n3,n4,n6,n8,n9,n12,n16}

}                                                                                                  OPTIONAL,

-- R1 45-2: Inclusion of current SpCell in the L1 measurement report

currentSpCellInclL1-Report-r18                        ENUMERATED {supported}                       OPTIONAL,

-- R4 39-1: SSB based L1-RSRP measurements for multiple cells with RTD > CP

multiCellL1-measRTD-greaterThan-CP-r18                ENUMERATED {supported}                       OPTIONAL,

-- R4 39-2: SSB based inter-frequency L1-RSRP measurements without measurement gaps

interFreqSSB-L1-MeasWithoutGaps-r18                   ENUMERATED {supported}                       OPTIONAL,

-- R4 39-3-1: Number of frequency layers for L1-RSRP measurement

maxFreqLayersL1-Meas-r18                              SEQUENCE {

supportedMaxIntraInterFreqLayersWithoutGaps-r18        INTEGER (1..8)                           OPTIONAL,

supportedMaxInterFreqLayersWithGaps-r18                INTEGER (1..8)                           OPTIONAL

}                                                                                                  OPTIONAL,

-- R4 39-3-2: Number of neighbour cells to be measured per frequency layer

maxNeighCellsPerFreqLayerL1-Meas-r18                  SEQUENCE {

supportedMaxNeighCellsPerFreqLayersWithoutGaps-r18     INTEGER (1..8)                           OPTIONAL,

supportedMaxNeighCellsPerFreqLayersWithGaps-r18        INTEGER (1..8)                           OPTIONAL

}                                                                                                  OPTIONAL,

-- R4 39-3-3: Number of total cells to be measured

supportedMaxCellsWithoutGapsL1-Meas-r18               INTEGER (1..24)                              OPTIONAL,

-- R4 39-3-4: Number of SSB resources for L1-RSRP measurement within a slot

supportedMaxSSB-WithinSlotL1-Meas-r18                 ENUMERATED {n1,n2,n3,n4,n5,n6,n7,n8,n16,n32,n48,n64}        OPTIONAL,

dummy                                                 SEQUENCE {

supportedMaxSSB-PerFreqLayerWithoutGaps-r18            INTEGER (1..8)                            OPTIONAL,

supportedMaxSSB-PerFreqLayerWithGaps-r18               INTEGER (1..8)                            OPTIONAL

}                                                                                                   OPTIONAL,

-- R4 39-3-6: Number of total SSB resources to be measured

supportedMaxSSB-L1-Meas-r18                           ENUMERATED {n2,n4,n8,n12,n16,n32,n64}         OPTIONAL,

-- R1 49-13: Default QCL assumption for multi-cell scheduling by DCI format 1_3

qcl-MultiCellDCI-1-3-r18                              ENUMERATED {diff, both}                       OPTIONAL,

-- R1 49-14: Support of BWP switch indication by DCI format 0_3/1_3

bwp-SwitchingDCI-0-3-And-1-3-r18                      ENUMERATED {supported}                        OPTIONAL

}

CA-ParametersNR-v1860 ::= SEQUENCE {

-- R4 39-3-5: Number of SSB resources for L1-RSRP measurement per frequency layer

maxSSB-PerFreqLayerL1-Meas-r18                  SEQUENCE {

supportedMaxSSB-PerFreqLayerWithoutGaps-r18     ENUMERATED {n1,n2,n3,n4,n5,n6,n7,n8,n12,n16,n20,n24}  OPTIONAL,

supportedMaxSSB-PerFreqLayerWithGaps-r18        INTEGER (1..8)                                  OPTIONAL

}                                                                                                   OPTIONAL

}

CA-ParametersNR-v1900 ::=                          SEQUENCE {

codebookParametersType1SP-SchemeA-PerBC-r19        CodebookParametersType1SP-SchemeA-r19            OPTIONAL,

codebookParametersType1SP-SchemeB-PerBC-r19        CodebookParametersType1SP-SchemeB-r19            OPTIONAL,

codebookParametersType1MP-PerBC-r19                CodebookParametersType1MP-r19                    OPTIONAL,

codebookParameterseType2ExtPerBC-r19               CodebookParameterseType2Ext-r19                  OPTIONAL,

codebookParametersfeType2ExtPerBC-r19              CodebookParametersfeType2Ext-r19                 OPTIONAL,

codebookParameterseType2DopplerExtPerBC-r19        CodebookParameterseType2DopplerExt-r19           OPTIONAL,

codebookParametersHybridBF-Type1SP-PerBC-r19       CodebookParametersHybridBF-Type1SP-r19           OPTIONAL,

codebookParametersHybridBF-eType2PerBC-r19         CodebookParametersHybridBF-eType2-r19            OPTIONAL,

-- R1 58-3-1: CSI prediction for UE-sided inference when N4=1

aiml-CSI-PredictionPerBC-r19                      ENUMERATED {supported}                            OPTIONAL,

-- R1 59-1-6: First PUCCH and second PUSCH from different PUCCH groups

diffGroupPUCCH-PUSCH-r19                          ENUMERATED {supported}                            OPTIONAL,

-- R1 59-2-1-6: CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports

timeRestriction128PortPerBC-r19                   ENUMERATED {supported}                            OPTIONAL,

-- R1 59-2-1-7: Group-specific 3-bit scaling factors for up to 128 ports

groupScalingFactorPerBC-r19                       ENUMERATED {rank1, rank1and2}                     OPTIONAL,

-- R1 59-2-2-3a: Configuration of MR always-reported resources with Rel-15 Type-I SP codebook

mr-AlwaysReportedType1SP-PerBC-r19                ENUMERATED {supported}                            OPTIONAL,

-- R1 59-2-2-3b: Configuration of MR always-reported resources with Rel-16 eType-II codebook with R=1

mr-AlwaysReported-eType2PerBC-r19                 ENUMERATED {supported}                             OPTIONAL,

-- R1 59-2-3-1: CJTC Dd report

cjtc-DdReportPerBC-r19                        SEQUENCE {

minRangeDdInCyclicPrefix-r19                  ENUMERATED {half, full},

maxResolutionDd-r19                           ENUMERATED {n32,n64,n128,n256},

scalingFactor-r19                             INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-1a: CJTC Dd report processing

cjtc-DdReportProcessingPerBC-r19              SEQUENCE {

maxNumberTRS-Resource-r19                     ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberTRS-ResourceAcrossCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-2: CJTC FO report

cjtc-FO-ReportPerBC-r19                       SEQUENCE {

minRangeFO-r19                                ENUMERATED {ppmDot1, ppmDot2},

maxResolutionFO-r19                           ENUMERATED {n16,n32,n256},

scalingFactor-r19                             INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-2a: CJTC FO report processing

cjtc-FO-ReportProcessingPerBC-r19            SEQUENCE {

maxNumberTRS-Resource-r19                     ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberTRS-ResourceAcrossCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-3: CJTC wideband PO report

cjtc-PO-ReportWidebandPerBC-r19               SEQUENCE {

maxResolution-r19                             ENUMERATED {n16, n32},

scalingFactor-r19                             INTEGER (1..2),

maxSlotDuration-r19                           INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-3a: CJTC wideband PO report processing

cjtc-PO-ReportWidebandProcessingPerBC-r19     SEQUENCE {

maxNumberCSI-RS-Configured-r19                ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberCSI-RS-ConfiguredAcrossCC-r19        ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-4: CJTC subband PO report

cjtc-PO-ReportSubbandPerBC-r19                SEQUENCE {

maxResolution-r19                             ENUMERATED {n16, n32},

minSubbandSize-r19                            ENUMERATED {n1,n2,n4,n8,n16},

scalingFactor-r19                             INTEGER (1..2),

maxSlotDuration-r19                           INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-5: CJTC Dd+FO report

cjtc-DdFO-ReportPerBC-r19                     SEQUENCE {

minRangeDdInCyclicPrefix-r19                  ENUMERATED {half, full},

maxResolutionDd-r19                           ENUMERATED {n32,n64,n128,n256},

minRangeFO-r19                                ENUMERATED {ppmDot1, ppmDot2},

maxResolutionFO-r19                           ENUMERATED {n16,n32,n256},

scalingFactor-r19                             INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-5a: CJTC Dd+FO report processing

cjtc-DdFO-ReportProcessingPerBC-r19           SEQUENCE {

maxNumberTRS-Resource-r19                     ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberTRS-ResourceAcrossCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                   OPTIONAL,

-- R1 59-2-3-7: Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering

linked-CJTC-Dd-eType2CJT-JointPerBC-r19           ENUMERATED {supported}                            OPTIONAL,

-- R1 59-2-3-7a: Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering

linked-CJTC-Dd-eType2CJT-SeparatePerBC-r19        ENUMERATED {supported}                            OPTIONAL,

-- R1 59-2-3-8: Separate triggering with configuration of 1-bit indicator per CSI trigger state

linked-CJTC-Dd-eType2CJT-SeparatePerStatePerBC-r19 ENUMERATED {supported}                           OPTIONAL,

-- R1 59-2-3-10: Relaxed timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT

timelineRelax-CJTC-Dd-eType2CJT-PerBC-r19         SEQUENCE {

scs15kHz-r19                                      ENUMERATED {n2,n4,n8}                         OPTIONAL,

scs30kHz-r19                                      ENUMERATED {n4,n8,n14,n28}                    OPTIONAL,

scs60kHz-r19                                      ENUMERATED {n8,n14,n28}                       OPTIONAL,

scs120kHz-r19                                     ENUMERATED {n14,n28,n56}                      OPTIONAL,

scs480kHz-r19                                     ENUMERATED {n56,n112,n224}                    OPTIONAL,

scs960kHz-r19                                     ENUMERATED {n112,n224,n448}                   OPTIONAL

}                                                                                                   OPTIONAL,

-- R1 59-2-4: Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH

nonCodebook-CSI-RS-SRS-PerBC-Enh-r19              SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER(0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

-- R1 63-1: NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility

-- (LTM) procedure

intraFreqL1-MeasConfigPeriodicCSI-RS-r19          SEQUENCE {

supportedMaxIntraFreqCellsConfig-r19               INTEGER (1..8),

supportedMaxIntraFreqCellsPerReport-r19            INTEGER (1..4),

supportedMaxReportBeamsPerReportedCell-r19         INTEGER (1..4),

supportedMaxReportBeamsReports-r19                 ENUMERATED {n1,n2,n3,n4,n6,n8,n9,n12,n16},

supportedMaxAperiodic-LTM-CSI-ReportConfig-r19     INTEGER (0..4),

supportedMaxPeriodic-LTM-CSI-ReportConfig-r19      INTEGER (1..4),

supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r19 INTEGER (0..4)

}                                                                                                    OPTIONAL,

-- R1 63-2: NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for

-- L1-L2 Triggered Mobility (LTM) procedure

intraFreqL1-MeasConfigSP-CSI-RS-r19                   SEQUENCE {

supportedMaxAperiodic-LTM-CSI-ReportConfig-r19         INTEGER (0..4),

supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r19    INTEGER (0..4)

}                                                                                                    OPTIONAL,

-- R1 63-8: Inclusion of current SpCell in the L1 measurement report based on CSI-RS(s)

currentSpCellInclL1-ReportCSI-RS-r19                  ENUMERATED {supported}                         OPTIONAL,

-- R1 67-6: Simultaneous NZP-CSI-RS resource counting

simultaneousNZP-CSI-RS-r19                            ENUMERATED {supported}                         OPTIONAL,

-- R1 67-8: Simultaneous NZP-CSI-RS resource counting NES

simultaneousNZP-CSI-RS-NES-r19                        ENUMERATED {supported}                         OPTIONAL,

-- R4 46-1: MPR enhancement for activated carrier

mpr-ActiveCarrierEnh-r19                              ENUMERATED {supported}                         OPTIONAL,

-- R4 46-2: FR2 MPR-Improvement Downlink Independent

mpr-DL-Independent-r19                                ENUMERATED {supported}                         OPTIONAL,

-- R4 46-3: FR2 MPR Improvement Activation Dependent

mpr-ActivateDependent-r19                             ENUMERATED {supported}                         OPTIONAL,

-- R4 47-1: Support network control of requirement for UE supporting intraBandNR-CA-non-collocated-r19

intraBandNR-CA-non-collocated-r19                     ENUMERATED {supported}                         OPTIONAL,

-- R4 48-1: Rx beam Type

atg-RxBeamType-r19                                    ENUMERATED {rxType1, rxType2}                  OPTIONAL,

-- R4 50-1: Lower bound of measurement periodicity of 10ms for the deactivated measurement requirement in

-- fast measurement window on OD-SSB SCell

od-SSB-FastMeasWinLowerBound-r19                      ENUMERATED {supported}                         OPTIONAL,

-- R4 52-1: Number of CSI-RS resources for L1-RSRP measurement within a slot

maxCSI-RS-ResourceL1-Meas-r19                        ENUMERATED {n1,n2,n3,n4,n5,n6,n7,n8,n16,n32,n48,n64} OPTIONAL,

-- R4 52-2: Number of total CSI-RS resources to be measured

totalCSI-RS-ResourceL1-Meas-r19                       ENUMERATED {n2,n4,n8,n12,n16,n32,n64}          OPTIONAL

}

CrossCarrierSchedulingSCell-SpCell-r17 ::= SEQUENCE {

supportedSCS-Combinations-r17              SEQUENCE {

scs15kHz-15kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs15kHz-30kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs15kHz-60kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs30kHz-30kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL,

scs30kHz-60kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL,

scs60kHz-60kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL

},

pdcch-MonitoringOccasion-r17               ENUMERATED {val1, val2}

}

PDCCH-BlindDetectionMixedList-r16::=       SEQUENCE {

pdcch-BlindDetectionCA-MixedExt-r16        CHOICE {

pdcch-BlindDetectionCA-Mixed-v16a0                PDCCH-BlindDetectionCA-MixedExt-r16,

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-v16a0 PDCCH-BlindDetectionCA-MixedExt-r16

}                                                                                             OPTIONAL,

pdcch-BlindDetectionCG-UE-MixedExt-r16     SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-v16a0                PDCCH-BlindDetectionCG-UE-MixedExt-r16,

pdcch-BlindDetectionSCG-UE-Mixed-v16a0            PDCCH-BlindDetectionCG-UE-MixedExt-r16

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCA-MixedExt-r16 ::=    SEQUENCE {

pdcch-BlindDetectionCA1-r16                INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                INTEGER (1..15)

}

PDCCH-BlindDetectionCG-UE-MixedExt-r16 ::= SEQUENCE {

pdcch-BlindDetectionCG-UE1-r16             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r16             INTEGER (0..15)

}

PDCCH-BlindDetectionMCG-SCG-r17 ::=        SEQUENCE {

pdcch-BlindDetectionMCG-UE-r17             INTEGER (1..15),

pdcch-BlindDetectionSCG-UE-r17             INTEGER (1..15)

}

PDCCH-BlindDetectionMixed-r17::=           SEQUENCE {

pdcch-BlindDetectionCA-Mixed-r17           PDCCH-BlindDetectionCA-Mixed-r17                   OPTIONAL,

pdcch-BlindDetectionCG-UE-Mixed-r17        SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-v17       PDCCH-BlindDetectionCG-UE-Mixed-r17,

pdcch-BlindDetectionSCG-UE-Mixed-v17       PDCCH-BlindDetectionCG-UE-Mixed-r17

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCG-UE-Mixed-r17 ::=    SEQUENCE {

pdcch-BlindDetectionCG-UE1-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r17             INTEGER (0..15)

}

PDCCH-BlindDetectionCA-Mixed-r17 ::=       SEQUENCE {

pdcch-BlindDetectionCA1-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA2-r17                INTEGER (1..15)                                    OPTIONAL

}

PDCCH-BlindDetectionMixed1-r17::=          SEQUENCE {

pdcch-BlindDetectionCA-Mixed1-r17          PDCCH-BlindDetectionCA-Mixed1-r17                  OPTIONAL,

pdcch-BlindDetectionCG-UE-Mixed1-r17       SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed1-v17      PDCCH-BlindDetectionCG-UE-Mixed1-r17,

pdcch-BlindDetectionSCG-UE-Mixed1-v17      PDCCH-BlindDetectionCG-UE-Mixed1-r17

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCG-UE-Mixed1-r17 ::=   SEQUENCE {

pdcch-BlindDetectionCG-UE1-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE3-r17             INTEGER (0..15)

}

PDCCH-BlindDetectionCA-Mixed1-r17 ::=      SEQUENCE {

pdcch-BlindDetectionCA1-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA2-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA3-r17                INTEGER (1..15)                                    OPTIONAL

}

PDCCH-BlindDetectionMixed2-r18 ::=         SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-r18       PDCCH-BlindDetectionCG-UE-MixedExt-r16,

pdcch-BlindDetectionSCG-UE-Mixed-r18       PDCCH-BlindDetectionCG-UE-MixedExt-r16

}

SimulSRS-ForAntennaSwitching-r16 ::= SEQUENCE {

supportSRS-xTyR-xLessThanY-r16       ENUMERATED {supported}                     OPTIONAL,

supportSRS-xTyR-xEqualToY-r16        ENUMERATED {supported}                     OPTIONAL,

supportSRS-AntennaSwitching-r16      ENUMERATED {supported}                     OPTIONAL

}

TwoPUCCH-Grp-Configurations-r16 ::=  SEQUENCE {

pucch-PrimaryGroupMapping-r16        TwoPUCCH-Grp-ConfigParams-r16,

pucch-SecondaryGroupMapping-r16      TwoPUCCH-Grp-ConfigParams-r16

}

TwoPUCCH-Grp-Configurations-r17 ::=  SEQUENCE {

primaryPUCCH-GroupConfig-r17         PUCCH-Group-Config-r17,

secondaryPUCCH-GroupConfig-r17       PUCCH-Group-Config-r17

}

TwoPUCCH-Grp-ConfigParams-r16 ::=    SEQUENCE {

pucch-GroupMapping-r16               PUCCH-Grp-CarrierTypes-r16,

pucch-TX-r16                         PUCCH-Grp-CarrierTypes-r16

}

CarrierTypePair-r16 ::=             SEQUENCE {

carrierForCSI-Measurement-r16       PUCCH-Grp-CarrierTypes-r16,

carrierForCSI-Reporting-r16         PUCCH-Grp-CarrierTypes-r16

}

PUCCH-Grp-CarrierTypes-r16 ::=       SEQUENCE {

fr1-NonSharedTDD-r16                 ENUMERATED {supported}                     OPTIONAL,

fr1-SharedTDD-r16                    ENUMERATED {supported}                     OPTIONAL,

fr1-NonSharedFDD-r16                 ENUMERATED {supported}                     OPTIONAL,

fr2-r16                              ENUMERATED {supported}                     OPTIONAL

}

PUCCH-Group-Config-r17 ::=           SEQUENCE {

fr1-FR1-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL,

fr2-FR2-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL,

fr1-FR2-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL

}

CombinationCarrierType-r18 ::=       SEQUENCE {

schedulingCellCarrierType-r18        ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1, fr2-2},

scheduledCellCarrierType-r18         ENUMERATED {licensed-fdd-fr1, licensed-tdd-fr1, unlicensed-tdd-fr1, fr2-1, fr2-2}

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

CA-ParametersNR field description

codebookParametersPerBC For a given supported band combination, this field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type, amongst the supported CSI-RS resources included in codebookParametersPerBand in MIMO-ParametersPerBand.

dummy The field is not used in the specification and the network ignores the received value.

#### –	CA-ParametersNRDC

The IE CA-ParametersNRDC contains dual connectivity related capabilities that are defined per band combination.

CA-ParametersNRDC information element

-- ASN1START

-- TAG-CA-PARAMETERS-NRDC-START

CA-ParametersNRDC ::=                         SEQUENCE {

ca-ParametersNR-ForDC                       CA-ParametersNR                              OPTIONAL,

ca-ParametersNR-ForDC-v1540                 CA-ParametersNR-v1540                        OPTIONAL,

ca-ParametersNR-ForDC-v1550                 CA-ParametersNR-v1550                        OPTIONAL,

ca-ParametersNR-ForDC-v1560                 CA-ParametersNR-v1560                        OPTIONAL,

featureSetCombinationDC                     FeatureSetCombinationId                      OPTIONAL

}

CA-ParametersNRDC-v15g0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v15g0                   CA-ParametersNR-v15g0                        OPTIONAL

}

CA-ParametersNRDC-v1610 ::= SEQUENCE {

-- R1 18-1: Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode1-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1a: Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode2-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1b: Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-DynamicPwrSharing-r16      ENUMERATED {short, long}       OPTIONAL,

asyncNRDC-r16                            ENUMERATED {supported}         OPTIONAL

}

CA-ParametersNRDC-v1630 ::=                         SEQUENCE {

ca-ParametersNR-ForDC-v1610                 CA-ParametersNR-v1610                        OPTIONAL,

ca-ParametersNR-ForDC-v1630                 CA-ParametersNR-v1630                        OPTIONAL

}

CA-ParametersNRDC-v1640 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1640                  CA-ParametersNR-v1640                        OPTIONAL

}

CA-ParametersNRDC-v1650 ::=                  SEQUENCE {

supportedCellGrouping-r16                    BIT STRING (SIZE (1..maxCellGroupings-r16))  OPTIONAL

}

CA-ParametersNRDC-v16a0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v16a0                  CA-ParametersNR-v16a0                        OPTIONAL

}

CA-ParametersNRDC-v16j0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v16j0                  CA-ParametersNR-v1690                        OPTIONAL

}

CA-ParametersNRDC-v1700 ::=                   SEQUENCE {

-- R1 31-9: Indicates the support of simultaneous transmission and reception of an IAB-node from multiple parent nodes

simultaneousRxTx-IAB-MultipleParents-r17     ENUMERATED {supported}                       OPTIONAL,

condPSCellAdditionNRDC-r17                   ENUMERATED {supported}                       OPTIONAL,

scg-ActivationDeactivationNRDC-r17           ENUMERATED {supported}                       OPTIONAL,

scg-ActivationDeactivationResumeNRDC-r17     ENUMERATED {supported}                       OPTIONAL,

beamManagementType-CBM-r17                   ENUMERATED {supported}                       OPTIONAL

}

CA-ParametersNRDC-v1720 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1700                  CA-ParametersNR-v1700                        OPTIONAL,

ca-ParametersNR-ForDC-v1720                  CA-ParametersNR-v1720                        OPTIONAL

}

CA-ParametersNRDC-v1730 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1730                   CA-ParametersNR-v1730                       OPTIONAL

}

CA-ParametersNRDC-v1760 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1760                  CA-ParametersNR-v1760

}

CA-ParametersNRDC-v1780 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1780                  CA-ParametersNR-v1780                        OPTIONAL

}

CA-ParametersNRDC-v17b0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v17b0                   CA-ParametersNR-v1740                        OPTIONAL

}

CA-ParametersNRDC-v1800 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1800                  CA-ParametersNR-v1800                        OPTIONAL,

-- R1 55-6d: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for

-- SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring on all the serving cells

pdcch-BlindDetectionNRDC-r18                 SEQUENCE(SIZE (1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF

PDCCH-BlindDetectionMixed1-r18      OPTIONAL

}

CA-ParametersNRDC-v1830 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1830                  CA-ParametersNR-v1830                        OPTIONAL

}

CA-ParametersNRDC-v1900 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1900                  CA-ParametersNR-v1900                        OPTIONAL

}

PDCCH-BlindDetectionMixed1-r18::=            SEQUENCE {

pdcch-BlindDetectionCG-UE-Mixed-r18          SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-r18         INTEGER (1..15),

pdcch-BlindDetectionSCG-UE-Mixed-r18         INTEGER (1..15)

}

}

-- TAG-CA-PARAMETERS-NRDC-STOP

-- ASN1STOP

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If a version of this field (i.e., with or without suffix) is absent for a band combination, the corresponding ca-ParametersNR field version in BandCombination is applicable to the UE configured with NR-DC for the band combination. If a version of this field (i.e., with or without suffix) is present for a band combination but does not contain any parameters, the UE does not support the corresponding field version when configured with NR-DC for the band combination.

featureSetCombinationDC If this field is present for a band combination, it reports the feature set combination supported for the band combination when NR-DC is configured. If this field is absent for a band combination, the featureSetCombination in BandCombination (without suffix) is applicable to the UE configured with NR-DC for the band combination.

#### –	CarrierAggregationVariant

The IE CarrierAggregationVariant informs the network about supported "placement" of the SpCell in an NR cell group.

CarrierAggregationVariant information element

-- ASN1START

-- TAG-CARRIERAGGREGATIONVARIANT-START

CarrierAggregationVariant ::=          SEQUENCE {

fr1fdd-FR1TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1FDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1TDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR2TDD  ENUMERATED {supported}                      OPTIONAL

}

-- TAG-CARRIERAGGREGATIONVARIANT-STOP

-- ASN1STOP

#### –	CodebookParameters

The IE CodebookParameters is used to convey codebook related parameters.

CodebookParameters information element

-- ASN1START

-- TAG-CODEBOOKPARAMETERS-START

CodebookParameters ::=             SEQUENCE {

type1                                  SEQUENCE {

singlePanel                           SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode1andMode2},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

},

multiPanel                            SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode2, both},

nrofPanels                            ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

}                                                                                                               OPTIONAL

},

type2                                  SEQUENCE {

supportedCSI-RS-ResourceList        SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                 ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}              OPTIONAL

}                                                                                                                   OPTIONAL,

type2-PortSelection                  SEQUENCE {

supportedCSI-RS-ResourceList        SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                 ENUMERATED {wideband, widebandAndSubband}

}                                                                                                                   OPTIONAL

}

CodebookParameters-v1610 ::=        SEQUENCE {

supportedCSI-RS-ResourceListAlt-r16  SEQUENCE {

type1-SinglePanel-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type1-MultiPanel-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-r16                            SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-PortSelection-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL

}                                                                                                                                       OPTIONAL

}

CodebookParametersAddition-r16 ::=      SEQUENCE {

etype2-r16                             SEQUENCE {

-- R1 16-3a Regular eType 2 R=1

etype2R1-r16                           SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 16-3a-1 Regular eType 2 R=2

etype2R2-r16                           SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                  OPTIONAL,

-- R1 16-3a-2: Support of parameter combinations 7-8

paramComb7-8-r16                       ENUMERATED {supported}      OPTIONAL,

-- R1 16-3a-3: Support of rank 3,4

rank3-4-r16                            ENUMERATED {supported}      OPTIONAL,

-- R1 16-3a-4: CBSR with soft amplitude restriction

amplitudeSubsetRestriction-r16         ENUMERATED {supported}      OPTIONAL

}                                                                      OPTIONAL,

etype2-PS-r16                          SEQUENCE {

-- R1 16-3b Regular eType 2 R=1 PortSelection

etype2R1-PortSelection-r16             SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 16-3b-1 Regular eType 2 R=2 PortSelection

etype2R2-PortSelection-r16             SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                  OPTIONAL,

-- R1 16-3b-2: Support of rank 3,4

rank3-4-r16                            ENUMERATED {supported}      OPTIONAL

}                                                                      OPTIONAL

}

CodebookComboParametersAddition-r16 ::= SEQUENCE {

-- R1 16-8 Mixed codebook types

type1SP-Type2-null-r16                 SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-Type2PS-null-r16               SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R1-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R2-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R1PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R2PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-Type2-Type2PS-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2-null-r16                 SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2PS-null-r16               SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R1-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R2-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R1PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R2PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2-Type2PS-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL

}

CodebookParametersfetype2-r17 ::= SEQUENCE {

-- R1 23-9-1  Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

fetype2basic-r17        SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16),

-- R1 23-9-2  Support of M=2 and R=1 for FeType-II

fetype2R1-r17           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-4  Support of R = 2 for FeType-II

fetype2R2-r17           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-3  Support of rank 3, 4 for FeType-II

fetype2Rank3Rank4-r17   ENUMERATED {supported}     OPTIONAL

}

CodebookComboParameterMixedType-r17 ::= SEQUENCE {

-- R1 23-9-5 Active CSI-RS resources and ports for mixed codebook types in any slot

type1SP-feType2PS-null-r17              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R1-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R2-null-r1          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M1-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M1-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M2R1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-null-r17              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R1-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R2-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M1-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M1-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M2R1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMultiTRP-r17::= SEQUENCE {

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

--  {Codebook 2, Codebook 3} =(NULL, NULL}

nCJT-null-null                        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-null-null                     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--    {Codebook 2, Codebook 3} = {( {"Rel 16 combinations in FG 16-8"}

nCJT-Type2-null-r16                   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2PS-null-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-Type2PS-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-null-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2-null-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1PS-null-r16           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2PS-null-r16           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-Type2PS-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- {Codebook 2, Codebook 3} = {"New Rel17 combinations in FG 23-9-5"}

nCJT-feType2PS-null-r17               SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R1-null-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R2-null-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M1-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M2R1-r17        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M2R1-r17     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-null-r17            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R1-null-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R2-null-r1        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M2R1-r17     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M2R1-r17  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersAdditionPerBC-r16::=  SEQUENCE {

-- R1 16-3a Regular eType 2 R=1

etype2R1-r16                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3a-1 Regular eType 2 R=2

etype2R2-r16                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3b Regular eType 2 R=1 PortSelection

etype2R1-PortSelection-r16      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3b-1 Regular eType 2 R=2 PortSelection

etype2R2-PortSelection-r16      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParametersAdditionPerBC-r16::= SEQUENCE {

-- R1 16-8 Mixed codebook types

type1SP-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersfetype2PerBC-r17 ::= SEQUENCE {

-- R1 23-9-1	Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

fetype2basic-r17    SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16),

-- R1 23-9-2	Support of M=2 and R=1 for FeType-II

fetype2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-4	Support of R = 2 for FeType-II

fetype2R2-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMixedTypePerBC-r17 ::= SEQUENCE {

-- R1 23-9-5 Active CSI-RS resources and ports for mixed codebook types in any slot

type1SP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R2-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R2-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMultiTRP-PerBC-r17::= SEQUENCE {

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

--  {Codebook 2, Codebook 3} =(NULL, NULL}

nCJT-null-null               SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-null-null            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--    {Codebook 2, Codebook 3} = {( {"Rel 16 combinations in FG 16-8"}

nCJT-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-null-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2-null-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1PS-null-r16  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2PS-null-r16  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-Type2PS-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- {Codebook 2, Codebook 3} = {"New Rel17 combinations in FG 23-9-5"}

nCJT-feType2PS-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R1-null-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R2-null-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M2R1-r17  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R2-null-r1  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersetype2DopplerCSI-r18 ::= SEQUENCE {

-- R1 40-3-2-1: Support of Rel-16-based doppler CSI

eType2Doppler-r18                          SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

valueY-P-SP-CSI-RS-r18                 INTEGER (1..3),

valueY-A-CSI-RS-r18                    INTEGER (1..3),

scalingfactor-r18                      ENUMERATED {n1, n2, n4}

},

-- R1 40-3-2-1a: Support of Rel-16-based doppler measurement with N4>1

eType2DopplerN4-r18  SEQUENCE {

supportedCSI-RS-ReportSettingList1-r18 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

SupportedCSI-RS-ReportSetting-r18,

supportedCSI-RS-ReportSettingList2-r18 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

SupportedCSI-RS-ReportSetting-r18

}                                                                                                            OPTIONAL,

-- R1 40-3-2-1a-1: DD unit size when A-CSI-RS is configured for CMR N4>1

ddUnitSize-A-CSI-RS-CMR-r18                ENUMERATED {supported}                                            OPTIONAL,

-- R1 40-3-2-1b: Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for

-- Rel-16-based doppler measurement

maxNumberAperiodicCSI-RS-Resource-r18      ENUMERATED {n4, n8, n12}                                          OPTIONAL,

-- R1 40-3-2-2: Support R=2 for Rel-16-based doppler codebook

eType2DopplerR2-r18  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 40-3-2-3: Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook

eType2DopplerX1-r18                        ENUMERATED {supported}                                            OPTIONAL,

-- R1 40-3-2-3a: Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook

eType2DopplerX2-r18                        ENUMERATED {supported}                                            OPTIONAL,

--R1 40-3-2-7: support of l = (n - nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook

eType2DopplerL-N4D1-r18                    ENUMERATED {supported}                                            OPTIONAL,

-- R1 40-3-2-8: Support of L=6 for Rel-16 based doppler codebook

eType2DopplerL6-r18                        ENUMERATED {supported}                                            OPTIONAL,

-- R1 40-3-2-9: Support of rank equals 3 and 4 for Rel-16 based doppler codebook

eType2DopplerR3R4-r18                      ENUMERATED {supported}                                            OPTIONAL

}

CodebookParametersfetype2DopplerCSI-r18 ::= SEQUENCE {

-- R1 40-3-2-4: Support of Rel-17-based doppler CSI

feType2Doppler-r18  SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

valueY-A-CSI-RS-r18                    INTEGER (1..3),

scalingfactor-r18                      ENUMERATED {n1, n2, n4}

},

-- R1 40-3-2-4b: Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for

-- Rel-17-based doppler CSI

maxNumberAperiodicCSI-RS-Resource-r18      ENUMERATED {n4, n8, n12}                                          OPTIONAL,

-- R1 40-3-2-5: Support of M=2 and R=1 for Rel-17-based doppler codebook

feType2DopplerM2R1-r18                     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 40-3-2-6: Support R=2 for Rel-17-based doppler codebook

feType2DopplerR2-r18  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--R1 40-3-2-7a: Support of l = (n - nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook

feType2DopplerL-N4D1-r18  ENUMERATED {supported}                                                             OPTIONAL,

-- R1 40-3-2-10: Support of rank equals 3 and 4 for Rel-17 based doppler codebook

feType2DopplerR3R4-r18                     ENUMERATED {supported}                                            OPTIONAL

}

CodebookParametersetype2CJT-r18 ::=    SEQUENCE {

-- R1 40-3-1-1: Basic feature for Rel-16-based CJT type-II codebook

eType2CJT-r18                          SEQUENCE {

supportedCSI-RS-ResourceList-r18       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

scalingfactor-r18                      ENUMERATED {n1, n1dot5, n2},

maxNumberNZP-CSI-RS-MultiTRP-CJT-r18   INTEGER (2..4)

},

-- R1 40-3-1-1a: Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset

eType2CJT-FD-IO-r18                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-2: Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1

eType2CJT-FD-FO-r18                    ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-3: Support R=2 for Rel-16-based CJT codebook

eType2CJT-R2-r18                      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-4: Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook

eType2CJT-PV-Beta-r18                 ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-9: Support for 2NN1N2 >32 for Rel-16 based CJT codebook

eType2CJT-2NN1N2-r18                  ENUMERATED {n64,n96,n128}                                             OPTIONAL,

-- R1 40-3-1-12: Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook

eType2CJT-Rank3Rank4-r18              ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-14: Support of Support of L=6 for Rel-16-based CJT type-II codebook

eType2CJT-L6-r18                      ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-15: dynamic selection of N<=N_TRP for Rel-16-based CJT type-II codebook

eType2CJT-NN-r18                      ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-17: Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-16-based CJT

-- type-II codebook

eType2CJT-NL-SD-r18                   ENUMERATED {n2,n4}                                                    OPTIONAL,

-- R1 40-3-1-23: Unequal number of spatial basis selection configuration for multi-TRP CJT

eType2CJT-Unequal-r18                  ENUMERATED {supported}                                                OPTIONAL

}

CodebookParametersfetype2CJT-r18 ::=   SEQUENCE {

-- R1 40-3-1-5: Basic feature for Rel-17-based CJT type-II codebook

feType2CJT-r18                         SEQUENCE {

supportedCSI-RS-ResourceList-r18    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

scalingfactor-r18                      ENUMERATED {n1, n1dot5, n2},

maxNumberNZP-CSI-RS-MultiTRP-CJT-r18   INTEGER (2..4)

},

-- R1 40-3-1-5a: Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset

feType2CJT-FD-IO-r18                   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-6: Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1

feType2CJT-FD-FO-r18                   ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-7: Support of M=2 and R=1 for Rel-17-based CJT codebook

feType2CJT-M2R1-r18                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-8: Support of R=2 for Rel-17-based CJT codebook

feType2CJT-R2-r18                      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)              OPTIONAL,

-- R1 40-3-1-9a: Support for 2NN1N2 >32 for Rel-17 based CJT codebook

feType2CJT-2NN1N2-r18                  ENUMERATED {n64,n96,n128}                                             OPTIONAL,

-- R1 40-3-1-13: Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook

feType2CJT-Rank3Rank4-r18              ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-16: dynamic selection of N<=N_TRP for Rel-17-based CJT type-II codebook

feType2CJT-NN-r18                      ENUMERATED {supported}                                                OPTIONAL,

-- R1 40-3-1-18: Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-17-based CJT

-- type-II codebook

feType2CJT-NL-r18                      ENUMERATED {n2,n4}                                                    OPTIONAL,

-- R1 40-3-1-23a: Unequal number of port selection configuration for multi-TRP CJT

feType2CJT-Unequal-r18                 ENUMERATED {supported}                                                OPTIONAL

}

CodebookComboParametersCJT-r18::= SEQUENCE {

-- R1 40-3-1-11: Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot

--  {Codebook 1} = Type I SP

cjt-Type1SP-eType2R1-null      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-eType2R2-null      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-feType2R1M1-null   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-feType2R1M2-null   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1SP-feType2R2M2-null   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--  {Codebook 1} = Type I MP

cjt-Type1MP-eType2R1-null      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-eType2R2-null      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-feType2R1M1-null   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-feType2R1M2-null   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

cjt-Type1MP-feType2R2M2-null   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersHARQ-ACK-PUSCH-r18::= SEQUENCE {

-- R1 55-4a: Multiplexing Type-1 HARQ-ACK codebook in a PUSCH for PDSCH scheduled after UL grant

multiplexingType1-r18                   ENUMERATED {supported}                                               OPTIONAL,

-- R1 55-4b: Multiplexing Type-2 HARQ-ACK codebook in a PUSCH for PDSCH scheduled after UL grant

multiplexingType2-r18                   ENUMERATED {supported}                                               OPTIONAL,

-- R1 55-4c: Multiplexing Type-3 HARQ-ACK codebook in a PUSCH for PDSCH scheduled after UL grant

multiplexingType3-r18                   ENUMERATED {supported}                                               OPTIONAL,

-- R1 55-4d: Determining a different PUCCH resource to transmit HARQ-ACK for PDSCH scheduled after UL grant

pucch-DiffResource-PDSCH-r18            ENUMERATED {supported}                                               OPTIONAL,

-- R1 55-4e: Determining different codebook size to transmit HARQ-ACK for PDSCH scheduled after UL grant

diffCB-Size-PDSCH-r18                   ENUMERATED {supported}                                               OPTIONAL

}

CodebookParametersType1SP-SchemeA-r19 ::=   SEQUENCE {

-- R1 59-2-1-1: Enhanced Type-I SP codebook for 64 ports – Scheme-A

enhType1SP64PortsSchemeA-r19                SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxRank-r19                                 INTEGER (4..8),

maxNumberResource-r19                       ENUMERATED {n2, n4},

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 59-2-1-1a: Enhanced Type-I SP codebook for 48 ports – Scheme-A

enhType1SP48PortsSchemeA-r19                SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxRank-r19                                 INTEGER (4..8),

maxNumberResource-r19                       INTEGER (1..8),

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-1b: Enhanced Type-I SP codebook for 128 ports – Scheme-A

enhType1SP128PortsSchemeA-r19               SEQUENCE {

supportedCSI-RS-ResourceExtList-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxRank-r19                                 INTEGER (4..8),

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL

}

CodebookParametersType1SP-SchemeB-r19 ::= SEQUENCE {

-- R1 59-2-1-1c: Enhanced Type-I SP codebook for 64 ports – Scheme-B

enhType1SP64PortsSchemeB-r19                SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxRank-r19                                 INTEGER (4..8),

maxNumberResource-r19                       ENUMERATED {n2, n4},

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 59-2-1-1d: Enhanced Type-I SP codebook for 48 ports – Scheme-B

enhType1SP48PortsSchemeB-r19                SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxRank-r19                                 INTEGER (4..8),

maxNumberResource-r19                       ENUMERATED {n2,n3},

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-1e: Enhanced Type-I SP codebook for 128 ports – Scheme-B

enhType1SP128PortsSchemeB-r19               SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxRank-r19                                 INTEGER (4..8),

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL

}

CodebookParametersType1MP-r19 ::= SEQUENCE {

-- R1 59-2-1-2: Enhanced Type-I MP codebook for 64 ports

enhType1MP64Ports-r19                       SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxPanel-r19                                ENUMERATED {n2, n4},

maxNumberResource-r19                       ENUMERATED {n2, n4},

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 59-2-1-2a: Enhanced Type-I MP codebook for 48 ports

enhType1MP48Ports-r19                       SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxPanel-r19                                ENUMERATED {n2, n4},

maxNumberResource-r19                       ENUMERATED {n2,n3},

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-2b: Enhanced Type-I MP codebook for 128 ports

enhType1MP128Ports-r19                      SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxPanel-r19                                ENUMERATED {n2, n4},

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL

}

CodebookParameterseType2Ext-r19 ::= SEQUENCE {

-- R1 59-2-1-3: Extended Rel-16 eType-II codebook for 64 Tx ports

eType2-64PortExt-r19                        SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

maxNumberResource-r19                       ENUMERATED {n2, n4},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 59-2-1-3a: Extended Rel-16 eType-II codebook for 48 Tx ports

eType2-48PortExt-r19                        SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

maxNumberResource-r19                       ENUMERATED {n2,n3},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-3b: Extended Rel-16 eType-II codebook for 128 Tx ports

eType2-128PortExt-r19                       SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-3-1: PMI sub-bands with R=2 for extended Rel-16 eType-II codebook for up to 128 ports

eType2R2Ext-r19                             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)                      OPTIONAL,

-- R1 59-2-1-3-3-2: Parameter combinations 7-8 for extended Rel-16 eType-II codebook for up to 128 ports

eType2ExtPC7-8-r19                          ENUMERATED {supported}                                                    OPTIONAL,

-- R1 59-2-1-3-3: Rank 3,4 for extended Rel-16 eType-II codebook for up to 128 ports

eType2R3R4Ext-r19                           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)                      OPTIONAL

}

CodebookParametersfeType2Ext-r19 ::= SEQUENCE {

-- R1 59-2-1-4: Extended Rel-17 FeType-II codebook with 64 Tx ports

feType2-64PortExt-r19                       SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

maxNumberResource-r19                       ENUMERATED {n2, n4},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 59-2-1-4a: Extended Rel-17 FeType-II codebook with 48 Tx ports

feType2-48PortExt-r19                       SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

maxNumberResource-r19                       ENUMERATED {n2,n3},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-4b: M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports

feType2-M2R1Ext-r19                         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)                       OPTIONAL,

--R1 59-2-1-4c: M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports

feType2-M2R2Ext-r19                         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)                       OPTIONAL,

-- R1 59-2-1-4d: Rank 3,4 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64ports

feType2-R3R4Ext-r19                         ENUMERATED {supported}                                                    OPTIONAL

}

CodebookParameterseType2DopplerExt-r19 ::= SEQUENCE {

-- R1 59-2-1-5: Extended Rel-18 eType-II Doppler codebook for 64 Tx ports

eType2Doppler-64PortExt-r19                 SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

valueY-P-SP-CSI-RS-r19                      INTEGER (1..3),

valueY-A-CSI-RS-r19                         INTEGER (1..3),

scalingfactor-r19                           ENUMERATED {n1, n2, n4},

maxNumberResource-r19                       ENUMERATED {n2, n4},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 59-2-1-5a: Extended Rel-18 eType-II Doppler codebook for 48 Tx ports

eType2Doppler-48PortExt-r19                 SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

valueY-P-SP-CSI-RS-r19                      INTEGER (1..3),

valueY-A-CSI-RS-r19                         INTEGER (1..3),

scalingfactor-r19                           ENUMERATED {n1, n2, n4},

maxNumberResource-r19                       ENUMERATED {n2,n3},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-5b: Extended Rel-18 eType-II Doppler codebook for 128 Tx ports

eType2Doppler-128PortExt-r19                SEQUENCE {

supportedCSI-RS-ResourceExtList-r19         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

processingCapability-r19                    ENUMERATED {cap1, cap2},

valueY-P-SP-CSI-RS-r19                      INTEGER (1..3),

valueY-A-CSI-RS-r19                         INTEGER (1..3),

scalingfactor-r19                           ENUMERATED {n1, n2, n4},

supportedCSI-RS-ResourceListPerCC-r19       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-5c: N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerN4Ext-r19  SEQUENCE {

supportedCSI-RS-ReportSettingList1-r19  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

SupportedCSI-RS-ReportSettingExt-r19,

supportedCSI-RS-ReportSettingList2-r19  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

SupportedCSI-RS-ReportSettingExt-r19

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-5d: DD unit size d=1 when A-CSI-RS is configured for CMR N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports

ddUnitSize-A-CSI-RS-CMR-Ext-r19             ENUMERATED {supported}                                                    OPTIONAL,

-- R1 59-2-1-5e: Maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for extended

-- Rel-18 Type-II Doppler codebook for up to 128 ports

maxNumberAperiodicCSI-RS-ResourceExt-r19    ENUMERATED {n4, n8, n12}                                                  OPTIONAL,

-- R1 59-2-1-5f: PMI subband R=2 for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerR2Ext-r19  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 59-2-1-5g: X=1 based on first and last slot of WCSI for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerX1Ext-r19                      ENUMERATED {supported}                                                    OPTIONAL,

-- R1 59-2-1-5h: X=2 CQI based on 2 slots for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerX2Ext-r19                      ENUMERATED {supported}                                                    OPTIONAL,

--R1 59-2-1-5i: l = (n – nCSI,ref ) for CSI reference slot for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerL-N4D1Ext-r19                  ENUMERATED {supported}                                                    OPTIONAL,

-- R1 59-2-1-5j: L=6 for CSI reference slot for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerL6Ext-r19                      ENUMERATED {supported}                                                    OPTIONAL,

-- R1 59-2-1-5k: Rank 3 and 4 for CSI reference slot for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerR3R4Ext-r19                    ENUMERATED {supported}                                                    OPTIONAL,

-- R1 59-2-1-5l: Processing timeline for CSI reference slot for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2DopplerProcessingTimelineExt-r19      SEQUENCE {

valueW-r19                                  SEQUENCE{

scs-15kHz                                   ENUMERATED {value1, value2}                                       OPTIONAL,

scs-30kHz                                   ENUMERATED {value1, value2}                                       OPTIONAL,

scs-60kHz                                   ENUMERATED {value1, value2}                                       OPTIONAL,

scs-120kHz                                  ENUMERATED {value1, value2}                                       OPTIONAL

},

timeRelaxation-r19                          ENUMERATED {cap1, cap2}

}                                                                                                                     OPTIONAL,

-- R1 59-2-1-5m: Maximum periodicity of CMR when configured as periodic CSI-Rs for extended Rel-18 Type-II Doppler codebook for up to 128 ports

eType2MaxPeriodicityCMR-r19                 ENUMERATED {sl4, sl5, sl8, sl10, sl20}                                    OPTIONAL

}

-- R1 59-2-2-1: Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook

CodebookParametersHybridBF-Type1SP-r19 ::=  SEQUENCE {

maxNumberCRI-Report-r19                     INTEGER (1..4),

supportedCSI-RS-ResourceHybridList-r19      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxValueKs-r19                              INTEGER (2..8)

}

-- R1 59-2-2-2: Hybrid BF (CRI-based) with Rel-16 eType-II codebook

CodebookParametersHybridBF-eType2-r19 ::=   SEQUENCE {

maxNumberCRI-Report-r19                     INTEGER (1..2),

supportedCSI-RS-ResourceHybridList-r19      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER

(0..maxNrofCSI-RS-ResourcesAlt-1-r16),

maxValueKs-r19                              INTEGER (2..8)

}

CodebookVariantsList-r16 ::=          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-Resource

CodebookVariantsListExt-r19 ::=       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-ResourceExt-r19

CodebookVariantsListAggregate-r19 ::= SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-ResourceAggregate-r19

CodebookVariantsListHybrid-r19 ::=    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-ResourceHybrid-r19

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

SupportedCSI-RS-ReportSetting-r18 ::= SEQUENCE {

maxN4-r18                            ENUMERATED {n1, n2, n4, n8},

maxNumberTxPortsPerResource-r18      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand-r18        INTEGER (1..64),

totalNumberTxPortsPerBand-r18        INTEGER (2..256)

}

SupportedCSI-RS-ResourceExt-r19 ::=  SEQUENCE {

maxNumberResourcesPerBand-r19        INTEGER (1..256),

totalNumberTxPortsPerBand-r19        INTEGER (64..1024)

}

SupportedCSI-RS-ResourceAggregate-r19 ::=    SEQUENCE {

maxNumberTxPortsPerAggregatedResource-r19    ENUMERATED {p48, p64, p128},

maxNumberAggregatedResources-r19             INTEGER (1..64),

totalNumberTxPorts-r19                       INTEGER (48..1024)

}

SupportedCSI-RS-ReportSettingExt-r19 ::=     SEQUENCE {

maxN4-r19                                    ENUMERATED {n1, n2, n4, n8},

maxNumberTxPortsPerAggregatedResource-r19    ENUMERATED {p48, p64, p128},

maxNumberAggregatedResources-r19             INTEGER (2..64),

totalNumberTxPorts-r19                       INTEGER (64..1024)

}

SupportedCSI-RS-ResourceHybrid-r19 ::= SEQUENCE {

maxNumberTxPortsPerResource-r19        ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources-r19                 INTEGER (1..256),

totalNumberTxPorts-r19                 INTEGER (64..1024)

}

-- TAG-CODEBOOKPARAMETERS-STOP

-- ASN1STOP

CodebookParameters field descriptions

supportedCSI-RS-ResourceListAlt This field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS-Resource defined in CodebookVariantsList. The value 0 corresponds to the first entry of CodebookVariantsList. The value 1 corresponds to the second entry of CodebookVariantsList, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC (but optional for single CC) and codebookParametersPerBand.

supportedCSI-RS-ResourceExtList-r19, supportedCSI-RS-ResourceListPerCC-r19 This field indicates the alternative list of SupportedCSI-RS-ResourceExt supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS-ResourceExt defined in CodebookVariantsListExt. The value 0 corresponds to the first entry of CodebookVariantsListExt. The value 1 corresponds to the second entry of CodebookVariantsListExt, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC (but optional for single CC) and codebookParametersPerBand.

supportedCSI-RS-ResourceHybridList-r19 This field indicates the alternative list of SupportedCSI-RS-ResourceHybrid supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS- ResourceHybrid defined in CodebookVariantsListHybrid. The value 0 corresponds to the first entry of CodebookVariantsListHybrid. The value 1 corresponds to the second entry of CodebookVariantsListHybrid, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC (but optional for single CC) and codebookParametersPerBand.

#### –	DL-PRS-MeasurementWithRxFH-RRC-Connected

The IE DL-PRS-MeasurementWithRxFH-RRC-Connected is used to convey the capabilities supported by the UE for PRS measurement with Rx frequency hopping within a measurement gap and measurement reporting in RRC_CONNECTED for RedCap UEs.

DL-PRS-MeasurementWithRxFH-RRC-Connected information element

-- ASN1START

-- TAG-DL-PRS-MEASUREMENTWITHRXFH-RRC-CONNECTED-START

DL-PRS-MeasurementWithRxFH-RRC-Connected-r18 ::= SEQUENCE {

maximumPRS-BandwidthAcrossAllHopsFR1-r18         ENUMERATED {mhz40, mhz50, mhz80, mhz100}            OPTIONAL,

maximumPRS-BandwidthAcrossAllHopsFR2-r18         ENUMERATED {mhz100, mhz200, mhz400}                 OPTIONAL,

maximumFH-Hops-r18                               ENUMERATED {n2, n3, n4, n5, n6}                     OPTIONAL,

processingDuration-r18                           SEQUENCE {

processingPRS-SymbolsDurationN3-r18              ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

processingDurationT3-r18                         ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160, ms320, ms640, ms1280}

}                                                                                                    OPTIONAL,

rf-RxRetuneTimeFR1-r18                           ENUMERATED {n70, n140, n210}                        OPTIONAL,

rf-RxRetuneTimeFR2-r18                           ENUMERATED {n35, n70, n140}                         OPTIONAL,

numOfOverlappingPRB-r18                          ENUMERATED {n0, n1, n2, n4}                         OPTIONAL,

...

}

-- TAG-DL-PRS-MEASUREMENTWITHRXFH-RRC-CONNECTED-STOP

-- ASN1STOP

#### –	ERedCapParameters

The IE ERedCapParameters is used to indicate the UE capabilities supported by eRedCap UEs.

ERedCapParameters information element

-- ASN1START

-- TAG-EREDCAPPARAMETERS-START

ERedCapParameters-r18::=                   SEQUENCE {

-- R1 48-1: eRedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1

supportOfERedCap-r18                       ENUMERATED {supported},

-- R1 48-2: eRedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1

eRedCapNotReducedBB-BW-r18                 ENUMERATED {supported}                            OPTIONAL,

eRedCapIgnoreCapabilityFiltering-r18       ENUMERATED {supported}                            OPTIONAL

}

-- TAG-EREDCAPPARAMETERS-STOP

-- ASN1STOP

#### –	FeatureSetCombination

The IE FeatureSetCombination is a two-dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

Each FeatureSet contains either a pair of NR or E-UTRA feature set IDs for UL and DL.

In case of NR, the actual feature sets for UL and DL are defined in the FeatureSets IE and referred to from here by their ID, i.e., their position in the featureSetsUplink / featureSetsDownlink list in the FeatureSet IE.

In case of E-UTRA, the feature sets referred to from this list are defined in TS 36.331 [10] and conveyed as part of the UE-EUTRA-Capability container.

The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Ids and FeatureSetDownlinkPerCC-Ids. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of carriers supported by the UE is also restricted by the bandwidth class indicated in the associated BandCombination, if present.

In feature set combinations the UE shall exclude entries with same or lower capabilities, since the network may anyway assume that the UE supports those.

NOTE 1:	The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:	The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

NOTE 3:	The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

FeatureSetCombination information element

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

#### –	FeatureSetCombinationId

The IE FeatureSetCombinationId identifies a FeatureSetCombination. The FeatureSetCombinationId of a FeatureSetCombination is the position of the FeatureSetCombination in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability). The FeatureSetCombinationId = 0 refers to the first entry in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability).

NOTE:	The FeatureSetCombinationId = 1024 is not used due to the maximum entry number of featureSetCombinations.

FeatureSetCombinationId information element

-- ASN1START

-- TAG-FEATURESETCOMBINATIONID-START

FeatureSetCombinationId ::=         INTEGER (0.. maxFeatureSetCombinations)

-- TAG-FEATURESETCOMBINATIONID-STOP

-- ASN1STOP

#### –	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                           ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy8                                  ENUMERATED {supported}                                                  OPTIONAL,

scellWithoutSSB                         ENUMERATED {supported}                                                  OPTIONAL,

csi-RS-MeasSCellWithoutSSB              ENUMERATED {supported}                                                  OPTIONAL,

dummy1                                  ENUMERATED {supported}                                                  OPTIONAL,

type1-3-CSS                             ENUMERATED {supported}                                                  OPTIONAL,

pdcch-MonitoringAnyOccasions            ENUMERATED {withoutDCI-Gap, withDCI-Gap}                                OPTIONAL,

dummy2                                  ENUMERATED {supported}                                                  OPTIONAL,

ue-SpecificUL-DL-Assignment             ENUMERATED {supported}                                                  OPTIONAL,

searchSpaceSharingCA-DL                 ENUMERATED {supported}                                                  OPTIONAL,

timeDurationForQCL                      SEQUENCE {

scs-60kHz                           ENUMERATED {s7, s14, s28}                                               OPTIONAL,

scs-120kHz                          ENUMERATED {s14, s28}                                                   OPTIONAL

}                                                                                                           OPTIONAL,

pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-30kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-60kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-120kHz                              ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL

}                                                                                                           OPTIONAL,

dummy3                                  DummyA                                                                  OPTIONAL,

dummy4                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyB                        OPTIONAL,

dummy5                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyC                        OPTIONAL,

dummy6                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyD                        OPTIONAL,

dummy7                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyE                        OPTIONAL

}

FeatureSetDownlink-v1540 ::= SEQUENCE {

oneFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

additionalDMRS-DL-Alt                   ENUMERATED {supported}                       OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-DL       ENUMERATED {supported}                       OPTIONAL,

pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE {

scs-15kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-30kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-60kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-120kHz                              ENUMERATED {set1, set2, set3}                OPTIONAL

}                                                                                    OPTIONAL,

pdsch-SeparationWithGap                 ENUMERATED {supported}                       OPTIONAL,

pdsch-ProcessingType2                   SEQUENCE {

scs-15kHz                               ProcessingParameters                         OPTIONAL,

scs-30kHz                               ProcessingParameters                         OPTIONAL,

scs-60kHz                               ProcessingParameters                         OPTIONAL

} OPTIONAL,

pdsch-ProcessingType2-Limited           SEQUENCE {

differentTB-PerSlot-SCS-30kHz           ENUMERATED {upto1, upto2, upto4, upto7}

} OPTIONAL,

dl-MCS-TableAlt-DynamicIndication       ENUMERATED {supported}                       OPTIONAL

}

FeatureSetDownlink-v15a0 ::= SEQUENCE {

supportedSRS-Resources              SRS-Resources                                    OPTIONAL

}

FeatureSetDownlink-v15t0 ::= SEQUENCE {

zeroSlotOffsetAperiodicSRS          ENUMERATED {supported}                           OPTIONAL

}

FeatureSetDownlink-v1610 ::=   SEQUENCE {

-- R1 22-4e/4f/4g/4h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 1

cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16   SEQUENCE {

scs-15kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-30kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-60kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-120kHz-r16       ENUMERATED {one, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

-- R1 22-3e/3f/3g/3h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 2

cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16   SEQUENCE {

scs-15kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-30kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-60kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-120kHz-r16       ENUMERATED {one, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

intraFreqDAPS-r16                  SEQUENCE {

intraFreqDiffSCS-DAPS-r16          ENUMERATED {supported}            OPTIONAL,

intraFreqAsyncDAPS-r16             ENUMERATED {supported}            OPTIONAL

}                                                                        OPTIONAL,

intraBandFreqSeparationDL-v1620    FreqSeparationClassDL-v1620           OPTIONAL,

intraBandFreqSeparationDL-Only-r16 FreqSeparationClassDL-Only-r16        OPTIONAL,

-- R1 11-2: Rel-16 PDCCH monitoring capability

pdcch-Monitoring-r16               SEQUENCE {

pdsch-ProcessingType1-r16          SEQUENCE {

scs-15kHz-r16                      PDCCH-MonitoringOccasions-r16 OPTIONAL,

scs-30kHz-r16                      PDCCH-MonitoringOccasions-r16 OPTIONAL

}                                                                    OPTIONAL,

pdsch-ProcessingType2-r16      SEQUENCE {

scs-15kHz-r16                  PDCCH-MonitoringOccasions-r16     OPTIONAL,

scs-30kHz-r16                  PDCCH-MonitoringOccasions-r16     OPTIONAL

}                                                                    OPTIONAL

}                                                                        OPTIONAL,

-- R1 11-2b: Mix of Rel. 16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers

pdcch-MonitoringMixed-r16          ENUMERATED {supported}                OPTIONAL,

-- R1 18-5c: Processing up to X unicast DCI scheduling for DL per scheduled CC

crossCarrierSchedulingProcessing-DiffSCS-r16  SEQUENCE {

scs-15kHz-120kHz-r16               ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-15kHz-60kHz-r16                ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-30kHz-120kHz-r16               ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-15kHz-30kHz-r16                ENUMERATED {n2}                   OPTIONAL,

scs-30kHz-60kHz-r16                ENUMERATED {n2}                   OPTIONAL,

scs-60kHz-120kHz-r16               ENUMERATED {n2}                   OPTIONAL

}                                                                        OPTIONAL,

-- R1 16-2b-1: Support of single-DCI based SDM scheme

singleDCI-SDM-scheme-r16           ENUMERATED {supported}                OPTIONAL

}

FeatureSetDownlink-v16k0 ::= SEQUENCE {

-- R1 22-8: For SRS for CB PUSCH and antenna switching on FR1 with symbol level offset for aperiodic SRS transmission

offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8a: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot and constrained timeline for SRS for CB

-- PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16           ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8b: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16    ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8c: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 with a DCI gap and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8d: All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap and constrained timeline

-- for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16 SEQUENCE {

scs-15kHz-r16                                 ENUMERATED {set1, set2, set3}                      OPTIONAL,

scs-30kHz-r16                                 ENUMERATED {set1, set2, set3}                      OPTIONAL,

scs-60kHz-r16                                 ENUMERATED {set1, set2, set3}                      OPTIONAL

}                                                                                                        OPTIONAL

}

FeatureSetDownlink-v1700 ::= SEQUENCE {

-- R1 36-2: Scaling factor to be applied to 1024QAM for FR1

scalingFactor-1024QAM-FR1-r17 ENUMERATED {f0p4, f0p75, f0p8}             OPTIONAL,

-- R1 24 feature for existing UE cap to include new SCS

timeDurationForQCL-v1710     SEQUENCE {

scs-480kHz                   ENUMERATED {s56, s112}                  OPTIONAL,

scs-960kHz                   ENUMERATED {s112, s224}                 OPTIONAL

}                                                                        OPTIONAL,

-- R1 23-6-1	SFN scheme A (scheme 1) for PDSCH and PDCCH

sfn-SchemeA-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1-1	SFN scheme A (scheme 1) for PDCCH only

sfn-SchemeA-PDCCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1a	Dynamic switching - scheme A

sfn-SchemeA-DynamicSwitching-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1b	SFN scheme A (scheme 1) for PDSCH only

sfn-SchemeA-PDSCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2	SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH

sfn-SchemeB-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2a	Dynamic switching - scheme B

sfn-SchemeB-DynamicSwitching-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2b	SFN scheme B (TRP based pre-compensation) for PDSCH only

sfn-SchemeB-PDSCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-2-1d	PDCCH repetition for Case 2 PDCCH monitoring with a span gap

mTRP-PDCCH-Case2-1SpanGap-r17    SEQUENCE {

scs-15kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-30kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-60kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-120kHz-r17                   PDCCH-RepetitionParameters-r17      OPTIONAL

}                                                                        OPTIONAL,

-- R1 23-2-1e	PDCCH repetition for Rel-16 PDCCH monitoring

mTRP-PDCCH-legacyMonitoring-r17  SEQUENCE {

scs-15kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-30kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL

}                                                                        OPTIONAL,

-- R1  23-2-4	Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP

mTRP-PDCCH-multiDCI-multiTRP-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 33-2:	Dynamic scheduling for multicast for PCell

dynamicMulticastPCell-r17        ENUMERATED {supported}                  OPTIONAL,

-- R1 23-2-1	PDCCH repetition

mTRP-PDCCH-Repetition-r17        SEQUENCE {

numBD-twoPDCCH-r17               INTEGER (2..3),

maxNumOverlaps-r17               ENUMERATED {n1,n2,n3,n5,n10,n20,n40}

}                                                                        OPTIONAL

}

FeatureSetDownlink-v1720 ::=                SEQUENCE {

-- R1 25-19: RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS

rtt-BasedPDC-CSI-RS-ForTracking-r17         ENUMERATED {supported}                                                   OPTIONAL,

-- R1 25-19a: RTT-based Propagation delay compensation based on DL PRS for RTT-based PDC and SRS

rtt-BasedPDC-PRS-r17                        SEQUENCE {

maxNumberPRS-Resource-r17                   ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPRS-ResourceProcessedPerSlot-r17   SEQUENCE {

scs-15kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-30kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-60kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-120kHz-r17                              ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL

}

}                                                                                                                    OPTIONAL,

-- R1 33-5-1: SPS group-common PDSCH for multicast on PCell

sps-Multicast-r17                           ENUMERATED {supported}                                                   OPTIONAL

}

FeatureSetDownlink-v1730 ::=                SEQUENCE {

-- R1 25-19b: Support of PRS as spatial relation RS for SRS

prs-AsSpatialRelationRS-For-SRS-r17         ENUMERATED {supported}                                                   OPTIONAL

}

FeatureSetDownlink-v17d0 ::=                SEQUENCE {

-- R1 23-8-4	Maximum 2 SP and 1 periodic SRS sets for antenna switching

srs-AntennaSwitching2SP-1Periodic-r17       ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-9	Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R

srs-ExtensionAperiodicSRS-r17               ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-10	1 aperiodic SRS resource set for 1T4R

srs-OneAP-SRS-r17                           ENUMERATED {supported}                             OPTIONAL

}

FeatureSetDownlink-v1800 ::=                    SEQUENCE {

-- R1 40-1-14a: Dynamic switching - scheme A

dynamicSwitchingA-r18                           ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-1-14b: Dynamic switching - scheme B

dynamicSwitchingB-r18                           ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-3-2-11: Aperiodic CSI report timing relaxation for doppler codebook based on Type-II codebook

aperiodicCSI-TimeRelaxation-r18                 SEQUENCE {

valueW-r18                                           SEQUENCE{

scs-15kHz                               ENUMERATED {value1, value2}                                              OPTIONAL,

scs-30kHz                               ENUMERATED {value1, value2}                                              OPTIONAL,

scs-60kHz                               ENUMERATED {value1, value2}                                              OPTIONAL,

scs-120kHz                              ENUMERATED {value1, value2}                                              OPTIONAL

},

timeRelaxation-r18                          ENUMERATED {cap1, cap2}

}                                                                                                                        OPTIONAL,

-- R1 40-4-1: Basic feature of Rel.18 enhanced DMRS ports for PDSCH for scheduling of mapping type A

pdsch-TypeA-DMRS-r18                            ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1a: Basic feature of Rel.18 enhanced DMRS ports for PDSCH for scheduling of mapping type B

pdsch-TypeB-DMRS-r18                            ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1b: 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH

pdsch-1SymbolFL-DMRS-Addition2Symbol-r18        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1c: Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH

pdsch-AlternativeDMRS-Coexistence-r18           ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1d: 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH

pdsch-2SymbolFL-DMRS-r18                        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1e: 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH

pdsch-2SymbolFL-DMRS-Addition2Symbol-r18        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1f: 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH

pdsch-1SymbolFL-DMRS-Addition3Symbol-r18        ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1g: DMRS type for Rel.18 enhanced DMRS ports for PDSCH

pdsch-DMRS-Type-r18                             ENUMERATED {etype1, etype1And2}                                          OPTIONAL,

-- R1 40-4-1h: 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8

pdsch-1PortDL-PTRS-r18                          ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1i: 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8

pdsch-2PortDL-PTRS-r18                          ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-1j: Support 1 symbol FL DMRS and 2 additional DMRS symbols for one port for scheduling of mapping type A

mappingTypeA-1SymbolFL-DMRS-Addition2Symbol-r18 ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-2: Capability on the maximum number of configured DMRS types for PDSCH across all DL DCI formats per cell

maxNumberDMRS-AcrossAllDL-DCI-r18               INTEGER (2..4)                                                           OPTIONAL,

-- R1 40-4-4: Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports

pdsch-ReceptionWithoutSchedulingRestriction-r18 ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-4a: Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA

pdsch-ReceptionSchemeA-r18                      ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-4b: Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB

pdsch-ReceptionSchemeB-r18                      ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-5: Rel-18 DL DMRS with single DCI based M-TRP

dmrs-MultiTRP-SingleDCI-r18                     ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-5a: Additional row(s) for antenna ports (0,2,3) for Rel.18 DL DMRS ports for single-DCI based M-TRP

dmrs-MultiTRP-AdditionRows-r18                  ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-7: Rel-18 DL DMRS with M-DCI based M-TRP

dmrs-MultiTRP-MultiDCI-r18                      ENUMERATED {supported}                                                   OPTIONAL,

-- R1 40-4-12: Support of Rel-18 DMRS and PDSCH processing capability 2 simultaneously

simulDMRS-PDSCH-r18                             SEQUENCE {

scs-15kHz-r18                                   INTEGER (0..4)                                                       OPTIONAL,

scs-30kHz-r18                                   INTEGER (0..5)                                                       OPTIONAL,

scs-60kHz-r18                                   INTEGER (0..7)                                                       OPTIONAL

}                                                                                                                        OPTIONAL,

-- R1 53-1: Support RLM/BM/BFD and gapless L3 intra-frequency measurements based on CD-SSB outside active BWP without interruptions

bwpOperationMeasWithoutInterrupt-r18            ENUMERATED {supported}                                                   OPTIONAL,

-- R1 55-6: (2, 2) span-based PDCCH monitoring with additional restriction(s)

pdcch-MonitoringSpan2-2-r18                     SEQUENCE{

pdsch-ProcessingType1-r18                       SEQUENCE{

scs-15kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL,

scs-30kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL

},

pdsch-ProcessingType2-r18                       SEQUENCE{

scs-15kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL,

scs-30kHz-r18                                   ENUMERATED {supported}                                           OPTIONAL

}

}                                                                                                                        OPTIONAL,

-- R1 55-6b: Mix of Rel-16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers

pdcch-MonitoringMixed-r18                       ENUMERATED {supported}                                                    OPTIONAL,

-- R1 55-6h: PDCCH repetition for Rel-16 PDCCH monitoring

mTRP-PDCCH-legacyMonitoring-r18                 SEQUENCE {

scs-15kHz-r18                                   PDCCH-RepetitionParameters-r17                                       OPTIONAL,

scs-30kHz-r18                                   PDCCH-RepetitionParameters-r17                                       OPTIONAL

}                                                                                                                        OPTIONAL,

-- R4 42-1: Support of SCell without SS/PBCH block for inter-band CA

scellWithoutSSB-InterBandCA-r18                 CHOICE {

supportOfSingleGroup                            ENUMERATED {referenceBand, scellWithoutSSB, both},

supportOfMultipleGroups                         ENUMERATED {referenceBand1, scellWithoutSSB1, referenceBand2, scellWithoutSSB2}

}                                                                                                                        OPTIONAL,

dummy                                           SEQUENCE (SIZE (1..maxBandsMRDC)) OF Dummy-PDCCH-RACH-DL-Info-r18        OPTIONAL

}

FeatureSetDownlink-v1830 ::=        SEQUENCE {

-- R4 39-4: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-AffectedBands-TargetBandList-r18    SEQUENCE (SIZE (1..maxBandsMRDC)) OF ENUMERATED {noInterruption, interruption}

OPTIONAL,

-- R4 39-4a: Interruption due to RF retuning for PDCCH- ordered RACH

pdcch-RACH-SwitchingTime-TargetBandList-r18    SEQUENCE (SIZE (1..maxBandsMRDC)) OF ENUMERATED {ms0, ms0dot25, ms0dot5, ms1, ms2, notSupported}

OPTIONAL,

-- R4 39-5: the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained

-- in any of UE's configured UL BWP(s) of active serving cells

pdcch-RACH-PrepTime-TargetBandList-r18         SEQUENCE (SIZE (1..maxBandsMRDC)) OF ENUMERATED {ms1, ms3, ms5, ms10, notSupported}

OPTIONAL

}

FeatureSetDownlink-v1860 ::=      SEQUENCE {

-- R1 40-5-5: Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching

srs-AntennaSwitching8T8R2SP-1Periodic-r18                   ENUMERATED {supported}                              OPTIONAL

}

FeatureSetDownlink-v1900 ::=      SEQUENCE {

-- R1 59-4-4a: Support two TAs enhancement for intra-cell beam management

twoTA-IntraCellBM-r19             ENUMERATED {supported}                                                                OPTIONAL,

-- R1 59-4-4b: Support two TAs enhancement for inter-cell beam management operation

twoTA-InterCellBM-r19             ENUMERATED {supported}                                                                OPTIONAL,

-- R1 59-4-4d: PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell

-- without CORESETPoolIndex

interCellCRFA-r19                 ENUMERATED {supported}                                                                OPTIONAL,

-- R1 59-3-3-1: Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching

srs-AntennaSwitching3T6R2SP-1Periodic-r19                  ENUMERATED {supported}                                       OPTIONAL,

-- R1 59-3-3a-1: Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching

srs-AntennaSwitching3T3R2SP-1Periodic-r19                  ENUMERATED {supported}                                       OPTIONAL

}

PDCCH-MonitoringOccasions-r16 ::= SEQUENCE {

period7span3-r16                  ENUMERATED {supported}                 OPTIONAL,

period4span3-r16                  ENUMERATED {supported}                 OPTIONAL,

period2span2-r16                  ENUMERATED {supported}                 OPTIONAL

}

PDCCH-RepetitionParameters-r17 ::= SEQUENCE {

supportedMode-r17                  ENUMERATED {intra-span, inter-span, both},

limitX-PerCC-r17                   ENUMERATED {n4, n8, n16, n32, n44, n64, nolimit}                      OPTIONAL,

limitX-AcrossCC-r17                ENUMERATED {n4, n8, n16, n32, n44, n64, n128, n256, n512, nolimit}    OPTIONAL

}

DummyA ::=      SEQUENCE {

maxNumberNZP-CSI-RS-PerCC                   INTEGER (1..32),

maxNumberPortsAcrossNZP-CSI-RS-PerCC        ENUMERATED {p2, p4, p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256},

maxNumberCS-IM-PerCC                        ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousCSI-RS-ActBWP-AllCC    ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26,

n28, n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52,

n54, n56, n58, n60, n62, n64},

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC ENUMERATED {p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256}

}

DummyB ::=       SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode1AndMode2},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyC ::=        SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p8, p16, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode2, both},

supportedNumberPanels               ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyD ::=                 SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}                          OPTIONAL,

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyE ::=    SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

Dummy-PDCCH-RACH-DL-Info-r18 ::=        CHOICE {

notSupported                            NULL,

supported                               SEQUENCE {

-- R4 39-4: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-AffectedBands-r18            ENUMERATED {noIntrruption, interruption},

-- R4 39-4a: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-SwitchingTimeList-r18        ENUMERATED {ms0, ms0dot25, ms0dot5 , ms1, ms2}                OPTIONAL,

-- R4 39-5: the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained

-- in any of UE's configured UL BWP(s) of active serving cells

pdcch-RACH-PrepTime-r18                 ENUMERATED {ms1, ms3, ms5, ms10}                              OPTIONAL

}

}

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

FeatureSetDownlink field descriptions

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

supportedSRS-Resources Indicates supported SRS resources for SRS carrier switching to the band associated with this FeatureSetDownlink. The UE is only allowed to set this field for a band with associated FeatureSetUplinkId set to 0.

#### –	FeatureSetDownlinkId

The IE FeatureSetDownlinkId identifies a downlink feature set. The FeatureSetDownlinkId of a FeatureSetDownlink is the index position of the FeatureSetDownlink in the featureSetsDownlink list in the FeatureSets IE. The first element in that list is referred to by FeatureSetDownlinkId = 1. The FeatureSetDownlinkId=0 is not used by an actual FeatureSetDownlink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetDownlinkId information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKID-START

FeatureSetDownlinkId ::=            INTEGER (0..maxDownlinkFeatureSets)

-- TAG-FEATURESETDOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetDownlinkPerCC information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-START

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                                                  OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                                           OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                                         OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                                                   OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                                                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1700 ::=   SEQUENCE {

supportedMinBandwidthDL-r17             SupportedBandwidth-v1700                                                OPTIONAL,

broadcastSCell-r17                     ENUMERATED {supported}                                                  OPTIONAL,

-- R1 33-2g: MIMO layers for multicast PDSCH

maxNumberMIMO-LayersMulticastPDSCH-r17  ENUMERATED {n2, n4, n8}                                                 OPTIONAL,

-- R1 33-2h: Dynamic scheduling for multicast for SCell

dynamicMulticastSCell-r17               ENUMERATED {supported}                                                  OPTIONAL,

supportedBandwidthDL-v1710              SupportedBandwidth-v1700                                                OPTIONAL,

-- R4 24-1/24-2/24-3/24-4/24-5

supportedCRS-InterfMitigation-r17       CRS-InterfMitigation-r17                                                OPTIONAL

}

FeatureSetDownlinkPerCC-v1720 ::=   SEQUENCE {

-- R1 33-2j: Supported maximum modulation order used for maximum data rate calculation for multicast PDSCH

maxModulationOrderForMulticastDataRateCalculation-r17  ENUMERATED {qam64, qam256, qam1024}                  OPTIONAL,

-- R1 33-1-2: FDM-ed unicast PDSCH and group-common PDSCH for broadcast

fdm-BroadcastUnicast-r17            ENUMERATED {supported}                                                  OPTIONAL,

-- R1 33-3-2: FDM-ed unicast PDSCH and one group-common PDSCH for multicast

fdm-MulticastUnicast-r17            ENUMERATED {supported}                                                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1730 ::=           SEQUENCE {

-- R1 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

intraSlotTDM-UnicastGroupCommonPDSCH-r17    ENUMERATED {yes, no}                    OPTIONAL,

-- R1 33-5-3: One SPS group-common PDSCH configuration for multicast for SCell

sps-MulticastSCell-r17                      ENUMERATED {supported}                  OPTIONAL,

-- R1 33-5-4: Up to 8 SPS group-common PDSCH configurations per CFR for multicast for SCell

sps-MulticastSCellMultiConfig-r17           INTEGER (1..8)                          OPTIONAL,

-- R1 33-1-1: Dynamic slot-level repetition for broadcast MTCH

dci-BroadcastWith16Repetitions-r17          ENUMERATED {supported}                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1780 ::=           SEQUENCE {

supportedBandwidthDL-v1780                  SupportedBandwidth-v1700                OPTIONAL

}

FeatureSetDownlinkPerCC-v1800 ::=           SEQUENCE {

-- R1 40-2-1: Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement

multiDCI-IntraCellMultiTRP-TwoTA-r18        ENUMERATED {supported}                                          OPTIONAL,

-- R1 40-2-2: Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement

multiDCI-InterCellMultiTRP-TwoTA-r18        INTEGER (1..2)                                                  OPTIONAL,

-- R1 40-2-6: Rx timing difference larger than CP length

rxTimingDiff-r18                            ENUMERATED {supported}                                          OPTIONAL,

-- R1 55-7: Two QCL TypeD for CORESET monitoring in multi-DCI based multi-TRP

multiDCI-MultiTRP-CORESET-Monitoring-r18    ENUMERATED {supported}                                          OPTIONAL,

broadcastNonServingCell-r18                 ENUMERATED {supported}                                          OPTIONAL,

-- R4 30-1: Supports scheduling restriction relaxation and measurement restriction relaxation

schedulingMeasurementRelaxation-r18         ENUMERATED {supported}                                          OPTIONAL

}

FeatureSetDownlinkPerCC-v1840 ::=           SEQUENCE {

supportedBandwidthDL-v1840                  SupportedBandwidth-v1840                OPTIONAL,

supportedMinBandwidthDL-v1840               SupportedBandwidth-v1840                OPTIONAL

}

FeatureSetDownlinkPerCC-v1900 ::=           SEQUENCE {

supportedBandwidthDL-v1900                  SupportedBandwidth-v1900                OPTIONAL,

supportedMinBandwidthDL-v1900               SupportedBandwidth-v1900                OPTIONAL,

-- R1 67-1: 32 DL HARQ processes for TN in FR1 and FR2-1

support32-DL-HARQ-ProcessTN-r19             ENUMERATED {supported}                  OPTIONAL,

maxNumberMIMO-LayersPDSCH-v1900             ENUMERATED {sixlayers}                  OPTIONAL

}

MultiDCI-MultiTRP-r16 ::=           SEQUENCE {

maxNumberCORESET-r16                ENUMERATED {n2, n3, n4, n5},

maxNumberCORESETPerPoolIndex-r16    INTEGER (1..3),

maxNumberUnicastPDSCH-PerPool-r16   ENUMERATED {n1, n2, n3, n4, n7}

}

CRS-InterfMitigation-r17 ::=        SEQUENCE {

-- R4 24-1 CRS-IM (Interference Mitigation) in DSS scenario

crs-IM-DSS-15kHzSCS-r17             ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-2 CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-15kHzSCS-r17          ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-3 CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-NWA-15kHzSCS-r17      ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-4 CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-30kHzSCS-r17          ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-5 CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-NWA-30kHzSCS-r17      ENUMERATED {supported}                                                  OPTIONAL

}

-- TAG-FEATURESETDOWNLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC-Id

The IE FeatureSetDownlinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetDownlinkPerCC-Id of a FeatureSetDownlinkPerCC is the index position of the FeatureSetDownlinkPerCC in the featureSetsDownlinkPerCC. The first element in the list is referred to by FeatureSetDownlinkPerCC-Id = 1, and so on.

FeatureSetDownlinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-ID-START

FeatureSetDownlinkPerCC-Id ::=      INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETDOWNLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-DownlinkId

The IE FeatureSetEUTRA-DownlinkId identifies a downlink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-DownlinkId = 1. The FeatureSetEUTRA-DownlinkId=0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-DownlinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRADOWNLINKID-START

FeatureSetEUTRA-DownlinkId ::=      INTEGER (0..maxEUTRA-DL-FeatureSets)

-- TAG-FEATURESETEUTRADOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-UplinkId

The IE FeatureSetEUTRA-UplinkId identifies an uplink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-UplinkId = 1. The FeatureSetEUTRA-UplinkId =0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-UplinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRAUPLINKID-START

FeatureSetEUTRA-UplinkId ::=                    INTEGER (0..maxEUTRA-UL-FeatureSets)

-- TAG-FEATURESETEUTRAUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports for that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present). The number of entries in the new list(s) shall be the same as in the original list(s).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=    SEQUENCE {

featureSetsDownlink                 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink               OPTIONAL,

featureSetsDownlinkPerCC            SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC            OPTIONAL,

featureSetsUplink                   SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink                   OPTIONAL,

featureSetsUplinkPerCC              SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC              OPTIONAL,

...,

[[

featureSetsDownlink-v1540           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540         OPTIONAL,

featureSetsUplink-v1540             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540             OPTIONAL,

featureSetsUplinkPerCC-v1540        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540        OPTIONAL

]],

[[

featureSetsDownlink-v15a0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0         OPTIONAL

]],

[[

featureSetsDownlink-v1610           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610         OPTIONAL,

featureSetsUplink-v1610             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1610             OPTIONAL,

featureSetDownlinkPerCC-v1620       SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620      OPTIONAL

]],

[[

featureSetsUplink-v1630             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1630             OPTIONAL

]],

[[

featureSetsUplink-v1640             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1640             OPTIONAL

]],

[[

featureSetsDownlink-v1700           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1700         OPTIONAL,

featureSetsDownlinkPerCC-v1700      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1700      OPTIONAL,

featureSetsUplink-v1710             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1710             OPTIONAL,

featureSetsUplinkPerCC-v1700        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1700        OPTIONAL

]],

[[

featureSetsDownlink-v1720           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1720         OPTIONAL,

featureSetsDownlinkPerCC-v1720      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1720      OPTIONAL,

featureSetsUplink-v1720             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1720             OPTIONAL

]],

[[

featureSetsDownlink-v1730           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1730         OPTIONAL,

featureSetsDownlinkPerCC-v1730      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1730      OPTIONAL

]],

[[

featureSetsDownlinkPerCC-v1780      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1780      OPTIONAL,

featureSetsUplinkPerCC-v1780        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1780        OPTIONAL

]],

[[

featureSetsDownlink-v1800           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1800         OPTIONAL,

featureSetsDownlinkPerCC-v1800      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1800      OPTIONAL,

featureSetsUplink-v1800             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1800             OPTIONAL,

featureSetsUplinkPerCC-v1800        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1800        OPTIONAL

]],

[[

featureSetsDownlink-v1830           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1830         OPTIONAL

]],

[[

featureSetsDownlinkPerCC-v1840      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1840      OPTIONAL,

featureSetsUplinkPerCC-v1840        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1840        OPTIONAL

]],

[[

featureSetsUplink-v1850             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1850             OPTIONAL,

featureSetsUplinkPerCC-v1850        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1850        OPTIONAL

]],

[[

featureSetsDownlink-v1860           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1860         OPTIONAL

]],

[[

featureSetsDownlinkPerCC-v1900      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1900      OPTIONAL,

featureSetsUplinkPerCC-v1900        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1900        OPTIONAL,

featureSetsDownlink-v1900           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1900         OPTIONAL,

featureSetsUplink-v1900             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1900             OPTIONAL

]]

}

FeatureSets-v15t0 ::=    SEQUENCE {

featureSetsDownlink-v15t0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15t0         OPTIONAL

}

FeatureSets-v16d0 ::=    SEQUENCE {

featureSetsUplink-v16d0             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v16d0             OPTIONAL

}

FeatureSets-v16k0 ::=    SEQUENCE {

featureSetsDownlink-v16k0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v16k0         OPTIONAL

}

FeatureSets-v17d0 ::=    SEQUENCE {

featureSetsDownlink-v17d0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v17d0         OPTIONAL

}

-- TAG-FEATURESETS-STOP

-- ASN1STOP

#### –	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=                SEQUENCE {

featureSetListPerUplinkCC           SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor                       ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy3                              ENUMERATED {supported}                                                  OPTIONAL,

intraBandFreqSeparationUL           FreqSeparationClass                                                     OPTIONAL,

searchSpaceSharingCA-UL             ENUMERATED {supported}                                                  OPTIONAL,

dummy1                              DummyI                                                                  OPTIONAL,

supportedSRS-Resources              SRS-Resources                                                           OPTIONAL,

twoPUCCH-Group                      ENUMERATED {supported}                                                  OPTIONAL,

dynamicSwitchSUL                    ENUMERATED {supported}                                                  OPTIONAL,

simultaneousTxSUL-NonSUL            ENUMERATED {supported}                                                  OPTIONAL,

pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-30kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-60kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-120kHz                                ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL

}                                                                                                           OPTIONAL,

dummy2                               DummyF                                                                 OPTIONAL

}

FeatureSetUplink-v1540 ::=           SEQUENCE {

zeroSlotOffsetAperiodicSRS           ENUMERATED {supported}                     OPTIONAL,

pa-PhaseDiscontinuityImpacts         ENUMERATED {supported}                     OPTIONAL,

pusch-SeparationWithGap              ENUMERATED {supported}                     OPTIONAL,

pusch-ProcessingType2                SEQUENCE {

scs-15kHz                            ProcessingParameters                       OPTIONAL,

scs-30kHz                            ProcessingParameters                       OPTIONAL,

scs-60kHz                            ProcessingParameters                       OPTIONAL

}                                                                               OPTIONAL,

ul-MCS-TableAlt-DynamicIndication    ENUMERATED {supported}                     OPTIONAL

}

FeatureSetUplink-v1610 ::=       SEQUENCE {

-- R1 11-5: PUsCH repetition Type B

pusch-RepetitionTypeB-r16        SEQUENCE {

maxNumberPUSCH-Tx-r16            ENUMERATED {n2, n3, n4, n7, n8, n12},

hoppingScheme-r16                ENUMERATED {interSlotHopping, interRepetitionHopping, both}

}                                                                              OPTIONAL,

-- R1 11-7: UL cancelation scheme for self-carrier

ul-CancellationSelfCarrier-r16       ENUMERATED {supported}                    OPTIONAL,

-- R1 11-7a: UL cancelation scheme for cross-carrier

ul-CancellationCrossCarrier-r16      ENUMERATED {supported}                    OPTIONAL,

-- R1 16-5c: The maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for Mode 2

ul-FullPwrMode2-MaxSRS-ResInSet-r16  ENUMERATED {n1, n2, n4}                   OPTIONAL,

-- R1 22-4a/4b/4c/4d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 1

cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16    SEQUENCE {

scs-15kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-30kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-60kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-120kHz-r16       ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL

} OPTIONAL,

-- R1 22-3a/3b/3c/3d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 2

cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16    SEQUENCE {

scs-15kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-30kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-60kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-120kHz-r16       ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL

} OPTIONAL,

supportedSRS-PosResources-r16              SRS-AllPosResources-r16             OPTIONAL,

intraFreqDAPS-UL-r16                             SEQUENCE {

dummy                                            ENUMERATED {supported}    OPTIONAL,

intraFreqTwoTAGs-DAPS-r16                        ENUMERATED {supported}    OPTIONAL,

dummy1                                           ENUMERATED {supported}    OPTIONAL,

dummy2                                           ENUMERATED {supported}    OPTIONAL,

dummy3                                           ENUMERATED {short, long}  OPTIONAL

}                                                                              OPTIONAL,

intraBandFreqSeparationUL-v1620                  FreqSeparationClassUL-v1620   OPTIONAL,

-- R1 11-3: More than one PUCCH for HARQ-ACK transmission within a slot

multiPUCCH-r16                        SEQUENCE {

sub-SlotConfig-NCP-r16                ENUMERATED {set1, set2}              OPTIONAL,

sub-SlotConfig-ECP-r16                ENUMERATED {set1, set2}              OPTIONAL

}                                                                              OPTIONAL,

-- R1 11-3c: 2 PUCCH of format 0 or 2 for a single 7*2-symbol subslot based HARQ-ACK codebook

twoPUCCH-Type1-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3d: 2 PUCCH of format 0 or 2 for a single 2*7-symbol subslot based HARQ-ACK codebook

twoPUCCH-Type2-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3e: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for a single 2*7-symbol HARQ-ACK codebooks

twoPUCCH-Type3-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3f: 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and

-- 11-3e

twoPUCCH-Type4-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3g: SR/HARQ-ACK multiplexing once per subslot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK

-- are supposed to be sent with different starting symbols in a subslot

mux-SR-HARQ-ACK-r16                   ENUMERATED {supported}                   OPTIONAL,

dummy1                                ENUMERATED {supported}                   OPTIONAL,

dummy2                                ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4c: 2 PUCCH of format 0 or 2 for two HARQ-ACK codebooks with one 7*2-symbol sub-slot based HARQ-ACK codebook

twoPUCCH-Type5-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4d: 2 PUCCH of format 0 or 2 in consecutive symbols for two HARQ-ACK codebooks with one 2*7-symbol sub-slot based HARQ-ACK

-- codebook

twoPUCCH-Type6-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4e: 2 PUCCH of format 0 or 2 for two subslot based HARQ-ACK codebooks

twoPUCCH-Type7-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4f: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for HARQ-ACK codebooks with one 2*7-symbol

-- subslot based HARQ-ACK codebook

twoPUCCH-Type8-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4g: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for two subslot based HARQ-ACK codebooks

twoPUCCH-Type9-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4h: 2 PUCCH transmissions in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot which are not covered

-- by 11-4c and 11-4e

twoPUCCH-Type10-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4i: 2 PUCCH transmissions in the same subslot for two subslot based HARQ-ACK codebooks which are not covered by 11-4d and

-- 11-4f

twoPUCCH-Type11-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 12-1: UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer

ul-IntraUE-Mux-r16                    SEQUENCE {

pusch-PreparationLowPriority-r16      ENUMERATED {sym0, sym1, sym2},

pusch-PreparationHighPriority-r16     ENUMERATED {sym0, sym1, sym2}

}                                                                              OPTIONAL,

-- R1 16-5a: Supported UL full power transmission mode of fullpower

ul-FullPwrMode-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 18-5d: Processing up to X unicast DCI scheduling for UL per scheduled CC

crossCarrierSchedulingProcessing-DiffSCS-r16    SEQUENCE {

scs-15kHz-120kHz-r16                  ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-15kHz-60kHz-r16                   ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-30kHz-120kHz-r16                  ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-15kHz-30kHz-r16                   ENUMERATED {n2}                      OPTIONAL,

scs-30kHz-60kHz-r16                   ENUMERATED {n2}                      OPTIONAL,

scs-60kHz-120kHz-r16                  ENUMERATED {n2}                      OPTIONAL

}                                                                              OPTIONAL,

-- R1 16-5b: Supported UL full power transmission mode of fullpowerMode1

ul-FullPwrMode1-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 16-5c-2: Ports configuration for Mode 2

ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16  ENUMERATED {p1-2, p1-4, p1-2-4} OPTIONAL,

-- R1 16-5c-3: TPMI group for Mode 2

ul-FullPwrMode2-TPMIGroup-r16         SEQUENCE {

twoPorts-r16                          BIT STRING(SIZE(2))                      OPTIONAL,

fourPortsNonCoherent-r16              ENUMERATED{g0, g1, g2, g3}               OPTIONAL,

fourPortsPartialCoherent-r16          ENUMERATED{g0, g1, g2, g3, g4, g5, g6}   OPTIONAL

}                                                                                  OPTIONAL

}

FeatureSetUplink-v1630 ::=       SEQUENCE {

-- R1 22-8: For SRS for CB PUSCH and antenna switching on FR1 with symbol level offset for aperiodic SRS transmission

offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8a: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot and constrained timeline for SRS for CB

-- PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16           ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8b: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16    ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8c: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 with a DCI gap and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16       ENUMERATED {supported}                   OPTIONAL,

dummy                                                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-9: Cancellation of PUCCH, PUSCH or PRACH with a DCI scheduling a PDSCH or CSI-RS or a DCI format 2_0 for SFI

partialCancellationPUCCH-PUSCH-PRACH-TX-r16                 ENUMERATED {supported}                   OPTIONAL

}

FeatureSetUplink-v1640 ::=              SEQUENCE {

-- R1 11-4: Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based +

-- sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE

twoHARQ-ACK-Codebook-type1-r16          SubSlot-Config-r16      OPTIONAL,

-- R1 11-4a: Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different

-- priorities at a UE

twoHARQ-ACK-Codebook-type2-r16          SubSlot-Config-r16      OPTIONAL,

-- R1 22-8d: All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap and constrained timeline

-- for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16 SEQUENCE {

scs-15kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL,

scs-30kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL,

scs-60kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL

}                                                                                                           OPTIONAL

}

FeatureSetUplink-v16d0 ::=       SEQUENCE {

pusch-RepetitionTypeB-v16d0      SEQUENCE {

maxNumberPUSCH-Tx-Cap1-r16       ENUMERATED {n2, n3, n4, n7, n8, n12},

maxNumberPUSCH-Tx-Cap2-r16       ENUMERATED {n2, n3, n4, n7, n8, n12}

}                                                                                         OPTIONAL

}

FeatureSetUplink-v1710 ::= SEQUENCE {

-- R1 23-3-1	Multi-TRP PUSCH repetition (type A) -codebook based

mTRP-PUSCH-TypeA-CB-r17                ENUMERATED {n1,n2,n4}                              OPTIONAL,

-- R1 23-3-1-2	Multi-TRP PUSCH repetition (type A) - non-codebook based

mTRP-PUSCH-RepetitionTypeA-r17         ENUMERATED {n1,n2,n3,n4}                           OPTIONAL,

-- R1 23-3-3	Multi-TRP PUCCH repetition-intra-slot

mTRP-PUCCH-IntraSlot-r17               ENUMERATED {pf0-2, pf1-3-4, pf0-4}                 OPTIONAL,

-- R1 23-8-4	Maximum 2 SP and 1 periodic SRS sets for antenna switching

srs-AntennaSwitching2SP-1Periodic-r17  ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-9	Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R

srs-ExtensionAperiodicSRS-r17          ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-10	1 aperiodic SRS resource set for 1T4R

srs-OneAP-SRS-r17                      ENUMERATED {supported}                             OPTIONAL,

-- R4 16-8 UE power class per band per band combination

ue-PowerClassPerBandPerBC-r17          ENUMERATED {pc1dot5, pc2, pc3}                     OPTIONAL,

-- R4 17-8 UL transmission in FR2 bands within an UL gap when the UL gap is activated

tx-Support-UL-GapFR2-r17               ENUMERATED {supported}                             OPTIONAL

}

FeatureSetUplink-v1720 ::= SEQUENCE {

-- R1 25-3: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8

pucch-Repetition-F0-1-2-3-4-RRC-Config-r17         ENUMERATED {supported}                 OPTIONAL,

-- R1 25-3a: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication

pucch-Repetition-F0-1-2-3-4-DynamicIndication-r17  ENUMERATED {supported}                 OPTIONAL,

-- R1 25-3b: Inter-subslot frequency hopping for PUCCH repetitions

interSubslotFreqHopping-PUCCH-r17                  ENUMERATED {supported}                 OPTIONAL,

-- R1 25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH

semiStaticHARQ-ACK-CodebookSub-SlotPUCCH-r17       ENUMERATED {supported}                 OPTIONAL,

-- R1 25-14: PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH

phy-PrioritizationLowPriorityDG-HighPriorityCG-r17 INTEGER(1..16)                         OPTIONAL,

-- R1 25-15: PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH

phy-PrioritizationHighPriorityDG-LowPriorityCG-r17 SEQUENCE {

pusch-PreparationLowPriority-r17                   ENUMERATED{sym0, sym1, sym2},

additionalCancellationTime-r17                     SEQUENCE {

scs-15kHz-r17                                      ENUMERATED{sym0, sym1, sym2}   OPTIONAL,

scs-30kHz-r17                                      ENUMERATED{sym0, sym1, sym2, sym3, sym4}    OPTIONAL,

scs-60kHz-r17                                      ENUMERATED{sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7, sym8} OPTIONAL,

scs-120kHz-r17                                     ENUMERATED{sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7, sym8, sym9,

sym10, sym11, sym12, sym13, sym14, sym15, sym16}    OPTIONAL

},

maxNumberCarriers-r17                              INTEGER(1..16)

}                                                                                         OPTIONAL,

-- R4 17-5 Support of UL DC location(s) report

extendedDC-LocationReport-r17                      ENUMERATED {supported}                 OPTIONAL

}

FeatureSetUplink-v1800 ::=                         SEQUENCE {

-- R1 40-3-3-1a: Supported maximum delay value larger than D_basic

maxDelayValueBeyondD-Basic-r18                     ENUMERATED {sl2,sl3,sl4,sl5,sl6,sl10}                        OPTIONAL,

-- R1 40-3-3-2: Number of delay values

tdcp-NumberDelayValue-r18                          INTEGER (2..4)                                               OPTIONAL,

-- R1 40-3-3-4: Phase report

phaseReportMoreThanOne-r18                         ENUMERATED {supported}                                       OPTIONAL,

-- R1 40-3-3-6: Maximum number of TRS resource sets in a report configuration

maxNumberTRS-ResourceSet-r18                       INTEGER (2..3)                                               OPTIONAL,

-- R1 40-3-3-7: Maximum number of TDCP report settings per-BWP

maxNumberTDCP-PerBWP-r18                           INTEGER (1..4)                                               OPTIONAL,

-- R1 40-4-6c: DMRS type for Rel.18 enhanced DMRS ports for PUSCH

pusch-DMRS-TypeEnh-r18                             SEQUENCE {

dmrs-Type-r18                                      ENUMERATED {etype1, both},

pusch-TypeA-DMRS-r18                               SEQUENCE {

-- R1 40-4-6: Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling mapping of type A for Rel.18 enhanced

-- DMRS ports

dmrs-TypeA-r18                                     ENUMERATED {supported},

-- R1 40-4-6d: 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH

pusch-2SymbolFL-DMRS-r18                           ENUMERATED {supported}                               OPTIONAL,

-- R1 40-4-6e: 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH

pusch-2SymbolFL-DMRS-Addition2Symbol-r18           ENUMERATED {supported}                               OPTIONAL,

-- R1 40-4-6f: 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH

pusch-1SymbolFL-DMRS-Addition3Symbol-r18           ENUMERATED {supported}                               OPTIONAL,

-- R1 40-4-6k: 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for

-- PUSCH

pusch-1SymbolFL-DMRS-BeyondOnePort-r18             ENUMERATED {supported}                               OPTIONAL

}                                                                                                           OPTIONAL,

-- R1 40-4-10: DMRS port configuration for PUSCH with 8Tx

dummy                                              ENUMERATED {rel15, both}                                 OPTIONAL,

-- R1 40-4-6a: Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports

pusch-TypeB-DMRS-r18                               ENUMERATED {supported}                                   OPTIONAL,

-- R1 40-4-6g: 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4

pusch-rank-1-4-1Port-r18                           ENUMERATED {supported}                                   OPTIONAL,

-- R1 40-4-6h: 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8

pusch-rank-5-8-1Port-r18                           ENUMERATED {supported}                                   OPTIONAL,

-- R1 40-4-6i: 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4

pusch-rank-1-4-2Port-r18                           ENUMERATED {supported}                                   OPTIONAL,

-- R1 40-4-6j: 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8

pusch-rank-5-8-2Port-r18                           ENUMERATED {supported}                                   OPTIONAL

}                                                                                                               OPTIONAL,

-- R1 40-4-13: Support Rel-18 UL DMRS with single-DCI based M-TRP

ul-DMRS-SingleDCI-M-TRP-r18                        ENUMERATED {supported}                                       OPTIONAL,

-- R1 40-4-14: Support Rel-18 UL DMRS with M-DCI based M-TRP

ul-DMRS-M-DCI-M-TRP-r18                            ENUMERATED {supported}                                       OPTIONAL,

-- R1 40-5-5: Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching

srs-AntennaSwitching8T8R2SP-1Periodic-r18                   ENUMERATED {supported}                              OPTIONAL,

-- R1 40-6-4: Single-DCI based STx2P SFN scheme for PUCCH

pucch-SingleDCI-STx2P-SFN-r18                      ENUMERATED {pf0-2, pf1-3-4, pf0-4}                           OPTIONAL,

-- R1 41-4-6: Positioning SRS bandwidth aggregation in RRC_CONNECTED

posSRS-BWA-RRC-Connected-r18                       PosSRS-BWA-RRC-Connected-r18                                 OPTIONAL,

-- R1 41-4-7: Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED

posSRS-BWA-IndependentCA-RRC-Connected-r18         PosSRS-BWA-IndependentCA-RRC-Connected-r18                   OPTIONAL,

-- R1 41-4-9: Indicate which other bands in the band combination are affected due to the need of a guard period

posSRS-BWA-AffectedBandList-r18                    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR         OPTIONAL,

-- R1 45-5a: RACH-based early TA acquisition with simultaneous transmission

rach-EarlyTA-BandList-r18                          SEQUENCE (SIZE (1..maxBandsMRDC)) OF BOOLEAN                 OPTIONAL,

-- R1 49-6: Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting

-- HARQ-ACK codebooks with different priorities by DCI format 1_3

simultaneous-2-1-HARQ-ACK-CB-r18                   SubSlot-Config-r16                                           OPTIONAL,

-- R1 49-6a: Two HARQ-ACK codebooks with two sub-slot based HARQ-ACK codebook simultaneously constructed for supporting

-- HARQ-ACK codebooks with different priorities by DCI format 1_3

simultaneous-2-2-HARQ-ACK-CB-r18                   SubSlot-Config-r16                                           OPTIONAL,

-- R1 49-7: UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical

-- layer for DCI format 1_3/0_3

ul-IntraUE-MuxEnh-r18                              SEQUENCE {

pusch-PreparationLowPriority-r18                   ENUMERATED {sym0, sym1, sym2},

pusch-PreparationHighPriority-r18                  ENUMERATED {sym0, sym1, sym2}

}                                                                                                               OPTIONAL,

-- R4 27-1 TxDiversity for 4Tx

txDiversity4Tx-r18                                 ENUMERATED {supported}                                       OPTIONAL,

-- R4 41-2: Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions without modified spectrum flatness requirement

powerBoosting-pi2BPSK-QPSK-r18                     ENUMERATED {supported}                                       OPTIONAL,

-- R4 41-3: Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions with modified spectrum flatness requirement shaping

powerBoosting-pi2BPSK-QPSK-Modified-r18            ENUMERATED {supported}                                       OPTIONAL,

-- R4 44-1 TxDiversity for 2Tx

txDiversity2Tx-r18                                 ENUMERATED {supported}                                       OPTIONAL,

ue-PowerClassPerBandPerBC-v1820                    ENUMERATED {pc5}                                             OPTIONAL

}

FeatureSetUplink-v1850 ::=                             SEQUENCE {

-- R1 40-4-10: DMRS port configuration for PUSCH with 8Tx

pusch-DMRS8Tx-r18                                      ENUMERATED {rel15, both}                                 OPTIONAL,

-- R1 40-7-1h: UE 8Tx PUSCH processing capability for codebook

additionalTime-CB-8TxPUSCH-r18                     SEQUENCE {

scs-15kHz-r18                                      ENUMERATED {sym1, sym2, sym4}                            OPTIONAL,

scs-30kHz-r18                                      ENUMERATED {sym1, sym2, sym4, sym8}                      OPTIONAL,

scs-60kHz-r18                                      ENUMERATED {sym2, sym4, sym8, sym16}                     OPTIONAL,

scs-120kHz-r18                                      ENUMERATED {sym4, sym8, sym16, sym32}                   OPTIONAL,

scs-480kHz-r18                                      ENUMERATED {sym16, sym32, sym64, sym128}                OPTIONAL,

scs-960kHz-r18                                      ENUMERATED {sym32, sym64, sym128, sym256}               OPTIONAL

}                                                                                                               OPTIONAL,

-- R1 40-7-2b: UE 8Tx PUSCH processing capability for non-codebook

additionalTime-NonCB-8TxPUSCH-r18                     SEQUENCE {

scs-15kHz-r18                                      ENUMERATED {sym1, sym2, sym4}                            OPTIONAL,

scs-30kHz-r18                                      ENUMERATED {sym1, sym2, sym4, sym8}                      OPTIONAL,

scs-60kHz-r18                                      ENUMERATED {sym2, sym4, sym8, sym16}                     OPTIONAL,

scs-120kHz-r18                                      ENUMERATED {sym4, sym8, sym16, sym32}                   OPTIONAL,

scs-480kHz-r18                                      ENUMERATED {sym16, sym32, sym64, sym128}                OPTIONAL,

scs-960kHz-r18                                      ENUMERATED {sym32, sym64, sym128, sym256}               OPTIONAL

}                                                                                                               OPTIONAL

}

FeatureSetUplink-v1900 ::=                             SEQUENCE {

-- R1 59-2-1-8: SRS Port Grouping

srs-PortGrouping-r19                                  ENUMERATED {xt8r, xt6r, both}                             OPTIONAL,

-- R1 59-2-4: Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH

nonCodebook-CSI-RS-SRS-Enh-r19                        ENUMERATED {supported}                                    OPTIONAL,

-- R1 59-3-3-1: Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching

srs-AntennaSwitching3T6R2SP-1Periodic-r19             ENUMERATED {supported}                                    OPTIONAL,

-- R1 59-3-3a-1: Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching

srs-AntennaSwitching3T3R2SP-1Periodic-r19                   ENUMERATED {supported}                              OPTIONAL,

-- R1 59-3-4: M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based

mTRP-PUSCH-TypeA-CB-3Port-r19                         INTEGER (1..2)                                            OPTIONAL,

-- R1 59-3-4a: M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based

mTRP-PUSCH-RepetitionTypeA-3Port-r19                  INTEGER (1..3)                                            OPTIONAL,

-- R1 59-3-6: PTRS of 3-antenna-port PUSCH transmission

threePortsPTRS-PUSCH-r19                              INTEGER (1..2)                                            OPTIONAL,

-- R1 59-3-7: UL full power transmission mode of fullpower

ul-FullPwrMode-3Port-r19                              ENUMERATED {supported}                                    OPTIONAL

}

SubSlot-Config-r16 ::=                  SEQUENCE {

sub-SlotConfig-NCP-r16                  ENUMERATED {n4,n5,n6,n7}              OPTIONAL,

sub-SlotConfig-ECP-r16                  ENUMERATED {n4,n5,n6}                 OPTIONAL

}

SRS-AllPosResources-r16 ::=               SEQUENCE {

srs-PosResources-r16                      SRS-PosResources-r16,

srs-PosResourceAP-r16                     SRS-PosResourceAP-r16                OPTIONAL,

srs-PosResourceSP-r16                     SRS-PosResourceSP-r16                OPTIONAL

}

SRS-PosResources-r16 ::=                       SEQUENCE {

maxNumberSRS-PosResourceSetPerBWP-r16                ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSRS-ResourcesPerBWP-PerSlot-r16             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16          ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r16  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceAP-r16 ::=                SEQUENCE {

maxNumberAP-SRS-PosResourcesPerBWP-r16         ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-PerSlot-r16 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceSP-r16 ::=                       SEQUENCE {

maxNumberSP-SRS-PosResourcesPerBWP-r16               ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSP-SRS-PosResourcesPerBWP-PerSlot-r16       ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-Resources ::=                           SEQUENCE {

maxNumberAperiodicSRS-PerBWP                ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot        INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP                 ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot         INTEGER (1..6),

maxNumberSemiPersistentSRS-PerBWP           ENUMERATED {n1, n2, n4, n8, n16},

maxNumberSemiPersistentSRS-PerBWP-PerSlot   INTEGER (1..6),

maxNumberSRS-Ports-PerResource              ENUMERATED {n1, n2, n4}

}

DummyF ::=                                  SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP           INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP          INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP     INTEGER (0..4),

simultaneousCSI-ReportsAllCC                INTEGER (5..32)

}

PosSRS-BWA-RRC-Connected-r18 ::=                  SEQUENCE {

numOfCarriersIntraBandContiguous-r18              ENUMERATED {two, three, twoandthree},

maximumAggregatedBW-TwoCarriersFR1-r18            ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100,

mhz160, mhz180, mhz190, mhz200}                  OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18            ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600, mhz800}   OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18          ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240, mhz300}   OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18          ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400,

mhz600, mhz800, mhz1000, mhz1200}

OPTIONAL,

maximumAggregatedResourceSet-r18                  ENUMERATED {n1, n2, n4, n8, n12, n16},

maximumAggregatedResourcePeriodic-r18             ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceAperiodic-r18            ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceSemi-r18                 ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourcePeriodicPerSlot-r18      ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maximumAggregatedResourceAperiodicPerSlot-r18     ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maximumAggregatedResourceSemiPerSlot-r18          ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

...

}

PosSRS-BWA-IndependentCA-RRC-Connected-r18 ::=    SEQUENCE {

numOfCarriersIntraBandContiguous-r18              ENUMERATED {two, three, twoandthree},

maximumAggregatedBW-TwoCarriersFR1-r18            ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100,

mhz160, mhz180, mhz190, mhz200}                     OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18            ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600, mhz800}      OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18          ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240, mhz300}      OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18          ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400,

mhz600, mhz800, mhz1000, mhz1200}

OPTIONAL,

maximumAggregatedResourceSet-r18                  ENUMERATED {n1, n2, n4, n8, n12, n16},

maximumAggregatedResourcePeriodic-r18             ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceAperiodic-r18            ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourceSemi-r18                 ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64},

maximumAggregatedResourcePeriodicPerSlot-r18      ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maximumAggregatedResourceAperiodicPerSlot-r18     ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maximumAggregatedResourceSemiPerSlot-r18          ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

guardPeriod-r18                                   ENUMERATED {n0, n30, n100, n140, n200},

powerClassForTwoAggregatedCarriers-r18            ENUMERATED {pc2, pc3}                                           OPTIONAL,

powerClassForThreeAggregatedCarriers-r18          ENUMERATED {pc2, pc3}                                           OPTIONAL,

...

}

-- TAG-FEATURESETUPLINK-STOP

-- ASN1STOP

FeatureSetUplink field descriptions

featureSetListPerUplinkCC Indicates which features the UE supports on the individual UL carriers of the feature set (and hence of a band entry that refers to the feature set). The UE shall hence include at least as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetUplinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

#### –	FeatureSetUplinkId

The IE FeatureSetUplinkId identifies an uplink feature set. The FeatureSetUplinkId of a FeatureSetUplink is the index position of the FeatureSetUplink in the featureSetsUplink list in the FeatureSets IE. The first element in the list is referred to by FeatureSetUplinkId = 1, and so on. The FeatureSetUplinkId =0 is not used by an actual FeatureSetUplink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetUplinkId information element

-- ASN1START

-- TAG-FEATURESETUPLINKID-START

FeatureSetUplinkId ::=                  INTEGER (0..maxUplinkFeatureSets)

-- TAG-FEATURESETUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC

The IE FeatureSetUplinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetUplinkPerCC information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-START

FeatureSetUplinkPerCC ::=               SEQUENCE {

supportedSubcarrierSpacingUL            SubcarrierSpacing,

supportedBandwidthUL                    SupportedBandwidth,

channelBW-90mhz                         ENUMERATED {supported}                      OPTIONAL,

mimo-CB-PUSCH                           SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH            MIMO-LayersUL                               OPTIONAL,

maxNumberSRS-ResourcePerSet             INTEGER (1..2)

}                                                                                   OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH         MIMO-LayersUL                               OPTIONAL,

supportedModulationOrderUL              ModulationOrder                             OPTIONAL

}

FeatureSetUplinkPerCC-v1540 ::=       SEQUENCE {

mimo-NonCB-PUSCH                      SEQUENCE {

maxNumberSRS-ResourcePerSet           INTEGER (1..4),

maxNumberSimultaneousSRS-ResourceTx   INTEGER (1..4)

} OPTIONAL

}

FeatureSetUplinkPerCC-v1700 ::=   SEQUENCE {

supportedMinBandwidthUL-r17       SupportedBandwidth-v1700                          OPTIONAL,

-- R1 23-3-1-3	FeMIMO: Multi-TRP PUSCH repetition (type B) - non-codebook based

mTRP-PUSCH-RepetitionTypeB-r17    ENUMERATED {n1,n2,n3,n4}                          OPTIONAL,

-- R1 23-3-1-1 -codebook based Multi-TRP PUSCH repetition (type B)

mTRP-PUSCH-TypeB-CB-r17           ENUMERATED {n1,n2,n4}                             OPTIONAL,

supportedBandwidthUL-v1710        SupportedBandwidth-v1700                          OPTIONAL

}

FeatureSetUplinkPerCC-v1780 ::=   SEQUENCE {

supportedBandwidthUL-v1780        SupportedBandwidth-v1700                          OPTIONAL

}

FeatureSetUplinkPerCC-v1800 ::=   SEQUENCE {

-- R1 40-2-7: Two TAs for multi-DCI STxMP PUSCH+PUSCH

twoPUSCH-MultiDCI-STx2P-TwoTA-r18      ENUMERATED {supported}                       OPTIONAL,

-- R1 40-6-1: Single-DCI based STx2P SDM scheme for PUSCH-codebook

pusch-CB-SingleDCI-STx2P-SDM-r18       SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             ENUMERATED {n1,n2,n4},

maxNumberLayerPerPanel-r18                  INTEGER (1..2),

maxNumberNZP-PUSCH-PortsPerSet-r18          ENUMERATED {n1,n2,n4},

maxNumberSRS-AntennaPortsPerSet-r18         ENUMERATED {n1,n2,n4}

}                                                                                   OPTIONAL,

-- R1 40-6-1a: Single-DCI based STx2P SDM scheme for PUSCH-noncodebook

pusch-NonCB-SingleDCI-STx2P-SDM-r18    SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             INTEGER (1..4),

maxNumberLayerPerPanel-r18                  INTEGER (1..2),

maxNumberSimulSRS-OneResourcePerSet-r18     INTEGER (1..4),

maxNumberSimulSRS-TwoResourcePerSet-r18     INTEGER (1..8)

}                                                                                   OPTIONAL,

-- R1 40-6-2: Single-DCI based STx2P SFN scheme for PUSCH-codebook

pusch-CB-SingleDCI-STx2P-SFN-r18       SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             ENUMERATED {n1,n2,n4},

maxNumberLayerPerSet-r18                    INTEGER (1..2),

maxNumberSRS-AntennaPortsPerSet-r18         ENUMERATED {n1,n2,n4},

maxNumberNZP-PUSCH-PortsPerSet-r18          ENUMERATED {n1,n2,n4}

}                                                                                   OPTIONAL,

-- R1 40-6-2a: Single-DCI based STx2P SFN scheme for PUSCH-noncodebook

pusch-NonCB-SingleDCI-STx2P-SFN-r18    SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             INTEGER (1..4),

maxNumberLayerPerSet-r18                    INTEGER (1..2),

maxNumberSimulSRS-OneResourcePerSet-r18     INTEGER (1..4),

maxNumberSimulSRS-TwoResourcePerSet-r18     INTEGER (1..8)

}                                                                                   OPTIONAL,

-- R1 40-6-3a: codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18   SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             ENUMERATED {n1, n2, n4},

maxNumberLayerOverlapping-r18               INTEGER (1..2),

maxNumberNZP-PUSCH-Overlapping-r18          ENUMERATED {n1, n2, n4},

maxNumberPUSCH-PerCORESET-PerSlot-r18       SEQUENCE {

scs-60kHz-r18                             ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL,

scs-120kHz-r18                            ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL

}                                                                              OPTIONAL,

maxNumberTotalLayerOverlapping-r18          INTEGER (2..4),

maxNumberSRS-AntennaPortsPerSet-r18         ENUMERATED {n1,n2,n4}

}                                                                                   OPTIONAL,

-- R1 40-6-3b: Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

twoPUSCH-NonCB-MultiDCI-STx2P-DG-DG-r18    SEQUENCE {

maxNumberSRS-ResourcePerSet-r18             INTEGER (1..4),

maxNumberLayerOverlapping-r18               INTEGER (1..2),

maxNumberSimulSRS-ResourcePerSet-r18        INTEGER (1..4),

maxNumberPUSCH-PerCORESET-PerSlot-r18       SEQUENCE {

scs-60kHz-r18                             ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL,

scs-120kHz-r18                            ENUMERATED {n1,n2,n3,n4,n7}     OPTIONAL

}                                                                              OPTIONAL,

maxNumberTotalLayerOverlapping-r18          INTEGER (2..4)

}                                                                                   OPTIONAL,

-- R1 40-6-6: Out-of-order operation for multi-DCI based STx2P PUSCH+PUSCH

twoPUSCH-MultiDCI-STx2P-OutOfOrder-r18           ENUMERATED {supported}             OPTIONAL,

codebookParameter8TxPUSCH-r18        SEQUENCE {

-- R1 40-7-1: Basic features for Codebook-based 8Tx PUSCH

codebook-8TxBasic-r18                        SEQUENCE {

maxNumberPUSCH-MIMO-Layer-r18                INTEGER (1..8),

maxNumberSRS-Resource-r18                    INTEGER (1..2),

srs-8TxPorts-r18                             ENUMERATED {noTDM, both}

},

-- R1 40-7-1a: Codebook-based 8Tx PUSCH-codebook1

codebook1-8TxPUSCH-r18               SEQUENCE {

codebookN1N4-r18                     ENUMERATED {ng1n4n1,ng1n2n2,both}      OPTIONAL,

srs-8TxPorts-r18                     ENUMERATED {noTDM, both}

},

-- R1 40-7-1b: Codebook-based 8Tx PUSCH-codebook2

codebook2-8TxPUSCH-r18                   ENUMERATED {supported}                 OPTIONAL,

-- R1 40-7-1c: Codebook-based 8Tx PUSCH-codebook3

codebook3-8TxPUSCH-r18                   ENUMERATED {supported}                 OPTIONAL,

-- R1 40-7-1d: Codebook-based 8Tx PUSCH-codebook4

codebook4-8TxPUSCH-r18                   ENUMERATED {supported}                 OPTIONAL,

-- R1 40-7-1e: UL full power transmission mode 0

ul-FullPwrTransMode0-r18             ENUMERATED {supported}                     OPTIONAL,

-- R1 40-7-1f: UL full power transmission mode 1

ul-FullPwrTransMode1-r18             ENUMERATED {supported}                     OPTIONAL,

-- R1 40-7-1g: UL full power transmission mode 2 with 1/2/4 resources

ul-FullPwrTransMode2-r18             ENUMERATED {n1,n2,n4}                      OPTIONAL,

-- R1 40-7-1g-1: SRS resources for UL full power transmission mode 2

ul-SRS-TransMode2-r18               BIT STRING (SIZE(3))                       OPTIONAL,

-- R1 40-7-1g-2: TPMI group(s) which delivers full power for codebook2

tpmi-FullPwrCodebook2-r18            ENUMERATED {first, second}                 OPTIONAL

}                                                                                   OPTIONAL,

-- R1 40-7-2: Basic features for Non-Codebook-based 8Tx PUSCH

nonCodebook-8TxPUSCH-r18             SEQUENCE {

maxNumberPUSCH-MIMO-Layer-r18        INTEGER (1..8),

maxNumberSRS-Resource-r18            INTEGER (1..8),

maxNumberSimultaneousSRS-r18         INTEGER (1..8)

}                                                                                   OPTIONAL,

-- R1 40-7-2a: Association between CSI-RS and SRS for non-codebook case

nonCodebook-CSI-RS-SRS-r18           ENUMERATED {supported}                            OPTIONAL,

-- R1 40-7-3: CBG based 2 CWs PUSCH with rank >4

cgb-2CW-PUSCH-r18                    ENUMERATED {supported}                         OPTIONAL

}

FeatureSetUplinkPerCC-v1840 ::=          SEQUENCE {

supportedBandwidthUL-v1840               SupportedBandwidth-v1840                   OPTIONAL,

supportedMinBandwidthUL-v1840            SupportedBandwidth-v1840                   OPTIONAL

}

FeatureSetUplinkPerCC-v1850 ::=          SEQUENCE {

-- R1 40-6-3a-1: UE STxMP processing capability for codebook

twoPUSCH-CB-MultiDCI-STx2P-AdditionalTime-r18        CHOICE {

scs-60kHz-r18               ENUMERATED {sym1, sym4, sym8, sym16},

scs-120kHz-r18              ENUMERATED {sym4, sym8, sym16, sym32},

scs-480kHz-r18              ENUMERATED {sym16, sym32, sym64, sym128},

scs-960kHz-r18              ENUMERATED {sym32, sym64, sym128,sym256}

}                                                                                   OPTIONAL,

-- R1 40-6-3b-2: UE STxMP processing capability for non-codebook

twoPUSCH-NonCB-MultiDCI-STx2P-AdditionalTime-r18        CHOICE {

scs-60kHz-r18               ENUMERATED {sym1, sym4, sym8, sym16},

scs-120kHz-r18              ENUMERATED {sym4, sym8, sym16, sym32},

scs-480kHz-r18              ENUMERATED {sym16, sym32, sym64, sym128},

scs-960kHz-r18              ENUMERATED {sym32, sym64, sym128,sym256}

}                                                                                   OPTIONAL

}

FeatureSetUplinkPerCC-v1900 ::=          SEQUENCE {

supportedBandwidthUL-v1900               SupportedBandwidth-v1900                   OPTIONAL,

supportedMinBandwidthUL-v1900            SupportedBandwidth-v1900                   OPTIONAL,

-- R1 67-1a: 32 UL HARQ processes for TN in FR1 and FR2-1

support32-UL-HARQ-ProcessTN-r19          ENUMERATED {supported}                     OPTIONAL,

-- R1 59-3-1: Non-codebook based PUSCH transmission for 3TX for single TRP

nonCodebook-3TxPUSCH-SingleTRP-r19       SEQUENCE {

maxNumberLayer-r19                       INTEGER (1..3),

maxNumberSRS-Resource-r19                INTEGER (1..3),

maxNumberSimultaneousSRS-r19             INTEGER (1..3)

}                                                                                   OPTIONAL,

-- R1 59-3-2: Codebook based PUSCH transmission for 3TX for single TRP

codebook-3TxPUSCH-SingleTRP-r19          SEQUENCE {

maxNumberPUSCH-MIMO-Layer-r19            INTEGER (1..3),

maxNumberSRS-Resource-r19                INTEGER (1..2)

}                                                                                   OPTIONAL,

-- R1 59-3-5: M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based

codebook-3PortPUSCH-TypeB-r19            INTEGER (1..2)                             OPTIONAL,

-- R1 59-3-5a: M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based

mTRP-PUSCH-RepetitionTypeB-3Port-r19     INTEGER (1..3)                             OPTIONAL

}

-- TAG-FEATURESETUPLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC-Id

The IE FeatureSetUplinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetUplinkPerCC-Id of a FeatureSetUplinkPerCC is the index position of the FeatureSetUplinkPerCC in the featureSetsUplinkPerCC. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.

FeatureSetUplinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-ID-START

FeatureSetUplinkPerCC-Id ::=            INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETUPLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQBANDINDICATOREUTRA-START

FreqBandIndicatorEUTRA ::=  INTEGER (1..maxBandsEUTRA)

-- TAG-FREQBANDINDICATOREUTRA-STOP

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used by the network to request NR CA, NR non-CA and/or MR-DC band combinations for specific NR and/or E-UTRA frequency bands and/or up to a specific number of carriers and/or up to specific aggregated bandwidth. This is also used to request feature sets (for NR) and feature set combinations (for NR and MR-DC). For NR sidelink communication, this is used by the initiating UE to request sidelink UE radio access capabilities from the peer UE. This is also used to request lower MSD capability for specific NR frequency bands for the UE supporting lower MSD.

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=                SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::=         CHOICE {

bandInformationEUTRA            FreqBandInformationEUTRA,

bandInformationNR               FreqBandInformationNR

}

FreqBandInformationEUTRA ::=    SEQUENCE {

bandEUTRA                       FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL,   -- Need N

ca-BandwidthClassUL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL    -- Need N

}

FreqBandInformationNR ::=       SEQUENCE {

bandNR                          FreqBandIndicatorNR,

maxBandwidthRequestedDL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxBandwidthRequestedUL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxCarriersRequestedDL          INTEGER (1..maxNrofServingCells)        OPTIONAL,   -- Need N

maxCarriersRequestedUL          INTEGER (1..maxNrofServingCells)        OPTIONAL    -- Need N

}

AggregatedBandwidth ::=         ENUMERATED {mhz50, mhz100, mhz150, mhz200, mhz250, mhz300, mhz350,

mhz400, mhz450, mhz500, mhz550, mhz600, mhz650, mhz700, mhz750, mhz800}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

#### –	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::= ENUMERATED { mhz800, mhz1200, mhz1400, ..., mhz400-v1650, mhz600-v1650}

FreqSeparationClassDL-v1620 ::= ENUMERATED {mhz1000, mhz1600, mhz1800, mhz2000, mhz2200, mhz2400}

FreqSeparationClassUL-v1620 ::= ENUMERATED {mhz1000}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	FreqSeparationClassDL-Only

The IE FreqSeparationClassDL-Only is used to indicate the frequency separation between lower edge of lowest CC and upper edge of highest CC of DL only frequency spectrum in a frequency band.

FreqSeparationClassDL-Only information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASSDL-Only-START

FreqSeparationClassDL-Only-r16 ::= ENUMERATED {mhz200, mhz400, mhz600, mhz800, mhz1000, mhz1200}

-- TAG-FREQSEPARATIONCLASSDL-Only-STOP

-- ASN1STOP

#### –	FR2-2-AccessParamsPerBand

The IE FR2-2-AccessParamsPerBand is used to convey FR2-2 related parameters specific for a certain frequency band (not per feature set or band combination).

FR2-2-AccessParamsPerBand information element

-- ASN1START

-- TAG-FR2-2-ACCESSPARAMSPERBAND-START

FR2-2-AccessParamsPerBand-r17 ::=       SEQUENCE {

-- R1 24-1: Basic FR2-2 DL support

dl-FR2-2-SCS-120kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-1a: Basic FR2-2 UL support

ul-FR2-2-SCS-120kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-2: 120KHz SSB support for initial access in FR2-2

initialAccessSSB-120kHz-r17             ENUMERATED {supported}            OPTIONAL,

-- R1 24-1b: Wideband PRACH for 120 kHz in FR2-2

widebandPRACH-SCS-120kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-1c: Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2

multiRB-PUCCH-SCS-120kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-1d: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2

multiPDSCH-SingleDCI-FR2-2-SCS-120kHz-r17 ENUMERATED {supported}          OPTIONAL,

-- R1 24-1e: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2

multiPUSCH-SingleDCI-FR2-2-SCS-120kHz-r17 ENUMERATED {supported}          OPTIONAL,

-- R1 24-4: 480KHz SCS support for DL

dl-FR2-2-SCS-480kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-4a: 480KHz SCS support for UL

ul-FR2-2-SCS-480kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-3: 480KHz SSB support for initial access in FR2-2

initialAccessSSB-480kHz-r17             ENUMERATED {supported}            OPTIONAL,

-- R1 24-4b: Wideband PRACH for 480 kHz in FR2-2

widebandPRACH-SCS-480kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-4c: Multi-RB support PUCCH format 0/1/4 for 480 kHz in FR2-2

multiRB-PUCCH-SCS-480kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-4f: Enhanced PDCCH monitoring for 480KHz in FR2-2

enhancedPDCCH-monitoringSCS-480kHz-r17  ENUMERATED {supported}            OPTIONAL,

-- R1 24-5: 960KHz SCS support for DL

dl-FR2-2-SCS-960kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-5a: 960KHz SCS support for UL

ul-FR2-2-SCS-960kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-5c: Multi-RB support PUCCH format 0/1/4 for 960 kHz in FR2-2

multiRB-PUCCH-SCS-960kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-5f: Enhanced PDCCH monitoring for 960KHz in FR2-2

enhancedPDCCH-monitoringSCS-960kHz-r17  SEQUENCE {

pdcch-monitoring4-1-r17                 ENUMERATED {supported}        OPTIONAL,

pdcch-monitoring4-2-r17                 ENUMERATED {supported}        OPTIONAL,

pdcch-monitoring8-4-r17                 ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

-- R1 24-6: Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access

type1-ChannelAccess-FR2-2-r17           ENUMERATED {supported}            OPTIONAL,

-- R1 24-7: Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access

type2-ChannelAccess-FR2-2-r17           ENUMERATED {supported}            OPTIONAL,

-- R1 24-10: Reduced beam switching time delay

reduced-BeamSwitchTiming-FR2-2-r17      ENUMERATED {supported}            OPTIONAL,

-- R1 24-8: 32 DL HARQ processes for FR 2-2

support32-DL-HARQ-ProcessPerSCS-r17     SEQUENCE {

scs-120kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-480kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-960kHz-r17                          ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

-- R1 24-9: 32 UL HARQ processes for FR 2-2

support32-UL-HARQ-ProcessPerSCS-r17		SEQUENCE {

scs-120kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-480kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-960kHz-r17                          ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

...,

[[

-- R4 15-1: 64QAM for PUSCH for FR2-2

modulation64-QAM-PUSCH-FR2-2-r17            ENUMERATED {supported}        OPTIONAL

]]

}

-- TAG-FR2-2-ACCESSPARAMSPERBAND-STOP

-- ASN1STOP

#### –	HighSpeedParameters

The IE HighSpeedParameters is used to convey capabilities related to high speed scenarios.

HighSpeedParameters information element

-- ASN1START

-- TAG-HIGHSPEEDPARAMETERS-START

HighSpeedParameters-r16 ::= SEQUENCE {

measurementEnhancement-r16       ENUMERATED {supported}   OPTIONAL,

demodulationEnhancement-r16      ENUMERATED {supported}   OPTIONAL

}

HighSpeedParameters-v1650 ::= CHOICE {

intraNR-MeasurementEnhancement-r16       ENUMERATED {supported},

interRAT-MeasurementEnhancement-r16      ENUMERATED {supported}

}

HighSpeedParameters-v1700 ::= SEQUENCE {

-- R4 18-1: Enhanced RRM requirements specified for CA for FR1 HST

measurementEnhancementCA-r17            ENUMERATED {supported}   OPTIONAL,

-- R4 18-2: Enhanced RRM requirements specified for inter-frequency measurement in connected mode for FR1 HST

measurementEnhancementInterFreq-r17     ENUMERATED {supported}   OPTIONAL

}

-- TAG-HIGHSPEEDPARAMETERS-STOP

-- ASN1STOP

#### –	IMS-Parameters

The IE IMS-Parameters is used to convey capabilities related to IMS.

IMS-Parameters information element

-- ASN1START

-- TAG-IMS-PARAMETERS-START

IMS-Parameters ::=         SEQUENCE {

ims-ParametersCommon       IMS-ParametersCommon                  OPTIONAL,

ims-ParametersFRX-Diff     IMS-ParametersFRX-Diff                OPTIONAL,

...

}

IMS-Parameters-v1700 ::=   SEQUENCE {

ims-ParametersFR2-2-r17    IMS-ParametersFR2-2-r17               OPTIONAL

}

IMS-ParametersCommon ::=   SEQUENCE {

voiceOverEUTRA-5GC                  ENUMERATED {supported}                OPTIONAL,

...,

[[

voiceOverSCG-BearerEUTRA-5GC        ENUMERATED {supported}                OPTIONAL

]],

[[

voiceFallbackIndicationEPS-r16       ENUMERATED {supported}                   OPTIONAL

]]

}

IMS-ParametersFRX-Diff ::= SEQUENCE {

voiceOverNR                ENUMERATED {supported}                OPTIONAL,

...

}

IMS-ParametersFR2-2-r17 ::= SEQUENCE {

voiceOverNR-r17             ENUMERATED {supported}               OPTIONAL,

...

}

-- TAG-IMS-PARAMETERS-STOP

-- ASN1STOP

#### –	InterRAT-Parameters

The IE InterRAT-Parameters is used convey UE capabilities related to the other RATs.

InterRAT-Parameters information element

-- ASN1START

-- TAG-INTERRAT-PARAMETERS-START

InterRAT-Parameters ::=             SEQUENCE {

eutra                               EUTRA-Parameters                OPTIONAL,

...,

[[

utra-FDD-r16                        UTRA-FDD-Parameters-r16         OPTIONAL

]]

}

EUTRA-Parameters ::=                SEQUENCE {

supportedBandListEUTRA          SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA,

eutra-ParametersCommon              EUTRA-ParametersCommon                                      OPTIONAL,

eutra-ParametersXDD-Diff            EUTRA-ParametersXDD-Diff                                    OPTIONAL,

...

}

EUTRA-ParametersCommon ::=      SEQUENCE {

mfbi-EUTRA                          ENUMERATED {supported}          OPTIONAL,

modifiedMPR-BehaviorEUTRA           BIT STRING (SIZE (32))          OPTIONAL,

multiNS-Pmax-EUTRA                  ENUMERATED {supported}          OPTIONAL,

rs-SINR-MeasEUTRA                   ENUMERATED {supported}          OPTIONAL,

...,

[[

ne-DC                               ENUMERATED {supported}          OPTIONAL

]],

[[

nr-HO-ToEN-DC-r16                   ENUMERATED {supported}          OPTIONAL

]]

}

EUTRA-ParametersXDD-Diff ::=        SEQUENCE {

rsrqMeasWidebandEUTRA               ENUMERATED {supported}          OPTIONAL,

...

}

UTRA-FDD-Parameters-r16 ::=                SEQUENCE {

supportedBandListUTRA-FDD-r16              SEQUENCE (SIZE (1..maxBandsUTRA-FDD-r16)) OF SupportedBandUTRA-FDD-r16,

...

}

SupportedBandUTRA-FDD-r16 ::=           ENUMERATED {

bandI, bandII, bandIII, bandIV, bandV, bandVI,

bandVII, bandVIII, bandIX, bandX, bandXI,

bandXII, bandXIII, bandXIV, bandXV, bandXVI,

bandXVII, bandXVIII, bandXIX, bandXX,

bandXXI, bandXXII, bandXXIII, bandXXIV,

bandXXV, bandXXVI, bandXXVII, bandXXVIII,

bandXXIX, bandXXX, bandXXXI, bandXXXII}

-- TAG-INTERRAT-PARAMETERS-STOP

-- ASN1STOP

#### –	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-Parameters-v1610 ::= SEQUENCE {

mac-ParametersFRX-Diff-r16      MAC-ParametersFRX-Diff-r16  OPTIONAL

}

MAC-Parameters-v1700 ::= SEQUENCE {

mac-ParametersFR2-2-r17         MAC-ParametersFR2-2-r17     OPTIONAL

}

MAC-Parameters-v17b0 ::= SEQUENCE {

mTRP-PUSCH-PHR-Type1-Reporting-r17      ENUMERATED {supported}      OPTIONAL

}

MAC-Parameters-v17c0 ::= SEQUENCE {

directSCellActivationWithTCI-r17        ENUMERATED {supported}      OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                         ENUMERATED {supported}      OPTIONAL,

dummy                                   ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction                  ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate                      ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery                 ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16         ENUMERATED {supported}     OPTIONAL,

preEmptiveBSR-r16                        ENUMERATED {supported}     OPTIONAL,

autonomousTransmission-r16               ENUMERATED {supported}     OPTIONAL,

lch-PriorityBasedPrioritization-r16      ENUMERATED {supported}     OPTIONAL,

lch-ToConfiguredGrantMapping-r16         ENUMERATED {supported}     OPTIONAL,

lch-ToGrantPriorityRestriction-r16       ENUMERATED {supported}     OPTIONAL,

singlePHR-P-r16                          ENUMERATED {supported}     OPTIONAL,

ul-LBT-FailureDetectionRecovery-r16      ENUMERATED {supported}     OPTIONAL,

-- R4 8-1: MPE

tdd-MPE-P-MPR-Reporting-r16              ENUMERATED {supported}     OPTIONAL,

lcid-ExtensionIAB-r16                    ENUMERATED {supported}     OPTIONAL

]],

[[

spCell-BFR-CBRA-r16                      ENUMERATED {supported}     OPTIONAL

]],

[[

srs-ResourceId-Ext-r16                   ENUMERATED {supported}     OPTIONAL

]],

[[

enhancedUuDRX-forSidelink-r17            ENUMERATED {supported}     OPTIONAL,

--27-10: Support of UL MAC CE based MG activation request for PRS measurements

mg-ActivationRequestPRS-Meas-r17         ENUMERATED {supported}     OPTIONAL,

--27-11: Support of DL MAC CE based MG activation request for PRS measurements

mg-ActivationCommPRS-Meas-r17            ENUMERATED {supported}     OPTIONAL,

intraCG-Prioritization-r17               ENUMERATED {supported}     OPTIONAL,

jointPrioritizationCG-Retx-Timer-r17     ENUMERATED {supported}     OPTIONAL,

survivalTime-r17                         ENUMERATED {supported}     OPTIONAL,

lcg-ExtensionIAB-r17                     ENUMERATED {supported}     OPTIONAL,

harq-FeedbackDisabled-r17                ENUMERATED {supported}     OPTIONAL,

uplink-Harq-ModeB-r17                    ENUMERATED {supported}     OPTIONAL,

sr-TriggeredBy-TA-Report-r17             ENUMERATED {supported}     OPTIONAL,

extendedDRX-CycleInactive-r17            ENUMERATED {supported}     OPTIONAL,

simultaneousSR-PUSCH-DiffPUCCH-groups-r17 ENUMERATED {supported}    OPTIONAL,

lastTransmissionUL-r17                   ENUMERATED {supported}     OPTIONAL

]],

[[

harq-RTT-TimerDL-ForNTN-MulticastMBS-r17 ENUMERATED {supported}     OPTIONAL

]],

[[

sr-TriggeredByTA-ReportATG-r18           ENUMERATED {supported}     OPTIONAL,

extendedDRX-CycleInactive-r18            ENUMERATED {supported}     OPTIONAL,

additionalBS-Table-r18                   ENUMERATED {supported}     OPTIONAL,

delayStatusReport-r18                    ENUMERATED {supported}     OPTIONAL,

cg-RetransmissionMonitoringDisabling-r18 ENUMERATED {supported}     OPTIONAL,

non-IntegerDRX-r18                       ENUMERATED {supported}     OPTIONAL

]],

[[

multipleEntryDelayStatusReport-r19       ENUMERATED {supported}     OPTIONAL,

lcp-PriorityAdjustment-r19               ENUMERATED {supported}     OPTIONAL,

ul-RateControl-r19                       ENUMERATED {supported}     OPTIONAL,

ul-RateQuery-r19                         ENUMERATED {supported}     OPTIONAL,

delayStatusReportNonDelayReportingData-r19         ENUMERATED {supported}     OPTIONAL

]]

}

MAC-ParametersFRX-Diff-r16 ::=  SEQUENCE {

directMCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

-- R1 19-1: DRX Adaptation

drx-Adaptation-r16          SEQUENCE {

non-SharedSpectrumChAccess-r16      MinTimeGap-r16              OPTIONAL,

sharedSpectrumChAccess-r16          MinTimeGap-r16              OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersFR2-2-r17 ::=  SEQUENCE {

directMCG-SCellActivation-r17           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r17     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r17           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r17     ENUMERATED {supported}      OPTIONAL,

drx-Adaptation-r17       SEQUENCE {

non-SharedSpectrumChAccess-r17      MinTimeGapFR2-2-r17         OPTIONAL,

sharedSpectrumChAccess-r17          MinTimeGapFR2-2-r17         OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic                     ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer             ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                           ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                          ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations               ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants                ENUMERATED {supported}     OPTIONAL,

...,

[[

secondaryDRX-Group-r16                  ENUMERATED {supported}     OPTIONAL

]],

[[

enhancedSkipUplinkTxDynamic-r16         ENUMERATED {supported}     OPTIONAL,

enhancedSkipUplinkTxConfigured-r16      ENUMERATED {supported}     OPTIONAL

]],

[[

dummy1                                  ENUMERATED {supported}     OPTIONAL,

dummy2                                  ENUMERATED {supported}     OPTIONAL

]]

}

MinTimeGap-r16 ::=    SEQUENCE {

scs-15kHz-r16                         ENUMERATED {sl1, sl3}        OPTIONAL,

scs-30kHz-r16                         ENUMERATED {sl1, sl6}        OPTIONAL,

scs-60kHz-r16                         ENUMERATED {sl1, sl12}       OPTIONAL,

scs-120kHz-r16                        ENUMERATED {sl2, sl24}       OPTIONAL

}

MinTimeGapFR2-2-r17 ::= SEQUENCE {

scs-120kHz-r17                        ENUMERATED {sl2, sl24}       OPTIONAL,

scs-480kHz-r17                        ENUMERATED {sl8, sl96}       OPTIONAL,

scs-960kHz-r17                        ENUMERATED {sl16, sl192}     OPTIONAL

}

MAC-ParametersPerBand-r18 ::=  SEQUENCE {

ptm-Retransmission-r18                ENUMERATED {supported}       OPTIONAL,

ptm-RetransmissionInactive-r18        ENUMERATED {supported}       OPTIONAL,

...

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v15t0 ::=          SEQUENCE {

measAndMobParametersCommon-v15t0        MeasAndMobParametersCommon-v15t0        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE {

concurrentPerUE-OnlyMeasGap-r17         ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17    ENUMERATED {supported}

}                                                                               OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

nr-NeedForGapNCSG-Reporting-r17         ENUMERATED {supported}                  OPTIONAL,

eutra-NeedForGapNCSG-Reporting-r17      ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-1 per FR Network controlled small gap (NCSG)

ncsg-MeasGapPerFR-r17                   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-2 Network controlled small gap (NCSG) supported patterns

ncsg-MeasGapPatterns-r17                BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-1-3 Network controlled small gap (NCSG) supported NR-only patterns

ncsg-MeasGapNR-Patterns-r17             BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL,

rrm-RelaxationRRC-ConnectedRedCap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 25-3: Parallel measurements with multiple measurement gaps

parallelMeasurementGap-r17              ENUMERATED {n2}                         OPTIONAL,

condHandoverWithSCG-NRDC-r17            ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-r17              ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-ENDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NEDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NRDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NPN-r17          ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R4 25-1: Parallel measurements on multiple SMTC-s for a single frequency carrier

parallelSMTC-r17                        ENUMERATED {n4}                         OPTIONAL,

-- R4 19-2-1 Concurrent measurement gaps for EUTRA

concurrentMeasGapEUTRA-r17              ENUMERATED {supported}                  OPTIONAL,

serviceLinkPropDelayDiffReporting-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-4 Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter

ncsg-SymbolLevelScheduleRestrictionInter-r17  ENUMERATED {supported}            OPTIONAL

]],

[[

eventD1-MeasReportTrigger-r17           ENUMERATED {supported}                  OPTIONAL,

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               OPTIONAL

]],

[[

interSatMeas-r17                            ENUMERATED {supported}              OPTIONAL,

deriveSSB-IndexFromCellInterNon-NCSG-r17    ENUMERATED {supported}              OPTIONAL

]],

[[

-- R4 31-1 Enhanced L3 measurement reporting for unknown SCell activation if the valid L3 measurement results are available

l3-MeasUnknownSCellActivation-r18           ENUMERATED {supported}              OPTIONAL,

-- R4 31-3 Shorter measurement interval for unknown SCell activation

shortMeasInterval-r18                       ENUMERATED {supported}              OPTIONAL,

nr-NeedForInterruptionReport-r18            ENUMERATED {supported}              OPTIONAL,

measSequenceConfig-r18                      ENUMERATED {supported}              OPTIONAL,

cellIndividualOffsetPerMeasEvent-r18        ENUMERATED {supported}              OPTIONAL,

eventD2-MeasReportTrigger-r18               ENUMERATED {supported}              OPTIONAL,

-- R4 32-1: Concurrent gaps with Pre-MG in a FR

concurrentMeasGapsPreMG-r18                 ENUMERATED {supported}              OPTIONAL,

-- R4 32-2: Support for dynamic collisions

dynamicCollision-r18                        ENUMERATED {supported}              OPTIONAL,

-- R4 32-3: Concurrent gaps with NCSG in a FR

concurrentMeasGapsNCSG-r18                  ENUMERATED {supported}              OPTIONAL,

-- R4 32-4: Inter-RAT EUTRAN measurements without gap and outside active DL BWP

eutra-NoGapMeasurementOutsideBWP-r18        ENUMERATED {supported}              OPTIONAL,

-- R4 32-5: Inter-RAT EUTRAN measurement without gap and within active DL BWP

eutra-NoGapMeasurementInsideBWP-r18         ENUMERATED {supported}              OPTIONAL,

-- R4 32-6: Effective measurement window for inter-RAT EUTRAN measurements

eutra-MeasEMW-r18                           BIT STRING (SIZE(6))                OPTIONAL,

-- R4 32-7: Simultaneous reception of NR data and EUTRAN CRS with different numerology

concurrentMeasCRS-InsideBWP-EUTRA-r18       ENUMERATED {supported}              OPTIONAL,

-- R4 39-2a: SSB based inter-frequency L1-RSRP measurements with measurement gaps

ltm-InterFreqMeasGap-r18                    ENUMERATED {supported}              OPTIONAL,

dummy-ltm-FastUE-Processing-r18             SEQUENCE {

fr1-r18                                    ENUMERATED {ms10, ms15},

fr2-r18                                    ENUMERATED {ms10, ms15},

fr1-AndFR2-r18                             ENUMERATED {ms20, ms30}

}                                                                                OPTIONAL,

rach-LessHandoverInterFreq-r18              ENUMERATED {supported}               OPTIONAL,

enterAndLeaveCellReport-r18                 ENUMERATED {supported}               OPTIONAL,

bestCellChangeReport-r18                    ENUMERATED {supported}               OPTIONAL,

secondBestCellChangeReport-r18              ENUMERATED {supported}               OPTIONAL

]],

[[

ltm-InterFreq-r18                           ENUMERATED {supported}               OPTIONAL,

ltm-MCG-NRDC-r18                            ENUMERATED {supported}               OPTIONAL,

ltm-RACH-LessDG-r18                         ENUMERATED {supported}               OPTIONAL,

ltm-RACH-LessCG-r18                         ENUMERATED {supported}               OPTIONAL,

ltm-Recovery-r18                            ENUMERATED {supported}               OPTIONAL,

ltm-ReferenceConfig-r18                     ENUMERATED {supported}               OPTIONAL,

ltm-MCG-NRDC-Release-r18                    ENUMERATED {supported}               OPTIONAL,

-- R4 39-7: Faster UE processing time during cell switch

ltm-FastUE-Processing-r18                   SEQUENCE {

fr1-r18                                    ENUMERATED {ms10, ms15}          OPTIONAL,

fr2-r18                                    ENUMERATED {ms10, ms15}          OPTIONAL,

fr1-AndFR2-r18                             ENUMERATED {ms20, ms30}          OPTIONAL

}                                                                                OPTIONAL,

ntn-NeighbourCellInfoSupport-r18            ENUMERATED {supported}               OPTIONAL

]],

[[

ltm-interFreqL1-OnlyInBC-r18                ENUMERATED {supported}               OPTIONAL

]],

[[

nr-CGI-Reporting-HSDN-r19                   ENUMERATED {supported}               OPTIONAL,

eutra-CGI-Reporting-HSDN-r19                ENUMERATED {supported}               OPTIONAL,

ltm-KeyUpdateMCG-r19                        ENUMERATED {supported}               OPTIONAL,

ltm-KeyUpdateSCG-r19                        ENUMERATED {supported}               OPTIONAL,

cltm-EarlyTA-Indication-r19                 INTEGER (1..8)                       OPTIONAL,

cltm-ExecutionConditionL1-r19               ENUMERATED {supported}               OPTIONAL,

cltm-ExecutionConditionL3-r19               INTEGER (1..2)                       OPTIONAL,

ltm-EventMeasAndReport-r19                  ENUMERATED {supported}               OPTIONAL,

ltm-RecoveryWithKeyUpdate-r19               ENUMERATED {supported}               OPTIONAL,

-- R4 49-1: Simultaneous L3 measurement on three  carriers for measurements without measurement gap under CA/DC operation

threeCarrierMeasWithoutGap-r19              SEQUENCE {

fr1-CA-NR-DC-r19                            ENUMERATED {supported}           OPTIONAL,

fr1-FR2-CA-r19                              ENUMERATED {supported}           OPTIONAL,

fr1-FR2-NR-DC-r19                           ENUMERATED {supported}           OPTIONAL

}                                                                                OPTIONAL,

-- R4 49-3: L3 serving cell and neighbor cells measurement and report on one serving carrier per-band for

-- intra-frequency measurements without measurement gap

multiCarrierSingleReportWithoutGap-r19      ENUMERATED {supported}               OPTIONAL,

-- R4 52-3: Skip SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement

skipSSB-L1-RSRP-Meas-r19                    ENUMERATED {neighbour, both}         OPTIONAL,

gapOccasionCancelRatioReport-r19            ENUMERATED {supported}               OPTIONAL,

twoSMTC-Periodicities-r19                   ENUMERATED {supported}               OPTIONAL,

reportClosestReferenceLocations-r19         ENUMERATED {supported}               OPTIONAL

]]

}

MeasAndMobParametersCommon-v15t0 ::=    SEQUENCE {

intraF-NeighMeasForSCellWithoutSSB      ENUMERATED{supported}                   OPTIONAL

}

MeasAndMobParametersXDD-Diff ::=        SEQUENCE {

intraAndInterF-MeasAndReport            ENUMERATED {supported}                  OPTIONAL,

eventA-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

...,

[[

handoverInterF                          ENUMERATED {supported}                  OPTIONAL,

handoverLTE-EPC                         ENUMERATED {supported}                  OPTIONAL,

handoverLTE-5GC                         ENUMERATED {supported}                  OPTIONAL

]],

[[

sftd-MeasNR-Neigh                       ENUMERATED {supported}                  OPTIONAL,

sftd-MeasNR-Neigh-DRX                   ENUMERATED {supported}                  OPTIONAL

]],

[[

dummy                                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                  ENUMERATED {supported}              OPTIONAL,

dummy                                       ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                       ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}          OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL,

-- R4 6-2: Support of beam level Early Measurement Reporting

idleInactiveNR-MeasBeamReport-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

increasedNumberofCSIRSPerMO-r16             ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersFR2-2-r17 ::=           SEQUENCE {

handoverInterF-r17                          ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC-r17                         ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC-r17                         ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r17               ENUMERATED {supported}              OPTIONAL,

...

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560      OPTIONAL

}

MeasAndMobParametersMRDC-v1610 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1610      MeasAndMobParametersMRDC-Common-v1610        OPTIONAL,

interNR-MeasEUTRA-IAB-r16                  ENUMERATED {supported}                       OPTIONAL

}

MeasAndMobParametersMRDC-v1700 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1700      MeasAndMobParametersMRDC-Common-v1700        OPTIONAL

}

MeasAndMobParametersMRDC-v1730 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1730   MeasAndMobParametersMRDC-Common-v1730           OPTIONAL

}

MeasAndMobParametersMRDC-v1810 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1810   MeasAndMobParametersMRDC-Common-v1810           OPTIONAL

}

MeasAndMobParametersMRDC-v1900 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1900   MeasAndMobParametersMRDC-Common-v1900           OPTIONAL

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1610 ::=   SEQUENCE {

condPSCellChangeParametersCommon-r16        SEQUENCE {

condPSCellChangeFDD-TDD-r16                 ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeFR1-FR2-r16                 ENUMERATED {supported}                  OPTIONAL

}                                                                                       OPTIONAL,

pscellT312-r16                              ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1700 ::=   SEQUENCE {

condPSCellChangeParameters-r17              SEQUENCE {

inter-SN-condPSCellChangeFDD-TDD-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFDD-TDD-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL

}                                                                                       OPTIONAL,

condHandoverWithSCG-ENDC-r17                        ENUMERATED {supported}              OPTIONAL,

condHandoverWithSCG-NEDC-r17                        ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1730 ::= SEQUENCE {

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                             OPTIONAL

}

}

MeasAndMobParametersMRDC-Common-v1810 ::=           SEQUENCE {

mn-ConfiguredMN-TriggerSCPAC-r18                    ENUMERATED {supported}              OPTIONAL,

mn-ConfiguredSN-TriggerSCPAC-r18                    ENUMERATED {supported}              OPTIONAL,

sn-ConfiguredSCPAC-r18                              ENUMERATED {supported}              OPTIONAL,

mn-ConfiguredMN-TriggerSCPAC-afterSCG-release-r18   ENUMERATED {supported}              OPTIONAL,

mn-ConfiguredReferenceConfigSCPAC-r18               ENUMERATED {supported}              OPTIONAL,

sn-ConfiguredReferenceConfigSCPAC-r18               ENUMERATED {supported}              OPTIONAL,

condHandoverWithCandSCG-Addition-r18                ENUMERATED {supported}              OPTIONAL,

condHandoverWithCandSCG-FR1-FR2-Change-r18          ENUMERATED {supported}              OPTIONAL,

condHandoverWithCandSCG-FDD-TDD-Change-r18          ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1900 ::=       SEQUENCE {

-- R4 49-1: Simultaneous L3 measurement on three  carriers for measurements without measurement gap under CA/DC operation

threeCarrierMeasWithoutGap-r19                  ENUMERATED {supported}                  OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                          OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}                   OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}                   OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

#### –	MIMO-Layers

The IE MIMO-Layers is used to convey the number of supported MIMO layers.

MIMO-Layers information element

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=   ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=   ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

#### –	MIMO-ParametersPerBand

The IE MIMO-ParametersPerBand is used to convey MIMO related parameters specific for a certain band (not per feature set or band combination).

MIMO-ParametersPerBand information element

-- ASN1START

-- TAG-MIMO-PARAMETERSPERBAND-START

MIMO-ParametersPerBand ::=          SEQUENCE {

tci-StatePDSCH                      SEQUENCE {

maxNumberConfiguredTCI-StatesPerCC  ENUMERATED {n4, n8, n16, n32, n64, n128}                                   OPTIONAL,

maxNumberActiveTCI-PerBWP           ENUMERATED {n1, n2, n4, n8}                                                OPTIONAL

}                                                                                                              OPTIONAL,

additionalActiveTCI-StatePDCCH              ENUMERATED {supported}                                             OPTIONAL,

pusch-TransCoherence                        ENUMERATED {nonCoherent, partialCoherent, fullCoherent}            OPTIONAL,

beamCorrespondenceWithoutUL-BeamSweeping    ENUMERATED {supported}                                             OPTIONAL,

periodicBeamReport                          ENUMERATED {supported}                                             OPTIONAL,

aperiodicBeamReport                         ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUCCH                          ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUSCH                          ENUMERATED {supported}                                             OPTIONAL,

dummy1                                      DummyG                                                             OPTIONAL,

maxNumberRxBeam                             INTEGER (2..8)                                                     OPTIONAL,

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

scs-15kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-30kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-60kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-120kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL

}                                                                                                              OPTIONAL,

maxNumberNonGroupBeamReporting              ENUMERATED {n1, n2, n4}                                            OPTIONAL,

groupBeamReporting                          ENUMERATED {supported}                                             OPTIONAL,

uplinkBeamManagement                        SEQUENCE {

maxNumberSRS-ResourcePerSet-BM              ENUMERATED {n2, n4, n8, n16},

maxNumberSRS-ResourceSet                    INTEGER (1..8)

}                                                                                                              OPTIONAL,

maxNumberCSI-RS-BFD                 INTEGER (1..64)                                                            OPTIONAL,

maxNumberSSB-BFD                    INTEGER (1..64)                                                            OPTIONAL,

maxNumberCSI-RS-SSB-CBD             INTEGER (1..256)                                                           OPTIONAL,

dummy2                              ENUMERATED {supported}                                                     OPTIONAL,

twoPortsPTRS-UL                     ENUMERATED {supported}                                                     OPTIONAL,

dummy5                              SRS-Resources                                                              OPTIONAL,

dummy3                              INTEGER (1..4)                                                             OPTIONAL,

beamReportTiming                    SEQUENCE {

scs-15kHz                           ENUMERATED {sym2, sym4, sym8}                                              OPTIONAL,

scs-30kHz                           ENUMERATED {sym4, sym8, sym14, sym28}                                      OPTIONAL,

scs-60kHz                           ENUMERATED {sym8, sym14, sym28}                                            OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym56}                                           OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetDL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationDL                                               OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetUL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationUL                                               OPTIONAL

}                                                                                                              OPTIONAL,

dummy4                              DummyH                                                                     OPTIONAL,

aperiodicTRS                        ENUMERATED {supported}                                                     OPTIONAL,

...,

[[

dummy6                              ENUMERATED {true}                                                          OPTIONAL,

beamManagementSSB-CSI-RS            BeamManagementSSB-CSI-RS                                                   OPTIONAL,

beamSwitchTiming                    SEQUENCE {

scs-60kHz                           ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL

}                                                                                                              OPTIONAL,

codebookParameters                  CodebookParameters                                                         OPTIONAL,

csi-RS-IM-ReceptionForFeedback      CSI-RS-IM-ReceptionForFeedback                                             OPTIONAL,

csi-RS-ProcFrameworkForSRS          CSI-RS-ProcFrameworkForSRS                                                 OPTIONAL,

csi-ReportFramework                 CSI-ReportFramework                                                        OPTIONAL,

csi-RS-ForTracking                  CSI-RS-ForTracking                                                         OPTIONAL,

srs-AssocCSI-RS                     SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource  OPTIONAL,

spatialRelations                    SpatialRelations                                                           OPTIONAL

]],

[[

-- R1 16-2b-0: Support of default QCL assumption with two TCI states

defaultQCL-TwoTCI-r16               ENUMERATED {supported}                                                     OPTIONAL,

codebookParametersPerBand-r16       CodebookParameters-v1610                                                   OPTIONAL,

-- R1 16-1b-3: Support of PUCCH resource groups per BWP for simultaneous spatial relation update

simul-SpatialRelationUpdatePUCCHResGroup-r16    ENUMERATED {supported}                                         OPTIONAL,

-- R1 16-1f: Maximum number of SCells configured for SCell beam failure recovery simultaneously

maxNumberSCellBFR-r16                           ENUMERATED {n1,n2,n4,n8}                                       OPTIONAL,

-- R1 16-2c: Supports simultaneous reception with different Type-D for FR2 only

simultaneousReceptionDiffTypeD-r16              ENUMERATED {supported}                                         OPTIONAL,

-- R1 16-1a-1: SSB/CSI-RS for L1-SINR measurement

ssb-csirs-SINR-measurement-r16      SEQUENCE {

maxNumberSSB-CSIRS-OneTx-CMR-r16    ENUMERATED {n8, n16, n32, n64},

maxNumberCSI-IM-NZP-IMR-res-r16     ENUMERATED {n8, n16, n32, n64},

maxNumberCSIRS-2Tx-res-r16          ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberSSB-CSIRS-res-r16          ENUMERATED {n8, n16, n32, n64, n128},

maxNumberCSI-IM-NZP-IMR-res-mem-r16 ENUMERATED {n8, n16, n32, n64, n128},

supportedCSI-RS-Density-CMR-r16     ENUMERATED {one, three, oneAndThree},

maxNumberAperiodicCSI-RS-Res-r16    ENUMERATED {n2, n4, n8, n16, n32, n64},

supportedSINR-meas-r16              ENUMERATED {ssbWithCSI-IM, ssbWithNZP-IMR, csirsWithNZP-IMR, csi-RSWithoutIMR}  OPTIONAL

}                                                                                                              OPTIONAL,

-- R1 16-1a-2: Non-group based L1-SINR reporting

nonGroupSINR-reporting-r16              ENUMERATED {n1, n2, n4}                                                OPTIONAL,

-- R1 16-1a-3: Non-group based L1-SINR reporting

groupSINR-reporting-r16                 ENUMERATED {supported}                                                 OPTIONAL,

multiDCI-multiTRP-Parameters-r16        SEQUENCE {

-- R1 16-2a-0: Overlapping PDSCHs in time and fully overlapping in frequency and time

overlapPDSCHsFullyFreqTime-r16          INTEGER (1..2)                                                     OPTIONAL,

-- R1 16-2a-1: Overlapping PDSCHs in time and partially overlapping in frequency and time

overlapPDSCHsInTimePartiallyFreq-r16    ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-2: Out of order operation for DL

outOfOrderOperationDL-r16               SEQUENCE {

supportPDCCH-ToPDSCH-r16                ENUMERATED {supported}                                         OPTIONAL,

supportPDSCH-ToHARQ-ACK-r16             ENUMERATED {supported}                                         OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 16-2a-3: Out of order operation for UL

outOfOrderOperationUL-r16               ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-5: Separate CRS rate matching

separateCRS-RateMatching-r16            ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-6: Default QCL enhancement for multi-DCI based multi-TRP

defaultQCL-PerCORESETPoolIndex-r16      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-7: Maximum number of activated TCI states

maxNumberActivatedTCI-States-r16        SEQUENCE {

maxNumberPerCORESET-Pool-r16            ENUMERATED {n1, n2, n4, n8},

maxTotalNumberAcrossCORESET-Pool-r16    ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL

}                                                                                                              OPTIONAL,

singleDCI-SDM-scheme-Parameters-r16         SEQUENCE {

-- R1 16-2b-1b: Single-DCI based SDM scheme - Support of new DMRS port entry

supportNewDMRS-Port-r16                     ENUMERATED {supported1, supported2, supported3}                                        OPTIONAL,

-- R1 16-2b-1a: Support of s-port DL PTRS

supportTwoPortDL-PTRS-r16                   ENUMERATED {supported}                                         OPTIONAL

}                                                                                                              OPTIONAL,

-- R1 16-2b-2: Support of single-DCI based FDMSchemeA

supportFDM-SchemeA-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2b-3a: Single-DCI based FDMSchemeB CW soft combining

supportCodeWordSoftCombining-r16            ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2b-4: Single-DCI based TDMSchemeA

supportTDM-SchemeA-r16                      ENUMERATED {kb3, kb5, kb10, kb20, noRestriction}                   OPTIONAL,

-- R1 16-2b-5: Single-DCI based inter-slot TDM

supportInter-slotTDM-r16                    SEQUENCE {

supportRepNumPDSCH-TDRA-r16                 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16},

maxTBS-Size-r16                             ENUMERATED {kb3, kb5, kb10, kb20, noRestriction},

maxNumberTCI-states-r16                     INTEGER (1..2)

}                                                                                                              OPTIONAL,

-- R1 16-4: Low PAPR DMRS for PDSCH

lowPAPR-DMRS-PDSCH-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6a: Low PAPR DMRS for PUSCH without transform precoding

lowPAPR-DMRS-PUSCHwithoutPrecoding-r16      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6b: Low PAPR DMRS for PUCCH

lowPAPR-DMRS-PUCCH-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6c: Low PAPR DMRS for PUSCH with transform precoding & pi/2 BPSK

lowPAPR-DMRS-PUSCHwithPrecoding-r16         ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings

csi-ReportFrameworkExt-r16                  CSI-ReportFrameworkExt-r16                                         OPTIONAL,

-- R1 16-3a, 16-3a-1, 16-3b, 16-3b-1, 16-8: Individual new codebook types

codebookParametersAddition-r16              CodebookParametersAddition-r16                                     OPTIONAL,

-- R1 16-8: Mixed codebook types

codebookComboParametersAddition-r16         CodebookComboParametersAddition-r16                                OPTIONAL,

-- R4 8-2: SSB based beam correspondence

beamCorrespondenceSSB-based-r16             ENUMERATED {supported}                                             OPTIONAL,

-- R4 8-3: CSI-RS based beam correspondence

beamCorrespondenceCSI-RS-based-r16          ENUMERATED {supported}                                             OPTIONAL,

beamSwitchTiming-r16                        SEQUENCE {

scs-60kHz-r16                               ENUMERATED {sym224, sym336}                                    OPTIONAL,

scs-120kHz-r16                              ENUMERATED {sym224, sym336}                                    OPTIONAL

}                                                                                                              OPTIONAL

]],

[[

-- R1 16-1a-4: Semi-persistent L1-SINR report on PUCCH

semi-PersistentL1-SINR-Report-PUCCH-r16     SEQUENCE {

supportReportFormat1-2OFDM-syms-r16         ENUMERATED {supported}                                     OPTIONAL,

supportReportFormat4-14OFDM-syms-r16        ENUMERATED {supported}                                     OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 16-1a-5: Semi-persistent L1-SINR report on PUSCH

semi-PersistentL1-SINR-Report-PUSCH-r16     ENUMERATED {supported}                                         OPTIONAL

]],

[[

-- R1 16-1h: Support of 64 configured PUCCH spatial relations

spatialRelations-v1640                      SEQUENCE {

maxNumberConfiguredSpatialRelations-v1640   ENUMERATED {n96, n128, n160, n192, n224, n256, n288, n320}

}                                                                                                          OPTIONAL,

-- R1 16-1i: Support of 64 configured candidate beam RSs for BFR

support64CandidateBeamRS-BFR-r16            ENUMERATED {supported}                                         OPTIONAL

]],

[[

-- R1 16-2a-9: Interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP

maxMIMO-LayersForMulti-DCI-mTRP-r16         ENUMERATED {supported}                                         OPTIONAL

]],

[[

supportedSINR-meas-v1670                    BIT STRING (SIZE (4))                                          OPTIONAL

]],

[[

-- R1 23-8-5	Increased repetition for SRS

srs-increasedRepetition-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-6	Partial frequency sounding of SRS

srs-partialFrequencySounding-r17            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-7	Start RB location hopping for partial frequency SRS

srs-startRB-locationHoppingPartial-r17      ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-8	Comb-8 SRS

srs-combEight-r17                           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-9-1	Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) per band information

codebookParametersfetype2-r17               CodebookParametersfetype2-r17                                  OPTIONAL,

-- R1 23-3-1-2a    Two associated CSI-RS resources

mTRP-PUSCH-twoCSI-RS-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2    Multi-TRP PUCCH repetition scheme 1 (inter-slot)

mTRP-PUCCH-InterSlot-r17                    ENUMERATED {pf0-2, pf1-3-4, pf0-4}                             OPTIONAL,

-- R1 23-3-2b    Cyclic mapping for multi-TRP PUCCH repetition

mTRP-PUCCH-CyclicMapping-r17                ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2c    Second TPC field for multi-TRP PUCCH repetition

mTRP-PUCCH-SecondTPC-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-5-2    MTRP BFR based on two BFD-RS set

mTRP-BFR-twoBFD-RS-Set-r17                  SEQUENCE {

maxBFD-RS-resourcesPerSetPerBWP-r17         ENUMERATED {n1, n2},

maxBFR-r17                                  INTEGER (1..9),

maxBFD-RS-resourcesAcrossSetsPerBWP-r17     ENUMERATED {n2, n3, n4}

}                                                                                                          OPTIONAL,

-- R1 23-5-2a    PUCCH-SR resources for MTRP BFRQ - Max number of PUCCH-SR resources for MTRP BFRQ per cell group

mTRP-BFR-PUCCH-SR-perCG-r17                 ENUMERATED{n1, n2}                                             OPTIONAL,

-- R1 23-5-2b    Association between a BFD-RS resource set on SpCell and a PUCCH SR resource

mTRP-BFR-association-PUCCH-SR-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-3    Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC)

sfn-SimulTwoTCI-AcrossMultiCC-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-4    Default DL beam setup for SFN

sfn-DefaultDL-BeamSetup-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-4a    Default UL beam setup for SFN PDCCH(FR2 only)

sfn-DefaultUL-BeamSetup-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-1    SRS triggering offset enhancement

srs-TriggeringOffset-r17                    ENUMERATED {n1, n2, n4}                                        OPTIONAL,

-- R1 23-8-2    Triggering SRS only in DCI 0_1/0_2

srs-TriggeringDCI-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-9-5    Active CSI-RS resources and ports for mixed codebook types in any slot per band information

codebookComboParameterMixedType-r17         CodebookComboParameterMixedType-r17                            OPTIONAL,

-- R1 23-1-1    Unified TCI [with joint DL/UL TCI update] for intra-cell beam management

unifiedJointTCI-r17                         SEQUENCE{

maxConfiguredJointTCI-r17                   ENUMERATED {n8, n12, n16, n24, n32, n48, n64, n128},

maxActivatedTCIAcrossCC-r17                 ENUMERATED {n1, n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1  23-1-1b    Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE

unifiedJointTCI-multiMAC-CE-r17             SEQUENCE{

minBeamApplicationTime-r17                  ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336}

OPTIONAL,

maxNumMAC-CE-PerCC                          ENUMERATED {n2, n3, n4, n5, n6, n7, n8}

} OPTIONAL,

-- R1 23-1-1d    Per BWP TCI state pool configuration for CA mode

unifiedJointTCI-perBWP-CA-r17               ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1e    TCI state pool configuration with TCI pool sharing for CA mode

unifiedJointTCI-ListSharingCA-r17           ENUMERATED {n1,n2,n4,n8}                                       OPTIONAL,

-- R1 23-1-1f    Common multi-CC TCI state ID update and activation

unifiedJointTCI-commonMultiCC-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1g    Beam misalignment between the DL source RS in the TCI state

unifiedJointTCI-BeamAlignDLRS-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1h    Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS

unifiedJointTCI-PC-association-r17          ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1i    Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH

unifiedJointTCI-Legacy-r17                  ENUMERATED {supported}                                         OPTIONAL,

-- 23-1-1m    Indication/configuration of R17 TCI states for SRS

unifiedJointTCI-Legacy-SRS-r17              ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1j    Indication/configuration of R17 TCI states for CORESET #0

unifiedJointTCI-Legacy-CORESET0-r17         ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1c    SCell BFR with unified TCI framework  (NOTE; pre-requisite is empty)

unifiedJointTCI-SCellBFR-r17                ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1a    Unified TCI with joint DL/UL TCI update for inter-cell beam management

unifiedJointTCI-InterCell-r17               SEQUENCE{

additionalMAC-CE-PerCC-r17                  ENUMERATED {n0, n1, n2, n4},

additionalMAC-CE-AcrossCC-r17               ENUMERATED {n0, n1, n2, n4}

}                                                                                                          OPTIONAL,

-- R1  23-10-1    Unified TCI with separate DL/UL TCI update for intra-cell beam management

unifiedSeparateTCI-r17                      SEQUENCE{

maxConfiguredDL-TCI-r17                     ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n128},

maxConfiguredUL-TCI-r17                     ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64},

maxActivatedDL-TCIAcrossCC-r17              ENUMERATED {n1, n2, n4, n8, n16},

maxActivatedUL-TCIAcrossCC-r17              ENUMERATED {n1, n2, n4, n8, n16}

} OPTIONAL,

-- R1  23-10-1b    Unified TCI with separate DL/UL TCI update for intra-cell beam management with more than one MAC-CE

unifiedSeparateTCI-multiMAC-CE-r17          SEQUENCE{

minBeamApplicationTime-r17                  ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336},

maxActivatedDL-TCIPerCC-r17                 INTEGER (2..8),

maxActivatedUL-TCIPerCC-r17                 INTEGER (2..8)

}                                                                                                          OPTIONAL,

-- R1 23-10-1d    Per BWP DL/UL-TCI state pool configuration for CA mode

unifiedSeparateTCI-perBWP-CA-r17            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-10-1e    TCI state pool configuration with DL/UL-TCI pool sharing for CA mode

unifiedSeparateTCI-ListSharingCA-r17        SEQUENCE {

maxNumListDL-TCI-r17                        ENUMERATED {n1,n2,n4,n8}                                   OPTIONAL,

maxNumListUL-TCI-r17                        ENUMERATED {n1,n2,n4,n8}                                   OPTIONAL

} OPTIONAL,

-- R1 23-10-1f    Common multi-CC DL/UL-TCI state ID update and activation with separate DL/UL TCI update

unifiedSeparateTCI-commonMultiCC-r17    ENUMERATED {supported}                                             OPTIONAL,

-- 23-10-1m    Unified TCI with separate DL/UL TCI update for inter-cell beam management with more than one MAC-CE

unifiedSeparateTCI-InterCell-r17            SEQUENCE {

k-DL-PerCC-r17                              ENUMERATED {n0, n1, n2, n4},

k-UL-PerCC-r17                              ENUMERATED {n0, n1, n2, n4},

k-DL-AcrossCC-r17                           ENUMERATED {n0, n1, n2, n4},

k-UL-AcrossCC-r17                           ENUMERATED {n0, n1, n2, n4}

}                                                                                                          OPTIONAL,

-- R1  23-1-2    Inter-cell beam measurement and reporting (for inter-cell BM and mTRP)

unifiedJointTCI-mTRP-InterCell-BM-r17       SEQUENCE {

maxNumAdditionalPCI-L1-RSRP-r17             INTEGER (1..7),

maxNumSSB-ResourceL1-RSRP-AcrossCC-r17      ENUMERATED {n1,n2,n4,n8}

}                                                                                                          OPTIONAL,

-- R1  23-1-3    MPE mitigation

mpe-Mitigation-r17                          SEQUENCE {

maxNumP-MPR-RI-pairs-r17                    INTEGER (1..4),

maxNumConfRS-r17                            ENUMERATED {n1, n2, n4, n8, n12, n16, n28, n32, n48, n64}

}                                                                                                          OPTIONAL,

-- R1  23-1-4    UE capability value reporting

srs-PortReport-r17                          SEQUENCE {

capVal1-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal2-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal3-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal4-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 23-2-1a    Monitoring of individual candidates

mTRP-PDCCH-individual-r17                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-2-1b    PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot

mTRP-PDCCH-anySpan-3Symbols-r17             ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-2-2    Two QCL TypeD for CORESET monitoring in PDCCH repetition

mTRP-PDCCH-TwoQCL-TypeD-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1-2b    CSI-RS processing framework for SRS with two associated CSI-RS resources

mTRP-PUSCH-CSI-RS-r17                       SEQUENCE {

maxNumPeriodicSRS-r17                       INTEGER (1..8),

maxNumAperiodicSRS-r17                      INTEGER (1..8),

maxNumSP-SRS-r17                            INTEGER (0..8),

numSRS-ResourcePerCC-r17                    INTEGER (1..16),

numSRS-ResourceNonCodebook-r17              INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 23-3-1a    Cyclic mapping for Multi-TRP PUSCH repetition

mTRP-PUSCH-cyclicMapping-r17                ENUMERATED {typeA,typeB,both}                                  OPTIONAL,

-- R1 23-3-1b    Second TPC field for Multi-TRP PUSCH repetition

mTRP-PUSCH-secondTPC-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1c     Two PHR reporting

mTRP-PUSCH-twoPHR-Reporting-r17             ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1e    A-CSI report

mTRP-PUSCH-A-CSI-r17                        ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1f    SP-CSI report

mTRP-PUSCH-SP-CSI-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1g    CG PUSCH transmission

mTRP-PUSCH-CG-r17                           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2d    Updating two Spatial relation or two sets of power control parameters for PUCCH group

mTRP-PUCCH-MAC-CE-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2e    Maximum number of power control parameter sets configured for multi-TRP PUCCH repetition in FR1

mTRP-PUCCH-maxNum-PC-FR1-r17                INTEGER (3..8)                                                 OPTIONAL,

-- R1 23-4    IntCell-mTRP

mTRP-inter-Cell-r17                         SEQUENCE {

maxNumAdditionalPCI-Case1-r17               INTEGER (1..7),

maxNumAdditionalPCI-Case2-r17               INTEGER (0..7)

}                                                                                                          OPTIONAL,

-- R1 23-5-1    Group based L1-RSRP reporting enhancements

mTRP-GroupBasedL1-RSRP-r17                  SEQUENCE {

maxNumBeamGroups-r17                        INTEGER (1..4),

maxNumRS-WithinSlot-r17                     ENUMERATED {n2,n3,n4,n8,n16,n32,n64},

maxNumRS-AcrossSlot-r17                     ENUMERATED {n8, n16, n32, n64, n128}

}                                                                                                          OPTIONAL,

-- R1 23-5-2c    MAC-CE based update of explicit BFD-RS    mTRP-PUCCH-IntraSlot-r17  => per band

mTRP-BFD-RS-MAC-CE-r17                      ENUMERATED {n4, n8, n12, n16, n32, n48, n64 }                  OPTIONAL,

-- R1 23-7-1    Basic Features of CSI Enhancement for Multi-TRP

mTRP-CSI-EnhancementPerBand-r17             SEQUENCE {

maxNumNZP-CSI-RS-r17                        INTEGER (2..8),

cSI-Report-mode-r17                         ENUMERATED {mode1, mode2, both},

supportedComboAcrossCCs-r17                 SEQUENCE (SIZE (1..16)) OF CSI-MultiTRP-SupportedCombinations-r17,

codebookModeNCJT-r17                        ENUMERATED{mode1,mode1And2}

}                                                                                                          OPTIONAL,

-- R1 23-7-1b    Active CSI-RS resources and ports in the presence of multi-TRP CSI

codebookComboParameterMultiTRP-r17          CodebookComboParameterMultiTRP-r17                             OPTIONAL,

-- R1 23-7-1a    Additional CSI report mode 1

mTRP-CSI-additionalCSI-r17                  ENUMERATED{x1,x2}                                              OPTIONAL,

-- R1 23-7-4    Support of Nmax=2 for Multi-TRP CSI

mTRP-CSI-N-Max2-r17                         ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-7-5    CMR sharing

mTRP-CSI-CMR-r17                            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-11    Partial frequency sounding of SRS for non-frequency hopping case

srs-partialFreqSounding-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1-24 feature: Extend beamSwitchTiming for FR2-2

beamSwitchTiming-v1710                      SEQUENCE {

scs-480kHz                                  ENUMERATED {sym56, sym112, sym192, sym896, sym1344}        OPTIONAL,

scs-960kHz                                  ENUMERATED {sym112, sym224, sym384, sym1792, sym2688}      OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature: Extend beamSwitchTiming-r16 for FR2-2

beamSwitchTiming-r17                        SEQUENCE {

scs-480kHz-r17                              ENUMERATED {sym896, sym1344}                               OPTIONAL,

scs-960kHz-r17                              ENUMERATED {sym1792, sym2688}                              OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature: Extend beamReportTiming for FR2-2

beamReportTiming-v1710                      SEQUENCE {

scs-480kHz-r17                              ENUMERATED {sym56, sym112, sym224}                         OPTIONAL,

scs-960kHz-r17                              ENUMERATED {sym112, sym224, sym448}                        OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature:    Extend maximum number of RX/TX beam switch DL for FR2-2

maxNumberRxTxBeamSwitchDL-v1710             SEQUENCE {

scs-480kHz-r17                              ENUMERATED {n2, n4, n7}                                    OPTIONAL,

scs-960kHz-r17                              ENUMERATED {n1, n2, n4, n7}                                OPTIONAL

}                                                                                                          OPTIONAL

]],

[[

-- R1-23-1-4a:	Semi-persistent/aperiodic capability value report

srs-PortReportSP-AP-r17                     ENUMERATED {supported}                                         OPTIONAL,

maxNumberRxBeam-v1720                       INTEGER (9..12)                                                OPTIONAL,

-- R1-23-6-5	Support implicit configuration of RS(s) with two TCI states for beam failure detection

sfn-ImplicitRS-twoTCI-r17                   ENUMERATED {supported}                                         OPTIONAL,

-- R1-23-6-6	QCL-TypeD collision handling with CORESET with 2 TCI states

sfn-QCL-TypeD-Collision-twoTCI-r17          ENUMERATED {supported}                                         OPTIONAL,

-- R1-23-7-1c	Basic Features of CSI Enhancement for Multi-TRP - number of CPUs

mTRP-CSI-numCPU-r17                         ENUMERATED {n2, n3, n4}                                        OPTIONAL

]],

[[

supportRepNumPDSCH-TDRA-DCI-1-2-r17         ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                   OPTIONAL

]],

[[

codebookParametersetype2DopplerCSI-r18      CodebookParametersetype2DopplerCSI-r18                         OPTIONAL,

codebookParametersfetype2DopplerCSI-r18     CodebookParametersfetype2DopplerCSI-r18                        OPTIONAL,

codebookParametersetype2CJT-r18             CodebookParametersetype2CJT-r18                                OPTIONAL,

codebookParametersfetype2CJT-r18            CodebookParametersfetype2CJT-r18                               OPTIONAL,

codebookComboParametersCJT-r18              CodebookComboParametersCJT-r18                                 OPTIONAL,

codebookParametersHARQ-ACK-PUSCH-r18        CodebookParametersHARQ-ACK-PUSCH-r18                           OPTIONAL,

-- R1 40-1-1: Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI

-- codepoint per CC

tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 SEQUENCE {

maxNumberConfigJointTCIPerCC-PerBWP-r18     ENUMERATED {n8,n12,n16,n24,n32,n48,n64,n128},

maxNumberActiveJointTCI-AcrossCC-r18        ENUMERATED {n2,n4,n6,n8,n16,n32}

}                                                                                                          OPTIONAL,

-- R1 40-1-1a: Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI

-- codepoints per CC

tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18 SEQUENCE {

tci-StateInd-r18                            ENUMERATED {withAssignment, withoutAssignment},

maxNumberActiveJointTCI-PerCC-r18           INTEGER (2..8)

}                                                                                                          OPTIONAL,

-- R1 40-1-1c: DCI format 1_1 and if supported 1_2 configured with TCI selection field

tci-SelectionDCI-r18                        ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-1-2: Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI

-- codepoint per CC

tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18 SEQUENCE {

maxNumConfigDL-TCI-PerCC-PerBWP-r18         ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n128},

maxNumConfigUL-TCI-PerCC-PerBWP-r18         ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64},

maxNumActiveDL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16},

maxNumActiveUL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-2a: Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple

-- activated TCI codepoints per CC

tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18  SEQUENCE {

maxNumActiveDL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16},

maxNumActiveUL-TCI-AcrossCC-r18             ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-3: Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP

tci-SelectionAperiodicCSI-RS-r18            ENUMERATED {perResource, perResourceSet, both}                 OPTIONAL,

-- R1 40-1-3a: Per aperiodic CSI-RS resource/resource set configuration for TCI selection in M-DCI based MTRP

tci-SelectionAperiodicCSI-RS-M-DCI-r18      ENUMERATED {perResource, perResourceSet, both}                 OPTIONAL,

-- R1 40-1-4: Two TCI states for CJT Tx scheme for PDSCH

twoTCI-StatePDSCH-CJT-TxScheme-r18          ENUMERATED {cjtSchemeA, cjtSchemeB, both}                      OPTIONAL,

-- R1 40-1-7: Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI

-- codepoint per CORESETPoolIndex per CC

tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18  SEQUENCE {

mTRP-Operation-r18                                        ENUMERATED {intraCell, intraCellAndInterCell},

maxNumberConfigJointTCIPerCC-PerBWP-r18                   ENUMERATED {n8,n12,n16,n24,n32,n48,n64,n128},

maxNumberActiveJointTCIAcrossCC-PerCORESET-r18            ENUMERATED {n1,n2,n4,n8,n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-7a: Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI

-- codepoints per CORESETPoolIndex per CC

tci-JointTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18        INTEGER (2..8)                               OPTIONAL,

-- R1 40-1-8: TRP-specific BFR with unified TCI framework with Unified TCI

tci-TRP-BFR-r18                             ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-1-9: Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI

-- codepoint per CORESETPoolIndex per CC

tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18  SEQUENCE {

mTRP-Operation-r18                          ENUMERATED {intraCell, intraCellAndInterCell},

maxNumConfigDL-TCI-PerCC-PerBWP-r18         ENUMERATED {n8, n12, n16, n24, n32, n48, n64, n128},

maxNumConfigUL-TCI-PerCC-PerBWP-r18         ENUMERATED {n8, n12, n16, n24, n32, n48, n64},

maxNumActiveDL-TCI-AcrossCC-r18             ENUMERATED {n1, n2, n4, n8, n16},

maxNumActiveUL-TCI-AcrossCC-r18             ENUMERATED {n1, n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1 40-1-9a: Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI

-- codepoints per CORESETPoolIndex per CC

tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18   SEQUENCE {

maxNumConfigDL-TCI-PerCC-PerBWP-r18         INTEGER (1..8),

maxNumConfigUL-TCI-PerCC-PerBWP-r18         INTEGER (1..8)

}                                                                                                          OPTIONAL,

-- R1 40-1-12: Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP

commonTCI-SingleDCI-r18                     INTEGER (1..4)                                                 OPTIONAL,

-- R1 40-1-13: Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP

commonTCI-MultiDCI-r18                      INTEGER (1..4)                                                 OPTIONAL,

-- R1 40-1-14: Two PHR reporting for STx2P

twoPHR-Reporting-r18                        ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-3: TAG ID indication via absolute TA command MAC CE

spCell-TAG-Ind-r18                          ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-4: PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for

-- inter-cell

interCellCrossTRP-PDCCH-OrderCFRA-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-4a: PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA for

-- intra-cell

intraCellCrossTRP-PDCCH-OrderCFRA-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-2-9: Overlapping UL transmission reduction

overlapUL-TransReduction-r18                ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-3-2-12: Supported maximum periodicity of CMR when configured as periodic CSI-RS

maxPeriodicityCMR-r18                       ENUMERATED {sl4, sl5, sl8, sl10, sl20}                         OPTIONAL,

-- R1 40-3-3-1: TDCP (Time Domain Channel Properties) report

tdcp-Report-r18                             SEQUENCE {

valueX-r18                                  INTEGER (1..2),

maxNumberActiveResource-r18                 INTEGER (2..32)

}                                                                                                          OPTIONAL,

-- R1 40-3-3-5: Number of CSI-RS resources for TDCP

tdcp-Resource-r18                           SEQUENCE {

maxNumberConfigPerCC-r18                    ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberConfigAcrossCC-r18                 INTEGER (1..32),

maxNumberSimultaneousPerCC-r18              ENUMERATED {n2, n4, n6, n8, n12, n16, n20, n24, n28, n32}

}                                                                                                          OPTIONAL,

-- R1 40-3-1-24: Timeline for regular eType-II-CJT CSI, or for port selection FeType-II-CJT CSI

timelineRelax-CJT-CSI-r18                   ENUMERATED {n0,n2}                                             OPTIONAL,

-- R1 40-4-11: Joint configuration of Rel.18 DMRS ports and Rel.18 dynamic switching between DFT-S-OFDM and CP-OFDM for PUSCH

jointConfigDMRSPortDynamicSwitching-r18     ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-1: SRS comb offset hopping

srs-combOffsetHopping-r18                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-1a: Comb offset hopping time-domain behavior when repetition factor R>1

srs-combOffsetInTime-r18                    ENUMERATED {srs, rsrs, both}                                   OPTIONAL,

-- R1 40-5-1b: SRS comb offset hopping combined with group/sequence hopping

srs-combOffsetCombinedGroupSequence-r18     ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-1c: Comb offset hopping within a subset

srs-combOffsetHoppingWithinSubset-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2: SRS cyclic shift hopping

srs-cyclicShiftHopping-r18                  ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2a: Smaller cyclic shift granularity for cyclic shift hopping

srs-cyclicShiftHoppingSmallGranularity-r18  ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2b: SRS cyclic shift hopping combined with group/sequence hopping

srs-cyclicShiftCombinedGroupSequence-r18    ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-2c: Cyclic shift hopping within a subset

cyclicShiftHoppingWithinSubset-r18          ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-5-3: SRS cyclic shift hopping combined with SRS comb offset hopping

srs-cyclicShiftCombinedCombOffset-r18       ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-1-1: 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH-codebook

pusch-CB-2PTRS-SingleDCI-STx2P-SDM-r18      ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-1a-1: 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH-noncodebook

pusch-NonCB-2PTRS-SingleDCI-STx2P-SDM-r18   ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-1b: Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH

pusch-NonCB-SingleDCI-STx2P-SDM-CSI-RS-SRS-r18 SEQUENCE {

maxNumberPeriodicSRS-Resource-PerBWP-r18      INTEGER (1..8),

maxNumberAperiodicSRS-Resource-PerBWP-r18     INTEGER (1..8),

maxNumberSemiPersistentSRS-ResourcePerBWP-r18 INTEGER (0..8),

valueY-SRS-ResourceAssociate-r18              INTEGER (1..16),

valueX-CSI-RS-ResourceAssociate-r18           INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 40-6-3b-1: Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH

twoPUSCH-NonCB-Multi-DCI-STx2P-CSI-RS-Resource-r18  SEQUENCE {

maxNumberPeriodicSRS-r18                      INTEGER (1..8),

maxNumberAperiodicSRS-r18                     INTEGER (1..8),

maxNumberSemiPersistentSRS-r18                INTEGER (0..8),

simultaneousSRS-PerCC-r18                     INTEGER (1..16),

simultaneousCSI-RS-NonCB-r18                  INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 40-6-1-2: New UL DMRS port entry for single-DCI based SDM scheme for Rel-15 DMRS port and/or Rel-18 DMRS port

dmrs-PortEntrySingleDCI-SDM-r18             ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-2-1: 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH-codebook

pusch-CB-2PTRS-SingleDCI-STx2P-SFN-r18      ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-2a-1: 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH-codebook

pusch-NonCB-2PTRS-SingleDCI-STx2P-SFN-r18   ENUMERATED {supported}                                         OPTIONAL,

-- R1 40-6-2b: Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH

pusch-NonCB-SingleDCI-STx2P-SFN-CSI-RS-SRS-r18 SEQUENCE {

maxNumberPeriodicSRS-Resource-PerBWP-r18      INTEGER (1..8),

maxNumberAperiodicSRS-Resource-PerBWP-r18     INTEGER (1..8),

maxNumberSemiPersistentSRS-ResourcePerBWP-r18 INTEGER (0..8),

valueY-SRS-ResourceAssociate-r18              INTEGER (1..16),

valueX-CSI-RS-ResourceAssociate-r18           INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 40-6-3c: Codebook multi-DCI based STx2P PUSCH+PUSCH - Fully overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18       ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3d: Codebook multi-DCI based STx2P PUSCH+PUSCH - Fully overlapping PUSCHs in time and partially overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18    ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3e: Codebook multi-DCI based STx2P PUSCH+PUSCH - Partially overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18    ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3f: Codebook multi-DCI based STx2P PUSCH+PUSCH - Partially overlapping PUSCHs in time, partially overlapping in frequency

twoPUSCH-CB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3g: Codebook multi-DCI based STx2P PUSCH+PUSCH - Partially overlapping PUSCHs in time, partially or non-overlapping

-- in frequency

twoPUSCH-CB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18     ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3h: Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG

twoPUSCH-CB-MultiDCI-STx2P-CG-CG-r18                         ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3i: Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG

twoPUSCH-CB-MultiDCI-STx2P-CG-DG-r18                         ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3j: Noncodebook multi-DCI based STx2P PUSCH+PUSCH - Fully overlapping PUSCHs in time and fully overlapping in frequency

twoPUSCH-NonCB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18    ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3k: Noncodebook multi-DCI based STx2P PUSCH+PUSCH - Fully overlapping PUSCHs in time and partially overlapping in

-- frequency

twoPUSCH-NonCB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18 ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3l: Noncodebook multi-DCI based STx2P PUSCH+PUSCH - Partially overlapping PUSCHs in time and fully overlapping in

-- frequency

twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18 ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3m: Noncodebook multi-DCI based STx2P PUSCH+PUSCH - Partially overlapping PUSCHs in time, partially overlapping in

-- frequency

twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 ENUMERATED {supported}                     OPTIONAL,

-- R1 40-6-3n: Noncodebook multi-DCI based STx2P PUSCH+PUSCH - Partially overlapping PUSCHs in time, non-overlapping in frequency

twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18  ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3o: Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG

twoPUSCH-NonCB-MultiDCI-STx2P-CG-CG-r18                      ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-3p: Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG

twoPUSCH-NonCB-MultiDCI-STx2P-CG-DG-r18                      ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-4a: Dynamic indication of repetition number for SFN scheme for PUCCH

pucch-RepetitionDynamicIndicationSFN-r18                     ENUMERATED {supported}                        OPTIONAL,

-- R1 40-6-5: Support grouped-based beam reporting for STx2P

groupBeamReporting-STx2P-r18                                 SEQUENCE {

groupL1-RSRP-Reporting-r18                                   ENUMERATED {jointULandDL, ulOnly, both},

maxNumberBeamGroups-r18                                      INTEGER (1..4),

maxNumberResWithinSlotAcrossCC-r18                           ENUMERATED {n2,n3,n4,n8,n16,n32,n64},

maxNumberResAcrossCC-r18                                     ENUMERATED {n8,n16,n32,n64,n128}

}                                                                                                          OPTIONAL

]],

[[

-- R1 40-4-1k: Simultaneous Configuration of Rel-18 DL DMRS and DCI format 1_3

simulConfigDMRS-DCI-1-3-r18                                  ENUMERATED {supported}                        OPTIONAL

]],

[[

codebookParametersType1SP-SchemeA-r19         CodebookParametersType1SP-SchemeA-r19                          OPTIONAL,

codebookParametersType1SP-SchemeB-r19         CodebookParametersType1SP-SchemeB-r19                          OPTIONAL,

codebookParametersType1MP-r19                 CodebookParametersType1MP-r19                                  OPTIONAL,

codebookParameterseType2Ext-r19               CodebookParameterseType2Ext-r19                                OPTIONAL,

codebookParametersfeType2Ext-r19              CodebookParametersfeType2Ext-r19                               OPTIONAL,

codebookParameterseType2DopplerExt-r19        CodebookParameterseType2DopplerExt-r19                         OPTIONAL,

codebookParametersHybridBF-Type1SP-r19        CodebookParametersHybridBF-Type1SP-r19                         OPTIONAL,

codebookParametersHybridBF-eType2-r19         CodebookParametersHybridBF-eType2-r19                          OPTIONAL,

-- R1 58-3-1: CSI prediction for UE-sided inference when N4=1

aiml-CSI-Prediction-r19                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-1-1: UE-initiated/event-driven beam management for Event-2 based measurement and report for Mode A

uei-ModeA-Event2-r19                          INTEGER (1..64)                                                OPTIONAL,

-- R1 59-1-2: UE-initiated/event-driven beam management Mode B

uei-ModeB-r19                                 SEQUENCE {

scs15kHz-r19                                  ENUMERATED {n0, n1, n2, n4, n8, n16}                       OPTIONAL,

scs30kHz-r19                                  ENUMERATED {n0, n2, n4, n8, n16, n32}                      OPTIONAL,

scs60kHz-r19                                  ENUMERATED {n0, n4, n8, n32, n64}                          OPTIONAL,

scs120kHz-r19                                 ENUMERATED {n0, n8, n16, n32, n64, n128}                   OPTIONAL,

scs480kHz-r19                                 ENUMERATED {n0, n32, n64, n128, n256, n512}                OPTIONAL,

scs960kHz-r19                                 ENUMERATED {n0, n64, n128, n256, n512}                     OPTIONAL

}                                                                                                            OPTIONAL,

-- R1 59-1-3: Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window.

uei-TriggerEventDetermination-r19             INTEGER (1..64)                                                OPTIONAL,

-- R1 59-1-4: UE-initiated/event-driven beam management for Event-1 based measurement and report for Mode A

uei-ModeA-Event1-r19                          ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-1-5: UE-initiated/event-driven beam management for Event-7 based measurement and report for Mode A

uei-ModeA-Event7-r19                          BIT STRING (SIZE (8))                                          OPTIONAL,

-- R1 59-1-7: 1-bit condition met indication in RSRP report format for each report of CRI/SSBRI for Event-2 and Event-7

event2ConditionIndication-r19                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-1-6: CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports

timeRestriction128Port-r19                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-1-7: Group-specific 3-bit scaling factors for up to 128 ports

groupScalingFactor-r19                        ENUMERATED {rank1, rank1and2}                                  OPTIONAL,

-- R1 59-2-2-3a: Configuration of MR always-reported resources with Rel-15 Type-I SP codebook

mr-AlwaysReportedType1SP-r19                  ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-2-3b: Configuration of MR always-reported resources with Rel-16 eType-II codebook with R=1

mr-AlwaysReported-eType2-r19                  ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-1: CJTC Dd report

cjtc-DdReport-r19                             SEQUENCE {

minRangeDdInCyclicPrefix-r19                  ENUMERATED {half, full},

maxResolutionDd-r19                           ENUMERATED {n32,n64,n128,n256},

scalingFactor-r19                             INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-1a: CJTC Dd report processing

cjtc-DdReportProcessing-r19                  SEQUENCE {

maxNumberTRS-Resource-r19                     ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberTRS-ResourceAcrossCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-2: CJTC FO report

cjtc-FO-Report-r19                            SEQUENCE {

minRangeFO-r19                                ENUMERATED {ppmDot1, ppmDot2},

maxResolutionFO-r19                           ENUMERATED {n16,n32,n256},

scalingFactor-r19                             INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-2a: CJTC FO report processing

cjtc-FO-ReportProcessing-r19                 SEQUENCE {

maxNumberTRS-Resource-r19                     ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberTRS-ResourceAcrossCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-3: CJTC wideband PO report

cjtc-PO-ReportWideband-r19                    SEQUENCE {

maxResolution-r19                             ENUMERATED {n16, n32},

scalingFactor-r19                             INTEGER (1..2),

maxSlotDuration-r19                           INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-3a: CJTC wideband PO report processing

cjtc-PO-ReportWidebandProcessing-r19          SEQUENCE {

maxNumberCSI-RS-Configured-r19                ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberCSI-RS-ConfiguredAcrossCC-r19        ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-4: CJTC subband PO report

cjtc-PO-ReportSubband-r19                     SEQUENCE {

maxResolution-r19                             ENUMERATED {n16, n32},

minSubbandSize-r19                            ENUMERATED {n1,n2,n4,n8,n16},

scalingFactor-r19                             INTEGER (1..2),

maxSlotDuration-r19                           INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-5: CJTC Dd+FO report

cjtc-DdFO-Report-r19                         SEQUENCE {

minRangeDdInCyclicPrefix-r19                 ENUMERATED {half, full},

maxResolutionDd-r19                           ENUMERATED {n32,n64,n128,n256},

minRangeFO-r19                                ENUMERATED {ppmDot1, ppmDot2},

maxResolutionFO-r19                           ENUMERATED {n16,n32,n256},

scalingFactor-r19                             INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-5a: CJTC Dd+FO report processing

cjtc-DdFO-ReportProcessing-r19                SEQUENCE {

maxNumberTRS-Resource-r19                     ENUMERATED {n2,n4,n6,n8,n10,n12},

maxNumberTRS-ResourceAcrossCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n64},

maxNumberCSI-RS-ResourcePerCC-r19             ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32},

maxNumberCSI-RS-ResourceAcrossCC-r19          ENUMERATED {n2,n4,n6,n8,n12,n16,n20,n24,n28,n32,n64},

valueX-r19                                    INTEGER (1..2)

}                                                                                                            OPTIONAL,

-- R1 59-2-3-6a: New CJT QCL assumptions for PDSCH pre-compensation for Scheme-C

cjt-QCL-PDSCH-SchemeC-r19                     ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-6b: New CJT QCL assumptions for PDSCH pre-compensation for Scheme-D

cjt-QCL-PDSCH-SchemeD-r19                     ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-6c: New CJT QCL assumptions for PDSCH pre-compensation for Scheme-E

cjt-QCL-PDSCH-SchemeE-r19                     ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-7: Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering

linked-CJTC-Dd-eType2CJT-Joint-r19            ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-7a: Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering

linked-CJTC-Dd-eType2CJT-Separate-r19         ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-8: Separate triggering with configuration of 1-bit indicator per CSI trigger state

linked-CJTC-Dd-eType2CJT-SeparatePerState-r19 ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-2-3-10: Relaxed timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT

timelineRelax-CJTC-Dd-eType2CJT-r19           SEQUENCE {

scs15kHz-r19                                  ENUMERATED {n2,n4,n8}                                      OPTIONAL,

scs30kHz-r19                                  ENUMERATED {n4,n8,n14,n28}                                 OPTIONAL,

scs60kHz-r19                                  ENUMERATED {n8,n14,n28}                                    OPTIONAL,

scs120kHz-r19                                 ENUMERATED {n14,n28,n56}                                   OPTIONAL,

scs480kHz-r19                                 ENUMERATED {n56,n112,n224}                                 OPTIONAL,

scs960kHz-r19                                 ENUMERATED {n112,n224,n448}                                OPTIONAL

}                                                                                                            OPTIONAL,

-- R1 59-3-1a: Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP

nonCodebook-CSI-RS-SRS-3TxPUSCH-r19           SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources))

OF SupportedCSI-RS-Resource                    OPTIONAL,

-- R1 59-4-1a: PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s)

pathlossOffsetPUCCH-PUSCH-SRS-JointTCI-r19    ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-1b: PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state(s)

pathlossOffsetPUCCH-PUSCH-SRS-SeparateTCI-r19 ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-2a: Path Loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s)

pathlossOffsetPRACH-JointTCI-r19              ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-2b: Path Loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s)

pathlossOffsetPRACH-SeparateTCI-r19           ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-7a: Extended value range of starting bit of block in DCI format 2_3

extendedStartBitDCI-2-3-r19                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-3: Two SRS closed-loop power control adjustment states separate from PUSCH

twoSRS-PwrControlAdjust-r19                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-5: Overlapping UL transmission reduction

overlapUL-TransReductionEnh-r19               ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-6: MAC-CE update of PL offset value(s)

pathlossOffsetUpdate-r19                      ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-7b: DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes

twoSRS-TPC-DCI-2-3-r19                        ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-8: DCI format 1_1 to indicate TPC command for SRS associated with a separate SRS CLPC adjustment state

srs-TPC-CLPC-AdjustmentState-r19              ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-9a: DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode

twoSRS-DCI-1-1-Separate-r19                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-9b: DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode

twoSRS-DCI-1-1-Joint-r19                      ENUMERATED {supported}                                         OPTIONAL,

-- R1 59-4-11: Support of including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission

-- and Type 1 PHR based on reference PUSCH

pathlossOffsetPHR-r19                         ENUMERATED {supported}                                         OPTIONAL

]]

}

MIMO-ParametersPerBand-v17b0 ::=        SEQUENCE {

-- R1  23-1-1b    Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE

unifiedJointTCI-multiMAC-CE-v17b0       SEQUENCE{

minBeamApplicationTimeJointTCI-v17b0    CHOICE {

fr1-v17b0                               SEQUENCE {

scs-15kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70} OPTIONAL,

scs-30kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70} OPTIONAL,

scs-60kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70} OPTIONAL

},

fr2-v17b0                               SEQUENCE {

scs-60kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70,

sym84, sym98, sym112, sym224, sym336}                                OPTIONAL,

scs-120kHz-v17b0                        ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70,

sym84, sym98, sym112, sym224, sym336}                                OPTIONAL

}

},

maxNumMAC-CE-PerCC-v17b0                        ENUMERATED {n2, n3, n4, n5, n6, n7, n8}

}                                                                                                                          OPTIONAL,

-- R1  23-10-1b    Unified TCI with separate DL/UL TCI update for intra-cell beam management with more than one MAC-CE

unifiedSeparateTCI-multiMAC-CE-v17b0    SEQUENCE{

minBeamApplicationTimeSeparateTCI-v17b0 CHOICE {

fr1-v17b0                               SEQUENCE {

scs-15kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70} OPTIONAL,

scs-30kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70} OPTIONAL,

scs-60kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70} OPTIONAL

},

fr2-v17b0                               SEQUENCE {

scs-60kHz-v17b0                         ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70,

sym84, sym98, sym112, sym224, sym336}                                OPTIONAL,

scs-120kHz-v17b0                        ENUMERATED {sym1, sym2, sym4, sym7, sym14, sym28, sym42, sym56, sym70,

sym84, sym98, sym112, sym224, sym336}                                OPTIONAL

}

},

maxActivatedDL-TCIPerCC-v17b0                 INTEGER (2..8),

maxActivatedUL-TCIPerCC-v17b0                 INTEGER (2..8)

}                                                                                                                          OPTIONAL

}

DummyG ::=                          SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx   ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}

}

BeamManagementSSB-CSI-RS ::=        SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n0, n8, n16, n32, n64},

maxNumberCSI-RS-Resource            ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberCSI-RS-ResourceTwoTx       ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}                                       OPTIONAL,

maxNumberAperiodicCSI-RS-Resource   ENUMERATED {n0, n1, n4, n8, n16, n32, n64}

}

DummyH ::=                          SEQUENCE {

burstLength                         INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..128)

}

CSI-RS-ForTracking ::=              SEQUENCE {

maxBurstLength                      INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..256)

}

CSI-RS-IM-ReceptionForFeedback ::=              SEQUENCE {

maxConfigNumberNZP-CSI-RS-PerCC                 INTEGER (1..64),

maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC      INTEGER (2..256),

maxConfigNumberCSI-IM-PerCC                     ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousNZP-CSI-RS-PerCC           INTEGER (1..64),

totalNumberPortsSimultaneousNZP-CSI-RS-PerCC    INTEGER (2..256)

}

CSI-RS-ProcFrameworkForSRS ::=                  SEQUENCE {

maxNumberPeriodicSRS-AssocCSI-RS-PerBWP         INTEGER (1..4),

maxNumberAperiodicSRS-AssocCSI-RS-PerBWP        INTEGER (1..4),

maxNumberSP-SRS-AssocCSI-RS-PerBWP              INTEGER (0..4),

simultaneousSRS-AssocCSI-RS-PerCC               INTEGER (1..8)

}

CSI-ReportFramework ::=                         SEQUENCE {

maxNumberPeriodicCSI-PerBWP-ForCSI-Report       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForCSI-Report      INTEGER (1..4),

maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4),

maxNumberPeriodicCSI-PerBWP-ForBeamReport       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForBeamReport      INTEGER (1..4),

maxNumberAperiodicCSI-triggeringStatePerCC      ENUMERATED {n3, n7, n15, n31, n63, n128},

maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4),

simultaneousCSI-ReportsPerCC                    INTEGER (1..8)

}

CSI-ReportFrameworkExt-r16 ::=                      SEQUENCE {

maxNumberAperiodicCSI-PerBWP-ForCSI-ReportExt-r16   INTEGER (5..8)

}

PTRS-DensityRecommendationDL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29),

sampleDensity1                      INTEGER (1..276),

sampleDensity2                      INTEGER (1..276),

sampleDensity3                      INTEGER (1..276),

sampleDensity4                      INTEGER (1..276),

sampleDensity5                      INTEGER (1..276)

}

SpatialRelations ::=                    SEQUENCE {

maxNumberConfiguredSpatialRelations     ENUMERATED {n4, n8, n16, n32, n64, n96},

maxNumberActiveSpatialRelations         ENUMERATED {n1, n2, n4, n8, n14},

additionalActiveSpatialRelationPUCCH    ENUMERATED {supported}                              OPTIONAL,

maxNumberDL-RS-QCL-TypeD                ENUMERATED {n1, n2, n4, n8, n14}

}

DummyI ::=               SEQUENCE {

supportedSRS-TxPortSwitch           ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, tr-equal},

txSwitchImpactToRx                  ENUMERATED {true}                                       OPTIONAL

}

CSI-MultiTRP-SupportedCombinations-r17 ::= SEQUENCE {

maxNumTx-Ports-r17                         ENUMERATED {n2, n4, n8, n12, n16, n24, n32},

maxTotalNumCMR-r17                         INTEGER (2..64),

maxTotalNumTx-PortsNZP-CSI-RS-r17          INTEGER (2..256)

}

-- TAG-MIMO-PARAMETERSPERBAND-STOP

-- ASN1STOP

MIMO-ParametersPerBand field descriptions

codebookParametersPerBand For a given frequency band, this field this field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resources indicated by this field are referred by codebookParametersperBC in CA-ParametersNR to indicate the supported CSI-RS resource per band combination.

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework CSI related capabilities which the UE supports on each of the carriers operated on this band. If the network configures the UE with serving cells on both FR1 and FR2 bands these values may be further limited by the corresponding fields in fr1-fr2-Add-UE-NR-Capabilities.

supportNewDMRS-Port Presence of this field set to supported1, supported2 or supported3 indicates that the UE supports the new DMRS port entry {0,2,3}.

#### –	ModulationOrder

The IE ModulationOrder is used to convey the maximum supported modulation order.

ModulationOrder information element

-- ASN1START

-- TAG-MODULATIONORDER-START

ModulationOrder ::= ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATIONORDER-STOP

-- ASN1STOP

#### –	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]]

}

MRDC-Parameters-v1580 ::= SEQUENCE {

dynamicPowerSharingNEDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v1590 ::=	SEQUENCE {
