3GPP TSG RAN WG2 Meeting #131bis	R2-2507906

Prague, Czech Republic, 13th – 17th October, 2025

# Title:	Draft LS on Security parameter in A-IoT paging

# Response to:	-

# Release:	Release 19

# Work Item:	Ambient_IoT_solutions

Source:	CMCC [to be RAN WG2]

To:	SA WG3

Cc:	CT WG1

Contact Person:

Name:	Ningyu Chen

E-mail Address:	chenningyu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

# 1. Overall Description:

RAN2 thanks SA3 on security parameter in paging request message (R2-2505064/ S3-252392).

RAN2 would like to ask SA3 the following question:

Question: Whether the 128 bits security parameter has to be included in every paging message?

From RAN2 point of view, while it is feasible to contain 128 bits security parameter in A-IoT paging message from signaling perspective, RAN2 has following concerns and downsides with making the 128 bits mandatory for every paging message:

- Deployment: secure indoor deployments, where it may not be needed to always keep re-authenticating a specific known device in every paging message if it has just completed the authentication.

- Overhead: high overhead to transmit 128 bit RANDAIOT_n in A-IoT paging message and to transmit 128 bit RANDAIOT_d and 32bit RESAIOT in D2R message. And it should be noted there is no segmentation in R2D message.

- Complexity: complexity for devices to perform authentication calculations.

- Power consumption: power consumption overhead with authentication

- Time consuming: time consuming for devices to perform authentication calculations

- Coverage reduction: coverage reduction due to larger payload size of the A-IoT paging message

RAN2 would like to clarify that RAN2 has defined different types of paging messages:

- Paging for one, multiple or all devices: A-IoT paging allows the A-IoT reader to trigger one, multiple or all A-IoT device(s) to perform A-IoT CBRA (Contention Based Random Access) or A-IoT CFA (Contention Free Access). The A-IoT paging message may include zero or one paging identifier, i.e., AIoT Device Permanent Identifier or Filtering Information as specified in TS 23.369. If a paging identifier is included, the A-IoT paging message may be addressed to a single A-IoT device or a group of A-IoT devices. If no paging identifier is included, the A-IoT paging message is addressed to all A-IoT devices.

- Subsequent paging: To support device re-access after access failure for the same A-IoT service request, gNB-reader may send the subsequent paging message containing the same transaction id as the previous paging message.

# 2. Actions:

# To SA3:

ACTION: RAN2 respectfully asks SA3 to take the above concerns into consideration and provide answers for the question above.

# 3. Dates of Next RAN2 Meetings:

RAN2#132				17th to 21st November 2025			Dallas, US

RAN2#133				9th to 13rd February 2026			Gothenburg, SE