3GPP TSG-RAN WG2 Meeting #131bis	R2-2506816

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:	8.1.1

Source:	CATT

Title:	Open issues on UE capability for Mob Ph4

Document for:	Discussion and Decision

Introduction

This document is the report of the following offline discussion.:

[AT131bis][105][MOB] Capabilities (CATT)

Scope: Discuss open issues in R2-2506816, RIL V400, and P2 and P3 of R2-2507550.

Intended outcome: Discussion summary in R2-2507720 and CRs (if possible) in R2-2507721 (331) and R2-2507722.

Deadline: CB on Thursday.

Open issues

According to [1],the following open issues are identified by companies,.

| Issue number | Issue description and potential resolution | Rapporteur comment |
| --- | --- | --- |
| Ericsson-1 | [Issue]: For cltm-ExecutionConditionL1-r19, Is it so that if UE signal this capability it shall also indicate support of some of the L1 measurements capability? [potential resolution]: UE also indicate support at least the intra-frequency L1 measurements capability | This issue was raised by Ericsson during CR review but not included in the final CR due to no time to align the understanding between companies. The Rapp thinks this is a valid issue to address and the potential resolution is clear. The Rapp is making a proposal on this. No contributions from companies on this are expected. Proposal 1: (Ericsson-1)For cltm-ExecutionConditionL1-r19, UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 for at least one band combination. |
| Samsung-1 | In Rel18, there are the following capabilities about MCG LTM with NR-DC. ltm-MCG-NRDC-r18 Indicates whether the UE supports LTM for MCG with RACH with NR-DC configured as defined in TS 38.331 [9] and TS 38.321 [8]. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18.. ltm-MCG-NRDC-Release-r18 Indicates whether the UE supports LTM for MCG with the release of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18.. In Rel-18, there was a restriction for MCG LTM as below mrdc-SecondaryCellGroupConfig This field is used to configure and release an SCG in NR-DC and NE-DC. In an RRCReconfiguration message within an LTM-Config IE associated with the MCG, if this field is present its value can only be set to release. The restriction is removed in Rel-19, without any conditions. i.e. MCG LTM for intra-CU LTM, Inter-CU MCG LTM and CLTM may include mrdc-SecondaryCellGroupConfig set to setup. Since network needs to identify UEs which supports SCG addition/modification during MCG LTM, a new capability similar to the above capabilities may be added, as below. ltm-MCG-NRDC-Setup-r19 Indicates whether the UE supports LTM for MCG with the setup of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18. As an alternative, newly defined Rel-19 capabilities may specify the behaviour. i.e. A UE supporting ltm-KeyUpdateMCG-r19 or cltm-ExecutionConditionL1-r19 or cltm-ExecutionConditionL3-r19 along with ltm-MCG-NRDC-r18 supports the setup of SCG during MCG LTM, as below. ltm-KeyUpdateMCG-r19 Indicates whether the UE supports security key change during MCG LTM cell switch execution. A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 in at least one band. A UE supporting this feature and ltm-MCG-NRDC-r18 supports setup of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received.. cltm-ExecutionConditionL1-r19 Indicates whether the UE supports the evaluation of LTM conditions evaluation based on L1 measurements. The UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 for at least one band. A UE supporting this feature and ltm-MCG-NRDC-r18 supports setup of NR-DC configuration as zpart of LTM execution when conditional LTM cell switch is executed.. cltm-ExecutionConditionL3-r19 Indicates whether the UE supports the evaluation of LTM conditions evaluation based on L3 measurements, by indicating the maximimum number of trigger events for the same execution condition. A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 for at least one band. A UE supporting this feature and ltm-MCG-NRDC-r18 supports setup of NR-DC configuration as part of LTM execution when conditional LTM cell switch is executed.. Out of these alternatives, we prefer a separate capability, since according to the RRC CR, Intra-CU MCG LTM can also add/modify SCG. Without the capability, some restriction also may need to be specified in RRC for this case, which is not required. | The Rapp thinks no new capability is needed as if the UE indicates the support of ltm-KeyUpdateMCG-r19 and ltm-MCG-NRDC-r18,then the NW can know the mrdc-SecondaryCellGroupConfig can be set to value other than release.Otherwise,it can only set it to release. The Rapp is making a proposal on this. No contributions from companies on this are expected. Proposal 2: (Samsung-1)If the UE indicates the support of ltm-KeyUpdateMCG-r19 and ltm-MCG-NRDC-r18,then the NW can know the mrdc-SecondaryCellGroupConfig can be set to value other than release. Otherwise, it can only set it to release. |
| Samsung-2 | For R18 reporting, there are some restrictions on the maximum number of LTM-CSI-ReportConfig for the number of periodic/aperiodic/semi-persistent reporting. For e.g. there are following capabilities -supportedMaxAperiodic-LTM-CSI-ReportConfig-r18 indicates maximum number of aperiodic LTM-CSI-ReportConfig; -supportedMaxPeriodic-LTM-CSI-ReportConfig-r18 indicates maximum number of periodic LTM-CSI-ReportConfig; -supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r18 indicates maximum number of semi-persistant LTM-CSI-ReportConfig; For L3 measurement configuration, even though there are no capabilities for the maximum number of measurement identities, there is a RRC constant which caps the maxNrofMeasId to 64. For NR-DC, this is shared by both MN and SN triggered measurement configuration. For event triggered L1 MR reporting, there is no capability on the maximum number of configurations. If we consider that the upper cap is based on the number of LTM-CSI-ReportConfig possible, it can be up to 48 based on RRC constant, and for NR-DC it can be up to 96, which is larger than even the maximum number of measurement identifiers for L3 measurements. This value seems to be not reasonable, and we think some constraints, preferably through capability reporting is required. UE can inform the maximum number of LTM Event configurations that it can support using capability reporting. | This is valid issue to clarify. The Rapp is making a proposal on this. No contributions from companies on this are expected. Proposal 3: (Samsung-2)RAN2 discuss whether to define a new UE capability for the supported maximum number of LTM Event configurations during the meeting. |

Issue Ericsson-1

For issue Ericsson-1(i.e., For cltm-ExecutionConditionL1-r19, Is it so that if UE signal this capability it shall also indicate support of some of the L1 measurements capability?), the Rapp’s proposal are as follows, according to [1],

Proposal 1: (Ericsson-1)For cltm-ExecutionConditionL1-r19, UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 for at least one band combination.

Issue Samsung-1

For issue Samsung-1(i.e., the needs of a new UE capability for including mrdc-SecondaryCellGroupConfig set to setup),the CR rapp’s proposals are as follows, according to [1],

Proposal 2: (Samsung-1)If the UE indicates the support of ltm-KeyUpdateMCG-r19 and ltm-MCG-NRDC-r18,then the NW can know the mrdc-SecondaryCellGroupConfig can be set to value other than release. Otherwise, it can only set it to release.

However, it is argued in contribution [2] that a separate UE capability should be introduced for MCG LTM with NR-DC configuration. And suggest RAN2 to discuss if case 4 (UE does not yet have NR-DC configured and UE executes MCG LTM upon which NR-DC is setup) is a valid case and decide whether to introduce a separate UE capability for it.

R2-2507550	Remaining CP issues in R19 mobility	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: A separate UE capability is introduced for MCG LTM with the setup of NR-DC configuration. Proposal 3: RAN2 to discuss if case 4 (UE does not yet have NR-DC configured and UE executes MCG LTM upon which NR-DC is setup) is a valid case and decide whether to introduce a separate UE capability for it.

The Rapp still thinks no new capability is needed as if the UE indicates the support of ltm-KeyUpdateMCG-r19 and

ltm-MCG-NRDC-r18,then the NW can know the mrdc-SecondaryCellGroupConfig can be set to setup. Otherwise, it can only set it to release or be absent.

Therefore, it is proposed that,

Proposal 2: (Samsung-1)If the UE indicates the support of ltm-KeyUpdateMCG-r19 and ltm-MCG-NRDC-r18,then the NW can know the mrdc-SecondaryCellGroupConfig can be set to value setup. Otherwise, it can only set it to release or be absent.

Issue Samsung-2

For issue Samsung-2(i.e., whether to define a new UE capability for the supported maximum number of LTM Event configurations), The Rapp would like to check companies’ views on this and conclude this issue based on the majority view,

Proposal 3: (Samsung-2)RAN2 discuss whether to define a new UE capability for the supported maximum number of LTM Event configurations during the meeting.

Issue v400

According to [3],the issue V400 is raised as follows,

If UE supports Rel-18 intra-CU LTM recovery but does not support Rel-19 inter-CU LTM recovery, the network will not configure attemptLTM-Switch-r18 to the UE when inter-CU LTM candidates are present because inter-CU LTM recovery is not supported by UE. This kind of network implementation prevents intra-CU LTM-based recovery from functioning as the UE cannot do intra-CU LTM-based recovery in this case. Therefore, a separate indicator for inter-CU LTM recovery should be introduced in the RRC specification instead of reusing the existing attemptLTM-Switch-r18 IE. Proposal 1: [V400] A separate indicator to enable inter-CU LTM recovery should be introduced in the RRC specification instead of reusing the existing attemptLTM-Switch IE.

The Rapp would like to check companies’ views on this and conclude this issue based on the majority view.

Proposal 4: [V400] RAN2 discuss whether a separate indicator to enable inter-CU LTM recovery should be introduced in the RRC specification instead of reusing the existing attemptLTM-Switch IE.

Conclusion

Proposal 1: (Ericsson-1)For cltm-ExecutionConditionL1-r19, UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 for at least one band combination.

Proposal 2: (Samsung-1)If the UE indicates the support of ltm-KeyUpdateMCG-r19 and ltm-MCG-NRDC-r18,then the NW can know the mrdc-SecondaryCellGroupConfig can be set to value other than release. Otherwise, it can only set it to release.

Proposal 3: (Samsung-2)RAN2 discuss whether to define a new UE capability for the supported maximum number of LTM Event configurations during the meeting.

Reference

R2-2506816,Report of [POST131][115][MOB] Open issues on UE capability (CATT)

R2-2507550,Remaining CP issues in R19 mobility	, MediaTek

R2-2507015,Discussion on RRC open issues for R19 mobility, vivo