3GPP TSG-RAN WG3 Meeting #129bis	R3-256602

Prague, CZ, October 13-17, 2025

Agenda item:	10.5.2

Source:	Tejas Networks

Title:	Federated Learning Framework for AI/ML for RAN

Document for:	Discussion

# 1	Introduction

This TDoc discusses the benefits of introducing Federated Learning framework in 6G RAN.

# 2	Discussion

2.1       Limitations of Existing AI/ML Frameworks

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

As per the TS 38.300 all the AI/ML for NG RAN use cases were based on the following AI/ML framework [1]:

The drawbacks of the above-mentioned solutions in TS 38.300[1] are:

- In solution 1, raw data needs to be sent to OAM from gNBs for training, which causes data privacy issues.

- In solution 2, the absence of insights from other gNBs will hinder the overall performance of the models within the gNBs.

Introducing Federated Learning is a promising solution for this. Various types of Federated Learning, such as Horizontal Federated Learning and Vertical Federated Learning, have already been implemented among NWDAFs and AFs in SA2.

## 2.2	Federated Learning

Federated Learning (FL) is a machine learning approach where multiple devices or nodes collaboratively train a shared model while keeping their data local. Instead of sending raw data to a central server, each participant trains the model on its own data and only shares model updates (like gradients or weights), which are then aggregated to improve the global model. This enhances data privacy, security, and efficiency, especially in distributed environments like mobile networks. Figure 1 illustrates the general architecture of Federated Learning.

Fig. 1: General Architecture of Federated Learning

2.3       Federated Learning in 6G RAN

OAM

OAM

6G RAN NODE n

6G RAN NODE 1

6G RAN NODE 1

Fig. 2: Federated Learning Model

In the Federated Learning approach, the OAM (Operation, Administration, and Maintenance) functions as the server, while the 6G RAN nodes act as the clients. Each 6G RAN node collects its own local data and trains its model based on that data. Instead of sending the raw data to the OAM, the 6G RAN nodes send only the model weights (i.e., the learned parameters) to the OAM. The OAM then aggregates the model weights from all participating 6G RAN nodes to create an aggregated model. These aggregated model parameters are sent back to the 6G RAN nodes, which continue the training process on their local data. The iterative process continues until the global model in the OAM converges.

This approach eliminates the need for transmitting raw data from the 6G RAN nodes to the OAM, ensuring that sensitive user data remains local and enhancing both privacy and efficiency. We propose that RAN3 studies Federated Learning for the use cases such as UE traffic prediction, allowing 6G RAN nodes to collaboratively create a global model, tailored to the unique data and conditions of each individual 6G RAN nodes.

Proposal 1: RAN3 shall study and undertake normative work on Federated Learning for AI/ML for 6G RAN, where the OAM acts as the FL server and the 6G RAN Nodes serve as FL clients, the model training will occur in both the OAM and the 6G RAN nodes, while the inference will take place in the 6G RAN nodes.

# 3	Conclusions

Proposal 1:  RAN3 shall study and undertake normative work on Federated Learning for AI/ML for 6G RAN, where the OAM acts as the FL server and the 6G RAN Nodes serve as FL clients, the model training will occur in both the OAM and the 6G RAN nodes, while the inference will take place in the 6G RAN nodes.

# References

[1]   TS 38.300 	NR and NG-RAN Overall description; Stage-2