3GPP TSG-RAN WG3 Meeting #129bis	R3-256696

Prague, CZ, 13th – 17th October 2025

Agenda item:	12.2.3

Source: 	Samsung

Title: 	Discussion on other handover enhancements

Document for:	Discussion and decision

Introduction

Aside from multi-hop UE trajectory and intra-CU LTM, there could be other mechanisms for handover enhancements by the assistance of AI/ML. This paper provides discussion on other handover enhancements for AI/ML-based mobility.

Discussion

AI/ML could be leveraged to assist with the mobility optimization, including cell-based UE trajectory prediction which was discussed in Rel18, intra-CU LTM as discussed in AI12.2.2, etc. In this section, when considering other enhancements, we are actually open to discuss any mechanism that was not touched before, and we can start from inter-CU LTM in this section, considering the intra-CU LTM is already a main topic of this SID.

In our discussion paper for intra-CU LTM [1], we are basically proposing to use AI/ML for predicting the TA value for the UE to connect to candidate/target cell(s), which would save the TA acquisition procedures for the UE and minimize the interruption and latency for handover. For inter-CU LTM, we believe similar mechanism could be applied, based on the inter-CU LTM related procedures.

Proposal 1: Similar mechanism for predicting TA value could be applied to inter-CU LTM as for intra-CU LTM.

For inter-CU LTM, XnAP impacts were introduced in Rel-19 to enable to source NG-RAN node and target NG-RAN node to coordinate about LTM configuration information, cell switch command notification, TA value information and so on. In order to allow the TA value prediction for inter-CU LTM, it is reasonable that the input and output information could be the same as what we defined for intra-CU LTM. An issue to be settled for inter-CU LTM is which node shall perform the prediction of TA value, and this could also impact how the data collection procedure is performed between two gNBs. Since there is no conclusion about which node shall perform the prediction of TA value in intra-CU LTM, it is too early to decide which node to perform TA value prediction in inter-CU LTM. So, we tend to postpone the discussion of this point in inter-CU LTM, and wait for the discussion in intra-CU LTM.

Proposal 2: Which node to perform the prediction of TA value could be pending the discussion of intra-CU LTM.

Once we have a clear vision about how to perform TA value prediction for intra-CU LTM, the corresponding data collection procedures could be applied to inter-CU LTM, with necessary enhancements in XnAP. For example, if it is the source DU to perform the TA value prediction, for intra-CU LTM, some of the input information would be collected from the candidate gNB-DU via the gNB-CU, involving F1AP enhancement. Similarly, for inter-CU LTM, if it is the source gNB-DU to perform TA value prediction, the input information may need to be collected from candidate cells in the target NG-RAN node, which means XnAP data collection procedure shall be enhanced. We believe the enhancements would be straight-forward.

Proposal 3: After the mechanism of AI/ML-based intra-CU LTM is determined, the enhancements for inter-CU LTM would be straight-forward.

Conclusion

In this paper, the following proposals are provided:

Proposal 1: Similar mechanism for predicting TA value could be applied to inter-CU LTM as for intra-CU LTM.

Proposal 2: Which node to perform the prediction of TA value could be pending the discussion of intra-CU LTM.

Proposal 3: After the mechanism of AI/ML-based intra-CU LTM is determined, the enhancements for inter-CU LTM would be straight-forward.

References

[1] R3-256695