3GPP TSG-RAN WG3 Meeting #129bis	R3-256719

Prague, Czech Republic, 13th – 17th October, 2025

Title:	Discussion on possible RAN-CN interface evolution towards Service Based Architecture

Source:	FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra

Agenda item:	10.3.2

Document Type:	Discussion and Decision

# 1	Introduction

The Service-Based Architecture (SBA) model, which has already demonstrated its effectiveness in the 5G Core Network (5CN) and in OAM domains since Rel-15, is widely recognized for enabling modularity, improved service flexibility, and streamlined integration processes. In contrast, the current architecture of the RAN and the interface towards the Core Network (CN) remains anchored to a Protocol-Based approach, characterized by tightly coupled interfaces and limited architectural agility.

In contribution [1], the rationale for investigating the adoption of SBA principles in the RAN domain has been introduced, outlining potential advantages, key constraints, and preliminary requirements. That analysis also highlighted the need of identifying which parts of the RAN could realistically benefit from a service-oriented design, taking into account some constraints like strict latency and performance demands, addressed in document [2].

This contribution builds on those foundations and focuses on exploring possible architectural directions to introduce SBA between the RAN and the CN. It aims to outline some alternatives in implementation scenarios, starting from current Network Functions (NFs) within the RAN.

This contribution is grounded on ongoing activities within the research project ORIGAMI [3]. The ORIGAMI project brings together leading industry and academic partners across the European mobile telecommunications ecosystem and is funded by the European Commission under the 6G Smart Networks and Services Joint Undertaking (SNS JU). Running from 2024 to 2026, ORIGAMI focuses on advancing architectural models for next-generation mobile networks and addressing key barriers to the deployment of successful 6G technologies. Specifically, ORIGAMI aims to study 6G architectural innovations, like the Global Service-Based Architecture (GSBA), that enables solutions of Network Intelligence (NI) designed to enhance network performance, security, and flexibility. Beyond technological advances, ORIGAMI is committed to establishing global unified standards, supporting the green transition, improving affordability and accessibility, and fostering the development of groundbreaking applications to create strong ecosystem and novel business models for operators to generate new revenue opportunities and allows operator networks to evolve into future generation with advanced capabilities. This holistic approach positions ORIGAMI at the forefront of shaping SBA that will underpin future 6G networks, as addressed in [4].

# 2	Discussion

## Potential SBA Proposals for the RAN

For the introduction of a SBA approach in the RAN, the current 5G System (5GS) architecture depicted in Figure 1 can be considered as a starting point, where there is the evidence of the difference between the service-based design of the 5GC and the protocol-based architecture of the RAN.

While the 5GC already embraces the SBA paradigm, enabling flexible service exposure, dynamic discovery of network functions, and modular integration, the RAN still relies on tightly specified interfaces and procedural communication models between its NFs. This results in a less flexible and less scalable system design.

Figure 1 illustrates the existing 5GS architectural model, with the CN represented through a clear SBA approach. The RAN, shown in the red dashed ellipse, interfaces with the 5GC through standardized reference points (e.g., N2, N3), and internally does not yet adopt a service-based interaction model. This observation motivates the idea of applying the Service-Based paradigm also to the RAN, by rethinking both:

- the external interfaces between RAN and CN, ensuring alignment with SBA principles where applicable, addressed in this contribution

- the internal architecture of the RAN, with a focus on potential reorganization of functional blocks and NFs into service-oriented structure based on services to be provided and consumed each other, addressed in [2]

Figure 1 – Starting point: current 5GS architecture and the proposed SBA extension to RAN

From a technical standpoint, applying the SBA paradigm to the RAN implies a fundamental shift in how RAN NFs interact, both internally and with external domains such as the 5GC. In particular, this evolution could involve:

- Service Identification from Signalling Procedures – The transition towards SBA requires an initial step of analysing existing signalling procedures to identify the underlying functionalities they implement. These functionalities can then be mapped into discrete services, allowing a clear definition of which RAN modules act as service producers or consumers. This decomposition is essential for modularizing RAN behaviour and enabling service exposure, invocation, and reusability across functional boundaries

- Service Exposure and Discovery: RAN NFs could expose their capabilities as discoverable services via standardized APIs, enabling other NFs (within RAN or external domains) to dynamically locate and invoke such services. This would require introducing mechanisms, similar to the Network Function Repository Function (NRF) used in the 5GC

- Consumer/Provider Roles within RAN: functions such as the one related to the gNB Central Unit Control Plane (CU-CP), gNB Distributed Unit (DU), or higher-layer schedulers could act as service providers, offering functionalities (e.g., resource management, UE context handling, QoS provisioning) that other RAN functions consume via service calls, rather than relying on tightly coupled protocol procedures

- RAN Interface re-architecture: traditional point-to-point interfaces like F1, E1 and Xn could be re-evaluated and potentially redesigned as service-based reference points, allowing more flexible and extensible communication patterns, including support for multi-tenancy, programmability, and network slicing

- Interworking with SBA-based CN: applying SBA to the RAN would also allow more coherent and native integration with the CN, possibly enabling RAN services to be easily invoked or orchestrated, especially in the context of AI/ML-driven network automation and intent-based management

This conceptual transition raises several architectural and performance-related challenges, including service granularity, latency budgets, reliability, and control/user plane separation.

In the following, the interaction between the RAN and the CN is addressed, while in [2] internal interfaces and functional splits within the RAN architecture are analysed, following a top-down approach.

SBA applied to RAN-CN N2 interface

In this scenario, the SBA is applied to the N2 interface, which connects the RAN and the CN in the control plane domain. The goal is to introduce service-based interactions between the CU-CP in the RAN and the AMF (Access and Mobility Management Function) in the CN, leveraging the existing SBA capabilities already defined in the 5GC.

As illustrated in Figure 2, a logical Service Bus is introduced between the RAN and CN. While the internal RAN architecture remains unchanged (maintaining existing interfaces and splits such as CU-CP, CU-UP, and DU), the communication over N2 is redefined in terms of service exposure and consumption:

- The AMF exposes services, i.e. as a producer, (e.g., UE context management, registration handling) that the CU-CP can discover and invoke, i.e. as a consumer

- The CU-CP, in turn, can expose services, i.e. as a producer, (e.g., RAN assistance information, local measurements, or configuration parameters) that may be consumed by the AMF, i.e. as a consumer

This results in the definition of two service-based reference points (SAP – Service Access Point) indicated as:

- Nngamf*: this SAP refers to services exposed by the AMF and consumed by the CU-CP

- Nngcucp: this SAP refers to services exposed by the CU-CP and consumed by the AMF

Figure 2 – High level scheme of SBA applied to N2 interface

The motivation for applying SBA specifically to N2 (and possibly also to N1 between UE and CN) lies in the fact that these interfaces do not have strict real-time or low-latency constraints. Additionally, the AMF is already implemented as a service-based NF in the Core. Furthermore, the AMF is already implemented as a service-based NF in the 5GC, making it a natural candidate for supporting service-oriented interactions with RAN nodes.

From an architectural perspective, in current deployments, a single AMF typically manages multiple CU-CP instances. Under a traditional P2P model, this would require the AMF to maintain dedicated connections and interface configurations for each CU-CP it interacts with. This can lead to tight coupling, limited scalability, and higher operational complexity.

By adopting a service-based model, the AMF can expose well-defined services on the N2 interface (through the Nngamf* SAP), which can be discovered and consumed dynamically by any authorized CU-CP over the shared Service Bus. Similarly, each CU-CP can register its own services (e.g., RAN context or mobility-related inputs), enabling bidirectional, loosely coupled interactions based on the producer/consumer paradigm.

For example, during UE registration or mobility procedures (e.g., inter-gNB handovers), the CU-CP may need to interact with the AMF selected for a given UE. With SBA, the CU-CP can simply discover and invoke the required AMF services without needing pre-established bindings or manual configuration. Similarly, the AMF can leverage services exposed by different CU-CPs to retrieve RAN context information or apply mobility optimization policies dynamically.

This loosely coupled interaction model enables:

- Dynamic association between AMF and CU-CP instances, based on UE location, load, or policy

- Simplified lifecycle management for RAN–CN integration, particularly during onboarding or reconfiguration of new RAN nodes

- Improved support for roaming or inter-PLMN scenarios, where service discovery and secure exposure mechanisms can reduce complexity in cross-domain communication

- Multi-operator or shared RAN deployments, where a consistent and secure exposure model is needed across administrative domains

This scenario represents a first step toward SBA adoption in the RAN, targeting a well-bounded and evolution-friendly interface (N2), while minimizing impact on the existing RAN functional structure. It also establishes the foundation for extending SBA to additional RAN interfaces in future scenarios, such as E1, Xn-C and F1-C, as addressed in [2].

The analysis highlights how the principles of the SBA, originally defined within the 5GC, can be applied for the interface between RAN and CN to increase modularity, enable more flexible function interaction, and support scalable deployment models.

In particular, the SBA paradigm proves especially beneficial in architectural points where services are dynamically consumed by multiple entities, i.e., scenarios involving 1:N or N:1 relationships, as the ones existing on N2.

Observation 1: SBA approach could be beneficial to evolve current RAN-CN interface to enable higher flexibility, e.g. allowing a dynamic interaction with multiple service consumers and producers

Based on the aspects outlined above, the introduction of SBA in the RAN-CN interface should be considered.

Proposal 1: RAN3 to study SBA introduction in the RAN-CN interface.

# 3	Conclusion

In this paper we discuss the scenario of SBA introduction in the RAN-CN and the following is proposed:

Proposal 1: RAN3 to study SBA introduction in the RAN-CN interface.

# 4	References

[1]	R3-256544, Discussion on Feasibility of RAN Service Based Architecture (FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra)

[2]	R3-256545, Discussion on possible RAN internal architecture evolution towards Service Based Architecture (FiberCop, Jio Platforms, KT Corp., Qualcomm Inc., Telstra)

[3]	https://sns-origami.eu/

[4]	3GPP TR 38.914, “Study on 6G Scenarios and requirements” (Rel-20)