3GPP TSG-RAN3 WG3 Meeting #129bis	R3-256726

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	14.2

Source: 	Qualcomm Incorporated

Title: 	Architecture, functionalities and signaling to support Topology 2

Document for:	Discussion/Decision

# Background

For Rel-20 Ambient_IoT_Solutions_Ph2 WI, following are the RAN3 related objectives mentioned in the WID RP-252894:

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

In this paper, we provide our views for the above objectives and the corresponding proposals.

# Discussion

## Architecture for Topology 2

For Topology 1, RAN3 and SA2 supported both a direct connectivity option and an indirect connectivity option (via the AMF) for the signaling between gNB and AIOTF as specified in TS 23.369.

For Topology 1, both direct-connectivity and indirect-connectivity (via AMF) options between gNB and AIOTF were supported in R19.

However, for Topology 2, SA2 captured as part of the architectural assumptions in TR 23.700-30 to support only the indirect connectivity option.

Architectural assumptions for support of Topology 2 for Key Issue#1:

- No Rel-19 AIoT Device impact is expected.

- The following topology as defined in TR 38.848 [10] is assumed:

- BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control

- Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied.

- The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline.

SA2 agreed to support the indirect connectivity option for Topology 2.

From our understanding, the direct connectivity option was also discussed in SA2, but there was no consensus to add it in the architectural assumptions yet. Considering this might be discussed again in the upcoming SA2 meetings, RAN3 can agree to support at least the indirect connectivity option and wait for SA2 progress on the direct connectivity option.

RAN3 agrees to support indirect connectivity option for Topology 2. Whether direct connectivity option is also to be supported for Topology 2 depends on further SA2 progress.

## Inventory for Topology 2

### Background (Topology 1)

For Topology 1, the following is captured in TS 23.369:

5.3.3      NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also select the RAN reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN selection information (AIoT Area list, RAN Reader list, and, optionally, the location served by each RAN Reader and each AIoT Area) via OAM or local configuration.

When the AIOTF receives an AIoT service request, based on the received Target Area information in the request and the NG-RAN selection information, the AIOTF selects the NG-RAN node(s) to handle the request, and determines a Requested Service Area Information for each selected NG-RAN node.

If the RAN Readers within an AIoT Area is configured in AIOTF, the AIOTF can directly select the RAN Reader(s).

The Requested Service Area Information provided to each NG-RAN node can be empty, a list of AIoT Areas, a list of RAN Readers, or both a list of AIoT Areas and a list of RAN Readers.

Based on the above specification text, the following are our observations:

For Topology 1, AIOTF can receive from AF (via the NEF) a “Target Area Information” in the Naiotf_AIoT_Inventory service operation indicating the targeted area for inventory.

For Topology 1, AIOTF selects NG-RAN node(s) and optionally RAN readers based on the Target Area Information received from AF (via the NEF) and the OAM-configured NG-RAN selection information.

### Area-based and UE-specific inventory request

For Topology 2 as well (similar to Topology 1), RAN3 should assume that the AIOTF can receive the “Target Area Information” indicating the area targeted for performing inventory and then the node selecting UE readers (AIOTF or gNB depending on the UE reader selection discussion) can select appropriate UE readers within the target area for performing inventory. We call this area-based inventory request in this paper.

In addition, RAN3 should also consider UE-specific inventory request e.g., a scenario where a human worker in the factory/supermarket presses a handheld UE reader and this handheld UE reader is to be used for performing inventory. There is no UE reader “selection” in this case.

In fact, SA2 consider both area-based and UE-specific inventory request during their Rel-19 study, and the following aspects were captured as conclusions by SA2 in the R19 TR 23.700-13:

8.3.4       Principles on the procedures to support AIoT services

All the procedures follow have the following steps:

1.   The AF makes a service request to the NEF, including parameters as defined in clause 8.3.5.

2.   The NEF determines an AIOTF for the requested operation and invokes a new service operation on the AIOTF.

3.   For the requested operation the AIOTF:

1).  Performs initial reader selection by either:

-     The AF provide information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader.

NOTE 1:  Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

-     If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

-     If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected, then the request is rejected.

For Topology 1, in TR 23.700-13, SA2 considered the scenario of a single UE Reader ID provided to the AIOTF by the AF (via the NEF) during A-IoT service request (e.g., upon a human worker pressing a button in a handheld UE reader to trigger inventory)

- We therefore propose the following:

RAN3 should consider both UE-specific inventory request and area-based inventory request for Topology 2.

As mentioned in the SA2 conclusions, in case a single UE reader ID is provided to the AIOTF from the AF (via the NEF), then that is used as the UE reader for performing A-IoT service operations (e.g., inventory). In this case, there is no UE reader “selection” procedure as such. Therefore, the following is proposed:

In case of UE-specific inventory request i.e., upon receiving a single UE reader ID from AF (via the NEF), AIOTF simply indicates that UE reader ID to gNB (via the AMF) in NG: Inventory Request. There is no UE reader “selection” in this case.

However in case of area-based inventory request, upon receiving the Target Area Information from AF (via the NEF), AIOTF should be able to select UE readers for performing inventory, similar to how AIOTF can select RAN readers directly (e.g., if OAM has provisioned AIOTF with the NG-RAN selection information which includes A-IoT area list, RAN reader list, location of RAN readers).

In contrast to the selection of RAN readers (which is based on OAM provisioned information), AIOTF can obtain the UE reader’s “location” by interacting with the UE reader’s serving AMF to obtain the UE Reader’s serving gNB or even obtain granular UE location by interacting with LMF (either directly or indirectly).

In case of area-based inventory request i.e., upon receiving the Target Area Information from AF (via the NEF), AIOTF can select UE reader(s) for inventory (e.g., after interaction with AMF serving the target area to obtain the list of UE readers within the target area and/or after interaction with LMF to obtain UE reader’s location).

### NGAP signaling to support Inventory for T2

It is assumed that the UE reader’s subscription/authorization information is stored at the UDM by the AMF and hence the UDM has knowledge of the UE reader’ serving AMF. AIOTF can obtain the UE reader’s serving AMF by interacting with UDM.

In case of area-based inventory request, after selecting UE reader(s), AIOTF should indicate the list of selected UE reader(s) to the each UE reader’s serving AMF(s)), which then forwards it to each UE reader’s serving gNB. Hence the following is proposed:

In case of area-based inventory request, after selecting UE reader(s), AIOTF can indicate the selected UE reader ID(s) to each UE reader’s serving AMF (e.g., via Naiotf_AIoT_Inventory service operation), which then sends the NG: Inventory Request to UE reader’s serving gNB.

In case of UE-specific inventory request, even though there is no UE reader “selection” as mentioned in the previous section, the AIOTF can still indicate to AMF the UE reader ID to use, which then indicates it to the UE reader’s serving gNB.

In both cases (area-based and UE-specific inventory request), AMF receives the UE reader ID(s) (e.g., SUPI/GPSI) from AIOTF and should map it to the corresponding NGAP ID(s) (i.e., RAN UE NGAP ID, AMF UE NGAP ID) before sending the NG: Inventory Request. AMF should maintain the mapping between the UE Reader ID(s) received from AIOTF and the NGAP ID(s).

For each UE Reader selected to perform inventory, AMF maintains the mapping between UE Reader ID received from AIOTF (e.g., GPSI/SUPI) and the RAN UE NGAP ID/AMF UE NGAP ID.

In case multiple UE readers are selected by the AIOTF to perform inventory for a specific A-IoT session, RAN UE NGAP ID, AMF UE NGAP ID are included along with the existing Inventory Request Transfer container for each UE reader the AIOTF is interested in performing inventory. Hence the following is proposed:

Enhance NG: Inventory Request to include {RAN UE NGAP ID, AMF UE NGAP ID} outside the Inventory Request Transfer Container for each UE reader the AIOTF is interested in performing inventory.

Upon receiving the UE Reader’s NGAP ID(s) in NG: Inventory Request, gNB understands that this inventory session requires the use of the indicated UE reader, creates an RRC message for carrying Inventory Request to the UE Reader and sends it to the UE reader.

Similar to how the RAN Reader ID is indicated in the NG: Inventory Response/Report, the UE reader ID (the NGAP ID(s) in this case) can be indicated in the NG: Inventory Response/Report. Therefore, the following is proposed:

Enhance NG: Inventory Response/Inventory Report to include {RAN UE NGAP ID, AMF UE NGAP ID} for identifying the UE reader from which the inventory response/report is sent.

## NGAP signaling to support Command for T2

Similar to the proposals for inventory procedure, AIOTF can indicate to AMF the UE reader ID(s) to be used for the command procedure. AMF can map these UE reader ID(s) received from AIOTF to the NGAP ID(s) and indicate it to the gNB in NG: Command Request.

Enhance NG: Command Request to include {RAN UE NGAP ID, AMF UE NGAP ID} along outside the Command Request Transfer Container to indicate the UE Reader via which the AIOTF is interested in sending the A-IoT Command.

Include RAN UE NGAP ID and AMF UE NGAP ID in NG: Command Response as well

## UE Reader mobility

In R20 A-IoT WID, there is an objective for RAN3 to specify necessary signaling support for inter-gNB RRC-connected UE Reader Mobility.

UE readers can be mobile and can be served by a new gNB upon mobility (e.g., upon moving from one corner of the factory to another corner).

An ongoing A-IoT session (e.g. inventory session) should not be terminated just because the UE reader connects to a new gNB due to mobility. Otherwise, the AF/AIOTF needs to start a new inventory session, perform reader selection yet again, configure the UE reader and target gNB with the A-IoT session parameters yet again every time there is UE reader mobility. A-IoT session parameters can include all things related to the A-IoT session and which are required at the gNB such as the A-IoT session ID, A-IoT Device Identification information, assistance information received from AIOTF e.g., expected number of devices, time interval for result aggregation, Correlation ID.

Also, in case the source gNB has stored some inventory reports received from UE reader over RRC and has not yet sent it to the AIOTF, there needs to be a mechanism to avoid the loss of stored inventory reports. In other words, the stored inventory reports should not be discarded every time there is a serving gNB change upon UE reader mobility. Otherwise, this would be a very inefficient system and lead to unnecessary and repeated inventory procedures. Therefore, the following is proposed:

RAN3 should discuss signaling enhancements over Xn to ensure A-IoT session continuity (i.e., to avoid terminating the ongoing A-IoT session and the AIOTF to configure target gNB with A-IoT session parameters after UE mobility).

RAN3 should discuss signaling enhancements over Xn to avoid loss of stored A-IoT data at source gNB upon UE reader mobility.

## UE Reader authorization

The following was captured in the SA2 interim conclusion in TR 23.700-13 for the UE Reader authorization in T2:

- The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

- The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

- Editor's note:    How to authorize the UE Reader will be determined later.

RAN3 assumes that a UE’s A-IoT related subscription capabilities (e.g., an indication whether the UE is allowed to act as A-IoT Reader) is stored at the UDM and that the AIOTF/AMF interacts with the UDM to check UE’s A-IoT related subscription upon receiving an Inventory Request from AF. This UE reader authorization information should be passed to the gNB serving the UE reader, so that the serving gNB can provision for resources for this authorized UE reader. This is similar to how authorization information is passed from AMF to the gNB for other features such as V2X, ProSe, IAB etc. Therefore, the following is proposed.

AMF provides UE reader related authorization information to the UE reader’s serving gNB via NGAP signaling via a new “A-IoT Authorized” IE with code points {authorized, not-authorized} in the INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACKNOWLEDGE messages over NGAP.

Introduce a new “A-IoT Authorized” IE in necessary messages over XnAP and F1AP as well

## UE Reader capabilities

Just like any feature supported by UE, there would be a discussion needed on UE capabilities, both the AS capabilities and the NAS capabilities. Although the details are under remit of other WGs (e.g., RAN2 for AS capabilities and SA2/CT1 for NAS capabilities), RAN3 needs to discuss (in coordination with other WGs) on which node stores the A-IoT related capabilities as this also will have a say in the UE reader selection discussion

RAN3 can assume that UE Reader can provide its A-IoT related AS capabilities to gNB via RRC signaling e.g., as part of UE capability procedure. UE reader’s AS capabilities can include e.g., list of frequencies supported by UE Reader for AIoT communication, supported band combinations for Uu + AIoT, co-existence of A-IoT with other NR features etc. It is not clear whether these UE reader’s radio capabilities also need to be known/stored at the AMF/AIOTF.

UE reader’s capabilities may include e.g., list of frequencies supported by UE Reader for AIoT communication, supported band combinations for Uu + AIoT, co-existence of A-IoT with other NR features etc.

Also, it is not yet clear whether there would be any A-IoT related NAS capabilities and whether the UE Reader would provide such A-IoT related NAS capabilities to AMF via NAS signaling e.g., during registration procedure. A-IoT related NAS capabilities can include e.g., whether the UE Reader is capable of forwarding A-IoT NAS PDU(s) for DO-DTT operations such as Inventory/Command, whether UE Reader is capable of forwarding A-IoT NAS PDU(s) for DO-A operations).

Since this involves cross WG involvement, we propose the following:

RAN3 to LS SA2 to check whether AIOTF/AMF needs to be aware of UE Reader’s capabilities e.g., for selecting the appropriate UE Reader(s) upon receiving an Inventory/Command Request. Also check with SA2 whether there would be any NAS capabilities, in addition to radio capabilities.

## 2.7 Requested Reader Type

As R20 is now specifying support for UE readers in addition to RAN readers specified in R19, there needs to be a mechanism to indicate the desired type of reader when an AF sends an A-IoT related service request (e.g., for performing inventory/command). For instance, the AF might be interested in only using RAN readers or only using UE readers or it might want to use both. This can for example depend on the time of day e.g., only RAN readers are used in the night hours when there are not many workers in the factory, both UE readers and RAN readers can be used in work hours. Or this could be area-specific e.g., only UE readers are used in some restricted area of the factory. This can help AIOTF in eventually selecting the right type of readers. For this purpose, we propose the following:

Besides indicating the type of Reader (RAN reader or UE reader) via Requested Reader Type, AF might be interested in a specific reader type e.g., indoor/outdoor reader, DO-DTT/DO-A-capable reader or both, reader supporting certain device types (e.g., device 1 or 2a or 2b, passive or active devices or a combination of them), reader(s) supporting a certain coverage, reader(s) supporting a certain Tx/Rx power. Therefore, the following is proposed:

AIOTF can receive from AF (via the NEF) a “Requested Reader Type” to indicate the desired type of readers for performing an A-IoT service operation e.g., UE Reader(s) only, gNB Reader(s) only, Both UE Reader(s) and gNB Reader(s), readers supporting device 1/2a/2b, readers supporting DO-DTT/DO-A

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Architecture for Topology 2

Observation 1:	For Topology 1, both direct-connectivity and indirect-connectivity (via AMF) options between gNB and AIOTF were supported in R19.

Observation 2:	SA2 agreed to support the indirect connectivity option for Topology 2.

Proposal 1:	RAN3 agrees to support indirect connectivity option for Topology 2. Whether direct connectivity option is also to be supported for Topology 2 depends on further SA2 progress.

2.2 Inventory for Topology 2

2.2.1	Background (Topology 1)

Observation 3:	For Topology 1, AIOTF can receive from AF (via the NEF) a “Target Area Information” in the Naiotf_AIoT_Inventory service operation indicating the targeted area for inventory.

Observation 4:	For Topology 1, AIOTF selects NG-RAN node(s) and optionally RAN readers based on the Target Area Information received from AF (via the NEF) and the OAM-configured NG-RAN selection information.

2.2.2	Area-based and UE-specific inventory request

Observation 5:	For Topology 1, in TR 23.700-13, SA2 considered the scenario of a single UE Reader ID provided to the AIOTF by the AF (via the NEF) during A-IoT service request (e.g., upon a human worker pressing a button in a handheld UE reader to trigger inventory)

We therefore propose the following:

Proposal 2:	RAN3 should consider both UE-specific inventory request and area-based inventory request for Topology 2.

Proposal 3:	In case of UE-specific inventory request i.e., upon receiving a single UE reader ID from AF (via the NEF), AIOTF simply indicates that UE reader ID to gNB (via the AMF) in NG: Inventory Request. There is no UE reader “selection” in this case.

Proposal 4:	In case of area-based inventory request i.e., upon receiving the Target Area Information from AF (via the NEF), AIOTF can select UE reader(s) for inventory (e.g., after interaction with AMF serving the target area to obtain the list of UE readers within the target area and/or after interaction with LMF to obtain UE reader’s location).

2.2.3	NGAP signaling to support Inventory for T2

Proposal 5:	In case of area-based inventory request, after selecting UE reader(s), AIOTF can indicate the selected UE reader ID(s) to each UE reader’s serving AMF (e.g., via Naiotf_AIoT_Inventory service operation), which then sends the NG: Inventory Request to UE reader’s serving gNB.

Proposal 6:	For each UE Reader selected to perform inventory, AMF maintains the mapping between UE Reader ID received from AIOTF (e.g., GPSI/SUPI) and the RAN UE NGAP ID/AMF UE NGAP ID.

Proposal 7:	Enhance NG: Inventory Request to include {RAN UE NGAP ID, AMF UE NGAP ID} outside the Inventory Request Transfer Container for each UE reader the AIOTF is interested in performing inventory.

Proposal 8:	Enhance NG: Inventory Response/Inventory Report to include {RAN UE NGAP ID, AMF UE NGAP ID} for identifying the UE reader from which the inventory response/report is sent.

2.3 NGAP signaling to support Command for T2

Proposal 9:	Enhance NG: Command Request to include {RAN UE NGAP ID, AMF UE NGAP ID} along outside the Command Request Transfer Container to indicate the UE Reader via which the AIOTF is interested in sending the A-IoT Command.

Proposal 10:	Include RAN UE NGAP ID and AMF UE NGAP ID in NG: Command Response as well

2.4 UE Reader mobility

Proposal 11:	RAN3 should discuss signaling enhancements over Xn to ensure A-IoT session continuity (i.e., to avoid terminating the ongoing A-IoT session and the AIOTF to configure target gNB with A-IoT session parameters after UE mobility).

Proposal 12:	RAN3 should discuss signaling enhancements over Xn to avoid loss of stored A-IoT data at source gNB upon UE reader mobility.

2.5 UE Reader authorization

Proposal 13:	AMF provides UE reader related authorization information to the UE reader’s serving gNB via NGAP signaling via a new “A-IoT Authorized” IE with code points {authorized, not-authorized} in the INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACKNOWLEDGE messages over NGAP.

Proposal 14:	Introduce a new “A-IoT Authorized” IE in necessary messages over XnAP and F1AP as well

2.6 UE Reader capabilities

Observation 6:	UE reader’s capabilities may include e.g., list of frequencies supported by UE Reader for AIoT communication, supported band combinations for Uu + AIoT, co-existence of A-IoT with other NR features etc.

Proposal 15:	RAN3 to LS SA2 to check whether AIOTF/AMF needs to be aware of UE Reader’s capabilities e.g., for selecting the appropriate UE Reader(s) upon receiving an Inventory/Command Request. Also check with SA2 whether there would be any NAS capabilities, in addition to radio capabilities.

2.7 Requested Reader Type

Proposal 16:	AIOTF can receive from AF (via the NEF) a “Requested Reader Type” to indicate the desired type of readers for performing an A-IoT service operation e.g., UE Reader(s) only, gNB Reader(s) only, Both UE Reader(s) and gNB Reader(s), readers supporting device 1/2a/2b, readers supporting DO-DTT/DO-A