3GPP TSG-RAN3 Meeting #129-bis	R3-256757

# Prague, Czech Republic, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0200 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on positioning data collection for case 3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support opportunistic positioning data collection, a Positioning Data Collection Needed IE was introduced as a top-level IE in the NRPPa MEASUREMENT RESPONSE and MEASUREMENT REPORT messages, this indicates that the data collection request applies to all TRPs involved in the positioning measurement. However, current IE can not support request positioning data for specific TRP(s), which is needed with the following purposes - For model training, even TRPs not selected for positioning can benefit through opportunistic positioning data collection. - For performance monitoring, it is typically sufficient to monitor only a subset of TRPs involved in positioning. Therefore, indicating the request TRP(s) in the Positioning Data Collection Needed IE is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Requested TRP List in the Positioning Data Collection Needed IE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support requesting positioning data for specific TRP(s) according to the actual needs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.106, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.106	Positioning Data Collection Needed

This information element contains the requested positioning information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested Positioning Data Collection | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents a requested positioning information: First bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Third bit: LoS/NLoS information Value ‘1’ indicates ‘needed’, Value ‘0’ indicates ‘not needed’. Other bits reserved for future use. |
| Requested TRP List |  | 0..1 |  | Indicates the TRP(s) that need positioning data. |
| >Requested TRP Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<skip the unchanged parts>

PositioningDataCollectionNeeded ::= SEQUENCE {

requested-PosDataCollection 		BIT STRING (SIZE(8)),

Requested-TRP-List 					Requested-TRP-List,

iE-extensions		ProtocolExtensionContainer { { PositioningDataCollectionNeeded-ExtIEs } } OPTIONAL,

...

}

PositioningDataCollectionNeeded-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Requested-TRP-List ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF Requested-TRP-Item

Requested-TRP-Item ::= SEQUENCE {

tRP-ID				TRP-ID,

iE-extensions		ProtocolExtensionContainer { { Requested-TRP-Item-ExtIEs } } OPTIONAL,

...

}

Requested-TRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>