3GPP TSG-RAN WG3 #129bis	R3-256777

Prague, Czech Republic, 13th – 17th October 2025

Agenda item:		10.2.1

Title:		6G overall RAN architecture principle and requirements

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Following the RAN-WG level study item on the 6G Radio, the study in RAN3 is now started from Q4’2026. (Study Item “Study on 6G Radio” in RP-252912).

This contribution discusses RAN architecture principle and requirement.

# 2. Discussion

2.1	Naming proposal

In the agreed SID for WG level “Study on 6G Radio” has given “6GR” as a shortform to express 6G Radio, which is similar to the “NR” that is using in 5G. Similarly, RAN for 6G, has been called 6G-RAN.

The node within the 6G-RAN, we will call it 6G RAN Node.

If we will have split architecture, we may call the entities in the 6G RAN 6G-NB, 6G-NB-CU, 6G-NB-DU, 6G-NB-UP.

The CN for 6G, may be SA side will give a naming, so far we call it 6GC.

Proposal 1: 6GR and 6G RAN have been used in WG level SID “Study on 6G Radio”. If we will have split architecture, we may call the entities in the 6G RAN 6G-NB, 6G-NB-CU, 6G-NB-DU, 6G-NB-UP etc.. So far we call the CN for 6G as 6GC.

2.2	Principles and requirements for overall 6G RAN architecture.

The deployment scenarios is now currently studied in RAN Plenary SID “Study on 6G Scenarios and Requirements”, which will then capture in TR38.914. Nevertheless, the RAN Architecture deployment scenario is to be studied in RAN3. The RAN Architecture scenarios is basically similar to 5G simply because 3GPP we will need to continue in 6G to provide services in similar or better form that have been available in 5G. Take 5G as the base for further study will be more efficient.

Figure 1: general deployment options of 6G RAN Architecture

Similar to 5G, 6G system will also have split of RAN and CN. CN is air interface agnostic, RAN is responsible for air interface related functions. RAN and CN are holding different responsibilities to serve a variety of services, it will need to ease the real deployment of the RAN and CN independently therefore need to allow for the RAN and CN to evolve independently. RAN and CN is connected by means of 6GNG interface.

RAN provide user plane and control plane protocol towards the UE.

Mobility of UE within RAN is fully controlled by the RAN with RAN internal interfaces and air interface with UE.

RAN architecture holds logical separation of signalling and data transport networks.

All RAN internal interfaces shall be fully open for multivendor inter-operability.

The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP. (RP-252881)

Proposal 2: it is proposed to capture the principles and requirement in the TR38.xxx (TR for RAN3)

# 3. Conclusion

In this contribution we discussed some principle and requirements for architecture perspective.

Proposal 1: 6GR and 6G RAN have been used in WG level SID “Study on 6G Radio”. If we will have split architecture, we may call the entities in the 6G RAN 6G-NB, 6G-NB-CU, 6G-NB-DU, 6G-NB-UP etc.. So far we call the CN for 6G as 6GC.

Proposal 2: it is proposed to capture the principles and requirement in the TR38.xxx (TR for RAN3)

“6G system has split of RAN and CN. CN is air interface agnostic, RAN is responsible for air interface related functions. RAN and CN are holding different responsibilities to serve a variety of services, it will need to ease the real deployment of the RAN and CN independently therefore need to allow for the RAN and CN to evolve independently. RAN and CN is connected by means of 6GNG interface.

RAN provide user plane and control plane protocol towards the UE.

Mobility of UE within RAN is fully controlled by the RAN with RAN internal interfaces and air interface with UE.

RAN architecture holds logical separation of signalling and data transport networks.

All RAN internal interfaces shall be fully open for multivendor inter-operability.

The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP. (RP-252881)”

# References

[2] RP-252882, TR38.914v0.2.0

[2] RP-252912, SID: Study on 6G Radio