3GPP TSG-RAN WG3 #129bis	R3-256786

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		14.2

Title:		Discussion on Topology 2 of Ambient IoT

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The RAN3 objectives for A-IoT Topology 2 enhancement mentioned in WID [1] that

RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

This contribution will discuss the issue related to these objectives.

# 2. Discussion

2.1 The baseline of A-IoT Topology 2 discussion

The system architecture agreed in the study phase for topology 2 is:

### 6.4.2 Support of Topology 2 Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB. The following definitions apply: A-IoT-enabled gNB: A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface. A-IoT-enabled UE: A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface. Figure 6.4.2-1 Logical system architecture for topology 2 NOTE 1: Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed. NOTE 2: The A-IoT CN could include AMF, UPF and A-IoTF, which is up to SA2 decision. NOTE 3: Details of the radio resource management performed by the A-IoT enabled gNB are subject to RAN1 and RAN2 mechanisms. For Topology 2, architecture and protocol aspects of split RAN architecture are not studied. In Topology 2, XX interface is NG interface.

The terminology of “A-IoT-enabled gNB” and “A-IoT-enabled UE”, and the system architecture should be a baseline for the Rel-20 AIoT topology 2 discussion.

Proposal 1: The terminology of “A-IoT-enabled gNB” and “A-IoT-enabled UE”, and the system architecture should be a baseline for Rel-20 AIoT topology 2 discussion.

Since the WI only includes the RRC-based solution, and excludes the NAS-based solution and UP-based solution, the protocol stack for the RRC-based solution agreed in TR 38.769 should be a baseline for Rel-20 AIoT topology 2 discussion.

##### 6.4.2.1.1 Solution 1: RRC based solution Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa. Figure 6.4.2.1.1-1: RRC based solution of Topology 2 In this solution, XX interface is NG-C interface. NOTE: Whether "XXAP" is realised by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision.

Proposal 2: The protocol stack for RRC-based solution agreed in TR 38.769 should be a baseline for Rel-20 AIoT topology 2 discussion.

In addition, the inventory and command procedures for the RRC-based solution of topology 2 are also captured in the 38.769 that:

#### 6.5.3.1 Candidate procedures for A-IoT Inventory for Topology 2 ##### 6.5.3.1.1 RRC based solution Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution #### 6.5.3.2 Candidate procedures for A-IoT Command for Topology 2 ##### 6.5.3.2.1 RRC based solution Figure 6.5.3.2-1: Message flow for A-IoT Command in Topology 2 (RRC-based solution)

And the inventory and command procedures above should be the baseline for the topology 2 discussion in Rel-20.

Proposal 3: The inventory and command procedure for RRC-based solution agreed in TR 38.769 should be a baseline for Rel-20 AIoT topology 2 discussion.

2.2 UE Reader authorization

During the Rel-19 study phase, progress has been made in both RAN (as documented in TR 38.769 Cluster 6.3.6) and SA (TR 23.700-13 Cluster 8.1.3.4) that:

### 6.3.6 Topology 2 aspects on the interface between UE reader and RAN It is assumed that the intermediate UE authorization is performed by upper layers, according to [151] and [152]. It is not studied, from RAN2 signalling perspective, the scenario to support bistatic mode of operation (e.g., D2T2-A1).

#### 8.1.3.4 Interim Conclusion for authorization The following interim conclusion are agreed for the UE Reader authorization in Topology 2: - The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader. - The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader. Editor's note: How to authorize the UE Reader will be determined later.

Based on our understanding, the procedure is as follows: A UE reports its AIoT capability to the AMF during the registration procedure. The AMF then checks the UE's subscription data and its AIoT capability (which may involve coordination between the AMF and UDM) to authorize it as a UE reader. Subsequently, the AMF sends the authorization result to the RAN node. This signaling could be implemented using the existing UE CONTEXT REQUEST message.

Observation 1: The process of AIoT capability reporting and UE authorization has no impact on RAN3 specifications. However, the AMF sending the authorization result to the RAN does have an impact on the NGAP protocol.

Proposal 4: The AMF should include the UE's authorization result in the INITIAL CONTEXT SETUP REQUEST message sent to the RAN.

2.3 The definition of UE reader ID

In Topology 1, as a gNB serves one or multiple readers and the gNB ID cannot uniquely identify a RAN reader, the RAN reader ID is defined as “Global gNB ID + Reader Index”. The index is assigned by the AIoT RAN, and the AIoTF becomes aware of the RAN reader ID via OAM.

For Topology 2, however, a single UE is typically capable of supporting only one reader. Therefore, RAN3 needs to discuss whether the UE reader ID should follow the same format as Topology 1 (“Global gNB ID + Reader Index”, where the Global gNB ID is that of the UE's serving gNB) or simply reuse the legacy UE ID. The legacy UE ID refers to the RAN UE NGAP ID over the NG interface and the C-RNTI over the Uu interface.

From our perspective, reusing the legacy UE ID as the reader ID is sufficient for Topology 2. Since one UE corresponds to one reader, the legacy UE ID can uniquely identify the reader. Moreover, considering the mobility of UE readers, this approach avoids the need to update the reader ID during inter-gNB handovers, which would be necessary if a “Global gNB ID + Reader Index” format is used.

Proposal 5: A UE shall support only one reader.

Proposal 7: The legacy UE ID (i.e., the RAN UE NGAP ID on the NG interface and the C-RNTI on the Uu interface) can uniquely identify a UE reader.

2.4 UE reader selection

Similar to Topology 1, the AIoT CN may send the UE reader(s) information or a requested service area to the AIoT-enabled gNB.

If the AIoT CN provides a single UE reader ID, as captured in 23.700-13, the AIoT-enabled RAN shall use the indicated UE as the UE reader for the AIoT service.

If the AIoT CN provides a list of UE reader IDs, the AIoT-enabled RAN has two options: the first option is that The AIoT RAN uses all the indicated UEs to perform the AIoT operation while the second option is that The AIoT RAN may perform a down-selection from the provided UE Reader list.

Proposal 8: If the AIoT CN send a single UE reader ID to the AIoT enabled RAN, the AIoT enabled RAN should consider the the indicated UE as the UE readers to provide the AIoT service.

Proposal 9: If the AIoT CN provides a list of UE reader IDs, the AIoT-enabled RAN has two options:

Option 1: The AIoT RAN uses all the indicated UEs to perform the AIoT operation.

Option 2: The AIoT RAN may perform a down-selection from the provided list.

Additionally, if only the target AIoT area is provided without any UE reader IDs, RAN3 can follow the principle from Topology 1. Specifically, the AIoT RAN may select authorized UE(s) within the target area to act as UE readers. However, whether the RAN should select all authorized UEs or a subset of them based on implementation requires further discussion.

Proposal 10: If the target AIoT area is provided, the AIoT RAN may select authorized UE(s) within that area as UE reader(s) to perform the AIoT operation. (Note: Whether the RAN should select all authorized UEs or a subset based on implementation requires further discussion.).

Finally, if the AIoT CN provides neither a target AIoT area nor UE reader(s), the AIoT RAN may select all authorized UEs it serves (consistent with Topology 1) or select UE readers based on its own implementation.

Proposal 11: If neither the target AIoT area nor UE reader(s) are provided, the AIoT RAN may select all authorized UEs it serves or select UE readers based on implementation.

2.5 UE reader mobility

The UE reader mobility is another aspect need to discussed in topology 2 as mentioned in WID [1] that:

RAN2 - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. RAN3 - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

From the WID [1], we can deduce that the UE reader should be able to perform the configured AIoT operation after the UE reader handover to another cell or gNB.

However, the A-IoT services should be highly location-dependent. For instance, a service might be configured to a specific factory area or storage room. In such cases, if a UE Reader handover out of the AIoT service area, the UE Reader is expected to stop the AIoT operation.

Proposal 12: If a UE Reader handover out of the intended service area, the UE Reader is expected to stop the AIoT operation.

The AIoT RAN will allocate resource for UE reader to perform AIoT operation, and the AIoT resource is only valid within the serving cell. So, when the UE handover to another cells or gNB, it will trigger the new serving cell or gNB to allocate the AIoT radio resource for the UE reader. Since the inter-gNB resource coordination is not specified, some assistance information for the AIoT resource allocation should be discussed in case of inter-gNB UE handover.

Proposal 13: Some assistance information for the AIoT resource allocation over Xn-AP should be discussed in case of inter-gNB handover.

# 3. Conclusion

Proposal 1: The terminology of “A-IoT-enabled gNB” and “A-IoT-enabled UE”, and the system architecture should be a baseline for Rel-20 AIoT topology 2 discussion.

Proposal 2: The protocol stack for RRC-based solution agreed in TR 38.769 should be a baseline for Rel-20 AIoT topology 2 discussion.

Proposal 3: The inventory and command procedure for RRC-based solution agreed in TR 38.769 should be a baseline for Rel-20 AIoT topology 2 discussion.

Observation 1: The process of AIoT capability reporting and UE authorization has no impact on RAN3 specifications. However, the AMF sending the authorization result to the RAN does have an impact on the NGAP protocol.

Proposal 4: The AMF should include the UE's authorization result in the INITIAL CONTEXT SETUP REQUEST message sent to the RAN.

Proposal 5: A UE shall support only one reader.

Proposal 7: The legacy UE ID (i.e., the RAN UE NGAP ID on the NG interface and the C-RNTI on the Uu interface) can uniquely identify a UE reader.

Proposal 8: If the AIoT CN send a single UE reader ID to the AIoT enabled RAN, the AIoT enabled RAN should consider the the indicated UE as the UE readers to provide the AIoT service.

Proposal 9: If the AIoT CN provides a list of UE reader IDs, the AIoT-enabled RAN has two options:

Option 1: The AIoT RAN uses all the indicated UEs to perform the AIoT operation.

Option 2: The AIoT RAN may perform a down-selection from the provided list.

Proposal 10: If the target AIoT area is provided, the AIoT RAN may select authorized UE(s) within that area as UE reader(s) to perform the AIoT operation. (Note: Whether the RAN should select all authorized UEs or a subset based on implementation requires further discussion.).

Proposal 11: If neither the target AIoT area nor UE reader(s) are provided, the AIoT RAN may select all authorized UEs it serves or select UE readers based on implementation.

Proposal 12: If a UE Reader handover out of the intended service area, the UE Reader is expected to stop the AIoT operation.

Proposal 13: Some assistance information for the AIoT resource allocation over Xn-AP should be discussed in case of inter-gNB handover.

# References

[1] RP-252894, Solutions for Ambient IoT (Internet of Things) in NR