3GPP TSG-RAN WG3 Meeting #129bis	R3-256787

Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | x | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NG interface suspend and resume indication for hard feeder link switchover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-19, the NR NTN regenerative payload is defined as terminating the NG interface toward the 5GC via the feeder link. Therefore, the hard feeder link switchover between gNBs and NTN gateways may lead to an interruption of the NG interface. The NGAP interruption can be known by the onboard gNB, but AMF cannot be aware of it if there is no enhancement, which will lead to the packet loss from AMF to the onboard gNB. Many companies want to introduce a suspend indication when the onboard gNB detects that the hard feeder link switchover is about to happen and sends the suspend indication to the AMF. AMF will suspend the DL data transmission until it receives a resume indication from the gNB. We believe this is an effective way to solve the DL packet loss caused by the hard feeder link switchover. However, objecting companies think the AMF can get some information about the hard feeder link switchover from the OAM, and the hard feeder link switchover is predictable. In TS 38.300 B.4 specifies that the OAM will provide “The time window of the successive switch overs (feeder link, service link) and the identifier and time window of all serving satellites (resp. HAPS) and NTN Gateways” to a given Quasi-Earth-fixed NTN payload and provide “Schedule of successive serving NTN Gateways/gNBs and Schedule of successive switch overs (feeder link, service link)” to a given earth-moving NTN payload. There is no clue indicating that the OAM can provide the pause/resume indication for the NGAP interruption or some interruption time window caused by the hardfeer link switchover. We know the above information can be helpful to predict the hard feeder link switchover, but the interruption of the hard feeder link switchover cannot be accurately predicted by the preconfiguration. Furthermore, objecting companies also propose that a suspension indication from AMF to UPF per UE per PDU session will be initiated when the AMF receives the suspend indication from the gNB. We think this procedure cannot be avoided, even for the OAM solution. Overall, we still support the suspend/resume indication over NGAP to solve the NGAP interruption caused by hard feeder link switchover. And RAN3 should not consider the suspend/resume indication in the gNB moving out of the AMF serving area scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding stage 2 description about the suspend/resume indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 2 description for the suspend/resume indicaiton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.4.2, 16.14.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.413 CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

End of Change