3GPP TSG-RAN WG3 Meeting #129bis	R3-256790

Prague, Czech Republic, 13 – 17 October 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1339 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-IoT device identifier privacy protection impact on RAN3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.4.2 of TS 33.369, there is the following statement on the AIoT device identifier protection for inventory with filtering information that: “For the protection of AIoT device permanent ID during the inventory procedure described in clause 5.2.2, the following change shall apply: - In step 4, the AIoT device determines it needs to reply to the NG-RAN based on the received filtering information. - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, filtering information is used as a device identification information if the AIOTF received it in step 0. NOTE 2: The AIOTF identifies the AIoT device by checking the received RESAIoT. Therefore, device identification information is not needed in the D2R and Inventory Report message.” In the inventory procedure with group devices, if the device ID privacy protection is active, the A-IoT device should not report its permanent ID to the RAN Reader, and the RAN Reader should also not include the device's permanent IDs in the inventory report. In addition, there is the following statement on the A-IoT Device identifier protection with Temp ID update during Individual inventory that “For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1. - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile.” In the inventory procedure with a single device, if the device ID privacy protection is active, a temporary device ID will be sent from AIoT CN to the AIoT device. The AIoT device will generate a temporary ID to determine whether it needs to reply to the paging message. If it needs to reply, the AIoT device will send a D2R message to the RAN reader, but not include the device's permanent ID. And the inventory report will not include the device's permanent ID. In addition, as TS 33.369 specified that “Depending on the situation and deployment scenario, the network operator can choose which paging procedure to use.”, whether to use the device ID privacy protection depending on the network operator. From the RAN3 point of view, the indication about whether the privacy protection is active or not should be indicated in the INVENTORY REQUEST message from the A-IoT CN to the RAN Reader. And the AIoT NAS PUD (i.e., the permanent A-IoT device ID) in the inventory report is not mandatory |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding an Identifier protection IE in the Single Device IE of A-IoT Device Identification Requested IE in INVENTORY REQUEST message. Adding an Identifier protection IE in the Group Devices IE of A-IoT Device Identification Requested IE in INVENTORY REQUEST message. Adding some text description about Identifier protection IE in the Inventory Request Successful Operation section. Change the mandatary A-IoT NAS PDU IE in the INVENTORY REPORT message to an optional IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is an misalignment between SA3 and RAN3 in terms of A-IoT device ID privacy protection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.20.1.2, 9.3.6.1, 9.3.6.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 38.410 CR 0047 TS 38.423 CR 1069 TS 38.473 CR 1070 TS 38.401 CR 0267 TS 38.420 CR 0038 TS 38.470 CR 0120 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 8.20.1	Inventory Request

#### 8.20.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform A-IoT paging over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.20.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

Upon receiving the Inventory Assistance Information IE contained in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

Interaction with Inventory Report procedure

When starting the requested inventory, the NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY REPORT message(s) as received from the INVENTORY REQUEST message

If the Follow on Command Indication IE set to "true" is included in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall consider that there will be command to be transmitted to the device(s), and shall allocate and include a RAN A-IoT Device NGAP ID IE in each Device Report Item IE within the Inventory Report Transfer IE in the INVENTORY REPORT message(s).

Upon receiving the Identifier protection IE contained in the Inventory Request Transfer IE of the INVENTORY REQUEST message and Single Device IE or Group Device IE is included in the A-IoT Device Identification Requested IE, the NG-RAN node shall consider the requested A-IoT device will not include the device identification information in the D2R message, and the INVENTORY REPORT message will not include the A-IoT NAS PDU IE.

Next Change

### 9.3.6	AIOTF Related IEs

#### 9.3.6.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | M |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| Identifier protection | O |  | ENUMERATED (true, ...) | Whether to apply AIoT Device identifier protection with Temp ID update during Individual inventory defined in TS 33.369 [z] if Single Device IE is included in the A-IoT Device Identification Requested IE. Whether to apply AIoT Device identifier protection for inventory with filtering information defined in TS 33.369 [z] if Group Devices IE is included in the A-IoT Device Identification Requested IE. | YES | reject |

Next Change

#### 9.3.6.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Reader Report List |  | 1 |  |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | O |  | OCTET STRING | This IE shall be absent if the Identifier protection IE is present in the Inventory Request Transfer IE of INVENTORY REQUEST message, as defined in TS 33.369. |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |
| Inventory Complete Indication | O |  | ENUMERATED (true, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-FollowonCommandIndication,

id-AIoT-IdentifierProtection,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-NASPDU,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

Omit unrelated part

AIoTAreaID ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

nID					NID							OPTIONAL,

aIoTAreaCode		OCTET STRING (SIZE (3)),

iE-Extensions		ProtocolExtensionContainer { { AIoTAreaID-ExtIEs} } OPTIONAL,

...

}

AIoTAreaID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-CorrelationIdentifier ::= OCTET STRING-- needs to be replaced by a constrained type definition --

AIoT-FollowonCommandIndication ::= ENUMERATED {true, ...}

AIoT-IdentifierProtection ::= ENUMERATED {true, ...}

RAN-AIOT-Device-NGAP-ID ::= INTEGER (0..4294967295)

Omit unrelated part

AIoT-DeviceReportList ::= SEQUENCE (SIZE(1..maxnoofDevices)) OF AIoT-DeviceReportItem

AIoT-DeviceReportItem ::= SEQUENCE {

aIoT-NASPDU					AIoT-NASPDU												OPTIONAL,

rAN-AIOT-Device-NGAP-ID		RAN-AIOT-Device-NGAP-ID									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-DeviceReportItem-ExtIEs} }	OPTIONAL,

...

}

AIoT-DeviceReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Omit unrelated part

InventoryRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { InventoryRequestTransferIEs} },

...

}

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	mandatory	}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		}|

{ ID id-AIoT-IdentifierProtection			CRITICALITY reject	TYPE AIoT- IdentifierProtection					PRESENCE	optional		},

...

}

Next Change

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Omit unrelated part

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

Omit unrelated part

id-InventoryRequestTransfer							ProtocolIE-ID ::= 500 --to be allocated

id-InventoryResponseTransfer						ProtocolIE-ID ::= 501 --to be allocated

id-InventoryFailureTransfer							ProtocolIE-ID ::= 502 --to be allocated

id-InventoryReportTransfer							ProtocolIE-ID ::= 503 --to be allocated

id-CommandRequestTransfer							ProtocolIE-ID ::= 504 --to be allocated

id-CommandResponseTransfer							ProtocolIE-ID ::= 505 --to be allocated

id-CommandFailureTransfer							ProtocolIE-ID ::= 506 --to be allocated

id-AIOTSessionReleaseCommandTransfer					ProtocolIE-ID ::= 507 --to be allocated

id-AIOTSessionReleaseCompleteTransfer					ProtocolIE-ID ::= 508 --to be allocated

id-AIOTSessionReleaseRequestTransfer				ProtocolIE-ID ::= 509 --to be allocated

id-AIoT-CorrelationIdentifier							ProtocolIE-ID ::= 510 --to be allocated

id-AIOTFIdentifier									ProtocolIE-ID ::= 511 --to be allocated

id-AIoT-DeviceIdentificationRequested					ProtocolIE-ID ::= 512 --to be allocated

id-AIoT-RequestedServiceAreaInformation					ProtocolIE-ID ::= 513 --to be allocated

id-AIoT-InventoryAssistanceInformation					ProtocolIE-ID ::= 514 --to be allocated

id-AIoT-FollowonCommandIndication						ProtocolIE-ID ::= 515 --to be allocated

id-RAN-AIOT-Device-NGAP-ID								ProtocolIE-ID ::= 516 --to be allocated

id-AIoT-CommandAssistanceInformation					ProtocolIE-ID ::= 517 --to be allocated

id-AIoT-NASPDU										ProtocolIE-ID ::= 518 --to be allocated

id-AIoT-Support											ProtocolIE-ID ::= 519 --to be allocated

id-AIOTFName											ProtocolIE-ID ::= 520 --to be allocated

id-IdentifierProtection									ProtocolIE-ID ::= 5xx --to be allocated

End Change