3GPP TSG-RAN WG3 Meeting #129 bis	R3-256799

# Prague, Czech Republic, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1538 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE performance for AI/ML |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN3#128 meeting, Reply LS on per-UE UE performance metrics (R3-253816) was sent to SA5 for clarifications about the granularity of Average Packet Delay UL/DL and Average Packet Loss UL/DL. In this meeting, SA5 replied the LS in R3-256518. Therefore, the semantics description of the some UE performance IEs need to be revised according to SA5 reply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following is the summary of changes: Add clarificaiton in the semantics description of the following IEs: - Average Packet Loss DL - Average Packet Delay UL - Average Packet Delay DL Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) as the change affects only the definitions of packet loss and packet delay. The CR is backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE performance may not be derived correctly in gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.179, 9.2.3.187 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of 1st change>

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.1. |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.2. |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | INTEGER (0..1000000, ...) | Corresponds to DL PDCP SDU Drop Rate as specified in TS 28.558 [58] clause 6.3.1.6. |

<Start of 2nd change>

#### 9.2.3.187	Average Packet Delay

This IE indicates the RAN part of the average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Corresponds to the Average Packet Delay as specified in TS 28.558 [58], clause 6.3.1.1. Per-UE metric is derived by aggregating per-DRB per-UE measurements. |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Corresponds to the Average Packet Delay as specified in TS 28.558 [58], clause 6.3.1.1. Per-UE metric is derived by aggregating per-DRB per-UE measurements. |

<End of change>