3GPP TSG-RAN WG3 Meeting #129bis	R3-256814

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	12.2.1

Source:	InterDigital

Title:	Multi hop UE trajectory Feedback

Document for:	Discussion and Agreement

# 1	Introduction

For Release 20 RAN plenary approved a AI/ML for NG-RAN study item. The objectives are:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

    - multi-hop UE trajectory across gNBs;

    - intra-CU LTM;

    - handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

# 2. 	Discussion

In release 18 we decided for the sake of time to restrict UE trajectory prediction to go across 1 gNB due to the complexity of getting trajectory feedback back from more distant gNBs through other intermediary gNBs.

In solving the problem, we need to look at the cases where there are AI/ML models in the neighbour gNBs and cases where there is not.

For the sake of discussion let’s look at the following example.

- There are 3 gNBs, gNB A, gNB B, and gNB C.

- gNB A and gNB B both have AI/ML models producing UE trajectory predictions, gNB C may or may not have AI/ML models producing UE trajectory predictions.

- gNB A’s model predicts that a particular UE will traverse cells A1 and A2 in gNB A, then cell B1 in gNB B, and then cells C1 and C2 in gNB C.

- When the UE traverses into cell B1 the gNB B model predicts the UE will traverse cells C1, C2, C3 and C4 in gNB C.

- When looking for feedback for its prediction gNB A would trigger during the handover a request to gNB B for a value of 3 in the number of visited cells in the UE Trajectory Collection Configuration IE

- When looking for feedback for its prediction gNB B would trigger during the handover request to gNB C for a value of 4 in the number of visited cells in the UE Trajectory Collection Configuration IE

- Additionally, gNB B would trigger a data collection for UE trajectory to fulfil gNB-A’s original request with a value of 2 cells and a reduced reporting time based on the time the UE spent in gNB-B.

If gNB B does not have a UE trajectory prediction model, it would set up only one trajectory collection to gNB C with a Number of visited cells value of 2 so it could fulfil gNB A’s request.

Even if you chain 3 or 4 gNBs in a trajectory prediction the complexity at each gNB would be the same it would have a trajectory collection going backward to the originator and a trajectory collection going forward linked to the backward collection.

This way of linking forward and backward trajectory prediction could be done with no or little stage 3 impact, but you could have optimizations these can be further discussed, now or in the work item.

As an optimization gNB B could in the message that sets up the trajectory collection to gNB C could pass along the already visited cells so that gNB C would only have to append its cells to the message back to gNB B. Or gNB B could build it from the stored results while the UE was in gNB B and append the gNB C results.

Another optimization could be to optionally include a second UE Trajectory Collection Configuration in the configuration in the handover in the same measurement you can get two reports one for gNB A’s model and one for gNB B’s model.

Proposal 1: The TP in the annex should be agreed as a baseline of the description of one solution for trajectory collection over multiple gNBs:

# Annex

## Hop-By-Hop solution

When gNB-A sets up trajectory feedback over gNB-B and then while the trajectory feedback criteria has not yet been fulfilled the gNB-B wants to handover the UE to a cell in gNB-C The gNB-B setups a trajectory collection to gNB-C that will fulfil the rest of gNB-As request. When gNB C fulfils the request then gNB B can append it to what it collected while the UE traversed through gNB B, or could just relay it along if the UE trajectory is kept along the way. . This method would allow gNB B to fulfil gNB A’s request and would still be able to ask gNB C for an additional trajectory collection to fulfil feedback for its model if it has one. This method would also have very little stage 3 impact though some optimizations and stage 2 impact might be considered during the normative phase.

X.Y-1 Hop by Hop