3GPP TSG-RAN WG3 Meeting #129-bis	R3-256815

Prague, Czech Republic, 13th - 17th Oct, 2025	Revison of R3-255652

Title: 	Further discussion on paging capability loss issue

Source: 	Huawei, China Unicom, China Telecom

Agenda item:	9.2.14

Document Type:	Discussion

# 1. Introduction

The last SA2-169 meeting sent the subsequent LS on paging capability loss as follows [1].

1. Overall Description: SA2 thanks RAN2 for this LS and their earlier LS in R2-2501388=S2-2504576. In the latest LS, RAN 2 state that: In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”. The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN. It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution. Hence this RAN2 solution is highly undesirable compared to what was proposed in RAN2’s earlier LS in R2-2501388=S2-2504576. SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF. 2. Actions: To RAN3, RAN2 SA2 asks RAN3 and RAN2 to provide views on the potential solution described above.

The above SA2 LS was a response to RAN2 LS on the UE radio capability for paging information in [2], especially related to LP-WUS.

1. Overall Description: RAN2 further discussed the issue of UE radio paging capability loss mentioned in the LS S2-2412876 and acknowledged that based on the legacy signalling, a gNB implementing Release X may not be able to forward the radio paging capabilities for a feature introduced after Release X, which results in UE missing the paging message. For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. - We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”. 2. Actions: To SA2, RAN3, CT1 RAN2 respectfully asks SA2, RAN3, CT1 to take the updated conclusions and agreements above into account.

This contribution provides our further views on the paging loss issue and reply to SA2.

# 2. Discussion

## 2.1 Recap the progress in RAN3-129 meeting

The issue from SA2 LS is mainly related to the different release NG-RAN node deployment when supporting different UE Radio Paging Capabilities for pre-19 features, e.g., PEI, (e)RedCap etc. The previous RAN3-129 meeting already discussed the issue, and had the following agreement and listed open issues as follows, where it was agreed that the NG-RAN node could check the missing paging radio capability in the initial context setup procedure starting from Rel17.

The solution for missing UE capability for paging features will be introduced from rel 17 for initial context setup. The following may be captured in the reply LS sent in the next meeting: RAN3 note that the possibility of paging loss due to paging capability mismatch may still exist in case that paging message is sent to the gNB before paging capability mismatch resolution. - RAN3 will agree corresponding CRs and reply LS in the next meeting based on the above, considering following aspects: - Whether the extension of the solution for Path Switch Request procedure and Handover Resource Allocation procedure is needed or not. - Whether the optimization for the solution proposed by Vodafone in R3-255529/R3-255530/R3-255531 is supported or not.

Also, this agreement is aligned with the solution mentioned in [1] as excerpted below, which requires the NG-RAN node to check whether the UE Radio Capability for Paging (URCP) contains the full paging capability information according to the received UE radio capabilities. If not, the NG-RAN uploads the updated URCP to the AMF.

SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF.

## 2.2 Further responses to SA2 solution

Another somehow relevant thing is the per-RAT URCP information which was discussed at the previous RAN3-129 meeting. It was agreed that when the NG-RAN checks the URCP does not contain paging information on all NG-RAN RATs that the UE supports, it should upload all “UE Radio Capability for Paging” as a single Information Element, instead of just paging information on the missing NG-RAN RATs, in the Initial UE context request procedure.

| R3-255712 | Correction on UE radio capability for paging information (CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson) | CR1313r, TS 38.413 v16.16.0, Rel-16, Cat. F moved from 9.2 Rev in R3-255820 Change source to TSG to “R3” Rev in R3-255885 Agreed unseen |
| --- | --- | --- |
| R3-255713 | Correction on UE radio capability for paging information (CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson) | CR1314r, TS 38.413 v17.12.0, Rel-17, Cat. A moved from 9.2 Rev in R3-255821 Change source to TSG to “R3” Add “and” before “use it for subsequent RAN paging” Rev in R3-255886 Agreed unseen |
| R3-255714 | Correction on UE radio capability for paging information (CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson) | CR1315r, TS 38.413 v18.6.0, Rel-18, Cat. A moved from 9.2 Rev in R3-255822 Change source to TSG to “R3” Add “and” before “use it for subsequent RAN paging” Rev in R3-255887 Agreed unseen |

Hence these two implementations can be performed by the NG-RAN node together – not only checking the missing NG-RAN RAT URCP, but also checking the missing paging related features.

- Observation 1: in the Initial UE context procedure, the NG-RAN can check the missing paging related features that both the RAN-node and UE support, upon receiving the URCP based on the received UE radio capabilities. This can be implemented together with the checking of the missing NG-RAN RAT URCP.

There are also some discussions on the handover procedures. We consider this is needed also, so that the NG-RAN node can take an aligned checking operation also for connected mobility in addition to the initial access procedure, to reduce the risk of the missing paging radio capability features. Note that the URCP is included in the Core Network Assistance Information for RRC INACTIVE IE (CNAI). Hence the target/new NG-RAN node can also check the URCP in the handover request message or in the path switch request acknowledge message.

In sum, the interactions with the UE Radio Capability Info Indication procedure can be added, when the received URCP does not contain all the paging related features for the following procedures:

- Initial Context Setup,

- Handover Resource Allocation

- Path Switch Request

RAN3 to agree to add interactions with the UE Radio Capability Info Indication procedure, when the received URCP does not contain all the paging related features for the following procedures:

- Initial Context Setup (for the top level URCP)

- Handover Resource Allocation (for the URCP contained in the CNAI)

- Path Switch Request (for the URCP contained in the CNAI)

- RAN3 should discuss from which release the changes begin. Given that the R17 is agreed for the initial context setup message, then the same should apply for all three messages above.

RAN3 to agree R17 onwards CRs of checking missing paging related features.

- Last, it should also be noted that the above solution is only applicable to RRC connected UEs in case of initial access, handover, reestablishment or context relocation. While for idle UE, the paging loss cannot be avoided, when the idle UE moves from the old release gNB to a new release gNB. Hence it is better to reply this point to the SA2.

RAN3 to reply that the paging loss cannot be avoided, when the idle UE moves from the old release gNB to a new release gNB.

- Based on the above discussion, RAN3 should reply to SA2 with the above proposals so that SA2 can update the clause 5.4.4.3 in TS 23.501.

RAN3 to send reply to SA2 a) inform RAN3 agreements, b) ask to update the TS 23.501 clause 5.4.4.3 if needed.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation 1: in the Initial UE context procedure, the NG-RAN can check the missing paging related features that both the RAN-node and UE support, upon receiving the URCP based on the received UE radio capabilities. This can be implemented together with the checking of the missing NG-RAN RAT URCP.

- RAN3 to agree to add interactions with the UE Radio Capability Info Indication procedure, when the received URCP does not contain all the paging related features for the following procedures:

- Initial Context Setup (for the top level URCP)

- Handover Resource Allocation (for the URCP contained in the CNAI)

- Path Switch Request (for the URCP contained in the CNAI)

RAN3 to agree R17 onwards CRs of checking missing paging related features.

RAN3 to reply that the paging loss cannot be avoided, when the idle UE moves from the old release gNB to a new release gNB.

RAN3 to send reply to SA2 a) inform RAN3 agreements, b) ask to update the TS 23.501 clause 5.4.4.3 if needed.

- Companion CRs are provided in [4-9], and the draft LS is provided in the Annex.

# 4. Reference

- S2-2506087 Response to Reply LS on paging capability loss issue, SA2

- R2-2503172 Reply LS on paging capability loss issue, RAN2

- S2-2506082 Reply LS on Handling of UE Radio Capability for Paging, SA2

- R3-256816	Avoiding paging capability loss for CN paging	Huawei, China Unicom, China Telecom

- R3-256817	Avoiding paging capability loss for CN paging	Huawei, China Unicom, China Telecom

- R3-256818	Avoiding paging capability loss for CN paging	Huawei, China Unicom, China Telecom

- R3-256819	Avoiding paging capability loss for CN paging	Huawei, China Unicom, China Telecom

- R3-256820	Avoiding paging capability loss for CN paging	Huawei, China Unicom, China Telecom

- R3-256821	Avoiding paging capability loss for CN paging	Huawei, China Unicom, China Telecom

# 5. Annex – Reply LS to SA2

3GPP TSG-RAN WG3 Meeting #129-bis <TDdoc#> Prague, Czech Republic, 13th - 17th Oct, 2025 Title: [draft] Reply LS on paging capability loss issue Response to: LS S2-2506087 on paging capability loss issue Release: Rel-19 Work Item: NR_LPWUS-Core, TEI19 Source: Huawei [to be RAN3] To: SA2 Cc: RAN2, CT1 Contact person: Feng Han Hanfeng3@huawei.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: R3-25xxxx # 1 Overall description RAN3 thanks SA2 and RAN2 for their subsequent LS on the paging capability loss issue. Regarding the proposed solution mentioned by SA2, RAN3 would like to provide following further conclusions. RAN3 agreed that the NG-RAN can check the missing paging related features and report all the UE Radio Capability for Paging that both the RAN-node and UE support, after checking the received UE radio capabilities, for the following procedures. - Initial Context Setup - Handover Resource Allocation - Path Switch Request RAN3 would like to highlight that for idle UE mobility, the paging loss cannot be avoided, when the idle UE moves from the old pre-19 release gNB to a new release gNB. The agreed CRs are attached for reference. # 2 Actions To SA2: ACTION: RAN3 respectfully asks to take above information into account, and provide feedback if any. # 3 Dates of next RAN3 meetings Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/ RAN3#130 2025-11-17 - 2025-11-21 Dallas, US RAN3#131 2026-02-09 – 2026-02-13 Stor-Göteborg, SE

# 6. Annex – from TS 38.300 v18.6.0

#### –	UERadioPagingInformation

This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs.

Direction: gNB to/ from 5GC and gNB to/from gNB

UERadioPagingInformation message

-- ASN1START

-- TAG-UE-RADIO-PAGING-INFORMATION-START

UERadioPagingInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioPagingInformation            UERadioPagingInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v15e0-IEs                      OPTIONAL

}

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v1700-IEs          OPTIONAL

}

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)     OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1800-IEs                   OPTIONAL

}

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1840-IEs                   OPTIONAL

}

UERadioPagingInformation-v1840-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                              OPTIONAL

}