3GPP TSG-RAN WG3 Meeting #129-bis	R3-256905

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.2.2

Source: 	Lenovo

Title: 	6G RAN Logical Architecture

Document for:	Discussion and Approval

1. Introduction

A new study on 6G radio was approved at the RAN#108 meeting and then revised at the RAN#109 meeting with the following objectives [1].

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML - Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN NOTE: Coordination with TSG-SA and SA WGs will be done, as necessary. 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026. TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

In this contribution, we discuss the 6G RAN logical architecture.

2. Discussion

Functions hold by 6G RAN are related to what architecture/protocol should be supported by the 6G RAN architecture. To initiate the discussions on the number of functionalities to be supported by the 6G, it would be useful to take the 5G gNB functions as a reference at first stage.

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity;

-	Tight interworking between NR and E-UTRA;

-	Maintain security and radio configuration for User Plane CIoT 5GS Optimisation.

Among the functions, the DC was discussed without agreements. RAN plenary also agreed to make a decision on DC by September 2026. Therefore, these functions can be added later to avoid occupying too much time of technical study. For the tight interworking between different RATs and the UP CIoT optimization, there is no need to cover them at the very beginning. For NAS messages distribution, there may be some enhancements e.g., to support distributed NAS, which depends on SA2’s discussion.

Proposal 1: NR functions are considered as a reference to 6GR, while the DC, tight interworking between 6GR and other RAT (e.g. NR or E-UTRA), and UP CIoT optimization are not considered at the beginning.

Motivated by the relatively recent uptake by many operators of 5GC, it is reasonable to assume that the 6GC is an evolution of the 5GC to reduce the investments. In line with the assumption, the interfaces related to the 6G RAN can be the evolution of the interfaces related to the 5G RAN, which have been successfully deployed. For example,

-	The 6G RAN is connected to the 6GC via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface.

-	The 6G RAN is connected to the 6GC via an NG-U like interface for user plane, e.g., by means of the evolution of NG-U interface.

-	The different 6G RAN nodes (e.g., 6G BS) are interconnected with each other via an Xn like interface, e.g., by means of the evolution of Xn interface.

The 6G network are expected to include new service types beyond traditional communication, such as data collection, sensing and AI. The characteristics of those new services are quite different from legacy functions. Service-based interface (SBI) between the RAN and CN for the new functions supporting the new service types, which can avoid too many P2P interface definitions and unnecessary transparent forwarding among network entities, thus enable efficient coordination between the RAN and CN. In addition, the RAN can more directly expose information to the CN.

Figure 1 shows an example of the overall 6G RAN architecture.

Figure 1: Overall 6G RAN Architecture

Proposal 2: 6G RAN can inherit the following interfaces, with potential enhancements:

-	The 6G RAN node is connected to the 6GC via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface. The 6G RAN node is also connected to the 6GC via a SBI interface to support the new services such as AI, ISAC and etc.

-	The 6G RAN node is connected to the 6GC via an NG-U like interface for user plane, e.g., by means of the evolution of NG-U interface.

-	The different 6G RAN nodes (e.g., 6G BS) are interconnected with each other via an Xn like interface, e.g., by means of the evolution of Xn interface.-

Compared with the 5G RAN, the 6G RAN should support some new services, e.g., AI, sensing.

It is highly probable that 6G-RAN nodes and devices will be equipped with AI-related capabilities. For the AI service, it typically requires high computing demand to guarantee the quality, low latency to support the real-time communication, and high security on the exposed AI data. To satisfy these requirements, the 6G RAN can provide new functions to support the AI services. For example, the new function supports the AI computing resource management, online AI model training, AI model inference, AI data collection and exposure, etc.

Proposal 3: The 6G RAN architecture should support AI functions, e.g., computing resource management, model training, model inference, data collection and exposure.

For the sensing service, it typically requires data privacy, low latency and flexible deployment (e.g., BS mono-static, BS-BS bi-static, BS-UE bi-static, UE-BS bi-static, UE mono-static and UE-UE bi-static). To satisfy these requirements, the 6G RAN can provide a new function to support the sensing service. For example, the new function supports the sensing measurement configuration, sensing datacollection to obtain the cloud of points or sensing results, etc.

Proposal 4: The 6G RAN architecture should support sensing functions, e.g., sensing measurement configuration, sensing data collection.

For these new RAN functions, there are two possible deployment options.

-	Option 1: New RAN functions are integrated into 6G BS. With this option, the 6G BS provides the new RAN functions, and no new node in the 6G RAN is introduced. The benefit is to keep the 6G BS as a single point towards the 6GC and other 6G BS, while the drawback is the low efficiency of deployment and collaboration between different 6G BSs for support new services.

-	Option 2: The new RAN functions are deployed independently from the 6G BS providing communication service. With this option, one or more RAN nodes will be introduced in the 6G RAN architecture. The benefit is a unified RAN node for easy deployment, while the drawback is to introduce new interface between the 6G BS and the new RAN node.

Proposal 5: RAN3 to discuss the following options to support the new RAN functions related to AI service and sensing service.

-	Option 1: The 6G BS provides the new RAN functions.

-	Option 2: Introduce a node different from the 6G BS in the 6G RAN to provide the new RAN functions.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: NR functions are considered as a reference to 6GR, while the DC, tight interworking between 6GR and other RAT (e.g. NR or E-UTRA), and UP CIoT optimization are not considered at the beginning.

Proposal 2: 6G RAN can inherit the following interfaces, with potential enhancements:

-	The 6G RAN node is connected to the 6GC via an NG-C like interface for control plane, e.g., by means of the evolution of NG-C interface. The 6G RAN node is also connected to the 6GC via a SBI interface to support the new services such as AI, ISAC and etc.

-	The 6G RAN node is connected to the 6GC via an NG-U like interface for user plane, e.g., by means of the evolution of NG-U interface.

-	The different 6G RAN nodes (e.g., 6G BS) are interconnected with each other via an Xn like interface, e.g., by means of the evolution of Xn interface.

Proposal 3: The 6G RAN architecture should support AI functions, e.g., computing resource management, model training, model inference, data collection and exposure.

Proposal 4: The 6G RAN architecture should support sensing functions, e.g., sensing measurement configuraiton, sensing data collection.

Proposal 5: RAN3 to discuss the following options to support the new RAN functions related to AI service and sensing service.

-	Option 1: The 6G BS provides the new RAN functions.

-	Option 2: Introduce a node different from the 6G BS in the 6G RAN to provide the new RAN functions.

4. Reference

- RP-252912, Revised SID: Study on 6G Radio.