3GPP TSG-RAN WG3 Meeting #129bis	R3-256947

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	13.2

Source:	Hanbat National University

Title:	Network architecture enhancements for NR ISAC

Document for:	Discussion and Decision

# Introduction

The study item on “Study on integrated sensing and communications (ISAC) for NR” was approved in RAN#108 meeting and it contains the network architecture aspects in its scope [1]. as follows:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this contribution, we discuss considerations for network architecture aspects to support NR ISAC.

# Architecture enhancements for ISAC

Looking forward to 5G-Advanced (and consequently 6GR), we consider network architecture enhancements to support monostatic (and preferably bistatic with minimal impacts) ISAC operations across multiple gNBs. These enhancements enable accurate and efficient sensing while managing interference through enhanced capabilities on existing interfaces. Key architectural considerations include inter-gNB signaling (Xn interface), RAN-core network integration (NG interface), and internal gNB splits (F1/E1).

## gNB monostatic sensing operation

To support monostatic sensing (TX and RX at the same site), we introduce gNB‑local behaviors only, realized as implementation features inside an existing gNB. Any sensing session can be based on a local sensing configuration set such as time/frequency resources, BWP, slot format, beam information, etc. The above operation can be done internally within gNB. Assuming CU/DU separation scenario, existing F1 interface can be used to deliver sensing configuration to the DU/TRP that will transmit and then receive echoes in scheduled sensing windows. For the self-interference handling, analog/digital cancellation and receiver protection will remain implementation details with no architectural impacts.

Interference management is an important issue to achieve higher sensing sensitivity and there are some exemplary scenarios of such interference management that can be done without inter-gNB coordination. The gNB may use local scheduling information to reserve sensing window so that the interference from other communication-related channels. The gNB may also rely on autonomous resource selection techniques which locally observes the neighboring environment and avoids their activity. These realizes gNB monostatic ISAC operation entirely inside a gNB using existing CU/DU roles and no architectural changes.

Proposal 1: RAN3 is encouraged to support the gNB monostatic sensing operation using the existing gNB functions without major architectural impacts.

## Bistatic sensing operation with no/minimal architecture impacts

To satisfy the bistatic applicability to gNB bistatic sensing without additional architectural impacts, we consider the existing CU/DU splitting function while avoiding introducing new network functions or inter-gNB coordination.

One of such examples is the intra-gNB multi-TRP bistatic sensing involving multiple DUs/TRPs under one CU. One DU/TRP acts as transmitter (Tx‑TRP) and one or more other DU/TRPs under the same gNB act as receivers (Rx‑TRP). This realizes a bistatic geometry using only intra‑gNB control (i.e., between CU and DU via existing F1 interface).

This intra-gNB coordination also enables, for example, interference muting, which can greatly reduce the impact of interference from other TRPs by delivering sensing-related messages among the TRPs, thereby achieving improved sensing performance. The existence of inter-cell interference in combination of interference muting was discussed in RAN1#122 meeting [2].

While supporting bistatic operation, everything remains inside a single gNB’s administrative boundary. There is no inter‑gNB signaling and no NG/Xn changes. This leverages the CU/DU split and multi‑TRP capabilities already implicit in the NR architecture.

Proposal 2: RAN3 is encouraged to support the gNB bistatic sensing operation with no/minimal architectural impacts (e.g., intra-gNB coordination).

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN3 is encouraged to support the gNB monostatic sensing operation using the existing gNB functions without major architectural impacts.

Proposal 2: RAN3 is encouraged to support the gNB bistatic sensing operation with no/minimal architectural impacts (e.g., intra-gNB coordination).

# References

- RP-251861 “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108.

- R1-2506480, “Summary #5 on evaluations for NR ISAC,” RAN1#122.