3GPP TSG-RAN WG3 Meeting #129bis	R3-256978

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1566 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Slice UE Performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, FiberCop, BT, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” RAN3 introduced in TS 38.423 a set of metrics representing the per-UE per-slice UE performance that could be requested by the source gNB to be reported by the target gNB via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision pertaining a UE served by certain slice(s). However, there is no clear understanding on how a gNB could derive per-UE per-slice Slice Average Packet Loss DL/UL metrics. Following the LS sent by the RAN3 in R3-253816, SA5 provided the reply LS in R3-256518 (S5-253854) and confirmed that it is feasible to specify in TS 28.558 the packet loss/drop measurements with a per-DRB per-UE granularity, performed at PDCP layer and that such measurements could be aggregated to reflect per-UE metrics. Moreover, these measurements can also be provided per supported S-NSSAI. This CR clarifies in the semantics description of Slice Average Packet Loss DL and Slice Average Packet Loss UL IEs within the Slice Based UE Performance IE that the corresponding measurements refer to TS 28.558 clauses 6.3.1.6.1.1 and 6.3.1.7.1, respectively. The CR also corrects the name of the measurement in TS 28.558 used as a reference for the semantics description of the Slice Average Packet Delay IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The semantics description of Slice Average Packet Loss DL and Slice Average Packet Loss UL IEs within the Slice Based UE Performance IE is updated so that the corresponding measurements refer to TS 28.558 clauses 6.3.1.6.1.1 and 6.3.1.7.1, respectively. The semantics description of the Slice Average Packet Delay IE is improved to include the correct name of the measurement in TS 28.558 used as a reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear how Slice Average Packet Loss DL/UL and Slice Average Packet Delay DL/UL are derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.238 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.238	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.1. |
| Slice Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.2. |
| Slice Average Packet Delay | O |  | Average Packet Delay 9.2.3.187 | Corresponds to the Packet delay per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.1. |
| Slice Average Packet Loss DL | O |  | INTEGER (0..1000000, ...) | Corresponds to DL PDCP SDU Drop Rate in gNB per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.6.1.1. |
| Slice Average Packet Loss UL | O |  | INTEGER (0..1000000, ...) | Corresponds to UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.7.1. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>