3GPP TSG-RAN WG3 Meeting #129bis	R3-257051

Prague, Czech, 13th – 17th October 2025

Agenda Item:	10.3.1

Source: 				LG Electronics Inc.

Title:			General principles and requirements for 6G RAN-CN interface

Document for:		Discussion and Decision

1.	Introduction

New 6G WG SID on Study on 6G Radio was agreed at RAN #108 [1]. This SID was updated in RAN #109 to reflect the agreement on the additional migration in [2]. One of the objectives in this SID is to study the 6G Radio Access Network architecture, interface protocols and procedures as Reader as follows:

… - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary …

In this contribution, we will discuss the general principle and requirements on the RAN-CN interface, and provide our view on it.

2.	Discussion

In 5G initial phase, EN-DC was considered essential for smooth migration from 4G to 5G as shown in Figure 1 [3]. This was because EN-DC allows reuse of existing 4G infra-structure to offer wide coverage and essential services such as IMS voice/MMTEL services while 5G is on the way to expand its coverage.

Figure 1: Options 3 and 3A in TR 38.801 [3]

However, this architecture option causes the delay of 5G standalone deployment as illustrated in Figure 2 [3]. In other words, dual connectivity brings huge RAN complexity and specification effort with less performance. It also causes extra OPEX and complexity due to upgrade of 5G NSA towards 5G SA.

Figure 2: Option 2 in TR 38.801 [3]

To avoid this problem, it was agreed to study the single technology framework based on a stand-alone architecture according to the 6G WG SID [2]. Hence, the basic system architecture in 6G can be considered as follows:

Figure 3: Basic system architecture for 6G

Observation 1: The basic system architecture in 6G can be considered as follows:

In 5G study, the following general principles for the NG interface were considered and captured in TR 38.801 [3]. We think that this principle can be used as a baseline for 6G RAN-CN interface.

Clause 7.3.2 of TR 38.801 [3]: … ### 7.3.2 General principles The general principles for the specification of the NG interface are as follows: - the NG interface shall be open; - the NG interface shall support the exchange of signalling information between the New RAN and NGC; - from a logical standpoint, the NG is a point-to-point interface between a New RAN node and an NGC node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the New RAN and NGC; - the NG interface shall support control plane and user plane separation; - the NG interface shall separate Radio Network Layer and Transport Network Layer; - the NG interface shall be future proof to fulfil different new requirements and support of new services and new functions; - the NG interface shall be decoupled with the possible New RAN deployment variants. - the NG Application Protocol shall support modular procedures design and use a syntax allowing optimized encoding /decoding efficiency. NOTE: Whether and how to document the application protocol for NG with regards to S1AP will be decided in the normative phase. …

In addition, in RAN#109, the 6G requirements for architecture and migration were captured in TR 38.914 [4]. These requirements should be also taken into account to design the RAN-CN interface.

Observation 2: The general principle for the NG interface in 5G study can be used as a baseline for 6G RAN-CN interface.

First principle is that the RAN-CN interface should be open as in 5G. It is very clear requirement to support multi-vendor interoperability. That is, even if the 6G RAN and 6G CN NF (e.g., 6G AMF) of different vendors may be deployed in the same network, the exchange of signalling information between the 6G RAN and 6G CN NF should be supported via the specified RAN-CN interface.

Observation 3: The RAN-CN interface should be open.

Second principle is that the RAN-CN interface should support the exchange of signalling information between the 6G RAN and 6G CN. It is also a reasonable requirement for the RAN-CN interface. But, there is one open issue to be discussed. That is how the signaling information is exchanged between the 6G RAN and 6G CN. In 5G, the point-to-point (P2P) interface (i.e., NG interface) is supported between the NG-RAN and AMF. Therefore, the following principle was captured in TR 38.801.

- From a logical standpoint, the NG is a point-to-point interface between a New RAN node and an NGC node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the New RAN and NGC

However, in 5G NG interface, all signalings should be redundantly processed at the AMF due to the P2P interface, thus causing additional signaling latency. In addition, the SCTP which is used for the NG interface is far from the cloud-native protocol. In 6G, since it is important to adopt flexible scaling, improved resource utilization, and the virtualization of RAN NFs, cloud-friendly protocol should be considered in the transport layer even if the P2P interface is adopted for 6G.

To solve this problem, another possible option is to introduce the service-based interface (SBI) between 6G RAN and each NF in 6G CN. The service based interface has a cloud native design. The SBI enables scalable and flexible network deployments to meet various requirements for new services. In 5G, the SBI is already adopted in the core network. All NFs in 5GC are inter-connected via this SBI. Each NF exposes its NF services to other NFs. Then, this NF service can be requested by another NF to provide a certain NF service. It is also possible that the NF subscribes to NF service offered by another NF. Therefore, the exchange of signalling information between the 6G RAN and 6G CN can be supported by SBI. Anyhow, the details on RAN-CN interface options are further discussed in [5]. For now, it is better to have the Editor’s note on this issue.

Observation 4: The RAN-CN interface should support the exchange of signalling information between the 6G RAN and 6G CN.

Observation 5: It is better to have the Editor’s note on how to exchange the signalling information between the 6G RAN and 6G CN.

Observation 6: The RAN-CN interface should support the cloud-friendly design.

Next principle in 5G is that the NG interface shall support control plane and user plane separation. Our understanding is that this should be supported in 6G as well for better scalability. Also, the requirement on this issue is already captured in TR 38.914.

Observation 7: The RAN-CN interface shall support control plane and user plane separation.

In 6G, the new services (e.g., AI/ML, Sensing) are considered. We think that if the service-based interface between 6G RAN and 6G CN is considered for these new services (i.e., beyond-connectivity services), it can take full advantage of cloud-based services. For example, in Sensing service, the sensing function in 6G CN can request some sensing operations (e.g., object detection, object tracking, environment sensing) together with the sensing assistance information to the sensing entity (e.g., in 6G RAN) over the SBI. After performing the sensing operations requested from the sensing function, the sensing entity can directly provide the measurement (or sensing) results to the sensing function without the help of the 6G AMF. Even if the P2P interface is considered, this interface should be enhanced to support these new services. One similar example in 5G is that new NGAP siganlings are newly defined to support Rel-19 A-IoT service. Therefore, our view is that the following principle in 5G should be supported in 6G.

- The NG interface shall be future proof to fulfil different new requirements and support of new services and new functions

Observation 8: The RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions.

Also, in 5G the network slicing is introduced to allow differentiated treatment depending on each customer requirements. Several functionalities related to the network slicing are developed in 5G. According to the current SA2 6G SID [6], it is also agreed to support the network slicing in 6G.

SP-250806 [6]: … The study includes the following high level work tasks, and the conclusion will consolidate the 6G architecture among all work tasks: WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network: NOTE 1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. 1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services. 1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access. 1.3. Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access. 1.4. Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G. In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G. …

During the study, SA2 will determine which functionality for the network sling should be supported in 6G. That is, the details on the network slicing functionality supported in 6G are pending to SA2 decision. Anyhow, the RAN-CN interface should support the operation of network slicing based on SA2 decision.

Observation 9: The RAN-CN interface should support the operation of network slicing.

As mentioned before, we think that the 6G RAN should advance toward a fully virtualized and modular design. It simplifies the introduction of a new services without impacting other services. In other words, this would allow RAN functions to be dynamically deployed at optimal locations based on network needs and real-time conditions. Therefore, the RAN-CN interface should support the signaling exchange between 6G RAN and 6G CN irrespective of how the RAN NFs are constructed within the 6G RAN. That is, the RAN-CN interface should be decoupled with the possible RAN NF deployment variants.

Observation 10: The RAN-CN interface should be decoupled with the possible RAN NF deployment variants.

Regarding the separation of the Radio Network Layer and Transport Network Layer, we think that this should be also supported in 6G to avoid unnecessary tight coupling between the RNL and TNL.

Observation 11: The RAN-CN interface shall separate Radio Network Layer and Transport Network Layer.

In order to capture the above observations, the following proposal is suggested to RAN3:

Proposal 1: It is proposed to capture the following principles into TR 38.8xx [7]:

- The RAN-CN interface should be open.

- The RAN-CN interface should support the exchange of signalling information between the 6G RAN and 6G CN.

  - Editor’s note: It is FFS how to exchange the signalling information between the 6G RAN and 6G CN.

- The RAN-CN interface should support the cloud-friendly design.

- The RAN-CN interface shall support control plane and user plane separation.

- The RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions.

- The RAN-CN interface should support the operation of network slicing.

- The RAN-CN interface should be decoupled with the possible RAN NF deployment variants.

- The RAN-CN interface shall separate Radio Network Layer and Transport Network Layer.

3.	Conclusion

In this contribution, we focused on the general principle and requirements on the RAN-CN interface, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to capture the following principles into TR 38.8xx [7]:

- The RAN-CN interface should be open.

- The RAN-CN interface should support the exchange of signalling information between the 6G RAN and 6G CN.

  - Editor’s note: It is FFS how to exchange the signalling information between the 6G RAN and 6G CN.

- The RAN-CN interface should support the cloud-friendly design.

- The RAN-CN interface shall support control plane and user plane separation.

- The RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions.

- The RAN-CN interface should support the operation of network slicing.

- The RAN-CN interface should be decoupled with the possible RAN NF deployment variants.

- The RAN-CN interface shall separate Radio Network Layer and Transport Network Layer.

4.	Reference

- RP-251881, “SID: Study on 6G Radio”, RAN (NTT DOCOMO, CMCC, AT&T, Vodafone), June, 2025.

- RP-252912, “Revised SID: Study on 6G Radio”, RAN (NTT DOCOMO, CMCC, AT&T, Vodafone), September, 2025.

- TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”, v14.0.0.

- TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0.

- R3-257052, “Consideration on RAN-CN interface options”, LG Electronics, October, 2025.

- SP-250806, “Study on Architecture for 6G System”, SA (ZTE), June, 2025.

- TR 38.8xx, “Study on 6G Radio RAN3 aspects”, v0.0.0.

5.	Appendix: Text proposal to TR 38.8xx [7]

This appendix provides the text proposal to TR 38.8xx [7] based on the proposals of this contribution.

----------------Start of the First Change---------------

# 6	RAN Architecture and Interfaces

Editor’s note: The aim of this section is to describe all aspects related to the Interface between gNB to CN.

## 6.1	General Principles and Requirements

The general principles for the specification of the RAN-CN interface are as follows:

- The RAN-CN interface should be open.

- The RAN-CN interface should support the exchange of signalling information between the 6G RAN and 6G CN.

Editor’s note: It is FFS how to exchange the signalling information between the 6G RAN and 6G CN.

- The RAN-CN interface should support the cloud-friendly design.

- The RAN-CN interface shall support control plane and user plane separation.

- The RAN-CN interface shall be future proof to fulfil different new requirements and support of new services and new functions.

- The RAN-CN interface should support the operation of network slicing.

- The RAN-CN interface should be decoupled with the possible RAN NF deployment variants.

- The RAN-CN interface shall separate Radio Network Layer and Transport Network Layer.

-----------------End of the First Change---------------