3GPP TSG-RAN WG3 Meeting #129bis	R3-257103

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	12.2.1

Source:	Nokia

Title:	(TP to TR 38.745) Multi-hop UE trajectory across gNBs

Document for:	Other

# 1	Introduction

Rel-20 NG-RAN AI/ML study focuses on AI/ML optimization of aspects pertaining to AI/ML mobility. In particular, one of the objectives is to study multi-hop UE trajectory across gNBs.

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

One of the key enablers of AI/ML for NG-RAN in Rel-18 is related to UE Trajectory. Predicted UE Trajectory information may be sent in Rel-18 from a source node to a target node during a handover. Correspondingly, a source node may request measured UE Trajectory information from a target node for a handed over UE. The measured UE Trajectory is cell-based and is measured over the cells of the target node. This enables the source node predict UE Trajectory over the target node cells, but also to monitor the performance of its prediction.

One of the fundamental limitations of Rel-18 predicted (single-hop) trajectory is that it is sent to a target node at a time of a UE handover. So, when the trajectory is sent to the target node the UE, for which the trajectory is provided, is already known. This means that even though a source node can use its calculated predicted trajectory to determine the best target cell, the node receiving predicted trajectory has limited possibilities to use the predicted trajectory information for its own resource optimization, e.g., limited to intra-gNB subsequent handovers and resource reservation. Another limitation of this approach is that the trajectory is, optionally, sent to the target node without the target having to request for it. The assumption for this design was that predicted trajectory information served the role of “assistance information” provided to the target node of a handover, if this information is available at the source. Finally, even though different UE Trajectory granularities were considered, based on cell level, beam level, or detailed location level, in Rel-18 we finally considered only cell level predicted and measured UE trajectory. This decision was driven by the considered use cases which only involved cell level decisions.

In the rest of this paper, we provide our views for various aspects related to multi-hop UE Trajectory that could be studied in Rel-20.

# 2	Discussion

## 2.1 Predicted Cell-based Multi-hop Trajectory

During Rel-19 study item phase, we had identified two main methods to introduce multi-hop UE trajectory in a simplified way. These options are summarized next:

- The initial source node sends measured UE Trajectory requests to all known nodes where its served UEs will connect according to their predicted trajectory. In each Handover Request message the initial source node and subsequent serving nodes include the UE context ID at the initial source node so that the measured UE Trajectories are forwarded directly from the serving nodes to the initial source node. Predicted UE Trajectory is sent in a hop-by-hop fashion from each node to the next node where a UE is handed over to.

- Reuse existing Data Collection procedures to propagate the request for measured UE Trajectory in a hop-by-hop basis until an exit condition is met. Each node concatenates in chronological order its own measured UE Trajectory and the measured UE trajectory received from its neighbouring node and reports it to the former node. Predicted UE Trajectory is sent in a hop-by-hop fashion from each node to the next node where a UE is handed over to.

In our view, the full potential of predicted multi-hop trajectory can be exploited when a node can anticipate traffic (in terms of trajectories) that will pass through its cells. Therefore, we think that RAN3 should examine alternatives that enable a node to manage its resources well in advance before a UE moves to the node, e.g., to enable cell switch-off actions, resource reservation, load management, etc. and not at the time of a handover. In order to avoid unnecessary or unrequested information being sent to the network nodes, we think that a node should only receive predicted multi-hop trajectory, if it has requested it. As can be shown in Figure 1, in practice, there could be a very large number of trajectories passing through a node, growing exponentially with the number of the allowed hops defining the multi-hop. We therefore think that in order to limit the complexity of the algorithm predicting multi-hop trajectory at a network node the number of hops of the requested predicted multi-hop UE trajectories should be limited.

Figure 1 Multi-hop trajectory passing from gNB1. n hop trajectory would exclude all paths that are farther from gNB 41.

Proposal 1: RAN3 to study how to enable a node to subscribe to other nodes for predicted multi-hop trajectories of a specific number of hops towards itself.

Another aspect to be considered is the granularity of predicted UE trajectory. For Rel-20, as a starting point, predicted multi-hop UE trajectory has cell-based granularity. If use cases justify a finer granularity, it can be considered on a case-by-case basis. For instance, RAN3 could later evaluate whether the intra-CU LTM use case could benefit from beam-based multi-hop trajectory prediction.

Proposal 2: As a starting point, predicted multi-hop UE trajectory has cell-based granularity. Beam-based granularity can be evaluated if a use case motivates it, e.g., intra-CU LTM.

We illustrate the above in the following example in Figure 2. gNB1 requests predicted 2-hop trajectories from its one-hop neighbour, gNB 21, that pass through itself. gNB 21 also checks with its one hop neighbours (gNB 31 and gNB 32) in this example for one-hop trajectories that pass from gNB 21. Each node, gNB 31 and gNB 32, reply to gNB 21 of a list of predicted trajectories that pass from gNB 21. gNB 21 aggregates those trajectories and responds back to gNB 1. The benefit of this approach is that the UE context does not need to be maintained over a long period after the UE has left the source gNB, it simplifies multi-hop trajectory prediction and at the same time it provides a flow over a multi-hop path of gNBs.

Figure 2 Example of multi-hop trajectory

Proposal 3: Consider predicted multi-hop UE trajectories not associated to a single UE.

## 2.2 Training Data to Predict Cell-based Multi-hop Trajectory

Similarly to the Rel-18 discussions, in order to predict multi-hop cell-based UE Trajectory a node should have access to measured trajectories over multiple hops away from the node making the predictions. Currently, this is not possible since measured UE Trajectory is sent from a target node to a source node, and involves only a measured trajectory over the cells of the target node. Similarly, to the predicted multi-hop trajectory case, in practice, there could be a very large number of measured trajectories passing through a node, growing exponentially with the number of the allowed hops defining the multi-hop. We therefore think that the number of hops of measured multi-hop UE trajectory should be limited.

In addition, training an AI/ML Model for UE Trajectory prediction could take advantage of information available both for active as well as idle/inactive UEs. This seems important in order to create and maintain a complete and accurate UE trajectory prediction. Keeping track of the time that UE spent in idle/inactive state as opposed to connected state helps the network keep track the UE movement in more detail and also could allow it to determine whether a mobility decision was taken by the UE or by the network. For example, network should be able to obtain trajectory information of those UEs that have crossed the base station’s coverage area in idle or inactive state and should further be aware of how long a UE stayed in a given cell when it was in idle, inactive or in connected state.

Proposal 4: RAN3 to study how to enable the availability of measured UE trajectory for UEs in RRC Connected, RRC Idle and RRC Inactive states, over a specific number of hops.

Mobility history information (specified in TS 38.331) indicates how a UE stores a list of visited cells, with the most recently visited cell being stored first, including cells visited in RRC Idle, RRC Inactive and RRC Connected states together with a time spent in each cell:

VisitedCellInfoList information element

-- ASN1START

-- TAG-VISITEDCELLINFOLIST-START

VisitedCellInfoList-r16 ::= SEQUENCE (SIZE (1..maxCellHistory-r16)) OF VisitedCellInfo-r16

VisitedCellInfo-r16 ::=  SEQUENCE {

visitedCellId-r16        CHOICE {

nr-CellId-r16            CHOICE {

cgi-Info                 CGI-Info-Logging-r16,

pci-arfcn-r16            PCI-ARFCN-NR-r16

},

eutra-CellId-r16         CHOICE {

cellGlobalId-r16         CGI-InfoEUTRA,

pci-arfcn-r16                PCI-ARFCN-EUTRA-r16

}

}                                        OPTIONAL,

timeSpent-r16            INTEGER (0..4095),

...,

[[

visitedPSCellInfoListReport-r17    VisitedPSCellInfoList-r17                   OPTIONAL

]]

}

VisitedPSCellInfoList-r17 ::= SEQUENCE (SIZE (1..maxPSCellHistory-r17)) OF VisitedPSCellInfo-r17

VisitedPSCellInfo-r17 ::=    SEQUENCE {

visitedCellId-r17            CHOICE {

nr-CellId-r17                CHOICE {

cgi-Info-r17                 CGI-Info-Logging-r16,

pci-arfcn-r17                PCI-ARFCN-NR-r16

},

eutra-CellId-r17         CHOICE {

cellGlobalId-r17         CGI-InfoEUTRALogging,

pci-arfcn-r17            PCI-ARFCN-EUTRA-r16

}

}                                                 OPTIONAL,

timeSpent-r17            INTEGER (0..4095),

...

}

-- TAG-VISITEDCELLINFOLIST-STOP

-- ASN1STOP

VisitedCellInfoList field descriptions

timeSpent This field indicates the duration of stay in the cell or in any cell selection state and/or camped on any cell state in NR or E-UTRA approximated to the closest second. If included in VisitedPSCellInfo, it indicates the duration of stay in the PSCell or without any PSCell. If the duration of stay exceeds 4095s, the UE shall set it to 4095s.

visitedCellId This field indicates the visited cell id including NR and E-UTRA cells.

Therefore, in addition to UE History Information, Mobility History information can be used to collect measured UE trajectory information across RRC states.

Proposal 5: RAN3 to consider usage of Mobility History Information in addition to UE History Information as input to obtain predicted multi-hop UE trajectory for UEs across RRC states.

# 3	Conclusion

In this paper, we make the following observations and proposals:

Proposal 1: RAN3 to study how to enable a node to other nodes for predicted multi-hop trajectories of a specific number of hops towards itself.

Proposal 2: As a starting point, predicted multi-hop UE trajectory has cell-based granularity. Beam-based granularity can be evaluated if a use case motivates it, e.g., intra-CU LTM.

Proposal 3: Consider predicted multi-hop UE trajectories not associated to a single UE.

Proposal 4: RAN3 to study how to enable the availability of measured UE trajectory for UEs in RRC Connected, RRC Idle and RRC Inactive states, over a specific number of hops.

Proposal 5: RAN3 to consider usage of Mobility History Information in addition to UE History Information as input to obtain predicted multi-hop UE trajectory for UEs across RRC states.

# Annex – TP to TR 38.745

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 4	Use cases and Solutions

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor Note: Capture the description of use case

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs that UE will visit and these cells are listed in chronological order. A node subscribes to other nodes for predicted multi-hop trajectories of a specific number of hops, terminating to itself. To increase the number of training data for multi-hop UE trajectory, UE trajectory is calculated not only from RRC Connected UEs, but also from UEs in RRC Idle or RRC Inactive states.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>