3GPP TSG-RAN WG3 Meeting #129bis	R3-257197

Prague, Czech, Oct 13 - 17, 2025

Agenda Item:	9.1.3

Source:	ZTE Corporation

Title:	Response to R3-256741, R3-256742, R3-256745, R3-256746

Document for:	Discussion

- Introduction

This contribution responses to R3-256741, R3-256742, R3-256745, R3-256746.

2	Discussion

In R3-256741, R3-256742, R3-256745 and R3-256746[1][2][3][4], the issue of restriction on the time-based handover for IoT NTN and NR NTN has been discussed.

RAN3#119 Introducing time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact. RAN3#119bis RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS38.413. Regarding the duplicated issues, IoT NTN shall wait for the corresponding progress in NR NTN.

Based on the previous conclusion in R18, a source gNB can only prepare one potential target cell for NG HO, and the principle should also be applied to IoT NTN. Since the above conclusion is only applicable to the NG and S1 based handover and there is no corresponding restriction on X2 and Xn based handover, it is reasonable to remove the “single” from the description given as below.

When a time-based trigger condition is used, the source eNB may signal the corresponding parameters to a target eNB via the Source eNB to Target eNB Transparent Container in an S1-based handover, see TS 23.401 [17], or X2 based handover. The source eNB signals the corresponding CHO configuration to the UE in the RRC Connection Reconfiguration message during handover. NOTE: For S1 based handover, only a single target cell is prepared when a time-based trigger condition is used.

However, to follow the previous common understanding “RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS 38.413.”, comparing with adding a dedicated NOTE in stage 2 specifications, it is more appropriate to add the corresponding procedural description in the TS 36.413 and TS 38.413 for IoT NTN and NR NTN respectively. And one example for IoT NTN could be given as below.

8.4.2 Handover Resource Allocation 8.4.2.1 General The purpose of the Handover Resource Allocation procedure is to reserve resources at the target eNB for the handover of a UE. 8.4.2.2 Successful Operation >>unchanged part skipped<< If the Direct Forwarding Path Availability IE is included in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUEST message, the target eNB may use the information to assign tunnel endpoints in case of inter-system handover. If the HANDOVER REQUEST message contains within the Source eNB to Target eNB Transparent Container IE the Time Based Handover Information IE, the target eNB may use this information to allocate necessary resources for the incoming time-based handover with only one single target cell indicated in the Source eNB to Target eNB Transparent Container IE. If the Time Reference Distribution Information IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 23.401 [11].

Proposal: Add the procedural description in TS 36.413 and TS 38.413 to reflect RAN3’s previous common understanding “RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS38.413.”

3	Summary

Proposal: Add the procedural description in TS 36.413 and TS 38.413 to reflect RAN3’s previous common understanding “RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS38.413.”

4	Reference

[1] R3-256741, Correction of time-based handover for IoT NTN, Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

[2] R3-256742, Correction of time-based handover for IoT NTN, Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

[3] R3-256745, Correction of time-based handover for NR NTN, Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

[4] R3-256746, Correction of time-based handover for NR NTN, Huawei, Deutsche Telekom, Jio Platforms, CATT, Ericsson

5	Annex

CR on time-based NG HO for NR NTN (R18)

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

# Prague, Czech Republic, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | xxxx | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on time-based NG HO for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, it was agreed to introduce the time-based NG HO. And RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS 38.413. However, there is no corresponding procedural description on this common understanding in TS 38.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedural description in the Handover Resource Allocation procedure to restrict that a source gNB can only prepare on potential target cell for time-based NG HO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restriction is not reflected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

>>unchanged part skipped<<

If the HANDOVER REQUEST message contains within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE the Time Based Handover Information IE, the target NG-RAN node may use this information to allocate necessary resources for the incoming time-based handover with only one single target cell indicated in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE.

If the Candidate Relay UE Information List IE is included in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure the path switch to indirect path as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

CR on time-based NG HO for NR NTN (R19)

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

# Prague, Czech Republic, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on time-based NG HO for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, it was agreed to introduce the time-based NG HO. And RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS 38.413. However, there is no corresponding procedural description on this common understanding in TS 38.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedural description in the Handover Resource Allocation procedure to restrict that a source gNB can only prepare on potential target cell for time-based NG HO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restriction is not reflected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

>>unchanged part skipped<<

If the HANDOVER REQUEST message contains within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE the Time Based Handover Information IE, the target NG-RAN node may use this information to allocate necessary resources for the incoming time-based handover with only one single target cell indicated in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE.

If the Candidate Relay UE Information List IE is included in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure the path switch to indirect path as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

CR on time-based S1 HO for IoT NTN (R18)

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

# Prague, Czech Republic, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on time-based NG HO for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, it was agreed to introduce the time-based NG HO. And RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS 38.413. And this principle should also apply for IoT NTN. However, there is no corresponding procedural description on this common understanding in TS 36.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedural description in the Handover Resource Allocation procedure to restrict that a source eNB can only prepare on potential target cell for time-based S1 HO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restriction is not reflected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target eNB for the handover of a UE.

8.4.2.2	Successful Operation

>>unchanged part skipped<<

If the Direct Forwarding Path Availability IE is included in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUEST message, the target eNB may use the information to assign tunnel endpoints in case of inter-system handover.

If the HANDOVER REQUEST message contains within the Source eNB to Target eNB Transparent Container IE the Time Based Handover Information IE, the target eNB may use this information to allocate necessary resources for the incoming time-based handover with only one single target cell indicated in the Source eNB to Target eNB Transparent Container IE.

If the Time Reference Distribution Information IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 23.401 [11].

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

CR on time-based S1 HO for IoT NTN (R19)

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

# Prague, Czech Republic, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on time-based NG HO for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, it was agreed to introduce the time-based NG HO. And RAN3 understands a source gNB can only prepare one potential target cell for NG HO as stated in TS 38.413. And this principle should also apply for IoT NTN. However, there is no corresponding procedural description on this common understanding in TS 36.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedural description in the Handover Resource Allocation procedure to restrict that a source eNB can only prepare on potential target cell for time-based S1 HO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restriction is not reflected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target eNB for the handover of a UE.

8.4.2.2	Successful Operation

>>unchanged part skipped<<

If the Direct Forwarding Path Availability IE is included in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUEST message, the target eNB may use the information to assign tunnel endpoints in case of inter-system handover.

If the HANDOVER REQUEST message contains within the Source eNB to Target eNB Transparent Container IE the Time Based Handover Information IE, the target eNB may use this information to allocate necessary resources for the incoming time-based handover with only one single target cell indicated in the Source eNB to Target eNB Transparent Container IE.

If the Time Reference Distribution Information IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context and use it as defined in TS 23.401 [11].

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>