3GPP TSG-RAN WG3 Meeting #129bis	R3-257263

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0487 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on WAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Clause 6.1.7 contains such text “backhaul and access of WAB-node”, but there is no clear definition on the access link and backhaul link of WAB. - The protocol stack figure title in clause 6.1.7 should be dedicated to WAB, rather than a general protocol stack for NG-RAN. - The report of additional ULI may not be required by CN, if reported via non-UE associated procedures, so the text of updating additional ULI in section 12.5 is not proper. - In clause 12.7.1, the last sentence emphasize that the conitnuity of OAM connection should be ensured as the WAB-node moves, this sentence itself is correct, but it is somehow redundant because similar sentence has already been captured in clause 12.2.1 - In clause 12.8, the last sentence indicates that the WAB-gNB should be configurable with respect to whether it should accept or reject Xn setup requests received from WAB-gNBs, we think “should” is improper, because the determination of whether to accept or reject Xn connection from other WAB-gNBs can also be up to the WAB-gNB’s implementation. A proper implementaiton for WAB-gNB to make such decision can be e.g., based on its own moving status. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description for access and backhaul link in clause 6.1.7. - Add “for WAB” in the title of protocol stack figures in clause 6.1.7. - Update the description for the report of updated additional ULI to CN in clause 12.5. - Remove the last sentence in clause 12.7.1. - Change “should” to be “can” in the last sentence of clause 12.8. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WAB part is misleading, e.g., unclear of “if required by CN” for additional ULI report, unknown definition of access and backhaul link for WAB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.7, 12.5, 12.7.1, 12.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Revert the change about removing “report additional ULI via legacy procedures” in 12.7.1. |  |  |  |  |  |  |  |  |

Start of Change

6.1.7	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu access radio link.

The WAB-MT is served by a BH-gNB via backhaul radio link. The WAB-gNB’s traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.7-1 shows the WAB architecture for 5GS.

Figure 6.1.7-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.  The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.7-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.7-2: Protocol stacks for NG Control plane and NG User plane transport of WAB

Figure 6.1.7-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

Figure 6.1.7-3: Protocol stacks for Xn Control plane and Xn User plane transport of WAB

.

Next Change

12.5	User Location Information for UEs served by a WAB-gNB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case Additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via legacy procedures, to the core network.

12.7	WAB-node mobility

12.7.1	WAB-MT mobility

The WAB-MT reuses legacy mobility procedures defined for the UE. During the WAB-node’s movement, when the BH PDU session(s) of WAB-MT are re-established, the co-located WAB-gNB may need to update the IP address(es) used for its traffic. In case IPsec tunnel mode is used to protect the WAB-gNB’s traffic, MOBIKE (IETF RFC 4555 [29]) can be used to avoid the change of inner IP address(es) used for this traffic. Otherwise, following procedures can be used for handling the IP address change of the WAB-gNB’s traffic:

-	NG-C and Xn-C can be migrated to the new IP address(es) via legacy procedures defined in TS 38.412 [37] and TS 38.422 [38], respectively.

-	NG-U GTP-U tunnels can be migrated via the legacy NGAP PDU Session Resource Modify Indication procedure.

Next Change

## 12.8	Xn connection management

A WAB-gNB can establish an Xn connection with the BH-gNB serving the co-located WAB-MT, and with the neighbouring gNBs. During the setup or update of its Xn connections, the WAB-gNB can include an ID of the WAB-MT, to indicate that it is a WAB-gNB. In case the peer gNB is the WAB-MT’s BH-gNB, the WAB-MT ID makes the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB. The WAB-MT ID consists of the C-RNTI assigned to the WAB-MT by the BH-gNB, and the cell ID of BH-gNB´s cell serving the WAB-MT.

Establishment of Xn connections between two WAB-gNBs can be avoided. To achieve this, the WAB-gNB may reject the Xn setup initiated by another WAB-gNB, e.g., based on the presence of the WAB-MT ID received in the XN SETUP REQUEST message.

A WAB-gNB can be configurable with respect to whether it should accept or reject Xn setup requests received from WAB-gNBs.

End of Change