3GPP TSG-RAN3 Meeting #129bis																R3-25xxxx

Prague, CZ, 13-17 October, 2025

Agenda Item:	12.2.1

Source:	ZTE Corporation

Title:	(TP to TR 38.745) Consideration on Multiple-hop UE trajectory

Document for:	Text Proposal

# Text Proposal

<<<<<<<<<<<<<<<<<<<< First Changes >>>>>>>>>>>>>>>>>>>>

# 1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., multiple-hop UE trajectory, AI/ML assisted intra-CU LTM, and other handover enhancements.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 4.1	Multi-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>