3GPP TSG RAN WG4 Meeting # 116							R4-2509121

Bengaluru (Bangalore), India, August 25-29, 2025

Agenda item:			7.7.3

Source:	SES, Fraunhofer IIS, Lockheed Martin

Title:	Summary of Ku Band Co-existence Analysis Results

Document for:	Discussion and Decision

# Introduction

To progress with the standardisation of the Ku band, two critical components namely the ACLR and ACS values have to be calculated. This is achieved by simulating a fictitious Terrestrial Network (TN) adjacent to a Non-Terrestrial Network (NTN) and from the simulated interference levels, the values of the two parameters ACLR and ACS can be calculated. The relation between ACIR, ACLR and ACS is explained in section 5.3 in more detail.

The co-existence study of the Ku Band is comprised of two phases:

- Calibration of the simulator platform against agreed parameters

- Calculation of the ACLR and ACS values from the ACIR parameter

The calibrations are performed in two segments, one for the NTN and the other for the TN components, separate from each other.

In this contribution we report on the calibration results of the Ku Band co-existence analyses available to date, for both NTN and TN segments.

While preliminary ACLR and ACS (or ACIR) results have been provided by various companies for the last RAN4#115 meeting, updated ACLR/ACS/ACIR results are expected for this RAN4#116 meeting based on the calibrated NTN and TN simulation setups.

# Ku Band Spectrum

In this section, we outline the Ku band downlink and uplink spectrum, as well as the position of the TN interferer.

Figure 1 below depicts the locations of the interferers utilized in the coexistence simulation.

Referring to Figure 1, in this simulation exercise, the downlink TN and NTN interferers are positioned at 11 GHz, while the uplink interferers are at 14 GHz. However, these frequency points are hypothetical and used solely for the co-existence analysis.

In real-world scenarios and typical deployment contexts, the expected frequency allocations for Ku band NTN and possible TN deployments are shown in Figure 2.

As shown in Figure 2, under typical realistic conditions, the minimal separations between interfering NTN and TN frequencies are as follows:

ITU Regions 1 and 3

- DL Separation: 700 MHz

- UL Separation 300 MHz

ITU Region 2

- DL separation 200 MHz

- UL Separation 300 MHz

As illustrated in Figure 1, the coexistence simulation involving frequency overlaps between NTN and TN—at 11 GHz for downlink and 14 GHz for uplink—represents the most adverse case for ACLR and ACS computations. In the Ku band, as illustrated in Figure 2, frequency overlap between NTN and TN is impractical and unrealistic. In practical settings, frequency gaps between NTN and TN span from 200 MHz to 700 MHz, varying based on uplink or downlink and the specific ITU Region.  Such considerable frequency separations yield more advantageous ACLR and ACS outcomes, making NTN-TN coexistence simpler than the scenarios analyzed in this document.

Figure 1 Location of Terrestrial and Non-Terrestrial Uplink and Downlink Interferers

Figure 2 Realistic NTN Deployment Frequencies and possible TN deployment, subject to ITU and regional Regulations

# NTN and TN Calibration Results

In this section we present summary of the calibration results for comparison. The calibration phase has been performed for NTN and TN components separate from each other. Assumptions and discussions regarding methodology and results for calibration are elaborated in more detail in section 5. The numerical results of the calibration phase contributed by different companies, which build the basis for the summary in the following subsections, are documented in the attached Excel workbook to this TDoc.

For NTN, calibrations have been performed for GEO and LEO satellites. Coupling loss and Geometry SINR have been evaluated for a 45x45 element phased array and parabolic antenna for both downlink and uplink. The results of the NTN calibration phase are summarized in section 3.1 for NTN uplink and section 3.2 for NTN downlink.

For TN, uplink at 14 GHz and downlink at 11 GHz have been calibrated with results for coupling loss and Geometry SINR summarized in sections 3.3 and 3.4, respectively.

## Non-Terrestrial Network Uplink (NTN UL)

The following sections illustrate the calibration results for the different scenarios of the NTN UL case at 14 GHz.

### GEO 45x45 UL

The following Figure 3 depicts the calibration results for GEO satellite for a 45x45 element phase array and uplink at 14 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for GEO 45x45 UL.

(a)

(b)

Figure 3 GEO 45x45 UL Calibration: (a) Coupling Loss, (b) Geometry SINR

### LEO 45x45 UL

The following Figure 4 depicts the calibration results for LEO satellite for a 45x45 element phase array and uplink at 14 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for LEO 45x45 UL.

(a)

(b)

Figure 4 LEO 45x45 UL Calibration: (a) Coupling Loss, (b) Geometry SINR

### GEO Parabolic UL

The following Figure 5 depicts the calibration results for GEO satellite for a parabolic antenna and uplink at 14 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for GEO Parabolic UL.

(a)

(b)

Figure 5 GEO Parabolic UL Calibration: (a) Coupling Loss, (b) Geometry SINR

### LEO Parabolic UL

The following Figure 6 depicts the calibration results for LEO satellite for a parabolic antenna and uplink at 14 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for LEO Parabolic UL.

(a)

(b)

Figure 6 LEO Parabolic UL Calibration: (a) Coupling Loss, (b) Geometry SINR

## Non-Terrestrial Network Downlink (NTN DL)

The following sections illustrate the calibration results for the different scenarios of the NTN DL case at 11 GHz.

### GEO 45x45 DL

The following Figure 7 depicts the calibration results for GEO satellite for a 45x45 element phase array and downlink at 11 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for GEO 45x45 DL.

(a)

(b)

Figure 7 GEO 45x45 DL Calibration: (a) Coupling Loss, (b) Geometry SINR

### LEO 45x45 DL

The following Figure 8 depicts the calibration results for LEO satellite for a 45x45 element phase array and downlink at 11 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for LEO 45x45 DL.

(a)

(b)

Figure 8 LEO 45x45 DL Calibration: (a) Coupling Loss, (b) Geometry SINR

### GEO Parabolic DL

The following Figure 9Figure 5 depicts the calibration results for GEO satellite for a parabolic antenna and downlink at 11 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for GEO Parabolic DL.

(a)

(b)

Figure 9 GEO Parabolic DL Calibration: (a) Coupling Loss, (b) Geometry SINR

### LEO Parabolic DL

The following Figure 10 depicts the calibration results for LEO satellite for a parabolic antenna and downlink at 11 GHz. The calibration results for coupling loss and SINR show good correlation among companies. Specifics of the calibration results are discussed in more detail in section 5.

Good correlation of NTN calibration results for LEO Parabolic DL.

(a)

(b)

Figure 10 LEO Parabolic DL Calibration: (a) Coupling Loss, (b) Geometry SINR

## Terrestrial Network Uplink (TN UL)

The following Figure 11 illustrates the calibration plots for the TN UL case at 14 GHz. The TN calibration results show good correlation among companies for coupling loss and geometry SINR.

Good correlation of TN calibration results for TN uplink.

(a)

(b)

Figure 11 TN UL Calibration: (a) Coupling Loss, (b) Geometry SINR

## Terrestrial Network Downlink (TN DL)

The following Figure 12 illustrates the calibration plots for the TN DL case at 11 GHz. The TN calibration results show good correlation among companies for coupling loss and geometry SINR.

Good correlation of TN calibration results for TN downlink.

(a)

(b)

Figure 12 TN DL Calibration: (a) Coupling Loss, (b) Geometry SINR

# Preliminary Co-existence Analysis Results

For the previous RAN4#115 meeting, companies have provided preliminary results for ACLR and ACS. The results have been summarized in [2] and are shown in the following

Table 1 for reference.

In the phase between the RAN4#115 and the RAN4#116 meetings, the main focus was on converging the NTN and TN calibration results as discussed in section 3.

Based on the converged stand-alone calibrated NTN and TN setups, with results documented in  section 3, it is expected that companies are going to update their ACLR and ACS results which are then to be discussed in the RAN4#116 meeting.

Companies are expected to update ACLR and ACS results based on calibrated NTN and TN setups.

Table 1 Summary of preliminary Ku Band Co-existence results from [2]

| ACIR（worst case btw 35/90 deg） |  |  | Xiaomi R4-25057620 | Qualcomm R4-2505747 | ZTE R4-2506258 | Ericsson R4-2507672 |
| --- | --- | --- | --- | --- | --- | --- |
| scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array | 3.6 | 25/25 | 15 |  |
|  |  | Parabolic | 9.6 | 20/15 | 21 | 3 |
|  | LEO | Phase array | 0 | 25/0 | 0 |  |
|  |  | Parabolic | 2.1 | 20/0 | 0 | 8 |
| scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array | 10.3 | 20/15 | 28.21 |  |
|  |  | Parabolic | 9.9 | 20/15 | 28.7 | 16 |
|  | LEO | Phase array | 39.5 | 25/25 | 38.46 |  |
|  |  | Parabolic | 37.7 | 30/25 | 38.46 | 22 |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array | 18.3 | 45/45 | 18.37 |  |
|  |  | Parabolic | 25 | 40/40 | 38.58 | 35 |
|  | LEO | Phase array | 19.3 | 45/45 | 24.36/>40 |  |
|  |  | Parabolic | 26.2 | 40/40 | 41.32 | 35 |
| scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array | 0 | 0 | 0 |  |
|  |  | Parabolic | 0 | 0 | 0 | 0 |
|  | LEO | Phase array | 0 | 0 | 0 |  |
|  |  | Parabolic | 0 | 0 | 0 | 0 |

# Discussion

## Calibration Assumptions

The calibration assumptions for the Ku band co-existence study are documented in [1] in Annex C.1 for both NTN and TN. With respect to applicability of specific assumptions for calibration, additional discussions and clarifications have been made for specific issues which are summarized in the following sections.

### NTN Calibration Assumptions

#### Clutter Loss and Shadow Fading for NTN

For NTN calibration, an elevation angle of 35 degrees is used for both GEO and LEO scenarios (see [1], Annex C.1). The large scale model of TS 38.811 is taken as basis for propagation modelling.

The clutter loss and shadow fading were selected in the simulation because:

- An urban propagation scenario was selected in the simulation assumptions for NTN calibration with an elevation of 35 degrees [2],

- The use case scenario was a Land based Mobile VSAT in an urban environment

The values for LOS probability are based on the urban scenario in TS 38.811, Table 6.6.1-1. Shadow fading and clutter loss parameters are based on the Ka band values as provided in TS 38.811, Tables 6.6.2-2 (urban scenario), as no Ku band values are given and channel effects in Ku band are similar to Ka band.

Clutter loss is only considered under NLOS conditions and is set to 0 dB for LOS conditions (based on TS 38.811, section 6.6.2). TS 38.811, Table 6.6.1-1 provides values for LOS probability in steps of 10 degrees of elevation. At a satellite elevation of 35 degrees, as selected for NTN calibration, the values for 30 and 40 degrees apply for UEs (nearest reference angles depending on the actual elevation), i.e. 49.3% and 61.3%. This corresponds to NLOS probabilities of 50.7% and 39.7%, respectively. Due to this significant NLOS probability, clutter loss, therefore, is non-negligible for this case and needs to be factored into the simulation. Based on TS 38.811, Table 6.6.2-2 clutter loss of 37.5 and 35.8 dB is considered for 30 and 40 degrees of elevation under NLOS conditions.

In a scenario with 90 degrees elevation, on the other hand, LOS probability based on TS 38.811, Table 6.6.1-1 is 99.2% for an urban scenario and clutter loss is virtually non-existent (NLOS probability only 0.8%) and, thus, does not significantly impact propagation in this case.

Update Ku band co-existence simulation assumptions in [1] to clarify that clutter loss and shadow fading with values for Ka band from TS 38.811 are included in the NTN propagation modeling for both calibration and co-existence simulations.

### TN Calibration Assumptions

TN calibration assumptions have been based on the way forward in [2], i.e.  ISD = 450m has been used by companies, which is later used in the TN/NTN co-existence evaluation study. However, the running document [1] states ISD = 750m in Table C.1-2 for TN calibration. The documentation of TN calibration assumptions in [1] should therefore be updated.

Update Ku band co-existence simulation assumptions in [1] to use ISD = 450m for TN calibration.

## NTN Calibration Results

### Coupling loss

Coupling loss calibration results for NTN UL and DL provided by various companies have converged and we now have good correlation between them. Different shapes of the CDF curves for coupling loss as illustrated in sections 3.1 and 3.2 above can be observed, depending on how and whether clutter loss is included in the NTN channel model.

As discussed in the previous section 5.1.1, NLOS probabilities of 50.7% and 39.7% apply for 30 and 40 degrees of elevation (nearest reference angles) apply for the NTN calibration scenario. Based on TS 38.811, Table 6.6.2-2 clutter loss of 37.5 and 35.8 dB is considered for 30 and 40 degrees of elevation under NLOS conditions.

This explains the “stretched” CDF curves for coupling loss as documented in sections 3.1 and 3.2 for NTN calibration, as the underlying PDFs show a bimodal distribution with values centred around the cases with and without clutter loss and a difference according to the constant clutter loss to be considered, i.e. 35.8 to 37.5 dB for 39.7% to 50.7% of the cases (NLOS).

In the case of 90 degrees of elevation, when clutter loss is virtually non-existent, or when clutter loss is not taken into account, a unimodal PDF would result in an S-shaped CDFs.

The observed “stretched” CDF curves for coupling loss in the NTN calibration are due to the underlying bimodal PDFs of clutter loss distributions.

### SINR Results

For NTN Calibration, FRF=1 shall be used for calibration, whereas FRF=2 is used in the co-existence analysis. For FRF=1, interference for all neighbouring beams to the centre beam has to be accounted for in SINR calibration.

Geometry SINR results for NTN calibration show good correlation among companies, as observed in sections 3.1 and 3.2. As SINR depends on coupling loss, similar effects can be observed for CDF curves dependent on the way clutter loss is factored in

The observed “stretched” CDF curves for SINR in the NTN calibration are due to the underlying bimodal PDFs of clutter loss distributions.

Further discussions have been conducted related to which UEs have to be taken into account for SINR calculations in an NTN scenario. In case UEs show very low SINR, e.g. below -10 dB, a UE would not be able to close the link with the satellite. In this case, the respective UEs may be excluded from the SINR calculations. The details may require further discussion.

Discuss which UEs to include in co-existence simulations dependent on SINR.

## ACLR/ACS Relationship

Adjacent Channel Leakage Ratio (ACLR) and Adjacent Channel Selectivity (ACS) requirements can be derived from ACIR (see also [3], [4], [5]), as the relation between ACIR, ACLR, and ACS in linear format can be expressed in the form of the following Equation 1:

|  | ACIR=11ACLR+1ACS | Equation (1) |
| --- | --- | --- |

As shown in Equation 1 above, ACIR is the combination of ACLR and ACS.  First, the ACIR is evaluating assuming 5% and 50% throughput loss.  Afterwards, once a tolerable ACIR is derived, a negotiation between different companies would commence on how this will be split between ACLR and ACS independently for uplink and downlink.

As discussed in section 4, updated ACLR and ACS results based on the calibrated NTN and TN setups are expected to be contributed by companies for this RAN4#116 meeting.

# Conclusion

The following observations and proposals have been made in this document:

Observation 1	Good correlation of NTN calibration results for GEO 45x45 UL.

Observation 2	Good correlation of NTN calibration results for LEO 45x45 UL.

Observation 3	Good correlation of NTN calibration results for GEO Parabolic UL.

Observation 4	Good correlation of NTN calibration results for LEO Parabolic UL.

Observation 5	Good correlation of NTN calibration results for GEO 45x45 DL.

Observation 6	Good correlation of NTN calibration results for LEO 45x45 DL.

Observation 7	Good correlation of NTN calibration results for GEO Parabolic DL.

Observation 8	Good correlation of NTN calibration results for LEO Parabolic DL.

Observation 9	Good correlation of TN calibration results for TN uplink.

Observation 10	Good correlation of TN calibration results for TN downlink.

Observation 11	The observed “stretched” CDF curves for coupling loss in the NTN calibration are due to the underlying bimodal PDFs of clutter loss distributions.

Observation 12	The observed “stretched” CDF curves for SINR in the NTN calibration are due to the underlying bimodal PDFs of clutter loss distributions.

Proposal 1	Companies are expected to update ACLR and ACS results based on calibrated NTN and TN setups.

Proposal 2	Update Ku band co-existence simulation assumptions in [1] to clarify that clutter loss and shadow fading with values for Ka band from TS 38.811 are included in the NTN propagation modeling for both calibration and co-existence simulations.

Proposal 3	Update Ku band co-existence simulation assumptions in [1] to use ISD = 450m for TN calibration.

Proposal 4	Discuss which UEs to include in co-existence simulations dependent on SINR.

The calibration phase of the Ku band co-existence study has progressed well, and we have obtained good correlation between results provided by various companies for NTN and TN. We can therefore proceed with the co-existence analysis.

The documentation of calibration assumptions for Ku band in TR 38.863 should be based on the actual used methodology and assumptions as discussed in section 5.1 and TR 38.863 should therefore be amended accordingly.

Further and updated Ku band ACLR and ACS results are expected for this RAN4#116 meeting by companies based on the calibrated NTN and TN setups and are discussed once available.

The TR 38.863 is then to be amended with new Ku Band ACLR and ACS values.

# References

- R4-2508774, “Ku band co-existence simulation assumptions (running document)”, Ericsson, 3GPP RAN4#115, Malta, May 2025.

- R4-2508773, “Way Forward for [115][313] NR_NTN_Ku_Band_Coexistence”, Intelsat (Moderator), 3GPP RAN4#115, Malta, May 2025.

- ECC Report 310, “Evaluation of receiver parameters and the future role of receiver characteristics in spectrum management, including in sharing and compatibility studies”, 31st Jan 2020.

- ECC Report 356, “Technical analysis to support the ECC Recommendation on receiver resilience to transmission on adjacent frequency ranges”, 10th May 2024.

- L. Sormunen, H. Martikainen, J. Puttonen and D. Panaitopol, "Co-existence of Terrestrial and Non-Terrestrial Networks on Adjacent Frequency Bands," 2022 11th Advanced Satellite Multimedia Systems Conference and the 17th Signal Processing for Space Communications Workshop (ASMS/SPSC), Graz, Austria, 2022, pp. 1-6, doi: 10.1109/ASMS/SPSC55670.2022.9914774.