3GPP TSG-RAN WG4 Meeting #116	                                                                                            R4-2509363

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on coexistence for NTN support HPUE

Agenda item:	7.6.3.1

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on coexistence study for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on coexistence study for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Tx requirements for NTN support HPUE was concluded during the previous meetings, as shown below:

Issue 3-2: Co-ex results update and related proposals in CR drafting - Proposals - Option 1: Consider to capture Ericsson’s updated simulation results in the TR 38.863 and simulation results collection document for NB-IoT NTN HPUE with 1.5 km isolation distance assumption. (Ericsson) - Option 2: Consider capturing the finding – throughput degradation vs ACLR relaxation along with the justification, in the TR 38.863 in Table 6b.5.1-2 with the following: “Note 3 – It is observed in some company co-existence results that the 5%-ile throughput degradation is slightly higher than 5% with zero km isolation distance assumption. It was however agreed to progress with this ACLR as it faciliates UE RF implementation.” (Ericsson) - Option 3-1: Clarify if the NTN UE can be used within the physical coverage of TN network or the Tx of NTN UE should be disabled when NTN UE travels into the physical locations of TN cell coverage. (Nokia) - Option 3-2: Clarify whether the requirement only applies for NTN UE outside of TN network coverage based on Zero isolation distance. Also Clarify what requirement should be applied for NTN UE if it can be used within the coverage of TN. (Nokia) Way forward: - Agree on Option 1 and 2, further refine wording in CR drafting. - Further discuss Option 3-1 and Option 3-2.

During the RAN4 #115 meeting, some companies identified the issue of NTN HPUE potentially roaming into TN cells. Normally, the UE ACLR requirements will guarantee the coexistence between devices. However, during the coexistence study, companies have observed that when the isolation distance is assumed to be 0 km, the current NTN HPUE ACLR assumption may result in some throughput degradation. Since current ACLR assumption will facilitate HPUE RF implementation, the NTN HPUE ACLR assumption has been accepted by RAN4. This raises a potential issue: from the perspective of specification, there is no other scheme to ensure coexistence between devices.

Some companies have proposed potential action on NTN HPUE behaviour. However, from a specification perspective, there does not appear to be a clear solution for controlling NTN HPUE behaviour when roaming into a TN cell. Some clarification might be helpful, such as what behaviour NTN HPUE should adopt when entering TN cells, or additional NTN UPUE requirement applicability. Anyway, there are currently no potential solutions to the above issues from a specification perspective.

Observation 1: For all NTN HPUE RF requirements related to coexistence with TN cells, ACLR is the most important requirements to be specified.

- Conclusion

This contribution provides our views on how to conduct the UE coexistence study for NR NTN HPUE. The following proposals are concluded as follows:

Observation 1: For all NTN HPUE RF requirements related to coexistence with TN cells, ACLR is the most important requirements to be specified.

- References

- R4-2508008, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #115