3GPP TSG RAN WG4 Meeting #116		R4-2509382

Bangalore, India, Aug 25th – 29th , 2025

Title: 	Finalized consideration on BS RF requirements for LP-WUS

Agenda Item:	7.24.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With approaching to the end of the work in Rel-19, discussions on BS RF requirements for LP-WUS were almost concluded in RAN4#115 where only two issues are left open [1]:

Issue 1-2: Whether/how to define EVM requirements for LP-WUS - Options - Option 1: - No BS LP-WUS EVM requirement is specified. - The background and consideration of LP-WUS transmitted signal quality are captured in TR 38.774. - Option 2: - BS LP-WUS EVM requirement as follows, no conformance test for this requirement. - For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK-4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2. Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier Modulation scheme for LP-WUS; Required EVM. OOK-4; [FFS] %. - WF - Down select the options in next meeting Issue 1-4: Restriction on the applicability of LP-WUS operation - WF - Option 1: - It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance. - Option 2 - No restriction of LP-WUS operation, this is BS implementation issue. - WF - Down select the options in next meeting

In this contribution, we present our analysis on these two open issues, and in particular, we propose a new method based on system simulations to determine the required EVM requirement for LP-WUS.

# 2 Discussion

Looking back to the history of specifying EVM requirements for different modulation schemes in 3GPP, there are two different methods which have been used for this purpose.

- Method #1: EVM values were selected using analysis by system simulation to verify that no more than a 5% performance loss would result in terms of average and cell-edge throughputs in typical deployment scenarios [2]. This method has been used from early days to determine EVM requirements for lower modulation schemes such as QPSK etc.

- Method #2: EVM values were selected using link-level simulations to justify potential gains of a higher order modulation scheme against a lower order modulation scheme, e.g., 1024QAM [3][4].

Observation 1: Two different methods have been used for selecting EVM values, one from system level performance perspective for a low order modulation scheme, the other from link level performance perspective for a high order modulation scheme.

Throughput is the main metric in both methods, however, due to the different nature of LP-WUS, it cannot act as the performance metric for determining EVM for LP-WUS, therefore the above two methods cannot be directly used.

Observation 2: The two methods cannot be directly used for determining EVM requirement for LP-WUS due to non-applicability of throughput as the performance metric.

For the same consideration, MDR is chosen as the performance metric to determine the required SNR for LP-WUR [5], and furthermore, the required SNR is set to -4.5dB for REFSENS requirement [6]. Enlightened by these considerations, one possible way is to modify Method #1 which is system-level performance based by looking at CDF at required SNR for different arbitrary EVM values, and then checking the acceptable performance loss.

Proposal 1: RAN4 to decide whether or not to introduce EVM requirement for LP-WUS based on system level instead of link level simulations.

Proposal 2: RAN4 to consider a new method to determine EVM value for LP-WUS by modifying the system-level performance based method and checking CDF loss at the required SNR for different arbitrary EVM values.

As one instance, we have conducted such system level simulations with the following main parameters:

Table – 1 Main system simulation parameters

| Parameter | Value |
| --- | --- |
| Site distance | 500 meters |
| Carrier frequency | 3.5 GHz |
| BS Channel bandwidth | 10 MHz |
| SCS | 30 kHz |
| Number of LP-WUS PRBs | 11 |
| Propagation model | UMa |
| BS height | 25 meters |
| BS transmission power | 43 dBm |
| UE height | 1.5 meters |
| UE noise figure | 11 |
| Required SNR for LP-WUS | -4.5 dB |

Fig. 1 and Fig. 2 illustrate how CDF curves look like and the corresponding cumulative probabilities at the required SNR for EVM 0% and 20% respectively. Fig. 3 shows CDF loss at the required SNR versus the change of EVM values.

By checking Fig. 3, 1% CDF loss at the required SNR compared with the case of 0% EVM would be an acceptable level.

Proposal 3: RAN4 to determine EVM value for LP-WUS at 1% CDF loss at the required SNR compared with the case of 0% EVM.

From Fig. 3, EVM value is 28.5% with 1% CDF loss at the required SNR -4.5dB, hence we propose to set EVM requirement for LP-WUS as 28.5%.

Proposal 4: RAN4 to set EVM requirement as 28.5% for LP-WUS.

Fig. 1, CDF of SINR with EVM = 0%

Fig. 2, CDF of SINR with EVM = 5%

Fig. 3, EVM Vs CDF loss at the required SNR

Regarding the issue on whether or not to restrict the applicability of LP-WUS operation with regards to a highest modulation scheme, we think that this is not necessary. If such a restriction is introduced, there could be two kinds of consequences:

- Supporting a higher order modulation scheme and LP-WUS are mutually exclusive in a static way

- Scheduling a UE with a higher order modulation scheme stops LP-WUS operation in a dynamic way

Both are unnecessary; therefore, no restriction should be introduced.

Proposal 5: No restriction of LP-WUS operation, this is BS implementation issue.

# 3 Conclusion

In this contribution, we have the following observations and proposals for BS RF requirements for LP-WUS:

Observation 1: Two different methods have been used for selecting EVM values, one from system level performance perspective for a low order modulation scheme, the other from link level performance perspective for a high order modulation scheme.

Observation 2: The two methods cannot be directly used for determining EVM requirement for LP-WUS due to non-applicability of throughput as the performance metric.

Proposal 1: RAN4 to decide whether or not to introduce EVM requirement for LP-WUS based on system level instead of link level simulations.

Proposal 2: RAN4 to consider a new method to determine EVM value for LP-WUS by modifying the system-level performance based method and checking CDF loss at the required SNR for different arbitrary EVM values.

Proposal 3: RAN4 to determine EVM value for LP-WUS at 1% CDF loss at the required SNR compared with the case of 0% EVM.

Proposal 4: RAN4 to set EVM requirement as 28.5% for LP-WUS.

Proposal 5: No restriction of LP-WUS operation, this is BS implementation issue.

# Reference

[1] R4-2508668, “WF on NR LP-WUS BS RF”, Huawei

[2] Stefania Sesia et. al., LTE – The UMTS Long Term Evolution, from theory to practice, A John Wiley and Sons, 2009

[3] R4-2108077, “WF on Link level simulation assumptions” (for DL 1024QAM for NR FR1), CATT

[4] R4-2118913, “Simulation results for 1024QAM”, Nokia

[5] R4-2414309, “WF on LP-WUS UE RF requirements”, vivo

[6] R4-2507936, “WF for [115][136] NR_LPWUS_UERF”, vivo