3GPP TSG-RAN WG4 Meeting #116														R4-2509440

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.14.3.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#115, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor.

# Discussion

In the WF, the following agreements and open issues were captured.

## Sub-topic 1-1 RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF ### Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode. ### Issue 1-1-2: Status report of fast beam sweeping Agreement: - No need to define UE status report of fast beam sweeping. ### Issue 1-1-3: UE capability for Rx BSF optimization Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Although in the WF, it was agreed that a new UAI is used to indicate overheating, it is a bit strange that the power saving aspect was not supported by UAI. As we stated in previous meetings, power saving is a good motivation for the UE to inform the network that it needs to fall back to single panel operation. In addition, if you look at how the UAI IE, i.e., “multiRx-PreferenceFR2-r18,” for R18 multi-RX reception WI was created, it does not need to explicitly capture the reason, whether it is due to overheating or power saving. Therefore, it is better to consider both overheating and power saving in UAI.

With the support of power saving by UAI, we are OK not to discuss the UE behaviour after reporting UAI for “power saving”.

Proposal 1: It is proposed to introduce a new UAI indicating either “overheating” or “power saving” issues. Like “overheating” issue, it is OK not to discuss the UE behaviour after reporting UAI for “power saving.”

On the timer-based solution, the value of T1 can considered based on the current intra-frequency and inter-frequency measurement requirements.

- Intra-frequency measurement: the measurement period is specified as max(400 ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra for the no DRX case. For PC3, Mmeas_period_w/o_gaps is 24, if SMTC period is 40ms, i.e., a value in the middle of the range of 5ms up to 160ms, each measurement period is 960ms long, which does not consider other scaling factors such as Kp, Klayer1_measurement, or CSSFintra.

- Inter-frequency measurement: the measurement period for measurements with gaps is specified as Max(400 ms, Ceil(Kgap  Mmeas_period_inter)  Max(MGRP , SMTC period))  CSSFinter for the no DRX case. For PC3, Mmeas_period_w/o_gaps is 40, if SMTC period and MGRP are both assumed to be 40ms, each measurement period is 1600ms long, which does not consider other scaling factors such as Kgap or CSSFinter.

If we consider the fact that FBS is triggered when serving cell quality is below a threshold, meaning it is likely the UE may need to speed up measurements in preparation for a possible handover soon. To assist the network in such a decision, it is reasonable to assume the UE needs to perform 3 to 5 such measurements. Therefore, the value of T1 can be considered as 3 to 5 times of measurement period. This would translate into a range of 3-10 seconds. If other scaling factors are considered, the value should be further extended. If further scaling factor of 3 is considered, the value of T1 is in the range of 10-30s.

Furthermore, UE mobility needs to be considered. For FR2, the UE is expected to be either stationary or move slowly with a speed < 15~30km/h (or 4.1~8.3m/s). Considering the maximum speed of 8.3m/s, if the value of T1 is 30s, the UE would have moved around 250m. For FR2-1, the maximum cell coverage is 1500m (based on the 5us propagation delay difference used to derive MRTD). When the UE is close to or at the cell edge, this period would mean the UE should start handover procedure or have completed the handover procedure.

Finally, the purpose of introducing the timer-based solution is to help UE save power. This means the timer T1 should not be too long.

Proposal 2: The range of T1 can be 10-30s.

UE capability

The open issue regarding UE capability is whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency). This comes from the fact that while this feature should be per-band based, it is not as straightforward how a UE would support and a network would configure the UE for the following three cases as mentioned in [2]:

- Case 1: Intra-band intra-frequency: PCell and target carrier for L3 measurement are in the same band and intra-frequency.

- Case 2: Intra-band inter-frequency: PCell and target carrier for L3 measurement are in the same band and inter-frequency.

- Case 3: Inter-band: PCell and target carrier for L3 measurement are in different bands and inter-frequency.

In our view, when the network configures the UE to perform L3 intra-frequency or inter-frequency measurements, the network would check UE’s capability and then know what UE behavior can be expected. And the network configuration and UE behavior for the above three cases are summarized in the following table:

|  | Network configuration | UE behavior |
| --- | --- | --- |
| Case 1: Intra-band intra-frequency | Whether the network configures it to the UE depends on whether UE supports this feature for the band in question. | Follow network configuration and enable FBS if it is supported. |
| Case 2: Intra-band inter-frequency | Whether the network configures it to the UE depends on whether UE supports this feature for the band in question. | Follow network configuration and enable FBS if it is supported. |
| Case 3: Inter-band | Assuming serving cell in Band A and target cell in Band B Sub-case 1: The UE supports the feature for both bands. The network configures it to the UE. Sub-case 2: The UE supports the feature for neither band. The network does not configure it to the UE. Sub-case 3: The UE supports the feature for Band A but not for Band B. The UE would support FBS in Band A, but not Band B. There is no confusion at the network side because the network knows the UE capability. Sub-case 4: The UE supports the feature for Band B but not for Band A. The UE would support FBS in Band B, but not Band A. Again, there is no confusion at the network side because the network knows the UE capability. | Follow network configuration and enable FBS for the band where it is supported. |

From the above table, no confusion is forseen.

On the other hand, it is better not to define the feature in a fragmented way. Therefore, we prefer not to define separate UE capabilities.

Proposal 3: It is preferred not to define separate UE capabilities for (intra-frequency) and (intra-frequency + inter-frequency), as there is no such a need.

Another issue is that a UE may report different BSF values for different bands, since the UE capability is per-band based. However, the current requirement, e.g., the measurement period for measurements with gaps is specified as Max(400 ms, Ceil(Kgap  Mmeas_period_inter)  Max(MGRP , SMTC period))  CSSFinter for the no DRX case, which does not differentiate between different bands. In this case, how should the requirement be specified? One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

Proposal 4: It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: It is proposed to introduce a new UAI indicating either “overheating” or “power saving” issues. Like “overheating” issue, it is OK not to discuss the UE behaviour after reporting UAI for “power saving.”

Proposal 2: The range of T1 can be 10-30s for the timer-based solution.

Proposal 3: It is preferred not to define separate UE capabilities for (intra-frequency) and (intra-frequency + inter-frequency), as there is no such a need.

Proposal 4: It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

# References

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2506702, Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, Huawei, HiSilicon