# 3GPP TSG-RAN WG4 Meeting #116	R4-2509488

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.20.6

Source:	Apple

Title:	On UE Demodulation and CSI Reporting Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 the UE demodulation and CSI reporting requirements for MIMO Phase 5 WI were discussed and way forward [1] was agreed. In this contribution we present our views on the UE demodulation and CSI reporting requirements for this WI.

# 2. Discussion

### General/ Scope

Based on the discussion in RAN4#115, the table below summarizes of scope of UE demod and CSI reporting requirements for MIMO Phase 5 WI.

| Topic | Description | UE Demod Requirements Impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | No |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | Yes – PMI reporting requirements for Type I SP |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | FFS – PMI reporting requirements for eType II and eType II-Doppler codebooks |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | No |
|  | SRS Port grouping | FFS – PDSCH demod requirements |
| CJT Calibration reporting | Aperiodic standalone CJT calibration reporting | FFS – PDSCH demod and CSI reporting requirements |
| 3TX | 3-antenna-port codebook-based UL transmission | No |
| Asymmetric DL sTRP/UL mTRP | Asymmetric DL sTRP/UL mTRP | No |

### CSI Enhancements

For CSI enhancements, the following are introduced in RAN1:

- PMI reporting for 48, 64, 128 ports for the following codebooks

  - Rel-15 Type I-SP (Single panel)

  - Rel-15 Type I-MP (multi panel)

  - Rel-16 eType II

  - Rel-17 FeType II PS (port selection)

  - Rel-18 Type II-Doppler

  - Rel-18 Type II-CJT

- CRI reporting enhancement for up to 128 ports for Type I and eType II codebooks

- Fixed SRS port group mapping for SRS antenna switching when > 4 layers/ 2 2CW is supported

Codebook enhancement

For codebook refinement for up to 128 ports it has been agreed to define PMI reporting requirements for typeI-SinglePanel-r19. And FFS if requirements are introduced for eType II and Type II-Doppler codebook for > 32 ports. It is not necessary to define requirements for all codebooks with increased number of TX. Hence, we propose not to define requirements for eType II and Type II-Doppler codebook with larger number of TX.

- It is not necessary to introduce PMI reporting requirements for all codebooks with increased number of TX ports

Do not define requirements with eType II or Type II-Doppler codebook with > 32 TX ports in Rel-19.

SRS Port grouping

Issue 1-1-9: Clarify if UE demodulation/CSI reporting requirements are needed for SRS port grouping

Way forward:

-	Option 1: RAN4 introduce PDSCH demodulation requirements for SRS port grouping

-	Option 1A: RAN4 to discuss reuse of existing demod requirements for SRS port grouping scheme.

-	Option 1B: RAN4 to discuss the new performance requirements for R19 8Rx UE supporting SRS port grouping, FFS the test setup.

-	Option 2: RAN4 do not introduce UE demodulation/CSI reporting requirements for SRS port grouping

It is FFS if requirements should be introduced for SRS port grouping. SRS port grouping feature introduces fixed port grouping for low complexity 6RX /8RX receiver for > 4 MIMO layers. To properly verify this the test setup will need to have the following –

- SRS antenna switching

- Reciprocity based SVD precoder for PDSCH

Neither of these are introduced for demod requirements so far.

We cannot reuse the exiting requirements for testing SRS port grouping in our understanding.

- For testing SRS port grouping with UE demod requirements, we would need to introduce (1) SRS antenna switching (2) Reciprocity based SVD precoder which have not been done so far for UE demodulation requirements.

- Existing requirements cannot be reused to test SRS port grouping.

We can further discuss the feasibility of the test set up and introduction of requirements to test SRS port grouping in Rel-19.

Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19.

### CJT Reporting Enhancement

For this feature RAN1 has introduced the following enhancement:

Issue 1-1-10: Clarify if UE demodulation requirements are needed for UE reporting enhancement for CJT

Way forward:

-	Option 1: RAN4 do not introduce demodulation requirements for reporting enhancements for CJT deployments

-	Option 2: RAN4 further discussion on whether to introduce new PDSCH demodulation performance requirements for PDSCH transmission Scheme C/Scheme D/Scheme E.

Issue 1-1-11: Clarify if CSI reporting requirements are needed for UE reporting enhancements for CJT

Way forward:

-	Option 1: RAN4 do not introduce CSI reporting requirements for UE reporting enhancements for CJT

-	Option 2: RAN4 introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements.

-	RAN4 do not introduce new CQI/RI and dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements.

-	Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

Requirements for CJT reporting enhancements is discussed in RRM with evaluation for number of samples for defining delay and accuracy requirements for delay and frequency offset. For phase offset reporting RRM requirements will be de-prioritized in Rel-19

Agreements in RAN4#115 for MIMO Phase 5 RRM:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting.

- RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

Introduction of RRM requirements for delay and accuracy would be sufficient to verify CJT calibration reporting feature. We don’t see the necessity to introduce a UE demod requirement for the same as the test procedure would be complicated and we still need to evaluate if there is a reasonable and significant performance difference observed in link level simulations with compensation.

- RAN4 will define RRM requirements for delay and accuracy for time and frequency offset reporting for CJT calibrtion

- RRM requirements would be sufficient to verify implementation of this feature

- UE demodulation requirements would need further study on test procedure and evaluation if performance delta is observed with compensation in link level simulations.

Do not introduce UE demodulation requirements for CJT calibration reporting in Rel-19.

# 3. Conclusion

In this paper, we present our views on the scope of UE demodulation and CSI reporting requirements for MIMO Phase 5 WI. Our observations and proposals are captured below:

- It is not necessary to introduce PMI reporting requirements for all codebooks with increased number of TX ports

- Do not define requirements with eType II or Type II-Doppler codebook with > 32 TX ports in Rel-19.

- For testing SRS port grouping with UE demod requirements, we would need to introduce (1) SRS antenna switching (2) Reciprocity based SVD precoder which have not been done so far for UE demodulation requirements.

- Existing requirements cannot be reused to test SRS port grouping.

Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19.

- RAN4 will define RRM requirements for delay and accuracy for time and frequency offset reporting for CJT calibrtion

- RRM requirements would be sufficient to verify implementation of this feature

- UE demodulation requirements would need further study on test procedure and evaluation if performance delta is observed with compensation in link level simulations.

Do not introduce UE demodulation requirements for CJT calibration reporting in Rel-19.

# Reference

- R4-2508627, “Way Forward for [115][329] NR_MIMO_demod”, Samsung.