# 3GPP TSG-RAN WG4 Meeting #116	R4-2509505

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.3.2

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#115, and the open issues have been summarized in the topic summary for [115][212] NR_RRM_Ph5_Part1 [2].

RAN4 received one LS from RAN2 (R2-2504728 Reply LS on CSSF optimization for NR RRM Phase 5), and the question from RAN2 is:

RAN2 have discussed the RRC configuration to support the CSSF optimization and reached the following agreements: Introduce the UE specific RRC configuration (per UE) to enable/disable the CSSF optimization feature. About the network indication of a specific SCC per-band for measurement, RAN2 discussed the following two alternatives: - Alternative 1: Introduce new RRC configuration to indicate which SCC to be measured. - When CSSF optimization feature is enabled, UE only performs the serving and intra-frequency neighbor measurement on the indicated SCC per band. - Alternative 2: No additional RRC signaling is needed. - When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement. - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.. Majority’s view in RAN2 is Alternative 2, but RAN2 did not reach a consensus. Therefore, RAN2 would like to ask RAN4 to decide which alternative to adopt for SCC indication and provide feedback during August meeting so that RAN2 can prepare the RRC CR for approval in August meeting.

In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

### Issue 1-1: issues related with solution 1 (measure one serving CC per band)

Reply LS to RAN2

The agreement for solution 1 was:

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

In the previous agreements, which kind of network indications can be assumed are still unclear, e.g., dynamic indication or semi-static indication. In our view, the indication to enable this CSSF enhancement can be a semi-static indication, since it will not change that often after configuring to UE. However, for network indication to measurement one SCC, we think the benefit for this R19 CSSF enhancement is obvious if dynamic indication can be used, e.g., MAC CE. It means network can configure multiple MOs for different SCCs with reporting to UE during SCell additions, and dynamic indication can allow network to guide UE measurement on different SCCs from time to time without RRC reconfiguration. The main motivation for switching the to-be-measured SCC dynamically is to monitor the interference level change (e.g., RSRQ) among the different SCCs for future SCell activation/deactivation. However, During the last meeting, the companies expressed reluctance to consider adopting the dynamic indication method. The alternative is like what were supported by majority companies in the LS, and if RRC reconfiguration is used for indication the SCell for measurement, network can also use reporting configuration to indicate which SCell shall be measured, i.e., network uses RRC reconfiguration for measurement report on a specific SCC to imply that the serving and neighbor cell on this SCC shall be measured. In that sense, we don’t think followings are valid/typical use cases:

- neither PCC nor PSCC is in band, and

- if there is no SCCs on which UE is configured to report SSB based measurements in band, or

- if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

If no SCC is configured with measurement report, it’s no way to verify which SCC is really measured, and then it’s pointless to have network indication to UE to decide which SCC shall be measured without report in the spec. If such cases happens we think majority’s view from RAN2 makes more sense and simpler for UE implementation, i.e., alt2 in RAN2 LS is preferrable.

Proposal 1: adopt Alt2: about the network indication of a specific SCC per-band for measurement: no additional RRC signaling is needed.

- When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement.

Proposal 2: the previous solution 1 shall be revised to,

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Issue 1-1-3: Expand solution 1 to FR1 case

[Wayforward]: Further discuss in next meeting: If extend for FR1: Aspect 1: co-located or non-collocated - Option 1: only limit to serving cell measurement (E///) - Option 2: cover both serving cell and neighboring cell measurements - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA. - Companies are encouraged to check the spec in this meeting. Aspect 2: Network configuration on enable/disable the feature - Option 1: the configuration is the same for different FRs - Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///) - Under the condition that the UE capability is per FR [or per band]. - Option 2 will be adopted if the solution 1 will be extended to FR1.

RAN4 agreed in #113 meeting that, after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable. In our view, the solution 1 is controlled by network, e.g., enable such CSSF enhancement and indicate which SCC to be measure in certain band, and therefore it would be also beneficial to expand it to FR1 case as well. If the channel conditions are quite different from network perspective and individual measurement shall be performed on all CCs in the same band, network can also disable such feature to UE and UE will follow the Rel-15 spec to perform serving and neighbor cell measurements. Based on the discussion in last meeting, we also agree that the intra-band contiguous case would be the most typical scenario to apply solution 1 for FR1 due to the channel similarity. Thus, we propose to expand the solution 1 to FR1 intra-band contiguous CA. In clause 7.6.4 of TS 38.133, it clearly says only co-located deployment is applied for intra-band contiguous CA. We also think this feature can be enable/disabled by network separately for FR1 and FR2, e.g., some network may assume FR1 has quite different interference level(due to load difference) within same band, network may choose to enable this feature for FR2 only.

Proposal 3: expand the solution 1 to FR1 intra-band contiguous CA.

Proposal 4: The solution 1 in FR1 assumes:

- Collocated case only

- Covering both serving cell and neighboring cell measurements

- Network configuration to enable/disable solution 1 can be different for different FRs, and the UE capability can be per-UE capability which can be differentiated between different FRs

# Detailed design for capability of CSSF enhancement

Issue 1-3-1: UE capability for CSSF enhancement option 1 (measure one carrier per band)

In last RAN4 meeting, it was agreed that,

Agreement in AdHoc: - For CSSF enhancement solution 1 in Rel-19, a new optional per-UE capability, apply for FR2-1 only. Note: the UE capability can be revisited if this feature is expanded to FR1 case.

Regarding the UE capability of solution 1, we assume UE needs a separated CSSF enhancement capability from the 3 searcher solution, and if both FR1 and FR2 are considered for solution 1, we believe it would be more flexible to allow UE indicating a per-FR capability since FR1 and FR2 probably has quite different architecture and design on the chipset. Thus, the solution 1 capability can be designed as an optional per-FR capability.

Proposal 5: to have a new optional per-UE capability which can be differentiated between different FRs for CSSF enhancement solution 1 in Rel-19.

In previous meeting, the agreements for solution 1 and solution 3 under CSSF enhancement were:

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19. Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Based on the template shared by moderator of the feature list thread, the following detailed capability design is proposed as in the annex of this paper.

Proposal 6: the detailed design for capability of CSSF enhancement can be referred to annex in this paper.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: adopt Alt2: about the network indication of a specific SCC per-band for measurement: no additional RRC signaling is needed.

- When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement.

Proposal 2: the previous solution 1 shall be revised to,

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Proposal 3: expand the solution 1 to FR1 intra-band contiguous CA.

Proposal 4: The solution 1 in FR1 assumes:

- Collocated case only

- Covering both serving cell and neighboring cell measurements

- Network configuration to enable/disable solution 1 can be different for different FRs, and the UE capability can be per-UE capability which can be differentiated between different FRs

Proposal 5: to have a new optional per-UE capability which can be differentiated between different FRs for CSSF enhancement solution 1 in Rel-19.

Proposal 6: the detailed design for capability of CSSF enhancement can be referred to annex in this paper.

# References

[1] R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#115

[2] R4-2505561, Topic summary for [115][212] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#115

# Annex – UE capability of CSSF enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | [49-x] | L3 serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap | Support of serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. Support of separate indications for FR1 and FR2-1. |  | Yes |  | UE does not support serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are [configured in a same band]. | Per-UE | No | Yes |  | UE indicating this capability shall meet the corresponding enhanced requirements in TS38.133 [Clause 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4] | Optional with capability signaling |