3GPP RAN4 Meeting #116		R4-2509570

Bengaluru, India, August 25-29, 2025

Agenda item:	7.10.3.2

Source:	Apple

Title:	On Rel-19 NTN OTA test methodology

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

During the RAN #103 meeting a new Work Item on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 was approved with the following scope related to IoT/NR NTN devices [1]:

- Study and define test methodology and radiated performance metric for FR1 NTN devices (including NR NTN and IoT NTN) - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop preliminary Measurement Uncertainty (MU) (RAN5)

Over the course of the meetings in this work item the sub-topic related NTN TRP/TRS achieved agreements captured in the following way forward documents: [2], [3], [4], and [5].  RAN4 #115 reached the following agreements [6]:

Issue 2-1-1: Details on NTN usage scenario declaration Agreements: - RAN4 to adopt the following rules for scenario declaration in NTN OTA testing: - DUT shall declare to support only one scenario - The declaration of scenario applicability shall be consistent with product manual usage scenario - The declaration of scenario shall be clearly stated in the test report Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2 Agreements: - For Scenario 2 UEs: - The DUT shall be mounted in a suitable hand phantom - The elevation tilt of the default position of the DUT can be: - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below - [±15, ±30, ±45] degrees; Other tilt angles are not precluded - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center Agreements: the partial sphere metric is using the  =0° (top pole) as a center Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS - Note1: the phi range should be 360 degrees - Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2. Issue 2-1-6: Whether Using integration of EIRP/EIS or CDF? Agreements: - Integration is baseline. - If different range is declared in issue 2-1-5, how to ensure the metric comparable should be discussed and resolved. Issue 2-2-2: Whether Dwell time is considered during NTN OTA testing Agreements: - RAN4 further discuss and align understanding of NTN antenna tuning, if any. - RAN4 further discuss whether Dwell time for NTN OTA test case is needed or not. a decision in RAN4 is needed. Issue 2-2-3: NTN MU Agreements: - For NTN OTA, there is no MU update identified. Partial sphere metric related MU may be further discussed, if needed.

Additionally, a draft CR was agreed during RAN4 #115, capturing the structure of the agreed usage scenarios and placeholder clauses for the corresponding performance metric [7].

This contribution provides our views related to this objective.

# 2	Discussion

Given the clear delineation of the scenarios applicable to NTN devices (see Issue 3-1-2 in [3]) and the agreement by RAN4 to introduce the OEM declaration (see Issue 3-1-1 in [4]), the OEM declaration of scenario applicability has been formulated for the TR, as captured in the draft CR in [7]:

The applicability of the OTA performance metric for IoT NTN and NR NTN UEs is also determined by this scenario declaration.  For handheld UEs declaring support of Scenario 1 and Scenario 3, the performance metric assumes the angles of arrival of the desired signal are likely from any direction and relative to any orientation of the device.  For this scenario RAN4 has already agreed that the full sphere TRP and TRS performance metrics apply.

Observation 1:	For Scenario 1 and 3 UEs, RAN4 has already agreed that the full-sphere TRP and TRS metrics apply.

For handheld UEs declaring support of Scenario 2, our past contribution described potential design optimizations that can be applicable [8].  Based on these observations, an argument can be made for defining the Scenario 2 radiated power and radiated sensitivity metrics as partial integrals over a subset of elevation angles, as an example, or other similar schemes.  Given this observation, it follows that the design and performance of such implementation-specific schemes can vary widely depending on form factor design, region, band, and satellite constellation.  Additional band-specific aspects, such as the impact of duplexer separation on the accuracy of such device positioning algorithms, should also be considered by RAN4.

Proposal 1:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles.

The draft CR has included a section to define applicability as follows [7]:

In our understanding, UHRP is a special case of PsRP when θ ∈ [0,180]o and φ ∈ [0,360] o.  Referring to Issue 2-1-5, our preference is Option 2 (multiple elevation angular ranges declared by the OEM), which implies that in total we have the following PsRP definitions:

UHRP: θ ∈ [-90, 90]o and φ ∈ [0, 360] o

PsPR60: θ ∈ [-60, 60]o and φ ∈ [0, 360] o

PsRP30: θ ∈ [-30, 30]o and φ ∈ [0, 360] o

Also based on the agreement from RAN4 #115 (Issue 2-1-2), we have the following possible range of tilt angles for the DUT initial positioning:

Tilt: θT ∈ [-45, -30, -15, 0, 15, 30, 45] o

Combining these aspects together, the potential OEM declaration and applicability matrix can be as follows:

| Scenario / description | OTA performance metric | Applicability of mechanical modes | Additional parameters declared by the OEM | Comments |
| --- | --- | --- | --- | --- |
| Scenario 1 both head+hand mode and hand only mode | Full sphere | Talk mode (head+hand phantom) and browsing mode (hand phantom) | None | Legacy TRP/TRS metric |
| Scenario 2 hand only mode targeted for specific UE orientation usage | UHRP or PsRP, depending on additional parameters declared by the OEM | Browsing mode (hand phantom) | DUT tilt for initial positioning is declared as one of the following values: θT ∈ [-45, -30, -15, 0, 15, 30, 45]o | New DUT positioning guidelines per Issue 2-1-2 in [6] |
|  |  |  | Elevation range: θ ∈ [-θR, θR] o, where θR is declared as one of the following values: θR ∈ [90, 60, 30]o | If the OEM declares θR=90, then the UHRP metric applies Otherwise, the corresponding PsRP metric applies |
| Scenario 3 hand only mode targeted for arbitrary UE orientation usage | Full sphere | Browsing mode (hand phantom) | None | Legacy TRP/TRS metric |

Proposal 2:	TR 38.870 should capture the OEM declaration and applicability matrix.

# 3	Conclusions

This contribution provides our views on the topic of TRP/TRS methodology development for IoT/NR NTN devices.  The following observation and proposal are made:

Observation 1:	For Scenario 1 and 3 UEs, RAN4 has already agreed that the full-sphere TRP and TRS metrics apply.

Proposal 1:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles.

Proposal 2:	TR 38.870 should capture the OEM declaration and applicability matrix.

# 4	References

RP-240841, “New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3,” Qualcomm, 3GPP RAN #103, March 2024

R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #112bis, October 2024

R4-2419808, “WF for [113][325] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #113, November 2024

R4-2502370, “Way Forward for [114][327] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #114, February 2025

R4-2505426, “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3,” vivo, 3GPP RAN4 #114bis, April 2025

R4-2508782, “Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3,” vivo, 3GPP RAN4 #115, May 2025

R4-2508779, “Draft CR to 38.870 on NTN OTA performance metric and test method,” vivo, 3GPP RAN4 #115, May 2025

R4-2417757, “On NTN TRP TRS test methodology,” Apple, 3GPP RAN4 #113, November 2024

TR38.821, “Solutions for NR to support non-terrestrial networks (NTN)”