3GPP TSG-RAN WG4 Meeting #116	R4-2509634

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM core requirements for ATG CA operation

Agenda item:	7.8.5

Document for:	Discussion

Introduction

In this contribution, we provide our views on remaining issue for ATG with DL CA operation.

Discussion

Sharing factor for Klayer1_measurement - effective non-overlapping duration

Open issue in RAN4#115: - Proposal 1: For the ‘SSB-ToMeasure and SS-RSSI-Measurement are configured’, use MRTD as the reference when calculate the margin for non-overlapping duration - Proposal 1-1: - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g., - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement - Proposal 1-2: - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured], - Klayer1_measurement=1, - - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or; - - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or; - Proposal 2: - Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement. - If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells. - If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin - Proposal 3: - reuse the legacy specification, i.e. - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

As mentioned in moderator summary [2] in the last meeting, when reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols configured by SSB-ToMeasure and SS-RSSI_Measurement, Klayer1_measurement can be 1. Additionaly, when reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cell, Klayer1_measurement can be 1 since the recived timing different from serving and neighbor cells is less than 1 symobl in TN FR2 scenario. For ATG UE with anteena array, considering up to 300km ISD, receive timing difference from serinving and neighbor cells could be much larger than 1 symbol. However, if deriveSSB-IndexFromCell is enable, it could be assumed that the timing difference between serving and neibhbor cells is less than 1 symbol. Therefore, depending on true or not for deriveSSB-IndexFromCell, following condition could be considered.

- Klayer1_measurement = 1,

  - when deriveSSB-IndexFromCell is enable for measured carrier, if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and X symbol before each consecutive SSB symbols and the RSSI symbols, and X symbol after each consecutive SSB symbols and the RSSI symbols of serving cells, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

    - when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], X=2

    - otherwise, X=1

RAN4 can consider two ways; one is not to consider effective non-overlapping symbols within SMTC window for Klayer1_measurement, other is to consider effective non-overlapping symbols within SMTC window only when deriveSSB-IndexFromCell is enable for measured carrier. We prefer the latter.

- Proposal 1: RAN4 to decide whether to define effective non-overlapping symbols within SMTC window for Klayer1_measurement

  - Alt1: not to define effective non-overlapping symbols within SMTC window for Klayer1_measurement

  - Alt2: define Klayer1_measurement

    - Klayer1_measurement = 1,

      - when deriveSSB-IndexFromCell is enable for measured carrier, if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and X symbol before each consecutive SSB symbols and the RSSI symbols, and X symbol after each consecutive SSB symbols and the RSSI symbols of serving cells, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

        - when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], X=2

        - otherwise, X=1

Scaling factor P value – when skipping neighbor SCell measurement

For ATG UE capable antennaArrayType, Navailable of the scaling factor P for measurement period for BFD, CBD, L1-SINR, and L1-RSRP for reporting is determined by the overlap of SSB/CSI-RS for BFD, CBD, L1-SINR, and L1-RSRP for reporting and MG/SMTC occasion is determined, e.g., Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W.

In the last meeting, RAN4 agreed to introduce network signaling to indicate whether an ATG UE supporting antennaArrayType-r18 on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC. Therefore, the definition of Navailable should be updated since the measurement of SSB/CSI-RS for BFD, CBD, L1-SINR, and L1-RSRP for reporting is possible even in the SCell SMTC occasions because Rx beam sweeping in SCell SMTC occasions is not performed. For this case, Navailable could be

- Navailable is

-	if the measured carrier is the SCC with servingcellMO configured, and [skippingSCCneighborCellMeas] is set to [‘enable’] by network

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or if UE capable of [two simultaneous separate Rx beams]

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion of other serving cell within the window W

The corresponding draft CR [3] for L1-RSRP for reporting is provided in our companion paper.

- Proposal 2: In case signaling for skipping neighbor cell measurement is provided by network, Navailable of scaling factor P for BFD, CBD, L1-SINR, and L1-RSRP for reporting is

- Navailable is

-	if the measured carrier is the SCC with servingcellMO configured, and [skippingSCCneighborCellMeas] is set to [‘enable’] by network

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or if UE capable of [two simultaneous separate Rx beams]

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion of other serving cell within the window W

Conclusion

In this contribution, we provide views on remaining issue for ATG with CA operation, and we propose

- Proposal 1: RAN4 to decide whether to define effective non-overlapping symbols within SMTC window for Klayer1_measurement

  - Alt1: not to define effective non-overlapping symbols within SMTC window for Klayer1_measurement

  - Alt2: define Klayer1_measurement

    - Klayer1_measurement = 1,

      - when deriveSSB-IndexFromCell is enable for measured carrier, if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and X symbol before each consecutive SSB symbols and the RSSI symbols, and X symbol after each consecutive SSB symbols and the RSSI symbols of serving cells, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

        - when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], X=2

        - otherwise, X=1

- Proposal 2: In case signaling for skipping neighbor cell measurement is provided by network, Navailable of scaling factor P for BFD, CBD, L1-SINR, and L1-RSRP for reporting is

- Navailable is

-	if the measured carrier is the SCC with servingcellMO configured, and [skippingSCCneighborCellMeas] is set to [‘enable’] by network

-	if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or if UE capable of [two simultaneous separate Rx beams]

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	otherwise,

-	the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion of other serving cell within the window W

Reference

[1] R4-2508263, “WF on RRM requirements for NR_ATG_enh,” CMCC

[2] R4-2505560, “Topic summary for [115][211] NR_ATG_enh,” CMCC

[3] R4-2509635, “Draft CR on L1-RSRP measurements for ATG CA requirements (Clause 9.5D),” LG Electronics